AMCP WORKING GROUP F mEETING

(Montreal, Canada, August 18-29, 2003)

Agenda Item 7:  Universal Access Transceiver (UAT)

Agenda Item 5:  5 GHz Band Issues

- CONSIDER ADDITIONAL ALLOCATIONS FOR THE AERONAUTICAL MOBILE (R) SERVICE IN PARTS OF THE BANDS BETWEEN 108 MHZ AND 6 GHZ

(Presented by the United States of America)

SUMMARY This paper presents aviation’s opportunity to confirm spectrum allocations for next-generation aviation systems. Of particular interest to the United States are the universal access transceiver (UAT), the airport network and location equipment (ANLE), and aeronautical fixed links.

- INTRODUCTION

  - At the 2003 World Radiocommunication Conference (WRC), the agenda for the 2007 WRC was determined.  Item 1.6 of that agenda is:

1.6	to consider additional allocations for the aeronautical mobile (R) service in parts of the bands between 108 MHz and 6 GHz, in accordance with Resolution [COM7/6] (WRC-03) and, to study current satellite frequency allocations, that will support the modernization of civil aviation telecommunication systems, taking into account Resolution [COM7/7] (WRC-03);

This agenda item, and the studies referenced in the associated resolutions (see attachments 1-4), offer aviation an opportunity to confirm spectrum allocations for next-generation aviation systems.  Of particular interest are the universal access transceiver (UAT) and the airport network and location equipment (ANLE).

- DISCUSSION

  - Universal Access Transceiver (UAT)

    - The primary function of the UAT is to support civil aviation goals of providing more situational awareness to the pilot.  In particular, UAT facilitates Automatic Dependent Surveillance – Broadcast (ADS-B), a surveillance architecture in which participating aircraft periodically broadcast their position and intent information to other aircraft and/or ground facilities.  In the UAT, this has been accomplished via a high-capacity (on the order of one megabits/second) data-link that is specifically designed to operate in the potentially high-pulsed environment expected in its design frequency range.

    - In particular, the UAT has been designed to operate at 978 MHz, in the lower portion of the 960-1215 MHz band.  That portion is currently allocated internationally for Aeronautical Radionavigation Service (ARNS) and for “airborne electronic aids to air navigation and any directly associated ground-based facilities” (S5.328).  The band is also used in the United States (US) and many other countries for the Joint Tactical Information Distribution System/Multifunctional Information Distribution System (JTIDS/MIDS) on a non-interference basis (e.g., via US224 in the US).  The UAT is currently undergoing the International Civil Aviation Organization (ICAO) Standards and Recommended Practices (SARPs) process under the auspices of a subgroup of the Aeronautical Communications Panel (ACP) Working Group C.

    - In the US, Federal Government use of radio spectrum is managed by the National Telecommunications and Information Administration (NTIA).  In the NTIA Manual of Regulations and Procedures for Federal Radio Frequency Management, it is stated that “Government agencies planning the use of, conducting experiments related to, or developing and procuring telecommunications requiring the use of radio frequencies shall take all reasonable measures to ensure that such systems will neither cause nor receive harmful interference to or from other authorized users when placed in their intended operational environment.”  This requirement is further reinforced by the Office of Management and Budget Circular No. A-11 which specifies that agencies “must obtain certification by the National Telecommunications and Information Administration, Department of Commerce that the radio frequency is available before you submit estimates for the development or procurement of major communications-electronics systems”.

    - NTIA certification of spectrum support is granted in four stages as the system under consideration matures.  These stages are:  Stage 1:  Conceptual, Stage 2:  Experimental, Stage 3:  Developmental, and Stage 4:  Operational.  Systems must fully meet the Circular A-11 “certification” requirement before they achieve Stage 4.  The UAT message and equipment standards have been specifically tailored to ensure it would operate compatibly in its intended environment.  As a direct result of that approach, NTIA granted Stage 4 certification to the UAT on April 16, 2003.  Government agencies can now procure and use UAT in the United States.  Consistent with current rules for the 960-1215 MHz band, the Federal Aviation Administration will administer such use, including the assignment of ground station uplink slots to preclude self-interference.

    - It must also be recognized however that the 960-1215 MHz band is shared between government and non-government users.  Consistent with existing practice, non-government use of the band is regulated by the Federal Communications Commission (FCC).  In the recent proceeding to update Part 87 (aviation systems) of the FCC rules, submissions were made to enable UAT use of the band.  As that Proceeding is not yet complete, it is expected that near-term use of the band by UAT will require some form of “blanket waiver”.  The FAA is currently initiating discussions with the FCC to facilitate that process.

    - Spectrum support for the UAT in the US was predicated on a decision by the regulatory authorities that as an ADS-B system, UAT was an “aid to navigation”, and authorized to use the band under footnote 5.328.  Other State representatives have provided preliminary indications that their regulators do not concur with the US assessment.  If those indications should prove true, ICAO should take advantage of the opportunity offered by agenda item 1.6 to facilitate international acceptance of UAT.  In addition, it may be beneficial to examine the utility of allowing aeronautical mobile (route) communications in the 960-977 MHz portion of the band – a portion currently not utilized by ICAO-standard ARNS systems.

  - Airport Network And Location Equipment (ANLE)

    - In its most basic form, ANLE is a high integrity, “safety rated” wireless local area network (LAN) for the airport area, combined with a connected grid of multilateration sensors.  The former would provide the cockpit with access to appropriate information via a high-bandwidth internet-like connection.  The latter would use those same transmissions to derive 3-dimensional position of the terminal – position that could then be broadcast via the same data link to provide all users with situational awareness on the airport surface.  Adding simple transmitters to other surface-movement vehicles would allow for the development of a high-fidelity complete picture of the airport surface environment.

    - As noted in the referenced Resolutions, current attention has focused on fielding an ANLE-type system in the 5091-5150 MHz band.  The frequency band 5000-5250 MHz is allocated on an international basis to the aeronautical radionavigation service (ARNS).  Currently only the 5030-5150 MHz portion has a defined ARNS function; namely the microwave landing system (MLS), with only the 5030-5091 MHz portion containing defined MLS channels.  As a result, the aviation community is exploring other applications in the 5091-5150 MHz band, and defining uses for the 5150-5250 MHz band.

    - Additional considerations:

- The 5000-5250 MHz band is currently protected for ARNS, although it would likely not be feasible to implement an aeronautical safety system in the 5150-5250 MHz segment of this band due to other incompatible services

- The 5 GHz band is not well suited for long-range air-ground applications due to the high level of power required in the aircraft to overcome atmospheric attenuation.  It is suitable however for short-range applications such as those associated with an airport movement area.  In addition, it is possible that short-range air-ground communications, for example those currently being studied by Eurocontrol, could also be accommodated.

- 5GHz LANs are currently commercially available (though not to the specifications proposed for ANLE).  As a result, with a properly implemented priority/preemption/precedence scheme for the pilot, it may be possible to have a system that is shared by both the cockpit and commercial (e.g., internet access) users in the terminal area.  Such sharing could result in very cost-effective equipment for the aviation community, and speed regulatory acceptance of such a proposal.

The FAA is currently trying to arrange proof-of-concept testing in conjunction with the National Aeronautics and Space Administration (NASA), currently slated for 2004.  In addition, we are assessing the viability of using Matlab-based computer models to assess system viability.

  - Aeronautical Fixed Links

    - Another application being considered for the 5091-5150 MHz band, is the provision of aeronautical fixed service links.  These links would be utilized to support short-range data transfer between sensors/systems, precluding the need to disturb airport tarmac for cabling.  While it is possible that such sensors/systems could be nodes in an airport network such as ANLE, it may prove advantageous to reserve a portion of the spectrum for less-complex stations.

- CONCLUSION/RECOMMENDATION

  - In conclusion, aviation should take the opportunity offered by the studies called for in the 2007 WRC agenda, to determine if future navigation/surveillance/communication systems can be accommodated within the bands currently available for use by aeronautical systems as a first step without placing undue constraints to services to which the frequency bands are currently allocated.  In case the first step above would not lead to satisfactory results, study the frequency bands currently not available for use by aeronautical systems, subject to not constraining the existing and planned use of the bands, taking account of existing use and future requirements in these bands.

  - The Working Group is invited to note this information, and to support the tasking of ACP Working Group F with coordinating ICAO efforts in support of the referenced International Telecommunication Union (ITU) studies.

Attachment 1

RESOLUTION  [COM7/6]  (WRC-03)

Consideration of the frequency range between 108 MHz and 6 GHz 
for new aeronautical applications

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that new technologies are needed to support communications and air navigation, including airborne and ground surveillance applications;

b)	that the new technologies to support air navigation in considering a) may not conform to the definition of aeronautical radionavigation in the Radio Regulations;

c)	that the current aeronautical mobile band from 117.975-137 MHz will become saturated in some areas of the world and will no longer be able to support the increasing and new requirements;

d)	that new aviation security requirements are currently being defined internationally;

e)	that new applications and concepts in air traffic management which are data intensive are envisioned;

f)	that proposals to meet two current aviation safety objectives, i.e. to provide more information to the pilot and cockpit, and to reduce runway incursions, are being considered in the band 5 091-5 150 MHz;

g)	that there are requirements to support data links that carry critical aeronautical data from systems such as air traffic control radars, wind shear radars, remote maintenance monitoring systems, runway lighting, low-level wind shear alert systems, automated weather surface observing systems, or automatic weather observation systems,

recognizing

that it may not be feasible to introduce some of these new aviation applications in certain bands used by aeronautical safety communications, radionavigation and surveillance due to the possible incompatibility problems or possible spectrum congestion,

noting

a)	that past experience has shown that the range 108 MHz to 6 GHz is suitable for aviation requirements;

b)	that Resolutions 114 (Rev.WRC-03) and [COM5/2] call for compatibility studies in the relevant aeronautical bands,

resolves

that WRC-07 considers additional allocations for the aeronautical mobile (R) service in parts of the bands between 108 MHz and 6 GHz, taking into account considering c) to g) above,

further resolves to invite ITU-R

1	to investigate, as a first step, the bands currently available for use by aeronautical systems in the frequency range between 108 MHz and 6 GHz in order to determine whether additional allocations to the aeronautical mobile (R) service are required and can be accommodated in these bands without placing undue constraints to services to which the frequency bands are currently allocated;

2	to further investigate, in case the first step above would not lead to satisfactory results, also the frequency bands currently not available for use by aeronautical systems, subject to not constraining the existing and planned use of such bands, taking account of existing use and future requirements in these bands;

3	to investigate how to accommodate the requirements for aeronautical systems in the band 5 091-5 150 MHz,

further invites

all members of the Radiocommunications Sector and especially the International Civil Aviation Organization (ICAO) to contribute to these studies,

requests the Secretary-General

to bring this Resolution to the attention of ICAO.

ADD	COM7/353/8	(B13/361/8)	(R12/397/7)

attachment 2

RESOLUTION  [COM7/7]  (WRC-03)

Study of current satellite frequency allocations that will support 
the modernization of civil aviation telecommunication systems

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that some developing countries still lack an appropriate communication infrastructure that meets the evolving requirements of modern civil aviation;

b)	that the cost of providing and maintaining such an infrastructure, in particular a terrestrial infrastructure, is increasingly expensive, particularly in remote regions;

c)	that satellite communication systems provide a real possibility to meet the demands of the International Civil Aviation Organization (ICAO) communication, navigation, surveillance and air traffic management (CNS/ATM), especially in the areas where a terrestrial communication infrastructure is not available,

further considering

a)	that allocations to aeronautical services and for applications exist over a broad range of frequency bands;

b)	that technologies now exist where different services can be accommodated by efficient use of a single spacecraft or satellite network;

c)	that the benefits of establishing and utilizing satellite communication systems for civil aviation would also bring additional benefits for developing and sparsely populated countries by enabling the development of telecommunication systems in conjunction with the civil aviation systems,

noting

1	that Recommendation 34 (WRC-95) states that future world radiocommunication conferences, whenever possible, should allocate frequency bands to the most broadly defined services with a view to providing maximum flexibility in spectrum use;

2	that Resolution 20 (Rev.WRC-2000) resolves to instruct the Secretary-General “to encourage ICAO to continue its assistance to developing countries which are endeavouring to improve their aeronautical telecommunications”;

3	that there is a need to provide long-term spectrum availability for aeronautical satellite communications for safety purposes,

resolves to invite WRC-07

1	to examine the possibility of broadening the services and applications of the use of current satellite frequency allocations in order to allow the expansion of ICAO CNS/ATM systems that can also support other non-aeronautical telecommunication services;

2	to take appropriate actions, based on the results of the examination specified under resolves 1,

invites ITU-R

1	to study, as a matter of urgency, the current satellite frequency allocations that could meet aeronautical requirements to support the modernization of civil aviation telecommunication systems, especially those in developing countries, and to study in particular those radio frequencies that could be used to support both ICAO CNS/ATM systems and other non-aeronautical telecommunication services,

further invites

1	BDT to also examine this issue and provide assistance, as appropriate, that would facilitate developing countries to participate in the work of ITU-R on this matter;

2	ICAO, the International Air Transport Association (IATA), administrations and other organizations concerned to participate in the studies identified in invites ITU-R above,

requests the Secretary-General

to bring this Resolution to the attention of ICAO.

ADD	COM7/353/9	(B13/361/9)	(R12/397/8)

Attachment 3

RESOLUTION  [COM5/2]  (WRC-03)

Use of the band 108-117.975 MHz by aeronautical services

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	the current allocation of the frequency band 108-117.975 MHz to the aeronautical radionavigation service;

b)	the current requirements of FM broadcasting systems operating in the frequency band 87-108 MHz;

c)	that digital sound broadcasting systems are capable of operating in the frequency band at about 87-108 MHz as described in Recommendation ITU-R BS.1114;

d)	the need for the aeronautical community to provide additional services by enhancing navigation and surveillance systems through a communication data link;

e)	the need for the broadcasting community to provide digital terrestrial sound broadcasting services,

recognizing

a)	that precedence must be given to the aeronautical radionavigation service operating in the frequency band 108-117.975 MHz;

b)	that, in accordance with Annex 10 of the Convention of the International Civil Aviation Organization (ICAO) on international civil aviation, all aeronautical systems must meet standards and recommended practices (SARPs) requirements;

c)	that within ITU-R, compatibility criteria between FM broadcasting systems operating in the frequency band 87-108 MHz and the aeronautical radionavigation service operating in the frequency band 108-117.975 MHz already exist, as indicated in the most recent version of Recommendation ITU-R SM.1009;

d)	that all compatibility issues between FM broadcasting systems and ICAO standard ground-based systems for the transmission of radionavigation-satellite differential correction signals have been addressed,

noting

a)	that aeronautical systems are converging towards a communication data link environment to support aeronautical navigation and surveillance functions, which need to be accommodated in existing radio spectrum;

b)	that some administrations are planning to introduce digital sound broadcasting systems in the frequency band at about 87-108 MHz;

c)	that no compatibility criteria currently exist between FM broadcasting systems operating in the frequency band 87-108 MHz and the planned additional aeronautical systems in the adjacent band 108-117.975 MHz using aircraft transmission;

d)	that no compatibility criteria currently exist between digital sound broadcasting systems capable of operating in the frequency band at about 87-108 MHz and aeronautical services in the band 108-117.975 MHz;

e)	that surveillance functions include the observation of aircraft location and velocity, and weather conditions for the purpose of air traffic control and situational awareness/collision avoidance between aircraft,

resolves

1	that the provisions of this Resolution and of No. 5.BA03 shall enter into force on 5 July 2003;

2	that any additional aeronautical systems1 planned to operate in the frequency band 108-117.975 MHz shall, as a minimum, meet the FM broadcasting immunity requirements contained in Annex 10 of the ICAO Convention on International Civil Aviation for existing aeronautical radionavigation systems operating in this frequency band;

3	that additional aeronautical systems operating in the band 108-117.975 MHz shall place no additional constraints on the broadcasting service or cause harmful interference to stations operating in the bands allocated to the broadcasting service in the frequency band 87-108 MHz and No. 5.43 does not apply to systems identified in recognizing d);

4	that frequencies below 112 MHz shall not be used for these additional aeronautical systems excluding the ICAO systems identified in recognizing d) until all potential compatibility issues with the lower adjacent frequency band 87-108 MHz have been resolved,

invites ITU-R

to study any compatibility issues between the broadcasting and aeronautical services that may arise from the introduction of these additional aeronautical systems as referenced in noting a), or appropriate digital sound broadcasting systems, as described in Recommendation ITU-R BS.1114 and to develop new or revised ITU-R Recommendations as appropriate,

instructs the Secretary-General

to bring this Resolution to the attention of ICAO.

ADD           COM5/209/14            (B3/239/26)            (R2/297/98)

Attachment 4

RESOLUTION  114  (Rev.WRC-03)

Studies on compatibility between new systems of the aeronautical radionavigation service and the fixed-satellite service (Earth-to-space) 
(limited to feeder links of the non-geostationary mobile-satellite systems 
in the mobile-satellite service) in the frequency band 5 091-5 150 MHz

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	the current allocation of the frequency band 5 000-5 250 MHz to the aeronautical radionavigation service;

b)	the requirements of both the aeronautical radionavigation and the fixed-satellite (Earth-to-space) (limited to feeder links of non-geostationary satellite (non-GSO) systems in the mobile-satellite service (MSS)) services in the above-mentioned band,

recognizing

a)	that precedence must be given to the microwave landing system (MLS) in accordance with No. 5.444 of the Radio Regulations and to other international standard systems of the aeronautical radionavigation service in the frequency band 5 030-5 150 MHz;

b)	that, in accordance with Annex 10 of the Convention of the International Civil Aviation Organization (ICAO) on international civil aviation, it may be necessary to use the frequency band 5 091-5 150 MHz for the MLS if its requirements cannot be satisfied in the frequency band 5 030-5 091 MHz;

c)	that the fixed-satellite service providing feeder links for non-GSO systems in the MSS will need access to the frequency band 5 091-5 150 MHz in the short term,

noting

a)	that Recommendation ITU-R S.1342 describes a method for determining coordination distances between international standard MLS stations operating in the band 5 030-5 091 MHz and fixed-satellite service earth stations providing Earth-to-space feeder links in the band 5 091-5 150 MHz;

b)	the small number of fixed-satellite service stations to be considered;

c)	the development of new systems that will provide supplemental navigation information integral to the aeronautical radionavigation service,

resolves

1	that administrations authorizing stations providing feeder links for non-GSO systems in the MSS in the frequency band 5 091-5 150 MHz shall ensure that they do not cause harmful interference to stations of the aeronautical radionavigation service;

2	that the allocation to the aeronautical radionavigation service and the fixed-satellite service in the frequency band 5 091-5 150 MHz should be reviewed at a future competent conference prior to 2018;

3	that studies be undertaken on compatibility between new systems of the aeronautical radionavigation service and systems of the fixed-satellite service providing feeder links of the non-GSO systems in the MSS (Earth-to-space),

invites administrations

when assigning frequencies in the band 5 091-5 150 MHz before 1 January 2018 to stations of the aeronautical radionavigation service or to stations of the fixed-satellite service providing feeder links of the non-GSO systems in the MSS (Earth-to-space), to take all practicable steps to avoid mutual interference between them,

invites ITU-R

to study the technical and operational issues relating to sharing of this band between new systems of the aeronautical radionavigation service and the fixed-satellite service providing feeder links of the non-GSO systems in the MSS (Earth-to-space),

invites

1	ICAO to supply technical and operational criteria suitable for sharing studies for new aeronautical systems;

2	all Members of the Radiocommunication Sector, and especially ICAO, to participate actively in such studies,

instructs the Secretary-General

to bring this resolution to the attention of ICAO.

MOD           COM5/303/15            (B7/324/11)            (R4/351/120)

1 	In the context of this Resolution, the term “additional aeronautical systems” refers to systems that transmit navigational information in support of air navigation and surveillance functions in accordance with recognized international aviation standards.