|  | International Civil Aviation Organization INFORMATION PAPER | ACP-WGF18/IP -06 12-22 May 2008. |
| --- | --- | --- |

AERONAUTICAL COMMUNICATIONS PANEL (ACP)

16th MEETING OF WORKING GROUP F

Montreal, Canada 12 – 22 May 2008

| Agenda Item 3: | Development of ICAO position for WRC-11 |
| --- | --- |

UK Progress with Respect to the Development of Spectrum Requirements for Un-manned Aerial Systems

(Presented by John Mettrop)

SUMMARY

This paper presents information on the progress made within the UK on defining the spectrum requirement for Unmanned Aerial Systems

ACTION

The meeting is invited to note the information paper and consider whether it is appropriate to use a similar methodology in defining their UAS spectrum requirement.

- INTRODUCTION

  - WRC-07 agreed on a future Conference Agenda Item for WRC-11 to investigate the requirements for and additional spectrum allocations to meet the expected demands of Un-manned Aerial Systems (UAS).  This agenda item was given to ITU Working Party 5B to progress.  In their first meeting of this study cycle Working Party 5B identified a work schedule that would have to be met in order for the work on identifying additional spectrum requirements to be complete in time for WRC-11.  This work schedule calls for the identification of the amount of spectrum required to have been identified by the second meeting of Working Party 5B in October 2008.

  - The scope of the agenda item enables appropriate spectrum to be identified and allocated, where necessary, to support both the command and control as well as the sense and avoid functionality of an UAS. This paper reports on the progress made within the UK on defining a methodology for the identification of UAS spectrum needs.

- discussion

  - In the UK we are attempting to define the spectrum requirement for UAS from the perspective of the information required, message size, update rate and density.  Knowing these parameters for each type of UAS a queuing model can then be used to simulate the traffic and estimate the amount of spectrum required.

  - The types of message and message length are based on ARINC standards and defined for the various phases of flight appropriate to that type of UAS in both normal and failure mode of flight (e.g for a transatlantic flight these would be airport surface, airport/TMA, en-route and oceanic/polar).  These messages are then prioritised into one of four levels according the time period within which a message must get through and the update rate defined.  An example of the type of this is attached.

  - Having defined the message types, message length, update rate and time period for a message to get through a queuing model is then used to calculate the amount of spectrum required to satisfy the throughput requirements.  This then defines the spectral requirement for each UAS in both normal and failure mode for each different type of UAS.  Dependent on the number of different types of UAS for which figures have been defined it may be necessary to then group similar UAS and average their bandwidth requirements.

  - Using the density figure for each type of UAS, the type of spectrum it will require, and making some assumptions about the coverage area of either a group station or satellite spot beam as well as the reuse rate the total spectrum requirement can then be estimated.

  - Currently the attached spreadsheet has been sent out to those within the UK who are interested in UAS for them to complete for the type of UAS they plan on producing.  These figures are due to be completed by the 13th June so that a combined list can be prepared for the CEPT PT C meeting in late June which hopefully will also have a first approximation of the overall spectrum requirement.  Work will then continue between June and September to further refine the model.

  - One decision that has been made is that for aircraft in the cruise plot extracted rather than raw video will be sent, if necessary, from the aircraft to the ground controller.  This decision was taken based on the practicality of flying a UAS via a video sent via a satellite link as well as the overall bandwidth requirement.  Raw video might however be necessary in a localised area such as an airport for either landing the aircraft when the landing aid fails or for taxing and avoiding obstacles.

- Action by the meeting

  - The meeting is asked to note the information provided and use it as required, any contribution to the process would be welcome

- Attachment

Note