|  | INTERNATIONAL TELECOMMUNICATION UNION | AMCP WGF7/WP4 |
| --- | --- | --- |
|  | RADIOCOMMUNICATION STUDY GROUPS | Document 8D/TEMP/113(Rev.1)-E Document 8B/TEMP/93-E* 1 November 2001 English only |

Received:	9 October 2001

Source:	Documents 8D/161 and 8D/197

Working Party 8D

draft new recommendation

Sharing between stations of the radionavigation-satellite service
(Earth-to-space) and radar of the radiolocation service and
the aeronautical radionavigation service in 
the frequency band 1 300-1 350 MHz

Summary

This Recommendation gives a methodology for selecting the location of RNSS uplink stations in the 1 300-1 350 MHz band, in order to assist their coordination with radiolocation and aeronautical radionavigation radar systems.

The ITU Radiocommunication Assembly,

considering

a)	that the band 1 300-1 350 MHz is allocated on a primary basis to the aeronautical radionavigation service for use by ground-based radar systems;

b)	that WRC-2000 (Istanbul) has added a primary allocation to the radionavigation-satellite service (Earth-to-space) in the 1 300-1 350 MHz band;

c)	that WRC-2000 has raised the status of the radiolocation service from secondary to primary in the 1 300-1 350 MHz band;

d) 	that the allocation to the radiolocation service is used by terrestrial as well as airborne radar systems;

e)	that there is a potential for interference between uplink stations in the radionavigation-satellite service and radar systems of the aeronautical radionavigation and radiolocation services;

f)	that radar systems of the aeronautical radionavigation and radiolocation services can be protected with the implementation of adequate separation distances;

g)	that a limited number of ground-based beacons in the radionavigation-satellite service are expected to be deployed globally,

h)	that Appendix S7 shall be used to determine the affected administrations for the coordination of specific RNSS earth stations in the Earth-to-space direction under S9.17,

recognizing

a)	that WRC-2000 added S5.337A stating the use of the band 1 300-1 350 MHz by earth stations in the radionavigation-satellite service and by stations in the radiolocation service shall not cause harmful interference to, nor constrain the operation and development of, the aeronautical-radionavigation service,

b)	that operational and practical difficulties exist in coordinating RNSS uplink stations and airborne radars,

recommends

1	that the methodologies in Annexes 1 and 2 be taken into account, when selecting the location of RNSS uplink stations in the range 1 300-1 350 MHz, in order to assist their coordination with radiolocation and aeronautical radionavigation radar systems.

ANNEX 1

Sharing between stations of the radionavigation-satellite service (Earth-to-space) and terrestrial radar systems of the radiolocation and aeronautical radionavigation service in the frequency band 1 300-1 350 MHz

# 1	Introduction

New radionavigation satellite systems will use the band 1 300-1 350 MHz for the transmission by uplink stations of information such as navigation, synchronization or integrity data, to a constellation of medium Earth-orbiting satellites.

This study provides an analysis of

the interference created by uplink stations into receiving terrestrial radar.

It results from the study that a separation distance permits to avoid excess interference into terrestrial radar.

# 2	Systems characteristics

## 2.1	Radiolocation and aeronautical radionavigation radars

The radar parameters used in this document are those given in Recommendation ITU-R M.1463, "Characteristics and protection criteria for radars operating in the radiodetermination service in the frequency band 1 215-1 400 MHz", covering radars of the radiolocation and aeronautical radionavigation service.

In order to calculate radar perturbation thresholds, the following formula and a –6 dB protection criteria (I/N) as given in Recommendation ITU-R M.1463 have been used:

P threshold (dBm)=10log(KT0FB)+24

Table 1

Radar-receiving parameters

|  | Bandwidth (MHz) | Reception antenna gain (dBi) | Perturbation threshold (dBm) |
| --- | --- | --- | --- |
| System 1 | 0.780 | 33.5 | 119.1 |
| System 2 | 0.690 | 38.9 | 119.6 |
| System 3 | 4.4, 6.4 | 38.2 | 108.8, 107.2 |
| System 4 | 1.2 | 32.5 | 115.7 |
| Wind profile radars | 2.5 | 33.5 | 114.5 |

## 2.2	Typical RNSS radio uplink stations

–	Transmitted power:	57.1 dBm

–	Antenna type:	omnidirectional with a physical isolation for low elevation (typically 50 dB attenuation with a choke-ring (see Annex 3 for the description of a choke ring))

–	Orientation:		zenith

–	Maximum gain:	<3 dBi

–	Gain for elevation:	below 10°       1 dBi

–	Modulation:		spread spectrum (1.023 and 10.23 Mchips/s)

–	Polarization:	LHCP

–	Height:			2 m

–	Network:		less than 20 uplink stations regularly spaced around the world. Each uplink station transmits on the same frequency to a constellation of medium Earth-orbiting satellites.

The total power is used to transmit  two signals, one spread with a 1.023 Mchips/s code and the other spread with a 10.23 Mchips/s code. While the 10.23 Mchips/s code signal has a power of 53 dBm, the 1.023 Mchips/s has a power of 55 dBm. As a worst-case illustration and for the purpose of this study, frequency of the uplink stations is taken as the same as the radar. However, the impact of a frequency shift is studied.

## 2.3	RNSS satellite receiver

–	Modulation:		spread spectrum (1.023 and 10.23 Mchips/s)

–	Polarization:	LHCP (TBC)

–	Minimal power received:	126 dBm

–	Receiver antenna gain:	0 dBi

–	RF filter 3 dB bandwidth:	±15 MHz

–	Front end amplifier saturation level:	40 dBm

# 3	Interference of RNSS uplink stations into radar

## 3.1	Compatibility study

In order to assess the separation distance necessary to protect radar reception, the propagation loss is calculated as:

L = Pt + Gt  At  FLt+ Gr  FLr + Rb  Dpol – Pthreshold
		= P interfering – Pthreshold	(1)

where:

Pt:	transmitting interfering power (dBm)

Gt:	transmitting interfering gain (dBi) in the radar direction

At:	physical isolation at low elevation (due to choke ring) (dB)

FLt:	transmitting feeder losses (dB)

Pthreshold:	perturbation threshold (dBm)

Gr:	reception gain (dBi)

FLr:	reception feeder losses (dB)

Rb:	rejection factor (dB)

Dpol:	polarization coupling factor (dB).

The rejection factor Rb represents the amount of the RNSS emission total power which is filtered by the radar receiver. Thus, it takes into account the radar receiver bandwidth, the frequency offset between the radar and the RNSS uplink emission central frequencies and the RNSS signal NPSD, Normalized Power Spectral Density. For a square BPSK modulation (expected for RNSS):

Rb=NPSD(f)df=f0R−Br2f0R+Br21fc⋅sinc²((f−f0T)fc)df=f0R−f0T−Br2f0R−f0T+Br21fc⋅sinc²(ffc)df	(2)

where:

Br:	reception bandwidth

It is to be noticed that the slow code (code rate = 1.023 Mchips/s) is also a short code (1 023 chips). Consequently, the spectrum of the corresponding signal has 1 kHz line components. Some of these line components have a power level greater than the (sinc x)2 values, but the average of lines keeps approximately a (sinc x)2 shape. Taking into account that the reception bandwidth is large with respect to the 1 kHz intervals, a great number of line components are averaged and formula (2) is adequate to compute Rb.

Following the determination of L, one can evaluate the corresponding separation distance. This is done through Recommendation ITU-R P.452-8, "Prediction procedure for the evaluation of the microwave interference between stations on the surface of the Earth at frequencies above about 0.7 GHz", as well as Recommendation ITU-R P.526-5, "Propagation by diffraction". The method proposed in Recommendation ITU-R P.452-8, Table 5, has been used in order to derive the overall prediction taking into account the path type (line-of-sight, line-of-sight with sub-path diffraction or trans-horizon).

In the table hereafter are given the main hypotheses that have been taken into account in the application of the above Recommendations:

Table 2

Main hypotheses for protection distances calculations

| Model | Parameter | Value | Comment |
| --- | --- | --- | --- |
| Tropospheric scatter | Path centre sea-level refractivity: No | 360 | Compromise among all the continents' worst values. |
| Ducting/layer reflection | Over-sea surface duct coupling corrections for the interfering and the interfered-with stations (Act, Acr) | 0 dB | It is assumed that the distance from each terminal to the coast along the great-circle interference-path is more than 5 km. |
|  | Terrain roughness parameter, that is the maximum height of the terrain above the smooth Earth surface (hm) | <10 m | We assume that the Earth's surface is smooth. |
| Additional clutter losses | Additional losses due to local ground clutters such as buildings, vegetation,… (Aht, Ahr) | 0 dB | We assume the worst case that there are no local ground obstacles between the interfering and the interfered-with stations. |
| All | Refractive index lapse rate over the first 1 Km of the atmosphere (N) | 80 | Worst case value. |
| All | Required time percentage for which the calculated basic transmission loss is not exceeded: p | 1% |  |
| All | Median effective Earth Radius Factor (K50) | 2.0389 | Obtained with N = 80 |

## 3.2	Calculation of separation distance beyond which protection is assured (protection distance)

Formula (1) is applied for all radars given in Recommendation ITU-R M.1463, for both worst-case co-frequency operations and for 3 MHz frequency off-set operation between the centre frequency of the radar and the RNSS emission. In Attachment 1 of this Annex are given the tables with the required loss calculations in order to protect each type of radar.

The methodology in Recommendation ITU-R P.452-8, as mentioned in § 3.1, is applied in order to calculate the protection distances from the required loss. In the figures below are given some examples of the results in terms of protection distances for several heights (above mean sea-level) of the interfering station and the interfered-with radars and for two worst-case required losses (see Attachment 1 of this Annex):

	161.1 dB, which is the worst case (Radar 2) with co-frequency operation of the radar and the RNSS station;

	149.7 dB, which is the worst case (Radar 3, receiving bandwidth of 6.4 MHz) with a 3 MHz offset between the central frequencies of the radar and the RNSS emission.

- .

For the understanding of the figures below,

•	h(m, amsl) Rx is the height in metres above mean sea level of the interfered-with radar;

•	h(m, amsl) Tx is the height in metres above mean sea level of the transmitting uplink RNSS station.

FIGURE 1

Radar protection distances for a required loss of 161.1 dB

FIGURE 2

Radar protection distances for a required loss of 149.7 dB

## 3.3	Conclusions

The results in the section before show the radar protection distances for several interfered-with radars and the interfering RNSS uplink stations heights, from 1 m to 1 000 m above mean sea-level. These distances go from 50 km to 325 km, depending on RNSS uplink stations and radar system heights. For different parameters (radar receiving parameters, RNSS earth station transmitting parameters, antenna heights, etc.), the methodology contained in this Annex should be applied to calculate the required protection distance. Also, in the section above it has been assumed that there are no local obstacles between the interfering RNSS stations and the interfered-with radars. The choice of an adequate location (i.e. a naturally protected zone) could allow for a closer radar protection distance, and could be studied on a case-by-case basis taking into account the specific path profile.

σq2=δq212=(2A)2(2n)2⋅12=6.25⋅σb2(2n)2⋅310log(σq2σb2)=10log(6.25)−10log(3)−20nlog(2)=3.2−6n

-

---

--

A2⋅τ+(1−τ)⋅σb2=A26.25with τ<16.25

σb2=A2⋅1/6.25−τ1−τ

σq2=δq212=(2A)2(2n)2⋅12=1−τ16.25−τ⋅σb2(2n)2⋅310log(σq2σb2)=10log(1−τ)−10log(16.25−τ)−10log(3)−20nlog(2)

ATTACHMENT 1 to Annex 1

System 1

Required loss between radar and uplink stations

|  | Surface-based radar typical values (frequency-offset = 0 MHz) |  |  |  |  | Surface-based radar typical values (frequency-offset = 3 MHz) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Code 10.23 Mcs | Code 1.023 Mcs | Addition of codes | Code 10.23 Mcs |  | Code 1.023 Mcs | Addition of codes |
| Pt |  | 53.0 | 55.0 |  | 53.0 |  | 55.0 |  |
| At |  | 50.00 | 50.00 |  | 50.00 |  | 50.00 |  |
| Gt |  | 1.0 | 1.0 |  | -1.0 |  | 1.0 |  |
| Flt |  | 0.0 | 0.0 |  | 0.0 |  | 0.0 |  |
| Gr |  | 33.5 | 33.5 |  | 33.5 |  | 33.5 |  |
| FLr |  | 0.5 | 0.5 |  | 0.5 |  | 0.5 |  |
| Rb |  | 11.2 | 1.8 |  | 12.4 |  | 24.4 |  |
| Dpol |  | 0.0 | 0.0 |  | 0.0 |  | 0.0 |  |
| P interfering |  | 24.8 | 36.2 | 36.5 | 23.6 |  | 13.6 | 24 |
| P threshold |  | 119.1 | 119.1 | 119.1 | 119.1 |  | 119.1 | 119.1 |
| Required loss (dB) |  | 143.9 | 155.2 | 155.5 | 142.6 |  | 132.7 | 143.0 |

System 2

Required loss between radar and uplink stations

|  | Surface-based radar typical values (frequency-offset = 0 MHz) |  |  |  |  | Surface-based radar typical values (frequency-offset = 3 MHz) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Code 10.23 Mcs | Code 1.023 Mcs | Addition of codes | Code 10.23 Mcs |  | Code 1.023 Mcs | Addition of codes |
| Pt |  | 53.0 | 55.0 |  | 53.0 |  | 55.0 |  |
| At |  | 50.00 | 50.00 |  | 50.00 |  | 50.00 |  |
| Gt |  | 1.0 | 1.0 |  | 1.0 |  | 1.0 |  |
| Flt |  | 0.0 | 0.0 |  | 0.0 |  | 0.0 |  |
| Gr |  | 38.9 | 38.9 |  | 38.9 |  | 38.9 |  |
| FLr |  | 0.5 | 0.5 |  | 0.5 |  | 0.5 |  |
| Rb |  | 11.7 | 2.2 |  | 13 |  | 25.6 |  |
| Dpol |  | 0.0 | 0.0 |  | 0.0 |  | 0.0 |  |
| P interfering |  | 29.7 | 41.2 | 41.5 | 28.4 |  | 17.8 | 28.8 |
| P threshold |  | 119.6 | 119.6 | 119.6 | 119.6 |  | 119.6 | 119.6 |
| Required loss (dB) |  | 149.3 | 160.8 | 161.1 | 148 |  | 137.4 | 148.4 |

System 3

Required loss between radar and uplink stations

|  | Surface-based radar typical values Rx Bw=4.4 MHz (worst case) (frequency-offset = 0 MHz) |  |  |  |  | Surface-based radar typical values Rx Bw=6.4 MHz (worst case) (frequency-offset = 3 MHz) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Code 10.23 Mcs | Code 1.023 Mcs | Addition of codes | Code 10.23 Mcs |  | Code 1.023 Mcs | Addition of codes |
| Pt |  | 53.0 | 55.0 |  | 53.0 |  | 55.0 |  |
| At |  | 50.00 | 50.00 |  | 50.00 |  | 50.00 |  |
| Gt |  | 1.0 | 1.0 |  | 1.0 |  | 1.0 |  |
| Flt |  | 0.0 | 0.0 |  | 0.0 |  | 0.0 |  |
| Gr |  | 38.2 | 38.2 |  | 38.2 |  | 38.2 |  |
| FLr |  | 0.5 | 0.5 |  | 0.5 |  | 0.5 |  |
| Rb |  | 3.9 | 0.2 |  | 3.6 |  | 1.7 |  |
| Dpol |  | 0.0 | 0.0 |  | 0.0 |  | 0.0 |  |
| P interfering |  | 36.8 | 42.5 | 43.5 | 37.1 |  | 41 | 42.5 |
| P threshold |  | 108.8 | 108.8 | 108.8 | 107.2 |  | 107.2 | 107.2 |
| Required loss (dB) |  | 145.7 | 151.3 | 152.4 | 144.4 |  | 148.3 | 149.7 |

System 4

Required loss between radar and uplink stations

|  | Surface-based radar typical values (frequency-offset = 0 MHz) |  |  |  |  | Surface-based radar typical values (frequency-offset = 3 MHz) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Code 10.23 Mcs | Code 1.023 Mcs | Addition of codes | Code 10.23 Mcs |  | Code 1.023 Mcs | Addition of codes |
| Pt |  | 53.0 | 55.0 |  | 53.0 |  | 55.0 |  |
| At |  | 50.00 | 50.00 |  | 50.00 |  | 50.00 |  |
| Gt |  | 1.0 | 1.0 |  | 1.0 |  | 1.0 |  |
| Flt |  | 0.0 | 0.0 |  | 0.0 |  | 0.0 |  |
| Gr |  | 32.5 | 32.5 |  | 32.5 |  | 32.5 |  |
| FLr |  | 0.5 | 0.5 |  | 0.5 |  | 0.5 |  |
| Rb |  | 9.3 | 0.8 |  | 10.6 |  | 20.7 |  |
| Dpol |  | 0.0 | 0.0 |  | 0.0 |  | 0.0 |  |
| P interfering |  | 25.7 | 36.2 | 36.6 | 24.4 |  | 16.3 | 25 |
| P threshold |  | 115.7 | 115.7 | 115.7 | 115.7 |  | 115.7 | 115.7 |
| Required loss (dB) |  | 141.4 | 151.9 | 152.3 | 140.1 |  | 132 | 140.7 |

Wind profile radars

Required loss between radar and uplink stations

|  | Surface-based radar typical values (frequency-offset = 0 MHz) |  |  |  |  | Surface-based radar typical values (frequency-offset = 3 MHz) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Code 10.23 Mcs | Code 1.023 Mcs | Addition of codes | Code 10.23 Mcs |  | Code 1.023 Mcs | Addition of codes |
| Pt |  | 53.0 | 55.0 |  | 53.0 |  | 55.0 |  |
| At |  | 50.00 | 50.00 |  | 50.00 |  | 50.00 |  |
| Gt |  | 1.0 | 1.0 |  | 1.0 |  | 1.0 |  |
| Flt |  | 0.0 | 0.0 |  | 0.0 |  | 0.0 |  |
| Gr |  | 33.5 | 33.5 |  | 33.5 |  | 33.5 |  |
| FLr |  | 0.5 | 0.5 |  | 0.5 |  | 0.5 |  |
| Rb |  | 6.2 | 0.4 |  | 7.4 |  | 18.3 |  |
| Dpol |  | 0.0 | 0.0 |  | 0.0 |  | 0.0 |  |
| P interfering |  | 29.8 | 37.6 | 38.3 | 28.6 |  | 19.7 | 29.1 |
| P threshold |  | 114.5 | 114.5 | 114.5 | 114.5 |  | 114.5 | 114.5 |
| Required loss (dB) |  | 144.3 | 152.1 | 152.7 | 143.1 |  | 134.2 | 143.6 |

-

ATTACHMENT 3 to Annex 1

Computation of the interfering radar power (side lobes) 
received at the satellite altitude

Radar 1 (worst case) in the 1 300-1 350 MHz band

| Frequency | MHz | 1 328.0 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Radar antenna max gain | dBi | 34.5 |  |  |  |
| Radar antenna elevation | deg | 2.5 |  |  |  |
| Elevation (deg) | deg | 5.0 | 10.0 | 30.0 | 90.0 |
| Psi | deg | 14.3 | 14.1 | 12.4 | 0.0 |
| Distance | km | 24 442.0 | 23 906.9 | 21 997.6 | 19 400.0 |
| Peak transmitted power | dBm | 97 | 96.5 | 96.5 | 96.5 |
| Antenna gain emission | dBi |  |  |  |  |

ANNEX 2

Sharing between stations of the radionavigation-satellite service 
(Earth-to-space) and airborne radar of the radiolocation service 
in the frequency band 1 300-1 350 MHz

# 1	Introduction

This Annex provides an analysis of

the interference created by uplink stations into receiving airborne radars.

It results from the study that a separation distance between the airborne radars service area border and the uplink RNSS stations permits the avoidance of excess interference into airborne radars.

# 2	System characteristics

## 2.1	Airborne radiolocation radars

The systems listed in Table 5 are representative systems of airborne radars that are operational or planned for the 1 300-1 350 MHz band.

### 2.1.1	System A

System A is an airborne, non-coherent, side-looking radar used for ocean surveillance. The radar will be carried on aircraft with an operation altitude of 15 500 to 25 500 feet in over-water operations at ranges of 50 to 500 n.m. from shore.

### 2.1.2	System B

System B is a multichannel airborne radar measurement system with operations at altitudes not to exceed 10 000 feet.

### 2.1.3	System C

System C is an airborne radar that has two operator-selectable, frequency-diverse pairs. The diverse frequencies have 15 MHz separation. In one mode, all four frequencies are used to eliminate second-time-around echoes. The radar is carried on a tethered balloon which is moored at an altitude of 10 000 to 15 000 feet.

### 2.1.4	System D

System D is an airborne radar that is normally operated above 30 000 feet (typically 30 000-40 000 feet).

TABLE 3

Airborne radiolocation technical characteristics

| Parameter | System A | System B | System C | System D |
| --- | --- | --- | --- | --- |
| Frequency range (MHz) | 1 300-1 350 MHz | 1 215-1 350 MHz | 1 215-1 400 MHz | 1 215-1 400 MHz |
| Output power | 15 kW | 19.2 kW | 8.4 kW | 27 kW |
| Emission BW | 19 MHz | 1 MHz | 3 MHz | 6, 12 MHz |
| Modulation | Pulsed Linear FM | Linear FM spread spectrum | Pulse w/linear FM chirp | - |
| Pulse rate (pps) | 55 | 250 | 369 | 300-1 000 |
| Pulse width | 28 usec | 50 usec | 260.4 usec | 5-50 usec |
| Rise time | 0.04 usec | 250 nsec | 0.1 usec | - |
| Fall time | 0.04 usec | 20 nsec | 0.1 usec | - |
| Antenna gain (dBi) | 20/22 | 27 | 35 | 33 |
| Antenna heights (ft) | 25 500 | 10 000 | 15 000 | 40 000 |
| Protection ratio (I/N) (dB) | –6 | –6 | –6 | - |
| Noise figure (dB) | 4 | 4 | 3.87 | 5 |
| Reflector size | 4'  2.5' & 2'  9' | 4'  4' | 3 metre | - |
| Beam type | Phased array | Phased array | Parabolic | Pencil |
| Beamwidth | 39 deg (az) or 12 deg (az) 13 deg (el) | 5.20 deg (az) 10.45 deg (el) | NOTE 3 | 3.5 |
| Scan | NOTE 1 | NOTE 2 | 360 degree horizontal | 360 AZ 60 EL |
| RPM | Fixed | 2 | 5 | 5 |
| IF Bandwidth (3 dB) | 14 MHz | 9 MHz | 20 MHz | - |
| NOTE 1 - There are two possible antennas for this radar, both of which are mounted at a right angle to the flight track. The first antenna has a beamwidth, off boresight shape of 60 × 21 degrees of azimuth with an elevation of 14 × 27 degrees elevation off of boresight. The second antenna has a beamwidth off boresight shape of 36 × 14 degrees of azimuth with an elevation of 14 × 27 degrees elevation off boresight. |  |  |  |  |
| NOTE 2 - The array is an active aperture capable of scanning plus or minus 60 degrees in the azimuth plane. Since the array, when mounted on the aircraft, will not rotate, there will be a 240 degree region around the rear of the array which is naturally blanked at all times. The main beam can be pointed in any sector of a 120 degree region (plus or minus 60 degree from array boresight) in front of the array. |  |  |  |  |
| NOTE 3 - Horizontal – first major side lobe 13.5 dBi@2.7 degree, vertical – 3.5 degree 3 dB spoiled. |  |  |  |  |

# 3	Interference of RNSS uplink stations into airborne radars

Considering the airborne radar characteristics given in § 2 of this Annex, we can derive the required loss in dB in order to protect these airborne radars from RNSS uplink stations beacons. These required loss values have been obtained with the formula in § 3.1 of Annex 1 and they are given in Attachment 1 of Annex 2.

## 3.1	Results

Taking into account the maximum height (supposed to be amsl) of each kind of airborne radar, we have computed the required distance between the border of the service area of the airborne radars, and the uplink RNSS stations using the free space loss propagation model (which is the worst case). For all the calculations we have assumed that the height (amsl) of the transmitting RNSS earth station is 1 000 m.

TABLE 4

Protection distances

|  | System A |  | System B |  | System C |  | System D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Offset = 0 MHz | Offset = 3 MHz | Offset = 0 MHz | Offset = 3 MHz | Offset = 0 MHz | Offset = 3 MHz | Offset = 0 MHz | Offset = 3 MHz |
| Required loss | 132.9 dB | 132.7 dB | 139.5 dB | 139.2 dB | 144.5 dB | 144.5 dB | 145.8 dB | 143.5 dB |
| Protection distance | 79 km | 77 km | 169km | 163km | 301 km | 301 km | 349 km | 277 km |

## 3.2	Conclusions

From these examples, the worst-case situation is given by System D type radar systems. In this case, a distance of 349 km between the area where  System D type airborne radars are in operation and RNSS uplink stations permits the avoidance of harmful interference into radars. For different parameters (radar receiving parameters, RNSS earth stations receiving parameters, etc.) other distances will be obtained.

In addition, it should be noted that other mitigation techniques such as the frequency planning shall reduce the level of interference into airborne radars.

The link budgets shown above are worst case since the received peak power has been calculated taking into account the maximum radar antenna gain. In practice, radar antenna gain in the direction of the receiving satellite will be lower than this value. Assuming the radar antenna gain of System A type radars in the satellite direction is at least 1 dB lower than the maximum gain, the Received Peak Power will be under both the front end amplifier saturation level (

ATTACHMENT 1 to Annex 2

System A

Required loss between A type airborne radars and uplink stations

|  | Airborne radar typical values (frequency-offset = 0 MHz) |  |  |  |  | Airborne radar typical values (frequency-offset = 3 MHz) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Code 10.23 Mcs | Code 1.023 Mcs | Addition of codes | Code 10.23 Mcs |  | Code 1.023 Mcs | Addition of codes |
| Pt |  | 53.0 | 55.0 |  | 53.0 |  | 55.0 |  |
| At |  | 50.00 | 50.00 |  | 50.00 |  | 50.00 |  |
| Gt |  | 1.0 | 1.0 |  | 1.0 |  | 1.0 |  |
| Flt |  | 0.0 | 0.0 |  | 0.0 |  | 0.0 |  |
| Gr |  | 22 | 22 |  | 22 |  | 22 |  |
| FLr |  | 0.5 | 0.5 |  | 0.5 |  | 0.5 |  |
| Rb |  | 0.6 | 0.1 |  | 1.1 |  | 0.1 |  |
| Dpol |  | 0.0 | 0.0 |  | 0.0 |  | 0.0 |  |
| P interfering |  | 23.9 | 26.4 | 28.4 | 23.4 |  | 26.4 | 28.2 |
| P threshold |  | 104.5 | 104.5 | -104.5 | 104.5 |  | 104.5 | 104.5 |
| Required loss (dB) |  | 128.4 | 131 | 132.9 | 128 |  | 130.9 | 132.7 |

System B

Required loss between B type airborne radars and uplink stations

|  | Airborne radar typical values (frequency-offset = 0 MHz) |  |  |  |  | Airborne radar typical values (frequency-offset = 3 MHz) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Code 10.23 Mcs | Code 1.023 Mcs | Addition of codes | Code 10.23 Mcs |  | Code 1.023 Mcs | Addition of codes |
| Pt |  | 53.0 | 55.0 |  | 53.0 |  | 55.0 |  |
| At |  | 50.00 | 50.00 |  | 50.00 |  | 50.00 |  |
| Gt |  | 1.0 | 1.0 |  | 1.0 |  | 1.0 |  |
| Flt |  | 0.0 | 0.0 |  | 0.0 |  | 0.0 |  |
| Gr |  | 27 | 27 |  | 27 |  | 27 |  |
| FLr |  | 0.5 | 0.5 |  | 0.5 |  | 0.5 |  |
| Rb |  | 1.4 | -0.1 |  | 2.3 |  | 0.2 |  |
| Dpol |  | 0.0 | 0.0 |  | 0.0 |  | 0.0 |  |
| P interfering |  | 28.1 | 31.4 | 33.1 | 27.2 |  | 31.3 | 32.7 |
| P threshold |  | 106.4 | 106.4 | 106.4 | 106.4 |  | 106.4 | 106.4 |
| Required loss (dB) |  | 134.5 | 137.8 | 139.5 | 133.6 |  | 137.7 | 139.2 |

System C

Required loss between C type airborne radars and uplink stations

|  | Airborne radar typical values (frequency-offset = 0 MHz) |  |  |  |  | Airborne radar typical values (frequency-offset = 3 MHz) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Code 10.23 Mcs | Code 1.023 Mcs | Addition of codes | Code 10.23 Mcs |  | Code 1.023 Mcs | Addition of codes |
| Pt |  | 53.0 | 55.0 |  | 53.0 |  | 55.0 |  |
| At |  | 50.00 | 50.00 |  | 50.00 |  | 50.00 |  |
| Gt |  | 1.0 | 1.0 |  | 1.0 |  | 1.0 |  |
| Flt |  | 0.0 | 0.0 |  | 0.0 |  | 0.0 |  |
| Gr |  | 35 | 35 |  | 35 |  | 35 |  |
| FLr |  | 0.5 | 0.5 |  | 0.5 |  | 0.5 |  |
| Rb |  | 0.4 | 0 |  | 0.5 |  | 0 |  |
| Dpol |  | 0.0 | 0.0 |  | 0.0 |  | 0.0 |  |
| P interfering |  | 37.1 | 39.5 | 41.4 | 37 |  | 39.5 | 41.4 |
| P threshold |  | 103.1 | 103.1 | 103.1 | 103.1 |  | 103.1 | 103.1 |
| Required loss (dB) |  | 140.2 | 142.6 | 144.5 | 140.1 |  | 142.5 | 144.5 |

System D

Required loss between D type airborne radars and uplink stations

|  | Airborne radar typical values Rx Bw = 6 MHz (worst case) (frequency-offset = 0 MHz) |  |  |  |  | Airborne radar typical values Rx Bw = 6 MHz (worst case) (frequency-offset = 3 MHz) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Code 10.23 Mcs | Code 1.023 Mcs | Addition of codes | Code 10.23 Mcs |  | Code 1.023 Mcs | Addition of codes |
| Pt |  | 53.0 | 55.0 |  | 53.0 |  | 55.0 |  |
| At |  | 50.00 | 50.00 |  | 50.00 |  | 50.00 |  |
| Gt |  | 1.0 | 1.0 |  | 1.0 |  | 1.0 |  |
| Flt |  | 0.0 | 0.0 |  | 0.0 |  | 0.0 |  |
| Gr |  | 33 | 33 |  | 33 |  | 33 |  |
| FLr |  | 0.5 | 0.5 |  | 0.5 |  | 0.5 |  |
| Rb |  | 2.7 | 0.1 |  | 3.8 |  | 3.1 |  |
| Dpol |  | 0.0 | 0.0 |  | 0.0 |  | 0.0 |  |
| P interfering |  | 32.8 | 37.4 | 38.7 | 31.7 |  | 34.4 | 36.3 |
| P threshold |  | 107.2 | 107.2 | 107.2 | 107.2 |  | 107.2 | 107.2 |
| Required loss (dB) |  | 140.0 | 144.5 | 145.8 | 138.9 |  | 141.6 | 143.5 |

ANNEX 3

Description of a choke ring antenna design providing high attenuation at low elevation, for RNSS uplinks in the 1 300-1 350 MHz bandwidth

# 1	Introduction, purpose of the choke ring

Earth-to-space uplinks can be used for RNSS purposes, with MEO satellites for instance. The power transmitted in this band would be typically around 57 dBm. The ground antennas will be pointed toward zenith. These antennas are supposed to be omnidirectional, with a gain of 1 dB  for an elevation angle o of 10 degrees, without the help of a choke ring. In order to properly protect radar links, it is necessary to have a choke ring providing an attenuation of 50 dB for the 10° elevation.

# 2	Description of the choke ring

A choke ring is made of one or several cylindrical elements placed around the radiating element of an antenna. They are used classically for GPS reception antennas, in order to mitigate multipath and attenuate the antenna gain at low elevations. The diameter of these choke rings is limited to about 20 to 50 cm in the case of GPS reception. This diameter corresponds to 1 or 2 wavelengths of the GPS signal. The choke ring operates here in the near-field area, and, as such, has limited attenuation performances.

For the purpose of the mentioned RNSS uplinks, the attenuation at low elevation shall be increased. This can be done with a choke ring cylinder having a big diameter, compared to the wavelength of the transmitted signal, as shown on the figure hereafter. In such a case, the choke ring operates in the far-field area, and can be considered as a real physical mask. The necessary diameter is a few metres (about 5 metres in the considered example). The elevation angle  of the choke ring seen from the antenna phase center is close to about 25 degrees (o < ). A cylindrical wall centered on the radiating element of the antenna makes the choke ring. (The height of the wall is about 1 metre in the considered example). The material of the wall is preferably absorbing, but simple concrete could also be used. The inside of the wall is covered with absorber. This absorber can be made of large pyramids, for instance. Commercial absorbers providing an attenuation of about 50 dB in L band are available, and compatible with the radiated power without inflammation risk due to the radiated power. The edge of the wall is designed to limit diffraction: a ridged or corrugated wall can be used, for instance, and an extra absorber can be put on the top of the wall if necessary. The overall structure is covered by a radiotransparent radome. This structure allows therefore an attenuation better than 50 dB for a 10° elevation angle, taking only into account the effects of the absorber and the corrugation. A second choke ring could be even added if a significant performance improvement were necessary.

Moreover, it can be noted that some meteorological radars also use transmitting and receiving antenna elements inside separated walls covered by absorbers. This concept is also called tunnel antenna. It allows an isolation of about 100 dB between both antennas, for an elevation angle below 5°.

____________