ICAO WORKSHOP ON

NATIONAL AVIATION SAFETY PLAN DEVELOPMENT

Handout 4:

Facilitated Exercise III: Develop List of Prioritized Safety Enhancement Initiatives

Context

Using the results from the Facilitated Exercises I and II, and the documentation provided during this workshop, complete the tasks below.

Time allocated: 3h00

Documentation / References

- Doc 10131, Chapter 2, Sections 2.7 and 2.8

- Doc 10161, Appendices A and B

Your tasks

- Based on list of prioritized national safety issues (from Exercise I) and the national goals, targets and indicators (from Exercise II), review the excepts from the Global Aviation Safety Roadmap (Doc 10161), presented Appendix A;

- Select series of safety enhancement initiatives (SEIs), and their specific actions, that will enable the achievement of the national targets;

- Review the list of selected SEIs and assign them an order of priority – Select those you will keep for the action plan as a prioritized list of SEIs; and

- Complete the form in the Appendix B.

— — — — — — — —

Presentation by: Team DETZ-Achievers

SELECTION OF SEIs AND ACTIONS & ASSIGNMENT OF PRIORITY

| SEI Number and Name | Actions | Priority (and why?) |
| --- | --- | --- |
| SEI-5 — Qualified technical personnel to support effective safety oversight |  5A — Establish an effective system to identify and track qualifications and training of existing technical personnel (CE-4)  5B — Identify the gaps in qualified technical personnel and training requirements necessary to implement the oversight mandate (CE-4)  5C — Establish a compensation scheme for the attraction and retention of qualified technical personnel (CE-4)  5D — Make use of RSOOs, RAIOs, or equivalent means, to secure qualified technical personnel to perform those functions which cannot be performed by the State acting on its own (CE-4)  5E — Establish human resource plans to support hiring and retention of the appropriate number of qualified technical personnel required (CE-4)  5F — Implement training policies and programmes for technical personnel and verify that the type and frequency of training successfully completed (i.e. initial, recurrent, specialized and on-the-job training) are sufficient to acquire/maintain the required qualifications and level of competence corresponding to the assigned duties and responsibilities of technical personnel (CE-4) | P1 This SEI receives our 1st Priority, with consideration to it being part of the Foundation PQs (Establishment) |
| SEI-9 — Continued implementation of and compliance with ICAO SARPs at the national level |  9A — Increase the level of compliance with ICAO SARPs and the EI of CEs at the national level (all CEs, emphasis on CE-4 and CE-8 in AIG & AGA)  9B — Implement regulatory oversight and enforcement processes (CE-7 and CE-8)  9C — Establish a system to resolve safety issues identified via accident and incident investigations, surveillance activities, safety reports and other means (CE-8) | P2 Improves overall safety, and will prevent the State from getting an SSC |
| SEI-17 — Availability of safety data and safety information to support safety management activities at the national level (step 1) |  17A — Establish national laws, regulations and policies protecting safety data, safety information and related sources, in accordance with Appendix 3 of Annex 19  17B — Establish a State mandatory occurrence reporting system  17C — Establish safety data collection and processing systems (SDCPS) to capture, store, aggregate, and enable the analysis of safety data and safety information to support their safety performance management activities  17D — Establish and maintain a process to identify hazards from collected safety data  17E — Establish and utilise a process to ensure the assessment of safety risks associated with identified hazards  17F — Establish a State confidential voluntary safety reporting system providing data to the safety database | P3 This is with consideration that an SDCPS will support most audit areas (OPS, AIR, AIG, ANS & AGA) and satisfies the requirements of CE8. It will further assist in the monitoring SEI in all HRC. |
| 3. MID-AIR COLLISION (MAC) (Mitigate contributing factors to MAC accidents and incidents) | - Implement the following MAC safety actions: - Establish guidance and regulations to ensure aircraft are equipped with airborne collision avoidance system (ACAS/ TCAS/ ADS-B/ PBN/ Satellite Surveillance), in accordance with Annex 6 - Ensure adherence to ACAS/TCAS warning procedures - Promote the improvement of air traffic control (ATC) systems, procedures and tools to enhance conflict management - Promote the improvement of communications systems and procedures, such as controller-pilot datalink - Validate the effectiveness of the SEIs through the analysis of MORs and VORs and accident/incident investigations (apply safety management methodologies) - Identify additional contributing factors, for example: - Traffic conditions - traffic density, complexity, mixture of aircraft types and capabilities, etc. - ATC performance related to workload, competence, teamwork, procedures, commitment, etc., as well as the influence of air navigation services providers' (ANSP) safety management - Flight crew training and corporate culture with workload, competence, teamwork, procedures, commitment etc., and the influence of aircraft operator’s safety management - ATC systems - flight data processing, communication, short term conflict alert (STCA), etc., as well as the interaction with the human operators and the aircraft systems, and the procurement policy of the ANSP - Aircraft equipment - autopilots, transponders and ACAS, but also aircraft performance (e.g. rate-of-climb) and their physical size - Navigation infrastructure - both coverage and quality - Surveillance - both coverage and quality - Develop and implement further SEIs to mitigate the risk of the identified contributing factors, if any, for MAC - Conduct continuous evaluations of the performance of the SEIs | P4 Besides it being a G-HRC, State X has increased reports associated with air-proximity issues and loss of and inadequate aircraft separation, including possible events that can lead to incidents associated with unstable approaches, hard-landings, and runway excursions (which State-X is experiencing) |
| 2. LOSS OF CONTROL IN-FLIGHT (LOC-I) (Mitigate contributing factors to the risk of CFIT) | - Implement the following LOC-I safety actions: - Require upset prevention and recovery training in all full flight simulator type conversion and recurrent training programmes - Require more time devoted to training for the pilot monitoring role - Validate the effectiveness of the SEIs in the industry through MORs and VORs systems and accident/incident investigations (apply safety management methodologies) - Identify additional contributing factors, for example: - Distraction - Adverse weather - Complacency - Inadequate standard operating procedures (SOPs) for effective flight management - Insufficient height above terrain for recovery - Lack of awareness of or competence in procedures for recovery from unusual aircraft attitudes - Inappropriate flight control inputs in response to a sudden awareness of an abnormal bank angle - Develop and implement further SEIs to mitigate the risk of the identified contributing factors, if any, for LOC-I, for example: - Increase the effectiveness of regulatory oversight - Improve regulations - Conduct continuous evaluations of the performance of the SEIs | P5 Besides it being a G-HRC, State X has increased reports associated with unstable approaches, hard-landings, and runway excursions |
| SEI-14 — Strategic allocation of resources to start SSP implementation |  14A — Appoint responsible entity for the management of the SSP  14B — Establish a process for planning and allocation of resources to enable SSP implementation and identify areas where resources are needed  14C — Obtain resources from national and appropriate authorities’ leadership and stakeholders within the State to support SSP implementation  14D — Work with the ICAO Regional Office to make use of available means (e.g. Technical Cooperation Bureau) to acquire assistance needed for SSP implementation  14E — Work with RSOO, other States and other organizations, as appropriate to train qualified technical personnel to fulfil their duties and responsibilities regarding SSP implementation  14F — Implement SSP training to SSP Coordinator(s) | P6 This will help the State to meet GASP Goal #3; however, this is not audited (USOAP-CMA) |
| SEI-13 — Start of SSP implementation at the national level |  13A — Secure State-level commitment to improve safety  13B — Conduct initial SSP gap analysis (checklist) then the detailed SSP self-assessment  13C — Establish an SSP implementation team  13D — Implement SSP training and promotion to Team, Safety Board & Stakeholder as appropriate  13E — Identify and involve relevant stakeholders  13F — Develop an implementation plan for the SSP  13G — Issue SMS regulations for service providers and verify SMS implementation  13H — Identify and share safety management best practices | P7 This SEI is a continuation of SEI-14 |
| SEI-18 — Availability of safety data and safety information to support safety management activities at the national level (step 2) |  18A — Establish the safety objectives to be achieved through the SSP  18B — Develop safety performance measurement methodologies, aligned with the regional safety metrics, using the established safety risk management process (see SEI-17E)  18C —Develop safety performance indicators and safety performance targets using the established safety risk management process  18D — Ensure the establishment of mandatory safety reporting systems by service providers  18E — Encourage establishment of voluntary safety reporting systems as part of service providers’ SMS  18F — Promote safety awareness and the two-way communication, sharing and exchange of safety-relevant information within the State’s aviation organizations and encourage sharing of safety information with industry within the State  18G — Contribute information on operational safety risks, including SSP safety performance indicators, and emerging issues, to the RASG | P8 This SEI supports the SSP, and therefore, to be implemented together with the SSP |

— END —