FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

Seventeenth Working Group meeting

El Cairo, Egypt, 30 August - 07 September 2023

Agenda Item 3b):	Development of Material for ITU-R Studies – FSMP.003.02.

WRC-23 AI 1.7 VHF AMS(R)S

Update dynamic analysis in protection of receiving AMS(R )S satellites from MSS, SOS, SRS and MetSat satellite unwanted emissions in the band 137-138 MHz

(Presented by: Manuel Garcia)

(Prepared by: ANSPs (ENAIRE), Industry (Indra, STARTICAL),

Communications Service Providers (Collins and SITA))

SUMMARY

Studies on WRC-23 agenda item 1.7 are performed within several ICAO and ITU-R groups since July 2020. Important progress has been made at the recent meeting of ITU-R WP 5B by including technical interference studies on several technologies (voice, datalink) and covering the whole band 117.975-137 MHz towards potential AMS(R)S allocation. This paper presents the outcomes of the study performed by Spain, recalled and extended in Attachment, regarding the use of the VDLM2 CSC (Operating worldwide around the 136.975 MHz frequency) in the Space-based solution considering the results of dynamic analysis performed in protection of receiving AMS(R)S stations and the different mechanisms implemented in the current ATN/VDLM2 network to guaranty the integrity of the ATC information exchanged. The main conclusions of the document are: - the percentage of the time that interferer signal levels from space systems operating above 137MHz can potentially cause disruption to the AMS(R)S, is negligible. - given the above, protocolar mechanisms of the higher layers of the VDLM2 standard are capable of maintaining adequate service performance levels for AMS(R)S. The study provides a rationale for supporting the operation of AMS(R)S systems in the band 117.975-137MHz not requiring any new restriction to existing VHF systems operating in the frequency band 137-138 MHz.

- INTRODUCTION

  - Several ICAO groups are involved in the development of SB-VHF technical and operational studies:

  - FSMP (Frequency Spectrum Management Panel):

- The Frequency Spectrum Management Panel (FSMP) is established to manage aeronautical frequency spectrum in order to ensure sufficient access to the resource for the provision of aeronautical communication, navigation and surveillance services (CNS) in an efficient and safe manner.

- FSMP is preparing the ICAO position regarding WRC-23 AI 1.7 VHF AMS(R)S (Aeronautical Mobile Satellite (Route) Services). Several Organizations (CAAS Singapur, AirServices Australia, ENAIRE, Indra, STARTICAL,..) have been providing contributions in different meetings to develop all the technical and operational topics regarding this concept.

- FSMP is also the ICAO liaison with ITU regarding regulatory activities performed in relation to ITU WRC-2023.

  - CG-SV (Satellite VHF Coordination Group):

- FSMP WG/13 agreed that a Satellite VHF Coordination Group (CG-SV) should be formed to allow for continued information exchange regarding the AMS(R)S (Aeronautical Mobile Satellite (R) Services) concept, and to allow for current and new questions to be considered and agreed responses developed. The CG-SV goal is to develop a comprehensive agreed set of frequently asked questions (FAQs) in order that all the different aeronautical stakeholders could find updated information about this concept. This activity is being coordinated with the DCIWG/PT-T/FVSG group.

  - FVSG (Future VHF Sub Group):

- Under CP-DCIWG, PT-T aims to develop ICAO provisions which are necessary to support global/regional adoptions of future aeronautical communication technologies operating in VHF frequency band. The FVSG will review and, as necessary, develop Proposals for Amendment (PfA) and Change Proposals (CPs) to ICAO provisions, such as Annex 10, as related to future aeronautical communication technologies to be operated in the VHF frequency band.

- The FVSG will be responsible in providing required SARPs validation reports and in developing any necessary ICAO guidance materials.  Additionally, in reference to ITU WRC-2023 Agenda Item 1.7, the FVSG will also provide necessary technical inputs and materials supporting the works of ICAO DCIWG and ICAO FSMP.

- The FVSG is also expected to assist future ICAO activities in support of the implementation of the new aeronautical VHF communication technologies.

- A Jobcard compiling the necessary activities was developed to address this topic.

  - During the previous FSMP/WG15 meeting, a working paper (WP14) was presented proposing an approach to minimize the impact on existing systems while still being able to operate an economically viable Data Link Service under the potential future AMS(R)S allocation in the VHF frequency band.

  - During the previous FSMP/WG16 meeting, a working paper (WP16) was presented developing the concept of operation for SB-VHF datalink that was analysed considering the potential impact on current avionics and datalink operational procedures concluding that:

- Satellite based datalink services could be integrated in existing CSPs datalink (POA/VDLm2) infrastructure using current operational procedures without any modification in current avionics, with potentially and not strictly necessary, database update of the datalink avionic (also supported by standard operational procedures).

- Based on the considerations explained in the document, satellite based datalink services could be provided assuring no interference with the current terrestrial VDL infrastructure.

- DISCUSSION

  - The objective of this paper is to present the outcomes of a study, performed by Spain in the frame of the PDNR ITU-R [Space_VHF] and extended by Spain as presented in the Attachment. The study consists in an initial analysis related to the use of the VDLM2 CSC (Operating worldwide around the 136.975 MHz frequency) under an AMS(R)S allocation. The study considers, in particular, the results of dynamic analysis performed in protection of receiving AMS(R)S stations and the different mechanisms implemented in the current ATN/VDLM2 network to guaranty the integrity of the ATC information exchanged.

  - In addition to this, the study analyses the effects of different protocol mechanisms that are part of the  VDLM2 standards at layers above the physical layer to illustrate how these can mitigate the impact of interferences from external systems not requiring any new restriction to existing VHF systems operating in the frequency band 137-138 MHz.For this purpose, an informal working group was created with the following participants:

- ANSPs (ENAIRE).

- Industry (Indra, STARTICAL).

- Communications Service Providers (Collins and SITA).

  - The working group has discussed the study presented as an Attachment to this paper with the following structure:

- 1. INTRODUCTION. It states that this document focuses on the analysis of the interference environment created by space stations of the adjacent band services into the space receiver of the future AMS(R)S and incorporates the results of additional complementary work that has been done on the dynamic analysis on protection of AMS(R)S satellites reception operating below 137MHz due to unwanted emissions from satellites operating MSS, SOS, SRS and METSAT services in the band 137-138MH.

- 2. INTERFERENCE ANALYSIS THRESHOLDS FOR THE REVERSE TECHNICAL COMPATIBILITY STUDY OF INTERFERENCE FROM ADJACENT BAND SERVICES INTO SPACE RECEIVERS OF THE AMS(R)S. This section brings additional comparative cases to further reconfirm results obtained in the reported cases of study 1 as contained in the Section 11 of PDNR ITU-R M.[SPACE-VHF].

- 3. COMPARATIVE ASSESSMENT OF INTERFERENCE INTO AMS(R) SPACE SYSTEMS OPERATING VOICE SERVICES AND INTO AMS(R) SPACE SYSTEMS OPERATING DIGITAL DATA LINK FROM THE ADJACENT BAND SYSTEMS SERVICES OPERATING IN THE BAND 137-138 MHz.. Considering the rationale described in the previous section where the characteristics of the Voice and Data service signals are set, a new comparative assessment of probability of interferences exceeding the interferences threshold identified in previous section is carried out.

- 4. IMPACT OF POTENTIAL INTERFERENCES OVER THE AMS(R)S VHF DATA LINK COMMUNICATION (VDL) SERVICES. This section explains the rational for using the CSC frequency in the Space-based solution and the different mechanisms implemented in the current ATN/VDLM2 network to guaranty the integrity of the ATC information exchanged. It also describes the different protection mechanisms implemented in Datalink network in order to mitigate the impact of network failures, including VHF interferences.

  - The main conclusions of the study are recalled hereafter:

- The percentage of the time that interferer signal levels from space systems operating above 137MHz can potentially cause disruption to the AMS(R)S, is negligible.

- The result of this comparative additional scenarios proposed by Spain in Attachment allow concluding that the interference created by out of band unwanted emissions of adjacent satellite services into space receivers of AMS(R)S would not create any relevant degradation of the AMS(R)S when operating VDLM2 digital data link. When operating Voice services below 136.9 MHz, it is noted that such operation requires lower interfering signal levels than for data link and while still will be possible to operate voice signals at their higher frequency channelling foreseen so far by ICAO, a proper frequency planning exercise is recommended.

- Regarding the provision of Datalink services and, as it has been demonstrated in the study performed by Spain and presented in Attachment, and considering the elements of the previous bullet point, protocolar mechanisms of the higher layers of the VDLM2 standard (e.g. frame retransmissions, error detection and correction mechanisms) are capable of maintaining adequate service performance levels for provision of AMS(R)S.

- It is recommended that the use of the CSC channel by AMS(R)S stations be limited to signalling communications with aircraft to indicate the availability of an alternative frequency.

- It is highlighted that, given the present state of the intra-system studies regarding the concurrent provision of both AM(R)S and AMS(R)S over a given geographical area, it is expected that stations participating in AMS(R)S and stations participating to AM(R)S are always assigned separate channels until further studies on the effects of operating differently are performed and discussed.

The elements presented above tend to advocate for AMS(R)S allocation to the full band 117.975-137MHz,  without requiring any new restrictions to existing VHF systems operating in the frequency band 137-138 MHz.

- ACTION BY THE MEETING

  - The meeting is invited to:

  - note and review the contents of this working paper;

  - endorse the main conclusions in paragraph 2.4.

  - take the elements presented in the Attachment and, in particular, the conclusions recalled in this paper as input to the discussions of the FSMP/WG 17 analysing the feasibility of an AMS(R)S operation in the 117.975-137MHz frequency band, in particular regarding the use of the 136.975 MHz center frequency for the operation of VDLM2 systems from space.

— END —

ATTACHMENT

Update dynamic analysis in protection of receiving AMS(R )S satellites from MSS, SOS, SRS and MetSat satellite unwanted emissions in the band 137-138 MHz.