FREQUENCY SPECTRUM MANGEMENT PANEL (FSMP)

Seventeenth Working Group Meeting

Cairo, Egypt – 30 August – 7 September 2023

Agenda Item 7):	ICAO Frequency Spectrum Handbook (Doc 9718)

a) Restructuring and update of Volume I

Relevant International Standards

(Presented by John Mettrop)

SUMMARY

This document provides information on international standards that are applicable to aeronautical systems and that this information be incorporated into the ICAO Frequency Spectrum Handbook.

INTRODUCTION

Traditionally ICAO has established Standards and Recommended Practices (SARPs) for aeronautical systems such as VHF communications, distance measuring equipment etc where interoperability between 2 pieces of equipment that potential could have been produced by different manufacturers. This paper suggests that whilst SARPs may only contain systems that require interoperability between equipment supporting that system there is a need, that the Frequency Spectrum Handbook can satisfy, to record the characteristics and relevant standards for all systems used by aviation to support safety of life.

DISCUSSION

Traditionally ICAO and their SARPs have concentrated on aeronautical safety systems that require interoperability between equipment potentially produced by different manufacturers. RTCA and EUROCAE Minimum Aviation Systems Performance Standards and Minimum Operational Performance Standards extend the systems covered but do not cover all systems that aviation use to support safety of life operations.

If aviation wishes to ensure that standards that are relevant to aeronautical safety of life systems, ensure that they are in conformance with the relevant radio regulatory standards and identify those aeronautical systems that do not have standards there is a need to identify those standards that do apply. Additionally, this will help with the identification of the values for the various characteristics that are selected to define the technical and operational characteristics required to support sharing studies.

It is therefore proposed that a list of the various standards that are applicable to aeronautical safety systems be included as part of the Frequency Spectrum Handbook. An initial version is included in the annex to this document which is based on the results of the recent survey undertaken by EUROCAE and RTCA.

ACTION BY THE MEETING

The meeting is invited to:

note and review the contents of this working paper.

Support the proposal to include information on standards applicable to aeronautical systems in the Frequency Spectrum Handbook

Annex

List of Standards

| System | Frequency Band | Service | Standards |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ICAO Annex 10 | RTCA | EUROCAE | ETSI | Standard 4 | Standard 5 |
| NDB | 130–535 kHz | ARNS | Volume I, Chapter 3.4 & 3.9 | DO-179 | ED-51 |  |  |  |
| HF Comms | 2 850–22 000 kHz | AM(R)S | Volume III, Part II, Chapter 3 |  |  |  |  |  |
| HF Datalink |  |  | Volume III, Part I, Chapter 11 | DO-265 | None |  |  |  |
| Marker Beacon | 74.8–75.2 MHz | ARNS | Volume I, Chapter 3.6 | DO-143 | Document 1/WG7 |  |  |  |
| ILS Localizer | 108–112 MHz |  | Volume I, Chapter 3.1 | DO-195 | ED-46B |  |  |  |
| VOR Ground | 108–117.975 MHz | ARNS | Volume I, Chapter 3.3 | DO-196 | ED-22B |  |  |  |
| VOR Airborne |  |  |  | None | ED-52 |  |  |  |
| GBAS Ground | 108–117.975 MHz | AM(R)S | Volume I, Chapter 3.7 |  | ED-114B |  |  |  |
| GBAS Airborne |  |  |  | DO-253D |  |  |  |  |
| Analogue Voice | 117.975–137 MHz | AM(R)S | Volume III, Part II, Chapter 2 | DO-186B | ED-23C |  |  |  |
| VDL Mode 2 |  |  | Volume III, Part I, Chapter 6 | DO-281C | ED-92C | EN 301 841-2 (ground only) |  |  |
| VDL Mode 4 |  |  | Volume III, Part I, Chapter 6 |  |  |  |  |  |
| Narrowband Digital Link |  |  |  | DO-224D | None |  |  |  |
| ILS Glide Path | 328.6–335.4 MHz | ARNS | Volume I, Chapter 3.1 | DO-192 | ED-47B |  |  |  |
| Search & Rescue | 406–406.1 MHz | MSS | Volume III, Part II, Chapter 5 | DO-204B | ED-62B |  |  |  |
| DME | 960-1215 MHz | ARNS | Volume I, Chapter 3.5 | DO-189 | ED-54 & 57 |  |  |  |
| UAT | 978 MHz | AM(R)S | Volume III, Part I, Chapter 12 | DO-282C | None |  |  |  |
| ADS-B | 1030 & 1090MHz | AM(R)S |  | DO-260C | ED-102B |  |  |  |
| SSR |  | ARNS | Volume IV, Chapter 3 | DO-144A | None | EN 303 363-1 |  |  |
| ACAS |  | AM(R)S | Volume IV, Chapter 4 | DO-385 | ED-256 |  |  |  |
| TCAS II |  |  |  | DO-185B | ED-143 |  |  |  |
| Mode S |  |  | Volume III, Part I, Chapter 5 | DO-181F | ED-73F |  |  |  |
| MLAT |  | AM(R)S | Volume IV, Chapter 6 | None | ED-117A |  |  |  |
| GNSS | 1164-1215 MHz | RNSS | Volume I, Chapter 3.7 | DO-292 |  |  |  |  |
|  | 1215-1300 MHz |  |  | DO-292 |  |  |  |  |
| Primary Radar | 1215-1240 MHz | RNS |  | DO-381 |  |  |  |  |
|  | 1240-1260 MHz |  |  |  |  |  |  |  |
|  | 1260-1300 MHz |  |  |  |  |  |  |  |
|  | 1300-1350 MHz | ARNS |  |  |  |  |  |  |
|  | 1350-1370 MHz |  |  |  |  |  |  |  |
| Satellite Comms | 1 518 – 1 559 MHz | MSS |  | DO-262F Appendix E | ED-243C Appendix E |  |  |  |
|  | 1525-1559 MHz |  | Volume III, Part I, Chapter 4 | DO-262F Appendix E ED-243C Appendix E |  |  |  |  |
| GNSS | 1 559 - 1 610 MHz | RNSS | Volume I, Chapter 3.7 | DO-229E, 235C, 253D, 316 & 384 | None |  |  |  |
| Satellite Comms | 1610-1626.5MHz | MSS | Volume III, Part I, Chapter 4 | DO-262F Appendix D | ED-243C Appendix D |  |  |  |
|  | 1626.5-1660.5 MHz |  | Volume III, Part I, Chapter 4 | DO-262F Appendix E | ED-243C Appendix E |  |  |  |
|  | 1668-1675 MHz |  | Volume III, Part I, Chapter 4 | DO-262F Appendix E | ED-243C Appendix E |  |  |  |
| Primary Radar | 2700-2900 MHz | ARNS |  | DO-381 |  | EN 303 347-2, EN 303 364-2 |  |  |
| Radio Altimeter | 4200-4400 MHz | ARNS |  |  | ED-30 |  |  |  |
| WAIC |  | AM(R)S |  |  |  |  |  |  |
| Air to Air Radar |  | ARNS |  | DO-366A |  |  |  |  |
| AeroMACS | 5000-5150 MHz | AM(R)S | Volume III, Part I, Chapter 7 | DO-346A | ED-233A |  |  |  |
| UAS Comms | 5030-5091 MHz | AM(R)S |  | DO-362A |  |  |  |  |
| MLS |  | ARNS | Volume I, Chapter 3.11 |  |  |  |  |  |
| Airborne Weather Radar | 5350-5460MHz | ARNS | None | DO-220A |  |  |  |  |
| Air to Air Radar |  |  |  | DO-366A |  |  |  |  |
| Airborne Weather Radar | 5460-5470 MHz | RNS | None | DO-220A |  |  |  |  |
| Air to Air Radar |  |  |  | DO-366A |  |  |  |  |
| Air to Air Radar | 8 750-8 850 MHz | ARNS |  | DO-366A |  |  |  |  |
| Primary Radar | 9000-9200 MHz | ARNS | Volume I, Chapter 3.2 | DO-381 |  | EN 303 364-3 |  |  |
| Primary Radar | 9 300-9 500 MHz | RNS |  |  |  | EN 303 364-3 |  |  |
| Air to Air Radar |  |  |  | DO-366A |  |  |  |  |
| Airborne Weather Radar |  |  |  | DO-220A |  |  |  |  |
| Helicopter ground beacon reply | 9 310 MHz |  |  | DO-172 |  |  |  |  |
| Helicopter airborne approach radar | 9 375 MHz |  |  | DO-172 |  |  |  |  |
| Air to Air Radar | 13.25 – 13.4 GHz | ARNS |  | DO-366A |  |  |  |  |
| Air to Air Radar | 15.40 – 15.70 GHz |  |  | DO-366A |  |  |  |  |
| Primary Radar |  |  |  | DO-381 |  |  |  |  |
| Airborne Weather Radar | 15.50 – 15.70 GHz |  |  | DO-220A |  |  |  |  |
| Air to Air Radar | 24.45 – 24.65 GHz | RNS |  |  |  |  |  |  |
| Primary Radar |  |  |  |  |  |  |  |  |
| Air to Air Radar | 32.30 – 33.40 GHz |  |  | DO-366A |  |  |  |  |
| Primary Radar |  |  |  | DO-381 |  |  |  |  |

- — END —