|  | International Civil Aviation Organization WORKING PAPER | FSMP-WG/13-WP/10. |
| --- | --- | --- |

FREQUENCY SPECTRUM MANAGEMENT PANEL

WORKING GROUP

thirteenth MEETING

Virtual,21 to 25 February 2022

Agenda Item 5		Any Other Business

Radio Equipment Directive and it’s Implications for Aeronautical Standards

(Presented by John Mettrop)

SUMMARY

This paper brings to the attention of the FSMP a change in the European regulations with respect to placing aeronautical equipment on the market within Europe.. ACTION The FSMP-WG is invited to: - Review the content of this paper. - Consider the impact on the development of standards for aeronautical radio equipment - Endorse that this paper be submitted to the RPASP for further consideration due to potential impacts on UAS command and non-payload communication as well as detect and avoid systems operating in non-aeronautical frequency bands

INTRODUCTION

In 2014 the EU adopted the Radio Equipment Directive (2014/53) which established a regulatory framework for placing radio equipment on the market in the EU. All radio equipment within the scope of this directive that is placed on the EU market must be compliant with the directive that came into force on the 13 June 2017.

discussion

Whilst the Radio Equipment Directive (RED) applies to aeronautical ground equipment, and hence ETSI is developing standards for aeronautical ground equipment, however, airborne equipment was via the EASA Regulation (2016/2008 exempted from the RED. However recent changes to the EASA regulation and the introduction of a new Delegated Regulation that supplements the RED in  the area of cyber security have changed the regulatory map within Europe as summarised in the following table.

|  |  |  | EASA Regulation 2018/1139 Requirement for Security | Radio Equipment Directive 2014/53 as Amended | Delegated Regulation (EU) xxxx/xxx |
| --- | --- | --- | --- | --- | --- |
| Equipment located on manned aircraft |  |  | No | Exempt | Exempt |
| Equipment located on Un-manned Aircraft | That is intended to operate only on frequencies allocated by the Radio Regulations for protected aeronautical use |  | No | Exempt | Exempt |
|  | That operates on frequencies that are not allocated by the Radio Regulations for protected aeronautical use |  | No | Included | Included Article 1 (1) applies |
| Equipment located on the ground Providing ATM/ANS |  | Within the Single European Sky airspace | Included Annex VIII Para 3.3.1 & 3.3.4 | Included Article 3 3 (d) | Included Article 1 (1) applies |
|  |  | Outside of the Single European Sky airspace | No | Included Article 3 3 (d), (e) & (f) | Included Article 1 in total applies |
| Equipment not providing ATM/ANS |  |  | No | Included Article 3 3 (d), (e) & (f) | Included Article 1 in total applies |

For a more detailed explanation of the situation see the attached internal briefing note on the issue.

Conclusions

The recent changes in the European radio regulatory framework has brought more aeronautical equipment under the scope of the RED especially when considering unmanned aircraft using frequencies not allocated to an aeronautical service.

Additionally, the inclusion of the cyber-security requirements through the Delegated Regulation may have wider reaching implications especially when considering the requirement being not only place on equipment that can be directly connected to the internet but also that which can indirectly be connected.  For instance what is the implication for ADS-B that may be a closed system within aviation but because it is also received by various internet based display services?

Whilst I have not had the chance to consider in conjunction with various other entities within the CAA the implications and therefore provide a definitive brief I believe it is valuable to bring this to the attention of the group at the earliest possible opportunity.

ACTION BY THE MEETING

The FSMP is invited to:

  - Review the content of this paper.

  - Consider the impact on the development of standards for aeronautical radio equipment

  - Endorse that this paper be submitted to the RPASP for further consideration due to potential impacts on UAS command and non-payload communication as well as detect and avoid systems operating in non-aeronautical frequency bands

- ATTACHMENT