FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

THIRD MEETING

Virtual (on-line) Meeting, 7 September 2022

Agenda Item 3:	Brief Status Report from FSMP WG/15

Meeting Report of FSMP WG/15 (22 August – 1 September 2022)

(Presented by the Rapporteur of FSMP WG/15)

SUMMARY

This paper attaches the Draft Meeting Report of FSMP WG/15, 22 August – 1 September 2022. The meeting was held at the IATA Offices in Montreal. The meeting was conducted in a hybrid setting. 99 meeting attendees in total, including 14 Panel Members. 29 people attended in-person. The main focus of the meeting included: - Preparations for the ITU World Radiocommunication Conference in 2023 (WRC-23), including draft updates to the ICAO Position for the conference and development of material for related ITU studies. The major items of interest include WRC-23 Agenda Item (AI) 1.6, Suborbital vehicles, WRC-23 AI 1.7 Space Based VHF and WRC-23 AI 1.8, Fixed Satellite Service for RPAS C2-link communications. Updates to the ICAO Position will be finalized by FSMP WG/16 in Q2 2023; - continued work on the Radio Altimeter / 5G studies; and - finalization of a draft new SARPs package for Wireless Aircraft Intra-Communications.

ATTACHMENT:  Draft REPORT of FSMP WG/15

- —-----—

|  | International Civil Aviation Organization REPORT | 1 September 2022. |
| --- | --- | --- |

FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

FIFTEENTH MEETING OF THE WORKING GROUP OF FSMP

(FSMP-WG/15)

Virtual

22 August-1 September, 2022

DRAFT REPORT

1.	Opening and Working Arrangements

1.1	The hybrid (in-person and electronic/virtual) meeting was opened by Mr Loftur Jonasson from the ICAO Secretariat, Montreal and Mr Mike Biggs, the Rapporteur of Working Group FSMP (FSMP-WG).  The in-person portion of the meeting was held at the IATA offices in Montreal, Canada and the meeting greatly appreciated the facilities. Mr Jonasson acted as the Secretary of the meeting. Mr Biggs welcomed the group and provided introductory remarks and meeting information. Lastly, it was noted that as for last few meetings, developing a run-of-meeting schedule was much simpler due to membership providing input contributions in a timely manner, and the cooperation was appreciated.

1.2	The meeting was held in English. After the opening of the meeting the agenda was approved by the group. The agenda is contained in Appendix A.

1.3	The list of papers submitted for consideration by FSMP-WG/15 is contained in Appendix B. The list of participants is in Appendix C.

1.4	The material in this report is organized by meeting agenda item number, and does not necessarily reflect the order of discussions.  Actions captured during discussions are shown in Appendix D, together with status of prior-meeting(s) actions. Regarding the latter, an extensive review of old action items was performed and a number were deleted/updated.

2.	Agenda Item 2 – ICAO WRC-23 Position

2.1 	Identified conflicts between administration preliminary proposals and ICAO Position

2.1.1	No contributions were received on this topic.

2.2	Discussion of possible future agenda item proposals from administrations

2.2.1	No contributions were received on this topic.

2.3	Modifications/Updates to the ICAO WRC-23 Position

2.3.1	Three papers (WP06, WP09 and WP23) were received with proposed changes to the ICAO Position. In addition, several proposals were considered that were not resolved at FSMP-WG/14, and one proposal was pulled from WP11. A Flimsy was developed that combined all proposals and was used by the meeting for the review. An updated Flimsy is contained in Appendix E containing all the agreed language as well as some proposals highlighted in yellow that were not resolved. That document will be used as a basis for any further contributions and discussions at FSMP-WG/16.

3.	Agenda Item 3 – Development of (planned) material for ITU-R studies

3.1	WRC-23 AI 1.6 Suborbital Vehicles

3.1.1	WP04 presented a Liaison Statement from ITU-R WP5B, providing a copy of the latest version of the working document towards a preliminary draft new Report, entitled “Regulatory, operational, and technical studies of radiocommunications for suborbital vehicles.” The meeting appreciated the information.

3.1.2	WP20 noted that, within Europe, the ECHO (European Concept of Higher airspace Operations) research project conducted under the SESAR umbrella was currently working on an air traffic management concept of operations for higher airspace operations, including potential future suborbital flights. The paper further suggested that FSMP liaise to that group to get information on which systems were envisioned to be on the suborbital vehicle to facilitate that process. An informal drafting group developed a proposed liaison, the text of which is contained in Appendix G.

3.2	WRC-23 AI 1.7 VHF AMS(R)S

3.2.1 	A large number of papers were received on the topic of WRC-23 agenda item 1.7. To manage the discussion, they were broken into 3 groups: (a) papers associated with ICAO and/or ITU coordination, (b) papers associated with technical/operational performance issues, and (c) papers associated with liaison to ITU-R WP5B. Once all of the papers within a certain category were introduced, then the meeting held an overall discussion.

3.2.2	Coordination

3.2.2.1	The meeting agreed to structure discussion of “coordination” on two specific areas: (a) Coordination procedures within ITU, (b) Coordination (and/or planning) within ICAO.  In particular it was noted that decisions on the former needed to be made in time to implement at WRC-23, whereas at least some parts of the latter could be deferred until after the Conference.

3.2.2.2	WP02 proposed a set of questions to help guide contributions to FSMP on the expected ICAO coordination process for any new VHF AMS(R)S services.  The paper recommended that the outcome of discussions should be liaised to the ITU-R, and eventually documented in a draft update of Annex 10, Vol. V.

3.2.2.3	WP11 reported on progress made on studies for technical compatibility of potential new allocation in the frequency band 117.975 – 137 MHz to the aeronautical mobile satellite (on route) service with respect to other services operating in adjacent bands. Based on that progress, suggestions were made for updates to the ICAO Position. The meeting could not conclude regarding selecting specific CPM Methods since that text was not yet complete, however there was agreement to add to the Position on AI1.7 to oppose any proposals that systems operating under the new AMS(R)S allocation cannot cause interference to, and/or claim protection from, systems operating in non aeronautical-safety services. The paper also included material to be used in the preparation of the Liaison Statement.

3.2.2.4	WP15 noted that ITU-R WP5B has been working on a draft report ITU-R M.[SPACE-VHF] containing interference studies and specifically addressing the expectation that ICAO would conduct any necessary planning exercise regarding in band sharing services. The paper addressed in-band sharing between AMS(R)S, AM(R)S and AM(OR)S to assist ICAO in that planning effort. It was also noted that the preliminary analysis could serve as response from ICAO to WP 5B by confirming that frequency planning, potential allotment plan and coordination of operational frequencies for all services involved will be undertaken and completed by ICAO at a future stage.

3.2.2.5	WP17 identified a number of concerns regarding the studies currently being undertaken on WRC-23 AI1.7.  During the discussion, it was identified a number of missing topics and inconsistencies (mostly on the regulatory front) regarding the studies currently being undertaken on WRC-23 AI1.7, and noted that there appeared to be a need of further clarification on the overall regulatory framework and some technical assumptions, that needs to be addressed in order to allow a positive decision at the conference.

3.2.2.6	WP18 paper investigated potential coordination mechanisms available within the Radio Regulations that could be used for the coordination of systems operating in the AMS(R)S with systems operating in the AM(R)S. For each potential coordination mechanism, the information that would have to be generated and the advantages/disadvantages of each mechanism were identified.

3.2.2.7	WP22 provided a comparison between RR Nos. 9.11A and 9.21 based on studies performed in another aeronautical frequency band. The paper concluded that for the case of AMS(R)S under WRC-23 AI 1.7, RR No. 9.11A coordination would be a preferable approach.

3.2.2.8	WP24 was a response to WP 03 of FSMP-WG/14, in particular sharing information on the current VHF frequency coordination procedure carried out in Asia Pacific (APAC) Region for the frequency band 117.975 – 137 MHz. In addition, the paper proposed that a coordination procedure similar to that currently used for AM(R)S could also be used for coordinating between future AMS(R)S, AM(OR)S and AM(R)S.

3.2.3	Technical/operational issues

3.2.3.1	WP12 highlighted that ANSPs and airlines are requiring more and more datalink communications for increased ATC automation, allowing extra capacity while improving safety. The paper noted that complementary use of VHF voice and VHF data (CPDLC and ADS-C) with surveillance information through ADS-B would facilitate the introduction of functionalities such as Trajectory Based Operations (TBO), Free Route Airspace (FRA) and 4D Trajectory Datalink (4DTRAD) in oceanic airspace, in regions where this can be shown to bring productivity benefits. The paper generated considerable discussion by the meeting, in particular the meeting suggested that when discussing satellite VHF the community should focus on explaining the potential benefits that could not be achieved via other means. There was general support to continue studies regarding the full 117.975-137 MHz band. Studies should properly address the challenges ahead, while distinguishing between actions that will have to be undertaken by the aeronautical community at any appropriate time (like the above noted topics), with respect to those other mandatory actions to be done on time for WRC23 AI 1.7 (technical compatibility studies). Care should be taken that aviation proposals should not be structured such that any remaining issue with digital datalink in the 136-137 MHz band results in objection to any AMS(R)S allocation at all.

3.2.3.2	WP13 presented the results of technical studies related to the spectrum sharing of Space-based VHF system with AM(R)S within the 117.975 – 137 MHz frequency band. The paper focused on assessing Doppler impacts, concluding that for the envisioned satellite system it would not be an issue. The paper also provides evidence that Space Based VHF systems will be able to handle Doppler without a need for aircraft avionics modifications.

3.2.3.3	WP14 proposed an approach to minimize the impact on existing systems while still being able to operate an economically viable Data Link Service under the potential future AMS(R)S allocation in the VHF frequency band.

3.2.3.4	During FSMP WG/13 it was agreed to create a Satellite VHF Coordination Group (CG-SV) to develop a set of frequently asked questions (FAQs) regarding the AMS(R)S (Aeronautical Mobile Satellite (R) Services) concept. IP05 (including attachment) contains the current collection of questions and answers (Q&As). This material will be updated regularly to be delivered in different ICAO and/or ITU groups that are working on the WRC-23 Agenda item 1.7 topic.

3.2.3.5	The presentations of the papers resulted in considerable discussion. In the end the meeting noted a number of points:

- Consistent with the agenda item Resolution, implementation shall be made without changes to VDL Mode 2 avionics, and the proponents believe, subject to confirmation, this will be possible.

- In the view of the proponents, implementation of the space based VHF system could require configuration changes to existing VDL Mode 2 avionics radios that wanted to participate in the satellite system, and making those configuration changes should be fairly simple. Those changes would not impact the ability of those radios, or other non-modified VDL Mode 2 radios, to operate with terrestrial VDL Mode 2 networks. The group agrees that further investigation on this aspect is needed, including whether the envisioned changes are within the scope of the Resolution.

- No changes to VDL Mode 2 emissions would be required

3.2.3.6	Representatives from the current terrestrial VDL Mode 2 service providers (i.e., SITA and Collins) indicated that a space-based VDL Mode 2 service could be feasible.  While one service provider was confident of the implementation, the other noted it still had outstanding questions that had yet to be addressed for both the technical implementation and the protection of, and coordination with, existing terrestrial services.

3.2.4	Liaison to WP5B

3.2.4.1	WP03 contained a Liaison Statement from ITU-R WP5B requesting information regarding the aeronautical VHF coordination process(es) between AM(R)S assignments and between AM(R)S and AM(OR)S assignments, with the goal of understanding how possible new AMS(R)S assignments could be included.  The document also included a copy of the Preliminary draft new Report ITU-R M.[Space-VHF].

3.2.4.2	WP16 provided a proposed response to the liaison from ITU-R WP5B contained in WP03. The proposed responds answered specific questions and provided information on the elements being studied. The material was included in the drafting group discussions outlined below.

3.2.4.3 During discussions, some participants made emphasis that the ongoing studies at the ITU address the allocation to AMS(R)S and ITU would not conclude on endorsing any particular system implementation, nor assigning frequencies.

3.2.5	To progress the material a drafting group was formed which met several times during the meeting and via Email correspondence. The agreed output is shown in Appendix I.

3.3	WRC-23 AI 1.8 Resolution 155/FSS for UAS

3.3.1	WP05 provides background on the provisions in ITU regarding the treatment of allocations which are used or planned to be used for Aeronautical Radiocommunication. The purpose was to respond to questions during FSMP WG/14 in the context of the potential use of FSS for C2 links for the operation of unmanned aircraft systems (UAS).  In particular, a debate on the use of recognized “aeronautical spectrum” versus “non-aeronautical spectrum” for safety purposes was highlighted. After discussion the meeting agreed that under certain specific conditions non-safety radiocommunication services could be used to support aeronautical safety communications, however it would need to be considered on a case-by-case basis.

3.4	WRC-23 AI 1.9 Wideband HF

3.4.1	No contributions were received on this topic.

3.5	WRC-23 AI1.10 Non-safety AMS

3.5.1	No contributions were received on this topic.

3.6	WRC-23 AI9.2 Radio Regulations clean-up

3.6.1	WP19 identified various inconsistencies within the Radio Regulations with some current aeronautical operating practices and technical requirements. Where those inconsistencies do not affect other services, the paper proposes how the current text could be modified to bring it into line with current operational practices. The meeting agreed that the paper offered a good basis for developing an element in the ICAO WRC-23 Position on agenda item 9.2 and participants were encouraged to bring specific proposals to FSMP-WG/16 [AI 15-02].

3.7	Other

3.7.1	WP01 was a liaison from ITU-R WP5B requesting information on the appropriate parameters to use for the protection of aeronautical Non-Directional Beacon receivers. This information is required for impact studies regarding Wireless Power Transmission for mobile device charging using non-beam magnetic inductive and magnetic resonant charging inter alia in the frequency band 315 - 405 kHz. Given the short deadline, Email coordination was performed with experts from the Navigation Systems Panel, and elements for a reply liaison are contained in Appendix H.

3.7.2	IP06 noted that Fixed Service (FS) systems were moved from the 24 GHz band after WRC-19 to a number of frequency bands including the 32 GHz band used by enhanced vision systems (EVS)/enhanced flight vision systems (EFVS). As a result, there are ongoing studies in ITU where ICAO support is needed. To help with those studies, the presentation provided information on one EFVS including operating characteristics and expected performance. The meeting appreciated the information.

3.7.3	WP21 expanded on the issues raised in IP06, noting that in addition to the frequency band 31.8-33.4 GHz (32 GHz band), the 95-100 GHz is also allocated to Radionavigation and FIXED services and being considered for EFVS. It was noted that EFVS are already identified in ICAO Annex 6, however specific frequencies are not listed. Finally, it was highlighted that RR No. 5.547A requires administrations to work to preclude interference to 32 GHz airborne radars. The paper provided an approach for a coordination process that accommodates FS and EFVS in the 31.8-33.4 GHz band, taking into account that the EFVS requires 320 MHz of bandwidth, indicating that a similar approach may be possible in other bands. To be considered in ITU studies, however, the frequency bands need to be recognized as bands identified for EFVS/EVS applications through a clear position from ICAO. After discussion the meeting agreed that this would most appropriately be done through additions to the Spectrum Handbook (Doc. 9718). The following links were provided for information:

Preliminary draft revision of Recommendation ITU-R F.1520-3 - Radio-frequency arrangements for systems in the fixed service operating in the band 31.8-33.4 GHz

https://www.itu.int/md/R19-WP5C-C-0274/en

M.1466 : Characteristics of and protection criteria for radars operating in the radionavigation service in the frequency band 31.8-33.4 GHz

https://www.itu.int/rec/R-REC-M.1466

Report CCE FH vs EFVS_32GHz

Report of the EMC Consultation Committee Electromagnetic Compatibility (ECC)

Coexistence between the fixed service and the radio navigation system in the frequency band 31.8- 33.4 GHz

Part A: Interference from the fixed service on the EFVS

https://www.anfr.fr/fileadmin/mediatheque/documents/etudes/2019-10-17_RapportCCE_FHvsEFVS_PartieA_FinalV3.pdf

4.	Agenda Item 4 – Radio Altimeter issues

4.0.1	IP03 provided radio altimeter (RA) interference susceptibility characteristics as measured at Japanese Sub-6 frequency conditions. Japanese Sub-6 frequency bands are 3.7 GHz–4.1 GHz and 4.5 GHz–4.6 GHz, (i.e., both sides of the RA band with the 100 MHz guard band). In particular in-band and out-of-band electromagnetic interference susceptibility was measured for 12 RAs used by large fixed-wing aircraft and 8 RAs used by medium/small fixed-wing aircraft and helicopters. The measurement procedures and the interference criteria were fundamentally same as those described in the RTCA 5G Interference Assessment Report, and ARINC429 altitude output or analog voltage output were obtained and evaluated. The meeting appreciated the information and in particular that there were now altimeter susceptibility results measured by different stakeholders that displayed similar levels. The meeting asked to be kept informed as data analyses proceed.

4.0.2	IP07 provided an initial comparison of published RF performance data for aviation and non-aviation receivers operating adjacently in the C-band. The intent was to show that in terms of adjacent band signal rejection, the radio altimeters were in general better than the performance specified in the 3GPP mobile standards. The meeting appreciated the analysis and noted it would be useful material to counter current criticism in some fora that “altimeters are looking outside their band”.

4.0.3	IP09 noted that ICAO has received studies from several States and organizations regarding the potential for 5G interference to radio altimeters. Those studies generally concluded that some models of radio altimeters will not operate as required if new cellular broadband technologies (5G) are deployed in frequency bands close to the frequencies used by radio altimeter’s operation (4.2-4.4 GHz). In reply to a request from the Secretariat of the ICAO HQ and considering the importance of this topic globally, the RADALT AG and ICAO MID office developed the draft guidance material presented in the IP and provided to FSMP for their information. The meeting appreciated the material, in particular that it addressed both technical issues and the operational impacts, and suggested that it could be updated and form the basis for an electronic bulletin or manual on the topic. All participants were given to the action [AI 15-01] to review the information regarding their State, and provide any updates as soon as possible to allow them to be reflected in the published version of the ICAO MID document. Those update should be sent to Mie Utsunomiya at (mutsunomiya@icao.int).

4.1	Report from correspondence group on radio altimeters (CG-RA)

4.1.1	IP08 provided a report of CG-RA and had the stated purpose to give a “full” picture of the issue and provide some guidance examples that could minimise the impact on Radio-Altimeter for all stakeholders. In particular it provided a catalogue of recent meetings, studies and test efforts on the topic and references where more information could be found. The meeting greatly appreciated the information and a large discussion ensued. It was noted that the mobile community in France was pushing for all radio altimeter related restrictions to be removed by July 2023. The meeting asked the CG to continue its work.

4.2	National efforts to implement broadband mobile near 4200-4400 MHz

4.2.1	Mitigation measures - No contributions were received on this topic

4.2.2	Safety cases/compatibility analyses - No contributions were received on this topic

5.	Agenda Item 5 -- 5 GHz Planning

5.1 	AeroMACS

5.1.1	No contributions were received on this topic

5.2	Global UAS/Remotely Piloted Aircraft Systems (RPAS) channel plan

5.2.1	WP10 was in response to FSMP Action Item 11-06 for members ‘to consider RPAS use of more than just the “2x10 MHz” for AMS(R)S communications in the 1.5/1.6 GHz L-Band’. The paper notes that at FSMP-WG/13 a number of criteria were identified to assess the suitability of a frequency band (see Section 5.5 of the FSMP WG/13 Meeting Report) for AMS(R)S, and suggests that FSMP responds to the RPAS panel that satellite-based RPAS C2 Links may be considered for operation within the MSS in the frequency bands 1 525-1 544/1 545-1 559 MHz and 1 626.5-1 645.5/1 646.5-1 660.5 MHz, provided there is a capability for the system to fall back to operating only in a sub-part of this band where priority is given within the Radio Regulations in accommodating the spectrum requirements of the AMS(R)S. The meeting agreed in principle the RPAS use of these bands, based on,  the “generic AMS(R)S” use of the band (i.e., “Option 2” as documented in paragraph 5.5 of the FSMP-WG/13 report, now included in Annex 10, Volume III), however that would require a change to Annex 10, Volume V, Chapter 5. The trigger to begin such a change would be a request from the RPAS Panel.

5.2.2	IP02 (with attachment) provides an update on the discussion between EUROCAE WG-105 and RTCA SC-228 in regard to a compatible technical solution for the shared use of the 5030-5091MHz band by terrestrial, high-altitude airborne relay, and satellite based C2 Links. The paper included two options that were still being considered (splitting the band by frequency between AMS(R)S and AM(R)S or splitting the band in time between the two services). The meeting appreciated the information.

6.	Agenda Item 6 -- Interference from Non-Aeronautical Sources

6.1	Mobile service communications adjacent to 1518 MHz satellite communications frequency band (see action item 14-04)

6.1.1	WP08 provided proposed edits to the text of the draft Guidelines/Information Note attached in Appendix H of the Report from FSMP-WG/12.  The paper also suggests that consideration be given by FSMP on how the draft Guidelines/Note, once finalised and agreed, may be circulated by ICAO (e.g., through an ICAO State Letter or via another means). After discussion the meeting agreed that the document would be updated to:

- Add back the example of an administration only protecting SATCOM use at the gates;

- Include any updated material from the WP4C (September) and WP5B (November) meetings; and

- a revised version will be submitted early for FSMP-WG/16. That version will show track-changes versus the version from WG/14 with the comments from France to that meeting accepted.

Regarding how to disseminate the final product, it was mentioned that an electronic bulletin may be most appropriate.

6.2	LED lighting systems

6.2.1	No contributions were received on this topic

7. 	Agenda Item 7 -- Revision of ICAO Frequency Spectrum Handbook (Doc 9718)

7.0.1	No papers were received on the Spectrum Handbook topic, however a general discussion was had to re-baseline the material that had been received, and to solicit new contributions for FSMP-WG/16. General details on the topics discussed are contained in Appendix J. A correspondence group was established and members should contact Andy Roy (ACR@asri.aero) to participate.

7.1	Restructure of Volume I

7.1.1	No contributions were received on this topic

7.2	Update of Volume II

7.2.1	No contributions were received on this topic

8.	Agenda Item 8 – Any other business

8.1	WP07 reported that FSMP WG/14 had agreed on a set of draft new proposed SARPs for WAIC and that those have been liaised with the Navigation Systems Panel (NSP) and the Communications Panel – Data Communication Infrastructure Working Group (CP-DCIWG) for comments.  The draft new SARPs were liaised on 11 May 2022, with 30 June indicated as deadline for comments.  As of 12 August, no comments have been received. The Secretariat has undertaken an editorial review of the draft new WAIC SARPs package.  The draft SARPs as submitted to FSMP WG/14 had been written for insertion into Annex 10 Vol III, however FSMP WG/14 agreed that the WAIC SARPs would be a better fit for Annex 10 Volume V.  The editorial review focused mainly on this aspect, while also taking note of the guidance provided in the “ANC Guide to the drafting of SARPS and PANS” and Assembly Resolution 39-22.  Some minor edits were also made to the Impact Assessment and Implementation Plan (see Annex 2 of Attachment) and the Validation Statement (see Annex 3 of Attachment). The meeting performed a final review of the draft new WAIC SARPs package, to facilitate a decision by FSMP/3, which will be held on 7 September 2022. The agreed material is contained in Appendix F.

8.2	IP01 contains the report of the June 2022 meeting of the ICAO NSP Spectrum Working Group, SWG. The following topics discussed in the SWG have particular relevance for the FSMP:

‒	Review of LDACS SARPs provided in the frame of inter-panel coordination by ICAO NSP

‒	Status of the development of revised LDACS versus DME/TACAN Compatibility Testing Plan

‒	Review of the recently published ICAO Doc 9718, Vol. II by ICAO NSP

‒	Proposed amendment to Annex 10, Volume V consequential to the recent publication of the update of DOC 9718, Vol. II. This proposal was circulated to the FSMP for Email coordination with a deadline of October 1.

The meeting appreciated the update.

8.3	IP10 provides a summary of, and invites industry to provide inputs on, the contents of the Innovation, Science and Economic Development Canada (ISED) notice seeking comments regarding an application by Ligado Networks (Canada) Inc. (Ligado Canada) for authority to provide Ancillary Terrestrial Component (ATC) mobile services over a portion of its licensed Mobile Satellite Services (MSS) spectrum in the L-band (1526-1536 MHz in the MSS downlink, and 1627.5-1637.5 MHz and 1646.5-1656.5 MHz in the MSS uplink). The paper noted that in a similar proceeding in the United States, the aviation industry expressed strong concerns with Ligado’s change to the usage of its spectrum from satellite to also providing terrestrial due to concerns with interference to GPS and satellite safety systems, and that the proposal for Canada was for significantly higher power. The meeting appreciated the information.

8.4	To assist in the discussion planned for FSMP/3 (7 September, 2022), the FSMP job cards were reviewed. Based on that review, the meeting made suggested revisions and those will be circulated to the Panel members for their consideration.

9.	Date of next meeting and administrative issues

9.1	FMSP-WG/16 is scheduled for [13-24 February][20 February-3 March], 2023 in hybrid (in-person plus virtual) format at the ICAO Regional Office in [Bangkok, Thailand][Cairo, Egypt]. It should be noted for virtual participants that the working hours will be the standard 0900-1700 in the local [Bangkok][Cairo] time zone. That meeting will include a Workshop which will be held the first two days. For planning purposes FSMP-WG/17 is tentatively planned for [21 August-1 September][28 August-8 September] at the ICAO Regional Office in [Cairo, Egypt][Bangkok, Thailand] again in a hybrid format and with a Workshop the first two days.

9.2	Due to the pending retirement of FSMP-WG Rapporteur Mr. Mike Biggs, a meeting of FSMP Panel Members was held to select a new Rapporteur. Members agreed on the selection of Mr. Andrew Roy (ASRI) as the new Rapporteur, and Mr. Christian Fleury (France) as the new Vice-Rapporteur.

9.3	The meeting was reminded that as agreed by FSMP-WG/12, papers for FSMP-WG/16 are due one week prior to the meeting. In exceptional circumstances, a framework detailing at minimum the paper summary and introduction will be provided one week prior to the meeting, with the full paper provided no later than one work day before the start of the meeting. Any papers received after this deadline will not be accepted unless agreed by the meeting on the first day. No papers received after the first day of the meeting will be accepted.

APPENDICES

Appendix A – Agenda

Appendix B – List of Working Papers, Information Papers and Flimsies

Appendix C – List of Participants

Appendix D – Action Item List

Appendix E – Draft modifications to the ICAO WRC-23 Position

Appendix F  - Draft WAIC SARPS package

Appendix G – Elements for a proposed Liaison to ECHO project

Appendix H– Elements for RLS to 5B on NDB parameters

Appendix I – Elements for RLS to WP5B on WRC-23 agenda item 1.7

Appendix J – Discussion of ICAO Spectrum Handbook

APPENDIX A

INTERNATIONAL CIVIL AVIATION ORGANIZATION

FIFTEENth working Group Meeting of the

Frequency Spectrum Management Panel

(FSMP-WG/15)

(Hybrid meeting, Montréal, QC, Canada, 22 August – 1 September, 2022)

AGENDA

Agenda Item 1		Opening and Working Arrangements

a)	Action Item Review

b) 	Status of tasks identified on Job Cards

Agenda Item 2		ICAO WRC-23 Position

a) 	Identified conflicts between administration preliminary proposals and ICAO Position

b)	Discussion of possible future agenda item proposals from administrations

c)	Modifications/Updates to the ICAO WRC-23 Position

Agenda Item 3		Development of (planned) Material for ITU-R Studies on:

a)	WRC-23 AI1.6 Suborbital Vehicles

b)	WRC-23 AI1.7 VHF AMS(R)S

c)	WRC-23 AI1.8 FSS for UAS

d)	WRC-23 AI1.9 Wideband HF

e)	WRC-23 AI1.10 Non-safety AMS

f)	WRC-23 AI9.2 Radio Regulations clean-up

g)	Other

Agenda Item 4		Radio Altimeter issues

b)	  Report from correspondence group on radio altimeters (CG-RA)

a)	  National efforts to implement broadband mobile near 4200-4400 MHz

- Mitigation measures

- Safety Cases/Compatibility Analyses

Agenda Item 5		5 GHz Band Planning

a)	AeroMACS

b)	Global UAS/Remotely Piloted Aircraft Systems (RPAS) channel plan

Agenda Item 6		Interference from Non-Aeronautical Sources

a)	Mobile service communications adjacent to 1518 MHz satellite communications frequency band (see action item 14-04)

b)	LED lighting systems

Agenda Item 7		Revision of ICAO Frequency Spectrum Handbook (Doc 9718)

a)	Restructure of Volume I

b)	Update of Volume II

Agenda Item 8		Any Other Business

APPENDIX B

List of Papers

“*” papers available on closed website(s)

List of Working Papers

| Paper # | Source | Title | Agenda Item |
| --- | --- | --- | --- |
| 1 | M. Biggs | Liaison Statement from ITU-R WP5B on parameters of non-directional beacon (NDB) | 3g |
| 2 | A.Roy | ICAO frequency coordination process flow and WRC-23 Agenda Item 1.7 (WRC-23) | 3b |
| 3 | Secretary | Liaison Statement from ITU-R WP5B: Studies on WRC-23 agenda item 1.7 | 3b |
| 4 | Secretary | Liaison Statement from ITU-R WP5B: WRC-23 agenda item 1.6 Regulatory, operational, and technical studies of radiocommunications for suborbital vehicles | 3a |
| 5 | F. Butsch | Safety aspects of Aeronautical Radiocommunication in the light of provision of ITU | 3c |
| 6 | M. Biggs | Proposed modifications to the ICAO WRC-23 Position | 2c |
| 7 | Secretary | Secretariat review of the draft WAIC SARPs | 8 |
| 8 | P. Deedman | IMT-MSS L-band draft ICAO guidelines, provides proposed edits to the text of the draft Guidelines/Information Note attached in Appendix H of the Report from FSMP-WG/12 | 6a |
| 9 | P. Deedman | PROPOSAL TO UPDATE THE ICAO WRC-23 POSITION | 2c |
| 10 | K. Masrani | L-band MSS frequency use for C2 Link SARPs | 5b |
| 11 | ENAIRE, Indra and SITA | Proposed position of ICAO on WRC-23 AI 1.7 VHF AMS(R)S for the ITU R CPM | 3b |
| 12 | ENAIRE, Indra and SITA | ANSP and Airline operational requirements to be supported by Satellite-Based CNS concept | 3b |
| 13 | ENAIRE, Indra and SITA | SB-VHF technical studies, spectrum sharing and Doppler effect | 3b |
| 14 | SITA, ENAIRE, and Indra | Space Based VHF Data Link and Terrestrial VHF Data Link co-existence in the 136 – 137 MHz Frequency Band | 3b |
| 15 | ENAIRE, Indra and SITA | Frequency assignments planning and coordination exercises on AI 1.7 VHF ASMS(R )S | 3b |
| 16 | ENAIRE, Indra and SITA | Proposed response to the Liaison Statements from ITU-R Working Party 5B on WRC-23 AI 1.7 VHF AMS(R)S | 3b |
| 17 | J. Mettrop | Current Concerns with the Studies undertaken with Respect to WRC-23 Agenda Item 1.7 | 3b |
| 18 | J. Mettrop | Coordination of Systems Operating in the Aeronautical Mobile Satellite (Route) Service with Systems operation in the Aeronautical Mobile (Route) Service within the Frequency Band 117.975-137 MHz | 3b |
| 19 | J. Mettrop | Proposed Updates to the Provisions Related to Aeronautical Services in the Radio Regulations under WRC-23 Agenda Item 9.2 | 3f |
| 20 | J. Andre | POTENTIAL SPECTRUM NEEDS FOR SUBORBITAL VEHICLES | 3a |
| 21 | C.Fleury | EFVS in the Frequency band 31.8-33.4GHz (Ka) and the 95-100GHz (W), The frequency band 31.8-33.4 GHz (32 GHz band) and the 95-100 GHz are allocated to Radionavigation and FIXED services | 3g |
| 22 | J. Andre | Possible ITU-R coordination for the VHF per satellite | 3b |
| 23 | A.Roy | UPDATES TO ICAO WRC-23 POSITION | 2c |
| 24 | C.N. Yeo, H. Foo, J. Chong and M. Kelly | COORDINATION OF VHF FREQUENCY ASSIGNMENTS IN APAC REGION | 3b |
| 25 |  |  |  |
| 26 |  |  |  |
| 27 |  |  |  |
| 28 |  |  |  |
| 29 |  |  |  |
| 30 |  |  |  |

List of Information Papers, Presentations and Flimsies

| Paper # | Source | Topic | Agenda Item |
| --- | --- | --- | --- |
| 1 | F. Butsch | Meeting Report of the NSP SPECTRUM WORKING GROUP (SWG) | 8 |
| 2 | A.Malaga, M. Neale, D. Nellis | Compatible AM(R)S and AMS(R)S operation supporting Remotely Piloted Aircraft Systems in the 5 030-5 091 MHz frequency spectrum | 5b |
| 3 | S. Futatsumori | Radio altimeter interference susceptibility test results at Japanese Sub-6 5G frequency conditions | 4 |
| 4 | ENAIRE, Indra and SITA | Sharing Studies and operation for Space-based VHF Communications in the 117.975 to 137 MHz Frequency Band | 3b |
| 5 | ENAIRE on behalf of CG-SV group | Questions and Answers (Q&As) regarding the AMS(R)S (Aeronautical Mobile Satellite (R) Services) concept | 3b |
| 6 | N. Moulin | Progress of EFVS works in the 32Ghz band | 3g |
| 7 | N. Shrout | Comparative Performance Review of C-Band Receivers | 4 |
| 8 | C.Fleury | Report from the CG-RA | 4a |
| 9 | Secretary | ICAO MID Draft Guidance on Safeguarding measures to protect Radio Altimeter from potential harmful interference from Cellular 5G Communications | 4 |
| 10 | J. Cramer | Ligado Request for Ancillary Terrestrial Component Authority in Canada | 8 |
| 11 |  |  |  |
| 12 |  |  |  |

| PRESENTATIONS |  |  |  |
| --- | --- | --- | --- |
| 1 |  | For IP02 | 5b |
| 2 |  | For IP04 | 3b |

| FLIMSIES |  |  |  |
| --- | --- | --- | --- |
| 1 | Meeting | Proposed modifications to the ICAO WRC-23 Position | 2c |
| 2 | C. Tourigny | Elements for RLS to WP5B on AI1.7 | 3b |
| 3 | M. Biggs | Elements for RLS to WP5B on NDBs | 3g |
| 4 | M. Biggs | Proposed disposition of action items | 8 |
| 5 | J. Andre | Elements for LS to project ECHO | 3a |
| 6 | A.Roy | Spectrum Handbook updating | 7 |
| 7 |  |  |  |
| 8 |  |  |  |
| 9 |  |  |  |
| 10 |  |  |  |
| 11 |  |  |  |

APPENDIX C

Fifteenth meeting of the Working Group of the Frequency Spectrum Management Panel (FSMP-WG/15)

Montreal, Canada,    22 August – 1 September, 2022

LIST OF PARTICIPANTS

| COUNTRY/ ORGANIZATION | * | NO | NAME | DESIGNATION-JOB TITLE/ TELEPHONE | EMAIL |
| --- | --- | --- | --- | --- | --- |
| AUSTRALIA |  |  | Matthew Kelly | Air Services Australia | matthew.kelly@airservicesaustralia.com |
| BRAZIL | * |  | Vahe Antoine Yaghdjian | Telecommunications Engineer Department of Air Space Control – DECEA Technical Subdepartment – Technical Coordination Division Rio de Janeiro - Brazil Tel: +21- 2101–6487/21-99955–3305 | vahevay@decea.gov.br vahe.antoine@gmail.com |
|  | * |  | Gerson Monteiro Siqueira | CNS Advisor Department of Air Space Control – DECEA Technical Subdepartment Rio de Janeiro - Brazil Tel: +55 21- 2101–6665 | monteirogms@decea.gov.br |
|  |  |  | Carlos Evangelista da Silva Junior | Specialist on Regulation of Telecommunications Services ANATEL - Agência Nacional de Telecomunicações | carlosjunior@anatel.gov.br |
|  |  |  | Valeria Leite | Aeronautics and Space Institute Brazil - FAB | valeriavcmnl@fab.mil.br |
| CANADA |  |  | Seyed Rastaghi | Nav Canada | Seyed.Rastaghi@navcanada.ca |
|  |  |  | John Taylor | Aeronautical Spectrum Regulations Specialist Canada Tel: +1-613-993-4061 | john@johntaylorconsulting.ca |
|  | * |  | Abed Ferr | Nav Canada | Abed.ferr@navcanada.ca |
| CHINA |  |  | Zhao Liang | FSMP Member Aviation Data Communication Corporation of CAAC | zhaoliang@adcc.com.cn |
|  |  |  | Cui KaiTao | The Second Research Institute of CAAC Civil Aviation Administration of China | cuikaitao@caacsri.com |
|  |  |  | Liu Rui | Aviation Data Communication Corporation of CAAC | liur@adcc.com.cn |
| FRANCE | * |  | Christian Fleury | IESSA - Spectrum Manager France DGAC/ Direction des services de la Navigation Aérienne Toulouse – France Tel: +33-0-562-14-55-41 Cell: +330625164420 | christian.fleury@aviation-civile.gouv.fr |
|  | * |  | Ivan Martin | Thales SIX | ivan.martin@thalesgroup.com |
|  |  |  | Josep GINE i CORTIELLA | ATR | JOSEP.GINE-I-CORTIELLA@atr-aircraft.com |
|  | * |  | Jérome ANDRE | ANFR | Jerome.ANDRE@anfr.fr |
|  |  |  | Benoit LOUVET | Thales Alenia SPACE | Benoit.Louvet@thalesaleniaspace.com |
|  |  |  | Christine Mengelle | Thales Alenia Space | christine.mengelle@thalesaleniaspace.com |
|  |  |  | Nicolas Moulin | Dassault Aviation | Nicolas.Moulin@dassault-aviation.com |
| GERMANY |  |  | Dr. Felix Butsch | Head of Frequency Management DFS Deutsche Flugsicherung GmbH, (German Air Navigation Services) SIS/MF (Spectrum and Frequency Management Office) - Am DFS-Campus 10 D-63225 Langen, GERMANY Tel: +496103707-1533 Cell: +49-172 6677682 | Felix.Butsch@DFS.DE |
|  |  |  | Martin Weber | Federal Network Agency, Germany | Martin.weber@bnetza.de |
|  |  |  | René Woerfel | AUDENS Telecommunications, Germany | Rene.woerfel@audens.de |
|  |  |  | Christian Beitz | Federal Supervisory Authority for Air Navigation Services, Germany | Christian.Beitz@baf.bund.de |
|  |  |  | Sergio Bovelli | Airbus, Germany | sergio.bovelli@airbus.com |
| INDIA |  |  | S. K. Mallick | GM (CNS-Automn) | gmcnsauto@aai.aero; skmalik@aai.aero |
|  |  |  | A. S. Yadav | Jt. GM (CNS) | ​asyadav@aai.aero |
|  |  |  | C. P. Dwivedi | Jt. GM (CNS) | cpdwivedi@aai.aero |
| INDONESIA |  |  | Abdul Aziz | Air Navigation Inspector - Directorate of Air Navigation (DGCA) Indonesia Tel: +6282113222432 | azizsabdul@gmail.com |
|  |  |  | Bimantoro | Air Navigation Inspector - Directorate of Air Navigation (DGCA) Indonesia Tel: +6285945063999 | bimzink@gmail.com |
| JAPAN |  |  | Katsuyuki Arakawa | Special Assistant to the Director JAPAN Civil Aviation Bureau Phone: +81-3-5253-8756 | arakawa-k24fe@mlit.go.jp |
|  |  |  | Yoshiaki Saeki | Chief Ministry of Internal Affairs and Communications TEL: +81-3-5253-5902 | y.saeki@soumu.go.jp |
|  |  |  | Fumie Ono | Research Manager National Institute of Information and Communications Technology (NICT) Phone: +81-42-327-6816 | fumie@nict.go.jp |
|  |  |  | Toshio Nomi | Senior Engineer Japan Radio Air Navigation Systems Association (JRANSA) Tel: +81-3-5214-1353 | nomi.toshio@jransa.or.jp |
|  |  |  | Naruto Yonemoto | Principal Researcher Japan Electronic Navigation Research Institute (ENRI) Tel: +81-422-41-3174 | yonemoto@mpat.go.jp |
|  |  |  | Hiroyuki Tsuji | Director Japan National Institute of Information and Communications Technology (NICT) Tel: +81-42-327-6034 | tsuji@nict.go.jp |
|  |  |  | Shunichi Futatsumori | Chief Researcher Japan Electronic Navigation Research Institute (ENRI) Tel: +81-422-41-3174 | futatsumori@mpat.go.jp |
|  |  |  | Furen Tang | Senior Researcher Mitsubishi Research Institute, Inc. Phone: +81-3-6858-3729 | furen_tang@mri.co.jp |
|  |  |  | Takayuki WADA | Deputy Director Ministry of Internal Affairs and Communications TEL: +81-3-5253-5902 | t5-wada@soumu.go.jp |
|  |  |  | Hiroki Tsukada | Official Ministry of Internal Affairs and Communications TEL: +81-3-5253-5902 | h.tsukada@soumu.go.jp |
| ROMANIA |  |  | Aurelian Sorinel Calinciuc | Spectrum Manager Vice-Chair ITU-R SG5 Tel: +40721291274 | aurelian.calinciuc@ancom.org.ro |
| SINGAPORE | * |  | Cheng Nam YEO | Consultant Aeronautical Telecommunications and Engineering Civil Aviation Authority of Singapore Tel: +65-9776-9376 | yeo_cheng_nam@caas.gov.sg |
|  |  |  | Henry FOO | Deputy Director (Aviation Technology and Development) Civil Aviation Authority of Singapore Tel: +65-8161-1958 | henry_foo@caas.gov.sg |
|  | * |  | John CHONG | Head (Masterplan & Enterprise Architecture) Civil Aviation Authority of Singapore Tel: +65-9369-6482 | john_chong@caas.gov.sg |
|  |  |  | Kok Pin PUAH | Head (Aviation Technology and Development) Civil Aviation Authority of Singapore Tel: +65-9116-0541 | puah_kok_pin@caas.gov.sg |
| SOUTH AFRICA |  |  | Lisa Cokisa Tele | Senior Engineer/Project Manager ATNS Head Office Johannesburg - South Africa Tel: +2711 6071134 Cell: +2783 779 3920 | lisat@atns.co.za |
|  |  |  | Takalani Tshikalaha | ATNS | TakalaniT@atns.co.za |
|  |  |  | Thomas Matabane | Frequency Spectrum, CNS CAA South Africa Tel: +27 11 545 1066 Cell: +27 71 801 1507 | Matabanet@caa.co.za |
| SPAIN | * |  | Manuel Garcia Martín | División de Comunicaciones Jefe de División ENAIRE Tel: +34 913 213 261 | mangarcia@enaire.es |
|  |  |  | Mario Garcia | ENAIRE | jmgcano@enaire.es |
|  | * |  | José Luis Chinchilla | STARTICAL | jlchinchilla@indra.es |
|  |  |  | Miguel Muñoz | STARTICAL | mmunozm@startical.com |
|  |  |  | Jose Luis Ruiz Dou | INDRA | jruizd@indra.es |
|  |  |  | Joan Manuel Cebrian | INDRA | jcebrian@indra.es |
|  |  |  | Julián Seseña | INDRA | jsesena@hi2.es |
| THAILAND |  |  | Chavalit Ithiapa | Officer, Communications, Navigation and Surveillance Standards Division Air Navigation Services Standards Department Civil Aviation Authority of Thailand Tel: +66 (0) 2568-8800 ext. 2510 Fax: +66 (0) 2568-8847 | chavalit.i@caat.or.th |
| UNITED KINGDOM | * |  | John Ernest Mettrop | Spectrum Policy Specialist UK Civil Aviation Authority Tel: +44-0-1293 573477 | John.Mettrop@caa.co.uk |
|  |  |  | Stephen Parry | Spectrum Manager, UK NATS Tel: +44 1489 616 454 | stephen.parry@nats.co.uk |
|  |  |  | Paul Deedman | Inmarsat | Paul.deedman@inmarsat.com |
| UNITED STATES | * |  | Michael Biggs | Senior Electronics Engineer Federal Aviation Administration USA Tel:+1-202-267-8241 | michael.biggs@faa.gov |
|  | * |  | Chris Tourigny | Federal Aviation Administration USA | chris.tourigny@faa.gov |
|  |  |  | Donald Nellis | Electronics Engineer Federal Aviation Administraton USA Tel. +1 202 267 9779 | Donald.Nellis@faa.gov |
|  |  |  | Damon Ladson | Technology Policy Advisor United States of America Tel: +1 301 728 7253 | dladson@hwglaw.com |
|  |  |  | Don Jansky | Jansky-Barmat Telecommunications Inc. USA Tel: +1 202-415-1834 | don@jansky-barmat.com |
|  |  |  | Michael Neale | Aces Inc. | michael.neale@aces-inc.com |
|  |  |  | Dr.. Alfonso Malaga | Aces Inc | Alfonso.malaga@aces-inc.com |
|  | * |  | Scott Kotler | Director, Technical Regulatory Affairs Lockheed Martin Arlington, VA 22202 USA Tel: +1-703-789-3923 | scott.kotler@lmco.com |
|  |  |  | Bruce Lamb | Telecoms Specialist NTIA Tel: +1 202-317-0036 | blamb@ntia.gov |
|  |  |  | Gregory Baker | Federal Communications Commission USA | Gregory.Baker@fcc.gov |
|  |  |  | Louis Bell | Electronics Engineer Federal Communications Commission Tel. +1 202 418 1641 | louis.bell@fcc.gov |
|  |  |  | Eric P.Y. Lee | Electronics Engineer Terrestrial Team Lead (Acting), International Spectrum Policy Division NTIA - Office of Spectrum Management\] Mobile: +1 (202) 360-0836 | elee@ntia.gov |
|  |  |  | John Nelsen | John A Nelsen NELSEN Advisors LLC P.O. Box 553 Wayne, PA 19087-0553 USA Tel: +1 610-613-2058 | nelsenj@gmail.com |
|  |  |  | Robert J. Kerczewski | The Aerospace Corporation 2310 E. El Segundo Blvd. El Segundo, CA 90245-4609 United States Tel: +1 440 832 9295 | robert.j.kerczewski@aero.org |
|  |  |  | Sai Kalyanaraman, Ph.D. | Technical Fellow, R&T, Avionics COLLINS AEROSPACE 855 35th Street NE, Cedar Rapids, Iowa, 52498 U.S.A Tel: +1 319-263-8152 | Sai.Kalyanaraman@collins.com |
|  | * |  | Joseph Cramer | Regional Director The Boeing Company Tel: +1-703-465-3486 | Joseph.Cramer@Boeing.com |
| ASECNA |  |  | Eric Damiba | CNS and MET Senior Manager ASECNA | damibaeri@asecna.org |
| ASRI | * |  | Andrew Roy | Director of Engineering Aviations Spectrum Resources Inc. Tel: +1-443-951-0340 | ACR@asri.aero |
|  | * |  | Kris Hutchison | President Aviations Spectrum Resources Inc. Tel: +1 443 951 0322 | KEH@asri.aero |
|  | * |  | Nic Shrout | Aviations Spectrum Resources Inc. | NJS@asri.aero |
| EUROCAE |  |  | Anna Guégan | Technical Programme Manager EUROCAE Le Triangle 9-23 rue Paul Lafargue 93200 Saint-Denis France | anna.guegan@eurocae.net |
| EUROCONTROL |  |  | Raffi Khatcherian | Head of International Spectrum Management EUROCONTROL Tel: +32 478241421 | raffi.khatcherian@eurocontrol.int |
|  | * |  | John Micallef | EUROCONTROL | john.micallef@eurocontrol.int |
| IATA | * |  | Noppadol Pringvanich | Global Head Air Traffic Management Engineering and Aviation Radio Spectrum International Air Transport Association Montreal, QC - Canada Tel: +1-438- 258-4714 | pringvanin@iata.org |
| ICCAIA | * |  | Claude Pichavant | ICCAIA Advisor Senior Expert – COM SUR Tel: +33622452389 | claude.pichavant@airbus.com |
|  |  |  | Dr. David Redman | Director, AVSI USA Tel: +1-979-218-2272 | dredman@tamu.edu |
|  | * |  | Marcella Ost | Dir., Spectrum Policy & Regulatory Affairs Global Spectrum Management The Boeing Company 45 O’Connor Street, Suite 1220 Ottawa, K1P1A4 613-790-2270 | Marcella.s.ost@boeing.com |
|  |  |  | Stephen R. Leger | Principal Manager, Connected Aviation Solutions COLLINS AEROSPACE 2551 Riva Road, Annapolis, MD 21401 USA | stephen.leger@collinsaerospace.com |
|  |  |  | Ty E. Robertshaw | Sr. Program Manager Government Services COLLINS AEROSPACE 2551 Riva Road, Annapolis, MD 21401 USA | ty.robertshaw@collins.com |
| ITU | * |  | Nikolai Vassiliev | International Telecommunication Union (ITU) Place des Nations CH-1211 Geneva Switzerland | nikolai.vassiliev@itu.int |
|  |  |  | Nelson Malaguti | International Telecommunication Union (ITU) Place des Nations CH-1211 Geneva Switzerland Tel: +41 022 73051 98 | nelson.malaguti@itu.int |
|  |  |  | Karlis Bogens | International Telecommunication Union (ITU) Place des Nations CH-1211 Geneva Switzerland | karlis.bogens@itu.int |
| RTCA |  |  | Albert Secen | RTCA Inc | asecen@rtca.org |
| SITA | * |  | Andrew Yang | SITA Montreal Canada Tel: +1 514285 9723 Mobile: +1 514 465 4888 | Andrew.Yang@sita.aero |
|  | * |  | Denis Martel | SITA | denis.martel@sita.aero |
|  |  |  | Abderrahmane Ledjiar | SITA | Abderrahmane.Ledjiar@sita.aero |
|  |  |  | Eric Wawrzynkowski | SITA | Eric.Wawrzynkowski@sita.aero |
|  |  |  | Nicolas Van Wambeke | SITA | Nicolas.Van_Wambeke@sita.aero |
|  | * |  | Rudolphe Domingue | SITA | Rudolphe.Domingue@sita.aero |
| ICAO | * |  | Loftur E. Jónasson | International Civil Aviation Organization (ICAO) Tel: +1-514-954-8219 Ext. 7130 | ljonasson@icao.int |
|  | * |  | Mie Utsunomiya | International Civil Aviation Organization (ICAO) Tel: +1-514-954-8219 Ext. 6082 | mutsunomiya@icao.int |
|  | * |  | Bruno Oliveira Alves Ferreira | International Civil Aviation Organization (ICAO) Tel: +1-514-954-8219 Ext. 7586 | bferreira@icao.int |

*    In-Person Attendance

-END-

APPENDIX D

ACTION ITEM LIST

| Number | Description | Actionee | Due Date | Status |
| --- | --- | --- | --- | --- |
| 03-03 | Provide comment on the spectrum sharing approach between terrestrial and satellite RPAS C2 systems for the 5 030-5 091 MHz as proposed in FSMP-WG/3 WP10 and FSMP-WG/4 WP17 | All | FSMP-WG/15 | Ongoing FSMP-WG/12 IP01 and WP11 provide status information CLOSE - ongoing |
| 04-05 | Provide input to complete the equipment physical characteristics (e.g., weight) table shown in the Annex of FSMP-WG4/WP26. | All | FSMP-WG15 | CLOSE – no contributions |
| 04-06 | With regard to the action to “Conduct an aircraft fleet equipage impact analysis and develop detailed transition plans based on industry input and expected safety benefit” in the radio altimeter job card, initiate outreach to the airlines and aircraft manufacturers. | IATA and ICCAIA | FSMP-WG15 | Ongoing CLOSE – no contributions |
| 05-06 | Explore restructuring of Doc 9718 Volume 1. | Secretary/A. Roy/correspondence group | 2022 (complete) | FSMP-WG/12 WP05 provides updates CLOSE – on agenda |
| 09-05 | Provide material for the relevant structures specified in WG/09 WP17 and WP20 to the spectrum handbook correspondence group before the next FSMP meeting. | All | FSMP-WG/15 | CLOSE – on agenda |
| 09-06 | Review system characteristics structure in WG/09 WP20, and provide necessary system information to complete the section for each system. | K. Masrani (Inmarsat), D. Ladson (Iridium), U. Schwark (WAIC), A. Roy (HF and VHF), and Secretary (for other panels to review) | FSMP-WG/15 | Partially addressed by FSMP-WG/10 WP14, FMSP-WG/11 WP25. CLOSE – part of standing handbook agenda item |
| 09-09 | Provide updated/expanded example outlining the approach for aviation system protection suggested in FSMP-WG/2 WP24. | J. Mettrop | FSMP-WG/15 | CLOSE – no contributions |
| 09-10 | Provide comparison of aviation system emission masks with respect to relevant ITU requirements. | J. Mettrop | FSMP-WG/16 | FSMP-WG/12 WP18 and WP19 & FSMP-WG/13 WP10 and WP12 provide information |
| 10-01 | Catalogue aviation concerns with the current language in Resolution 155 (Rev. WRC-19). Participants to send their email address to John Mettrop (john.mettrop@caa.co.uk) by 28 August, 2020 if they want to be part of the FSMP correspondence group. | All | FSMP-WG/15 | Termed the group CG-155. FSMP-WG/12 WP13 progressed questions to help delineate ITU tasks from tasks for ICAO/Administration CLOSE – no contributions, on agenda |
| 10-02 | Provide comments on increased role for ICAO proposed in the longer-term HF frequency coordination approach contained in FSMP-WG/10 WP17. | All | FSMP-WG/16 | CLOSE – no contributions received |
| 10-03 | Report on current use in States of the 1300-1350 MHz frequency band for radars and longer term plans. | All | FSMP-WG/16 | Potential to be WRC-27 AI band so there is some urgency to this. Put on agenda? |
| 10-06 | Provide contributions describing how ultrawideband systems are being authorized in your State. | All | FSMP-WG/15 | FSMP-WG/11 IP01 CLOSE – no contributions |
| 10-07 | Monitor ITU-R WP5A deliberations on radiocommunications systems for train and trackside (RSTT) as frequency bands being considered are adjacent to those used by aeronautical safety systems | All | Ongoing | CLOSE – no contributions |
| 10-08 | Develop contributions on how to ensure protection of aeronautical systems that are not included in the ITU MIFR. | All | FSMP-WG/16 |  |
| 11-04 | Participate in the radio altimeter correspondence group (CG-RA) | All | Ongoing | Send Email to christian.fleury@aviation-civile.gouv.fr |
| 11-05 | Participate in ITU-R WP 3K-3M-9 correspondence group which is modelling air-ground propagation paths | All | Ongoing | CLOSE – no contributions received |
| 11-06 | Consider RPAS use of more than just the “2x10 MHz” for AMS(R)S communications in the 1.5/1.6 GHz L-Band | All | FSMP-WG/15 | CLOSE – FSMP-WG/15 WP10 |
| 11-08 | Review Recommedation ITU-R SM.1535 and determine if updates are required | All | FSMP-WG/15 | Noted that any identified changes should be input to ITU-R by State(s) and not ICAO. CLOSE – no contributions |
| 12-02 | Provide comments and updates to the national power levels contained in Figure 1 of FSMP-WG/12 WP17 to Sai.Kalyanaraman@collins.com | All | ASAP | CLOSE – no contributions |
| 12-03 | Provide contributions to FSMP-WG/13 regarding the proposals on SM.1541 contained in FSMP-WG/12 WP18 | All | ASAP to author or to FSMP-WG/15 | FSMP-WG/13 WP09 applies CLOSE – no contributions |
| 12-04 | Provide contributions to FSMP-WG/13 regarding the proposals on adding SARPS spectral characteristics requirements contained in FSMP-WG/12 WP19 | All | ASAP to author or to FSMP-WG/15 | CLOSE – no contributions |
| 12-06 | Provide contributions to WG/13 to further develop and answer outstanding questions on the aviation RF sharing framework in Flimsy08 | All | FSMP-WG/15 | CLOSE – no contributions |
| 12-07 | Provide contributions to WG/13 to provide material on a methodology to develop aviation system protection criteria and spectral masks. | All, Lead: J. Mettrop | FSMP-WG/15 | CLOSE – part of standing handbook agenda item |
| 12-08 | Review and provide comments on the “ITU Res 155 tasks” material contained in Appendix G to the report of FSMP-WG/12. | All | FSMP-WG/15 | CLOSE – on agenda |
| 12-09 | Review the text of the Job Card FSMP-006-001 and determine if changes are needed.. | All | FSMP-WG/15 | CLOSE – no contributions |
| 12-10 | Determine if generic guidance can be developed to help aviation authorities analyse potential 5G impacts on radio altimeters given their specific State implementation |  | FSMP-WG/16 | FSMP-WG/15 IP09 applies |
| 13-01 | Provide examples of national specifications or regulations on adjacent-band signal rejection. | FSMP participants who are also spectrum regulators in their country | FSMP-WG/16 |  |
| 13-02 | Develop repository on the FSMP website for storage of reference material on 5G/Radio altimeter issue | Secretary | ASAP or by FSMP-WG/15 | CLOSED |
| 13-03 | Develop educational material for States explaining the 5G/RA issue. | CG-RA | FSMP-WG/16 | Similar to 12-10, combine? IP09 applies |
| 13-04 | Develop a set of frequently asked questions (FAQs) regarding satellite-VHF. | CG-SV | ongoing | To be part of the CG-SV send an Email to the coordinators at: mangarcia@enaire.es and olivier.pellay@airbus.com FSMP-WG/15 IP05 applies |
| 13-05 | Catalog existing aviation system transmit and receive masks | J. Mettrop | FSMP-WG/15 | CLOSE – part of standing handbook agenda item |
| 14-01 | Liaise the draft WAIC SARPS material to the ICAO NSP and CP | Secretary | ASAP so any concerns can be addressed at FSMP-WG/15 | CLOSED FSMP-WG/15 WP07 |
| 14-02 | Consider the proposed Note for the RPAS C2 link SARPS as contained in FSMP-WG/14 WP07 in order to resolve at FSMP-WG/15. | All | FSMP-WG/15 | CLOSE, deadline is past. States can comment on the SARPS |
| 14-03 | Liaise FSMP-WG/14 WP06 to RPASP WG3 to solicit input to the under-development ITU-R Handbook on detect and avoid systems | Secretary | ASAP | FSMP Liaison was sent, waiting on response |
| 14-04 | Participate in CG-MSS to complete the Note providing information on ensuring compatibility between IMT operated below 1518 MHz and aeronautical MSS services above 1525 MHz. | All | FSMP-WG/16 | To participate send an email to Kamlesh Masrani Kamlesh.Masrani@inmarsat.com FSMP-WG/15 WP08 |
| 14-05 | Material on proposed coordination approach for new AMS(R)S under WRC-23 1.7. Include proposed liaison to WP5B on the approach. | All | FSMP-WG/15 | CLOSED FSMP-WG/15 Flimsy 2 |
| 14-06 | Review proposal from SP captured in FSMP-WG/14 WP19 and provide comments as necessary. | All | By 11 May 2022 to author (chris.tourigny@faa.gov) | CLOSED |
| 15-01 | Review the material in IP09 and provide any necessary updates. | All | ASAP to allow incorporation in published version. |  |
| 15-02 | Bring specific proposals for the ICAO Position on WRC-23 AI9.2 to FSMP-WG/16 | All | FSMP-WG/16 |  |

APPENDIX E

Draft modifications to the ICAO WRC-23 Position

APPENDIX F

Draft WAIC SARPS package

APPENDIX G

Elements for a proposed Liaison to ECHO project

APPENDIX H

Elements for RLS to 5B on NDB parameters

APPENDIX I

Elements for a RLS to WP5B on WRC-23 agenda item 1.7

APPENDIX J

Discussion of ICAO Spectrum Handbook