Introductory text to be developed before liaise to NSP and CP
ANNEX 1

Proposed Modifications to Annex 10 to the Convention on International Civil Aviation

Insert new text as follows

CHAPTER xx.	WIRELESS AVIONICS INTRA-COMMUNICATIONS (WAIC)

xx.1	DEFINITIONS

Wireless Avionics Intra-Communications (WAIC) – WAIC is defined as radiocommunication between two or more aircraft stations located on board a single aircraft; supporting the safe operation of the aircraft.

WAIC System – A WAIC System provides wireless communications between points on board a single aircraft for aircraft applications related to the safety and regularity of flight using the aeronautical mobile (route) service (AM(R)S) allocation in the frequency band 4 200 – 4 400 MHz. A WAIC System may be comprised of one or more WAIC Networks necessary for establishing, maintaining and securing wireless communications. A WAIC System is understood as the entirety of all WAIC components on board the same aircraft, so that a single aircraft contains only a single WAIC System.

WAIC Network – A WAIC Network comprises interrelated WAIC Components, e.g., components used for wireless communications, security or network management.

WAIC Component – Any tangible entity of a WAIC Network on board an aircraft.

WAIC Node – A WAIC Node is a specific category of a WAIC Component establishing wireless communications between aircraft applications or parts thereof.

| Origin: FSMP | Rationale: The above definitions are specific to WAIC and are provided in addition to the general definitions given in Chapter 1, Part 1 of Annex 10 Volume III. |
| --- | --- |

xx.2	INTRODUCTION

xx.2.1	WAIC Systems provide wireless communications between points on board a single aircraft for aircraft applications related to the safety and regularity of flight using the aeronautical mobile (route) service (AM(R)S) allocation in the frequency band 4 200 – 4 400 MHz. WAIC Systems are not allowed to communicate off board a given aircraft. This frequency band is shared with existing radio altimeters, which must be protected from WAIC emissions.

xx.2.2	These Standard and Recommended Practices (SARPs) define the requirements that ensure that WAIC Systems and radio altimeters can provide their intended functions while multiple aircraft are in mutual radio range. Coexistence requirements between WAIC Systems and radio altimeters installed on board the same aircraft are covered by established airworthiness certification processes and are outside the scope of these SARPs.

xx.3	GENERAL

xx.3.1	WAIC Systems shall comply with the applicable provisions of the ITU Radio Regulations.

xx.3.2	WAIC shall only be used for communications between two or more points on a single aircraft.

xx.3.3	WAIC Systems shall not cause harmful interference to radio altimeter systems on other aircraft while in operation in the frequency band 4 200 – 4 400 MHz.

Note:	Compliance with xx.3.3 is achieved by limiting the power of WAIC emissions below the level at which altimeter performance may be affected. The RTCA document DO-378A and the EUROCAE document ED-260A specify the power spectral density limit for a WAIC system that is consistent with xx.4.2 below, and provide one acceptable method of demonstrating compliance with that power spectral density limit.

xx.3.4	A WAIC system located on board one aircraft shall maintain its intended function while subject to emissions from WAIC and radio altimeter systems located on board other aircraft.

Note:	The RTCA document DO-378A and the EUROCAE document ED-260A provide one acceptable method of demonstrating compliance with xx.3.4 via test. Alternatively, the critical coexistence scenario described in DO-378A  and ED-260A may also be used to develop appropriate analyses to demonstrate compliance with xx.3.4.

| Origin: FSMP | Rationale: The sections above describe the basic function and purpose of WAIC. |
| --- | --- |

xx.4	RADIO FREQUENCY (RF) CHARACTERISTICS

xx.4.1	Frequency Band: WAIC systems shall operate in the frequency band 4 200 – 4 400 MHz.

xx.4.2	System

xx.4.2.1 Total Radiated Power: The power of the total emissions of all WAIC transmitters on board an aircraft shall not exceed an equivalent isotropic radiated power of -20dBm, assuming a point source located at the geometrical center of the aircraft.

Note: The RTCA document DO-378A and the EUROCAE document ED-260A provide one acceptable method of demonstrating compliance with xx.4.2.

xx.4.2.2 Occupied bandwidth: The overall occupied bandwidth shall be maintained completely within the allocated frequency band 4 200 – 4 400 MHz including any offsets such as Doppler shift or frequency tolerances. Where the occupied bandwidth is defined as the bandwidth for which 99% of the signal energy falls within the lower and upper frequency limits.

Note:	The Radio Regulations define the occupied bandwidth as “The width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage β/2 of the total mean power of a given emission”. The value of β/2 being 0.5%.

xx.4.3 Transmitter

xx.4.3.1 Necessary bandwidth: The necessary bandwidth (NB) shall be calculated according to Appendix 1 of the ITU-R Radio Regulations and applies to the emissions of a single WAIC transmitter.

xx.4.3.2	 Unwanted emission limits in the out-of-band and spurious domains: The boundary between the out-of-band and spurious domains shall be determined according to Annex 1 of Appendix 3 of the ITU-R Radio Regulations. The required attenuation of the mean power of any unwanted emission relative to the total mean power P shall meet or exceed the following conditions:

50% of NB < f < 150% of NB:	Linear increase (in dB) from 24dB to 35dB within a reference bandwidth of 4kHz (Note 1)

150% of NB < f <  start of the spurious domain: 35dB within a reference bandwidth of 4kHz (Note 1)

Spurious domain:	56+10log(P) or 40dB whichever is less stringent measured in a RBW of 1MHz (Note 2)

Note 1:	Reference bandwidth of 4 kHz within the out-of-band domain in accordance with Annex 11 of Recommendation ITU.R SM.1541-6. The parameter f is the frequency separation from the center frequency of the transmit signal.

Note 2: 	Reference bandwidth of 1 MHz within the spurious domain in accordance with Appendix 3 paragraph 7 of the ITU-R Radio Regulations and determination of attenuation for low power device radio equipment (Category A) in accordance Appendix 3 paragraph 13 of the ITU-R Radio Regulations.

xx.4.4	Out-of-Band Interference Tolerance of a WAIC Receiver

Note: These requirements are for WAIC equipment and define an RF environment in which WAIC equipment must meet its performance requirements without taking into account any mitigation afforded by its installation. Additional guidance for performance of installed WAIC equipment is specified in relevant minimum operational performance standards.

xx4.4.1 In-band interference: receivers shall tolerate interference from sources operating outside of the frequency band 4  200 ‐ 4 400 MHz whose total combined emitted power falling within the frequency band 4 200 ‐ 4 400 MHz as measured at the receiver does not exceed a power spectral density of ‐120 dBm / MHz.

xx.4.4.2 Total interference from out-of-band sources:  receivers shall tolerate interference from sources operating outside of the frequency band 4 200 – 4 400 MHz whose total combined power as measured at the receiver does not exceed -20 dBm.

| Origin: FSMP | Rationale: The sections above describe the minimum RF characteristics of WAIC transmitter and receiver. |
| --- | --- |

— — — — — — — —

ANNEX 2

Impact and Implementation Assessment

Proposed Text to be Submitted to ANC

# PART 1: IMPACT ASSESSMENT

1.1	What is the problem/opportunity that this proposal is designed to address?

Please include reference to Jobcard / ASBU / work programme item, as applicable

(Reference: Job Card FSMP 07.01 – Develop and maintain SARPs and guidance to prevent WAIC / Radio Altimeter interference) Resolution 424 of the World Radiocommunication Conference 2015 established a new aeronautical mobile route service allocation in the frequency band 4200 – 4400 MHz, reserved exclusively for Wireless Avionics Intra Communications (WAIC), and invited ICAO to develop SARPs for WAIC taking into account Recommendation ITU-R M.2085. As specified in Resolution 424, WAIC cannot cause harmful interference to, nor claim protection from radio altimeters operating in the same band as part of the pre-existing frequency allocation to the aeronautical radionavigation service. These proposed SARPs specify conditions that will assure such safe coexistence between future WAIC installations and radio altimeters. WAIC is a new type of wireless on-board communication serving aircraft functions related to safety and regularity of flight. To-date, wireless data links have been feasible only for non-critical and non-essential aircraft functions. Availability of a new route allocation for WAIC will allow the use of wireless links for a wide range of aircraft systems. Possible applications of WAIC, as discussed in detail in Report ITU-R M.2283, include a variety of sensing, monitoring and control functions. The ability to deploy new sensors without additional wiring will allow introduction of new sensing and monitoring capabilities, leading to increased safety and efficiency of flight. Dissimilar redundant wireless links will make it easier to design safety critical systems by reducing common failure modes in data links. Aircraft reconfiguration and retrofit will be made more efficient when fewer wired data links have to be physically rerouted. Reduction of wire weight will also be consistent with ICAO environmental contributions to Sustainable Development Goals. Other, not yet fully understood benefits of WAIC will likely become apparent once the technology becomes commercially available. In order to realize the many potential benefits of WAIC, ICAO needs to develop and approve SARPs. Per Resolution 424 (WRC-15) WAIC can operate only in compliance with ICAO SARPs. In order to fully invest in technical development of WAIC, the industry must have a high level of certainty about technical requirements that WAIC equipment and networks must satisfy. Therefore, the approval of these SARPs will be a crucial and necessary step towards practical deployment of WAIC systems.

1.2	What is the overall impact of this proposal on the strategic objectives of ICAO, namely:

|  | Positive / Negative / Negligible/None | Rationale: Please provide an explanation for your choice and highlight any caveats or limitations in the selection |
| --- | --- | --- |
| Safety | Positive | WAIC is an enabling technology introducing a new communications medium between aircraft functions. WAIC will enable expanded intra-system communications and the introduction of new sensors/actuators. WAIC-related safety impacts can only be assessed in association with future intended functions. The current level of safety will be at least maintained. Depending on the actual function WAIC is used for, safety may be improved for particular circumstances, e.g., by allowing dissimilar redundancy or adding new safety functionalities. |
| Security | Negligible/None | WAIC Systems do not provide communication between two aircraft. Specific equipment security capabilities and system implementations are defined in the WAIC MOPS to protect systems against specific threats. |
| Environment | Positive | Enhanced sensing, without additional weight penalty of new wiring, will enable more energy-efficient operation of aircraft systems, thus reducing the overall fuel consumption and carbon emissions. |
| Efficiency | Positive | Reduction of wiring, with associated harnesses and connectors, will simplify the design effort needed to add and route new wiring, and will reduce assembly and installation labor. New sensing and monitoring functions made possible by WAIC will facilitate predictive and condition-based maintenance, reducing gate delays and improving air transport efficiency. |

Note: In the following questions ‘States’ applies to the adoption and oversight of new SARPs. ‘Industry’ applies to the service provision and use, whether State owned or not (e.g., ANSPs, airlines aerodromes, meteorology, general aviation, etc.). With respect to financial costs for States, it refers to the cost to develop, implement, maintain, and consider oversight issues associated with the proposed change. For Industry, it refers to the cost of implementing the change, where compliance is required by the State, which may translate in costs for equipage, human resources, training, documentation, aircraft modifications or upgrades, operations and airworthiness for example.

1.3	What is the overall impact on resources (financial, personnel, etc.) of this proposal for:

|  | Increase/decrease/negligible/unknown | Rationale: Please provide an explanation for your choice and highlight any caveats or limitations in the selection |
| --- | --- | --- |
| States | Negligible/No impact | No changes to existing systems or infrastructure will be needed when WAIC Systems are introduced. WAIC involves communication only between points on a single aircraft, and does not require any new infrastructure. |
| Industry | Negligible/No impact | The proposed WAIC Systems will be implemented on new aircraft being delivered from the aircraft manufacturers and also potentially on retrofit aircraft, with no impact on other aircraft that does not use WAIC. |

1.4	In your opinion, do the benefits of this proposal justify the cost of implementing the proposal from the perspective of:

|  | Answer | Rationale: Please provide an explanation for your choice and highlight any caveats or limitations in the selection |
| --- | --- | --- |
| States | Yes | WAIC will allow design of more efficient and intelligent aircraft systems with enhanced sensing and monitoring functionalities, facilitating technical progress in aviation and improving efficiency of air travel. |
| Industry | Yes | The new AM(R)S allocation for WAIC was established due to industry request. The availability of SARPs for WAIC will allow taking advantage of the many potential benefits of WAIC. |

# PART 2: IMPLEMENTATION PLAN

To assist ICAO and States ensure this proposal will be effectively implemented please answer the following questions.

Note: The ANC recognizes that panel experts may feel limited in their ability to answer some or all of these questions, however, encourages the panels to provide their views. If still unsure, it is acceptable to leave one or more blank. The answers presented to the ICAO Council with the proposed amendment will be further developed by ICAO.

2.1	What supporting documentation is required for this proposed amendment?

Please include reference to any documents that require initial release/amendment e.g., ICAO Document or Circular name and number, industry specification, etc.

- Add a new chapter to Annex 10 Aeronautical Telecommunications, Volume V on the frequency band 4200 – 4400 MHz. That chapter is planned to contain two sub-sections, one on the Wireless Avionics Intra-Communications (WAIC), and one on radio altimeters.

2.2	What other guidance, training and support activities do you recommend ICAO undertake to ensure the effective implementation of this proposed amendment?

Please include reference to any existing support/promotional programmes and whether it is required globally or regionally e.g., regional seminars, ikits, etc.

2.3	What are the essential steps to be followed by a State in order to implement this proposed amendment?

Please include the major steps e.g., amendment of national legislation, change of oversight procedures, training of oversight personnel, required competencies, etc.

- National radio frequency spectrum regulations need to be adapted to reflect the changes in the Radio Regulations for WAIC agreed by WRC-15 for WAIC.

2.4	What is the timeframe needed to implement this proposal by:

|  | Answer | Rationale: For the State, the timeframe is the length of time needed to implement in the national regulatory framework For industry, the timeframe is the length of time needed for industry to start implementing in their operations |
| --- | --- | --- |
| States | 1 - 2 Years | National radio frequency spectrum regulations need to be adapted to reflect the changes in the Radio Regulations for WAIC agreed by WRC-15 for WAIC. |
| Industry | 1 - 2 Years | The lack of SARPs is the main factor preventing the industry from developing and deploying WAIC solutions. Once these SARPs are approved, the expectation within the industry is to have first WAIC solutions available within 2 years. |

# PART 3: AUDIT PLAN

Note: This section will be completed by ICAO prior to the presentation of any proposed changes to SARPs or PANS. The Panel Secretary will coordinate with the relevant experts in ICAO.

3.1	Does this proposal require an amendment of the USOAP CMA protocol questions to assess effective implementation by States?

Please include reference to existing PQs that may need amendment or description of any new PQs that may be required. State ‘Not applicable’ if no impact

- Not applicable

ANNEX 3

Validation Statement

Proposed Text to be Submitted to ANC

WAIC SARPS VALIDATION REPORT

- INTRODUCTION

  - Wireless Avionics Intra-Communications (WAIC) is a new class of wireless data links intended for communication between various functions on-board a single aircraft. A new aeronautical mobile route service allocation for WAIC within the radio frequency band of 4 200 – 4 400 MHz was established by the 2015 World Radiocommunication Conference (WRC-15).

  - The same radio frequency band includes an aeronautical radionavigation service allocation reserved for radio altimeters. The new allocation for WAIC requires that WAIC cannot cause harmful interference to nor claim protection from radio altimeters. Therefore, in development of SARPs for WAIC the main technical task was to establish and validate conditions under which WAIC shall not harmfully interfere with altimeters.

  - This report summarizes studies that were undertaken to characterize susceptibility of radio altimeters with respect to interference from WAIC and to validate technical requirements to be included in WAIC SARPs. The particular technical parameter under consideration was the total radiated power emitted from a WAIC system on board a single aircraft. A series of experimental studies investigated the highest power that may be emitted from a WAIC-equipped aircraft that allows normal operation of altimeters on other aircraft. Results of those studies were presented to and analyzed by FSMP.

  - Prior to finalization of these SARPs, the EUROCAE Working Group 96 and RTCA Special Committee 236 jointly developed a Minimum Aviation System Performance Standard (MASPS) for WAIC, which is now published as ED-260 and DO-378. That joint committee reviewed the same experimental study and confirmed its validity. The technical specifications in WAIC SARPs are consistent with conditions specified within this WAIC MASPS.

- WAIC POWER LIMIT

  - Resolution 424 (WRC-15) invited ICAO to take into account Recommendation ITU-R M.2085 when developing WAIC SARPs. That Recommendation specifies that the maximum equivalent isotropically radiated power (EIRP) spectral density generated by a WAIC system installed on board a single aircraft must not exceed 6 dBm/MHz, or equivalently 4mW/MHz. The WAIC SARPs recommends that the power of the total emissions of all WAIC transmitters on board an aircraft shall not exceed an equivalent isotropic radiated power of -20dBm assuming a point source located at the geometrical center of the aircraft, which complies with the limit in Recommendation ITU-R M.2085.

  - ED-260 and DO-378 adopt the total EIRP limit for WAIC systems and provide a practical verification procedure how to ascertain whether the limit is satisfied.

  - The validation study summarized herein took as the basis the power limit from ITU-R M.2085. The goal was to confirm experimentally whether that limit assures safe operation of actual radio altimeter implementations aboard other aircraft under worst case operational scenarios. Thus, the much lower limit provide by the WAIC SARPs will provide an even greater margin to prevent interference.

- INTERFERENCE SUSCEPTIBILITY STUDY

  - The experimental study was performed by the Aerospace Vehicle Systems Institute (AVSI). The project team included three leading altimeter manufacturers (Honeywell, Rockwell Collins, Thales), two airframers (Airbus, Embraer), as well as equipment manufacturers (Lufthansa Technik, UTC, Zodiac) and NASA specialists.

  - The study was performed at Texas A&M University to enable objective and repeatable testing in an independent academic setting.

  - A collection of commercial altimeters was placed within a calibrated test bench. Controlled interference signals were injected into altimeter receivers’ additive to their return signals.

  - Through analysis and experimental iteration, the project team established the worst-case operational scenario under which multiple WAIC-equipped aircraft generate most severe interference received by altimeters on a victim aircraft. That worst-case scenario involves landing when the multiple WAIC-aircraft aircraft parked or taxiing at the airport, consistent with ICAO regulations for aerodromes, generate the most severe composite interference environment.

  - The study crucially considered interference from multiple altimeters installed on multiple aircraft at the airport, as well as from multiple redundant altimeters aboard the landing (victim) aircraft. That was a primary concern for FSMP, as other altimeters represent the most severe interference source for a victim altimeter, often exceeding the effects of potential interference from WAIC.

  - The study used a set of very conservative assumptions and was thoroughly reviewed by the altimeter experts on the AVSI team. It was also reviewed and accepted by a wider team on the joint WG-96 and SC-236 committee, including experts from EASA and FAA.

  - Details of the study were reviewed by FSMP in form of a series of Information Papers submitted by AVSI.

- VALIDATION OF REMAINING SARPS REQUIREMENTS

  - Additional parameters specified by this SARPs were experimentally found to have no effect on the interference sensitivity of radio altimeters. However, to properly implement WAIC systems, these additional parameters are required to meet international radio frequency spectrum regulation and have been verified by FSMP members as sufficient to satisfy these regulations.

  - Specifically, the unwanted emissions limits were thoroughly discussed to assure compliance with applicable ITU-R recommendations and national regulations. The final version of the emissions mask satisfies all the requirements introduced by FSMP members representing different national regulatory bodies.

  - Out-of-band interference tolerance was similarly thoroughly examined discussed to assure compliance with applicable ITU-R regulations. The final version of the requirement addresses all concerns voiced by FSMP members.

  - The FSMP reviewed and validated these SARPs for regulatory compliance.

- CONCLUSIONS

  - The AVSI study verified that if EIRP spectral density generated by WAIC aboard a single aircraft does not exceed 6dBm/MHz, then performance of altimeters aboard other aircraft will not be negatively affected. Thus, a total EIRP of -20dBm/200 MHz will be well below the threshold for interference.

  - The conclusion holds in most unfavorable worst case operational scenarios, under a set of conservative worst case assumptions.

  - FSMP concludes that the -20 dBm EIRP limit on the total emissions of all WAIC transmitters on board an aircraft is an appropriate requirement for WAIC and satisfies the condition of protecting radio altimeters.

  - All requirements in WAIC SARPs were validated by FSMP for regulatory compliance.