| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 5B/TEMP/129 Subject: WRC-23 agenda item 1.7 Resolution 428 (WRC-19) | Annex 29 to Document 5B/355-E |
|  | 8 June 2021 |
|  | English only |
| Annex 29 to the Working Party 5B Chairman’s Report |  |
| working document towards Preliminary Draft new report [SPACE VHF] WORKING DOCUMENT RELATED TO WRC-23 Agenda Item 1.7 |  |
| Space-based aeronautical VHF communications in 117.975-137 MHz frequency band |  |

Scope

[To be populated later]

[Editor’s note: Presented below are the draft main topics to provide a structured outline that can be populated later or upon availability of the results of technical studies, simulations and other operational characteristics within ICAO and ITU-R WP 5B. This is a live working document that will require significant input, revision and update to ensure the draft PDNR can reach maturity for adoption in the ITU-R as a published report.]

Glossary of abbreviations

ADS-B:	Automatic dependent surveillance – broadcast

AM(R)S:	Aeronautical mobile (route) service

AMS(R)S:	Aeronautical mobile satellite (route) service

ANSP:	Air navigation service provider

ATC:	Air traffic control

DCPC:	Direct controller to pilot communications

EPFD:	Effective power flux density

FIR:	Flight information region

ICAO:	International Civil Aviation Organization

LEO:	Low earth orbit

MSS:	Mobile satellite service

RR:	Radio Regulations

SATCOM:      Satellite communications

VDES:	VHF data exchange system

Relevant ITU-R Recommendations

| ITU-R M.1231-0 | Interference criteria for space-to-Earth links operating in the mobile-satellite service with non-geostationary satellites in the 137-138 MHz band |
| --- | --- |
| ITU-R M.1232-0 | Sharing criteria for space-to-Earth links operating in the mobile-satellite service with non-geostationary satellites in the 137-138 MHz band |
| ITU-R M.2092-0 | Technical characteristics for a VHF data exchange system in the VHF maritime mobile band |
| ITU-R P.531-14 | Ionospheric propagation data and prediction methods required for the design of satellite networks and systems |
| ITU-R RA.769-2 | Protection criteria used for radio astronomical measurements |
| ITU-R SM.1541-6 | Unwanted emissions in the out-of-band domain |

# 1	Description of space-based VHF communications concept

## 1.1	General concept

Space-based VHF communication is a concept in which aircraft operating in remote regions and oceanic areas provide communications from the aircraft to air traffic control (ATC) via satellite using VHF frequencies within the 117.975 – 137 MHz range.  This concept, when implemented, is expected to enhance air traffic management and flight operations in oceanic and remote airspace and will complement current aviation use of satellite-based navigation and surveillance technologies.

While currently there are other long-range communication systems, such as HF and SATCOM, available to facilitate communications between aircraft to ATC in remote and oceanic airspace, the performance of these current systems is not adequate to safely support close aircraft-to-aircraft separation in a similar fashion as to what is being applied  in dense airspace where terrestrial-based VHF communications infrastructure is predominant. Therefore, this leads to constraints in airspace capacity and efficiency in oceanic and remote areas, where it is not practical to deploy VHF terrestrial infrastructure.

Figure 1 is an illustration of the space-based VHF communication concept. The space segment is able to receive and transmit to standard VHF radios already installed on board aircraft and is designed to behave as if it was just another VHF tower located in the sky, with a larger footprint than terrestrial towers.

Figure 1

The space-based VHF communication concept

This report provides studies on the use of low-Earth orbiting satellites to relay air traffic control messages between the pilot and the controller. Operation of the space-based VHF system within the frequency band 117.975-137 MHz, currently allocated to the aeronautical mobile (Route) service (AM(R)S), aims to utilize existing on-board VHF radios used for provision of terrestrial-based VHF communications, thus being compliant with the ITU-R Resolution 428.

A new allocation to the aeronautical mobile satellite (Route) service AMS(R)S within this frequency band 117.975-137 MHz is being considered under the agenda item 1.7 of the WRC-23.

AMS(R)S links between aircraft and satellite (shown as “Satellite VHF COM” in Figure 1) will operate within the frequency band 117.975-137 MHz and are the subject of considerations contained in this Report. Feeder links between earth station and satellite is not implemented within the VHF band, but rather in an appropriate separate allocation (for instance FSS) which is out of scope of this Report.

## 1.2	High-level objectives

The following objectives and characteristics are considered for the space-based VHF system under study:

–	The applications provided are within AMS(R)S communications. Voice is the most critical VHF communication application in terms of safety and dependability.

–	No change is made on:

•	aircraft avionic equipment, RF setup and applicable specifications.

•	terrestrial base stations specifications, and configuration of base stations located in flight information regions (FIRs) which do not make use of the space-based VHF service.

–	No or minimal change would be made on:

•	operational aspects for pilots and controllers.

•	terrestrial base stations configuration in FIRs with space-based VHF service.

The service area covered by the space-based VHF system will depend on satellite constellation architecture and design. It can be limited to one or several FIRs, regional or global. The space-based VHF system is primarily intended to cover oceanic and remote areas where terrestrial service is not available, but it could also be used to backup terrestrial stations in case they are affected by a catastrophic event.

To achieve the above objectives, the following tasks need to be carried out.

–	ITU-R carry out sharing and compatibility studies for co-existence between potential new AMS(R)S systems in the frequency band 117.975-137 MHz (Earth-to-space and space-to-Earth) and existing primary in-band and adjacent band non-ICAO services considering existing and planned systems.

–	Based on the results of the studies, ITU-R provides technical and regulatory proposals towards a possible new AMS(R)S allocation within the frequency band 117.975-137 MHz.

–	ICAO participation would be beneficial in ITU-R sharing and compatibility studies to provide aeronautical operational requirements, between new AMS(R)S and existing aeronautical systems, and other relevant available operational characteristics.

# 2	Current use of the VHF frequency band 117.975-137 MHz

Below is the Radio Regulations (RR) table of allocations and associated footnotes for the band 117.975-137 MHz. It shows that services allocated in this band on a primary basis are:

–	Aeronautical mobile (R) service throughout all the band.

–	Aeronautical mobile (OR) service in the bands 132-136 MHz and 136-137 MHz in certain countries listed respectively in RR Nos. 5.201 and 5.202.

| Allocation to Services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 117.975-137 AERONAUTICAL MOBILE (R) 5.111 5.200 5.201 5.202 |  |  |

5.111	The carrier frequencies 2 182 kHz, 3 023 kHz, 5 680 kHz, 8 364 kHz and the frequencies 121.5 MHz, 156.525 MHz, 156.8 MHz and 243 MHz may also be used, in accordance with the procedures in force for terrestrial radiocommunication services, for search and rescue operations concerning manned space vehicles. The conditions for the use of the frequencies are prescribed in Article 31.

The same applies to the frequencies 10 003 kHz, 14 993 kHz and 19 993 kHz, but in each of these cases emissions must be confined in a band of  3 kHz about the frequency.     (WRC-07)

5.200	In the band 117.975-137 MHz, the frequency 121.5 MHz is the aeronautical emergency frequency and, where required, the frequency 123.1 MHz is the aeronautical frequency auxiliary to 121.5 MHz. Mobile stations of the maritime mobile service may communicate on these frequencies under the conditions laid down in Article 31 for distress and safety purposes with stations of the aeronautical mobile service.     (WRC-07)

5.201	Additional allocation:  in Armenia, Azerbaijan, Belarus, Bulgaria, Estonia, the Russian Federation, Georgia, Hungary, Iran (Islamic Republic of), Iraq (Republic of), Japan, Kazakhstan, Mali, Mongolia, Mozambique, Uzbekistan, Papua New Guinea, Poland, Kyrgyzstan, Romania, Senegal, Tajikistan, Turkmenistan and Ukraine, the frequency band 132-136 MHz is also allocated to the aeronautical mobile (OR) service on a primary basis. In assigning frequencies to stations of the aeronautical mobile (OR) service, the administration shall take account of the frequencies assigned to stations in the aeronautical mobile (R) service.     (WRC-19)

5.202	Additional allocation:  in Saudi Arabia, Armenia, Azerbaijan, Bahrain, Belarus, Bulgaria, the United Arab Emirates, the Russian Federation, Georgia, Iran (Islamic Republic of), Jordan, Mali, Oman, Uzbekistan, Poland, the Syrian Arab Republic, Kyrgyzstan, Romania, Senegal, Tajikistan, Turkmenistan and Ukraine, the frequency band 136-137 MHz is also allocated to the aeronautical mobile (OR) service on a primary basis. In assigning frequencies to stations of the aeronautical mobile (OR) service, the administration shall take account of the frequencies assigned to stations in the aeronautical mobile (R) service.     (WRC-19)

# 3	Current Use of the frequency bands adjacent to 117.975-137 MHz

## 3.1	Radiocommunication services operating in the 108-117.975 MHz frequency band based on the RR Table of Allocations

Below is the RR table of allocations and associated footnotes for the band 108-117.975 MHz. It shows that services allocated in this band on a primary basis are:

–	Aeronautical radio navigation service

–	Aeronautical mobile (R) service.

| Allocation to Services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 108-117.975 MHz AERONAUTICAL RADIONAVIGATION 5.197 5.197A |  |  |

5.197	Additional allocation:  in the Syrian Arab Republic, the band 108-111.975 MHz is also allocated to the mobile service on a secondary basis, subject to agreement obtained under No. 9.21. In order to ensure that harmful interference is not caused to stations of the aeronautical radionavigation service, stations of the mobile service shall not be introduced in the band until it is no longer required for the aeronautical radionavigation service by any administration which may be identified in the application of the procedures invoked under No. 9.21.    (WRC-12)

5.197A	Additional allocation:  the band 108-117.975 MHz is also allocated on a primary basis to the aeronautical mobile (R) service, limited to systems operating in accordance with recognized international aeronautical standards. Such use shall be in accordance with Resolution 413 (Rev.WRC-07)*. The use of the band 108-112 MHz by the aeronautical mobile (R) service shall be limited to systems composed of ground-based transmitters and associated receivers that provide navigational information in support of air navigation functions in accordance with recognized international aeronautical standards.     (WRC-07)

## 3.2	Radiocommunication services operating in the 137-143.6 MHz frequency bands based on the Radio Regulations table of allocations

Below is the RR table of allocations and associated footnotes for the band 137-143.6 MHz. It shows that services allocated in this band on a primary basis are:

Primary services in 137-138 MHz:

–	Aeronautical mobile (OR) service in certain countries under RR No. 5.206

–	Broadcasting service in Australia under RR No. 5.207

–	Fixed service in certain countries under RR No. 5.204 and No. 5.205

–	Meteorological satellite service (space-to-Earth)

–	Mobile satellite service (space-to-Earth)

–	Mobile service in certain countries under RR No. 5.204 and No. 5.205

–	Space operation service (space-to-Earth)

–	Space research service (space-to-Earth)

Primary services in 138-143.6 MHz:

–	Aeronautical mobile (OR) service in Region 1

–	Broadcasting service in Australia under RR No. 5.207

–	Fixed service in Region 2, Region 3, and certain countries in Region 1 under RR No. 5.212 and No. 5.214

–	Land mobile service in certain countries in Region 1 under RR No. 5.211

–	Mobile service in Region 2, Region 3, and certain countries in Region 1 under RR No. 5.212

–	Maritime mobile service in certain countries in Region 1 under RR No. 5.211

–	Radio location service in Region 2 and in China under RR No. 5.213

| Allocation to Services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 137-137.025 SPACE OPERATION (space-to-Earth) 5.203C METEOROLOGICAL-SATELLITE (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.208A 5.208B 5.209 SPACE RESEARCH (space-to-Earth) Fixed Mobile except aeronautical mobile (R) 5.204 5.205 5.206 5.207 5.208 |  |  |  |
| 137.025-137.175 MHz SPACE OPERATION (space-to-Earth) 5.203C METEOROLOGICAL-SATELLITE (space-to-Earth) SPACE RESEARCH (space-to-Earth) Fixed Mobile except aeronautical mobile (R) Mobile-satellite (space-to-Earth) 5.208A 5.208B 5.209 5.204 5.205 5.206 5.207 5.208 |  |  |
| 137.175-137.825 MHz SPACE OPERATION (space-to-Earth) 5.203C 5.209A METEOROLOGICAL-SATELLITE (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.208A 5.208B 5.209 SPACE RESEARCH (space-to-Earth) Fixed Mobile except aeronautical mobile (R) 5.204 5.205 5.206 5.207 5.208 |  |  |
| 137.825-138 MHz SPACE OPERATION (space-to-Earth) 5.203C METEOROLOGICAL-SATELLITE (space-to-Earth) SPACE RESEARCH (space-to-Earth) Fixed Mobile except aeronautical mobile (R) Mobile-satellite (space-to-Earth) 5.208A 5.208B 5.209 5.204 5.205 5.206 5.207 5.208 |  |  |
| 138-143.6 MHz AERONAUTICAL MOBILE (OR) | 138-143.6 FIXED MOBILE RADIOLOCATION Space research (space-to-Earth) | 138-143.6 FIXED MOBILE Space research (space-to-Earth) 5.207 5.213 |
| 5.210 5.211 5.212 5.214 |  |  |

5.203C	The use of the space operation service (space-to-Earth) with non-geostationary satellite short-duration mission systems in the frequency band 137-138 MHz is subject to Resolution 660 (WRC-19). Resolution 32 (WRC-19) applies. These systems shall not cause harmful interference to, or claim protection from, the existing services to which the frequency band is allocated on a primary basis.     (WRC-19)

5.204	Different category of service:  in Afghanistan, Saudi Arabia, Bahrain, Bangladesh, Brunei Darussalam, China, Cuba, the United Arab Emirates, India, Indonesia, Iran (Islamic Republic of), Iraq, Kuwait, Montenegro, Oman, Pakistan, the Philippines, Qatar, Singapore, Thailand and Yemen, the frequency band 137-138 MHz is allocated to the fixed and mobile, except aeronautical mobile (R), services on a primary basis (see No. 5.33).     (WRC-19)

5.205	Different category of service:  in Israel and Jordan, the allocation of the band 137-138 MHz to the fixed and mobile, except aeronautical mobile, services is on a primary basis (see No. 5.33).

5.206	Different category of service:  in Armenia, Azerbaijan, Belarus, Bulgaria, Egypt, the Russian Federation, Finland, France, Georgia, Greece, Kazakhstan, Lebanon, Moldova, Mongolia, Uzbekistan, Poland, Kyrgyzstan, the Syrian Arab Republic, Slovakia, the Czech Rep., Romania, Tajikistan, Turkmenistan and Ukraine, the allocation of the band 137-138 MHz to the aeronautical mobile (OR) service is on a primary basis (see No. 5.33).     (WRC-2000)

5.207	Additional allocation:  in Australia, the band 137-144 MHz is also allocated to the broadcasting service on a primary basis until that service can be accommodated within regional broadcasting allocations.

5.208	The use of the band 137-138 MHz by the mobile-satellite service is subject to coordination under No. 9.11A.     (WRC-97)

5.208A	In making assignments to space stations in the mobile-satellite service in the frequency bands 137-138 MHz, 387-390 MHz and 400.15-401 MHz and in the maritime mobile-satellite service (space-to-Earth) in the frequency bands 157.1875-157.3375 MHz and 161.7875-161.9375 MHz, administrations shall take all practicable steps to protect the radio astronomy service in the frequency bands 150.05-153 MHz, 322-328.6 MHz, 406.1-410 MHz and 608-614 MHz from harmful interference from unwanted emissions as shown in the most recent version of Recommendation ITU-R RA.769-2.     (WRC-19)

5.208B*	In the frequency bands:

137-138 MHz,
		157.1875-157.3375 MHz,
		161.7875-161.9375 MHz,
		387-390 MHz,
		400.15-401 MHz,
		1 452-1 492 MHz,
		1 525-1 610 MHz,
		1 613.8-1 626.5 MHz,
		2 655-2 690 MHz,
		21.4-22 GHz,

Resolution 739 (Rev.WRC-19) applies.     (WRC-19)

5.209	The use of the bands 137-138 MHz, 148-150.05 MHz, 399.9-400.05 MHz, 400.15-401 MHz, 454-456 MHz and 459-460 MHz by the mobile-satellite service is limited to non-geostationary-satellite systems.     (WRC-97)

5.209A	The use of the frequency band 137.175-137.825 MHz by non-geostationary-satellite systems in the space operation service identified as short-duration mission in accordance with Appendix 4 is not subject to No. 9.11A.     (WRC-19)

5.210	Additional allocation:  in Italy, the Czech Rep. and the United Kingdom, the bands 138-143.6 MHz and 143.65-144 MHz are also allocated to the space research service (space-to-Earth) on a secondary basis.     (WRC-07)

5.211	Additional allocation:  in Germany, Saudi Arabia, Austria, Bahrain, Belgium, Denmark, the United Arab Emirates, Spain, Finland, Greece, Guinea, Ireland, Israel, Kenya, Kuwait, Lebanon, Liechtenstein, Luxembourg, North Macedonia, Mali, Malta, Montenegro, Norway, the Netherlands, Qatar, Slovakia, the United Kingdom, Serbia, Slovenia, Somalia, Sweden, Switzerland, Tanzania, Tunisia and Turkey, the frequency band 138-144 MHz is also allocated to the maritime mobile and land mobile services on a primary basis.    (WRC-19)

5.212	Alternative allocation:  in Angola, Botswana, Cameroon, the Central African Rep., Congo (Rep. of the), Eswatini, Gabon, Gambia, Ghana, Guinea, Iraq, Jordan, Lesotho, Liberia, Libya, Malawi, Mozambique, Namibia, Niger, Oman, Uganda, Syrian Arab Republic, the Dem. Rep. of the Congo, Rwanda, Sierra Leone, South Africa, Chad, Togo, Zambia and Zimbabwe, the frequency band 138-144 MHz is allocated to the fixed and mobile services on a primary basis.    (WRC-19)

5.213	Additional allocation:  in China, the band 138-144 MHz is also allocated to the radiolocation service on a primary basis.

5.214	Additional allocation:  in Eritrea, Ethiopia, Kenya, North Macedonia, Montenegro, Serbia, Somalia, Sudan, South Sudan and Tanzania, the frequency band 138-144 MHz is also allocated to the fixed service on a primary basis.    (WRC-19)

# 4	Aircraft VHF transmitter and receiver characteristics

## 4.1	Aircraft VHF transmitter characteristics

The same antenna pattern is considered for aircraft VHF transmitters and receivers.

In terms of transmitted power, the minimum aircraft transmit output powers for voice are 16 watts for 200 nautical miles maximum range, and 4 watts for 100 nautical miles maximum range. The first figure of 16 watts is retained in this report, as the range between aircraft and satellite will exceed 200 nautical miles as shown in next sections.

## 4.2	Aircraft VHF receiver characteristics

### 4.2.1	Aircraft VHF receiver antenna

Aircraft are usually equipped with two or three VHF antennas, in which case at least one of them is located on top of the aircraft, and one on the bottom. In the case of three VHF antennas, their typical location installed on a generic aircraft is shown in Figure 2 below provided by ICAO.

Figure 2

Typical VHF antenna location on aircraft

The aircraft VHF receiving antenna pattern is obviously an essential element to consider in the studies. The performances of available products show that:

–	Relatively low gains are achieved

–	Radiation patterns are globally omni-directional, and more precisely

•	omnidirectional in azimuth

•	cosinusoidal in elevation, meaning a theoretical zero is achieved at aircraft zenith (90° elevation).

As a guide, ICAO has provided the following general characteristics of VHF antenna to be used as a baseline for ITU-R studies. ICAO has also confirmed the co-sinusoidal shape and consequential null at aircraft zenith, which has an important implication on the performance of the satellite VHF link: one can assume that the AMS(R)S downlink operation is expected to be ‘off-zenith’ between the aircraft and the satellite.

–	Frequency range: 118-137 MHz

–	Polarization: Vertical

–	Radiation pattern: Omni directional

–	Gain: −1 dBi

### ** ICAO Note to ITU-R WP-5B:

### No further inputs from Section 4.2.2 onwards at this time

____________________

*	This provision was previously numbered as No. 5.347A. It was renumbered to preserve the sequential order.