FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

Seventeenth Working Group meeting

Cairo, Egypt, 30 August to 07 September 2023

Agenda Item 3b: WRC-23 AI 1.7 VHF AMS(R)S

Space-Based VHF Proof of Concept Testing

(Presented by Matthew Kelly)

SUMMARY

In June 2023 Skykraft Space-Based VHF Proof of Concept test satellites were launched and in July 2023 Skykraft successfully transmitted an AM DSB Modulated signal within the VHF Airband from a LEO satellite and received it at a ground station. This paper presents details of the testing and information of the success of the testing mission. This paper provides confidence that technology is being developed and will be implemented to take advantage of the new AMS(R)S allocation being considered at WRC-23.

INTRODUCTION

    - On the 13 June 2023 at 7:35am (AEST) the Skykraft Block 3 payload was launched on SpaceX’s Transporter-8 Mission. In July 2023 Skykraft successfully transmitted an AM DSB Modulated signal within the VHF Airband from a LEO satellite and received it at a ground station, with follow on testing in August 2023.

DISCUSSION

    - Airservices Australia (Airservices) put a number of controls and processes in place to operationally monitor the frequencies in use at the time of testing. Air Traffic Controllers were notified of the testing and a procedure was in place to report any interference and then to cease the testing.

    - Airservices has not experienced any interference to our terrestrial VHF systems as a result of the Space-Based VHF testing that is currently underway. Airservices and Skykraft have been working together to coordinate test frequencies and protocols to mitigate any risk to our in-service VHF system. The tests did not involve connecting to any Airservices equipment or test beds. The Skykraft Space-Based VHF service is still in development and Airservices is not using it operationally.

    - The testing included transmitting a pre-recorded voice message from the Satellite, to a hand held 3-element Yagi antenna on the ground. The radiated signal level is kept under 5 Wattts to minimise impact on the prototype satellite. This confirms that telecommands are correct, emitter is operating and that the collection system is able to record signals adequately.

    - Early indications from these test are showing that Space-Based VHF is feasible, with the received signal levels within the expected range and gaining an improved understanding of voice quality. These will be further refined with satellites of nominal operational space-based VHF capability, including increased power and antenna gain. The launch of these satellites is scheduled for June 2024.

    - The satellite under test is a model intended to demonstrate the viability of design and fabrication techniques, with Air Traffic Management payloads not fully developed. The existing satellite VHF and ADS-B payloads are placeholders for the systems that are being built for the operational service, however they are not capable enough to draw firm conclusions or publish data from.

    - The attached document provides additional details on the nominal link budgets calculated from this POC.

— END —