|  | International Civil Aviation Organization INFOMATION PAPER | FSMP-WG/3 IP/01 2016-07-20. |
| --- | --- | --- |

FREQUENCY SPECTRUM MANGEMENT  PANEL (FSMP)

Third Meeting of the Working Group of FSMP

(FSMP-WG/3)

Montreal, Canada, 6 – 14 September 2016

| Agenda Item | 7: Any other business |  |
| --- | --- | --- |

Information about an initiative

to revise Recommendation ITU-R SM.1009

- Compatibility between the sound-broadcasting service in the band of about 87-108 MHz and the aeronautical services in the band 108-137 MHz

Presented by: Felix Butsch

Prepared by: Martin Weber and Felix Butsch

SUMMARY

This paper provides information about an initiative to revise Recommendation ITU-R SM.1009 “Compatibility between the sound-broadcasting service in the band of about 87-108 MHz and the aeronautical services in the band 108-137 MHz”. Attached are: - German input paper to the recent meeting of ITU Working Party 1A on the initiative to revise ITU Recommendation ITU-R SM.1009. - Liaison Statement by WP1A to Working Parties 5B and 6A inviting to provide views about the proposed revision until 22nd November 2016.

INTRODUCTION

The latest update of Recommendation ITU-R SM.1009-1 was already approved in 1995. It is evident that technology advanced significantly over the past 20 years, which changes characteristics of systems in broadcasting as well as in aviation.  Since that time, technology of broadcasting service and the aeronautical services has been improved. New sound processing methodologies have been introduced for the broadcasting service. In addition new digital transmission systems (e.g. DARC, DRM+, HD-Radio) have been developed. This leads to changes of the spectrum characteristics of sound broadcasting signals and to a different interference potential.

In Germany the Federal Network Agency (BNetzA) in collaboration with the Federal Supervisory Authority for Air Navigation Services (BAF) with contributions by the German Air Traffic Service Provider (DFS) undertook an extensive investigation on the various aspects of the compatibility between analogue and digital broadcasting systems operating in the frequency band 87 – 108 MHz and aeronautical radionavigation systems in the adjacent band 108 – 137 MHz. (More details on the digital radio broadcast signal types considered in this investigation can be found in ICAO ACP WG/F, WP13 “Digital Broadcasting systems in the 87.5-108 MHz band”).

The result of these studies were presented to the ITU-R WP 1A (Document 1A/28-E), which is responsible for the Recommendation ITU-R SM .1009. In order to raise awareness of the aeronautical community on this matter the above mentioned contribution is attached.

ITU-R WP 1A carried the document forward as an Annex to the chairman’s report, in addition a liaison statement was send to ITU-R WP 5B and 6A for comments (Document 5B/98).  Responses are requested prior to the next planned meeting of WP 1A in 22-30 November 2016. For convenience this document is also attached.

Attachments:

- German input paper to the recent meeting of ITU Working Party 1A on the initiative to revise ITU Recommendation ITU-R SM.1009.

- Liaison Statement by WP1A to Working Parties 5B and 6A inviting to provide views about the proposed revision until 22nd November 2016.

— END —