|  | International Civil Aviation Organization REPORT | 7 April 2017. |
| --- | --- | --- |

FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

FOURTH MEETING OF THE WORKING GROUP OF FSMP

(FSMP-WG/4)

Bangkok, Thailand

29 March - 7 April, 2017

REPORT

1.	Introduction

1.1	The meeting was opened by Mr Loftur Jonasson from the ICAO Secretariat, Montreal and Mr Mike Biggs, the Rapporteur of Working Group FSMP (FSMP-WG).  Mr Jonasson acted as the Secretary of the meeting assisted by Mr. Li Peng and Mr. Frederic Lecat of the ICAO Asia and Pacific Office. Following introductions, Mr Biggs welcomed the group and provided introductory remarks, meeting information and housekeeping details.

1.2	The meeting was held in English. After the opening of the meeting the agenda was approved by the group. The agenda is contained in Appendix A.

1.3	The list of papers submitted for consideration by FSMP-WG is contained in Appendix B. The list of participants is in Appendix C.

1.4	The material in this report is organized by meeting agenda item number, and does not necessarily reflect the order of discussions.  The meeting conducted a review of the actions from the last meeting. Actions captured during discussions are shown in Appendix D, together with status of prior-meeting(s) actions.

2.	Agenda Item 2 – Completion of ICAO Position for WRC-19

2.1	Flimsy 02Rev1 provided comments from States on the draft ICAO Position for WRC-19.  All of the comments were discussed and resolved.  As a result of discussions, the only change to the ICAO Position was with respect to paragraph 3 of the position on WRC-19 agenda item 1.7. The that paragraph was modified to read “To ensure that any change to the regulatory provisions and spectrum allocations resulting from this agenda item do not preclude the use of SOS any particular allocations for space planes if the this radiocommunication service is deemed appropriate for such use.”

2.2	The meeting also identified changes to Note 4 in the Position that should be considered during the next update. Those changes would modify the note to read: “Note 4.― No impact on aeronautical safety services has been identified from WRC-19 Agenda Items 1.1, 1.2, 1.3, 1.4, 1.5, 1.6, 1.15, 2, 3, 5, 6, 7, 9.1 (Issue 9.1.1), 9.1 (Issue 9.1.2), 9.1 (Issue 9.1.5), 9.1 (Issue 9.1.7), 9.1 (Issue 9.1.8), 9.1 (Issue 9.1.9), 9.2 and 9.3 which are therefore not addressed in this position.”.

3.	Agenda Item 3 – Revision of the ICAO Frequency Spectrum Handbook (Doc 9718 Vol I)

3.1	WP10 provided comments and proposed edits to the HF usage and HF descriptions in Chapter 7, together with context for those changes. As part of the discussion the meeting agreed that while Appendix 27 to the Radio Regulations may require some minor updates, attempting to make modifications would require a tremendous amount of effort with likely very little benefit or support.

3.2	WP18 proposed changes related to GNSS (GLONASS) and COSPAS-SARSAT material in Chapter 7.

3.3	WP25 suggested modified text for the VSAT section in Chapter 7 of the DOC 9718, with examples of how important the system is for ATM infrastructure in the African region.

3.4	The proposals in WPs 10, 18, and 25, together with other changes received during the planned edit period before the meeting, were combined into a single Flimsy 05 and discussed by the meeting.  The resulting agreed modifications to the Handbook Volume 1 are contained in Appendix E. Of particular note is an addition to Chapter 9 that “Non safety of life services, willing to share a safety of life band have to comply with the aviation safety requirements applicable in that band  including certification of radio equipment, software and radio operators, as well as assumption of liability.”

3.5	The meeting also discussed a number of topics that should be considered for the next update of the Handbook.  These included: (a) restructure of the material to have a volume on policy/strategy, a volume on spectrum management, and a volume on frequency management; (b) addition of material on aviation use of VSAT to Chapter 7 (see AI 04-03); (c) improve the format of Chapter 7 including ensuring consistency across the bands; and (d) update to Table 9-1 (Chapter 9.6).

4.	Agenda Item 4 – Radio Altimeter and WAIC Issues

4.1	Presentation 1 provided a brief history of the Aerospace Vehicle Systems Institute (AVSI) and its role in wireless avionics intra-communications (WAIC), a description of tests investigating WAIC interference on radio altimeters, and their future plans for additional testing.  This work directly supports the FSMP’s radio altimeter and WAIC job cards. This presentation was a general overview, with more detail on certain aspects provided in Presentation 2 and 3.

4.2	Presentation 2 provided some initial results from on-going radio altimeter susceptibility testing. Those laboratory tests, which will be expanded to include flight testing, introduce simulated WAIC signals into radio altimeters and catalogue the performance impacts. The meeting thanked AVSI for the information, and reminded them that an adjacent band interference rejection mask for the altimeters would also be a useful outcome of the testing to support the FSMP job card on the topic (see section 4.4 below).

4.3	Presentation 3 provided information on on-going interference path loss (IPL) measurements to determine the expected isolation between WAIC transmitters and radio altimeters – either on the same aircraft, or on nearby aircraft in an airport ground environment.  Initial results have indicated a worst case (i.e., minimum) IPL of about 80 dB for signals in the 4 200 to 4 400 MHz range.

4.4	Flimsy 8 provided the FSMP Job Card on Radio Altimeters as approved by the Air Navigation Commission (ANC). With regard to the version agreed by FSMP/2, the ANC-approved version includes an action to “Conduct an aircraft fleet equipage impact analysis and develop detailed transition plans based on industry input and expected safety benefit.” That action is assigned to the FSMP, with support from AIRP and FLTOPSP, and with a deadline of Q1 2019. During discussion the meeting noted that the primary information for that action would likely need to come from IATA and/or ICCAIA (see AI 04-06).

5.	Agenda Item 5 – Development of (planned) material for ITU-R Studies

5.1	WP1 contained a Liaison Statement from ITU-R WP1A, titled: Assessment of the impact of Unwanted Radio Frequency energy generated by non-radiocommunication equipment to radiocommunication services. In the document, WP1A requested information from ICAO on (a) details of instances of disturbances and degradation within their experience caused by electrical/electronic equipment and telecommunication distribution systems operating over metallic conductors, and (b) the noise floor they anticipate in order to satisfy their planned operational requirements, especially where there are precise expectations on the noise floor.

5.1.1	After discussion, the meeting agreed to a short term (1 month) action (AI 04-01) for all FSMP members to provide the FSMP Rapporteur with information via Email on items (a) and (b) above. He will then compile that information into a simple response back to WP1A.

5.2	IP3 provided an overview of NASA’s UAS Command and Control Communications (C2) Communications Project Phase 2. In previous FSMP meetings, Phase 1 focusing on

line-of-sight C2 communications had been briefed. Phase 2 focuses on beyond-line-of-sight C2 communications, and is now underway. The meeting appreciated the information and asked to be kept informed on future developments. In addition, concern was raised about assumed compatibility criteria for the L-band data link versus other incumbent aviation systems. Of particular note were that the DME protection criteria, and the +/- 10 MHz guardband assumption around 1030/1090 MHz, used in the RTCA command and control datalink MOPS did not agree with assumptions used in other analyses.  The meeting agreed that the issue needed to be further studied.

5.3	Fixed Satellite Service for UAS/RPAS

5.3.1	WP2 provided a liaison from ITU-R WP4A to ICAO titled “ICAO UAS/RPAS SARPS activities”. In the liaison WP4A thanked ICAO for its update on the activities regarding the issue of preparation of its Standards and Recommended Practices (SARPs) for UAS CNPC links. Regarding however the ICAO request for  various information on technical and operational characteristics for satellite based unmanned aircraft CNPC links, WP 4A stated that they believe that preparation and finalization of such characteristics for UAS CNPC links falls within the mandate of WP 5B. WP 4A stands prepared to examine the final characteristics once concluded/agreed by WP 5B. The meeting noted the response.

5.3.2	WP3 provided a reply liaison from WP5B to ICAO titled “Unmanned Aeronautical Vehicle system characteristics and interference environment”. In the response, WP5B stated that they had considered the ICAO request for information with respect to the characteristics of unmanned aeronautical vehicle satellite links and the interference environment within which they will have to operate. Unfortunately, at the time WP5B was not in a position to provide any characteristics. Work has been initiated to identify such characteristics and WP5B will inform ICAO as and when they become available.  WP 5B further asked if ICAO could indicate, where possible, the appendix 4 to the Radio Regulations reference for the data elements that have been provided in Documents 5B/115 and 5B/116 for which WP 5B were requested to provide values. The meeting noted that the ICAO Remotely Piloted Aircraft Systems Panel (RPASP) had addressed the requested actions (see WP12 below).

5.3.3	WP11, from Working Group 2 (WG2) of the RPASP presented a proposed Liaison Statement to ITU-R that provides information related to the development of SARPS for unmanned aircraft systems (UAS) control and non-payload communications (CNPC) links in accordance with ITU-R Resolution 155 (WRC-15).  The attachment to the Liaison Statement contains candidate SARPS that apply to terrestrial and satellite based UAS CNPC links. The meeting discussed the text and material for a proposed liaison from ICAO on the topic is contained in Appendix F.  The material will also be sent “for information only” to WP4A.

5.3.4	WP12, from the co-rapporteurs of RPASP WG2, provided proposed text for a reply liaison to ITU-R WP5B in response to their liaison regarding information required to support RPAS SARPS development.  The meeting discussed the text and material for a proposed liaison from ICAO on the topic is contained in Appendix G. The material will be sent to WP5B “for information and action as appropriate”, and “for information” to WP4A.

5.4	GADSS

5.4.1	WP4 provided the text of a liaison from WP5B to ICAO regarding Questions about the systems to be used and the spectrum requirements of Global Aeronautical Distress and Safety System (GADSS)”.

5.4.2	IP5 provided the latest version of the GADSS Concept of Operations (ConOps). It was noted that the version provided included redline/strikeout, but it was expected that a clean version would be available for submission to WP5B in May 2017.

5.4.3	Flimsy 4 provided responses to an ICAO survey regarding GADSS. That survey was sent to a number of companies and asked the following questions: (a) Does your company offer or plan to offer equipment that will satisfy normal aircraft tracking requirements? If yes, in what frequency band does it/will it operate?; (b) Does your company offer or plan to offer equipment that will satisfy ATS surveillance systems requirements? If yes, in what frequency band does it/will it operate?; (c) Does your company offer or plan to offer equipment that will satisfy autonomous distress tracking (ADT) requirements?  If yes, in what frequency band does it/will it operate?; (d) Does your company offer or plan to offer equipment that will satisfy data retrieval such as data streaming?  If yes, in what frequency band does it/will it operate?; (e) Has your company identified any frequency spectrum issues? If so, please provide more details. The meeting agreed that the responses could assist in the ICAO response to WP5B (see 5.4.1 above).

5.4.4	WP16 provided an overview of the international Cospas-Sarsat (C/S) system and suggested elements for C/S to meet the requirements expressed by GADSS regarding the distress phase. In addition, a proposed answer back to ITU-R WP5B in connection with WRC19 agenda item 1.10, showing the fact that C/S may meet all the basic distress tracking requirements.

5.4.5	WP19 discussed the potential need for additional new spectrum for each of the GADSS ConOps functions to assist the FSMP in providing a draft response to ITU-R WP 5B Liaison Statement 5B/TEMP/62.

5.4.6	WP27 discussed that while it was likely there would be no change to Article 5 of the Radio Regulation due to GADSS, there may be changes required to other Articles. In particular possible changes to portions of Chapters VI, VII and VIII (i.e., portions of Articles 21-45) were delineated. Those changes were separated into changes that could (a) be addressed under WRC-19 agenda item 1.10, (b) be accommodated perhaps under WRC-19 agenda item 9.2, and (c) should be pursued on a more long-term basis. The meeting agreed to an action (AI 04-02) to review the material prior to FSMP-WG5 in Sep 2017, noting that it would be an agenda item for that meeting.

5.4.7	All of the documents were presented, and then the meeting had a general discussion on GADSS. After that discussion, elements for a possible response to the WP5B liaison statement were agreed. Those elements are given in Appendix H.

5.5	Proposed updates of Recommendation ITU-R SM.1009

5.5.1	WP5 provided the text of a liaison from WP5B to WP1A regarding proposed revision of recommendation ITU-R SM.1009-1. That Recommendation outlines the criteria for establishing compatibility between FM Broadcast stations in the band 87 – 108 MHz and the aeronautical services operating in the band 108 – 137 MHz. It was noted that the ICAO liaison to the November 2016 meeting of WP5B, based on material developed by FSMP-WG3, was well-represented in the WP5B response to WP1A.

5.5.2	WP20 presented some views and concerns of the aviation community on the proposed revision of SM.1009-1, and presented elements of a draft reply liaison to ITU-R WP 5B for consideration of the meeting.

5.5.3	The meeting was informed that the current expectation was that WP1A would not modify SM.1009-1, and instead the new material would be simply included in a new report as a procedure used by one administration. After discussion the meeting agreed that no further FSMP action was currently required, however the situation should be monitored closely.

6.	Agenda Item 6:  RF Handbook Volume II (Doc 9718 Vol II), Frequency Assignment Planning

6.1	WP7, based on studies performed by both Eurocontrol and the EUR frequency Management Group (FMG), proposed a revision of the frequency assignment planning criteria for VHF air/ground communication systems operating in the frequency band 117.975 – 137 MHz from what is currently in Doc 9718, Volume II. Details on this revision were earlier present to ACP WG-F 32 in 2015. After discussion, the meeting agreed to the proposed modifications.

6.2	WP13 proposed to incorporate in Doc 9718, Volume II, guidance material on the implementation and use of back-up frequencies that was developed and approved by APANPIRG (Conclusion 26/49), with the view to provide guidance on a global basis when assigning backup frequencies. After some small modifications to the material, the meeting approved its addition (including the example template) to 9718 Volume II.

6.3	IP07 discussed attempts to optimize VHF assignments in the US by testing the RF performance of VHF voice radios.  The initial tests results included includes Adjacent Channel Rejection (ACR), intermodulation attenuation, and Out of Band Emissions (OOBE) masks.  After some discussion and noting that additional testing was planned, the meeting asked to be kept informed as the testing developed.

7.	Agenda Item 7:  5 GHz Band Planning

7.1	AeroMACS Status

7.1.1	No papers were presented on this topic.

7.2	Global UAS/RPAS channel plan

7.2.1	IP1 provided the results of trials on ground station handover during unmanned aircraft (UA) operations.  In particular, the experimental system demonstrated how 5 GHz control and non-payload communications (CNPC) links could be handed off between multiple ground stations to ensure seamless operation of the UA. The paper generated considerable discussion about how small UA would be handled, especially given the numbers foreseen. It was agreed that classically the small UA would not operate in non-segregated airspace, nor require international standardization.  However, due to the potential for impact to ICAO systems and/or aviation safety it was recognized that there will likely need to be some ICAO role in the development of small-UAS rules.  The meeting asked to be kept informed about progress with the UA trials described in the paper, as well as any compatibility studies, if any, with current and future systems foreseen to be performed by the authors. Further it was requested that FSMP be provided with a comparison of the signal format presented in IP1 to the guidelines being issued by RTCA in its UAS command and control datalink MOPS.

7.2.2	WP09 provided considerations to be taken in the establishment of channel plans that utilize the aeronautical mobile (R) and aeronautical mobile-satellite (R) service allocations in the 5030-5091 MHz band for terrestrial and satellite systems supporting UAS C2 links, taking into account the presence of MLS and identifying operational constraints related to its presence. Included were discussions on protection of microwave landing systems, and how to equitably share the band between the terrestrial and satellite C2 links. The meeting appreciated the material and asked to be kept informed as the situation develops. Further developments on the topic will be made through work of the RPASP for the establishment of amendment proposals to Annex 10, Volume V in liaison with FSMP. Analysis of the impacts on aeronautical systems such as AeroMACS and MLS will be established by the FSMP.

8	Agenda Item 8: New provisions to support aeronautical radiocommunications

8.1	WP17 informed the FSMP about a Remotely Piloted Aircraft Systems Panel (RPASP) Job Card (RPASP.002.04, WPE 9371) regarding “Amendment proposals to Annex 10, Volume V”.  In the paper the RPASP proposed:

- to start assessing - between the FSMP and the RPASP - the C2 Link spectrum details, including spectrum management considerations, which could be part of Annex 10 Volume V; This assessment could take the form of a common FMSP/RPASP working paper or any other form the FSMP may consider suitable; and

- to include any other C2 Link spectrum considerations in the new proposed Annex 10 Volume VI, which is dedicated to RPAS C2 Link SARPs.

After discussion the meeting agreed with this proposed way forward.

9.	Agenda Item 9:  Interference from non-aeronautical sources

9.1	PMSE

9.1.1	A number of papers were input dealing with the consideration by some States to operate programme making and special events (PMSE) equipment in the 960-1164 MHz band. Each of those papers was introduced, and then the meeting held a general discussion of the topic.

9.1.2	WP6 introduced a letter from the CEPT electronics communications committee (ECC) working group FM (WGFM) requesting information on aeronautical, regulatory, legal and technical matters related to the possible sharing the frequency band 960-1164 MHz with low power audio PMSE applications (Wireless microphones and In-Ear monitors). The meeting agreed to develop material that could be used for an ICAO response to WGFM.

9.1.3	Presentation 4 provided an overview of PMSE and the rationale for the current push to share the 960-1164 MHz band. It was understood that the approach was in-part driven by a desire for a global long-term band, and the mistaken impression that aviation will move away from DME as GNSS progresses, and as a result the band will become available. Of particular interest were the results of studies that have shown historically the PMSE community and PMSE equipment do not always meet their operating requirements/limitations. During discussion one participant indicated that he thought the characterization of the radio regulator proposing PMSE in the DME band was not correct in the presentation. Instead, he felt the regulator had been cooperatively addressing the aviation concerns.

9.1.4	WP8 provided additional information on the WGFM-PMSE efforts, and in particular the view of an airframe manufacturer on potential impacts.  The paper invited the meeting to develop material which could be submitted as a contribution to the next meeting of WGFM.

9.1.5	WP15 addressed operational considerations on the potential PMSE use of the 960 – 1164 MHz frequency band. The paper focused on DME and GNSS navigation functions and the potential consequences for disruption of normal aircraft operations stemming from degradation or loss of any of these navigation systems while operating in required navigation performance (RNP) airspace. The meeting was invited to use elements as part of a response to WGFM.

9.1.6	WP21 focused on regulatory and legal issues with respect to the feasibility of introducing low power audio PMSE in the band 960-1164MHz. Again the meeting was invited to use elements as part of a response to WGFM.

9.1.7	WP23 presented information from the Data Communications Infrastructure Working Group of the ICAO Communications Panel. That Working Group is developing standards for a future L-band terrestrial aeronautical data link system using spectrum already allocated to aeronautical services. The candidate system will need to co-exist with existing civil and military aviation systems which also make use of this spectrum. Extensive evaluations have been conducted to ensure compatibility between the new aeronautical data link system and the other aviation systems using this spectrum. Because of that, the paper noted the great concern about the possibility of PMSE using the same spectrum in certain States. In particular, i) accommodation of PMSE would likely negatively impact performance of the new aeronautical data link, and ii) mitigation or minimization of that impact to meet aviation safety standards would be very difficult. The FSMP was asked to take the necessary actions to ensure that the integrity of the L-band aeronautical spectrum is preserved and that adequate spectrum is available for future aeronautical communications.

9.1.8	WP24 argued that opening the 960-1164 MHz band to commercial, non-safety terrestrial applications represents a fundamental change. That change would have significant economic impact, as civil aviation equipment would need to be modified to accommodate new failure mechanisms; re-certification, revision of procedures, training, etc.  In addition PMSE sharing of the 960-1164 MHz band would constrain incumbent Civil and Military users of the band and prevent aviation from planned modernisation.

9.1.9	After discussion the meeting formed a drafting group to develop elements that could be used in an ICAO response to WGFM. That drafting group met a number of times, and held extensive discussions on all the contributions to compile text. The drafting group output, as agreed by the FSMP-WG4 and including final edits by the FSMP Secretary, is provided in Flimsy 9. The meeting appreciated the hard work of the drafting group and noted that it was vital that the aviation community (in particular the European Civil Aviation Authorities) fully engage and participate in the on-going debates about possible introduction of PMSE in the 960-1164 MHz band. The meeting also asked IATA to bring the issue to the attention of the airlines to enlist their support.

9.2	WP22 provided information on a recent case of interference to aviation VHF communications in the United States. After considerable investigation, the source was determined to be LED-based lights that were used to replace fluorescent tube lighting in a maintenance shelter. The meeting was invited to note that LED bulbs can be a source of broadband interference, and that some LED lights, designed as direct replacement for conventional bulbs and using conventional sockets, are not readily apparent as being LED. The meeting appreciated the information and asked to be kept informed as more information becomes available. It was also suggested the information could be presented to WP5B by the author at a future meeting.

9.3	Flimsy6 presents information from the recent Munich Satellite Navigation Summit that discussed the vulnerabilities of GNSS from the increasing use of jamming devices and that consensus was reached on the need for a terrestrial back-up to GNSS. Technical characteristics for such a system were also defined.  The meeting was asked to note the critical importance and availability of terrestrial navigation systems used by the aviation transportation industry, and that those systems must be protected from interference and disruption. The meeting agreed and noted the paper provided yet another reason for being cautious about allowing non-aviation systems into frequency bands used by aviation navigation systems (see, for example, discussion in section 9.1 above).

10.	Any Other Business

10.1	IP2 provided a summary of the final results of the L-Band and C-Band air-ground channel model development based on data gathered through a propagation flight measurement campaign performed by NASA. The material will be used to assist future studies.

10.2 	IP4 contained the report of the last meeting of the ICAO NSP Spectrum Working Group (SWG), which was held at ICAO Headquarters, Montreal from 30th November to 2nd December 2016. The main topics of the meeting were GBAS VDB Frequency coordination criteria, Insertion of GNSS RFI Mitigation Plan into GNSS Manual, LDACS, Potential need for revision of frequency planning Rules for ILS-VOR, and Electromagnetic Compatibility of Beidou with Mobile Satellite Service.

10.3	IP06 reported that Phase 1 of the Command and Control Communications (C2) Subproject of NASA’s UAS Integration in the National Airspace System Project included the development and testing of prototype C2 radio systems. The paper provided an overview of the functionality and testing of the fifth and final Phase 1 generation of the prototype radio system. That radio was used for the RTCA UAS C2 MOPS validation testing.

10.4	WP14 provided an initial proposal for HF Ground Stations technical parameters for protection from adjacent channel emissions and interference.  The information was provided for development and possible agreement within ICAO before deciding if suitable for formalizing externally (i.e. an ITU-R Recommendation). The meeting agreed with the material but suggested that instead of ITU, it be catalogued in Doc 9718, Volume II. The author was given an action to make a proposal for that addition at FSMP-WG5 (see AI 04-04).

10.5	WP26 addressed the need to develop a vision of the global air traffic management system that ideally should be in place by 2060, the rationale for such a vision and the potential benefits. The paper generated considerable discussion and debate. One driver for the approach was to try to capture benefits in terms of cost savings for removing on-board equipment. To assist with that analysis, an action was assigned (see AI 04-05) to provide equipment physical characteristics (e.g., weight) as noted in the Annex to the paper.

11.	Date of next meeting

11.1	The next meeting, FSMP-WG/5, is tentatively scheduled for 4-8 September 2017 at the DGAC offices (Farman) in Paris, France.  It will be followed by a 2 day spectrum workshop for the MID/EUR regions 11-12 September, 2017 at the ICAO Paris Office (Neully).  Papers for FSMP-WG/5 are due one week prior to the meeting.

APPENDICES

Appendix A – Agenda

Appendix B – List of Working Papers, Information Papers and Flimsies

Appendix C – List of Participants

Appendix D – Action Item List

Appendix E – Modifications to Doc 9718, Volume I

Appendix F – Elements for ICAO response to ITU WPP5B regarding status of UAS SARPS development

Appendix G – Elements for ICAO response to ITU WPP5B regarding proposed UAS SARPS text

Appendix H– Elements for proposed ICAO Liaison to WP5B regarding GADSS

APPENDIX A

INTERNATIONAL CIVIL AVIATION ORGANIZATION

fourth working Group Meeting of the

Frequency Spectrum Management Panel

(FSMP-WG/4)

(Bangkok, Thailand  29 March-7 April, 2017)

AGENDA

Agenda Item 1: Opening and working arrangements

- Action Item review

Agenda Item 2: Completion of ICAO Position for WRC-19

- Review and address State Letter responses

Agenda Item 3: Revision of the ICAO Frequency Spectrum Handbook (Doc 9718 Vol I)

- Updates to the Frequency Spectrum Policy Statements (Chapter 7)

- Updates to the Frequency Spectrum Strategy (Chapter 8)

(the two items above to be presented to ANC and Council for approval in May/June, in a package together with ICAO Position for WRC-19)

Agenda Item 4: Radio Altimeter and WAIC issues

- Status of compatibility testing

Agenda Item 5: Development of (planned) material for ITU-R Studies on:

- FSS for UAS

- GADSS

- Status of proposed update to Recommendation ITU-R SM.1009

Agenda Item 6: RF Handbook Volume II (Doc 9718, Vol. II), Frequency Assignment Planning

- Further development in preparation for a second edition

Agenda Item 7: 5 GHz Band Planning

- AeroMACS status

- Global UAS/RPAS channel plan

Agenda Item 8: New provisions to support aeronautical radiocommunications

- LDACS, status update CP WG-T

Agenda Item 9: Interference from non-aeronautical sources

Agenda Item 10: Any Other Business

APPENDIX B

List of Papers

List of Working Papers

| Paper # | Source | Title | Agenda Item |
| --- | --- | --- | --- |
| 1 | Secretary | Liaison from WP1A titled: Assessment of the impact of Unwanted Radio Frequency energy generated by NON radiocommunication equipment to radiocommunication services (Questions ITU-R 221-2/1 & ITU-R 236/1) | 5 |
| 2 | Secretary | Liaison Statement from ITU-R WP4A, titled: ICAO UAS/RPAS SARPs activities | 5 |
| 3 | Secretary | Reply Liaison Statement from ITU-R WP5B, titled: Unmanned Aeronautical Vehicle System Characteristics and Interference Environment | 5 |
| 4 | Secretary | Liaison Statement from ITU-R WP5B, titled: QUESTIONS ABOUT THE SYSTEMS TO BE USED, AND THE SPECTRUM REQUIREMENTS, OF GADSS | 5 |
| 5 | Secretary | Liaison Statement from ITU-R WP5B, titled: PROPOSED REVISION OF RECOMMENDATION ITU-R SM.1009-1 | 5 |
| 6 | Secretary | Letter from CEPT Electronics Communications Committee – Working Group FM, titled: Request for information on aeronautical, regulatory, legal and technical matters related to the possible sharing the frequency band 960-1164 MHz with wireless microphones | 10 |
| 7 | Secretary | Revision of adjacent frequency assignment planning criteria | 6 |
| 8 | C. Pichavant | Programme Making and Special Event (PMSE) in the 960 – 1164 MHz frequency band | 9 |
| 9 | C. Fleury, N. Van Wambeke, M. Neale, D. Nellis | Considerations for establishment of channel plans for the AMS(R)S and AM(R)S allocations in the 5030-5091 MHz frequency band for use by satellite and terrestrial systems supporting UAS C2 Links | 7 |
| 10 | A.Roy | Aviation HF usage and proposed edits to HF descriptions in Chapter 7 | 3 |
| 11 | RPASP WG-2 | Draft Liaison Statement to ITU-R WP-4A and WP-5B Containing Candidate Terrestrial and Satellite Based C2 Link SARPS | 5 |
| 12 | M. Neale, D. Colin (RPASP WG2) | LIAISON TO ITU-R WP5B IN RESPONSE TO THEIR LIAISON REGARDING INFORMATION REQUIRED TO SUPPORT RPAS SARPS DEVELOPMENT | 5 |
| 13 | Secretary | Backup Frequencies | 6 |
| 14 | G. Baker | PROPOSAL FOR NEW ITU-R RECOMMENDATION FOR HF GROUND STATION PROTECTION CRITERIA | 10 |
| 15 | J. Taylor | Operational Considerations on the Potential Use of Programme Making and Special Event (PMSE) in the 960 – 1164 MHz frequency band | 9 |
| 16 | C. Fleury | Global Aeronautical Distress and Safety System (GADSS) and the Cospas-Sarsat system | 5 |
| 17 | Secretary | Job Card RPASP.002.04, WPE 9371: “Amendment proposals to Annex 10, Volume V” | 8 |
| 18 | D. Muzychenko | PROPOSED CHANGES TO ICAO DOC 9718, VOL. I | 3 |
| 19 | J. Taylor | ITEMS FOR CONSIDERATION ON WRC-19 AGENDA ITEM 1.10 | 5 |
| 20 | J. Taylor | ITEMS FOR CONSIDERATION ON THE PROPOSED REVISION OF ITU-R RECOMMENDATION SM.1009-1 | 5 |
| 21 | C. Fleury | PRELIMINARY INVESTIGATIONS ON REGULATORY AND LEGAL ISSUES ON THE FEASIBILITY OF INTRODUCTING LOW POWER PMSE IN THE BAND 900-1164 MHz | 9 |
| 22 | M. Biggs | Case Study: LED lighting interference to aviation VHF communications | 9 |
| 23 | Secretary | CONCERNS OVER THE PROSPECTIVE USE OF AERONAUTICAL SPECTRUM FOR NON-SAFETY SERVICES | 9 |
| 24 | R. Khatcherian | Importance of having an ICAO position on PMSE sharing the 960-1164 MHz band | 9 |
| 25 | L. Tele | VSAT for Aeronautical usage | 3 |
| 26 | J. Mettrop | Future ATM System | 10 |
| 27 | J. Mettrop | World Radiocommunication Conference Report of the Director Agenda Item 1.10 | 5 |

List of Information Papers, Presentations and Flimsies

| Paper # | Source | Topic | Agenda Item |
| --- | --- | --- | --- |
| 1 | T. Takayama | Handover system of plural ground stations in plural ua operation | 7 |
| 2 | R. Kerczewski | Air-Ground Channel Models for L-Band and C-Band – Final Results | 10 |
| 3 | R. Kerczewski | Overview of NASA’s UAS C2 Communications Project Phase 2 | 5 |
| 4 | F. Butsch | Report of ICAO NSP Spectrum Working Group (30 Nov- 2 Dec, 2016) | 10 |
| 5 | Secretary | GADSS Concept of Operations (Ver 5.13+) | 5 |
| 6 | R. Kerczewski | UAS C2 Radio System – Final Phase 1 Development and Testing | 10 |
| 7 | A.Roy | Radio Testing to optimize VHF coordination | 6 |

| PRESENTATIONS |  |  |  |
| --- | --- | --- | --- |
| 1 | D. Redman | Update on AVSI WAIC-Radio Altimeter Coexistence Testing | 4 |
| 2 | M. Franceschini | WAIC Radio Altimeter Interference Susceptibility Testing | 4 |
| 3 | M. Franceschini | Measurement of Interference Path Loss between WAIC System installed inside the Aircraft Cabin and Radio Altimeters within the frequency band 4 200 – 4 400 MHz | 4 |
| 4 | R. Khatcherian | Use of Wireless Programme Making Special Event (PMSE) Equipment in the L-Band | 9 |

| FLIMSIES |  |  |  |
| --- | --- | --- | --- |
| 1 | Secretary | Spectrum policy/strategy updates | 3 |
| 2 | Secretary | ICAO Position State responses | 2 |
| 3 | Secretary | Draft ICAO WRC-19 Position | 2 |
| 4 | Secretary | Responses to ICAO GADSS survey | 5 |
| 5 | Rapporteur | Proposed 9718 Volume I updates | 3 |
| 6 | J. Taylor | GNSS Interference update | 10 |
| 7 | Rapporteur | Proposed elements for an ICAO response to WP5B | 5 |
| 8 | Secretary | Updated FSMP job card on radio altimeters | 4 |
| 9 | Chmn PMSE drafting gp | Proposed elements for an ICAO response to WGFM on PMSE | 9 |

APPENDIX C

Fourth meeting of the Working Group of the Frequency Spectrum Management Panel (FSMP-WG/4)

Bangkok, Thailand, 29 March to 7 April 2017

Attendance List

| Name / Title | Address / Telephone / E-mail |
| --- | --- |
| Australia |  |
| Eddy D’Amico | Airservices Australia GPO Box 367 Canberra ACT 2601 Australia Tel: +61 2 6268 5443 Mobile: +61 427 706 887 E-mail: eddy.damico@airservicesaustralia.com |
| Bangladesh |  |
| Mohammad Hamidul Haque Director (Communication) | Civil Aviation Authority of Bangladesh CAAB Headquarters, Kurmitola Dhaka 1229 Bangladesh Tel: +880 (2) 890 1403 Fax: +880 (2) 890 1411 E-mail: dcom@caab.gov.bd, Hamidul61@yahoo.com |
| A.K.M. Manzur Ahmed Deputy Director (Planning) | Civil Aviation Authority of Bangladesh CAAB Headquarters, Kurmitola Dhaka 1229 Bangladesh Tel: +880 (17) 2629 0536 Fax: +880 (2) 890 1411 E-mail: ahmedcaab@gmail.com, ddplan@caab.gov.bd |
| Beni Madhab Biswas Deputy Director (Telecom & OPS) | Civil Aviation Authority of Bangladesh CAAB Headquarters, Kurmitola Dhaka 1229 Bangladesh Tel: +880 (2) 890 1422 Fax: +880 (2) 890 1428 E-mail: ddcomops@caab.gov.bd |
| Mohammad Mehedi Hasan Assistant Programmer | Ministry of Civil Aviation and Tourism Room # 1908, Floor #19, Building #06 Bangladesh Secretariat, Dhaka Bangladesh Tel: +880 (2) 954 5024 Fax: +880 (2) 951 5499 E-mail: mehedi@mocat.gov.bd |
| Brazil |  |
| Vahe Antoine Yaghdjian | Departamento de Controle do Espaço Aéreo – DECEA Av. General Justo, 160 - Centro Rio de Janeiro, CEP: 20021-130 Brazil Tel: +55 (21) 999 55 3305 E-mail: vahevay@decea.gov.br |
| Gustavo Erivan Bezerra Lima CNS Manager | Avenue Gen Justo, 160 Centro Rio De Janeiro BJ 20021-130 Brazil Tel: +55 (21) 2101 6581 Mobile: +55 21 980 920 073 E-mail: erivangebl@decea.gov.br |
| Cambodia |  |
| Neang To Chief of Bureau of CNS | State Secretariat of Civil Aviation #44 Phnom Penh International Airport Russian Federation Blvd., Phum Ta Ngoun Sangkat Kakap, Khan Porsenchey Phnom Penh Cambodia Tel: +855 (23) 890 198 Fax: +855 (23) 890 102 E-mail: neangto.ans@gmail.com |
| Khuon Sineath | State Secretariat of Civil Aviation #44 Phnom Penh International Airport Russian Federation Blvd., Phum Ta Ngoun Sangkat Kakap, Khan Porsenchey Phnom Penh Cambodia Tel: +855 016 933 552 E-mail: sineathk@cats.com.kh |
| Heng Mengkong Official of CNS | State Secretariat of Civil Aviation #44 Phnom Penh International Airport Russian Federation Blvd., Phum Ta Ngoun Sangkat Kakap, Khan Porsenchey Phnom Penh Cambodia Tel: +855 (23) 890 198 Fax: +855 (23) 890 102 E-mail: hengmengkong@gmail.com |
| Canada |  |
| John Taylor Aeronautical Spectrum Inspector | Transport Canada Twr C, Place de Ville 330 Sparks Street Ottawa, Ontario K1A 0N8, Canada Tel.: +613 993-4061 E-mail: john.taylor@tc.gc.ca |
| China |  |
| Zhang Jia | Civil Aviation Administration of China NO.155 Dongsi West Avenue, Dongcheng District, Beijing PRC Tel: +86-10-64092664 Fax: +86-10-64092644 E-mail: zhangjia@caac.gov.cn |
| Zhao Liang | ADCC of Air Traffic Management Bureau 14F Baiyan Building,No.238 Beisihuan Zhong Road, Haidian Dist, Beijing PRC Tel: +86-10-82325050 Fax: +86-10-82325050 E-mail: zhaoliang@adcc.com.cn |
| France |  |
| Christian Fleury | DTI.CNS.FRS 1 Av ou Docteur Maurice Grynfogel.BP 53 584\ 31035 Toulouse France Tel: +33 5 62 14 5561 E-mail: christian.fleury@aviation-civile.gouv.fr |
| Alexandre Guignot | DGCA/France 13, rue de condé 59110 La Madeeleine France Tel: +33 769 457 085 E-mail: alexander.guignot@aviation-civile.gouv.fr |
| Nicolas Van-Wambeke | Thales Alenia Space Toulouse France Tel : +33 (0) 5 34 35 42 94 Mobile : +33 (0) 6 71 36 23 96 E-mail : Nicolas.Van-Wambeke@thalesaleniaspace.com |
| Ivan Martin | 4, Avenue des Louvresses 92622 Gennevilliers France Tel: +33 6753 58171 E-mail: ivan.martin@thalesgroup.com |
| Germany |  |
| Felix Butsch Head of Frequency Management | DFS Deutsche Flugsicherung Dept. SIS/MF Am DFS-Campus 10 D-63225 Langen Tel: +49-6103-707-1533 E-mail: Felix.Butsch@DFS.de |
| Martin Weber | Bundesnetzagentur Im Gewerbepark A 15 D-93059 Regensburg Germany Tel: +49 941 4626 230 Mobile: +49 172 6752 583 E-mail: martin.weber@bnetza.de |
| Indonesia |  |
| Suyanti Aviany Inspector of Air Navigation | Directorate General of Civil Aviation Indonesia Medan Merdeka Barat No. 8 Jakarta Indonesia Mobile: +62 811 974 852 Fax: +62 21 350 6579 E-mail: aviakennia@gmail.com |
| Mr. Bimantoro Inspector of Air Navigation | Directorate General of Civil Aviation Indonesia Medan Merdeka Barat No. 8 Jakarta Indonesia Mobile: +62 85 9450 6399 Fax: +62 (21) 350 6579 E-mail: bimantoro@dephub.go.id, bimzink@gmail.com |
| Japan |  |
| Tsuneaki Watanabe Special Assistant to the Director | CNS Planning Office, Air Traffic Service Department Civil Aviation Bureau Ministry of Land, Infrastructure, Transport and Tourism of Japan Japan Tel.: +81-3-5253-8742 Fax: +81-3-5253-1663 E-mail: watanabe-t43rr@mlit.go.jp |
| Hiroyuki Tsuji Planning Manager | Research Planning and Promotion Office National Institute of Information and Communications Technology (NICT) Incorporated Administrative Agency 4-2-1, Nukui-Kitamachi Koganei, Tokyo 184-8795 Japan Tel: +81 (42) 327 6034 Fax: +81 (42) 327 6128 E-mail: tsuji@nict.go.jp |
| Taichi Takayama | Mitsubishi Research Institute, Inc. 10-3 Nagatacho 2-chome Chiyoda-ku Tokyo, 100-8141 Japan Tel.: +81 3 6705 6039 takayama@mri.co.jp |
| Hiroshi Okochi | JRANSA Kaiji Center Building 4-5 Kojimachi, Chiyoda-ku Tokyo 102 0083 Japan Tel.: +81 3 5214 1353 E-mail: ookouchi@jransa.or.jp |
| Naruto Yonemoto | Electronic Navigation Research Institute (ENRI), National Institute of Maritime, Port and Navigation Technology (MPAT) 7-42-23, Jindaiji-higashi, Chofu, Tokyo, 182-0012 Japan Tel: +81-422-41-3174 E-mail: yonemoto@mpat.go.jp |
| Korea |  |
| Shim Yongsup Researcher | National Radio Research Agency Ministry of Science ICT and Future Planning 767, Bitgaram-ro Naju-si, Jeollanan-do Republic of Korea Tel: +82 (61) 338 4623 Fax: +82 (61) 338 4619 E-mail: wave0908@korea.kr |
| Malaysia |  |
| Sahrol Nizal Ab Rashid Senior Assistant Director | Department of Civil Aviation Malaysia No. 27, Persiaran Perdana, Aras 4 Block Podium B, Presint 4 62618 Putrajaya Malaysia Tel: +603 8871 4278 Fax: +603 8881 0530 E-mail: sahrol@dca.gov.my |
| Syahroni Bin Che Ros Assistant Director | Department of Civil Aviation Malaysia No. 27, Persiaran Perdana, Aras 4 Block Podium B, Presint 4 62618 Putrajaya Malaysia Tel: +603 8871 4313 Fax: +603 8881 0530 E-mail: syahroni@dca.gov.my |
| Netherlands |  |
| Gerlof Osinga | Radio Communication Agency PO Box 450 9700 AL GRONINGEN The Netherlands Tel: +31 6 535 484 95 E-mail: gerlof.osinga@agentschaptelecom.nl |
| Philippines |  |
| Charlemagne P. Gilo | Civil Aviation Authority of the Philippines (CAAP) Air Traffic Service Civil Aviation Authority of the Philippines MIA Road, Pasay City 1300 Philippines Tel: +63 2 879 9282 E-mail: charlemagne.gilo@gmail.com |
| Josue R. Sugui | Civil Aviation Authority of the Philippines Air Navigation Service ANS Technical Center Building Old Mia Road, Pasay City 1300 Philippines Tel: +63 (2) 944 2186 Mobile: +63 939 5454488 E-mail: josuesugui@yahoo.com |
| Romania |  |
| Aurelian Sorinel Calinciuc | National Authority for Management and Regulation in Communications of Romania – ANCOM Bucurest 3, 2 Delea Noua Street 030925 Romania Tel.: +40 721 291 274 E-mail: aurelian.calinciuc@ancom.org.ro |
| Russian Federation |  |
| Dmitriy Muzychenko Chief, Avionics Electromagnetic Compatability Assurance Sector | Russian Federation Tel: +7 903 174 8224 Fax: E-mail: muzychenko@atminst.ru |
| South Africa |  |
| Lisa Tele | Air Traffic& Navitation Services (S. Africa) Block C., East Gate Office Park South Boulevard Road Bruma, Johannesburg South Africa Tel: +2711 607 1134 E-mail: lisat@atns.co.za |
| Takalani Tshikalaha | ATNS Head Office, East Gate Office Park Block C, South Boulevard Road Bruma, Johannesburg, 2198 South Africa Tel: +2711 607 1140 E-mail: takalanit@atns.co.za |
| Thailand |  |
| Paveen Juntama ANS Specialist | The Civil Aviation Authority of Thailand 333/105 Lak Si Plaza Khamphaeng Phet 6 Road Talad Bang Khen, Lak Si Bangkok 10210 Thailand Tel: +66 (2) 568 8800 Ext. 8825 E-mail: paveen.ju@caat.or.th |
| Paisan Praneetpongrang Senior Director Air Traffic Services Engineering Bureau | The Aeronautical Radio of Thailand Ltd. 102 Ngamduplee, Tungmahamek Bangkok 10120 Thailand Tel: +66 (2) 287 8890 Fax: +66 (2) 287 8862 E-mail: paisan.pr@aerothai.co.th |
| Chainan Chaisompong Air Traffic Engineering Manager | The Aeronautical Radio of Thailand Ltd. 102 Ngamduplee, Tungmahamek Bangkok 10120 Thailand Tel: +66 (2) 287 8391 Fax: +66 (2) 287 8681 E-mail: chainan.ch@aerothai.co.th |
| United Kingdom |  |
| John Mettrop Spectrum Engineer | K6 CAA House 45-59 Kingsway WC2B 6TE London, England Tel: +44 2074536531 E-mail: john.mettrop.caa.co.uk |
| Kamlesh Masrani | Inmarsat 99 City Road London, EC1Y 1AX United Kingdom Tel : +44 20 7728 1338 E-mail: kamlesh.masrani@inmarsat.com |
| United States |  |
| Michael Biggs | Federal Aviation Administration (FAA) AJW-1C3, Room 7E-513 800 Independence Avenue SW Washington, DC 20591 United States Tel: +202 267-8241 E-mail: michael.biggs@faa.gov |
| Robert Kerczewski Project Manager | NASA Glenn Research Center 21000 Brookpark Road, MS 54-1 Cleveland, OH 44135 United States Tel: +216 433-3434 E-mail: rkerczewski@nasa.gov |
| Joseph Cramer | The Boeing Company 929 Long Bridge Drive Arlington, VA 22202, USA Tel: +1 703 465 3486 E-mail: joseph.cramer@boeing.com |
| Robert W. Denny | US Department of Commerce/NTIA 1401 Constitution Ave, NW Washington, DC 20230 United States Tel: +1 202 482 3803 E-mail: rdenny@ntia.doc.gov |
| Scott Kotler | Lockheed Martin Corporation 2121 Crystal Drive Suite 100 Arlington, VA 22202 United States Tel : +703 413 3102 E-mail: scott.kotler@lmco.com |
| Damon Ladson | Harris, Wiltshire & Grannis, LLP 1919 M St. NW, 8th Floor Washington, DC 20036 United States Tel: +1 202 730 1315 E-mail: Dladson@hwglaw.com |
| Louis Bell | Federal Communications Commission United States Tel: +1 202-418-1641 E-mail: Louis.Bell@fcc.gov |
| David Redman Director, Aerospace Vehicle Systems Institute | Aerospace Vehicle Systems Institute (AVSI) 754 HRBB/3141 TAMU College Station, Texas 77843-3141 United States Tel: +1 (979) 862 2316 Mobile: +1 (979) 218 2272 Fax: +1 (979) 393 4075 E-mail: dredman@avsi.aero |
| Michael Franceschini Engineering Fellow | Honeywell International 29 Harbor Circle Centerport, New York 11721 United States Tel: +1 (631) 745 8996 Mobile: +1 631 697 0970 E-mail : michael.franceschini@honeywell.com |
| Michael Neale (remote participant) | E-mail : michael.neale@aces-inc.com |
| Aviation Spectrum Resources, Inc. (ASRI) |  |
| Andrew Roy Director, Engineering Services | Aviation Spectrum Resources, Inc. 180 Admiral Cochrane Drive Suite 300 Annapolis, MD 21401 United States Tel: +1 443 951 0340 E-mail: ACR@asri.aero |
| Kris Hutchison | Aviation Spectrum Resources, Inc. 180 Admiral Cochrane Drive Suite 300 Annapolis, MD 21401 United States Tel: +1 443 951 0322 E-mail: KEH@asri.aero |
| Greg Baker | Aviation Spectrum Resources, Inc. 180 Admiral Cochrane Drive Suite 300 Annapolis, MD 21401 United States Tel: +1 443 951 0335 E-mail: GDB@asri.aero |
| EUROCONTROL |  |
| Raffi Khatcherian ATM senior expert Spectrum Management | EUROCONTROL Headquarters Rue de la Fusée, 96 1130 Brussels Belgium Tel: +32 2 7293687 Mobile: +32 478241421 E-mail: Raffi.khatcherian@eurocontrol.int |
| IATA |  |
| Noppadol Pringvanich | IATA 800 Place Victoria P.O. Box 113 Montreal, QC Canada Tel: +1 514 653 6542 E-mail: pringvanin@iata.org |
| Tim Totten | UPS 825 Lotus Ave Louisville KY, 40213 United States Tel: +1.502.359.8772 Mob: +1.502.419.5694 E-mail: ttotten@ups.com |
| ICCAIA |  |
| Claude Pichavant Member of ACP Senior Expert Communications & Surveillance Communications-Surveillance Department | Airbus 316 Route de Bayonne 31060 Toulouse Cédex, France Tel: +33 (0)5 61 93 57 88 E-mail: claude.pichavant@airbus.com |
| Luiz Fernando de Souza | Flight Test Instrumentation EMBRAER- Brazil Avenida Bhigadeino Faria Lima 2170 Sao Jose de Campos-SP Brazil Phone +55 16 98167 2275 lfsouza@embraer.com.br |
| ITU |  |
| Zhou Xingguo | International Telecommunication Union Place des Nations CH-1211 Geneva, Switzerland Tel: +41 (22( 730 5068 E-mail: xingguo.zhou@itu.int |
| Nelson Malaguti | International Telecommunication Union Place des Nations CH-1211 Geneva, Switzerland Tel: +41 022 73051 98 E-mail: nelson.malaguti@itu.int |
| ICAO |  |
| Loftur Jonasson FSMP Secretary | International Civil Aviation Organization (ICAO) Headquarters 999 Robert-Bourassa Blvd Montreal, Quebec, Canada H3C 5H7 Tel: + 1514 954 8219, Ext. 7130 E-mail: ljonasson@icao.int |
| Robert Witzen Consultant | International Civil Aviation Organization (ICAO) Headquarters 999 Robert-Bourassa Blvd Montreal, Quebec, Canada H3C 5H7 Tel: +1 (515) 708 2296 Mobile: +1 4502262048 E-mail: r.witzen@videotron.ca |
| Steve Mitchell Consultant | E-mail: steve.mitchell_spectrum@yahoo.com |
| Li Peng Regional Officer CNS | International Civil Aviation Organization Asia and Pacific Office 252/1, Vibhavadee Road Ladyao, Chatuchak Bangkok 10900 Thailand Tel: +66 (2) 537 8189 Ext.158 Fax: +66 (2) 537 8199 E-mail: PLi@icao.int |
| Frederic Lecat Regional Officer CNS | International Civil Aviation Organization Asia and Pacific Office 252/1, Vibhavadee Road Ladyao, Chatuchak Bangkok 10900 Thailand Tel: +66 (2) 537 8189 Ext.155 Fax: +66 (2) 537 8199 E-mail: FLecat@icao.int |

APPENDIX D

ACTION ITEM LIST

| Number | Description | Actionee | Due Date | Status |
| --- | --- | --- | --- | --- |
| 32-3 | Review proposed VHF adjacent channel planning criteria contained in WG-F/32 WP11/IP02 in order to allow for a decision on whether to update the Spectrum Handbook. | All | FSMP-WG/4 | CLOSED |
| 32-5 | Consider the issue of interference to radio altimeters as presented in WG-F/32 WP21. Provide input on issues such as: should ICAO develop altimeter standards? What is the best way for ICAO to raise the visibility of the safety issue? Do high intensity radiated field (HIRF) requirements apply to radio systems, and if so, how? | All | FSMP-WG/5 | Still open |
| 32-6 | Poll internal data bases and compile list of reported interference to radio altimeter systems. Include geographic locations of instances. Also present a paper to the RASG-MID/4 meeting on the issue. | IATA | FSMP-WG/3 | CLOSED [23 reports 2014/2015; some altimeters more susceptible than others.] |
| 32-7 | Provide input to J. Mettrop to support the examination of cost savings from removing a generic avionics box (ref. WG-F/32 IP08) | All | FSMP-WG/3 | CLOSED (see AI 04-05) |
| 32-8 | Review the proposal to develop a definition of “aviation safety system” and provide draft inputs as appropriate | All | FSMP-WG/5 | Still open. Input provided in WG-FSMP/1 Flimsy 1 for Comm systems |
| 01-1 | Update Doc 9718, Volume I, Section 9.5 “Special Cases” to include material on VSAT vs IMT. | B. Sougue L. Tele | FSMP-WG/3 | CLOSED [FSMP-WG4 WP25] |
| 01-3 | Provide examples of unintentional radiators causing interference to aviation wireless systems (reference WF-F/32 WP02 and IP14) | All | FSMP-WG/5 | [Action 04-01] |
| 01-5 | Provide notional characteristics for the line-of-sight (LOS) and beyond-LOS (BLOS) RPAS systems to be used to begin investigating adjacent channel compatibility with AeroMACS in 5091-5150 MHz. | A. Munro | FSMP-WG/5 | [material in WG3-WP10/Appx, but study has not been done] |
| 02-1 | Compile listing of any civil aviation use of 5 150-5 250 MHz, 5 350-5 470 MHz and/or 5 850-6 425 MHz frequency bands | All | FSMP-WG/5 | CLOSED |
| 02-2 | Report on results of radio altimeter interference reporting effort | IATA | FSMP-WG/3 | CLOSED |
| 02-4 | Research ICAO efforts going on with respect to space planes (“legal committee”) | Secretary | FSMP-WG/3 | Closed [nothing happening] |
| 02-6 | Develop a timeline for FSMP actions regarding resolves 18 of Resolution 155 (WRC-15) | All | FSMP-WG/5 |  |
| 02-11 | Develop a simple example outlining the approach for aviation system protection suggested in WP24. | J. Mettrop | FSMP-WG/5 |  |
| 03-01 | Review ITU Radio Regulations Articles related to aviation and determine if revisions are needed. Reflect changes in next update to ICAO Position, and in Doc 9718. | All | FSMP-WG/4 | [see paper on GADSS] CLOSED (see AI 04-02] |
| 03-02 | Provide submission to WP5B supporting elevation of the preliminary draft revision of Recommendation ITU-R M.1466 to draft revision. | Secretary | Oct. 2016 | Closed |
| 03-03 | Provide comment on the spectrum sharing approach between terrestrial and satellite RPAS C2 systems for the 5 030-5 091 MHz as proposed in FSMP-WG/3 WP10 and FSMP-WG/4 WP17 | All | FSMP-WG/5 |  |
| 03-04 | Provide comments/revisions to Spectrum Handbook Volume I for review at FSMP-WG/4. [Goal is to have stable final draft out of FSMP-WG/4] | All | Feb 1, 2017 | Closed [papers to mtg] |
| 04-01 | Provide the FSMP Rapporteur with information via Email on (a) details of instances of disturbances and degradation within their experience caused by electrical/electronic equipment and telecommunication distribution systems operating over metallic conductors, and (b) the noise floor they anticipate in order to satisfy their planned operational requirements, especially where there are precise expectations on the noise floor. | All | May 7, 2017 |  |
| 04-02 | Review the material contained in FSMP-WG4 WP27 and provide input on possible changes to Chapters VI-VIII of the Radio Regulations resulting from the introduction of GADSS. | All | FSMP-WG5 |  |
| 04-03 | Develop material on aviation use of VSAT for Chapter 7 of doc 9718, Volume I, for the next update. | Lisa Tele | FSMP-WG8 |  |
| 04-04 | Develop proposed approach to insert the material in FSMP-WG4/WP14 on HF characteristics into Doc 9718 Volume II | Greg Baker | FMSP-WG5 |  |
| 04-05 | Provide input to complete the equipment physical characteristics (e.g., weight) table shown in the Annex of FSMP-WG4/WP26. | All | FSMP-WG5 |  |
| 04-06 | With regard to the action to “Conduct an aircraft fleet equipage impact analysis and develop detailed transition plans based on industry input and expected safety benefit” in the radio altimeter job card, initiate outreach to the airlines and aircraft manufacturers. | IATA and ICCAIA | FSMP-WG6 |  |

APPENDIX E

Modifications to Doc 9718, Volume I

Document FSMP-WG04-Flimsy05_doc 9718 - combined v2 rev7.doc embedded below:

APPENDIX F

Elements for ICAO response to ITU WPP5B regarding status of UAS SARPS development

Document R15-WP5B-C-FSMP-RPAS_SARPS (1).doc embedded below:

APPENDIX G

Elements for ICAO response to ITU WPP5B regarding proposed UAS SARPS text

Document R15-WP5B-C-FSMP FSS_UAS.doc embedded below:

APPENDIX H

Elements for proposed ICAO Liaison to WP5B regarding GADSS

[This material should be liaised to ITU-R WP5B for “information and action as appropriate”, and sent also to WP4C “for information”.]

The International Civil Aviation Organization (ICAO) would like to thank ITU-R Working Party (WP) 5B for the information and requests titled “QUESTIONS ABOUT THE SYSTEMS TO BE USED, AND THE SPECTRUM REQUIREMENTS, OF GADSS” (see 5D/417).

ICAO notes that in order for WP 5B to start the studies on WRC-19 agenda item 1.10, WP 5B requests from ICAO the following generic information about GADSS as identified below:

Does ICAO plan to use new systems for implementation of the Concept of Operations (ConOps) for the GADSS?  If so:

1)	Will these systems operate in the frequency bands having been already allocated to the aeronautical services, or are new frequency bands required for their operation?

2)	What are the assumed operational characteristics and performance requirements of such systems that will be used for implementation of the ConOps for the GADSS?

In response to WP 5B, ICAO would like to provide information based on the latest version of the ConOps. It should be noted that development of that ConOps is still in progress.

I.          Four functions have been identified in the ConOps which could result in a requirement for additional radio spectrum:

- Normal tracking is identified as the means of flight following an aircraft throughout its entire flight within the context of availability of position reporting at least once in 15 minutes. This function is deemed to be under the purview of the airline or company operating the aircraft. More than one method is available for an airline or company to accomplish normal tracking of a flight.

- Surveillance is a specific air traffic control function that is accomplished in accordance with ICAO SARP’s relative to airspace separations standards, technical standards and also the onboard avionics certification. The surveillance function may be used to provide normal tracking.

- Distress tracking in the ConOps context is during conditions of emergency operations of an aircraft.  It is not technology-specific and will allow for various solutions, including a triggered transmission system, but needs to be capable of transmitting location information in the event of aircraft electrical power loss and making the information available to the relevant authorities such as search and rescue (SAR) Regional Coordination Centres and air traffic services.  Principal methods being considered within ICAO are the use of emergency position-indicating radiobeacons (EPIRBs) and emergency locator transmitter (distress tracking) (ELT(DT)), both of which operate on 406.1 MHz.  Two questions remain: one is how to implement the most appropriate triggering mechanism of the system(s) while in flight, and two, if COSPAS-SARSAT is able to undertake the function of distress tracking.

- Data retrieval in respect to recovery of data contained in the flight recorders is a function of the ConOps that is still under discussion within the responsible operational and technical Panels of ICAO.

II.           In order to ensure global interoperability and lawful operation of radio equipment on-board aircraft, the equipment needs to conform to agreed performance standards, operate in correct frequency bands, be licensed by appropriate authorities, and be operated by licensed personnel if appropriate.

The below table shows which types of frequency bands could be considered for the various categories of functions specified under the GADSS

| Function | Spectrum Category |
| --- | --- |
| Normal Tracking | A |
| Surveillance | B |
| Distress Tracking | C |
| Data Retrieval (not used for real-time functions) | A |

A: any type of spectrum properly allocated, on a primary basis, for the function being performed

B: only protected aeronautical safety spectrum can be used.

C: only protected aeronautical safety spectrum, or protected distress spectrum (e.g., 406.1 MHz), can be used

*This chart is not intended to imply that any new spectrum allocations are necessary to support GADSS.

III.           ICAO is pleased to advise WP 5B that at this moment in time it is foreseen no new spectrum will be required to address normal tracking, surveillance and distress tracking. Until there is maturity regarding an agreed method to accomplish data retrieval however, it remains premature to determine any potential for additional new spectrum requirements for that function. ICAO would look forward to providing further information to WP 5B at its next meeting in November 2017 regarding whether any additional spectrum may be required for that function.

Finally, it should be noted that in addition to consideration of Article 5 of the Radio Regulation (RR), aviation has been investigating whether there may be changes required to other RR Articles due to the introduction of GADSS. In particular possible changes to portions of RR Chapters VI, VII and VIII (i.e., portions of Articles 21-45) have been identified and are currently being considered within ICAO by the relevant expert groups.