FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

Twelfth Working Group meeting

Web Meeting, 4 – 15 October 2021

Agenda Item 5:	Development of (planned) Material for ITU-R Studies on:

b)	WRC-23 AI 1.7 VHF AMS(R)S

Reply Liaison Statement from ITU-R WP5B to ICAO

Elements for studies on WRC-23 agenda item 1.7

(Presented by the Secretary)

SUMMARY

Attached is a reply liaison statement from ITU-R WP5B to ICAO, discussing and requesting information from ICAO on the elements being studied under WRC-23 agenda item 1.7 on space-based aeronautical VHF communications system within the 117.975-137 MHz frequency band. Action: FSMP WG/12 is invited to review the attached liaison statement and take action as appropriate.

______________

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 5B/TEMP/133 Subject: WRC-23 agenda item 1.7 Resolution 428 (WRC-19) |  |
|  | 8 June 2021 |
|  | English only |
| ITU-R Working Party 5B |  |
| REPLY Liaison Statement to ICAO |  |
| Elements for studies on WRC-23 agenda item 1.7 |  |

Working Party (WP) 5B would like to thank ICAO for its liaison statement contained in Document 5B/268 related to studies under WRC-23 agenda item 1.7 on space-based aeronautical VHF communications system within the 117.975-137 MHz frequency band. WP 5B reviewed the material provided by ICAO and would like to provide the following information and comments.

Status of studies on agenda item 1.7

During its May 2021 meeting, WP 5B has progressed on main assumptions regarding the architecture and parameters of a possible AMS(R)S system within 117.975-137 MHz, intended to relay VHF aeronautical mobile communications and to complement terrestrial infrastructures over oceanic and remote areas, without modification to aircraft equipment. Taking into account the different elements provided by ICAO regarding antenna pattern for aircraft VHF equipment, performance requirement for this equipment, and overall availability considerations, WP 5B has determined that an AMS(R)S system could most-likely rely on non-geostationary satellites and has defined reference link budgets for satellite-to-aircraft (downlink) and aircraft-to-satellite (uplink) VHF links. Related elements have been contained in an updated version of corresponding working document being developed by WP 5B (see attachment), which also includes first considerations on compatibility studies with existing primary services in-band and in adjacent bands, for review and comments.

Compatibility between AMS(R)S and AM(R)S in-band, and between AMS(R)S and ARNS in adjacent-band

In particular, WP 5B has reflected the view provided by ICAO regarding compatibility study between AMS(R)S within the frequency band 117.975-137 MHz and, on one side AM(R)S in-band, on the other side ARNS in the adjacent frequency band 108-117.975 MHz. WP 5B understands that ICAO frequency planning and coordination will ensure these compatibilities and that comprehensive sharing studies are not necessary, provided that typical compatibility information, such as being developed through WP 5B working document, is available.

In relation to this and to the existing organisation of the band 117.975-137 MHz by ICAO between different aeronautical application, WP 5B would like to seek ICAO’s view, if any, on the extent of a possible AMS(R)S allocation in this frequency band. Provided that compatibility with primary services in adjacent band is ensured, AMS(R)S may have an allocation in the entire band 117.975-137 MHz, which would give maximum flexibility to ICAO in future planning exercises. On the other hand, there may be certain AM(R)S applications not compatible with AMS(R)S and already firmly planned in certain sub-bands, in which case AMS(R)S would not seem necessary in these sub-bands.

Power considered for aircraft VHF transmitter

For the aircraft-to-satellite link budget, WP 5B has taken the assumption of an aircraft transmit output powers of 16 watts, taken from past WP 5B studies related to aeronautical VHF aeronautical mobile communications. WP 5B would like to request ICAO to confirm this particular assumption.

Satellite Doppler shift and latency time

In addition, WP 5B has noted that the non-geostationary orbit considered for AMS(R)S will cause a Doppler shift, which depends on satellite altitude. At the altitude of 600 km, it would be of ±4 kHz maximum at VHF range. This altitude will also correspond to a latency time due to propagation comprised between 4 ms (at zenith) and 18.9 ms (at horizon).

WP 5B would like to draw ICAO’s attention on these elements and would like to know if any information is available regarding their possible impact on VHF aeronautical mobile systems.

Working Party 5B looks forward to continued fruitful cooperation with ICAO regarding this work.

| Status: For action |  |
| --- | --- |
| Deadline: Next WP 5B meeting (November 2021) |  |
| Contact: Loftur Jonasson (ICAO) | E-mail: LJonasson@icao.int |

Attachment:	Working document towards a preliminary draft new Report ITU-R M.[Space-VHF] (Annex 29 to WP 5B Chairman’s Report, Doc. 5B/355)

______________