FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

Fourteenth Working Group meeting

Web Meeting, 25 – 29 April 2022

Agenda Item 6:		Any other Business

b)	MSS Guidelines 1525-1559 MHz

IMT-MSS L-band draft ICAO guidelines

(Presented by Kamlesh Masrani)

SUMMARY

This paper offers edits to the text of the draft Guidelines/Information Note attached in Appendix H of the Report from FSMP-WG/12. Consideration should be given by FSMP on how the draft Guidelines/Note, if finalised and agreed, may be circulated by ICAO (e.g., through an ICAO State Letter or via another means). To progress the work in finalising the draft ICAO Guidelines/Note, it is proposed to initiate a correspondence group to support discussions between FSMP meetings.

INTRODUCTION

FSMP has been developing a possible draft Guidance/Information Note on the protection of aircraft earth stations supporting aeronautical safety services in the frequency band 1 525-1 544 / 1 545-1 559 MHz, from the planned deployment of IMT base stations and user equipment around airports operating in the frequency bands below 1 518 MHz.

The latest draft of this note is contained in Appendix H of the FSMP-WG/12 Meeting Report and is attached to this input document for convenience.  FSMP-WG/12 had agreed for the note to be carried forward for future reference and members were encouraged to provide contributions to future FSMP meetings.

This paper proposes to progress the work on the draft guidelines with a view to finalizing them, if possible, at FSMP WG/15.

DISCUSSION

To progress the development of a possible Guidance/Information Note by ICAO, it is proposed that FSMP WG/14 assigns time during the meeting to discuss what further information and edits would be necessary to finalise the note, if possible, at FSMP WG/15.

To assist the discussions, Inmarsat offers the following comments on the existing draft text:

- we are in general agreement with the current text of the draft Guidance/Information Note, and observe the inclusion of the Editor’s Note and other square-bracketed text that would need to be resolved.

- we note the lack of consensus and slow progress within the ITU-R in finalising the Draft New Recommendation ITU-R M.[REC.MSS & IMT L-band COMPATIBILITY].  We view this delay, in part, is a result of the submission of inflexible and non-constructive proposals by some parties seeking to benefit in not having an ITU-R Recommendation agreed on this matter in a timely manner.  Given such a delay, Inmarsat encourages ICAO to finalise any protection measures and timelines that it considers as necessary to ensure the continued protection and operation of existing aeronautical L-band satcom receivers in the frequency band 1 525-1 544 / 1 545-1 559 MHz, from the planned deployment of IMT systems around airports in the frequency bands below 1 518 MHz.

- we propose the introduction of text in the draft note (see attached proposal) that avoids the possibility of IMT/mobile user terminal transmissions operating up to and inside the aircraft.  It is recognised any such geographical restrictions adopted by an Administration need only apply in areas around airports and not generally throughout the whole country, to minimise the impact of the requested protection measures on IMT.

- to propose the introduction of text in the draft note (see attached proposals) that protects the sensitivity of aeronautical satellite receivers by ensuring there are maximum limits on the unwanted emission levels that fall into the 1 525-1 559 MHz band from IMT transmissions.

- we observe that several countries have completed, or are in the process of completing, spectrum auctions for IMT in the frequency band 1 492-1 518 MHz.  For example, auctions have been completed in Switzerland, Slovenia, Austria, Denmark, and Latvia.  An auction in Belgium is currently underway.  Brazil is finalising the national regulations to make this band available for private mobile networks.  Many other countries around the world have general plans to make available the 1 500 MHz band for mobile systems in the coming years.  In some cases, the national spectrum regulator is consulting with the aviation community (including the likes of Inmarsat) to establish the technical conditions for compatibility, but in other cases they are not.  We have found in some cases where the spectrum regulator has engaged with the airport operators or the national aviation regulator, but those bodies do not have sufficient information available on the interference issue to effectively discuss the protection requirements with the spectrum regulator.

- the auctions completed so far have adopted differing protection conditions of aeronautical L-band MSS earth stations operating above 1 525 MHz.  Where PFD limits are being applied to protect the operation of such satcom terminals, the timelines in the transition between ‘Phase 1’ PFD limits to ‘Phase 2’ PFD limits are far shorter (i.e., 5-7 years) than those identified within the draft Guidance/Information Note.  We have also found that some airports where aeronautical L-band MSS service usage have been omitted from inclusion in the national protection measures.

Given the general lack of clear and agreed guidelines for spectrum regulators, the operation of existing aeronautical L-band MSS earth-station receivers operating around airports in the frequency band 1 525-1 559 MHz is put at the risk of harmful interference from IMT.  This will likely lead to disruption in the use of L-band satcom aviation services in future, as IMT and terrestrial mobile systems become more widely deployed around airports in the 1 492-1 518 MHz frequency band.

ACTION BY THE MEETING

The meeting is invited to:

note and review the contents of this working paper, together with the proposed edits to the draft Guidance / Information Note on the protection of aeronautical MES receivers from the introduction of IMT/LTE in the frequency band below 1518 MHz;

Agree to the formation of a formal correspondence group following FSMP WG/14 to progress on the work on the draft note, and with a view to finalising it at FSMP WG/15m if possible;

To discuss the manner in which the draft guidelines may be circulated by ICAO (e.g., whether this could be through an ICAO State Letter).

- Attached (2 documents):

- — END —