	[image: image1.png]



	[image: image1.png]
International Civil Aviation Organization

WORKING PAPER
	FSMP-WG/2 WP/06
2016-02-08 




ATMRPP-WG/WHL/4-WP/


.././06
FSMP-WG/2 WP/06
- 2 -

- 3 -
FSMP/1-WP/xxx

FREQUENCY SPECTRUM MANGEMENT  PANEL (FSMP)
SECOND MEETING OF THE WORKING GROUP
Montreal, Canada, 15 to 19 February 2016
Agenda Item 3:
Development of ICAO Position for WRC-19
DRAFT ICAO POSITION ON WRC-19 AGENDA ITEM 1.16
(Presented by John Taylor)

	SUMMARY

	Agenda item 1.16 is to consider issues related to wireless access systems, including radio local area networks (WAS/RLAN), in the frequency bands between 5 150 MHz and 5 925 MHz, and take the appropriate regulatory actions, including additional spectrum allocations to the mobile service, in accordance with Resolution 239 [COM6/22] 



1. INTRODUCTION

WRC-15 agreed an agenda item to address the considerable growth in the demand for Wireless Access Systems including radio local area networks (WAS/RLAN) applications with multimedia capabilities, in the frequency bands between 5 150 and 5 925 MHz.
2. DISCUSSION

In the lead up to WRC-15, the results of ITU-R studies indicate that the minimum spectrum need for WAS/RLAN in the 5 GHz frequency range in the year 2018 is estimated at 880 MHz; this figure includes 455-580 MHz already utilized by non-IMT mobile broadband applications operating within the 5 GHz range resulting in 300-425 MHz additional spectrum being required. Further, compatibility studies performed by ITU-R in preparation for this conference indicate that when assuming the use of WAS/RLAN mitigation measures limited to the regulatory provisions of Resolution 229 (Rev.WRC-12), sharing betweenWAS/RLAN and the EESS (active) systems in the frequency bands 5 350 to 5 470 MHz would not be feasible, as well as being insufficient to ensure protection of certain radar types in this frequency band; for these cases, 

sharing may only be feasible if additional WAS/RLAN mitigation measures are implemented, however, no agreement was reached on the applicability of any additional WAS/RLAN mitigation techniques.

From an aviation perspective, the band the frequency band 5 150-5250 MHz is also allocated worldwide on a primary basis to the aeronautical radionavigation service and to the fixed-satellite service (No. 5.447A) also that the band the frequency band 5 350 -5 460 MHz is allocated worldwide to the aeronautical radionavigation service on a primary basis, with 5 460 – 5 470 MHz allocated worldwide to the radiohavigation service on a primary basis.
The intention of the studies is to attempt to show compatibility with incumbent services with outdoor WAS/RLAN systems, using appropriate mitigation measures.
ICAO Position:
To ensure, on the basis of agreed studies and appropriate mitigation measures that any use of WAS/RLAN outdoor systems does not cause harmful interference to aeronautical services operating below 5 150 MHz or to the ARNS operating in 5 350 – 5 470 MHz.
3. ACTION BY THE MEETING

3.1 The meeting is invited to:

a) note and review the contents of this working paper;

b) endorse the proposed draft Position above; 
c) agree to continue work on this proposal, with a view to finalization during future meetings of the WG/WHL. 

— END —

(2 pages)

FSMP-WG02-WP06_Draft Position on 1.16.doc

