FREQUENCY SPECTRUM MANGEMENT PANEL (FSMP)

Eighteenth Working Group Meeting

Montreal, Canada, 6 – 16 February 2024

Agenda Item 8):	ICAO Frequency Spectrum Handbook (Doc 9718)

Restructure and update of Volume I

Handbook update related to foreign object debris (FOD) detection system.

(Presented by Katsuyuki Arakawa)

SUMMARY

This document proposes the draft modifications of the ICAO Frequency Spectrum Handbook related to foreign object debris (FOD) detection system operating in the frequency range 92-100 GHz.

INTRODUCTION

Report ITU-R M.2501-0 was approved by the ITU-R study group 5 on 16 December 2021 which provides the technical and operational characteristics of foreign object debris (FOD) detection system operating in the frequency range 92-100 GHz for sharing and compatibility studies between FOD detection system and passive services, as well as among active services.

Recommendation ITU-R M.2162-0, Technical and operational characteristics of radiolocation systems operating in the frequency range 92-100 GHz and radionavigation systems operating in the frequency range 95-100 GHz, will be published.

DISCUSSION

Foreign object debris (FOD) detection system operating in the frequency range 92-100 GHz can provide such performance as high detection sensitivity, short detection response time, sufficient coverage of surveillance runway area and high location accuracy for safety airport operation. Table 1 summarizes the technical and operational characteristics of FOD detection system operating in the frequency range 92-100 GHz.

- TABLE 1: Technical and operational characteristics of foreign object debris detection system operating in the frequency range 92-100 GHz.

| Parameters | Values |
| --- | --- |
| Frequency range (GHz) | 92 … 100 |
| Channel bandwidth (GHz) | 0.58 … 7.98 |
| Channel plan | See Fig. 1 |
| Transmit peak power (mW) | 100-200 |
| Sweep frequency (FMCW) (kHz) | 1.250 |
| Antenna type | Cassegrain |
| Antenna gain (dBi) | 44 |
| Antenna pattern | Rec. ITU-R F.699 |
| Antenna height (m) | 4 … 8 |
| Full width at half maximum antenna gain (3 dB beamwidth) (degrees) | Elevation: 1.0, Azimuth: 1.0 |
| Antenna rotation speed (rpm) | 15 |
| Detection distance (m) | 200 … 500 |
| Radiated rotation angle in azimuth (degree) | ±60 |
| Radar cross section specification (dB/m2) | −20 |
| Range resolution (cm) | 3 … 50 |
| Emission bandwidth (−3 dB) (MHz) | 1 |
| Emission bandwidth (−20 dB) (MHz) | 3.5 |
| Adjacent channel leakage ration (dBc) | < −70 |
| Receiver noise figure (dB) | 10 |
| I/N protection criteria (dB) | −6 |

- FIGURE 1: Example of channel plan for foreign object debris detection system operating in the frequency range 92-100 GHz.

- 1 This channel plan will be used if mitigation techniques are available to resolve the coexistence issues with EESS (active).

Considering the approval by the Radiocommunication Assembly 2023, the ICAO Frequency Spectrum Handbook should be updated to include a description of foreign object debris (FOD) detection system.

This document proposes the draft modifications of the ICAO Frequency Spectrum Handbook in the attachment to this document.

ACTION BY THE MEETING

The meeting is invited to:

note and review the contents of this working paper.

Disscuss to the further development of the modifications of the Frequency Spectrum Handbook provided in the attachment to this document.

integrate this modifications into the other proposals for update of the ICAO frequency spectrum handbook.

- — END —

Attachment

proposal for the ICAO Frequency Spectrum Handbook related to FOD detection system.

Chapter 7

STATEMENT OF FREQUENCY ALLOCATIONS,

TECHNICAL DETAILS AND ICAO POLICY

…

SECTION 7-I.    LIST OF FREQUENCY BANDS

| Band | Service | Aviation use | Section 7-II page no. |
| --- | --- | --- | --- |
| 31.8–33.4 GHz 92-95 GHz 95–100 GHz | RNS RLS RNS RLS | ASDE/airborne radar enhanced flight vision system (EFVS) Foreign Object Debris (FOD) Detection airborne radar enhanced flight vision system (EFVS) Foreign Object Debris (FOD) Detection | 7-165 7- 171 7-173 |
| AM(R)S: Aeronautical mobile (route) service AMS(R)S: Aeronautical mobile-satellite (route) service ARNS: Aeronautical radionavigation service MSS: Mobile-satellite service RLS: Radiolocation service RNS: Radionavigation service RNSS: Radionavigation-satellite service |  |  |  |

…

SECTION 7-II.    CIVIL AVIATION FREQUENCY

ALLOCATIONS — ICAO POLICY STATEMENTS AND

RELATED INFORMATION

…

Band: 92–95 GHz

Service: Radiolocation (FOD)

Allocation:

| GHz 92–95 |  |  |
| --- | --- | --- |
| Allocation to Services |  |  |
| Region 1 | Region 2 | Region 3 |
| 92-94 | FIXED 5.338A MOBILE RADIO ASTRONOMY RADIOLOCATION 5.149 |  |
| 94-94.1 | EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) Radio astronomy 5.562 5.562A |  |
| 94.1-95 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION 5.149 |  |

…

Band: 95–100 GHz

Service: Radionavigation (EFVS), Radiolocation (FOD)

Allocation:

| GHz 95–100 |  |  |
| --- | --- | --- |
| Allocation to Services |  |  |
| Region 1 | Region 2 | Region 3 |
| 95-100 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION RADIONAVIGATION RADIONAVIGATION-SATELLITE 5.149 5.554 |  |

…

Attachment G

TECHNICAL INFORMATION

AND FREQUENCY-SHARING CRITERIA

…

Band: 92-95 GHz

Technical Information:

Service: Radiolocation

Aviation use: Foreign object debris detection system (FOD)

Annex 10:

SARPs:

Frequency plan:

Channelization:

Planning criteria:

RTCA:

Eurocae:

ARINC characteristic:

ITU Res./Rec.: Recommendation ITU-R M.2162-0 Technical and operational characteristics of radiolocation systems operating in the frequency range 92-100 GHz and radionavigation systems operating in the frequency range 95-100 GHz

ITU-R:  ITU Report M.2501-0, Technical and operational characteristics of the foreign object debris detection system operating in the frequency range 92-100 GHz

Other material:

Band: 95-100 GHz

Technical Information:

Service: Radiolocation, Radionavigation

Aviation use: Airport surface detection equipment (ASDE), Enhanced flight vision systems (EFVS), foreign object debris detection system (FOD)

Annex 10:

SARPs:

Frequency plan:

Channelization:

Planning criteria:

RTCA:

Eurocae:

ARINC characteristic:

ITU Res./Rec.: Recommendation ITU-R M.2162-0 Technical and operational characteristics of radiolocation systems operating in the frequency range 92-100 GHz and radionavigation systems operating in the frequency range 95-100 GHz

ITU-R:  ITU Report M.2501-0 Technical and operational characteristics of the foreign object debris detection system operating in the frequency range 92-100 GHz

Other material: