FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

Eighteenth Working Group Meeting

Montreal, Canada, 06 to 16 February 2024

Agenda Item 8	ICAO Frequency Spectrum Handbook (Doc 9718)

b)	Update of Volume II

Proposed changes to DOC 9718, Volume II on GBAS frequency assignment planning

(Presented by Christophe MACABIAU, on behalf of NSP)

SUMMARY

This paper presents modifications to the Handbook on Radio Frequency Spectrum Requirements for Civil Aviation, Volume II, Chapter 6 (Doc 9718) on GBAS/VDB frequency assignment planning. These modifications include: • Clarifications and corrections of the DOC for the GBAS/VDB • Optional use of the Airborne Correction Factor in frequency assignment planning. The Appendix to the paper was reviewed and approved by the 11th meeting of the Navigation Systems Panel (NSP) Joint Working Groups (JWGs/11) for submission to FSMP for review and inclusion in the next edition of Doc 9718, Volume II. Action by the FSMP meeting is in paragraph 3.

- INTRODUCTION

  - At the NSP 10th Joint Working Groups (JWGs/10) meeting (22-26 and 31 May 2023) the Conventional Navigation and Testing Working Group (CNTWG) reviewed JWGs/10 – WP/08 which presented material related to the frequency assignment planning of GBAS/VDB in relation to using the Airborne Contribution Factor (ACF) when assessing compatibility between a desired GBAS/VDB facility and an undesired (potential interfering) ILS-Localizer, VOR or GBAS/VDB facility.

    - This paper discusses various mitigation factors that should be taken into account for GBAS/VDB frequency assignment planning, in particular when the GBAS/VDB is used to support approach and landing operations of aircraft. These mitigation factors would allow for GBAS/VDB frequency assignments to be made without the necessity for using the ACF in assessing compatibility between the GBAS/VDB and other facilities operating in the same frequency band (Localizer and VOR. These mitigation factors include:

- The aircraft orientation is directed towards the airport/runway where the GBAS/VDB is operating

- The actual field strength of the GBAS/VDB signal level is higher than the minimum as required by Annex 10

- Variations in the propagation conditions result in less transmission loss for the desired signal (set at 95% of the time) and larger attenuation of the undesired signal (set at 5% of the time)

- In some cases, the predicted interference in peripheral areas of the DOC may be operationally insignificant

1.1.2		In addition, it was noted that currently GBAS/VDB facilities are satisfactorily operating while the ACF has not been considered (APAC and EUR Regions as well as in the United States)

1.1.3		The CNTWG, when considering the various mitigation factors that could be taken into account when coordinating frequency assignments for the GBAS/VDB, expressed no concerns when the frequency assignment planning for GBAS/VDB is performed without using the ACF but noting that, in particular cases, States may require protection of GBAS/VDB facilities that take into account the ACF. This should be reflected in modifications to the GBAS/VDB frequency assignment planning material in the Handbook, Volume II (Doc. 9718, Handbook on Radio Frequency Spectrum Requirements for Civil Aviation, Volume II, Frequency assignment planning criteria for aeronautical radio communications and navigation systems, Volume II, Second Edition, 2022).

1.2		The CNTWG also considered proposals as presented in JWGs/10-WP/21 for amending the Handbook on the actual DOC for the GBAS/VDB. The CNTWG had no concerns with the proposed changes.

1.3		Following the discussion at JWGs/10, the NSP 11th Joint Working Groups (JWGs/11) meeting (28 November – 1 December and 5 December 2023), considered JWGs/11-WP/6, which proposed modifications to the Handbook, Chapter 6 (GBAS) in conformity with the original proposals as in JWGs/10-WP/08 and JWGs/10-WP/21.

1.4		JWGs/11, after reviewing the proposed modifications and introducing some amendments, reflected in JWGs/11-Flimsy/10, approved them for submission to FSMP for review and inclusion in the next edition of Doc 9718, Volume II.

1.5		The proposed modifications, as amended by Flimsy 10 Revision 1, are presented in the Appendix for FSMP action as detailed in paragraph 3.

- DISCUSSION OF PROPOSED MODIFICATIONS TO CHAPTER 6 OF DOC 9718, VOLUME II

- 2.1	Designated Operational Coverage for GBAS.

- 2.1.1	The proposed modifications to paragraph 6.2 (Designated Operational Coverage; DOC) clarify the DOC that may be considered in actual frequency assignment planning, in particular when at an airport multiple GBAS approaches and landing operations are configured for multiple runways and served by a single GBAS/VDB facility. The proposed modifications also correct an error in Figure 6-2 (Minimum required DOC for a GBAS and clarify the use of extended GBAS/DOC.

- 2.1.2	The modifications are in line with the material that was presented to the NSP in NSP/7, Flimsy 30, Rev. 1 and JWGs/10-WP/21

- 2.2	FM immunity

- 2.2.1	A minor modification to paragraph 6.4 on FM immunity points the reader to the ITU-R Recommendation SM.1841 which contains material specific to the compatibility between GBAS/VDB operating in the frequency band between 108 MHz and 117.975 MHzand FM broadcasting stations operating in the frequency band below 108 MHz.

- 2.3	Airborne Correction Factor. Modifications are presented to paragraph 6.6 (on protection requirements for the GBAS/VDB. These modifications include the following:

- 2.3.1	The whole of the section 6.6.2 on the Airborne Contribution Factor has been replaced to the new paragraph 6.11. The material in this (new) paragraph can be used in cases where the assumptions for GBAS/VDB frequency assignment planning as in the new paragraph 6.10.1 cannot be applied.

- 2.4	Protection requirements for GBAS/VDB receivers.

- 2.4.1	Missing data from Table 6.2 on the Co- and adjacent channel protection requirements for GBAS/VDB receivers has been added based on the material that was presented to the NSP with NSP WGW/12 WP/08 ((SESAR 15.3.6 on “Measurement results of the compatibility between GBAS/VDB with ILS and VHF-COM). It should be noted that in the SESAR Report the adjacent channel interference from undesired localizer facilities is much better (lower) than those presented in Table 6.2.

- 2.5	Interference from VHF communication systems.

- 2.5.1	The calculation results in Table 6.4 on Minimum separation distances between aircraft has been brought in line with the D/U ratios as in Table 6.2.

- 2.6	Calculation of minimum separation distances

- 2.6.1	In section 6.9 on the Calculation of minimum separation distances the Tables presenting the minimum required separation distances (Table 6.5 on the Minimum separation distances between desired and undesired GBAS stations, Table 6.6 on the Minimum separation distances between the DOC of the desired GBAS station and the location of the undesired VOR or Localizer (17 dBW), Table 6.7 on the Minimum separation distances between the DOC of the desired GBAS station and the location of the undesired VOR or Localizer (27 dBW) and Table 6.8 on the Minimum separation distances between the DOC of the desired GBAS station and the location of the undesired VOR or Localizer (30 dBW) were brought in line with the protection requirements as in Table 6.2 (Protection requirements for GBAS/VDB receivers without considering the Airborne Correction Factor).

- 2.7	Frequency assignment planning methodology for a GBAS/VDB

- 2.7.1	Section 6.10 on the Frequency assignment planning methodology for a GBAS/VDB was modified to include a paragraph explain the assumptions on the use of GBAS/VDB that allow for not using the ACF in frequency assignment planning based on the experience with GBAS/VDB implementations.

- 2.8	Same Airport Compatibility

- 2.8.1	In this section Step 4 has been added to point to the need for a “Same airport compatibility analysis” that may be required

- 2.8.2	No proposals have been developed for an ICAO publication of this material.

- 3.	ACTION BY THE MEETING

- 3.1	The meeting is invited to review and agree to the inclusion in the next edition of Doc 9718, Volume II, of the proposed modifications to Chapter 6 as shown in the Appendix to this paper, which reproduces JWGs/11-Flimsy/10.

- APPENDIX

END —