FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

Twentieth Working Group Meeting

Bangkok, Tailand, 26 Feburary to 7 March 2025

Agenda Item 8:	Any other business

Information for Specification of ASDE in Japan in consideration for possible frequency bands for Detect and Avoid (DAA) Radar

(Presented by Katsuyuki Arakawa)

SUMMARY

This IP provides information for the specification of Airport Surface Detection Equipment (ASDE) in Japan in the frequency Band 24.45-24.65 GHz in consideration for possible frequency bands for the Detect and Avoid (DAA) radar system.

INTRODUCTION

Various types of Detect and Avoid (DAA)  radar systems are under development around the world. Possible frequency bands for DAAs were discussed at the FSMP-WG19, and the following frequency band were tentatively selected as candidates.

| Band | Service | Aviation use | Section 7-II page no. |
| --- | --- | --- | --- |
| *1 215–1 400 MHz | RLS/ RNSS ARNS | GNSS Primary surveillance radar † | 7-79 |
| *2 700–3 300 MHz | ARNS/RNS/ RLS | Primary surveillance radar † | 7-109 |
| *5 350–5 470 MHz | ARNS | Airborne weather radar †† | 7-127 |
| 8 750–8 850 MHz | ARNS/RLS | Airborne Doppler radar †† | 7-129 |
| 9 000–9 500 MHz | ARNS/RNS | Precision approach radar/ airborne weather radar/ ASDE † | 7-131 |
| 13.25–13.4 GHz | ARNS | Airborne Doppler radar †† | 7-143 |
| 15.4–15.7 GHz | ARNS/RLS | ASDE/other systems ††† | 7-149 |
| 24.25–24.65 GHz | RNS | ASDE ††† | 7-159 |
| 31.8–33.4 GHz | RNS | ASDE/ enhanced flight vision system (EFVS) ††† | 7-165 |
| AM(R)S: Aeronautical mobile (route) service AMS(R)S: Aeronautical mobile-satellite (route) service ARNS: Aeronautical radionavigation service MSS: Mobile-satellite service RLS: Radiolocation service RNS: Radionavigation service RNSS: Radionavigation-satellite service |  |  |  |

†	These radars could include systems that provide Detect and Avoid surveillance.

††	These radars could include systems that provide airborne Detect and Avoid surveillance.

†††	Both ground based and airborne DAA can be used in this band.

DISCUSSION

Among those frequency bands that could be used for DAA radar, 24.25–24.65 GHz band is also considered for shared use with ASDE.

ASDEs in Japan use frequency band 24.25–24.65 GHz at varuous airports such as Haneda and Narita international, and so that DAA radar in the same frequency band should not affect performance of ASDEs in Japan.

The DAA radar systems should be designed based on  the sharing study in accordance with the following specification of ASDEs in Japan.

Specification of ASDEs in Japan

| - Parameter | - Value |
| --- | --- |
| - Frequency range | - 24.25 – 24.65 GHz |
| - Peak transmitter power | - 3 kW |
| - Pulse width | - 20 ns |
| - Pulse repetition frequency | - 25,300 pps |
| - Receiver noise figure | - 2 dB |
| - Rx noise power | - -105 dBm |
| - Horizontal beamwidth | - 0.3 degrees |
| - Polarization | - Circular |
| - Antenna gain | - 45dBi |
| - Antenna Rotations | - 60rpm |

— END —