| FLIMSY 1r4 [Source: Annex 7 to the Nov 2017 WP5B Chairman’s Report] |  |
| --- | --- |
| Working document towards Draft CPM text |  |
| AGENDA ITEM 1.10 |  |

(WP 5B / WP 4A, WP 4B, WP 4C, WP 5A, WP 5C, WP 5D, WP 6A, WP 7C,
WP 7B, WP 7D
(WP 3M))

1.10	to consider spectrum needs and regulatory provisions for the introduction and use of the Global Aeronautical Distress and Safety System (GADSS), in accordance with Resolution 426 (WRC-15);

Resolution 426 (WRC-15): Studies on spectrum needs and regulatory provisions for the introduction and use of the Global Aeronautical Distress and Safety System

# 5/1.10/1	Executive summary

In accordance with Resolution 426 (WRC-15), ITU-R considered spectrum needs and regulatory provisions for the introduction and use of the global aeronautical distress and safety system (GADSS).

One method is proposed which states that no changes to Article 5 are required but other Articles of the Radio Regulations (RR)  may need some modifications and additional Articles may be required.

# 5/1.10/2	Background

The International Civil Aviation Organization (ICAO) has developed a concept of operations (ConOps) to support future development of a GADSS.

The ConOps describes in particular the following functions:

–	Aircraft tracking:

Typically leverages existing technologies to assist in the timely identification and location of aircraft. Provides an automated reporting function every 15 mins or less. Aircraft tracking may be accomplished by multiple different systems over the duration of a flight.

–	Autonomous distress tracking:

An automated method of position reporting at intervals of one minute or less to support search and rescue (SAR), triggered by indications that an aircraft is in distress which may result in an accident. Distress tracking aims to establish the location of a potential accident site within a 6 nautical mile (11.11 km) radius.

–	Post flight localization and recovery:

A combination of both the immediate need to locate and rescue possible survivors after an air accident using emergency location beacons and other methods to an accuracy of <1 nautical mile (<1.85 km), and the timely collection of aircraft components and data that will assist in the accident investigation.

–	Procedures and information management:

The method of data collection and notification of flight tracking data to the relevant SAR, and rescue coordination centres.

The ConOps is the guidelines for the development of ICAO performance-based standards, outlining specific technical and operational requirements that an aircraft must meet. It  does not identify specific systems proposed to contribute to GADSS. ICAO intends to use systems operating under appropriate existing allocations in accordance with the provisions of the RR, including the use of emergency position-indicating radio beacons operating in the 406-406.1 MHz frequency band.

# 5/1.10/3	Summary and Analysis of the results of ITU-R studies

As ICAO has concluded that the GADSS requirements can be satisfied using systems operating within existing aeronautical frequency allocations or distress spectrum, and also that for WRC-19 no additional spectrum allocations are required, then no changes are required to the Radio Regulations Article 5.

Possible changes to portions of RR Chapters VII  have been identified to facilitate GADSS implementation.

5/1.10/3.1  Relevant ITU-R Recommendations and Reports

Working Document towards a Preliminary Draft New Report ITU-R M.[GADSS}

# 5/1.10/4	Methods to satisfy the agenda item

There is one method to satisfy the agenda item.

## 5/1.10/4.1 	Method A

Invites ITU-R 1 b) of Resolution 426 (WRC-15) calls for the analysis of the existing allocations to the relevant aeronautical services in order to determine whether any additional spectrum is required.

Regarding this question, no additional spectrum is needed to support GADSS, and as a result no changes to Article 5 of the RR are proposed.

Invites ITU-R 2 of Resolution 426 (WRC-15) calls for studies of the existing regulatory provisions to determine whether it might be necessary to apply additional regulatory measures.

Regarding this question, in order to facilitate its introduction, modification of the RR is proposed to include GADSS as a distress and safety communications system in Chapter VII – Distress and safety communications.

# 5/1.10/5	Regulatory and procedural considerations

ARTICLE 5

Frequency allocations

NOC

ARTICLE 30

General Provisions

Section I – Introduction

MOD

30.1	§ 1	Nos. 30.4-30.13 and Articles 31, 32, 33 and 34 of this Chapter contain the provisions for the operational use of the global maritime distress and safety system (GMDSS), whose functional requirements, system elements and equipment carriage requirements are set forth in the International Convention for the Safety of Life at Sea (SOLAS), 1974, as amended. These Articles also contain provisions for initiating distress, urgency and safety communications by means of radiotelephony on the frequency 156.8 MHz (VHF channel 16). (WRC-19)

ADD

30.1A	Article 34A of this Chapter contains the provisions for the global aeronautical distress and safety system (GADSS), whose functional requirements are set forth in the Convention on International Civil Aviation, as amended.     (WRC-19)

ADD

ARTICLE 34A

Global Aeronautical Distress and Safety System

34A.1	The GADSS concept of operations determines performance requirements for the radiocommunication systems being used for conducting functions such as

aircraft tracking,

autonomous distress tracking, and

post flight localization and recovery. (WRC-19)

34A.2	The type of radiocommunication service to be used by systems contributing to the GADSS depends on the requirements of the specific GADSS function. Systems contributing to the GADSS shall not operate under the provisions of Article 4.4. (WRC-19)

SUP

RESOLUTION 426 (WRC-15)

Studies on spectrum needs and regulatory provisions for the introduction and use of the Global Aeronautical Distress and Safety System

____________________

Version 6.0.

In carrying out these studies it has been noted that other aeronautical-related provisions within the RR may not reflect the current/future plans for aeronautical use. These are outside the scope of agenda item 1.10 and as a result no related WRC-19 action is proposed.