FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

Seventeenth Working Group meeting

Cairo, Egypt, 30 August – 8 September 2023

Agenda Item 8:	Any Other Business

LDACS/GNSS compatibility – Outcomes of NSP JWGs/10

Presented by Guillaume Novella (France)

Prepared by Guillaume Novella, Julien Lesouple, Christophe Macabiau

SUMMARY

This paper recaps the discussion on LDACS vs GNSS compatibility which took place at NSP JWGs/10 and at RTCA SC-159 WG-6.

INTRODUCTION

FSMP WG/16 WP/07 analysed the compliance of LDACS ground-to-air (forward link) and air-to-ground (return link) links with GNSS regulatory framework. It was concluded that there was no major issue concerning the LDACS ground-to-air link with the assumptions we made (ITU Res 417 met, LDACS-GNSS distance >500 ft, assumptions that LDCAS ground station antenna pattern is identical to DME beacon pattern, assumptions on LDACS ground stations network distribution over Europe from LDACS paper), but the compliance of LDACS air-to-ground link with GNSS interference mask deserves further discussion.

FSMP WG/16 took the action (Action item 16-07) to forward WP/07 on compatibility between GNSS and LDACS to NSP GNSS SARPs WG for any comments on hypothesis concerning LDACS which were assumed.

The topic has been discussed by NSP GNSS SARPs WG (WP/13, submitted by France). This contribution summarizes the discussion that took place at NSP JWGs/10 and at RTCA SC-159 WG-6 on the compatibility analysis between LDACS and GNSS.

- FSMP WG/16 WP/07 and NSP JWGs/10 WP/13 are attached in annex.

DISCUSSION

The main concern risen in NSP JWGs/10 WP/13 is the compliance of LDACS air-to-ground signal, as defined in Draft LDACS SARPs, with GNSS interference power thresholds (also called the GNSS RFI mask) published in the Annex 10 to the ICAO convention. The GNSS interference mask provides the maximum power at the GNSS antenna port of an external interference that can be tolerated by a GNSS receiver. This set of power interference thresholds is derived by evaluating how much margin exists on the effective C/N0 for all aeronautical GNSS receivers to operate while complying with the minimum performance requirements. The equivalent wide-band noise margin identified is converted in a maximum tolerable power level from external interference. In the case of GNSS L5/E5a, the C/N0 margin is extremely thin.

The LDACS air-to-ground link received power at the GNSS antenna port is given by (Eq 1).

|  | PLDACS=EIRPLDACS∙PAPR∙PL∙GRx∙Lpolar | (Eq 1) |
| --- | --- | --- |

- EIRPLDACS is the maximum average EIRP of airborne transmitter (41 dBm according to Draft LDACS SARPs).

- PAPR refers to the peak-to-average power ratio in order to take into account that for OFDM, the peak power can exceed the average power.

- PL refers to propagation losses.

- GRx is the GNSS receiver antenna gain.

- Lpolar refers to polarization losses.

Discussion at NSP JWGs/10 and RTCA SC-159 WG-6 mainly focused on:

- A relevant value to be used for PAPR in the link budget between LDACS airborne transmitter and GNSS airborne receiver.

- The interpretation of the GNSS interference mask to consider a relevant threshold in the compatibility analysis.

Discussion on peak-to-average power ratio

PAPR complementary cumulative density function (CCDF) is represented in Figure 1.

Figure 1: LDACS PAPR CCDF

A rule of thumb for GNSS is to tolerate a maximum rate of interference above the threshold of 10-6 h-1 (see ITU Recommendation M.1903).

In NSP JWGs/10 WP/13, two approaches were presented. The first approach considered only the instantaneous 10-6 probability of exceedance by the peak envelope power, meaning that it was considered that if at any time the peak envelope power during 1 LDACS sample exceeds the RFI mask, then continuity of GNSS operation is compromised. With this approach, a margin PAPR=11 dB would be required according to Figure 1. This is a conservative approach because, since LDACS symbol period is 1.6 µs according to draft LDACS SARPs, and this 1.6 µs duration is much lower than typical GNSS coherent integration time of 1ms to 20 ms, exceeding the GNSS interference threshold during one LDACS symbol does not necessarily induce a disruption of continuity of GNSS service.

- In the second approach, a disruption of GNSS service continuity is supposed to occur when the LDACS interference power at the antenna port averaged over the integration time Ti exceeds the threshold. In this analysis, Ti is set to 1 ms in order to address the GNSS L5/E5a signal acquisition function, which is the GNSS function using the shortest integration time, therefore inducing the smallest power averaging. Assuming independent LDACS samples, it was demonstrated in NSP JWGs/10 WP/13 that PAPR=2 dB is sufficient to respect the maximum GNSS service continuity disruption rate of 10-6 per hour. Assuming independence between LDACS samples and therefore also independence of occurrences of events of no exceedance of 1ms-average LDACS RL power over 1 hour may not be the most realistic assumption. But no other guaranteed model for time correlation scheme of LDACS RL power is known by the authors at this stage so we prefer to keep this model of independence for now.

- It is mentioned in WP-02 that the assumption of independence between LDACS samples is not correct. As illustrated in Figure 2, comparing the LDACS power averaged over 1 ms (normalized) computed from generated samples from matlab software with the theoretical approach assuming independence, preliminary simulations indeed tend to confirm this statement, and it may be considered to lower this value down to 0.5 dB at a latter stage, but further validations are needed.

Figure 2: Comparison of CCDF of LDACS power averaged over 1 ms, from LDACS Matlab software and from the theoretical approach assuming independence

It can be noted that LDACS SARPs indicated that the duty cycle of LDACS RL is limited. However, in absence of further information (transmission duration, duty cycle value, dependence with phase of flight), this assumption is not taken into account in order to stay conservative.

Discussion on the interpretation of the GNSS interference mask

RTCA SC-159 WG-6 brought clarification of the interpretation and usage of the GNSS interference mask. The interference threshold to be applied to LDACS signal is derived from the GNSS interference mask as follows:

- A 6 dB power level margin is applied to the RFI mask, in order to account for the unknowns (known and unknown), whatever the interfering system to be considered. According to ITU-R Rec M.1903, this margin may be consumed by variations in the GNSS C/N0,eff, and therefore, no credit is assigned the interfering signal because of the existence of the safety margin. This margin is therefore applicable to all interference sources not accounted in the RFI mask.  In the case of LDACS Return Link, this margin may cover uncertainties in modelling (such as propagation model near aircraft fuselage), uncertainties in LDACS parameters since Draft LDACS SARPs are not yet definitive and because of a lack of step back and long-term data with LDACS, as well as other phenomenon that are hardly predictable. Again, this margin is applied to all interference sources not accounted in the RFI mask. Historically, this 6 dB margin was based on the interference environment before LDACS was considered. As a consequence, the margin to be taken into account to cover known unknowns and unknown unknowns cannot be under 6 dB.

- A 3 dB power level apportionment margin should also be added for LDACS, to accommodate for other interference impacting the GNSS receiver. For example, DO-292 already identified at least two external sources that have a non-negligible impact on GNSS receiver performance. The impact of these 2 known external sources is modelled as an equivalent wide band noise affecting the GNSS correlator outputs. DO-292 estimates the equivalent in-band noise level induced by on-board PEDs and terrestrial emitters at -207.2 dBW/Hz, whereas the maximum equivalent noise level from interference not accounted in the RFI mask that can be tolerated by a GNSS receiver is -201.58 dBW/Hz (this is the input value to derive the interference mask). Therefore, the room left for other interference sources is -203 dBW/Hz. A similar approach applies for out-of-band sources of RFI such as LDACS. These 3 numbers are given as an indication that known external interference besides LDACS already exist, and that the margin on the equivalent wide-band noise induced by RFI is thin because the noise induced at the correlator outputs is already close to the maximum tolerable. Thus a minimum back-off of 3dB on LDACS RL received power is recommended.

From the discussion above, the relevant LDACS RL interference threshold to be considered are given in Table 1.

|  | - 964 MHz | - 979 MHz | - 1010 MHz |
| --- | --- | --- | --- |
| - Interference threshold (dBm) | - -1 | - -1 | - -3.29 |

Table 1: GNSS interference threshold for LDACS RL signal

Results

Applying assumptions of 2.5 and 2.6, and assuming the LDACS antenna is mounted on the top of the fuselage, the comparison between the LDACS RL received power and the interference threshold is presented in Table 2.

|  |  | Frequency (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 964 | 979 | 1010 |
| Distance between LDACS and GNSS antennas (m) | 0,5 | 10,90 | 10,76 | 10,49 |
|  | 1 | 4,88 | 4,74 | 4,47 |
|  | 1,968 | -1,00 | -1,14 | -1,41 |
|  | 2 | -1,14 | -1,28 | -1,55 |
|  | 2,445 | -2,89 | -3,02 | -3,29 |
|  | 5 | -9,10 | -9,24 | -9,51 |
|  | 10 | -15,12 | -15,26 | -15,53 |
|  | 20 | -21,14 | -21,28 | -21,55 |
| Interference threshold (dBm) |  | -1 | -1 | -3,29 |

Table 2: Comparison between LDACS RL received power and interference threshold

From Table 2, a minimum separation of 2.45 m would be necessary between LDACS and GNSS antennas to respect the interference threshold.

When the LDACS antenna is on the bottom of the aircraft fuselage, no compatibility issue has been identified.

Discussion at NSP also highlighted that it would be relevant to address this isolation matter in certification or other industrial document/standard.

ACTION BY THE MEETING

The meeting is invited to:

- note and review the contents of this working paper;

ANNEX

— END —