FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

Eleventh Working Group meeting

Web Meeting, 1 – 12 March 2021

Agenda Item 5:	Development of (planned) Material for ITU-R Studies on:

b)	WRC-23 AI1.7 VHF AMS(R)S

Liaison Statement from ITU-R WP5B

on

WRC-23 Agenda Item 1.7

(Presented by the Secretary)

SUMMARY

Attached is a liaison statement from ITU-R WP5B on WRC-23 Agenda Item 1.7. Action: FSMP WG/11 is invited to review the attached liaison statement and take action as appropriate.

______________

| Radiocommunication Study Groups |  |  |
| --- | --- | --- |
| Source: Document 5B/TEMP/45(Rev.2) Subject: WRC-23 agenda item 1.7 LS ICAO |  |  |
|  |  | 26 November 2020 |
|  |  | English only |
| Working Party 5B |  |  |
| Liaison statement to ICAO |  |  |
| WRC-23 agenda item 1.7 |  |  |
| Questions on a space-based aeronautical VHF communications system in 117.975-137 MHz frequency band |  |  |

In order to meet the evolving requirements of modern civil aviation, satellite systems may be considered within the band 117.975-137 MHz for the relay of aeronautical VHF communications over oceanic and remote areas. Under WRC-23 agenda item 1.7 and Resolution 428 (WRC-19), Working Party (WP) 5B is tasked to define the relevant technical characteristics and to study compatibility between such satellite systems and existing primary services in-band and in adjacent bands.

At its last meeting in November 2020, WP 5B has therefore initiated the development of [elements of a working document related to AI 1.7 WRC-23] working document towards a preliminary draft new Report ITU-R M.[Space-VHF] (see Attachment). The preliminary parameters concentrate on the provision of voice communications, considered as the essential application to provide, and on the satellite to aircraft downlink path, as it would be the new active element introduced in the overall picture of aeronautical VHF stations.

In order to duly complete its task on defining the relevant technical characteristics and to perform sharing and compatibility studies, under WRC-23 agenda item 1.7, WP 5B would like to question ICAO on the following elements:

Available measured pattern for aircraft VHF antenna

Available information indicate that aircraft VHF antenna have a relatively low maximum gain (typically 3 to 5 dBi), an omnidirectional radiation pattern in azimuth, and a cosinusoidal radiation pattern in elevation (meaning a theoretical zero is achieved at aircraft zenith).

Working Party 5B would like to know if measured (i.e. taking account of aircraft body) receiving and transmitting VHF antenna radiation patterns can be provided by ICAO, both for upper and lower aircraft VHF antennas. Ideally, several typical patterns would be desirable, for the different classes of aircraft that fly over oceanic and remote areas (from small transoceanic business jets to large aircraft).

Aircraft VHF receiver performance requirement

In terms of aircraft receiver sensitivity, the recommendation contained in section 2.3.2.2.1 of Annex 10 Volume III Part II to the Convention on International Civil Aviation recommends an airborne sensitivity level of 30 microvolts per metre in the case of “extended range VHF facilities”, assumed to correspond to the satellite case.

In addition, the “Minimum operational Performance Specification (MOPS) for airborne VHF receiver-transmitter operating in the frequency range 117.975-137 MHz” developed by EUROCAE and RTCA specify that minimal receiver sensitivity to achieve a SINAD of 6 dB is -93 dBm/ 25 kHz.

Working Party 5B would like to get confirmation from ICAO on which sensitivity is the most relevant to be used for the studies in the Attachment.

Required availability performance

Working Party 5B would like to know from ICAO if a reference availability target can be identified for aeronautical VHF voice communications, and if such a reference could depend upon time and location.

Working Party 5B looks forward to continuing fruitful cooperation with ICAO regarding this work.

| Status: For action |  |
| --- | --- |
| Deadline: Next WP 5B meeting (May 2021) |  |
| Contact: Mr. Loftur Jonasson | E-mail: LJonasson@icao.int |

Attachment:	Working document towards a preliminary draft new Report ITU-R M.[Space-VHF]

______________