FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

Eleventh Working Group meeting

Web Meeting, 1 - 12 March 2021

Agenda Item 10:              Any Other Business

Title: UPDATE ON THE USE OF ULTRA-WIDEBAND SYSTEMS IN INDIA

(Presented by Airports Authority of India)

SUMMARY

This paper informs the FSMP on the use of ultra wide band (UWB) devices and their regulations in India.

INTRODUCTION

Globally, the use of ultra-wideband (UWB) systems/devices is significantly on the increase and their proliferation has the potential to cause harmful interference to safety critical aeronautical systems. As the impact of UWB devices is not fully understood at this time, the last FSMP-WG/10 meetng assigned an action item (AI10-06) to states to indicate in the next FSMP-WG/11 meeting how the Radio Frequency Regulatory Autorities handle the UWB system authorizations at the respective states.

This information paper provides information on how UWB devices are authorized in India.

DISCUSSION

The very low power UWB devices permitted to operate between the frequency range 1.6 GHz to 10.6 GHz and above 10.6 GHz are:

- Generic  ultra-wideband device such as personal computers, handheld terminals, cable modems, set-top boxes, indoor access points etc.

- Location tracking system- for location tracking of people or objects.

- Ultra-wideband device installed in Road and Rail Vehicle

- Material sensing device -a radio determination device designed to detect the location of objects within a structure or to determine the physical properties of a material

- Building material analysis device-  a material sensing device that is used to detect the location of objects within a building structure, or to determine the physical properties of building material.

2.2 		The use of UWB devices is licence-exempted and the designated frequency bands are available on a non-interference, non-protected and shared on non-exclusive basis.

2.3 		Decision 2014/702/EU, dated 07 October 2014 (amending Decision 2007/131/EC) of the European Commission is applied, as a whole, to regulate the use of UWB devices in India. It is pertinent to note that now European Commission has repealed Decision 2007/131/EC vide Decision 2019/785 of 14 May 2019.

CONCLUSION

The meeting is invited to note the contents of the paper for information.