FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

Ninth Working Group meeting

Montréal, Canada, 22 – 30 August 2019

Agenda Item 9:	Revision of ICAO Frequency Spectrum Handbook (Doc 9718)

ICAO SPECTRUM HANDBOOK – VOL. C UPDATES ON TECHNICAL CHARACTERISTICS

(Presented by Andrew Roy)

SUMMARY

Provides an updated structure and additional information that should be included when documenting aviation system characteristics. The paper also includes some additional material that may have considerations for other parts of Vol. B that are not already in the existing Handbook

INTRODUCTION

With feedback from FSMP WG/8, the ICAO Handbook Vol. C – System Characteristics has been modified to reflect the thoughts process and discussions.  The updated generic structure in Annex A is then used in Annex B as an example with the radio altimeter given the ongoing focus for that system

DISCUSSION

Since the last meeting and the relative feedback, the proposed structure of the system parameters has been updated to incorporate:

Revised reference model structure

Operational information on which types of airspace users are equipped with the system

Clarification on when the system is used.

By implication of the above points, it should also be noted that some part of the new handbook may need a ‘dummies guide’ to airspace usage and phases of flight.  While this wouldn’t be comprehensive, it would allow for guard rails to ensure other industries are not taking an overly favourable view when conducting interference analysis.  This is still a concept, and would need exploring further to see if it is a viable options.

Once agreed at this meeting, it is proposed that service providers and those with specialist knowledge of certain systems be asked to provide the necessary data for the next FSMP meeting in Q1 2020.

SUMMARY

The updating of the ICAO handbook is an ongoing process that requires significant effort to research and develop.  While the structure appears to be nearing an agreed form, contributions from relevant FSMP system experts are essential to support the work on this text at future FSMP meetings.

ACTION BY THE MEETING

The meeting is invited to:

note and review the contents of this working paper;

Provide feedback in the meeting on the discussion and supplement document’s format/content.

Recommend actions be given to relevant system experts/service providers to fill in necessary system parameters to Annex A.

— END —

# Annex A – Generic System Parameter Template

# [System Name] (Frequency Range)

## QUICK REFERENCE

| System name | System name |
| --- | --- |
| Description | One sentence description of the system |
| Technologies used | RF emissions (CW, pulsed, etc.) |
| International Spectrum Allocation | International allocation |
| ICAO regulations on usage | Headline ICAO annex that requires system |

## System Summary

Short description of system operation (approx. half a page) covering broad principle of operations and how it integrates into the airframe or airspace management.

## Equipage and usage

Airspace users operating the equipment:  List of airframe types that operate and use the system

Usage:  Phases of flight the system is used in

Additional operational notes:

- Additional notes on equipage or usage that may clarify system for non-aviation readers.

## Historical information

Background on the system and its introduction.  Previous system designs no longer in use can be described here (e.g. 100 kHz VHF channels, etc.)

## Radio Altimeter Reference model for interference studies

System parameters

| ### Parameter | ### Value (FMCW) | ### Value (Pulsed) | ### Source (if available) |
| --- | --- | --- | --- |
| Max transmit power (peak W) |  |  |  |
| Receiver sensitivity or noise floor |  |  |  |
| Noise figure (dB) |  |  |  |
| Protection criteria for Desensitization I/N (dB)* |  |  |  |
| Feeder cable losses (dB) |  |  |  |

### Transmitter emissions mask

| Parameter | Value | Source (if available) |
| --- | --- | --- |

### Receiver selectivity mask

| Interference frequency (MHz) | RF filter attenuation (dB) |
| --- | --- |

### Antenna pattern

| Parameter | Value | Source (if available) |
| --- | --- | --- |
| Derived antenna pattern |  |  |
| 3 dB beamwidth (Deg) |  |  |
| Polarization |  |  |
| Maximum gain (dBi) |  |  |
| Antenna FDR (dB) |  |  |

## Technical References:

### ICAO Annex 10

RTCA:

EUROCAE:

ARINC characteristic:

ITU Res./Rec.:

Other relevant material or developments:

# Annex B – Radio Altimeter Example

# Radio Altimeter (4 200–4 400 MHz)

## QUICK REFERENCE

| System name | Radio Altimeter |
| --- | --- |
| Description | A navigational avionics system that provides a continuous report of the aircraft’s height above terrain. |
| Technologies used | Radar based (FMCW or pulsed) |
| International Spectrum Allocation | ARNS |
| ICAO regulations on usage | Annex 6 |

## System Summary

Radio altimeters serve an essential safety-of-life function in landing/take-off, low level maneuvering, and avoiding changes in terrain that may not be visible at night or during bad weather.  The radio altimeter has significantly improved aviation safety for all aircraft types since its introduction, and is now essential for automated landings that increases the safety and efficiency of air travel.

A radar-based system, commercial radio altimeter avionics use Frequency Modulated Continuous Wave (“FMCW”) or pulses to continually report the distance to the terrain below the aircraft.  Both transmitter and receiver are mounted on the underside of the aircraft, pointing in a downwards direction slightly ahead of the aircraft’s direction of travel.  The system operates in all phases of flight, from operation on the ground to high cruising altitudes, and adapts its operation depending on the reported altitude.  Low level flight is the most critical, requiring less than 3 ft of resolution to ensure accurate navigation and landing by either the pilot or autopilot depending on the aircraft approach type.  This is achieved by the altimeters increasing the swept or pulsed signal bandwidth to nearly the full 200 MHz allocation.

For these applications, a good interference rejection performance is essential. Integrity standards of the order of one failure in 1019 operations are not uncommon. The use of a wide frequency band is an essential feature in effective designs to achieve high orders of interference rejection and freedom from disruptive effects due to the high levels of pollution of the radio environment which exist in densely populated areas.  The system also feeds the Ground Proximity Warning System (GPWS), alerting pilots to possible obstructions that may endanger the aircraft.

## Equipage and usage

Airspace users operating the equipment:  Widebody commercial aircraft, narrow body commercial aircraft, private/business aircraft, helicopters, general aviation, drones

Usage:  The altimeter is operational in all phases of flight and while the aircraft is active on the ground.

Additional operational notes:

- The radio altimeter is part of the Minimum Equipment List (“MEL”) for many aircraft, with medium to large aircraft are often fitted with a minimum of two altimeters operating simultaneously for redundancy.  Therefore, all aircraft should be assumed to be carrying at least one radio altimeter.

## Historical information

The radio altimeter was widely introduced after a number of aviation incidents up to the 1970’s of aircraft flying unintentionally into the ground, a circumstance formally known as Controlled Flight Into Terrain (CFIT).  Since its global adoption, CFIT accidents of large commercial aircraft have dramatically reduced from several incidents a year. Use of this band for radio altimeters is expected to continue for the long term.

## Radio altimeter reference model for interference studies

System parameters

| ### Parameter | ### Value (FMCW) | ### Value (Pulsed) | ### Source (if available) |
| --- | --- | --- | --- |
| Max transmit power (peak W) | 1 | 100 | Altimeter A3 and A4 worst case from ITU-R Rec M.2059 |
| Receiver sensitivity or noise floor | -125 dBm | -95 dBm | Altimeter D2 and D4 worst case from ITU-R Rec M.2059 |
| Noise figure (dB) | 10 |  | Altimeter A1 and A6 worst cases from ITU-R Rec M.2059 |
| Protection criteria for Desensitization I/N (dB)* | –6 |  | From ITU-R Rec M.2059 |
| Feeder cable losses (dB) | 3 |  | Typical values from ITU-R Rec M.2059 Attenuation applies to both transmit and receive functions |
| Front End Overload threshold (dBm)* | -56 | -53 | Altimeter A3 and D3 worst cases from ITU-R Rec M.2059 |
| False Altitudes threshold (dBm/ 100 Hz)* | –143 | N/A | From ITU-R Rec M.2059 |
| Additional safety margin to be used in interference studies (dB) | 6 |  |  |

* All three criteria must be met for protection of the system

### Transmitter emissions mask

| Parameter | Value (FMCW) | Value (Pulsed) | Source (if available) |
| --- | --- | --- | --- |
| Waveform repetition frequency (Hz or pulses per second) | 12 - 1623 | 6000 - 25000 | Altimeter A3, A6 and D4 worst cases from ITU-R Rec M.2059 |
| 3 dB emission bandwidth (MHz) | 175 | 31 | Altimeter D3 and D4 worst cases from ITU-R Rec M.2059 |
| 20 dB emission bandwidth (MHz) | 185 | 105 | Altimeter D3 and D4 worst cases from ITU-R Rec M.2059 |
| 40 dB emission bandwidth (MHz) | 196 | 195 | Altimeter D3 and D4 worst cases from ITU-R Rec M.2059 |

### Receiver selectivity mask

| Interference frequency (MHz) | RF filter attenuation (dB) |
| --- | --- |
| ≤ 4 200 | Attenuated at 24 dB per octave to a maximum of 40 dB |
| 4 200 | 0 |
| 4 300 | 0 |
| 4 400 | 0 |
| ≥ 4 400 | Attenuated at 24 dB per octave to a maximum of 40 dB |

### Antenna pattern

| Parameter | Value | Source (if available) |
| --- | --- | --- |
| Derived antenna pattern |  | Based on ITU-R Rec M.2059 = The radio altimeter gain at an angle measured from the vertical axis (dBi) = The total 3dB beamwidth of the antenna (rad) = The maximum antenna gain (dBi) |
| 3dB beamwidth | 60 | ITU-R Rec M.2059 |
| Polarization | Vertical |  |
| Maximum gain | 10 | Typical based on ITU-R Rec M.2059 and ARINC 707 guidance |
| Antenna FDR | 0 | Has been suggested it is minimal in the adjacent bands, therefore assuming negligible at this stage |

## Technical References:

### Annex 10

- SARPs: Development work initiated in 2016

- Frequency plan: None

- Planning criteria:  None

RTCA:

- DO-155, Minimum Performance Standards — Airborne Low-range Radar Altimeters (1974)

- DO-161A, Minimum Performance Standards — Airborne Ground Proximity Warning Equipment (1976)

EUROCAE:

- ED-30, MPS for Airborne Low-Range Radio (Radar) Altimeter Equipment (1980), Amend. 1 (1980)

- ED-83, Recommendations on Ground Collision Avoidance Systems

ARINC characteristic:

- 594-4, Ground Proximity Warning System (GPWS)

- 723-3, Ground Proximity Warning System

- 707-6, Radar Altimeter (RALT) Supplement 6

ITU-R Res./Rec.:

- Recommendation M.2059-0: Operational and technical characteristics and protection criteria of radio altimeters utilizing the band 4 200-4 400 MHz (02/2014)

- Report [BL/8] (Düsseldorf 1990)

Other relevant material or developments:

Cable loss can vary from 1 to 7 dB depending on installation.  Lower cable loss improves wanted signal return but lowers isolation from other onboard radio altimeters.  Therefore a 3 dB values is a tradeoff between the two and is understood to be a common installation guideline.