FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

Twentieth Working Group Meeting

Bangkok, Thailand, 26 February -7 March 2025

Agenda Item 3b:	Development of material for ITU-R studies – Non-WRC material for the ITU

ITU Resolution 609

(Presented by Guillaume Novella)

SUMMARY

This WP provides an overview of the ITU Resolution 609 situation. The Res 609 epfd (equivalent power flux density) threshold protecting the aeronautical radionavigation service from emission of the radionavigation satellite service in the 1164 – 1215 MHz band is reached. This WP encourages involvement of aeronautical stakeholders to follow the way forward which may be proposed by Res 609 consultation meetings.

INTRODUCTION

ITU Resolution 609 addresses the protection of aeronautical radionavigation service systems from the equivalent power flux-density produced by radionavigation-satellite service networks and systems in the 1 164-1 215 MHz frequency band. It provides an aggregate equivalent power flux density threshold that RNSS stations shall not exceed in order to protect DME. It also provides a process to check that RNSS emissions remain below the epfd, and describes the actions to be taken when the epfd threshold is exceeded.

At the 21st consultation meeting of Res 609, for the first time in the history of Resolution 609, the maximum aggregate epfd values computed from meeting inputs exceeded the regulatory limit of -121.5 dB(W/(m²∙MHz)). This situation conducted administrations to take actions to remain compliant with Res 609, and to think to a way forward so that the band 1164 – 1215 MHz remain accessible for new RNSS entrants.

This WP gives an overview of the RNSS environment in the 1164 – 1215 MHz band, and briefs FSMP with the decisions of Res 609 21st consultation meeting (Nov 2024).

DISCUSSION

RNSS situation in the 1164 – 1215 MHz band

RNSS allocation in the 1164 – 1215 MHz was provided by ITU WRC 2000.  First RNSS systems in the 1164 – 1215 MHz band came into service at the end of the years 2000’s and with the development of LEO PNT systems, the number of new projects has significantly increased in recent years. Table 1 provides a status of RNSS systems operating or planning to operate in the 1164 – 1215 MHz band. The information is extracted from the output document of Res 609 21st consultation meeting “List of RNSS systems characteristics” attached in Annex. Further information are contained in the document “RNSS systems or network status 21st CM” also attached in Annex.

| - RNSS System/Network | - Number of satellites | - Operational system ? |
| --- | --- | --- |
| - Galileo-2G | - 36 non-GSO | - Yes |
| - NAVSTAR GPS | - 36 non-GSO | - Yes |
| - GLONASS-M | - 24 non-GSO | - Yes |
| - COMPASS | - Regional system: 7GSO, 7 IGSO, 4 MEO - Global system: 5 GSO, 3 IGSO, 27 MEO | - Yes |
| - QZSS | - 11 non GSO - 2 GSO | - Yes - QZSS-GS-A1 (90.5E) planned for 2025 |
| - LM-RPS-107.3W | - 1 | - Yes |
| - INMARSAT GSO networks | - 7 | - Yes - Inmarsat-8 satellites planned for 2027 |
| - GAGAN | - 3 | - Yes (2011 – 2015) |
| - IRNSS | - 8 non-GSO - 4 GSO | - Yes (4 non-GSO have been launched) - GSO: 2014 - 2016 |
| - LUX-G6-2-E | - 1 GSO | - Yes |
| - LUX-G7-9-E2 | - 1 GSO | - Yes |
| - LUX-G9-38-A | - 1 GSO | - Yes |
| - RAGGIANA-18 | - 1 GSO | - Yes (Jan 2017) |
| - ALCOMSAT-24.8W | - 1 GSO | - Yes (Dec 2017) |
| - NIGCOMSAT-1R | - 1 GSO | - Yes (2011) |
| - MEASAT-1C | - 1 GSO | - Yes (2022) |
| - F-SAT-RN-5W | - 1 GSO | - Planned to be brought into use within 18 months after the start of the tentatively-scheduled Consultation Meeting |
| - CENTISPACE-1 | - 2 non-GSO | - Yes (1 non-GSO have been launched) |
| - USASAT-80C-1 | - 1 GSO | - Yes (July 2020) |
| - PAKSAT-MM1-38.2E-KA | - 1 GSO | - Yes (June 2024) |
| - NSAT01.001 | - 1 non-GSO | - Yes (Expected Q4 2025) |
| - SATNET LEO | - 504 non-GSO | - Yes |
| - GALILEO-L3 | - 2 non-GSO | - Yes |
| - F-SAT-RN-13E | - 1 GSO | - Planned to be brought into use within 18 months after the start of the tentatively-scheduled Consultation Meeting |
| - LUX-G15-7-E | - 1 GSO | - Planned to be brought into use within 18 months after the start of the tentatively-scheduled Consultation Meeting (moving this satellite into this position) |
| - SYNCHROCUBE | - Demonstrator: 1 non-GSO - Full constellation: 24 non-GSO | - Demonstrator expected end of 2024. Launch of the final constellation should starts in 2025. |
| - KOREASAT-116N | - 1 GSO | - Planned for Q1 2025 |
| - KOREASAT-128.2A | - 1 GSO | - Planned to be brought into use within 18 months after the start of the tentatively-scheduled Consultation Meeting |
| - SDCM-KFD | - 3 GSO | - Expected in 2025 |
| - EU-LNAV | - Demonstrator2025 : 1 non-GSO - Demonstrator 2026: 1 non-GSO - Full constellation: 263 non-GSO | - Yes (launch expected end of 2025) - No - No |
| - KPS | - 5 nong-GSO - 1 GSO | - No (GSO planned for 2029) |
| - NSAT01 | - 96 non-GSO | - No (first launches expected Q4 2026, operational constellation in 2030) |
| - XONA PULSAR | - Demonstrator: 1 non-GSO - Final phase: 258 (option 1) / 160 (option 2) non-GSO | - Yes (launch schedules for June 2025) - Yes |

Table 1: RNSS systems operating or planning to operate in the 1164 - 1215 MHz frequency band

EPFD calculation at the 21st Res 609 consultation meeting

Record of decisions of the 21st Res 609 consultation meeting indicates:

- “For the first time in the history of Resolution 609, the calculated maximum aggregate epfd values exceeded the regulatory limit of -121.5 dB(W/(m²∙MHz)). Using the input data to CM 21, exceedances were calculated for center frequencies of 1176 MHz and 1191 MHz. The calculated exceedance at 1176 MHz, where most existing operational systems and networks concentrate their transmit power, was predicted during CM 20.  The calculated exceedance at 1191 MHz is a result of including two new systems (EU LNAV and XONA PULSAR) employing LEO satellites for which the corresponding frequency assignments are expected to be brought into use.”

The following principles has been adopted in order to remain compliant with Res 609 epfd threshold:

- Reduction of power for all satellites that exceeds the maximum per-satellite pfd level in Recommendation 608 (Rev. WRC-07).

- Use of actual operational link characteristics for computation of maximum aggregate epfd instead of filled characteristics.

- Reduction in the list of candidate systems, e.g. in cases when a demonstrator and the full constellation will not operate simultaneously, or when a system has been replaced by a new system.

The previous principles resulted in the following actions:

- Power reduction:

  - Reduction of e.i.r.p. density level submitted to RES 609 CM for  India’s IRNSS satellites (GSOs and non-GSOs) and Korean KPS

  - Reduction of 1.67 dB epfd by XONA PULSAR (full constellation) and 0.3 dB epfd by XONA PULSAR (IOV) satellite

  - Reduction of 0.63 dB epfd at 1191 MHz and neighboring frequencies by EU LNAV SAF modification

- Non-simultaneous operations of RNSS systems:

  - Commitment not to operate full GALILEO-2G constellation at the same time with GALILEO-L3 satellites

  - Commitment not to operate full constellation of France’s SYNCHROCUBE(*) at the same time with Demonstrator 2024

  - Commitment not to operate full constellation of UAE’s NSAT01.001 at the same time with NSAT01

  - Commitment for the IOD (EU LNAV) satellites not to operate simultaneously with the full EU LNAV constellation

  - Commitment for the  XONA PULSAR (IOV)  satellite not to operate simultaneously with the XONA PULSAR full constellation

Considering theses actions, the epfd obtained at the Res 609 consultation meeting is given in Figure 1.

Figure 1: Res 609 21st CM epfd

It is important to note that the aggregate epfd reaches the epfd threshold for several frequencies (1176 MHz, 1191 MHz).

Way forward proposed by the Res 609 consultation meeting

Since little room is left for new RNSS entrants, some ways forward are considered to remain below the Res 609 epfd threshold.

- use of actual operational link characteristics (e.g., maximum operational power levels, instead of filed parameters);

- Use of other portion of the 1164 – 1215 MHz band;

- revisions to the epfd calculation methodology to avoid the addition of different worst cases and revisiting some assumptions (e.g. orbit propagation model);

- geographical analysis of peak aggregate RNSS epfd areas with a view to informing future RNSS systems about areas where RNSS emissions are closest to the epfd limit.

Discussion and evaluation of these methods are expected to take place ahead of Res 609 22nd consultation meeting (2025).

ACTION BY THE MEETING

The meeting is invited to:

note and review the contents of this working paper;

Pay attention to the ways forward taken by Res 609 consultation meetings.

— END —

ANNEX: REFERENCE DOCUMENTS

- Report to BR

- Record of Decisions

- List of RNSS systems characteristics

- RNSS systems or networks status 21st CM

- ITU Resolution 609: 
https://www.itu.int/en/ITU-R/space/Res609%20CM%20Documents/RES-609_e.pdf

- ITU Resolution 608:
https://www.itu.int/en/ITU-R/space/Res609%20CM%20Documents/REC-608_e.pdf

ANNEX: CALCULATION OF RES 609 EPFD THRESHOLD

The EPFD threshold value of -121.5 dBW/MHz/m² is derived in Annex 2 of ITU-R Recommendation M.1639. This calculation is reminded in the table below.

| Parameter | Value | Reference |
| --- | --- | --- |
| DME RNSS interference threshold (at antenna port) | -129 dB(W/MHz) | Eurocae ED-54 §3.16.4 |
| Maximum DME/TACAN antenna gain including polarization mismatch | 3.4 dBi | (5.4 dBi antenna gain – 2 dB polarization mismatch) |
| Effective area of 0 dBi antenna at 1176 MHz | -22.9 dBm² |  |
| RNSS (all systems) aggregate epfd in 1 MHz | -109.5 dB(W/m²/MHz) |  |
| Safety margin | 6 dB | ITU Rec M.1477 |
| Apportionment of RNSS interference to all the interference sources | 6 dB | Apportion 25% of total permissible interference to RNSS |
| Maximum RNSS aggregate epfd | -121.5 dB(W/m²/MHz) |  |

An operational system is one that has frequency assignments in the 1164-1215 MHz band that have been brought into use or that are anticipated to be brought into use within 18 months after the start of the Consultation Meeting. For details, see §§11 b), 12b) and 15 of the Consultation Meeting Terms of Reference.