FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

Fifteenth Working Group meeting

Montreal (hybrid meeting), 22 Aug – 1 Sep 2022

Agenda Item 8:	Any Other Business

Secretariat review of the draft WAIC SARPs

(Presented by the FSMP Secretary)

SUMMARY

FSMP WG/14 agreed on a set of draft new proposed SARPs for WAIC and that those should be liaised with the Navigation Systems Panel (NSP) and the Communications Panel – Data Communication Infrastructure Working Group (CP-DCIWG) for comments, if any. The draft new SARPs were liaised on 11 May 2022, with 30 June indicated as deadline for comments. As of 12 August, no comments have been received. The Secretariat has undertaken an editorial review of the draft new WAIC SARPs package. The draft SARPs as submitted to FSMP WG/14 had been written for insertion into Annex 10 Vol III, however FSMP WG/14 agreed that the WAIC SARPs would be a better fit for Annex 10 Volume V. The editorial review focused mainly on this aspect, while also taking note of the guidance provided in the “ANC Guide to the drafting of SARPS and PANS” and Assembly Resolution 39-22. Some minor edits were also made to the Impact Assessment and Implementation Plan (see Annex 2 of Attachment) and the Validation Statement (see Annex 3 of Attachment). FSMP WG/15 is invited to do a final review of the attached draft new WAIC SARPs package, to facilitate a decision by FSMP/3, which will be held on 7 September 2022. Action: FSMP WG/15 to review the proposed edits to the draft new WAIC SARPs, as attached to this WP.

ATTACHMENT:

Annex 1: Proposed Modifications to Annex 10 Volume V to the Convention on International Civil Aviation

Annex 2: Impact and Implementation Assessment

Annex 3: Validation Statement

______________

ATTACHMENT (including Annexes 1 to 3)

ANNEX 1

Proposed Modifications to Annex 10 Volume V to the Convention on International Civil Aviation

Insert new text as follows

CHAPTER 1.	DEFINITIONS

…

Wireless Avionics Intra-Communications (WAIC) – Radiocommunication between two or more aircraft stations located on board a single aircraft for aircraft applications related to the safety and regularity of flight using the aeronautical mobile (route) service (AM(R)S) allocation in the frequency band 4 200 – 4 400 MHz; supporting the safe operation of the aircraft.

WAIC System – A system which provides wireless communications between points on board a single aircraft. A WAIC System may be comprised of one or more WAIC Networks necessary for establishing, maintaining and securing wireless communications. A WAIC System is understood as the entirety of all WAIC components on board the same aircraft, so that a single aircraft contains only a single WAIC System.

WAIC Network – A network comprising of interrelated WAIC Components, e.g., components used for wireless communications, security or network management.

WAIC Component – Any tangible entity of a WAIC Network on board an aircraft.

WAIC Node – A specific category of a WAIC Component establishing wireless communications between aircraft applications or parts thereof.

| Origin: FSMP | Rationale: The above definitions are specific to WAIC and are being added to the definitions in Chapter 1 of Annex 10 Volume V. |
| --- | --- |

CHAPTER 4.    UTILIZATION OF FREQUENCIES ABOVE 30 MHz

…

4.5    UTILIZATION IN THE FREQUENCY BAND 4 200 – 4 400 MHz

4.5.1    Utilization for Radio Altimeters

To be developed

4.5.2     Utilization for Wireless Avionics Intra-communication (WAIC) systems

Note:		The below provisions for WAIC define the requirements that ensure that WAIC Systems and radio altimeters can provide their intended functions while multiple aircraft are in mutual radio range. Coexistence between WAIC Systems and radio altimeters installed on board the same aircraft is addressed by the specific implementation and established airworthiness certification processes.

4.5.2.2	WAIC Systems shall comply with the applicable provisions of the Radio Regulations.

4.5.2.3	WAIC shall only be used for communications related to the safety and regularity of flight, between two or more points on a single aircraft.

4.5.2.4	WAIC Systems shall not cause harmful interference to radio altimeter systems and WAIC systems on other aircraft.

Note:	Compliance with Error! Hyperlink reference not valid.4.5.2.4 is achieved by limiting the power of WAIC emissions below the level at which altimeter performance may be affected, consistent with 4.5.2.6 Error! Hyperlink reference not valid.below. and provide one acceptable method of demonstrating compliance with that power spectral density limit. EUROCAE/RTCA documents ED-260A/DO-378A provide one acceptable method of demonstrating compliance with that power limit.

4.5.2.5	A WAIC system located on board one aircraft shall maintain its intended function while subject to emissions from WAIC and radio altimeter systems located on board other aircraft.

Note:	EUROCAE/RTCA documents ED-260A/DO-378A provide one acceptable method of demonstrating compliance with 4.5.2.5 via test. Alternatively, the critical coexistence scenario described in those documents may also be used to develop appropriate analyses to demonstrate compliance with 4.5.2.5.

| Origin: FSMP | Rationale: This Rationale Deleted |
| --- | --- |

4.5.2.6	Radio frequency (RF) characteristics of WAIC systems

4.5.2.6.1	WAIC systems shall operate in the frequency band 4 200 – 4 400 MHz.

4.5.2.6.2	The power of the total aggregate emissions of all WAIC transmitters on board an aircraft shall not exceed an equivalent isotropic radiated power of -20dBm, assuming a point source located at the geometrical center of the aircraft.

Error! Hyperlink reference not valid.

4.5.2.6.3	The overall occupied bandwidth shall be maintained completely within the allocated frequency band 4 200 – 4 400 MHz including any offsets such as Doppler shift or frequency tolerances. Where the occupied bandwidth is defined as the bandwidth for which 99% of the signal energy falls within the lower and upper frequency limits.

Note:	The Radio Regulations define the occupied bandwidth as “The width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage β/2 of the total mean power of a given emission”. The value of β/2 being 0.5%.

4.5.2.6.4

The necessary bandwidth (NB) of the WAIC transmitter shall be calculated according to Appendix 1 of the Radio Regulations.

4.5.2.6.5	 The boundary between the out-of-band and spurious domains shall be determined according to Annex 1 of Appendix 3 of the Radio Regulations. The required attenuation of the mean power of any unwanted emission relative to the total mean power P shall meet or exceed the following conditions:

50% of NB < f < 150% of NB:	Linear increase (in dB) from 24dB to 35dB within a reference bandwidth of 4kHz (Note 1)

150% of NB < f <  start of the spurious domain: 35dB within a reference bandwidth of 4kHz (Note 1)

Spurious domain:	56+10log(P) or 40dB whichever is less stringent measured in a RBW of 1MHz (Note 2)

Note 1:	Reference bandwidth of 4 kHz within the out-of-band domain in accordance with Annex 11 of Recommendation ITU.R SM.1541-6. The parameter f is the frequency separation from the center frequency of the transmit signal.

Note 2: 	Reference bandwidth of 1 MHz within the spurious domain in accordance with Appendix 3 paragraph 7 of the Radio Regulations and determination of attenuation for low power device radio equipment in accordance Appendix 3 paragraph 13 of the Radio Regulations.

4.5.2.7				Out-of-Band Interference Tolerance of a WAIC Receiver

Note: These requirements are for WAIC equipment and define an RF environment in which WAIC equipment must meet its performance requirements without taking into account any mitigation afforded by its installation.

4.5.2.7.1 Receivers shall tolerate interference from sources operating outside of the frequency band 4  200 ‐ 4 400 MHz whose total combined emitted power falling within the frequency band 4 200 ‐ 4 400 MHz as measured at the receiver does not exceed a power spectral density of ‐120 dBm / MHz.

4.5.2.7.2 Receivers shall tolerate interference from sources operating outside of the frequency band 4 200 – 4 400 MHz whose total combined power as measured at the receiver does not exceed -20 dBm.

| Origin: FSMP | Rationale: In order to ensure safe operation, thesections above describe the purpose, basic functions and minimum RF characteristics of the WAIC transmitter and the WAIC receiver, necessary to prevent interference between a WAIC system onboard one aircraft and radio altimeters onboard other aircraft. No guidance is required to ensure coexistence of WAIC systems onboard multiple aircraft. |
| --- | --- |

— — — — — — — —

ANNEX 2

Impact and Implementation Assessment

Proposed Text to be Submitted to ANC

PART 1: IMPACT ASSESSMENT

1.1	What is the problem/opportunity that this proposal is designed to address?

Please include reference to Jobcard / ASBU / work programme item, as applicable

(Reference: Job Card FSMP 07.01 – Develop and maintain SARPs and guidance to prevent WAIC / Radio Altimeter interference) Resolution 424 of the World Radiocommunication Conference 2015 established a new aeronautical mobile route service allocation in the frequency band 4200 – 4400 MHz, reserved exclusively for Wireless Avionics Intra Communications (WAIC), and invited ICAO to develop SARPs for WAIC taking into account Recommendation ITU-R M.2085. As specified in Resolution 424, WAIC cannot cause harmful interference to, nor claim protection from radio altimeters operating in the same band as part of the pre-existing frequency allocation to the aeronautical radionavigation service. These proposed SARPs specify conditions that will assure such safe coexistence between future WAIC installations and radio altimeters. WAIC is a new type of wireless on-board communication serving aircraft functions related to safety and regularity of flight. To-date, wireless data links have been feasible only for non-critical and non-essential aircraft functions. Availability of a new route allocation for WAIC will allow the use of wireless links for a wide range of aircraft systems. Possible applications of WAIC, as discussed in detail in Report ITU-R M.2283, include a variety of sensing, monitoring and control functions. The ability to deploy new sensors without additional wiring will allow introduction of new sensing and monitoring capabilities, leading to increased safety and efficiency of flight. Dissimilar redundant wireless links will make it easier to design safety critical systems by reducing common failure modes in data links. Aircraft reconfiguration and retrofit will be made more efficient when fewer wired data links have to be physically rerouted. Reduction of wire weight will also be consistent with ICAO environmental contributions to Sustainable Development Goals. Other, not yet fully understood benefits of WAIC will likely become apparent once the technology becomes commercially available. In order to realize the many potential benefits of WAIC, ICAO needs to develop and approve SARPs. Per Resolution 424 (WRC-15) WAIC can operate only in compliance with ICAO SARPs. In order to fully invest in technical development of WAIC, the industry must have a high level of certainty about technical requirements that WAIC equipment and networks must satisfy. Therefore, the approval of these SARPs will be a crucial and necessary step towards practical deployment of WAIC systems.

1.2	What is the overall impact of this proposal on the strategic objectives of ICAO, namely:

|  | Positive / Negative / Negligible/None | Rationale: Please provide an explanation for your choice and highlight any caveats or limitations in the selection |
| --- | --- | --- |
| Safety | Positive | WAIC is an enabling technology introducing a new communications medium between aircraft functions. WAIC will enable expanded intra-system communications and the introduction of new sensors/actuators. WAIC-related safety impacts can only be assessed in association with future intended functions. The current level of safety will be at least maintained. Depending on the actual function WAIC is used for, safety may be improved for particular circumstances, e.g., by allowing dissimilar redundancy or adding new safety functionalities. |
| Security | Negligible/None | WAIC Systems do not provide communication between two aircraft. Specific equipment security capabilities and system implementations are defined in the WAIC MOPS to protect systems against specific threats. |
| Environment | Positive | Enhanced sensing, without additional weight penalty of new wiring, will enable more energy-efficient operation of aircraft systems, thus reducing the overall fuel consumption and carbon emissions. |
| Efficiency | Positive | Reduction of wiring, with associated harnesses and connectors, will simplify the design effort needed to add and route new wiring, and will reduce assembly and installation labor. New sensing and monitoring functions made possible by WAIC will facilitate predictive and condition-based maintenance, reducing gate delays and improving air transport efficiency. |

Note: In the following questions ‘States’ applies to the adoption and oversight of new SARPs. ‘Industry’ applies to the service provision and use, whether State owned or not (e.g., ANSPs, airlines aerodromes, meteorology, general aviation, etc.). With respect to financial costs for States, it refers to the cost to develop, implement, maintain, and consider oversight issues associated with the proposed change. For Industry, it refers to the cost of implementing the change, where compliance is required by the State, which may translate in costs for equipage, human resources, training, documentation, aircraft modifications or upgrades, operations and airworthiness for example.

1.3	What is the overall impact on resources (financial, personnel, etc.) of this proposal for:

|  | Increase/decrease/negligible/unknown | Rationale: Please provide an explanation for your choice and highlight any caveats or limitations in the selection |
| --- | --- | --- |
| States | Negligible/No impact | No changes to existing systems or infrastructure will be needed when WAIC Systems are introduced. WAIC involves communication only between points on a single aircraft, and does not require any new infrastructure. |
| Industry | Negligible/No impact | The proposed WAIC Systems will be implemented on new aircraft being delivered from the aircraft manufacturers and also potentially on retrofit aircraft, with no impact on other aircraft that does not use WAIC. |

1.4	In your opinion, do the benefits of this proposal justify the cost of implementing the proposal from the perspective of:

|  | Answer | Rationale: Please provide an explanation for your choice and highlight any caveats or limitations in the selection |
| --- | --- | --- |
| States | Yes | WAIC will allow design of more efficient and intelligent aircraft systems with enhanced sensing and monitoring functionalities, facilitating technical progress in aviation and improving efficiency of air travel. |
| Industry | Yes | The new AM(R)S allocation for WAIC was established due to industry request. The availability of SARPs for WAIC will allow taking advantage of the many potential benefits of WAIC. |

# PART 2: IMPLEMENTATION PLAN

To assist ICAO and States ensure this proposal will be effectively implemented please answer the following questions.

Note: The ANC recognizes that panel experts may feel limited in their ability to answer some or all of these questions, however, encourages the panels to provide their views. If still unsure, it is acceptable to leave one or more blank. The answers presented to the ICAO Council with the proposed amendment will be further developed by ICAO.

2.1	What supporting documentation is required for this proposed amendment?

Please include reference to any documents that require initial release/amendment e.g., ICAO Document or Circular name and number, industry specification, etc.

- Supporting material in Doc 9718 Vol I. - EUROCAE/RTCA documents ED-260A/DO-378A.

2.2	What other guidance, training and support activities do you recommend ICAO undertake to ensure the effective implementation of this proposed amendment?

Please include reference to any existing support/promotional programmes and whether it is required globally or regionally e.g., regional seminars, ikits, etc.

- Awareness of new SARPS provided through Regional Spectrum and WRC preparatory workshops.

2.3	What are the essential steps to be followed by a State in order to implement this proposed amendment?

Please include the major steps e.g., amendment of national legislation, change of oversight procedures, training of oversight personnel, required competencies, etc.

- National radio frequency spectrum regulations need to be adapted to reflect the changes in the Radio Regulations for WAIC agreed by WRC-15 for WAIC.

2.4	What is the timeframe needed to implement this proposal by:

|  | Answer | Rationale: For the State, the timeframe is the length of time needed to implement in the national regulatory framework For industry, the timeframe is the length of time needed for industry to start implementing in their operations |
| --- | --- | --- |
| States | 1 - 2 Years | National radio frequency spectrum regulations need to be adapted to reflect the changes in the Radio Regulations for WAIC agreed by WRC-15 for WAIC. |
| Industry | 1 - 2 Years | The lack of SARPs is the main factor preventing the industry from developing and deploying WAIC solutions. Once these SARPs are approved, the expectation within the industry is to have first WAIC solutions available within 2 years. |

# PART 3: AUDIT PLAN

Note: This section will be completed by ICAO prior to the presentation of any proposed changes to SARPs or PANS. The Panel Secretary will coordinate with the relevant experts in ICAO.

3.1	Does this proposal require an amendment of the USOAP CMA protocol questions to assess effective implementation by States?

Please include reference to existing PQs that may need amendment or description of any new PQs that may be required. State ‘Not applicable’ if no impact

- No changes are envisaged in the protocol questions.

ANNEX 3

Validation Statement

Proposed Text to be Submitted to ANC

WAIC SARPS VALIDATION REPORT

- INTRODUCTION

  - Wireless Avionics Intra-Communications (WAIC) is a new class of wireless data links intended for communication between various functions on-board a single aircraft. A new aeronautical mobile route service allocation for WAIC within the radio frequency band of 4 200 – 4 400 MHz was established by the 2015 World Radiocommunication Conference (WRC-15).

  - The same radio frequency band includes an aeronautical radionavigation service allocation reserved for radio altimeters. The new allocation for WAIC requires that WAIC cannot cause harmful interference to nor claim protection from radio altimeters. Therefore, in development of SARPs for WAIC the main technical task was to establish and validate conditions under which WAIC shall not harmfully interfere with altimeters.

  - This report summarizes studies that were undertaken to characterize susceptibility of radio altimeters with respect to interference from WAIC and to validate technical requirements to be included in WAIC SARPs. The particular technical parameter under consideration was the total radiated power emitted from a WAIC system on board a single aircraft. A series of experimental studies investigated the highest power that may be emitted from a WAIC-equipped aircraft that allows normal operation of altimeters on other aircraft. Results of those studies were presented to and analyzed by FSMP.

  - Prior to finalization of these SARPs, the EUROCAE Working Group 96 and RTCA Special Committee 236 jointly developed a Minimum Aviation System Performance Standard (MASPS) for WAIC, which is now published as ED-260A and DO-378A. That joint committee reviewed the same experimental study and confirmed its validity. The technical specifications in WAIC SARPs are consistent with conditions specified within this WAIC MASPS.

- WAIC POWER LIMIT

  - Resolution 424 (WRC-15) invited ICAO to take into account Recommendation ITU-R M.2085 “Technical conditions for the use of wireless avionics intra-communicaiton systems operating in the aeronautical mobile (R) service in the frequency band 4 200 – 4 400 MHz” when developing WAIC SARPs. That Recommendation specifies that the maximum equivalent isotropically radiated power (EIRP) spectral density generated by a WAIC system installed on board a single aircraft must not exceed 6 dBm/MHz, or equivalently 4mW/MHz. The WAIC SARPs recommends that the power of the total emissions of all WAIC transmitters on board an aircraft shall not exceed an equivalent isotropic radiated power of -20dBm assuming a point source located at the geometrical center of the aircraft, which complies with the limit in Recommendation ITU-R M.2085.

  - ED-260A and DO-378A adopt the total EIRP limit for WAIC systems and provide a practical verification procedure how to ascertain whether the limit is satisfied.

  - The validation study summarized herein took as the basis the power limit from ITU-R M.2085. The goal was to confirm experimentally whether that limit assures safe operation of actual radio altimeter implementations aboard other aircraft under worst case operational scenarios. Thus, the much lower limit provide by the WAIC SARPs will provide an even greater margin to prevent interference.

- INTERFERENCE SUSCEPTIBILITY STUDY

  - The experimental study was performed by the Aerospace Vehicle Systems Institute (AVSI). The project team included three leading altimeter manufacturers (Honeywell, Rockwell Collins, Thales), two airframers (Airbus, Embraer), as well as equipment manufacturers (Lufthansa Technik, UTC, Zodiac) and NASA specialists.

  - The study was performed at Texas A&M University to enable objective and repeatable testing in an independent academic setting.

  - A collection of commercial altimeters was placed within a calibrated test bench. Controlled interference signals were injected into altimeter receivers’ additive to their return signals.

  - Through analysis and experimental iteration, the project team established the worst-case operational scenario under which multiple WAIC-equipped aircraft generate most severe interference received by altimeters on a victim aircraft. That worst-case scenario involves landing when the multiple WAIC-aircraft aircraft parked or taxiing at the airport, consistent with ICAO regulations for aerodromes, generate the most severe composite interference environment.

  - The study crucially considered interference from multiple altimeters installed on multiple aircraft at the airport, as well as from multiple redundant altimeters aboard the landing (victim) aircraft. That was a primary concern for FSMP, as other altimeters represent the most severe interference source for a victim altimeter, often exceeding the effects of potential interference from WAIC.

  - The study used a set of very conservative assumptions and was thoroughly reviewed by the altimeter experts on the AVSI team. It was also reviewed and accepted by a wider team on the joint WG-96 and SC-236 committee, including experts from EASA and FAA.

  - Details of the study were reviewed by FSMP in form of a series of Information Papers submitted by AVSI.

- VALIDATION OF REMAINING SARPS REQUIREMENTS

  - Additional parameters specified by this SARPs were experimentally found to have no effect on the interference sensitivity of radio altimeters. However, to properly implement WAIC systems, these additional parameters are required to meet international radio frequency spectrum regulation and have been verified by FSMP members as sufficient to satisfy these regulations.

  - Specifically, the unwanted emissions limits were thoroughly discussed to assure compliance with applicable ITU-R recommendations and national regulations. The final version of the emissions mask satisfies all the requirements introduced by FSMP members representing different national regulatory bodies.

  - Out-of-band interference tolerance was similarly thoroughly examined discussed to assure compliance with applicable ITU-R regulations. The final version of the requirement addresses all concerns voiced by FSMP members.

  - The FSMP reviewed and validated these SARPs for regulatory compliance.

- CONCLUSIONS

  - The AVSI study verified that if EIRP spectral density generated by WAIC aboard a single aircraft does not exceed 6dBm/MHz, then performance of altimeters aboard other aircraft will not be negatively affected. Thus, a total EIRP of -20dBm/200 MHz will be well below the threshold for interference.

  - The conclusion holds in most unfavorable worst case operational scenarios, under a set of conservative worst case assumptions.

  - FSMP concludes that the -20 dBm EIRP limit on the total emissions of all WAIC transmitters on board an aircraft is an appropriate requirement for WAIC and satisfies the condition of protecting radio altimeters.

  - All requirements in WAIC SARPs were validated by FSMP for regulatory compliance.