FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

Eighth Working Group meeting

Montréal, Canada, 21 – 29 January 2019

Agenda Item 7:	Interference From Non-Aeronautical Sources

- Programme Making and Special Events (PMSE)

UK Ofcom Safety Assurance Case (SAC)

(Presented by the Secretary)

SUMMARY

In December 2018 the UK Spectrum Regulatory Authority (UK Ofcom) and UK Civil Aviation Authority (UK CAA) released a joint communication indicating UK Ofcom’s decision to operate PMSE within the 960 – 1164 MHz band which is used by aeronautical radionavigation and surveillance systems. This decision is based on the acceptance of UK CAA of a Safety Assurance Case (SAC) developed by UK Ofcom. Prior to UK CAA’s acceptance of the SAC document, the ICAO Secretariat had a brief opportunity to review it and provide comments. In this context it should be noted that the SAC document is not considered equivalent to a Safety Case, which, according to the ICAO and EASA regulatory frameworks has to be developed by the affected Air Traffic Service Providers and approved by the national Civil Aviation Authority. During the Secretariat review it was noted that the SAC was based on a number of assumptions, concluding that it is only valid if those assumptions remain accurate. The Secretariat reviewed the key assumptions contained in the SAC and found some of those not to be accurate. The outcome of this brief review was promptly communicated to the UK CAA. FSMP WG/8 is invited to review and comment on the SAC developed by UK Ofcom (Attachment 1) and the ICAO Secretariat review of it (Attachment 2).

INTRODUCTION

UK Ofcom has for some time been developing a SAC to assess and quantify the risks of introducing the operation of audio PMSE equipment in the 960 – 1164 MHz band in the UK.  This frequency band is used by a number of safety critical aeronautical communication, navigation and surveillance systems, including Distance Measuring Equipment (DME), Secondary Surveillance Radar (SSR) and Airborne Collision Avoidance Systems (ACAS), just to name a few.

In accordance with the ICAO regulatory framework (ICAO Annex 11 “Air Traffic Services”, Annex 19 “Safety Management”, Doc 9734 “Safety Oversight Manual”, Doc 9859 “Safety Management Manual”) the development of Safety Cases is the responsibility of the Air Traffic Service Providers, as being the entities having full knowledge of the use and vulnerabilities of their systems and services.  Attachment 3 contains a few relevant excerpts from the ICAO provisions referenced above.

While the SAC developed by UK Ofcom is not considered the equivalent to a Safety Case, it is understood that Air Traffic Service Providers may be able to use information contained in the SAC when developing their own required Safety Cases.

A copy of the SAC was provided by UK CAA to the ICAO Secretariat in October 2018.  The Secretariat provided comments back, highlighting serious concerns with a number of key assumptions the SAC is based on.  To date, it is not known to which extent those comments were taken into account.

In December 2018, UK Ofcom and UK CAA released a communication, indicating that the CAA has reviewed the SAC and is satisfied that any proposed change to the radio spectrum environment can be implemented and maintained without interference to aeronautical services and that as a result, assignments to PMSE in the 960 to 1164 MHz band will commence in early 2019.

DISCUSSION

The SAC (Attachment 1) developed by UK Ofcom is “predicated on a number of assumptions relating to the environment and type of PMSE use and is therefore valid only if those assumptions remain accurate.” (Ref: Executive Summary of the SAC).

Two “critical assumptions include” (Ref: Executive Summary of the SAC):

“That PMSE users who will utilise equipment capable of tuning to this frequency range are professionals (only the largest events will require such equipment).”

“There will be around 945 assignments of PMSE equipment in the 960-1164 MHz band on a peak day.”

Analysis of the first assumption

The first assumption is made on the basis that PMSE is operated by professional sound stage engineers managing large venues.  However, no evidence is provided that only “professionals” will operate PMSE equipment in the frequency band 960 – 1164 MHz.

Contrary to the assumption, and as evident from the description of PMSE use on the UK Ofcom website, PMSE is also used for other public or private events, at any location, e.g. sporting, music, theatrical, religious, political, hobby and  corporate retailing.

PMSE is used extensively by amateur performers, conferencing personnel and other users which are not required to have the training or skillset expected of professional sound stage managers in order to provide high fidelity reproduction of high profile events.

The typical PMSE users are not frequency spectrum professionals and may not be aware of spectrum management rules, or even the web-based frequency-assignment licensing application as provided by UK Ofcom.

There is no certification of competency or licensing requirement in order to purchase PMSE equipment.  No specific ownership certification or licensing is being proposed for PMSE equipment operating in the 960-1164 MHz band.  Hence anyone will be able to purchase and operate 960 – 1164 MHz PMSE equipment.

In summary, the SAC only addresses the professional users, which may be assumed to possess a certain skillset and knowledge of their equipment as well as the associated frequency management rules and regulations to operate in accordance with the use cases contained in the study.  However since there is no certification requirement for the competency of users, and no licensing required for ownership of the equipment, the expectation clearly cannot be validated that all users will possess the required knowledge and skillset to operate inside of the licensing rules.

The linked Report (page 27), available on the Ofcom website makes reference to “illegal” use.  Success/failure case analysis taking this fact into account is completely missing from the safety assurance study.

Analysis of the second half of the first assumption and the second assumption

Here it is assumed that the 960 – 1164 MHz band will be used as a top-up band only, by loading the existing 470 MHz band to capacity first, hence only a very small portion (7%) of all PMSE assignments would use the frequencies within the 960 – 1164 MHz band.

There is no PMSE equipment ownership licensing or enforcement mechanism being proposed that would steer the loading of the frequency bands.  It can be expected that a significant portion of the existing 700 MHz band equipment slated for removal from PMSE service will be replaced by 960-1164 MHz equipment.  It is also reasonable to expect that a significant portion of new PMSE equipment purchases will target this new band.

The web-based licensing tool does not provide means to steer use towards the 470 MHz band first.  Users would utilize the tuning range of the equipment they have available before seeking additional equipment.  In other words, the distribution between the 470 MHz band and the new 960 – 1164 MHz band would be governed mainly by the proportional ownership of equipment in the two frequency bands.

As evident from the above, it can safely be assumed that a much larger use of the 960-1164 MHz band will be expected than the one postulated in the SAC.

Review of the SAC as a whole

The quick review contained in Attachment 2 is not intended to reassess the assumptions contained in the SAC or the “success cases” and “failure cases” derived from those.  However, it is evident from the review of those assumptions, that there is a problem with a number of the core assumptions and that hence the outcome of the SAC as a whole is in doubt.

ACTION BY THE MEETING

The meeting is invited to:

review the UK Ofcom SAC contained in Attachment 1 to this working paper; and

review and validate the ICAO Secretariat analysis of the UK Ofcom SAC contained in Attachment 2 to this working paper.

- Attachment 1:

- Attachment 2:

- Attachment 3:

— END —