	[image: image7.png]



	[image: image7.png]
International Civil Aviation Organization

WORKING PAPER
	FSMP/WG-9 WP/25
2019-08-26 




ATMRPP-WG/WHL/4-WP/


.././06
FSMP-WG/9 WP/25
- 8 -

- 3 -
FSMP WG/9 WP/25

FREQUENCY SPECTRUM MANGEMENT  PANEL (FSMP)
NINTH WORKING GROUP MEETING
Montreal, Canada, 22 to 30 August 2019
Agenda Item 9c..:
Revision of the ICAO Frequency Spectrum Handbook (Doc 9718)

Volume B - Spectrum
Spectrum Guidance for the Development of Aeronautical systems 
(Presented by John Mettrop)

	SUMMARY

	This document proposes that the text contained in the attachment to this document be included in Volume B Section 5 of the New ICAO Frequency Spectrum Handbook.


1. INTRODUCTION

1.1 The development of a new ICAO Frequency spectrum Handbook to replace Doc 9718 (existing handbook) has opened the opportunity to include additional or expand on existing elements that experience, since the publication, of the current handbook has shown would be useful to the aviation community. This document makes a proposal for the inclusion of text addressing the process for addressing new aeronautical or non-aeronautical , system specific, proposals for sharing/adjacent band co-existence.

2. DISCUSSION

2.1 As acknowledged by this panel, spectrum is a finite resource that is in high demand from a number of industries including aviation. The existing frequency bands that are allocated to aeronautical services tend to be in what is regarded as the sweet spot in the spectrum where the trade of between range, power and capacity is close to optimal for a number of applications. Additionally aeronautical systems are perceived as in-efficient users of that spectrum and therefore a number of industries and radio regulators believe there is scope for sharing in or adject frequency band co-existence with a number of  frequency bands allocated to aeronautical services and this group has discussed a number of examples of this. 
2.2 There is therefore increasing pressure coming from both aeronautical and non-aeronautical sources to accept additional/new shaing of frequency bands allocated to aeronautical systems or the increased use of adjacent frequency bands. The response to such proposals has been varied and in many cases un-coordinated internationally with little guidance material on which authorities can base decisions as to how to proceed. It is therefore intended to include in the new frequency handbook a section that provides guidance on the process that should be followed in addressing such proposals 

3. ACTION BY THE MEETING
3.1 The meeting is invited to:

a) note and review the contents of this working paper.
b) Incoporate the attachment to this document into the Volume B of Section 5 of the draft new Frequency Spectrum Handbook

c) Contribute to the further development of this section of Volume B of the new Frequency spectgrum Handbook.

— END —

Attachment
Suggested text for the New Version of the ICAO Spectrum Handbook – Vol. B, Section 5
SECTION 5

Process for Consideration of Sharing in Frequency Bands used by or Adjac ent to 
Aeronautical Safety Systems Outside of the world Radiocommunication Conference Process.
5.1 
Introduction

This process provides guidance on how to address a proposal for the introduction of a new or modified system either in or adjacent to a frequency band allocated to one or more aeronautical services.  The process applies to applications made either on behalf of an aeronautical or non-aeronautical system.
5.2
Overview

The process, as illustrated below in principle the same whether it is being applied to an aeronautical or non-aeronautical proposal.  However, there are variances, as indicated in some of the process steps, that reflect the differences in responsibilities and powers between radio regulators and aeronautical authorities.
Each step in the process is described, and where appropriate expanded upon in sections 5.3 – 5.14

[image: image1.emf]Proposal made for  in/adjacent 

frequency band use 

Theoretical study

Harmful interference 

predicted?

Practical testing

Co-existence feasible & 

viable?

Develop mitigation solutions/

planning criteria

Reduction in safety?

Develop/modify equipment 

standards

Socio-economic 

value offsets reduction

 in safety?

Develop safety support argument

Implement & monitor

Reject proposal

Yes

Yes

Yes

No

No

No

No

Potential

Yes with 

mitigation

Yes without 

mitigation


5.3
 Proposal Made for In/Adjacent Frequency Band Use
	Aeronautical
	Non-Aeronautical

	
[image: image2.emf]Proposal for a new 

aeronautical system

In an appropriately 

assigned existing 

aeronautical 

frequency band?

Initial impact 

assessment

Potential to affect 

use of aeronautical 

systems?

Seek a WRC agenda 

item

No

No

Yes

Yes

Select desired frequency 

band


	
[image: image3.emf]Requirement for a new/

modified allocation 

identified

In/adjacent to 

an aeronautical 

frequency band?

Initial impact 

assessment

Potential to affect 

use of aeronautical 

systems?

No furthe action

No

No

Yes

Yes



	
	

	
	


5.4
Theoretical Study
5.5
Harmful Interference Predicted?

5.6
Practical Testing

5.7
Co-existence Feasible and Viable?

5.8
Develop Mitigation Solutions/Planning Criteria

5.9
Reduction in Safety?

5.10
Socio-Economic Value Offsets Reduction in Safety?

5.11
Develop/Modify Equipment Standards

5.12
Develop Safety Support Argument

	Aeronautical
	Non-Aeronautical

	
	
[image: image4.emf]Confidence in 

agreement & safety 

support argument 

Develop safety support 

argument

No

Yes

Develop  agreement on 

the operation of the 

new system

1

To implement

 &monitor



	
	

	
	


5.13
Implement and Monitor

	Aeronautical
	Non-Aeronautical

	
[image: image5.emf]Implementation 

operating within 

agreed bounds?

Review implementation

No

Yes

Implement new/

modified system

Remedial action 

possible?

Develop & implement 

mitigation

Yes

Rethink requirement/

solution

No


	
[image: image6.emf]Implementation 

operating within 

agreed bounds?

Review implementation

No

Yes

Implement proposed 

change

Remedial action 

possible?

Develop & implement 

mitigation

Yes

Radio regulator  

implement 

proposed change?

Develop aeronautical 

mitigation necessary to 

maintain safety

Implement mitigation Drop proposal

1

From develop 

safety support 

argument

No



	
	

	
	


5.14
Reject Proposal

— END —
(8 pages)


Proposal for a new aeronautical system
In an appropriately assigned existing aeronautical  frequency band?
Initial impact assessment
Potential to affect use of aeronautical systems?
Seek a WRC agenda item
No
No
Yes
Yes

Select desired frequency band



Confidence in agreement & safety support argument
Develop safety support argument
No
Yes
Develop  agreement on the operation of the new system
1
To implement
 &monitor



Implementation operating within agreed bounds?
Review implementation
No
Yes
Implement new/modified system
Remedial action possible?
Develop & implement mitigation
Yes
Rethink requirement/solution
No



Implementation operating within agreed bounds?
Review implementation
No
Yes
Implement proposed change
Remedial action possible?
Develop & implement mitigation
Yes
Radio regulator  implement proposed change?
Develop aeronautical mitigation necessary to maintain safety
Implement mitigation
Drop proposal
1
From develop safety support argument
No



Requirement for a new/modified allocation identified
In/adjacent to  an aeronautical  frequency band?
Initial impact assessment
Potential to affect use of aeronautical systems?
No furthe action
No
No
Yes
Yes



Proposal made for  in/adjacent frequency band use
Theoretical study
Harmful interference predicted?
Practical testing
Co-existence feasible & viable?
Develop mitigation solutions/planning criteria
Reduction in safety?
Develop/modify equipment standards
Socio-economic  value offsets reduction  in safety?
Develop safety support argument
Implement & monitor
Reject proposal

Yes
Yes
Yes
No
No
No
No
Potential
Yes with mitigation
Yes without mitigation



