FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

Ninth Working Group meeting

Montréal, Canada, 22 – 30 August 2019

Agenda Item 8:	Interference from Non-Aeronautical Sources

...

b) Mobile service communications adjacent to 1 518 MHz satellite communications frequency band

Proposals on the development of guidelines that may be used by spectrum regulators to provide protection of aeronautical L-band Mobile Earth Stations (MES) from the introduction of IMT/LTE in the frequency band below 1 518 MHz.

(Presented by Kamlesh Masrani, Inmarsat Global Limited.)

SUMMARY

This paper provides information on the status of studies within the ITU-R Working Party 4C and 5D groups to address the potential for interference to Mobile Earth Stations (MES) operating in the frequency band 1 518-1 559 MHz, from the planned introduction of IMT/LTE transmissions in the frequency band 1 492-1 518 MHz. Action: - FSMP WG/9 is invited to review the protection levels being considered within the ITU-R WP 4C and 5D groups, along with the protection levels specified within ECC Report 299. - FSMP WG/9 is also invited to review the proposed revisions to the general guidelines being developed within ICAO FSMP, to ensure the protection of aeronautical L-band MES at airports from the potential for interference arising from the introduction of IMT/LTE transmissions below 1 518 MHz (as identified in ICAO FSMP Action 08-06). - FSMP WG/9 is invited to consider sending responses to the liaison statements sent to ICAO by the ECC regarding ECC Report 299 and the plans for the introduction of new MSS terminals with improved resilience, and by ITU-R WP 5D on the work being conducted in the ITU-R.

INTRODUCTION

At the Eight Working Group Meeting (WG/8) of the Frequency Spectrum Management Panel (FSMP) in Montréal, Canada, over 21-29 January 2019, the meeting agreed to continue the development of general guidelines to be included as a part of the Handbook on the protection of aeronautical L-band MES.  The meeting also identified that Annex 3 of Working Paper 16 from FSMP WG/8 may be used as a suitable framework in the development of such guidelines (see ICAO FSMP Action 08-06).

This paper proposes revisions to the limits contained in Annex 3 of FSMP-WG/8 Working Paper 16 on the protection of aeronautical L-band MES.  The proposed revisions are based on the latest outputs available from ITU-R Working Party 4C and 5D on the development of a Preliminary Draft New Recommendation and Report on MSS and IMT L-band compatibility.

It is noted the work within the ITU-R is still on-going and will be carried forward to the next ITU-R study cycle.  However, given that the status of the studies are sufficiently complete to identify the protection measures necessary to facilitate compatibility at airports, between IMT systems planned below 1 518 MHz and aeronautical MES operating above 1 518 MHz, it is proposed to progress the development of ICAO FSMP guidelines based on the protection measures that are considered as appropriate from an aviation perspective.  Such measures would encompass the protection of currently operating aeronautical L-band MESs at airports in the frequency band 1 518-1 559 MHz (including those providing aeronautical safety communications within the frequency band 1 525-1 559 MHz), as well as future, less stringent, protection requirements at airports resulting of the introduction of next generation aeronautical MES designed to have a higher receiver blocking tolerance of high power transmissions in adjacent frequency bands.

DISCUSSION

Following the last FSMP Working Group Meeting in January 2019, ITU-R WP 4C met in June 2019 and WP 5D met in February 2019 and July 2019.  Unfortunately, the meetings were unable to complete the development of the Draft new Recommendation and Report ITU-R M.[REP/REC MSS & IMT L-BAND COMPATIBILITY], and these documents, along with other relevant documents, are carried forward for further consideration to the next meetings of WP 4C and WP 5D in the next WRC study cycle.  An extract of the WP 5D meeting report from the July 2019 meeting summarising the status of discussions on MSS and IMT L-band compatibility, together with a copy of the latest revision to the draft new Recommendation and Report being developed is provided Annex 1.

By way of summary, the WP 5D meeting report identifies the status of the PDN Report ITU-R M.[REP MSS & IMT L-BAND COMPATIBILITY] may be considered as stable and ready for approval.  However, a few items remain where different views were expressed which could not be resolved in time at the WP 5D July 2019 meeting.  For the PDN Recommendation ITU-R M.[REC MSS & IMT L-BAND COMPATIBILITY], WP 5D identifies: a number of concerns that were expressed; that the document still contains a large number of square brackets and Editor’s Notes; and only around half of the document has so far been reviewed by WP 5D. WP 5D therefore considered not to elevate the document to a preliminary draft new Recommendation and has updated its work plan accordingly.

Also, it should be noted that WP 5D was unable to reach a consensus to approve the draft Revision of Recommendation ITU-R M.1036-5 on frequency arrangements for implementation of the terrestrial component of International Mobile Telecommunications (IMT) in the bands identified for IMT in the Radio Regulations (RR).

On the status of the Revision of Recommendation ITU-R M.1036-5, three significant issues are identified by WP 5D as needing to be resolved, which are:

- how to treat the operation of IMT systems in frequency bands allocated to the mobile service other than those identified in the RR;

- the identification of the frequency arrangements for IMT in the frequency band 1 427-1 518 MHz, and whether they should be included in the draft revision to ITU-R M.1036-5 or included as a part of the next revision of the Recommendation, following the finalization of the ITU-R documents on compatibility between IMT and MSS at L-band; and

- the identification of frequency arrangements in the frequency bands 1 980-2 010 MHz and 2 170-2 200 MHz, which have been identified for both the terrestrial component of IMT and the satellite component of IMT, pending completion of studies on the mitigation techniques to avoid harmful interference between the two components.

Resolution of the Bullet 2 issue above is relevant to the development of guidelines within ICAO on MSS & IMT L-band compatibility since it relates to the frequency arrangements for IMT in the frequency band 1 492-1 518 MHz and what additional frequency separation or guard band (e.g. between 0 MHz to 6 MHz) below 1 518 MHz is appropriate, together with any additional measures that administrations may consider necessary to facilitate the adjacent band compatibility by IMT with MSS above 1 518 MHz.

WP 5D agreed to submit the draft Revision of Recommendation ITU-R M.1036-5 to the Study Group 5 meeting scheduled for September 2019, in order for these remaining issues to be addressed and potentially the revision to then be considered for approval under Resolution ITU-R 1-7.  See Annex 2 for a copy of the draft revision, together with an extract of the text concerning the proposed frequency arrangements for IMT in the frequency band 1 427-1 518 MHz.

PROPOSAL

As WP 4C and WP 5D have not finalised their work on IMT/MSS compatibility at L-band, and noting the next meeting of WP 5D is scheduled for February 2020, while the next meeting of WP 4C is scheduled in May 2020, it is proposed that FSMP progresses the work on the development of general guidelines within ICAO FSMP on the protection of aeronautical systems at airports based on the current status of studies available within the ITU-R and CEPT.  It is not anticipated the overall technical conclusions of such studies will change as a result of any further study.

To this end, Annex 3 contains proposed edits for consideration within FSMP updating the draft guidelines.  The proposals are made using revision marking from the limits and protection measures identified in Annex 3 of Working Paper 16 from FSMP WG/8.

It is further noted FSMP has received two liaison statements (provided in FSMP-WG09-WP05) from the Electronic Communications Committee (ECC) of The European Conference of Postal and Telecommunications Administrations (CEPT) and from ITU-R Working Party 5D on the issue of potential blocking of Mobile Earth Stations operating in bands adjacent to 1 518 MHz (including 1 525-1 559 MHz) at sea ports and airports.  These liaison statements relate to guidelines that may be developed within FSMP and, as such, would need to be duly considered by the meeting.

It is proposed that FSMP may consider responding to the ECC, thanking them for their liaison statement, and noting that the duration of identified short time period for the first phase (e.g., 7 years) that is intended to protect currently operating aeronautical L-band MES is not considered sufficient from an aeronautical perspective given the high costs involved changing aviation equipage.  It may be noted the relevant standardisation bodies are in the process updating the adjacent channel selectivity requirements for aviation receivers in order to tolerate better the operation of high power adjacent channel signals.

It is also proposed that any response to the ECC includes the following sentiments: ICAO is pleased to see the development of example PFD limits for protection of aeronautical MESs, noting that those in Section A2.2 (based on measured performance of aeronautical L-band MES receivers) provide adequate protection to existing aircraft MESs for single and multiple LTE channels.

It is proposed that FSMP may considering responding to WP 5D, in particular to highlight the need for protection of aeronautical MESs used for safety related applications in the band 1525-1559 MHz.

ACTION BY THE MEETING

The meeting is invited to:

note and review the contents of this working paper;

continue the work on FSMP Action 08-06 reflecting the latest available output from the ECC, as well as the ITU-R studies;

consider a response to ECC on their liaison statement contained in FSMP-WG09-WP05, and noting that the duration of identified short time period for the first phase (e.g. 7 years) that is intended to protect currently operating aeronautical L-band MES is not considered sufficient from an aeronautical perspective given the high costs involved changing aviation equipage.  It may be noted the relevant standardisation bodies are in the process updating the adjacent channel selectivity requirements for aviation receivers in order to tolerate better the operation of high power adjacent channel signals; and

considering responding to ITU-R WP 5D, in particular to highlight the need for protection of aeronautical MESs used for safety related applications in the band 1525-1559 MHz.

Annex 1.  Report and working documents under development within ITU-R WP 4C and WP 5D on potential for interference from IMT transmissions below 1 518 MHz to MES receivers operating above 1 518 MHz.

- Attach:

- WP5D (July 2019) Meeting Report Extract;

- PDN Recommendation MSS - IMT L-band; and

- PDN Report MSS - IMT L-band.

- Annex 2.  Draft revision to ITU-R M.1036-5 on IMT frequency arrangements that will be considered at the September 2019 meeting of Study Group 5.

- SG 5 at its meeting in September 2019 will need to decide whether to include the following frequency assignments of IMT systems in the band 1 427-1 518MHz within a revision to ITU-R 1036-5, or to consider the arrangements within the next revision.

- Extract of frequency arrangments for consideration in Section 4 of ITU-R M.1036-5, covering the frequency band 1 427-1 518 MHz.

“The recommended frequency arrangements for implementation of IMT in the band 1 427-1 518 MHz are provided in Table 4 and in Fig. 4, noting the implementation aspects in section 1 above.

TABLE 4

Frequency arrangements in the band 1 427-1 518 MHz

| Frequency arrangements | Paired arrangements (FDD) |  |  |  | Un-paired arrangements (TDD) (MHz) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mobile station transmitter (MHz) | Centre gap (MHz) | Base station transmitter (MHz) | Duplex separation (MHz) |  |  |
| G1 | External | – | 1 427-1 517 | – | None |
| G2 | 1 427-1 470 | 5 | 1 475-1 518 | 48 | None |
| G3 |  |  |  |  | 1 427-1 517 |

NOTE 1 – With respect to IMT in the frequency band 1 492-1 518 MHz and the MSS in the frequency band 1 518-1 525 MHz, ITU-R studies were conducted in accordance with Resolution 223 (Rev.WRC-15) and provide possible technical measures to facilitate adjacent band compatibility. Frequency arrangements in this band take into account the results of these studies.

Based on these studies, administrations may consider additional frequency separation below 1 518 MHz at the upper part of G1, G2, or G3 (e.g. a total separation of 0 MHz to 6 MHz). This is one of a number of possible measures to facilitate adjacent band compatibility. (See Report ITU-R M.[REP.MSS & IMT L-BAND COMPATIBILITY] [and Recommendation ITU-R M.[REC.MSS & IMT L-BAND COMPATIBILITY]]).”

fIGURE 4
(see notes to Table 4)

- Attachment: Input to SG5 on Draft Revision to ITU-R Rec M.1036-5

Annex 3.  Draft guidelines that may be considered by spectrum regulators to ensure the protection of aeronautical MES receivers operating in the frequency band 1 518-1 525 MHz, from the introduction of IMT/LTE in the frequency band below 1518 MHz.

[Editors note.  A summary of the key outstanding issues to be addressed within WP 4C and 5D (as identified in the draft PNDR Report and Recommendation on IMT/MSS sharing at L-band) is provided below.  It is copied for information for ICAO FSMP to assist in the development of any guidelines on protection limits for aeronautical MES receivers.

| Issue | Aspects for consideration within WP 4C & 5D |
| --- | --- |
| Guard band | • Need to specify whether from IMT or MSS, or both. • Need to suggest specific values in range 0-6 MHz. • Should be consistent with Recommendation ITU-R M.1036. |
| Base station emission limits (standard and enhanced?) | • Ideally only a single option, but if multiple options, need to specify the applicable scenario for each. • Maximum frequency for unwanted emission limits TBD. • Need feedback from WP 5D on feasibility of meeting limits. |
| UE emission limits | • Ideally only a single option, but if multiple options, need to specify the applicable scenario for each. • Maximum frequency for unwanted emission limits TBD. • Need feedback from WP 5D on feasibility of meeting limits. |
| Protection measures for ship earth stations and aircraft earth stations | • Need for additional protection measures for ports/airports • May include pfd values and other measures (to be based on Report). |
| IMT/MES equipment improvements | • Should be a recommendation on MES blocking, possibly with options. • Need to talk about timing of implementation for MES. • Need to talk about timing of improvement in IMT UW emissions. |

]

Introduction

To be developed.

[Editor’s Note: general introductory text describing the use of the band 1 518-1 559 MHz (including the provision of aeronautical safety communications within the frequency band 1 525-1 559 MHz) by aeronautical MES, and the testing of terminal functions correctly when the aircraft is on the ground is to be included.]

- Measured blocking levels of most susceptible terminal identified within Draft ECC report 299.

[Editor’s note:  it is proposed to delete the results of measurements contained in Draft ECC Report 299 in order to simplify the guidelines.  Instead, reference on the blocking performance of the most susceptible aeronautical L-band MES receiver that was measured is included alongside the protection PFD limits identified below.]

- IMT Base Station PFD Limits to protect aeronautical MES from receiver blocking

The recommended approach to protection of aeronautical L-band MES is through the application of power flux density (PFD) limits.  The geographic area to be protected should be defined, which is normally the area within an airport when aircraft are at the gate, taxing, or on the runway.  The PFD produced by IMT base stations must be kept below the PFD limit value at the boundary of the protected area and any point inside the area.  Operators of IMT base stations located outside the protected area will need to assess the PFD at the boundary of, and inside, the area to ensure that the PFD limit is not exceeded.  The PFD approach allows for the mobile operator to take into account the actual IMT base station characteristics (such as antenna height and downtilt) and the actual propagation loss (e.g. taking account of local clutter).  In some cases, it may be possible for operators to deploy IMT base stations inside buildings within the defined area (e.g. inside the airport terminal) while still ensuring the PFD limits outside the building is met, when the building wall loss is taken into account.

[Editor’s Note: A figure to illustrate the application of the PFD limit is to the added.]

It is anticipated that national licensing authorities will ensure that the protected areas and associated PFD limits are defined and included in the IMT operator’s licence conditions.  This should be done in consultation with the relevant national aviation safety representatives and/or with the airport operator.

The PFD limits in this section are based on the blocking measurements of the most susceptible terminals, as performed by some Satcom manufacturers and presented in ECC Report 299.  The blocking criterion for current aeronautical L-band MES receiver design with respect to IMT base stations operating a single IMT channel transmission in the frequency band 1 512-1 517 MHz is -50.3 dBm, while the blocking criterion with respect to IMT base stations operating multiple IMT channel transmissions is -55.5 dBm.  For next generation aeronautical L-band MES receivers that are expected to be more resilient to blocking effects and hence would lead to more relaxed constraints on IMT base stations, the blocking criterion with respect to IMT emissions in the band 1 512-1 517 MHz is -30 dBm.

The PFD limits are identified in two parts:  Phase 1 limits are derived for the protection of existing aeronautical L-band MES equipment, while Phase 2 limits are derived based on the protection requirements for next generation aeronautical L-band MES equipment.

Note: the PFD limits identified in Tables 1 and 2 are based on a MES with an assumed antenna gain values towards the horizon of either 3 dBi or 17 dBi.  The limits are replicated from Annex 1 of PDN Recommendation ITU-R M.[REC.MSS & IMT L-band COMPATIBILITY].

The protection limits for aeronautical MES with an assumed maximum gain of 3 dBi towards the horizon are identical to the PFD levels specified in Annex 2, Option 2 within ECC Report 299 (i.e., based on measured blocking performance of aeronautical L-band MESs).

For cases where the aeronautical L-band MES antenna gain towards the horizon can exceed 3 dBi, in particular when such aeronautical MES have high gain antennas (with maximum gain up to 17 dBi) and are used in airports having a low elevation angle towards the associated MSS satellite, the lower PFD limits derived for an assumed gain of 17 dBi towards the horizon would be appropriate as an aeronautical MES protection requirement.

Table 1

PFD limits for IMT BS with single IMT channel transmission

| Phase |  | Phase 1 |  |  | Phase 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | MSS terminal antenna gain (dBi) | PFD limit for BS emissions in the band 1 492-1 502 MHz (dBW/m2) | PFD limit for BS emissions in the band 1 502-1 512 MHz (dBW/m2) | PFD limit for BS emissions in the band 1 512-1 517 MHz (dBW/m2) | PFD limit for BS emissions in the band 1 492-1 502 MHz (dBW/m2) | PFD limit for BS emissions in the band 1 502-1 512 MHz (dBW/m2) | PFD limit for BS emissions in the band 1 512-1 517 MHz (dBW/m2) |
| Airports | 3 | -28.9 | -42.9 | -58.2 | No limit required | -27.9 | -37.9 |
|  | 17 | -42.9 | -56.9 | -72.2 | No limit required | -41.9 | -51.9 |

Table 2

PFD limits on IMT BS with multiple IMT channel transmissions

| Phase | MSS terminal antenna gain (dBi) | Phase 1 |  | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | PFD limit for emissions in the band 1 492-1 512 MHz (dBW/m2) | PFD limit for emissions in the band 1 512-1 517 MHz (dBW/m2) | PFD limit for emissions in the band 1 492-1 512 MHz (dBW/m2) | PFD limit for emissions in the band 1 512-1 517 MHz (dBW/m2) |
| Airports | 3 | -53.5 | -63.4 | -30.9 | -40.9 |
|  | 17 | -67.5 | -77.4 | -44.9 | -54.9 |

Notes: the maximum PFD should be considered within airport in places where aircraft are likely to be positioned and calculated at height of 10 m above local ground level.

[Editors note. Further guidance may need to be provided on the propagation model and clutter to be used in calculating the PFD.  The time percentage associated with the propagation model may be considered to be for attenuation not exceeded for 20% of time, e.g., equating to long term propagation conditions, to avoid the need to consider short term propagation enhancements.]

Recommended time lines in the application of Phase 1 limits and the transition to Phase 2 protection limits.

It is important the Phase 1 PFD limits protect existing aviation equipment and the transition to Phase 2 PFD limits be conducted at a date that is supportable by aviation equipage schedules.  Member States should discuss appropriate timelines for protection measures with their national aeronautical industries (including the airlines and their trade bodies) to assess the potential impact, cost and the best way forward, so that they can take the appropriate steps with no unnecessary delay to reduce the duration of the Phase 1 PFD limits to an acceptable and reasonable timeframe.

By way of general guidance, it is recognised that the longstanding position of airlines and operators has been to accommodate updates to aircraft systems as part of the natural lifecycle of airframes and avionics. Such a timeframe is normally between 20-30 years for each aircraft, given the significant capital cost of purchasing or changing an operational aircraft.  Any changes to aeronautical L-band satellite communication equipment requires updates to the performance standards, procedures, regulations and guidelines which govern air transport.  Design of new equipment must be developed to the new aviation standards and be certified as safe for use by a competent authority.  Once new equipment design has been certified, it has to be incorporated into the production of new aircraft for line-fit delivery, mandated for installation and made available to the airline industry.  For retrofit installations, a supplementary type certificate (STC) must be obtained from EASA/FAA for each type and variant of aircraft.  Currently there are no certified aeronautical L-band satcom terminals available that comply with the new receiver blocking requirement of -30 dBm from mobile-base station transmissions at 1 512-1 517 MHz.

- Conditions on unwanted emissions for mobile base station transmissions in the 1 492-1 517 MHz.

In addition to receiver blocking effects, there is a potential for interference associated with the unwanted emissions of IMT base stations.  Hence, it is necessary to define the maximum level of unwanted emissions from IMT base station transmissions falling within an aeronautical MES receiver pass band.

It is noted that ECC has specified unwanted e.i.r.p density limits from base station transmissions, per sector in a multi-sector site, falling within the band 1 518-1 520 MHz of -0.8 dBm/MHz, and within the band 1 520-1 559 MHz of -30 dBm/MHz, and with a maximum in band e.i.r.p of 58 dBm per cell for base-station transmissions operating in band 1 512-1 517 MHz.

[ICAO FSMP] considers these limits are not sufficient to protect MES receivers as it results in an unacceptable potential for interference into the 1 518-1 559 MHz frequency band.

[ICAO FSMP] recommends unwanted e.i.r.p density of out-of-band emissions from IMT base stations operating in the frequency band 1 492-1 518 MHz are no higher than -41 dBm/MHz into the frequency bands 1 518-1 525 MHz and 1 525-1 559 MHz.  This unwanted emissions limit is in line with Option B of recommends 2 of [Working Document towards a Preliminary Draft New] Recommendation ITU-R M.[REC.MSS & IMT L-band COMPATIBILITY].  The limit, combined with the geographical separation that can be expected between the aircraft and base stations would ensure the MES receiver degradation due to IMT unwanted emissions is to acceptable levels.

- Identification of a guard band for IMT operation.

The recommended limits for IMT out-of-band emissions will require a guard band to allow for practical filters to achieve the required reduction in base station emissions.  Moreover, a guard band is necessary to allow for future MES receivers to be designed to be resilient to high IMT blocking levels below 1 518 MHz.  A minimum guard band of 3 MHz is recommended as providing a reasonable balance between maximising spectrum for IMT and the practical implementation of the necessary filtering in the base station and in the MES.  The guard band should be applied below 1518 MHz.

The measurements on aeronautical MES receivers contained in ECC Report 299  show aviation MES receivers are more sensitive (by around 15 dB) to interference received from the topmost IMT/LTE channel in 1 512-1 517 MHz relative to use of IMT/LTE channels below 1 512 MHz.  An appropriately-sized guard band below 1518 MHz (e.g., 3-6 MHz) could be an effective means of mitigating (but not entirely eliminating) some interference to aviation MES receivers from new IMT/LTE systems.  Other restrictions in the operation in LTE transmissions in the band 1 512-1 517 MHz within the vicinity of airports, including operation limited to low-power indoor mobile base-station use within airports, also would reduce the potential for interference to aircraft L-band satcom receivers as compared to unrestricted operation of LTE/IMT in channels below 1 517 MHz.

National spectrum regulators may consider not licensing the uppermost channels of the band 1 427- 1518 MHz (and in particular the topmost channel), recognising that in some countries there is limited demand from mobile operators for use of this band in the immediate term.  The uppermost channels may be licensed at a later date, which may also take advantage of the more relaxed “Phase 2” PFD limits anticipated after deployment of improved aeronautical L-band MES.  Avoiding use of the uppermost IMT channels would also ease the IMT base station filter requirements.

TDD UE operation permissible under a TDD operation identified in draft revision to ITU-R Recommendation 1036-5.

- [Editor’s note: in case Study Group 5 approves revision to ITU-R M.1036-5 with a TDD frequency arrangement for IMT operation in the band 1 427-1 518 MHz, the following text is proposed for consideration]:

- It is noted that Draft Recommendation [ITU-R M.1036-6] contains several different frequency arrangements for IMT operation in the 1 427-1 518 MHz band.  It is recommended that administration only allocate the band 1 492-1 517 MHz for IMT base-station transmissions/user equipment reception, in order to prevent the use of mobile equipment transmissions in the band immediately adjacent to 1 518 MHz.  This would restrict the operation of IMT mobile transmissions up to the aircraft door (and potentially inside the fuselage), avoiding potentially unmanaged sources of interference arising from uncoordinated IMT mobile transmissions in bands adjacent to 1 518 MHz.  Frequency arrangement G1 (“SDL”) and G2 (“FDD”) meet this requirement, whereas frequency arrangement G3, which is based on a TDD arrangement would not meet this requirement and is therefore not recommended.

— END —

There is an early WP 5D scheduled but the Working Group on Spectrum Aspects of WP 5D is scheduled to meet next in February 2020.

https://www.ecodocdb.dk/download/8f411ee1-4d48/ECC%20Report%20299.pdf

ECC Decision (17)06 (https://www.ecodocdb.dk/download/4f052b0b-2c6c/ECCDEC1706.pdf)