### ENR 1 GENERAL RULES AND PROCEDURES

#### 1 GENERAL RULES

The air traffic rules and procedures Atlantis FIR conform to Annex 2 and 11 to the Convention on International Civil Aviation and to those portions of the Procedures for Air Navigation Services, Rules of the Air and Air Traffic Services and the Regional Supplementary Procedures applicable to the Region.

#### 2 VISUAL FLIGHT RULES

A VFR flight may only be carried out when flight visibility and distance of aircraft from clouds are equal to or greater than the values specified in the following table:

| Altitude band | Airspace Class | Flight visibility | Distance from cloud |
| --- | --- | --- | --- |
| At and above FL 100 | A, B, C, D, E, F, G | 5 NM | 5000 ft horizontally 1000 ft vertically |
| Below FL 100 and above 3000 ft AMSL, or 1000 ft above terrain, whichever is the higher | A, B, C, D, E, F, G | 3 NM | 5000 ft horizontally 1000 ft vertically |
| At and below 3000 ft AMSL, or 1000 ft above terrain, whichever is the higher | A, B, C, D, E | 3 NM | 5000 ft horizontally 1000 ft vertically |
|  | F, G | 3 NM | Clear of cloud and with the surface in sight |

#### 3 INSTRUMENT FLIGHT RULES

##### Off Route Clearance

During weekends, FRI 1700-MON 0700 (FRI 1600-MON 0600) and night hours, traffic conditions permitting, ATC may clear flights via off route trajectories within controlled airspace, in order to promote economical flight operations

##### Off Route Clearance

##### An IFR flight intending to join controlled airspace in the Atlantis FIR shall make a request for joining clearance on the appropriate frequency at least 10 minutes before the aircraft is estimated to reach the joining position. ATS routes may only be joined over a designated reporting point.

A request for joining clearance shall contain:

- aircraft identification

- aerodrome of departure and destination.

- ATS route.

- joining position and ETA joining position.

- joining level.

##### Wake Turbulence – ATC separation minima

The wake turbulence separation minima applied by ATC are conform to those contained in ICAO Doc 4444-ATM/501 (PANS-ATM) chapter 5.8 and 8.7.3.4.

##### Formation flights

Formation flights along ATS routes within the Atlantis FIR, will be accepted, provided that:

- Aircraft are not carrying passengers for compensation or for hire.

- Prior arrangements have been made between the pilots concerned.

- Prior to the execution of such formation flights pilots have received permission from all ATC units concerned. Permission for Atlantis FIR can be obtained as described in paragraph 1.5 below.

- The formation will be conform the standard formation criteria. (A standard formation is one in which a proximity of no more than 0,5 NM laterally or longitudinally and within 100 ft vertically from the formation leader is maintained by each aircraft in the formation).

- The formation leader shall squawk the assigned transponder code.

- A proper ICAO flight plan has been submitted.

A formation flight will be handled by ATC as a single aircraft, with increased radar separation (1 NM). When individual control is requested, advisory information will be issued to assist pilots in attaining standard ATC separation.

When pilot-reports indicate that standard ATC separation has been established normal ATC clearances will be issued.

Flights with a specific character, requiring special handling by ATC, such as calibration flights, check flights, test flights, etc. must be coordinated at least 24 HR in advance with:

| Post: | Air Traffic Control Operational Helpdesk P.O. Box 752 Atlantis |
| --- | --- |

Subject flights in the Atlantis FIR above FL 245 must in addition be coordinated at least 24 HR in advance with Atlantis FIC by filling the approval form available via the website

##### POSITION REPORTING

###### IFR flights on ATS routes

Position reports shall be made:

- Immediately after a frequency change (initial call).

- Over compulsory reporting points, unless otherwise instructed (see item c).

- As instructed by ATC.

Position reports shall contain:

- Initial call:

  - Aircraft identification.

  - Actual level and, when climbing or descending, cleared level.

- Subsequent reports:

  - Aircraft identification.

  - Position and time.

  - Level, except when the aircraft is in level flight and this level has previously been reported.

In addition after clearance to change level pilots shall report: leaving previously assigned level and reaching cleared level.

##### Communication Failure

###### Flying in IMC

Pilots of an IFR flight in IMC, or not able to comply, shall maintain the last assigned speed and level, or minimum flight altitude if higher, for a period of 7 minutes following:

- the time the last assigned level or minimum flight altitude is reached; or

- the time the transponder is set to code 7600; or

- the aircraft's failure to report its position over a compulsory reporting point;

Whichever is later, and thereafter adjust level and speed in accordance with the filed flight plan.

###### Flying in VMC

The pilot of an IFR flight in VMC shall:

- select transponder code 7600;

- continue to fly in VMC;

- land at the nearest suitable aerodrome; and

- report its arrival time by the most expeditious means to the appropriate ATS unit.

###### Arriving flights

For IFR flights with a destination situated in the Atlantis FIR, communication failure procedures are contained in the paragraph "Instrument approach procedures" for each airport.