### GEN 3.3 AIR TRAFFIC SERVICES

#### 1 INTRODUCTION

The authority responsible for the administration of air traffic services provided for civil aviation is the Organization Air Traffic Control under the authority of the Minister of Infrastructure and Environment.

Post: 	Air Traffic Control

ATLANTIS

Tel:  +00 1 (070) 456 6480

Fax:  +00 1 (070) 456 6750

Email: ATC@ Atlantis-ATC.fish

URL: https://www.Atlantis-ATC.fish

The air traffic rules and procedures in the ATLANTIS FIR conform to Annex 2 and 11 to the Convention on International Civil Aviation and to those portions of the Procedures for Air Navigation Services, Rules of the Air and Air Traffic Services and the Regional Supplementary Procedures applicable to the region.

#### 2 ARE OF RESPONSIBILITY

Air traffic services in the following paragraphs are provided within the airspace encompassed by the ATLANTIS FIR.

#### 3 TYPES OF SERVICE

##### Air traffic control services

The appropriate air traffic control unit provides air traffic control service in the CTAs, TMAs and CTRs.

##### Provision of UTC time-check at controlled aerodromes

Before taxiing for takeoff, the pilot shall ensure that the time used by the aircraft systems is set and checked by synchronizing it with GPS time (corrected to UTC). The synchronization shall be carried out with the aircraft's aviation approved GPS device.

If the aircraft is not equipped with GPS and the pilot requires the current time (UTC), the pilot shall ask ATC for a time-check.

##### Flight information and alerting service

The appropriate ATC unit provides flight information and alerting service in controlled airspace.

##### Aerodrome flight information service (AFIS)

Aerodrome flight information service (AFIS) is the term used to describe the provision of information useful for the safe and efficient conduct of IFR operations at those uncontrolled aerodromes designated for use by international general aviation. From the information received the pilot will be able to decide the appropriate course of action to be taken to ensure the safety of flight.