WORKING COPY OF

EUR ANP, VOLUME II, FASID

SUPPLEMENT TABLE CNS 1 — AERONAUTICAL MOBILE SERVICE HF AIR-GROUND AND VHF GENERAL PURPOSE REQUIREMENTS

SUPPLEMENT TABLE CNS 2 — AERONAUTICAL MOBILE SERVICE (VHF) — AERODROME AND APPROACH CONTROL SERVICE

SUPPLEMENT TABLE CNS 3 — AERONAUTICAL MOBILE SERVICE (VHF) — FLIGHT INFORMATION AND AREA CONTROL SERVICE

SUPPLEMENT TABLE CNS 4a — RADIO NAVIGATION AIDS ASSOCIATED WITH THE BASIC ATS ROUTE NETWORK

SUPPLEMENT TABLE CNS 4b — INSTRUMENT APPROACH PROCEDURES AND ASSOCIATED RADIO NAVIGATION SERVICES AND FACILITIES

RECORD OF AMENDMENTS

Note: A consolidated text of this section, containing the following approved amendments to the EUR ANP, will be officially disseminated on an annual basis.  This document is produced solely as reference material to assist States in the preparation of proposals for amendment to the EUR ANP.

AMENDMENTS

| P. f. Amdt. Serial No. | Originator | Date of Approval letter | Date Entered |  | P. f. Amdt. Serial No. | Originator | Date of Approval letter | Date Entered |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F05/27-CNS Table CNS 3 | Switzerland | 30/11/05 | 10/05/06 |  | State Letter EUR/NAT 11-0718.TEC | Czech Republic, Estonia, Finland, Former Yugoslav Republic of Macedonia, Greece, Italy, Kazakhstan, Lithuania, Luxembourg, Poland, Portugal, Republic of Moldova, Romania, Russian Federation, Sweden, Switzerland, United Kingdom, |  | 13/02/12 |
| F05/26-CNS Table CNS 2 | Uzbekistan | 19/01/06 | 10/05/06 |  |  |  |  |  |
| F05/27-CNS AOP Table CNS 2 | Switzerland | 21/09/06 | 13/02/07 |  |  |  |  |  |
| State Letter TEC/CNS/COM-08-0005.SLG | Cyprus, Czech Republic, Denmark, Estonia, Finland, Germany, Greece, Monaco, Poland, Portugal, Romania, Republic of Moldova, Serbia, Sweden, Turkey, United Kingdom | 21/01/08 | 15/06/09 |  |  |  |  |  |
| State Letter EUR/NAT 10-442.TEC | Belarus, Denmark, Greece, Latvia, Norway, Romania, Slovakia, Sweden, Switzerland, Turkey, Ukraine, United Kingdom, | 18/08/10 | 18/08/10 |  |  |  |  |  |
| State Letter EUR/NAT 10-910.TEC | Armenia, Belgium, Estonia, Germany, Ireland, Kyrgyzstan Rep, Norway, Poland, Romania, Russian Federation, Spain, Sweden, Switzerland, The Netherlands, Turkey, Ukraine, United Kingdom | 02/03/2011 | 02/03/11 |  |  |  |  |  |

SUPPLEMENT Table CNS 1 — AERONAUTICAL MOBILE SERVICE

HF AIR-GROUND AND VHF GENERAL PURPOSE REQUIREMENTS

EXPLANATION OF THE TABLE

Column

1	Name/location of station.

2	HF network designation based on ITU Appendix 27.

Note.— The inclusion of a network designator applicable to a given aeronautical station indicates that all frequencies of that network are assigned to that station, which may implement any number of frequencies of that network based on pertinent technical considerations (e.g. propagation conditions, sunspot cycle, etc.) without coordination with ICAO. The implementation of discontinuance of guard of specific discrete frequencies should be announced by NOTAM.

3	General purpose (GP) VHF requirement.

4	Supplementary information.

| Location | HF | VHF GP | Remarks |
| --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |
| ARMENIA |  |  |  |
| GUMRI | MID-1/2/3 |  |  |
| YEREVAN | MID-1/2/3 |  |  |
| AZERBAIJAN |  |  |  |
| BAKU/Bina | MID 1/2/3 |  |  |
| GYANDZHA | MID 1/2/3 |  |  |
| NAKHICHEVAN | MID 1/2/3 |  |  |
| YEVLAKH | MID 1/2/3 |  |  |
| BELARUS |  |  |  |
| MINSK | EUR |  |  |
| CZECH REPUBLIC |  |  |  |
| PRAHA MCC |  | X |  |
| BECHYNE | EUR |  |  |
| GEORGIA |  |  |  |
| TBILISI | MID-1/2 |  |  |
| GREECE |  |  |  |
| ATHINAI | EUR, AFI 1, 1D WI* | X | *OPC |
| IRELAND |  |  |  |
| SHANNON ACC |  | X |  |
| KAZAKHSTAN |  |  |  |
| AKTOBE | MID-3 |  |  |
| ASTANA | RDARA |  |  |
| SHYMKENT | RDARA |  |  |
| KYRGYZSTAN |  |  |  |
| BISHKEK | MID-2/3 |  |  |
| LATVIA |  |  |  |
| RIGA | EUR |  |  |
| MALTA |  |  |  |
| MALTA ACC | EUR |  |  |
| NORWAY |  |  |  |
| SEIVAAG | D/ 1C |  |  |
| ANDENES | D /1C |  |  |
| BERLEVAAG | D /1C |  |  |
| KAPP LINNE | 1C |  |  |
| JAN MAYEN | 1C |  |  |
| BJORNOYA | 1C |  |  |
| PORTUGAL |  |  |  |
| LISBOA ACC |  | X |  |
| SANTA MARIA ACC | NAT A | X |  |
|  | NAT E |  |  |
|  | A-3/5.6/9/13.3/18 MHz |  |  |
|  | E-3/6.6/9/11.3/13.3 MHz |  |  |
| ROMANIA BUCURESTI |  | X |  |
| RUSSIAN FEDERATION |  |  |  |
| SANKT PETERBURG | EUR |  |  |
| VELIKYE LUKY | EUR |  |  |
| VOLOGDA | EUR |  |  |
|  | NCA-1 |  |  |
| ARKHANGELSK | EUR |  |  |
| MURMANSK | EUR |  |  |
| SYKTYVKAR | EUR |  |  |
|  | NCA-1 |  |  |
| MOSCOW | EUR |  |  |
|  | MID-3 |  |  |
|  | NCA-1 |  |  |
|  | VNCA |  |  |
| SAMARA | MID-3 |  |  |
| YEKATERINBURG | NCA-1 |  |  |
| IVDEL | NCA-1 |  |  |
| SURGUT | NCA-2 |  |  |
| KHANTY MANSIYSK | NCA-1 |  |  |
|  | NCA-2 |  |  |
| NOVOSIBIRSK | NCA-2 |  |  |
|  | VNCA |  |  |
| KOLPASHEVO | NCA-2 |  |  |
| BARNAUL | NCA-2 |  |  |
| KRASNOYARSK | NCA-2 |  |  |
| PODKAMENNAYA TUNGUSKA | NCA-2 |  |  |
|  | NCA-3 |  |  |
| YENISEYSK | NCA-2 |  |  |
|  | NCA-3 |  |  |
| TURUKHANSK | NCA-2 |  |  |
| IRKUTSK | NCA-2 |  |  |
|  | NCA-3 |  |  |
|  | EA-1 |  |  |
|  | EA-2 |  |  |
| KIRENSK | NCA-2 |  |  |
|  | NCA-3 |  |  |
| CHITA | NCA-3 |  |  |
|  | NCA-3 (2) |  |  |
| ULAN-UDE | NCA-3 |  |  |
| KHABAROVSK | NCA-3 |  |  |
|  | NCA-3 (2) |  |  |
|  | NP |  |  |
|  | VNCA |  |  |
| EKIMCHAN | NCA-3 |  |  |
| YUZHNO-SAKHALINSK | NCA-3 (2) |  |  |
| PETROPAVLOVSK-KAMCHATSKY | NCA-3 (2) |  |  |
| CHULMAN | NCA-3 |  |  |
| UKRAINE |  |  |  |
| KYIV |  |  |  |
| LVOV |  |  |  |
| ODESSA |  |  |  |
| SIMFEROPOL |  |  |  |
| UZBEKISTAN |  |  |  |
| SAMARKAND | MID-2/3 |  |  |
| TASHKENT | MID-2/3 VNCA |  |  |

SUPPLEMENT Table CNS 2 — AERONAUTICAL MOBILE SERVICE (VHF) —

AERODROME AND APPROACH CONTROL SERVICE

EXPLANATION OF THE TABLE

Column

1	Name/location of station.

2	Channel(s) for aerodrome control (TWR).

3	Channel(s) for aerodrome flight information service (AFIS).

4	Channel(s) for approach control, low (APP/L).

5	Channel(s) for approach control, intermediate (APP/I).

6	Channel(s) for approach control, high (APP/H).

7	Channel(s) for automatic terminal information service (ATIS).

8	Supplementary information.

Note 1.— In cases where more than one function is combined in one ATS radio control position, the channel concerned is listed only in that column corresponding to the highest requirements in operational range and height.

Note 2.— The planning criteria to be used for operational coverage of the various services are those shown in the Table of Uniform Values contained in Attachment B to Part V.II — ATS, unless otherwise specifically indicated.

Note 3.— The specific frequencies assigned to the channels are shown in Supplement Table COM 2 of Part IV — CNS.

Symbols/abbreviations used and their meaning

#	Aerodrome and ATS listed in Table AOP 1, but for which no channel requirement is presented under Table ATS 2.

*	TWR service provided on APP channel.

a	Air traffic services requirement is not in accordance with the basic operational requirements, planning criteria and methods of application (RCM).

AS	Aerodrome surface communications channel allotted also.

CVFR	Controlled VFR flights.

DEP	For departing traffic.

HEL	For helicopter operations.

ILS	ILS immediate access channel.

ILS-CH	Use of ILS voice channel.

VOR	Use of VOR speech facility.

| Location | TWR | AFIS | APP |  |  | ATIS | Remarks |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | I | H |  |  |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  |
| ALBANIA |  |  |  |  |  |  |  |  |
| TIRANA/Tirana | 1 |  |  |  |  | 1 |  |  |
| ARMENIA |  |  |  |  |  |  |  |  |
| GUMRI | 1 |  | 1 | 1 |  | 1 |  |  |
| YEREVAN/ZVARTNOTS | 1 | 1 | 1 | 1 | 1 | 1 |  |  |
| AUSTRIA |  |  |  |  |  |  |  |  |
| DORNBIRN/Hohenems/Dornbirn | 1 |  |  |  |  |  | IGA 10 NM/4000 ft |  |
| GRAZ/Graz | 1 |  |  | 1 |  |  |  |  |
| INNSBRUCK/Innsbruck | 1 |  |  | 1 |  |  |  |  |
| KLAGENFURT/Klagenfurt | 1 |  |  | 1 |  |  |  |  |
| LINZ/Linz | 1 |  |  |  | 1 |  |  |  |
| ST JOHANN IN TIROL/St Johann in Tirol | 1 |  |  |  |  |  | IGA 10 NM/4000 ft |  |
| SALZBURG/Salzburg | 1 |  | 1x | 1 |  |  | x 1988 |  |
| VOESLAU/Voeslau | 1 |  |  |  |  |  | IGA 10 NM/4000 ft |  |
| WELS/Wels | 1 |  |  |  |  |  | IGA 10 NM/4000 ft |  |
| WIEN/Schwechat | 2 |  |  | 3 |  | 1 |  |  |
| ZELL AM SEE/Zell am See | 1 |  |  |  |  |  | IGA 10 NM/4000 ft |  |
| AZERBAIJAN |  |  |  |  |  |  |  |  |
| BAKU/Bina | 1 |  | 1 |  | 1 | 2 |  |  |
| GYANDZHA/Gyandzha | 1 | 1 |  |  |  | 1 |  |  |
| NAKHICHEVAN/Nakhichevan | 1 | 1 |  |  |  | 1 |  |  |
| YEVLAKH/Yevlakh | 1 | 1 |  |  |  |  |  |  |
| BELARUS |  |  |  |  |  |  |  |  |
| BREST | 2 |  |  |  | 1 | 2 |  |  |
| GOMEL | 1 |  |  | 2 | 1 | 2 |  |  |
| GRODNO | 1 |  |  | 1 | 1 | 1 |  |  |
| MINSK-1 | 1 |  |  |  |  | 1 |  |  |
| MINSK-2 | 2 1+ |  | 1X |  | 1X | 2 | X APPROACH and RADAR control also for Minsk-1:+AS |  |
| MOGILEV | 1;1+ |  |  |  | 1* | 1 | +AS |  |
| ORSHA | 1 |  |  | 1x |  |  | xAPP VITEBSK |  |
| VITEBSK | 1 |  | 1 | 1 | 2 | 1 |  |  |
| BELGIUM |  |  |  |  |  |  |  |  |
| ANTWERPEN/Deurne | 1 |  |  |  |  |  |  |  |
| BALEN/Keiheuvel |  | 1 |  |  |  |  | IGA 15 NM |  |
| BRUXELLES/Grimbergen | 1 |  |  |  |  |  | IGA 15 NM |  |
| CHARLEROI/Gosselies | 1 |  | 1x |  |  |  | x Radar |  |
| GENK/Zwarbert |  | 1 |  |  |  |  |  |  |
| KORTRIJK/Wevelgem |  | 1 |  |  |  |  | IGA 15 NM |  |
| LIEGE/Bierset | 1* |  | 1 |  |  |  |  |  |
| OOSTENDE/Oostende | 1 |  | 1 |  |  |  |  |  |
| SAINT HUBERT |  | 1 |  |  |  |  | IGA 15 NM |  |
| SPA/La Sauvenière |  | 1 |  |  |  |  | IGA 15 NM |  |
| TOURNAI/Maubray |  | 1 |  |  |  |  | IGA 15 NM |  |
| BOSNIA AND HERZEGOVINA |  |  |  |  |  |  |  |  |
| BANJA LUKA | 1 |  | 1 |  | 1 | 1 |  |  |
| BIHAC/Zeljava | 1 |  | 1 |  | 1 | 1 |  |  |
| MOSTAR/Ortijes | 1 |  | 1 |  | 1 | 1 |  |  |
| SARAJEVO/Butmir | 1 |  | 1 |  | 1 | 1 |  |  |
| TUZLA/Dubravet | 1 |  | 1 |  | 1 | 1 |  |  |
| BULGARIA |  |  |  |  |  |  |  |  |
| BURGAS/Sarafovo | 1 |  | 1 | 1 | 1x | 1 | x Radar |  |
| SOFIA/Vrajdebna | 1 |  | 1 | 1 | 1x | 1 | x Radar |  |
| VARNA/Acsakovo | 1 |  | 1 | 1 | 1x |  | x Radar |  |
| CROATIA |  |  |  |  |  |  |  |  |
| BRAZ/Bol | 1 |  |  |  |  |  |  |  |
| DUBROVNICK/Cilipi | 1 |  | 1 | 3 | 1 |  |  |  |
| LOSINJ/Losinj | 1 |  | 1 |  |  |  |  |  |
| OSIJEK/Klisa | 1 |  | 1 |  |  |  |  |  |
| OSIJEK/Cepin | 1 |  |  | 1 |  |  |  |  |
| PULA/Pula | 1 |  |  | 1 | 2 | 1x | x VOR |  |
| RIJEKA/Krk | 1 |  |  | 1 |  |  |  |  |
| SPLIT/Kastel | 2 |  | 1 | 2 | 2 | 1x | x VOR |  |
| VRSAR/Crljenka | 1 |  |  |  |  |  |  |  |
| ZADAR/Zemunik | 1 |  | 1 |  | 1 |  |  |  |
| ZAGREB/Pleso | 1 |  |  | 2 | 1 | 1 |  |  |
| CYPRUS |  |  |  |  |  |  |  |  |
| LARNACA/Larnaca | 1 |  | 1 |  |  | 1x | x VOR |  |
| NICOSIA/Nicosia (CLOSED UFN) | 1 |  | 1 |  |  |  | AS |  |
| PAPHOS/Paphos | 1 |  | 1 |  |  |  |  |  |
| CZECH REPUBLIC |  |  |  |  |  |  |  |  |
| BRNO/Turany | 2 |  |  | 2x |  | 1 |  |  |
| ČÁSLAV | 2 |  |  | 1 |  |  |  |  |
| KARLOVY VARY | 2 | 1 |  | 1 |  | 1 |  |  |
| NÁMÉŠT | 2 |  |  | 1 |  |  |  |  |
| OSTRAVA/Mosnov | 1 |  |  | 2x |  | 1 |  |  |
| PARDUBICE | 2 |  | 1 | 1 |  |  |  |  |
| PLZEŇ/Line | 2 | 1 |  |  |  |  |  |  |
| PRAHA/Kbely | 2 |  | 1 | 1 | 2 |  |  |  |
| PRAHA/Ruzyne | 3 |  | 1 | 4x |  | 1 |  |  |
| PŘEROV | 2 |  |  | 1 |  |  |  |  |
| DENMARK |  |  |  |  |  |  |  |  |
| ALBORG/Alborg | 1 |  |  | 1 | 1 | 1 |  |  |
| AARHUS/Aarhus | 1* |  |  | 1 |  |  |  |  |
| BILLUND/Billund | 1 |  |  |  | 1 | 1 |  |  |
| BORNHOLM/Rønne | 1 |  |  | 1 |  |  |  |  |
| ESBJERG/Esbjerg |  | 1 |  |  |  |  |  |  |
| KARUP/Karup | 1 |  | 1 |  | 1 |  |  |  |
| KØBENHAVN/Kastrup | 6 |  |  | 1 | 4 | 2 |  |  |
| KØBENHAVN/Roskilde | 2 |  |  | 1 |  | 1 |  |  |
| KOLDING/Vamdrup |  | 1 |  |  |  |  |  |  |
| MARIBO/Maribo |  | 1 |  |  |  |  |  |  |
| ODENSE/Odense |  | 1 |  |  |  |  |  |  |
| SINDAL/Sindal |  | 1 |  |  |  |  |  |  |
| SKYDSTRUP/Vojens | 1 |  |  |  | 2 | 1 |  |  |
| SØNDERBORG/Sønderborg |  | 1 |  |  |  |  |  |  |
| STAUNING/Stauning |  | 1 |  |  |  |  |  |  |
| ESTONIA |  |  |  |  |  |  |  |  |
| KARDLA |  | 1 |  |  |  |  |  |  |
| KURESSAARE | 1 | 1 | 1 |  |  |  |  |  |
| PARNU |  | 1 |  |  |  |  |  |  |
| TALLINN/Ulemiste | 1 |  | 1 |  | 1 | 1 |  |  |
| TARTU/Ulenurme | 1 |  | 1 |  |  |  |  |  |
| FINLAND |  |  |  |  |  |  |  |  |
| HELSINKI/Helsinki-Malmi | 1* |  |  |  | 1x |  |  |  |
| HELSINKI/Helsinki-Vanta | 1 |  |  |  | 1 1xx | 1+ | + VOR xx Radar |  |
| IVALO/Ivalo | 1* |  | 1 |  |  |  |  |  |
| MARIEHAMN/Mariehamn | 1* |  |  | 1 |  |  |  |  |
| OULU/Oulu | 1 |  |  | 1 |  |  |  |  |
| ROVANIEMI/Rovaniemi | 1 |  |  | 1 | 1 |  |  |  |
| TAMPERE/Tampere-Pirkkala | 1 |  |  | 1 | 1 |  |  |  |
| TURKU/Turku | 1* |  |  | 1 |  |  |  |  |
| VAASA/Vaasa | 1* |  |  | 1 |  |  |  |  |
| FRANCE |  |  |  |  |  |  |  |  |
| AJACCIO/Campo dell’Oro | 1 |  | 1 |  | 1x | 1 | x 90/250 |  |
| BÂLE-MULHOUSE/Bâle-Mulhouse | 2 |  | 1xxx |  | 1+ | 1xx | xx FM 250 + TMA FL 195 xxx Radar |  |
| BASTIA/Poretta | 1 |  | 1 |  |  | 1 |  |  |
| BEAUVAIS/Tillé | 1 |  | 1 |  |  |  |  20 NM/4000 ft |  |
| BIARRITZ/Bayonne-Anglet | 1 |  | 1 |  |  |  |  |  |
| BORDEAUX/Mérignac | 1 |  | 2xxx |  | 1x | 1xx | x FL 195 xx 60 NM/FL 250 xxx Radar |  |
| BREST/Guipavas | 1x |  | 1 |  |  |  | x 20 NM/4000 ft |  |
| CAEN/Carpiquet | 1x |  | 1 |  |  |  | IGA x 20 NM/4000 ft |  |
| CALAIS/Dunkerque | 1 |  | 1 |  |  |  | IGA |  |
| CALVI/Sainte-Catherine | 1 |  | 1 |  |  |  | IGA |  |
| CANNES/Mandelieu | 2 |  | 1 |  |  | 1 | IGA |  |
| CHAMBERY/Aix-les-Bains | 1 |  | 1 |  |  |  | IGA |  |
| CHERBOURG/Maupertus | 2 |  | 1 |  |  |  |  |  |
| CLERMONT-FERRAND/Aulnat | 1 |  | 1 | 1 |  | 1 |  |  |
| DEAUVILLE/Saint-Gatien | 1 |  |  | 1 |  |  |  |  |
| DINARD/Pleurtuit-Saint-Malo | 1 |  |  | 1 |  | 1 |  |  |
| DÔLE/Tavaux | 1 |  |  |  |  |  |  |  |
| GRENOBLE/Saint-Geoirs | 1 |  | 1 |  |  | 1 |  |  |
| LANNION | 1 |  | 1 |  |  |  | IGA |  |
| LA ROCHELLE/Laleu | 1 |  | 1 |  |  |  | IGA |  |
| LE HAVRE/Octeville | 1 |  | 1 |  |  |  |  |  |
| LE TOUQUET/Paris-Plage | 1 |  | 1 |  |  |  |  |  |
| LILLE/Lesquin | 1 |  | 1x |  |  | 1 | x Radar |  |
| LYON/Bron | 1 |  | 1 |  | 1 | 1 | IGA |  |
| LYON/Satolas | 2 |  | 1 2xx | 1xx |  | 1x | x 60 NM/FL 250 ft xx Radar |  |
| MARSEILLE/Provence | 1x 1 |  | 2 | 1xxx 1 | 1xx | 1 | x 20 NM/3000 ft xx 60 NM/FL 250 xxx Radar |  |
| METZ/Frescaty | 1 |  | 1 |  |  |  | IGA – aerodrome not contained in Table AOP 1) |  |
| MONTPELLIER/Fréjorgues | 1 |  | 1 |  |  | 1x | x 60 NM/FL 200 |  |
| NANCY/Essey | 1 |  | 1 |  |  |  | IGA |  |
| NANTES/Château-Bougon | 1x 1 |  | 1xx |  |  | 1 | x 20 NM/3000 ft xx Radar |  |
| NICE/Côte d'Azur | 1 |  | 1xx |  | 1 1xx | 1 | xx Radar |  |
| NÎMES/Garons | 1 |  | 1x 1 |  |  |  | x Radar |  |
| PARIS/Charles-de-Gaulle | 1x 1+ 2 |  | 2xx | 2 1x | 3 | 1 | + 20 NM/3000 ft ILS x 20 NM/3000 ft xx Radar |  |
| PARIS/Le Bourget | 1x 1 |  | 1xx |  |  | 1 | x 20 NM/3000 ft (IGA) xx Radar |  |
| PARIS/Orly | 2 |  | 1 2xx | 1x 1xx | 1 2xx | 1 | x FL 150 xx Radar |  |
| PAU/Pont-Long-Uzein | 1 |  | 1 1x | 1 |  | 1 | IGA |  |
| PERPIGNAN/Rivesaltes | 1 |  |  | 1x |  |  | x Radar |  |
| POITIERS/Biard | 1 |  | 1 |  |  |  |  |  |
| QUIMPER/Pluguffan | 1x |  | 1 |  |  |  | x 20 NM/4000 ft |  |
| REIMS/Champagne | 1 |  |  | 1 |  |  | IGA |  |
| RENNES/Saint-Jacques | 1 |  | 1 |  |  |  |  |  |
| SAINT-BRIEUC/Armor | 1 |  | 1 |  |  |  |  |  |
| SAINT-ETIENNE/Bouthéon | 1 |  | 1 |  |  |  | IGA |  |
| SAINT-NAZAIRE/Montoir | 1 |  |  |  |  |  | IGA |  |
| STRASBOURG/Entzheim | 2 |  |  | 1x |  | 1 | x Radar |  |
| TARBES/Ossun-Lourdes | 1 |  | 1 | 1x |  | 1 | x Radar |  |
| TOULOUSE/Blagnac | 1 |  | 1 1x | 1x | 1x/  | 1 | x Radar  100 NM |  |
| TOURS/Saint-Symphorien | 1 |  |  | 1 |  |  | IGA |  |
| TOUSSUS-LE-NOBLE | 2 |  | 2 |  |  | 1 | IGA |  |
| GEORGIA |  |  |  |  |  |  |  |  |
| TBILISI/Novoalexeyevka | 1 |  |  |  | 1 | 1 |  |  |
| GERMANY |  |  |  |  |  |  |  |  |
| ALLENDORF/Eder |  | 1 | 1 |  |  | 1 | IGA ; APP by Langen RADAR |  |
| AUGSBURG | 1 |  |  |  |  | 1 |  |  |
| BARTH |  | 1 |  |  |  |  | IGA |  |
| BAUTZEN |  | 1 |  |  |  |  | IGA |  |
| BAYREUTH |  | 1 |  |  |  |  | IGA |  |
| BERLIN/Schönefeld | 3 | 1 | 1 | 1 | 4 | 1 | AS, APP by Bremen RADAR |  |
| BERLIN/Tegel | 2 |  | 1 |  | 1 | 1 | AS , APP by Bremen RADAR |  |
| BIELEFELD |  | 1 |  |  |  |  | IGA |  |
| BRAUNSCHWEIG-Wolfsburg | 1 |  |  |  |  |  |  |  |
| BREMEN | 2 |  | 1 |  | 1 | 1 | AS |  |
| BREMERHAVEN |  | 1 |  |  |  |  |  |  |
| COBURG-Brandensteinsebene |  | 1 |  |  |  | 1 | IGA |  |
| COTTBUS-Drewitz |  | 1 |  |  |  |  |  |  |
| DONAUESCHINGEN-Villingen |  | 1 |  |  |  |  | APP provided by Zürich |  |
| DONAUWÖRTH HEL |  | 1 |  |  |  | 1 |  |  |
| DORTMUND | 2 |  | 1 |  |  | 1 | AS, APP by Langen RADAR |  |
| DRESDEN | 2 |  |  |  | 1 | 1 | AS |  |
| DÜSSELDORF | 1 |  | 1 | 2 |  | 1 | AS |  |
| EGELSBACH |  | 1 |  |  |  |  | AS |  |
| EGGENFELDEN |  | 1 |  |  |  | 1 | IGA |  |
| EMDEN |  | 1 |  |  |  |  | IGA |  |
| ERFURT | 2 |  |  |  |  | 1 | AS |  |
| ESSEN |  | 2 |  |  |  |  |  |  |
| FLENSBURG |  | 2 |  |  |  |  |  |  |
| FRANKFURT-Hahn | 2 |  | 1 |  |  | 1 | AS, APP by Langen RADAR |  |
| FRANKFURT | 4 |  | 1 | 3 | 1 | 2 | AS |  |
| FREIBURG |  | 1 |  |  |  |  |  |  |
| FRIEDRICHSHAFEN/ | 1 |  |  | 1 |  | 1 | AS , APP provided by Zürich |  |
| HAMBURG-Finkenwerder | 1 |  | 1 |  |  |  | AS, APP by Bremen RADAR |  |
| HAMBURG | 2 |  | 1 | 1 |  | 1 | AS, APP by Bremen RADAR |  |
| HANNOVER | 2 |  |  | 2 | 1 | 2 | AS, APP by Bremen RADAR |  |
| HERINGSDORF | 1 | 1 |  |  |  |  |  |  |
| HOF-Plauen | 1 |  |  |  |  |  |  |  |
| KARLSRUHE-BADEN/Baden | 1 |  | 1 |  |  | 1 | APP provided by Strassbourg |  |
| KASSEL-Calden | 1 |  |  |  |  |  |  |  |
| KIEL/Holtenau |  | 1 |  |  |  |  | IGA |  |
| KÖLN/Bonn | 2 |  | 1 1 | 2 1 |  | 1 | AS, APP by Langen RADAR |  |
| KONSTANZ |  | 1 |  |  |  |  | IGA |  |
| LAHR | 1 |  | 1 |  |  |  | APP provided by Strasbourg |  |
| LANDSHUT LEIPZIG-Altenburg | 1 | 1 |  |  |  |  |  |  |
| LEIPZIG/Halle | 3 |  |  | 1 | 1 | 1 | AS |  |
| LÜBECK MAGDEBURG | 1 | 1 |  |  |  | 1 | AS IGA |  |
| MAGDEBURG/Cochstedt | 1 |  |  |  |  |  | AS |  |
| MANNHEIM City | 1 | 1 | (1) |  |  | 1 | APP by Langen RADAR |  |
| MEMMINGEN | 1 |  |  |  |  | 1 | AS |  |
| MENGEN-Hohentengen |  | 1 |  |  |  |  | IGA |  |
| MÖNCHENGLADBACH | 1 |  | (1) |  |  | 1 | AS, APP by Langen RADAR |  |
| MÜNCHEN | 3 |  | 2 | 2 | 1 | 1 | AS |  |
| MÜNSTER/ Osnabrück | 1 |  | 1 |  |  | 1 | AS , APP by Langen RADAR |  |
| NIEDERRHEIN | 1 |  | (1) |  |  | 1 | APP by Langen RADAR |  |
| NIEDERSTETTEN | 2 | 1 |  |  |  |  |  |  |
| NORDHOLZ | 3 |  |  |  |  |  |  |  |
| NÜRNBERG | 2 |  |  |  | 1 | 1 |  |  |
| OBERPFAFFENHOFEN | 2 |  |  |  |  |  |  |  |
| PADERBORN | 2 |  | (1) |  |  | 1 | AS, APP by Langen RADAR |  |
| SAARBRÜCKEN | 2 |  | (1) |  |  | 1 | APP by Langen RADAR |  |
| SCHWÄBISCH-HALL |  | 1 | (1) |  |  | 1 | IGA, APP by Langen RADAR |  |
| SCHWERIN-PARCHIM | 1 |  |  |  |  |  |  |  |
| SIEGERLAND |  | 1 |  | 1 |  | 1 | IGA, APP Langen Radar |  |
| STRAUBING |  | 1 |  |  |  | 1 | IGA |  |
| STUTTGART | 3 |  | (1) | 1 | 2 | 2 | APP by Langen RADAR |  |
| TRIER |  | 2 |  |  |  |  | IGA |  |
| WESTERLAND/Sylt | 1 | 1 |  |  |  | 1 | IGA |  |
| WILHELMSHAVEN |  | 1 |  |  |  | 1 |  |  |
| WORMS/ |  | 1 |  |  |  |  | IGA |  |
| ZWEIBRÜCKEN | 1 |  | (1) |  |  | 1 | APP by Langen RADAR |  |
|  |  |  |  |  |  |  | () shared Channel |  |
| GIBRALTAR (United Kingdom) |  |  |  |  |  |  |  |  |
| GIBRALTAR/North Front | 1 |  |  |  | 1 |  |  |  |
| GREECE |  |  |  |  |  |  |  |  |
| ALEXANDROUPOLIS/ Dimokritos | 1* 1*x |  |  | 1 1x |  |  | x RGA |  |
| ALMIROS/Nea Anchialos | 1*x |  |  |  | 1 1x |  | x RGA |  |
| ANDRAVIDA | 1* 1*x |  |  |  | 2 1x |  | x RGA |  |
| ARAXOS | 1 1x |  |  |  |  |  | APP service provided by Andravida x RGA |  |
| ATHINAI APP |  |  |  |  | 2 4x 2xx |  | (ATHINAI TMA) 90NM/FL280 x Radar, 90NM/L280 xx VFR, 80 NM/FL200 |  |
| ATHINAI/ Eleftherios Venizelos | 3 1x 1xx 5xxx |  |  |  |  | 1 | APP service provided by Athinai APP x RGA xx VFR 50NM/FL250 xxx AS |  |
| CHANIA/Ioannis Daskalogiannis | 1* 1* x 1xx |  |  |  | 1 1x | 1 | x RGA xx AS |  |
| CHIOS/Omiros | 1 |  |  | 1 |  |  |  |  |
| ELEFSIS | 2 1x |  |  |  |  |  | APP service provided by Athinai APP x RGA |  |
| IOANNINA/King Pyrros | 1 |  |  |  | 1 |  |  |  |
| IRAKLION/Nikos Kazantzakis | 2 1*x 2xxx |  |  |  | 2xx 1x | 1 | xx Radar x RGA xxx AS |  |
| KALAMATA | 1* 1*x |  |  |  | 1 1x |  | x RGA |  |
| KARPATHOS |  | 1 1x |  |  |  |  | x RGA |  |
| KAVALA/Megas Alexandros | 1 1*x |  |  |  | 1 1x | 1 | x RGA |  |
| KEFALINIA | 1 1x |  |  |  |  | 1 | APP service provided by Andravida x RGA |  |
| KERKIRA/Ioannis Kapodistrias | 1 1*x 2xxx |  |  |  | 2xx 1x | 1 | xx Radar x RGA xxx AS |  |
| KITHIRA |  | 1 |  |  |  |  | APP service provided by Kalamata |  |
| KOS/Ippokratis | 1 1*x 1xx |  | 1 1x |  |  | 1 | x RGA xx AS |  |
| LIMNOS/Ifaistos | 1* 1*x |  |  | 1 1x |  |  | x RGA |  |
| MIKONOS | 1 1*x |  |  | 1 1x |  | 1 | x RGA |  |
| MITILINI/Odysseas Elytis | 1* 1*x 1xx |  |  | 1 1x |  |  | x RGA xx AS |  |
| PREVEZA/Aktion | 1* 1*x |  |  | 1 1x |  |  | x RGA |  |
| RODOS/Diagoras | 1 1*x 2xxx |  |  |  | 2xx 1x | 1 | xx Radar x RGA xxx AS |  |
| SAMOS/Aristarchos of Samos | 1* 1*x 1xx |  | 1 1x |  |  | 1 | x RGA xx AS |  |
| SANTORINI | 1* 1*x |  |  | 1 1x |  | 1 | x RGA |  |
| SITIA |  | 1 |  |  |  |  | APP service provided by Iraklion |  |
| SKIATHOS/Alexandros Papadiamandis | 1 1*x |  |  | 1 1x |  |  | x RGA |  |
| THESSALONIKI/Makedonia | 2 1*x 2xxx |  |  | 1xx 1x | 1xx | 1 | xx Radar x RGA xxx AS |  |
| ZAKINTHOS/Dionisios Solomos | 1 1x |  |  |  |  | 1 | APP service provided by Andravida x RGA |  |
| HUNGARY |  |  |  |  |  |  |  |  |
| BUDAPEST/Ferihegy | 1 |  | 1 | 1 | 1 | 1 |  |  |
| IRELAND |  |  |  |  |  |  |  |  |
| Ireland West | 1 |  |  |  |  |  |  |  |
| CORK/Cork | 11 |  |  | 2 |  |  | 1 AS |  |
| Donegal | 1 |  |  |  |  | 1 |  |  |
| DUB/IN/Dublin | 11 |  | 1 | 2 |  | 1 | 1 AS |  |
| Galway | 1 |  |  |  |  |  |  |  |
| Kerry | 1 |  |  |  |  | 1 |  |  |
| SHANNON/Shannon | 11 |  |  |  | 2 | 1 | 1 AS |  |
| Sligo | 1 |  |  |  |  |  |  |  |
| Waterford | 1 |  |  |  |  | 1 |  |  |
| Weston | 1 |  |  |  |  | 1 |  |  |
| ITALY |  |  |  |  |  |  |  |  |
| ALBENGA/Albenga |  | 1 |  |  |  |  |  |  |
| ALGHERO/Alghero | 1 |  | 1 |  |  | 1 |  |  |
| ANCONA/Falconara | 2 |  | 1 |  |  | 1 |  |  |
| AOSTA/Aosta |  |  |  |  |  | 1 |  |  |
| BARI/Palese Macchie | 1 |  | 2 |  |  | 1 |  |  |
| BERGAMO/Orio al Serio | 2 |  | 2 |  |  | 1 |  |  |
| BOLOGNA/Borgo Panigale | 2 |  | 2 |  |  | 1 |  |  |
| BOLZANO/Bolzano |  | 1 |  |  |  |  |  |  |
| BRESCIA/Montichiari | 2 |  | 3 |  |  |  |  |  |
| BRINDISI/Casale | 2 |  | 2 |  |  |  |  |  |
| CAGLIARI/Elmas | 3 |  |  |  | 2 |  |  |  |
| CATANIA/Fontanarossa | 2 |  |  |  | 2 | 1 |  |  |
| COMO |  |  |  |  |  |  |  |  |
| CROTONE/Crotone |  | 1 | 2 |  |  |  |  |  |
| CUNEO | 1 |  | 2 |  |  | 1 |  |  |
| ELBA/Marina di Campo |  | 1 |  |  |  |  |  |  |
| FIRENZE/Peretola | 2 |  | 1 |  |  | 1 |  |  |
| FOGGIA/Gino Lisa |  | 1 | 1 |  |  |  |  |  |
| FORLI/Forli | 2 |  | 3 |  |  |  |  |  |
| GENOVA/Sestri | 1 |  | 2 |  |  | 1 |  |  |
| GROSSETO | 1 |  | 2 |  |  |  |  |  |
| LAMEZIA/Terme | 1 |  | 2 |  |  |  |  |  |
| LAMPEDUSA |  | 2 |  |  |  |  |  |  |
| MILANO/Linate | 4 |  | 4 |  |  | 1 |  |  |
| MILANO/Malpensa | 5 |  | 5 |  |  | 1 | x 15 NM/4000 ft |  |
| NAPOLI/Capodichino | 2 |  | 3 |  |  | 1 |  |  |
| OLBIA/Costa Smeralda | 2 |  | 2 |  |  | 1 |  |  |
| ORISTANO FENOSU |  | 2 |  |  |  |  |  |  |
| PADOVA |  | 1 | 1 |  |  |  |  |  |
| PALERMO/Punta Raisi | 2 |  | 2 |  |  | 1 |  |  |
| PANTELLERIA/Pantelleria | 2 |  |  |  |  |  |  |  |
| PARMA/Parma | 1 |  | 1 |  |  |  |  |  |
| PERUGIA/S. Egidio | 1 |  | 1 |  |  |  |  |  |
| PESCARA/Pescara | 1 |  | 1 |  |  |  |  |  |
| PISA/San Guisto | 3 |  | 2 |  | 2 |  |  |  |
| REGGIO CALABRIA/Reggio Calabria | 1 |  | 1 |  |  |  |  |  |
| REGGIO EMILIA |  | 1 |  |  |  |  |  |  |
| RIMINI/Miramare | 3 |  | 3 |  | 2 |  |  |  |
| ROMA/Ciampino | 4 |  | 5 |  |  |  |  |  |
| ROMA/Fiumicino | 6 |  | 6 |  |  |  |  |  |
| ROMA/Urbe | 1 |  |  |  |  |  |  |  |
| SALERNO/Pontecagnano |  | 1 | 2 |  |  |  |  |  |
| SIENA/Ampugnano |  | 1 | 2 |  |  |  |  |  |
| TARANTO/Grottaglie |  |  | 1 |  |  |  |  |  |
| TORINO/Caselle | 2 |  | 2 |  |  | 1 |  |  |
| TORTOLI’/Arbatrax |  | 1 |  |  |  |  |  |  |
| TRAPANI/Birgi | 2 |  |  |  | 1 |  |  |  |
| TREVISO/S.Angelo | 2 |  | 2 |  | 2 |  |  |  |
| TRIESTE/Ronchi De Legionari | 1 |  | 1 |  |  |  |  |  |
| VENEZIA/S. Nicolo |  | 1 |  |  |  |  |  |  |
| VENEZIA/Tesséra | 2 |  | 1 |  |  |  |  |  |
| VERONA/Villafranca | 2 |  | 3 | 1 |  |  |  |  |
| KAZAKHSTAN |  |  |  |  |  |  |  |  |
| AKTAU | 1 |  |  |  |  | 1 | x ILS-CH |  |
| AKTOBE | 1 |  |  |  |  | 1 |  |  |
| ALMATY | 1 |  |  | 1 |  | 1 | No information |  |
| ASTANA | 1 |  |  | 1 |  | 1 | No information |  |
| ATYRAU | 1 |  |  |  |  | 1 | No information |  |
| KARAGANDA | 1 |  |  |  |  |  | No information |  |
| KOKSHETAU | 1 |  |  |  |  |  | No information |  |
| KOSTANAY | 1 |  |  |  |  |  | No information |  |
| KYZYLORDA | 1 |  |  |  |  |  | No information |  |
| PAVLODAR | 1 |  |  |  |  |  | No information |  |
| PETROPAVLOVSK | 1 |  |  |  |  |  | No information |  |
| SEMEY | 1 |  |  |  |  |  | No information |  |
| TARAZ | 1 |  |  |  |  |  | No information |  |
| URALSK | 1 |  |  |  |  |  | No information |  |
| UST-KAMENOGORSK | 1 |  |  |  |  |  | No information |  |
| SHYMKENT | 1 |  |  |  |  | 1 | No information |  |
| ZHEZKAZGAN | 1 |  |  |  |  |  | No information |  |
| KYRGYZSTAN |  |  |  |  |  |  |  |  |
| BISHKEK/Manas | 1 |  |  | 1 |  | 1 |  |  |
| OSH/Osh | 1 |  |  | 1 |  | 1 |  |  |
| TAMCHY/Ysykkul | 1* |  |  | 1* |  |  |  |  |
| LATVIA |  |  |  |  |  |  |  |  |
| LIEPAJA | 0 |  | 1 |  |  | 0 |  |
| RIGA | 2 |  | 0 | 0 | 3 | 1 |  |
| VENTSPILS | 0 | 1 | 0 |  |  | 1 |  |
| TUKUMS |  | 1 |  |  |  | 1 |  |
| LITHUANIA |  |  |  |  |  |  |  |  |
| KAUNAS | 1 |  | 1 |  |  |  |  |  |
| PALANGA | 1 |  | 1 |  |  |  |  |  |
| SIAULIAI | 1 |  |  |  |  |  |  |  |
| VILNIUS | 1 |  | 1 |  |  | 1 |  |  |
| LUXEMBOURG |  |  |  |  |  |  |  |  |
| LUXEMBOURG/Luxembourg WILTZ/Noertrange | 1 | 1 | 1x |  |  | 1 | x Radar |  |
| MALTA |  |  |  |  |  |  |  |  |
| MALTA/Luqa | 1 |  |  | 1 |  |  |  |  |
| MONACO |  |  |  |  |  |  |  |  |
| Monaco | 1 |  |  |  |  |  | HEL |  |
| NETHERLANDS |  |  |  |  |  |  |  |  |
| AMSTERDAM/Schipol | 6 |  | 3 | 3 |  | 2x | x VOR/VOT |  |
| AMELAND |  | 1 |  |  |  |  | IGA 15 NM/3000 ft |  |
| DEVENTER/Teuge |  | 1 |  |  |  |  | IGA 15 NM/3000 ft |  |
| DRACHTEN |  | 1 |  |  |  |  | IGA 15 NM/3000 ft |  |
| GRONINGEN/Eelde | 1 |  | 1 | 1 |  | 1x |  |  |
| GRONINGEN/Oostwold |  | 1 |  |  |  |  | IGA 15 NM/3000 ft |  |
| HILVERSUM/Hilversum |  | 1 |  |  |  |  | IGA 15 NM/3000 ft |  |
| HOEVE/Seppe |  | 1 |  |  |  |  | IGA 15 NM/3000 ft |  |
| HOOGEVEEN/Hoogeveen |  | 1 |  |  |  |  | IGA 15 NM/3000 ft |  |
| LELYSTAD/Lelystad | 2* | 1 |  | 1* |  | 1* | IGA 15 NM/3000 ft /* ATS planned |  |
| MAASTRICHT/Zuid Limburg | 1 |  | 1 | 1 |  | 1 |  |  |
| MIDDLEBURG/Midden-Zeeland |  | 1 |  |  |  |  | IGA 15 NM/3000 ft |  |
| ROTTERDAM/Rotterdam | 1 |  | 1 |  |  | 1x | x VOR IGA 15 NM/3000 ft |  |
| TEXEL/Texel |  | 1 |  |  |  |  | IGA 15 NM/3000 ft |  |
| WEERT/Budel |  | 1 |  |  |  |  | IGA 15 NM/3000 ft |  |
| NORWAY |  |  |  |  |  |  |  |  |
| ALTA | 1 |  | 1 |  |  |  |  |  |
| BERGEN/Flesland | 1 |  |  | 3 |  |  |  |  |
| BODØ | 1 |  |  |  | 3 |  |  |  |
| Fagernes, Leirin |  | 1 |  |  |  |  |  |  |
| HARSTAD/NARVIK, Evenes | 1 |  | 1 |  |  |  |  |  |
| HAUGESUND, Karmøy | 1 |  |  | 1x |  |  |  |  |
| KIRKENES/Høybuktmoen | 1 |  | 1 |  |  |  |  |  |
| KRISTIANSAND/Kjevik | 1 |  |  | 1 |  |  |  |  |
| KRISTIANSUND, Kvernberget | 1 |  |  | 1 |  |  |  |  |
| MOLDE, Arø |  | 1 |  | 1x |  |  |  |  |
| OSLO/Gardermoen | 2 |  |  |  | 5x |  |  |  |
| MOSS, Rygge | 1 |  |  | 1x |  |  |  |  |
| Røros | 1 |  | 1 |  |  |  |  |  |
| Sandefjord, Torp | 1 |  |  | 1x |  |  |  |  |
| STAVANGER/Sola | 1 |  |  | 2 |  |  |  |  |
| SVALBARD/Longyear |  | 1 |  |  |  |  |  |  |
| TROMSØ | 1 |  |  |  | 1 |  |  |  |
| TRONDHEIM/Vaernes | 1 |  |  |  | 2 |  |  |  |
| ALESUND/Vigra | 1 |  |  |  | 1 |  |  |  |
| POLAND |  |  |  |  |  |  |  |  |
| BYDGOSZCZ/Szwederowo | 1 |  |  | 1 |  |  |  |  |
| GDANSK/Rebiechowo | 1 |  | 1 |  | 1 | 1 |  |  |
| KATOWICE/Pyrzowice | 1 |  |  | 1 |  | 1 |  |  |
| KRAKOW/Balice | 1 |  | 1 | 1 | 1 | 1 |  |  |
| LODZ/Lublinek | 1 |  |  | 1 |  |  |  |  |
| MIELEC MODLIN |  | 1 1 |  |  |  |  |  |  |
| POZNAN/Lawica | 1 |  | 1 | 1 |  | 1 |  |  |
| RZESZOW/Jasionka | 1 |  |  | 1 |  | 1 |  |  |
| SZCZECIN/Goleniow | 1 |  |  | 1 |  | 1 |  |  |
| SWIDNIK |  | 1 |  |  |  |  |  |  |
| WARSZAWA/Babice |  | 1 |  |  |  |  |  |  |
| WARSZAWA/Okeçic | 1 |  | 1 | 1 | 1x | 1 | x 60 NM |  |
| WROCLAW/Strachowice ZIELONA GORA/Babimost | 1 1 |  |  | 1 1 |  | 1 |  |  |
| PORTUGAL |  |  |  |  |  |  |  |  |
| FARO/Faro | 1 |  | 2 |  | 1 | 1 |  |  |
| FUNCHAL/Ilha de Madeira | 1 |  |  | 1 | 2 | 1 |  |  |
| LISBOA/Lisboa | 3 |  |  |  | 5 | 1 |  |  |
| PORTO/Porto | 2 |  | 1 |  | 1 | 1 |  |  |
| PORTO SANTO/Ilha de Porto Santo, Madeira | 1 |  | 1 |  |  |  |  |  |
| Bragança |  | 1 |  |  |  |  |  |  |
| Villa real |  | 1 |  |  |  |  |  |  |
| Portimao |  | 1 |  |  |  |  |  |  |
| Chaves |  | 1 |  |  |  |  |  |  |
| Evora |  | 1 |  |  |  |  |  |  |
| REPUBLIC OF MOLDOVA |  |  |  |  |  |  |  |  |
| CHISINAU | 1 |  | 1 | 1 |  | 1 |  |  |
| BALTI | 1 |  |  |  |  |  |  |  |
| CAHUL | 1 |  |  |  |  |  |  |  |
| ROMANIA |  |  |  |  |  |  |  |
| ARAD/Arad | 2 |  |  | 2 |  | 1 |  |
| BACAU | 2 |  |  | 1 |  | 1 |  |
| BAIA MARE | 1 |  |  | 2 |  | - | 1 APP is also TWR |
| BUCURESTI APP | - |  |  | 2 |  | - |  |
| BUCURESTI/Baneasa | 2 |  |  | 2 |  | 1 |  |
| BUCURESTI/Otopeni | 5 |  |  | - |  | 1 |  |
| CLUJ | 2 |  |  | 4 |  | 1 |  |
| CONSTANTA/M. Kogalniceanu | 2 |  |  | 2 |  | 1 |  |
| CRAIOVA | 1 |  |  | 1 |  | - |  |
| IASI | 2 |  |  | - |  | - |  |
| ORADEA | 2 |  |  | 2 |  | - |  |
| SATU MARE | 2 |  |  | 1 |  | - |  |
| SIBIU | 2 |  |  | 2 |  | - |  |
| SUCEAVA | 3 |  |  | 1 |  | - |  |
| TIMISOARA/Giarmata | 2 |  |  | 1 |  | 1 |  |
| TIRGU MURES/Vidrasau | 1 |  |  | 1 |  | - |  |
| TULCEA | - |  |  | 2 |  | - | 1 APP is also TWR |
| RUSSIAN FEDERATION |  |  |  |  |  |  |  |  |
| ABAKAN | 1 |  | * | 1 |  | 1 | Krasnoyarsk ACC Serves as Radar |  |
| ANADYR/Ugolny | * |  | 1 |  |  | 2 | 1 APP is also TWR |  |
| ANAPA/Vityazevo | 1 |  | 1 |  | * | 2 | АСС Rostov Serves as Anapa-Approach Н |  |
| ARKHANGELSK/Talagi | 1 |  | 1 | 1 |  | 1 |  |  |
| ASTRAKHAN | 1 |  | 1 | 1 |  | 1 | + 1 (Reserve FREQ) + 1 (Emergency FREQ) |  |
| BARNAUL | 1 |  | * | * |  |  | 1 TWR is also APP, Radar + 2 (Reserve FREQ) + 1 (Emergency FREQ) |  |
| BEGISHEVO | 1 |  | * | 1 |  | 1 | 1 TWR is also Radar + 1 (Emergency FREQ) + 2 (Reserve FREQ) |  |
| BELGOROD | 1 |  | 1 | 1 |  |  | + 1 (Emergency FREQ) + 1 (Reserve FREQ) |  |
| BLAGOVESHCHENSK/ Ignatyevo | 1 |  | * |  |  | 1 | 1 TWR is also Radar |  |
| BRATSK | 2 |  | * | * |  |  | 1 TWR is also APP, Radar + 1 (Transit) + 1 (MET) |  |
| BRYANSK | 1 |  |  |  |  |  |  |  |
| CHEBOKSARY | 1 |  | * |  |  |  | 1 TWR is also Radar |  |
| CHELYABINSK/Balandino | 1 |  | * | 2 |  |  | 2 APP is also Radar; + 1 (MET) |  |
| CHITA/Kadala | 1 |  | 1 |  |  | 1 | + 1 (Emergency FREQ) + 1 (VOLMET) |  |
| ELISTA | 1 |  |  |  |  | 1 |  |  |
| GROZNY/Severny | 1 |  | 1 |  |  | 1 |  |  |
| IRKUTSK | 2 |  | 1 |  | 1 | 2 | + 1 (Emergency FREQ) + 1 (Reserve FREQ) + 1 (VOLMET) |  |
| KALININGRAD/Khrabrovo | 1 |  | 1 |  |  | 2 | + 1 (Emergency FREQ) + 1 (Reserve FREQ) + 1 (Transit) |  |
| KAZAN | 2 |  | 1 | * |  | 1 | 1 Radar is also APP; + 1 (Emergency FREQ) + 2 (Reserve FREQ) |  |
| KEMEROVO | 1 |  |  | 1 |  |  | + 2 (Reserve FREQ) + 1 (MET) |  |
| KHABAROVSK/Novy | 2 |  | 1 | 2 |  | 1 | + 1 (VOLMET) + 1 (Transit) + 1 (AS) |  |
| KHANTY-MANSIYSK | 1 |  | 1 |  |  | 1 |  |  |
| KRASNODAR/Pashkovskiy | 3 |  | 1 | 2 |  | 2 | + 1 (Emergency FREQ) + 2 (Reserve FREQ) |  |
| KRASNOYARSK/Yemelyanovo | 2 |  | 1 | 1 |  | 1 | + 1 (Emergency FREQ) + 2 (Reserve FREQ) + 1 (Transit) |  |
| KURSK/Vostochny | 1 |  | 1 | * |  |  | 1 Radar is also APP; + 1 (Emergency FREQ) + 1 (MET) |  |
| MAGADAN/Sokol | 1 |  |  | 1 |  | 1 | + 1 (Emergency FREQ) + 2 (Reserve FREQ) + 1 (VOLMET) |  |
| MAGNITOGORSK | 1 |  |  |  |  |  | + 1 (Emergency FREQ) + 1 (Reserve FREQ) + 1 (MET) |  |
| MAKHACHKALA/Uytash | 1 |  | * | 1 |  | 2 | 1 TWR is also Radar |  |
| MINERALNYYE VODY | 2 |  | 1 | 1 |  | 2 | + 1 (Transit) |  |
| MOSCOW/Domodedovo | 2 |  | 1 | 1 |  | 2 | + 1 (Emergency FREQ) + 2 (Reserve FREQ) + 1 (VOLMET) + 1 (Transit) + 3 (APN) |  |
| MOSСOW/Sheremetyevo | 4 |  | 1 | 2 |  | 2 | + 1 (Emergency FREQ) + 1 (Reserve FREQ) + 1 (VOLMET) + 3 (Transit) + 3 (APN) |  |
| MOSCOW/Vnukovo | 5 |  | 1 | 1 |  | 2 | + 1 (Emergency FREQ) + 2 (Reserve FREQ) |  |
| MURMANSK | 1 |  |  |  |  | 1 |  |  |
| NALCHIK | 1 |  | * | 1 |  |  | 1 APP is also Radar |  |
| NIZHNEVARTOVSK | 1 |  |  | 1 |  | 1 | + 1 (Transit) |  |
| NIZHNY NOVGOROD/ Strigino | 2 |  | 1 | 2 |  | 1 | + 1 (Transit) |  |
| NOVOSIBIRSK/Tolmachevo | 2 |  | 1 | 2 |  | 2 | + 1 (VOLMET) |  |
| OMSK/Tsentralny | 2 |  | 1 | * |  | 1 | 1 Radar is also APP; + 1 (Emergency FREQ) + 1 (Reserve FREQ) |  |
| ORENBURG/Orenburg | 1 |  |  |  |  | 1 | + 1 (Transit) |  |
| ORSK | 1 |  | * |  |  | 1 | 1 TWR is also Radar; + 1 (Emergency FREQ) + 2 (Reserve FREQ) + 1 (Transit) |  |
| OSTAFYEVO | 1 |  | * |  |  | 1 | 1 TWR is also Radar; + 1 (Transit) |  |
| PERM / Bolshoye Savino | 1 |  | * | 1 |  | 1 | 1 APP is also Radar |  |
| PETROPAVLOVSK KAMCHATSKY / Yelizovo | 1 |  | 1 |  |  | 1 |  |  |
| PETROZAVODSK/Besovets | 1 |  |  |  |  |  |  |  |
| PROVIDENIYA BAY | 1 |  |  |  |  |  |  |  |
| PSKOV | 1 |  |  |  |  |  |  |  |
| ROSTOV-NA-DONU | 3 |  | 1 | 2 |  | 1 | + 1 (Emergency FREQ) + 1 (Reserve FREQ) + 2 (Transit) |  |
| SAMARA/Kurumoch | 2 |  | 1 | 2 |  | 2 | + 1 (Emergency FREQ) + 1 (VOLMET) + 1 (Transit) |  |
| SANKT-PETERBURG/Pulkovo | 4 |  | 1 | 2 |  | 2 | + 1 (Emergency FREQ) + 2 (Reserve FREQ) |  |
| SARATOV/Tsentralny | 2 |  | 1 |  |  | 1 | + 1 (Transit) |  |
| SOCHI | 3 |  | 1 | 1 |  | 2 | + 1 (Transit) |  |
| STAVROPOL/Shpakovskoye | 2 |  | * | 1 |  |  | 1 APP is also Radar |  |
| SURGUT | 2 |  | 1 | 1 |  | 1 | + 1 (Emergency FREQ) + 2 (Reserve FREQ) + 1 (Transit) |  |
| SYKTYVKAR | * |  | 1 | * |  |  | 1 Radar is also TWR, APP; + 1 (Emergency FREQ) + 1 (Transit) + 1 (MET) |  |
| TYUMEN/Roshchino | 2 |  | 1 | 1 |  | 1 |  |  |
| UFA | 3 |  | 1 | * |  | 2 | Ufa-Control serves as APP; + 1 (Transit) |  |
| ULAN-UDE/Mukhino | 1 |  | 1 | 1 |  |  | + 1 (MET) |  |
| ULYANOVSK/Vostochny | 1 |  | 1 | 1 |  |  |  |  |
| VLADIKAVKAZ/Beslan | 1 |  | 1 |  |  | 1 | + 1 (Transit) |  |
| VLADIVOSTOK/Knevichi | 2 |  | 1 |  |  | 2 | + 1 (Emergency FREQ) + 1 (Reserve FREQ) + 1 (VOLMET) |  |
| VOLGOGRAD/Gumrak | 2 |  | 1 | 1 |  | 2 | + 1 (Emergency FREQ) + 1 (Reserve FREQ) |  |
| VORONEZH/Chertovitskoye | 2 |  |  | 1 |  | 1 | + 1 (Emergency FREQ) + 1 (Reserve FREQ) + 1 (Transit) |  |
| YAKUTSK | 2 |  | 1 | 1 |  | 1 |  |  |
| YAROSLAVL/Tunoshna | * |  | * | 1 |  | 1 | 1 APP is also TWR, Radar |  |
| YEKATERINBURG/Koltsovo | 2 |  | 1 | 2 |  | 1 | + 1 (Emergency FREQ) |  |
| YUZHNO-SAKHALINSK/ Khomutovo | * |  | 1 | * |  | 1 | 1 Radar is also TWR, APP |  |
| SAN MARINO |  |  |  |  |  |  |  |  |
| No information |  |  |  |  |  |  |  |  |
| SERBIA |  |  |  |  |  |  |  |  |
| BEOGRAD/Nikola Tesla | 2 |  | 1 | 1 |  | 1 |  |  |
| NIS/Konstantin Veliki | 1 |  | 1 |  |  | 1 |  |  |
| PRISTINA/Pristina | 1 |  | 1 |  |  |  |  |  |
| VRSAC/Vrsac | 1 |  | 1 |  |  |  |  |  |
| SLOVAKIA |  |  |  |  |  |  |  |  |
| BRATISLAVA/M.R. Stefanik | 3 |  | 2x |  |  | 1 | x Radar |  |
| KOSICE | 1 |  | 1x |  |  |  | x Radar |  |
| PIESTANY | 1 |  |  |  |  |  |  |  |
| POPRAD/TATRY | 3 |  |  |  |  |  |  |  |
| SLIAC | 1 |  | 2x |  |  |  | x Radar |  |
| ZILINA | 1 |  |  |  |  |  |  |  |
| SLOVENIA |  |  |  |  |  |  |  |  |
| LJUBLJANA/Brnik | 1 |  | 1 |  |  |  |  |  |
| MARIBOR/Maribor | 1* |  | 1 |  |  |  |  |  |
| PORTOROZ/Portoroz | 1* |  | 1 |  |  |  |  |  |
| SPAIN |  |  |  |  |  |  |  |  |
| ALICANTE/Alicante | 1 |  |  |  | 1 | 1 | + AS |  |
|  | 1+ |  |  |  |  |  |  |  |
| ALMERIA/Almeria | 1* |  | 1 |  |  |  | + AS |  |
|  | 1+ |  |  |  |  |  |  |  |
| AVILES/Asturias | 1* |  | 2 |  |  |  | + AS |  |
|  | 1+ |  |  |  |  |  |  |  |
| BARCELONA/Barcelona | 2 |  | 1 | 1 | 5 | 1 | + AS |  |
|  | 4+ |  |  |  |  | 1& | & DEP |  |
| BILBAO/Bilbao | 1 |  |  | 2 |  | 1 | + AS |  |
|  | 1+ |  |  |  |  |  |  |  |
| BURGOS/Burgos | 1+ | 1 |  | 1 |  |  | + AS |  |
|  |  |  |  |  |  |  | Aerodromes not contained in Table AOP-1 |  |
| CORDOBA/Cordoba | 1+ | 1 |  |  |  |  | + AS |  |
|  | 1 |  |  |  |  |  | Aerodrome not contained in Table AOP-1 |  |
| GERONA/Costa Brava | 1 |  |  | 1 |  | 1 | + AS |  |
|  | 1+ |  |  |  |  |  |  |  |
| GRANADA/Granada | 1+ |  | 1 |  |  |  | + AS |  |
| HUESCA/Huesca |  |  | 1 |  |  |  | Aerodrome not contained in Table |  |
|  |  |  |  |  |  |  | AOP-1 |  |
| IBIZA/Ibiza | 1 |  |  |  | 2 | 1 | + AS |  |
|  | 2+ |  |  |  |  |  |  |  |
| JEREZ/Jerez | 1 |  |  |  |  | 1 | + AS |  |
|  | 1+ |  |  |  |  |  |  |  |
| LA CORUÑA/La Coruña | 1 |  |  |  |  |  | + AS |  |
|  | 1+ |  |  |  |  |  |  |  |
| LEON/Léon | 1 |  |  |  |  |  | Aerodrome not contained in Table AOP-1 |  |
| LOGROÑO/Logroňo |  |  | 1 |  |  |  | Aerodrome not contained in Table AOP-1 |  |
| MADRID/Barajas | 5 |  | 4 | 3 | 5 | 1 | & DEP |  |
|  | 6+ |  |  |  |  | 1 & | + AS |  |
| MADRID/Cuatro Vientos | 1 |  |  |  |  | 1 | IGA |  |
|  | 1+ |  |  |  |  |  | + AS |  |
|  | 1 ∏ |  |  |  |  |  | ∏ HEL MIL |  |
| MAHON/Menorca | 1* |  | 1 |  |  | 1 | + AS |  |
|  | 1 |  |  |  |  |  |  |  |
|  | 1+ |  |  |  |  |  |  |  |
| MALAGA/Málaga | 2 |  | 1 | 1 | 1 | 1 | + AS |  |
|  | 3+ |  |  |  |  |  |  |  |
| MELILLA/Melilla | 1 |  |  |  |  |  | + AS |  |
|  | 1+ |  |  |  |  |  | ∏ VFD |  |
|  | 1 ∏ |  |  |  |  |  | Aerodrome not contained in Table AOP-1 |  |
| MURCIA/San Javier | 1 |  |  |  | 1 |  | + AS |  |
|  | 1* |  |  |  |  |  |  |  |
|  | 1+ |  |  |  |  |  |  |  |
| PALMA DE MALLORCA/ Palma de Mallorca | 3 |  |  |  |  |  |  |  |
|  | 3+ |  | 2 | 1 | 4 | 1 | + AS |  |
| PAMPLONA/Noain-Pamplona | 1* |  | 1 |  |  |  | IGA – Aerodrome not contained in Table AOP-1 |  |
| REUS/Reus | 1 |  | 1 |  |  |  | + AS |  |
|  | 1+ |  |  |  |  |  |  |  |
| SABADELL/Sabadell | 1 |  |  |  |  |  | IGA |  |
|  | 1+ |  |  |  |  |  | + AS |  |
| SALAMANCA/Matacan | 1 |  | 1 |  |  |  | + AS |  |
|  | 1+ |  |  |  |  |  |  |  |
| SAN SEBASTIAN/San Sebastian | 1* |  | 1 |  |  |  | + AS |  |
|  | 1+ |  |  |  |  |  |  |  |
| SANTANDER/Santander | 1 |  |  | 1 |  |  | + AS |  |
|  | 1+ |  |  |  |  |  |  |  |
| SANTIAGO/Santiago | 1 |  | 1 |  | 1 |  | + AS |  |
|  | 1+ |  |  |  |  |  |  |  |
| SEVILLA/Sevilla | 1 |  | 1 |  | 2 | 1 | + AS |  |
|  | 1+ |  |  |  |  |  |  |  |
| VALENCIA/Valencia | 1 |  | 1 |  | 3 | 1 | + AS |  |
|  | 1+ |  |  |  |  |  |  |  |
| VALLADOLID/Villanubla | 1 |  |  |  |  |  |  |  |
| VIGO/Vigo | 1 |  |  |  |  |  | + AS |  |
|  | 1+ |  |  |  |  |  |  |  |
| VITORIA/Vitoria | 1 |  |  |  |  |  | + AS |  |
|  | 1+ |  |  |  |  |  |  |  |
| ZARAGOZA/Zaragoza | 1 |  |  | 2 | 1 |  | + AS |  |
|  | 1+ |  |  |  |  |  |  |  |
| SWEDEN |  |  |  |  |  |  |  |  |
| ARVIDSJAUR | 1x | 1x |  |  |  |  | x TWR/AFIS |  |
| BORLÄNGE | 2 |  |  |  |  |  |  |  |
| ESKILSTUNA |  | 1 |  |  |  |  |  |  |
| GÄLLIVARE |  | 1 |  |  |  |  |  |  |
| GÄVLE |  | 1 |  |  |  |  |  |  |
| GÖTEBORG/Landvetter | x3 |  |  |  | 2 | 1+ | + VFR x FL 200 Also Säve |  |
| GÖTEBORG/Säve | 1 |  |  |  | 1x |  | IGA x FL 200 |  |
| HAGFORS |  | 1 |  |  |  |  |  |  |
| HALMSTAD | 2 |  |  |  |  |  |  |  |
| HEMAVAN-TÄRNABY |  | 1 |  |  |  |  |  |  |
| JÖNKÖPING | 1x |  | 1xx |  |  | 1 | x 1985 xx 1985 Also TWR |  |
| KALMAR | 1 |  | 1 |  |  |  | No information |  |
| KARLSTAD | 1* |  |  |  |  |  |  |  |
| KIRUNA | 1* |  |  |  | 1 |  | IGA |  |
| KRAMFORS-SOLLEFTEÅ |  | 1 |  |  |  |  |  |  |
| KRISTIANSTAD | 1 |  |  |  |  |  |  |  |
| LIDKÖPING |  | 1 |  |  |  |  |  |  |
| LINKÖPING/Malmen | 3 |  | 1 |  |  |  |  |  |
| LINKÖPING/Saab | 4 |  |  |  |  |  |  |  |
| LJUNGBYHED | 2 |  |  | 1 |  | 1 |  |  |
| LULEÅ/Kallax | 1 |  |  | 2 |  |  |  |  |
| LYCKSELE |  | 1 |  |  |  |  |  |  |
| MALMÖ/Sturup | 1 |  | 1x |  |  | 1+ | x FL 60 + VOR |  |
| MORA/Siljan |  | 1 |  |  |  |  |  |  |
| NORRKÖPING/Kungsängen | 1* |  | 1 |  |  |  |  |  |
| OSKARSHAMN |  | 1 |  |  |  |  |  |  |
| PAJALA-YLLÄS |  | 1 |  |  |  |  |  |  |
| RONNEBY | 2 |  | 2 | 1 |  |  |  |  |
| SKELLEFTEÅ | 1 |  |  |  |  |  |  |  |
| SKÖVDE |  | 1 |  |  |  |  |  |  |
| STOCKHOLM/Arlanda | X7 |  | 1x | 1 | 2xx | X2 | x FL 60 xx FL 200 |  |
| STOCKHOLM/Bromma | 1 |  | 1x |  |  | 1 | IGA x FL 60 |  |
| STOCKHOLM/ Skavsta | 1 |  |  |  |  | 1 |  |  |
| STOCKHOLM/Västerås | 1 |  | 1 |  |  |  |  |  |
| STORUMAN |  | 1 |  |  |  |  |  |  |
| SUNDSVALL-HÄRNÖSAND | 2 |  |  | 1 |  | 1+ | + VOR |  |
| SÅTENÄS | 1 |  | 2 |  |  |  |  |  |
| SÖDERHAMN |  | 1 |  |  |  |  |  |  |
| TROLLHÄTTAN-VÄNERSBORG | 1 |  |  |  |  |  |  |  |
| UMEA/Umea | 2 |  |  | 1 |  |  |  |  |
| VILHELMINA |  | 1 |  |  |  |  |  |  |
| VISBY/Visby | 1 |  |  |  | 1 |  |  |  |
| VÄXJÖ/Kronoberg | 1 |  |  |  |  |  |  |  |
| ÅRE ÖSTERSUND | 1 |  |  | 1 |  |  |  |  |
| ÄNGELHOLM | 1 |  |  | 1 |  |  |  |  |
| ÖREBRO | 1 |  | 1 |  |  |  |  |  |
| ÖRNSKÖLDSVIK | 1 |  |  |  |  |  |  |  |
| SWITZERLAND |  |  |  |  |  |  |  |  |
| BASEL-MULHOUSE/Basel-Mulhouse |  |  |  |  |  |  | see under France: Bâle-Mulhouse |  |
| BERN/Bern-Belp | 1 |  | 1 |  |  | 1 |  |  |
| BUOCHS/Buochs | 1 |  |  |  |  |  |  |  |
| GENÈVE/Genève | 4 |  |  | 5 | 1 | 1 |  |  |
| GRENCHEN/Grenchen | 3 |  |  |  |  | 1 | IGA |  |
| LA CHAUX-de-FONDS/La Chaux-de-Fonds-Les Eplatures | 1 |  |  |  |  |  | IGA |  |
| LUGANO/ Lugano-Agno | 1 |  |  |  |  | 1 |  |  |
| SAMEDAN/Samedan | 1 |  |  |  |  |  | IGA |  |
| SION/Sion | 1 |  |  | 1 |  |  |  |  |
| ST. GALLEN/St. Gallen-Altenrhein | 1 |  |  |  |  | 1 |  |  |
| ZÜRICH/Zürich | 5*;4+ |  | 1 | 5 | 1 | 2 | 5 ATC; 4 AMS |  |
| TAJIKISTAN |  |  |  |  |  |  |  |  |
| DUSHANBE/Dushanbe | 1 |  |  | 1 |  | 1 |  |  |
| THE FORMER YUGOSLAV REPUBLIC OF MACEDONIA |  |  |  |  |  |  |  |  |
| OHRID/Ohrid | 1* |  | 1 |  |  |  |  |  |
| SKOPJE/Petrovac | 1 |  | 1 |  |  |  |  |  |
| TURKEY |  |  |  |  |  |  |  |  |
| ATATÜRK | 4 |  |  | 3 |  | 1 |  |  |
| ESENBOĞA | 2 |  |  | 4 |  | 1 |  |  |
| A.MENDERES | 2 |  |  | 6 |  | 1 |  |  |
| ANTALYA | 2 |  |  | 5 |  | 1 |  |  |
| DALAMAN | 2 |  |  | 4 |  | 1 |  |  |
| ADANA | 2 |  |  | 1* |  | 1 | * at Adana/ İncirlik |  |
| TRABZON | 1 |  |  | 1 |  | 1 |  |  |
| MİLAS-BODRUM | 2 |  |  |  |  | 1 |  |  |
| S. DEMİREL | 1 |  |  |  |  |  |  |  |
| N.KAPADOKYA | 1 |  |  |  |  | 1 |  |  |
| ADIYAMAN | 2 |  |  |  |  |  |  |  |
| AGRI | 1 |  |  |  |  |  |  |  |
| B. YENİŞEHİR | 1 |  |  |  |  |  |  |  |
| ÇANAKKALE | 1 |  |  |  |  |  |  |  |
| ÇARDAK | 1 |  |  | 1 |  |  |  |  |
| ÇORLU | 1 |  |  |  |  |  |  |  |
| ELAZIĞ | 1 |  |  |  |  |  |  |  |
| ERZİNCAN | 1 |  |  |  |  |  |  |  |
| ERZURUM | 1 |  |  |  |  | 1 |  |  |
| G.ANTEP | 2 |  |  | 2 |  | 1 |  |  |
| K.M ARAŞ | 1 |  |  |  |  |  |  |  |
| KARS | 1 |  |  |  |  |  |  |  |
| KÖRFEZ/EDREMİT | 1 |  |  |  |  |  |  |  |
| MARDİN | 1 |  |  |  |  |  |  |  |
| MERZİFON | 1 |  |  |  |  |  |  |  |
| MUŞ | 1 |  |  |  |  |  |  |  |
| SAMSUN ÇARŞAMBA | 2 |  |  | 2 |  | 1 |  |  |
| SİİRT | 1 |  |  |  |  |  |  |  |
| SİNOP | 1 |  |  |  |  |  |  |  |
| SİVAS | 1 |  |  |  |  |  |  |  |
| Ş.URFA-GAP | 1 |  |  | 1 |  |  |  |  |
| TOKAT | 1 |  |  |  |  |  |  |  |
| UŞAK | 1 |  |  |  |  |  |  |  |
| (VAN (F.MELEN) | 1 |  |  | 1 |  |  |  |  |
| ZONGULDAK/ÇAYCUMA | 1 |  |  |  |  |  |  |  |
| BATMAN | 1 |  |  |  |  |  |  |  |
| GÖKÇEADA | 1 |  |  |  |  |  |  |  |
| TURKMENISTAN |  |  |  |  |  |  |  |  |
| ASHKABAD/Ashkabad |  |  |  |  |  |  | No information |  |
| KRASNOVODSK |  |  |  |  |  |  | No information |  |
| TASHAUZ |  |  |  |  |  |  | No information |  |
| UKRAINE |  |  |  |  |  |  |  |  |
| CHERKASY |  |  | 1* |  |  |  |  |  |
| CHERNIVTSI |  |  | 1* |  |  |  |  |  |
| DNIPROPETROVS’K | 2 |  |  | 1 |  | 1 | AS |  |
| DONETS’K | 2 |  | 1 |  | 1 | 1 | AS |  |
| IVANO-FRANKIVS’K | 1 |  | 1 |  |  | 1 |  |  |
| KHARKIV | 1 |  | 1 | 1 | 1 | 2 |  |  |
| KRYVYI RIH |  |  | 1* |  |  | 1 |  |  |
| KYIV (Antonov) | 1 |  | 1 |  |  |  |  |  |
| KYIV (Boryspil’) | 4 1,2 |  | 2 |  | 3 | 2 | 1 AS, |  |
|  |  |  |  |  |  |  | 2 DEP |  |
| KYIV (Zhuliany)) | 2 |  | 1 |  |  | 1 | AS |  |
| L ‘VIV | 2 |  |  | 1 |  | 2 | AS |  |
| LUHANS’K | 1 |  |  |  | 1 | 1 |  |  |
| MARIUPOL’ |  |  | 1* |  |  |  |  |  |
| MYKOLAIV |  |  | 1* |  |  | 1 |  |  |
| ODESSA | 2 |  | 1 |  | 1 | 2 | AS |  |
| RIVNE |  |  | 1* |  |  |  |  |  |
| SIMFEROPOL’ | 2 |  | 1 |  | 1 | 2 | AS |  |
| UZHOROD | 1 |  | 1 |  |  |  |  |  |
| ZAPORIZHZHIA | 1 |  | 1 |  |  | 1 |  |  |
| UNITED KINGDOM |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 1 vor |  |
|  |  |  |  |  |  |  | 2 Departure ATIS |  |
| ABERDEEN | 1 |  | 13 |  | 1 | 212 | 1 VOR 2 Departure ATIS 3 Radar |  |
| Belfast Aldergrove | 1 |  | 1 1 |  | 1 | 1 | 1 Radar |  |
| Biggin Hill | 1 |  | 1 |  |  | 1 1 | 1 Departure ATIS |  |
| Birmingham | 1 |  | 3 1 |  |  | 1 | 1 1 Radar |  |
| Blackpool | 1 |  | 2 1 |  |  | 1 | 1 1 Radar |  |
| Bournemouth | 1 |  | 2 1 |  |  | 1 | 1 1 Radar |  |
| Bristol | 1 |  |  | 2 1 |  | 1 | 1 2 Radar |  |
| Cambridge | 1 |  | 1 | 1 1 |  | 1 | 1 1 Radar |  |
| Cardiff | 1 |  |  | 2 1 |  | 1 | 1 1 Radar |  |
| Coventry | 1 |  |  | 2 1 |  |  | 1 1 Radar |  |
| Durham Tees Valley | 1 |  |  | 2 1 |  | 1 | 1 1 Radar |  |
| East Midlands | 1 |  |  | 1 1 | 1 | 1 | 1 Radar |  |
| Edinburgh | 1 |  | 2 1 |  |  | 1 | 1 1 Radar |  |
| Exeter | 1 |  | 1 1 | 1 |  | 1 | 1 Radar |  |
| Glasgow | 1 |  | 3 1 |  |  | 1 | 1 2 Radar |  |
| Humberside | 1 |  |  | 2 1 |  | 1 | 1 1 Radar |  |
| Isle of Man | 1 |  | 3 1 |  |  | 1 | 1 2 Radar |  |
| Leeds Bradford | 1 |  | 2 1 |  |  | 1 | 1 1 Radar |  |
| Liverpool | 1 |  | 2 1 |  |  | 1 | 1 1 Radar |  |
| London City | 2 |  | 2 1 |  |  | 1 | 1 1 Radar |  |
| London Gatwick | 1 1 |  | 1 | 2 |  | 1 | 1 Also APP |  |
|  |  |  |  |  |  |  | 1 2 on VOR |  |
|  |  |  |  |  |  |  | 2 1 Departure ATIS |  |
| London Heathrow | 3 |  | 23 |  |  | 412 | 3 1 Radar |  |
| London Luton | 1 |  | 2 1 |  |  | 1 | 1 1 Radar |  |
|  |  |  |  |  |  |  | 1 1 Radar |  |
| London Stansted | 2 |  | 1 1 | 1 |  | 22 | 2 1 VOR |  |
| Lydd | 1 |  | 1 |  |  | 1 |  |  |
|  |  |  |  |  |  |  | 1 Radar |  |
|  |  |  |  |  |  |  | 2 1 VOR |  |
| Manchester | 2 |  | 11 | 21 |  | 233 | 3 1 Departure ATIS |  |
| Manston | 1 |  | 1 1 | 1 |  | 1 | 1 1 Radar |  |
| Newcastle | 1 |  |  |  | 2 1 | 1 | 1 1 Radar |  |
| Norwich | 1 |  | 1 1 |  | 1 | 1 | 1 1 Radar |  |
| Plymouth | 1 |  |  | 1 |  |  |  |  |
| Prestwick | 2 |  |  | 2 1 |  |  | 1 1 Radar |  |
| Shoreham | 1 |  | 1 |  |  | 1 |  |  |
| Southampton | 1 |  | 1 |  |  | 1 1 | 1 VOR |  |
| SOUTHEND | 1 |  | 21 |  |  | 1 | 1 1 Radar |  |
| Sumburgh | 1 |  |  | 2 1 |  | 1 | 1 2 Radar |  |
| UZBEKISTAN |  |  |  |  |  |  |  |  |
| BHUKHARA | 1 |  |  | 1 |  |  |  |  |
| SAMARKAND/Samarkand | 1 |  | 1 | 1 |  |  |  |  |
| TASHKENT/Yuzhny | 1 | 1 | 1 | 1 | - | 1 |  |  |
| URGENCH | 1 |  |  | 1 |  |  |  |  |
| TERMEZ | 1 |  |  |  |  |  |  |  |

SUPPLEMENT Table CNS 3 — AERONAUTICAL MOBILE SERVICE (VHF) —

FLIGHT INFORMATION AND AREA CONTROL SERVICE

EXPLANATION OF THE TABLE

Column

1	Name/location of station.

2	Channel(s) for flight information service (FIS).

3	Channel(s) for area control service in the lower airspace (ACC/L).

4	Channel(s) for area control service in the upper airspace (ACC/U).

5	Supplementary information.

Note 1.— In cases where more than one function is combined in one ATS radio control position, the channel concerned is listed only in that column corresponding to the highest requirements in operational range and height.

Note 2.— The planning criteria to be used for operational coverage of the various services are those shown in the Table of Uniform Values contained in Attachment B to Part V.II — ATS, unless otherwise specifically indicated.

Note 3.— The specific frequencies assigned to the channels are shown in Supplement Table COM 2 of Part IV — CNS.

Note 4.— Whenever the FIS channel does not cover the entire FIR, its area of use is indicated broadly either by E, W, N or S or by indicating the sector number as used to describe the area of coverage of ACC channels.

Symbols/abbreviations used and their meaning

DEP	For departing aircraft

ER	Extended range VHF

HEL	For helicopter operations

.

| Location | Lower airspace |  | Upper airspace | Remarks |
| --- | --- | --- | --- | --- |
|  | FIR | ACC-L |  |  |
| 1 | 2 | 3 | 4 | 5 |
| ALBANIA |  |  |  |  |
| Tirana |  | 1(1) |  | ACC |
| ARMENIA |  |  |  |  |
| Yerevan ACC | 1 | 1(1) | 1(1) | ** FL 150 |
| Yerevan AFIS | 1 | 1 |  | ** FL 115 |
| AUSTRIA |  |  |  |  |
| Wien ACC | 1 1W | 1(1)* 1(2)* 1(3)* 1(4)* | 1(1) 1(2) 1(3) | * FL 300 |
| AZERBAIJAN |  |  |  |  |
| Baku ACC |  | 1 | 1(1) 1(2) 1(3) 1(4) 1 |  |
| BELARUS |  |  |  |  |
| Minsk ACC | 4* |  | 1 | * FIS: below FL50-GND |
|  |  |  | 2(S1S) | + VOLMET |
|  |  |  | 2(S2W1) |  |
|  |  |  | 2(S3N) |  |
|  |  |  | 2(S4C) |  |
|  |  |  | 2(S5W2) |  |
|  |  |  | 1(S6C2) |  |
|  |  |  | 2(S7W3) |  |
|  |  |  | 2(S8S2) |  |
|  |  |  | 1(S9S3) |  |
|  |  |  | 2+ |  |
| BOSNIA AND HERZEGOVINA |  |  |  |  |
| Sarajevo ACC | 1 | 1(LE) Note 1 | 1 (UN) Notes 2, 3 1 (US) Notes 2, 3 1 (UW) Notes 2, 3 1 (TUN) Notes 2, 4 1 (TUS) Notes 2, 4 1 (TUW) Notes 2, 4 | Note 1: up to FL 195 Note 2: ATS provided by Zagreb ACC Note 3: up to FL 360 Note 4: above FL 360 |
| BULGARIA |  |  |  |  |
| Sofia ACC | 1 | 1(1) 1(2) † 1(2) †† 1(3) | 1 * 1 1(1) 1(2) 1(3) | † Also APP/L †† Radar * Training |
| Varna ACC | 1 | 1 1(1) | 2 |  |
| CROATIA |  |  |  |  |
| Zagreb ACC | 1 | 1(LE) 1(LS) 1(LW) 1(LN) | 1(UN) Note 3 1(US) Note 3 1(UW) Note 3 1(TUN) Note 4 1 (TUS) Note 4 1 (TUW) Note 4 | Note 3: up to FL 360 Note 4: above FL 360 |
| CYPRUS |  |  |  |  |
| Nicosia ACC |  | 1(1)LW | 1(1)W 1(2)E 1S |  |
| CZECH REPUBLIC |  |  |  |  |
| Praha ACC | 2(1*) | 5 (5*) | 8 (12*)2**(12*) | * the Number of ACC elementary sectors** UHF |
| PRAHA MACC | 1 | 4 | 2 |  |
| DENMARK |  |  |  |  |
| Copenhagen ACC | 1(1) 1(2) 3(4) | 1(1) | 4(1) 3(2) 3(3) 2(4) |  |
| ESTONIA |  |  |  |  |
| Tallin ACC |  | 1(2) | 1(2) |  |
| FINLAND |  |  |  |  |
| Tampere ACC |  |  | 1(1) 1(2) 1(1+2) |  |
|  |  |  | 2(1) 2(2) 1(3) 1(4) 1(5) 1(4+5) 1(1+2+3+4+5) |  |
| FRANCE |  |  |  |  |
| Bordeaux ACC | 1(1+2) 1(3+4) | 1(1) 2(2) 3(4) 2(3+4) 1(1+2+3+4) | 2(2) 1(1+4) 2(3+4) 1(1+2+3) 1(1+3+4) 1(1+2+3+4) |  |
| Brest ACC | 1(2) 1(1+2) | 1(2) 1(1+2) | 1(2) 6(1+2) |  |
| France UAC |  |  | 1(1)ø 1(2)ø 1(3)ø 3(4)† 1(5)† 1(6)† 2(7)† 1(8)*** 1(9)*** 1(9) 1(10)#***†† 3(14)ø 2(15)ø 2(16)† 1(17)*** 1(17)***øø 1(18)ø 1(1+2)ø** 1(1+2+13)ø 1(3+14)ø 1(4+5+16)†*** 1(4+16)† 1(5+6)†** 1(8+9) 1(8+9+17) 1(8+17)** 4(10+11)#*** 1(11+12+13+18)ø *** o 1(12+13)ø*** 1(12+13)ø** 1(15+18)ø 1(15+18)ø*** | ø Paris/Reims † Marseille *** FL 600  Bordeaux # Brest †† ER S-SW Bordeaux øø S of 46N ** FL 320 o SAT/SST |
| Marseille ACC | 1(1+4) 1(3) 1(2) | 1(1) 5(2) 3(3) 2(4) | 2(1) 3(2) 2(3) 2(4) 1(1+4) 1(2+3) 1(1+2+4) 1(1+2+3+4) |  |
| Paris ACC | 1(1+2+6) 1(3+4) 1(5) | 1(1) 3(3) 1(4) 2(5) 1(6) 1(1+2) 2(3+4) 1(5+6) | 1(1) 1(4) 3(5) 3(6) 2(1+2) 1(3+4) | Refers to upper airspace sectors in Paris and Reims ACCs |
| Reims ACC | 1 | 1(1) 1(2) | 3(4+5) 1(1+2+6) 3(1+2+3) 1(4+5+6) |  |
| GEORGIA |  |  |  |  |
| Sukhumi ACC |  | 1 1(1) 1(2) | 1(1) 1(1) |  |
| Tbilisi |  | 1(1) | 1(1) 1† | † Volmet |
| GERMANY |  |  |  |  |
| Bremen ACC | 3 (1) | 18 (16) |  |  |
|  |  | 1 |  |  |
|  |  | 1 |  | APP EDDH |
|  |  | 1 |  | APP EDDB, EDDT |
|  |  | 1 |  | APP EDDW |
|  |  | 1 |  | APP EDDV |
|  |  |  | 1† | † Volmet Bremen/Berlin |
| Largen ACC | 4 (2) | 18(7) |  |  |
|  |  | 1 |  |  |
|  |  | 1 |  | APP EDGS |
|  |  | 1 |  | APP EDFQ |
|  |  | 1 |  | EDFH |
|  |  | 1 |  | EDDR and EDRZ |
|  |  | 1 |  | EDFM and ETOR |
|  |  | 1 |  | EDTY and EDDS |
|  |  | 1 |  | EDDG |
|  |  | 2 |  | APP EDLW and EDLP |
|  |  | 1 |  | APP EDDK |
|  |  |  |  | APP EDLN and EDLV |
|  |  | 2 |  | And ETNG and ETNN |
|  |  |  | 2† | † Volmet Frankfurt |
| Munchen ACC | 4(2) | 11(4) |  |  |
|  |  |  | 13(3)* | *Up to FL 195 |
|  |  | 1 |  | APP EDQC, EDQD, EDQM |
|  |  |  |  | And EDDN |
|  |  | 1 |  | APP EDDP, EDAC, EDDE |
|  |  |  |  | APP EDDC, EDAB |
|  |  |  |  | (Hannover) |
| Maastricht UAC (German Part) |  |  | 7(6) | Jever,Deco, Hostein, Celle Ruhr,Münster,Solling, Flovo |
|  |  |  | 2 | Lippe Radar |
| Karlsruhe UAC |  |  | 28(4) | East |
|  |  |  |  | Center |
|  |  |  |  | West |
| GREECE |  |  |  |  |
| Athinai ACC | 3x 1xx | 1‡ (KRKL) 1‡ (KFNL) 1‡ (MILL) 1‡ (RDSL) | 3*(U) 1‡ (KRKU) 1‡ (KFNU) 1‡ (PLH) 1‡ (MILU) 1‡ (RDSU) 1‡(SIT) 1‡(KAV) | *common U & L channels for all Athinai ACC (LGGG) x common ch for both LGGG and LGMD Accs xx planned ‡ ch and operational coverage are shown in Supplement Table COM2 of Part IV – CNS |
| Makedonia ACC | 3x 1xx | 1‡(WL) 1‡(LMOL) 1‡(KVLL) | 2**(U) 1‡(TSLU) 1‡(SKPU) 1‡(LMOU) 1‡(KVLU) | **common U & L channels for all Makedonia ACC (LGMD) |
| HUNGARY |  |  |  |  |
| Budapest ACC | 1E 1W | 1(1)E 1(2)W 1(3)S 1† | 1(1)E 1(2)W 1(3)S | † TMA FL 200 |
| IRELAND |  |  |  |  |
| Shannon ACC | 1S 1N | 1(1)N 1(2)S | 1* 1* 1* 1* | * FL 640 W of 0800W |
|  |  |  | 1* |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
| Dublin ACC | 1 | 1(1)N |  |  |
|  |  | 1(2)S |  |  |
| ITALY |  |  |  |  |
| Brindisi ACC | 1N-S 1S | 1(2) 1(3) 1(1+2) |  |  |
| Italia UAC |  |  | 1(5) 2(6) 1(7) 1(7) 1(8) 1(10) 1(11)* 1(12) 1(13) 1(14) 1(15)† 1(1+2) 1(2+1) 1(3+4) 1(4+3) 1(5+6) 1(8+11)** 1(9+10) 1(13+14)ø |  Also Roma ACC-L(7) * Also Roma ACC-L(1) † Also Roma ACC-L(9) ** Also Roma ACC-L(8) ø Also Roma ACC-L(4) |
| Milano | 1E 1SE 2W | 2(1) 1(2) 2(4) 1(5) 1(5+3) | 1(3) 1(9) 1(10) |  |
| Roma ACC | 1N 1S 1SW | 1(2) 1(3) 1(5) 1(10) 1(11) 1(12) |  |  |
| KAZAKHSTAN |  |  |  |  |
| AKTOBE | 1 |  |  |  |
| ALMATY | 1 |  |  |  |
| ASTANA | 1 |  |  |  |
| SHYMKENT | 1 |  |  |  |
| KYRGYZSTAN |  |  |  |  |
| Bishkek ACC |  | 1 | 1 |  |
|  |  | 1 (East) | Since 2012 |  |
|  |  | 1 (West) | Since 2012 |  |
| Osh ACC |  |  | 1 | Till 2012 |
| LATVIA |  |  |  |  |
| Riga ACC |  | 1(2) E 1(2) W 1(2) N | 1(2) E 1(2) W 1(2) N | E – East sector W – West Sector N – North sector |
| LITHUANIA |  |  |  |  |
| Vilnius ACC | 1(1) | 1(1)* 1(2)* | 2(1) 2(2) | * FL 200 |
| MALTA |  |  |  |  |
| Malta ACC |  |  | 1† 1‡ | † FIS ‡ ER SE |
| NETHERLANDS |  |  |  |  |
| Amsterdam ACC | 6ø | 14* | 6 | FIS above North Sea and within Amsterdam FIR *incl. backup up to FL245 |
| Nieuw Milligen ACC |  | 4† |  | †‡ FL 120 |
| NORWAY |  |  |  |  |
| Bodø ACC | 1 |  | 4 | Combined Upper/Lower |
| Oslo ACC | 1 |  | 5 | Combined Upper/Lower |
| Stavanger ACC | 2 |  | 3 | Combined Upper/Lower |
| POLAND |  |  |  |  |
| Warszawa ACC | 2E 1W 1N 1S |  | 1(G) 1(J) 1(S) 1(T) 1(D) 1(C) 1(B) 1(R) 1(E) 1* | Depend on volume of traffic combinations of sectors are used |
|  |  |  |  | * Volmet Warszawa |
| PORTUGAL |  |  |  |  |
|  | 1 | 6 | 6 |  |
|  |  | 2(1) | 2(1) | Upper north |
|  |  | 2(2) | 2(2) | Upper centre |
|  |  | 2(3) | 2(3) | South |
|  |  |  | 1(4) |  |
|  |  | 2(4) | 2(4) | Demos |
|  |  | 2(5) | 2(5) | Veram |
|  |  |  | 1(6) |  |
|  |  | 2(6) | 2(6) | madeira |
| REPUBLIC OF MOLDOVA |  |  |  |  |
| Chisinau ACC | 1 | 1 | 1 |  |
| ROMANIA |  |  |  |  |
| Bucuresti ACC | 1* 7 1 | - 3(2) 2(3) 2(4) 2(5) 2(7) 2(8) 2(9) 5(10) 1(2)* | 2(2)* 3(2)2(3) 4(4)6(5) 1(5+6+7) 4(6) 4(7) 3(8) 2(9) 9 9 9 - | *BACAU LOCATION ALSO USED FOR ACC-L *CONSTANTA LOCATION |

| Location | Lower airspace |  | Upper airspace | Remarks |
| --- | --- | --- | --- | --- |
|  | FIR | ACC-L |  |  |
| 1 | 2 | 3 | 4 | 5 |
| RUSSIAN FEDERATION |  |  |  |  |
| Sankt-Peterburg ACC |  |  | 1(North) |  |
|  |  |  | 1(South-East) |  |
|  |  |  | 1(South-West) |  |
|  |  |  | 2(West) |  |
|  |  |  | 1(Petrozavodsk) |  |
| Velikiye Luki ACC |  |  | 1 |  |
|  |  |  | 1R |  |
| Vologda ACC |  |  | 1R |  |
|  |  |  | 1(South) |  |
|  |  |  | 2(North) |  |
| Arkhangelsk ACC |  | 1 |  | 6000 m or below |
|  |  |  | 1 | Above 6000m |
| Kotlas ACC |  |  | 1 |  |
|  |  | 1 |  |  |
| Leshukonskoye ACC |  | 1 |  |  |
| Naryan-Mar ACC |  |  | 1 |  |
| Murmansk ACC |  |  | 1 |  |
|  |  |  | 1(Oceanic) |  |
| Syktyvkar ACC |  | 1(1) |  | Below 8850 m |
|  |  |  | 1(1) | 8850 m or above |
|  |  |  | 1(2) |  |
|  |  |  | 1(3) |  |
| Vorkuta ACC |  |  | 1(North) |  |
|  |  |  | 1(Sout) |  |
| Kaliningrad ACC |  |  | 1 |  |
|  |  |  | 1R |  |
| Moscow ACC |  |  | 2R |  |
|  |  |  | 1(North 1) |  |
|  |  |  | 1(Gorkiy 2) |  |
|  |  |  | 1(Gorkiy 3) |  |
|  |  |  | 1(Gorkiy 4) |  |
|  |  |  | 1(Gorkiy 5) |  |
|  |  | 1(Gorkiy 6) |  |  |
|  |  |  | 1(Penza 1) |  |
|  |  | 1(Penza 2) |  |  |
|  |  |  | 1(Voronezh 1) |  |
|  |  |  | 1(Voronezh 2) |  |
|  |  |  | 1(Voronezh 3) |  |
|  |  | 1(Voronezh 4) |  |  |
|  |  |  | 1(Kharkiv 1) |  |
|  |  |  | 1(Kharkiv 2) |  |
|  |  |  | 1(Kyiv 1) |  |
|  |  |  | 1(Kyiv 2) |  |
|  |  |  | 1(West 1) |  |
|  |  |  | 1(West 2) |  |
|  |  |  | 1(Leningrad 1) |  |
| Rostov-na-Donu АСС |  |  | 2R |  |
|  |  |  | 1(1) | below 9300 |
|  |  |  | 1(1А) | 9300 m or above |
|  |  |  | 1(2) |  |
|  |  |  | 1(3) | below 9300 |
|  |  |  | 1(3А) | 9300 m or above |
|  |  |  | 1(4) |  |
|  |  |  | 1(5) |  |
|  |  |  | 1(5)R |  |
|  |  |  | 1(6) |  |
|  |  |  | 1(7) |  |
|  |  |  | 1(8) |  |
|  |  |  | 2(8)R |  |
|  |  |  | 1(9) |  |
|  |  |  | 2(9)R |  |
|  |  |  | 1(10) |  |
|  |  |  | 1(11) |  |
|  |  |  | 1(12) |  |
| Samara ACC |  |  | 1(1(South)) |  |
|  |  |  | 1(1(South))R |  |
|  |  |  | 1(2(West)) |  |
|  |  |  | 1(2(West))R |  |
|  |  |  | 1(3(East)) |  |
|  |  |  | 1(3(East))R |  |
|  |  |  | 1(4(Saratov)) |  |
| Saratov АСС |  | 1 |  | below 6100 |
| Kazan ACC |  |  | 1(North) |  |
|  |  |  | 1(North)R |  |
|  |  |  | 1(South) |  |
|  |  |  | 1(South)R |  |
| Penza ACC |  |  | 1R |  |
|  |  | 1 | 1 |  |
| Orenburg ACC |  |  | 1 |  |
| Ufa ACC |  |  | 1(West) | 7300 m or above |
|  |  | 1(West) |  | Below 7300 m |
|  |  | 1(West)R |  | Below 7300 m |
|  |  |  | 1(East) | 7300 m or above |
|  |  | 1(East) |  | Below 7300 m |
|  |  | 1(East)R |  | Below 7300 m |
| Yekaterinburg ACC |  |  | 1O/R |  |
|  |  |  | 1(Ural 1) |  |
|  |  |  | 1(Ural 2) |  |
|  |  |  | 1(Ural 3) |  |
|  |  |  | 1(Ural 4) |  |
| Kirov ACC |  |  | 1(West) |  |
|  |  |  | 1(East) |  |
| Perm ACC |  |  | 1(North) |  |
|  |  |  | 1(South) |  |
| Chelyabinsk ACC |  |  | 1(East) |  |
|  |  |  | 1(West) |  |
| Magnitogorsk ACC |  |  | 1 |  |
| Tyumen ACC |  |  | 1(1) |  |
|  |  |  | 1(2) |  |
|  |  |  | 1(3) |  |
|  |  |  | 1(4) |  |
| Kamenny Mys ACC |  |  | 1 |  |
| Salekhard ACC |  |  | 1 |  |
| Surgut ACC |  |  | 1 |  |
| Tarko-Sale ACC |  |  | 1 |  |
| Khanty-Mansiysk ACC |  |  | 1 |  |
| Novosibirsk ACC |  |  | 1(1) |  |
|  |  |  | 1(2) |  |
|  |  |  | 1(3) |  |
|  |  |  | 1(4) |  |
|  |  |  | 1(5) |  |
|  |  |  | 1(6) | Above 4500 |
| Barnaul ACC |  |  | 1 |  |
| Omsk ACC |  |  | 1(1) |  |
|  |  |  | 1(3) |  |
| Krasnoyarsk ACC |  |  | 1(1) |  |
|  |  |  | 1(2) |  |
|  |  |  | 1(3) |  |
|  |  |  | 1(4) |  |
|  |  |  | 1(5) |  |
|  |  |  | 1(6) |  |
|  |  |  | 1(7) |  |
| Norilsk ACC |  |  | 1(West) |  |
|  |  |  | 1(North-East) |  |
| Tura ACC |  |  | 1 |  |
| Irkutsk ACC |  |  | 1(1) |  |
|  |  |  | 1(1)R |  |
|  |  |  | 1(2) |  |
|  |  |  | 1(2)R |  |
|  |  |  | 1(3) |  |
|  |  |  | 1(3)R |  |
|  |  |  | 1(4) |  |
|  |  |  | 1(5) |  |
|  |  |  | 1(6) |  |
|  |  |  | 1(7) |  |
|  |  |  | 2(7)R |  |
| Chita ACC |  |  | 1 |  |
| Khabarovsk ACC |  |  | 2(1) |  |
|  |  |  | 1(2) |  |
|  |  |  | 2(3) |  |
|  |  |  | 2(4) |  |
|  |  |  | 1(5) |  |
|  |  |  | 2(6) |  |
|  |  |  | 1(7) |  |
|  |  |  | 2(8) |  |
|  |  |  | 1(9) |  |
|  |  |  | 1(10) |  |
|  |  |  | 1(11) |  |
|  |  |  | 1(12) |  |
|  |  |  | 1(13) |  |
|  |  |  | 1(14) |  |
| Magadan ACC |  |  | 1(North) |  |
|  |  |  | 1(North 1) |  |
|  |  |  | 1(North 2) |  |
|  |  |  | 1(South) |  |
| Anadyr ACC |  | 1 |  |  |
| Keperveyem ACC |  | 1 |  |  |
| Markovo ACC |  | 1 |  |  |
| Omolon ACC |  | 1 |  | 4500 or below |
| Pevek ACC |  | 1 |  |  |
| Shmidta Mys ACC |  | 1 |  |  |
| Petropavlovsk- Kamchatsky ACC |  |  | 1(West) |  |
|  |  |  | 1(East) |  |
| Milkovo ACC |  | 1 |  |  |
| Ossora ACC |  | 1 |  |  |
| Ust-Kamchatsk ACC |  | 1 |  |  |
| Yakutsk ACC |  |  | 1R |  |
|  |  |  | 1(West) |  |
|  |  |  | 1(East) |  |
|  |  |  | 1(North 1) |  |
|  |  |  | 1(North 2) |  |
| Mirny ACC |  |  | 1R |  |
|  |  |  | 1(North) |  |
|  |  |  | 1(South) |  |
| Nyurba ACC |  |  | 1 |  |
|  |  |  | 1R |  |
| Olekminsk ACC |  |  | 1 |  |
|  |  |  | 1R |  |
| Poliarny ACC |  |  | 1 |  |
| Tiksi ACC |  |  | 1 |  |
|  |  |  | 2O/R |  |
| Chulman ACC |  |  | 1R |  |
|  |  |  | 1(North) |  |
|  |  |  | 1(South) |  |

| Location | Lower airspace |  | Upper airspace | Remarks |
| --- | --- | --- | --- | --- |
|  | FIR | ACC-L |  |  |
| 1 | 2 | 3 | 4 | 5 |
| SERBIA |  |  |  |  |
| Beograd ACC |  | 1(1)* 1(2)* 1(3)* | 1(1) 1(2) 1(3) 1(4) 1(5) 1(6) 1(7) 1(8) 1(9) | * FL 285 |
| SLOVAKIA |  |  |  |  |
| Bratislava ACC | 1 | 4* | 3* 1+ | * Radar + VOLMET |
| SLOVENIA |  |  |  |  |
| No information |  |  |  |  |
| SPAIN |  |  |  |  |
| Barcelona ACC | 1 | 1 | 1(1) ø* 1(1) x 1(2) øø 1(3) [ 1(1+2+3) ( 1(4) † 1(4+5) { 1 (7) ‡ 1(8) > 1 (9) 1(10 + 11 )  3(11) xx 1(6+7+8+9) ‡‡ 1 (1+11) 1(6+8) | ø Lérida North x West A1 øø West A2 [ West A3 ( West A1-A3 † Central 1 { Central 1-2 ‡ East South > Mediterranean North Mediterranean South  Levante xx Levante 2 ‡‡ East N&S -- Mediterranean N&S |
|  |  |  | 2* | *FL 350 |
|  |  |  | 2** | ** VOLMET |
| Madrid ACC | 1 |  | 1(1)oo 3(8)++ 1(6+7)† 1(11)‡ 2(9)ø 3(5)øø 2(4) 2(7)‡‡ 3(12)# | oo Santiago ++ Castejon † Toledo ‡ Málaga ø Sevilla øø Zamora  Zaragoza ‡‡ Villatobas # Yeste |
|  |  |  | 1(3) - | - Domingo |
|  |  |  | 2(14) - - | - - Pamplona |
|  |  |  | 1(2) x 2(13) xx | X Cantábrico xx Asturias |
|  |  |  | 1(10) [ | [ Martín |
|  |  |  | 1(5+6) ( | ( Toledo + Zamora |
|  |  |  | 1(2+3) 1(4+14) 1(6+7+8) 2(1+13) 1(9+10+11+12) 1(3)* 1(2)* 3** | *FL350 ** VOLMET |
| SWEDEN |  |  |  |  |
| Malmö ACC |  | 2 (51)* 1 (53)* 1 (55)* 1 (57)* 1 (58)* 1 (61)* | 2 (52) 1 (54) 1 (56+57+58) 1 (56+57) 1 (58) | * FL285 |
| Stockholm ACC |  | 1 (25)* 1 (27)* 1 (28)* | 2** 1 (21) 1 (22) 1 (23) 1 (24) 1 (26) | * FL285 ** Volmet |
| SWITZERLAND |  |  |  |  |
| Genève | 3 | 5 | 7 |  |
| Zürich | 3 | 6 | 7 |  |
| THE FORMER YUGOSLAV REPUBLIC OF MACEDONIA |  |  |  |  |
| Skopje ACC | 1 | 1 | 2 |  |
| TURKEY |  |  | 2 (SOU) * | *above FL 325 |
| Ankara ACC |  | 2 (SOL) 1 (CEL) 2 (SEL) 1 (NEL) 2 (NOL) 2 (WEL) | 1 (CEU) 1 (SEU)* 1 (NEU)* 1 (NOU)* 2 (WEU)* |  |
|  |  |  | 3 (UEE + UWE) * | * above FL 325 |
| Istanbul ACC |  | 3(LEE + LWE) 3 (EAS) + 2 (WES) + 2 (NOR) + 3 (SOU) + |  | + Izmir |
| UKRAINE |  |  |  |  |
| L'viv ACC | 1 (N) 1 (S) | 1 (E) 1 (W+E) | 1 (C1+C2) 1 (C2) |  |
| Dnipropetrovs’k ACC | 1 | 1 (1) 1* 1** | 1 (2) 1(3) 1 (4) | * APP Kharkiv ** APP Donets'k |
| Kyiv ACC | 1 (N) 1 (S) | 1 (LIV+CEN) 1 (MOW+NTH) 1 (ODS+SOU) | 1 (CEN) 1 (NTH) 1 (SOU) |  |
| Odesa ACC | 1 | 1 (2+3) | 1 (1) 1 (3) |  |
| Simferopol’ ACC | 1 (N) 1 (S) | 1 (4+5) | 1 (1) 1 (2) 1 (3) 1 (5) |  |
| UNITED KINGDOM |  |  |  |  |
| London ACC | 2 | 1 | 1 |  |
|  | 1 | 1 | 1 |  |
|  | 1 | 2 | 1 |  |
|  | 1 | 1 | 1 |  |
|  | 1 | 1 | 1 |  |
|  | 2 | 1 | 1 |  |
|  | 1 | 1 | 1 |  |
|  |  | 1 | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 2 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 2 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 2 |  |
|  |  |  | 1 |  |
|  |  | 1 | 1 1 |  |
| Scottish ACC | 1 | 1 | 1 |  |
|  | 1 | 2 | 1 |  |
|  | 1 |  | 1 |  |
|  | 1 |  | 2 |  |
|  |  |  | 1 |  |
|  |  |  | 2 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
| Manchester ACC |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 |  |
|  |  |  | 1 1 |  |

Supplement Table CNS 4a — RADIO NAVIGATION AIDS ASSOCIATED WITH

THE BASIC ATS ROUTE NETWORK

Note.— All of the requirements for facilities and services associated with each of the newly created States in the European region have been placed under listings associated with those new States. It is understood that some of these requirements may no longer exist and new ones may need to be added. Such amendments will be carried out through the normal amendment process in consultation with the Secretariat.

Introduction

1.This table shows, State by State in alphabetical order, those radio navigation aids which are required for navigation on the agreed basic ATS route network. as defined in the Basic ANP, Table ATS 1.

2.All navaids listed are shown with their minimum designated operational coverage (DOC) required in connexion with the purpose(s) to be served by them.

3.Required DOCs listed in this table are for planning purposes only and therefore should not, in any case, be used for operational purpose(s).

4.In this context, the DOC provided (Supplement Table COM 3 and COM 4 of Part IV — CNS of EUR FASID refer) may be different from the requirement contained in the  Basic ANP, Table ATS 1 (19 of Part IV — CNS of EUR FASID refers).  While consistent with the requirement, the implemented facility providing the service may also be different (e.g. a VORTAC in lieu of a VOR/DME).

Explanation of the Table

5.Under each State, the required navaids are listed in alphabetical order, followed by indications of their type, their geographical coordinates and their designated operational coverage (range and height).

6.The types of navaids are indicated in this table by the following abbreviations:

DME: Distance measuring equipment

L: Locator

NDB: Non-directional beacon

VOR: VHF omnidirectional radio range.

7.The geographical coordinates in this table are indicated to the nearest minute only.

8.The planned designated operational range (first number) is expressed in nautical miles (NM) for NDB, VOR and DME.  The planned designated operational height (second number) is expressed in hundreds of feet for VOR and DME.

Examples:

—	NDB 50

—	VOR/DME 100/250

—	VOR 100/250

—	(100/500: National requirement) (Note 1)

—	VOR 5000 N 2000 W 120/500 (Note 2) ATE

—	DME 5000 N 2000 W 80/250 (Note 2) AT

Note 1.— Specific coverage required for national purpose(s) should be shown in brackets for the purpose of frequency allocation (1 to 4 above refer).

Note 2.— Collocated navaids may need different coverages as they may be required for different purposes.

| Location | Type | Geographical coordinates |  | Operational range/height |
| --- | --- | --- | --- | --- |
| ALBANIA |  |  |  |  |
| Tirana | VOR | 4120N | 01946E | 100/500 |
| ARMENIA |  |  |  |  |
| Andranik | NDB | 3949N | 04458E | 150 |
| Gyumri | NDB | 4043N | 04350E | 150 |
|  | DVOR/DME | 4043N | 04350E | 150 |
| Megri | NDB | 3857N | 04621E | 150 |
| Sevan | NDB | 4033N | 04456E | 150 |
| Yerevan/Zvartnots | VOR/DME | 4009N | 04425E | 200 |
|  | NDB | 4009N | 04425E | 150 |
| Talin | NDB | 4023N | 04353E | 150 |
| AUSTRIA |  |  |  |  |
| Bruck | NDB | 4804N | 01643E | 40 |
| Freistadt | VOR/DME | 4826N | 01408E | 60/500 |
| Gleichenberg | NDB | 4653N | 01548E | 40 |
| Graz | VOR/DME | 4657N | 01527E | 60/500 |
|  | NDB | 4655N | 01528E | 40 |
| Innsbruck | NDB | 4714N | 01124E | 40 |
|  | L | 4717N | 01130E | 25 |
| Klagenfurt | VOR/DME | 4636N | 01434E | 60/500, 80/500 NW |
|  | NDB | 4638N | 01432E | 40 |
|  | L | 4638N | 01423E | 25 |
|  | L | 4640N | 01413E | 25 |
| Kuhtai | NDB | 4713N | 01102E | 25 |
| Linz | VOR/DME | 4814N | 01406E | 60/500, 80/500 W & NW |
|  | NDB | 4814N | 01419E | 40 |
| Rattenberg | NDB | 4726N | 01157E | 25 |
| Salzburg | VOR/DME | 4800N | 01254E | 60/500, 80/500 E |
|  | NDB | 4758N | 01254E | 40 |
|  | L | 4749N | 01259E | 25 |
|  | L | 4753N | 01257E | 25 |
| Sollenau | VOR/DME | 4753N | 01617E | 60/500, 40/500 N |
| Steinhof | NDB | 4813N | 01615E | 25 |
| Stockerau | VOR/DME | 4825N | 01601E | 60/500 |
| Villach | VOR/DME | 4642N | 01355E | 60/500, 80/500 NW-N-SE |
|  | NDB | 4642N | 01355E | 60 |
| Wagram | VOR/DME | 4819N | 01630E | 60/250 |
| Wien | VOR/DME | 4806N | 01638E | 40/150 |
|  | L | 4809N | 01628E | 25 |
| AZERBAIJAN |  |  |  |  |
| Akhsu | NDB | 4034N | 04823E | 120 |
| Baku | DVOR/DME | 4028N | 05003E | 400 |
|  | NDB | 4025N | 05004E | 150 |
| Gyandzha | NDB | 4024N | 04619E | 150 |
| Khanlar | NDB | 4037N | 04620E | 120 |
| Nakhchivan | NDB | 3912N | 04527E | 150 |
| Yevlakh | NDB | 4038N | 04708E | 120 |
| BELARUS |  |  |  |  |
| Brest | NDB | 5208N | 02350E | 80 |
|  | NDB | 5205N | 02358E | 80 |
| Gomel | VOR/DME | 5231N | 03103E | 150/500 |
|  | NDB | 5233N | 03057E | 80 |
|  | NDB | 5231N | 03105E | 80 |
| Grodno | NDB | 5339N | 02403E | 80 |
|  | NDB | 5333N | 02404E | 80 |
| Ivenets | NDB | 5353N | 02644E | 80 |
| Minsk-1 | NDB | 5354N | 02729E | 80 |
|  | NDB | 5351N | 02732E | 80 |
| Minsk-2 | NDB | 5355N | 02758E | 80 |
|  | NDB | 5351N | 02805E | 80 |
|  | VOR/DME | 5353N | 02801E | 150/500 |
| Mogilev | VOR/DME | 5357N | 03006E | 150/500 |
| Pinsk | VOR/DME | 5209N | 02607E | 150/500 |
| Vitebsk | VOR/DME | 5507N | 03021E | 150/500 |
|  | NDB | 5506N | 03017E | 80 |
|  | NDB | 5509N | 03026E | 80 |
| BELGIUM |  |  |  |  |
| Affligem | VOR/DME | 5054N | 0408E | 90E, 40/250 |
| Antwerpen | NDB | 5110N | 0434E | 50 |
|  | TVOR/DME | 5111N | 0428E | 40/250 |
|  | L | 5111N | 0429E | 25 |
| Brasschaat | NDB | 5122N | 0429E | 50 |
| Bruno | VOR/DME | 5107N | 0451E | 40/250 |
| Bruxelles | VOR/DME | 5054N | 0432E | 100/500 |
|  | L | 5055N | 0437E | 25 |
|  | L | 5050N | 0428E | 25 |
|  | L | 5056N | 0436E | 25 |
| Charleroi | NDB | 5029N | 0433E | 50 |
|  | L | 5028N | 0429E | 25 |
| Chievres | VOR | 5034N | 0350E | 60/500 |
| Costa | VOR/DME | 5121N | 0321E | 60/500 |
| Dender | NDB | 5053N | 0402E | 50 |
| Flora | VOR/DME | 5053N | 0508E | 50/250 |
| Huldenberg | VOR/DME | 5045N | 0438E | 32 NNW-NE, 40/250 |
| Goetsenhoven | NDB | 5047N | 0457E |  |
| Gosly | VOR/DME | 5027N | 0426E | 30/260 |
| Kleine Brogel | NDB | 5113N | 0533E | 35 |
| Koksy | VOR | 5106N | 0239E | 80/500; 100 SE |
| Kortrijk | NDB | 5049N | 0313E | 10 |
| Liege | NDB | 5042N | 0533E | 25 |
|  | VOR/DME | 5039N | 0528E | 40/250 |
|  | L | 5039N | 0528E | 25 |
| Mackel | NDB | 5058N | 0330E | 50 |
| Nicky | VOR/DME | 5110N | 0411E | 60/500, 100 E |
|  | NDB | 5110N | 0411E | 50 |
| Nieuwrode | L | 5057N | 0450E |  |
| Olno | VOR/DME | 5035N | 0543E | 60/500, 80 SW-NW |
| Oostende | NDB | 5113N | 0300E | 50 |
|  | TVOR | 5112N | 0251E |  |
|  | L | 5112N | 0250E | 25 |
|  | L | 5112N | 0254E | 25 |
| Spa | NDB | 5029N | 0555E | 10 |
| Sprimont | VOR/DME | 5031N | 0537E | 60/500, 80 SW-NW |
| St. Truiden | NDB | 5047N | 5011E |  |
| Werchter | L | 5059N | 0425E |  |
| Weelde | NDB | 5124N | 0458E |  |
| Zoersel | NDB | 5116N | 0446E |  |
| BOSNIA AND HERZEGOVINA |  |  |  |  |
| Banja Luka | VOR/DME | 4456N | 01717E | 40/250 |
| Bratjevac | NDB | 4450N | 01548E | 25 |
| Bihac | NDB | 4449N | 01549E | 25 |
| Derventa | VOR/DME | 4459N | 01758E | 80/600 |
| Domanovici | NDB | 4308N | 01751E | 50 |
| Dubrave | NDB | 4427N | 01842E | 25 |
| Gacko | NDB | 4308N | 01833E | 60 |
| Kiseljak | VOR | 4401N | 01804E | 25/250 |
| Kobljaca | VOR | 4352N | 01810E | 25/250 |
|  | NDB | 4352N | 01810E | 25 |
| Laktasi | L | 4455N | 01718E | 25 |
| Mostar | VOR/DME | 4315N | 01751E | 80/500 |
|  | NDB | 4313N | 01751E | 25 |
| Omarska | VOR/DME | 4457N | 01648E | 80/500 |
| Sarajevo | VOR/DME | 4356N | 01827E | 80/500 |
|  | NDB | 4356N | 01827E | 60 |
|  | L | 4350N | 01818E | 25 |
| Topola | NDB | 4505N | 01715E | 50 |
| Turija | NDB | 4428N | 01829E | 50 |
| Vilusi | L | 4457N | 01717E | 25 |
| Zilvinice | NDB | 4427N | 01840E | 25 |
| BULGARIA |  |  |  |  |
| Bailovo | VOR/DME | 4240N | 02349E | 100/500 |
| Bozhourishte | NDB | 4247N | 02312E | 50 |
| Burgas | VOR/DME | 4234N | 02731E | 80/500W, 100/500 E |
|  | NDB | 4238N | 02735E | 40 |
|  | L | 4235N | 02732E | 10 |
|  | L | 4231N | 02727E | 25 |
|  | L | 4233N | 02730E | 10 |
| Devnya | NDB | 4315N | 02739E | 40 |
| Golyama | VOR/DME | 4306N | 02413E | 100/500 |
| Gorna | VOR/DME | 4309N | 02543E | 100/500 |
|  | NDB | 4310N | 02537E | 40 |
|  | L | 4309N | 02541E | 10 |
|  | L | 4308N | 02549E | 25 |
|  | L | 4309N | 02544E | 10 |
| Kalotina | VOR/DME | 4257N | 02253E | 80/500, 100/500 E |
| Plovdiv | VOR/DME | 4203N | 02453E | 100/500 |
|  | DME | 4204N | 02452E | 60/250 |
|  | NDB | 4202N | 02454E | 40 |
|  | L | 4203N | 02452E | 10 |
| Russe | NDB | 4339N | 02559E | 40 |
| Sofia | VOR/DME | 4242N | 02323E | 100/500 |
|  | NDB | 4242N | 02328E | 40 |
|  | L | 4242N | 02326E | 10 |
|  | L | 4242N | 02318E | 25 |
|  | L | 4242N | 02323E | 10 |
| Vakarel | NDB | 4235N | 02342E | 40 |
| Varna | VOR/DME | 4314N | 02749E | 150/500, 80/500 S-SW |
|  | L | 4315N | 02745E | 40 |
|  | L | 4314N | 02748E | 10 |
|  | L | 4313N | 02754E | 25 |
|  | L | 4314N | 02751E | 10 |
| CROATIA |  |  |  |  |
| Bokanjac | L | 4410N | 01516E | 25 |
| Breza | NDB | 4525N | 01421E | 50 |
| Cavtat | L | 4235N | 01813E | 15 |
| Cres | NDB | 4454N | 01425E | 50 |
| Cepin | L | 4531N | 01833E | 25 |
| Drvenik | NDB | 4327N | 01609E | 25 |
| Dubrovnik | VOR/DME | 4233N | 01817E | 80/500, 100/500 N |
| Gruda | L | 4232N | 01820E | 15 |
| Kakma | NDB | 4400N | 01530E | 50 |
| Kavran | NDB | 4453N | 01401E | 25 |
| Kolocep | NDB | 4240N | 01802E | 50 |
| Kozala | L | 4520N | 01427E | 15 |
| Losinj | NDB | 4432N | 01428E | 50 |
| Pisarovina | NDB | 4536N | 01551E | 50 |
| Pula | L | 4454N | 01352E | 15 |
|  | VOR/DME | 4454N | 01355E | 80/500, 100/500 NE-SE+SW-NW |
|  | NDB | 4453N | 01345E | 50 |
| Rijeka | VOR/DME | 4512N | 01435E | VOR 60/500; DME 60/250 |
|  | L | 4508N | 01439E | 25 |
| Sali | NDB | 4536N | 01510E | 30 |
| S.Kraljevec | L | 4548N | 01610E | 15 |
| Split | VOR/DME | 4330N | 01618E | 100/500 |
| Trogir | NDB | 4330N | 01614E | 50 |
| Valtura | L | 4454N | 01356E | 15 |
| Varazdin | NDB | 4619N | 01623E | 25 |
| Velika Barna | VOR/DME | 4545N | 01709E | 80/500 |
| V.Gorica | L | 4543N | 01603E | 15 |
| Vrsar | NDB | 4513N | 01339E | 25 |
| Zadar | VOR/DME | 4406N | 01522E | 70/500 |
|  | L | 4406N | 01522E | 25 |
| Zagreb | VOR/DME | 4554N | 01618E | 80/500, 150/500 NE & E-S |
|  | NDB | 4554N | 01619E | 100 |
| CYPRUS |  |  |  |  |
| Larnaca | VOR/DME | 3453N | 3338E | 200/500 |
|  | NDB | 3459N | 3344E | 50 |
|  | NDB | 3450N | 3334E | 150 |
| Pafos | VOR/DME | 3443N | 3231E | 200/500 |
|  | NDB | 3443N | 3229E | 100 |
| CZECH REPUBLIC |  |  |  |  |
| Brno | VOR/DME | 4009N | 1642E | 80/250 |
| Cheb | VOR/DME | 5004N | 1224E | 60/500, 80/500 E |
| Desna | VOR/DME | 4858N | 1533E | 80/250 |
| Frydlant | VOR/DME | 5044N | 1502E | 50/500, 80/500 S |
| Holesov | NDB | 4919N | 1732E | 25 |
| Neratovice | VOR/DME | 5019N | 1424E | 25/100 |
| Nora | L | 4940N | 1803E | 50 |
| Pardubice | L | 5001N | 1546E | 25 |
|  | NDB | 5001N | 1548E | 25 |
| Praha | L | 5007N | 1417E | 25 |
| Praha/Nort | NDB | 5009N | 1422E | 25 |
| Praha/Ruzyne | VOR/DME | 5006N | 1416E | 100/500 |
| Rona | L | 4940N | 1803E | 50 |
| Vlasim | VOR/DME | 4942N | 1504E | 80/500 |
|  | NDB | 4942N | 1504E | 50 |
| Vozice | VOR/DME | 4933N | 1452E | 60/250, 40/250 NW |
|  | NDB | 4933N | 1452E | 80 |
| Zabreh | VOR/DME | 4956N | 1803E | 80 |
| DENMARK |  |  |  |  |
| Alborg | VOR/DME | 5706N | 1000E | 100/500 |
|  | L | 5705N | 0941E | 20 |
| Alsie | VOR | 5454N | 1000E | 60/500, 80/500 N-NE & SW |
| Billund | NDB | 5545N | 0917E | 50 |
|  | L | 5544N | 0901E | 15 |
| Codan | VOR/DME | 5500N | 1223E | 60/500 |
| Donna | NDB | 5528N | 0508E | 25 |
| Esjberg | L | 5531N | 0825E | 20 |
|  | L | 5533N | 0842E | 20 |
| Gedser | VOR | 5437N | 1156E | 60/500 |
| Karup | L | 5618N | 0914E | 20 |
|  | L | 5618N | 9858E | 25 |
| København/Kastrup | VOR/DME | 5535N | 1237E | 60/500 |
| Kolding | L | 5527N | 0920E | 15 |
| Maribo | L | 5442N | 1130E | 15 |
| Nora | VOR | 5606N | 1215E | 80/500 N, 60/500 S |
| Odense | L | 5531N | 1028E | 20 |
| Odin | VOR | 5535N | 1039E | 80/500 N, 60/500 S |
|  | DME | 5535N | 1039E | 80/250 |
| Ramme | VOR | 5629N | 0811E | 60/500, 150/500 W-N |
| Ringo | VOR/DME | 5528N | 1150E | 80/500 |
| Rønne | L | 5502N | 1454E | 20 |
| Roskilde | L | 5537N | 1200E | 30 |
| Sindal | L | 5730N | 1009E | 15 |
| Skive | L | 5635N | 0908E | 15 |
|  | L | 5630N | 0914E | 25 |
| Sonderborg | L | 5501N | 0942E | 15 |
|  | L | 5456N | 0950E | 15 |
| Stauning | L | 5559N | 0819E | 15 |
|  | L | 5559N | 0826E | 15 |
| Skrydstrup | VOR | 5514N | 0913E | 80/500 |
|  | L | 5512N | 0928E | 20 |
| Thisted | L | 5703N | 0852E | 40 |
|  | L | 5705N | 0837E | 15 |
| Tirstrup | L | 5619N | 1028E | 20 |
|  | L | 5617N | 1047E | 20 |
| Trano | VOR/DME | 5546N | 1126E | 60/500 |
| Vesta | VOR/DME | 5536N | 0818E | 80/500, 100/500 N-E, |
| 200/500 W-NW | E |  |  |  |
| ESTONIA |  |  |  |  |
| Kärdla | NDB | 5857N | 2252E | 80 |
|  | NDB | 5859N | 2251E | 25 |
| Kuressaare | VOR/DME | 5814N | 2231E | 50/500, 80/500 |
|  | NDB | 5813N | 2231E | 25 |
| Pärnu | VOR/DME | 5825N | 2428E | 30/500 |
|  | NDB | 5830N | 2434E | 80 |
|  | NDB | 5826N | 2430E | 25 |
| Tallinn/Ülemiste | VOR/DME | 5925N | 2450E | 80/500 |
|  | NDB | 5925N | 2448E | 30 |
|  | NDB | 5925N | 2453E | 30 |
|  | DME | 5925N | 2449E | 25 |
|  | DME | 5925N | 2451E | 25 |
| Tartu/Ülenurme | NDB | 5819N | 2646E | 80 |
|  | NDB | 5819N | 2643E | 25 |
|  | DME | 5818N | 2642E | 25 |
| Vöhma | VOR/DME | 5839N | 2534E | 80/100 |
| FINLAND |  |  |  |  |
| Anton | DME | 6052N | 2508E | 60/500 |
| Enodak | VOR/DME | 6823N | 2328E | 100/500 |
| Enontekiö | DME | 6822N | 2326E | 25/100 |
| Halli | VOR/DME | 6151N | 2448E | 100/500 |
| Heka | NDB | 6015N | 2529E | 25 |
| Helsinki | VOR/DME | 6020N | 2457E | 100/500, 150/500 N |
| Helsinki-Vantaa | DME | 6018N | 2456E | 25/100 |
| Helsinki-Vantaa | DME | 6019N | 2458E | 25/100 |
| Hetta | L | 6825N | 2332E | 30 |
| Ivalo | DME | 6836N | 2725E | 25/100 |
| Joensuu | DME | 6239N | 2938E | 25/100 |
| Josie | VOR/DME | 6240N | 2937E | 100/250 |
| Jyvaskyla | DME | 6223N | 2541E | 25/100 |
| Kadis | DME | 6008N | 2504E | 100/500 |
| Kainuu | VOR/DME | 6417N | 2739E | 60/500 |
| Kajaani | DME | 6417N | 2740E | 25/100 |
| Kantola | VOR/DME | 6600N | 2914E | 60/500 |
| Kauhava | VOR/DME | 6307N | 2303E | 60/500 |
| Kaunis | DME | 6827N | 2727E | 100/500 |
| Kemi | L | 6550N | 2436E | 40 |
| Kemi-Tornio | DME | 6547N | 2435E | 25/100 |
| Kittila | L | 6737N | 2454E | 30 |
| Kruunu | L | 6348N | 2310E | 50 |
| Kruunupyy | DME | 6343N | 2308E | 25/100 |
| Kuopio | DME | 6260N | 2748E | 25/100 |
| Kuusamo | DME | 6559N | 2913E | 25/100 |
| Linna | VOR/DME | 6157N | 2855E | 60/500 |
| Luonet | VOR/DME | 6225N | 2537E | 100/500 |
| Marie | VOR/DME | 6008N | 1955E | 60/500 |
| Orimaa | DME | 6050N | 2546E | 60/500 |
| Oulu | DME | 6456N | 2520E | 25/100 |
| Oulu | VOR/DME | 6456N | 2522E | 100/500, 150/500N |
| Pirkka | VOR/DME | 6125N | 2335E | 60/500 |
| Porvoo | VOR/DME | 6018N | 2535E | 60/500 |
| Previk | VOR/DME | 6128N | 2148E | 60/500 |
| Rovani | VOR/DME | 6634N | 2549E | 60/500, 80/500 N |
| Rovaniemi | DME | 6634N | 2550E | 25/100 |
| Rusko | VOR/DME | 6031N | 2215E | 60/500 |
| Seinajoki | L | 6238N | 2256E | 25 |
| Turku | DME | 6031N | 2216E | 25/100 |
| Utti | VOR/DME | 6054N | 2655E | 60/500 |
| Vaasa | DME | 6303N | 2145E | 25/100 |
| Vaasa | VOR/DME | 6302N | 2146E | 100/500 |
| Vehkaa | VOR/DME | 6259N | 2749E | 60/500 |
| Vihti | DME | 6028N | 2415E | 60/500 |
| Vilmas | VOR/DME | 6102N | 2807E | 60/500 |
| FRANCE |  |  |  |  |
| Abbeville | VOR/DME | 5008N | 0151E |  |
| Agen Gaudonville | VOR/DME | 4353N | 0052E |  |
| Agen La Garenne | L | 4410N | 0035E |  |
| Aix-les-Milles | L | 4330N | 0522E |  |
| Ales Deaux | L | 4404N | 0409E |  |
| Ajaccio/Campo dell’Oro | NDB | 4156N | 0848E |  |
|  | L | 4155N | 0848E |  |
|  | L | 4156N | 0848E |  |
| Ajaccio/Coti Chiavari | VOR/DME | 4146N | 0846E |  |
| Albi le Sequestre | L | 4355N | 0207E |  |
| Amberieu | TACAN | 4559N | 0520E |  |
| Amboise | VOR/DME | 4726N | 0104E |  |
|  | NDB | 4726N | 0104E |  |
| Amiens Glisy | L | 4951N | 0229E |  |
| Angers | VOR | 4732N | 0051W |  |
| Angers Marce | L | 4733N | 0019W |  |
| Angouleme Brie Champniers | L | 4544N | 0012E |  |
| Annecy Meythet | L | 4552N | 0601E |  |
| Autun | VOR/DME | 4648N | 0416E |  |
| Auxerre Branches | L | 4751N | 0330E |  |
| Avignon Caumont | L | 4354N | 0454E |  |
| Avignon Pujaut | VOR | 4400N | 0445E |  |
| Avord | TACAN | 4703N | 0238E |  |
| Bale/Mulhouse | VOR/DME | 4738N | 0730E |  |
|  | NDB | 4735N | 0732E |  |
|  | L | 4735N | 0732E |  |
| Bastia Poretta | VOR/DME | 4234N | 0929E |  |
|  | L | 4234N | 0929E |  |
| Beauvais Tille | VOR | 4926N | 0209E |  |
|  | L | 4926N | 0206E |  |
| Bergerac Roumaniere | L | 4450N | 0030E |  |
| Besancon La Veze | L | 4712N | 0604E |  |
| Beziers Vias | L | 4319N | 0322E |  |
| Biarritz Bayonne Anglet | VOR/DME | 4328N | 0131W |  |
|  | NDB | 4328N | 0132W |  |
| Blois Le Breuil | L | 4740N | 0112E |  |
| Bordeaux/Merignac | VOR/DME | 4450N | 0044W |  |
|  | L | 4449N | 0044W |  |
|  | L | 4450N | 0044W |  |
|  | L | 4450N | 0044W |  |
| Boulogne-sur-Mer | VOR | 5037N | 0154E |  |
| Bourges | L | 4704N | 0223E |  |
| Boursonne | VOR/DME | 4911N | 0303E |  |
| Bray sur Seine | VOR/DME | 4824N | 0318E |  |
| Brest/Guipavas | L | 4827N | 0427W |  |
| Bretigny sur Orge | TACAN | 4835N | 0220E |  |
| Caen Carpiquet | VOR | 4910N | 0027W |  |
|  | L | 4911N | 0028W |  |
| Cahors Lalbenque | L | 4421N | 0129E |  |
| Calais/Dunkerque | L | 5057N | 0157E |  |
| Calvi Sainte Catherine | L | 4231N | 0848E |  |
| Cambrai Epinoy | VORTAC | 5014N | 0309E |  |
| Cannes-Tanneron | VOR | 4334N | 0652E |  |
| Carcassone Salvaza | L | 4313N | 0219E |  |
| Castres Mazamet | L | 4333N | 0218E |  |
| Cazaux | TACAN | 4432N | 0109W |  |
| Chalons Vatry | VOR/DME | 4847N | 0411E |  |
|  | L | 4847N | 0411E |  |
|  | L | 4846N | 0413E |  |
| Chambery Aix les Bains | VOR/DME | 4553N | 0546E |  |
|  | L | 4538N | 0553E |  |
| Charleville Meziere | L | 4947N | 0440E |  |
| Chartres La Loupe | VOR/DME | 4829N | 0059E |  |
| Chateaudun | VOR/DME | 4804N | 0123E |  |
|  | NDB | 4804N | 0122E |  |
| Chateauroux Deols | NDB | 4651N | 0143E |  |
| Chatillon sur Marne | VOR/DME | 4908N | 0335E |  |
| Cherbourg/Maupertus | L | 4939N | 0129W |  |
| Clermont Ferrand Auvergne | VOR/ DME | 4547N | 0311E |  |
|  | L | 4547N | 0309E |  |
| Cognac Chateaubernard | VOR/DME | 4540N | 0019W |  |
|  | NDB | 4539N | 0020W |  |
| Colmar Houssen | L | 4806N | 0721E |  |
| Colmar Meyenheim | TACAN | 4756N | 0724E |  |
| Coulommiers Voisins | VOR/DME | 4851N | 0301E |  |
| Creil | VORTAC | 4915N | 0231E |  |
| Deauville Saint Gatien | VOR | 4919N | 0019E |  |
| Dieppe | VOR | 4956N | 0110E |  |
| Dieppe Saint Aubin | L | 4956N | 0110E |  |
| Digne | VOR | 4400N | 0606E |  |
| Dijon Longvic | VOR/DME | 4717N | 0506E |  |
| Dinard Pleurtuit Saint Malo | VOR/DME | 4835N | 0205W |  |
|  | L | 4836N | 0205W |  |
| Dole Tavaux | L | 4703N | 0527E |  |
| Epernon | VOR/DME | 4837N | 0139E |  |
| Epinal Mirecourt | VOR | 4819N | 0604E |  |
|  | L | 4819N | 0603E |  |
| Etampes Mondesir | NDB | 4823N | 0207E |  |
| Evreux Fauville | VORTAC | 4902N | 0113E |  |
| Figari Sud Corse | VOR/DME | 4130N | 0905E |  |
|  | L | 4130N | 0905E |  |
| Figeac | L | 4440N | 0147E |  |
| Gaillac Castelnau de Montmirail | VOR | 4357N | 0150E |  |
| Granville | L | 4855N | 0129W |  |
| Grenoble Saint Geoirs | L | 4522N | 0521E |  |
|  | L | 4522N | 0521E |  |
| Grostenquin | VOR/DME | 4859N | 0643E |  |
| Hericourt | NDB | 4734N | 0644E |  |
| Hyeres Le Palyvestre | TACAN | 4306N | 0608E |  |
|  | NDB | 4302N | 0609E |  |
| Istres Le Tube | TACAN | 4332N | 0456E |  |
| L’Aigle | VOR | 4848N | 0032E |  |
| La Chatre | VOR | 4642N | 0209E |  |
| La Rochelle Ile de Re | L | 4611N | 0113W |  |
| La Tour du Pin-Lyon Est | VOR | 4529N | 0526E |  |
| Landivisiau | TACAN | 4832N | 0409W |  |
| Lannion | L | 4845N | 0329W |  |
| Lanveoc Poulmic | TACAN | 4817N | 0427W |  |
| Laval Entrammes | L | 4802N | 0045W |  |
| Le Havre Octeville | L | 4932N | 0005E |  |
| Le Luc Le Cannet | VOR/DME | 4323N | 0615E |  |
| Le Mans Arnage | L | 4757N | 0012E |  |
| Le Touquet Paris Plage | L | 5031N | 0138E |  |
| Lille Lesquin | VOR/DME | 5034N | 0306E |  |
|  | L | 5034N | 0305E |  |
| Limoges | VOR/DME | 4549N | 0102E |  |
| Limoges Bellegarde | VOR | 4605N | 0123E |  |
|  | L | 4551N | 0110E |  |
|  | L | 4551N | 0110E |  |
| Lorient Lann Bihoue | NDB | 4745N | 0328W |  |
|  | TACAN | 4746N | 0326W |  |
| Luxeuil | VOR | 4741N | 0618E |  |
| Luxeuil Saint Saveur | TACAN | 4747N | 0621E |  |
| Lyon/Bron | L | 4544N | 0456E |  |
| Lyon Saint-Exupery | VOR/DME | 4545N | 0505E |  |
|  | L | 4545N | 0505E |  |
| Marseille Provence | VOR/DME | 4326N | 0513E |  |
|  | NDB | 4326N | 0513E |  |
|  | NDB | 4326N | 0513E |  |
|  | NDB | 4326N | 0513E |  |
|  | L | 4326N | 0514E |  |
| Martigues | VOR/DME | 4323N | 0505E |  |
| Melun Villaroche | VOR | 4837N | 0239E |  |
| Mende Brenoux | NDB | 4434N | 0328E |  |
| Mende-Nasbinals | VOR/DME | 4436N | 0310E |  |
| Merville Calonne | L | 5037N | 0239E |  |
|  | L | 5037N | 0239E |  |
| Metz Frescaty | TACAN | 4904N | 0608E |  |
| Metz Nancy Lorraine | L | 4900N | 0616E |  |
|  | L | 4900N | 0616E |  |
| Mont de Marsan | TACAN | 4355N | 0030W |  |
| Montbeliard Courcelles | L | 4730N | 0656E |  |
| Montdidier | VOR | 4933N | 0229E |  |
| Montelimar Ancone | VOR/DME | 4433N | 0447E |  |
|  | NDB | 4433N | 0447E |  |
| Montlucon Domerat | NDB | 4613N | 0222E |  |
| Montmedy-Dun sur Meuse | VOR | 4924N | 0508E |  |
| Montpellier Mediterranee | VOR/DME | 4335N | 0358E |  |
|  | L | 4335N | 0358E |  |
| Monts d’Arree | VOR | 4820N | 0336W |  |
| Morlaix Ploujean | L | 4836N | 0349W |  |
| Moulins Montbeugny | VOR/DME | 4632N | 0330E |  |
|  | NDB | 4632N | 0330E |  |
| Nancy Essey | L | 4842N | 0614E |  |
| Nancy Ochey | TACAN | 4835N | 0557E |  |
| Nantes Atlantique | VOR/DME | 4709N | 0136W |  |
|  | NDB | 4710N | 0136W |  |
| Nevers | VOR | 4709N | 0256E |  |
| Nevers Fourchambault | L | 4709N | 0256E |  |
| Nice Cote d’Azur | VOR/DME | 4340N | 0712E |  |
|  | VOR/DME | 4340N | 0715E |  |
|  | NDB | 4340N | 0714E |  |
|  | L | 4340N | 0714E |  |
| Nimes Garons | L | 4345N | 0425E |  |
|  | TACAN | 4345N | 0425E |  |
| Orange Caritat | NDB | 4408N | 0453E |  |
|  | TACAN | 4408N | 0452E |  |
| Orleans Bricy | NDB | 4759N | 0145E |  |
|  | TACAN | 4759N | 0146E |  |
| Orleans Saint Denis de l’Hotel | L | 4759N | 0145E |  |
| Paris Charles-de-Gaulle | VOR/DME | 4901N | 0230E |  |
|  | L | 4901N | 0233E |  |
|  | L | 4901N | 0236E |  |
|  | VOR/DME | 4900N | 0237E |  |
|  | L | 4901N | 0234E |  |
|  | L | 4901N | 0231E |  |
| Paris Le Bourget | VOR/DME | 4858N | 0227E |  |
|  | L | 4858N | 0228E |  |
| Paris/Orly | VOR/DME | 4844N | 0223E |  |
|  | L | 4844N | 0223E |  |
|  | L | 4844N | 0224E |  |
| Pau Pyrenees | L | 4323N | 0026W |  |
|  | L | 4323N | 0026W |  |
| Perigueux Bassillac | L | 4512N | 0048E |  |
| Perpignan Rivesaltes | VOR/DME | 4245N | 0252E |  |
|  | L | 4245N | 0252E |  |
| Pithiviers | VOR | 4809N | 0216E |  |
| Poitiers Biard | VOR | 4635N | 0018E |  |
|  | NDB | 4642N | 0024E |  |
| Pontoise Cormeilles en Vexin | VOR | 4906N | 0202E |  |
| Quimper Pluguffan | VOR/DME | 4757N | 0411W |  |
|  | L | 4759N | 0411W |  |
| Rambouillet-Les Bordes | VOR/DME | 4839N | 0200E |  |
| Reims Champagne | VORTAC | 4919N | 0403E |  |
| Rennes Saint Jacques | VOR/DME | 4804N | 0144W |  |
|  | L | 4804N | 0145W |  |
| Roanne-Renaison | L | 4603N | 0400E |  |
|  | VOR | 4604N | 0400E |  |
| Rochefort Saint Agnant | L | 4556N | 0100W |  |
| Rodez Marcillac | L | 4425N | 0228E |  |
|  | L | 4425N | 0228E |  |
| Rolampont | VOR/DME | 4754N | 0515E |  |
| Rouen Vallee de Seine | VOR | 4928N | 0117E |  |
| Saint Brieuc Armor | L | 4832N | 0252W |  |
| Saint Dizier-Robinson | NDB | 4838N | 0453E |  |
|  | TACAN | 4838N | 0453E |  |
| Saint Etienne Boutheon | L | 4532N | 0418E |  |
|  | L | 4532N | 0418E |  |
| Saint Inglevert | NDB | 5053N | 0145E |  |
| Saint Nazaire Montoir | L | 4718N | 0210W |  |
| Saint Tropez | VOR/DME | 4313N | 0636E |  |
| Saint Yan | L | 4625N | 0400E |  |
| Sauveterre de Guyenne | VOR | 4441N | 0009W |  |
| Solenzara | TACAN | 4156N | 0924E |  |
| Strasbourg Entzheim | VOR/DME | 4830N | 0734E |  |
|  | L | 4832N | 0737E |  |
| Tarbes Lourdes Pyrenees | VOR | 4320N | 0009W |  |
|  | NDB | 4310N | 0001W |  |
| Tarbes-Lahitte | VOR | 4328N | 0001W |  |
| Thiers | VOR | 4553N | 0333E |  |
| Toul Rosieres | TACAN | 4848N | 0600E |  |
| Toulouse Blagnac | VOR/DME | 4341N | 0119E |  |
|  | L | 4356N | 0126E |  |
|  | NDB | 4330N | 0129E |  |
|  | NDB | 4335N | 0123E |  |
|  | NDB | 4335N | 0123E |  |
| Toulouse Francazal | TACAN | 4333N | 0122E |  |
| Tours Saint Symphorien | NDB | 4725N | 0043E |  |
|  | TACAN | 4725N | 0043E |  |
| Toussus le Noble | VOR | 4845N | 0206E |  |
| Troyes Barberey | VOR | 4815N | 0358E |  |
|  | L | 4819N | 0401E |  |
| Valence Chabeuil | L | 4456N | 0458E |  |
| Valenciennes Denain | L | 5019N | 0328E |  |
| Vannes Meucon | L | 4743N | 0243W |  |
| Villacoublay Velizy | NDB | 4846N | 0211E |  |
| Vichy Charmeil | L | 4611N | 0324E |  |
| Vienne Reventin | VOR | 4533N | 0453E |  |
| GEORGIA |  |  |  |  |
| Ali | NDB | 4206N | 4339E | 80 |
| Batumi | NDB | 4136N | 4137E | 80 |
| Gori | NDB | 4201N | 4408E | 80 |
| Kutaisi | VOR/DME | 4210N | 4227E | 100/500 |
|  | L | 4211N | 4231E | 30 |
|  | L | 4210N | 4233E | 80 |
| Mukhrani | NDB | 4155N | 4434E | 80 |
| Tbilisi | VOR/DME | 4139N | 4458E | 100/500 |
|  | L | 4139N | 4458E | 30 |
|  | L | 4138N | 4500E | 80 |
|  | L | 4141N | 4456E | 30 |
|  | L | 4143N | 4454E | 80 |
| Tsnori | NDB | 4138N | 4601E | 80 |
| GERMANY |  |  |  |  |
| Allersberg | VOR/DME | 4913N | 1113E | 60/500, 80/500 (100-135) |
| Arpe | VOR | 5101N | 0818E | 40/300 |
| Augsburg | NDB | 4825N | 1056E | 25 |
| Barmen | VOR/DME | 5120N | 0711E | 40/500, 60/500 NW |
| Bayreuth | VOR | 4959N | 1138E | 40/250, 50/250 W |
|  | NDB | 4958N | 1134E | 25 |
| Berlin/Schönefel | NDB | 5224N | 1337E | 25 |
| Berlin/Tegel | VOR/DME | 5234N | 1317E | 100/250, 80/250 N & S |
| Berlin/Tegel-East | NDB | 5234N | 1323E | 50 |
| Berlin/Tegel-West | NDB | 5233N | 1309E | 100 |
| Berlin/Tempelhof | VOR/DME | 5229N | 1324E | 200/500, 150/500 N & S |
| Bottrop | NDB | 5135N | 0701E | 40 |
| Braunschweig | L | 5219N | 1036E | 20 |
| Bremen | VOR/DME | 5303N | 0847E | 60/250 |
|  | NDB | 5303N | 0855E | 25 |
|  | L | 5303N | 0840E | 25 |
| Brünkendorf | VOR/DME | 5302N | 1133E | 60/250, 100/250 SE |
|  | NDB | 5304N | 1124E | 75 |
| Bückeburg | NDB | 5217N | 0906E | 25 |
| Charlie/Frankfurt | VOR | 4955N | 0902E | 60/500 |
| Cola/Köln | VOR/DME | 5047N | 0736E | 40/250, 60/250 W |
| Dinkelsbühl | VOR/DME | 4909N | 1014E | 60/500 |
| Dortmund | VOR/DME | 5143N | 0735E | 40/500, 60/500 SE, 80/500 NW |
| Dortmund-Wickede | NDB | 5132N | 0738E | 25 |
|  | DME | 5132N | 0738E | 25/100 |
| Dresden | VOR/DME | 5102N | 1338E | 80/250 |
|  | NDB | 5112N | 1351E | 15 |
| Düsseldorf | VOR/DME | 5117N | 0645E | 40/250 |
|  | NDB | 5119N | 0642E | 15 |
|  | L | 5121N | 0654E | 15 |
|  | L | 5114N | 0639E | 15 |
| Elbe | VOR/DME | 5339N | 0936E | 60/500, 80/500 S & SW |
| Erding | VOR/DME | 4820N | 1157E | 40/250 |
| Erfurt | NDB | 5058N | 1105E | 60 |
|  | L | 5059N | 1052E | 15 |
|  | VOR/DME | 5028N | 1105E | 40/250 |
| Erlangen | VOR/DME | 4939N | 1109E | 60/500 |
| Eurach | VOR/DME | 4744N | 1115E | 60/300 |
| Frankfurt/Main | VOR/DME | 5003N | 0836E | 100/500, 50/500 SW |
|  | NDB | 5003N | 0839E | 40 |
|  | L | 5004N | 0841E | 15 |
|  | L | 5001N | 0826E | 15 |
| Friedland | VOR/DME | 5346N | 1335E | 60/500 |
| Friedrichshafen | NDB | 4741N | 0932E | 40 |
|  | DME | 4740N | 0931E | 25 |
| Fulda | VOR/DME | 5036N | 0934E | 100/500 |
| Fürstenwalde | VOR/DME | 5225N | 1408E | 100/500 |
| Gedern | VOR/DME | 5025N | 0915E | 80/500, 70/500 W |
| Germinghausen | VOR | 5110N | 0754E | 60/250 |
| Giessen | NDB | 5038N | 0849E | 40 |
| Glückstadt | NDB | 5351N | 0927E | 30 |
| Gotem | DVOR/DME | 5120N | 1136E | 60NM |
| Hamburg | VOR/DME | 5341N | 1012E | 60/500, 80/500 S |
|  | NDB | 5341N | 1005E | 50 |
|  | L | 5335N | 0953E | 15 |
| Hamburg-Finkenwerder | DME | 5332N | 0950E | 25/100 |
| Hamm | VOR/DME | 5151N | 0743E | 40/500, 60/500 SE |
| Hammelburg | NDB | 5006N | 0947E | 30 |
| Hannover | L | 5228N | 0948E | 15 |
|  | L | 5228N | 0933E | 15 |
|  | DME | 5228N | 0941E | 25/100 |
| Hehlingen | VOR/DME | 5222N | 1048E | 60/500, 80/500 SW |
| Helgoland | VOR/DME | 5411N | 0755E | 60/500, 200/500 SW-N |
|  | NDB | 5411N | 0753E | 100 |
| Hermsdorf | NDB | 5055N | 1422E | 25 |
| Hof | NDB | 5017N | 1146E | 15 |
|  | DME | 5017N | 1151E | 25/100 |
| Karlsruhe | VOR | 4900N | 0835E | 40/500, 60/500 NW & S-E |
| Kassel-Calden | DME | 5128N | 0927E | 25/100 |
| Kempten | VOR/DME | 4745N | 1021E | 60/500, 80/500 SE |
| Kiel | L | 5423N | 1007E | 15 |
| Kirn | VOR/DME | 4951N | 0722E | 40/500 |
| Klasdorf | DVOR/DME | 5201N | 1334E | 60 NM |
| Köln-Bonn | VOR/DME | 5052N | 0709E | 25/150 |
|  | L | 5048N | 0714E | 15 |
|  | L | 5054N | 0714E | 15 |
| König | NDB | 4946N | 0906E | 40 |
| Leipzig | VOR/DME | 5126N | 1228E | 100/250 |
|  | L | 5124N | 1221E | 20 |
|  | L | 5126N | 1207E | 20 |
| Leine | VOR/DME | 5215N | 0953E | 40/500, 60/500 S |
| Lemwerder | DME | 5309N | 0837E | 25/100 |
| Lichtenau | NDB | 5112N | 0942E | 30 |
| Lima/Düsseldorf | NDB | 5122N | 0624E | 25 |
| Lübeck | VOR | 5357N | 1040E | 40/500, 60/500 NE |
|  | L | 5348N | 1042E | 15 |
| Luburg/Stuttgart | VOR/DME | 4855N | 0920E | 40/500, 60/500 N-E |
| Magdeburg | VOR/DME | 5159N | 1148E | 80/500 |
| Maisach | VOR/DME | 4816N | 1119E | 40/300, 60/300 NW |
| Mansbach | NDB | 5047N | 0955E | 30 |
| Metro | VOR | 5017N | 0851E | 60/250 |
| Michaelsdorf | VOR | 5418N | 1100E | 60/500 |
| Mike | NDB | 4834N | 1136E | 25 |
| München | VOR/DME | 4811N | 1149E | 60/500, 40/500 N, 80/500 SE-W |
|  | NDB | 4810N | 1149E | 50 |
|  | L | 4806N | 1136E | 15 |
|  | DME | 4822N | 1148E | 25/100 |
|  | DME | 4821N | 1147E | 25/100 |
|  | NDB | 4821N | 1154E | 25 |
|  | NDB | 4823N | 1155E | 25 |
|  | NDB | 4820N | 1139E | 25 |
|  | NDB | 4821N | 1140E | 25 |
| München/Moosburg | VOR/DME | 4834N | 1216E | 60/500 |
| München/Milldorf | VOR/DME | 4814N | 1220E | 60/500 |
| Münster/Osnabrück | L | 5210N | 0748E | 20 |
|  | DME | 5208N | 0741E | 25/100 |
| Nattenheim | VOR/DME | 5001N | 0632E | 60/500 |
| Neckar | NDB | 4920N | 0844E | 50 |
| Nienburg | VOR | 5238N | 0922E | 40/250 |
| Nördlingen | NDB | 4850N | 1025E | 50 |
| Nörvenich | VOR/DME | 5051N | 0642E | 60/500 |
| Nürnberg | VOR | 4931N | 1049E | 40/150 |
|  | L | 4931N | 1058E | 15 |
|  | DME | 4930N | 1106E | 40/150 |
| Oberpfaffenhofen | DME | 4805N | 1118E | 25/100 |
|  | NDB | 4805N | 1117E | 20 |
| Osnabrück | VOR | 5212N | 0817E | 60/500, 80/500 NE |
| Paderborn-Lippstadt | NDB | 5137N | 0838E | 30 |
|  | DME | 5137N | 0838E | 25/100 |
| Ried | VOR/DME | 4947N | 0833E | 60/250 (DVOR) |
| Roding | VOR/DME | 4902N | 1232E | 60/500 |
| Rothenbach | NDB | 4929N | 1115E | 50 |
| Rüdesheim | NDB | 5002N | 0757E | 25 |
| Saarbrücken | VOR/DME | 4913N | 0707E | 25/100, 40/100 N |
|  | NDB | 4913N | 0707E | 40 |
|  | L | 4913N | 0713E | 25 |
| Siegerland | L | 5041N | 0808E | 25 |
| Solling | NDB | 5141N | 0931E | 50 |
| Spessart | NDB | 4952N | 0921E | 40 |
| Stuttgart | L | 4843N | 0920E | 25 |
|  | L | 4840N | 0907E | 25 |
|  | DME | 4841N | 0913E | 40/150 |
| Sulz | VOR | 4823N | 0839E | 60/250 |
| Sylt | L | 5451N | 0825E | 25 |
| Tango/Stuttgart | VOR/DME | 4837N | 0916E | 60/500, 80/500 E |
|  | NDB | 4837N | 0916E | 50 |
| Taunus | VOR/DME | 5015N | 0810E | 40/250, 60/250 NW |
| Trent | VOR | 5431N | 1315E | 100/500 |
| Walda | VOR/DME | 4835N | 1108E | 80/500, 60/500 W-NW |
| Warburg | VOR/DME | 5130N | 0907E | 80/500 |
| Weser | VOR/DME | 5321N | 0853E | 60/500 |
| Wipper | VOR | 5103N | 0717E | 25/150, 40/250 SE |
| Wunstorf | NDB | 5228N | 0927E | 25 |
| Würzburg | VOR | 4943N | 0957E | 60/300 |
| GREECE |  |  |  |  |
| Alexandroupolis | VOR/DME | 4051N | 2557E | 60/500, 100/500 W |
| Anchialos | VOR/DME | 3913N | 2248E | 40/250 |
| Araxos | VOR/DME | 3809N | 2125E | 80/500, 150/500 SE-W-NW |
| Athinai | VOR/DME | 3753N | 2348E | 120/500 |
| Chania | VOR/DME | 3531N | 2411E | 50/250 |
| Chios | VOR/DME | 3821N | 2609E | 40/250 |
| Didimon | VOR/DME | 3729N | 2313E | 80/500 |
| Fiska | VOR/DME | 4106N | 2300E | 80/500, 150/500 SE |
| Ioannina | VOR/DME | 3942N | 2049E | 60/250 |
| Iraklion | VOR/DME | 3521N | 2511E | 60/250 |
| Kalamata | VOR/DME | 3704N | 2202E | 40/250 |
| Karistos | VOR/DME | 3802 | 2432 | 120/500 |
| Karpathos | NDB | 3525N | 2709E | 25 |
| Kastoria | VOR/DME | 4027 | 02117 | 40/250 |
| Kavala | VOR/DME | 4055N | 2437E | 40/250 |
| Kea | VOR/DME | 3734N | 2418E | 80/500, 120/500 N-SE |
| Kefalina | VOR/DME | 3807N | 2030E | 100/500 |
| Kerkira | VOR/DME | 3927N | 2004E | 100/500 |
| Korinthos | NDB | 3756N | 2256E | 50 |
| Kozani | VOR/DME | 4017N | 2150E | 40/250 |
| Larissa (MIL) | NDB | 3939N | 2226E | 100 |
| Lesvos | VOR | 3914N | 2625E | 60/500, 100/500 SW |
| Limnos | VOR/DME | 3955N | 2514E | 60/500, 100/500 S |
| Mesta | VOR/DME | 3815N | 2554E | 60/500, 150/500 S |
| Mikonos | VOR/DME | 3726N | 2521E | 40/200 |
| Milos | VOR/DME | 3645N | 2431E | 80/500, 120/500 E-NE |
| Paleohora | VOR/DME | 3513N | 2341E | 200/500, 100/500 N |
| Rodos | VOR/DME | 3620N | 2805E | 200/500 |
| Samos | VOR/DME | 3741N | 2654E | 50/250 |
| Santorini | VOR/DME | 3624N | 2529E | 40/250 |
| Sitia | VOR/DME | 3504N | 2612E | 80/500, 150/500 NW, 200/500 E-S |
| Skopelos | VOR/DME | 3911N | 2337E | 150/500 |
| Thessaloniki | VOR/DME | 4027N | 2259E | 100/500, 150/500 SE |
| Tripolis | VOR | 3724N | 2220E | 60/500, 150/500 E |
| Zakinthos | VOR/DME | 3745N | 2053E | 40/250 |
| HUNGARY |  |  |  |  |
| Békés | DVOR/DME | 4648N | 2104E | 100/500 |
|  | NDB | 4648N | 2104E | 100/500 |
| Bekescsaba | L | 4640N | 2110E | 25 |
| Budaors | L | 4727N | 1900E | 25 |
| Budapest-Ferihegy | TVOR/DME | 4727N | 1915E | 40/250 |
|  | ILS/DME | 4727N | 1915E | 25/6.25 |
|  | ILS/DME | 4726N | 1915E | 25/6.25 |
|  | ILS/DME | 4727N | 1913E | 25/6.25 |
|  | ILS/DME | 4725N | 1918E | 25/6.25 |
|  | L | 4729N | 1909E | 25 |
|  | L | 4729N | 1911E | 25 |
|  | L | 4727N | 1913E | 25 |
|  | L | 4727N | 1915E | 25 |
|  | L | 4725N | 1918E | 25 |
|  | L | 4723N | 1922E | 25 |
|  | L | 4725N | 1916E | 25 |
|  | L | 4724N | 1917E | 25 |
| Bugac | VOR/DME | 4641N | 1941E | 100/500 |
|  | NDB | 4641N | 1941E | 100/500 |
| Debrecen | L | 4729N | 2136E | 25 |
|  | L | 4727N | 2134E | 25 |
| Györ | VOR/DME | 4740N | 1743E | 100/500 |
|  | NDB | 4740N | 1743E | 100/500 |
| Jaszbereny | NDB | 4730N | 1954E | 60 |
| Kaposujlak | L | 4622N | 1744E | 25 |
| Monor | VOR/DME | 4720N | 1924E | 100/500 |
|  | NDB | 4720N | 1924E | 100/500 |
| Nagycserkesz | NDB | 4758N | 2132E | 60 |
| Nyiregyhaza | L | 4758N | 2142E | 25 |
|  | L | 4756N | 2142E | 25 |
|  | VOR | 4759N | 2142E | Planned |
| Pecspogany | NDB | 4600N | 1814E | 30 |
| Pusztaszabolcs | VOR/DME | 4709N | 1845E | 100/500 |
|  | NDB | 4709N | 1845E | 100/500 |
| Sagvar | VOR/DME | 4650N | 1807E | 100/500 |
|  | NDB | 4650N | 1807E | 100/500 |
| Sajohidvég | VOR/DME | 4800N | 2100E | 100/500 |
|  | NDB | 4800N | 2100E | 100/500 |
| Sarmellek | L | 4640N | 1710E | 25 |
| Siofok/Kiliti/Sagvar | ILS/DME | 4652N | 1805E | 25/100 |
| Szeged | L | 4616N | 2005E | 25 |
|  | L | 4618N | 2005E | 25 |
| Tapiosap | VOR/DME | 4730N | 1927E | 100/500 |
|  | NDB | 4730N | 1927E | 100/500 |
| IRELAND |  |  |  |  |
| Baldonnel | VOR/DME | 5318N | 0627W | 60 |
| Clonmel | NDB | 5227N | 0729W | 50 |
| Connaught | VOR/DME | 5355N | 0849W | 100/500, 300/700 (180 T-360 T) |
|  | L | 5354N | 0856W | 25 |
|  | L | 5355N | 0842W | 25 |
| Cork | VOR/DME | 5150N | 0830W | 100/500, 300/700 (180 T-360 T) |
|  | L | 5154N | 0832W | 25 |
|  | L | 5145N | 0826W | 25 |
| Donegal | NDB | 5503N | 0820W | 25 |
|  | DME | 5503N | 0820W | 25 |
| Dublin | VOR/DME | 5330N | 0618W | 100/500, 300/700 (180 T-360 T) |
|  | L | 5326N | 0626W | 25 |
|  | L | 5325N | 0608W | 25 |
|  | NDB | 5316N | 0606W | 50 |
|  | NDB | 5332N | 0627W | 45 |
|  | NDB | 5331N | 0607W | 30 |
|  | NDB | 5339N | 0614W | 30 |
|  | DME | 5339N | 0614W | 30 |
| Galway | NDB | 5318N | 0857W | 30 |
|  | DME | 5318N | 0857W | 30 |
| Kerry | NDB | 5211N | 0931W | 25 |
|  | DME | 5211N | 0931W | 25 |
| Shannon | VOR/DME | 5243N | 0853W | 100/500, 300/700 (180 T-360 T) |
|  | NDB | 5234N | 0912W | 50 |
|  | NDB | 5254N | 0856W | 40 |
|  | L | 5245N | 0849W | 25 |
| Sligo | DME | 5417N | 0836W | 20 |
|  | NDB | 5417N | 0836W | 20 |
| Waterford | NDB | 5211N | 0705W | 15 |
|  | DME | 5211N | 0705W | 25 |
| ITALY |  |  |  |  |
| Albenga | NDB | 4403N | 0813E | 50 |
| Alghero | VOR/TAC | 4038N | 0815E | 200/500SE, 150/500 |
|  | NDB | 4035N | 0816E | 100 |
| Amendola | NDB | 4130N | 1550E | 25 |
| Ancona | VOR/DME | 4335N | 1328E | 80/500, 100/500 SE |
|  | NDB | 4335N | 1328E | 50 |
| Bari | VOR/DME | 4108N | 1645E | 40/250 |
|  | L | 4106N | 1639E | 25 |
| Bergamo/Orio al Serio | VOR/DME | 4540N | 0942E | 40/250 |
|  | L | 4539N | 0951E | 25 |
| Bologna | VOR/DME | 4432N | 1117E | 60/500, 80/500 NW-SE |
|  | NDB | 4434N | 1112E | 30 |
| Bolsena | VOR/TAC | 4237N | 1203E | 100/500 |
| Bolzano | VOR/DME | 4618N | 1119E | 100/500 |
|  | NDB | 4628N | 1119E | 60 |
| Brindisi | VOR/TAC | 4037N | 1800E | 200/600 |
|  | NDB | 4036N | 1801E | 78 |
| Cagliari | VOR/DME | 3915N | 0903E | 25/100 |
|  | L | 3913N | 0906E | 25 |
| Campagnano | VOR/DME | 4207N | 1223E | 20/250, 70/250 NW |
| Caraffa | VOR/TAC | 3845N | 1622E | 200/600 |
|  | NDB | 3845N | 1622E | 150 |
| Carbonara | VOR/DME | 3907N | 0930E | 80/500,100/500 N, 130/500 NE,SW |
|  | NDB | 3906N | 0931E | 100 |
| Catania | VOR/DME | 3727N | 1458E | 100/500 |
|  | NDB | 3727N | 1458E | 50 |
| Chioggia | VOR/DME | 4504N | 1217E | 60/500, 100/500 SE |
|  | NDB | 4504N | 1217E | 50 |
| Crotone | VOR/DME | 3900N | 1705E | 60/500, 100/500 NW, 150/500 E |
|  | L | 3900N | 1704E | 25 |
| Decimomannu | NDB | 3922N | 0858E | 50 |
| Elba | VOR/TAC | 4244N | 1024E | 200/600 |
| Ferrara | NDB | 4449N | 1137E | 50 |
| Firenze | VOR/TAC | 4402N | 1100E | 200/600 |
| Firenze/Peretola | VOR/DME | 4349N | 1112E | 50/100 |
| Forli | L | 4415N | 1155E | 25 |
| Genova | VOR/DME | 4425N | 0905E | 80/500, 100/500 NE & S |
|  | NDB | 4425N | 0905E | 100 |
| Genova/Sestri | VOR/DME | 4425N | 0851E | 25/100 |
|  | L | 4420N | 0917E | 70 |
| Lamezia | VOR/DME | 3854N | 1616E | 25/100 |
| Lampedusa | VOR/DME | 3530N | 1238E | 80/400 |
|  | NDB | 3530N | 1237E | 50 |
| Latina | VOR/DME | 4132N | 1255E | 60/250 |
| Milano/Linate | VOR/DME | 4528N | 0917E | 450/250 |
|  | L | 4521N | 0917E | 25 |
| Milano/Malpensa | VOR/DME | 4539N | 0844E | 40/250 |
|  | L | 4533N | 0845E | 25 |
| Napoli | VOR/DME | 4053N | 1418E | 40/250 |
|  | L | 4051N | 1414E | 25 |
| Novara | NDB | 4525N | 0848E | 50 |
|  | DME | 4525E | 0848E | 50/250 |
| Olbia/Costa Smeralda | VOR/DME | 4053N | 0930E | 50/250 |
|  | NDB | 4054N | 0931E | 50 |
| Ostia | NDB | 4148N | 1214E | 50 |
|  | VOR/DME | 4148N | 1214E | 100/500, 200/500S |
| Padova | NDB | 4524N | 1151E | 25 |
|  | DME | 4524N | 1151E | 25/100 |
| Palermo | VOR/DME | 3802N | 1311E | 100/500, 200/500S |
| Palermo/Punta Raisi | VOR/DME | 3810N | 1305E | 40/250 |
| Pantelleria | VOR/DME | 3649N | 1158E | 80/500, 150/500S |
|  | NDB | 3649N | 1157E | 50 |
| Parma | NDB | 4449N | 1018E | 50 |
| Pescara | VOR/DME | 4226N | 1411E | 80/500, 120/500 E |
| Pisa | VOR/DME | 4341N | 1023E | 40/500, 80/500 S |
| Pomigliano | NDB | 4056N | 1423E | 25 |
| Ponza | TAC | 4055N | 1257E | 200/600 |
| Reggio Calabria | VOR/DME | 3805N | 1539E | 40/250, 80/250 N |
|  | NDB | 3801N | 1558E | 50 |
| Roma/Fiumiccino | L | 4155N | 1214E | 25 |
|  | L | 4153N | 1212E | 25 |
|  | L | 4150N | 1221E | 25 |
| Romagnano | L | 4538N | 0824E | 25 |
| Ronchi dei Legionari | VOR/DME | 4546N | 1329E | 40/250 |
|  | L | 4546N | 1322E | 25 |
| Saronno | VOR/DME | 4539N | 0901E | 120/500, 80/500 E-S |
|  | L | 4539N | 0901E | 25 |
| Sorrento | VOR/DME | 4035N | 1420E | 120/500, 60/500 NW-N |
|  | NDB | 4035N | 1420E | 100 |
| Tarquinia | VOR/DME | 4213N | 1144E | 40/250 |
| Teano | VOR/DME | 4118N | 1358E | 60/500, 100/500 E |
|  | NDB | 4118N | 1358E | 50 |
| Torino Poirino | VOR/DME | 4455N | 0752E | 80/500, 100/500 SE |
|  | NDB | 4455N | 0752E | 50 |
| Torino Caselle | VOR/DME | 4513N | 0739E | 45/250 |
|  | L | 4507N | 0739E | 25 |
| Trapani/Birgi | VOR/TAC | 3754N | 1231E | 80/500 |
| Trezzo | VOR/DME | 4534N | 0930E | 60/500, 100/500 N |
| Venezia | L | 4527N | 1217E | 25 |
| Vicenza | VOR/DME | 4538N | 1141E | 80/500 |
|  | NDB | 4538N | 1141E | 50 |
| Vieste | VOR/DME | 4155N | 1603E | 80/500 |
| Voghera | VOR/DME | 4458N | 0858E | 60/500 |
| KAZAKHSTAN |  |  |  |  |
| Akchi | NDB | 4358N | 07620E | 150 |
| Aktau | VOR/DME | 4352N | 05106E | 250/250 |
|  | NDB | 4350N | 05109E | 150 |
| Aktyubinsk | VOR/DME | 5016N | 05711E | 150/500 |
|  | NDB | 5017N | 05709E | 80 |
| Almaty | VOR/DME | 4322N | 07705E | 150/500 |
|  | NDB | 4323N | 07706E | 80 |
| Aralsk | NDB | 4650N | 06136E | 150 |
| Arkalyk | NDB | 5019N | 06657E | 80 |
| Astana | VOR/DME | 5100N | 07126E | 250/250 |
|  | NDB | 5104N | 07132E | 150 |
| Atyrau | VOR/DME | 4706N | 05151E | 150/500 |
|  | NDB | 4705N | 05151E | 80 |
| Ayaguz | VOR | 4756N | 08027E | 150/500 |
|  | NDB | 4756N | 08027E | 80 |
| Balkhash | VOR/DME | 4653N | 07459E | 150/500 |
|  | NDB | 4652N | 07457E | 80 |
| Ekibastuz | NDB | 5137N | 07516E | 150 |
| Karaganda | VOR/DME | 4942N | 07322E | 250/250 |
|  | NDB | 4942N | 07325E | 150 |
| Kokshetau | NDB | 5322N | 06939E | 150 |
| Kostanay | NDB | 5310N | 06335E | 80 |
| Kyzylorda | VOR/DME | 4442N | 06536E | 250/250 |
|  | NDB | 4441N | 06532E | 150 |
| Novokazalinsk | VOR/DME | 4550N | 06208E | 150/500 |
|  | NDB | 4550N | 06208E | 80 |
| Pavlodar | NDB | 5214N | 07707E | 150 |
| Petropavlovsk | NDB | 5446N | 06916E | 150 |
| Semipalatinsk | NDB | 5021N | 08019E | 150 |
| Shalkar | NDB | 4750N | 05931E | 150 |
| Shymkent | VOR/DME | 4222N | 06930E | 150/500 |
|  | NDB | 4223N | 06925E | 80 |
| Taraz | NDB | 4253N | 07115E | 80 |
| Tole bi | NDB | 4342N | 07345E | 150 |
| Uralsk | VOR/DME | 5109N | 05133E | 250/250 |
|  | NDB | 5111N | 05136E | 150 |
| Ust-Kamenogorsk | NDB | 5000N | 08233E | 80 |
| Uzunagach | NDB | 4336N | 07620E | 80 |
| Zhezkazgan | VOR/DME | 4743N | 06746E | 150/500 |
|  | NDB | 4744N | 06748E | 80 |
| KYRGYZSTAN |  |  |  |  |
| Balykchi | L | 4226N | 07608E | 80 |
| Bishkek | VOR/DME | 4304N | 07431E | 100/500 |
|  | NDB | 4303N | 07424E | 80 |
|  | NDB | 4304N | 07433 | 80 |
|  | L | 4252N | 07431E | 80 |
| Chaldovar | L | 4249N | 07333E | 80 |
| Dzhalal-abad | L | 4056N | 07258E | 80 |
| Kalinovka | L | 4242N | 07522E | 80 |
| Karakol | L | 4231N | 07821E | 80 |
| Narynges | L | 4110N | 07210E | 80 |
| Osh | NDB | 4038N | 07244E | 80 |
| LATVIA |  |  |  |  |
| Daugavpils | VOR/DME | 5556N | 2640E |  |
| Liepaja | VOR/DME | 5631N | 2105E |  |
| Riga | DVOR/DME | 5655N | 2357E |  |
| Trakshi | VOR/DME | 5646N | 2621E |  |
| Ventspils | VOR/DME | 5722N | 2133E |  |
| LITHUANIA |  |  |  |  |
| Kaunas | VOR/DME | 5458N | 2404E | 100/500 |
|  | L | 5458N | 2410E | 25 |
| Klaipèda | VOR/DME | 5543N | 2115E | 60/500 |
| Palanga | L | 5559N | 2106E | 80 |
| Palanga | DME | 5557N | 2105E | 50 |
| Vilnius | DVOR/DME | 5438N | 2518E | 80/500 |
|  | L | 5439N | 2518E | 25 |
| Siauliai | DVOR/DME | 5553N | 2325E | 80/500 |
| LUXEMBOURG |  |  |  |  |
| Diekirch | DVOR/DME (DIK) | 4952N | 0608E | 70/500, 100/500 NW |
|  | NDB (DIK) | 4952N | 0608E | 50 |
| Luxembourg | DVOR/DME (LUX) | 4938N | 0615E | 60/250 |
|  | NDB (LE) | 4938N | 0615E | 25 |
|  | NDB (LW) | 4937N | 0611E | 25 |
|  | NDB (ELU) | 4941N | 0621E | 25 |
|  | NDB (WLU) | 4034N | 0603E | 25 |
| MALTA |  |  |  |  |
| Gozo | VOR/DME | 3602N | 1412E | 300/500 |
| Malta | NDB | 3549N | 1432E | 40 |
| NETHERLANDS |  |  |  |  |
| Amsterdam/Schiphol | VOR/DME | 5217N | 0445E | 60/250 |
|  | L | 5209N | 0446E | 15 |
|  | L | 5220N | 0502E | 15 |
|  | L | 5213N | 0433E | 15 |
|  | L | 5228N | 0445E | 15 |
| Den Helder | VOR/DME | 5244N | 0446E | 100/250, 60/250 SE-SW |
| Eelde | VOR/DME | 5310N | 0640E | 60/500, 150/500 N-NE |
|  | L | 5308N | 0636E | 15 |
| Enko | NDB | 5240N | 0514E | 25 |
| Haamstede | VOR/DME | 5143N | 0351E | 80/250, 50/250 SE |
| Lake | NDB | 5231N | 0534E | 25 |
| Maastricht | VOR/DME | 5058N | 0558E | 40/250 |
|  | L | 5101N | 0553E | 15 |
|  | L | 5056N | 0548E | 15 |
| Nieuwkoop | L | 5209N | 0446E | 15 |
| Nyke | NDB | 5214N | 0532E | 25 |
| Pampus | VOR/DME | 5220N | 0506E | 80/500, 120/500 SW, |
| 150/500 NW-N | T,E |  |  |  |
| Rekken | VOR/DME | 5208N | 0646E | 50/500, 60/500 SE |
| Rotterdam | VOR/DME | 5158N | 0429E | 50/250, 100/250 W-N |
|  | NDB | 5154N | 0433E | 25 |
|  | L | 5157N | 0425E | 15 |
|  | L | 5152N | 0413E | 15 |
| Spijkerboor | VOR/DME | 5232N | 0451E | 100/500, 150/500 NW |
|  | NDB | 5232N | 0450E | 75 |
| Stad | NDB | 5145N | 0415E | 15 |
| Thorn | NDB | 5111N | 0550E | 25 |
| Twenthe | NDB | 5216N | 0653E | 25 |
| Woensdrecht | NDB | 5127N | 0421E | 25 |
| NORWAY |  |  |  |  |
| Advent | NDB | 7811N | 1601E | 50 |
| Aalesund/Lepsøy | L | 6236N | 0613E | 30 |
| Alta | DVOR/DME | 6959N | 2322E | 60/500 |
| Alta | NDB | 7000N | 2318E | 50 |
| Andøya | VOR/DME | 6917N | 01608E | 200/500 |
| Askøy | NDB | 6025N | 0511E | 50 |
| Banak | VOR/DME | 7011N | 2457E | 60/250 |
| Bardufoss | DVOR/DME | 6904N | 01828E | 40/500 |
| Bergerud | NDB | 5951N | 01116E | 50 |
| Bjørnøya | NDB | 7430N | 01905E |  |
| Bodø | DVOR/DME | 6716N | 1422E | 200/500 |
| Bodø/Ilstad | L | 6716N | 1443E | 30 |
| Brandbu | DVOR/DME | 6028N | 01020E | 150/500 |
| Bratta | NDB | 6003N | 0518E | 50 |
| Brønnøy | DVOR/DME | 6527N | 1213E |  |
| Drammen | DVOR/DME | 5944N | 1007E | 100/500, 150/500 S |
| Evenes | DVOR/DME | 6827N | 1641E | 60/250 |
| Evenes/Fjellstad | L | 6836N | 1640E | 30 |
| Fauske | NDB | 6716N | 01508E | 50 |
| Fleinvar | NDB | 6710N | 01346E | 50 |
| Flesland | DVOR/DME | 6018N | 0513E | 100/500 |
| Florø | VOR/DME | 6135N | 00501E | 100/500 |
| Flornes | NDB | 6328N | 01120E | 50 |
| Gardermoen | DVOR/DME | 6012N | 1104E | 60/250 |
| Graakallen | NDB | 6325N | 01016E | 50 |
| Hammerfest | DVOR/DME | 7042N | 02341E | 100/500 |
| Hansmark | NDB | 6939N | 1854E | 30 |
| Hestvik | NDB | 6956N | 0210E | 50 |
| Isfjord | NDB | 7804N | 01336E | 50 |
| Kalhovd | VOR/DME | 6003N | 0822E | 60/500 |
| Karmøy | DVOR/DME | 5921N | 00512E | 100/500 |
| Kirkenes | VOR/DME | 6943N | 2952E | 60/500 |
| Kobbe | NDB | 6930N | 01849E | 50 |
| Kristiansand/Birkeland | L | 5818N | 0814E | 30 |
| Kvernberget | DVOR/DME | 6307N | 0749E | 100/500 |
| Longyear | NDB | 7815N | 1524E | 50 |
| Lyse | VOR/DME | 5908N | 0653E | 60/500 |
| Mesnali | DVOR/DME | 6106N | 01039E | 100/500 |
| Morskogen | NDB | 6027N | 01131E | 50 |
| Odden | NDB | 6824N | 01607E | 50 |
| Odderøy | NDB | 5808N | 0800E | 50 |
| Opaker | NDB | 6012N | 01131E |  |
| Rambu | NDB | 6230N | 1133E |  |
| Rennesøy | NDB | 5908N | 0539E | 50 |
| Rørvik | VOR/DME | 6450N | 01109E | 40/250 |
| Seida | VOR/DME | 7014N | 2804E | 60/500 |
| Sigdal | DVOR/DME | 6001N | 00938R | 100/500 |
| Skagen | VOR/DME | 6835N | 01502E | 60/250 |
| Skien | DVOR/DME | 5911N | 00934E | 60/250 |
| Slettnes | VOR/DME | 7105N | 02814E | 60/250 |
| Sokna | NDB | 6014N | 00955E | 50 |
| Solberg | NDB | 6001N | 01058E | 50 |
| Sola | DVOR/DME | 5853N | 0538E | 60/500, 200/500 S-W |
| Stokka | DVOR/DME | 6558N | 1228E | 60/250 |
| Støtt | NDB | 6656N | 01326E | 50 |
| Svensheia | DVOR/DME | 5805N | 0754E | 150/500, 250/500 SW |
| Tolga | VOR/DME | 6228N | 1053E | 60/250 |
| Torp | DVOR/DME | 5910N | 01016E | 100/500 |
| Torp/Dalen | L | 5916N | 1015E | 30 |
| Torp/Sandefjord | L | 5906N | 1016E | 50 |
| Tromsø | DVOR/DME | 6942N | 1900E | 100/500 |
| Tromsø/Kvalsund | L | 6949N | 01900E | 30 |
| Tromsø/Lanes | L | 6943N | 1857E | 25 |
| Trondheim | VOR/DME | 6332N | 1053E | 150/500, 200/500 N |
| Trondheim/Hegra | L | 6328N | 1105E | 25 |
| Tyrifjord | NDB | 5956N | 01016E |  |
| Vardefjell | DVOR/DME | 6553N | 01318E | 60/500 |
| Varhaug | NDB | 5838N | 00538E | 50 |
| Vigra | DVOR/DME | 6234N | 0608E | 100/500, 200/500 SW-N |
| Vollo | DVOR/DME | 6033N | 00508E | 50/400 |
| POLAND |  |  |  |  |
| Bydgoszcz | NDB | 5306N | 1803E | 25 |
| Chociwel | NDB | 5329N | 1520E | 40 |
| Czempin | VOR/DME | 5208N | 1644E | 60/500 |
| Darlowo | DVOR/DME | 5425N | 1623E | 80/500 |
| Drezdenko | DME | 5204N | 1657E | 80/500 |
| Gdansk | NDB | 5421N | 1836E | 40 |
|  | LCTR | 5422N | 1830E | 25 |
| Grudziadz | DVOR/DME | 5331N | 1847E | 80/500 |
| Jablonka | VOR/DME | 4929N | 1941E | 60/500 |
| Jedrzejow | VOR/DME | 5039N | 2015E | 80/500 (100/500, 150/500 E: National requirement) |
| Karnice | VOR/DME | 5157N | 2027E | 60/500 |
| Kartuzy | VOR/DME | 5418N | 1812E | 60/500 |
| Katowice | NDB | 5028N | 1909E | 40 |
|  | LCTR | 5028N | 1906E | 25 |
| Kmiecin | VOR | 5412N | 1909E | 60/500 |
| Krakow | NDB | 5006N | 1955E | 40 |
|  | LCTR | 5005N | 1949E | 25 |
| Linin | VOR/DME | 5156N | 2110E | 60/500 |
| Lodz | VOR/DME | 5148N | 1939E | 100/500, 80/500 E (150/500, 80/500 E: National requirement) |
| Mragowo | DVOR/DME | 5347N | 02108E | 80/500 |
| Poznan | DVOR/DME | 5225N | 1650E | 80/500 |
| Rzeszow | DVOR/DME | 5006N | 2201E | 80/500 |
|  | NDB | 5007N | 2208E | 40 |
|  | LCTR | 5007N | 2203E | 25 |
| Siedlce | VOR/DME | 5209N | 2212E | 80/500, 150/500 E (150/500, 80/500 SW-NW: National requirement) |
| Slubice | DVOR/DME | 5223N | 1435E | 80/500 (150/500: National requirement) |
| Suwalki | DVOR/DME | 5404N | 2254E | 86/500 |
| Szczecin | NDB | 5333N | 1458E | 40 |
| Trzebielino Trzebnica | DME VOR/DME | 5104N 5118N | 1657E 1707E | 40/100 60/500 |
| Warszawa | DVOR/DME | 5210N | 2058E | 80/500 |
|  | LCTR | 5210N | 2055E | 25 |
| Wroclaw | NDB | 5105N | 1657E | 40 |
|  | LCTR | 5106N | 1655E | 25 |
| Zaborowek | DVOR/DME | 5216N | 2039E | 80/500 (80/500, 150/500 NE: National requirement) |
| Zelona Góra | LCTR | 5209N | 1550E | 25 |
| PORTUGAL |  |  |  |  |
| Arruda | NDB | 3900N | 0902W | 50 |
| Beja | VOR/DME | 3808N | 0756W | 80/500, 150/500 N |
| Caparica | NDB | 3839N | 0913W | 250 |
| Espichel | VOR/DME | 3825N | 0911W | 80/500, 200/500 SW-W |
| Faro | VOR/DME | 3701N | 0759W | 60/500, 200/500 S-W |
|  | NDB | 3700N | 0756W | 250 |
| Fatima | VOR/DME | 3940N | 0830W | 60/500 |
| Funchal | VOR/DME | 3245N | 1642W | 200/500 |
|  | NDB | 3245N | 1642W | 50 |
| Lisboa | VOR/DME | 3853N | 0910W | 80/500 |
|  | L | 3851N | 0906W | 25 |
| Nisa | VOR/DME | 3934N | 0755W | 60/500,200/500 W |
| Porto | VOR/DME | 4116N | 0841W | 80/500, 200/500 W |
|  | NDB | 4121N | 0842W | 250 |
|  | L | 4105N | 0838W | 25 |
| Porto Santo | VOR/DME | 3305N | 1621W | 200/500 |
|  | NDB | 3304N | 1621W | 250 |
| Sagres | VOR/DME | 3705N | 0857W | 60/500, 200/500 S-W |
| Viseu | VOR/DME | 4043N | 0753W | 100/500, 200/500 W |
| REPUBLIC OF MOLDOVA |  |  |  |  |
| Chisinau | DVOR/DME | 46° 55’34”N | 028°54’16”E | 200/FL400 |
| ROMANIA |  |  |  |  |
| Arad | DVOR/DME | 4611N | 2109E | 175/500 |
|  | NDB | 4611N | 2109E | 50 |
| Bacau | DVOR/DME | 4631N | 2650E | 175/500 |
|  | NDB | 4629N | 2656E | 50 |
|  | L | 4630N | 2655E | 25 |
| Baia Mare | NDB | 4740N | 2321E | 100 |
|  | L | 4740N | 2326E | 25 |
| Brasov | DVOR/DME | 4534N | 2534E | 175/500 |
| Bucuresti/Baneasa | L | 4431N | 2608E | 25 |
|  | L | 4432N | 2614E | 25 |
|  | L | 4428N | 2557E | 25 |
| Bucuresti/Otopeni | L | 4434N | 2613E | 25 |
|  | L | 4435N | 2608E | 25 |
|  | L | 4435N | 2614E | 25 |
|  | L | 4435N | 2604E | 25 |
|  | L | 4434N | 2600E | 25 |
|  | L | 4433N | 2559E | 25 |
| Cluj | DVOR/DME | 4648N | 2347E | 175/500 |
| Constanta | VOR/DME | 4418N | 2829E | 150/500 |
|  | L | 4426N | 2830E | 50 |
| Craiova | DVOR/DME | 4419N | 2355E | 100 |
|  | NDB | 4419N | 2400E | 50 |
| Deva | DVOR/DME | 4550N | 2258E | 86 |
| Floresti | VOR/DME | 4430N | 2542E | 150/500 |
| Galati | DVOR/DME | 4525N | 2756E | 175/500 |
| Iasi | DME | 4711N | 2737E | 50 |
|  | NDB | 4714N | 2735E | 50 |
|  | L | 4712N | 2736E | 25 |
| Oradea | NDB | 4706N | 2155E | 100 |
|  | L | 4703N | 2154E | 25 |
| Satu Mare | DVOR/DME | 4744N | 2254E | 150/500 |
|  | NDB | 4743N | 2254E | 75 |
| Sibiu | NDB | 4547N | 2409E | 100 |
| Strejnic | VOR/DME | 4455N | 2559E | 80/500 |
| Suceava | DVOR/DME | 4740N | 2622E | 86 |
| Timisoara | NDB | 4549N | 2118E | 50 |
|  | L | 4550N | 2114E | 25 |
|  | L | 4548N | 2122E | 25 |
| Targu Jiu | VOR/DME | 4504N | 2320E | 150/500 |
| Targu Mures | NDB | 4627N | 2418E | 80 |
|  | L | 4628N | 2423E | 25 |
| Tulcea | DVOR/DME | 4504N | 2842E | 50 |
|  | DME | 4503N | 2843E | 25 |
| RUSSIAN FEDERATION |  |  |  |  |
| Abaken | VOR/DME | 5344N | 09123E | 300 km / 14000 m |
| Achinsk | NDB | 5616N | 09036E | 150 km |
| Aginskoy | NDB | 5515N | 09452E | 150 km |
| Agoy | NDB | 4408N | 03901E | 150 km |
| Alatyr | NDB | 5449N | 04634E | 150 km |
| Algasovo | NDB | 5341N | 04140E | 150 km |
| Artemovskiy | NDB | 5720N | 06154E | 150 km |
| Bagayevskiy | NDB | 4719N | 04021E | 150 km |
| Bakchar | NDB | 5701N | 08203E | 150 km |
| Balakovo | NDB | 5153N | 04747E | 150 km |
| Barabinsk | NDB | 5521N | 07818E | 150 km |
| Baykit | NDB | 6142N | 09629E | 150 km |
| BAZARNYYE MATAKI | NDB | 5453N | 04954E | 150 km |
| BAZHENOVO | NDB | 5538N | 07305E | 150 km |
| BELOZERSK | NDB | 6001N | 03745E | 150 km |
| BELY | NDB | 5550N | 03255E | 150 km |
| BERINGOVSKIY | NDB | 6301N | 17918E | 150 km |
| BLAGOVESHCHENSK/Ignatyevo | VOR/DME | 5023N | 12725E | 300 km / 14000 m |
| BOGDANOVO | NDB | 5706N | 03742E | 150 km |
| BOGUCHANY | NDB | 5822N | 09722E | 150 km |
| BRATSK | VOR/DME | 5622N | 10141E | 300 km / 14000 m |
| BUINSK | NDB | 5459N | 04815E | 150 km |
| BUTURLINO | NDB | 5534N | 04454E | 150 km |
| CHAPAYEV | NDB | 5615N | 07356E | 150 km |
| CHELYABINSK/Balandino | VOR/DME | 5518N | 06131E | 300 km / 14000 m |
| CHERNUKHA | NDB | 5536N | 04346E | 150 km |
| CHERNYSHEVKA | NDB | 4412N | 13307E | 150 km |
| CHERUSTI | NDB | 5533N | 03959E | 150 km |
| CHUDOVO | NDB | 5906N | 03138E | 150 km |
| DOBRYNSKOYE | NDB | 5616N | 04035E | 150 km |
| DUDINKA | NDB | 6924N | 08613E | 150 km |
| DZHUBGA | NDB | 4418N | 03842E | 150 km |
| EKIMCHAN | NDB | 5308N | 13249E | 150 km |
| ELISTA | VOR/DME | 4622N | 04421E | 300 km / 14000 m |
| GAGARIN | NDB | 5532N | 03501E | 150 km |
| GOREVOYE | NDB | 5638N | 09239E | 150 km |
| GORKA | NDB | 5949N | 03220E | 150 km |
| IGARKA | VOR/DME | 6726N | 08638E | 300 km / 14000 m |
| ISHIM | NDB | 5608N | 06924E | 150 km |
| KALAUS | NDB | 4327N | 04512E | 150 km |
| KARGOPOL | NDB | 6132N | 03857E | 150 km |
| KARMANOVO | NDB | 5549N | 03451E | 150 km |
| KAZAN | VOR/DME | 5536N | 04916E | 300 km / 14000 m |
| KEM | NDB | 6456N | 03429E | 150 km |
| KHABAROVSK | VOR/DME | 4832N | 13512E | 300 km / 14000 m |
| KHANTY-MANSIYSK | VOR/DME | 6102N | 06907E | 300 km / 14000 m |
| KIKERINO | NDB | 5926N | 02937E | 150 km |
| KIRISHI | NDB | 5926N | 03202E | 150 km |
| KISLOVODSK | NDB | 4357N | 04237E | 150 km |
| KIZLYAR | NDB | 4350N | 04642E | 150 km |
| KOBONA | NDB | 6001N | 03132E | 150 km |
| KOLYVAN | NDB | 5519N | 08242E | 150 km |
| KONSTANTINOVSK | NDB | 4735N | 04106E | 150 km |
| KOTLAS | VOR/DME | 6114N | 04642E | 300 km / 14000 m |
| KOTLY | NDB | 5936N | 02846E | 150 km |
| KOZHEVNIKOVO | NDB | 5614N | 08357E | 150 km |
| KRASNAYA GORBATKA | NDB | 5552N | 04145E | 150 km |
| KRASNOARMEYSK | NDB | 5102N | 04538E | 150 km |
| KRASNOYARSK | VOR/DME | 5610N | 09230E | 300 km / 14000 m |
| KRASNY SULIN | NDB | 4753N | 04006E | 150 km |
| KYZYL | DME | 5135N | 09412E | 300 km / 14000 m |
| LARIONOVO | NDB | 5602N | 03937E | 150 km |
| LAVRENTIYA | NDB | 6534N | 17100W | 150 km |
| LAZAREVSKOYE | NDB | 4354N | 03920E | 150 km |
| LYSKOVO | NDB | 5600N | 04501E | 150 km |
| MAGDAGACHI | NDB | 5328N | 12548E | 150 km |
| MAGNITOGORSK | VOR/DME | 5324N | 05845E | 300 km / 14000 m |
| MAKHACHALA/Uytash | VOR/DME | 4249N | 04738E | 300 km / 14000 m |
| MAKSIMKIN YAR | NDB | 5838N | 08643E | 150 km |
| MALINOVKA | NDB | 5642N | 08520E | 150 km |
| MALKA | NDB | 5319N | 15732E | 150 km |
| MALOYE SKURATOVO | NDB | 5333N | 03702E | 150 km |
| MAMA | NDB | 5819N | 11253E | 150 km |
| MAMADYSH | NDB | 5544N | 05123E | 150 km |
| MATVEYEVSKIY | NDB | 5456N | 08304E | 150 km |
| MENDELEYEVO | NDB | 5810N | 05458E | 150 km |
| MEZEN | NDB | 6552N | 04411E | 150 km |
| MIKHAYLOVSK | NDB | 5627N | 05907E | 150 km |
| MIRNY | VOR/DME | 6232N | 11401E | 300 km / 14000 m |
| MOGOCHA | NDB | 5343N | 11945E | 150 km |
| MOROZOVSK | VOR/DME | 4820N | 04148E | 300 km / 14000 m |
| MURAVEYKA | NDB | 4353N | 13315E | 150 km |
| MURAVLYANKA | NDB | 5345N | 03829E | 150 km |
| NARYAN-MAR | VOR/DME | 6738N | 05306E | 300 km / 14000 m |
| NERL | NDB | 5702N | 03759E | 150 km |
| NERYUNGRI/Chulman | VOR/DME | 5654N | 12452E | 300 km / 14000 m |
| NIKOLSK | NDB | 5244N | 10427E | 150 km |
| NIKOLSKOYE | NDB | 5402N | 04910E | 150 km |
| NIKOLSKOYE | NDB | 5510N | 16602E | 150 km |
| NIZHNEUDINSK | VOR | 5453N | 09903E | 300 km / 14000 m |
| NIZHNEVARTOVSK | VOR/DME | 6056N | 07628E | 300 km / 14000 m |
| NIZHNEYANSK | NDB | 7125N | 13608E | 150 km |
| NIZHNEYE SANCHELEYEVO | NDB | 5341N | 04925E | 150 km |
| NIZHNYAYA PESHA | NDB | 6645N | 04745E | 150 km |
| NORILSK/Alykel | VOR/DME | 6918N | 08718E | 300 km / 14000 m |
| NOVOLOKTI | NDB | 5423N | 08255E | 150 km |
| NOVOSIBIRSK/Tolmachevo | VOR/DME | 5500N | 08238E | 300 km / 14000 m |
| NOVOTYRYSHKINO | NDB | 5517N | 08223E | 150 km |
| NOVYVASYUGAN | NDB | 5835N | 07629E | 150 km |
| OKTYABRSKIY | NDB | 5414N | 03853E | 150 km |
| OLZONY | NDB | 5257N | 10512E | 150 km |
| OMSUKCHAN | NDB | 6227N | 15544E | 150 km |
| ONEGA | NDB | 6354N | 03807E | 150 km |
| OSA | NDB | 5322N | 10352E | 150 km |
| OSMINO | NDB | 5859N | 02906E | 150 km |
| PAVLOVSHCHINA | NDB | 5644N | 09333E | 150 km |
| PENZA | VOR/DME | 5306N | 04501E | 300 km / 14000 m |
| PEREDOVAYA | NDB | 4407N | 04128E | 150 km |
| PETROVSKOYE | NDB | 5143N | 04010E | 150 km |
| POCHINOK | NDB | 5743N | 03457E | 150 km |
| PODKAMENNAYA TUNGUSKA | VOR/DME | 6135N | 09000E | 300 km / 14000 m |
| PROVIDENIYABAY | NDB | 6422N | 17314W | 150 km |
| RAZDOLYE | NDB | 5226N | 10311E | 150 km |
| ROSHCHINO | NDB | 4553N | 13453E | 150 km |
| RYAZANSKAYA | NDB | 4457N | 03934E | 150 km |
| SALEKHARD | VOR/DME | 6635N | 06636E | 300 km / 14000 m |
| SAMBEK | NDB | 4745N | 03947E | 150 km |
| SANGAR | NDB | 6357N | 12725E | 150 km |
| SANKT-PETERBURG/Pulkovo | VOR/DME | 5948N | 03016E | 300 km / 14000 m |
| SARATOV/Tsentralny | VOR/DME | 5133N | 04602E | 300 km / 14000 m |
| SEROV | NDB | 5933N | 06033E | 150 km |
| SEVERNOYE | NDB | 5620N | 07821E | 150 km |
| SEYMCHAN | NDB | 6255N | 15225E | 150 km |
| SHARANGA | NDB | 5711N | 04630E | 150 km |
| SHUMERLYA | NDB | 5531N | 04626E | 150 km |
| SIROTINSKAYA | NDB | 4915N | 04340E | 150 km |
| SMOLENSKAYA | NDB | 4446N | 03848E | 150 km |
| SOBOLEVO | NDB | 5417N | 15557E | 150 km |
| SOLODNIKI | NDB | 4824N | 04517E | 150 km |
| SOSNOVKA | NDB | 5408N | 04638E | 150 km |
| SOSNOVSKOYE | NDB | 5548N | 04309E | 150 km |
| SREDNEBELOYE | NDB | 5038N | 12802E | 150 km |
| STARITSA | NDB | 5631N | 03455E | 150 km |
| SUKHOTINO | NDB | 5436N | 03719E | 150 km |
| TAZOVSKIY | NDB | 6729N | 07839E | 150 km |
| TERBUNY | NDB | 5208N | 03815E | 150 km |
| TIKHORETSK | NDB | 4551N | 04005E | 150 km |
| TILICHIKI | NDB | 6021N | 16600E | 150 km |
| TOBOLSK | NDB | 5808N | 06816E | 150 km |
| TROITSKOYE | NDB | 4927N | 13634E | 150 km |
| TURA/Gorny | NDB | 6417N | 10026E | 150 km |
| TURAN | NDB | 5209N | 09354E | 150 km |
| TURUKHANSK | VOR/DME | 6547N | 08756E | 300 km / 14000 m |
| UELKAL | NDB | 6530N | 17916W | 150 km |
| UFA | VOR/DME | 5432N | 05553E | 300 km / 14000 m |
| UKHTA | VOR/DME | 6333N | 05347E | 300 km / 14000 m |
| ULYANOVSK/Baratayevka | VOR/DME | 5415N | 04813E | 300 km / 14000 m |
| UST-BOLSHERETSK | NDB | 5249N | 15616E | 150 km |
| UST-KAMCHATSK | NDB | 5613N | 16241E | 150 km |
| UST-KUT | NDB | 5650N | 10548E | 150 km |
| UST-LABINSK | NDB | 4513N | 03940E | 150 km |
| UYAR | NDB | 5549N | 09418E | 150 km |
| VELIKIYELUKI | VOR | 5622N | 03036E | 300 km / 14000 m |
| VENEV | NDB | 5421N | 03813E | 150 km |
| VILYUYSK | NDB | 6344N | 12147E | 150 km |
| VOLGOGRAD/Gumrak | VOR/DME | 4846N | 04420E | 300 km / 14000 m |
| VOLOCHAYEVKA | NDB | 4834N | 13435E | 150 km |
| YAKUTSK | VOR/DME | 6205N | 12947E | 300 km / 14000 m |
| YARTSEVO | NDB | 6015N | 09011E | 150 km |
| YEGORLYKSKAYA | NDB | 4635N | 04039E | 150 km |
| YEKATERINBURG | VOR/DME | 5644N | 06047E | 300 km / 14000 m |
| YELSHANKA | NDB | 5148N | 04623E | 150 km |
| YENISEYSK | VOR/DME | 5828N | 09207E | 300 km / 14000 m |
| YUKHNOV | NDB | 5443N | 03512E | 150 km |
| ZABAYKALSK | VOR | 4938N | 11720E | 300 km / 14000 m |
| ZADONSK | NDB | 5223N | 03855E | 150 km |
| ZAKHAROVKA | NDB | 5444N | 03638E | 150 km |
| ZAYELTSOVSKIY | NDB | 5504N | 08253E | 150 km |
| ZELENGA | NDB | 4611N | 04836E | 150 km |
| ZENZELI | NDB | 4555N | 04703E | 150 km |
| ZHIGALOVO | NDB | 5448N | 10509E | 150 km |
| SERBIA AND MONTENEGRO |  |  |  |  |
| Aleksandrovac | NDB | 4327N | 2105E | 50 |
| Beograd | VOR/DME | 4448N | 2020E | 60/500 |
|  | L | 4442N | 2033E | 50 |
|  | L | 4454N | 2008E | 25 |
| Budisavci | VOR/DME | 4238N | 2030E | 80/500 |
| Metlika | NDB | 4539N | 1519E | 50 |
| Mitrovica | VOR/DME | 4500N | 1926E | 100/500 |
| Mojkovac | NDB | 4257N | 1935E | 80 |
| Niksic | NDB | 4246N | 1856E | 50 |
| Obrenovac | NDB | 4438N | 2009E | 50 |
| Pozarevac | NDB | 4437N | 2109E | 50 |
| Ribno | NDB | 4232N | 2103E | 100 |
| Titograd | VOR/DME | 4223N | 1916E | 60/500 |
|  | L | 4218N | 1915E | 50 |
| Tivat | NDB | 4225N | 1849E | 50 |
| Topola | VOR/DME | 4409N | 2045E | 40/500, 80/500 SE |
| Valjevo | VOR/DME | 4416N | 1955E | 60/500, 100/500 E-SW |
| Vranje | NDB | 4233N | 2155E | 60 |
| SLOVAKIA |  |  |  |  |
| Štefánik | DME | 4811N | 1713E | 75/500 |
| David (Stefanik) | L | 4811N | 1714E | 25 |
| Barka (Stefanik) | L | 4809N | 1715E | 25 |
| Štefánik North | NDB | 4813N | 1717E | 75 |
| Štefánik South | NDB | 4807N | 1718E | 25 |
| Dubove | NDB | 4851N | 1848E | 50 |
| Janovce | VOR/DME | 4811N | 1733E | 60/500 |
| Kostolany | NDB | 4831N | 1743E | 40 |
| Kosice | VOR/DME | 4838N | 2114E | 80/500 |
|  | NDB | 4835N | 2113E | 40 |
| Haniska (Kosice) | L | 4838N | 2114E | 25 |
| Nitra | VOR/DME | 4817N | 1803E | 80/500 |
|  | NDB | 4818N | 1803E | 50 |
| Piestany | NDB | 4834N | 1749E | 50 |
| Centr (Piestany) | L | 4836N | 1749E | 25 |
| Tatry | VOR/DME | 4904N | 2021E | 60/500 |
|  | NDB | 4904N | 2021E | 40 |
| Local Tatry | L | 4904N | 2016E | 25 |
| Tatry East | NDB | 4904N | 2027E | 40 |
| Tatry West | NDB | 4905N | 2004E | 40 |
| Sliac | VOR/DME | 4827N | 1907E | 60/500 |
| Hajniky (Sliac) | L | 4837N | 1908E | 25 |
| Sturovo | NDB | 4750N | 1841E | 80 |
| Zilina | NDB | 4912N | 1831E | 50 |
| Hlinik (Zilina) | L | 4914N | 1836E | 25 |
| SLOVENIA |  |  |  |  |
| Bistrica | VOR/DME | 4534N | 1410E | 100/500 |
|  | NDB | 4534N | 1410E | 50 |
| Dolsko | VOR/DME | 4605N | 1446E | 100/500, 150/500 E |
| Ljubljana | NDB | 4606N | 1440E | 50 |
|  | L | 4610N | 1433E | 25 |
| Maribor | L | 4622N | 1547E | 50 |
|  | L | 4628N | 1542E | 25 |
| Metlika | NDB | 4539N | 1519E | 50 |
| Portoroz | L | 4529N | 1337E | 50 |
| SPAIN |  |  |  |  |
| Alcobendas | NDB | 4035N | 0340W | 30 |
| Alicante | VOR/DME | 3816N | 0034W | 100/500, 120/500 S |
|  | NDB | 3817N | 0033W | 50 |
| Almeria | VOR | 3650N | 0216W | 60/250 |
|  | NDB | 3651N | 0223W | 60 |
| Andraitx | NDB | 3933N | 0224E | 30 |
| Arbancon | NDB | 4058N | 0308W | 60 |
| Arta | VOR/DME | 3939N | 0325E | 80/500W, 200/500 E |
| Asturias | VOR | 4334N | 0602W | 80/500, 200/500 N |
|  | NDB | 4333N | 0602W | 60 |
| Bagur | VOR/DME | 4157N | 0313E | 100/500 |
| Bailen | VOR/DME | 3809N | 0337W | 100/500 |
| Barahona | VOR/DME | 4119N | 0238W | 60/500 |
| Barcelona | VOR/DME | 4118N | 0205E | 100/500, 200/500 E |
|  | L | 4116N | 0159E | 15 |
| Bilbao | VOR/DME | 4318N | 0256W | 60/500, 150/500 N |
|  | NDB | 4319N | 0258W | 70 |
|  | NDB | 4311N | 0236W | 25 |
| Burgos | NDB | 4221N | 0338W | 50 |
| Caceres | VOR/DME | 3931N | 0626W | 80/500 |
| Calamocha | VOR | 4052N | 0117W | 60/500 |
| Calles | NDB | 3943N | 0059W | 50 |
| Castejon | VOR/DME | 4022N | 0233W | 100/500 |
| Costix | NDB | 3938N | 0255E | 50 |
| Domingo | VOR/DME | 4227N | 0253W | 80/500 |
| Girona/Costa Brava | VOR/DME | 4156N | 0246E | 80/250, 80/500 NW |
|  | NDB | 4201N | 0248E | 50 |
| Granada | VOR | 3711N | 0359W | 40/250 |
| Hinojosa del Duque | VOR/DME | 3830N | 0505W | 80/500 |
| Ibiza | VOR/DME | 3852N | 0122E | 80/500, 150/500 SE |
|  | NDB | 3855N | 0128E | 60 |
| Jerez | NDB | 3650N | 0601W | 50 |
|  | VOR | 3649N | 0602W | 40/250 |
| La Coruna | NDB | 4322N | 0820W | 40 |
| Lerida | NDB | 4133N | 0039E | 50 |
| Madrid | VOR/DME | 4019N | 0322W | 60/500 |
|  | L | 4023N | 0334W | 15 |
|  | L | 4024N | 0329W | 30 |
| Maella | VOR/DME | 4108N | 0010E | 80/500, 100/500 N |
| Mahon/Menorca | VOR/DME | 3952N | 0415E | 120/500, 60/500 NE |
|  | NDB | 3950N | 0413E | 60 |
| Malaga | VOR/DME | 3649N | 0422W | 100/500, 80/500 NE |
|  | NDB | 3640N | 0429W | 60 |
| Martin | VOR | 3703N | 0456W | 25/100 |
| Murcia/San Javier | NDB | 3741N | 0056W | 40 |
| Navas del Rey | NDB | 4022N | 0415W | 40 |
| Palma de Mallorca | VOR/DME | 3926N | 0245E | 80/500, 200/500 N & S |
|  | L | 3936N | 0249E | 25 |
| Pamplona | VOR/DME | 4244N | 0142W | 80/500, 100/500 E-S |
| Pollensa | VOR/DME | 3956N | 0307E | 60/250 |
| Reus | VOR | 4109N | 0110E | 40/500, 80/500 S, 120/500 N |
| Sabadell | NDB | 4131N | 0206E | 40 |
| Sagunto | NDB | 3940N | 0012W | 40 |
| San Sebastian | NDB | 4323N | 0148W | 50 |
| Santander | VOR | 4327N | 0354W | 60/500, 150/500 N |
|  | L | 4326N | 0351W | 25 |
| Santiago | VOR/DME | 4256N | 0826W | 100/500, 200/500 W-N |
|  | L | 4258N | 0826W | 30 |
| Sevilla | VOR/DME | 3726N | 0546W | 100/500, 150/500 NE, 200/500 SW |
|  | NDB | 3725N | 0548W | 40 |
| Somosierra | NDB | 4108N | 0335W | 50 |
| Toledo | VOR/DME | 3958N | 0420W | 60/500 |
| Valencia | VOR/DME | 3929N | 0029W | 100/500 |
| Vejer de la Frontera | VOR/DME | 3615N | 0558W | 60/500, 100/500 NE & S |
| Vigo | VOR | 4219N | 0836W | 40/250 |
|  | NDB | 4211N | 0838W | 50 |
| Villatobas | VOR | 3947N | 0327W | 60/500 |
| Vitoria | VOR | 4244N | 0252W | 40/250 |
| Yeste | VOR/DME | 3822N | 0221W | 60/500 |
| Zamora | VOR/DME | 4132N | 0538W | 100/500, 150/500 SW, 200/500 N |
| Zaragoza | NDB | 4144N | 0112W | 50 |
|  | NDB | 4136N | 0053W | 25 |
| SWEDEN |  |  |  |  |
| Alma | VOR | 5525N | 1334E | 60/500 |
| Arlanda | VOR/DME | 5939N | 1755E | 100/500 |
| Aros | VOR/DME | 5935N | 1639E | 80/500 |
| Backa | VOR/DME | 5733N | 1159E | 80/500 |
| Borlange | VOR/DME | 6025N | 1531E | 100/500 |
| Corner | NDB | 5916N | 1729E | 25 |
| Dunker | VOR | 5912N | 1701E | 80/500 |
| Goteborg/Landvetter | L | 5744N | 1221E | 15 |
| Goteborg/Save | L | 5749N | 1153E | 15 |
|  | L | 5744N | 1152E | 15 |
|  | DME | 5747N | 1152E | 25/60 |
| Hammar | VOR/DME | 6017N | 1824E | 100/500, 150/500 N |
| Harry | VOR | 5750N | 1242E | 60/250 |
| Jönköping | VOR/DME | 5745N | 1404E | 100/500 |
|  | L | 5749N | 1407E | 25 |
| Kalmar | VOR/DME | 5641N | 1617E | 80/500, 160/500 NE |
| Karlstad | VOR/DME | 5926N | 1320E | 120/500 |
|  | L | 5923N | 1316E | 25 |
| Kiruna | VOR/DME | 6749N | 2020E | 80/500 |
|  | L | 6753N | 2027E | 40 |
| Landvetter | VOR/DME | 5739N | 1218E | 100/500 |
| Lena | NDB | 5932N | 1722E | 25 |
| Lulea | VOR/DME | 6532N | 2208E | 60/500 |
| Lunde | NDB | 6253N | 1750E | 25 |
| Nacka | NDB | 5918N | 1811E | 25 |
| Natta | VOR/DME | 6645N | 2120E | 80/500 |
| Nolvik | VOR | 5747N | 1150E | 80/500 |
| Norrkoping | L | 5835N | 1623E | 25 |
|  | L | 5835N | 1608E | 15 |
| Nortel | VOR | 5945N | 1846E | 100/500 |
| Örnsköldsvik | VOR/DME | 6324N | 1900E | 60/500 |
| Ostersund | VOR/DME | 6312N | 1430E | 150/500 |
| Skelleftea | VOR/DME | 6438N | 2105E | 60/500 |
| Stockholm/Arlanda | L | 5939N | 1755E | 15 |
|  | L | 5934N | 1754E | 25 |
|  | L | 5940N | 1806E | 15 |
| Stockholm/Bromma | L | 5919N | 1803E | 25 |
|  | L | 5924N | 1749E | 25 |
| Storuman | VOR/DME | 6457N | 1742E | 100/500 |
| Sturup | VOR/DME | 5532N | 1323E | 80/500, |
| Sundsvall | VOR/DME | 6231N | 1727E | 100/500 |
|  | L | 6228N | 1729E | 25 |
| Sveda | VOR/DME | 5610N | 1235E | 100/500 |
| Tebby | VOR/DME | 5932N | 1812E | 100/500 |
| Trosa | VOR/DME | 5856N | 1730E | 80/500 |
| Umea | VOR/DME | 6347N | 2017E | 150/500 |
|  | L | 6351N | 2011E | 15 |
| Vassen | VOR/DME | 5818N | 1543E | 100/500 |
| Vasteras | L | 5931N | 1636E | 15 |
|  | L | 5940N | 1640E | 15 |
| Vaxjö | L | 5654N | 1443E | 15 |
|  | L | 5700N | 1446E | 30 25 |
| Vedby | VOR | 5609N | 1313E | 60/500 |
| Visby | VOR/DME | 5740N | 1821E | 80/500 |
|  | L | 5744N | 1824E | 25 |
| SWITZERLAND |  |  |  |  |
| Bern | NDB | 4654N | 00731E | 25 |
| Corvatsch | DME | 4625N | 00949E | 80/500 |
| Fribourg | VOR/DME | 4647N | 00713E | 40/500, 100/500 195º to 255º true |
| Geneva | VOR/DME | 4615N | 00608E | 50/250 |
| Gland | NDB | 4625N | 00615E | 25 |
| Grenchen | VOR/DME | 4711N | 00725E | 25/100 |
| Hochwald | VOR/DME | 4728N | 00740E | 60/500, 85/500 30° to 120° true |
| Kloten | VOR/DME | 4727N | 00833E | 50/250 |
| La Praz | DME | 4641N | 00625E | 80/500, 70/500 195° to 255° true |
| Les Eplatures | NDB | 4705N | 00648E | 15 |
| Montana | VOR/DME | 4619N | 00730E | 40/250 |
| Passeiry | VOR/DME | 4610N | 00600E | 80/500 |
| Sion | VOR/DME | 4613N | 00717E | 40/250 |
| St-Prex | VOR/DME | 4628N | 00627E | 100/500 |
| Trasadingen | VOR/DME | 4741N | 00826E | 100/500 |
| Weissfluhjoch | DME | 4650N | 00948E | 80/500 |
| Willisau | VOR/DME | 4711N | 00754E | 50/250, 80/250 0° to 150° true |
| Zurich East | VOR/DME | 4736N | 00849E | 80/500 |
| TAJIKISTAN |  |  |  |  |
| Dushanbe | NDB | 3829N | 6848E | 80 |
|  | VOR/DME | 3832N | 6449E | 100/500 |
| Fayzabad | NDB | 3833N | 6919E |  |
| Khudzhand | NDB | 4013N | 06942E | 80 |
| Oktyabrsky | NDB | 3832N | 6824E |  |
| Pugus | NDB | 3851N | 6850E |  |
| Ura Tyube | NDB | 4003N | 06857E | 120 |
| THE FORMER YUGOSLAV REPUBLIC OF MACEDONIA |  |  |  |  |
| Izdeglavje | NDB | 4120N | 2049E | 50 |
| Ohrid | VOR/DME | 4110N | 2044E | 25/250 |
| Petrovec | L | 4155N | 2138E | 50 |
| Prilep | NDB/DME | 4120N | 2127E | 80 |
| Sinko | VOR/DME | 4158N | 2222E | 50/450 |
| Skopje | VOR/DME | 4156N | 2138E | 50/450 |
| TURKEY |  |  |  |  |
| Adana | VOR/DME | 3656N | 03513E | 60/500, 100/500 W |
|  | NDB | 3658N | 03516E | 50 |
| Adiyaman | VOR/DME | 3743N | 03828E | 50/500 |
|  | NDB | 3743N | 03828E | 50 |
| Afyon | VOR/DME | 3848N | 03033E | 150/500 |
|  | NDB | 3848N | 03033E | 75 |
| Agri | VOR/DME | 3940N | 04301E |  |
| Ankara/Ank | NDB/DME | 3957N | 03250E | 100/500 |
| Antalya | VOR/DME | 3655N | 03048E | 80/500, 200/500 S |
|  | NDB | 3653N | 03027E | 50 |
| Baglum/Bag | VOR/DME | 4004N | 03249E | 100/500,150/500 NW & SE |
| Balikesir | NDB/DME | 3938N | 02756E |  |
| Batman | VOR/DME | 3756N | 04107E | 70/500 |
| Beykoz | VOR/DME | 4108N | 02909E | 80/500, 120/500 SE |
|  | NDB | 4108N | 02909E | 75 |
| Beypazari | NDB | 4010N | 03156E | 75 |
| Biga | VOR/DME | 4017N | 02722E | 50/500, 80/500 S |
| Canakkale | VOR/DME | 4009N | 02626E | 50/500 |
|  | NDB | 4009N | 02626E | 75 |
| Cardak | VOR/DME | 3747N | 02942E |  |
|  | NDB | 3747N | 02942E |  |
| Carsamba | VOR/DME | 4116N | 03632E | 60/500, 150/500 W & E |
| Caycuma | NDB/DME | 4130N | 03205E | 75 |
| Cekmece | NDB | 4100N | 02832E | 75 |
| Corlu | VOR/DME | 4109N | 02756E | 75/500 |
| Cubuk/Buk | VOR/DME | 4015N | 03306E | 60/500, 120/500 W & E |
|  | NDB | 4015N | 03306E | 75 |
| Dalaman | VOR/DME | 3641N | 02847E | 80/250 |
|  | NDB | 3641N | 02847E | 50 |
| Diyarbakir | VOR/DME | 3752N | 04013E | 100/500 |
|  | NDB | 3752N | 04013E | 75 |
| Edremit | VOR/DME | 3933N | 02701E | 50/500 |
|  | NDB | 3933N | 02701E | 75 |
| Elazig | VOR/DME | 3843N | 03913E | 100/500 |
| Erzincan | VOR/DME | 3943N | 03932E |  |
| Erzurum | VOR/DME | 3958N | 04112E | 150/500 |
| Eskisehir | VOR/DME | 3949N | 03031E | 100/500 |
|  | NDB | 3949N | 03031E | 75 |
| Gaziantep | VOR/DME | 3657N | 03728E | 60/500, 150/500 NW |
| Gemerek | VOR/DME | 3909N | 03602E | 100/500, 150/500 NW |
| Golbasi | NDB | 3943N | 03249E | 75 |
| Haymana | NDB/DME | 3926N | 03230E | 75 |
| Inebolu | VOR/DME | 4157N | 03342E | 150/500, 80/500 S |
| Isparta | VOR/DME | 3751N | 03022E | 75/500 |
| Istanbul | VOR/DME | 4058N | 02848E | 60/500, 120/500 N |
|  | L | 4058N | 02849E | 50 |
| Istanbul/SBH | VOR/DME | 4054N | 02919E | 50 |
| Izmir/Imr | VOR/DME | 3819N | 02700E |  |
|  | NDB | 3223N | 02709E |  |
| Izmir/Men | VOR/DME | 3818N | 02710E | 100/500 |
| Kahramanmaras | VOR/DME | 3732N | 03657E | 75/500 |
|  | NDB | 3732N | 03657E | 75 |
| Kars | VOR/DME | 4034N | 04307E | 75/500 |
|  | NDB | 4034N | 04307E | 75 |
| Kayseri | VOR/DME | 3847N | 03531E | 100/250 |
| Kiyikislacik | DVOR/DME | 3717N | 02733E | 100/500 |
| Konya | VOR | 3800N | 03234E |  |
| Malatya | VOR | 3828N | 03807E |  |
|  | NDB | 3828N | 03807E |  |
| Mardin | NDB/DME | 3714N | 04038E | 100 |
| Milas/Bodrum | VOR/DME | 3715N | 02740E | 75/500 |
| Mus | VOR/DME | 3845N | 04140E | 75/500 |
|  | NDB | 3845N | 04140E | 75 |
| Mut | VOR/DME | 3652N | 03318E | 150/500 |
| Siirt | VOR/DME | 3755N | 04153E | 150/500 |
|  | NDB | 3759N | 04151E | 75 |
| Sinop | VOR/DME | 4201N | 03505E | 75/500 |
|  | NDB | 4201N | 03505E | 75 |
| Sivas | VOR/DME | 3947N | 03654E | 60/500, 150/500 W & E |
| Tekirdag | VOR/DME | 4057N | 02726E | 80/500 |
| Tokat | NDB | 4018N | 03622E |  |
| Trabzon | VOR/DME | 4100N | 03948E | 150/500 |
|  | NDB | 4100N | 03948E | 75 |
| Tuzkoy | VOR/DME | 3846N | 03432E | 75/500 |
|  | NDB | 3846N | 03432E | 75 |
| Urfa | VOR/DME | 3706N | 03851E | 75/500 |
|  | NDB | 3706N | 03851E | 75 |
| Usak | NDB/DME | 3841N | 02929E |  |
| Van | VOR/DME | 3828N | 04319E | 60/500, 80/500 NW |
|  | NDB | 3828N | 04320E | 75 |
| Yalova | VOR/DME | 4034N | 02922E | 80/500, 100/500 E-S |
|  | NDB | 4034N | 02922E | 25 |
| Yenisehir | VOR/DME | 4016N | 02936E | 50/500 |
| TURKMENISTAN |  |  |  |  |
| Askhabad | VOR/DME |  |  |  |
|  | NDB | 3759N | 5821E | 100 |
| Dashkhowuz | NDB |  |  |  |
| Serdar | NDB | 3900N | 5618E | 100 |
| Turkmenabat | NDB | 3905N | 6337E | 100 |
|  | VOR/DME |  |  |  |
| Turkmenbashi | NDB | 4005N | 5305E | 100 |
|  | VOR/DME |  |  |  |
| UKRAINE |  |  |  |  |
| Alushta | NDB | 4441N | 3424E | 100 |
| Auly | NDB | 4835N | 3429E | 100 |
| Bibrka | NDB | 4939N | 2417E | 100 |
| Bakhmach | DME | 5104N | 3253E | 200 |
| Bohdanivka | NDB | 5038N | 3054E | 75 |
| Cherniakhiv | NDB | 5027N | 2842E | 100 |
| Chervonye | NDB | 5004N | 3124E | 75 |
| Davydo-Mykil's'ke | NDB | 4836N | 3951E | 100 |
| Dmytrovka | NDB | 4529N | 3503E | 75 |
| Dnipropetrovs’k | VOR/DME | 4822N | 3506E | 150/500 |
| Donets’k | VOR/DME | 4804N | 3744E | 150/500 |
| Dobrushyn | NDB | 4523N | 3322E | 75 |
| Helmiasiv | NDB | 4950N | 3151E | 100 |
| Ivano-Frankivs’k | VOR/DME | 4853N | 2442E | 80/250 |
| Kakhovka | NDB | 4649N | 3330E | 100 |
| Kamianets | NDB | 4842N | 2636E | 100 |
| Kaniv | NDB | 4945N | 3127E | 100 |
| Kerch | DME | 4522N | 3624E | 200 |
| Kharkiv | VOR/DME | 4956N | 3617E | 150/500 |
| Koshany | NDB | 5057N | 3059E | 100 |
| Koviahy | NDB | 4955N | 3533E | 100 |
| Krabor | NDB | 5036N | 2917E | 100 |
| Kryviy Rih | DME | 4803N | 3313E | 200 |
| Kyiv "Boryspil" | VOR/DME | 5017N | 3054E | 150/250 |
| Krasnoarmiis’k | NDB | 4818N | 3712E | 100 |
| Krasnyi Luch | NDB | 4811N | 3856E | 100 |
| Krasnohrad | NDB | 4923N | 3527E | 1001 |
| Lesnik | NDB | 4450N | 3415E | 75 |
| Liubymivka | NDB | 4538N | 3454E | 100 |
| L’viv | VOR/DME | 4949N | 2357E | 150/500 |
| Lykhacheve | NDB | 4922N | 3613E | 100 |
| Mariupol | DME | 4705N | 3727E | 200 |
| Nykolay | NDB | 4458N | 3337E | 100 |
| Nemyriv | NDB | 4858N | 2851E | 150 |
| Odesa | VOR/DME | 4626N | 3041E | 150/500 |
| Parutyne | NDB | 4642N | 3154E | 100 |
| Pavlivka | NDB | 4745N | 3714E | 100 |
| Pii | NDB | 4952N | 3108E | 100 |
| Rashivka | NDB | 5013N | 3353E | 100 |
| Semenivka | NDB | 5212N | 3233E | 150 |
| Serednie | NDB | 4832N | 2231E | 75 |
| Shepetivka | NDB | 5011N | 2703E | 150 |
| Shyriaieve | NDB | 4724N | 3017E | 100 |
| Simferopol | VOR/DME | 4503N | 3359E | 150/500 |
| Soloviivka | NDB | 5011N | 2934E | 100 |
| Stebliv | NDB | 4924N | 3105E | 100 |
| Tomakivka | NDB | 4749N | 3444E | 100 |
| Topchyne | NDB | 4856N | 3448E | 100 |
| Trypillia | NDB | 5007N | 3046E | 100 |
| Verkhnie | NDB | 4857 N | 2303E | 75 |
| Vinnytsia | DME | 4914N | 2837E | 200 |
| Yahotyn | NDB | 5016N | 3148E | 100 |
| Yalta | NDB | 4429N | 3408E | 100 |
| Zolochiv | NDB | 4948N | 2454E | 100 |
| UNITED KINGDOM |  |  |  |  |
| Aberdeen | VOR/DME | 5719N | 0216W | 90/500, 200/500 E |
|  | NDB | 5708N | 0023W | 15 |
| Alderney | NDB | 4943N | 0212W | 30 |
| Barkway | VOR/DME | 5159N | 0004E | 40/250 |
| Belfast | VOR/DME | 5440N | 0614W | 40/500, 200/500 SW |
|  | L | 5442N | 0605W | 15 |
| Benbecula | VOR/DME | 5729N | 0722W | 200/500W, 150/500 S-N |
| Berry Head | VOR/DME | 5024N | 0330W | 85/500 |
| Biggin | VOR/DME | 5120N | 0002E | 60/500, 100/500 NE-SE 125/500 W-NE |
| Birmingham | L | 5227N | 0145W | 25 |
|  | DME | 5227N | 0145W | 15 |
| Blackpool | L | 5346N | 0302W | 15 |
| Bovingdon | VOR/DME | 5144N | 0033W | 60/500 |
| Brecon | VOR/DME | 5144N | 0316W | 65/500 |
| Bristol/Lulsgate | L | 5123N | 0243W | 40 |
| Brookmans Park | VOR/DME | 5145N | 0006W | 40/500 80/500 |
| Burnham | NDB | 5131N | 0041W | 30 |
| Cardiff | L | 5124N | 0320W | 40 |
| Chittem | NDB | 5137N | 0031W | 25 |
| Clacton | VOR/DME | 5151N | 0109E | 100/500, 150/500 NW-NE |
| Compton | VOR/DME | 5130N | 0113W | 80/500,150/500 SW-NE |
| Cranfield | VOR | 5204N | 0037W | 50/250 |
| Daventry | VOR/DME | 5211N | 0107W | 60/500 75/500 NW-N |
| Dean Cross | VOR/DME | 5443N | 0320W | 60/500 W-N |
| Detling | VOR/DME | 5118N | 0036E | 60/500 |
| Dover | VOR/DME | 5110N | 0122E | 80/500, 200/500 NE |
| East Midlands | L | 5250N | 0127W | 10 |
| Edinburgh | L | 5559N | 0317W | 35 |
|  | L | 5554N | 0330W | 25 |
| Epsom | NDB | 5119N | 0022W | 25 |
| Garnston | VOR/DME | 5317N | 0057W | 80/250 |
| Glasgow | VOR/DME | 5552N | 0427W | 70/500, 300/700 SW-N |
|  | L | 5549N | 0433W | 25 |
|  | L | 5556N | 0420W | 15 |
| Goodwood | VOR/DME | 5051N | 0045W | 80/500 |
| Guernsey | VOR/DME | 4926N | 0236W | 50/500 |
|  | L | 4926N | 0238W | 30 |
| Henton | NDB | 5146N | 0047W | 30 |
| Honiley | VOR/DME | 5221N | 0140W | 60/500, 100/500 NW-N |
| Humberside | L | 5335N | 0021W | 15 |
| Inverness | VOR/DME | 5733N | 0402W | 60/250 |
| Isle of Man | VOR/DME | 5404N | 0446W | 60/500 |
|  | NDB | 5408N | 0430W | 25 |
|  | L | 5405N | 0437W | 20 |
| Jersey | VOR/DME | 4913N | 0203W | 60/500, 40/500 |
|  | L | 4912N | 0213W | 25 |
| Lambourne | VOR/DME | 5139N | 0009E | 40/500, 110/500 NW-SE |
| Land’s End | VOR/DME | 5008N | 0538W | 200/500 |
| Leeds/Bradford | NDB (L) | 5352N | 0139W | 25 |
| Lichfield | NDB | 5245N | 0143W | 50 |
| Liverpool | L | 5320N | 0243W | 25 |
| London | VOR/DME | 5129N | 0028W | 40/500, 110/500 NW-SE |
| Luton | L | 5154N | 0015W | 20 |
| Lydd | VOR/DME | 5100N | 0053E | 80/500, 100/500 S-W |
|  | NDB | 5058N | 0057E | 15 |
|  | L | 5058N | 0056E | 20 |
| Lyneham | NDB | 5130N | 0200W | 40 |
| Machrihanish | VOR/DME | 5526N | 0539W | 150/500, 200/500 SW-N |
| Manchester | VOR/DME | 5321N | 0216W | 90/500 |
|  | L | 5321N | 0216W | 15 |
| Manchester/Woodford | L | 5321N | 0210W | 15 |
| Mayfield | VOR/DME | 5101N | 0007E | 40/250, 60/20 SE-S |
| Midhurst | VOR/DME | 5103N | 0038W | 60/500, 100/500 SW-N |
| Newcastle | VOR/DME | 5502N | 0142W | 200/500 |
| Norwich | NDB | 5241N | 0117W | 20 |
| New Galloway | NDB | 5511N | 0410W | 35 |
| Ockham | VOR/DME | 5118N | 0027W | 70/250, 90/250 NE-E |
| Ottringham | VOR/DME | 5342N | 0006W | 100/600, 150/600 E, 200/600 NE |
| Perth | VOR | 5627N | 0322W | 80/500 |
| Pool Hill | VOR/DME | 5345N | 0206W | 115/500, 150/500 NE-SE |
| Prestwick | L | 5533N | 0441W | 30 |
|  | DME | 5530N | 0436W | 40 |
| Saint Abbs | VOR/DME | 5554N | 0212W | 50/500, 200/500 NE-SE 100/500 SW-NW |
| Scotstown Head | NDB | 5734N | 0149W | 80, 25 S-NW |
| Seaford | VOR/DME | 5046N | 0007E | 50/500, 120/500 SW-NW |
| Shoreham | DME | 5050N | 0017W | 10 |
|  | L | 5050N | 0017W | 10 |
| Southampton | VOR/DME | 5057N | 0120W | 100/500, 150/500 SW-NW |
| Southend | L | 5135N | 0042W | 20 |
| Stornoway | VOR/DME | 5812N | 0611W | 200/500, 105/500 S-SE |
| Strumble | VOR/DME | 5200N | 0502W | 85/500, 300/700 SW-NW, 150/500 NW-NE |
| Sumburgh | VOR/DME | 5952N | 0117W | 200/500 |
| Talla | VOR/DME | 5530N | 0321W | 100/500, 120/500 SW-NW |
| Tiree | VOR/DME | 5630N | 0653W | 90/500, 200/500 W |
| Trent | VOR/DME | 5303N | 0140W | 80/500, 100/500 |
| Turnberry* | VOR/DME | 5519N | 0447W | 60/500, 200/500 W-N |
| Wallasey | VOR/DME | 5324N | 0308W | 120/500 |
| Westcott | NDB | 5151N | 0058W | 30 |
| Whitegate | NDB | 5311N | 0237W | 25 |
| Wick | VOR/DME | 5828N | 0306W | 100/500, 160/500 NW-N |
| Woodley | NDB | 5127N | 0053W | 25NM |
| *Previously Prestwick VOR/DME |  |  |  |  |
| UZBEKISTAN |  |  |  |  |
| Andizhan | NDB | 4042N | 07215E | 70 |
| Bukhara | NDB | 3944N | 06428E | 80 |
| Bulungur | NDB | 3945N | 06716E | 75 |
| Dalverzin | NDB | 4025N | 06917E | 75 |
| Fergana | NDB | 4025N | 07145E | 80 |
| Guzar | NDB | 3837N | 06614E | 80 |
| Kakaydi | NDB | 3734N | 06730E | 80 |
| Karakhtay | NDB | 4055N | 06948E | 75 |
| Karshi | NDB | 3851N | 06546E | 80 |
| Kasan | NDB | 3903N | 06535E | 75 |
| Kokand | NDB | 4028N | 07103E | 75 |
| Muynak | NDB | 4346N | 05902E | 100 |
| Nagornaya | NDB | 3945N | 06624E | 75 |
| Namangan | NDB | 4058N | 07137E | 75 |
| Nukus | NDB | 4227N | 05939E | 80 |
| Nurata | NDB | 4034N | 06542E | 100 |
| Samarkand | NDB | 3942N | 06655E | 80 |
| Sergely | NDB | 4111N | 06913E | 75 |
| Sherabad | NDB | 3742N | 06703E | 80 |
| Syrdarya | NDB | 4049N | 06841E | 100 |
| Tamdy-Bulak | VOR/DME | 4146N | 06437E | 150/250, 80/250E |
|  | NDB | 4146N | 06437E | 100 |
| Tashkent | VOR/DME | 4116N | 06918E | 120/250, 80/260E |
|  | NDB | 4115N | 06913E | 80 |
| Termez | VOR/DME | 3717N | 06918E | 80/250N-S, 120/250 W-E |
|  | NDB | 3718N | 06913E | 80 |
| Toytepa | NDB | 4059N | 06923E | 75 |
| Ura-Tube | NDB | 4003N | 06858E | 75 |
| Urgench | VOR/DME | 4135N | 06039E | 150/250 |
|  | NDB | 4135N | 06041E | 80 |
| Urgut | NDB | 3926N | 06715E | 80 |