Draft Proposal for Amendment to ICAO Doc 4444, PANS-ATM, Chapter 6

Chapter 6

SEPARATION IN THE VICINITY OF AERODROMES

6.1	REDUCTION IN VISUAL SEPARATION IN THE

VICINITY OF AERODROMES

Appropriate ATS authorities may authorize for specific scenarios in the vicinity of an aerodrome, methods to separate aircraft when each aircraft is continuously visible to the aerodrome controller.

In addition to the circumstances mentioned in Chapter 5, 5.11.1, the separation minima detailed in Chapter 5, 5.4.1 and 5.4.2, may be reduced in the vicinity of aerodromes if:

- adequate separation can be provided by the aerodrome controller when each aircraft is continuously visible to this controller; or

Rationale:

The reference to Chapter 5, 5.4.1. and 5.4.2 reflects only partly the circumstances where the provisions of 6.1 a) to c) could apply. Although from an overall ATS planning and implementation perspective, reference to ‘reduction of separation minima’ could be used, presenting these options in 6.1 a) to c) as methods to separate traffic in the vicinity of aerodromes is providing for a direct applicability by operational personnel.

Moreover, the use of these methods to separate aircraft was always subject to conditions and limitations specific to the local environment. From a safety perspective as well as to ensure traceability in safety management systems, it is proposed that the method to separate aircraft when each aircraft is continuously visible to aerodrome controller is accompanied by specific scenarios an limitations authorised by the appropriate ATS authority.

It is also proposed to consolidate a common terminology to be used when referring to such methods to separate aircraft, to ensure consistency with its use in other provisions in PANS-ATM (e.g. paragraph 6.7.3.2.11, paragraph 15.7.2.4).

6.2	OWN SEPARATION

6.2.1	In addition to 6.1, the following methods to separate aircraft could be used in the vicinity of and aerodrome:

- each aircraft is continuously visible to flight crews of the other aircraft concerned and the pilots thereof report that they can maintain their own separation; or

- in the case of one aircraft following another, the flight crew of the succeeding aircraft reports that the other aircraft is in sight and own separation can be maintained.

Note.- See also Section 5.8

6.2.2	When applying the provisions in 6.2.1, if the projected flight path of an aircraft will cross or follow behind the flight path of another aircraft, at the same altitude or less than 300 m (1000 feet) below, the controller should issue a caution of possible wake turbulence when necessary, prior to instructing the flight crew to maintain own separation.

Note 1. — see Chapter 12, 12.3.4.19 e) for RT phrase to be used when passing caution of possible wake turbulence.

Note 2. — see 5.8.1.2 for the responsibilities of the pilot-in-command of the aircraft issued with a caution of possible wake turbulence.

6.23 ESSENTIAL LOCAL TRAFFIC

6.23.1	Information on essential local traffic known to the controller shall be transmitted without delay to departing and arriving aircraft concerned.

Note 1.— Essential local traffic in this context consists of any aircraft, vehicle or personnel on or near the runway to be used, or traffic in the take-off and climb-out area or the final approach area, which may constitute a collision hazard to a departing or arriving aircraft.

Note 2.— See also Chapter 5, Section

--END--

The text in blue was subject to a proposal for amendment to ICAO Doc 4444, discussed by ICAO EASPG01&02 in the context of responsibility for wake turbulence separation.