Draft Proposal for Amendment to ICAO Doc 7030 (EUR-SUPPs)

6.2.6	Separation in the vicinity of the aerodrome

(P-ATM – Chapter 6)

6.2.6.1	When implementing methods to separate aircraft in the vicinity of an aerodrome on the basis that each aircraft is continuously visible to the aerodrome controller, the appropriate ATS authority shall address, inter alia, the following requirements:

a) 	that weather conditions are such that the aircraft concerned are continuously visible to the aerodrome controller;

b)	to establish scenarios and limitations for the application of these methods and describe them accordingly in local instructions or manual of operations;

c)	that the use of such methods is consistent with the coordination and transfer of control conditions agreed with adjacent ATC units;

d)	that the approach controller is informed when such methods to separate aircraft are used by the aerodrome controller;

e)	that such methods cannot be used when wake turbulence separation have to be provided between the aircraft concerned;

f)	that as soon as it becomes apparent that these methods can no longer be used, another form of separation is established and the approach controller is informed, as required.

6.2.6.2	When pilots accept to maintain own separation in the vicinity of an aerodrome, the controller should, nevertheless, monitor the progress of flights and issue alternative instructions whenever the distance between aircraft will not ensure the applicable runway separation or at the request of the accepting ATC unit.

Rationale:

The proposal for introducing a new section in ICAO Doc 7030, EUR-SUPPs, is intended to provide a harmonized framework for developing and implementing methods to separate aircraft in the vicinity of aerodromes. It describes a number of pre-requisites and assessments that have to be made to ensure that the local instructions and operational manuals are providing the necessary clarity and understanding for use by air traffic controllers.

--END--