APPENDIX C – Amendments to the EUR Frequency Management Manual (EUR Doc 011)

# Introduction

  - The text below indicates the amendments to be included in EUR DOC011 and its supplement.

  - Amendments are arranged to show deleted text using strikeout (text to be deleted), and added text with grey shading (text to be inserted). Text with yellow shading correspond to additional proposed editorial changes not contained in the texts initially agreed by FMG.

# Amendment to the Supplement of EUR Doc 011 on “Information about Use”

Note 1: The content of this addition to the Supplement of EUR Doc011 was based on the one hand on p.7 of FMG25 SoD and on the other hand on the content of FL05 of FMG25 which was the outcome of a break-out session on this subject.

Note 2: the following material will constitute Chapter J in the Supplement to EUR Doc011.

## “Information about Use”

- Introduction

1.1. 	This section provides guidance on the exchange of information for systems intended to operate in the aeronautical bands but for which no planning criteria are defined in EUR Doc011.

- General

2.1. 	A State that intends to operate a system in the aeronautical bands that are subject to the ICAO EUR frequency planning but for which system no planning criteria currently exist in EUR Doc011 (e.g., air-to-air TACAN, UAT, RPAS) can provide information on the intended use as described in the following.

2.2. 	For these cases:

- Any such use is subject to interference analysis at its origin, i.e., it is the responsibility of the State that intends to operate such a system to ensure other States are not impacted.

- Any such use, which may affect frequency assignments recorded in the ICAO COM/NAV Tables outside the State of origin, is subject to the agreement of the affected States.

- In a case of harmful interference involving the application of such systems, there is an obligation to the State of origin to eliminate harmful interference.

- Provision of “Information about Use”

  - The provision of “Information about Use” is supported by a dedicated internet facility which permits the exchange of information between subscribed States.

  - States are not obliged to receive nor process such “Information about Use”.

  - The provision of “Information about Use” does not constitute a coordination request and it does not imply the agreement of affected States.

  - The “Information about Use” will contain the following elements:

- the frequency (mandatory)

- a location name (mandatory)

- a DOC (circular or polygonal) (mandatory)

- the planned period of use (mandatory)

- the name/type of the system to be used (mandatory)

- additional information about the system (optional)

- the bandwidth used (optional)

- type of modulation (optional)

- remarks (optional)

- radiated power (mandatory)

- elevation of ground station AMSL (mandatory)

- The analysis of the impact on neighbouring states (optional)

- the contact point to request termination of transmissions (optional)

# Amendment to EUR Doc 011 on Protected / Unprotected channels

ICAO EUR Doc 011 Part II: VHF AIR-GROUND COMMUNICATIONS FREQUENCY ASSIGNMENT PLANNING CRITERIA

…

2. 	Utilization of the frequency band 117.975 – 137 MHz

2.1 	The table is based on Annex 10, Volume V, 4.1.1 also incorporating regional agreements on specific uses of individual frequencies or sub-bands and the identification of the assignable 25 kHz and 8.33 kHz channels:

…

*OPC: Assignments used for OPC purposes shall not be co-ordinated on protected standard channels. 8.33 kHz channel spacing to be used wherever possible, between 131.405 – 131.9801 except for the ACARS channels 131.525, 131.725 and 131.825. The use of voice at the first adjacent 8,33 KkHz channel to ACARS is not possible. Care should be taken when considering the use of the second adjacent channel at 16,67 KkHz from the ACARS.

Unprotected assignments within this band can accommodate services other than OPC, where local planning conditions permit.

1 The band 131.405 – 131.980 is suggested in accordance with the proposed definition on Unprotected assignments. (FMG22/WP11)

- Services and frequency protection volumes

…

- Channels and assignments: Classification and treatment

  - Definitions

## Channel type

    - Assignable channels are classified as follows:

      - Standard channel is a channel where frequency compatibility between assignments on the same channel is assessed using distance separation criteria.

Note: A standard channel may be used commonly by a group of States for a specific purpose such that assignments of participating States on this channel are considered a priori compatible. However, compatibility with assignments of non-participating States is assessed using distance separation criteria.

      - Special purpose channel is a channel designated for a specific purpose where frequency compatibility between assignments on the same channel is assumed a priori. Special conditions may apply for the use of such channels.

        - Special purpose channels are:

- the Emergency channel (121.500);

- the SAR channel (123.100);

- the assignable channels of the data link sub-band;

- the channels of the OPC sub-band (131.400-131.975 MHz);

- the channels 122.500, 122.505 and 123.500, 123.505, that are reserved for regional light aviation.

## Assignment type

    - Frequency assignments are classified according to their status of protection as follows:

      - Protected frequency assignment: A frequency assignment whose stations are expected to operate free from harmful interference within the corresponding DOC, unless otherwise implied by the applicable planning and co-ordination rules or by bilateral agreement.

Note 1: Protected frequency assignments should preferably be made on standard channels.

Note 2: Restrictions apply to the protection of reception of protected frequency assignments proposed or registered on or close to a channel in (d) or (e) of paragraph 4.1.1.2.1. (See sub-section 4.2.3 below).

Note 3: Frequency assignments on the channels (a), (b) and (c) of paragraph 4.1.1.2.1 are considered by convention as protected frequency assignments.

      - Unprotected frequency assignment: A frequency assignment for which compatibility of reception is not assessed in the planning of aeronautical frequency assignments. Stations operating in accordance with such an assignment may expect interference from other aeronautical assignments.

  - Planning for protected and unprotected frequency assignments:

    - Any two unprotected frequency assignments are considered a priori compatible.

    - Protected and unprotected frequency assignments on standard channels:

      - The reception and the transmission of a protected frequency assignment are considered compatible with those of another protected frequency assignment, when sufficient distance or frequency separation exists to ensure the protection of the reception of both assignments.

      - The reception and the transmission of a protected frequency assignment are considered compatible with those of an unprotected frequency assignment with a DOC height greater than zero, when sufficient distance or frequency separation exists to ensure the protection of the reception of the protected assignment.

      - The reception and the transmission of an unprotected frequency assignment with a DOC height greater than zero are considered compatible with those of a protected frequency assignment, when sufficient distance or frequency separation exists to ensure the protection of the reception of the protected assignment.

      - New or modified unprotected frequency assignments on standard channels shall be coordinated with their actual DOC, i.e. the actual range and height greater than zero.

Note: Unprotected frequency assignments on standard channels that are planned to be used only on the ground should be coordinated with a height equal to 1 (i.e. 100ft).

        - Unprotected frequency assignments with a DOC height equal to zero operate on condition that they should not cause harmful interference to nor claim protection from any protected frequency assignment, regardless of distance separation. All transmissions associated to an unprotected frequency assignment with a DOC height equal to zero operating on a standard channel shall be immediately discontinued on reception of an interference report indicating the unprotected frequency assignment as the cause of the interference.

Note: Because of the above condition of operation, in a compatibility assessment registered unprotected frequency assignments with a DOC height equal to zero can be assumed compatible with any protected frequency assignment.

    - Protected frequency assignments on or close to the channels in (d) or (e) of paragraph 4.1.1.2.1:

      - The reception of a protected frequency assignment on one of the channels in (d) or (e) of paragraph 4.1.1.2.1 is considered a priori compatible with the presence of any other frequency assignment.

Note: No distance separation requirement is applied for the protection of the reception of such an assignment from any other assignments.

      - The reception and the transmission of a protected frequency assignment on a standard channel at frequency separation from a channel in (d) or (e) of paragraph 4.1.1.2.1 less than 25 kHz, are considered a priori compatible with the presence of any frequency assignment on the latter channel.

Note 1: No distance separation requirement is applied for the protection of the reception of such an assignment from assignments on the special purpose channel.

Note 2: Frequency assignments on a standard channel at 16.67 kHz frequency separation from a channel in (d) or (e) of paragraph 4.1.1.2.1 are not expected to be subject to harmful interference from frequency assignments on such a channel, provided the two 2nd-adjacent-channel frequency assignments are not deployed in the same airport and that no frequency offset is used.

      - The reception and the transmission of a protected frequency assignment on a standard channel at 25 kHz frequency separation from a channel in (d) or (e) of paragraph 4.1.1.2.1 are considered compatible with frequency assignments on the latter channel, when the distance separation foreseen in paragraph 6.1 is ensured between ground receivers of the former assignment and ground transmitters of the latter.

  - Guidance on frequency co-ordination

    - A proposed assignment can be objected when it is incompatible with an existing assignment.

Note 1: A proposed assignment is incompatible with the reception of an existing protected assignment when harmful interference is expected to the reception of the existing protected assignment caused by transmissions of the proposed assignment. A proposed protected assignment is incompatible with the transmission of an existing assignment when harmful interference is expected to the reception of the proposed protected assignment caused by transmissions of the existing assignment.

Note 2: In the case of a bilateral arrangement regarding two incompatible assignments, one proposed and the other existing, it is recommended that the content of the arrangement be recorded in a repository accessible to the Registration Office.

# 45. 	Co-channel separation distances between services

…

# 56.	Adjacent channel separation distances between services

…

67.		Operational Control

…

78.		Utilization of the Frequency Band 112 – 117.975 MHz

…

89.		VDL Mode 2 Assignments

…

910.	VDL Mode 4 Assignments

…

____________________

Under development (September 2021).