Appendix L — proposal for amendment to eur supps (doc 73030), Ch. 13 (AIS)

(paragraph 5.1.4 refers)

Chapter 13.	AERONAUTICAL INFORMATION SERVICES

# 13.1	NOTAM ADDRESSING AND DISTRIBUTION

    - In addition to the distribution to individual States, all NOTAM originated worldwide shall also be addressed to the European AIS Database (EAD) using the AFTN address EUECYIYN as destination address.

Note.— Bilateral addressing agreements between States remain unchanged.

    - The EAD shall ensure distribution of NOTAM to all relevant NOTAM Offices (NOF) within its area of responsibility (EAD Clients).

    - NOTAM originated by EAD Clients shall be channelled through the EAD system and therefore indicate the EAD AFTN origination address.

    - The EAD AFTN destination and origination addresses are as follows:

| Type of message | EAD destination address | EAD origination address (where applicable) |
| --- | --- | --- |
| NOTAM | EUECYIYN | EUECYIYN |
| SNOWTAM | EUECYIYS | EUECYIYN |
| ASHTAM | EUECYIYA | EUECYIYN |
| BIRDTAM | EUECYIYB | EUECYIYN |
| Freetext | EUECYIYX | EUECYIYN |
| Request for: - repetition of NOTAM - original version of NOTAM - list of valid NOTAM | EUECYRYX | EUECYIYN |
| Reply message | EUECYRYX | EUECYIYN |
| Note.— “Request for…” messages will be processed automatically by the EAD in case the standard request format is applied. |  |  |

Note 1.— Detailed procedures and information applicable to the European AIS Database (EAD) is contained in the EUROCONTROL “EAD Operational User Handbook”.

Note 2.— BIRDTAM is not an official ICAO term. BIRDTAM is an acronym for AFTN messages originated by military services based on a NATO Standard to provide information about bird strike risk/warning particularly in lower level flying areas. The EAD covers those messages for specific military clients.

EUR 13-2	Regional Supplementary Procedures

# 13.2	AERONAUTICAL CHART INFORMATION

13.2.1	Visual procedures

13.2.1.1 Information essential for the conduct of visual departures and visual approaches (e.g. significant obstacles, topographical and cultural features), including any specific limitations as prescribed by the appropriate authority (e.g. designated airspace, recommended tracks) shall be displayed on the visual approach chart and standard instrument departure (SID) chart or standard instrument arrival (STAR) chart, as appropriate.

--END--