Appendix X – Available Platforms providing Detection and Forecasting Information of Biomass Burning Smoke

Several different types of biomass burning information available at open data platforms

Biomass burning smoke (FU) impact analysis in a part of the ICAO Crisis Management Framework Document (EUR Doc 031) and describes impact on aircraft, airspace, aerodrome, flight operations, ANSP, persons and cargo. Immediate safety impact on an aircraft is due to reduction of horizontal and vertical visibility or contamination affecting cabin air requiring flight crew use of oxygen masks. Longer term safety impact is on an aircraft ventilation and pressurization systems which can become heavily contaminated.

Climate models (e.g. CIMP5 and CIMP6 - https://interactive-atlas.ipcc.ch) predict higher average temperature with less summer precipitation in southern parts of Europe, causing favourable conditions for severe biomass burning events. In the next decades we could expect even more frequent and severe fire events, at least in this region, so we believe we should be prepared and respond appropriately.

Improved FU information is only possible with the use of reliable, accurate and appropriate detection and forecasting capabilities. Capacity could be developed on national or international level.

Several open data products for the EUR region providing detection and forecasting information of biomass burning smoke are available. The products development and research focus mostly on air quality information. Nevertheless, the aviation industry could benefit from this already available information.

Special global satellite monitoring of fire areas is available at Copernicus platform https://gwis.jrc.ec.europa.eu/apps/gwis_current_situation/index.html .

On-site real time data over the EUR region is available at EUMETNET’s extensive ceilometer network platform https://e-profile.eu/#/cm_profile with point vertical back scatter measurement data. Given the known limitations of this type of measurements, the data could also be useful for the detection of air pollution particles (including biomass burning smoke) at a given point and the estimation of vertical extent of the particles layers.

Numerical modelling results for global biomass burning smoke areas is available at the Copernicus Atmosphere Monitoring Service (CAMS) https://atmosphere.copernicus.eu/charts/ . The charts show forecasts of biomass burning aerosol total optical depth at 550 nm.

Numerical modelling for daily particulate matter (PM) biomass burning concentration is available at https://www.regional.atmosphere.copernicus.eu/ . The forecast is an output from an ensemble model (so called CAMS regional Ensemble) based on eleven state-of-the-art numerical air quality models developed in Europe.

State MET providers are encouraged to further investigate existing and emerging modelling and detection capabilities within individual States with national organisations responsible for air quality services or similar (could be the same as MET ANSP or different entities).

States are also encouraged to improve coordination and information exchange between the MET service provider and the ATS provider, since FU could cause airspace flight restrictions or bans and information platforms above could help MET ANSP as useful additional information, if capabilities at national State level are not sufficient.

Some examples of available products from the summer 2023 period

Fires on island Rhodes (Greece) on the 23th of July 2023

Picture 1; Eumetsat Natural Colour RGB/MSG 0 degree                    Picture 2; CAMS ensemble forecast PM10 fire particles at surface

Source - https://view.eumetsat.int/		                                       Source - https://www.regional.atmosphere.copernicus.eu/

- Portugal on the 9th of August 2023

Picture 3; CAMS ensemble daily max PM10 fire particles

Source - https://www.regional.atmosphere.copernicus.eu

- Canada and USA on the 6th of June 2023

Picture 4; Copernicus biomass burning aerosol optical depth at 550 nm forecast

Source - https://atmosphere.copernicus.eu/global-forecast-plots