EUR Doc 011

INTERNATIONAL CIVIL AVIATION ORGANIZATION

European and North Atlantic Office

EUR Frequency Management Manual

for

Aeronautical Mobile

and

Aeronautical Radio Navigation

###### Services

Edition 2025

# Amendments

Procedure for the Amendment of the EUR Frequency Management Manual

Principles and procedures for the amendment of EUR Documents, as approved by EASPG, are contained in the EASPG Handbook, EUR Doc 001. Accordingly, amendments to the EUR Frequency Management Manual which have been approved by the FMG are formally endorsed by EASPG and/or PCG. Amendments to the EUR Frequency Management Manual shall be effected on the basis of an adequately documented proposal submitted to the FMG of the EASPG. Such proposals should include draft new text clearly identifying additions, modifications and deletions of existing text.

The latest edition of this Manual, including endorsed amendments, will be promulgated to FMG members by the ICAO Regional Office and access will be provided at the ICAO Website.

INTERNATIONAL CIVIL AVIATION ORGANIZATION

European and North Atlantic Office

Web		http://www.icao.int/EURNAT/

E-mail		icaoeurnat@icao.int

Tel		+33 1 46 41 85 85

Fax		+33 1 46 41 85 00

Mail		3 bis Villa Emile Bergerat

F-92522 Neuilly-sur-Seine Cedex

France

| Edition | Subject(s) | Approved |
| --- | --- | --- |
| 2002 | Introduction of EUR Frequency Management Manual | FMG/6 |
| 2003 | Amendment of criteria for 3rd adjacent 8.33 kHz COM channel; Addition of note on COM offset-carrier systems; Addition of note on extension of ILS Glide Path up to 15 NM; Amendment of DME planning criteria for different pulse code; and Amendment of planning criteria for identifications of radio navigation aids. | FMG/7 |
| 2004 | Amendment concerning VDL and 8.33 kHz for OPC; Amendment concerning VHF COM area to broadcast services separation distance; Amendment concerning NDB frequency co-ordination above 526.5 kHz; Amendment of GBAS VDB provisions; and Amendment of planning criteria for identifications according to Annex 11. | FMG/8 |
| 2005 | Amendment concerning ILS frequency assignment planning; Addition of note concerning VHF COM vertical separation; Addition of Block Planning Rules (Appendix to Part II); and Addition of offset carrier (CLIMAX) information. | FMG/9 |
| 2006 | Amendment concerning temporary guidance for GBAS/H planning criteria, Amendment concerning temporary use of ILS to GBAS planning co-ordination criteria, Amendment concerning GBAS VDB frequency planning assignment criteria, Amendment concerning Block Planning procedure | FMG/10 |
| 2007 | Amendment concerning Maritime Radio Beacons with DGPS transmissions Amendment concerning the RSBN4 System Amendment concerning ground-based augmentation system (GBAS) Amendment concerning the procedure to allocate or request a new UHF Channel for ATC communications in GAT | FMG/11 |
| 2008 | Amendment concerning the utilization of the frequency band 117.975 – 137 MHz Amendment concerning the coordination and registration procedure for aeronautical frequency |  |
| 2009 | Amendment concerning the utilization of the frequency band 117.975 – 137 MHz; Amendment concerning the VHF AirGround communications frequency assignment planning criteria; Amendment concerning Directional DME; Amendment concerning unwarranted objections; Amendment concerning country codes & addresses to be used for frequency coordination; Amendment concerning international coordination of mobile offshore NDB idents; Amendment concerning handling incomplete COM3 & COM4 coordination messages. | FMG/13 |
| 2010 | Amendment concerning DME. Amendment concerning GBAS material | FMG/14 |
| 2011 2011 (corrigended) | Amendment in Part II in relation to AS Assignments Amendment in Part III, Section 1, NDB & Locator Amendment in Appendix A part IV Amendment in Part II, Section 4 (para. 4.1.1.1 added) | FMG/15 |
| 2012 | Amendment in Part I, section 3 Amendment in Part II, section 1, 4 and 5 Removal of Appendix in Part II | FMG/16 |
| 2013 | Amendment in Part II, new section 7 “Utilization of the frequency band 112 – 117.975 MHz Note added to the title of Part III Amendment in Part I, section 3 Amendment in Part II, section 4, para 4.2.5 | FMG/17 |
| 2013 | Amendment in Part II, section 5 Amendment in Part II, section 2 , “8.33 in the OPC band” | FMG/18 |
| 2014 | Amendment in Part III, section 3, MLS | FMG/19 |
| 2014 | Amendment in Part II, section 2, addition of para 2.2 Amendment in Part II, section 1, addition of para 1.9 Amendment in Part IV, Section 1, 2, 3 and 4 Amendment in Part III, Section 7, identification of NDBs on maritime Vessels | FMG/20 |
| 2015 | Amendment in Part II, General, 8.33 kHz Climax Amendment in Part II, section 4, addition of text “consideration of terrain in compatibility assessment” Amendment in Part II, section 5, DSM-AM to DSB-AM Amendment in Part III, Part 7, Note on “NDB identifications” Amendment in Part IV, Appendix C “Mandatory Coordination Data”, deletion of Appendix D | FMG/21 |
| 2016 | Amendment in Part II, Section 2, utilization of frequency band 117.975-137 MHz | FMG/22 |
|  | Amendment in Part II, Section 3 and 4 Amendments in Part II, Section 7, Utilization of the Frequency band 112 – 117.975 MHz Amendment in Part II, addition of section 8 “VDL Mode 2 Assignments” Amendment in Part III, Section 1, NDB Amendment in Part III, Section 3, Addition of Note in Para 3.1.1 Amendment in Part III, Section 5, DME Amendment to the Supplement of EUR Doc 011, Insertion of section “B. Guidance on the Coordination of Common Assignments” |  |
| 2017 | Amendment in Part II, Section 1 Amendment in Part II, Section 3 and Section 8 Amendment in Part III, Section 2, ILS Frequency Assignment Planning Criteria Amendment in Part III, Section 5, DME Editorial Updates in Part III, Section 1 Amendment to the Supplement of EUR Doc 011, Insertion of section F “VDL Ground Station Installation Guidance” | FMG/23 |
| 2018 | Amendment List of Abbreviations and Part I, General Overview Amendment Part II, Section 1, Pilot Controlled Lighting Amendment Part II, Add Section 9, VLD Mode 4 Assignements Amendment part III, Section 5, DME Amendment part III, Section 4, GBAS Amendment to the Supplement of EUR Doc 011: - Addition of Guidance on DME First Adjacent-Channel Compatibility - Addition Section D ‘Prioritisation Method for DME Channels’ | FMG/24 |
| 2019 | Amendment Part III, Section 3, VOR Amendment Part III, Section 7, Identification of Radio Navigation Aids, deletion of a note Amendment to Appendix A to Part III Updates for Spain in Appendix A “Country Codes and AFTN Addresses” Amendment to the Supplement of EUR Doc 011: - Amendment in Part A Section 2.2 - Addition of Part I : “Convention for Indicating the angular limits of sectorization in range”, old method. | FMG/25 |
| 2020 | Amendments in entire document on : - Renaming of Table COM 2 into Table COM, Table COM 3 and COM 4 into Table NAV (EASPG Conclusion 1/20 refers) - Renaming of EANPG into EASPG - Removal of outdated references to old versions of EUR Air Navigation Plan (BASIC Vol I and FASID Vol II), with references of new EUR ANP Vol I and EUR ANP Vol II published in 2016. | FMG/26 |
| 2021 | Amendment in Part II: VHF Air-Ground Communications Frequency Assignment Planning Criteria, Addition of section 4 Channels and Assignments: Classification and Treatment | FMG/27 |
| 2022 | a) Updates to Appendix A: Country Codes and AFTN Addresses to be used for frequency coordination b) New allotment plan for VDL 2 channels: amendments to the EUR Doc 011 related to the allotment of the frequency 136.675 MHz for VDL 2, including the new channel allotment plan for data link (EASPG Decision 4/10 refers) c) Update in Part I, section 3 regarding RPAS C2 Link (EASPG Conclusion 4/16). | FMG/28 |
| 2023 | - Extension of temporary allotment of the frequency 136.675 MHz for VDL 2 until 31 December 2027 (EASPG Decision 5/18 refers). - Endorsement of new coordination procedures for Urgent Frequency Assignment and Temporary Frequency Assignment (EASPG Decision 5/19). The new procedures are included in the EUR Doc 011 supplement. - Usage of channels 123.065 and 123.135 (FMG Decision 29/2) | FMG/29 |

TABLE OF CONTENTS

Amendments	ii

List of abbreviations	viii

Part I	Scope and General Overview	1

1	Scope	1

2	The Frequency Management Group	2

3	General Rules for Co-ordination of Frequency Assignments	3

Part II	VHF Air-Ground Communications Frequency Assignment Planning Criteria	5

1	General	5

2	Utilization of the frequency band 117.975 – 137 MHz	7

3	Services and frequency protection volumes	10

4	Channels and Assignments: Classification and Treatment	12

5	Co-channel separation distances between services	15

6	Adjacent channel separation distances between services	19

7	Operational Control	21

8	Utilization of the Frequency Band 112 – 117.975 MHz	21

9	VDL Mode 2 Assignments	23

10	VDL Mode 4 Assignments	23

Part III	Radio Navigation Aid Frequency Assignment Planning Criteria	24

1	NDB and locator	24

2	ILS	34

3	VOR	42

4	GBAS	54

5	DME	64

6	MLS	80

7	Identifications of Radio Navigation Aids	81

8	Non-ICAO Standard Systems operating in Part or in Total within ARNS Bands	83

Part IV	Frequency Co-ordination and Registration Procedures	89

1	Introduction	89

2	General	89

3	Basic Co-ordination and Registration Procedures for Aeronautical Frequency Assignments	90

4	Procedure to allocate or request a New UHF Channel for ATC Communications in GAT	99

# List of abbreviations

Abbreviations which are defined in the Procedures for Air Navigation Services – ICAO Abbreviations and Codes (PANS-ABC) (Doc 8400) are used in accordance with the meanings and usages given therein. As far as possible, the abbreviations used in this document and outlined below are those which have the widest international use. Their meaning is explained below:

ACARS	Aircraft Communications Addressing and Reporting System

AM	Amplitude Modulation

ANP	Air Navigation Plan

CCIR	International Radio Consultative Committee (predecessor of ITU-R)

COG	EANPG Coordination Group (predecessor of the PCG)

dB	Decibel

DOC	Designated Operational Coverage

DPSK	Differential Phase Shift Keying

D/U	Desired to Undesired

eANP	Electronic Air Navigation Plan

EANPG	European Air Navigation Planning Group (predecessor of the EASPG)

EASPG 	European Aviation System  Planning Group

ECAC	European Civil Aviation Conference

EIRP	Equivalent Isotropically Radiated Power (ITU Radio Regulations 1.161)

EMRP	Effective Monopole Radiated Power (ITU Radio Regulations 1.163)

ERP	Effective Radiated Power (ITU Radio Regulations 1.162)

EUR	ICAO European region

EUR ANP	European Air Navigation Plan (Doc 7754)

FASID	Facilities and Services Implementation Document (predecessor of the eANP)

FCB	Frequency Co-ordinating Body (predecessor of the FMG and dissolved in 1995)

FMG	Frequency Management Group (established in 1995)

GBAS	Ground-based augmentation system (a GNSS element)

GHz	Gigahertz

IAOPA	International Aircraft Owners and Pilots Association

IATA	International Air Transport Association

ITU	International Telecommunication Union

ITU-R	ITU Radiocommunication Sector

kHz	Kilohertz

MHz	Megahertz

PCG	EASPG Programme Coordination Group

SAFIRE	Spectrum and Frequency Information Resource

SUM	Standard Updating Message

VDL	VHF Air-ground Digital Link

W	Watt

Conventions

Shall	Indicates that the requirement is mandatory to be able to claim compliance with this document.

Will 	Indicates an intention or assumption for something.

Should 	Indicates a desirable requirement that cannot be justifiably mandated.

May 	Indicates that may be at the choice for example of a State or frequency manager.

# Part I	Scope and General Overview

## Scope

  - Frequency assignment planning for the aeronautical frequency bands listed below are subject to ICAO EUR regional planning criteria and procedures:

| LF/MF | 255 – 495 kHz and 505 – 526.5 kHz | NDB and locator |
| --- | --- | --- |
| VHF | 108 – 117.975 MHz 117.975 – 137 MHz | ILS localizer (below 112 MHz), VOR and GBAS Air-ground communications |
| UHF | 328.6 – 335.4 MHz 960 – 1215 MHz | ILS glide path DME |
| SHF | 5030 – 5150 MHz | MLS |

  - Aeronautical HF air-ground communications frequencies are planned in accordance with Appendix 27 of the ITU Radio Regulations and are not subject to ICAO regional planning procedures.

  - A number of other frequency bands are used by aviation. In many cases, these bands are not allocated exclusively to aeronautical services. ICAO frequency assignment planning procedures, however, have been developed and are applied only for the bands listed under paragraph 1.1 above.

## The Frequency Management Group

  - The Frequency Management Group (FMG) is established by EANPG Decision 37/2 to pursue the tasks of the Group in the field of aeronautical frequency spectrum management in support to the relevant ICAO Strategic Objectives with the following TORs:

    - Ensure the continuous and coherent development of the relevant sections of the European eANP and other relevant regional documents, including EUR Doc 011 Frequency Management Manual, taking into account the evolving operational requirements in the EUR Region and the need for harmonization with the adjacent regions in compliance with the Global Air Navigation Plan;

    - Monitor and coordinate implementation of the relevant ICAO SARPs and regional procedures, facilities and services by the EUR States and where necessary promote and facilitate harmonization, taking due account of financial and institutional issues;

    - Identify any deficiencies in the aeronautical frequency spectrum management related matters in the EUR Region and coordinate the development and implementation of relevant action plans by the States to resolve them;

    - Foster implementation by facilitating the exchange of know-how and transfer of knowledge and experience among States of the Region;

    - Provide input to the work of appropriate ICAO bodies in the field of aeronautical frequency spectrum, according to the established procedures.

  - The main tasks of the FMG are:

- to establish co-ordinated frequency assignment plans for the EUR aeronautical mobile services and the EUR radio navigation aids service, and to make recommendations, as necessary, concerning frequency aspects of their implementation;

- to co-ordinate the frequency aspects of new requirements, as necessary;

- to give advice to States on questions of frequency assignment, rated coverage, etc., as necessary;

- to undertake specific tasks assigned to it by the EASPG;

- to advise the EASPG on frequency spectrum issues covering all aeronautical radio services, including satellite based facilities; and

- to work in liaison with relevant international organisations.

  - Each State within the region and relevant international organisations such as EUROCONTROL, IATA and IAOPA should nominate a FMG Member, being authorised with the responsibility for aeronautical frequency management. In order to take care of possible problems in the border area between Regions, the EUR FMG also includes experts from States adjacent to the European Region, in practice the States along the Mediterranean. Contact details can be found in a "List of FMG members and frequency experts" published by the ICAO Paris office.

## General Rules for Co-ordination of Frequency Assignments

  - Chapters 4 and 5 of Volume I of  the ICAO “Handbook on Radio Frequency Spectrum Requirements for Civil Aviation” (Doc 9718) provide an overview of institutional relations important to aeronautical frequency management and co-ordination of assignments.

  - In most cases a Telecommunications Administration / Radio Authority within a State is the responsible superior authority for use of the radio spectrum. This includes the authority to co-ordinate new assignments with other States under the rules of the ITU and after successful co-ordination to register the new assignment.

  - There is an arrangement for coordination between Civil Aviation representatives in the EUR region. The coordination processes are designed to ensure that States have an opportunity to assess compatibility with the planning rules contained in this document prior to a frequency being assigned for operational use. The EASPG has mandated the use of the Spectrum and Frequency Information (SAFIRE) web based tool to carry out coordination and registration of frequency assignments in the EUR Region. The basic aeronautical co-ordination procedure is depicted below.

Requesting State

sends proposal

to all States concerned

Requesting State negotiates

details of the proposal

Requesting State

withdraws the proposal

ICAO

registers

assignment

Requesting State

starts implementation

any

objection(s)

received

?

Requirement

for a new

assignment

yes

no

yes

Assignment granted.

ICAO and all States concerned informed

  - Co-ordination is deemed to have been completed and the assignment is considered to be granted when either:

- All concerned States have registered their acceptance, or

- A 4 week period has elapsed without an objection having been registered.

  - For the VHF communications band 117.975 - 137 MHz special procedures are applied due to the congestion problems in this band.  Block Planning Exercises are organised, as required, based on the mutual arrangements agreed between the States involved. These exercises are organized for those requirements for which an available channel can only be identified through shifting of one or more existing assignments to some other channel. The normal coordination procedure can be used if no shifts are involved.

  - Co-ordination must be made with all States that in some way may be affected by the proposed assignment. Additionally some States may have an interest in being informed about changes, although not directly affecting them, in order to update national databases or for other purposes.

  - After successful co-ordination of a new or modified assignment, the assignment is registered in SAFIRE database. All States and ICAO are informed about the registration of assignments through SAFIRE. The registered assignments are published in the appropriate frequency assignment table of the ICAO EUR Air Navigation Plan.

  - No frequency assignment for the provision of RPAS C2 links should be made in the bands 108-137 MHz and 960-1164 MHz prior to the development of adequate frequency planning criteria agreed by the FMG for use in the EUR Region.

# Part II	VHF Air-Ground Communications Frequency Assignment Planning Criteria

## General

  - The band 117.975 - 137 MHz is allocated to the aeronautical mobile (R) service and used mainly for air/ground voice communications and, to some extent, air/ground data communications.

  - References to documents:

-	Annex 10, Volume V, paragraph 4.1

(allotment table, channelling, protection criteria);

-

-	European Region Air Navigation Plan (Doc 7754), Volume I, Part III,  paragraphs 2.3 to 2.5 and 2.9 (general planning requirements) ;

-	European Region Air Navigation Plan (Doc 7754), Volume II, Part III,  paragraphs 2.42 to 2.46 and 3.58 to 3.61 (general planning considerations).

.

  - Depending on the type of service, the protected range can be from 16 NM to 260 NM or defined by the border of a FIR or sector. The protected altitude can vary from 3000 ft up to 45000 ft. The actual protection details can be obtained from the frequency assignment table.

    - Protection by distance separation has in the past been achieved by making sure that any other transmitter (on the ground or airborne) is below the radio horizon. Alternatively, a recently introduced change to Annex 10 allows the Regional use of a desired to undesired signal ratio of 14 dB, equivalent to a 5 to 1 distance ratio, if this separation distance is shorter than the distance to the radio horizon.

    - Risk of conflict normally only occurs between two aircraft, one in each service area, while the location of the ground transmitter is of less importance. For practical planning special software made available through EUROCONTROL or a table may be used, listing the required distance between combinations of services on the same channel. Also the adjacent channel is considered.

    - Some sub-bands are reserved for special purposes. From  121.5417 to 121.9917 MHz only ground-ground communication is allowed, typically for communications between the TWR and taxiing aircraft. The bands 131.400 to 131.975 MHz is used by airlines for OPC according to special rules. In several States, common frequencies are used for applications such as light aviation, gliding and ballooning activities, etc.

      - Frequency assignment should be done in the dedicated sub-bands. Due to the limitations of the sub-bands, States are strongly encouraged to introduce 8.33 kHz channel spacing to reduce frequency congestion.

      - It is recommended that States reduce the antenna height and transmit power to reduce the possibility of interference to other stations, but still be able to provide sufficient coverage.

      - In case of congestion of the dedicated AS sub-band, frequencies may be planned on a temporary basis outside the sub-band in accordance with the planning criteria.

    - Three channels are used for ACARS which is not an ICAO system. The uppermost part of the VHF COM band (136.700 – 136.975 MHz) is reserved for VDL Mode 2 and VDL Mode 4.

    - Frequency assignments can be reserved for a common use over several States. Guidance material on the allotment, coordination and compatibility assessment between common assignments is provided in the Supplement to EUR Doc011.

  - The risk of interference from broadcasting stations in the band 87 - 108 MHz, due to interference caused by unwanted emissions into the aeronautical band or generated in the airborne receiver, is generally not considered to be an operational problem for this type of communications.

  - Co-ordination must at least be made with States which may be affected by the proposal. Special care is required for assignments with a very large co-ordination distance (e.g. VOLMET) which may exist on almost any frequency in the band.

  - It has been agreed within the FMG to use a special procedure due to the congestion problems in this band. Requests for assignments which cannot be satisfied without shifts have to be submitted to a  Block Planning exercise , which is run about every six months and based on the mutual arrangements agreed between the States involved.

  - Call signs and designators are not considered as part of the frequency planning process in this band.

  - The ICAO planning criteria presented in this document do not provide protection against interference phenomena which may occur if communication facilities are co-located (e.g. interference caused by intermodulation). It is therefore possible that a proposed frequency is not acceptable due to specific local conditions.

  - 8.33 kHz CLIMAX.  Significant numbers of aircraft currently operating in European airspace are believed not to be equipped with radios intended for 8.33 kHz offset carrier operation. Consequently,the assignment of 8.33 kHz channels for operation in offset carrier mode shall not be permitted until further notice, whilst studies are conducted, except in cases where all aircraft receiving the service are equipped with a radio intended for 8.33 kHz offset carrier operation.

Note: Aircraft radios intended for 8.33 kHz offset carrier operation should be compliant with the requirements for EUROCAE ED-23C Class H1 or H2 receivers.

  - EANPG Conclusion 56/04 invites States to permit the use of TWR frequencies by vehicles involved in runway operations, where required:

Note: Any assignment of a TWR frequency to vehicles involved in runway operation should be subject to local safety assessment. Such assessment should address in particular training, operating procedures and the use of one language in radiotelephony communications between air crews, vehicle drivers and TWR controllers, and enforcement measures.

  - At aerodromes where a VHF pilot-controlled lighting systems are deployed, these should generally operate on the frequency assigned to the aerodrome. No separate frequency should be required.

  - Assignment of VHF Data Link (VDL) services. States shall only coordinate and assign VDL Mode 2 and 4 frequencies approved by the FMG and contained in the VHF data link channel plan.

## Utilization of the frequency band 117.975 – 137 MHz

  - The table is based on Annex 10, Volume V, 4.1.1 also incorporating regional agreements on specific uses of individual frequencies or sub-bands and the identification of the assignable 25 kHz and 8.33 kHz channels:

| Channel | Frequencies (MHz) | Worldwide General utilization | EUR special application or remarks |
| --- | --- | --- | --- |
| 118.000 – 121.460 inclusive | 118.000 - 121.4583 inclusive | Aeronautical Mobile Services | 119.700 MHz reserved for regional guard supplementary tower and approach services. |
| 121.500 | 121.500 | Emergency frequency |  |
| 121.540 – 121.990 inclusive | 121.5417 - 121.9916 inclusive | Aerodrome Surface Communications |  |
| 122.000 – 123.060 | 122.000 - 123.0583 inclusive | Aeronautical Mobile Services | 122.100 MHz reserved as common channel to provide supplementary TWR/APP services in a number of nations. 122.500 MHz reserved for regional light aviation. |
| 123.100 | 123.100 | Auxiliary frequency SAR |  |
| 123.140 – 136.490 | 123.1417 - 136.4916 inclusive | Aeronautical Mobile Services 123.450 MHz reserved for air-to-air communications for flights over remote and oceanic areas out of range of VHF ground stations | 123.300 MHz reserved as common channel to provide PAR services in a number of nations. 123.450 MHz reserved for air-to-air communications only for flights over remote and oceanic areas out of range of VHF ground stations 123.500 MHz reserved for regional light aviation 131.400 – 131.975 MHz reserved for operational control (OPC) communications* 131.525, 131.725 and 131.825 reserved for ACARS data link |
| 136.500 – 136.675** | 136.500 - 136.675 | Aeronautical Mobile Services | New assignments permitted only on a temporary basis.*** |
| 136.700 – 136.975** | 136.700 - 136.975 inclusive | Aeronautical Mobile Services 136.900 - 136.975 MHz reserved for air-ground data link communications on a worldwide basis. | reserved for air-ground data link communications (see paragraph 2.2.) |

Guard bands:

For 121.500: Channels 121.475, and 121.525, are not assignable; and

For 123.100: Channels 123.065, 123.075, 123.080, 123.085, 123.115, 123.125,  123.130, and 123.135 are not assignable, except for ATIS on channels 123.080 and 123.130.

*OPC:	Assignments used for OPC purposes shall not be co-ordinated on protected standard channels. 8.33 kHz channel spacing to be used wherever possible, between 131.405 – 131.980 except for the ACARS channels 131.525, 131.725 and 131.825. The use of voice at the first adjacent 8,33 kHz channel to ACARS is not possible. Care should be taken when considering the use of the second adjacent channel at 16,67 kHz from the ACARS.

Unprotected assignments within this band can accommodate services other than OPC, where local planning conditions permit.

Concerning the use for ATS purposes of the lower adjacent frequencies to 131.400 and the upper adjacent frequencies to 131.975, care should be exercised for the observance of adjacent channel compatibility requirements.

**Note:	8.33 kHz channel spacing is not to be used for frequencies 136.500 – 136.975 MHz inclusive (Annex 10, Volume V,

paragraph 4.1.6.1.1).

***Note:	the temporary allotment of the frequency 136.675 MHz for VDL 2 is extended until 31 December 2027 as coordinated with FMG members during FMG29 (EASPG Decision 5/18 refers):

- with reference to the EASPG Conclusion 3/13, the temporary reservation of channels 136.650 and 136.675 for VDL Mode 2 services be removed by end of 2027.

- until the end of 2027, no proposals for frequency assignments on channels 136.650 and 136.675 for voice communication services should be admitted and that such proposals on a temporary basis be readmitted as of 1 January 2028.

- channel 136.650 until the end of 2027 serves as guard channel. The possibility of allotting channel 136.650 to voice communication services prior to the end of 2027 is under consideration.

  - The table below contains the VHF data link channel plan in the sub-band 136.700 – 136.975 MHz

| Frequencies (MHz) | Usage | Remarks |
| --- | --- | --- |
| 136.700 | VDL Mode 2 | AIR |
| 136.725 | VDL Mode 2 | AIR |
| 136.750 | VDL Mode 2 | AIR |
| 136.775 | VDL Mode 2 | AIR |
| 136.800 | VDL Mode 2 | AIR |
| 136.825 | VDL Mode 2 | GND |
| 136.850 | VDL Mode 2 | AIR |
| 136.875 | VDL Mode 2 | GND |
| 136.900 | VDL Mode 2 | AIR |
| 136.925 | VDL Mode 4 VDL Mode 2 | common signalling channel (CSC) GND |
| 136.950 | VDL Mode 2 | AIR |
| 136.975 | VDL Mode 2 | common signalling channel (CSC) |

Note 1: 	GND means that the channel is planned for use mainly by ground stations providing airport coverage.

AIR means that the channel is planned for use mainly by ground stations providing en-route coverage.

Note 2: 	Services in all VDL Mode 2-allotted channels can be delivered by using dual DSP ID (dual squitter) technology.

Note 3: 	Subject to evaluation of local conditions and data traffic intensity, VDL services can be deployed on first-adjacent channels. The factor enabling the compatibility of 25 kHz-adjacent-channel services is the sufficient distant separation between receive and transmit antennas of such services on the ground, which is estimated up to about 3 NM.

Note 4: The use of channel 136.925 by VDL Mode 2 services requires the agreement of States in the ICAO EUR Region planning VDL Mode 4 services on this channel, in which case paragraph 4.2.12 of Part II of this Manual applies. The co-ordination of frequency assignments on channel 136.925 for VDL Mode 2 services should be accompanied with a precise specification of the DOC, when the use of this channel is not limited strictly on the ground. With the exception of this compatibility constraint between VDL Mode 2 and VDL Mode 4 services on channel 136.925, VDL co-channel and adjacent-channel frequency assignments by different States are considered a priori mutually compatible.

Note 5:  CSC is expected to be used primarily for signaling purposes. The exchange of data traffic on CSC is to be avoided unless under conditions of light loading of the channel.

Note 6:  In line with ICAO Annex 10 Volume III ATTACHMENT TO PART I. GUIDANCE MATERIAL FOR THE VHF DIGITAL LINK (VDL), Section 4.5 and in order to minimize the probability of simultaneous transmissions by hidden transmitters, an AIR channel should not be used for aerodrome surface communications except in areas of very low channel loading.

Note 7: In line with ICAO Annex 10 Volume III ATTACHMENT TO PART I. GUIDANCE MATERIAL FOR THE VHF DIGITAL LINK (VDL), Section 4.4 and in order to minimize the probability of simultaneous uplink transmissions from ground stations causing message collisions and affecting channel performance, the smallest number of ground stations possible should be used to provide en-route coverage.

## Services and frequency protection volumes

  - Information on frequency assignments includes designations of services for which the frequencies are used. The designations and abbreviations indicated below should be used:

    - Aerodrome

TWR		Aerodrome control service

AS		Aerodrome surface communications

PAR		Precision approach radar

AFIS		Aerodrome flight information service

    - Approach

APP		Approach control service

ATIS		Automatic terminal information service

    - En route

FIS		Flight information service

ACC		Area control service

    - Other functions

A/A		Air-to-air

A/G		Air-to-ground

EMERG	Emergency

OPC		Operational control

SAR		Search and rescue

VOLMET	Meteorological broadcast for aircraft in flight

DL		Data Link services

  - The DOC is that agreed to be the standard for the region (). Values different from those indicated may be used in some cases, as appropriate. Minimum required coverage volumes should be used where frequency congestion exists.

  - The frequency protection volume of a service is usually identical to the designated operational coverage. It defines the airspace where the frequency assignment planning process provides protection from other assignments.

  - A combined code was developed in the EUR Region for the definition of the service type and the frequency protection volume.

    - A letter provides information on the type of service:

A	area service (e.g. ACC);

B	broadcast service without airborne transmission (e.g. ATIS, VOLMET);

C	circular service (e.g. TWR, APP, AS);

E	European service (e.g. EMERG, SAR); and

U	Unprotected service (e.g. OPC).

    - A numeric part specifies the DOC (range (r)/height (h)) for all service types except for area services where the horizontal extension is defined as a polygon):

A-h	the vertical extension is provided as an integer multiple of 100 ft (e.g. A-450 is used for an area service with an upper limit at 45000 ft for sector Upper1);

B-r/h	the horizontal circular extension is expressed in NM and the vertical extension in integer multiples of 100 ft (e.g. B-60/200 is used for a broadcast service with a range of 60 NM and a vertical extension of 20000 ft);

C-r/h	as for B-r/h (e.g. C-40/150 is used for an approach service with a range of 40 NM and a vertical extension of 15000 ft);

ICAO Table COMcontains a specific field to indicate whether an assignment has protected or unprotected status. Unprotected services, such as OPC, are recorded in Table COM with the protection field set to “U”.

## Channels and Assignments: Classification and Treatment

  - Definitions

# Channel type

    - Assignable channels are classified as follows:

      - Standard channel is a channel where frequency compatibility between assignments on the same channel is assessed using distance separation criteria.

Note: A standard channel may be used commonly by a group of States for a specific purpose such that assignments of participating States on this channel are considered a priori compatible. However, compatibility with assignments of non-participating States is assessed using distance separation criteria.

      - Special purpose channel is a channel designated for a specific purpose where frequency compatibility between assignments on the same channel is assumed a priori. Special conditions may apply for the use of such channels.

        - Special purpose channels are:

- the Emergency channel (121.500);

- the SAR channel (123.100);

- the assignable channels of the data link sub-band;

- the channels of the OPC sub-band (131.400-131.975 MHz);

- the channels 122.500, 122.505 and 123.500, 123.505, that are reserved for regional light aviation.

## Assignment type

    - Frequency assignments are classified according to their status of protection as follows:

      - Protected frequency assignment: A frequency assignment whose stations are expected to operate free from harmful interference within the corresponding DOC, unless otherwise implied by the applicable planning and co-ordination rules or by bilateral agreement.

Note 1: Protected frequency assignments should preferably be made on standard channels.

Note 2: Restrictions apply to the protection of reception of protected frequency assignments proposed or registered on or close to a channel in (d) or (e) of paragraph 4.1.1.2.1. (See sub-section 4.2.3 below).

Note 3: Frequency assignments on the channels (a), (b) and (c) of paragraph 4.1.1.2.1 are considered by convention as protected frequency assignments.

      - Unprotected frequency assignment: A frequency assignment for which compatibility of reception is not assessed in the planning of aeronautical frequency assignments. Stations operating in accordance with such an assignment may expect interference from other aeronautical assignments.

  - Planning for protected and unprotected frequency assignments:

    - Any two unprotected frequency assignments are considered a priori compatible.

    - Protected and unprotected frequency assignments on standard channels:

      - The reception and the transmission of a protected frequency assignment are considered compatible with those of another protected frequency assignment, when sufficient distance or frequency separation exists to ensure the protection of the reception of both assignments.

      - The reception and the transmission of a protected frequency assignment are considered compatible with those of an unprotected frequency assignment with a DOC height greater than zero, when sufficient distance or frequency separation exists to ensure the protection of the reception of the protected assignment.

      - The reception and the transmission of an unprotected frequency assignment with a DOC height greater than zero are considered compatible with those of a protected frequency assignment, when sufficient distance or frequency separation exists to ensure the protection of the reception of the protected assignment.

      - New or modified unprotected frequency assignments on standard channels shall be coordinated with their actual DOC, i.e. the actual range and height greater than zero.

Note: Unprotected frequency assignments on standard channels that are planned to be used only on the ground should be coordinated with a height equal to 1 (i.e. 100ft).

        - Unprotected frequency assignments with a DOC height equal to zero operate on condition that they should not cause harmful interference to nor claim protection from any protected frequency assignment, regardless of distance separation. All transmissions associated to an unprotected frequency assignment with a DOC height equal to zero operating on a standard channel shall be immediately discontinued on reception of an interference report indicating the unprotected frequency assignment as the cause of the interference.

Note: Because of the above condition of operation, in a compatibility assessment registered unprotected frequency assignments with a DOC height equal to zero can be assumed compatible with any protected frequency assignment.

    - Protected frequency assignments on or close to the channels in (d) or (e) of paragraph 4.1.1.2.1:

      - The reception of a protected frequency assignment on one of the channels in (d) or (e) of paragraph 4.1.1.2.1 is considered a priori compatible with the presence of any other frequency assignment.

Note: No distance separation requirement is applied for the protection of the reception of such an assignment from any other assignment.

      - The reception and the transmission of a protected frequency assignment on a standard channel at frequency separation from a channel in (d) or (e) of paragraph 4.1.1.2.1 less than 25 kHz, are considered a priori compatible with the presence of any frequency assignment on the latter channel.

Note 1: No distance separation requirement is applied for the protection of the reception of such an assignment from assignments on the special purpose channel.

Note 2: Frequency assignments on a standard channel at 16.67 kHz frequency separation from a channel in (d) or (e) of paragraph 4.1.1.2.1 are not expected to be subject to harmful interference from frequency assignments on such a channel, provided the two 2nd-adjacent-channel frequency assignments are not deployed in the same airport and that no frequency offset is used.

      - The reception and the transmission of a protected frequency assignment on a standard channel at 25 kHz frequency separation from a channel in (d) or (e) of paragraph 4.1.1.2.1 are considered compatible with frequency assignments on the latter channel, when the distance separation foreseen in paragraph 6.1 is ensured between ground receivers of the former assignment and ground transmitters of the latter.

  - Guidance on frequency co-ordination

    - A proposed assignment can be objected when it is incompatible with an existing assignment.

Note 1: A proposed assignment is incompatible with the reception of an existing protected assignment when harmful interference is expected to the reception of the existing protected assignment caused by transmissions of the proposed assignment. A proposed protected assignment is incompatible with the transmission of an existing assignment when harmful interference is expected to the reception of the proposed protected assignment caused by transmissions of the existing assignment.

Note 2: In the case of a bilateral arrangement regarding two incompatible assignments, one proposed and the other existing, it is recommended that the content of the arrangement be recorded in a repository accessible to the Registration Office.

## Co-channel separation distances between services

DSM-AM TO DSB-AM

- The following principles for VHF planning criteria for determining the separation distances between RTF services with DSB AM carriers and 8.33 or 25 kHz channel spacing should be used in the EUR region. Co-channel separation criteria should be applied between an 8.33 kHz and a 25 kHz channel using the same frequency (co-frequency, e.g. channels 132.000 / 132.005).To protect a service with a circular operational coverage (circular service), the distance from the edge of the service to another airborne or ground transmitter should be 5 times the range of that circular service; if the other transmitter is below the radio horizon from that service edge and the radio horizon distance is also less than 5 times the circular service range then radio horizon distance should be used.

- In order to ensure the appropriate frequency planning and assignment of circular services, it is recommended that the operational coverage of circular services is such that the radius is in appropriate proportion to the height (note : further work on this issue to be carried out).

5.1.2 To protect a service with a non-circular operational coverage area, the ground or airborne transmitter of the other service should be below the radio horizon.

    - The protection criteria for both the requested service and the existing assignment should be met for a valid assignment.

- The following co-channel protection criteria should be used for planning purposes.

    - Circular service areas (except broadcast functions) should be planned so that the separation distance from the edge of one circular service area to another circular service should be 5 times the larger range from those respective DOCs or the sum of their radio horizon, whichever is least.

    - The separation distance between circular and area services should be calculated using the radio horizon method. The separation distance between the service edge of the two services should be the sum of the two relevant radio horizon distances (radio line-of-sight distance).

    - The separation distance between circular and broadcast services should be determined as follows:

a)	the separation distance between the aircraft receiver at the circular service edge and the ground broadcast transmitter (actual location, not broadcast service edge) should be a minimum of 5 times the circular service range or the radio line-of-sight distance, assuming a ground antenna height of 20 m (65 ft);

b)	the separation distance between an aircraft receiver at the broadcast service edge and an airborne transmitter at the circular service edge should be a minimum of 5 times the broadcast service range or the sum of the two radio horizon distances; and

c)	both criteria in a) and b) should be satisfied to make a valid assignment.

    - The separation distance between two area services should be calculated using the radio horizon method.

    - In the case of national aerodrome assignments, the co-channel protection criteria applied and the coordinated DOC should be sufficient to protect, but not exceed, that of its sub-ordinate services.

    - For broadcast service to broadcast service planning, the separation distance between the edge of one service and the ground station of the other should be calculated as follows:

a) for each service in turn, calculate:

1) the radio line-of-sight distance, assuming a ground antenna height of 20 m (65 ft); and

2) 5 times the service range; and

b) take the minimum of these figures for each service.

The broadcast service having the larger of these two figures determines the separation distance between the edge of that service and the ground station of the other.

    - The separation distance between area and broadcast services should be determined as follows:

a)	the separation distance between the aircraft receiver at the area service edge and the ground broadcast transmitter (actual location, not broadcast service edge) should be at least the radio line-of-sight distance, assuming a ground antenna height of 20 m (65 ft);

b)	the separation distance between an aircraft receiver at the broadcast service edge and an airborne transmitter at the area service edge should be a minimum of 5 times the broadcast service range or the sum of the two radio horizon distances; and

c)	both criteria in a) and b) should be satisfied to make a valid assignment.

Separation distances between services

The table below presents examples of minimum co-channel separation distances between edges of service areas. The DOCs were taken from Attachment B, Part V.II, European Region Air Navigation Plan, Volume II, FASID (except for VOLMET).

|  | AFIS or TWR | TWR | APP | APP | ATIS | APP | VOLMET | ACC | ACC | ACC | ACC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DOC range (NM) height (ft) | (a) 16 3000 | (b) 25 4000 | (c) 25 10000 | (d) 40 15000 | (e) 60 20000 | (f) 50 25000 | (g) 271 45000 | (h) 15000 | (i) 25000 | (j) 35000 | (k) 45000 |
| (a) AFIS or TWR | 80 | 125 | 125 | 200 | U241* Note 1 | 250 | 328* | 218* | 261* | 297* | 328* |
| (b) TWR | 125 | 125 | 125 | 200 | 252* | 250 | 339* | 229* | 272* | 308* | 339* |
| (c) APP | 125 | 125 | 125 | 200 | 297* | 250 | 384* | 274* | 317* | 353* | 384* |
| (d) APP | 200 | 200 | 200 | 200 | 300 | 250 | 412* | 302* | 345* | 381* | 412* |
| (e ) ATIS | 241* | 252* | 297* | 300 | 124*+ | 300 | 211*+ | 300 | 300 | 300 | 300 |
| (f ) APP | 250 | 250 | 250 | 250 | 300 | 250 | 455* | 345* | 388* | 424* | 455* |
| (g) VOLMET | 328* | 339* | 384* | 412* | 211*+ | 455* | 10*+ | 412* | 455* | 491* | 522* |
| (h ) ACC | 218* | 229* | 274* | 302* | 300 | 345* | 412* | 302* | 345* | 381* | 412* |
| (i) ACC | 261* | 272* | 317* | 345* | 300 | 388* | 455* | 345* | 388* | 424* | 455* |
| (j) ACC | 297* | 308* | 353* | 381* | 300 | 424* | 491* | 381* | 424* | 460* | 491* |
| (k ) ACC | 328* | 339* | 384* | 412* | 300 | 455* | 522* | 412* | 455* | 491* | 522* |

Note 1	Distances marked with an asterisk (*) are limited to the radio-horizon

(e.g. U241*U: 241 =  1.23*(+).

Note 2	All distances given in the table are in NM and are measured between service areas.

Note 3	Distances marked with (+) were calculated assuming a ground antenna height of 65 ft (20 m).

    - Consideration of terrain in compatibility assessment

    - Frequency managers are encouraged to take into consideration the effect of terrain when conducting frequency compatibility assessment. When assessing frequency compatibility between co-channel assignments, in cases where the path profile of the undesired co-channel transmission is intersected by terrain, the resulting undesired field strength at a receiver can be expected to be lower by at least 6 dB than that which would be achieved if the signal path was unobstructed. This equates to a 50% reduction in the undesired path length. The following approach to assessing compatibility in these cases is recommended.

    - The 5:1 Case

5.2.9.1 In cases where the co-channel planning rule requires that the separation distance between the edges of two services should be 5 times the larger of their respective DOCs (e.g. circular-versus-circular), compatibility can generally be assumed when:

a) The undesired signal path is intercepted by terrain; and

b) The distance between the edges of the DOCs is not less than 2.5 times the path length of the desired signal.

    - The RLOS case

5.2.10.1 In cases where the co-channel planning rules require that the minimum separation distance between DOC edges is the sum of the RLOS distances of the two services (e.g. area-versus-area), compatibility can generally be assumed when:

a) The undesired signal path is intercepted by terrain; and

b) The distance between the edges of the DOCs is not less than 5 times the path length of the desired signal.

5.2.10.2 In the case of area services where the length of the desired path is not known (i.e. where the ground station location has not been coordinated), it is recommended that the desired path length is taken to be 70% of the length of the maximum diagonal of the desired DOC polygon.

Notes:

- MANIF AFM provides a capability for conducting the compatibility assessment using the above criteria;

- A technical justification for the above recommendations is provided in the Supplement to ICAO EUR Doc 011.

    - Recording of terrain masking in SAFIRE

5.2.11.1 Where a State coordinates a new assignment that does not comply with the co-channel planning rules but, by reason of the terrain effects described above, is deemed to be compatible with another existing assignment or with a proposal under coordination, a relevant remark should be provided in the SAFIRE Remarks field simply by inserting the word "TERRAIN".

VDL TO VDL & OTHER SERVICES

    - The separation distance between VDL and DSB-AM as well as between VDL and another VDL service shall be calculated using the radio horizon method. The separation distance between the service edges of the two services shall be the sum of their radio horizon distances.

AERODROME SURFACE ASSIGNMENTS

    - Aerodrome Surface assignments should be planned according to the following guidance:

      - The band [121.5417 to 121.9916 MHz] inclusive is reserved exclusively for aerodrome  surface movement assignments (AS).

      - The DOC area should be coordinated as circular, with a radius of 5NM and a  maximum antenna height of 100ft (C-5/1).

      - AS assignments used for ATS purposes should be coordinated as protected.

      - AS assignments should be co-ordinated preferably in the dedicated band [121.5417 to  121.9916 MHz] inclusive. If it is necessary to plan an AS assignment outside of this band, it  should be coordinated on a temporary basis.

## Adjacent channel separation distances between services

DSM-AM to DSB-AM

The following planning criteria should be used for the deployment of DSB AM RTF services on adjacent channels:

- Services with equal channel spacing (either 8.33 or 25 kHz) and frequency separation of one (1) channel or with different channel spacing and frequency separation of 25 kHz:

- These services may have overlapping service areas provided that a separation distance of at least 10 NM is maintained between a ground receiver of one service and a ground transmitter of the other service, except when both services under consideration are on 8.33 kHz channels and at least one utilises offset carrier systems.

- In the case of two 8.33 kHz services where one or both utilises an offset carrier system, the separation distance between services should be determined on the basis of co-channel separation criteria.

Note: For international co-ordination purposes, the only requirement for States in this case is to notify to a neighbouring State the location of ground receivers and ground transmitters that are located within 10NM of the land mass of the neighbouring State  except when they are sited at the centre of  a circular service. Notwithstanding, States are urged to ensure compliance with the constraint in a) within their own territories.

- Services using different channel spacing and frequency separation of 8.33 kHz:

The distance separation of services as above should be determined on the basis of co-channel separation criteria.

- Services using different channel spacing and frequency separation of 16.67 kHz:

The separation distance between edges of service areas should be at least 10 NM except when both services under consideration are broadcast services;

A broadcast service may have an overlapping service area with another broadcast service provided that the ground transmitter of each service is at least 10 NM outside the edge of the service area of the other service.

Note 1:  These planning rules are based on the assumption that the ground transmitters and receivers are placed within the protected service volume.

Note 2:  As a consequence of the above criteria, adjacent channel services for ATS purposes should not be deployed in the same airport.

Note 3: Where a State proposes a new offset-carrier service where the location of the ground transmitter(s) may affect an existing foreign assignment, it is the responsibility of that State to ensure that sufficient technical data is made available to the adjacent State(s) through bi-lateral coordination to allow the other State to complete a compatibility assessment.”

VDL to VDL & Other Services

- Adjacent channel separation between VDL and DSB-AM systems operating in the same geographical area shall be in accordance with the following table.

| Vs. |  | Interference source |  |  |
| --- | --- | --- | --- | --- |
|  |  | DSB-AM | VDL Mode 2 (117.975-137 MHz) | VDL Mode 4 (130-137 MHz) |
| Victim | DSB-AM | - | 1 | 2 |
|  | VDL Mode 2 | 1 | - | - |
|  | VDL Mode 4 | 2 | - | - |

6.4.1. VDL services can be deployed on channels separated by 25 kHz, subject to sufficient distance separation between receive and transmit antennas of such services on the ground, which is estimated up to about 3 NM.

Note 1: 	The numbers in the table represent the guard channels (25 kHz). The next frequency that can be used without any frequency assignment planning constraint is 1 (one) higher (e.g. a desired DSB-AM station that is interfered by a VDL mode 2 aircraft station requires one 25 kHz guard band channel. The next frequency, 50 kHz separated from the VDL Mode 2 assigned frequency, can be used in the same designated operational coverage area without any frequency assignment planning constraint.

Note 2: 	Attention is drawn to the possibility of interference between DSB-AM and VDL when used onboard an aircraft on the surface of an airport. Interference can occur if the channel separation (guard band) is four or less. In this case, interference can be prevented by securing that the minimum field strength at the (receiving) antenna is at least -70 dBm.

Note 3: 	Any interference that can be caused by aircraft transmission of DSB-AM or VDL signals on the surface of an airport into a ground-based (i.e. not an aircraft) receiving station can be mitigated or removed through using cavity filters in the ground based receiver. These filters block the reception of unwanted signals.

Note 4: Due to the small size of the required distance separation between receive and transmit antennas of adjacent-channel VDL services, adjacent-channel VDL frequency assignments are considered mutually compatible for international co-ordination purposes.

## Operational Control

  - Dedicated frequencies for operational control purposes should be assigned to those aircraft operators who are required to maintain a system of Operational Control under the provisions of Annex 6, Part I.

  - Strict economy in the number and use of frequencies assigned for this purpose should be observed. Shared use of channels by different operators is a method which may help to achieve this goal. Eurocontrol can provide advice to States (see paragraph 1.3.3).

  - The Annex 10 provisions for frequency protection should not apply to OPC frequencies used in the EUR region.

## Utilization of the Frequency Band 112 – 117.975 MHz

  - In accordance with provisions of the Regional Supplementary Procedures (Doc 7030), based on an agreement reached at FMG/21, the assignment of VDL Mode 4 frequencies in the band 112 – 117.975 MHz shall not be permitted until further notice.

Note: Currently, all navigation aids operating in the band 112-117.975 MHz make use of ground based transmitters and no transmissions take place from the aircraft. There is the potential for aircraft-based VDL Mode 4 transmissions to cause harmful interference to the users of these existing ground-based navigation aids, and their monitors, unless a capability is provided to prohibit unintended VDL Mode 4 transmission outside of designated operational coverage. It is not currently clear whether this capability is available and further studies need to be conducted.

  - Subject to a satisfactory solution ensuring that VDL Mode 4 transmission do not cause harmful interference to other users of the band, the Regional Supplementary Procedures may be amended to remove the above restriction. In those circumstances, in accordance with the provisions of Annex 10 and the ITU Radio Regulations, the frequency band 112 – 117.975 MHz could be used for VDL Mode 4. When making frequency assignments to VDL Mode 4 in this frequency band, the following frequency assignment planning parameters apply:

  - VDL Mode 4 to VDL and VHF DSB/AM (air/ground voice)

    - Co-frequency

- Note: the frequency band 112 – 117.975 MHz is (currently) not to be used for VHF DSB/AM or VDL mode 3 systems as per provisions of Annex 10, Volume III and no co-frequency assignment planning to secure compatibility between VDL Mode 4 and VHF DSB/AM or VDL Mode 2 are necessary.

- Co-frequency use of frequencies for VDL Mode 4 (different VDL Mode 4 networks)

- The separation distance between VDL Mode 4 frequency assignments shall be calculated using the radio horizon method. The separation distance between the service edges of the two services shall be equal or greater than the sum of their respective distances to the radio horizon (see also paragraphs 4.2.8 above)

    - Adjacent frequency

- Adjacent channel compatibility of VDL Mode 4 with DSB/AM or VDL Mode 2 systems is secured when applying the provisions of paragraph 5.4 above.

  - VDL Mode 4 and ILS-Localizer

    - Co-frequency

- Note: Since, in accordance with the provisions of Annex 10 (Volumes I and II) the highest assignable frequency for the ILS Localizer is 111.950 MHz and the lowest assignable VDL Mode 4 frequency is 112 MHz, no co-frequency use of VDL Mode 4 and the ILS Localizer is expected.

    - Adjacent frequency

- Note:  Since, in accordance with the provisions of Annex 10 (Volumes I and III) the highest assignable frequency for the ILS Localizer is 111.950 MHz and the lowest assignable frequency for VDL Mode 4 is 112 MHz, the minimum frequency separation between an ILS-Localizer frequency and a VDL Mode 4 frequency is 50 kHz.

- VDL Mode 4 can be used without frequency assignment planning constraints when separated from an operational ILS-Localizer frequency with 50 kHz or more.

- Note: In certain cases VDL Mode 4 may be subject to harmful interference from transmissions from the ILS-Localizer transmitter if the separation distance is less than 2.5 NM. Such interference is typically transient in nature.

  - VDL Mode 4 and VOR

    - Co-frequency and first adjacent (25 kHz) frequency

- The separation distance between VDL Mode 4 and VOR frequency assignments shall be calculated using the radio horizon method. The separation distance between the service edges of the two services shall be equal or greater than the sum of their respective distance to the radio horizon.

    - Second adjacent frequency (50 kHz separation between VOR and VDL Mode 4 frequency assignments)

- VDL Mode 4 can be used without frequency assignment planning constraints when separated from an operational VOR station with 50 kHz or more.

- Note:  In certain cases VDL Mode 4 may be subject to harmful interference from transmissions from the VOR transmitter if the separation distance is less than 2.5 NM. Such interference is typically transient in nature.

  - VDL Mode 4 and GBAS

    - Co-frequency and first adjacent (25 kHz) frequency

- The co-frequency separation distance between VDL Mode 4 and GBAS frequency assignments shall be calculated using the radio horizon method. The separation distance between the service edges of the two services shall be equal or greater than the sum of their respective distance to the radio horizon.

- The first adjacent (25kHz) frequency channel separation distance shall be 10 Km between the service edge of the GBAS VDB and any VDL4 transmitter.

    - Second adjacent frequency (50 kHz separation between VDL Mode 4 and GBAS)

- VDL Mode 4 can be used without frequency assignment planning constraints when separated from an operational GBAS station with 50 kHz or more.

- Note:  VDL Mode 4 should not operate on the 2nd, 3rd or 4th adjacent channel to a VDB GBAS frequency at the surface of an airport in case the separation between a VDB GBAS and a VDL Mode 4 equipped aircraft can be less than 130 m. Note: This is an operating restriction that may affect the use of VDL Mode 4 on an airport.

## VDL Mode 2 Assignments

9.1	In order to support evolution of the VDL band plan, the VHF data link assignments should be coordinated as follows:

- Assignments on auxiliary frequencies should be made on temporary basis to support changes to the Data Link Allotment Plan accordingly;

- Assignments should be deleted if the frequencies are not used within one year;

- Discrete assignments should be made for each CSP at each location;

- Assignments should be coordinated as DL services with a DOC of C-1/0 and include the precise location of the ground station(s) and their EIRP;

- Assignments should include the CSP name and the ground station identifier in the Remarks field.

## VDL Mode 4 Assignments

10.1 	In order to support evolution of the VDL band plan, VDL Mode 4 assignments on the VDL4 Common Signalling Channel (CSC) 136.925 should be coordinated as follows:

- Assignments on channel 136.925 should be on a temporary basis to support changes to the Data Link Allotment Plan accordingly;

- Assignments should be deleted if the frequencies are not used within one year;

- States using VDL4 should create Assignments on channel 136.925. Assignments should be coordinated as DL services with the text “VDL4 CSC” in the SAFIRE Remarks field;

- Protected polygonal assignments should be created in the areas where aircraft will use VDL4 CSC;

- Discrete assignments could be made for each fixed ground station: they should include the precise location of the ground station(s) and their EIRP. In addition, they should contain the operator name in the Remarks field.

# Part III	Radio Navigation Aid Frequency Assignment Planning Criteria

## NDB and locator

- General

    - A number of sub-bands in the range of 255 – 526.5 kHz (excluding 495 - 505 kHz) are allocated to aeronautical radio navigation with various status. Only the sub-band 325 - 405 kHz is allocated exclusively to the aeronautical radio navigation service, the other sub-bands being shared with other services.

- As for the other users of the band, the sub-band 255 - 283.5 kHz is mainly used by broadcasting stations, while the sub-bands 283.5 - 315 kHz and 315 - 325 kHz are used by maritime beacons. The small segment 405 - 415 kHz is designated for radio direction-finding in the maritime radio navigation service, while the sub-bands in the range 415 - 526.5 kHz (excluding the segment 495 – 505 kHz) are used by the maritime mobile service, limited to radiotelegraphy.

- It is noted additionally that Rradio frequency carrier systems are widely used for remote control or transmission of speech or data over segments of high-voltage overhead power lines. In some cases frequencies in the LF/MF bands are used and this has been proved to be able to affect ADF indications in aircraft at several nautical miles from the power lines.

    - Organization of Planning

- In order to secure the international and the operational status of their assignments, States are required to apply the rules and procedures of the ITU Radio Regulations. This is particularly necessary for the shared sub-bands, since the ICAO EUR Table NAV is not supposed to include non-aeronautical assignments.

- For assignments in the aeronautical radio navigation service, States are required to apply the ICAO EUR FMG rules and procedures.

Note 1: Although in the shared sub-bands the FMG process is evidently partial and cannot result, if applied alone, in the required status, this process is necessary for the co-ordination of the identifications of the radio navigation aids.

Note 2: In the shared sub-bands, States are advised to avoid including assignments for non-aeronautical services (e.g. maritime services) in the FMG co-ordination process, as such assignments are not taken into account in compatibility assessments.

    - References to documents:

-	Annex 10, Volume I, Attachment C, paragraph 6.2.1.6

(field strength);

-	Annex 10, Volume V, section 3.2

(general about protection, sharing and frequency congestion);

-	Annex 10, Attachment A to Volume V

(protection, receiver characteristics, filter attenuation);

-	European Region Air Navigation Plan (Doc 7754), Volume II, Part III, paragraphs 3.58 to 3.60 (general planning considerations).

    - The range can be from 10 NM (nautical miles) up to 100 NM (in some rare cases more) and the operational range is listed in the national AIP and the frequency assignment table. The maximum altitude is not specified. Co-channel separation between two transmitters shall be such that an unwanted signal is more than 15 dB below the wanted signal within the specified service area (DOC). This protection criterion is also included in the ITU Radio Regulations. The bandwidth characteristics of the airborne receiver need to be taken into account which require in principle that existing transmitters within ± 7 kHz from the frequency of the wanted station be considered.

    - In the band 325 - 405 kHz, aviation is the only user and new assignments should therefore preferably be made in this segment. Use of a segment where aeronautical radio navigation is secondary should only be considered in areas where the primary service is not used, like inland areas where the distance to any maritime station is sufficient.

    - The co-ordination of LF/MF radio navigation aids to be applied for aeronautical purposes above 526.5 kHz shall be carried out between national radio regulators, applying the relevant ITU procedures. The co-ordination of the identification of such a radio navigation aid shall be carried out between FMG members according to the rules in Part IV.

    - For normal coverage ranges (15 - 50 NM) co-ordination should, as an absolute minimum, be made with States within a radius of 300 - 400 NM. Co-ordination should also be made with the national authority responsible for protection of the maritime services if a shared band segment is to be used.

    - Information on the planning of identifications can be found in section 7.

- Frequency Assignment Planning Principles

1.2.1	Wherever possible, frequencies at kHz points (i.e. integral multiples of 1 kHz) in the bands used for NDBs should be chosen to meet particular requirements. Frequency assignments at 0.5 kHz points may also be utilized, provided the full required protection can be ensured.

    - In the planning of frequency assignments, ADF receiver characteristics as specified in paragraph 3 of Attachment A to Annex 10, Volume V, should be assumed.

    - For the purpose of assessing the signal attenuation with distance, the LF/MF ground wave propagation curves agreed by the ITU-R should be used, taking into account the frequency of operation and the effect of mixed land/sea path where appropriate.

- Frequency Assignment Planning Criteria

    - The protection of a NDB against interference from another NDB can be calculated using the propagation curves in Recommendation ITU-R P.368-9. These curves (known also as CCIR curves) represent the propagation characteristics of a transmitter (beacon) with an effective monopole radiated power (e.m.r.p.) of 1 kW. These curves can be adjusted to describe the propagation characteristics (field strength as a function of the distance from the beacon) of a beacon with a given coverage area. Such adjustment needs to ensure that the field strength at the edge of the coverage area is the minimum required. For aeronautical beacons this field strength is 70 µV/m or 36.9 dB (µV/m). (Ref.: Annex 10, Volume I, section 3.4 and ITU Radio Regulations Appendix 12).

    - For frequency planning purposes, the CCIR curves for sea water and wet ground propagation are used, with the following characteristics:

Sea water:	σ = 5 S/m		Wet ground:	σ = 10-2 S/m

ε = 70					ε = 30

    - The protection ratio has to be calculated at the point P where the interference is maximum. There are two cases to consider:

i)	when the undesired beacon (B) is inside the coverage area of the desired beacon (A): at the location of that (undesired) beacon (see Figure 1)

A

B≡P

Figure 1

ii)	when the undesired beacon (B) is outside the coverage area of the desired beacon (A): at that point on the edge of the coverage area of the desired beacon closest to the undesired beacon (see Figure 2)

Figure 2

B

P

A

    - Calculation of the minimum protection ratio (D/U) shall ensure identification of the worst-case interference.

    - To ensure the compatibility of two beacons, it is necessary to consider successively the two cases where each of the beacons is treated as desired and the other one as undesired. In each case, the protection ratio needs to be calculated at the point of maximum interference. The lowest value of these two protection ratios is the value at risk and to be used in frequency planning.

    - Once the worst case protection ratio is known, the minimum frequency separation can be determined taking into account the ADF selectivity characteristics.

  - Description of the Calculations

    - As a first step, the ITU-R curves (see section 1.6) are adjusted to obtain field-strength values corresponding to the effective monopole radiated power of the beacon, necessary to obtain the minimum field strength required at the edge of coverage (36.9 dB µV/m).

Let a and b denote respectively the operational ranges of the beacons A and B.

The following symbols are additionally used:

f(R)		field strength at distance R, for e.m.r.p.= 1kW (from ITU-R curves).

fPAP(R)				field strength for beacon A at distance R.

fPBP(R)				field strength for beacon B at distance R.

f(a)				field strength  f(R) at R = a

f(b)				field strength  f(R) at R = b

fPAP(a); fPBP(b)			field strengths  fPAP(R) at R = a and fPBP (R) at R = b

[normally 36.9 dB µV/m].

f(1); fPAP(1); fPBPP(1)		field strengths f(R), fPAP(R) and fPBP(R) at R = 1 km.

f(R) at R = 1 km is 109.5 dB µV/m.

See also Recommendation ITU-R P.368-9.

The field strengths produced by beacons A and B at any distance R from the beacon satisfy:

fPAP(R) = f(R) - [f(a) - 36.9]						(1)

fPBP(R) = f(R) - [f(b) - 36.9]						(2)

Note: In general the field strength (f(R), f A(R) or f B(R)) depends not only on the path distance R (this dependence being the only explicit in the notation), but also on the frequency as well as on the geological constitution of the radio-path (proportion of sea-path over the total path distance R).

    - Determination of the field strengths f(a) and f(b)

On considering each beacon separately, the first step is the identification of the radial along which the proportion p of sea-path over a total path distance equal to the range of the beacon is lowest.

On the basis of the lowest proportion p, the mixed-path calculation method (see section 1.5) is then applied for the determination of the quantity f(a) or f(b) and the consequent dimensioning of the output power of the beacon (see section 1.7).

    - Calculation of the protection ratio when the undesired beacon is inside the coverage area of the desired beacon (see paragraph 1.3.3 i) above). In this case the maximum level of the undesired signal that contributes to the interference of the desired signal is at the location of the transmitter of the undesired beacon.

If we assume A to be the desired beacon and B the undesired beacon, the maximum level of the undesired signal is:

EU B = fPBP(1)	= f(1) - [f(b) - 36.9]

= 109.5 - [f(b) - 36.9]						(3)

The signal level of the desired facility A at this point of maximum interference is:

ED B = fPAP(Rs)	= f(Rs) - [f(a) - 36.9]			0<Rs<a			(4)

Rs = separation distance between A and B.

The protection ratio D/U is:	EBDB - EBUB		or	fPAP(Rs) - fPBP(1).

Substitution of (3) and (4) in this formula gives:

D/U = f(Rs) - f(a) + f(b) - 109.5							(5)

Note: For the application of formula (5), the quantity f(Rs) at the position of the undesired beacon is determined by the application of the mixed-path calculation method (see section 1.5).

    - Calculation of the protection ratio when the undesired beacon is outside the coverage area of the desired beacon (see paragraph 1.3.3 ii) above). In this case the maximum level of the undesired signal that contributes to the interference of the desired signal is at the location of point P in Figure 2.

At this point the signal level of the desired facility A is:

ED B = fPAP(a*) = f(a*) - [f(a) - 36.9]							(6)

(fA(a*),  f(a*) denote respectively the adjusted and the non-adjusted (corresponding to e.m.r.p.=1kW) field strengths at the particular point P).

The signal level of the undesired facility B at this point is:

EU	= fPBP(Rs - a)

= f(Rs - a) - [f(b) - 36.9]	,		a<Rs,				(7)

Rs = separation distance between beacon A and B.

The protection ratio D/U is:	EBDB - EBUB		or	fPAP(a*) - fPBP(Rs-a).

Substitution of (6) and (7) in this formula gives:

D/U = f(b) - f(Rs - a) + f(a*) - f(a).						(8)

Note: For the application of formula (8), the quantities f(a*) and f(Rs-a) are determined by the application of the mixed-path calculation method (see section 1.5), f(a*) corresponding to the propagation path AP, while f(Rs-a) corresponding to propagation path BP (see Figure 2).

    - Formulas (5) and (8) can be used to calculate the interference from beacon B (undesired) to beacon A (desired). When the outcome of the calculations shows that the D/U is equal or greater than 15 dB, the desired beacon is protected and the beacons can operate on the same frequency. When the outcome is less than 15 dB, a frequency separation between the frequencies assigned to these beacons has to be maintained.

The required frequency-separation depends upon the calculated D/U ratio and can be taken from the following table:

| D/U(dB) | frequency-separation (kHz) between desired and undesired signal |
| --- | --- |
| U>U 15 | 0 |
| < 15 to U>U 9 | 1 |
| < 9 to U>U (-5) | 2 |
| < (- 5) to U>U (-20) | 3 |
| < (-20) to U>U (-35) | 4 |
| < (-35) to U>U (-50) | 5 |
| < (-50) to U>U (-65) | 6 |
| < -65 | 7 |

Table 1 – Required Frequency Separation

The above relation between the D/U ratio and the required frequency-separation corresponds to the ADF-receiver selectivity curve in Annex 10, Volume V, Attachment A, paragraph 3.

Since the D/U ratio, for a combination of beacons with a range of a and b respectively, is a function of the distance between these beacons and the required frequency separation is a function of the D/U ratio, the minimum required frequency separation is a complex function of the separation distance.

  - Mixed Path Calculations

- The method described below pertains to a propagation path which contains both land and sea segments.

- Although ITU-R is, for this purpose, recommending the Millington method (reference Recommendation ITU-R P.368-9, Annex 2), a simplified method was developed and recommended to be used in calculations when mixed path conditions exist.

- In this simplified method the calculation of the signal-level of a beacon is based upon the following formula:

### f(R)M = f(R)L + { f(R)S - f(R)L}p					(9)

### where :

### f(R)M =	effective field strength of a beacon with EMRP=1kW at a point P, at distance R from the beacon

### f(R)L =field strength at P if the total path were over land.

### f(R)S =field strength at P if the total path were over sea.

### R = total path length.

### p = S/R where S is the total sea-path length.

  - ITU-R Propagation Curves

1.6.1 The ITU-R propagation curves (known also as CCIR curves) can be found in Recommendation ITU-R P.368-9. They were generated through the application of the computer program GRWAVE, which is made available by ITU-R.

1.6.2 The program GRWAVE was used as well for calculating the field strength values of Table 2 below for EMRP=1 kW. Field strength values at intermediate frequencies can be estimated by linear interpolation.

| SEPARATION DISTANCE | BEACON RANGE | SEA 300 kHz | SEA 400 kHz | SEA 500 kHz | LAND 300 kHz | LAND 400 kHz | LAND 500 kHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| KM | NM | DB(UV/M) | DB(UV/M) | DB(UV/M) | DB(UV/M) | DB(UV/M) | DB(UV/M) |
| 2 |  | 103.5 | 103.4 | 103.4 | 103.2 | 103 | 102.8 |
| 4 |  | 97.4 | 97.4 | 97.4 | 97.1 | 96.9 | 96.6 |
| 6 |  | 93.9 | 93.9 | 93.9 | 93.5 | 93.3 | 93 |
| 8 |  | 91.4 | 91.4 | 91.4 | 91 | 90.7 | 90.3 |
| 10 |  | 89.5 | 89.5 | 89.4 | 89 | 88.6 | 88.2 |
| 18.5 | 10 | 84.1 | 84.1 | 84.1 | 83.4 | 82.9 | 82.3 |
| 20 |  | 83.4 | 83.4 | 83.4 | 82.7 | 82.2 | 81.5 |
| 27.8 | 15 | 80.5 | 80.5 | 80.5 | 79.6 | 79 | 78.2 |
| 37 | 20 | 78 | 77.9 | 77.9 | 76.9 | 76.1 | 75.1 |
| 40 |  | 77.3 | 77.3 | 77.2 | 76.1 | 75.3 | 74.2 |
| 46.3 | 25 | 76 | 76 | 76 | 74.7 | 73.7 | 72.6 |
| 55.6 | 30 | 74.4 | 74.4 | 74.3 | 72.9 | 71.8 | 70.4 |
| 60 |  | 73.7 | 73.7 | 73.6 | 72.1 | 70.9 | 69.5 |
| 80 |  | 71.1 | 71 | 70.9 | 69.1 | 67.6 | 65.8 |
| 92.6 | 50 | 69.7 | 69.6 | 69.5 | 67.5 | 65.8 | 63.8 |
| 100 |  | 69 | 68.9 | 68.8 | 66.6 | 64.8 | 62.7 |
| 138.9 | 75 | 65.7 | 65.6 | 65.5 | 62.7 | 60.3 | 57.5 |
| 185.2 | 100 | 62.7 | 62.5 | 62.3 | 58.9 | 55.9 | 52.4 |
| 200 |  | 61.8 | 61.6 | 61.4 | 57.8 | 54.6 | 50.9 |
| 231.5 | 125 | 60.1 | 59.8 | 59.6 | 55.6 | 52 | 47.9 |
| 277.8 | 150 | 57.9 | 57.5 | 57.2 | 52.7 | 48.5 | 43.8 |
| 370.4 | 200 | 53.9 | 53.3 | 52.8 | 47.4 | 42.1 | 36.4 |
| 400 |  | 52.7 | 52.1 | 51.5 | 45.8 | 40.2 | 34.1 |
| 463 | 250 | 50.3 | 49.5 | 48.8 | 42.6 | 36.3 | 29.6 |
| 555.6 | 300 | 46.9 | 45.9 | 45 | 38.1 | 30.8 | 23.2 |
| 600 |  | 45.3 | 44.2 | 43.2 | 36 | 28.2 | 20.3 |
| 800 |  | 38.4 | 36.8 | 35.4 | 26.9 | 17.1 | 7.3 |
| 1000 |  | 31.8 | 29.7 | 28 | 18.2 | 6.2 | -5.3 |
| 1200 |  | 25.4 | 22.8 | 20.7 | 9.6 | -4.4 | -17.8 |
| 1400 |  | 19.1 | 16 | 13.5 | 1.2 | -14.9 | -30.1 |
| 1600 |  | 12.9 | 9.3 | 6.4 | -7.2 | -25.3 | -42.4 |
| 1800 |  | 6.7 | 2.7 | -0.7 | -15.5 | -35.6 | -54.5 |
| 2000 |  | 0.6 | -3.9 | -7.6 | -23.7 | -45.9 | -66.6 |

Table 2 – 1 kW EMRP Field Strength at a Given Separation Distance

  - Effective Monopole Radiated Power (EMRP)

    - Pursuant to the ITU Radio Regulations (Article 28.23), the power radiated by each radio beacon shall be adjusted to the value necessary to produce the stipulated field strength (36.9 dB (µV/m)) at the limit of its operational range.

    - The EMRP is calculated not for frequency co-ordination purposes but for the determination of the transmitter output power P of the beacon which is required for the desired operational coverage. The quantities EMRP and P in dBW are related as follows:

P = EMRP + cable and tuning losses – Gv,

where Gv is the gain of the antenna with regard to a short vertical monopole. In most practical cases Gv=0dB. Cable and tuning losses, on the other hand, are usually not less than 10 dB (see also Attachment C to Annex 10, Volume I, paragraph 5 of section 6.3.2).

    - The EMRP  in dBW can be calculated, using the ITU-R propagation curve for a 1 kW transmitter, as follows:

EMRP	=	30 - [f(a) - 36.9]

=	66.9 - f(a),

where

30 	= 	term for the conversion of the ITU-R propagation curves

from a 1 kW transmitter to a 1Watt transmitter in order to obtain an EMRP value in dB relative to 1 Watt.

36.9	=	minimum field strength in dB(µV/m) required at the edge of the coverage.

f(a) 	= 	field strength in dB(µV/m) at the edge of the beacon’s desired coverage

calculated for an EMRP of 1kW as described in paragraph 1.4.2.

  - Treatment of Assignments over Polygonal Areas

- In exceptional cases, frequency assignments can be co-ordinated over polygonal areas. In such a case :

- Frequency protection is accorded over the co-ordinated polygonal area.

- The envisaged distance (range) between receiver and transmitting beacon is specified at co-ordination in addition to the polygonal area. This range shall not exceed the length of the maximum diagonal of the polygonal area.

- The transmitting beacon can be anywhere inside the polygonal area.

- The following instructions are provided for the calculation of the required D/U ratio involving assignments over polygonal areas (referred to henceforth as area assignments) :

- The field strength f(a) at the range of an area assignment is calculated by considering all possible paths of length a within the polygonal area and by identifying the lowest sea-path proportion among those paths.

- Area assignment versus circular assignment.

- Protection of the area assignment

- If the centre of the circular assignment is inside the polygon of the area assignment, formula (5) is applicable with Rs being equal to the minimum of (i) the range a of the area assignment and (ii) the maximum distance between the centre of the circular assignment and the polygon. f(Rs) can be calculated by considering the radial with the lowest sea-path proportion among those radials of length Rs from the centre of the circular assignment which end up within the polygon.

### Note: When Rs=a, f(Rs)≥f(a). In general Rs=a does not imply f(Rs)=f(a) because the minimisation of the sea-path proportion is effected over different sets of possible paths of length a.

- If the centre of the circular assignment is outside the polygon of the area assignment, formula (8) is applicable with Rs-a being equal to the distance between the centre of the circular assignment and its closest point on the polygon. f(a*) can be calculated by considering the radial with the lowest sea-path proportion among those radials which originate from the point on the polygon closest to the centre of the circular assignment, have length equal to the range a of the area assignment and end up within the polygon. If the distance of this point from any other point on the polygon is less than the range a, the length of the radial for the calculation of f(a*) is reduced to the maximum value of this distance.

- Protection of the circular assignment

        - If the polygon of the area assignment overlaps with the coverage area of the circular assignment, formula (5) is applicable with Rs being equal to the minimum of (i) the range a of the circular assignment and (ii) the maximum distance between the centre of the circular assignment and the polygon. f(Rs) can be calculated by considering the radial with the lowest sea-path proportion among those radials of length Rs from the centre of the circular assignment which end up within the polygon.

      - If the polygon of the area assignment does not overlap with the coverage area of the circular assignment, formula (8) is applicable with Rs being equal to distance between the centre of the circular assignment and its closest point on the polygon. In this case the situation is very similar to the compatibility between two circular assignments.

- Area assignment versus area assignment

      - Protection of each area assignment

        - If the two polygonal areas overlap, formula (5) is applicable with Rs being equal to the minimum of (i) the range a of the area assignment to be protected and (ii) the maximum distance between any point in the intersection of the two polygonal areas and any other point on the polygon to be protected. f(Rs) can be calculated by considering the path with the lowest sea-path proportion among all possible paths of length Rs which originate from points in the intersection of the two polygonal areas and end up within the polygon to be protected.

        - If the two polygonal areas do not overlap, formula (8) is applicable with Rs-a being equal to distance between the two polygons. f(a*) can be calculated by considering the radial with the lowest sea-path proportion among those radials which originate from the point on the polygon to be protected closest to the opposite polygon, have length equal to the range a of the area assignment to be protected and end up within the polygon to be protected. If the distance of this point from any other point on the polygon to be protected is less than the range a, the length of the radial for the calculation of f(a*) is reduced to the maximum value of this distance.

## ILS

  - General

    - The band 108 - 112 MHz is used by ILS Localizer and VOR, on different channels and the ILS Glide Path is using the band 328.6 - 335.4 MHz. Within the band 108 - 112 MHz, channels on odd multiples of 100 kHz (108.10, 108.30, ...) are used for ILS. Normally channels spaced 100 kHz are used, however, in congested areas also the intermediate channels (50 kHz) may be used.

    - References to documents:

-	Annex 10, Volume I, section 3.1

(technical and operational specifications for ILS, in particular localizer and glide path frequency pairing, coverage as well as minimum field strengths);

- Doc 9718, Vol. II, section 3.4

(protection requirements for ILS versus other ILS or versus VOR);

-

-	European Region Air Navigation Plan (Doc 7754), Volume I, Part III, paragraph 2.9

(foundations of frequency assignment planning);

- European Region Air Navigation Plan (Doc 7754), Volume II, Part III, paragraph 2.47 (application of 50 kHz channel spacing).

    - The operational service area of an ILS localizer is defined Annex 10 and often formed as a "key-hole" out to 25 NM. The ILS Glide Path has shorter range and the frequency is fixed in relation to the ILS Localizer. The adjacent channel criteria require special attention, because the localizer to glide path frequency pairing does not make sure that facilities on the adjacent localizer channel are automatically using an adjacent glide path channel and vice versa (e.g. 330.65/110.95 MHz; 330.80/110.90 MHz; 330.95/111.95 MHz). Therefore, the frequency assignment planning for localizer and glide path has to be performed separately.

    - For the co-channel case, the wanted signal must be at least 20 dB stronger than an unwanted signal. The distance from the edge of the service area of the wanted station to another ILS on the same frequency should be more than 80 NM. Another ILS on the first adjacent channel (50 kHz) should not be closer than 5 NM.

    - The co-ordination should also take into account the risk of interference from broadcasting stations in the band 87.5 - 108 MHz, due to interference caused by unwanted emissions into the aeronautical band or generated in the airborne receiver. Rules for this are available from the Recommendation ITU-R SM.1009-1 “Compatibility between the sound-broadcasting service in the band of about 87 - 108 MHz and the aeronautical services in the band 108 - 118 MHz”.

    - Co-ordination should be made with States within a radius of at least 150 NM. Co-ordination should also be made with the authority responsible for the analysis of the broadcasting versus aeronautical situation.

    - Information on the planning of identifications can be found in section 7 below.

  - Frequency Assignment Planning Criteria

    - Frequencies for ILS facilities should be selected from the list at Annex 10, Volume I, 3.1.6.1 in accordance with paragraph 2.47 of  EUR ANP (Doc 7754), Volume II, Part III and paragraph 4.2.2.1 of Annex 10, Volume V. Where DME is provided, the appropriate DME channel selected from Table A at Annex 10, Volume I, Chapter 3 should be used.

    - The co-channel and adjacent channel geographical separations between ILS localizers or between an ILS localizer and a VOR on adjacent channels as well as  between ILS glide path installations should be based on the protection requirements contained in section 3.4 of Doc. 9718, Vol. II  for ILS localizers designed for 50 kHz channel spacing and for ILS glide paths designed for 150 kHz channel spacing.

  - Required Separation Distances

    - The required separation distances to be applied in the ICAO EUR Region are provided below. They originate from material that used to be contained in the Attachment C to Annex 10, Vol. I. This material was removed from Annex 10, Vol. I in 2025  by virtue of the amendment 94  to Annex 10, Vol. I.

    - Note: Doc 9718, Vol. II adopts a different methodology for calculating separation distances and, consequently, leads potentially to different results.

    - Table of required ILS localizer and glide path intra-system separation distances

| Equipment | Frequency Separation | Minimum separation between second facility and the service volume of the first facility (NM) |
| --- | --- | --- |
| Localizer | Co-Channel | 80 |
|  | 50 kHz | 5 |
| Glide Path | Co-Channel | 50 |
|  | 150 kHz | 1 |

Table 1:  Required ILS localizer and glide path intra-system separation distances

Note 1.	The above figures are based on the assumption of protection points for the localizer at 46 km (25 NM) distance and 1900 m (6250 ft) height and for the ILS glide path at 10 NM distance and 760 m (2500 ft) height.

Note 2.	States, in applying the separations shown in the table, should recognize the necessity to site the ILS and VOR facilities in a manner which will preclude the possibility of airborne receiver error due to overloading by high unwanted signal levels when the aircraft is in the initial and final approach phases.

Note 3.	States, in applying the separations shown in the table, should recognize the necessity to site ILS glide path facilities in a manner which will preclude the possibility of erroneous glide path indications due to reception of adjacent channel signals when the desired signal ceases to radiate for any reason while the aircraft is in the final approach phase.

.

Note 4.	Studies have shown that for an extended glide path range of 10 to 15 NM, a separation distance of 2 NM instead of 1 NM for the adjacent channel provides adequate protection. The co-channel is automatically protected by the localizer separation distance.

    - Compatibility between ILS localizer and VOR on adjacent channels

Note: For this compatibility case, FMG drew upon material that existed in Attachment C to Annex 10, Vol. I before amendment 94 (2025) as well as on the assumption that the D/U protection ratios for ILS localizers against intra-system interference are also applicable to interference caused by VOR (see section 3.4 of Doc 9718).

2.3.3.1 No separation requirement needs to be applied for the protection of the VOR facility.

2.3.3.2 For a VOR facility having an EIRP of 17 dBW, a separation distance of 10 NM is required from the service volume of an ILS localizer which provides service to 50 kHz channel-spacing receivers and is operated with frequency separation of 50 kHz (first-adjacent channel).

2.3.3.3 Although there exist instances of ILS and VOR installations with 100 kHz frequency separation located at shorter distances apart without interferences, for a VOR facility having an EIRP of 17 dBW, a separation distance of 5 NM is recommended from the service volume of an ILS localizer which provides service to 50 kHz channel-spacing receivers and is operated with frequency separation of 100 kHz (second-adjacent channel).

    - Note: For a VOR having a different value of EIRP, the required separation distance can be approximately obtained  by multiplying the above values by the factor 100,025(EIRP-17).

    - The horizontal extensions of the ILS localizer and glide path DOCs are shown in the figures below.

Horizontal extension of the ILS localizer DOC

Horizontal extension of the ILS glide path DOC

    - The glide path co-channel minimum separation distance of 50 NM does not require any specific action, because it is indirectly and automatically provided for by the application of the localizer co-channel minimum separation distance of 80 NM.

    - The adjacent channel glide path minimum separation distance of 1 NM does not require any specific action for about half of the adjacent channels, because it is indirectly and automatically covered by the adjacent localizer channel minimum separation distance of 5 NM.

Column A of the table below lists the ILS channels. Column B lists the adjacent glide path channels that require no specific action. Column C lists the adjacent glide path channels that do require specific action (i.e. to check if the minimum required separation distance of 1 NM is respected).

| A | B | C |
| --- | --- | --- |
| LLZ / GP (MHz) | adjacent GP / (LLZ) (MHz) | adjacent GP / (LLZ) (MHz) |
| 108.10 / 334.70 | 334.55 / (108.15) | 334.85 / (110.35) |
| 108.15 / 334.55 | 334.70 / (108.10) | 334.40 / (110.10) |
| 108.30 / 334.10 | 333.95 / (108.35) | 334.25 / (110.15) |
| 108.35 / 333.95 | 334.10 / (108.30) | 333.80 / (109.90) |
| 108.50 / 329.90 | 329.75 / (108.55) | 330.05 / (110.75) |
| 108.55 / 329.75 | 329.90 / (108.50) | 329.60 / (110.50) |
| 108.70 / 330.50 | 330.35 / (108.75) | 330.65 / (110.95) |
| 108.75 / 330.35 | 330.50 / (108.70) | 330.20 / (110.70) |
| 108.90 / 329.30 | 329.15 / (108.95) | 329.45 / (110.55) |
| 108.95 / 329.15 | 329.30 / (108.90) | - / (-) |
| 109.10 / 331.40 | 331.25 / (109.15) | 331.55 / (111.15) |
| 109.15 / 331.25 | 331.40 / (109.10) | 331.10 / (111.90) |
| 109.30 / 332.00 | 331.85 / (109.35) | 332.15 / (111.35) |
| 109.35 / 331.85 | 332.00 / (109.30) | 331.70 / (111.10) |
| 109.50 / 332.60 | 332.45 / (109.55) | 332.75 / (111.55) |
| 109.55 / 332.45 | 332.60 / (109.50) | 332.30 / (111.30) |
| 109.70 / 333.20 | 333.05 / (109.75) | 333.35 / (111.75) |
| 109.75 / 333.05 | 333.20 / (109.70) | 332.90 / (111.50) |
| 109.90 / 333.80 | 333.65 / (109.95) | 333.95 / (108.35) |
| 109.95 / 333.65 | 333.80 / (109.90) | 333.50 / (111.70) |
| 110.10 / 334.40 | 334.25 / (110.15) | 334.55 / (108.15) |
| 110.15 / 334.25 | 334.40 / (110.10) | 334.10 / (108.30) |
| 110.30 / 335.00 | 334.85 / (110.35) | - / (-) |
| 110.35 / 334.85 | 335.00 / (110.30) | 334.70 / (108.10) |
| 110.50 / 329.60 | 329.45 / (110.55) | 329.75 / (108.55) |
| 110.55 / 329.45 | 329.60 / (110.50) | 329.30 / (108.90) |
| 110.70 / 330.20 | 330.05 / (110.75) | 330.35 / (108.75) |
| 110.75 / 330.05 | 330.20 / (110.70) | 329.90 / (108.50) |
| 110.90 / 330.80 | 330.65 / (110.95) | 330.95 / (111.95) |
| 110.95 / 330.65 | 330.80 / (110.90) | 330.50 / (108.70) |
| 111.10 / 331.70 | 331.55 / (111.15) | 331.85 / (109.35) |
| 111.15 / 331.55 | 331.70 / (111.10) | 331.40 / (109.10) |
| 111.30 / 332.30 | 332.15 / (111.35) | 332.45 / (109.55) |
| 111.35 / 332.15 | 332.30 / (111.30) | 332.00 / (109.30) |
| 111.50 / 332.90 | 332.75 / (111.55) | 333.05 / (109.75) |
| 111.55 / 332.75 | 332.90 / (111.50) | 332.60 / (109.50) |
| 111.70 / 333.50 | 333.35 / (111.75) | 333.65 / (109.95) |
| 111.75 / 333.35 | 333.50 / (111.70) | 333.20 / (109.70) |
| 111.90 / 331.10 | 330.95 / (111.95) | 331.25 / (109.15) |
| 111.95 / 330.95 | 331.10 / (111.90) | 330.80 / (110.90) |

    - The ILS co-ordinates provided shall be those of the localizer antenna system.

    - The calculation of the required separations distances may be simplified by the following procedures (see figures below):

      - If the radiation direction (runway) is known, then the area where no co-channel localizer should be located may be defined as a circle with radius 92.5 NM (93 NM) around a point on the front course line 12.5 NM from the localizer antenna.

Note:	If the radiation direction (runway) is unknown, then a circle with a radius of 105 NM centred at the localizer antenna position defines the area where no co-channel localizer should be located.

      - If the radiation direction (runway) is known, then the area where no adjacent channel localizer should be located may be defined as a circle with radius 17.5 NM (18 NM) around a point on the front course line 12.5 NM from the localizer antenna.

Note:	If the radiation direction (runway) is unknown, then a circle with a radius of 30 NM centred at the localizer antenna position defines the area where no adjacent channel localizer should be located.

      - If the radiation direction (runway) is known, then the area where no adjacent channel glide path should be located may be defined as a circle with radius 7 NM around a point on the front course line 6 NM from the localizer antenna (assuming that the distance between the two antenna systems is 2 NM).

Note:	If the radiation direction (runway) is unknown, then a circle with a radius of 13 NM centred at the localizer antenna position defines the area where no adjacent channel glide path should be located.

ILS protection areas

Localizer

Glide path

## VOR

  - General

    - The band 108 - 112 MHz is used by ILS Localizer and VOR, on different channels, while the band 112 - 118 MHz is used only by VOR. Within the band 108 - 112 MHz, all channels on even multiples of 100 kHz (108.20, 108.40, ...) are used for VOR. Normally channels spaced 100 kHz are used, however, in congested areas also the intermediate channels (50 kHz) within this sub-band may be used ending in even tenths plus a twentieth of a Megahertz. The VOR channels in this sub-band are usually used for terminal VOR, with short range. Within the 112 - 118 MHz portion of the band all channels (100 and 50 kHz) are available for VOR services.

Note : In the EUR region, for the avoidance of harmful interference, it is not permitted for a VOR facility to operate, even on a temporary basis, on a frequency allotted to ILS localizers.

    - References to documents:

-	Annex 10, Volume I, section 3.3

(technical and operational specifications for VOR, in particular frequency band and channel spacing);

-	Annex 10, Volume V, sections 4.2.1 and 4.2.3

(available frequencies for VOR and frequency planning);

-	Annex 10, Attachment C to Volume I, section 3.1

(minimum field strength and coverage);

- Doc. 9718 Vol. II, section 4.4

(protection requirements for VOR versus other VOR or versus ILS localizer);

-	European Region Air Navigation Plan (Doc 7754), Volume I, Part III, paragraph 2.9 (foundations of frequency assignment planning);

-	European Region Air Navigation Plan (Doc 7754), Volume II, Part III, paragraph 2.47.

(application of 50 kHz channel spacing).

    - The DOC of a VOR can typically vary from 25 NM to approximately 100 NM and the protected altitude up to between 10 000 feet and 60 000 feet. Any unwanted signal must be at least 20 dB lower than the signal from the wanted station. Sectorization of the coverage may be used, in this case the range is different in different directions according to a system described in Appendix A to Part III. Information of the designated operational coverage of each station is available from Table NAV and published accordingly in the national AIP. Both the co-channel and the first adjacent channels on each side are considered in the planning.

    - The co-ordination should also take into account the risk of interference from broadcasting stations in the band 87.5-108 MHz, due to interference caused by unwanted emissions into the aeronautical band or generated in the airborne receiver. Rules for this are available from the Recommendation ITU-R SM.1009-1 “Compatibility between the sound-broadcasting service in the band of about 87 - 108 MHz and the aeronautical services in the band 108 - 118 MHz”.

    - Co-ordination should as a minimum be made with States within a radius of approximately 400 - 500 NM, and also be made with the authority responsible for the analysis of the broadcasting versus aeronautical situation.

    - Information on the planning of identifications can be found in section 7 below.

  - Basic Principles

    - Frequencies for VOR facilities should be selected at 50 kHz points, in accordance with Annex 10, Volume I, paragraph 3.3.2.1 and Volume V, paragraphs 4.2.1, 4.2.3 and 4.2.3.1. The associated DME channels should be selected from Table A of Chapter 3 of Annex 10, Volume I.

    - Where the designated operational range of a given station is not the same throughout 360°, the angular limits of sectorization in range should be indicated in accordance with the method described in Appendix A to Part III.

  - Frequency Assignment Planning Criteria

    - Introduction

Note: 113.250 MHz is reserved for possible future use as a VDL Mode 4 CSC. Assignments of VOR on this frequency are not permitted until further notice.

      - The 1976 EUR FCB Meeting agreed that, for VOR frequency planning purposes, the attached Tables should be used in replacement of existing material, for the calculation of geographical separation between VOR in the co-channel and adjacent channel cases in the EUR Region (cf. Resolution No. 10).

      - These tables take into account, as appropriate:

a)	The fact that 20 dB has been retained by the EUR FCB as the planned minimum D/U signal strength ratio, within the protected coverage of EUR VOR facilities;

b)	The relative VOR effective radiated power, covering power ratios up to 100 (i.e. ±20 dB), so that account can be taken of VORs whose EIRP varies, for example, from 200 W to 2 W;

c)	The adjacent channel protection ratio corresponding to the particular ground and airborne equipment situations.

      - As the geographical separations have been calculated on the basis of a typical ground VOR antenna and its vertical directivity taken into account, it is only necessary in the case of VOR facilities having typical ground VOR antennas to specify the total radiated power. The pertinent characteristics of the assumed ground VOR antenna are given in 3.3.2.2 and such characteristics are typical for range calculation purposes of existing known VOR ground facilities having single dipoles located above the counterpoise and for which the elevation angle of maximum radiation is about 25 to 30 degrees.

      - In the case of any facilities where the vertical pattern substantially differs from that of a single dipole (i.e. where a two-element antenna may be used to enhance the lower angle radiation), the radiated power could be suitably adjusted for the angles of elevation of most interest. A factor of 3 to 4 dB would appear to be appropriate, i.e. for geographical separation purposes a VOR with a two-element antenna and having a total radiated power of 25 W would be considered as having a total radiated power of 50/65 W.

      - Criteria for geographical separation between ILS and VOR facilities operating in the band 108 - 112 MHz on adjacent channels can be found in section 2.3.3, Part III of this manual (material for ILS).

    - Desired to Undesired Signal Ratio Contours

      - If two VOR facilities of equal radiated power are separated in distance, it is possible to construct desired/undesired facility signal ratio contours. Figure 1 shows the contours that will be obtained when the desired facility is at 0 NM and the undesired facility at 371 km (200 NM) (i.e. a geographical separation of 200 NM). The 20 dB contour is relevant to the co-channel case, the -34 dB contour relevant to the 50 kHz adjacent channel case (9960 harmonics in conformity with Annex 10, Vol.I), and the -20 dB contour relevant to the 50 kHz adjacent channel case (9960 Hz harmonics not in conformity with Annex 10, Vol.I). For other geographical separation distances the D/U ratio contours follow a similar pattern.

      - Also shown on Figure 1 is a "nominal range curve". The nominal range curve shows the range and altitude at which a power density of -113 dBW/m2 will be obtained at 108 MHz for a radiated power of 100 W (20 dBW), the ground VOR facility being assumed to have a circular horizontal pattern and a vertical pattern similar to a dipole 1.22 m (4 ft) above a 16 m (52 ft) diameter counterpoise which in turn is assumed to be 4.83 m (16 ft) above ground level. Under favourable conditions (e.g. path, airborne receiver sensitivity greater than that assumed), the range obtainable may exceed the "nominal range" by a factor of 1.15 or so. This greater range approximates to the horizon range based on a true earth radius.

      - The following deductions can be made from Figure 1:

a)	+20 dB Contour. This is applicable to the co-channel case. For the geographical separation distance of 371 km (200 NM), it would be possible to have cylindrical service volumes of 93 km (50 NM), 11278 m (37000 ft) or 139 km (75 NM), 3353 m (11000 ft). It will be noted that in the co-channel case, because of the direction of the +20 dB contour, the highest altitude to be protected is the controlling factor in determining the geographical separation distance.

b)	-20 and -34 dB contours. These are applicable to adjacent channel cases. Because the direction of the slope of the contour is the opposite of the co-channel case, the lowest altitude to be protected is the controlling factor. For a given distance separation, it is only necessary to protect those altitudes above the "horizon" curve because at lower altitudes there will be insufficient signal level to reliably actuate the airborne receiver (see 3.3.2.2 above).

    - The Co-channel Case

      - The geographical separation distances, at Tables 1 to 9 inclusive, have been derived by the application of a methodology that used to contained in the  Attachment C to Annex 10, Vol.I.  This material was removed from Annex 10, Vol.I by virtue of the amendment 94 to Annex 10, Vol.I. Related to the application of the geographical separation distances derived by this methodology, Attachment C to Annex 10, Vol.I contained the caveat that such  application will only be correct within the limitations set by the assumptions of the methodology. The assumptions include that the change of field strength with distance (Factor "S") at various altitudes of reception is only valid for angles of elevation at the VOR of up to about 5 degrees, but above the radio line of sight. In practice account need only be taken of this limitation for short range/high altitude facilities, where the altitude component is 15240 m (50000 ft) and above, and long range/high altitude facilities, where the altitude component is 12192 m (40000 ft) and above. Increased geographical separation distances (marked + in the tables) are recommended to assure adequate protection for the combinations specified, i.e. where the elevation angle exceeds 5 degrees. Decreased geographical separation distances (marked *) may be used for the combinations specified due to the higher rate of change of field strength (Factor "S") at or near the horizon.

Example:

Facility 1:	DOC 25 NM / 10000 ft, 100 W (20 dBW) EIRP

Facility 2:	DOC 100 NM / 50000 ft 400 W (26 dBW) EIRP

D/U = 20 dB

DB1B = 25 NM

DB2B = 100 NM

K = -6 dB (20 dBW - 26 dBW)

S (10000 ft) = 0.43 dB/NM (see paragraph 3.3.3.2 below)

S (50000 ft) = 0.18 dB/NM (see paragraph 3.3.3.2 below)

Separation distance: the greater of	2DB1B + (D/U-K)/S	or	2DB2B + (D/U+K)/S:

2*25+(20+6)/0.43 NM= 110 NM or 2*100+(20-6)/0.18 NM = 278 NM

Required minimum separation distance = 278 NM

      - The following table  supplies values of the factor S (rate of change of the field strength in dB with distance) for different altitudes:

| Altitude m (ft) | S dB/km (NM) |
| --- | --- |
| 1 200 (4 000) | 0.32 (0.60) |
| 3 000 (10 000) | 0.23 (0.43) |
| 4 500 (15 000) | 0.18 (0.34) |
| 6 000 (20 000) | 0.15 (0.29) |
| 7 500 (25 000) | 0.13 (0.25) |
| 9 000 (30 000) | 0.12 (0.23) |
| 12 000 (40 000) | 0.10 (0.19) |
| 18 000 (60 000) | 0.09 (0.17) |

Note: The above information was retrieved from Attachment C to Annex 10, Vol. I, prior to its removal by amendment 94.

    - The Adjacent-Channel Case

      - Tables 10 and 11 give the necessary geographical separation of adjacent-channel facilities for the following cases:

a)  50 kHz channel spacing, harmonic levels low (Table 10)

b)  50 kHz channel spacing, harmonic levels high (Table 11)

The undesired facility should always be outside the DOC of the desired facility.

      - In each table, the necessary geographical separation has been derived from the formula:

      - Separation distance from desired facility=2D+(D/U-K)/S,

where:

D is the range of the desired facility,

S is the value of the S factor at the lowest serviceable altitude at the range of the desired facility, and

K=EIRP of desired facility – EIRP of undesired facility,

      - in accordance with the method given in 3.3.4.3 below.

      - The derivation of adjacent channel geographical separation distances requires that account be taken of both the range and the lowest altitude at which the provision of the necessary desired/undesired (D/U) field strength ratio is required. The lowest altitude at which protection is to be provided is assumed to be that height corresponding to the horizon distance (based on true earth radius) for the range in question, experience having shown that this horizon distance is the normal maximum range achievable (see 3.3.2.2 and 3.3.2.3 b) above).

The horizon distance for a true earth radius is given by the formula:

D =

where H is in feet, D in NM.

This formula can be re-arranged as follows :

H = 0.9 D2

      - The following Table gives for the ranges specified, values of H together with corresponding S values to be inserted in the adjacent-channel geographical separation formula (as in the example below):

| D km(NM) | H m (ft) | S (dB/NM) |
| --- | --- | --- |
| 46 km (25) | 171 m (560) | 1.0 |
| 74 km (40) | 439 m (1 440) | 1.0 |
| 111 km (60) | 987 m (3 240) | 0.65 |
| 148 km (80) | 1 756 m (5 760) | 0.55 |
| 185 km (100) | 2 743 m (9 000) | 0.44 |
| 278 km (150) | 6 172 m (20 250) | 0.28 |
| 333 km (180) | 8 888 m (29 160) | 0.23 |
| 380 km (205) | 11 521 m ( 37 800) | 0.20 |
| 426 km (230) | 14 511 m (47 610) | 0.18 |

The above values D and S have been used in the derivation of geographical separation distances for the adjacent-channel case. . For low harmonic levels, an adjacent-channel D/U ratio of -34 dB was assumed, whereas for high harmonic levels, the adjacent-channel D/U ratio was taken to be -20 dB.

Example:

Facility 1:	DOC 25 NM / 10000 ft, 100 W (20 dBW) EIRP

Facility 2:	DOC 100 NM / 50000 ft 400 W (26 dBW) EIRP

D/U = -34 dB (Table 4-4 of Doc 9718, Vol.II)

D1 = 25 NM

D2 = 100 NM

K = -6 dB (20 dBW - 26 dBW)

S (25 NM) = 1.00 dB/NM (Paragraph 3.3.4.4)

S (100 NM) = 0.44 dB/NM (Paragraph 3.3.4.4)

Separation distance: the greater of	 2D1 - (34+K)/S	or	2D2 - (34-K)/S:

2*25-(34-6)/1.00 NM= 22 NM or 2*100-(34+6)/0.44 NM = 109 NM

Required minimum separation distance:		109 NM.

    - Sectorization

3.3.5.1 Application of the calculation methods described above may produce incorrect results if sectorization (key-holing) is used. The figure below can be used to explain the problem:

Consider A to be the desired VOR facility and B the undesired one. The “Critical Point” is most exposed to interference from facility B because it is the point of the DOC of facility A which is nearest to transmitter B. Its vulnerability is further enhanced by the distance from facility A which is the maximum possible within the DOC of A.

3.3.5.2 The correct assessment of compatibility for facility A requires the following calculations:

Co-channel:

Minimum separation distance between “Critical Point” and transmitter B:	D1+(20-K)/S;

(this is derived from a subtraction of D1 from 2D1+(20-K)/S).

Adjacent channel:

Minimum separation distance between “ Critical Point” and transmitter B:	D1-(34+K)/S;

(this is derived from a subtraction of D1 from 2D1-(34+K)/S);

where

D1 = range of facility A at the “Critical Point”.

VOR DESIRED/UNDESIRED SIGNAL RATIO CONTOURS

FACILITY SEPARATION 200 NM (K = 0)

EUR VOR GEOGRAPHICAL SEPARATION TABLES

(For circular coverage only)

CONTENTS

| Table 1 | Minimum co-channel geographical separation distances | 21,336 m (70,000 ft) |
| --- | --- | --- |
| Table 2 | Minimum co-channel geographical separation distances | 15,250 m (50,000 ft) |
| Table 3 | Minimum co-channel geographical separation distances | 12,192 m (40,000 ft) |
| Table 4 | Minimum co-channel geographical separation distances | 9,144 m (30,000 ft) |
| Table 5 | Minimum co-channel geographical separation distances | 7,620 m (25,000 ft) |
| Table 6 | Minimum co-channel geographical separation distances | 6,096 m (20,000 ft) |
| Table 7 | Minimum co-channel geographical separation distances | 4,572 m (15000 ft) |
| Table 8 | Minimum co-channel geographical separation distances | 3,048 m (10,000 ft) |
| Table 9 | Minimum co-channel geographical separation distances | 1,524 m (5,000 ft) |
| Table 10 | Minimum adjacent channel geographical separation distances | 50 kHz channel spacing (harmonic levels low) |
| Table 11 | Minimum adjacent channel geographical separation distances | 50 kHz channel spacing (harmonic levels high) |

Table 1

| Desired coverage |  | Minimum co-channel geographical separation in NM to provide protection for the indicated ratios (K) of desired to undesired radiated power |  |  |  |  |  |  | Remarks | Direct ray elevation angle |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Altitude m (ft) | Range NM | +20 dB | +12 dB | +6 dB | 0 dB | -6 dB | -12 dB | -20 dB |  |  |
| 21,336 (70,000) | 25 | 50 | 100 | 125 | 170 | 190 (225)+ | 245 (315)+ | 275 (345)+ |  | 27° |
|  | 40 | 80 | 130 | 155 | 200 (255)+ | 220 (300)+ | 275 (340)+ | 305 (365)+ |  | 15° |
|  | 60 | 120 | 170 | 195 | 240 (325)+ | 260 (350)+ | 315 (375)+ | 345 (390)+ |  | 11° |
|  | 80 | 160 | 210 (270)+ | 235 (325)+ | 280 (360)+ | 300 (380)+ | 355 (400)+ | 385 (415)+ |  | 9° |
|  | 100 | 200 | 250 (295)+ | 275 (350)+ | 320 (380)+ | 340 (410)+ | 395 (425)+ | 425 (435)+ |  | 6° |
|  | 150 | 300 | 350 | 375 | 420 | 440 | 495 | 525 |  | 3.5° |
|  | 230 | 460 | 510 | 535 | 580 (555)* | 600 (560)* | 655 (570)* | 685 (600)* |  | 1.3° |
|  | 280 | 560 | 610 | 635 | 680 (620)* | 700 (625)* | 755 (680)* | 785 (725)* | Horizon | 0.4° |
|  | 315 | 630 | 680 | 705 | 750 | 770 | 825 | 855 | Radio Horizon | 0° |

Table 2

| Desired coverage |  | Minimum co-channel geographical separation in NM to provide protection for the indicated ratios (K) of desired to undesired radiated power |  |  |  |  |  |  | Remarks | Direct ray elevation angle |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Altitude m (ft) | Range NM | +20 dB | +12 dB | +6 dB | 0 dB | -6 dB | -12 dB | -20 dB |  |  |
| 15,240 (50,000) | 25 | 50 | 90 | 125 | 160 | 190 (205)+ | 220 (260)+ | 270 (300)+ |  | 19° |
|  | 40 | 80 | 120 | 155 (190)+ | 190 (235)+ | 220 (270)+ | 250 (320)+ | 300 (335)+ |  | 13° |
|  | 60 | 120 | 160 (210)+ | 195 (250)+ | 230 (290)+ | 260 (310)+ | 290 (330)+ | 340 |  | 9.5° |
|  | 80 | 160 | 200 (225)+ | 235 (275)+ | 270 (310)+ | 300 (335)+ | 330 (355)+ | 380 |  | 6° |
|  | 100 | 200 | 240 | 275 | 310 | 340 | 370 | 420 |  | 4° |
|  | 150 | 300 | 340 | 375 | 410 | 440 (420)* | 470 (428)* | 520 (435)* |  | 2° |
|  | 205 | 410 | 450 | 485 | 520 (490)* | 550 (495)* | 580 (505)* | 630 (560)* |  | 1° |
|  | 240 | 480 | 520 | 555 (525)* | 590 (535)* | 620 (540)* | 650 (600)* | 700 (655)* | Horizon | 0.4° |
|  | 270 | 540 | 580 (565)* | 615 (570)* | 650 (620)* | 680 (650)* | 710 (680)* | 760 (730)* | Radio Horizon | 0° |

Table 3

| Desired coverage |  | Minimum co-channel geographical separation in NM to provide protection for the indicated ratios (K) of desired to undesired radiated power |  |  |  |  |  |  | Remarks | Direct ray elevation angle |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Altitude m (ft) | Range NM | +20 dB | +12 dB | +6 dB | 0 dB | -6 dB | -12 dB | -20 dB |  |  |
| 12.592 | 25 | 50 | 90 | 125 | 155 | 190 | 220 | 260 |  | 15° |
| (40.000) | 40 | 80 | 120 | 155 | 185 | 220 | 250 | 290 |  | 9° |
|  | 60 | 120 | 160 | 195 | 225 | 260 | 290 | 330 |  | 6° |
|  | 80 | 160 | 200 | 235 | 265 | 300 | 330 | 370 |  | 4.2° |
|  | 100 | 200 | 240 | 275 | 305 | 340 | 370 | 410 |  | 3° |
|  | 150 | 300 | 340 | 375 | 405 | 440 | 470 | 510 |  | 1.5° |
|  | 180 | 360 | 400 | 435 (425)* | 465 (435)* | 500 (440)* | 530 (450)* | 570 (485)* |  | 0.9° |
|  | 210 | 420 | 460 | 495 (470)* | 525 (475)* | 560 (480)* | 590 (535)* | 630 (580)* | Horizon | 0.4° |
|  | 245 | 490 | 530 (515)* | 565 (520)* | 595 (560)* | 630 (600)* | 660 (635)* | 700 (680)* | Radio horizon | 0° |

Table 4

| Desired coverage |  | Minimum co-channel geographical separation in NM to provide protection for the indicated ratios (K) of desired to undesired radiated power |  |  |  |  |  |  | Remarks | Direct ray elevation angle |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Altitude m (ft) | Range NM | +20 dB | +12 dB | +6 dB | 0 dB | -6 dB | -12 dB | -20 dB |  |  |
| 9.144 | 25 | 50 | 85 | 110 | 140 | 165 | 190 | 225 |  | 11° |
| (30.000) | 40 | 80 | 115 | 140 | 170 | 195 | 220 | 235 |  | 7° |
|  | 60 | 120 | 165 | 180 | 210 | 235 | 260 | 275 |  | 4.5° |
|  | 80 | 160 | 205 | 220 | 250 | 275 | 300 | 315 |  | 3° |
|  | 100 | 200 | 245 | 260 | 290 | 315 | 340 | 355 |  | 2.3° |
|  | 150 | 300 | 345 | 360 | 390 | 415 | 440 | 455 |  | 0.9° |
|  | 165 | 330 | 375 | 390 | 420 | 445 | 470 | 485 |  | 0.6° |
|  | 190 | 380 | 425 | 440 | 470 | 495 | 520 | 535 | Horizon | 0.3° |
|  | 210 | 420 | 465 | 480 | 510 | 535 | 560 | 575 | Radio Horizon | 0° |

Table 5

| Desired coverage |  | Minimum co-channel geographical separation in NM to provide protection for the indicated ratios (K) of desired to undesired radiated power |  |  |  |  |  |  | Remarks | Direct ray elevation angle |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Altitude m (ft) | Range NM | +20 dB | +12 dB | +6 dB | 0 dB | -6 dB | -12 dB | -20 dB |  |  |
| 7,620 | 25 | 50 | 80 | 105 | 130 | 155 | 180 | 210 |  | 9° |
| (25,000) | 40 | 80 | 110 | 135 | 160 | 185 | 210 | 240 |  | 6° |
|  | 60 | 120 | 150 | 175 | 200 | 225 | 250 | 280 |  | 3.8° |
|  | 80 | 160 | 190 | 215 | 240 | 265 | 290 | 320 |  | 2.5° |
|  | 100 | 200 | 230 | 255 | 280 | 305 | 330 | 360 |  | 1.8° |
|  | 150 | 300 | 330 | 355 | 380 | 405 | 430 | 460 |  | 0.6° |
|  | 170 | 340 | 370 | 395 | 420 | 445 | 470 | 500 | Horizon | 0.4° |
|  | 195 | 390 | 420 | 445 | 470 | 495 | 520 | 550 | Radio Horizon | 0° |

Table 6

| Desired coverage |  | Minimum co-channel geographical separation in NM to provide protection for the indicated ratios (K) of desired to undesired radiated power |  |  |  |  |  |  | Remarks | Direct ray elevation angle |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Altitude m (ft) | Range NM | +20 dB | +12 dB | +6 dB | 0 dB | -6 dB | -12 dB | -20 dB |  |  |
| 6,096 | 25 | 50 | 80 | 90 | 110 | 130 | 160 | 190 |  | 7° |
| (20,000) | 40 | 80 | 110 | 123 | 140 | 160 | 190 | 220 |  | 4.5° |
|  | 60 | 120 | 150 | 160 | 180 | 200 | 230 | 260 |  | 3° |
|  | 80 | 160 | 190 | 200 | 220 | 240 | 270 | 300 |  | 2° |
|  | 100 | 200 | 230 | 240 | 260 | 280 | 310 | 340 |  | 1.3° |
|  | 135 | 270 | 300 | 310 | 330 | 350 | 380 | 410 |  | 0.6° |
|  | 155 | 310 | 340 | 350 | 370 | 390 | 420 | 450 | Horizon | 0.2° |
|  | 175 | 350 | 380 | 390 | 410 | 430 | 460 | 490 | Radio Horizon | 0° |

Table 7

| Desired coverage |  | Minimum co-channel geographical separation in NM to provide protection for the indicated ratios (K) of desired to undesired radiated power |  |  |  |  |  |  | Remarks | Direct ray elevation angle |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Altitude m (ft) | Range NM | +20 dB | +12 dB | +6 dB | 0 dB | -6 dB | -12 dB | -20 dB |  |  |
| 4,572 | 25 | 50 | 75 | 90 | 110 | 130 | 145 | 170 |  | 6° |
| (15,000) | 40 | 80 | 105 | 120 | 140 | 160 | 175 | 200 |  | 3° |
|  | 60 | 120 | 145 | 160 | 180 | 200 | 215 | 240 |  | 2° |
|  | 80 | 160 | 185 | 200 | 220 | 240 | 265 | 280 |  | 1.3° |
|  | 100 | 200 | 225 | 240 | 260 | 280 | 305 | 320 |  | 0.8° |
|  | 120 | 240 | 265 | 280 | 300 | 320 | 345 | 360 |  | 0.5° |
|  | 140 | 280 | 305 | 320 | 340 | 360 | 385 | 400 | Horizon | 0.2° |
|  | 155 | 310 | 335 | 350 | 370 | 390 | 415 | 430 | Radio Horizon | 0° |

Table 8

| Desired coverage |  | Minimum co-channel geographical separation in NM to provide protection for the indicated ratios (K) of desired to undesired radiated power |  |  |  |  |  |  | Remarks | Direct ray elevation angle |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Altitude m (ft) | Range NM | +20 dB | +12 dB | +6 dB | 0 dB | -6 dB | -12 dB | -20 dB |  |  |
| 3,048 | 25 | 50 | 70 | 85 | 95 | 110 | 125 | 145 |  | 4° |
| (10,000) | 40 | 80 | 100 | 115 | 125 | 140 | 155 | 175 |  | 2° |
|  | 60 | 120 | 140 | 155 | 165 | 180 | 195 | 215 |  | 1.3° |
|  | 80 | 160 | 180 | 195 | 210 | 220 | 235 | 255 |  | 0.7° |
|  | 100 | 200 | 220 | 235 | 245 | 260 | 275 | 295 |  | 0.4° |
|  | 115 | 230 | 250 | 265 | 275 | 290 | 305 | 323 | Horizon | 0.2° |
|  | 130 | 260 | 280 | 295 | 305 | 320 | 335 | 355 | Radio Horizon | 0° |

Table 9

| Desired coverage |  | Minimum co-channel geographical separation in NM to provide protection for the indicated ratios (K) of desired to undesired radiated power |  |  |  |  |  |  | Remarks | Direct ray elevation angle |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Altitude m (ft) | Range NM | +20 dB | +12 dB | +6 dB | 0 dB | -6 dB | -12 dB | -20 dB |  |  |
| 1,524 | 25 | 50 | 65 | 75 | 85 | 95 | 105 | 115 |  | 2° |
| (5,000) | 40 | 80 | 95 | 105 | 115 | 125 | 135 | 145 |  | 1.2° |
|  | 60 | 120 | 135 | 145 | 155 | 165 | 175 | 185 |  | 0.5° |
|  | 75 | 150 | 165 | 175 | 185 | 195 | 205 | 215 |  | 0.2° |
|  | 85 | 170 | 185 | 195 | 205 | 215 | 225 | 235 | Horizon | 0.1° |
|  | 95 | 190 | 205 | 215 | 225 | 235 | 245 | 255 | Radio Horizon | 0° |

Table 10

| Range NM | Minimum adjacent channel geographical separation in NM to provide protection for the indicated ratios (K) of desired to undesired radiated power (harmonic levels low) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | +20 dB | +12 dB | +6 dB | 0 dB | -6 dB | -12 dB | -20 dB |
| 25 | 25 | 25 | 25 | 25 | 25 | 28 | 36 |
| 40 | 40 | 40 | 40 | 45 | 52 | 57 | 65 |
| 60 | 60 | 60 | 60 | 67 | 77 | 86 | 100 |
| 80 | 80 | 80 | 87 | 97 | 110 | 120 | 135 |
| 100 | 100 | 100 | 109 | 125 | 135 | 150 | 170 |
| 150 | 150 | 150 | 157 | 180 | 200 | 220 | 250 |
| 180 | 180 | 180 | 186 | 210 | 240 | 265 | 300 |
| 205 | 205 | 205 | 210 | 240 | 270 | 300 | 340 |
| 230 | 230 | 230 | 238 | 270 | 305 | 340 | 380 |

Table 11

| Range NM | Minimum adjacent channel geographical separation in NM to provide protection for the indicated ratios (K) of desired to undesired radiated power (harmonic levels high) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | +20 dB | +12 dB | +6 dB | 0 dB | -6 dB | -12 dB | -20 dB |
| 25 | 25 | 25 | 25 | 30 | 36 | 42 | 50 |
| 40 | 40 | 48 | 54 | 60 | 66 | 72 | 80 |
| 60 | 60 | 70 | 80 | 90 | 98 | 108 | 120 |
| 80 | 87 | 102 | 113 | 125 | 135 | 145 | 160 |
| 100 | 109 | 127 | 140 | 155 | 168 | 182 | 200 |
| 150 | 157 | 185 | 207 | 230 | 250 | 272 | 300 |
| 180 | 186 | 220 | 245 | 273 | 300 | 325 | 360 |
| 205 | 210 | 250 | 280 | 310 | 340 | 370 | 410 |
| 230 | 238 | 282 | 315 | 350 | 382 | 415 | 460 |

## GBAS

  - General

    - The GNSS ground-based augmentation system (GBAS) VHF data broadcast (VDB) frequency band is 108 to 117.975 MHz. The lowest assignable frequency is 108.025 MHz and the highest assignable frequency is 117.950 MHz – channel spacing is 25 kHz. This band is also used for ILS (below 112 MHz) and for VOR. GBAS VDB applies a time division multiple access (TDMA) technique with 8 time slots which are repeated every 0.5 seconds. The signal timing is synchronised with UTC.

Note: 113.250 MHz is reserved for possible future use as a VDL Mode 4 CSC. Assignments of GBAS on the frequencies 113.225, 113.250 and 113.275 MHz are not permitted until further notice.

    - References to documents:

-	Annex 10, Volume I, paragraph 3.7.3.5

(coverage, carrier frequency, channelling, field strength);

-	Annex 10, Appendix B to Volume I, paragraph 3.6.8.2.2.5-7

(co- and adjacent channel rejection requirements);

-	Doc 9718 Vol. II, section 6.6

(protection requirements for GBAS VDB);

-	Doc 9718 Vol. II, section 3.5

(protection requirements for ILS localizer versus GBAS VDB);

-	Doc 9718 Vol. II, section 4.5

(protection requirements for VOR versus GBAS VDB);

-	Annex 10, Attachment D to Volume I, section 7.9

( assignment of reference path data selector and reference station data selector);

-	Annex 10, Volume V, paragraph 4.2

(use of the band 108 – 117.975 MHz).

Note: The protection requirements and separation distances presented in this manual for GBAS VDB from other GBAS VDB or from other systems in the same or adjacent bands are in general different from those contained in ICAO Doc 9718, Vol. II.

    - The GBAS precision approach coverage is defined relative to the runway. However, it is recommended to use an omnidirectional designated operational coverage (DOC) with a range of 23 NM and up to a height of 10000 ft above threshold. The GBAS positioning service coverage is dependent upon the specific operations intended. Undesired signals from co-channel GBAS or VOR stations must be at least 26 dB below the desired GBAS signal. Adjacent channels up to ±100 kHz must be considered in the compatibility assessment.

    - The risk of interference from FM broadcasting stations in the band 87.5 – 108 MHz caused by unwanted emissions into the aeronautical band or generated in the airborne receiver should be taken into account. The relevant Recommendation for GBAS is ITU-R SM.1841.

    - Co-ordination should be made with States within a radius of about 500 NM and also with the authority responsible for compatibility between FM broadcasting and aeronautical radio navigation services.

    - Besides the frequency, co-ordination must also take into account reference path data selectors and reference station data selectors (RPDS and RSDS) and time slots.

    - Information on the planning of identifications can be found in section 7 below.

    - The prerequisite for any D/U calculation to be valid is that the minimal receive power is achieved at all points throughout the coverage.

    - Beyond Radio Line Of Sight (BRLOS) attenuation for GBAS signals BRLOS is 0.72/NM dB for the first 30 NM and 0.14 dB/NM  thereafter, as extracted from  ITU-R Recommendation P.528-2 125 MHz, 50% curves A and C. The BRLOS shall be calculated using the real site elevation of the antenna phase center.

The radio horizon distance (denoted by RBH1 in the above figure) of a receiving point P is given by:

RH1=1.23* h

Where:

RBH1 B= radio horizon distance of receiving point P in nautical miles (NM)

h = height of receiving point P in feet (ft) above Mean Sea Level

The radio horizon distance of the unwanted GBAS facility (RH2) is calculated by the same formula as above:

RH2=1.23*hGBAS unwanted

Where:

RH2 = radio horizon distance of unwanted GBAS facility in nautical miles (NM).

h GBAS-unwanted = height of GBAS antenna phase center in (ft) above Mean Sea Level

Two points are considered to be within radio line-of-sight (RLOS) if their separation distance is smaller or equal than the sum of their respective radio horizon distances.

Rs≤RRLOS= RH1+RH2

Two points are considered to be beyond radio line-of-sight (BRLOS) if their separation distance is greater than the sum of their respective radio horizon distances.

Rs>RRLOS  , with    RS=RH1+RH2+RE

Where:

Rs = separation distance in NM between point P and the undesired GBAS facility.

RBRLOS B= radio line-of-sight distance in NM, given the heights of point P and the unwanted GBAS facility (= sum of radio horizon distances from height P and hGBAS-unwanted).

RBE B= propagation distance in NM covered by diffraction (= radio path distance separating point P from the GBAS facility minus RRLOS)

    - The EIRP of a GBAS system shall be provided as Peak Envelope Power. Since MOPS specify the output power to be measured as mean power over a bit-sequence in the preamble, care must be taken that the PEP are used for calculation!

EIRP PEP[dBW] = P Transmitter-Output [dBW] - A Cable loss [dBW] + G Antenna gain [dBi]

    - Polarization of GBAS systems for civil aviation purposes shall use horizontal polarization (=GBAS/H).

Note: Exceptionally, for providing service to military aircraft, elliptical polarization (GBAS/E) could also be used. However, the applicable planning criteria of the following table do not cover this case.

    - Since only horizontal polarization is assumed to be in operation in the European Region, the consideration of cross polar isolation between a purely horizontal polarized GBAS system and a vertical polarized system, like VHF-COM will provide significantly reduced separation requirements. The temporary value for cross polarization isolation between horizontal and vertical polarization is assumed to be 10 dB.

    - To allow a use of Frequency Management Programs, temporary values for calculation are given in the table below. The appropriate GBAS vertical and horizontal polarization and the respective EIRP PEP have to be available for calculation. The temporary separation requirements based on EIRP PEP, polarization and system GBAS, ILS and VHF-COM, is provided in the table below.

| GBAS/H |  | VHF-COM | VDL-2 | VDL-3 | VDL-4 | ILS | VOR | GBAS/H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| h-pol | polarization | v-pol | v-pol | v-pol | v-pol | h-pol | h-pol | h-pol |
| 25 kHz | channelization | 8,33 & 25 kHz | 25 kHz | 25 kHz | 25 kHz | 50 kHz | 50 kHz | 25 kHz |
|  | EIRP | air 1 dBW gnd 19 dBW |  |  |  | No recommend-dations for EIRP | 17 dBW | 17 dBW |
| ±0 kHz co-chn |  |  |  |  |  | D/U,+10dB safety margin, ( x-pol.-loss-isolation) | D/U, ( x-pol.-loss-isolation) | D/U, ( x-pol.-loss-isolation) |
|  |  | not applicable |  |  |  |  |  |  |
|  |  |  |  |  |  | 26 +10 = 36 | 26 | 26 |
| ±25 kHz 1.adj.-chn. |  |  |  |  |  | D/U,+6dB safety margin, ( x-pol.-loss-isolation) | D/U, ( x-pol.-loss-isolation) | D/U, ( x-pol.-loss-isolation) |
|  |  | not applicable |  |  |  | 0 +6 = 6 | 0 | -18 |
| ±50 kHz 2.adj.-chn. |  | D/U,+6dB safety margin, ( x-pol.-loss-isolation) |  |  |  | D/U,+6dB safety margin, ( x-pol.-loss-isolation) | D/U, ( x-pol.-loss-isolation) | D/U, ( x-pol.-loss-isolation) |
|  |  | -53 +6(-10) = -57 |  |  |  | -34 +6 = -28 | -34 | -43 |
| ±75 kHz 3.adj.-chn. |  | D/U,+6dB safety margin, ( x-pol.-loss-isolation) |  |  |  | D/U,+6dB safety margin, ( x-pol.-loss-isolation) | D/U, ( x-pol.-loss-isolation) | D/U, ( x-pol.-loss-isolation) |
|  |  | -60 +6(-10) = -64 |  |  |  | -46 +6 = -40 | -46 | -46 |
| ±100 kHz 4.adj.-chn. |  | D/U,+6dB safety margin, ( x-pol.-loss-isolation) |  |  |  | D/U,+6dB safety margin, ( x-pol.-loss-isolation) | D/U, ( x-pol.-loss-isolation) |  |
|  |  | -63+6(-10) = -67 |  |  |  | -65 +6 = -59 | -65 | no restriction |
| ±125 kHz and beyond |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | no restriction | no restriction | no restriction |

The table is based on the following assumptions: - FMG/10 decided the standard polarization of GBAS in Europe to be horizontal. Therefore no D/U calculation criteria for vertical polarisation (GBAS/E) are available. - GBAS has similar characteristics to VDL-M2. - For ILS, a safety margin of 10 dB was added for co-channel and 6 for adjacent channel - Cross-polar-isolation of 10 dB . Acronyms: GBAS/E-h; GBAS elliptical polarization - horizontal component. GBAS/E-v; GBAS elliptical polarization - vertical component. GBAS/E; GBAS elliptical polarization. GBAS/H; GBAS horizontal polarization. adj.; adjacent channel. Chn.; channel.

  - GBAS to GBAS Frequency Assignment Planning Criteria

For a more refined calculation considering EIRP PEP separated by polarization shall be used as provided in the planning criteria Table in 4.1.

    - The signal ratios between desired and undesired signals (D/U) to protect GBAS precision approach VDB in the presence of an other GBAS station are (Annex 10 SARPs for ±100 kHz and beyond are more conservative):

| Frequency offset | Minimum D/U ratio required |
| --- | --- |
| 0 kHz (co-channel) | 26 dB |
| ±25 kHz (first adjacent channel) | -18 dB |
| ±50 kHz (second adjacent channel) | -43 dB |
| ±75 kHz (third adjacent channel) | -46 dB |
| ±100 kHz and beyond | planning freedom |

Note:	Criteria to achieve compatibility via time slot co-ordination are not presented here.

    - Appendix B to Part III describes the calculation for the determination of the required geographical separation distance. The example separation distances indicated in the table below were the result of calculations assuming a GBAS VDB field strength of 215 μV/m (-99 dBW/mP2P) corresponding to the minimum required field strength at the edge of coverage for the desired facility and of different radiated powers as indicated for the undesired facility, a constant attenuation factor of  0,72 dB/NM for the first 30 NM and 0,14 dB/NM thereafter for beyond radio line-of-sight, a GBAS VDB antenna height of 0 ft (phase centre) and a DOC of 23 NM/10000 ft. Signal polarization was assumed to be horizontal:

| Desired facility: GBAS station (DOC 23 NM /10000 ft) |  |  |  |
| --- | --- | --- | --- |
| Undesired facility: GBAS station |  |  |  |
| Frequency offset | Minimum required geographical separation distance between stations for different radiated powers (EIRP) of the undesired GBAS station |  |  |
|  | 50 W 47 dBm 17 dBW | 80 W 49 dBm 19 dBW | 100 W 50 dBm 20 dBW |
| 0 kHz (co-channel) | 194 NM | 198 NM | 200 NM |
| ±25 kHz (first adjacent channel) | 35 NM | 38 NM | 40 NM |
| ±50 kHz (second adjacent channel) | 24 NM | 24 NM | 24 NM |
| ±75 kHz (third adjacent channel) | 23 NM | 24 NM | 24 NM |
| ±100 kHz and beyond | no restrictions |  |  |

  - GBAS to VOR Frequency Assignment Planning Criteria

    - For a more refined calculation considering EIRP PEP separated by polarization shall be used as provided in the planning criteria Table in 4.1.

    - The signal ratios between desired and undesired signals (D/U) to protect GBAS receivers in the presence of a VOR signal and VOR receivers in the presence of a GBAS signal (Annex 10 SARPs and guidance material for ±100 kHz and beyond are not identical for GBAS – VOR and VOR – GBAS desired to undesired situations and they are more conservative if VOR is the undesired facility):

| Frequency offset | Minimum D/U ratio required |
| --- | --- |
| 0 kHz (co-channel) | 26 dB |
| ±25 kHz (first adjacent channel) | 0 dB |
| ±50 kHz (second adjacent channel) | -34 dB |
| ±75 kHz (third adjacent channel) | -46 dB |
| ±100 kHz (fourth adjacent channel) | -65 dB |
| ±125 kHz and beyond | planning freedom |

    - Appendix B to Part III describes the calculation for the determination of the required geographical separation distance. The example separation distances indicated in the tables below were calculated assuming a VOR field strength of 90 μV/m (-107 dBW/mP2P), corresponding to the minimum required field strength at the edge of coverage for the desired facility, a VOR DOC as indicated in the tables, a constant attenuation factor of  0.72 dB/NM for the first 30 NM and 0,14 dB/NM thereafter for beyond radio line-of-sight, a GBAS VDB and a VOR antenna height of 0 ft. GBAS signal polarization was assumed to be horizontal:

| Desired facility: VOR (DOC 25 NM / 15000 ft) |  |  |  |
| --- | --- | --- | --- |
| Undesired facility: GBAS station |  |  |  |
| Frequency offset | Minimum required geographical separation distance between stations for different radiated GBAS VDB powers (EIRP) of the undesired station |  |  |
|  | 50 W 47 dBm 17 dBW | 80 W 49 dBm 19 dBW | 100 W 50 dBm 20 dBW |
| 0 kHz (co-channel) | 236 NM | 240 NM | 242 NM |
| ±25 kHz (first adjacent channel) | 184 NM | 188 NM | 190 NM |
| ±50 kHz (second adjacent channel) | 30 NM | 31 NM | 32 NM |
| ±75 kHz (third adjacent channel) | 26 NM | 27 NM | 27 NM |
| ±100 kHz (fourth adjacent channel) | 25 NM | 25 NM | 25 NM |
| ±125 kHz and beyond | no restrictions |  |  |

| Desired facility: VOR (DOC 40 NM / 20000 ft) |  |  |  |
| --- | --- | --- | --- |
| Undesired facility: GBAS station |  |  |  |
| Frequency offset | Minimum required geographical separation distance between stations for different radiated GBAS VDB powers (EIRP) of the undesired station |  |  |
|  | 50 W 47 dBm 17 dBW | 80 W 49 dBm 19 dBW | 100 W 50 dBm 20 dBW |
| 0 kHz (co-channel) | 272 NM | 276 NM | 278 NM |
| ±25 kHz (first adjacent channel) | 220 NM | 224 NM | 226 NM |
| ±50 kHz (second adjacent channel) | 45 NM | 46 NM | 47 NM |
| ±75 kHz (third adjacent channel) | 41 NM | 42 NM | 42 NM |
| ±100 kHz (fourth adjacent channel) | 40 NM | 40 NM | 40 NM |
| ±125 kHz and beyond | no restrictions |  |  |

| Desired facility: VOR (DOC 50 NM / 25000 ft) |  |  |  |
| --- | --- | --- | --- |
| Undesired facility: GBAS station |  |  |  |
| Frequency offset | Minimum required geographical separation distance between stations for different radiated GBAS VDB powers (EIRP) of the undesired station |  |  |
|  | 50 W 47 dBm 17 dBW | 80 W 49 dBm 19 dBW | 100 W 50 dBm 20 dBW |
| 0 kHz (co-channel) | 300 NM | 304 NM | 306 NM |
| ±25 kHz (first adjacent channel) | 248 NM | 252 NM | 254 NM |
| ±50 kHz (second adjacent channel) | 55 NM | 56 NM | 57 NM |
| ±75 kHz (third adjacent channel) | 51 NM | 52 NM | 52 NM |
| ±100 kHz (fourth adjacent channel) | 50 NM | 50 NM | 50 NM |
| ±125 kHz and beyond | no restrictions |  |  |

| Desired facility: VOR (DOC 60 NM / 30000 ft) |  |  |  |
| --- | --- | --- | --- |
| Undesired facility: GBAS station |  |  |  |
| #### Frequency offset | Minimum required geographical separation distance between stations for different radiated GBAS VDB powers (EIRP) of the undesired station |  |  |
|  | 50 W 47 dBm 17 dBW | 80 W 49 dBm 19 dBW | 100 W 50 dBm 20 dBW |
| 0 kHz (co-channel) | 327 NM | 331 NM | 333 NM |
| ±25 kHz (first adjacent channel) | 275 NM | 279 NM | 281 NM |
| ±50 kHz (second adjacent channel) | 65 NM | 66 NM | 67 NM |
| ±75 kHz (third adjacent channel) | 61 NM | 62 NM | 62 NM |
| ±100 kHz (fourth adjacent channel) | 60 NM | 60 NM | 60 NM |
| ±125 kHz and beyond | no restrictions |  |  |

| Desired facility: VOR (DOC 70 NM / 40000 ft) |  |  |  |
| --- | --- | --- | --- |
| Undesired facility: GBAS station |  |  |  |
| Frequency offset | Minimum required geographical separation distance between stations for different radiated GBAS VDB powers (EIRP) of the undesired station |  |  |
|  | 50 W 47 dBm 17 dBW | 80 W 49 dBm 19 dBW | 100 W 50 dBm 20 dBW |
| 0 kHz (co-channel) | 368 NM | 372 NM | 374 NM |
| ±25 kHz (first adjacent channel) | 316 NM | 320 NM | 322 NM |
| ±50 kHz (second adjacent channel) | 75 NM | 76 NM | 77 NM |
| ±75 kHz (third adjacent channel) | 71 NM | 72 NM | 72 NM |
| ±100 kHz (fourth adjacent channel) | 70 NM | 70 NM | 70 NM |
| ±125 kHz and beyond | no restrictions |  |  |

| Desired facility: VOR (80 NM / 50000 ft) |  |  |  |
| --- | --- | --- | --- |
| Undesired facility: GBAS station |  |  |  |
| Frequency offset | Minimum required geographical separation distance between stations for different radiated GBAS VDB powers (EIRP) of the undesired station |  |  |
|  | 50 W 47 dBm 17 dBW | 80 W 49 dBm 19 dBW | 100 W 50 dBm 20 dBW |
| 0 kHz (co-channel) | 405 NM | 409 NM | 411 NM |
| ±25 kHz (first adjacent channel) | 353 NM | 357 NM | 359 NM |
| ±50 kHz (second adjacent channel) | 85 NM | 86 NM | 87 NM |
| ±75 kHz (third adjacent channel) | 81 NM | 82 NM | 82 NM |
| ±100 kHz (fourth adjacent channel) | 80 NM | 80 NM | 80 NM |
| ±125 kHz and beyond | no restrictions |  |  |

| Desired facility: VOR (100 NM / 50000 ft) |  |  |  |
| --- | --- | --- | --- |
| Undesired facility: GBAS station |  |  |  |
| Frequency offset | Minimum required geographical separation distance for different radiated GBAS VDB powers (EIRP) |  |  |
|  | 50 W 47 dBm 17 dBW | 80 W 49 dBm 19 dBW | 100 W 50 dBm 20 dBW |
| 0 kHz (co-channel) | 425 NM | 429 NM | 431 NM |
| ±25 kHz (first adjacent channel) | 373 NM | 377 NM | 379 NM |
| ±50 kHz (second adjacent channel) | 105 NM | 106 NM | 107 NM |
| ±75 kHz (third adjacent channel) | 101 NM | 102 NM | 102 NM |
| ±100 kHz (fourth adjacent channel) | 100 NM | 100 NM | 100 NM |
| ±125 kHz and beyond | no restrictions |  |  |

| Desired facility: VOR (150 NM / 50000 ft) |  |  |  |
| --- | --- | --- | --- |
| Undesired facility: GBAS station |  |  |  |
| Frequency offset | Minimum required geographical separation distance for different radiated GBAS VDB powers (EIRP) |  |  |
|  | 50 W 47 dBm 17 dBW | 80 W 49 dBm 19 dBW | 100 W 50 dBm 20 dBW |
| 0 kHz (co-channel) | 475 NM | 479 NM | 481 NM |
| ±25 kHz (first adjacent channel) | 423 NM | 427 NM | 429 NM |
| ±50 kHz (second adjacent channel) | 155 NM | 156 NM | 157 NM |
| ±75 kHz (third adjacent channel) | 151 NM | 152 NM | 152 NM |
| ±100 kHz (fourth adjacent channel) | 150 NM | 150 NM | 150 NM |
| ±125 kHz and beyond | no restrictions |  |  |

  - GBAS to ILS and ILS to GBAS Frequency Assignment Planning Criteria

    - For a more refined calculation considering EIRP PEP separated by polarization shall be used as provided in the planning criteria Table in 4.1. In absence of relevant Annex 10 material the following temporary criteria are provided, to allow use of  existing Frequency Management Programs (FEMP), which need D/U criteria as input values.

    - Results obtained by such programs are therefore to be used only with extreme caution and require additional validation. The required receive power density has to be provided at any point throughout the coverage.

    - The signal ratios between desired and undesired signals (D/U) to protect GBAS receivers in the presence of a ILS signal and ILS receivers in the presence of a GBAS signal (Annex 10 SARPs and guidance material for ±100 kHz and beyond are derived by adding 10 dB safety margin for co-channel and 6 dB safety margin for adjacent channel to the existing GBAS - VOR and VOR – GBAS criteria.

- The values separated by polarization are contained in the planning criteria Table in 4.1.

| Frequency offset | co-chn. | ±25 kHz (first adjacent channel) | ±50 kHz (second adjacent channel) | ±75 kHz (third adjacent channel) | ±100 kHz (fourth adjacent channel) | ±125 kHz and beyond |
| --- | --- | --- | --- | --- | --- | --- |
| GBAS vs. ILS | 36 dB | 6 dB | -28 dB | -40 dB | -59 dB | -59 dB |
| ILS vs. GBAS | 36 dB | 6 dB | -28 dB | -40 dB | -59 dB | -59 dB |

Table 1

  - GBAS to VHF Communications Frequency Assignment Planning Criteria

### 4.5.1	For a more refined calculation considering ERIP PEP separated by polarization shall be used as provided in the planning criteria Table in 4.1.

4.5.2	In absence of relevant Annex 10 material the following temporary criteria are provided, to allow use of of existing Frequency Management Programs (FEMP), which need D/U criteria as input values.

4.5.3	Results obtained by such programs are therefore to be used only with extreme caution and require additional validation. The required receive power density has to be provided at any point throughout the coverage.

4.5.4	The signal ratios between desired and undesired signals (D/U) to protect GBAS receivers in the presence of a VHF-COMsignal and VHF-COM receivers in the presence of a GBAS signal and beyond are derived by adding 10 dB safety margin for co-channel and 6 dB safety margin for adjacent channel to the Draft of VHF-COM to VDL-M.2 (= GBAS) criteria.

- The values separated by polarization are contained in the planning criteria Table in 4.1.

  - GBAS Data Selector and Time Slot Planning Criteria

4.6.1	Time slot co-ordination among GBAS VDB systems operating on the same frequency will not be used initially, i.e. all 8 time slots (A – H) are assigned to a station together with the VDB frequency.If time slot coordination were to be used, up to eight times reuse of the frequency would be possible. :

Note.— No geographic transmitter restrictions are expected between co-frequency, adjacent time slots provided the undesired VDB transmitting antenna is located at least 80 m from areas where the desired signal is at minimum field strength.

    - Reference path data selectors (RPDS) and reference station data selectors (RSDS) must not be duplicated within the protection region of a given frequency. As long as time slot co-ordination is not used to share frequencies, as indicated above, RPDS and RSDS (0 to 48) compatibility is automatically established via frequency co-ordination, i.e. all 49 data selectors are available together with the assigned VDB frequency.

  - Special Provisions

4.7.1	The provisions below apply as long as ICAO Annex 10 GBAS frequency assignment criteria have not been completed:

- frequencies should be selected from the frequency range of 112.050 – 116.400 MHz to protect ILS and VHF-COM(frequencies above 116.400 MHz may be used if the necessary calculation using the D/U criteria from table in 4.1. have indicated sufficient protection).;

- GBAS VDB systems with multiple antennas which are separated more than 1 NM must be considered individually.

    - Proposals for co-ordination of GBAS assignments must include at a minimum

- ITU Country Code;

- Location name and co-ordinates;

- Frequency;

- Equipment (i.e. GBAS/H or GBAS/E);

- Identification;

- DOC (23 NM / 10000 ft is recommended);

- Peak radiated power EIRP PEP in W for horizontal polarization;

•	Peak radiated power EIRP PEP in W for vertical polarization;

- antenna elevation above MSL of the phase center of the antenna;

- Time slot and data selectorinformation

## DME

  - General

    - The band 960 - 1215 MHz is allocated, though not exclusively, to the aeronautical radio navigation service and used mainly by the DME system. Within this band, two segments, around 1030 MHz and 1090 MHz, are reserved for SSR. Furthermore, some military systems make use of this band, e.g. TACAN (essentially a DME with additional pulses for bearing information) and data/voice systems employing frequency hopping techniques. Part 960-1164 MHz of the band is also allocated to the aeronautical mobile (route) service, for use by next generation aeronautical communication systems.

Note: In order to support compatibility between DME/TACAN assignments and GNSS L5/E5 aeronautical receivers, States are encouraged to apply the recommendations contained in section D of the Supplement to EUR Doc011 when planning new DME/TACAN assignments.

    - References to documents:

-	Annex 10, Volume I, section 3.5.3.3, paragraph 3.5.5.3.4.1

(channelling, interference rejection);

-	Annex 10, Volume V, section 4.3

(channel groups, pairing);

-	Doc 9718, Vol. II, section 5.14

-

(protection requirements for DME versus other DME.);

-	European Region Air Navigation Plan (Doc 7754), Volume I, Part III, paragraph  2.9 (foundations of frequency assignment planning) ;

-	European Region Air Navigation Plan (Doc 7754), Volume II, Part III, paragraph 2.47  (use of X and Y channels).

    - DME is in many cases co-located and frequency paired (Annex 10, Volume I, chapter 3, Table A) with another facility (VOR, ILS, MLS) and normally has the same protected range. The protection also takes into account the pulse coding and output power. Both the first and second adjacent channels are considered.

    - Co-ordination should follow the rules for the co-located equipment; for a stand-alone DME co-ordination should at least include States within a radius of 400 - 500 NM.

    - Information on the planning of identifications can be found in section 7 below.

  - Frequency Assignment Planning Criteria

Notes:	For frequency planning purposes,

- there is no difference between DME/N and DME/P.

- TACAN facilities are treated in the same way as DME stations.

- no criteria are defined for DME/W because it is not used in EUR Region.

    - Protection Requirements

      - The necessary desired to undesired (D/U) signal ratios are needed to protect the desired transponder reply signals at the airborne receiver from the various co-frequency/adjacent-frequency, same pulse code/different pulse code, undesired transponder reply signal combinations that may exist.

      - In making an assignment, each facility must be treated as the desired source with the other acting as the undesired source. If both satisfy their  individual D/U requirements, then the assignment can be made.

      - The channelling arrangement for DME, when considering X and Y channels only, is such that in the ground-to-air direction (transponder reply frequency) no use is made of multiplexing techniques on the same frequency, thus avoiding the situation where different pulse codings on the same or adjacent frequencies have to be studied when making frequency assignments. However, when assign4ments on DME W or Z channels have to be made, these assignments have to be checked against interference from DME X or Y channels and vice-versa.

      - Co-frequencyTPPT protection ratios (D/U)

The co-frequency protection ratios (D/U) are:

Same pulse code:	8 dB

Different pulse code:	8 dB	(column A of Table

5-3, Doc 9718, Vol. II).

Geographical separation distances are based upon the required D/U ratio, taking into consideration the EIRP of both the desired and the undesired DME and the appropriate propagation characteristics.

      - Adjacent frequency protection ratios (D/U)

The following values are assumed for the adjacent-channel emissions of the transponder:

200 mW (-7 dBW) on the first adjacent frequency

2 mW (-27 dBW) on the second adjacent frequency.

Note: The application of the above values in combination with a co-frequency protection ratio of 8 dB is equivalent to using the adjacent-channel D/U ratios given in Table 5-3, column A of Doc 9718, Vol.II for the same pulse code.

      - Designated Operational Coverage (DOC)

        - When the DME is associated with a VOR or an ILS the coverage should be at least that of the VOR or ILS to the extent practicable.

        - When the DME is associated with an MLS, the coverage should be omnidirectional up to the operational range of the MLS approach azimuth facility.  The protected height of the DME should be the same as for the MLS approach azimuth sector.

        - Where the designated operational range of a given DME facility is not the same throughout 360°, the angular limits of sectorization in range should be indicated in accordance with the method described in Appendix A to Part III.

        - The DOC for a DME with directional antenna should be specified on the basis of the particular operational requirements. The orientation of the DOC should be defined relative to the true (i.e. with respect to the true north – not the magnetic one) radiation direction (TRD) of the facility. When defining the DOC for a DME with directional antenna, the antenna characteristics of the DME facility need to be taken into account in order to ensure that throughout the DOC the available signal level will not be lower than the minimum signal level to be protected (-89 dBW/m²).

      - Calculation of EIRP

        - The EIRP (in dBW) in any particular direction α can be calculated by means of the general formula:

EIRP(α)= EIRPmax+ GAntenna, rel (α),

where

EIRPmax = PTransmitter Output – ALine + GAntenna, max .

PTransmitter Output is the output power of the transmitter in dBW, ALine is the attenuation of the transmission line in dB and GAntenna, max is the antenna gain in the main beam direction in dBi (with respect to an isotropic antenna) at the given frequency. Furthermore, GAntenna, rel (α) denotes the normalised antenna gain in dB relative to GAntenna, max, at the given vector direction α. In the case of DME, EIRP and PTransmitter Output are meant as PEP (Peak Envelope Power) values.

Note: GAntenna, rel  is always less than or equal to 0 dB. For an omnidirectional antenna, the normalized gain GAntenna, rel is taken equal to 0 dB.

        - The value of the foreseen EIRPmax shall be provided with the frequency co-ordination request. It is calculated as above, where PTransmitter Output should be sufficient to produce adequate power flux density throughout the DOC.

        - Typical EIRPmax values per category of facility are given below as guidance, based on standard transmitter output power, feeder line losses and antenna gain:

Landing DME:	27 dBW, Omnidirectional

29 dBW, Directional

En-route DME:  	37 dBW

TACAN:	40 dBW.

        - For the sake of spectrum economy, it is highly recommended that technical measures be taken so that the EIRPmax of landing DME with directional antenna does not exceed 29 dBW, despite the high gain provided by the beacon antenna.

    - Propagation model

      - Propagation characteristics

        - The same propagation conditions along the path of the desired and the undesired signals are assumed. Within the radio line-of-sight, free space attenuation is assumed. Beyond the radio line-of-sight, propagation is approximated by an attenuation rate of 1.6 dB/NM. This is derived from the propagation  1200 MHz/50% time curve in Rec. ITU-R P.528-2 (former CCIR Rec. 528-1). For radio propagation and radio horizon calculations a smooth Earth is assumed with an effective 4/3 Earth radius.

Within and beyond the radio line-of-sight situations are depicted below. Two points are considered to be within radio line-of-sight if their separation distance is smaller than the sum of their respective radio distances.

        - Calculation of the radio horizon distance:

The radio horizon distance of a point P is given by:

RBHB = 1.23 *

where

RBH	B= radio horizon distance in nautical miles (NM)

h	= height of point P in feet (ft)

For the calculation of the radio horizon distance of a receiving point to be protected (denoted by RH1 in the above figure), the above formula is applied with h being the altitude in feet.

The radio horizon distance of the DME facility (denoted by RH2 in the above figure) is calculated by the same formula as above. In general, the height of the phase centre of the facility’s antenna above ground level should be taken for the calculation of the facility’s radio horizon.

If a facility’s antenna height is not provided it will be assumed equal to 30 feet.

        - When radio line-of-sight conditions exist between the DME and the protection point P, the following formula may be used to calculate the power flux density at point P as:

PBdB = EIRP - 20 log R - 76.3	(1a)

where

PBdB	= power flux density in dBW/mP2P

EIRP	= effective isotropically radiated power in dBW

R	= distance of point P to DME in NM

        - For distances between a DME facility and a receiving point P greater than the sum of their respective radio horizon distances, the following formula may be used instead:

PBd   = EIRP - 20 log RBRLOSB - 1.6*RBEB - 76.3,	(1b)

where

RBRLOSB = sum of the radio horizon distances of point P and the DME facility

= maximum radio line-of-sight distance, given the heights of point P and the DME facility

RE = propagation distance in NM covered by diffraction (see above figure)

= radio path distance separating point P from the DME facility minus RRLOS

      - Additional means of calculation

As a means of improving the efficiency of spectrum utilization, in particular in frequency congested areas, compatibility of assignments could also be assessed on a bilateral basis using more realistic propagation models taking into account other signal attenuation elements such as terrain which can affect signal propagation significantly.

    - Calculation of Minimum Required Separation Distances

The conditions for the satisfactory operation of DME are expressed in terms of the protection ratio (D/U) in dB, which is equal to the difference in power flux densities of the desired and undesired DME facilities at the protection point P. The minimum acceptable value of D/U is used to calculate the minimum required separation distance between two DME facilities. In calculating separation distances, the conditions of paragraph 5.2.1 above shall be met. The table at the end of chapter 5 contains the co-frequency, different pulse code, and adjacent frequency, same and different pulse code, DME channels to consider when an assignment on a particular DME channel is proposed.

      - Co-frequency; desired and undesired DME facilities having the same pulse code.

        - Undesired DME facility within the radio line-of-sight

Application of formula (1a) to the desired and undesired DME as appropriate, results in:

D/U = K + 20 log ( RBuB/RBdB )	(2a)

where

D/U	= protection ratio in dB (minimum 8 dB)

K	= EIRP of the desired minus the EIRP of the undesired facility in dBW

RBuB	= distance between the edge of the DOC of the desired facility and the undesired facility in NM

RBdB	= operational range of desired facility in NM

Thus for the required separation distance RBsBit is obtained

RBsB = Rd + Ru = RBdB ( 1 + 10 P[ ( D/U - K) / 20 ]P ), or	(2b)

Rs = Rd + 10 T / 20 .

where

RBsB	= required separation distance between the desired and undesired facilities in NM and

T	= D/U - K + 20 log Rd.

        - Undesired DME facility beyond the radio line-of-sight

Application of formula (1a) to the desired DME and of (1b) to the undesired DME leads to:

D/U = K + 20 log ( RRLOSB/RBdB ) + 1.6 RBEB	(3a)

This can be expressed as the required separation distance RBsB:

RBsB = RBdB + RBRLOSB + [ T - 20 log RRLOS ]/ 1.6	(3b)

The numerical examples below are valid only for omnidirectional DME (the facilities antenna heights are assumed equal to 30ft above ground level):

a)	DME(1) coverage: 23 NM, 10 000 ft; EIRP =  27 dBW

DME(2) coverage: 23 NM, 10 000 ft; EIRP =  27 dBW

Since the two services have identical characteristics, one check is sufficient. For a D/U ratio of 8 dB, no EIRP difference: K=0 and thus T = 8 - 0 + 20 log (23) = 35.23; therefore 10T/20=58NM. The RRLOS is equal to 1.23*(+)=130NM >58NM. Hence the undesired facility and a receiving point at the edge of the service area of the desired facility can be within radio line of sight, the minimum separation distance between facilities being equal to RBdB + 10T/20 = 23 + 58 = 81NM.

b)	DME(1) coverage: 25 NM, 10 000 ft; EIRP =  27 dBW

DME(2) coverage: 120 NM, 40 000 ft; EIRP = 40 dBW

D/U = 8 dB.

i)	separation distance to protect DME(1):

T = 8 - (27-40) + 20 log (25) = 49, 10T/20= 281 NM.

RRLOS =1.23*(+)=130 NM.

Since 10T/20> RRLOS, DME(2) should be located beyond the radio line of sight of a receiving point at the edge of the service area of  DME(1).

Hence the minimum separation distance between facilities to protect DME(1) should be calculated with formula (3b): RBsB= RBdB+RRLOSB+[T-20 log RRLOS ]/1.6 = 25B +130 + 4 = 159NM.

ii)	separation distance to protect DME(2):

T = 8 - (40-27) + 20 log (120) = 36.6, 10T/20= 68NM.

RRLOS =1.23*(+)=253 NM.

Now 10T/20<RRLOS, with the result that DME(1) can be within radio line of sight of a receiving point at the edge of the service area of DME(2), the minimum separation distance between facilities to protect DME(2) being equal to RBsB= RBdB + 10T/20 = 120 + 68 = 188 NM.

To assure protection to both facilities, the  required separation distance between these facilities should be at least 188 NM.

      - Co-frequency; desired and undesired DME facility having different pulse codes

Since the required protection is provided at the airborne DME receiver, different pulse coding protection is only required when a DME X channel is interfered with a DME W channel (or DME Y versus DME Z) or vice versa (see also paragraph 5.2.1.3). The separation distances can be calculated along the same principles as indicated in paragraph 5.2.3.1. The protection ratio is 8 dB (see paragraph. 5.2.1.4).

      - Adjacent channel separation distances

        - First adjacent frequency

The minimum separation distance between DMEs operating on first-adjacent channels shall be 65 NM between beacons or 5NM from the DOC-edge of the desired beacon, whichever gives the lesser distance between beacons.

Notes:

1	The above criteria ensure that the D/U ratio within the operational service volume does not fall below the values given in Table 5-3, Doc 9718, Volume II.

2.	Since calculations for a DME/P show that all separation distances derived for the Initial Approach (IA) mode (-89 dBW/mP2P at 23 NM) are larger than for the Final Approach (FA) mode (-75 dBW/mP2P at 7 NM), it is sufficient to consider only the IA mode protection.

        - Second adjacent frequency

The second adjacent frequency separation distances are almost insignificant. However it is recommended that second adjacent frequency assignments on the same aerodrome should be avoided. For practical planning purposes, a minimum required separation distance between the facilities of 10 NM may be used.

      - Separation requirement for DME frequencies which are separated by 63 MHz

Annex 10, Attachment C to Volume I, paragraph 7.1.9 indicates that “where the reply frequency of one DME matches the interrogation frequency of a second DME, the two transponders should be separated by a distance greater than the radio horizon distance between them”. For this reason, a minimum separation distance of 15 NM (assuming an antenna height of 30 ft for both transponders) between the two transponders should be imposed in this case, which occurs when the reply frequencies of the two DMEs differ by 63 MHz.

This requirement is to prevent desensitization that may occur to the transponder that is receiving on the same frequency the other transponder is transmitting, irrespective of the pulse code.

For example:

downlink	uplink				downlink	uplink

18X(W)	1042		979		81X(W)	U1105U		1168

18Y(Z)		1042		U1105U		81Y(Z)		U1105U		1042

Channel 18Y(Z), transmitting on 1105 MHz may cause interference to the transponder receiver operating on channels 81X(W) or 81Y(Z). Similarly, channel 81Y(Z) may interfere with channels 18X(W) and 18Y(Z).

      - Sectorization

The material of paragraph 3.3.5 (VOR) on sectorization is also applicable for DME. However, the calculation of the separations distance must be adapted as indicated below:

The correct assessment of compatibility for facility A requires the following calculations:

Co-channel:

Minimum separation distance between “Critical Point” and transmitter B:

RBsB = RBdB*10P[(D/U– K)/20]P				if undesired facility within radio line-of-sight; or

RBsB = RBHB+[D/U-K-20 log (RBHB/RBdB)]/1.6		if undesired facility beyond radio line-of-sight;

(this is derived from a subtraction of Rd from RBsB = RBdB (1 + 10[(D/U-K)/20]) and from
RBdB+RBHB+[D/U-K-20 log (RBHB/RBdB)]/1.6.);

where

RBdB = operational range of facility A at the “ Critical Point”.

Co-frequency:

Minimum separation distance between “ Critical Point” and transmitter B:

RBsB = RBdB*10[(D/U-K)/20]			if undesired facility within radio line-of-sight; or

RBsB = RBHB+[D/U-K-20 log (RBHB/RBdB)]/1.6		if undesired facility beyond radio line-of-sight;

(this is derived from a subtraction of Rd from RBsB = RBdB (1 + 10[(D/U-K)/20]) and from
RBdB+RBHB+[D/U-K-20 log (RBHB/RBdB)]/1.6.).

First adjacent channel for DMEs with a DOC radius of 60 NM or less:

Minimum separation distance between “ Critical Point” and transmitter B:

Range of facility A plus 5 NM.

First adjacent channel for DMEs with a DOC radius of greater than 60 NM:

Minimum 65 NM separation distance between facilities A and B.

Second adjacent channel:

Minimum separation distance between transmitter A and transmitter B:

10 NM.

63 MHz separated facilities:

Minimum separation distance between transmitter A and transmitter B:

15 NM.

Compatibility of facility A (undesired facility) with B (desired facility) must also be considered.

If the “ Critical Point” lies on the direct line between the two facilities A and B, then the “normal” calculation method may be applied (i.e. point “P” is “ Critical Point”).

5.2.3.6  Use of DMEs with directional antennas

When omnidirectional coverage is not required, the use of a DME with directional antenna is encouraged, subject to the proviso that the operational requirements for the service to be provided by this facility are met. In Particular, they are useful for the replacement of ILS marker beacons. The directional DME could provide a means to mitigate frequency congestion.

The compatibility of DMEs with directional antennas can be examined basically along the same lines as those developed for omnidirectional DME. However, because of the variable antenna gain of a DME with directional antenna, the value of the quantity K (EIRP of the desired minus the EIRP of the undesired facility) in the above formulas depends in general on the position of the airborne receiver. It is noted in particular that because of the orientation of a DME with directional antenna, the minimum D/U value is not necessarily attained along the direct line connecting desired and undesired facilities, as opposed to the case of compatibility between omnidirectional DME.

In the presence of at least one DME with directional antenna, the following method should be applied. At first an appropriate number of points is selected along the edge of the DOC of the desired DME, regardless of the type of its antenna. Subsequently at all such points the required D/U criteria are checked. Compatibility is ensured if at all such points the required D/U criteria are met. The following figure depicts how the compatibility check is carried out for the protection of a directional DME.

Undesired

facility

Protection point P

TRD

Desired

facility

Direct line
between facilities

Currently (2010),  two specific antennas  can be used for directional DME.  One  is “Kathrein 716405”, known also as “FAN88” and the other one is “Kathrein 880 10003”.

Unless otherwise specified, the normalised antenna gain GAntenna, rel  of the facility’s directional antenna (in dB) will be calculated as follows:

For the transmission of desired signals:

GAntenna,rel,d (φ) = – 3.1·10-3 φ2 + 7.5·10-8 φ4 – 0.9·10-12 φ6,          for –180°≤φ≤180°.

For the transmission of undesired signals:

GAntenna,rel,u (φ) = – 2.45·10-3 φ2 + 8.1·10-8 φ4 – 10-12 φ6,              for –180°≤φ≤180°,

where φ denotes the azimuthal deviation in degrees from the direction of maximum radiation (see figure below). The above formulas for the calculation of the normalized antenna gain are frequency-independent approximations to the measured gain values of the  afore-mentioned antennas. In particular, GAntenna,rel,d and GAntenna,rel,u represent respectively an underestimate and an overestimate of the measured normalized antenna gain. If more precision is required, use can be made of the antenna manufacturer’s data sheets. Users of other antenna types should either submit detailed antenna gain data or verify that their normalized antenna gain is not less than GAntenna,rel,d and not greater than GAntenna,rel,u as above.

Note1:  For the protection of a DME with directional antenna, no compatibility check is required outside its DOC.

Note2: In the coordination messages, DME facilities with directional antennas should be clearly designated as DME-dir.

    - Table of Example Minimum Separation Distances used in the Planning of Omnidirectional DME

| Facility 1 | Facility 2 | Minimum required separation distance (NM) |  |  |
| --- | --- | --- | --- | --- |
| Desired | Undesired | Co-frequency | 1PstP adjacent frequency | 2PndP adjacent frequency |
| Cylindrical DOC and EIRP (30ft assumed for both facility heights) |  |  |  |  |
| DME (MLS) 23 NM/10000 ft 27 dBW 1) | 1) | 81 | 28 | 10 |
| 1) | 2) | 81 | 28 | 10 |
| 1) | 3) | 155 | 28 | 10 |
| 1) | 4) | 156 | 28 | 10 |
| DME (ILS) 25 NM/10000 ft 27 dBW 2) | 1) | 88 | 30 | 10 |
| 2) | 2) | 88 | 30 | 10 |
| 2) | 3) | 157 | 30 | 10 |
| 2) | 4) | 159 | 30 | 10 |
| DME (VOR) 100 NM/50000 ft 37 dBW 3) | 1) | 179 | 65 | 10 |
| 3) | 2) | 179 | 65 | 10 |
| 3) | 3) | 351 | 65 | 10 |
| 3) | 4) | 383 | 65 | 10 |
| TACAN 40 NM/25000 ft 40 dBW 4) | 1) | 62 | 45 | 10 |
| 4) | 2) | 62 | 45 | 10 |
| 4) | 3) | 111 | 45 | 10 |
| 4) | 4) | 140 | 45 | 10 |

Image - 63 MHz:

separation 28 km (15 NM)  .

Notes:

- Typical for a DME associated with a MLS

- Typical for a DME associated with an ILS

- Typical for a DME associated with a VOR

- Typical for a TACAN

The required separation distance for two facilities A and B is the greater of the separation distances between A and B or B and A (e.g. the required co-frequency separation distance between A being a DME associated with an ILS and B a DME associated with a VOR is the greater of 157 and 179 NM, i.e. 179 NM).

Table

| DME-channel | Reply FREQ (MHz) | Co-FREQ different pulse code | 1st adjacent FREQ same pulse code |  | 1st adjacent FREQ different pulse code |  | 2nd adjacent FREQ same pulse code |  | 2nd adjacent FREQ different pulse code |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 17X 17Y 17Z | 978 1104 1104 | - 17Z 17Y | 16X 16Y - | 18X 18Y 18Z | - - 16Y | 18W 18Z 18Y | 15X 15Y - | 19X 19Y 19Z | - - 15Y | - 19Z 19Y |
| 18X 18W 18Y 18Z | 979 979 1105 1105 | 18W 18X 18Z 18Y | 17X - 17Y 17Z | 19X - 19Y 19Z | - 17X 17Z 17Y | - 19X 19Z 19Y | 16X - 16Y - | 20X 20W 20Y 20Z | - 16X - 16Y | 20W 20X 20Z 20Y |
| 19X 19Y 19Z | 980 1106 1106 | - 19Z 19Y | 18X 18Y 18Z | 20X 20Y 20Z | 18W 18Z 18Y | 20W 20Z 20Y | 17X 17Y 17Z | 21X 21Y 21Z | - 17Z 17Y | - 21Z 21Y |
| 20X 20W 20Y 20Z | 981 981 1107 1107 | 20W 20X 20Z 20Y | 19X - 19Y 19Z | 21X - 21Y 21Z | - 19X 19Z 19Y | - 21X 21Z 21Y | 18X 18W 18Y 18Z | 22X 22W 22Y 22Z | 18W 18X 18Z 18Y | 22W 22X 22Z 22Y |
| 21X 21Y 21Z | 982 1108 1108 | - 21Z 21Y | 20X 20Y 20Z | 22X 22Y 22Z | 20W 20Z 20Y | 22W 22Z 22Y | 19X 19Y 19Z | 23X 23Y 23Z | - 19Z 19Y | - 23Z 23Y |
| 22X 22W 22Y 22Z | 983 983 1109 1109 | 22W 22X 22Z 22Y | 21X - 21Y 21Z | 23X - 23Y 23Z | - 21X 21Z 21Y | - 23X 23Z 23Y | 20X 20W 20Y 20Z | 24X 24W 24Y 24Z | 20W 20X 20Z 20Y | 24W 24X 24Z 24Y |
| 23X 23Y 23Z | 984 1110 1110 | - 23Z 23Y | 22X 22Y 22Z | 24X 24Y 24Z | 22W 22Z 22Y | 24W 24Z 24Y | 21X 21Y 21Z | 25X 25Y 25Z | - 21Z 21Y | - 25Z 25Y |
| 24X 24W 24Y 24Z | 985 985 1111 1111 | 24W 24X 24Z 24Y | 23X - 23Y 23Z | 25X - 25Y 25Z | - 23X 23Z 23Y | - 25X 25Z 25Y | 22X 22W 22Y 22Z | 26X 26W 26Y 26Z | 22W 22X 22Z 22Y | 26W 26X 26Z 26Y |
| 25X 25Y 25Z | 986 1112 1112 | - 25Z 25Y | 24X 24Y 24Z | 26X 26Y 26Z | 24W 24Z 24Y | 26W 26Z 26Y | 23X 23Y 23Z | 27X 27Y 27Z | - 23Z 23Y | - 27Z 27Y |
| 26X 26W 26Y 26Z | 987 987 1113 1113 | 26W 26X 26Z 26Y | 25X - 25Y 25Z | 27X - 27Y 27Z | - 25X 25Z 25Y | - 27X 27Z 27Y | 24X 24W 24Y 24Z | 28W 28X 28Y 28Z | 24W 24X 24Z 24Y | 28W 28X 28Z 28Y |
| 27X 27Y 27Z | 988 1114 1114 | - 27Z 27Y | 26X 26Y 26Z | 28X 28Y 28Z | 26W 26Z 26Y | 28W 28Z 28Y | 25X 25Y 25Z | 29X 29Y 29Z | - 25Z 25Y | - 29Z 29Y |
| 28X 28W 28Y 28Z | 989 989 1115 1115 | 28W 28X 28Z 28Y | 27X - 27Y 27Z | 29X - 29Y 29Z | - 27X 27Z 27Y | - 29X 29Z 29Y | 26X 26W 26Y 26Z | 30X 30W 30Y 30Z | 26W 26X 26Z 26Y | 30W 30X 30Z 30Y |
| 29X 29Y 29Z | 990 1116 1116 | - 29Z 29Y | 28X 28Y 28Z | 30X 30Y 30Z | 28W 28Z 28Y | 30W 30Z 30Y | 27X 27Y 27Z | 31X 31Y 31Z | - 27Z 27Y | - 31Z 31Y |
| 30X 30W 30Y 30Z | 991 991 1117 1117 | 30W 30X 30Z 30Y | 29X - 29Y 29Z | 31X - 31Y 31Z | - 29X 29Z 29Y | - 31X 31Z 31Y | 28X 28W 28Y 28Z | 32X 32W 32Y 32Z | 28W 28X 28Z 28Y | 32W 32X 32Z 32Y |
| 31X 31Y 31Z | 992 1118 1118 | - 31Z 31Y | 30X 30Y 30Z | 32X 32Y 32Z | 30W 30Z 30Y | 32W 32Z 32Y | 29X 29Y 29Z | 33X 33Y 33Z | - 29Z 29Y | - 33Z 33Y |
| 32X 32W 32Y 32Z | 993 993 1119 1119 | 32W 32X 32Z 32Y | 31X - 31Y 31Z | 33X - 33Y 33Z | - 31X 31Z 31Y | - 33X 33Y 33Z | 30X 30W 30Y 30Z | 34X 34W 34Y 34Z | 30W 30X 30Z 30Y | 34W 34X 34Z 34Y |
| 33X 33Y 33Z | 994 1120 1120 | - 33Z 33Y | 32X 32Y 32Z | 34X 34Y 34Z | 32W 32Z 32Y | 34W 34Z 34Y | 31X 31Y 31Z | 35X 35Y 35Z | - 31Z 31Y | - 35Z 35Y |
| 34X 34W 34Y 34Z | 995 995 1121 1121 | 34W 34X 34Z 34Y | 33X - 33Y 33Z | 35X - 35Y 35Z | - 33X 33Z 33Y | - 35X 35Z 35Y | 32X 32W 32Y 32Z | 36X 36W 36Y 36Z | 32W 32X 32Z 32Y | 36W 36X 36Z 36Y |
| 35X 35Y 35Z | 996 1122 1122 | - 35Z 35Y | 34X 34Y 34Z | 36X 36Y 36Z | 34W 34Z 34Y | 36W 36Z 36Y | 33X 33Y 33Z | 37X 37Y 37Z | - 33Z 33Y | - 37Z 37Y |
| 36X 36W 36Y 36Z | 997 997 1123 1123 | 36W 36X 36Z 36Y | 35X - 35Y 35Z | 37X - 37Y 37Z | - 35X 35Z 35Y | - 37X 37Z 37Y | 34X 34W 34Y 34Z | 38X 38W 38Y 38Z | 34W 34X 34Z 34Y | 38W 38X 38Z 38Y |
| 37X 37Y 37Z | 998 1124 1124 | - 37Z 37Y | 36X 36Y 36Z | 38X 38Y 38Z | 36W 36Z 36Y | 38W 38Z 38Y | 35X 35Y 35Z | 39X 39Y 39Z | - 35Z 35Y | - 39Z 39Y |
| 38X 38W 38Y 38Z | 999 999 1125 1125 | 38W 38X 38Z 38Y | 37X - 37Y 37Z | 39X - 39Y 39Z | - 37X 37Z 37Y | - 39X 39Z 39Y | 36X 36W 36Y 36Z | 40W 40X 40Y 40Z | 36W 36X 36Z 36Y | 40W 40X 40Z 40Y |
| 39X 39Y 39Z | 1000 1126 1126 | - 39Z 39Y | 38X 38Y 38Z | 40X 40Y 40Z | 38W 38Z 38Y | 40W 40Z 40Y | 37X 37Y 37Z | 41X 41Y 41Z | - 37Z 37Y | - 41Z 41Y |
| 40X 40W 40Y 40Z | 1001 1001 1127 1127 | 40W 40X 40Z 40Y | 39X - 39Y 39Z | 41X - 41Y 41Z | - 39X 39Z 39Y | - 41X 41Z 41Y | 38X 38W 38Y 38Z | 42X 42W 42Y 42Z | 38W 38X 38Z 38Y | 42W 42X 42Z 42Y |
| 41X 41Y 41Z | 1002 1128 1128 | - 41Z 41Y | 40X 40Y 40Z | 42X 42Y 42Z | 40W 40Z 40Y | 42W 42Z 42Y | 39X 39Y 39Z | 43X 43Y 43Z | - 39Z 39Y | - 43Z 43Y |
| 42X 42W 42Y 42Z | 1003 1003 1129 1129 | 42W 42X 42Z 42Y | 41X - 41Y 41Z | 43X - 43Y 43Z | - 41X 41Z 41Y | - 43X 43Z 43Y | 40X 40W 40Y 40Z | 44X 44W 44Y 44Z | 40W 40X 40Z 40Y | 44W 44X 44Z 44Y |
| 43X 43Y 43Z | 1004 1130 1130 | - 43Z 43Y | 42X 42Y 42Z | 44X 44Y 44Z | 42W 42Z 42Y | 44W 44Z 44Y | 41X 41Y 41Z | 45X 45Y 45Z | - 41Z 41Y | - 45Z 45Y |
| 44X 44W 44Y 44Z | 1005 1005 1131 1131 | 44W 44X 44Z 44Y | 43X - 43Y 43Z | 45X - 45Y 45Z | - 43X 43Y 43Z | - 45X 45Y 45Z | 42X 42W 42Y 42Z | 46X 46W 46Y 46Z | 42W 42X 42Z 42Y | 46W 46X 46Z 46Y |
| 45X 45Y 45Z | 1006 1132 1132 | - 45Z 45Y | 44X 44Y 44Z | 46X 46Y 46Z | 44W 44Z 44Y | 46W 46Z 46Y | 43X 43Y 43Z | 47X 47Y 47Z | - 43Z 43Y | - 47Z 47Y |
| 46X 46W 46Y 46Z | 1007 1007 1133 1133 | 46W 46X 46Z 46Y | 45X - 45Y 45Z | 47X - 47Y 47Z | - 45X 45Z 45Y | - 47X 47Z 47Y | 44X 44W 44Y 44Z | 48X 48W 48Y 48Z | 44W 44X 44Z 44Y | 48W 48X 48Z 48Y |
| 47X 47Y 47Z | 1008 1134 1134 | - 47Z 47Y | 46X 46Y 46Z | 48X 48Y 48Z | 46W 46Z 46Y | 48W 48Z 48Y | 45X 45Y 45Z | 49X 49Y 49Z | - 45Z 45Y | - 49Z 49Y |
| 48X 48W 48Y 48Z | 1009 1009 1135 1135 | 48W 48X 48Z 48Y | 47X - 47Y 47Z | 49X - 49Y 49Z | - 47X 47Z 47Y | - 49X 49Z 49Y | 46X 46W 46Y 46Z | 50X 50W 50Y 50Z | 46W 46X 46Z 46Y | 50W 50X 50Z 50Y |
| 49X 49Y 49Z | 1010 1136 1136 | - 49Z 49Y | 48X 48Y 48Z | 50X 50Y 50Z | 48W 48Z 48Y | 50W 50Z 50Y | 47X 47Y 47Z | 51X 51Y 51Z | - 47Z 47Y | - 51Z 51Y |
| 50X 50W 50Y 50Z | 1011 1011 1137 1137 | 50W 50X 50Z 50Y | 49X - 49Y 49Z | 51X - 51Y 51Z | - 49X 49Z 49Y | - 51X 51Z 51Y | 48X 48W 48Y 48Z | 52X 52W 52Y 52Z | 48W 48X 48Z 48Y | 52W 52X 52Z 52Y |
| 51X 51Y 51Z | 1012 1138 1138 | - 51Z 51Y | 50X 50Y 50Z | 52X 52Y 52Z | 50W 50Z 50Y | 52W 52Z 52Y | 49X 49Y 49Z | 53X 53Y 53Z | - 49Z 49Y | - 53Z 53Y |
| 52X 52W 52Y 52Z | 1013 1013 1139 1139 | 52W 52X 52Z 52Y | 51X - 51Y 51Z | 53X - 53Y 53Z | - 51X 51Z 51Y | - 53X 53Z 53Y | 50X 50W 50Y 50Z | 54X 54W 54Y 54Z | 50W 50X 50Z 50Y | 54W 54X 54Z 54Y |
| 53X 53Y 53Z | 1014 1140 1140 | - 53Z 53Y | 52X 52Y 52Z | 54X 54Y 54Z | 52W 52Z 52Y | 54W 54Z 54Y | 51X 51Y 51Z | 55X 55Y 55Z | - 51Z 51Y | - 55Z 55Y |
| 54X 54W 54Y 54Z | 1015 1015 1141 1141 | 54W 54X 54Z 54Y | 53X - 53Y 53Z | 55X - 55Y 55Z | - 53X 53Z 53Y | - 55X 55Z 55Y | 52X 52W 52Y 52Z | 56X 56W 56Y 56Z | 52W 52X 52Z 52Y | 56W 56X 56Z 56Y |
| 55X 55Y 55Z | 1016 1142 1142 | - 55Z 55Y | 54X 54Y 54Z | 56X 56Y 56Z | 54W 54Z 54Y | 56W 56Z 56Y | 53X 53Y 53Z | 57X 57Y 57Z | - 53Z 53Y | - 57Z 57Y |
| 56X 56W 56Y 56Z | 1017 1017 1143 1143 | 56W 56X 56Z 56Y | 55X - 55Y 55Z | 57X - 57Y 57Z | - 55X 55Y 55Z | - 57X 57Y 57Z | 54X 54W 54Y 54Z | 58X 58W 58Y 58Z | 54W 54X 54Z 54Y | 58W 58X 58Z 58Y |
| 57X 57Y | 1018 1144 | - - | 56X 56Y | 58X 58Y | 56W 56Z | - - | 55X 55Y | 59X 59Y | - 55Z | - - |
| 58X 58Y | 1019 1145 | - - | 57X 57Y | 59X 59Y | - - | - - | 56X 56Y | 60X 60Y | 56W 56Z | - - |
| 59X 59Y | 1020 1146 | - - | 58X 58Y | 60X 60Y | - - | - - | 57X 57Y | 61X 61Y | - - | - - |
| 60X 60Y | 1021 1147 | - - | 59X 59Y | 61X 61Y | - - | - - | 58X 58Y | 62X 62Y | - - | - - |
| 61X 61Y | 1022 1148 | - - | 60X 60Y | 62X 62Y | - - | - - | 59X 59Y | 63X 63Y | - - | - - |
| 62X 62Y | 1023 1149 | - - | 61X 61Y | 63X 63Y | - - | - - | 60X 60Y | - - | - - | 64Y 64X |
| 63X 63Y | 1024 1150 | - - | 62X 62Y | - - | - - | 64Y 64X | 61X 61Y | - - | - - | 65Y 65X |
| 64X 64Y | 1151 1025 | - - | - - | 65X 65Y | 63Y 63X | - - | - - | 66X 66Y | 62Y 62X | - - |
| 65X 65Y | 1152 1026 | - - | 64X 64Y | 66X 66Y | - - | - - | - - | 67X 67Y | 63Y 63X | - - |
| 66X 66Y | 1153 1027 | - - | 65X 65Y | 67X 67Y | - - | - - | 64X 64Y | 68X 68Y | - - | - - |
| 67X 67Y | 1154 1028 | - - | 66X 66Y | 68X 68Y | - - | - - | 65X 65Y | 69X 69Y | - - | - - |
| 68X 68Y | 1155 1029 | - - | 67X 67Y | 69X 69Y | - - | - - | 66X 66Y | 70X 70Y | - - | - - |
| 69X 69Y | 1156 1030 | - - | 68X 68Y | 70X 70Y | - - | - - | 67X 67Y | 71X 71Y | - - | - - |
| 70X 70Y | 1157 1031 | - - | 69X 69Y | 71X 71Y | - - | - - | 68X 68Y | 72X 72Y | - - | - - |
| 71X 71Y | 1158 1032 | - - | 70X 70Y | 72X 72Y | - - | - - | 69X 69Y | 73X 73Y | - - | - - |
| 72X 72Y | 1159 1033 | - - | 71X 71Y | 73X 73Y | - - | - - | 70X 70Y | 74X 74Y | - - | - - |
| 73X 73Y | 1160 1034 | - - | 72X 72Y | 74X 74Y | - - | - - | 71X 71Y | 75X 75Y | - - | - - |
| 74X 74Y | 1161 1035 | - - | 73X 73Y | 75X 75Y | - - | - - | 72X 72Y | 76X 76Y | - - | - - |
| 75X 75Y | 1162 1036 | - - | 74X 74Y | 76X 76Y | - - | - - | 73X 73Y | 77X 77Y | - - | - - |
| 76X 76Y | 1163 1037 | - - | 75X 75Y | 77X 77Y | - - | - - | 74X 74Y | 78X 78Y | - - | - - |
| 77X 77Y | 1164 1038 | - - | 76X 76Y | 78X 78Y | - - | - - | 75X 75Y | 79X 79Y | - - | - - |
| 78X 78Y | 1165 1039 | - - | 77X 77Y | 79X 79Y | - - | - - | 76X 76Y | 80X 80Y | - - | - 80Z |
| 79X 79Y | 1166 1040 | - - | 78X 78Y | 80X 80Y | - - | - 80Z | 77X 77Y | 81X 81Y | - - | - 81Z |
| 80X 80Y 80Z | 1167 1041 1041 | - 80Z 80Y | 79X 79Y - | 81X 81Y 81Z | - - 79Y | - 81Z 81Y | 78X 78Y - | 82X 82Y 82Z | - - 78Y | - 82Z 82Y |
| 81X 81Y 81Z | 1168 1042 1042 | - 81Z 81Y | 80X 80Y 80Z | 82X 82Y 82Z | - 80Z 80Y | - 82Z 82Y | 79X 79Y - | 83X 83Y 83Z | - - 79Y | - 83Z 83Y |
| 82X 82Y 82Z | 1169 1043 1043 | - 82Z 82Y | 81X 81Y 81Z | 83X 83Y 83Z | - 81Z 81Y | - 83Z 83Y | 80X 80Y 80Z | 84X 84Y 84Z | - 80Z 80Y | - 84Z 84Y |
| 83X 83Y 83Z | 1170 1044 1044 | - 83Z 83Y | 82X 82Y 82Z | 84X 84Y 84Z | - 82Z 82Y | - 84Z 84Y | 81X 81Y 81Z | 85X 85Y 85Z | - 81Z 81Y | - 85Z 85Y |
| 84X 84Y 84Z | 1171 1045 1045 | - 84Z 84Y | 83X 83Y 83Z | 85X 85Y 85Z | - 83Z 83Y | - 85Z 85Y | 82X 82Y 82Z | 86X 86Y 86Z | - 82Z 82Y | - 86Z 86Y |
| 85X 85Y 85Z | 1172 1046 1046 | - 85Z 85Y | 84X 84Y 84Z | 86X 86Y 86Z | - 84Z 84Y | - 86Z 86Y | 83X 83Y 83Z | 87X 87Y 87Z | - 83Z 83Y | - 87Z 87Y |
| 86X 86Y 86Z | 1173 1047 1047 | - 86Z 86Y | 85X 85Y 85Z | 87X 87Y 87Z | - 85Z 85Y | - 87Z 87Y | 84X 84Y 84Z | 88X 88Y 88Z | - 84Z 84Y | - 88Z 88Y |
| 87X 87Y 87Z | 1174 1048 1048 | - 87Z 87Y | 86X 86Y 86Z | 88X 88Y 88Z | - 86Z 86Y | - 88Z 88Y | 85X 85Y 85Z | 89X 89Y 89Z | - 85Z 85Y | - 89Z 89Y |
| 88X 88Y 88Z | 1175 1049 1049 | - 88Z 88Y | 87X 87Y 87Z | 89X 89Y 89Z | - 87Z 87Y | - 89Z 89Y | 86X 86Y 86Z | 90X 90Y 90Z | - 86Z 86Y | - 90Z 90Y |
| 89X 89Y 89Z | 1176 1050 1050 | - 89Z 89Y | 88X 88Y 88Z | 90X 90Y 90Z | - 88Z 88Y | - 90Z 90Y | 87X 87Y 87Z | 91X 91Y 91Z | - 87Z 87Y | - 91Z 91Y |
| 90X 90Y 90Z | 1177 1051 1051 | - 90Z 90Y | 89X 89Y 89Z | 91X 91Y 91Z | - 89Z 89Y | - 91Z 91Y | 88X 88Y 88Z | 92X 92Y 92Z | - 88Z 88Y | - 92Z 92Y |
| 91X 91Y 91Z | 1178 1052 1052 | - 91Z 91Y | 90X 90Y 90Z | 92X 92Y 92Z | - 90Z 90Y | - 92Z 92Y | 89X 89Y 89Z | 93X 93Y 93Z | - 89Z 89Y | - 93Z 93Y |
| 92X 92Y 92Z | 1179 1053 1053 | - 92Z 92Y | 91X 91Y 91Z | 93X 93Y 93Z | - 91Z 91Y | - 93Z 93Y | 90X 90Y 90Z | 94X 94Y 94Z | - 90Z 90Y | - 94Z 94Y |
| 93X 93Y 93Z | 1180 1054 1054 | - 93Z 93Y | 92X 92Y 92Z | 94X 94Y 94Z | - 92Z 92Y | - 94Z 94Y | 91X 91Y 91Z | 95X 95Y 95Z | - 91Z 91Y | - 95Z 95Y |
| 94X 94Y 94Z | 1181 1055 1055 | - 94Z 94Y | 93X 93Y 93Z | 95X 95Y 95Z | - 93Z 93Y | - 95Z 95Y | 92X 92Y 92Z | 96X 96Y 96Z | - 92Z 92Y | - 96Z 96Y |
| 95X 95Y 95Z | 1182 1056 1056 | - 95Z 95Y | 94X 94Y 94Z | 96X 96Y 96Z | - 94Z 94Y | - 96Z 96Y | 93X 93Y 93Z | 97X 97Y 97Z | - 93Z 93Y | - 97Z 97Y |
| 96X 96Y 96Z | 1183 1057 1057 | - 96Z 96Y | 95X 95Y 95Z | 97X 97Y 97Z | - 95Z 95Y | - 97Z 97Y | 94X 94Y 94Z | 98X 98Y 98Z | - 94Z 94Y | - 98Z 98Y |
| 97X 97Y 97Z | 1184 1058 1058 | - 97Z 97Y | 96X 96Y 96Z | 98X 98Y 98Z | - 96Z 96Y | - 98Z 98Y | 95X 95Y 95Z | 99X 99Y 99Z | - 95Z 95Y | - 99Z 99Y |
| 98X 98Y 98Z | 1185 1059 1059 | - 98Z 98Y | 97X 97Y 97Z | 99X 99Y 99Z | - 97Z 97Y | - 99Z 99Y | 96X 96Y 96Z | 100X 100Y 100Z | - 96Z 96Y | - 100Z 100Y |
| 99X 99Y 99Z | 1186 1060 1060 | - 99Z 99Y | 98X 98Y 98Z | 100X 100Y 100Z | - 98Z 98Y | - 100Z 100Y | 97X 97Y 97Z | 101X 101Y 101Z | - 97Z 97Y | - 101Z 101Y |
| 100X 100Y 100Z | 1187 1061 1061 | - 100Z 100Y | 99X 99Y 99Z | 101X 101Y 101Z | - 99Z 99Y | - 101Z 101Y | 98X 98Y 98Z | 102X 102Y 102Z | - 98Z 98Y | - 102Z 102Y |
| 101X 101Y 101Z | 1188 1062 1062 | - 101Z 101Y | 100X 100Y 100Z | 102X 102Y 102Z | - 100Z 100Y | - 102Z 102Y | 99X 99Y 99Z | 103X 103Y 103Z | - 99Z 99Y | - 103Z 103Y |
| 102X 102Y 102Z | 1189 1063 1063 | - 102Z 102Y | 101X 101Y 101Z | 103X 103Y 103Z | - 101Z 101Y | - 103Z 103Y | 100X 100Y 100Z | 104X 104Y 104Z | - 100Z 100Y | - 104Z 104Y |
| 103X 103Y 103Z | 1190 1064 1064 | - 103Z 103Y | 102X 102Y 102Z | 104X 104Y 104Z | - 102Z 102Y | - 104Z 104Y | 101X 101Y 101Z | 105X 105Y 105Z | - 101Z 101Y | - 105Z 105Y |
| 104X 104Y 104Z | 1191 1065 1065 | - 104Z 104Y | 103X 103Y 103Z | 105X 105Y 105Z | - 103Z 103Y | - 105Z 105Y | 102X 102Y 102Z | 106X 106Y 106Z | - 102Z 102Y | - 106Z 106Y |
| 105X 105Y 105Z | 1192 1066 1066 | - 105Z 105Y | 104X 104Y 104Z | 106X 106Y 106Z | - 104Z 104Y | - 106Z 106Y | 103X 103Y 103Z | 107X 107Y 107Z | - 103Z 103Y | - 107Z 107Y |
| 106X 106Y 106Z | 1193 1067 1067 | - 106Z 106Y | 105X 105Y 105Z | 107X 107Y 107Z | - 105Z 105Y | - 107Z 107Y | 104X 104Y 104Z | 108X 108Y 108Z | - 104Z 104Y | - 108Z 108Y |
| 107X 107Y 107Z | 1194 1068 1068 | - 107Z 107Y | 106X 106Y 106Z | 108X 108Y 108Z | - 106Z 106Y | - 108Z 108Y | 105X 105Y 105Z | 109X 109Y 109Z | - 105Z 105Y | - 109Z 109Y |
| 108X 108Y 108Z | 1195 1069 1069 | - 108Z 108Y | 107X 107Y 107Z | 109X 109Y 109Z | - 107Z 107Y | - 109Z 109Y | 106X 106Y 106Z | 110X 110Y 110Z | - 106Z 106Y | - 110Z 110Y |
| 109X 109Y 109Z | 1196 1070 1070 | - 109Z 109Y | 108X 108Y 108Z | 110X 110Y 110Z | - 108Z 108Y | - 110Z 110Y | 107X 107Y 107Z | 111X 111Y 111Z | - 107Z 107Y | - 111Z 111Y |
| 110X 110Y 110Z | 1197 1071 1071 | - 110Z 110Y | 109X 109Y 109Z | 111X 111Y 111Z | - 109Z 109Y | - 111Z 111Y | 108X 108Y 108Z | 112X 112Y 112Z | - 108Z 108Y | - 112Z 112Y |
| 111X 111Y 111Z | 1198 1072 1072 | - 111Z 111Y | 110X 110Y 110Z | 112X 112Y 112Z | - 110Z 110Y | - 112Z 112Y | 109X 109Y 109Z | 113X 113Y 113Z | - 109Z 109Y | - 113Z 113Y |
| 112X 112Y 112Z | 1199 1073 1073 | - 112Z 112Y | 111X 111Y 111Z | 113X 113Y 113Z | - 111Z 111Y | - 113Z 113Y | 110X 110Y 110Z | 114X 114Y 114Z | - 110Z 110Y | - 114Z 114Y |
| 113X 113Y 113Z | 1200 1074 1074 | - 113Z 113Y | 112X 112Y 112Z | 114X 114Y 114Z | - 112Z 112Y | - 114Z 114Y | 111X 111Y 111Z | 115X 115Y 115Z | - 111Z 111Y | - 115Z 115Y |
| 114X 114Y 114Z | 1201 1075 1075 | - 114Z 114Y | 113X 113Y 113Z | 115X 115Y 115Z | - 113Z 113Y | - 115Z 115Y | 112X 112Y 112Z | 116X 116Y 116Z | - 112Z 112Y | - 116Z 116Y |
| 115X 115Y 115Z | 1202 1076 1076 | - 115Z 115Y | 114X 114Y 114Z | 116X 116Y 116Z | - 114Z 114Y | - 116Z 116Y | 113X 113Y 113Z | 117X 117Y 117Z | - 113Z 113Y | - 117Z 117Y |
| 116X 116Y 116Z | 1203 1077 1077 | - 116Z 116Y | 115X 115Y 115Z | 117X 117Y 117Z | - 115Z 115Y | - 117Z 117Y | 114X 114Y 114Z | 118X 118Y 118Z | - 114Z 114Y | - 118Z 118Y |
| 117X 117Y 117Z | 1204 1078 1078 | - 115Z 115Y | 116X 116Y 116Z | 118X 118Y 118Z | - 116Z 116Y | - 118Z 118Y | 115X 115Y 115Z | 119X 119Y 119Z | - 115Z 115Y | - 119Z 119Y |
| 118X 118Y 118Z | 1205 1079 1079 | - 118Z 118Y | 117X 117Y 117Z | 119X 119Y 119Z | - 117Z 117Y | - 119Z 119Y | 116X 116Y 116Z | 120X 120Y - | - 116Z 116Y | - - 120Y |
| 119X 119Y 119Z | 1206 1080 1080 | - 119Z 119Y | 118X 118Y 118Z | 120X 120Y - | - 118Z 118Y | - -S 120Y | 117X 117Y 117Z | 121X 121Y - | - 115Z 115Y | - - 121Y |
| 120X 120Y | 1207 1081 | - - | 119X 119Y | 121X 121Y | - 119Z | - - | 118X 118Y | 122X 122Y | - 118Z | - - |
| 121X 121Y | 1208 1082 | - - | 120X 120Y | 122X 122Y | - - | - - | 119X 119Y | 123X 123Y | - 119Z | - - |
| 122X 122Y | 1209 1083 | - - | 121X 121Y | 123X 123Y | - - | - - | 120X 120Y | 124X 124Y | - - | - - |
| 123X 123Y | 1210 1084 | - - | 122X 122Y | 124X 124Y | - - | - - | 121X 121Y | 125X 125Y | - - | - - |
| 124X 124Y | 1211 1085 | - - | 123X 123Y | 125X 125Y | - - | - - | 122X 122Y | 126X 126Y | - - | - - |
| 125X 125Y | 1212 1086 | - - | 124X 124Y | 126X 126Y | - - | - - | 123X 123Y | - 1Y | - - | - - |
| 126X 126Y | 1213 1087 | - - | 125X 125Y | - 1Y | - - | - - | 124X 124Y | - 2Y | - - | - - |

Note:	The DME channelling and frequency pairing (see figure below) results in “anomalous” adjacent channel combinations around 63X/64X and 63Y/64Y (e.g. 63X and 64Y). The term “up-link FREQ” is used instead of “reply FREQ”.

## MLS

### Note: the MLS material has been temporarily removed, pending conclusion of the on-going work in various groups.  For any additional information on this subject please consult FMG/19 WP02 and the ICAO EUR FMG.

## Identifications of Radio Navigation Aids

  - References to Documents

-	Annex 11, Appendix 2, paragraph 2.2

-	Annex 10, Volume I, paragraphs 3.1.3.9, 3.3.6, 3.4.5, 3.5.2.5, 3.5.3.6, 3.11.4.8.2.1(n) and Appendix B, paragraph 3.6.3.4.1

(ILS, VOR, NDB, DME, MLS and GBAS);

-	European Region Air Navigation Plan (Doc 7754), Volume II, Part III.

  - Composition of Identifications

    - Identifications of navigation aids shall be so composed, if possible, as to facilitate association with the name of the point in plain language. The table below provides an overview of the requirements which have to be taken into account for identifications:

| Facility | Identification | Remarks |
| --- | --- | --- |
| NDB and Locator | 2 or 3 letters of the International Morse Code. See Note 2 below |  |
| ILS | 2 or 3 letters of the International Morse Code which may be preceded by the letter I. | Transmitted by the localizer. |
| ILS/DME | ILS: 2 or 3 letters of the International Morse Code which may be preceded by the letter I. DME: Identical to associated ILS. | Associated DME identification signal. |
| VOR | 2 or 3 letters of the International Morse Code. |  |
| VOR/DME (VOR/TACAN also called VORTAC) | VOR: 2 or 3 letters of the International Morse Code. DME: Identical to associated VOR. | Associated DME identification signal. |
| DME (TACAN) | Letters and numerals of the International Morse Code. | Independent DME identification signal for stand-alone DME facility. |
| MLS | 3 letters A to Z. The characters shall be transmitted as a digital word and encoded in accordance with the International Alphabet No. 5. A letter M is added at the beginning of the identification, but not transmitted. | No MLS configurations without associated DME are planned. |
| MLS/DME | MLS: 3 letters A to Z. DME: Identical to associated MLS. | See further details under MLS above. Associated DME identification signal. |
| ILS/MLS/DME | ILS: 3 letters of the International Morse Code which may start with the letter I. See Note below. MLS: Identical to associated ILS. DME: Identical to associated ILS. | Triple frequency pairing. See further details under MLS above. Associated DME identification signal. |
| GBAS | 4 characters (upper case letters, numeric digits, space) of the International Alphabet No. 5. Use of the Location Indicator of nearest airport is recommended. The fourth character may be a space (i.e. 3 letter identifications are acceptable). |  |

Note 1:	4 letter ILS identifications do not fit into the triple frequency pairing requirements because of the fixed number (3) of letters to be used for MLS and the associated DME.

    - A letter M is added ahead of the MLS identification as described above. This letter M is not transmitted. Since it is added to all MLS identifications, it does not provide any additional information which is not already available. It may be useful for operational purposes, but should not be included in the planning process for identifications of radio navigation aids. With such an arrangement, it is sufficient to co-ordinate and to register one single identification for each collocated MLS/DME and ILS/MLS/DME facility.

    - The identification TST is reserved for radio navigation facilities on test and should not be used for other purposes.

    - The two following combinations of letters should not be used for identifications (FMG Decision 4/2): GAT and OAT.

  - Duplication

    - Duplication of the same identification of a navigation aid should, as far as possible, be avoided in the EUR Region and in any case should not be permissible for two aids located less than 600 NM apart. Due to these provisions, an aircraft at the maximum flight level will never be within radio line-of-sight of more than one radio navigation facility using a specific identification.

    - The provisions of 7.3.1 above do not apply when two radio navigation aids operating in different bands of the frequency spectrum are situated at the same location, then their identifications are normally the same.

  - Co-ordination and Registration

    - Identifications should be subject to co-ordination and registration in the EUR Region in the same way as frequencies of navigation aids.

## Non-ICAO Standard Systems operating in Part or in Total within ARNS Bands

RSBN

8.1	RSBN4 is a system that can provide information for APP and En Route Navigation similar as ILS, VOR, DME and TACAN. RSBN4 uses for one system component the same frequencies as DME/TACAN operating below channel 40x which is, why interference with DME and TACAN is possible. This information was already presented as WP12 at the FMG-SG7 meeting in 2004, but unfortunately no conclusion about further actions were reached. Continued operation could have also an impact on a future ARNS system in the band.

Appendix A to Part III

Convention for indicating the angular limits of sectorization in range

1.	The range values given for ICAO category facilities are the ICAO recommended designated operational ranges and, for national facilities, the declared operational range requirement. In both cases the ranges are normally circular, i.e. of the same value throughout 360° degrees (°).

2.	Where, however, the designated or declared values of operational range are not the same in all sectors, the following method is used to indicate the angular limits of sectors.

Method

3. The angles are measured clockwise along a horizontal plane from the True North direction (Geographical North): 0° is True North, 90° is East, 180° is South, etc.

4. An angle is expressed by a three-digit number and can have any value between 000 and 359. 
Example: 90° is noted 090.

5. A sector is defined by a range value in NM followed by a pair of angles into brackets in degrees separated by “:”
Example: 80(130:180) means 80NM between 130° and 180°

6. [A sector is always defined clockwise.]

7. If several sectors have the same range, the range is followed by the sectors separated by a “+”
Example: 60(065:095 +160:200) means 60NM between 65° and 95° and between 160° and 200°

8. A sectorization is provided using a series of sectors. At the end of the sectorization a range value preceded by  “/”  can be provided, to indicate the range for all the angles outside the defined sectors
Example: 80(110:170)/60 means 80NM between 110° and 170° and 60NM elsewhere

9. Illustrative examples of sectorization

Illustrative examples of sectorization

(Figures below)

10. 	The sectorization as denoted is best described by the following examples.

Example 1. 80(000:090)/60 means 80 NM in sector 000° - 090° and 60 NM in other directions.

Example 2. 100(045:315)/60 means 100 NM in sector 315° - 045°, clockwise, and 60 NM in other directions.

Example 3. 100(045:315)/60 means 100 NM in sector 045° - 315°. Note that this is a sectorization that is the reverse of Example 2.

Example 4. 80(045:135+225:315)/60means 80 NM in sector 045° - 135 degrees, clockwise, and 225° - 315° also clockwise while 60 NM in what remains, that is the intermediate sectors 135° - 225° and 315° - 045°.

Note: The method applied before FMG#25 (2019) which had divided a circle into 24 equal radials, each designated with a letter taken from A to X, is kept for reference purposes in the Supplement to Doc 011. It should not be used anymore in coordination and registration of frequency assignments.

Appendix B

Generic calculation method for geographical separation distances

Propagation characteristics

It is assumed that propagation characteristics along the paths of the desired and undesired signals are identical. Calculation of attenuation is based on free space propagation from the transmitter up to the radio line-of-sight distance and on a constant attenuation factor beyond radio line-of-sight.

Calculation of required separation distance

The free space propagation formula for the power flux-density:

= 										(1)

where

PBfdB	= power flux-density [W/mP2P]

EIRP	= equivalent isotropically radiated power [W]

d	= distance from transmitter [m]

Formula (1) can be expressed in logarithmic terms and using NM instead of m for the distance:

PBfdB = EIRP – 20 log d – 10 log 4π – 20 log 1852

PBfdB = EIRP – 20 log d – 76.3								(2)

where

PBfdB	= power flux-density from desired transmitter [dBW/mP2P]

EIRP	= equivalent isotropically radiated power of the desired transmitter [dBW]

d	= distance from transmitter [NM]

The desired facility is protected against the undesired facility if:

D/U  PBfddB – PBfduB									(3)

where

D/U	= protection ratio [dB]

PBfddB	= power flux-density from desired transmitter [dBW/mP2P]

PBfduB	= power flux-density from undesired transmitter [dBW/mP2P]

Applying formula (2) to the critical point P (figure):

D/U  EIRPBdB – EIRPBuB – 20 log RBdB + 20 log RBuB

D/U  K + 20 log ()								(4)

where

EIRPBd	B= equivalent isotropically radiated power of the desired transmitter

Note:	The value of EIRPBdB is derived based on a required minimum field strength at the edge of coverage of the desired facitlity, assuming free space propagation. This value is fictious and does not correspond to the actually required radiated power. For example, for GBAS, the minimum required field strength at the edge of coverage at a range of 23 NM is 215 μV/m (-99 dBW/mP2P), which corresponds to 4 dBW (this value can be derived applying formula (2)).

EIRPBuB	= equivalent isotropically radiated power of the undesired transmitter

K	= EIRPBdB - EIRPBuB [dB]

RBdB	= operational range of the desired facility [NM]

RBuB	= distance of the undesired facility from the edge of the DOC of the desired facility [NM]

The required separation distance RBsB can be derived from formula (4):

D/U – K  20 log ()

(D/U – K)/20  log ()

10P(D/U-K)/20P  RBuB/RBdB			with	RBsB	= RBdB + RBuB

RBsB  RBdB (1 + 10P(D/U-K)/20P)								(5)

This formula applies, if the undesired transmitter is within radio line-of-sight from the edge of the DOC of the desired facility:

RBsB - RBdB  RBRLOSB				with	RBRLOSB	= 1.23 (+)

RBsB  RBdB + 1.23 (+)	[NM]

where

h	= height of the DOC of the desired facility [ft]

hBaB	= antenna height of the undesired facility [ft]

Beyond radio line-of-sight, a constant attenuation factor is used to calculate the path loss. Applying formula (2):

PBfduB = EIRPBuB – 20 log RBRLOSB – 76.3 – a(RBuB – RBRLOSB)

Applying formula (4):

D/U  K + 20 log () + a(RBuB – RBRLOSB)

D/U – K – a(RBsB – RBdB – RBRLOSB)  20 log ()

a(RBsB – RBdB – RBRLOSB)  –20 log () + D/U – K

RBsB  RBdB + RBRLOSB + [D/U – K – 20 log ()]/a

RBsB  RBdB + 1.23 (+) + [D/U – K – 20 log (1.23 (+)/RBdB)]/a		(6)

where

a	= attenuation factor beyond radio line-of-sight [dB/NM]

a can be derived from Recommendation ITU-R P.528-2 Propagation curves for aeronautical mobile and radionavigation services using VHF, UHF and SHF bands.

The following procedure should be applied to determine the required separation distance RBsB between two facilities:

- Determine RBsB = RBdB (1 + 10P(D/U-K)/20P);

- if RBsB  RBdB + 1.23 (+), then RBsB is the required separation distance;

- if RBsB > RBdB + 1.23 (+), then determine
RBsB = RBdB + 1.23 (+) + [D/U – K – 20 log (1.23 (+)/RBdB)]/a;

- repeat steps (1) to (3) with reversed roles for the desired and undesired facilities; and

- the greater separation distance resulting from the two runs desribed above is the required separation distance.

where

RBsB	= separation distance [NM]

RBdB	= operational range of the desired facility [NM]

D/U	= desired to undesired signal ratio required to protect the desired facility [dB]

K	= EIRP of the desired minus EIRP of the undesired transmitter [dB]

h	= height of the DOC of the desired facility [ft]

hBaB	= antenna height of the undesired facility [ft]

a	= attenuation factor beyond radio line-of-sight [dB/NM]

# Part IV	Frequency Co-ordination and Registration Procedures

## Introduction

  - Co-ordination and registration procedures for frequency assignments are usually agreed between States under the rules of the ITU. These procedures are agreed between States in order to assure mutual acknowledgement of (the status of) each other's frequency assignments and the corresponding rights and obligations under ITU rules. Within the aeronautical ICAO community in Europe, the FMG has established co-ordination procedures between States, which can be regarded as mirror procedures to the formal ITU versions. Although these procedures do not have the same formal status as the original ITU-ones, in practice they are treated by States concerned in much the same way. This document highlights the main parties involved in these procedures and their mutual relationship/responsibility.

## General

  - The main part of this document contains the basic co-ordination and registration procedure for use in the European Region.

  - The implementation of co-ordination and registration procedures is supported by SAFIRE, a dedicated internet-based facility developed and managed by EUROCONTROL on behalf of the FMG and the Registration Office (ICAO EANPG Conclusion 50/20 refers)*. The performance of SAFIRE is regularly reviewed by the FMG. States are strongly encouraged to register as users of SAFIRE in order to increase individual and overall efficiency.

  - Co-ordination of new or modified frequency assignments is required with all States concerned that  may be affected by the proposed assignment.

  - Special procedures apply if it is necessary to change the frequency of an existing assignment in order to satisfy a request for a new assignment or for a modification of another existing assignment. In these circumstances, the co-ordination activity should encompass also the States that may be involved in the required frequency shifts of existing assignments as well as the States that may be concerned with these frequency shifts. EUROCONTROL has established on behalf of the FMG specialized co-ordination mechanisms for the satisfaction of such frequency requests. States are encouraged to request the support of EUROCONTROL when faced with such frequency requests.

  - The ICAO Paris Office would be available to assist States in resolving any co-ordination difficulties or disputes.

* EANPG Conclusion 50/20 - Implementation of the SAFIRE for COM2 coordination:

That starting on 1 January 2009, the on-line system SAFIRE be used as the only means of frequency assignments coordination and registration for COM2 Tables in the ICAO EUR Region.

Note : Table COM2 was renamed Table COM as per EASPG Conclusion 1/20 (December 2020)

## Basic Co-ordination and Registration Procedures for Aeronautical Frequency Assignments

  - The basic procedure for the co-ordination and registration of aeronautical frequency assignments is depicted in the figure of Section 3 of Part I of this manual (page 3). As can be seen from this, the main parties involved are:

1)	States having a requirement for an addition or modification of an assignment to a service;

2)	all States concerned with a proposed  new assignment or modification of an existing assignment to a service;

3)	the Registration Office (ICAO EUR/NAT Office).

It is clarified that in the co-ordination and registration process participate also States from adjacent ICAO regions.

  - For the implementation of the co-ordination and registration procedures described below, the following arrangements apply:

1)	States registered to SAFIRE for a given category (COM, NAV) of assignments (referred to as SAFIRE States henceforth) make mandatorily use of the appropriate utilities of the facility. Under normal conditions,, SAFIRE States are expected to execute their tasks exclusively on SAFIRE.

2)	States not registered to SAFIRE for a given category of assignments (referred to as non-SAFIRE States henceforth) should  exchange co-ordination and registration messages as described in the following paragraph.

3)	With the exception of bilateral negotiations, the exchange of information between a SAFIRE State and a non-SAFIRE State or between two non-SAFIRE States is effected through the mediation of the Registration Office. In particular, requests,  reactions or notifications of non-SAFIRE States which concern SAFIRE States or other non-SAFIRE States are sent to the Registration Office, which in turn produces the appropriate entries into SAFIRE. Conversely for requests,  reactions or notifications of SAFIRE or non-SAFIRE States which concern non-SAFIRE States, the Registration Office retrieves the corresponding information entered in SAFIRE and generates the appropriate messages to the non-SAFIRE States concerned. In their communication with the Registration Office for co-ordination and registration purposes, non-SAFIRE States may usethe dedicated e-mail address icaoeurnat@paris.icao.int.

4)	 For the distribution of co-ordination-related material entered in SAFIRE to concerned non-SAFIRE States the Registration Office is supported by automation. Unless a non-SAFIRE State informs the Registration Office of the list of SAFIRE States from which it wishes to receive co-ordination-related messages, the Registration Office shall forward to such non SAFIRE State all SAFIRE co-ordination-related messages.

  - The various actions and responsibilities of each of the parties in the co-ordination and registration procedures are described in the paragraphs below.

  - A State having a requirement for an addition/modification of an assignment to a service is responsible for:

  - the request for co-ordination and registration to be submitted to all States concerned and the Registration Office (Note: Proposals establish priority with respect to their provisional protection according to their time of submission to the Registration Office);

  - in case any objections are raised:

    - negotiating with objecting State(s) on details of the proposed assignment with a view to solving identified incompatibilities;  or

    - the withdrawal of the objected proposal and corresponding notification to all States concerned and the Registration Office

Note 1: In case of objection, requesting States may extend their bilateral negotiations with objecting States beyond the 4-week period of paragraph 3.7 b) up until 3 months from the submission of the co-ordination request.

Note 2: Requests for modifications of the remarks field or of the location name field as well as VHF COM conversions to 8.33 kHz channel spacing on the same frequency may reach the Registration Office without prior co-ordination.

Note 3: Care of the Registration Office,  objected proposals, , are deleted from the pertinent databases after the lapse of  3 months from their submission.  Objected proposals under investigation by the Registration Office are not subject to removal.

  - Deletions of assignments are submitted to the Registration Office and notified to all States concerned.

  - All States concerned with a possible impact of a proposed addition/modification of an assignment on one or more of their registered assignments or proposals are responsible for:

  - the protection of their registered assignments and proposals against incompatibility with the proposed addition/modification;

  - the proper reaction to the requesting State and the Registration Office, based on the examination of a proposal as above, within 4 weeks from the notification of the co-ordination request (Note: No answer by the target date will be taken as an approval); and

  - the proper reaction to the requesting State and the Registration Office  in the eventof an improperly  registered proposal.

Note 1: Valid objections can be raised against proposals which do not contain all information required for compatibility assessment or contain implausible data (e.g. frequencies not complying with the appropriate pairing tables, in the case of paired services).

  - After the lapse of the 4-week period of paragraph 3.6b), non-objected proposals are registered in the appropriate ICAO EUR COM or NAV table of frequency assignments. Objected proposals are registered in the appropriate table only after all objections to these proposals are withdrawn.

Note 1: If bilateral negotiations are foreseen to last longer than 4 weeks, States having valid concerns about a proposal are advised to object this proposal, without prejudice to the outcome of the negotiations.

Note 2: Until SAFIRE supports fully the automation of this step, manual action will be required for the registration of proposals.

  - The Registration Office is responsible for:

  - the registration of assignments and in general for the maintenance of the COM tables, NAV Tables and associated databases, both in terms of form and content;

  - the archiving of the co-ordination registration requests of non-SAFIRE States; and

  - the publication on the ICAO web-site of the updated COM and NAV tables on a semiannual basis;

  - inter-regional co-ordination with adjacent ICAO Region Offices, in the sense of the exchange of critical information for the co-ordination and the registration of assignments;

  - providing the mediation described above in paragraph 3.3.3), as the need of information exchange arises between SAFIRE and non-SAFIRE states; and

  - the oversight of SAFIRE operation.

  - The format of the messages used for information exchange over AFTN should follow the guidelines described in the Appendices A, B and  C. Appendix D (templates and examples° clarifies the form and the content of the information to be exchanged.

  - A proposing State faced with an unwarranted objection may submit a registration request to the Registration Office with the corresponding notification to all States concerned. This request should be accompanied with the appropriate documentation. The Registration Office considers the request on condition that sufficient evidence is provided that the proposing State has diligently sought the withdrawal of the objection by the objecting State(s).

  - Taking due account of the views of the parties involved, the Registration Office performs then an examination of the case with respect to the conformity with the procedures, the technical requirements and the general principles governing the safe and efficient use of the aeronautical frequency spectrum.

  - In this task the Registration Office may ask the assistance of a group of experts on an ad-hoc or a permanent basis at which the representatives of the concerned parties may be present to introduce their views.

  - If the finding of the examination of the registration request is favourable, the proposal is registered by the Registration Office in the appropriate COM or NAV table with an indication of the objecting State(s). The assignment then enjoys full protection rights with respect to all States including the objecting States. The outcome of the examination shall be notified to the parties concerned.

  - Concerned States disagreeing with a finding of the Registration Office may request the review of it. The review of a finding may also be undertaken on the initiative of the Registration Office itself when it considers it justified.

  - On applying FMG decisions, the Registration Office is authorised to relegate a proposal or an assignment to the status of an objected proposal. Such a measure may be for instance required in preserving the rights of States in relation to the priority of submission of co-ordination material

  - For the basic steps of the co-ordination and registration procedure, the actions of SAFIRE and non-SAFIRE States are clarified below:

|  | Non-SAFIRE States | SAFIRE States |
| --- | --- | --- |
| Submission of a proposal | Send appropriate message to the Registration Office | Make appropriate entry into SAFIRE |
| Reaction to a proposal | Send appropriate message to the Registration Office | Make appropriate entry into SAFIRE |
| Registration request for a proposal | Send appropriate message to the Registration Office | Make appropriate entry into SAFIRE |

Appendix A: Country codes and AFTN addresses to be used for frequency co-ordination

| Organization/State | ITU Country Code | AFTN address | E-mail or fax |
| --- | --- | --- | --- |
| ICAO Regional Office (ICAO Paris) | - | LFPSYAYU | icaoeurnat@icao.int |
| EUROCONTROL | - | LKINEURO | frequency.management@eurocontrol.int |
| Albania | ALB | LATIYFYX | illir.bano@dgca.gov.al |
| Algeria | ALG | DAAAYKYD | fmg_enna_com234@enna-dz.com |
| Armenia | ARM |  | +37 41 228 2066 |
| Austria | AUT | LOWWYNYX | frequencies@austrocontrol.at |
| Azerbaijan | AZE |  | farhan.guliyev@azans.az |
| Belarus | BLR | UMMDYKYX | vkaliaha@ban.by |
| Belgium | BEL | EBVAYTYX | vdh@skeyes.be |
| Bosnia and Herzegovina | BIH | LQBHYATS | jasmina.bogdanic@bhansa.gov.ba |
| Bulgaria | BUL | LBSFYAYX | freqmng@bulatsa.com |
| Croatia | HRV | LDZAYATS | frqmng@crocontrol.hr |
| Cyprus | CYP | LCNCYAYX | cmarouchos@dca.mcw.gov.cy |
| Czech Republic | CZE | LKPRYAYX | frekvence@mdcr.cz |
| Denmark | DNK | EKCAYAYX | fkm@trafikstyrelsen.dk |
| Egypt | EGY | HECAYAYX |  |
| Estonia | EST | EETNYTYX |  |
| Finland | FIN | EFCAYAYX | radiotaajuudet@traficom.fi |
| France | F |  | vhf.dsna-dti@aviation-civile.gouv.fr |
| Georgia | GEO |  | m.mikhailov@airnav.ge |
| Germany | D | EDDAYGNA | frequm@baf.bund.de |
| Greece | GRC |  | d4d@hcaa.gr |
| Hungary | HNG | LHBPYEYX | caa@itm.gov.hu |
| Ireland | IRL | EIDWYAYX | michael.oconnor@iaa.ie |
| Israel | ISR | LLAAYYYX |  |
| Italy | I | LIIRYTYX | silvio.zappi@enav.it |
| Jordan | JOR | OJAMYAYX |  |
| Kazakhstan | KAZ |  |  |
| Kyrgyzstan | KYR |  | dga@infotel.kg |
| Latvia | LVA | EVRRYTYX | edgars.dreijers@latcaa.gov.lv |
| Lebanon | LBN | OLBAYFYX |  |
| Libya | LBY | HLLTYAYX |  |
| Lithuania | LTU |  | algirdas.naraskevicius@rrt.lt |
| Luxembourg | LUX | ELLXYAYF | Jose.hidalgo@av.etat.lu |
| Malta | MLT | LMMLYAYA | george.borg-mark@gov.mt |
| Moldova | MDA | LUKKYAYX | alla.toma@caa.gov.md |
| Morocco | MRC | GMMRYHDT | nmassali@dac-maroc.gov.ma |
| Netherlands | HOL | EHGGYIEZ | aviation@agentschaptelecom.nl |
| Norway | NOR |  | nfmn@caa.no |
| Poland | POL | EPWAYFMG | Katarzyna.grenda@pansa.pl |
| Portugal | POR | LPPTYVYW | fernando.simoes@nav.pt |
| Romania | ROU | LRBBYDYT | sergiu.dumitrescu@caa.ro |
| Russian Federation | RUS | UUUUYTYX | gaynov_ma@fana.ru |
| Serbia | SRB | LYBBYTYX | radmila.papulic@smatsa.rs |
| Slovak Republic | SVK | LZIBYQYC | sap@caa.sk |
| Slovenia | SVN | LJLAYAYX | jure.locniskar@sloveniacontrol.si |
| Spain | E | LEANZXIC | Safire_Spain@enaire.es |
| Sweden | S | ESALYAYX | Morgan.Sundell@transportstyrelsen.se |
| Switzerland | SUI | LSSOYAYF | safire_sui@bakom.admin.ch |
| Syrian Arab Republic | SYR | OSDSYTYX |  |
| The Former Yugoslav Republic of Macedonia | MKD | LWSKYKYT | patcev@dgca.gov.mk |
| Tunisia | TUN | DTTCYDNA | malikarekik@yahoo.fr |
| Turkey | TUR | LTAAYAYT | caglar.sisman@shgm.gov.tr; syd_hava@btk.gov.tr |
| Turkmenistan | TKM |  |  |
| Ukraine | UKR | UKKAYTYD | ukrcns@avia.gov.ua |
| United Kingdom | G | EGGAYTYF | spectrum@caa.co.uk |
| Uzbekistan | UZB |  |  |

Note 1: E-mail and fax are the only method of communication with ICAO EUR/NAT Office

Note2: Proposals for the assignment of frequencies or identifications have to be addressed to States concerned. However, the following States would like to get copies of all proposals and registration messages: BEL, D, F, G and HRV.

Appendix B:	A unique message reference system

The National Co-ordination Reference (NCR)

| Field name | Explanation | Number of characters |
| --- | --- | --- |
| Country Code | ITU Country Code (ref. Appendix A) | 1 or 3 letters |
| Type of service (COM Table number) | 2 communications (Table COM) 3 VHF/UHF/SHF navigation (Table NAV) 4 LF/MF navigation (Table NAV) | 1 digit (2, 3 or 4) |
| Sequential number | A unique sequential number for each type of service | 4 digits |
| Co-ordination phase | Indicates the phase of the co-ordination: P – Proposal S – Standard Updating Message | 1 |
| Type of amendment | Indicates the different amendments. Addition of a new assignment (ADDITION) Modification of an existing assignment (MODIFICATION) Deletion of an existing assignment (DELETION) Withdrawal of a proposal (WITHDRAWAL) | 10, 12 or 14 |

Note 1: A NCR should be associated with only one assignment. If a co-ordination message refers to more than one assignments then a separate NCR should be used for each one of them.

Note 2: Frequency changes of registered assignments may be co-ordinated in steps – as additions of a new assignment identical to the existing except the frequency and later, after the new frequency has become operational, with a deletion of the old assignment using different sequential numbers for addition and deletion.

Example:	HOL 2 0027 S (ADDITION)

| Country Code | Type of service | Sequential number | Co-ordination phase | Type of amendment |
| --- | --- | --- | --- | --- |
| HOL | 2 | 0027 | S | (ADDITION) |

This NCR should be used to register an additional assignment to the VHF communications list for the Netherlands. The NCR of the next co-ordination message of the Netherlands for the same type of service should start with HOL 2 0028.

Basic principles for the use of a NCR in the co-ordination process:

- Each addition of a new assignment or modification or deletion of an existing assignment shall be referenced with an individual NCR.

- The same NCR will be used for an assignment during a complete co-ordination process.

- If a proposal is withdrawn then the NCR sequential number of the withdrawal message shall be identical to the one used in the corresponding proposal.

Appendix C:	 Mandatory Coordination Data

It is mandatory to include the data fields set out in the tables D1 to D3 below in all frequency coordination proposals.

The mandatory data items marked * have no impact on compatibility with other frequency assignments. When modifying existing frequency assignments, changes to these data items are permitted without prior coordination with other States.

COM 2

National Coordination Reference

Country Code

Location name*

Channel

Service type

DOC

Coordinates (not applicable to area assignments)

Service area name (for all area assignments)

Polygon coordinates (for all area assignments)

Climax service and number of legs (for all Climax services)

Coordinates of transmitting and receiving ground stations (applicable to coordinations after 31 December 2015 in specified cases as set out below)

Table D-1 – Table COM Mandatory Data Fields

Coordination of COM  ground stations

The location of COM  transmitting and receiving ground stations shall be coordinated in the following cases:

- Area services where the transmitting ground stations are outside of the DOC;

- Circular services where the transmitting ground stations are not within 3NM of the DOC centre;

- Transmitting or receiving ground stations that lie within 10 NM of an adjacent State’s landmass;

- Cases where the inclusion of the transmitting or receiving ground station location is a precondition for accepting an incompatible proposal.

It is recommended that for Climax services, the position of the ground stations is coordinated where possible.

COM 3

| ILS | VOR | DME/TACAN | GBAS | MLS |
| --- | --- | --- | --- | --- |
| National Coordination Reference |  |  |  |  |
| Country Code |  |  |  |  |
| Facility type (e.g. ILS, VOR, ILS/DME, VORTAC etc.) |  |  |  |  |
| Location name* |  |  |  |  |
| DOC |  |  |  |  |
| Identification |  |  |  |  |
| Coordinates (not applicable to area assignments) |  |  |  |  |
| Service area name (for all area assignments) |  |  |  |  |
| Polygon coordinates (for all area assignments) |  |  |  |  |
| Frequency | Frequency | Channel | Frequency | Channel |
| - | EIRP | EIRP (PEP) | EIRP | - |
| Runway served (required in cases where TRD has not been coordinated) | - | - | - | Runway served (required in cases where TRD has not been coordinated) |
| - | - | Directional DME (Y/N) | Elevation (amsl) | - |
| True radiation direction (TRD) required for all new coordinations | - | TRD (required for directional DMEs only) | Antenna polarisation - GBAS/H (default) or GBAS/E | - True radiation direction (TRD) required for all new coordinations |
|  |  |  | Time slot number(s) |  |
|  |  |  | Data selector number(s) (RPDS and RSDS) |  |

Note: TRD and runway information not applicable to area assignments

Table D-2 – Table NAV Mandatory Data Fields

NAV

National Coordination Reference

Country Code

Location name*

Frequency

Service type – NDB or Locator

Identification

Coordinates (not applicable to area assignments)

Range

Service area name (for all area assignments)

Polygon coordinates (for all area assignments)

Table D-3 – Table NAV Mandatory Data Fields

## 4	Procedure to allocate or request a New UHF Channel for ATC Communications in GAT

4.1	 General Overview

4.1.1	The UHF band (225-400 MHz) within NATO Member States is collectively managed by the NATO Frequency Management Sub-Committee (FMSC), with the technical and administrative support of the Spectrum Management Branch (SMB) which is a permanent Staff located at NATO Headquarters (NATO HQ), Brussels.

4.1.2	When a request for assignment in the UHF band is received at SMB, it will be checked for correctness and run through the NUFAS tool (NATO UHF Assignment Software) that will identify a proper UHF channel taking into account all geographical, technical and administrative details provided in the request. Once the frequency is identified and assigned, the assignment record is stored in the SMB database and a copy is sent back to the national agency (NARFA or equivalent office) of the requesting State (CASE 1 for NATO State or CASE 2 for Non NATO State).

Important Note:

Since the band is managed centrally by SMB which maintains a central database of all UHF Air/Ground/Air assignments, the Nations do not normally nominate or propose frequencies in the requests; SMB assigns directly the best suitable frequency for the request.

4.2	Case 1: For NATO Member States

4.2.1	The procedure for NATO Member States to get a deconflicted frequency assignment in the NATO UHF band (225-400 MHz) is governed by a NATO Agreement which is of application throughout NATO Europe. Every NATO Member State has within its Ministry of Defence a National Radio Frequency Agency (NARFA), or equivalent office, which acts as the point of contact between the Nation and the SMB at NATO HQ. NARFAs collect requests from their National Users, either military or civilian, willing to operate in the UHF band, and forward them to the SMB. The format of the request coming from the national users to its NARFA is a national decision, but the format of the request forwarded to SMB must use the NATO “Spectrum Management Allied Data Exchange Format” (SMADEF). A free software tool (ARCADE), is distributed by SMB to help producing requests in SMADEF form.

4.3	Case 2: For Non NATO Members

4.3.1.	These States that are not Member of NATO but that belong to EUR/NAT ICAO Region may also get support from the SMB at NATO HQ in the process of deconflicting UHF channels to be used for Civilian Communications. This deconfliction is necessary due to the large coordination distances involved to avoid interferences.

4.3.2	The Appendix A is a simplified request form, based on the information normally provided in SMADEF format. It lists the information that, as a minimum, is necessary to the SMB to run their Frequency Assignment System in order to identify an available UHF channel. This form may be subject to changes that will be notified by NATO as necessary. An example of a filled-in questionnaire is also attached.

Notes:

- Other information may be required by SMB from those States on a case by case basis and this will be done by bilateral coordination with the Point Of Contact mentioned in the request form.

- Non NATO Member States who wish it are free to use the full SMADEF format and associated free software tool ARCADE, in place of the simplified form at Appendix X.

Appendix A: UHF Request Form:

| Items | Data | Example |
| --- | --- | --- |
| Geographical data |  |  |
| Station Name | Up to 24 characters | ORLY ARPT |
| Antenna Location | dddmmssE/W - ddmmssN/S | 0022157E - 484352N |
| Antenna Site Elevation (AMSL in meters) | MMMMM | 88 m |
| 1/2 Service Radius (in KM) | KKKK | 78 Km |
| 1 Aeronautical Service Height (in feet) | fffff | 25000ft |
| Technical data |  |  |
| Transmitter Nomenclature (name) | Not essential | R&S Serie 200 |
| Tuning lower limit (in MHz) | Mxxx.xxx | M225 |
| Tuning upper limit (in MHz) | Mxxx.xxx | M399.975 |
| Tuning increment | 25 or 50kHz | 25K |
| Maximum output power (in dBW) | xx.x | 17.0 |
| Antenna Nomenclature (name) | Not essential | HK001 |
| Antenna Type | E.g.UHF Broadband, Whip,.. | OMNIDIRECTIONAL |
| Antenna Polarisation | V, H, M,.. | V |
| Antenna Gain (in dBi) | -/+xx.xxdBi | +2dBi |
| Emission Designator (ITU code) | E.g. 6K00A3E-- | 6K00A3E-- |
| VHF Assigned at the site(to avoid harmonics) | Mxxx.xxx, Mxxx.xxx,… | M131.35, M123.875, M124.45, M118.7, M120.5, M121.7, M121.825, M127.75, M128.375, M121.05 |
| Forbidden or non desirable Freq(or band) | Mxxx.xxx to Mxxx.xxx | M225 to M230 M235.125 M380 to M385 M390 to M395 |
| Administrative data |  |  |
| Owner ITU Country code | C,CC or CCC | F |
| Call sign or ICAO Location identifier | LLLL | LFPO |
| Point Of Contact Data | Name, Tel, Fax, E-mail,… | M. DUCHEMIN Tel: +33.1.22.33.44.55 m.duchemin@skynet.fr |
| Customer Reference Number | If any | - |
| Operational Date | YYYYMMDD | 20070601 |
| Expiration Date (+ 5 Years by default) | YYYYMMDD | 20120630 |
| Remarks | All pertinent information not covered by the above item but helpful for the assignment and record process. | e.g.: This assignment will be used by ATC in charge of States Aircraft not equiped with 8.33 |

Note:	1	Also known as protected range and protected altitude
	2	If not a circle but a sector (polygon) please insert the coordinate points that 
		represent the boundary of the polygon.

-- END --

There can be several assignments on the same channel in a given location

TPPT Co-frequency and adjacent frequency are referred to the transponder reply frequency.