Proposal for amendment (PfA) to the EUR eANP Volume II (Part III CNS) new regional requirements section

…

MINIMUM OPERATIONAL NETWORK (MON) CONCEPT FOR ANSISG REGION

Using Conventional Navigation AIDS (NAVAID) for Contingency during Global Navigation Satellite System (GNSS) Outages

1. Introduction

The shift from traditional NAVAIDs-referenced navigation to Performance Based Navigation (PBN) observed in Eastern Europe has yielded considerable efficiency advantages for the States in the Region, ANSPs, and airspace users. These advantages encompass more direct routing, parallel flight paths, and enhanced flexibility in designing routes tailored to operational requirements.

The primary facilitator of PBN is the Global Navigation Satellite System (GNSS). Due to the vulnerabilities associated with GNSS, especially concerning Radio Frequency Interference (RFI), it is crucial to maintain a minimum number of conventional NAVAIDs as part of a Minimum Operational Network (MON) and, when necessary, to install additional NAVAIDs to guarantee minimum coverage. This approach will offer users alternatives during potential GNSS RFI disruptions.

2. Purpose

This document outlines the concept and preliminary design criteria for the Air Navigation Service Implementation Support Group (ANSISG) Minimum Operational Network (MON) for navigation. The MON is intended to create a foundational framework for NAVAIDs infrastructure throughout the Region of Air Navigation Service Implementation Support Group (ANSISG) member-States, in compliance with applicable ICAO standards and National Regulations.

The design of the ANSISG MON is based on the goals of ensuring safety and security in air traffic management and flight operations in the event of a GNSS failure.

3. Background

The Global Air Navigation Plan or GANP (ICAO Doc 9750) is described as “ICAO’s highest air navigation strategic document and the plan to drive the evolution of the global air navigation system, in line with the Global Air Traffic Management Operational Concept (GATMOC, ICAO Doc 9854) and the Manual on Air Traffic management System Requirements (ICAO Doc 9882)”. The GANP has the objective of delivering a future, harmonised global navigation capability based on RNAV and PBN supported by the GNSS. “The current single frequency GNSS capability provides the most accurate source of positioning that is available on a global basis. With suitable augmentation, as standardised within annexes, single frequency GNSS has the capability to support all phases of flight. The current GNSS has an extremely high availability, although it does not have adequate resilience to several vulnerabilities, most notably RFI and solar events causing ionospheric disturbances. Until a solution to ensure adequate GNSS resilience is available, it is essential that a terrestrial navigation infrastructure, suitably dimensioned to be capable of maintaining safety and continuity of aircraft operations, be provided.”1 The overview of the GANP context indicates that the role of ground-based NAVAIDs will evolve to support contingencies during potential GNSS disruptions. As a contingency capability, conventional NAVAIDs can be rationalised. However, to retain a MON designed to efficiently provide reversion service, it is important to recognise that a MON is not intended to serve as the primary navigation service, but as a recovery capability that aircraft can revert to, independent of GNSS.

4. Methodology

To develop this Concept, the MON Project Team reviewed various national and regional MON concepts, including concepts developed by IATA for Tajikistan and Uzbekistan, a regional concept from EUROCONTROL and ICAO guidelines.

ICAO Annex 10 (Aeronautical Telecommunications), Volume 1 (Radio Aids to Navigation – Seventh Edition, July 2018), Attachment H defines a global “Strategy for rationalisation of conventional radio navigation aids and evolution toward supporting performance-based navigation”. The objective of Attachment H is to provide guidance to the States for both the rationalisation and back-up of the terrestrial NAVAID infrastructure. The recommendations include the new roles foreseen for each type of NAVAID as part of the reversionary terrestrial infrastructure capable of maintaining safety and an adequate level of operations to support contingency operations.

The following key principles were considered for the development of the Concept:

- enable aircraft to continue RNAV or RNP operations with minor impacts by GNSS RFI.

- top-down vs bottom-up approach for GNSS contingency planning.

- support of multiple applications (DME/DME based PBN, etc).

- redundancy avoidance.

- critical Navaids MON should be reviewed at the regional level, not only national level.

- cross-border Navaids applicability should be considered.

- military Navaids integration into the MON where possible.

- use case analysis and airspace user consultation are essential.

5. Role of NAVAIDS

5.1 DME/DME

DMEs are commonly used to support various flight procedures, including both conventional and PBN operations. In conventional procedures, DME can be used in conjunction with ILS/DME, VOR/DME, and NDB/DME approaches. When paired with a VOR, ILS, or NDB, DME provides range information to support these procedures. DME can also enable the flying of DME arcs. In PBN procedures, DME/DME supports RNAV 5, RNAV 2, and RNAV 1 specifications, which can also serve as RNP 1 reversion. Additionally, with authorization from the State, DME can support RNP 1 and RNP AR missed approach procedures.

5.1.1 DME/DME in context of MON

The primary role of DME NAVAIDs is to support reversion for PBN operations in case of GNSS failure. Therefore, the main objective should be the definition of the DME/DME performance in airspace volumes where PBN operations are to continue. The DME/DME performance requirements can be defined in terms of accuracy, integrity, continuity, and availability of the navigation service. This performance level should be defined for each airspace volume served, based on the type of application to be supported and while considering continuity objectives set by the airspace concept. While the required navigation accuracy generates constraints on the geometry of the DME/DME pairs, it should be noted that the RNAV navigation specifications do not include integrity requirements for the DME/DME signal-in-space. However, the expected continuity and availability of service will drive the number and location of DMEs required to meet DME/DME RNAV coverage and redundancy requirements. The following minimum set of considerations is recommended when identifying the MON operational roles of the DME network:

- determine the enroute and terminal airspace where DME/DME RNAV service is needed for contingency operations during GNSS outages.

- identify the PBN NavSpec needed to support RNAV contingency operations.

- RNAV 5 operations in Free Route Airspace (FRA) or along ATS routes in En Route airspace volumes;

- RNAV 1 operations (SIDs/STARs) in terminal airspace volumes.

- identify the conventional NAVAIDs with DME where DME/DME RNAV service is desired.

- identify airports where approach and landing operations must continue during GNSS outages.

- identify where additional DMEs are required to support PBN instrument flight procedures.

5.2 VOR

For conventional procedures paired (or not) with a DME, VORs can support enroute operations (low level ATS routes), SIDs/STARs, non-precision approaches (NPA), missed approach procedures and even the transition up to intercepting the ILS. In all the previous cases they provide bearing information and enable homing to a beacon. When co-located with a DME, range and bearing information is available. For PBN procedures the only PBN navigation specification enabled by VOR, provided a co-located DME is present, is RNAV 5. VORs can be used in the enroute phase of flight (at or near the bottom of the enroute airspace, where achieving DME/DME coverage is challenging).

5.2.1 VOR in context of MON

As mentioned before, the only PBN navigation specification enabled by VORs is RNAV 5 (provided that a DME is co-located). Given these limitations, the ease in route phase to achieve proper DME/DME coverage, and the fact that only a few aircraft operators are certified to use exclusively RNAV 5 capabilities based on VOR/DME, a deep analysis is required to assess which VOR facilities should be retained to support either enroute or terminal As mentioned before, the only PBN navigation specification enabled by VORs is RNAV 5 (provided that a DME is co-located). Given these limitations, the ease in route phase to achieve proper DME/DME coverage, and the fact that only a few aircraft operators are certified to use exclusively RNAV 5 capabilities based on VOR/DME, a deep analysis is required to assess which VOR facilities should be retained to support either enroute, terminal or approach and landing operations, which will be conventional in the most cases. The main residual operational roles of the VOR in context of the MON are:

- providing alternative means of navigation during contingency operations.

- allow the operation of non-PBN capable aircraft during the transition to PBN. approach and landing operations, which will be conventional in the most cases.

As part of the overall MON strategy, it is recommended to perform a VOR network rationalisation analysis and identify where VOR(/DME) is needed to support:

- En route and TMA airspaces.

- RNAV 5 operations in FRA or on ATS routes.

- conventional ATS routes defined by VOR/DME which are required to be maintained.

- operations of State aircraft or aircraft of lower capabilities on ATS Routes.

- conventional instrument approach procedures at alternates for major aerodromes and/or for aerodromes where only RNP APCH procedures are planned.

5.3. NDBs

NDBs do not support PBN operations and their use as an aid to enroute navigation, terminal area operations or as an approach aid is generally considered obsolete. An increasing number of aircraft no longer provide an ADF capability on- board. Therefore, the retention of NDB-based ATS routes and procedures is not recommended. However, if a need for NDBs is identified in the airspace concept, the reason for this must be fully explored and justified. Additionally, the required facilities (including co-located DMEs) should also be identified.

5.4. ILS

Instrument Landing Systems (ILS) support Precision Approaches and Non-Precision Approaches (NPAs) for localiser (LOC) only approaches. They can be co-located with DMEs to provide range information. They provide lateral and vertical path guidance for a precision approach and can provide lateral path guidance for NPAs.

5.4.2. ILS context of MON

At airports where all approaches are based on GNSS (i.e. RNP APCH or GBAS), a GNSS outage could have a serious impact on operations. At most airports with air carrier operations, ILS will continue to provide the only means for 3-D operations. The evolution of the ILS/DME network must be considered when planning the overall DME network evolution because most ILS facilities are co-located with a Low Power DME (LPDME). The possibility of adding these DME facilities to support RNAV operations should be considered following an analysis of fleet equipage and changes in the operational management of these ILS/DME facilities.

When implementing GNSS-only instrument flight procedure at certain aerodrome it should be ensured that the ILS-equipped aerodrome exists in reasonable distance to be selected as a destination alternate

6. Navigation avionics equipage

This section illustrates some information on GNSS and PBN aircraft capability as declared in ICAO Flight Plans. This information is available since November 2012 when ICAO updated the FPL format with additional codes for Item 10 and 18.

Statistics in this section are based on an analysis of Navigation, GNSS and Performance Based Navigation (PBN) information in the ICAO Flight Plans available to the EUROCONTROL Network Manager (NM) and are presented quarterly in Network Manager reports.

Additional information can be obtained with the EUROCONTROL CNS dashboard (www.eurocontrol.int/services/communication-navigation-surveillance-cns-dashboard) which allows in-depth analysis – e.g. information on the proportion of capable-aircraft as well as the proportion of capable-flights; analysis for different periods of time, airports, airlines or aircraft types (depending on the user profile).

Despite the varied quality of declared PBN capability in Flight Plans (FPL), NM reports provide useful information in support to monitoring fleet capabilities and are correlated to the data available for IATA through periodical airlines survey.

6.1. Airborne equipment and capabilities for Europe

|  | 2025 Q1 |  | 2025 Q2 |  |
| --- | --- | --- | --- | --- |
|  | Flights | Aircraft | Flights | Aircraft |
| Total Nb of flights/Aircraft |  |  |  |  |
|  | 2,284,424 | 23,760 | 2,956,680 | 26,601 |
| GBAS |  |  |  |  |
|  | 12.80% | 9.71% | 11.72% | 9.29% |
| GNSS |  |  |  |  |
|  | 99.18% | 95.81% | 99.22% | 95.56% |
| INS |  |  |  |  |
|  | 87.38% | 71.54% | 87.79% | 70.57% |
| PBN |  |  |  |  |
|  | 99.31% | 95.17% | 99.30% | 94.88% |
| RNAV 5 |  |  |  |  |
| Any means | 98.13% | 91.76% | 98.11% | 91.15% |
| GNSS - only | 6.11% | 19.94% | 6.17% | 21.04% |
| GNSS and DME/DME | 90.73% | 67.21% | 90.74% | 65.64% |
| RNAV 1 |  |  |  |  |
| Any means | 98.00% | 87.92% | 98.00% | 87.38% |
| GNSS - only | 6.94% | 21.17% | 6.85% | 22.13% |
| Non-GNSS | 0.51% | 0.90% | 0.48% | 0.75% |
| RNP 1 |  |  |  |  |
| Any means | 88.63% | 76.06% | 89.66% | 75.24% |
| RNP APCH |  |  |  |  |
| Any means | 95.72% | 86.90% | 96.73% | 86.69% |
| LNAV-only | 8.54% | 12.91% | 8.32% | 13.05% |
| LNAV/VNAV | 84.25% | 62.29% | 85.42% | 61.04% |
| LPV | 12.46% | 29.93% | 12.62% | 32.10% |
| RNP AR APCH |  |  |  |  |
| Any means | 12.74% | 12.79% | 12.70% | 12.54% |

6.2. RNAV 5

Analysis indicates that 98 % of flights (91% of aircraft) can operate using the RNAV5 specification. The share of aircraft using all acceptable sensors exceeds 90%, which means that in the event of a GNSS signal loss or going beyond the coverage area of ground radio aids, these aircraft will continue RNAV operations along the route without the need for ATS intervention.

6.3. RNAV 1

Another indication was drawn - 98 % of flights (87% of aircraft) can operate using the RNAV1 specification. The share of aircraft using all acceptable sensors exceeds 87%, which means that in the event of a GNSS signal loss, these aircraft will continue operations within the departure/arrival routes using the ground radio navigation facilities (DME/DME) without the need for ATS intervention. For the remaining aircraft, it may be necessary to use traditional navigation or vectoring procedures.

According to IATA data for certain States (e.g. Uzbekistan) the percentage of RNAV 5 and RNAV 1 capable fleet exceeds 95%

7. Existing ground-based navigation networks of the ANSISG Region

For the purposes of the analysis the consideration was given to the navigation aids which can support PBN operations independently of GNSS (VOR/DME, DME). For the cross-border consideration the VOR/DME stations located in the FIRs of neighbouring States were considered as much as they can support the DME/DME network.

The data presented below forms the NAVAIDS network of the States supporting ANSISG MON either at national or cross-border level.

ALGERIA

|  | Name of station (VAR) | Id | Frequency (CH) | Hours of operation | Geographical Coordinates | Degrees with decimal fractions | ELEV DME antenna | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| ALGERIA | ALGER DVOR/DME 2°E (2023) | ALR | 112.5 MHZ / CH 72 X | H24 | 364127.59N 0031255.73E | 36.69099°N 3.21548°E | / |  |
|  | SIDI MHAMED DVOR/DME 2°E (2023) | SDM | 113.9 MHZ / CH 86X | H24 | 363747.69N 0025821.50E | 36.62992° N 2.97264° E | / |  |
|  | ZEMMOURI DVOR/DME 2°E (2023) | ANB | 113.5 Mhz / CH 82 X | H24 | 364956.80N 0074852.50E | 36.83245°N 7.81458°E | / |  |
|  | ANNABA CVOR/DME (2°E 2017) | ZEM | 116.6 MHZ / CH 113 X | H24 | 364742N 0033415E | 36.79500° N 3.57083° E | / |  |
|  | BEJAIA CVOR/DME | BJA | 113 Mhz CH 77X | H24 | 364252.45 N 0050449.43E | 36.71457° N 5.08040° E | 11,1 M |  |
|  | ADRAR CVOR/DME 1° W (2017) | ADR | 112.6 Mhz / CH 73 X | H24 | 274913.20N 0001209.93W | 27.82034° N -0.01943° W | / |  |
|  | BATNA DVOR/DME (2°E 2017) | BTN | 115.7 Mhz / CH 104 X | H24 | 354617.50N 0062037.66E | 35.77153° N 6.34379° E | / |  |
|  | BECHAR CVOR/DME (0°E 2017) | BCR | 113.9 Mhz / CH 86X | H24 | 314104.53N 0021540.59W | 31.68459° N -2.26128° W | / |  |
|  | B. B MOKHTAR CVOR/DME (0° E 2023) | MOK | 114.0 Mhz / CH87X | H24 | 212257.36N 0005702.49E | 21.38260° N 0.95069° E | 393 M |  |
|  | BISKRA CVOR/DME (2°E 2023) | BIS | 115.0 Mhz / CH 97 X | H24 | 344633.91N 0054549.43E | 34.77609° N 5.76373° E | / | BISKRA CVOR/DME (2°E 2023) |
|  | CHLEF CVOR/DME (1° E 2017) | CLF | 117.0 Mhz / CH 117X | H24 | 361248.3N 0011919.9E | 36.21342° N 1.32220° E | 163 M | CHLEF CVOR/DME (1° E 2017) |
|  | CONSTANTINE DVOR/DME (2°E 2017) | CSO | 115.5 Mhz / CH 102 X | H24 | 361735.75 N0063629.96E | 36.29326° N 6.60832° E | / | CONSTANTINE DVOR/DME (2°E 2017) |
|  | DJANET DVOR/DME (1°E 2017) | DJA | 114.1 Mhz CH 88X | H24 | 241715.82N 0092712.03E | 24.28773° N 9.45334° E | / | DJANET DVOR/DME (1°E 2017) |
|  | EL BAYADH DVOR/DME (1°E 2017) | BAY | 114.8 MHz / CH 95X | H24 | 334235.30N 0010445.07E | 33.70981° N 1.07919° E | / | EL BAYADH DVOR/DME (1°E 2017) |
|  | EL GOLEA DVOR/DME (1° E 2017) | MNA | 112.1 Mhz / CH 58X | H24 | 303330.77N 0025141.97E | 30.55855° N 2.86166° E | / | EL GOLEA DVOR/DME (1° E 2017) |
|  | EL OUED DVOR/DME (2°E / 2017) | ELO | 117.6 Mhz / CH 123X | H24 | 333037.64N 0064650.21E | 33.51046° N 6.78062° E | / | EL OUED DVOR/DME (2°E / 2017) |
|  | GHARDAIA DVOR/DME (1°E 2017) | GHA | 114.9 Mhz / CH 96 X | H24 | 322335.83N 0034640.37E | 32.39328° N 3.77788° E | 482 M | GHARDAIA DVOR/DME (1°E 2017) |
|  | HASSI MESSAOUD DVOR/DME (2.2°E 2023) | HME | 114.7 Mhz / CH 94 X | H24 | 314127.7N 0060830.88E | 31.69102° N 6.14191° E | 149 M | HASSI MESSAOUD DVOR/DME (2.2°E 2023) |
|  | ILLIZI CVOR/DME (2°E 2023) | ILZ | 115.6 Mhz / CH 103 X | H24 | 264312.18N 0083808.78E | 26.72005° N 8.63577° E | / | ILLIZI CVOR/DME (2°E 2023) |
|  | JIJEL DVOR/DME (2°E 2017) | JIL | 117.9 Mhz / CH 126X | H24 | 364751.3N 0055231.7E | 36.79758° N 5.87548° E | 17 M | JIJEL DVOR/DME (2°E 2017) |
|  | ORAN CVOR/DME (1°E 2023) | ORA | 114Mhz / CH 87 X | H24 | 353645.53N 0003917.96W | 35.61265° N -0.65499° W | / | ORAN CVOR/DME (1°E 2023) |
|  | OUARGLA CVOR/DME (2°E (2020) 6’) | OUR | 112.7 Mhz / CH 74X | H24 | 315630N 0052500 E | 31.94167° N 5.41667° E | / | OUARGLA CVOR/DME (2°E (2020) 6’) |
|  | SETIF CVOR/DME (1° E 2017) | STF | 114.6 MHZ / CH 93 X | H24 | 361036.00N 0051719.00 E | 36.17667° N 5.28861° E | 987 M | SETIF CVOR/DME (1° E 2017) |
|  | TAMENGHASSET DVOR/DME (1°E 2023) | TMS | 112.5 Mhz / CH72 X | H24 | 224827.40N 0052647.50E | 22.80761° N 5.44652° E | / | TAMENGHASSET DVOR/DME (1°E 2023) |
|  | TEBESSA DVOR/DME (3°E 2023) | TSB | 114.5 Mhz / CH 92 X | H24 | 352723.64N 0080407.05E | 35.45657° N 8.06863° E | / | TEBESSA DVOR/DME (3°E 2023) |
|  | TIARET CVOR/DME (1°E 2017) | TRB | 116.3 Mhz / CH 110X | H24 | 352051.92N 0013053.70E | 35.34775° N 1.51492° E | / | TIARET CVOR/DME (1°E 2017) |
|  | TIMIMOUN CVOR/DME (0°E 2017) | TIO | 112.8 Mhz / CH 75 X | H24 | 291333.89N 0001529.14E | 29.22608° N 0.25809° E | / | TIMIMOUN CVOR/DME (0°E 2017) |
|  | TINDOUF CVOR/DME (2°W (2017)) | TDF | 114.2 MHZ / CH 89X | H24 | 274236.26N 0080751.20W | 27.71007° N -8.13089° W | / | TINDOUF CVOR/DME (2°W (2017)) |
|  | TLEMCEN DVOR/DME (1°E 2023) | TLM | 116.5 Mhz / CH 112 X | H24 | 350054.07N 0012646.12W | 35.01502° N -1.44614° W | / | TLEMCEN DVOR/DME (1°E 2023) |
|  | TOUGGOURT CVOR/DME (2°E 2017) | TGU | 113.2 Mhz / CH 79X | H24 | 330325.90N 0060519.68E | 33.05719° N 6.08880° E | / | TOUGGOURT CVOR/DME (2°E 2017) |
|  | IN AMENAS CVOR/DME (2°E 2017) | IMN | 112.9 Khz / CH 76 X | H24 | 280359.6N 0093939.3E | 28.06656° N 9.66092° E | / | IN AMENAS CVOR/DME (2°E 2017) |
|  | IN SALAH CVOR/DME (1°E 2023) | NSL | 113.1 Mhz / CH 78X | H24 | 271448.26N 0023009.24E | 27.24674° N 2.50257° E | 278 M | IN SALAH CVOR/DME (1°E 2023) |
|  | BNI ABBAS CVOR/DME (0° W/2017) | BSS | 115.3 Mhz / CH 100X | H24 | 300125.60N 0021350.30W | 30.02378° N -2.23064° W | / | BNI ABBAS CVOR/DME (0° W/2017) |
|  | BORDJ OMAR DRISS CVOR/DME (0° E/2023) | BOD | 114.3 Mhz / CH 90X | H24 | 280758.97N 0065021.03E | 28.13305° N 6.83917° E | / | BORDJ OMAR DRISS CVOR/DME (0° E/2023) |
|  | MOSTAGHANEM CVOR/DME (1° W/2017) | MOS | 112.2 Mhz / CH 59X | H24 | 355355.13N 0000810.67E | 35.89864° N 0.13629° E | / | MOSTAGHANEM CVOR/DME (1° W/2017) |
|  | HASSI RMEL CVOR/DME (1°E 2017) | HRM | 115.4 Mhz / CH 101X | H24 | 32°56’15.62’’N 003°21’24.50’’E | 32.93767° N 3.35681° E |  | HASSI RMEL CVOR/DME (1°E 2017) |

ARMENIA

|  | Name of station (VAR) | Id | Frequency (CH) | Hours of operation | Geographical Coordinates | Degrees with decimal fractions | ELEV DME antenna | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| ARMENIA | GYUMRI DVOR/DME (7° E) | GRM | 113.400 MHz (CH 81 x) | H24 | 404346N 0435050E | 40.729444 43.847222 | 4950 FT | Coverage FL 410/110 KM (60 NM) Bearing information in sector 120º –240º may be disturbed at flight altitudes bellow 13 000 FT Coverage 50 NM |
|  | ZVARTNOTS DVOR/DME (7° E) | ZVR | 112.300 MHz (CH 70 x) | H24 | 400849N 0442017E | 40.146944 44.338056 | 2849 FT | Coverage FL 410/370 KM (200 NM) Bearing information in sector 320º –60º may be disturbed at flight altitudes bellow 13 000 FT |

AZERBAIJAN

|  | Name of station (VAR) | Id | Frequency (CH) | Hours of operation | Geographical Coordinates | Degrees with decimal fractions | ELEV DME antenna | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| AZERBAIJAN | BAKU (DVOR/DME) | BAK | 115 MHZ |  | 402514.80N 0500350.30E |  |  |  |
|  | GANJA (DVOR/DME) | GND | 115.8 MHZ |  | 404512.70N 0461743.80E |  |  |  |
|  | NAKHCHIVAN (DVOR/DME) | NAX | 116.5 MHZ |  | 391205.90N 0452627.40E |  |  |  |
|  | ZAQATALA (VOR/DME) | ZAQ | 114.10 MHZ |  | 413314.1N 0464012.0E |  |  |  |
|  | GABALA (DVOR/DME) | QBL | 112.5 MHZ |  | 404956.60N 0474305.80E |  |  |  |
|  | ZANGELAN (VOR/DME) | ZNG | 117.3 MHZ |  | 390624.6N 464426.1E |  |  |  |
|  | FUZULI (VOR/DME) | FZL | 112.8 MHZ |  | 393511.82N 0471227.33E |  |  |  |

GEORGIA

|  | Name of station (VAR) | Id | Frequency (CH) | Hours of operation | Geographical Coordinates | Degrees with decimal fractions | ELEV DME antenna | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| GEORGIA | BAKURIANI DME | BKU | 110.600 MHz (CH 43X) | H24 | 414141N 0433236E |  | 9000 FT | Coverage 108 NM. |
|  | BATUMI DME | BTM | 108.400 MHz (CH 21X) | H24 | 413623N 0413606E |  | 100 FT | Coverage 108 NM. Omnidirectional. |
|  | GUDAURI DME | GUD | 110.800 MHz (CH 45X) | H24 | 422930N 0442946E |  | 8900 FT | Coverage 108 NM. |
|  | KUTAISI DVOR/DME (7° E) | KTS | 113.600 MHz (CH 83X) | H24 | 421033N 0422905E |  | 200 FT | Coverage 108 NM. FRA(I) |
|  | POTI DME | PTI | 111.000 MHz (CH 47X) | H24 | 420942N 0414150E |  | 100 FT | Coverage 108 NM. |
|  | TBILISI DVOR/DME (7° E) | TBS | 113.700 MHz (CH 84X) | H24 | 414014N 0445649E |  | 1700 FT | Coverage 108 NM. FRA(I) |
|  | TSNORI DME | TSN | 108.600 MHz (CH 23X) | H24 | 413746N 0460056E |  | 800 FT | Coverage 108 NM. |

KAZAKHSTAN

|  | Name of station (VAR) | Id | Frequency (CH) | Hours of operation | Geographical Coordinates | Degrees with decimal fractions | ELEV DME antenna | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| KAZAKHSTAN | AKTAU DVOR/DME (8E/2021) | AKT | 113.3 MHZ CH 80X | H24 | 435220N 0510352E | 43.8722° 51.0644° | 100 FT |  |
|  | AKTOBE DVOR/DME (11E/2020) | AKB | 113,4 MHZ CH 81X | H24 | 501548N 0571055E | 50.2633° 57.1820° | 700 FT |  |
|  | ALMATY DVOR/DME (5E/2023) | ATA | 116.4 MHZ CH 111X | H24 | 432229N 0770507E | 43.3747° 77.0853° | 2200 FT |  |
|  | BEINEU VOR/DME (7E/1999) | BNU | 115 MHZ CH 97X | H24 | 452023N 0550721E | 45.3397° 55.1225° | 0 FT |  |
|  | JARKENT DVOR/DME | JRK | 114,8 MHZ CH 95X | H24 | 441344N 0795719E | 44.2289° 79.9553° | 2600 FT |  |
|  | KOKSHETAU VOR/DME (11E/2013) | KTU | 115,5 MHZ CH 102X | H24 | 532103N 0693701E | 53.3508° 69.6170° | 900 FT |  |
|  | KOSTANAY DVOR/DME (13E/2022) | KST | 114.8 MHZ CH 95X | H24 | 531113N 0633346E | 53.1869° 63.5628° | 600 FT |  |
|  | KYZYLORDA DVOR/DME (7E/2022) | KZO | 112.7 MHZ CH 74X | H24 | 444145N 0653349E | 44.6958° 65.5636° | 500 FT |  |
|  | PAVLODAR DVOR/DME (9E/2013) | PVL | 114 MHZ CH 87X | H24 | 521235N 0770542E | 52.2097° 77.0950° | 500 FT |  |
|  | PETROPAVLOVSK DVOR/DME (12E/2017) | PSK | 112,5 MHZ CH 72X | H24 | 544703N 0691309E | 54.7841° 69.2192° | 500 FT |  |
|  | SEMEY DVOR/DME (7E/2014) | SEM | 115,3 MHZ CH 100X | H24 | 502059N 0801438E | 50.3497° 80.2439° | 700 FT |  |
|  | SHYMKENT DVOR/DME (6E/2013) | SMK | 113 MHZ CH 77X | H24 | 422220N 0692631E | 42.3723° 69.4419° | 1400 FT |  |
|  | TALDYKORGAN DVOR/DME (5E/2014) | TDK | 116,1 MHZ CH 108X | H24 | 450622N 0782548E | 45.1061° 78.4300° | 2000 FT |  |
|  | TARAZ DVOR/DME (6E/2013) | TAR | 115,9 MHZ CH 106X | H24 | 425214N 0711654E | 42.8706° 71.2817° | 2200 FT |  |
|  | URALSK DVOR/DME (11E/2015) | URL | 114,2 MHZ CH 89X | H24 | 510855N 0513238E | 51.1486° 51.5439° | 200 FT |  |
|  | UST-KAMENOGORSK DVOR/DME (6E/2021) | UKM | 115 MHZ CH 97X | H24 | 500158N 0823031E | 50.0328° 82.5086° | 1000 FT |  |
|  | AKTAU DVOR/DME (8E/2021) | AKT | 113.3 MHZ CH 80X | H24 | 435220N 0510352E | 43.8722° 51.0644° | 100 FT |  |
|  | AKTOBE DVOR/DME (11E/2020) | AKB | 113,4 MHZ CH 81X | H24 | 501548N 0571055E | 50.2633° 57.1820° | 700 FT |  |
| KYRGYZSTAN | BISHKEK/MANAS VOR/DME (6°E/2023) | MNS | 113.4 MHZ CH 81X | H24 | 430400.92N 0743115.27E | 43.0669° 74.5209° |  |  |
|  | ISSYK-KUL DVOR/DME (5°E/2023) | ILK | 114 MHZ CH 87X | H24 | 423444.24N 0764019.64E | 42.57896° 76.67212° |  |  |
|  | OSH DVOR/DME (5°E/2023) | OSH | 112.4 MHZ CH 71X | H24 | 403622.21N 0724807.38E | 40.606169° 72.802050° |  |  |
| UZBEKISTAN | DJIZAK DVOR/DME | JZK | 112.2 MHz | HO | 401516.7 N 0675439.3 E | 40.25464° 67.91092° | 1100 FT |  |
|  | NAMANGAN DVOR/DME | NMA | 116.0 MHz | H24 | 405855.1 N 0713338.0 E | 40.98197° 71.56056° | 1600 FT |  |
|  | SAMARKAND DVOR/DME | SKD | 115.0 MHz | H24 | 394213.5 N 0665821.6 E | 39.70375° 66.97267° | 2200 FT |  |
|  | TASHKENT DVOR/DME | TKT | 113.2 MHz | H24 | 411516.1 N 0691513.0 E | 41.25447° 69.25361° | 1400 FT |  |
| RUSSIAN FEDERATION | ASTRAKHAN VOR/DME | AST | 116.10 MHz CH 108X | H24 | 461652N 0480033E | 46.28111° 48.00917° | 98 FT |  |
|  | BARABINSK БАРАБИНСК DME | SZ | 114.30 MHz CH 90X | H24 | 552125N 0781810E | 55.35694° 78.30278° | 492 FT |  |
|  | CHELYABINSK/Balandino VOR/DME | LBN | 113.30 MHz CH 80X | H24 | 551825N 0613104E | 55.30694° N, 61.51778° E | 787 FT |  |
|  | KAMYSHIN DME | KMN | 114.50 MHz CH 92X | H24 | 500312N 0452229E | 50.05333° 45.37472° | 492 FT |  |
|  | KRASNOARMEYSK DME | DK | 113.30 MHz CH 80Х | H24 | 510154N 0453838E | 51.03167° 45.64389° | 886 FT |  |
|  | MAGNITOGORSK VOR/DME | MGR | 114.10 MHz CH 88X | H24 | 532415N 0584527E | 53.40417° 58.90750° | 1378 FT |  |

KYRGYZSTAN

|  | Name of station (VAR) | Id | Frequency (CH) | Hours of operation | Geographical Coordinates | Degrees with decimal fractions | ELEV DME antenna | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| KYRGYZSTAN | BISHKEK/MANAS VOR/DME (6°E/2023) | MNS | 113.4 MHZ CH 81X | H24 | 430400.92N 0743115.27E | 43.066922° 74.520908° |  |  |
|  | ISSYK-KUL DVOR/DME (5°E/2023) | ILK | 114 MHZ CH 87X | H24 | 423444.24N 0764019.64E | 42.578956° 76.672122° |  |  |
|  | OSH DVOR/DME (5°E/2023) | OSH | 112.4 MHZ CH 71X | H24 | 403622.21N 0724807.38E | 40.606169° 72.802050° |  |  |
| KAZAKHSTAN | TARAZ DVOR/DME (6E/2013) | TAR | 115,9 MHZ CH 106X | H24 | 425214N 0711654E | 42.870556° 71.281667° | 2200 FT |  |
|  | ALMATY DVOR/DME (5E/2023) | ATA | 116.4 MHZ CH 111X | H24 | 432229N 0770507E | 43.374722° 77.085278° | 2200 FT |  |
|  | SHYMKENT DVOR/DME (6E/2013) | SMK | 113 MHZ CH 77X | H24 | 422220N 0692631E | 42.372222° 69.441944° | 1400 FT |  |
| UZBEKISTAN | SAMARKAND DVOR DME | SKD | 115.0 MHz | H24 | 394213.5N 0665821.6E | 39.703750° 66.972667° | 2200 FT | R=108NM |
|  | NAMANGAN DVOR/DME | NMA | 116.0 MHz | H24 | 405855.1N 0713338.0E | 40.981972° 71.560556° | 1600 FT | R=54NM |
|  | TASHKENT DVOR DME | TKT | 113.2 MHz | H24 | 411516.1N 0691513.0E | 41.254472° 69.253611° | 1400 FT | R=143NM |
| TAJIKISTAN | KHUJAND DME | HND | 114.30 MHz | H24 | 401250N 0694205E | 40.213889° 69.701389° | 477M |  |
|  | ISTARAVSHAN DME | IRN | 113.90 MHz | H24 | 400338N 0685644E | 40.060556° 68.945556° | 713M |  |
|  | DUSHANBE DME | DBE | 114.20 MHz | H24 | 383230N 0684919E | 38.541667° 68.821944° | 780M |  |
|  | DANGARA DME | DNR | 114.10 MHz | H24 | 381547N 0691319E | 38.263056° 69.221944° | 2303M |  |
|  | KULOB DME | KLB | 114.40 MHz | H24 | 375904N 0694806E | 37.984444° 69.801667° | 670M |  |
|  | MURGAB DME | MRB | 114.70 MHz | H24 | 380321N 0743033E | 38.055833° 74.509167° | 3742M |  |
|  | Aksu VOR/DME | AKS | 115.1 MHz CH 98X | H24 | N41°15.7′ E080°17.1′ | 41.260666° 80.286660° | 1 185M |  |
|  | YINING | YNN | 112.9 MHz | H24 | N43°57.97' E81°18.48' | 43.958105° 81.329016° |  |  |
|  | Yarkant VOR/DME | DSC | 112.5 MHz CH 72X | H24 | N38°13.3′ E077°04.3′ | 38.223317° 77.070385° | 1 317M |  |

REPUBLIC OF MOLDOVA

|  | Name of station (VAR) | Id | Frequency (CH) | Hours of operation | Geographical Coordinates | Degrees with decimal fractions | ELEV DME antenna | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| REPUBLIC OF MOLDOVA | CHISINAU DVOR/DME (7˚E) | KIV | 113.7MHZ CH 84X | H24 | 465534N 0285416E | 46.926111° 28.904444° | 400 FT | Designated Operational Coverage 200NM, FL400. FRA (I).* |
| ROMANIA | IAŞI DME | ISI | 1106.000 MHz (CH 82X) | H24 | 471404N 0273446E |  | 300 FT | Coverage 100 NM** Unusable in sector 200°-230° |
|  | STOLNICENI DME | STL | 1100.000MHz (76X) | H24 | 473409N 0271521E |  | 800 FT | Coverage 100 NM* |
|  | HADÂMBU DME | HDB | 1046.000MHz (22X) | H24 | 470054N 0272526E |  | 1500 FT | Coverage 100 NM* |
|  | ARINI DME | ARN | 1060.000MHz (36Y) | H24 | 462246N 0270924E |  | 1900 FT | Coverage 100 NM* |
|  | STRUNGA DME | STR | 1049.000MHz (25X) | H24 | 470940N 0265637E |  | 1200 FT | Coverage 100 NM* |
|  | BACĂU DME | BAC | 1145.000 MHz (CH 121X) | H24 | 463005N 0265505E |  | 700 FT | Coverage 100 NM** Unusable in sector 210°-290° |
|  | ISTRITA DME | ISA | 1129.000MHz (105Y) | H24 | 450709N 0263235E |  | 2500 | Coverage 100 NM* |
|  | ODOBEȘTI DME | ODB | 1054.000MHz (30Y) | H24 | 464152N 0271000E |  | 1600 FT | Coverage 100 NM* |
|  | VOR/DME Suceava | SCV | 112.300 MHz (CH 70X) | H24 | 474020N 0262139E |  | 1300 | Coverage 175 NM* FRA (I) |
|  | TOPOLOG DME | TPG | 1075.000MHz (51Y) | H24 | 445122N 0282438E |  | 1400 | Coverage 100 NM |

MOROCCO

|  | Name of station (VAR) | Id | Frequency (CH) | Hours of operation | Geographical Coordinates | Degrees with decimal fractions | ELEV DME antenna | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| MOROCCO | AGADIR/ AL MASSIRA DVOR/DME | ADM | 117,2 MHz (CH 119X) | H24 | 30 19 19N 009 23 00W |  | 90 m | 2700 m RWY centre 1100 m THR 27 |
|  | AL HOCEIMA / Chérif EL Idrissi VOR/DME | ALM | 115 MHz (CH 97X) | H24 | 35 11 17N 003 50 17W |  | ALT : 0 m |  |
|  | BENGUERIR VOR/DME | BGR | 115,400 MHz (CH 101X) | H24 | 32 09 28N 007 52 51W | ALT : 450 m |  |  |
|  | BÉN-MELLAL DVOR/DME | BML | 113,100 MHz (CH 78X) | H24 | 32 25 00.75 N 006 17 52.88 W |  | ALT : 540 m | - PWR DVOR : 100W PWR DME: 1KW |
|  | BENSLIMANE VOR/DME L | BNS CAE | 117,450 MHz (CH 121Y) 275 KHz | H24 H24 | 33 38 31N 007 12 29W 33 36 57N 007 06 49W |  | ALT : 210 m |  |
|  | BOUARFA VOR/DME | BRF | 117,0 MHz (CH 117X) | H24 | 32 30 51N 002 01 50W |  | ALT : 1110 m |  |
|  | CASABLANCA / MOHAMMED V DVOR/DME VOR/DME DME | CBL SLK CBA | 115,1 MHz (CH 98X) 112,500 MHz (CH 72X) 116,9 MHz (CH 116X) | H24 H24 H24 | 33°25’13.82 N 007°36’21.72 W 33 06 56N 007 30 22W 33 31 17N 007 40 38w |  | ALT 180 m ALT : 300 m ALT : 120 m |  |
|  | DAKHLA DVOR/DME | DKH | 115,8 MHz | H24 | 23 44 41N |  | ALT : 30 m | Antenna HGT : 13 m |
|  | # DAOURAT L | CSD | 345 KHz | H24 | 32 56 01N 008 03 54W |  |  |  |
|  | ESSAOUIRA / Mogador VOR/DME DVOR/DME | ESS MGR | 112,7 MHz (CH 74X) 109,0 MHz (CH 27X) | H24 H24 | 31 24 04N 009 41 13W 31 23 47.84 N 009 41 04.41 W |  | ALT : 120 m ALT : 120 m | Antenna HGT : 8,91m |
|  | ERRACHIDIA / Moulay Ali Chérif VOR/DME L | ERA KSR | 117,1 MHz (CH 118X) 293 KHz | H24 H24 | 31 55 27N 004 21 38W 31 55 31N 004 21 44W |  | ALT: 1020 m | DIST/THR 31: 3035 m DME PWR : 1 KW VOR PWR : 100 W Range : 200 NM DIST/THR 31: 2940 m PWR : 25 W |
|  | FES / SAISS VOR/DME L | FES FEZ | 115,7 MHz (CH 104X) 315 KHz | H24 H24 | 33 55 39N 005 00 49W 33 55 37N 004 53 27W |  | ALT : 570 m DIST/THR 27 : 6439 m |  |
|  | # GUELMIME VOR/DME | GLM | 114,3 MHz (CH 90X) | H24 | 29 00 38N 010 04 09W |  | ALT : 330 m | Exploitant / Operator ONDA / FRA |
|  | LAAYOUNE / Hassan 1er VOR/DME | LAY | 112,1 MHz (CH 58X) | H24 | 27 08 34N 013 13 05W |  | ALT : 60 m | Antenna HGT : 7 m |
|  | MARRAKECH / Ménara DVOR/DME | MAK | 113,3 MHz (CH 80X) | H24 | 31 36 46N 008 06 11W |  | ALT : 450 m |  |
|  | NADOR / EL AROUI DVOR/DME | ARI | 116,0 MHz (CH 107X) | H24 | 34 59 12N 003 01 45W |  | ALT : 180 m | VOR PWR : 100 W DME PWR : 1 KW |
|  | OUARZAZATE DVOR/DME L | OZT ORZ | 116,7 MHz (CH 114X) 384 KHz | H24 H24 | 30 56 24N 006 54 16W 30 55 38N 006 52 57W |  | ALT : 1140 m | 1116 m THR 30 / 307 m North side RCL DME Range: 200 NM PWR: 1 KW |
|  | OUJDA / Angads VOR/DME VOR/DME | OJD VDO | 117,5 MHz (CH 122X) 115,65 MHz (CH 103Y) | H24 H24 | 34 46 24N 001 57 01W 34 48 57N 001 57 38W |  | ALT : 480 m ALT : 480 m | DIST/THR 13 : 1004 m VOR PWR: 100 W DME PWR: 1000 W |
|  | RABAT / Salé DVOR/DME L | RBT SBI | 116,5 MHz (CH 112X) 332 KHz | H24 H24 | 34 03 00N 006 44 47W 34 10 35N 006 39 51W |  | ALT : 90 m | DIST/THR 21: 14300 m |
|  | SIDI SLIMANE VOR/DME | SMN | 115,5 MHz (CH 102X) | H24 | 34 14 15N 006 01 33W |  | ALT : 0 m | Operator : Royal Air Forces ALT : 7m |
|  | ES-SEMARA DVOR/DME L | SMR SMA | 108,2 MHz (CH 19X) 401 KHz |  | 26 43 51N 011 41 12W 26 44 31N 011 41 16W |  | ALT : 180 m | Exploitant / operator Royal Air Forces |
|  | TANGER / Ibn Batouta DVOR/DME L | TNR TAN | 108,050 MHz (CH 17Y) 374 KHz | H24 H24 | 35 44 03N 005 55 09W 35 43 16N 005 48 17W |  | ALT : 30 m | DVOR et DME TNR inutilisables entre RDL 310° et RDL 340° au-dessous. de 6000FT AMSL / DVOR and DME TNR not usable between RDL 310° and RDL 340° below 6000FT AMSL Antenna HGT: 27,40 m PWR : 50 W |
|  | TAN-TAN / Plage Blanche DVOR/DME | TNN | 112,3 MHz (CH 70X) | H24 | 28 25 42N 011 10 23W |  | ALT : 210 m | DIST/THR 03 :1179.43 m VOR PWR : 50 W DME PWR : 1000 W |
|  | TETOUAN / Saniat R‘mel DVOR/DME | TTN | 117,3 MHz (CH 120X) | H24 | 35 35 51N 005 19 08W |  | ALT : 0 m | Antenna HGT: 8,73 m |
|  | ZAGORA DVOR/DME | ZAG | 114.600 MHZ (CH 93X) | H24 | 30°15’34”N 005°50’08”W |  | 720 M |  |

TAJIKISTAN

|  | Name of station (VAR) | Id | Frequency (CH) | Hours of operation | Geographical Coordinates | Degrees with decimal fractions | ELEV DME antenna | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| TAJIKISTAN | KHUJAND DME | HND | 114.30 MHz | H24 | 401250N 0694205E | 40.213889° 69.701389° | 477M |  |
|  | ISTARAVSHAN DME | IRN | 113.90 MHz | H24 | 400338N 0685644E | 40.060556° 68.945556° | 713M |  |
|  | DUSHANBE DME | DBE | 114.20 MHz | H24 | 383230N 0684919E | 38.541667° 68.821944° | 780M |  |
|  | DANGARA DME | DNR | 114.10 MHz | H24 | 381547N 0691319E | 38.263056° 69.221944° | 2303M |  |
|  | KULOB DME | KLB | 114.40 MHz | H24 | 375904N 0694806E | 37.984444° 69.801667° | 670M |  |
|  | MURGAB DME | MRB | 114.70 MHz | H24 | 380321N 0743033E | 38.055833° 74.509167° | 3742M |  |

TÜRKIYE

|  | Name of station (VAR) | Id | Frequency (CH) | Hours of operation | Geographical Coordinates | Degrees with decimal fractions | ELEV DME antenna | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| TÜRKIYE | ADANA VOR/DME | ADA | 112.7 MHZ CH74X | H24 | 365626.1N 0351237.4E | 21 M |  | ADANA VOR/DME |
|  | ADIYAMAN VOR/DME | KHD | 113.6 MHZ CH83X | H24 | 374336.3N- 0382758.1E | 683 M | Coverage 50 NM | ADIYAMAN VOR/DME |
|  | AFYON VOR/DME | KFK | 112.2 MHZ CH59X | H24 | 384814.9N 0303248.6E | 1022 M |  |  |
|  | AĞRI VOR/DME | ARI | 116.7 MHZ CH114X | H24 | 393844.9N 0430137.5E | 1669 M | Coverage 50 NM |  |
|  | ANTALYA VOR/DME | AYT | 114.0 MHZ CH87X | H24 | 365514.9N 0304740.1E | 59 M |  |  |
|  | BAĞLUM VOR/DME | BAG | 113.1 MHZ CH78X | H24 | 400412.0N 0324835.6E | 1463 M |  |  |
|  | BEYKOZ VOR/DME | BKZ | 117.3 MHZ CH120X | H24 | 410737.1N 0290834.4E | 327 M | Coverage 50 NM |  |
|  | BIGA VOR/DME | BIG | 116.9 MHZ CH116X | H24 | 401703.8N 0272155.9E | 127 M | Coverage 50 NM |  |
|  | BİNGÖL VOR/DME | BNG | 112.1 MHZ CH58X | H24 | 385120.0N 0403602.8E | 1061 M |  |  |
|  | BURSA/Yenişehir VOR/DME | BRY | 115.3 MHZ CH100X | H24 | 401545.1N 0293538.5E | 235 M | Coverage 50 NM |  |
|  | CENGİZ TOPEL NDB | CTP | 307 KHZ | H24 | 404414.9N 0300416.9E |  |  |  |
|  | ÇANAKKALE VOR/DME | CNK | 111.2 MHZ CH49X | H24 | 400804.1N 0262535.6E | 14 M | Coverage 50 NM |  |
|  | ÇARDAK VOR/DME | CRD | 112.0 MHZ CH57X | H24 | 374723.9N 0294214.5E | 856 M | Coverage 200 NM |  |
|  | ÇARSAMBA VOR/DME | CRM | 112.8 MHZ CH75X | H24 | 411555.8N 0363255.9E | 10 M |  |  |
|  | ÇAYCUMA VOR/DME/ | CAY | 109.6 MHZ CH33X | H24 | 413049.6N 0320240.4E | 521 M |  |  |
|  | ÇILDIR VOR/DME | CLD | 113.85 MHZ CH85Y | H24 | 374852.7N 0275347.0E | 37 M |  |  |
|  | ÇORLU VOR/DME | CRL | 114.8 MHZ CH95X | H24 | 410902.7N 0275606.0E | 187 M | Coverage 75 NM |  |
|  | ÇUBUK VOR/DME | BUK | 114.3 MHZ CH90X | H24 | 401430.7N 0330617.7E | 1253 M |  |  |
|  | DALAMAN VOR/DME | DAL | 114.7 MHZ CH94X | H24 | 364122.9N 0284655.7E | 10 M |  |  |
|  | DİYARBAKIR VOR/DME | DYB | 109.4 MHZ CH31X | H24 | 375225.3N 0401230.1E | 665 M | Coverage 100 NM |  |
|  | EDREMİT VOR/DME | EDR | 109.4 MHZ CH31X | H24 | 393305.0N 0270014.8 | 16 M | Coverage 50 NM |  |
|  | ELAZIĞ VOR/DME | EZS | 114.7 MHZ CH94X | H24 | 384229.8N 0391326.7E | 1487 M |  |  |
|  | ERZİNCAN VOR\DME | ERN | 112.7 MHZ CH74X | H24 | 394229.1N 0393144.5E | 1153 M | Coverage 80 NM Coverage 100 NM |  |
|  | ERZURUM VOR/DME | ERZ | 115.5 MHZ CH102X | H24 | 395724.0N 0411225.7E | 1769 M |  |  |
|  | ESENBOĞA VOR/DME | ESB | 112.1 MHZ CH58X | H24 | 400847.8N 0330044.9E | 960 M |  |  |
|  | ESKİŞEHİR VOR/DME | ESR | 108.2 MHZ CH19X | H24 | 394847.5N 0303044.8E | 795 M | Coverage 100 NM |  |
|  | GAP VOR/DME | GAP | 113.2 MHZ CH79X | H24 | 372729.1N 0385416.9E | 835 M |  |  |
|  | GAZİANTEP VOR/DME | GAZ | 116.7 MHZ CH114X | H24 | 365705.8N 0372822.7E | 705 M |  |  |
|  | GAZİPAŞA NDB | GZP | 316 KHZ | H24 | 361815.8N 0321751.0E |  |  |  |
|  | GEMEREK VOR/DME | GEM | 115.1 MHZ CH98X | H24 | 390913.7N 0360142.6E | 1527 M | Coverage 75 NM |  |
|  | GÖKÇEADA VOR/DME | GKA | 109.0 MHZ CH27X | H24 | 401047.0N 0255525.2E | 430 M |  |  |
|  | HATAY VOR/DME | HTY | 112.05 MHZ CH57Y | H24 | 362146.6N 0361724.7E | 88 M |  |  |
|  | HAYMANA NDB/DME | HAY | 350 KHZ CH55X | H24 | 392611.9N 0323034.2E | 1303 M | Coverage 75 NM Coverage 65 NM |  |
|  | IĞDIR VOR/DME | GDR | 117.7 MHZ CH124X | H24 | 395821.4N 0435310.7E | 939 M |  |  |
|  | İNEBOLU VOR/DME | INB | 113.3 MHZ CH80X | H24 | 415705.1N 0334222.6E | 646 M | Coverage 150 NM |  |
|  | ISPARTA VOR/DME | IPT | 117.5 MHZ CH122X | H24 | 375031.8N 0302041.2E | 866 M | Coverage 75 NM |  |
|  | İSTANBUL VORTAC | IST | 112.5 MHZ CH72X | H24 | 405741.3N 0284838.5E |  |  |  |
|  | İZMİR VOR/DME | IMR | 113.7 MHZ CH84X | H24 | 381902.8N 0270026.0E | 1007 M |  |  |
|  | KARS VOR/DME | KAR | 113.8 MHZ CH85X | H24 | 403324.1N 0430614.3E | 1800 M | Coverage 75 NM |  |
|  | KASTAMONU VOR/DME | KST | 115.2 MHZ CH99X | H24 | 412101.1N 0334759.5E | 1032 M |  |  |
|  | KAYSERİ VOR/DME | KSR | 116.3 MHZ CH110X | H24 | 384632.3N 0353119.0E | 1065 M | Coverage 100 NM |  |
|  | KULA VOR/DME | DEN | 112.05 MHZ CH57Y | H24 | 383441.1N 0283609.7E | 954 M |  |  |
|  | MALATYA VOR/DME | ERH | 112.0 MHZ CH 57X | H24 | 382745.1N 0380643.7E | 832 M | Coverage 100 NM At field |  |
|  | MARDİN VOR/DME | MRD | 116.9 MHZ CH116X | H24 | 371339.0N 0403819.6E | 529 M | Coverage 100 NM |  |
|  | MİLAS-BODRUM VOR/DME | BDR | 116.7 MHZ CH114X | H24 | 371527.7N 0273815.9E | 9 M | Coverage 75 NM |  |
|  | MUŞ VOR/DME | MUS | 111.2 MHZ CH49X | H24 | 384459.7N 0413940.3E | 1276 M | Coverage 80 NM Coverage 100 NM |  |
|  | MUT VOR/DME | MUT | 112.3 MHZ CH70X | H24 | 365146.9N 0331730.6E | 1768 M | Coverage 50 NM |  |
|  | ORDU-GİRESUN NDB | ORI | 350 KHZ | H24 | 405752.4N 0380510.1E |  |  |  |
|  | RİZE-ARTVİN VOR/DME | ART | 110.45 MHZ CH41Y | H24 | 411048.8N 0405131.3E | 15 M |  |  |
|  | SİİRT VOR/DME | SRT | 114.3 MHZ CH90X | H24 | 375438.4N 0415255.1E | 962 M | Coverage 50 NM |  |
|  | SİNOP VOR/DME | SIN | 114.0 MHZ CH87X | H24 | 420122.9N 0350434.0E | 14 M | Coverage 75 NM |  |
|  | SİVAS VOR/DME | SIV | 114.2 MHZ CH89X | H24 | 394722.0N 0365336.3E | 1615 M |  |  |
|  | ŞIRNAK VOR/DME | RNA | 112.3 MHZ CH70X | H24 | 372154.3N 0420341.2E | 629 M |  |  |
|  | TEKİRDAĞ VOR/DME | EKI | 116.3 MHZ CH110X | H24 | 405703.8N 0272534.2E | 323 M |  |  |
|  | TOKAT VOR/DME | TKT | 115.0 MHZ CH97X | H24 | 401906.6N 0362209.6E | 568 M |  |  |
|  | TRABZON VOR/DME | TBN | 114.9 MHZ CH96X | H24 | 405943.0N 0394744.3E | 17 M | Coverage 75 NM |  |
|  | TUZKÖY VOR/DME | TZK | 115.3 MHZ CH100X | H24 | 384600.3N 0343231.5E | 950 M | Coverage 75 NM |  |
|  | VABEL VOR/DME | EZN | 112.05 MHZ CH57Y | H24 | 395613.8N 0385313.3E | 2252 M |  |  |
|  | VAN VOR/DME | VAN | 115.2 MHZ CH99X | H24 | 382758.0N 0431930.6E | 1659 M |  |  |
|  | YALOVA VOR/DME | YAA | 117.7 MHZ CH124X | H24 | 402830.1N 0291227.2E | 879 M |  |  |
|  | YÜKSEKOVA VOR/DME | YKV | 112.1 MHZ CH58X | H24 | 373257N 0441433E | 1865 M |  |  |
|  | ZAFER VOR/DME | KTH | 112.3 MHZ CH70X | H24 | 390624.4N 0300805.2E | 1018 M |  |  |

UZBEKISTAN

|  | Name of station (VAR) | Id | Frequency (CH) | Hours of operation | Geographical Coordinates | Degrees with decimal fractions | ELEV DME antenna | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| UZBEKISTAN | DJIZAK DVOR DME | JZK | 112.2 MHz | HO | N401516.7 E0675439.3 |  | 1100 | R=135М.миль |
|  | KHANABAD DVOR DME | HNB | 112.8 MHz | HO | N384941.1 E0655518.8 |  | 1300 | R=124М.мили |
|  | MUYNAK DVOR DME | MNK | 114.6 MHz | H24 | N434507.5 E0590207.1 |  | 200 | R=113М.миль |
|  | NAMANGAN DVOR DME | NMA | 116.0 MHz | H24 | N405855.1 E0713338.0 |  | 1600 | R=54М.мили |
|  | NAVOI VOR DME | NVI | 113.8 MHz | H24 | N400653.4 E0651035.2 |  | 1200 | R=174М.мили |
|  | SAMARKAND DVOR DME | SKD | 115.0 MHz | H24 | N394213.5 E0665821.6 |  | 2200 | R=108М.миль |
|  | TASHKENT DVOR DME | TKT | 113.2 MHz | H24 | N411516.1 E0691513.0 |  | 1400 | R=143М.мили |
|  | TASHKENT DVOR DME | TVS | 112.7 MHz | H24 | N411830.4 E0692146.8 |  | 1600 |  |
|  | TERMEZ VOR DME | TRZ | 113.4 MHz | H24 | N371712.1 E0671903.9 |  | 1100 | R=110М.миль |
|  | ZAAMIN CVOR DME | ZMN | 116.5 MHz | H24 | N400051.4 E0682422.1 |  | 1800 | R=123М.мили |
| TAJIKISTAN | KHUJAND DME | HND | 114.30 MHz | H24 | 401250N 0694205E | 40.213889° 69.701389° | 477M |  |
|  | ISTARAVSHAN DME | IRN | 113.90 MHz | H24 | 400338N 0685644E | 40.060556° 68.945556° | 713M |  |
| KYRGYZSTAN | OSH DVOR/DME (5°E/2023) | OSH | 112.4 MHZ CH 71X | H24 | 403622.21N 0724807.38E | 40.606169° 72.802050° |  |  |
| TURKMENISTAN | DASHOGUZ DVORDME | DSH | 112.00 | H24 | 414535.88N 0594742.85E |  | 300 |  |
|  | TURKMENABAT CVORDME | LBA | 109.20 | H24 | 385449.00N 0633522.00E |  | 600 |  |

8. MON design

Although GNSS (GPS) signal are used as the primary navigation sensor considering observed constellation parameters such as average availability of 6 satellites in view, accuracy and 100% RAIM availability, the significant impact of Radio Frequency Interference (RFI) to GNSS functionality indicated in the airspace of ANSISG Region requires the necessary measures to be taken to ensure that navigation service remains available for airspace users. Those measures shall include retaining a network of conventional navigation aids and related surveillance and communications infrastructure.

Consequently, matching up the local fleet avionics capability with a particular navigation specification requires that the infrastructure is available to support all potential airspace users. Specifically, VOR/DME infrastructure for RNAV5, and DME/DME infrastructure for RNAV5 and RNAV1 should be provided. However, if it would be cost prohibitive or impractical (terrain limitations etc.) to provide a specific type of infrastructure coverage.  The application of a cross-border approach to MON is recommended taking into account the available ground-based NAVAIDS in the neighbouring States. This will allow the continuity of PBN operations without reverting back to conventional navigation in the event of GPS outages.

in case of cross-border NAVAIDs, the States (ANSPs) should ensure that they have adequate information exchange on the status of navaids, e.g. in respect to maintenance or malfunctions, with the owner of that specific navaid.

ANSPs that share NAVAID services with neighbouring States should consider establishing agreements to include NAVAIDs from the adjoining State in its navigation solution and MON strategy. Collaboration between ANSPs from neighbouring States is essential for designing routes and procedures that cross international boundaries. Additionally, reliance on another State’s NAVAID infrastructure for GNSS contingency can help reduce the cost of implementation and life-cycle maintenance of its own NAVAID infrastructure. The shared use of NAVAIDs may require the establishment of bilateral agreements between the neighbouring ANSPs. ICAO has identified this relationship as a contingency requirement under Regional Air Navigation Plans.

Whereas ICAO Assembly Resolution A 37-11 urges States, inter alia, to implement approach procedures with vertical guidance (APV) (Baro-VNAV and/or augmented GNSS), including LNAV only minima, for all instrument runway ends, either as the primary approach or as a back-up for precision approaches, and considering that RNP APCH specification requires GNSS, States are encourage to maintain the Minimum Operational Network of ILS to ensure the safe diversion for aerodromes with stand-alone RNP APCH procedure.

8.1. ANSISG MON structure

ANSISG Regional Minimum Operational Network is composed of National Networks of:

- Algeria

- Armenia

- Azerbaijan

- Georgia

- Moldova (supported by cross-border service of Romania)

- Morocco

- Türkiye, and

- Sub-regional cross-border MON including Kazakhstan, Kyrgyzstan, Tajikistan and Uzbekistan

Algeria

The existing VOR/DME network of Algeria fully supports RNAV 5 operations and RNAV 1 operations at the main aerodromes

Armenia

Two existing VOR/DME stations fully supports RNAV 5 operations but may be not sufficient for RNAV 1. However, air space users may consider the closely located VOR/DME stations in Azerbaijan and Georgia for GNSS contingency purpose. The status of the NAVAIDS of neighbouring States should be clarified through the NOTAM at the preflight phase

Zvartnots DVOR-DME (ZVR)  FL195-FL280                                        Guymri DVOR-DME (ZVR)  FL195-FL280

Azerbaijan

The existing VOR/DME network of Azerbaijan fully supports RNAV 5 operations and RNAV 1 operations at the main aerodromes

Georgia

The existing VOR/DME network of Georgia fully supports RNAV 5 operations and RNAV 1 operations at the main aerodromes

Moldova (supported by cross-border service of Romania)

The existing VOR/DME network of Moldova with support by cross-border service of Romania fully supports RNAV 5 operations and RNAV 1 operations at the main aerodromes

Morocco

?????????????????????????

Türkiye

En-route: RNAV5 specification is applied

- Conventional route structure remains for contingency

- Radar coverage available all-around FIR, when needed/requested for Radar vectoring is provided to the IFR aircraft in case of loss of GNSS signal

TMAs with Radar Service: RNAV1 SID/STARs are applied.

Conventional SIDs/STARs are available as back-up to PBN procedures.

In case of loss of GNSS signal, Radar vectoring can be provided when needed

2.TMAs without Radar Service: RNP1 SID/STARs are applied.

- Conventional SID/STAR are available as the back-up procedure for loss of GNSS signal.

- DME-DME method is being implemented to support RNP1SIDsandSTARs in case of loss of GNSS signal

Sub-regional cross-border MON including Kazakhstan, Kyrgyzstan, Tajikistan and Uzbekistan

RNAV 5 specification is ensured by existing VOR/DME and DME network in case of GNSS outage considering the FMS resistance to degradation.

RNAV 1 is partially ensured by DME/DME network

All the States have the VOR/DME, DME deployment plans to improve the coverage. States are encourage to focus on DME/DME deployment as an optimal scenario.

Appendix A. Reference documents

ICAO DOC 9750 Global Air Navigation Plan (GANP)

ICAO DOC 9854 Global Air Traffic Management Operational Concept

ICAO DOC 9882 Manual on Air Traffic Management System Requirements

ICAO DOC 9992 Manual on the Use of PBN in Airspace Design

ICAO Annex 10, Volume I Radio Navigation Aids, Seventh Edition, July 2018

ICAO Annex 11, Air Traffic Services

ICAO Performance-based Navigation Manual, ICAO, Doc 9613, Edition 4, 2013

FAA PERFORMANCE BASED NAVIGATION, PBN NAS NAVIGATION STRATEGY 2016

European Navaid Infrastructure Planning Handbook including Minimum Operational Network (MON), PBN

HANDBOOK No. 4

European GNSS Contingency/Reversion Handbook for PBN Operations, PBN HANDBOOK No. 6

--END--