- …

- Utilization of the frequency band 117.975 – 137 MHz

  - The table is based on Annex 10, Volume V, 4.1.1 also incorporating regional agreements on specific uses of individual frequencies or sub-bands and the identification of the assignable 25 kHz and 8.33 kHz channels:

| Channel | Frequencies (MHz) | Worldwide General utilization | EUR special application or remarks |
| --- | --- | --- | --- |
| 118.000 – 121.460 inclusive | 118.000 - 121.4583 inclusive | Aeronautical Mobile Services | 119.700 MHz reserved for regional guard supplementary tower and approach services. |
| 121.500 | 121.500 | Emergency frequency |  |
| 121.540 – 121.990 inclusive | 121.5417 - 121.9916 inclusive | Aerodrome Surface Communications |  |
| 122.000 – 123.060 123.065 | 122.000 - 123.0583 123.0667 inclusive | Aeronautical Mobile Services | 122.100 MHz reserved as common channel to provide supplementary TWR/APP services in a number of nations. 122.500 MHz reserved for regional light aviation. Channel 123.065 reserved for air-to-air General Aviation communi-cations |
| 123.100 | 123.100 | Auxiliary frequency SAR |  |
| 123.140 123.135 – 136.490 | 123.1417 123.1333- 136.4916 inclusive | Aeronautical Mobile Services 123.450 MHz reserved for air-to-air communications for flights over remote and oceanic areas out of range of VHF ground stations | Channel 123.135 reserved for air-to-air General Aviation communi-cations 123.300 MHz reserved as common channel to provide PAR services in a number of nations. 123.450 MHz reserved for air-to-air communications only for flights over remote and oceanic areas out of range of VHF ground stations 123.500 MHz reserved for regional light aviation 131.400 – 131.975 MHz reserved for operational control (OPC) communications* 131.525, 131.725 and 131.825 reserved for ACARS data link |
| 136.500 – 136.675** | 136.500 - 136.675 | Aeronautical Mobile Services | New assignments permitted only on a temporary basis.*** |
| 136.700 – 136.975** | 136.700 - 136.975 inclusive | Aeronautical Mobile Services 136.900 - 136.975 MHz reserved for air-ground data link communications on a worldwide basis. | reserved for air-ground data link communications (see paragraph 2.2.) |

Guard bands:

For 121.500: Channels 121.475, and 121.525, are not assignable; and

For 123.100: Channels 123.065, 123.075, 123.080, 123.085, 123.115, 123.125 and 123.130, and 123.135 are not assignable, except for ATIS on channels 123.080 and 123.130.

*OPC:	Assignments used for OPC purposes shall not be co-ordinated on protected standard channels. 8.33 kHz channel spacing to be used wherever possible, between 131.405 – 131.980 except for the ACARS channels 131.525, 131.725 and 131.825. The use of voice at the first adjacent 8,33 kHz channel to ACARS is not possible. Care should be taken when considering the use of the second adjacent channel at 16,67 kHz from the ACARS.

Unprotected assignments within this band can accommodate services other than OPC, where local planning conditions permit.

Concerning the use for ATS purposes of the lower adjacent frequencies to 131.400 and the upper adjacent frequencies to 131.975, care should be exercised for the observance of adjacent channel compatibility requirements.

**Note:	8.33 kHz channel spacing is not to be used for frequencies 136.500 – 136.975 MHz inclusive (Annex 10, Volume V,

paragraph 4.1.6.1.1).

***Note:	The the temporary allotment of the frequency 136.675 MHz for VDL 2 is extended until 31 December 2027 as coordinated with FMG members during FMG29 (EASPG Decision 5/18 refers):

- with reference to the EASPG Conclusion 3/13, the temporary reservation of channels 136.650 and 136.675 for VDL Mode 2 services be removed by end of 2027.

- until the end of 2027, no proposals for frequency assignments on channels 136.650 and 136.675 for voice communication services should be admitted and that such proposals on a temporary basis be readmitted as of 1 January 2028.

- channel 136.650 until the end of 2027 serves as guard channel. The possibility of allotting channel 136.650 to voice communication services prior to the end of 2027 is under consideration.

-----------------------------

(footnote continued)