# WORK INSTRUCTIONS

ON MET OBSERVATIONS AND REPORTS

# Revision 01

2011

| PREPARED | REVISED | APPROVED |
| --- | --- | --- |

# SUMMARY

- Objectives

- Scope

- Responsibilities

- References

- Requirements

- Description of Activities

- Records

- Glossary

- Annexes

- OBJECTIVE

  - Establish the guidelines for conducting observations and preparing MET reports at an aeronautical meteorological station,in order to provide the following information: MET REPORT, SPECIAL, METAR, and SPECI.

- SCOPE

  - These work instructions apply to the aeronautical meteorological station (location indicator) of the(name of the aerodrome)airport.

- RESPONSIBILITIES

  - The meteorology technician (MET observer) is responsible for:

- Verifying the operating condition of communication equipment and media forthe preparation andtransmission of MET information (see activity 4101);

- Conducting observations and preparing MET (MET, SPECIAL, METAR, and SPECI)reports and sending them, on a timely basis,via the aeronautical fixed communication network. Likewise, recording MET parameters in meteorological templates (record R01-EMA…-01, act. No. 4102);

- Coordinatingwith the associated AMO the inclusion of trend forecasts, based onCAR/SAM FASID Table MET 1A (Act. No.4102);

- Keeping continuous watch over meteorological conditions (act. No. 4102);

- Informing the associated AMO(Table MET 1A of the CAR/SAM FASID)about observed meteorological conditions that might give rise to a windshearwarning and alert;

- Completing the records specified in these work instructions;

- Operating the conventional MET station in case of failure at the automatic MET observing station;

- Reportingany equipment and observation system failures, as well as operational occurrences,on the service sheet of the Integrated Safety Reporting service.

- REFERENCES

  - Management Manual

  - Drafting and presentation of MET/QMS documents (PR-4.2-AGC-1)

  - Control of MET/QMS documents (PR-4.2-AGC-2)

  - Control of MET/QMS records (PR-4.2-AGC-3)

  - Doc 9873 – Manual on the Quality Management System for the Provision of Meteorological Service to International Air Navigation

4.6	Latest version of ICAO Annexes:

- Annex 3 – Meteorological service for international air navigation, seventeenth edition;

- Doc 8896 – Manual of aeronautical meteorological practices;

- Doc 9837 – Manual on automatic meteorological observing systems at aerodromes;

- Doc 9328 – Manual on runway visual range observing and reporting practices; and

- Aeronautical information publication - AIP

4.7	Latest version of World Meteorological Organization (WMO) documents:

- WMO No 8 – Guide to meteorological instruments and methods of observation;

- WMO No 306, Volume I – Manual on codes;

- WMO No 731 – Guide on meteorological observation and information distribution systems at aerodromes;

- International Cloud Atlas (WMO No. 407).

Note: Or national regulatory reference.

- REQUIREMENTS

  - National aeronautical regulations or ICAO Annex 3.

  - ISO 9001:2008

- DESCRIPTION OF ACTIVITIES

  - Verify the operating condition of MET equipment and communication media (Activity 4101)

  - Surveillance, observation, drafting and dissemination of MET REPORT, METAR, SPECIAL, and SPECI reports; as well as recording of MET parameters in the established records (Activity 4102 and Activity 4103)

Note. – The coding of the activitiesof the aeronautical meteorological station starts at 4100, where the first digit corresponds to meteorology processes, according to the ECCAIRS ADREP document (ADREP: the ICAO Accident/Incident Data Reporting System, andECCAIRS: European Co-Ordination Centre for Aviation Incident Reporting Systems).

- RECORDS

| IDENTIFICATION | STORAGE | PROTECTION | RETRIEVAL | RETENTION | DISPOSAL |
| --- | --- | --- | --- | --- | --- |
| R01-EMA…-01 Record of routine and special observations | (Name of the climatological unit) | Printed | Chronological | Not defined | Climatological archive |
| #### R01-EMA…-0 2 #### METAR EMA… | Aeronautical meteorological station | Printed/Digital | Chronological | 30 days | Eliminate |
| R01-EMA…-03 MET REPORT EMA… | Aeronautical meteorological station | Printed/Digital | Chronological | 30 days | Eliminate |
| R01-EMA…-04 SPECI EMA… | Aeronautical meteorological station | Printed/Digital | Chronological | 30 days | Eliminate |
| R01-EMA...-05 SPECIAL EMA… | Aeronautical meteorological station | Printed/Digital | Chronological | 30 days | Eliminate |
| R02-EMA…-01 Service sheet EMA… | Aeronautical meteorological station | Printed/Digital | Chronological | 6 months | Eliminate |

- GLOSSARY

AFTN:

Aeronautical fixed telecommunication network. Telecommunication circuit that transmits operational messages in the form of text, according to international standards, for exclusive use of aviation.

AMHS:

(Aeronautical Message Handling System)

An aeronautical message management system.

# AWOS

Automated integrated observing systemfor the dissemination and display of meteorological information, which provides for manual insertion of data observations, including meteorological elements that cannot be observed using automatic means.

# Runway visual range (RVR):

The range over which the pilot of an aircraft on the centre line of a runway can see the runway surface markings or the lights delineating the runway or identifying its centre line.

# Meteorological parameter:

One of the atmospheric variables or phenomena that characterise the physical state of the atmosphere related to a specifiedlocation and time.

# Aeronautical meteorological station:

A station designated to make observations and meteorological reports for use in international air navigation.

# Meteorological report:

A statement ofobserved meteorological conditions related to a specified time and location.

# Meteorological message:

A message made up by a single meteorological bulletin, preceded by a preliminary lineand followed by the end-of-message signals.

METAR:

A routine aviation weather report.

# (Meteorological) observation:

# Evaluation of one or more meteorological parameters.

# Aerodrome meteorological office:

An office located at an aerodrome, designated to provide meteorological service for international air navigation.

# Routine observations and reports

Meteorological observations conducted at one-hour intervals.  They are disseminated in abbreviated plain languageat the aerodrome of origin as local routine reports (MET REPORT) oras METARs to other aerodromes other aerodromes outside of the aerodrome of origin.

# Special observations and reports

These are special meteorological observations made when significant changes occurin surface wind, visibility, runway visual range, current weather, clouds, or air temperature.  They are disseminated at the aerodrome of origin to other aerodromes outside of the aerodrome of origin as local special reports (SPECIAL), oras SPECI to other aerodromes outside of the aerodrome of origin.

# Requirement:

# An established need or expectation, generally implicit or mandatory.

SPECI:

A selected special aeronautical meteorological report.

TREND:

A trend-type landing forecast.

# Visibility:

Visibility for aeronautical purposes is the greater of:

- the greatest distanceat which a black object of suitable dimensions, situated near the ground, can be seen and recognised when observed against a bright background;

- the greatest distance at which lights in the vicinity of 1,000 candelas can be seen and identified against an unlit background.

Note.- The two distances have different values in air of a given extinction coefficient, and the latter b) varies with the background illumination.  The former a) is represented by the meteorological optical range (MOR).

# Prevailing visibility:

The highest visibility value observed according to the definition of “visibility”, obtained within a circle that covers at least half of the horizon or at least half of the surface of the aerodrome.  These areas could encompass adjacent or non-adjacent sectors.

- ANNEXES

Annex I:	Revision status control matrix

Annex II:	Record of routine and special observations

Annex III:	METAR EMA... report

Annex IV:	MET REPORT EMA…

Annex V:	SPECI EMA...

Annex VI:	SPECIAL EMA...

Annex VII:	Service sheet

ANNEX I

REVISION STATUS CONTROL MATRIX

CODE: R01-AGC.CD-05	REVISION: 00/XX-XX-2011

| REVISION | PREPARED | REVISED | APPROVED | REASON FOR THE CHANGE | DATE |
| --- | --- | --- | --- | --- | --- |
| 00 |  |  |  |  |  |
| 01 |  |  |  |  |  |

ANNEX II

RECORD OF ROUTINE AND SPECIAL OBSERVATIONS

CODE: R01-EMA…-01	REVISION: 00/XX-XX-2011

| Code | CCCC | GGggZ | dddffGfmfm | dndndnvdxdxdx | VVVVDv | RVR | W’W’ | NsNsNshshshs | NsNsNshshshs | NsNsNshshshs | T’T’ | TdTd | QPhPhPhPh | Supplemen-tary Information | OBS | HR | BH | PoPoPoPo |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

ANNEX III

METAR REPORT EMA....

CODE: R01-EMA...-02	REVISION: 00/XX-XX-2011

# Example

# GG SPIMCOLE

# 171554 SPIMZXOB

# SAPR01 SPIM 171500

# METAR SPIM 171500Z 20005KT CAVOK 26/20 Q1010 NOSIG RMK PP000=

# Note: SPIMCOLE (set of AFTN addresses for sendingMETAR information)

# ANNEX IV

MET REPORT EMA....

CODE: R01-EMA...-03	REVISION: 00/XX-XX-2011

# Example

# GG INFOMETX

# 171450  SPIMZXOB

# MET REPORT SPIM 171500Z WIND RWY 15 TDZ 200/5KT MAX 6KT RWY 33 TDZ 210/5KT MAX 6KT CAVOK T26 DP20 QNH 1010HPA TREND NOSIG=

# Note: INFOMETX (Set of AFTN addresses for sendingMET REPORT information)

# ANNEX V

SPECI EMA...

CODE: R01-EMA...-04	REVISION: 00/XX-XX-2011

# Example:

# GG SPIMCOLE

# 061109 SPIMZXOB

# SPPR01 SPIM 061110

# SPECI SPIM 061110Z 19005KT 0500 R15/0350DFG OVC001 17/17 Q1011=

# ANNEX VI

SPECIAL EMA...

CODE: R01-EMA....-05	REVISION: 00/XX-XX-2011

# Example:

# GG INFOMETX

# 061109 SPIMZXOB

SPECIAL SPIM 061110Z WIND RWY15 TDZ 19005KT MAX10KT VIS 500M RVR RWY15 TDZ 350M RWY33 700M T17 DP17 QNH 1011=

# ANNEX VII

SERVICE SHEET

CODE: R02-EMA…-01	REVISION: 00/XX-XX-2011

Example: