|  | International Civil Aviation Organization REPORT | 24 February, 2023. |
| --- | --- | --- |

FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

SIXTEENTH MEETING OF THE WORKING GROUP OF FSMP

(FSMP-WG/16)

Hybrid

15-24 February, 2023

REPORT

- Opening and Working Arrangements

  - The hybrid (in-person and electronic/virtual) meeting was opened by Mr Loftur Jonasson from the ICAO Secretariat, Montreal and Mr Andrew Roy, the Rapporteur of Working Group FSMP (FSMP-WG).  The in-person portion of the meeting was held at the ICAO Asia and Pacific (APAC) office in Bangkok, Thailand and the meeting greatly appreciated the facilities and the assistance provided by ICAO APAC office. Mr Jonasson acted as the Secretary of the meeting, assisted by Mr. Luo Yi and Mr. Derek S.L. How. Mr Roy welcomed the group and provided introductory remarks and meeting information. Lastly, it was noted that as for the last few meetings, developing a run-of-meeting schedule was much simpler due to membership providing input contributions in a timely manner, and the cooperation was appreciated.

  - The meeting was held in English. After the opening of the meeting the agenda was approved by the group. The agenda is contained in Appendix A.

  - The list of papers submitted for consideration by FSMP-WG/16 is contained in Appendix B. The list of participants is in Appendix C.

  - The material in this report is organized by meeting agenda item number and does not necessarily reflect the order of discussions.  Actions captured during discussions are shown in Appendix D, together with the status of prior-meeting(s) actions.

- Agenda Item 2 – ICAO WRC-23 Position

  - Identified conflicts between administration preliminary proposals and ICAO Position

    - No contributions were received on this topic.

    - Discussion of possible future agenda item proposals from administrations

    - No contributions were received on this topic.

  - Modifications/Updates to the ICAO WRC-23 Position

    - Eight contributions (WP06, WP10, WP11, WP14, WP15, WP17, WP20, WP22, WP23) were received with proposed changes to the ICAO Position. In addition, several proposals were considered that were not resolved at FSMP-WG/15. Flimsy01 was developed that combined all proposals and was used by the meeting for the review.  After an extensive discussion of multiple topics that included additional meeting time, multiple offline consultations, and topic specific Flimsies being generated (Flimsy02, 03, and 04), Flimsy01 incorporated these updates and was updated as per Appendix E, containing all the consensus language. Concerning WRC-23 AI 1.8, it was not possible to find a way to clarify the status of Article No. 4.10 in the specific ICAO position of this agenda item; however, in this regard, §3.4 of the ICAO position document was highlighted. The ICAO Secretariat was requested to send the update to the Air Navigation Commission (ANC), informing them on the relevant changes since the 2021 ICAO State Letter.

      - During the development of Flimsy01, significant discussion centred on WRC-23 agenda item 1.7.  After multiple sessions and offline opportunities for discussion, a consensus was only reached on agenda item 1.7 after a simplified compromise text was produced by the Chair for consideration of the meeting.  While the meeting reached a consensus to this text on a compromise basis, one meeting attendee still did not agree with the outcome and has provided the following statement for this report:

‘This attendee considers that the new proposed position of ICAO on AI 1.7 introduces a significant change on a very high level strategic consideration: the targeted spectrum. The new position raises a concerning artificial language on a potential reduction of the aeronautical spectrum which may be allocated by WRC 23 to the AMS(R)S. This is expressed in the new words “…all of parts of…” This new language is not in the current position of ICAO which clearly advocates, since 2020, in favor of supporting for a global allocation to the aeronautical mobile-satellite (route) service for both the Earth-to-space and space-to-Earth directions in the frequency band 117.975-137 MHz and that the use of the allocation be limited to the relaying of aeronautical VHF air traffic management communications. The new language proposed by FSMP at its #16th meeting (February 2023) indicates that ICAO would be supporting the allocation in part of the 117.975-137 MHz band. This element of the ICAO position should only be substantiated on high level strategic consideration on the needs of the aeronautical sector. Spain disagrees with the new introduction of a ICAO self-reduction of targeted spectrum, noting the advances on the understanding of the technical compatibility of AMS(R)S and AM(R)S regarding the use of data link over satellite and terrestrial systems as it has been presented in FSMP/WG16 WP18. The possibility to operate AMS(R)S up to 137 MHz is critical for efficient implementation of data link systems over satellite.

The above consideration are even more strange noting that this is the most and unique update of the position, as the other elements of the position (operation in accordance with international Standards and Recommended Practices and procedures protection of the operation of existing VHF systems in the band 117.975-137 MHz, are maintained, with similar language to the existing ICAO position.

Spain expects that, due to the strategic/institutional nature of the elements proposed for modification of ICAO position, the SUPERIOR COMMITTEE can reconsider the position and avoid the negative impact which such position would impose on the expectations that the WRC 23 allocation to AMS(R)S would allow operation of data link services over satellite, which require the operation till 137 MHz.’

    - Additionally, after the agreement had been reached on the ICAO position on WRC-23, one panel member requested the following statement to appear in the report:

‘I am concerned a lot of the discussion and issues we encountered at this meeting with finalizing the ICAO position for WRC-23 is a result of not properly assessing within aviation the proposals for WRC agenda items before seeking ICAO’s endorsement.  I have repeatedly raised these concerns during this WRC study cycle in the meeting, and would fully support a gating process for future proposals as under discussion in the ICAO ICNSS TF to ensure we have a suitable understanding of a system before attempting to obtain spectrum at the ITU.’

    - WP04 proposed to set up a correspondence group to develop text to take into consideration the ITU BR Director’s Report on GNSS RFI for consideration at the Conference Preparatory Meeting (CPM).  While the meeting agreed with the intent of the proposal, the membership noted that it was for States to raise the issue directly with the ITU for the attention of the BR Director, and not ICAO.  However, it was agreed that the FSMP membership could create a correspondence group (CG) for the issue to help collect the relevant information that may be helpful for the Administrations of individual panel members should they wish to submit something to ITU-R WP 4C.  The output document was created as Flimsy06 in Appendix F, creating the TORs for the CG and also some initial material generated.  It was noted that this early material had not been formally reviewed by the meeting and therefore had no formal status other than providing the meeting with some initial thoughts.  Additionally, it was stated that the output from the CG would have no formal status and no action would be taken by FSMP when the CG reported back at its next meeting.  Action Item 16-01 was created to encourage inputs to the CG to help develop the text.

- Agenda Item 3 – Development of (planned) material for ITU-R studies

  - WRC-23 AI 1.6 Suborbital Vehicles

    - No contributions were received on this topic.

  - WRC-23 AI 1.7 VHF AMS(R)S

    - A number of papers were received on the topic of WRC-23 agenda item 1.7. To manage the discussion, they were broken into 3 groups: (a) papers providing a system overview, (b) papers associated with specific technical/operational topics, and (c) papers associated with planning a way forward on the item. Once all of the papers within a certain category were introduced, then the meeting held an overall discussion.

    - SATCOM VHF System Overview:

      - WP18 presented a concept of operations for VHF SATCOM datalink that considered the potential impact on current avionics and datalink operational procedures.  It concluded that satellite-based datalink services could be integrated in existing low-density areas using current operational procedures without any modifications in current avionics other than standard operational procedures, and satellite-based datalink services could be provided assuring no interference with the current terrestrial VDL infrastructure.  There was a short discussion about use of the VHF Air-Ground Digital Link Mode 2 (VDLM2) common signalling channel (CSC) on 136.975 MHz and if a VHF SATCOM system would still be able to operate with only alternate VDLM2 and Plain Old ACARS (POA) frequencies.  The meeting appreciated the information and asked to be kept updated on the work.

      - IP05 presented an introduction to the SkyCraft network system and the plans to provide for a VHF SATCOM service.  It included the different VHF services it planned to provide, and details on the satellite constellation and coverage.  The meeting was grateful for the presentation and noted the contents of the information paper.

    - SATCOM VHF technical and operational topics:

      - WP09 proposed an AMS(R)S coordination threshold to facilitate frequency planning with the AM(R)S systems in the frequency band 117.975-137 MHz, based on the existing ICAO planning criteria of a desired to undesired signal (D/U) ratio of 20 dB. Several questions were received on the calculations and the potential variables for defining such a value, including if a specific coordination pfd was needed.

      - IP03 provided data contained in the ITU terrestrial online database (eTerrestrial), illustrating when coordination between AMS(R)S and terrestrial systems operating in the band 117.975-136.8 MHz would be possibly triggered in the framework of No. 9.11A.  There was a short discussion about the accuracy of the assignments for some administrations and the gaps in others, which would cause concerns if a 9.11A coordination process would not provide protection to unregistered stations.  Additional discussions focused on how an ITU-R Article 9 process might work and the conditions that would be applied.

    - SATCOM VHF - Finding a way forward on the agenda item:

      - WP08 was a continuation of a previous contribution FSMP WG/13/WP01, discussing the ongoing work within WP5B and the impact of agreements reached at WRC-19 AI 1.7.  The document discussed the interest and need for a guard band between the AMS(R)S service in the band 117.975-137 MHz and services in the 137-138 MHz band, supporting a No. 9.11A as the appropriate Article 9 coordination provision to be applied to this new primary AMS(R)S allocation.  The resulting discussion focused on adjacent band considerations and the effect of out of band interference from services above 137 MHz.

      - WP19 provided an extensive background on possible ITU and/or ICAO coordination considerations for any VHF SATCOM systems.  It also provided several concepts to be considered for frequency planning processes that could be used by ICAO.  There was a short discussion on the appropriateness of different coordination methods and how they would be applicable to any potential multi-provider environment.

      - WP24 explored the current elements surrounding the coordination and planning of VHF SATCOM services in relation to terrestrial networks, seeking clarifications on some of the concepts discussed.  Initial suggestions were proposed for discussion and potential incorporation into a Flimsy and LS to WP 5B as appropriate.  Several clarifications were received on the WP, including on the operational assumptions.

    - After all the papers were presented, the meeting as a whole expressed its appreciation for the additional information provided and noted that this helped focus the work on the ICAO position.  Further material to help with the development and understanding of the VHF SATCOM concept was encouraged at the next FSMP WG/17.

  - WRC-23 AI 1.8 Resolution 155/FSS for UAS

    - No contributions were received on this topic.

  - WRC-23 AI 1.9 Wideband HF

    - No contributions were received on this topic.

  - WRC-23 AI 1.10 Non-safety AMS

    - No contributions were received on this topic.

  - WRC-23 AI 9.2 Radio Regulations clean-up

    - No contributions were received on this topic.

  - Other

    - WP01 (and the proposed response in WP25) was a liaison statement from ITU-R WP5B, requesting information from ICAO with regard to Wireless Avionics Intra-Communication Systems (WAIC) transmitter technical characteristics as well as general aircraft deployment information including statistical data such as the number of simultaneous flights above oceans, for use in studies related to earth exploration-satellite service (EESS) (passive) and space research service (SRS) (passive).  WP25 provided a framework for an initial response with the draft Proposal for amendment to Annex 10 Volume V to include provisions for WAIC  as finalised by the FSMP and some preliminary estimates of aircraft oceanic density data.  While the meeting supported sending the information, it was agreed that additional time would be needed to develop a proper material for aircraft oceanic density values.  Therefore, elements for a reply liaison are contained in Flimsy05 - Appendix G, and an Action Item 16-02 was created to seek further input on oceanic density statistics to FSMP WG/17.

    - WP03 was a liaison statement from ITU-R WP5B, informing ICAO of work having been initiated on a working document toward a preliminary draft new Report ITU-R M.[AERO-PROVISIONS] with respect to Resolution 427 (WRC-19).  There was a resulting discussion on the work and how mature it could be for WRC-23 action.  It was agreed that Action Item 15-02 should be emphasised for the FSMP WG/17 meeting, seeking inputs on the what information may need to be considered on legacy or inconsistent Articles in the RR, and how it should be actioned noting other aviation topics at WRC-23.

- Agenda Item 4 – Radio Altimeter issues

  - Report from correspondence group on radio altimeters (CG-RA)

    - IP06 provided an update from the RA CG with a summary of RA activity since the FSMP WG/15 meeting.   In particular it provided a catalogue of recent meetings, studies and test efforts on the topic and references where more information could be found. The meeting greatly appreciated the information and a discussion ensued. The meeting asked the CG to continue its work collecting updates on relevant activities associated with the radar altimeter to help inform the future work of the FSMP.

  - National efforts to implement broadband mobile near 4200-4400 MHz

    - WP02 was an update on the Secretariat work with FSMP and the ICAO Middle East Regional Office (MID) Radio Altimeter (RADALT) Action Group (AG) to develop and finalize guidance material to protect radio altimeters from potential harmful interference from new cellular broadband technologies such as 5G.  The WP presented the latest draft of this guidance material.  The ICAO Secretariat requested any additional comments or edits be submitted to them directly for consideration.  After a discussion on the paper, the meeting endorsed the material broadly and agreed to provide comments to the ICAO Secretariat within 2 weeks after the meeting (Action Item 16-03).

    - WP05 provided a summary of the different radar altimeter interference scenarios and also some preliminary views of drawback and advantages for each of them. Comments and views were sought in order to try to align aviation community view on what scenarios are suitable to address the 5G/RA coexistence issue.  Discussion of the different scenarios reflected a range of views in the meeting on the appropriate considerations for a suitable interference scenario.  Given the points raised, WP26 was then introduced to continue the discussion.

  - Development of SARPS material for Annex 10, Vol. V

    - WP26 provided an initial framework and plan to begin the development of ICAO SARPs for the radar altimeters.  It also considered how the process related to the RTCA/Eurocae work and the ITU-R.  Lastly it requested the material be sent to the ICAO Flt Ops Panel to help support ongoing national efforts to define runway safety zones.  On the proposed SARPs framework, there was a short discussion on what category of parameters might be considered in any future ICAO SARPs.  As a result, it was requested that future contribution be sought to the meeting on appropriate material (Action Item 16-04) and also to send the document (along with WP05) to RTCA/EUROCAE for their consideration and input (Action Item 16-05).   Additionally, the request to seek initial ICAO comments on operational scenarios and what is needed to define runway safety zones resulted in Action Item 16-06 for the ICAO Secretariat to supply WP05 and WP26 to the Flt Ops Panel for comments and any interim guidance it could provide on the operational topics raised.  It was requested for a response back to the FSMP at its Aug 2023 meeting.

    - IP04 discussed the plans for a new Technical Standard Orders for radio altimetry equipment from the FAA when the standards at RTCA and EUROCAE are complete. These new standards are expected to be of interest to the Panel when developing proposed amendments for Annex 10.  Expanding on this, IP07 provided background information and progress of work within the joint effort of RTCA and EUROCAE to update radio altimeter standards.  The resulting discussion had several questions on the testing and potential performance on new altimeter designs.  It was noted that the DO-399 document being currently published is not a final performance standard, only an interim document that is intended for feedback in developing the final radar altimeter standard for 2024.  It was also stated that DO-399 is not a reflection of current altimeter performance.  Support of the development of a new RTCA/EUROCAE performance standard was encouraged to ensure all relevant information can be captured and accounted for in the new standard.

- Agenda Item 5 – 5 GHz Planning

  - AeroMACS

    - No contributions were received on this topic

  - Global UAS/Remotely Piloted Aircraft Systems (RPAS) channel plan

    - No contributions were received on this topic.

- Agenda Item 6 –  Interference from Non-Aeronautical Sources

  - Mobile service communications adjacent to 1518 MHz satellite communications frequency band (see action item 14-04)

    - WP12 and WP16 proposed updates to the note on MSS L-band from FSMP WG/12 Appendix H.  After a discussion on the contents of the papers, there were several concerns raised about the retrofits for aircraft operating in Europe and the uncertainty generated for aviation by the regional process to implement IMT around 1518 MHz.  It was agreed to continue the work in the CG, with a focus in the meeting to consolidate text between the two papers and remove track changes in areas that could be agreed to at this stage. The output document was created as Flimsy07 in Appendix H, noting that the work had not been formally reviewed by the meeting and therefore had no formal status other than requesting continued input as per the previous Action Item 14-04.

  - LED lighting systems

    - No contributions were received on this topic

- Agenda Item 7 – Revision of ICAO Frequency Spectrum Handbook (Doc 9718)

  - While no papers were received on the ICAO Spectrum Handbook due to other work commitments, the importance of the work was agreed and that would continue as both a CG and also in future meetings.  Therefore, contributions on the handbook updates were strongly encouraged for the FSMP WG/17 meeting, noting there would be capacity to properly consider material.

  - Restructure of Volume I

    - No contributions were received on this topic.

  - Update of Volume II

- 7.3.1	No contributions were received on this topic.Agenda Item 8 – Any other business

    - 8.1	WP07 discussed the compatibility between LDACS and GNSS, analyzing the compliance of LDACS signal, as defined in LDACS Draft SARPs, with GNSS regulatory framework.  With commentary on the interpretation of ITU-R Resolution 417 included in this WP, if LDACS ground station transmitted EIRP as specified in LDACS draft SARPs, the WP explained that specifications are compliant with Resolution 417. The WP is seeking input from the FSMP if any constraints on the airborne LDACS antenna location should be considered in order to protect the GNSS system based on its RFI receiver mask.  During the meeting, several clarifications were received resulting in updates posted to WP07.   Following a discussion several action items were proposed: Action 16-07 was for the ICAO Secretariat to send the paper to the NSP GNSS SARPs WG for any comments.

    - Action 16-08 was for the ICAO Secretariat to send the paper to the CP-DCIWG Project Team – Terrestrial (PT-T) meeting for feedback on the paper, and also clarify what actions had been taken by PT-T in response to the outstanding Resolves 7, in ITU-R Resolution 417 (Rev.WRC-12) with a request to have a response by the next FSMP WG/17 meeting.

  - WP13 presented the lasted ITU regulatory developments concerning the coexistence of the EFVS and the fixed service in the 31.8-33.4 GHz frequency band.  It invited the FSMP Member States to support the use of EFVS with their individual States in 31.8-33.4 GHz. After a short discussion, the meeting agreed with the points raised and encouraged attendees of member States to inform their national regulators of the work.

  - WP21 summarized discussions at the CP-DCIWG/6 meeting on 17-24 Oct on general compatibility issues that have been identified by PT-T, including missing specifications about sensitivity/susceptibility and interference rejection for existing L-band receivers and the very loose requirements on spectral masks and emissions. The CP-DCIWG/6 meeting recommended to bring this issue to the attention of FSMP in order to enable a more spectrum-efficient use of the L-band in the future.  In the discussion on the requests from CP-DCIWG, the meeting stated that the FSMP can support work towards spectrum strategy and high-level policy, but not the apparent requests for specific action plans for other panels on systems standards development.  The meeting recommended that an update be provided to the FSMP and other relevant panels from the CP-DCIWG with its recommendation for the type of actions that should be taken (Action Item 16-09), including what frequency bands and technical requirements should be considered by other panels developing SARPs for L-band CNS systems.

  - IP01 was presentation on the work of RTCA SC-242 and EUROCAE WG-124 on spectrum efficiency, reviewing existing RTCA/EUROCAE standards for spectrum considerations and then recommendations for future standards development and updates.  The meeting noted the presentation and expressed thanks for the provision of the material, noting its correlation with the FSMP work on updating the spectrum handbook.  Updates were requested at future FSMP meetings to ensure the work on the spectrum handbook was coordinated with the RTCA and EUROCAE material.

  - IP02 contained the January 2023 meeting report of the ICAO NSP Spectrum Working Group (SWG). It noted the following topics were discussed at the SWG meeting that may have particular relevance for the FSMP:  SARPS amendments after the moving frequency assignment planning criteria for ILS, VOR, DME and GBAS VDB to ICAO Doc 9718, Vol. II, GBAS VDB coverage volume specifications, and issues to be addressed in the context of analysing the compatibility of LDACS.  The meeting appreciated the update and noted the report, including that the ICAO secretariat are planning to publish a Corrrigendum of Doc 9718 Volume II in ideally 2023.  In addition, it was stated that NSP was in the process of reorganizing where the work of the SWG was conducted.  It was requested the ICAO Secretariat provide an update of the outcome to this decision to the next FSMP meeting in Aug 2023 under Action Item 16-10.

- Date of next meeting and administrative issues

  - FMSP-WG/17 is scheduled for 28 August-8 September, 2023 in hybrid (in-person plus virtual) format at the ICAO Regional Office in Cairo, Egypt. It should be noted for virtual participants that the main meeting working hours are expected to be standard working hours in the local Cairo time zone. That meeting will include a Workshop which will be held the first two days. For planning purposes FSMP-WG/18 is tentatively planned for Q1 2024 again in a hybrid format at ICAO HQ in Montreal Canada.

  - The meeting was reminded that papers for FSMP-WG/17 are due one week prior to the meeting start date. In exceptional circumstances, a framework detailing at minimum the paper summary and introduction will be provided one week prior to the meeting, with the full paper provided no later than one workday before the start of the meeting. Any papers received after this deadline will not be accepted unless agreed by the meeting on the first day. No papers received after the first day of the meeting will be accepted.

APPENDICES

Appendix A – Agenda

Appendix B – List of Working Papers, Information Papers and Flimsies

Appendix C – List of Participants

Appendix D – Action Item List

Appendix E – Final modifications to ICAO position on ITU WRC-23 Agenda Items

Appendix F – CG TORs and initial text for consideration the ITU BR Director’s Report on GNSS RFI

Appendix G – Elements for a proposed reply Liaison to ITU WP 5B

Appendix H – Proposed updates to the note on MSS L-band for continuation by CG

APPENDIX A

INTERNATIONAL CIVIL AVIATION ORGANIZATION

SIXTEENth working Group Meeting of the Frequency Spectrum Management Panel (FSMP-WG/16)

(Bangkok, Thailand, 15 – 24 February 2023)

AGENDA

Agenda Item 1		Opening and Working Arrangements

- Action Item Review

- Status of tasks identified on Job Cards

Agenda Item 2		ICAO WRC-23 Position – FSMP.002.02

- Identified conflicts between administration preliminary proposals and ICAO Position

- Discussion of possible future agenda item proposals from administrations

- Finalize modifications/updates to the ICAO WRC-23 Position

Agenda Item 3		Development of Material for ITU-R Studies – FSMP.003.02

- WRC-23 AI 1.6 Suborbital Vehicles

- WRC-23 AI 1.7 VHF AMS(R)S

- WRC-23 AI 1.8 FSS for UAS

- WRC-23 AI 1.9 Wideband HF

- WRC-23 AI 1.10 Non-safety AMS

- WRC-23 AI 9.2 Radio Regulations clean-up

- Other ITU-R material outside of WRC items

Agenda Item 4		Radio Altimeter issues – FSMP.006.02

- Report from correspondence group on radio altimeters (CG-RA)

- National efforts to implement broadband mobile near 4200-4400 MHz

  - Mitigation Measures

  - Safety Cases/Compatibility Analyses

- Development of SARPS material for Annex 10, Vol. V

Agenda Item 5		5 GHz Band Planning – FSMP.005.03

- AeroMACS

- Global RPAS C2 link channel plan

Agenda Item 6		Interference from Non-Aeronautical Sources – FSMP.004.03

- Mobile service communications adjacent to 1518 MHz satellite communications frequency band (see action item 14-04)

- LED lighting systems

- Other reported interference

Agenda Item 7	ICAO Frequency Spectrum Handbook (Doc 9718) – FSMP.001.02 & FSMP.005.03

- Restructure and update of Volume I

- Update of Volume II

Agenda Item 8		Any Other Business

APPENDIX B

List of Papers

“*” papers available on closed website(s)

List of Working Papers

| Paper # | Source | Topic | Agenda Item |
| --- | --- | --- | --- |
| 1 | A. Roy | LS from WP5B request for WAIC and RA information | 3 |
| 2 | Secretary | Publication on the ICAO Circular Guidance to protect RA | 4 |
| 3 | A. Roy | LS from WP5B Resolution427 (WRC-19) | 3 |
| 4 | EUROCONTROL | GNSS RFI | 2 |
| 5 | C. Pichavant | 5G vs RA scenario_Airbus | 4 |
| 6 | F. Butsch | Proposals to modify ICAO Draft Position on AI 1.8 to WRC-23 (color code) | 2 |
| 7 | G. Novella | LDACS | 8 |
| 8 | C. Fleury | ELEMENTS OF DISCUSSION ON WRC-23 AGENDA ITEM 1.7 France | 2 |
| 9 | M. Tran | AMSRS Coord Threshold | 2 |
| 10 | C. Tourigny and G. Baker | Proposed Updates to ICAO Draft Position on AI 1.7-9.1a-9.2 to WRC-23 | 2 |
| 11 | C. Fleury | French proposal to amend ICAO Draft Position to WRC-23 | 2 |
| 12 | C. Fleury | An update the note on_MSS L-Band | 6 |
| 13 | C. Fleury | Progress of EFVS works in the 32Ghz band | 8 |
| 14* | J. Mettrop | WRC-23 AI 1.7 VHF SATELLITE - Proposed Amendments to the Draft ICAO Position | 2 |
| 15* | J. Mettrop | WRC-23 AI 1.8 SATELLITE SPECTRUM FOR UAS CNPC- Review of Current Proposals Against the Preliminary ICAO Position | 2 |
| 16 | K. Masrani | IMT-MSS note | 6 |
| 17 | K. Masrani | Proposed Updates to ICAO Draft Position to WRC-23 (Inmarsat) | 2 |
| 18 | M. Garcia | Space Based VHF datalink concept | 2 |
| 19 | M. Garcia | Regulatory Frequency planning exercises AI 1.7 update | 2 |
| 20 | M. Garcia | Contribution to the ICAO position on the agenda item 1.7 | 2 |
| 21 | CP-DCIWG | Required System Specification Updates for the Legacy L-band Systems | 8 |
| 22 | J. Cramer | Proposal for ICAO Position for WRC-23 Agenda Item 2.9 | 2 |
| 23 | N. Shrout and K. Hutchison | Updates to ICAO Position - 1.7 | 2 |
| 24 | A. Roy | Elements for the Coordination and Planning of VDL SATCOM | 2 |
| 25 | A. Roy | Proposed Response to WP5B Liaison on WAIC | 3 |
| 26 | A. Roy | Planning for Radar Altimeter SARPs | 4 |

List of Information Papers, Presentations and Flimsies

| Paper # | Source | Topic | Agenda Item |
| --- | --- | --- | --- |
| 1 | RTCA/EUROCAE | New RTCA EUROCAE Joint Committee on Spectrum Efficiency | 8 |
| 2 | F. Butsch | ICAO NSP Spectrum Working Group meeting report_ | 8 |
| 3 | C. Fleury | Terrestrial stations registered in the MIFR in the band 117.975-136.8 MHz France | 2 |
| 4 | B. Clark and M. Bellman | Future Radio Altimeter Standards Status | 4 |
| 5 | C. Benson | Space-Based Air Traffic Management - | 2 |
| 6 | C. Fleury | CG-RA Report Feb 2023 V1.01 | 4 |
| 7 | ICCAIA | Status Update of RTCA SC239-EUROCAE WG119 | 4 |

| PRESENTATIONS |  |  |  |
| --- | --- | --- | --- |
|  | RTCA/EUROCAE | For IP01 – SC-242 WG-124 Spectrum Efficiency | 8 |
|  | C. Benson | For IP05 – Skykraft ATM | 2 |

| FLIMSIES |  |  |  |
| --- | --- | --- | --- |
| 1 |  | Final modifications to ICAO position on ITU WRC-23 Agenda Items | 2 |
| 2 |  | WRC-23 AI 1.8 Position Text Drafts | 2 |
| 3 |  | WRC-23 AI 10 Proposal Draft | 2 |
| 4 |  | WRC-23 AI 1.6 Position Text Drafts | 2 |
| 5 |  | Elements for a proposed reply Liaison to ITU WP 5B | 3 |
| 6 |  | CG TORs and initial text for consideration the ITU BR Director’s Report on GNSS RFI | 2 |
| 7 |  | Proposed updates to the note on MSS L-band for continuation by CG | 6 |

APPENDIX C

The Sixteenth Working Group Meeting of the Frequency Spectrum Management Panel (FSMP-WG/16)

(Bangkok, Thailand, 15-24 February 2023)

LIST OF PARTICIPANTS

|  |  | STATE/NAME | TITLE/ORGANIZATION | E-MAIL | MODE OF ATTENDANCE (V – Virtual I – In-person) |
| --- | --- | --- | --- | --- | --- |
|  |  | AUSTRALIA (2) |  |  |  |
|  |  | Mr. Matthew Kelly | Senior Asset Engineer Airservices Australia | matthew.kelly@airservicesaustralia.com; | I |
|  |  | Mr. Craig Benson | Director and Chief Innovation Officer Skykraft Australia | craig.benson@skykraft.com.au; | I |
|  |  | BANGLADESH (2) |  |  |  |
|  |  | Engr Md Anwar Hossain | Assistant Director (CNS-ANS Inspector) & Asst. NCMC Civil Aviation Authority of Bangladesh | ahossaincaab@gmail.com; | V |
|  |  | Mr. Abul Kalam Md Saiduzzaman | Deputy Director (ATM) Civil Aviation Authority of Bangladesh | akmsaiduz@gmail.com; | V |
|  |  | BELGIUM (1) |  |  |  |
|  |  | Ms. Sonja Boedt | CNS Expert Belgian Civil Aviation Authority | sonja.boedt@mobilit.fgov.be; | V |
|  |  | BRAZIL (1) |  |  |  |
|  |  | Mr. Vahe Antoine Yaghdjian | CNS Advisor Department of Airspace Control | vahe.antoine@gmail.com; | V |
|  |  | CANADA (4) |  |  |  |
|  |  | Mr. John Taylor | Aeronautical Spectrum & Regulations Specialist Transport Canada (UAS Division) | helma589@gmail.com; | I |
|  |  | Mr. Seyed Rastaghi | Manager, Spectrum Management NAV CANADA | Seyed.Rastaghi@navcanada.ca; | I |
|  |  | Mr. Ricardo Hisano | Senior Engineer Transport Canada | ricardo.hisano@tc.gc.ca; | V |
|  |  | Mr. Abed Ferr | Engineering Systems Specialist/ Spectrum Management NAV CANADA | abed.ferr@navcanada.ca; | V |
|  |  | CHINA (8) |  |  |  |
|  |  | Mr. Cui Kaitao | ENGINEER The Second Research Institute of CAAC | cuikaitao@caacsri.com; | V |
|  |  | Mr. Ye Jiaquan | Engineer The Second Research Institute of CAAC | yejq@caacsri.com; | V |
|  |  | Mr. Zhang Jia | Engineer Civil Aviation Administration of China | zhangjia135@139.com; | V |
|  |  | Ms. Liu Rui | Engineer ADCC OF CAAC | liurui@adcc.com.cn; | V |
|  |  | Mr. Liang Zhao | Engineer ADCC of CAAC | zhaoliang@adcc.com.cn; | V |
|  |  | Mr. Yinghan Xia | Engineer CETC | yinghan94max@hotmail.com; | V |
|  |  | Mr. Zhang ZhenNan | Embedded Engineer HaiGeWings | 643989461@qq.com; | V |
|  |  | Mr. Pengyu Wang | Engineer, CNS Department ATMB, CAAC | wangpengyu@atmb.net.cn; | V |
|  |  | HONG KONG CHINA (1) |  |  |  |
|  |  | Mr. Joseph Ho | Senior Electronics Engineer Civil Aviation Department of Hong Kong, China | jcyho@cad.gov.hk; | I |
|  |  | FRANCE (7) |  |  |  |
|  |  | Mr. Guillaume Novella | Spectrum Engineering DSNA | guillaume.novella@aviation-civile.gouv.fr; | I |
|  |  | Mr. Jerome Andre | Spectrum Engineer ANFR | JEROME.ANDRE@ANFR.FR ; | I |
|  |  | Mr. Christian Fleury | FSMP Panel Member and Vice Chair DGAC-DSNA | christian.fleury@aviation-civile.gouv.fr; | I |
|  |  | Mr. Louvet Benoit | Deputy Head Regulatory Affairs Thales Alenia Space | benoit.louvet@thalesaleniaspace.com; | V |
|  |  | Mrs. Christine Mengelle | Regulatory Affairs Thales Alenia Space | christine.mengelle@thalesaleniaspace.com; | V |
|  |  | Mr. Josep Gine | Radio Functions Engineer ATR | josep.gine-i-cortiella@atr-aircraft.com; | V |
|  |  | Mr. Martin Ivan | Aeronautical & Maritime Frequency Spectrum Regulatory Manager THALES | ivan.martin@thalesgroup.com; | V |
|  |  | GERMANY (5) |  |  |  |
|  |  | Dr. Felix Butsch | Head of Frequency Management DFS Deutsche Flugsicherung GmbH, (German Air Navigation Services) | Felix.Butsch@DFS.de; | V |
|  |  | Mr. René Woerfel | Advisor AUDENS TC | rene.woerfel@audens.de; | I |
|  |  | Mr. Martin Weber | Spectrum Expert Federal Network Agency | Martin.Weber@BNetzA.de; | I |
|  |  | Mr. Christian Beitz | Head of Unit Frequency Management and Protection of ANS-Facilities Federal Supervisory Authority for Air Navigation Services Germany | christian.beitz@baf.bund.de; | V |
|  |  | Mr. Sergio Bovelli | Head of Spectrum Management Aero-Connectivity Airbus | sergio.bovelli@airbus.com; | I |
|  |  | INDIA (2) |  |  |  |
|  |  | Mr. Ajay Kumar Kapur | General Manager (CNS) Airports Authority of India | akkapur@aai.aero; | I |
|  |  | Mr. Arvind Singh Yadav | Joint General Manager (CNS) Airports Authority of India | asyadav@aai.aero; | I |
|  |  | INDONESIA (2) |  |  |  |
|  |  | Mr. Bimantoro | Inspector of Air Navigation DGCA Indonesia | bimzink@gmail.com; | I |
|  |  | Mr. Abdul Aziz | Inspector of Air Navigation DGCA Indonesia | azizsabdul@gmail.com; | I |
|  |  | JAPAN (14) |  |  |  |
|  |  | Mr. Kuji, Satoshi | Special Assistant to the Director of the Airworthiness Division Civil Aviation Bureau of Japan | kuji-s46kd@mlit.go.jp; | V |
|  |  | Mr. Ken Kitamura | Special Assistant to the Director, Aircraft Engineering & Certification Center Airworthiness Division Japan Civil Aviation Bureau | kitamura-k97mv@mlit.go.jp; | V |
|  |  | Mr. Kenji Miyabe | Senior Aeronautical Engineer / Aircraft Engineering & Certification Center Airworthiness Division Japan Civil Aviation Bureau | miyabe-k973z@mlit.go.jp; | V |
|  |  | Mr. Katsuyuki Arakawa | Special Assistant to the Director JAPAN Civil Aviation Bureau (JCAB) | arakawa-k24fe@mlit.go.jp; | I |
|  |  | Mr. Manabu Kawamoto | Chief JAPAN Civil Aviation Bureau (JCAB) | kawamoto-m46vv@mlit.go.jp; | V |
|  |  | Mr. Yoshiaki Saeki | Chief Ministry of Internal Affairs and Communications | y.saeki@soumu.go.jp; | I |
|  |  | Mr. Takayuki Wada | Deputy Director Ministry of Internal Affairs and Communications | t5-wada@soumu.go.jp; | V |
|  |  | Mr. Naruto Yonemoto | Principal Researcher Electronic Navigation Research Institute, National Institute of Maritime, Port, and Aviation Technology (ENRI/MPAT） | yonemoto@mpat.go.jp; | V |
|  |  | Mr. Shunichi Futatsumori | Chief Researcher Electronic Navigation Research Institute, National Institute of Maritime, Port, and Aviation Technology (ENRI/MPAT） | futatsumori@mpat.go.jp; | V |
|  |  | Mr. Toshio Nomi | Senior Engineer Japan Radio Air Navigation Systems Association (JRANSA) | nomi.toshio@jransa.or.jp; | I |
|  |  | Mr. Fumio Tagawa | Senior Engineer Japan Radio Air Navigation Systems Association (JRANSA) | tagawa-f213@jransa.or.jp; | I |
|  |  | Mr. Yasuyoshi Nakatani | Chief Engineering Analysist Japan Radio Air Navigation Systems Association (JRANSA) | nakatani-y214@jransa.or.jp; | V |
|  |  | Mr. Hiroyuki Tsuji | Director National Institute of Information and Communications Technology (NICT) | tsuji@nict.go.jp; | V |
|  |  | Mr. Furen Tang | Researcher Mitsubishi Research Institute | furen_tang@mri.co.jp; | I |
|  |  | MALAYSIA (2) |  |  |  |
|  |  | Mr. Shairyzal Bin Mohamad @ Azizan | Assistant Director Civil Aviation Authority of Malaysia | shairyzal.azizan@caam.gov.my; | V |
|  |  | Mr. Syahroni Bin Chek Rus | Assistant Director Civil Aviation Authority of Malaysia | syahroni@caam.gov.my; | V |
|  |  | PAKISTAN (5) |  |  |  |
|  |  | Mr. Shamsuddin Hakro | A/Additional Director Com.OPS Civil Aviation Authority of Pakistan | Shamsuddin.hakro@caapakistan.com.pk; | V |
|  |  | Mr. Shahid Hussain | Sr.Joint Director Com.OPS Civil Aviation Authority of Pakistan | Shahid.hussain@caapakistan.com.pk; | V |
|  |  | Mr. M.Usman Rasheed | Deputy Director Com.OPS Civil Aviation Authority of Pakistan | usman.rasheed@caapakistan.com.pk; | V |
|  |  | Mr. Tanweer Akhtar | Deputy Director Com.OPS Civil Aviation Authority of Pakistan | Mt.akhtar@caapakistan.com.pk; | V |
|  |  | Mr. Nauman Zahid | Assistant Director Com.OPS Civil Aviation Authority of Pakistan | Nauman.zahid@caapakistan.com.pk; | V |
|  |  | PHILIPPINES (2) |  |  |  |
|  |  | Mr. Gilo, Charlemagne P. | Division Chief III Civil Aviation Authority of the Philippines | ats_atd@caap.gov.ph; | I |
|  |  | Mr. Norrick T.Baes | Communications Navigation Surveillance Systems Officer V Civil Aviation Authority of the Philippines | norrick_baes@caap.gov.ph; | V |
|  |  | ROMANIA (1) |  |  |  |
|  |  | Mr. Aurelian Sorinel Calinciuc | Spectrum Manager ANCOM | aurelian.calinciuc@ancom.ro; | V |
|  |  | SAUDI ARABIA (1) |  |  |  |
|  |  | Mr. Faris Abdulwahab Alzahrani | Air Navigation Safety Inspector General Authority of Civil Aviation | Faalzahrani@gaca.gov.sa; | V |
|  |  | SINGAPORE (3) |  |  |  |
|  |  | Mr. Henry Foo | Deputy Director (AT&D) Civil Aviation Authority of Singapore | Henry_FOO@caas.gov.sg; | I |
|  |  | Mr. Chong Ziyu John | DD(Masterplan and Research Innovation CAA Singapore | jczy0999@gmail.com; | I |
|  |  | Mr. Yeo Cheng Nam | Consultant (Aeronautical Telecommunications & Engineering) Civil Aviation Authority of Singapore | Yeo_cheng_nam@caas.gov.sg; | I |
|  |  | SOUTH AFRICA (2) |  |  |  |
|  |  | Ms. Lisa Cokisa Tele | Director PPMO ATNS | lisat@atns.co.za; | I |
|  |  | Mr. Thomas Matabane | Frequency Spectrum CNS Civil Aviation Authority of South African | matabanet@caa.co.za; | V |
|  |  | SPAIN (3) |  |  |  |
|  |  | Mr. José Luis Chinchilla García | ATM - Marketing and Innovation - Director Indra Sistemas | jlchinchilla@indra.es; | I |
|  |  | Mr. Julian Sesena | Advisor Holistic Innovation SLU | jsesena@hi2.es; | I |
|  |  | Mr. Manuel Garcia | Communications Division Head ENAIRE | mangarcia@enaire.es; | I |
|  |  | THAILAND (7) |  |  |  |
|  |  | Mr. Chavalit Ithiapa | Senior Navigation Aids Standards Division Officer Civil Aviation Authority of Thailand | chavalit.i@caat.or.th; | V |
|  |  | Mr. Nattaporn Pornsawat | Navigation Aids Standards Division Officer Civil Aviation Authority of Thailand | nattaporn.p@caat.or.th; | I |
|  |  | Mr. Phichpawis Plengsiriwat | Air Navigation Operations Planning Division Officer Civil Aviation Authority of Thailand | phichpawis.p@caat.or.th; | I |
|  |  | Mr. Chainan Chaisompong | Air Traffic Engineering Manager Aeronautical Radio of Thailand Ltd. | chainan.ch@aerothai.co.th; | I |
|  |  | Mr. Adul Wisetbuppha | Division Director Office of the National Broadcasting and Telecommunications Commission | Adul.w@nbtc.go.th; | V |
|  |  | Mr. Nattawut Wanich | Senior Engineer Officer Office of the National Broadcasting and Telecommunications Commission | Nattawut.w@nbtc.go.th; | V |
|  |  | Mr. Nathat Sangsuwan | Mis-level Inspection and Operation Officer Office of the National Broadcasting and Telecommunications Commission | Nathat.s@nbtc.go.th; | V |
|  |  | UNITED KINGDOM (3) |  |  |  |
|  |  | Mr. John Ernest Mettrop | Spectrum Policy Specialist UK Civil Aviation Authority | john.mettrop@caa.co.uk; | I |
|  |  | Mr. Kamlesh Masrani | Director, Spectrum Inmarsat Global Limited | kamlesh.masrani@inmarsat.com; | V |
|  |  | Mr. Stephen Parry | Spectrum Manager, Spectrum System Design Authority NATS | stephen.parry@nats.co.uk; | V |
|  |  | UNITED STATES (12) |  |  |  |
|  |  | Mr. Christopher Tourigny | US Panel Member, FSMP US Federal Aviation Administration | chris.tourigny@faa.gov; | I |
|  |  | Ms. Kathryn Bernazzani | General Engineer U.S. Department of Transportation/Volpe Center | kathryn.bernazzani@dot.gov; | V |
|  |  | Mr. Miles Bellman | General Engineer Federal Aviation Administration | miles.e.bellman@faa.gov; | V |
|  |  | Mr. Andrew Pegues | Foreign Affairs Officer U.S. Department of State | PeguesAH@state.gov; | V |
|  |  | Mr. Shayne A Campbell | Senior Air Traffic Representative Asia Pacific Federal Aviation Administration | shayne.a.campbell@faa.gov; | I |
|  |  | Mr. Gregory Baker | Electronics Engineer Federal Communications Commission | Gregory.Baker@fcc.gov; | I |
|  |  | Mr. Michael Neale | Spectrum Engineer Federal Aviation Administration | michael.neale@aces-inc.com; | V |
|  |  | Mr. Damon C. Ladson | Technical Policy Advisor HWG, LLP | dladson@hwglaw.com; | I |
|  |  | Mr. Bruce Lamb | Telecom Specialist NTIA | blamb@ntia.gov; | I |
|  |  | Mrs. Barbara Clark | Engineer Federal Aviation Administration | Barbara.Clark@faa.gov; | V |
|  |  | Mr. Donald O. Nellis Jr. | Electronics Engineer US Federal Aviation Administration | Donald.Nellis@faa.gov; | V |
|  |  | Mr. Michael Tran | Spectrum and aviation Engineer FAA (Mitre contractor for FAA) | mtran@mitre.org; | V |
|  |  | EUROCONTROL (2) |  |  |  |
|  |  | Mr. John Micallef | Administrator / Spectrum Expert EUROCONTROL | john.micallef@eurocontrol.int; | I |
|  |  | Mr. Raffi Khatcherian | Head of International Spectrum Management EUROCONTROL | raffi.khatcherian@eurocontrol.int; | I |
|  |  | IATA (5) |  |  |  |
|  |  | Mr. Blair Cowles | Regional Director - Operations, Safety & Security (Asia Pacific) International Air Transport Association | cowlesb@iata.org; | I |
|  |  | Mr. Diego Albert | Assistant Director Safety and Flight Operations International Air Transport Association | albertd@iata.org; | I |
|  |  | Mr. Kieran O'Carroll | Assistant Director Americas Region Flight Operations International Air Transport Association | ocarrollk@iata.org; | V |
|  |  | Mr. Prashant Sanglikar | Assistant Director Safety and Flight Operations IATA | sanglikarp@iata.org; | V |
|  |  | Ms. Ruby Sayyed | Head ATM Strategy International Air Transport Association | sayyedr@iata.org; | V |
|  |  | ICCAIA (5) |  |  |  |
|  |  | Mr. Claude Pichavant | Executive Expert CNS (Airbus) | claude.pichavant@airbus.com; | I |
|  |  | Mr. Joseph Cramer | Director The Boeing Company | Joseph.Cramer@Boeing.com; | I |
|  |  | Mr. Olivier Pellay | Spectrum Manager Airbus | olivier.pellay@airbus.com; | V |
|  |  | Dr. Sai Kalyanaraman | Sr. Technical Fellow Collins Aerospace | Sai.Kalyanaraman@collins.com; | V |
|  |  | Mr. Scott Kotler | Director, Regulatory Engineering Lockheed Martin Corporation | scott.kotler@LMCO.com; | V |
|  |  | ITU (2) |  |  |  |
|  |  | Mr. Nikolai Vassiliev | Chief, Terrestrial Services Department International Telecommunication Union | nikolai.vassiliev@itu.int; | I |
|  |  | Mr. Nelson Malaguti | Counsellor International Telecommunication Union | nelson.malaguti@itu.int; | V |
|  |  | ASRI (3) |  |  |  |
|  |  | Mr. Kris Hutchison | President Aviation Spectrum Resources, Inc. | keh@asri.aero; | I |
|  |  | Mr. Nicholas Shrout | RF Engineer Aviation Spectrum Resources, Inc. | njs@asri.aero; | I |
|  |  | Mr. Andrew Roy | Director of Engineering Aviation Spectrum Resources, Inc. | acr@asri.aero; | I |
|  |  | SITA (2) |  |  |  |
|  |  | Mr. Eric Wawrzynkowski | Spectrum Manager SITA | eric.wawrzynkowski@sita.aero; | V |
|  |  | Mr. Pablo De La Viuda | Senior Product Manager SITA For Aircraft | pablo.de_la_viuda@sita.aero; | V |
|  |  | ICAO (6) |  |  |  |
|  |  | Mr. Loftur Jonasson | Chief CNSS (FSMP Secretary) International Civil Aviation Organization | ljonasson@icao.int; | I |
|  |  | Mr. Luo Yi | Regional Officer CNS Asia and Pacific Office International Civil Aviation Organization | yluo@icao.int; | I |
|  |  | Ms. Soniya Nibhani | Regional Officer ANS Implementation (CNS) Asia and Pacific Office International Civil Aviation Organization | snibhani@icao.int; | I |
|  |  | Mr. How Sze Lung, Derek | Regional Officer CNS Asia and Pacific Office International Civil Aviation Organization | show@icao.int; | I |
|  |  | Ms. Zhong Wenhan, Nancy | Regional Officer CNS Asia and Pacific Office International Civil Aviation Organization | wzhong@icao.int; | I |
|  |  | Ms. Varapan Meefuengsart | Programme Assistant, CNS/MET Asia and Pacific Office International Civil Aviation Organization | vmeefuengsart@icao.int; | I |

-END-

APPENDIX D

ACTION ITEM LIST

| Number | Description | Actionee | Due Date | Status |
| --- | --- | --- | --- | --- |
| 10-03 | Report on current use in States of the 1300-1350 MHz frequency band for radars and longer term plans. | All | FSMP-WG/17 | Strongly encouraged given possible future WRC-27 agenda item. Relevant papers: WG/16 – WP22 |
| 10-08 | Develop contributions on how to ensure protection of aeronautical systems that are not included in the ITU MIFR. | All | FSMP-WG/17 | Possible options need to be investigated on generic MIFR registrations. Has potential relevance to WRC-23 AI 1.7. |
| 11-04 | Participate in the radio altimeter correspondence group (CG-RA) | All | Ongoing | Send Email to christian.fleury@aviation-civile.gouv.fr |
| 12-10 | Determine if generic guidance can be developed to help aviation authorities analyse potential 5G impacts on radio altimeters given their specific State implementation |  | FSMP-WG/17 | Addressed in WG/16 WP05 and 26, establishing a CG to coordinate possible material. |
| 13-01 | Provide examples of national specifications or regulations on adjacent-band signal rejection for receivers. | FSMP participants who are also spectrum regulators in their country | FSMP-WG/17 |  |
| 13-03 | Develop educational material for States explaining the 5G/RA issue. | CG-RA | CLOSED | Completed with contribution to ICAO Circular Letter on the topic |
| 13-04 | Develop a set of frequently asked questions (FAQs) regarding satellite-VHF. | CG-SV | CLOSED | No additional questions raised. |
| 14-03 | Liaise FSMP-WG/14 WP06 to RPASP WG3 to solicit input to the under-development ITU-R Handbook on detect and avoid systems | Secretary | ASAP | FSMP Liaison was sent, waiting for finalization of DAA Manual. Secretary to confirm current status and liaise to WP5B if finalized |
| 14-04 | Participate in CG-MSS to complete the Note providing information on ensuring compatibility between IMT operated below 1518 MHz and aeronautical MSS services above 1525 MHz. | All | FSMP-WG/18 | To participate send an email to Kamlesh Masrani Kamlesh.Masrani@inmarsat.com Relevant papers: WG/16 Flimsy07 |
| 15-01 | Review the material in IP09 and provide any necessary updates. | All | CLOSED | Addressed through input on WG/16 WP02 |
| 15-02 | Bring specific proposals for the ICAO Position on WRC-23 AI9.2 to FSMP-WG/16 on clean-up of aeronautical provisions of the RR. Seeking inputs on the what information may need to be considered and how it should be actioned noting other aviation topics at WRC-23. | All | FSMP-WG/17 |  |
| 16-01 | Encourage inputs to the new CG on GNSS RFI to help develop the text for a possible ITU-R Resolution at WRC-23. See paper WG/16 WP04 | All | Before FSMP WG/17 | CG Chair: Guillaume Novella |
| 16-02 | Bring specific data on aircraft oceanic density statistics to FSMP WG/17 to support a follow up LS to 5B on the data for EESS in 4.2-4.4 GHz. See paper WG/16 WP25 | All (Eurocontrol) | FSMP WG/17 |  |
| 16-03 | To provide comments to the ICAO Secretariat on the proposed ICAO Circular on radar altimeter interference. See paper WG/16 WP02 | All | Within 2 weeks after FSMP WG/16 |  |
| 16-04 | Contributions be sought on appropriate material for radar altimeter SARPs. See paper WG/16 WP26 | All | FSMP WG/17 |  |
| 16-05 | Send documents WG/16 WP05 and WP26 on possible new radar altimeter SARPS to RTCA/EUROCAE for their consideration and input | ICAO Secretariat | Before FSMP WG/17 |  |
| 16-06 | Send WG/16 WP05 and WP26 to the Flt Ops Panel for comments and any interim guidance it could provide on the radar altimeter operational topics raised by the next FSMP | ICAO Secretariat | Before FSMP WG/17 |  |
| 16-07 | Send the paper WG/16 WP07 to the NSP GNSS SARPs WG for any comments on interference assessments made with potential new LDACS | ICAO Secretariat | Before FSMP WG/17 |  |
| 16-08 | Send the paper WG/16 WP07 to the CP-DCIWG PT-T for any comments on interference assessments made with potential new LDACS, and also clarify what actions had been taken by PT-T in response to the outstanding Resolves 7, in ITU-R Resolution 417 (Rev.WRC-12). | ICAO Secretariat | Before FSMP WG/17 |  |
| 16-09 | From WG/16 WP21, request an update be provided to the FSMP and other relevant panels from the CP-DCIWG PT-T with its recommendation actions to be taken including what frequency bands and technical requirements should be considered by other panels developing SARPs for L-band CNS systems | ICAO Secretariat | FSMP WG/17 |  |
| 16-10 | Provide an update of the outcome from the NSP reorganizing the work of the Spectrum SWG. See paper WG/16 IP02 | ICAO Secretariat | FSMP WG/17 |  |

APPENDIX E

Final modifications to ICAO position on ITU WRC-23 Agenda Items

APPENDIX F

CG TORs and initial text for consideration the ITU BR Director’s Report on GNSS RFI

APPENDIX G

Elements for a proposed reply Liaison to ITU WP 5B

APPENDIX H

Proposed updates to the note on MSS L-band for continuation by CG