FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

Sixteenth Working Group meeting

Bangkok, Thailand, 15 – 24 February 2023

Agenda Item ..:	ICAO Position for WRC-23

Proposed Updates to ICAO Position – WRC-23 Agenda Item 1.7

(Presented by Nic Shrout and Kris Hutchison, ASRI)

SUMMARY

Proposes modifications to the ICAO Position for WRC-23 - Agenda Item 1.7

INTRODUCTION

The embedded document contains proposed edits to the ICAO WRC-23 Position on Agenda Item 1.7.  The proposal is intended to address the points discussed in section 2.

DISCUSSION

Frequency range of AMS(R)S operation:

Given the uncertainties regarding the possible frequency range of any AMS(R)S allocation, it is recommended to retain the ‘in all or part of’ language in the ICAO position. It is clear there are still multiple views on this topic, and ICAO should keep its options open leading up to and during WRC-23. It would be difficult for ICAO to support a specific frequency range now and then alter that position at the WRC.

Protection of existing AM(R)S assignments under Article 9

As noted, in the ITU-R BR Document 5B/726, if using measures under Article 9.11A, only stations registered in the ITU-R MIFR would be able to claim protection if they experienced interference.  However, use of the ITU-R MIFR for aeronautical assignments varies widely depending on the administration, and any attempts to require states to register all ATS and AOC frequencies would likely be unsuccessful.  More fundamentally, if registration in the MIFR is a control measure, then that should be implemented before ICAO makes any recommendation/endorsement of a method at the ITU-R that relies on it.  Therefore, it must be assumed that comprehensive use of the ITU-R MIFR is unfeasible for WRC-23 and other measures should be taken to ensure protection of terrestrial services.  Instead, suitable language should be included in the ITU-R Article 5 to ensure terrestrial services have suitable protection from satellite VHF in addition to any Article 9 requirements, ensuring AM(R)S services are declared primary to AMS(R)S services.

ACTION BY THE MEETING

The meeting is invited to:

note and review the contents of this working paper; and

endorse the proposed changes to the ICAO position for WRC-23 Agenda Item 1.7.

— END —

Asking for significant changes to existing national and regional VHF coordination processes and procedures to accommodate a new VHF SATCOM system is in contrast with the original ICAO proposal presented in 2019.