FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

Twentieth Working Group meeting

Bangkok, Thailand, 24 February to 7 March 2025

Agenda Item 4:	Radio Altimeters

Minimum Distance between IMT Base Stations and Radio Altimeters

(Presented by Damian Lopez & Abed Ferr from NAV CANADA)

SUMMARY

- The threat posed by WRC-27 Agenda Item 1.7 to Radio Altimeters is well understood by ICAO and has been the focus of multiple discussions. - This paper includes a contribution to ITU-R Working Party 5D providing a detailed numerical analysis that estimates the minimum distance required between IMT base stations operating in the 4 400-4 800 MHz band and Radio Altimeters.

INTRODUCTION

  - While there is factual evidence that Radio Altimeters can have their performance affected by nearby IMT base stations operating in the 4 400-4 800 MHz, there is little information available providing a rigorous analysis of interference and minimum allowed distance.

DISCUSSION

  - China’s recent contribution #472 (23 January 2025) to ITU-R Working Party 5D provides a detailed numerical analysis estimating the minimum distance required between IMT base stations operating in the 4 400-4 800 MHz band and Radio Altimeters operating in the adjacent 4 200-4 400 MHz band.

The contribution establishes an evaluation methodology of interference, determines the worst-case scenario, and selects all necessary technical parameters. The choice of Recommendation ITU-R P.528 as propagation model determines the type of analysis, which must be statistical (Monte Carlo) as opposed to purely mathematical. At first glance all assumptions appear to be reasonable. Notably, ICAO’s recommended 6 dB safety margin for aviation is included.

  - The analysis concludes by providing minimum distances required to prevent three different types of performance degradation to radio altimeters: 2.2 Km for front-end overload, 65 Km for false altitudes, and 74 Km for desensitization.

ACTION BY THE MEETING

  - The meeting is invited to:

note and review the contents of the attached document (contribution #472 to WP 5D);

consider replicating the analysis in the attached document, for verification; and

consider issuing a Liaison Statement to WP 5D in support of the attached document and its conclusions.

— END —