
[State CAA]

MANUAL OF VALIDATION AND SURVEILLANCE

of
FOREIGN Air OPERATORS
Intentionally Blank

[State CAA]

	


Intentionally Blank
Record of Amendments

	Amendment

Number
	Date of Amendment
	Pages Affected
	Date Entered
	Initials

	1
	
	
	
	

	2
	
	
	
	

	3
	
	
	
	

	4
	
	
	
	

	5
	
	
	
	

	6
	
	
	
	

	7
	
	
	
	

	8
	
	
	
	

	9
	
	
	
	

	10
	
	
	
	

	11
	
	
	
	

	12
	
	
	
	

	13
	
	
	
	

	14
	
	
	
	

	15
	
	
	
	

	16
	
	
	
	

	17
	
	
	
	

	18
	
	
	
	

	19
	
	
	
	

	20
	
	
	
	

	21
	
	
	
	

	22
	
	
	
	

	23
	
	
	
	

	24
	
	
	
	

	25
	
	
	
	


Intentionally Blank
TABLE OF CONTENTS

2Foreword


4Record of Amendments


6TABLE OF CONTENTS


8Chapter 1


81
General


81.1
[Flight Standards Directorate or as applicable]


81.1.1
General


81.1.2
Responsibilities


81.2
Application of the Manual


81.2.1
Definitions


91.2.2
Applicability


91.2.3
Policies and Procedures


91.3
Preparation and Distribution of the Manual


91.4
Manual Revisions


101.5
General Considerations


101.5.1
[Civil Aviation Regulations (CARs) or as appropriate]


101.5.2
Requirements


101.5.3
[State] Air Services Agreement Requirements


12Chapter 2


122
Validation of Air Operator Certificate


122.1
General


122.2
Validation Process


122.2.1
Documentation Required


132.2.2
Initial Application


132.2.3
Amendment to an Existing Validation


132.2.4
Review of Documentation


132.2.5
Evaluation of an Application by a Foreign Air Operator


142.2.6
Additional Measures


152.3
Reserved


152.4
Issuance or Amendment to a validation


152.5
Records Management


17Chapter 3


173
Surveillance of Foreign Air Operators


173.1
Introduction


173.2
Inspectors


183.3
Sharing of safety information (upon request)


183.4
Pre-inspection planning


193.5
Inspections


203.6
Action on Findings – Resolution of Safety Issues


22Appendix A - FOREIGN AIR OPERATOR APPLICATION


24Appendix B - Sample Validation of Air Operator Certificate


26Appendix C - Validation Process


28Appendix D - Guidance For Ramp Inspection


40Appendix E - Levels of seriousness and related actions


51AIRWORTHINESS WORKSHEET


53FLIGHT OPERATIONS WORKSHEET


54CABIN SAFETY WORKSHEET


56SAMPLE CORRESPONDENCE





Chapter 1
1 General
1.1 [Flight Standards Directorate or as applicable]

1.1.1 General

Pursuant to Article [number] of the Civil Aviation Law, the Minister is responsible for the regulation and supervision of all aeronautical matters within [State].  Pursuant to Part [XXX] of the [State] Civil Aviation Regulations (CARs), the [State CAA] has determined that all foreign aircraft operated in commercial air transport operations within [State] shall be operated and maintained in accordance with ICAO Standards, as well as the terms of an Air Operator Certificate (AOC) and associated Operations Specifications issued by the State of the Operator.  The Director General shall issue a formal validation of the Air Operator Certificate, which may include additional conditions and/or limitations which are intended to address unique and special requirements associated with the airspace of  [State]. Such conditions and/or limitations should not conflict with the AOC and the associated Operations Specifications issued by the State of the Operator.  The discharge of this responsibility has been delegated to the [Director, Chief as appropriate].
1.1.2 Responsibilities

The [Flight Standards Directorate or as applicable] is responsible for:
(1)
Promulgating functional direction for the regulation of foreign air operators operating in and out of [State];
(2)
Administration of the process related to issuance of the validation of air operator certificate, establishment of the foreign air operator surveillance programme and subsequently, implementation of the programme;

(3)
Communicating with foreign civil aviation authorities, and related coordinating and liaising with appropriate [State] government departments and agencies;
(4)
Participating in working groups, seminars, conferences, on matters relating to international air services and the regulation thereof; and
(5)
Providing assistance, when requested, to foreign civil aviation authorities (CAAs).

1.2 Application of the Manual

1.2.1 Definitions

For the purposes of this manual:
“Air operator certificate (AOC)” a certificate authorizing an operator to carry out specified commercial air transport operations. Note:  Issued by the State of the Operator;
“CAA” identifies Civil Aviation Authority of [State];
“CAA inspector” identifies, as applicable, an operations inspector, a cabin safety inspector, an airworthiness inspector or any other inspector who is duly authorized by the CAA for  the purposes of  conducting safety oversight of foreign air operations in [State];
“Charter” is a non-scheduled operation using a chartered aircraft. In a charter of an aircraft, a portion of or the entire capacity of the aircraft is hired or purchased privately by one or
more entities, which may re-sell it to the public (this occurs most frequently in non-scheduled passenger air operations which is why they are popularly known as “charter flights”). The situation in which the charterer is another air operator which has its own operating authority and charters the entire capacity of the aircraft, usually on short notice, is termed a sub‑charter.

“Lease” 
“ Damp lease” is generally understood to be a wet lease of an aircraft where the aircraft is operated under the AOC of the lessor, with the flight crew and possibly part of the cabin crew being provided by the lessor. Part or all of the cabin crew is provided by the lessee.

“ Dry lease” is understood to be the lease of an aircraft where the aircraft is operated under the AOC of the lessee. It is normally a lease of an aircraft without crew, operated under the custody and the operational and commercial control of the lessee, and using the lessee’s airline designator code and traffic rights.
“Wet lease” is generally understood to be a lease of an aircraft where the aircraft is operated under the AOC of the lessor. It is normally a lease of an aircraft with crew, operated under the commercial control of the lessee and using the lessee’s airline designator code and traffic rights. 

“Operations specifications”  -  The authorizations, conditions and limitations associated with the air operator certificate and subject to the conditions in the operations manual.

1.2.2 Applicability
The procedures and policy herein shall apply to the operation of any civil aircraft for the purpose of commercial air transport operations by any air operator whose Air Operator Certificate is issued and controlled by a civil aviation authority other than the [civil aviation authority of STATE].  Adherence to the guidance herein provided will enable CAA personnel to perform foreign air operator validation and surveillance in a uniform manner.
1.2.3 Policies and Procedures
The policies and procedures herein apply to all foreign commercial air operators falling within the scope of Part XXX of the Civil Aviation Regulations.  Because of the broad scope of foreign air operations and the many variables involved, it is impossible to provide detailed procedures for all requirements.  Therefore, CAA inspectors must have a sound knowledge of their inspection responsibilities and exercise sound judgement in applying the policy and procedures found in this manual.

1.3 Preparation and Distribution of the Manual

This manual is an official [State CAA] document, available to the public upon request.

1.4 Manual Revisions

Suggestions and comments for amendment of the Manual should be sent to the [Director General, Director as appropriate], who is responsible for revisions of this Manual.
1.5 General Considerations
1.5.1  [Civil Aviation Regulations (CARs) or as appropriate]
Pursuant to the authority of Part XXX of the CARs, a foreign air operator shall not operate an aircraft in commercial air transport operations in [State] contrary to the requirements of:

(1)
Part XXX;
(2)
Applicable paragraphs of Parts [STATE GENERAL OPERATING RULES];

(3)
The validation of air operator certificate; and
(4)
The rules of the State of Registry and the State of the Operator of the aircraft that give effect to the Standards and Recommended Practices contained in Annex 1 ( Personnel Licensing), Annex 6 ( Operation of Aircraft), Part I and Part III, Section II, and in Annex 8 (Airworthiness of Aircraft) to the Chicago Convention.

1.5.2 Requirements

To recommend the issuance of a validation of air operator certificate, CAA inspectors shall ensure that:

(1)
The foreign air operator has been issued an Air Operator Certificate (AOC) and associated operations specifications by the State of the Operator in accordance with pertinent national regulations of the State of the operator; 

(2)
The AOC authorizes the air operator to conduct operations to [State] as requested by the foreign air operator; and,
(3)
The State of the Operator is capable to carry out safety oversight of the foreign air operator, as determined by a review of the ICAO USOAP audit reports, and/or audits from another contracting State and/or knowledge of [State CAA] of the safety oversight capabilities of the State of the Operator.

1.5.3 [State] Air Services Agreement Requirements
1.5.3.1
All foreign air operators wishing to operate an air service within [State] or into and out of [State] must obtain an Air Services Agreement from the [State]. This requirement is entirely separate from the process of obtaining a validation of air operator certificate.
1.5.3.2
The Air Service Agreement should contain a safety clause addressing safety requirements that each party to an agreement would need to maintain and helps to ensure that aircraft using airspace and airports in another State are operated and maintained in accordance with ICAO Standards. Ongoing dialogue, as well as surveillance of air operations, would be required to maintain the validity of such an agreement.  The safety clause was first introduced by ICAO in 2001, so Air Service Agreements finalized prior to this date may not contain such a clause.
Note:  DOC 8335, Manual of Procedures for Operations Inspections, Certification and Continued Surveillance outlines the text of a model clause in Attachment VI-A.  
1.5.3.3
[State] may authorize flights not contained in the Air Services Agreement on a case by case basis.  Such flights would be of short duration for special events, for emergency purposes, etc., and a validation of the AOC in accordance with the procedures of this manual may or may not be required depending on the circumstances.  Nevertheless, the [State CAA] will determine that such operations can be conducted in a safe manner, after conducting a suitable risk assessment. 
Intentionally Blank
Chapter 2
2 Validation of Air Operator Certificate
2.1 General

2.1.1
Pursuant to Part XXX. of the CARs, all foreign air operators conducting commercial air transport operations in [State] shall do so under the terms and conditions of an AOC and associated Operations Specifications issued by the State of the Operator, and of a Validation of AOC with additional conditions and limitations as applicable, issued by [State Director General].

2.1.2
Pursuant to Article 11 of the ICAO Convention, each member State recognizes the other contracting States’ right to regulate aircraft that operate in their airspace.  Consequently, the foreign air operator must comply, where applicable, with the [State Civil Aviation Law and the CARs] when operating within [State]’s territory.

2.1.3
When evaluating an application of an operator from another State to operate within its territory, the [ State CAA] must examine both the safety oversight capabilities and record of the State of the Operator and, if different, the State of Registry, as well as the operational procedures and practices of the operator.

2.1.4
In the case where working/technical arrangements have been established, validations will be granted on the basis of such an agreement rather than under this manual.  
2.1.5
Where no working/technical arrangements exist, an administrative review of the relevant documentation as detailed below will be performed. Validation shall be granted in the absence of any significant negative findings / major deficiencies.


In the case of any significant negative findings / major deficiencies, the document review will be followed by additional measures as described in 2.2.6 prior to issuing a validation.

2.2 Validation Process
Note : Appendix C contains a flow chart of the approval process
2.2.1 Documentation Required
Foreign air operators shall be required to submit copies of the documentation below:
(1)
Air Operator Certificate and associated operations specifications;

(2)
Insurance certificate;

(3)
In case of wet-lease of aircraft, approval of CAA of the State of the Operator of the lessee, with identification of the operator that exercises operational control on the aircraft; and
(4)
Document authorizing the specific traffic rights, issued by [appropriate department] or resulting from a bilateral air transport agreement, if required, by the [State].
2.2.2 Initial Application
(1)
Applicants from a contracting State to the Chicago Convention shall make application in accordance with the application contained in Appendix A.

(2)
Application requirements for applicants from a State that is not a contracting State to the Chicago Convention will be determined on a case-by-case basis.

2.2.3 Amendment to an Existing Validation
Applicants requesting an amendment to an existing Validation (that is, a change in or addition to service, aircraft type, etc.) will be required to provide the following documentation: 
(1)
A copy of the relevant CAA authority to operate the aircraft into [State], if amended;
(2)
A completed [State] Foreign Operator’s Application Form;
(3)
In case of wet-lease of aircraft, approval of CAA of the State of the Operator of the lessee, with identification of the operator that exercises operational control on the aircraft; and
(4)
Copies of any amendment to the operations specifications relevant to the application and issued by the State of the Operator.
2.2.4 Review of Documentation

[State CAA] personnel shall confirm receipt of all requested documents.  Normally, certificates and licences issued or rendered valid by a Contracting State to the Chicago Convention will be recognized.  This recognition is predicated on the understanding that the requirements under which the certificates and licences were issued or rendered valid are equal to or exceed the minimum standards established by ICAO.
Note : If there is a concern about the validity of any aspect of the collected documentation and the air operator is unable to provide clarification, this concern is to be conveyed to the CAA of the applicant’s State of Registry or State of the Operator as applicable.
2.2.5 Evaluation of an Application by a Foreign Air Operator

2.2.5.1
When evaluating a new application by a foreign air operator [State CAA] must examine both the safety oversight capabilities and record of the State of the Operator and, if different, the State of Registry.  This is necessary in order for the [State CAA], in the terms of Article 33 to the Convention, to have confidence in the validity of the certificates and licences associated with the operator, its personnel and aircraft, in the operational capabilities of the operator and in the level of oversight applied to the activities of the operator by the State of the Operator.

Note:
ICAO has made available a Safety Oversight Audit (SOA) Secure Site internet website [http://www.icaosoa.ca/soamain] to provide all Contracting States the ability to access safety critical information collected from Contracting States during the Universal Safety Oversight Audit Programme (USOAP) audit.  This secure site contains final safety oversight audit reports including the audit findings, recommendations, State’s action plan and comments, as well as the comments of the Safety Oversight Audit Section on a State’s action plan in addition to all relevant information derived from the Audit Findings and Differences Database (AFDD).  The SOA Secure Site is also accessible through the ICAO Flight Safety Information Exchange (FSIX) website [http://www.icao.int/fsix].

2.2.5.2
If required [State] CAA may request to the State of the Operator for reports of any inspections that may have been conducted to obtain information on an operator from another State.

2.2.5.3 
If required [State] CAA may also request access to reports of audits of the operator in question conducted by independent internationally recognized aviation audit organizations and / or by other air operators, such as code-sharing partners.  Such non-regulatory audits should be used in conjunction with other information such as a report from the ICAO USOAP or other inspection results to evaluate the application.
2.2.6 Additional Measures
2.2.6.1
Additional measures will be taken by the [State] CAA if the initial review indicates significant deficiencies in the safety oversight system of the State of the Operator and, if different, the State of Registry or in the safety performance of the air operator.  
2.2.6.2
In the case of any significant negative findings / major deficiencies, the document review should 
be followed by discussions with the State of the Operator seeking resolution of such deficiencies prior to granting an approval. This information can be supplemented, in part, by the consideration of audits, including operator audits. The State may consider audits performed by other States, by internationally recognized audit organizations or by its CAA.

2.2.6.3
[State] CAA inspectors may also request additional documentation from the air operator that could demonstrate that deficiencies have been rectified. 
2.2.6.4
In case of unsatisfactory additional measures to rectify significant negative findings / major deficiencies, the application to operate by the foreign operator shall be denied and a letter forwarded to the State of the Operator and the State of Registry, if different, with a copy to the operator’s management with details of the significant findings.  An air operator may provide additional supporting documentation or evidence. 
2.3 Reserved
2.4 Issuance or Amendment to a validation
2.4.1
Pursuant to XXX of the [State CARs], the [Director General, Director as applicable], shall, upon determination that the applicant has met all of the safety and regulatory requirements, issue or amend the validation.  The validation remains valid from the date of issue as long as the Air Operator Certificate upon which it is based remains valid.  If the State of the Operator only issues AOCs of short validity period (e.g. one year) or if the AOC expires or is amended soon after the initial approval, it is incumbent on the air operator to provide the updated version of the AOC in a timely manner.  The surveillance programme will enable [State] to confirm that the AOC remains valid. 
2.4.2
If the [State] CAA decides to approve the service, it will issue an appropriate written validation (see Appendix “B”) to the operator. This document may include additional conditions and limitations for elements not listed in the operator’s AOC and its associated operations specifications but considered necessary for the safe operation within [State] territory.  
2.4.3
These conditions and limitations, that may be included, are intended to provide more detailed information and may address unique and special requirements associated with the airspace of [State] where the operations will occur. The issue of these conditions and limitations is part of the State approval of the operations. Such conditions and limitations should not conflict with the AOC and the operations specifications issued by the State of the Operator.
2.5 Records Management
Files are used to maintain records associated with the validation of air operator certificate. The file is opened on the day that the validation is issued and is to contain all material that supports or justifies the granting of a validation.  It contains the latest validation issued to the foreign air operator, justification for the validation, and copies of all superseded documents.
Intentionally Blank
Chapter 3
3 Surveillance of Foreign Air Operators
3.1 Introduction

3.1.1
In order to maintain a Validation of Air Operator Certificate issued by [State CAA], operators will be subject to appropriate surveillance.  This includes regular ramp checks and documentation reviews. In case any significant negative finding / major deficiency is detected during this process, [State CAA] staff shall take appropriate measures, to ensure significant negative findings / major deficiencies are rectified. 

3.1.2
While the State of the Operator which issued the AOC has primary responsibility for overseeing the safety of operations of its certificate holders, [State CAA] undertakes, in accordance with Article 12 to the Convention, to ensure that every aircraft flying over or manoeuvring within its territory shall comply with ICAO and [State] requirements.

3.2 Inspectors
3.2.1
All inspectors who conduct inspections of foreign aircraft should have significant  experience in inspection and surveillance of air operators and understand the difference between ramp inspections conducted on their own operators as part of their certificate management responsibilities and surveillance inspections conducted on aircraft of foreign operators. These inspectors must be specifically trained and authorized to conduct such inspections and possess appropriate credentials identifying them as inspectors employed by [State CAA].

3.2.2
The foreign operator’s ramp inspections shall be carried out in a similar manner to the ramp inspections of [State] operators, with some important differences, as the standards applied to foreign operators are based primarily on international standards and not on national regulations. Inspectors shall be sufficiency fluent in both spoken and written English language to be able to conduct the inspection. For Flight Operations Inspectors this would be equivalent to capability of at least ICAO operational level (level 4). The classroom training shall also be followed by On the Job Training. The inspector’s training file shall be annotated to indicate that the required training has been completed and attesting to the inspector’s qualifications to conduct surveillance of foreign air operators.


The inspectors shall be trained and knowledgeable in the following:

(a)
ICAO Convention and its Annexes 1, 6, 7,8 and 18;
(b)
Differences between ICAO standards and national regulations, which may be more detailed or restrictive;

(c)
Diplomacy, including dealing with potential language difficulties and cultural differences;

(d)
Sovereignty of foreign aircraft, which means that the inspector authority is limited to document, communicate and report findings, except as provided at 3.6.2.3  
(e)
Observing, recording and reporting procedures during inspections of foreign operators; 
(f)
Surveillance activities which are not linked to the certification process of the operator; and
(g)
National regulations related to Foreign Air Operators.
3.2.3 
Inspectors should exercise particular tact and diplomacy during contact with representatives of various foreign government agencies they may deal with, as well as with representatives of foreign air operators.
3.3
Sharing of safety information (upon request)
[State CAA] will share relevant safety findings regarding a foreign operator with other Contracting States.

3.3 Pre-inspection planning
3.4.1
Inspectors should prepare for an inspection by updating themselves on any recent changes to [State] regulations and/or ICAO SARPs with respect to operations by operators from other States.

3.4.2
A check should be made of the authority for the operator to operate the particular aircraft concerned.
3.4.3
The record of the operator’s history in [State] should be examined, including records of past aircraft inspections and, in particular, those of the specific aircraft concerned in the inspection to be conducted, to check for any outstanding actions or recurring trends that might warrant particular attention.

3.4.4
Ramp inspections customarily involve the aircraft and its crew, line station operations, servicing and maintenance and the ramp and gate area condition and activity. Time constraints may apply only to the inspection of the aircraft and crew. Determination should be made of the number of inspectors and the specializations to be involved, the distribution of tasks and the time to be allocated to each task.

3.4.5
Whilst the plan will include comprehensive inspections it might not be possible to cover all the desired elements in the time available for a particular inspection without causing unreasonable delay to the operation.  As inspections on aircraft of any one operator may be conducted at different airports by different inspectors, the overall inspection plan will need to take this into account.  Some elements should be covered at every inspection , others can be covered over a number of inspections.  Thus comprehensive records must be kept of all inspections of aircraft of a particular operator in [specify State records management location or central database, as applicable] accessible to and updated by the inspectors concerned. From these records it will be possible to plan the content of inspections so that a complete inspection of the aircraft of any one operator is undertaken over [define period for a complete inspection cycle.  All Foreign Air Operators will be inspected at least once each year. 
3.4.6
Selection of a particular aircraft to inspect should normally be done at random, in a non-discriminatory manner.  However, the [State] Inspectors shall apply principles of risk management to identify operations perceived to present a higher safety risk and, as a result, conduct additional inspection activities aimed at those operations, which can be linked to a specific:

(a)
State of the Operator;
(b)
aircraft type;
(c)
nature of operations (scheduled, non-scheduled, cargo, etc.);

(d)
foreign air operator; or

(e)
individual aircraft.
3.4.7
Inspectors may need to coordinate inspections with the appropriate customs and security departments.
3.4 Inspections

3.5.1
Routine surveillance will be conducted on all foreign air operators by [State CAA] inspectors at each international airport during ramp inspections.  All ramp inspections of aeroplanes will be conducted utilizing the Ramp Inspection Worksheets contained in Attachment I, II and/or Attachment III. The ramp inspection will be chiefly concerned with the aircraft documents and manuals, flight crew licenses, the apparent condition of the aircraft, and the presence and condition of mandatory safety equipment.
3.5.2
Special-purpose inspections, focused on a particular air operator, may be conducted where previous inspections have indicated a high level of non-conformances to requirements. In addition, reports from air traffic services, airport staff and/or incident reports may also result in a requirement for special-purpose inspections.  

3.5.3
It is to be noted that ramp inspections of foreign air operators are by their nature on-the-spot assessments which can not substitute or replace safety oversight responsibilities of the State of the Operator or the State of Registry.  Ramp inspections serve as pointers but they are not intended to, and they cannot, guarantee the airworthiness of a particular aircraft or the safety of the operator’s flight operations.
Detailed guidance on the conduct of inspections of foreign operators, are described in Appendix “D”.
3.5 Action on Findings – Resolution of Safety Issues
3.6.1
General
3.6.1.1
After a ramp inspection of a foreign air operator, inspector action resulting from findings will depend on the seriousness of the safety finding.  Specific action may also be needed where the State of Registry of the aircraft is different from the State of the Operator.

3.6.1.2
[State CAA] staff will use the information contained in Appendix E as a guide in determining the seriousness of a finding and in the course of action to be taken.  Should a foreign air operator not resolve a major finding in a timely manner then [State CAA] will consider revocation of the validation to operate to [State].
3.6.2 
Follow Up
3.6.2.1
If there is any finding from an inspection, the air operator will be advised in writing, and depending on the seriousness, with a copy to the appropriate foreign CAA of the State of the Operator and/or the State of Registry, as applicable, advising of the safety deficiency or observation and requesting remedial action or comment as appropriate.  If a response is not received from the air operator within thirty days then the foreign CAA should be contacted directly and requested to ensure that corrective action has been taken to rectify the situation.  (See Appendix E, for guidance regarding notification and Attachment IV for sample letters that will be used by inspectorate staff.)

3.6.2.2
In accordance with Article 16 of the Chicago Convention, Inspectors will not normally cause a delay to a particular flight to complete a Ramp Inspection. Where there is insufficient time to complete a particular inspection due to the late arrival of the aircraft or for some other reason, then another inspection should be planned for a subsequent flight, subject to 3.6.2.3.

3.6.2.3
Delays may be unavoidable where it has been determined that the aircraft has damage and is not airworthy. Annex 8, Part II, 3.6 allows the State to prevent a damaged foreign aircraft from resuming its flight operation on the condition that the CAA shall advise the State of Registry or State of the Operator as applicable. Annex 8 also requires that the State of Registry will consider the airworthiness of the aircraft and prohibit the aircraft from resuming flight until it is restored to an airworthy condition or permit the aircraft to resume its flight, if considered airworthy, or permit the aircraft to conduct a non-commercial air transport operation, under prescribed limiting conditions, to an aerodrome at which it will be restored to an airworthy condition.

Note:  Where the State of Registry has entered into an 83bis agreement with the State of the Operator then the State of Operator may substitute for the State of Registry.

Intentionally Blank
Appendix A - FOREIGN AIR OPERATOR APPLICATION

    FOREIGN AIR OPERATOR APPLICATION
PAGE 1 of 2
	Application for Air Transport Operations by a Foreign Operator

(To be completed by a foreign air operator for an approval to conduct operations in [State])

	Section 1A.  To be completed by the air operator

	1. Company registered name and trading name if different. Address of company: mailing address; telephone; fax; and e-mail.


	2. Address of the principal place of business including: telephone; fax; and e-mail.



	3.  Proposed Start Date of Operations:

(dd/mm/yyyy):
	4.  ICAO 3-letter Designator for aircraft operating agency:

	5.  Operational Management Personnel

	Name
	Title
	Telephone, fax and e-mail

	
	
	

	Section 1B. Type of approval requested - To be completed by the air operator, checking applicable boxes

	6
	Air operator intends to conduct commercial flights to and from aerodromes in [State]



	7.
Air Operator proposed types of operation:
	8.
Geographic areas of intended operations and proposed route structure:

	 FORMCHECKBOX 
  Passengers and Cargo

 FORMCHECKBOX 
  Cargo Only

 FORMCHECKBOX 
  Scheduled Operations

 FORMCHECKBOX 
  Charter Flight Operations

 FORMCHECKBOX 
  Dangerous Goods
	

	Section 1C on Page 2 to be completed by the air operator

	Signature:
	
	Date (dd/mm/yyyy):
	Name and Title:

	Section 2.  To be completed by the CAA

	Evaluated by (Name and Office):


	CAA Decision:
 FORMCHECKBOX 
 Approval granted


 FORMCHECKBOX 
 Not approved

	Remarks:



	Signature of CAA representative:
	Date (dd/mmm/yyyy):


FOREIGN AIR OPERATOR APPLICATION
PAGE 2 of 2
	Section 1C.  To be completed by the air operator

	9.
Provide location on board or provide separate documentation where individual aircraft nationality and registration marks are listed as part of the aircraft fleet operated within [State] territory under the air operator certificate:


Provide following information:

	Aircraft Type 
(make, model and series, or master series)
	RVSM-3
Approval
	ETOPS-3 
	Noise-4 Certification
(Annex 16 Ch.)
	Remarks

	[Aircraft type 1]
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	
	

	[Aircraft type 2]
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	
	

	[Aircraft type 3]
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	
	

	[Aircraft type 4]
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	
	

	Etc.
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	
	

	Attach copies of:

Air Operator Certificate and associated operations specifications;
Insurance certificate;

In case of wet-lease of aircraft: approval of CAA of the State of the Operator, with identification of the operator that exercises operational control on the aircraft; and

Document authorizing the specific traffic rights, issued by [appropriate department] or resulting from a bilateral  air transport agreement (if required by the State to which the operator is flying to).


	Note : 3 As approved by the State of the Operator

	           4 As approved by State of Registry 


Appendix B - Sample Validation of Air Operator Certificate
[State]

Validation of Air Operator Certificate 
No.   

This is to certify that

[The Company Registered name and Trading name if different]

meets the requirements of [State] Civil Aviation Regulations Part XXX and may conduct commercial air transport operations into, within, or from [State] territory in accordance with the Air Operator Certificate and associated operations specifications issued by the [State of Operator] and limitations and conditions stated in this Approval.

This Validation is issued on the basis of the [State of Operator] Air Operator Certificate number ____, and remains valid from the date of issue as long as the Air Operator Certificate remains valid.  Unless otherwise stated in the attached Conditions and Limitations the approvals and restrictions of the Air Operator Certificate and its associated Operations Specifications remain valid within [State], in so far as they are not repugnant to the Civil Aviation Regulations of [State].   

This Approval, may be cancelled, suspended, or revoked by the Director General at any time if the conditions of the issue are not met by the foreign air operator or if the Director General determines that such action is required in the interests of aviation safety. 

For the Director General of [State CAA]:

Signature:                     
Title:                         
Department:                   
Date issued:                   
CONDITIONS AND LIMITATIONS FOR OPERATIONS INTO, WITHIN, OR FROM [STATE] TERRITORY

	Company Registered name as Stated in the approval document:



	Address of Principal Place of Business 

	Mailing address:


	Telephone number:

Fax number:

E-mail:

	Trading name(s) if different than the Registered name stated on the Authorization, under which the operator may operate:



	The types of operations authorized:

 FORMCHECKBOX 
  Passenger and Cargo                                                    FORMCHECKBOX 
  Scheduled Operations                                       

 FORMCHECKBOX 
  Cargo only                                                                    FORMCHECKBOX 
  Dangerous Goods 
 FORMCHECKBOX 
  Charter Operations


	Conditions and limitations (if required) 
Note to States:  Only required if the Air Operator Certificate or Operations Specification does not adequately address certain [State] specific requirements that could impact on the safety of operations.

	Destination airport(s) in [State] to be served:



	Aircraft types authorized for use :



	Frequency of flights:   [ Note to State – only required if there is requirement to track the frequency of flights]



Appendix C - Validation Process

Intentionally Blank
Appendix D - Guidance For Ramp Inspection
1. General

The items to be checked during a Ramp Check are summarized below:
A - Flight Deck,

B - Cabin / Safety,

C - Aircraft External Condition, 

D - Cargo, and

E - General.

2. Detailed List


The detailed list contains information on the items to be checked. For each item, guidance is provided on how to perform the check. Each item is also provided with the applicable reference in ICAO Annexes, where available. However the specific references should be checked for the complete requirements. 

Note:  The references to ICAO Annexes are valid as of the Amendment number indicated below:

Annex 1 to Amendment Number 168
Annex 6 to Amendment Number 32
Annex 7 to Amendment Number 5
Annex 8 to Amendment Number 100 
Annex 9 to Amendment Number 20
Annex 15 to Amendment Number 34
Annex 18 to Amendment Number 9
3. Scope
It is not possible to cover all items on the list at every ramp inspection. Inspections should be planned to cover high risk items and to cover all other items over a series of inspections. It is essential that adequate records be kept and that there is complete coordination between all inspectors involved in ramp inspections of any one operator.
4. Items to be checked 
	Flight Deck:

General

	A 1. General Condition

Instructions: Check cleanliness, tidiness and general condition.

References: Nil.

	A 2. Emergency Exit
Instructions: Check if in compliance with ICAO Standards and Recommended Practices.

References: Annex 8, 4.1.7 - Emergency landing provisions.

	A 3. Equipment

Instructions: Check for the presence of the following equipment where required:

Two sensitive pressure altimeters with counter drumpointer or equivalent presentation (IFR operations)

Airborne collision avoidance system (ACAS); 

Cockpit voice recorder (CVR) and flight data recorder (FDR);

Emergency locator transmitter (ELT);

Ground proximity warning system (GPWS); and
Where a flight management computer (FMC) is provided - valid database.

References:

Altimeters    Annex 6, Part I, 6.9.1. c).

ACAS II    Annex 6, Part I, 6.18.

CVR and FDR    Annex 6, Part I, 6.3; and Part III, Section II, 4.3.

GPWS    Annex 6, Part I,  6.15.

ELT    Annex 6, Part I, 6.17 and Part III, Section II, 4.7.

Database    Annex 6, Part I, 7.4.2.




	Documentation

	A 4. Manuals

All required manuals

Instructions: Check for presence.  Check if manuals are up-to-date and accepted or approved as required. Flight manual data may be included in the operations manual which may itself be in several parts, some of which are dealt with in A5, 6 and 7 below.

References: 
Flight Manual - Annex 6, Part I, 6.2.3, 11.1 and Part III, Section II, 4.2.3, 9.1;

Operations Manual - Annex 6, Part I, 4.2.2, 6.2.3 and Appendix 2 and Part III, Section II, 2.2.2, 4.2.3 and Attachment H; and

Aircraft operating manual - Annex 6, Part I, 6.1.4 and Appendix 2, 2.2 and Part III, Section II, 4.1.4 and Attachment H, 2.2.

	A 5 Checklists

Instructions: Confirm checklists are available and up to date. Check if their content is in compliance with the requirement. Normal, non-normal and emergency checklists are sometimes combined in a 'Quick Reference Handbook';

Check the availability of an aircraft search procedure checklist; and

Confirm availability of the checklist of emergency and safety equipment.

References: 
Flight crew checklists - Annex 6, Part I, 4.2.5, 6.1.4 and Appendix 2, 2.2.2; and Part III, Section II, 2.2.5, 4.1.4 and Attachment H, 2.2.10.

Aircraft search procedure checklist - Annex 6, Part I, 13.3 and Part III, Section II, 11.1.

Checklist of emergency and safety equipment - Annex 6, Part I, Appendix 2, 2.2.10 and Part III, Attachment H, 2.2.8.

	A 6. Route Guide
Instructions: Check if a route guide, including charts, is available, suitable and up-to-date.

References: Annex 6, Part I, 6.2.3 and Appendix 2, 2.3.1; and Part III, Section II, 4.2.3 and Attachment H, 2.3.1.

	A 7. Minimum Equipment List (MEL)

Instructions: Check if the MEL is available, up-to-date and approved.

References: Annex 6, Part I, 6.1.3, Appendix 2, 2.2.9 and Attachment G; and Part III, Section II, 4.1.3, Attachment E and Attachment H, 2.2.7.


	A 8. Documents required to be carried on board

a) Certificate of registration

Instructions: Check for presence and accuracy and format.

References: Convention on International Civil Aviation, Article 29; and Annex 7, 7.

b) Identification plate

Instructions: Check presence and location.

Reference: Annex 7, 8.
c) Certificate of Airworthiness

Instructions: Check that the Certificate of Airworthiness of the aircraft is on board and valid.

References: Convention on International Civil Aviation, Articles 29 and 31; Annex 8, Part II, Chapter 3.

d) Crew member licences
Instructions: Check valid in: date; type rating; instrument rating; competency check; language proficiency endorsement; medical assessment; and format (see also item E 3 below).

References: Convention on International Civil Aviation, Article 29; Annex 1, 1.2.1, 1.2.5.1, 1.2.9, 2.1.3, 2.1.7 and Chapter 5; Annex 6, Part I, 9.4.4 and Part III, Section II, 7.4.4.
e) Journey log book or technical log and voyage report

Instructions: Check entries up to date, validity of maintenance release. Check number of deferred defects (specify in the report where necessary). Check that defect deferments include time limits and comply with the stated time limits. Where applicable, check compliance with the aircraft MEL.

References: Convention on International Civil Aviation, Article 29; Annex 6, Part I, 4.3.1 and 11.4; and Part III, Section II, 2.3.1 and 9.4.

f) Radio station licence
Instructions: Check available and up to date.

References: Convention on International Civil Aviation, Articles 29 and 30; Annex 6, Part I, 7.1; and Part III, Section II, 5.1.

g) Noise certification document or statement, where applicable
Instructions: Check available and valid.

References: Annex 6, Part I, 6.13; Part III, Section II, 4.11; and Annex 16, Volume I, Parts I and II.

h) Air Operator Certificate (certified true copy) and Operations Specifications (copy)
Instructions: Check available, applicable and valid.

References: Annex 6, Part I, 4.2.1, 6.1.2, Appendix 5, 7 and Appendix 6; and Part III, Section II, 2.2.1, 4.1.2, Appendix 1, 7 and Appendix 3.


	Flight preparation

	A 9. Operational flight plan
Instructions: Check for presence, accuracy and signature(s), and for adequate fuel and oil reserve planning and supply on board.

References: Annex 6, Part I, 4.3.3 and Appendix 2, 2.1.16; and Part III, Section II, 2.3.3 and Attachment H, 2.1.15.

	A 10. Mass and balance sheet
Instructions: Check for presence of load sheet and accuracy.

References: Annex 6, Part I, 4.3.1 and Appendix 2, 2.1.14; and Part III, Section II, 2.3.1 and Attachment H, 2.1.13.

	A 11. Aircraft performance limitations using current route, airport obstacles and runway analysis data
Instructions: Check for availability of aircraft performance information including limitations and runway performance analysis based on current airport data.
References: Annex 6, Part I, 5.1, 5.2 and 5.3; and Part III, Section II, 3.1 and 3.2.

	A 12. Cargo  manifest and , if applicable, passenger manifest
Instructions: Check for availability of completed cargo manifest and, if required, passenger manifest.

References: Annex 9, 2.12, 2.13 and 4.12 and Appendices 2 and 3.

	A 13. Pre-flight inspection
Instructions: Check for presence of pre-flight inspection or preparation forms (landing documents, air traffic service flight plan).

References: Annex 6, Part I, 4.3; and Part III, Section II, 2.3.

	A 14. Weather reports and forecasts
Instructions: Check for availability of weather reports and forecasts adequate for the flight.

References: Annex 6, Part I, 4.3.5.2; and Part III, Section II, 2.3.5.2.

	A 15. NOTAM (Notice to Airman)
Instructions: Check for availability of NOTAMs for the route of flight.

References: Annex 15, Chapter 1 – Definitions.


	Safety Equipment

	A 16. Portable Fire Extinguishers
Instructions: Check for presence, number, condition and expiry date.

References: Annex 6, Part I, 6.2.2 b); and Part III, Section II, 4.2.2 b).

	A 17. Life jackets/ Flotation devices
Instructions: Check for presence, condition and where applicable expiry date.

References: Annex 6, Part I, 6.5; and Part III, Section II, 4.3.

	A 18. Safety Harness
Instructions: Check for presence, condition and quantity.

References: Annex 6, Part I, 6.2.2; and Part III, Section II, 4.2.2.

	A 19. Oxygen equipment
Instructions: Check for presence, quantity and condition.

References: Annex 6, Part I, 4.3.8; and Part III, Section II, 2.3.8.

	A 20. Emergency flashlight
Instructions: Check for appropriate quantities of emergency flashlight. Check their condition if possible.

References: Annex 6, Part I, 6.10; and Part III, Section II, 4.4.2.


	B. Cabin / Safety

	B 1. General condition
Instructions: Check for cleanliness, tidiness and general condition.

References: Annex 8, Part III, 8.3.

	B 2 Cabin crew seats and safety harness
Instructions: Check for presence and compliance with the requirement.

References: Annex 6, Part I, 6.16; and Part III, Section II, 4.12.

	B 3. First aid kit/emergency medical kit
Instructions: Check for presence, condition, location and expiry date if available.
References: Annex 6,  6.2.2.; and Part III, Section II, 4.2.2.

	B 4. Portable fire extinguishers
Instructions: Check for presence, number, condition and expiry date if available.

References: Annex 6, Part I, 6.2.2; and Part III, Section II, 4.2.2.

	B 5. Life jackets/Flotation devices 
Instructions: Check for presence, condition and expiry date as applicable.

References: Annex 6, Part I, 6.5; and Part III, Section II, 4.5.

	B 6. Seat belts

Instructions: Check for presence and condition.

References: Annex 6, Part I, 6.2.2; and Part III, Section II, 4.2.2.

	B 7. Emergency exit lighting and marking, emergency flashlights

Instructions: Check for presence of emergency exit signs, lighting and marking, and emergency flashlights (one per cabin crew member). Where possible, check condition of floor path lighting / marking and of flashlights.

References: Annex 6, Part I, 6.10; Part III, Section II, 4.4.2; and Annex 8, Part III A, 4.1.7.3 and Part IIIB, D.6.3.

	B 8. Slides / Life Rafts and pyrotechnical distress signalling devices (as required)

Instructions: Check bottle gauge, slide bar and slide expiry date. Check presence of life raft, when required.

References: Annex 6, Part I, 6.5 and 6.6; Part III, Section II, 4.5.and 4.6; Annex 8, Part III A, 4.1.7 (and Part III D.6.2 to D.6.4).

	B 9. Oxygen supply - cabin crew and passengers

Instructions: Check for presence and condition where applicable.

References: Annex 6, Part I, 4.3.8 and 6.7; and Part III, Section II, 2.3.8 and 4.8 and Section III, 2.9 and 4.5.

	B 10. Emergency Briefing Cards

Instructions: Check for presence and accuracy.

References: Annex 6, Part I, 4.2.11.1 and 6.2.2; and Part III, Section II, 2.2.10 and Section III, 2.3.

	B 11. Cabin crew members
Instructions: Check that the number of cabin crew is appropriate. Check whenever possible that the location of cabin crew members allows to effect a safe and expeditious evacuation of the aircraft.

References: Annex 6, Part I, 12.1; and Part III, Section II, 10.1.

	B 12.  Access to emergency exits
Instructions: Check that appropriate access to emergency exits is provided and that it is not impeded.

References: Annex 8, Part III A, 4.1.7 (and Part III D.6.2 and D.6.3).

	B 13. Safety of cabin baggage

Instructions: Check that the the crew and the passengers do not carry oversized hand baggage for the stowage capacity of the aircraft. Check proper stowage of cabin baggage.

References: Annex 6, Part I, 4.8; and Part III, Section II, 2.7.

	B 14. Seating capacity
Instructions: Check that the number of persons boarding does not exceed the number permitted (number of seats normally, except specific circumstances).

References: Annex 6, Part I, 6.2.2; and Part III, Section II, 4.2.2.

	B 15. Security of the flight crew compartment door (if applicable)
Instructions: Check that the flight crew compartment door, if provided, is lockable. Where applicable, check that the flight crew compartment door is penetration resistant.

References: Annex 6, Part I, 13.2.


	C. Aircraft External Condition

	C 1. General condition
Instructions: Check general condition of the airframe: apparent corrosion; cleanliness; presence of ice, snow, frost; legibility of markings, etc.

References: For markings: Annex 7, 3, 4 and 5.

	C 2. Doors and hatches
Instructions: Check for passenger and cargo door condition, external markings, seals, operating instructions and condition of hatches.

References: Nil.

	C 3. Wings and Tail

Instructions: Check wings, vertical and horizontal stabilizers, including all flight control surfaces. Check for obvious damage, corrosion, disbonding, evidence of lightning strikes, dents, looseness of fittings, missing static discharges, etc.

References: Nil.

	C 4. Wheels, brakes and tires
Instructions: Inspect for damage, wear and signs of tire under inflation.

References: Nil.

	C 5. Undercarriage

Instructions: Visual inspection. Focus on lubrication, leakage & corrosion and wear on door fittings and hinges.

References: Nil.

	C 6. Wheel well

Instructions: Visual inspection. Focus on cleanliness, leakage & corrosion.

References: Nil.

	C 7. Intake & exhaust nozzle
Instructions: Visual inspection. Focus on damage, cracking, dents and loose/missing fasteners (intake) and LPT blades (where visible), obvious damage to sensors, jet pipe nozzle, exhaust, thrust reversers, etc.

References: Nil.

	C 8. Fan blades (if applicable)

Instructions: Visual inspection. Check for foreign object damage, cracks, cuts, corrosion, erosion etc.

References: Nil.

	C 9. Propellers (if applicable)

Instructions: Visual inspection. Check for corrosion, looseness of blades in hub, erosion, stone damage, anti/de-icing system, etc.
References: Nil.

	C 1O. Previous structural repairs

Instructions: Visual inspection. Note any previous repairs – check condition and verify compliance to standard practices.

References: Nil.

	C 11. Obvious damage
Instructions: Visual inspection. Note unassessed and unrecorded damage including corrosion, lightning strike damage, and bird strikes etc.

References: Annex 8, Part II, 3.6.

	C 12. Leakage
Instructions: Visual inspection: fuel, oil, hydraulic leaks. Inspect for toilet leaks at service locations.

References: Nil.


	D. Cargo

	D 1. General condition of cargo compartment and containers

Instructions: Check for cleanliness and general condition of cargo compartment and containers.  Check damage to compartment liners and condition of fire protection, detection & extinguishing system (if appropriate). Check condition of container locking devices.

References: Nil.

	D 2. Dangerous Goods

Instructions: If dangerous goods are on board, check that the pilot has received appropriate notification, Check that the operations manual includes relevant information as required by ICAO Annex 18.

References: Annex 6, Part I, Appendix 2, 2.1.35; Part III, Attachment H, 2.1.28; and Annex 18, 9.1 and 9.2.

	D 3. Safety of cargo on board

Instructions: Check that loads are properly distributed and safely secured.

References: Annex 6, Part I, 4.3.1; Part III, Section II, 2.3.1.


	E. General

	E 1. Additional Remarks

Instructions: Record and report any items of significant nature that may be observed which are not covered by this guidance.

References: Nil.

	E 2. Refuelling
Instructions: Check that the procedures relating to refuelling with passengers on board are complied with.

References: Annex 6, Part I, 4.3.7; and Part III, Section II, 2.3.7.

	E 3. Language for communication
Instructions: Check that all pilots, and those flight navigators required to use the radio telephone, are fluent in the language used for radiotelephony communications or in the English language.

References: Annex 1, 1.2.9 


Intentionally Blank
Appendix E - Levels of seriousness and related actions
	ACTIONS TO BE TAKEN AFTER FINDINGS DURING A RAMP INSPECTION OF A FOREIGN OPERATOR
	Actions

	
	Information to pilot-in-command
	Information to responsible CAA (State of the Operator and/or State of Registry) and operational management of the operator
	Corrective actions

	Seriousness of findings
	Minor
	Yes
	No
	No

	
	Significant
	Yes
	Yes

Letter to CAA and copy to operator’s management
	No

	
	Major
	Yes
	Yes

Letter to CAA and copy to Operator’s management. 

In case of aircraft damage affecting airworthiness, a direct communication with the State of Registry CAA will also be established and in accordance with Annex 8 the State of Registry CAA will establish conditions regarding return to flight status. Confirmation will be required by letter to the [State] CAA and copy to operator’s management.
	Yes

Actions consisting of operational restrictions, corrective actions before flight or at maintenance-base, grounding and/or withdrawal of validation of air operator certificate in the territory of the [State] will depend on the circumstances.


Examples of findings and resulting actions

The following table is based on the ramp check guidance contained in Appendix D. It describes various findings and gives examples of levels of seriousness and resulting actions that will be utilized by [State CAA].
	Item #
	Seriousness:(
Item description
	Minor


	Significant


	Major



	A
	Flight Deck:

General

	1
	General Condition
	Dirty and untidy
	
	Large unsecured objects (eg. cargo or baggage)

Unserviceable flight crew seats

	2
	Emergency Exits
	Not all exits are serviceable, but properly deferred in accordance with MEL provisions
	Not all exits are serviceable and MEL provisions not applied
	No emergency exits serviceable / no provisions in MEL for continued operation

	3
	Equipment:

· GPWS




· FMC



· ACAS/CVR/FDR/ELT
	Inoperative and in accordance with MEL provisions

Inoperative and in accordance with MEL provisions
	Inoperative and MEL provisions not applied


FMS database recently outdated (<28 days).

Inoperative and MEL provisions not applied
	Not installed

Forward looking GPWS 
required and not installed.

FMS database more than 28 days outdated.


Required and not installed.


	A
	Documentation

	4
	Manuals:

Flight manual



	
	No evidence of State of Registry approval

Incomplete but performance calculations possible
	Not on board and performance calculations not possible

	
	Operations Manual
	
	Incomplete (see Appendix 2 of ICAO Annex 6) or not approved by State of the Operator or not the current version
	Not on board

	5 
	Checklists
	Not within reach
	Not readily available and used or not the current version
	Not on board

	6
	Route Guide (Navigation charts)
	Not within reach
	Recently out of date (<= 28 days)

Photocopies of current charts
	Significantly out of date (> 28 days)

Not on board

	7
	Minimum Equipment List
	
	Not on board or MMEL used, but no deferred defects

MEL content does not reflect aircraft equipment fitted

MEL not approved
	Not on board or MMEL used, with deferred defects

	8a)
	Certificate of registration
	Non-certified copy
	Not on board

No English translation
	

	8b)
	Identification plate
	
	Not installed or not able to read
	

	8c)
	Certificate of airworthiness
	
	Not an original or certified true copy

No English translation
	Not on board

Out of date

	8d)
	Crew member licences (see also E. General – Language for communications)
	Form or content not in compliance with ICAO standard
	No English translation


	Not valid for the type of aircraft

Not on board or no proper validation from the registration authority

Expired or no class 1 medical assessment

	8e)
	Journey Log Book or equivalent technical log
	Minor defects not documented
	On board but not properly filled in
	Not on board or no equivalent document

Maintenance release expired or not valid

Defects MEL deadline expired

	8f)
	Radio station licence
	Non-certified copy
	Not on board
	

	8g)
	Noise certificate (where applicable)
	Not on board

No English translation
	
	

	8h)
	Air Operator Certificate 
	Non-certified copy
	Not accurate (out of date, incorrect operation type/route, incorrect aircraft or operator, etc…) or no English translation
	Not on board

	9
	Operational flight plan
	Copy not retained on ground
	Actual flight calculations but no actual documents

Lack of fuel monitoring data (arrival flight)

Fuel calculation unsatisfactory (departing flight)
	No or incomplete flight preparation


Required fuel calculation not available or not up dated for actual conditions

	10
	Mass and balance sheet and data
	
	Incorrect but within a/c limits
	Incorrect and outside operational limits or missing

Weight and balance data not available

	11
	Aircraft performance limitations using current route, airport obstacles and runway analysis data
	Incomplete but not affecting the operation on that date (e.g. no contaminated or wet runway data but these conditions are not present)
	Not current data or data validity date not available
	Not available

	12
	Cargo manifest and, if applicable, passenger manifest
	
	Some limited inaccuracy or missing data not affecting safety
	Not available or grossly inaccurate/incomplete

	13
	Pre-flight inspection
	Form on board but incomplete
	Not performed for inbound flight
	Not performed for outbound flight

	14
	Weather reports and forecasts
	Not the latest available data but valid
	Not printed but handwritten
	Not valid or not available

	15
	NOTAM (Notice to Airman)
	
	Some en-route relevant data missing
	Not available

	A
	Safety Equipment

	16
	Portable fire extinguishers
	Not easily accessible
	Expired

Not properly secured
	Empty or insufficient number or missing

Significantly low pressure

Not accessible

	17
	Life jackets/flotation device

(if required)
	Not directly accessible
	Expired, as applicable
	Not available for each cockpit crew member on board

	18
	Safety Harness
	
	Seat belt instead of harness
	Not available or serviceable for all flight crew members

	19
	Oxygen equipment

(if required)
	
	No direct access
	Not available or serviceable for all flight crew members

Oxygen quantity not sufficient

	20
	Emergency flashlight (night operations conducted by operator)
	Only one available
	Weak battery
	Not in cockpit or unserviceable


	B
	Safety / Cabin:

	1
	General Condition
	Dirty, untidy and in bad condition
	Loose carpet

Loose or damaged floor panel

Unserviceable seats (and not identified as such)
	Not possible to perform normal and abnormal duties unrestricted

	2
	Cabin crew seats and safety harness
	Harness/belt is difficult to operate 
	Strap or buckle worn out or damaged – item is not serviceable
	For any member of the minimum required cabin crew: a seat is not available; or
proper harness and seat belt not available or not serviceable

	3
	First aid kit/Emergency medical kit
	Expired

Incomplete

Not at the indicated location
	
	Not available

	4
	Portable fire extinguishers
	Not directly accessible
	Expired

Not correctly secured
	Empty, significantly low pressure or missing or not serviceable

	5
	Life jackets/ Flotation devices (if required)
	Not directly accessible
	Expired, as applicable
	Not available for each person to be carried

	6 
	Seat belts (passenger seats)
	Strap worm or buckle worn out or damaged

Not available or serviceable for all passenger seats and aircraft dispatched in accordance with MEL
	Not available or serviceable for all passengers and aircraft not despatched in accordance with MEL.
	Not available or not serviceable for any passenger

	7
	Emergency exit, lighting and marking, emergency flashlights
	
	Some emergency exit signs out of order

Insufficient number of emergency flashlights

emergency flashlights not correctly located

emergency flashlight batteries weak or flat
	Emergency facilities defects not acceptable according to MEL provisions

	8
	Slides/life-rafts (for long-range over water flights) and pyrotechnical distress signalling devices (as required)
	Not in specified location, as established by the State of the Operator
	Incorrectly installed
	Insufficient number

Not serviceable

	9
	Oxygen Supply (cabin crew and passengers)
	Insufficient quantity of oxygen or insufficient quantity of masks for passengers and crew members
	Insufficient quantity of oxygen or insufficient quantity of masks for passengers and crew members, and flight performed above level 250
	

	10
	Emergency briefing cards
	No enough emergency briefing cards for all passengers
	Briefing cards from another aircraft or from obviously different versions

Some information missing or incorrect
	No emergency briefing cards on board

	11
	Cabin crew members
	
	Cabin crew members not in specified location
	Insufficient number of cabin crew members

	12
	Access to emergency exits
	
	
	Impeded by luggage or cargo, etc

Impeded by seats

	13
	Safety of cabin baggage
	
	
	Not securely stowed

	14
	Seating capacity
	
	
	More seats than certified capacity

Insufficient serviceable seats for all passengers on board

	15 
	Security of the flight crew compartment door (if applicable)
	
	Door not installed or unserviceable (ref. Annex 6, 13.2)
	


	C
	Aircraft condition:

	1
	General external condition
	Minor defects
	The defects need not necessarily be corrected before flight (visible corrosion, marking not legible, etc.)
	Safety related defect (correction required before departure)

Inadequate de-icing

	2
	Doors and hatches
	Minor defects but serviceable
	Door operation instructions missing or unclear

Seal slightly damaged
	Unserviceable and not compatible with passenger number


Seal missing or badly damaged

	3
	Wings and tail
	Minor defects
	Poor condition (damage, missing bonding strips or static discharges, play, lack of lubrication, disbanding)
	Damage, corrosion, leaks or wear outside limits of MEL, SRM
, etc

	4
	Wheels, tires and brakes
	Minor defects
	Signs of under inflation

Incorrect tire pressure

Unusual wear and tear
	Tires worn out or damaged beyond limits

Brakes worn out, leaking or damaged beyond limits

Damaged components or missing parts (i.e., tie bolts, heat sensors…)

	5
	Undercarriage
	Minor defects
	Significant signs of leakage, strut under-pressure, corrosion and obvious lack of lubrication
	Damage, corrosion, missing parts and/or leakage outside limits

	6
	Wheel well
	Minor defects or dirty
	Signs of leakage, corrosion and obvious lack of lubrication
	Damage, wide spread corrosion, leakage outside limits

	7
	Intake and exhaust nozzle
	Minor defects
	Damage to casing or lining

Dents and cracks in exhaust area all within limits, but not recorded in Technical Log or equivalent

Minor leaks of oil and fuel
	Damage (nicks, dents, cracks, etc.) outside the MEL, AMM
, SRM, etc limits

Leakage out-of-limits

	8
	Fan Blades (if applicable)
	Minor defects
	Damage to fan blades within limits but not recorded in Technical Log or equivalent
	Damage (nicks, dents, cracks, etc) outside the MEL, AMM, SRM, etc. limits

	9
	Propellers (if applicable)
	Minor defects
	Damage to propellers within limits but not recorded in Technical Log or equivalent
	Damage (nicks, dents, cracks, etc.), leakage, looseness of blades outside the MEL, AMM, SRM, etc limits

	10
	Previous structural repairs
	Minor defects
	No information about temporary repairs, doubts about old repairs, and repairs acceptable for continuation of flight
	Improperly performed repairs or apparent unsatisfactory design. Damage to old repair



	11
	Obvious un-repaired damage
	Within limits
	Within limits but not recorded
	Unassessed and not recorded damage affecting airworthiness

	12
	Leakage
	Within limits
	Long standing water and lavatory leaks (blue ice)
	Leakage (oil, fuel, hydraulic, water) outside limits


	D
	Cargo:

	1
	General condition of cargo compartment and containers
	Partly defective lights

Minor defects but safe condition
	Partly damaged panelling

Partly damaged containers

Defective lights

Floor locks (partly) u/s

Limited access to cargo (for combi)

Dividing net or door protection net damaged
	Damaged panelling out-of-limits

Damaged containers

Structural damage out of limits

Defective or missing fire extinguishing system (where applicable)

Cargo area not used in accordance with classification

No access to cargo area (for combi)

No barrier net (combi and cargo aircraft)

No smoke barrier/curtain

Floor locks unserviceable and outside MEL limits

	2
	Dangerous Goods
	Unable to recognize dangerous goods presented to operator for shipment
(e.g., Crew unable to respond appropriately to questions from Inspector.)
	No dangerous goods regulations or references
	No or incomplete information to the captain of dangerous goods carried, in contradiction to Doc. 9284 provisions. 

Deficiencies: leakage, wrong packaging, label missing

Dangerous Goods not correctly secured

Loading not performed in accordance with ICAO Annex 18

Dangerous Goods carried without authorization or in contradiction to Annex 18 or Doc. 9284

	3
	Safety of cargo on board
	Minor damage to: lashing, tie down equipment, pallet/container and/or locks
	Damaged pallet, container or net
	Cargo not correctly secured and/or safely distributed: 

- lashing

- tie-down equipment

- pallets and containers

- locks

Load distribution/floor load limit exceeded

	E
	General:

	1
	Additional Remarks
	General findings with minor safety impact
	General findings with significant safety impact
	General findings with major safety impact

	2
	Refuelling
	Some Cabin crew not aware of refuelling with passengers on board
	No procedures in place for refuelling with passengers on board
	Procedures in place but not carried out

	3
	Language for communications
	
	Pilot licences with no language proficiency endorsement, in the English language or the language used in radiotelephony (except if implementation plan made available by State of issuance – until 5 March 2011)
	Pilots not fluent in the English language or the language used in radiotelephony


Attachment I

	RAMP INSPECTION
AIRWORTHINESS WORKSHEET
	CAA Logo
	[STATE CAA]


	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	Date:


	
	Aircraft make and model:
	
	Handling Agent:
	

	
	Operator: 


	
	Airframe serial no:


	
	Maintenance Support:


	

	
	State of the Operator:
	
	Nationality and Reg. marks:

	
	Station:
	

	
	Route from:


	
	Inbound flight no:
	
	
	

	
	Route to: 


	
	
	
	
	

	
	
	
	Check

(S / U / N) 
	S = satisfaction    U = unsatisfactory     N = not check
Remark
	

	
	A
	Flight Deck
	
	
	

	
	1
	General condition
	
	
	

	
	2
	Emergency exit
	
	
	

	
	3
	Equipment (GPWS, ACAS, , ELT,CVR/FDR,etc. )
	
	
	

	
	7
	Minimum Equipment List and deferred defect rectification
	
	
	

	
	8 *
	Documents
	
	
	

	
	9
	Operational flight plan
	
	
	

	
	10 *
	Mass and Balance
	
	
	

	
	11 *
	Aircraft Performance data
	
	
	

	
	12 *
	Cargo/ passenger manifest
	
	
	

	
	16
	Portable Fire Extinquishers
	
	
	

	
	17.
	Life Jackets/flotation devices
	
	
	

	
	18
	Safety Harness
	
	
	

	
	19
	Oxygen equipment
	
	
	

	
	20
	Emergency flashlight
	
	
	

	
	C
	Aircraft External Condition
	
	
	

	
	1 *
	General condition
	
	
	

	
	2
	Doors and hatches
	
	
	

	
	3 *
	Wings and Tail
	
	
	

	
	4 *
	Wheels, brakes and tires 
	
	
	

	
	5 *
	Undercarriage
	
	
	

	
	6 *
	Wheel well
	
	
	

	
	7 *
	Intake and exhaust nozzle
	
	
	

	
	8 *
	Fan blades (if applicable)
	
	
	

	
	9 *
	Propellers (if applicable)
	
	
	

	
	10
	Previous structural  repairs
	
	
	

	
	11 *
	Obvious damage
	
	
	

	
	12 *
	Leakage
	
	
	


	
	D
	Cargo
	
	
	

	
	1
	General condition of cargo compartment and containers
	
	
	

	
	2 *
	Dangerous goods
	
	
	

	
	3
	Safety of cargo on board
	
	
	


	
	E
	General
	
	
	

	
	1
	Additional remarks
	
	
	

	
	2
	Refuelling
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	Inspected by:


	
	
	
	Report No:


	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	Revision date 25 FEB 2009

	


Note 1: The elements of the list that are marked with an asterisk (*) are minimum items that should be addressed in a ramp inspection of an aircraft of an operator from another State. Time permitting, the remaining items should also be addressed to constitute a complete ramp inspection.

Note 2: If inspection is carried out as a team (airworthiness, cabin safety and flight operations), coordination between the inspectors is required to avoid duplication.

Note 3: If cabin safety inspector is not available during the ramp inspection, Cabin / Safety checklist for cabin items is to be used by either airworthiness inspector or flight operations inspector as appropriate.
	RAMP INSPECTION
FLIGHT OPERATIONS WORKSHEET
                     
	State Logo
	    Attachment II

[State CAA]


	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	Date:


	
	Aircraft make and model:


	
	Capt:
	

	
	Operator: 


	
	Nationality and Reg. marks:


	
	F/O:
	

	
	State of the Operator:


	
	Airframe serial no:


	
	F/E or S/O
	

	
	Route from:


	
	Handling agent:


	
	Sup/Capt:
	

	
	Route to:


	
	Station:
	
	Sup/F/O:
	

	
	Inbound flight no:
	
	
	
	
	

	
	
	
	Check
(S / U / N)
	S = satisfaction U = unsatisfactory N = not check

                            Remarks
	

	
	A
	Flight deck
	
	
	

	
	1
	General condition
	
	
	

	
	2
	Emergency exit
	
	
	

	
	3
	Equipment (GPWS, ACAS,  ELT, Cockpit door, FDR/CVR, etc)
	
	
	

	
	4
	Manuals
	
	
	

	
	5
	Checklists 
	
	
	

	
	6
	Route Guide
	
	
	

	
	7*
	Minimum Equipment List and deferred defect rectification
	
	
	

	
	8 *
	Documents
	
	
	

	
	9
	Operational Flight Plan
	
	
	

	
	10 *
	Mass and balance
	
	
	

	
	11 *
	Aircraft performance data
	
	
	

	
	12 *
	Cargo/Passenger maniafest
	
	
	

	
	13
	Pre-flight forms  (ATC flight plan, etc)
	
	
	

	
	14
	Weather reports and forecasts 
	
	
	

	
	15
	NOTAM (Notice to Airman)
	
	
	

	
	16
	Portable Fire Extinguishers
	
	
	

	
	17 
	Life jackets/Flotation Devices
	
	
	

	
	18
	Safety harness
	
	
	

	
	19
	Oxygen equipment
	
	
	

	
	20
	Emergency flashlight
	
	
	

	
	E
	General
	
	
	

	
	3
	Language of communication
	
	
	


	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	Inspected by:


	
	
	
	Report No:


	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	Revision date 25 FEB 2009

	


Attachment III
	RAMP INSPECTION
CABIN SAFETY WORKSHEET 
	State Logo
	[State CAA]

	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	Date:


	
	Aircraft make and model:


	
	Chartered by Operator:


	

	
	Operator: 


	
	Registration mark:


	
	Charter’s state:


	

	
	State of the Operator:


	
	Airframe serial no:


	
	Station:
	

	
	Route from:


	
	Inbound flight no:
	
	
	

	
	Route to:


	
	
	
	
	

	
	
	
	Check
(S/ U / N)
	S = satisfaction U = unsatisfactory N = not check

 Remarks
	

	
	B
	Safety/Cabin

	
	
	

	
	1
	General Condition
	
	
	

	
	
	
	
	
	

	
	2
	Cabin Crew’s seat and harness
	
	
	

	
	
	
	
	
	

	
	3
	First aid kit/Emergency Medical kit
	
	
	

	
	
	
	
	
	

	
	4
	Portable Fire extinguishers
	
	
	

	
	
	
	
	
	

	
	5
	Life jackets/Flotation device
	
	
	

	
	
	
	
	
	

	
	6
	Passenger seat belts
	
	
	

	
	
	
	
	
	

	
	7
	Emergency exit lighting and marking, emergency flashlights
	
	
	

	
	
	
	
	
	

	
	8
	Slides/Life-rafts and pyrotechnical signalling devices (as required)
	
	
	

	
	
	
	
	
	

	
	9
	Oxygen supply (Cabin crew and passengers)
	
	
	

	
	
	
	
	
	

	
	10
	Emergency Briefing cards
	
	
	

	
	
	
	
	
	

	
	11
	Cabin crew members
	
	
	

	
	
	
	
	
	

	
	12
	Access to emergency exits
	
	
	

	
	
	
	
	
	

	
	13
	Safety of cabin baggage
	
	
	

	
	
	
	
	
	

	
	14
	Seating capacity
	
	
	

	
	15
	Security of flight crew comparrment door
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	Inspected by:


	
	
	
	Report No:


	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	Revision date 25 Feb. 2009

	


Intentionally Blank
Attachment IV

SAMPLE CORRESPONDENCE
Minor Findings Sample Letter
File: 12345                                                                                    XX August 2008
Mr. John Doe
Station Manager…..

XWZ Airline 
Fax: 123456

Dear Mr. John Doe,
The [State CAA] conducts regular Ramp Inspections of Foreign Air Operators in [State].  These inspections are conducted under the authority of [State CAR XXX]  which is in conformance with Article 16 of the ICAO Convention.   The inspections are conducted using a detailed checklist which is based on certain aircraft operations Standards as contained in pertinent ICAO Annexes.  

A routine Ramp Inspection was conducted on your flight XYZ123, aircraft registration X-XXXX, prior to its departure from XXX airport on [date].  Attached for your information is a copy of the report which contains minor finding(s).  We would kindly request feedback on the report and that corrective action be taken by your airline within 30 days of receipt of this letter. 

Yours sincerely, 

[function] Civil Aviation Safety Inspector
[State ]
Significant Findings Sample Letter
File: 12345                                                                                    XX August 2008
Mr. John Doe

Station Manager ……..

XWZ Airline 
Fax: 123456

Dear Mr. John Doe,

The [State CAA] conducts regular Ramp Inspections of Foreign Air Operators in [State].  These inspections are conducted under the authority of [State CAR] which is in conformance with Article 16 of the ICAO Convention.   The inspections are conducted using a detailed checklist which is based on certain aircraft operations Standards as contained in pertinent ICAO Annexes.  

A routine Ramp Inspection was conducted on your flight XYZ123, aircraft registration X-XXXX, prior to its departure from XXX airport on [date].  Attached for your information is a copy of the report which contains significant finding(s).  We would request feedback on the report and that corrective action to be taken by your airline.   As you will note below we have also provided a copy of this letter to your Civil Aviation Authority, who may also wish to provide comments concerning these finding(s).  
We would request a reply within 30 days of receipt of this letter. 

Yours sincerely, 

[function] Civil Aviation Safety Inspector
[State ]
CC: Mr. ……

Director General 

[State CAA of aircraft operator and registration if different]
Serious Findings Sample Letter

File: 12345                                                                                    XX August 2008
Mr. ……

Director General 

[Foreign CAA]
Dear Sir,

The [State CAA] conducts regular Ramp Inspections of Foreign Air Operators in [State].  These inspections are conducted under the authority of [State CAR XXX], which is in conformance with Article 16 of the ICAO Convention.   The inspections are conducted using a detailed checklist which is based on certain aircraft operations Standards as contained in pertinent ICAO Annexes. 

A routine Ramp Inspection was conducted on XYZ airline, aircraft registration X-XXXX, prior to its departure from XXX airport on [date].  Attached for your information is a copy of the report which contains serious finding(s).  Due to the serious nature of the finding(s) we would kindly request [Foreign CAA] investigation of this matter and information concerning the corrective action that will be undertaken by the airline to prevent a re-occurrence of these finding(s).    As an interim measure we will increase our surveillance of XYZ airline.
Your assistance with this matter is appreciated and we look forward to receiving the results of your investigation of this safety matter as soon as possible.  
Yours sincerely, 
Mr. XYZ …..

Director General 

[State CAA]











































































No





No





Commercial Air Transport Operators from other States





Additional Inputs:�Findings or Deficiencies


from safety programmes (USOAP, FAA-IASA, �EU-SAFA, etc)





By the bilateral mutual recognition agreement 





Document Check





Further investigation, Audit (as per 2.2.6.2)





foreign operator APPROVAL





Additional measures: denial or other action 





Additional Inputs:


Findings or Deficiencies from safety programmes (USOAP, FAA-IASA, �EU-SAFA, etc)





Ramp Checks


Regular Document Checks


Other Safety indicators








Further investigation, Audit (as per 2.2.6.2)





foreign operator approval maintained / renewed





SURVEILLANCE





Additional measures: withdraw approval or�other action








Initial Check (USOAP, FAA -IASA, EU-SAFA, etc.)





Start of operations





B.A.?





Yes





Finding?





No





No





Yes





Yes





Yes





Signif.?





Finding?





No





No





No





Yes





Yes, significant finding/deficiency





Finding?





Finding?





Finding?





Yes





B.A. = Bilateral Agreement is signed.








� Structural repair manual


� Aircraft maintenance manual





COSCAP 2009-07 r2
PAGE  
COSCAP 2009-07 r2
58



