The amendment to Annex 14, Volume I, contained in this document was adopted by the Council of ICAO on 27 February 2013. Such parts of this amendment as have not been disapproved by more than half of the total number of Contracting States on or before 15 July 2013 will become effective on that date and will become applicable on 13 November 2014 as specified in the Resolution of Adoption. (State letter AN 4/1.2.24-13/20 refers.) Replacement pages incorporating Amendment 11-A will be forwarded as soon as practicable after the amendment becomes effective. Replacement pages incorporating Amendment 11-B are expected to be distributed in October 2014. MARCH 2013 INTERNATIONAL CIVIL AVIATION ORGANIZATION

AMENDMENT No. 11-B

TO THE

INTERNATIONAL STANDARDS

AND RECOMMENDED PRACTICES

AERODROMES

ANNEX 14

TO THE CONVENTION ON INTERNATIONAL CIVIL AVIATION

VOLUME I

AERODROME DESIGN AND OPERATIONS

AMENDMENT 11 TO THE INTERNATIONAL STANDARDS AND RECOMMENDED PRACTICES

ANNEX 14, VOLUME I — AERODROME DESIGN AND OPERATIONS

RESOLUTION OF ADOPTION

The Council

Acting in accordance with the Convention on International Civil Aviation, and particularly with the provisions of Articles 37, 54 and 90 thereof,

1.		Hereby adopts on 27 February 2013 Amendment 11 to the International Standards and Recommended Practices contained in the document entitled International Standards and Recommended Practices, Aerodrome Design and Operations which for convenience is designated Annex 14, Volume I to the Convention;

2.		Prescribes 15 July 2013 as the date upon which the said amendment shall become effective, except for any part thereof in respect of which a majority of the Contracting States have registered their disapproval with the Council before that date;

3.		Resolves that the said amendment or such parts thereof as have become effective shall become applicable on 14 November 2013;

4.		Requests the Secretary General:

- a)	to notify each Contracting State immediately of the above action and immediately after 15 July 2013 of those parts of the amendment which have become effective;

- b)	to request each Contracting State:

  - to notify the Organization (in accordance with the obligation imposed by Article 38 of the Convention) of the differences that will exist on 14 November 20131 between its national regulations or practices and the provisions of the Standards in the Annex as hereby amended, such notification to be made before 14 October 2013, and thereafter to notify the Organization of any further differences that arise;

  - to notify the Organization before 14 October 20132 of the date or dates by which it will have complied with the provisions of the Standards in the Annex as hereby amended;

- c)	to invite each Contracting State to notify additionally any differences between its own practices and those established by the Recommended Practices, when the notification of such differences is important for the safety of air navigation, following the procedure specified in subparagraph b) above with respect to differences from Standards.

— — — — — — — —

NOTES ON THE PRESENTATION OF THE

AMENDMENT TO ANNEX 14, VOLUME I

The text of the amendment is arranged to show deleted text with a line through it and new text highlighted with grey shading, as shown below:

|  | Text to be deleted is shown with a line through it. |  | text to be deleted |
| --- | --- | --- | --- |
|  | New text to be inserted is highlighted with grey shading. |  | new text to be inserted |
|  | Text to be deleted is shown with a line through it followed by the replacement text which is highlighted with grey shading. |  | new text to replace existing text |

TEXT OF AMENDMENT 11-B

TO THE

INTERNATIONAL STANDARDS

AND RECOMMENDED PRACTICES

AEROROMES

ANNEX 14

TO THE CONVENTION ON INTERNATIONAL CIVIL AVIATION

# VOLUME I

AERODROME DESIGN AND OPERATIONS

CHAPTER 1.    DEFINITIONS

. . .

Instrument runway. One of the following types of runways intended for the operation of aircraft using instrument approach procedures:

  - Non-precision approach runway. An instrument A runway served by visual aids and non-visual aid(s)  providing at least directional guidance adequate for a straight-in approach intended for landing operations following an instrument approach operation type A and a visibility not less than 1 000 m.

b)	Precision approach runway, category I. An instrument A runway served by ILS and/or MLS and visual aids and non-visual aid(s) intended for operations landing operations following an instrument approach operation type B with a decision height (DH) not lower than 60 m (200 ft) and either a visibility not less than 800 m or a runway visual range not less than 550 m.

c)	Precision approach runway, category II. An instrument A runway served by ILS and/or MLS and visual aids and non-visual aid(s) intended for operations landing operations following an instrument approach operation type B with a decision height (DH) lower than 60 m (200 ft) but not lower than 30 m (100 ft) and a runway visual range not less than 300 m.

d)	Precision approach runway, category III. An instrument A runway served by ILS and/or MLS visual aids and non-visual aid(s) intended for landing operations following an instrument approach operation type B to and along the surface of the runway and:

A —	intended for operations with a decision height (DH) lower than 30 m (100 ft), or no decision height and a runway visual range not less than 175 m.

B —	intended for operations with a decision height (DH) lower than 15 m (50 ft), or no decision height and a runway visual range less than 175 m but not less than 50 m.

C —	intended for operations with no decision height (DH) and no runway visual range limitations.

Note 1.— See Annex 10, Volume I, for related ILS and/or MLS specifications.

Note 2.1. — Visual aids need not necessarily be matched to the scale of non-visual aids provided. The criterion for the selection of visual aids is the conditions in which operations are intended to be conducted.

Note 2.— Refer to Annex 6 for instrument approach operation types.

. . .

Non-instrument runway. A runway intended for the operation of aircraft using visual approach procedures or an instrument approach procedure to a point beyond which the approach may continue in visual meteorological conditions.

Note.— Visual meteorological conditions (VMC) are described in Chapter 3 of Annex 2.

— END —

13 November 2014 for definitions of instrument and non-instrument runways.

13 October 2014 for definitions of instrument and non-instrument runways.