APPENDIX 1 TO CHAPTER 4

SUMMARY TABLE OF HAZARD IDENTIFICATION AND RISK MANAGEMENT

Hazard Identification and Risk Management Form - ATSRO

| Generic hazard 1 | Specific component of hazard | Hazard-related consequence | Current defences to control risk and the risk index | Further action to reduce risk and the resulting risk index | Responsible party |
| --- | --- | --- | --- | --- | --- |
| Non-RNAV aircraft flying on RNAV routes | Inability of the aircraft to maintain RNAV route | Loss of separation | - Integrated Aeronautical Information System (AIP, NOTAM, AIP, AIC Supplement and others) - FPL - En-route navigation charts - ATS surveillance service - ACAS/TCAS - Operational manuals of the operator and the ANSP 3C | - Direct access of ATS units to updated database of RNAV-approved aircraft - Improve surveillance in the necessary areas (implement VHF, HF, ADS-CPDLC, ADS-B, Radar and other types of communications) - Direct access of ARO/AIS units to updated database of RNAV-approved aircraft - Training of operators and air navigation service providers - Applicable contingency procedures upon identifying a non-RNAV aircraft on an RNAV route - Designate a responsible party in each State to maintain the database of RNAV-approved aircraft - Updating and, if applicable, development of the Operational Manuals of the ANSP, incorporating the appropriate procedures 2C | States Operators ANSPs |

<<<

| Generic hazard 2 | Specific component of hazard | Hazard-related consequence | Current defences to control risk and the risk index | Further action to reduce risk and the resulting risk index | Responsible party |
| --- | --- | --- | --- | --- | --- |
| Loss of RNAV capacity on board | Inability of the aircraft to maintain the RNAV route | Loss of separation | Alternate “conventional” navigation equipment Reversal to “conventional” navigation procedures ATS surveillance service ACAS - Operational manuals of the operator and the ANSP 3C | Standardisation of operator procedures in case of loss of RNAV capability; Improve surveillance in the necessary areas (implement VHF, HF, ADS-CPDLC, ADS-B, Radar and other types of communications) Applicable contingency procedures upon identifying an aircraft that has lost RNAV capability Training of operators, including training of the crew in the application of contingency procedures Training of air navigation service providers ATC simulations RNAV system integrity oversight plan 2C | States Operators ANSPs |

<<<<<

| Generic hazard 3 | Specific component of hazard | Hazard-related consequence | Current defences to control risk and the risk index | Further action to reduce risk and the resulting risk index | Responsible party |
| --- | --- | --- | --- | --- | --- |
| Lack of GND NAVAID coverage | Inability of the aircraft to maintain RNAV route | Loss of separation | - GNSS - IRS/INS - ATS surveillance - Integrated Aeronautical Information System (AIP, NOTAM, AIP Supplement, AIC, etc.) - NOTAM concerning satellite health - ACAS - Operational manuals of the operator and the ANSP - Operational manuals of the operator and the ANSP 3C | Standardisation of procedures for operators in case of inability to maintain the route; Ensure an appropriate coverage by: - Implementing the required GND NAVAIDs; or - Changing from a “conventional” route to an RNAV route, establishing the air navigation system requirements to fly the route. Improve surveillance in the required areas (implement VHF, HF, ADS-CPDLC, ADS-B, Radar and other types of communications) Training of operators and air navigation service providers Updating and, if applicable, drafting of the operational manuals of the ANSP, incorporating the appropriate procedures 2C | States Operators ANSPs |

<<<<<

| Generic hazard 4 | Specific component of hazard | Hazard-related consequence | Current defences to control risk and the risk index | Further action to reduce risk and the resulting risk index | Responsible party |
| --- | --- | --- | --- | --- | --- |
| NAVAID coverage failure | Inability of the aircraft to maintain RNAV route | Loss of separation | - Reversal procedures: - GNSS, in case of failure of GND NAVAIDs, for equipped aeroplanes - Failure of IRS/INS, GND NAVAIDs, for equipped aeroplanes - GND NAVAIDs, in case of GNSS and INS failure - Maintenance of GND NAVAIDs - NOTAM concerning satellite health - Flight inspection - Contingency procedures of the operator and the ANSPs. 3C | Standardisation of operator procedures in case the aircraft cannot maintain its route; Specific procedures to be applied in case of degradation of the navigation capability of the aircraft; Compliance with the Maintenance plan for the verification of ground radio aids; Updating and, if applicable, drafting of the operational manuals of the ANSP, incorporating the appropriate procedures Disseminating information about solar storms that might affect the satellite and HF systems 2C | States ANSPs Operators |

<<<<<

| Generic hazard 5 | Specific component of hazard | Hazard-related consequence | Current defences to control risk and the risk index | Further action to reduce risk and the resulting risk index | Responsible party |
| --- | --- | --- | --- | --- | --- |
| Adverse meteorological effects | Large deviations | Loss of separation | - NOTAM - MET/PIB/VOLMET briefings - ATM briefings between ATFMUs - AIREP - Contingency plan - ACAS - Operational manuals of the operator and the ANSP 4C | - Updating of LOAs between MET/ATM services - Alternate routes - QMS/MET/AIS - Updating of the operational manuals of the operator and the ANSP 2C | States Operators ANSPs |

<<<<<

| Generic hazard 6 | Specific component of hazard | Hazard-related consequence | Current defences to control risk and the risk index | Further action to reduce risk and the resulting risk index | Responsible party |
| --- | --- | --- | --- | --- | --- |
| Volcanic eruption | Volcanic ash Reduced availability of the route network | Engine failure Increase of ATC workload | ASHTAM MET/PIB/VOLMET briefings ATM briefings between ATFMUs AIREP Application of the procedures foreseen in Chapter 15 (15.8) of Doc 4444 Operational manuals of the operator and the ANSP Cancellation of operations on the affected routes 4C | - Re-routing of air traffic - Training of operators, including training of the crew on the application of contingency procedures - Determine ATC sector capacity - ATFM - Updating of LOAs between MET/ATM services - Develop regional contingency procedures - Alternate routes - Updating of the operational manuals of the operator - Updating and, if applicable, drafting of the operational manuals of the ANSP, incorporating the procedures foreseen in Chapter 15 (15.8) of Doc 4444 (4D) | States ANSPs Operators |

<<<<<

| Generic hazard 7 | Specific component of hazard | Hazard-related consequence | Current defences to control risk and the risk index | Further action to reduce risk and the resulting risk index | Responsible party |
| --- | --- | --- | --- | --- | --- |
| Discrepancy of aeronautical information concerning the route network | Lack of integrity of the data published by the States | Loss of separation | - Use of WGS84 - ATS surveillance service - Integrated aeronautical information system (AIP, NOTAM, AIP Supplement, AIC, etc.) - ACAS | - Implement quality management system (QMS) in AIS - Comply with AIRAC cycles | States ANSPs |
|  | Outdated airborne navigation database, if used |  | - Coordination between the States and the navigation database providers - Agreements between operators and navigation database providers - AIRAC cycles 4C | - Comply with AIRAC cycles - Improve and/or formalise coordination between States and navigation database providers - Publish specific navigation database regulations - Schedule operator inspections - Comply with navigation database regulations established by the States 3C | States Operators |

<<<<<

| Generic hazard 8 | Specific component of hazard | Hazard-related consequence | Current defences to control risk and the risk index | Further action to reduce risk and the resulting risk index | Responsible party |
| --- | --- | --- | --- | --- | --- |
| Inadequate civil/military coordination | - Unauthorised entry of civil aircraft in restricted airspaces - Unauthorised entry of military aircraft from restricted airspaces into ATS routes | Loss of separation | - Civil/military LOA - Civil/military coordination committee - Integrated aeronautical information system (AIP, NOTAM, AIP Supplement, AIC, etc.) - Operational manuals of the ANSP - Video charts - Annex 2 and 11 Application of the procedures foreseen in Chapter 16 (16.1) of Doc 4444 - ACAS - ATS surveillance 3C | - Updating of video charts - Updating of LOAs and general civil/military ATM procedures - ATFM - Training of ATCOs and pilots - Restructuring of segregated sectors/airspaces - Optimise or, if applicable, implement a civil/military coordination committee - Flexible use of airspace - Updating and, if applicable, drafting of the operational manuals of the ANSP, incorporating the procedures foreseen in Chapter 16 (16.1) of Doc 4444 and the standards and recommended practices of Annex 2 and 11. 2C | States ANSPs |

<<<<<

| Generic hazard 09 | Specific component of hazard | Hazard-related consequence | Current defences to control risk and the risk index | Further action to reduce risk and the resulting risk index | Responsible party |
| --- | --- | --- | --- | --- | --- |
| Inadequate airspace design | Increased ATCO workload | Operational error of ATCO | ATM/Airspace planning ATS surveillance Training of ATCOs SMS implementation standards (paragraph 2.27 of Annex 11) Capacity management (paragraph 3.1 of Doc 4444) 3D | - Analyse sector workload after route optimisation - Provide a sufficient number of air traffic controllers - Drafting of the safety plan for sectors - ATFM - Updating of ATCO training - Analysis of airspace capacity after RNAV route optimisation - Airspace redesign applying continuous descent operations (CDO) - Implementation of RNAV SIDs/STARs 2D | States ANSPs |

<<<<<

| Generic hazard 10 | Specific component of hazard | Hazard-related consequence | Current defences to control risk and the risk index | Further action to reduce risk and the resulting risk index | Responsible party |
| --- | --- | --- | --- | --- | --- |
| Navigation deviations due to technical errors | Deviation due to engine failure | Loss of separation | - ATS surveillance - Safety assessment - Operational manuals of the operator and the ANSP - Aircraft and operator approval process 4C | - Use of the lateral deviation (LD) reporting form - Monthly delivery of LD data to CARSAMMA - Training of ATCOs in the completion of the lateral deviation reporting form - Use of data available in CARSAMMA for visualising potential technical errors - Operator inspection programmes - Implement ATC system alarms to detect lateral deviations 2C | States ANSPs Operators |
|  | Deviation due to a contingency |  |  |  |  |

<<<<<

| Generic hazard 11 | Specific component of hazard | Hazard-related consequence | Current defences to control risk and the risk index | Further action to reduce risk and the resulting risk index | Responsible party |
| --- | --- | --- | --- | --- | --- |
| Navigation deviations due to operational errors | Lack of coordination between ACCs | Loss of separation | - Measures to reduce operational errors in the ATC coordination loop between adjacent ATCs (GREPECAS15/36) - ATS surveillance - Safety assessment - LOA - Operational manuals of the ANSP 4C | - Implement measures to reduce operational errors in the ATC coordination loop between adjacent ACCs (GREPECAS15/36) - Use of the lateral deviation (LD) reporting form - Investigation of each LD, taking the necessary risk mitigation measures, and monthly delivery of LD data to CARSAMMA - Training of ATCOs in the application of procedures to avoid ATC coordination errors and completion of the the lateral deviation reporting form - Use of data available in CARSAMMA to visualise potential errors - Updating and, if applicable, drafting of the operational manuals of the ANSP, incorporating procedures to reduce operational errors in the ATC coordination loop between adjacent ACCs - Implement ATC system alarms to detect lateral deviations - Implement AIDC 2C | States ANSPs |

<<<<<

| Generic hazard 12 | Specific component of hazard | Hazard-related consequence | Current defences to control risk and the risk index | Further action to reduce risk and the resulting risk index | Responsible party |  |
| --- | --- | --- | --- | --- | --- | --- |
| Outdated operational manuals of operators/ANSP | Erroneous application of procedures | Loss of separation | RNAV 5 approval procedures Safety oversight Audits and inspections (4C) | Implement SMS (2C) | States Operators ANSPs |

<<<<<