The Runway Condition Report (RCR) is the basis for all runway surface condition reporting, and is a comprehensive standardized report relating to runway surface condition and its effect on the aeroplane landing and take-off performance. [Text in italic is the ICAO definition of RCR]

WET – Depth at or below 3 mm is not reported.

3 mm

NOTAM – ‘WET (‘Slippery wet’ runway) NOTAM published. Give additional information in plain language remark section.

Note 1.   For STANDING WATER, 04 (4 mm) is the minimum depth value at and above which the depth is reported. (From 3 mm and below, the runway third is considered WET).

Preparation

- LAST RCR

- Significant changes

- Relevant NOTAMs	    Changes in published declared distances LDA  	       WET (‘Slippery wet’ runway)           Changed geometry, rutting, ponding

- Pilot braking action report after landing - AIREPs – Worse than reported - (When received, Initiate reassessment of the runway surface conditions)

- Prevailing weather condition

- Runway treatments performed		    Newly laid/resurfaced pavement	   Newly rejuvenated pavement        Trend monitoring evaluation

Runway condition assessment - Aeroplane performance section

- DRY						   DRY					   DRY

- WET (up to and including 3 mm)		   WET					   WET

- WET (‘Slippery wet’ runway)			   WET (‘Slippery wet’ runway)   	   WET (‘Slippery wet’ runway)

- STANDING WATER (more than 3 mm)		   STANDING WATER    		   STANDING WATER

Any downgrading due to AIREP, prevailing weather condition, runway surface treatment performed

Additional assessment – Situational awareness section

Reduced runway length (from NOTAM)   	Standardised fixed text: 	RWY nn [L] or nn [C] or nn [R] LDA REDUCED TO [n]nnn.

Example:			RWY 22L LDA REDUCED TO 1450.

Loose sand on the runway			Format/Example:		RWY nn [L] or nn [C] or nn [R] LOOSE SAND. / RWY 02R LOOSE SAND.

Taxiway conditions				Format:				TWY [nn]n POOR. / TWY B POOR.

Apron conditions				Format:				APRON [nnnn] POOR. / APRON NORTH POOR.

Plain language remarks using only allowable characters in capital letters:

A B C D E F G H I J K L M N O P Q R S T U V W X Y Z 0 1 2 3 4 5 6 7 8 9 / [oblique stroke] “.” [period] “ “ [space]

Note.Each information ends with a “.” [period]

RCR                                  //        /  /        /  /                      /             /

Aerodrome   MMDDhhmm     RWY      RWYCC       % Coverage             Depth in mm 	                             Runway surface description 1st, 2nd and 3rd third.		    Width of runway to which the

RWYCCs apply if less than published width

(If applicable)

……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

WET (on any RWY Third)   (Up to and including 3 mm of water)

Report RCR using ATM/ATS only.     ATM part: (Aerodrome operator Accountable manager; Management system addresses the coordination and interface.)

RCR                                 5 /5/5          50/75/100               NR/NR/NR       				 WET/WET/WET

Aerodrome   MMDDhhmm     RWY      RWYCC       % Coverage             Depth in mm 	                             Runway surface description 1st, 2nd and 3rd third.		    Width of runway to which the

RWYCCs apply if less than published width

(If applicable)

NOTAMs? (Reduced runway length LDA) (Rutting?) (Ponding?) (Location?)  ………………………………………………………………………………………………………………………………………………………………………………

……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

3 mm

3 mm

More than 3 mm (on any RWY Third)

WET (Slippery wet runway)

Report RCR using AIM/AIS and ATM/ATS.

AIM and ATM part: (Aerodrome operator Accountable manager; Management system addresses the coordination and interface.)

RCR                                 5 /2/3          50/50/50                  NR/04/NR                                                     WET/STANDING WATER/WET

Aerodrome   MMDDhhmm     RWY      RWYCC       % Coverage             Depth in mm 	                             Runway surface description 1st, 2nd and 3rd third.		    Width of runway to which the

RWYCCs apply if less than published width

(If applicable)

NOTAMs? (Reduced runway length LDA) (‘Slippery wet’ runway) (Rutting?) (Ponding?) (Location?)   ………………………………………………………………………………………………………………………………………

……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………