| 41 MEETING OF PERMANENT CONSULTATIVE COMMITTEE II: RADIOCOMMUNICATIONS May 22 to 26, 2023 Mexico City, Mexico |  |  | OEA/Ser.L/XVII.4.2.41 CITEL/GT/CMR-23/doc. 077/23 rev. 1 25 May 2023 Original: English |  |
| --- | --- | --- | --- | --- |
|  | DRAFT INTER-AMERICAN PROPOSAL FOR WRC-23 AGENDA ITEM 1.9 |  |  |
|  | (Item on the Agenda: 3.1 (SGT-2)) (Document submitted by the Coordinators) |  |  |

SGT 2 – Radiolocation, Maritime, Aeronautical

Coordinator:  Michael Razi (Canada)

Alternate Coordinator: Jose Hernandez (Mexico)

Rapporteur: [TBD]

Alternate Rapporteur: Gregory Baker (USA)

Agenda Item 1.9: to review Appendix 27 of the Radio Regulations and consider appropriate regulatory actions and updates based on ITU-R studies, in order to accommodate digital technologies for commercial aviation safety-of-life applications in existing HF bands allocated to the aeronautical mobile (route) service and ensure coexistence of current HF systems alongside modernized HF systems, in accordance with Resolution 429 (WRC-19);

BACKGROUND

Source: USA 5714

Support:

Brazil, Canada, USA

Aeronautical mobile (R) service (AM(R)S) frequency bands in the range 2 850 – 22 000 kHz are used for long-distance aeronautical communications in remote and oceanic areas. These aeronautical High Frequency (HF) channels are used for Air Traffic Control (ATC) communications to aircraft for most oceanic areas and are mandated as a minimum equipment needed for an aircraft to enter these areas by many international Civil Aviation authorities.

Source: Canada 5763

Support:

Brazil, Canada, USA

The availability of digital technologies provides the opportunity to modernize aeronautical wideband (i.e. multiple 3 kHz channel aggregated contiguously or non-contiguously) communications in the High Frequency (HF) bands towards faster data rates and better voice communications. There is an operational need for the modernization of data link services in the HF band for messages related to the safety and regularity of flight for use by international civil aviation. Current aeronautical HF systems are limited by the available technology and are insufficient to meet many modern aircraft information requirements without being augmented by aeronautical safety satellite communications.

Source: Brazil 5749r1

Support:

Brazil, Canada, USA

The development of advanced digital techniques, including new waveforms, allows the aggregation of both contiguous and non-contiguous channels. This opens the possibly for simultaneous transmission of voice and data, thus improving capacity, connectivity, and quality of HF communication systems. Aviation would like to take advantage of these developments to provide aircraft with additional capabilities and to improve the reliability, availability and continuity of communications especially when used in conjunction with existing L-band aviation SATCOM systems.

Source: Canada 5763

Support:

Brazil, Canada, USA

ITU-R reviewed and identified necessary modifications to Appendix 27 of the RR and assessed the possibility of inclusion of the relevant part of the current text of the Rules of Procedures and other changes to this Appendix on the use of wideband digital emissions. It was concluded that wideband digital communications could be facilitated by aggregating multiple individual contiguous or non-contiguous channels defined in the existing Plan while maintaining the existing regulatory requirements of these channels individually.

PROPOSAL

APPENDIX 27 (REV.WRC-19)*

Frequency allotment Plan for the aeronautical mobile (R)
service and related information

PART  I  –  General provisions

Section II  –  Technical and operational principles used
for the establishment of the Plan of allotment of frequencies
in the aeronautical mobile (R) service

A  –  Channel characteristics and utilization

Source: Brazil 5749r1, Canada 5763, USA 5714

Support:

Brazil, Canada, Uruguey, U.S.A.

2	Frequencies allotted

ADD	DIAP/1.9/1

27/18A		Individual contiguous or non-contiguous channels complying with the provisions of the Plan3 contained in this Appendix may be aggregated to provide wideband communication without changing the Plan of individual channels.

Source: Brazil 5749r1, Canada 5763, USA 5714

Support:

Brazil, Canada, Uruguey, U.S.A.

ADD	DIAP/1.9/2

_______________

3 	27/18A.1	In particular the provisions related to the protection (Part I, Section II B), to power limits (Nos. 27/60 and 27/61), to class of emissions (No. 27/58), to out-of-band spectrum mask (No. 27/74), to assigned frequency (No. 27/75), and to channel spacing (No. 27/11).

Source: Brazil 5749r1, Canada 5763, USA 5714

Support:

Brazil, Canada, Uruguey, U.S.A.

C  –  Classes of emission and power

1	Classes of emission

MOD	DIAP/1.9/3

27/57	1.1	Telephony – amplitude modulation:

double sideband	A3E*

single sideband, full carrier	H3E*

single sideband, suppressed carrier	J3E, J2E, J7E, J9E

Source: Brazil 5749r1, Canada 5763, USA 5714

Support:

Brazil, Canada, Uruguey, U.S.A.

MOD	DIAP/1.9/4

1.2	Telegraphy and data transmission

Source: Brazil 5749r1, Canada 5763, USA 5714

Support:

Brazil, Canada, Uruguey, U.S.A.

MOD	DIAP/1.9/5

27/58	1.2.1	Amplitude modulation:

telegraphy without the use of a modulating audio frequency (by on-off keying)	A1A, A1B**

–	telegraphy by the on-off keying of an amplitude modulating audio frequency or audio frequencies or by the on-off keying of the modulated emission and including selective calling, single sideband, full carrier	H2B

- multichannel voice frequency telegraphy, single sideband, suppressed carrier	J7A

–	telegraphy or data transmissions using any other single sideband, suppressed carrier modulation, under the condition that the reference frequency of the concerned transmission corresponds to the list of carrier (reference) frequencies (No. 27/18) and its occupied bandwidth does not exceed the upper limit of J3E emissions (No. 27/12), i.e. 2 800 Hz for each individual channel	J2B, J2D, J7B, J7D, J9B, J9D

Source: Brazil 5749r1, Canada 5763, USA 5714

Support:

Brazil, Canada, Uruguey, U.S.A.

2	Power

MOD	DIAP/1.9/6

27/60	2.1	Unless otherwise specified in Part II of this Appendix, the peak envelope powers supplied to the antenna transmission line shall not exceed the maximum values indicated in the Table below; the corresponding peak effective radiated powers being assumed to be equal to two-thirds of these values.

Source: Brazil 5749r1, Canada 5763

Support:

Brazil, Canada, Uruguey, U.S.A.

| Class of emission |  | Stations | Maximum peak envelope power |
| --- | --- | --- | --- |
| H2B, J3E, J7A, J2E, J7E, J9E, J2B, J2D, J7B, J7D, J9B, J9D A3E*, H3E* (100% modulation) |  | Aeronautical stations Aircraft stations | 6 kW 400 W |
| Other emissions such as A1A, A1B, F1B |  | Aeronautical stations Aircraft stations | 1.5 kW 100 W |
| * A3E and H3E to be used only on 3 023 kHz and 5 680 kHz. |  |  |

Source: Brazil 5749r1

Support:

Brazil, Canada, Uruguey, U.S.A.

Note: the “(100% modulation)” may require additional clarification.

Source: Brazil 5749r1, Canada 5763, USA 5714

Support:

Brazil, Canada, Uruguey, U.S.A.

SUP	DIAP/1.9/7

RESOLUTION 429 (WRC-19)

Consideration of regulatory provisions for updating Appendix 27 of the Radio Regulations in support of aeronautical HF modernization

Reasons:

Resolution 429 (WRC-19) is proposed to be suppressed as studies were completed and issues were resolved.

*	A3E and H3E to be used only on 3 023 kHz and 5 680 kHz.

**	A1A, A1B and F1B are permitted provided they do not cause harmful interference to the classes of emission H2B, J3E, J2E, J7E, J9E, J7A, J2B, J2D, J7B, J7D, J9B, and J9D. In addition, AlA, A1B and FlB emissions shall be in accordance with the provisions in Nos. 27/70 to 27/74 and care should be taken to place these emissions at or near the centre of the channel. However, a modulating audio frequency is permitted with single sideband transmitters, where the carrier is suppressed in accordance with No. 27/69.