GANP PORTAL

# ASBU ELEMENTS

| ACAS | ACDM | AMET | APTA |
| --- | --- | --- | --- |

| Functional Description | Enablers | Deployment Applicability | Performance Impact Assessment |
| --- | --- | --- | --- |

## ACAS

ACAS-B1/1	ACAS Improvements	Operational

Sixth edition of the GANP

Main Purpose	To provide airborne collision avoidance as a last resort safety net for pilots.

New Capabilities		The traffic alert and collision avoidance system (TCAS) version 7.1 provides short-term improvements to existing airborne collision avoidance systems (ACAS) to reduce nuisance alerts as well as to enhance the logic for some geometries (i.e., Uberlinghen accident). This will reduce trajectory deviations and increase safety in cases where there is a breakdown of separation.

Description		TCAS systems selectively interrogate nearby aircraft to determine their position and velocity (using Mode C/S replies); this information is passed through “threat logic” to determine proximate traffic, issue traffic alerts, and issue collision avoidance “resolution advisories” to flight crews. Resolution advisories provide flight crews with vertical guidance (climb, descend, remain level, do not descend/climb) as appropriate to avoid collisions.

Modern “hybrid surveillance” TCAS systems use ADS-B information to reduce the interrogations needed to perform some of these functions – however, resolution advisories are only issued based on interrogation/reply information (ADS-B data is not used).

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes New resolution advisories.

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

En-route

Departure

Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology need

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

### ENABLERS

Aircraft manufacturer

CAA

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

Certification	TCAS version

7.1 certification

SARPs for TCAS version 7.1, which provides short-term improvements to previous airborne collision avoidance systems (ACAS). Reference: ICAO Annex 6, ICAO Annex 10 Volume IV technical requirements (amendment 85 to Annex 10, Volume IV).

2014

Operational procedures

Operations  Procedures for

the operation of ACAS including phraseology

Procedures for the operation of ACAS including phraseology. References: ICAO Doc 4444 (PANS-ATM), ICAO Doc 8168 (PANS-OPS), ICAO Doc 9863 Airborne Collision Avoidance System (ACAS) Manual.

2010

Airborne system capability

Aircraft system

TCAS II version

7.1 avionics

TCAS II version 7.1 avionics. References: EUROCAE ED-143 or RTCA DO-185B

2008

Aircraft manufacturer

Regulatory provisions

Operational Approval

TCAS II version

Aircraft manufacturer

7.1 operational approval

Responsibility of State of the operator. References: ICAO Annex 6 Part I

CAA 	2014

Airborne system capability

Surveillance	Avionics for

Aircraft operator

extended hybrid surveillance (optional)

Avionics standards for Extended Hybrid Surveillance. References: EUROCAE ED- 221A or RTCA DO-300A, Change 1

2015

Training	-	Training requirements for TCAS version

DEPLOYMENT APPLICABILITY

Operational conditions: Main intended benefits:

operations

Pilot training for TCAS version 7.1, which provides information on new TCAS RA alert wording.

2012

Aircraft operator

ANSP

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Improve situational awareness of flight crew | Aircraft operator |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Improve mid-air collision avoidance (safety net) | ++ | KPI20: Number of aircraft accidents KPI23: Number of airprox/TCAS alert/loss of separation/near midair collisions/midair collisions (MAC) |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

ACAS-B2/1	New collision avoidance system	Operational

Sixth edition of the GANP

Main Purpose		To provide airborne collision avoidance as a last resort safety net for pilots (improving functionality provided in BBB and Block 0).

Implementation must minimize “nuisance alerts” while maintaining or improving existing levels of safety, and must be able to more quickly adapt to changes in procedures and the environment. Also, this successor system must be capable of accommodating reduced separation minima and other new procedures such as 4D trajectory management.

New Capabilities	Implementation of a new airborne collision avoidance system will enable more efficient operations and airspace procedures while complying with safety regulations. Fewer “nuisance alerts” will reduce pilot and controller workload as personnel spend less time responding to such alerts, increasing safety. Operation-specific collision avoidance logic can be engaged via the pilot’s use of their ADS- B-In system for a particular application.

Description		ACAS systems use ADS-B information and selective interrogations of nearby aircraft to determine their position and velocity; this information is passed through “threat logic” to determine proximate traffic, issue traffic alerts, and issue collision avoidance “resolution advisories” to flight crews.

Resolution advisories provide flight crews with vertical guidance (climb, descend, remain level, do not descend/climb) as appropriate to avoid collisions.

In order to achieve a high level of safety, the alerting criteria used by current ACAS systems often overlap with the horizontal and vertical separation associated with many safe and legal procedures (e.g., visual separation operations). ACAS monitoring data from the U.S. indicate that as many as 90% of observed resolution advisories (RAs) are due to the interaction between ACAS II alerting criteria and normal ATC separation procedures (e.g., 500 feet IFR/VFR separation, visual parallel approach procedures, level-off with a high vertical rate, or VFR traffic pattern procedures). This new ACAS system will address this deficiency.

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? No

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   En-route   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

ACAS-B1/1 - ACAS Improvements

Relation-operational need

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-benefit

ASUR-B2/1 - Evolution of ADS-B and Mode S

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

Certification	Certification for

for ACAS Xa/Xo

SARPs for ACAS Xa/Xo, a new airborne collision avoidance system that will enable more efficient operations and airspace procedures while ensuring safety.

References: ICAO Annex 10 Volume IV technical requirements

CAA

Aircraft manufacturer

2020

Operational procedures

Operations	Procedures for

ACAS Xa/Xo operation

Procedures for ACAS Xa/Xo operation. References: ICAO Doc 9863, 3rd edition- Airborne Collision Avoidance System (ACAS) Manual

ANSP

Aircraft manufacturer

Aircraft operator

2022

Aircraft operator

Airborne system capability

Aircraft system

Avionics for ACAS Xa/Xo

Avionics standards for ACAS Xa/Xo. References: EUROCAE/RTCA ED- 256/DO-385

2018

Training	-	Training requirements for ACAS Xo

Training on use of ACAS Xo is recommended/required. References: ICAO Doc 9863, 3rd edition

2020

Regulatory provisions

Operational Approval

ACAS Xo

operational approval

Responsibility of State of the aircraft operator. References: Future Annex 6

CAA 	2020

DEPLOYMENT APPLICABILITY

Operational conditions:

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Improve situational awareness of flight crew | Aircraft operator |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Improve mid-air collision avoidance (safety net) | ++ | KPI20: Number of aircraft accidents KPI23: Number of airprox/TCAS alert/loss of separation/near midair collisions/midair collisions (MAC) |

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

ACAS-B2/2	New collision avoidance capability as part of an overall detect and avoid system for RPAS

Operational

Sixth edition of the GANP

Main Purpose		As part of a detect and avoid system, to provide the airborne collision avoidance function as a last resort safety net for RPAS' pilots.

Implementation will minimize “nuisance alerts” while maintaining safety, and will quickly adapt to changes in procedures and the environment. Also, this system must accommodate the particularities of RPAS.

New Capabilities		Implementation of a new airborne collision avoidance function of a detect and avoid system will enable a new capability for RPAS. The systems will be tailored to provide the last resort collision avoidance function including horizontal resolution advisories (in addition to the vertical resolution advisories provided by current collision avoidance systems).

Description		ACAS systems for RPAS use multiple surveillance sensor inputs to determine the position and velocity of nearby aircraft; this information is passed through “threat logic” to determine proximate traffic, issue alerts, and issue collision avoidance “resolution advisories” to RPAS' pilots. Resolution advisories include both horizontal (turn left or right) and/or vertical guidance (climb, descend, remain level, do not descend/climb) as appropriate to avoid collisions.

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   En-route   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

ACAS-B2/1 - New collision avoidance system

Relation-technology need

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-technology benefit

ASUR-B2/1 - Evolution of ADS-B and Mode S

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

Certification	ACAS Xu

certification

SARPs for ACAS Xu, a new airborne collision avoidance system that will be part of an overall RPAS Detect & Avoid system. References: ICAO Annex 10 Volume IV technical requirements

CAA

Aircraft manufacturer

2022

Operational procedures

Operations	Procedures for

the operation of ACAS Xu

Procedures for the operation of ACAS Xu. References: ICAO Doc 9863 Airborne Collision Avoidance System (ACAS) Manual, 4rd edition; ICAO Doc 10019 Manual on Remotely Piloted Aircraft Systems (RPAS); Potential changes to ICAO Doc 4444 New definitions and procedures for Detect & Avoid Remain Well Clear (RWC); Potential changes to ICAO Doc 8168 New definitions and procedures for Detect & Avoid Remain Well Clear (RWC)

ANSP

Aircraft operator

2024

Airborne system capability

Aircraft system

Avionics for ACAS Xu

Avionics standards for ACAS Xu. References: EUROCAE/RTCA ED-yyy/ DO-xxx

2020

Aircraft manufacturer

Regulatory provisions

Operational Approval

ACAS Xu

operational approval

Responsibility of State of the operator. References: ICAO Annex 6 Part IV future editions

CAA 	2024

Training	-	Training requirements for new collision avoidance capability for RPAS

Operator training on Remain Well Clear, Detect & Avoid systems functionality, and related collision avoidance functionality.

2022

Aircraft operator

### DEPLOYMENT APPLICABILITY

Operational conditions:

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Improve situational awareness of flight crew | Aircraft operator |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Improve mid-air collision avoidance (safety net) | ++ | KPI20: Number of aircraft accidents KPI23: Number of airprox/TCAS alert/loss of separation/near midair collisions/midair collisions (MAC) |

## ACDM

Sixth edition of the GANP

Main Purpose 			To generate common situational awareness, which will foster improved decision making within aerodromes, by sharing relevant surface operations data among the local stakeholders involved in aerodrome operations.

Operational

Airport CDM Information Sharing (ACIS)

ACDM-B0/1

New Capabilities 	 Stakeholders will be able to collaborate and take actions towards the achievement of a set of defined milestones by being aware of the status of an individual flight measured against known target times and milestones.

Description 		This element represents the first collaboration step among stakeholders involved in aerodrome operations. It consists in the definition of common specific milestones for several flight events taking place during surface operations. The stakeholders involved have to, based on accurate operational data, achieve the agreed milestones.

Maturity Level 	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

OPERATIONS

Does it imply a change to levels of automation? No

PLANNING LAYERS

Tactical-During ops

Tactical-Pre ops

Pre-tactical

| Taxi-out | Departure | Arrival | Taxi-in | Turn-around |
| --- | --- | --- | --- | --- |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-information need

AMET-B0/1 - Meteorological observations products

Relation-information need

AMET-B0/2 - Meteorological forecast and warning products

Relation-operational benefit

SURF-B0/2 - Comprehensive situational awareness of surface operations

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational Operations

procedures

Surface

operation milestones procedure

Reference: Manual on Collaborative Air

Traffic Flow Management (ATFM) ICAO Doc 9971

2013

Ground

system infrastructur e

Airport

systems

ACIS system

A simple A-CDM dialog system to a more

advanced A-CDM Information sharing platform (ACISP) to achieve A-CDM information sharing.

2013

Ground handling agent

Aircraft operator

Ground handling agent

Aircraft operator

ANSP

ANSP

Airport operator

Airport operator

Training	-	Training requirements for ACIS

Training in the operational standards and procedures

2013

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |
| Ground handling agent |  |

Operational procedures

Phraseology	ACIS

Phraseology

Phraseology for the implementation of ACIS. References: Procedures for Air Navigation Services-Air Traffic Management (Doc 4444)

2013

Aircraft operator

ANSP

DEPLOYMENT APPLICABILITY

#### Operational conditions:

This element is expected to bring benefits in complex or even simple but constrained airports. Collaborative decision- making by information sharing can highly facilitate coordination of common operational solutions in order to improve access and equity to ATM resources.

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |  |  |
| --- | --- | --- | --- | --- |
| Direct benefits | Efficiency of operations | Airport operator | ANSP | Aircraft operator |
|  | Improve situational awareness of airport operator, aircraft operator and ANSP | Airport operator | ANSP | Aircraft operator |
| Indirect benefits | Increased safety | Airport operator | ANSP | Aircraft operator |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

Sixth edition of the GANP

Main Purpose

Airport CDM operations will be enriched by enhanced arrival information from the ATM network and,

at the same time, network operations will benefit from more accurate departure information from CDM airports.

New Capabilities	To connect airport operations to the ATM network.

Operational

Integration with ATM Network function

ACDM-B0/2

Description 		This element consists in feeding arrival information from the network into A-CDM and, at the same time, coordinate specific departure milestones. The involved stakeholders have to, based on accurate operational data, achieve the agreed milestones.

Maturity Level 	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

OPERATIONS

Does it imply a change to levels of automation? No

PLANNING LAYERS

Tactical-During ops

Tactical-Pre ops

Pre-tactical

| Taxi-out | Departure | Arrival | Taxi-in | Turn-around |
| --- | --- | --- | --- | --- |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

RSEQ-B0/1 - Arrival Management

Relation-operational need

RSEQ-B0/2 - Departure Management

Relation-benefit

SURF-B0/2 - Comprehensive situational awareness of surface operations

Relation-benefit

FICE-B0/1 - Automated basic inter facility data exchange (AIDC)

Relation-operational need

NOPS-B0/4 - Initial Airport/ATFM slots and A-CDM Network Interface

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational Operations	Procedures for

procedures	turnaround integration

Procedures for: integration of aircraft

turnaround with ATM/ATFM objectives. References: Manual on Collaborative Air Traffic Flow Management (ATFM) (Doc 9971)

2013

Operational Phraseology	Phraseology for

procedures	turnaround integration

Phraseology for the integration of the

turnaround within the network. . References: Procedures for Air Navigation Services-Air Traffic Management (Doc 4444).

ANSP

Aircraft operator

2013

Ground	-

system infrastructur e

A-CDM

system/platform- ATFM system interconnectivity

Interconnection of ACDM and the network

using data exchange models.

2013

Aircraft operator

ATM network function

Aircraft operator

ATM network function

ANSP

ANSP

Airport operator

Airport operator

Training	-	Training requirements for the integration of the turnaround

Training in the operational standards and procedures

2013

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |
| Ground handling agent |  |

DEPLOYMENT APPLICABILITY

#### Operational conditions:

This element is expected to bring benefits in complex or even simple but constrained airports surrounded by complex airspace. Collaborative decision-making by information sharing can highly facilitate coordination of common operational solutions in order to improve access and equity to ATM resources. The integration of aerodromes with the ATM network is naturally contributing to more accurate and efficient tactical and operational decision-making.

| Airport operator | ANSP |  |  |
| --- | --- | --- | --- |
| ATM network function |  | Aircraft operator |  |

Main intended benefits:

| Type Operational description Benefitting stakeholder(s) |  |
| --- | --- |
| Direct benefits | Efficiency of Operations Airport operator ANSP Aircraft operator |
|  | Improve situational awareness of network manager, airport operator, aircraft operator and ANSP |
| Indirect benefits Increased safety Airport operator ANSP Aircraft operator |  |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

Sixth edition of the GANP

Main Purpose

To enhance the planning and management of airport operations and allow their fully integration in the

ATM network and enhance collaboration between airport stakeholders.

New Capabilities	Airport stakeholders will be able to better communicate and coordinate among themselves to

develop and maintain dynamically joint plans and to execute those in their respective area of responsibility.

Operational

Airport Operations Plan (AOP)

ACDM-B2/1

Description 		This element consists of a collaborative airport operations plan (AOP) which encompasses “local” airport information and shared information with the ATM network in order to develop a synchronized view for the integration of local airport operations as well as aircraft operations into the overall ATM network.

The AOP includes an airport performance framework and steers with specific performance indicators and targets aligned with the regional/national performance frameworks, building upon A- CDM. Information on resources and aircraft operation plans is available to the different operational units on the airport and elsewhere in ATM.

The AOP may be managed and monitored by the Airport Operations Centre (APOC).

Maturity Level 	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

OPERATIONS

Does it imply a change to levels of automation? Yes

| Strategical | Pre-tactical |  | Tactical-Pre ops |
| --- | --- | --- | --- |
| Tactical-During ops |  |  |  |

| Taxi-out | Departure | Arrival | Taxi-in | Turn-around |
| --- | --- | --- | --- | --- |

PLANNING LAYERS

DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Evolution	ACDM-B0/1 - Airport CDM Information Sharing (ACIS) Relation-operational need	ACDM-B0/2 - Integration with ATM Network function

Relation-operational need	NOPS-B1/3 - Enhanced integration of Airport operations planning with

network operations planning

Relation-information need	AMET-B1/1 - Meteorological observations information Relation-information need	AMET-B1/2 - Meteorological forecast and warning information Relation-benefit	ACDM-B2/2 - Airport Operations Centre (APOC)

Relation-benefit	SURF-B1/1 - Advanced features using visual aids to support traffic management during ground operations

Relation-benefit	SURF-B1/5 - Enhanced vision systems for taxi operations

Relation-benefit	FICE-B2/2 - Filing Service

Relation-benefit	FICE-B2/4 - Flight Data Request Service

Relation-information need	AMET-B1/3 - Climatological and historical meteorological information Relation-information need	DAIM-B1/7 - NOTAM improvements

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Ground system infrastructure

Airport systems

AOP	Develop the Airport Operations Plan module

Airport operator 2019

Operational procedures

-	CDM information management processes

Definition of roles, responsibilities and processes to fully integrate airports in the ATM network. References: Manual on Collaborative (total) Airport Management (Doc xxxx)

Airport operator 2025

Aircraft operator

ANSP

ATM network function

Training	-	Training requirements for AOP

Training in the operational standards and procedure

Airport operator 2019

ATM network function

ANSP

Aircraft operator

Ground handling agent

### DEPLOYMENT APPLICABILITY

#### Operational conditions:

This element is expected to bring benefits in complex or even simple but constrained airports surrounded by complex airspace. Collaborative decision-making by information sharing can highly facilitate coordination of common operational solutions in order to improve access and equity to ATM resources. The full integration of aerodromes within the ATM network using standardized data and information is naturally contributing to more accurate and efficient tactical and operational decision-making.

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |  |  |
| --- | --- | --- | --- | --- |
| Direct benefits | Efficiency of Operations | Airport operator | ANSP | Aircraft operator |
|  | Improve situational awareness of stakeholders |  |  |  |
| Indirect benefits | Increased safety | Airport operator | ANSP | Aircraft operator |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

ACDM-B2/2	Airport Operations Centre (APOC)	Operational

Sixth edition of the GANP

Main Purpose		The integration of all stakeholders, both landside and airside, into a coherent decision making entity/process (and team), using the shared information and capabilities provided through the AOP.

New Capabilities		The APOC is an additional but important means by which the efficiency of the overall airport operations will be further enhanced. This will be achieved by bringing all stakeholders together in a physical facility, using the shared information and capabilities of the AOP (ensuring thereby a coherent overall airport performance monitoring), decision making and steering process, addressing all phases of operations (strategic planning, through operation to post operations).

Description		The APOC will bring stakeholders together in a physical entity (team) enabling them to better communicate and coordinate, to develop and dynamically maintain joint plans which are executed in their respective areas of responsibility at the airport.

Its main information source is the Airport Operations Plan, which integrates information from the appropriate process monitors, collating it into consistent, timely and reliable knowledge for the airport’s various operational units, in particular the APOC.

The APOC will be equipped with a real-time monitoring system, a decision support system and will apply a set of collaborative procedures that build upon the capabilities of the AOP. This will ensure that the management of landside and airside airport processes will be fully integrated.

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? No

### PLANNING LAYERS

Pre-tactical

Tactical-Pre ops

Tactical-During ops

DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

### OPERATIONS

| Taxi-out | Departure | Arrival | Taxi-in | Turn-around |
| --- | --- | --- | --- | --- |

Relation-information need	ACDM-B2/1 - Airport Operations Plan (AOP)

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Ground system infrastructur e

Ground system infrastructur e

Operations	Operational

procedures to support operations within the APOC

- APOC

information sharing system

- APOC

monitoring and decision support systems

Procedures for data sharing, management and decision making within the APOC. References: Manual on Collaborative (total) Airport Management

Implementation of a system for information sharing between the APOC actors.

Implementation of real-time monitoring and decision support systems within an operational facility able to accommodate representation from all stakeholders.

2025

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |
| Ground handling agent |  |

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |
| Ground handling agent |  |

2019

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |
| Ground handling agent |  |

2019

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |
| Ground handling agent |  |

Ground system infrastructur e

- Physical APOC	Designation and equipage of a physical

infrastructure as an APOC.

2019

DEPLOYMENT APPLICABILITY

Operational conditions:

On complex or even simple but constrained airports, Collaborative Decision Making by information sharing can highly facilitate coordination of common operational solutions in order to improve access and equity to ATM resources. Full Integration with ATM network using standardized data and information is naturally contributing to more accurate and efficient tactical and operational decision-making. Applicable in aerodromes with complex traffic and infrastructure as well as complexity within the surrounding airspace. The airport should also operate during a certain period at a level of saturation.

Main intended benefits:

ANSP

Airport operator

Training	-	Training requirements for APOC

Training and competency monitoring in the use of procedures, responsibilities and decision support tools.

2019

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
|  | Efficiency of Operations | Airport operator ANSP Aircraft operator |

| TDyirpeect benefits | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
|  | Improve situational awareness of airport operator, aircraft operator and ANSP | Airport operator ANSP Aircraft operator |
| Indirect benefits | Increased safety | Airport operator ANSP Aircraft operator |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

ACDM-B2/3	Total Airport Management (TAM)	Operational

Sixth edition of the GANP

Main Purpose		Total Airport Management (TAM) is an approach that takes a holistic view of airport performance management, integrating all stakeholders including the ATM network, local ATM, passenger terminal operations, service providers, passenger and baggage management and ground transportation. All stakeholders are integrated into a coherent planning and collaborative decision-making process using shared information and capabilities.

New Capabilities		TAM is an enhancement of the APOC with integration of the landside management aspects to support further improvement of the efficiency of the overall airport operation including passenger management. This will be achieved using the shared information and capabilities of the AOP, APOC and landside management thereby ensuring a coherent overall airport performance monitoring, decision making and steering process, addressing all phases of operations (strategic planning, through operation to post operations).

All essential airport processes from passenger check-in to aircraft turn-round work collaboratively with the common goal of ensuring that each departure meets its agreed 4D-trajectory. The airport is considered as one node of the overall air transport network. In order to ensure an overall Quality of Service (QoS) of an airport to the customers and to the air transport network, the integrated APOC concentrates on the initial strategic and pre-tactical planning phases using the most accurate information available, followed by the monitoring (and when required, reactive planning) of the tactical working process.

Description 		TAM will bring stakeholders together as physical entity (team) enabling them to better communicate and coordinate, to develop and dynamically maintain joint plans which are executed in their respective areas of responsibility at an airport.

Its main information source will be the Airport Operations Plan with the level of predictability allowed by TBO as well as Landside Management including Passenger management, which integrates information from the appropriate process monitors, collating it into consistent, timely and reliable knowledge for the airport’s various operational units, in particular the APOC.

TAM will be equipped with a real-time monitoring system, a decision support system and will apply a set of collaborative procedures that build upon the capabilities of the APOC. This will ensure that the management of landside and airside airport processes will be fully integrated including passenger management.

Maturity Level 	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

OPERATIONS

Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

Tactical-Pre ops

Pre-tactical

| Taxi-out | Departure | Arrival | Taxi-in | Turn-around |
| --- | --- | --- | --- | --- |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

SWIM-B2/1 - Information service provision

Relation-operational need

SWIM-B2/2 - Information service consumption

Relation-operational need

SWIM-B2/3 - SWIM registry

Relation-information benefit

FICE-B2/2 - Filing Service

Relation-information benefit

FICE-B2/4 - Flight Data Request Service

Relation-information need

AMET-B2/1 - Meteorological observations information

Relation-information need

AMET-B2/2 - Meteorological forecast and warning information

Relation-information benefit

FICE-B2/5 - Notification Service

ENABLERS

Enabler

Category

Enabler Type	Enabler Name

Description / References

Stakeholders	Year

Operational procedures

Operational procedures

Ground system infrastructur e

Ground system infrastructur e

Ground system infrastructur e

Information

Operations	TAM airside

operational procedures

Operations	Operational

procedures to support TAM landside airport management

- TAM information sharing system

- TAM monitoring and decision support systems

- TAM landside

systems

Flight and flow Flight Information

Procedures for data sharing, management and decision making within the APOC. References: Manual on Collaborative Air Traffic Flow Management (ATFM) (Doc 9971) Manual on the System-wide Information Management (SWIM) Concept (Doc 10039)

Landside Operational procedures for passenger and luggage management

Implementation of SWIM information services to support the exchange of information between the APOC actors

Implementation of real-time monitoring and decision support systems within an operational facility able to accommodate representation from all stakeholders.

Landside Management systems including passenger management, which integrate information from the appropriate process monitors, collating it into consistent, timely and reliable knowledge for the airport’s various operational units, in particular the APOC.

Reference: PANS-ATM (Doc 4444) and

2025

Aircraft operator

Airport operator

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |
| Ground handling agent |  |

2025

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |
| Ground handling agent |  |

2025

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |
| Ground handling agent |  |

2025

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |
| Ground handling agent |  |

2025

ATM network function

ANSP

2024

exchange model

information

Exchange Model (FIXM) Version x.x.x

FF-ICE Manual (Doc 9965).

DEPLOYMENT APPLICABILITY

Operational conditions:

On complex or even simple but constrained Airports, Collaborative Decision Making by Information Sharing can highly facilitate coordination of common terminal throughput solutions in order to improve airside operations. The Cooperative Airport Management is benefiting from Trajectory based operations and provids predictable performance to the TBO. Better predictability of the landside is achieved for the benefit of APOC processes and airside operations.

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Efficiency of oeprations | Airport operator ANSP Aircraft operator |
| Indirect benefits | Increased safety | Airport operator ANSP Aircraft operator |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

ACDM-B3/1	Full integration of ACDM and TAM in TBO	Operational

Sixth edition of the GANP

Main Purpose	To use the integration of ACDM in the overall synchronization of the ATM network, to contribute

to end-to-end stable, consistent and robust trajectory-based operations providing the adequate level of performance.

New Capabilities	ACDM is fully synchronized with TBO.

Description		All stakeholders are fully connected. All tactical decisions are fully synchronized and operations are fully trajectory-based. Aerodrome operations are considering the en-route to en-route view with the turnaround process, agree on, and subsequently manage the flights on the surface, to deliver expected surface event times with known impacts to the ATM system, and to ensure that the agreed trajectory is consistent with the Airport Operations Plan.

ACDM is contributing to the network based, efficiently-converging coordination process as a subcomponent of the overall ATM network synchronization process.

Maturity Level	Concept

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS

Pre-tactical

Tactical-Pre ops

Tactical-During ops

DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

### OPERATIONS

| Taxi-out | Departure | Arrival | Taxi-in | Turn-around |
| --- | --- | --- | --- | --- |

Evolution

-

Relation-operational need

SWIM-B2/1 - Information service provision

Relation-operational need

SWIM-B2/2 - Information service consumption

Relation-operational need

SWIM-B2/3 - SWIM registry

Relation-information need

DAIM-B2/5 - NOTAM replacement

Relation-information need

AMET-B2/1 - Meteorological observations information

Relation-information need

AMET-B2/2 - Meteorological forecast and warning information

Relation-information benefit

FICE-B2/2 - Filing Service

Relation-information benefit

FICE-B2/4 - Flight Data Request Service

Relation-information benefit

FICE-B2/5 - Notification Service

ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Information

Flight and flow Flight Information

References: PANS-ATM ICAO Doc 4444

ANSP

2020

exchange model

information

Exchange Model (FIXM) Version 4.2.0

- Procedures for Air Navigation Services and ICAO Doc 9965- FF-ICE Manual 1st Edition.

ATM network function

Aircraft operator

ATM network function

Operational procedures

Operations	Operational

procedures to support operations within a full integrated TAM

Procedures for data sharing, management and decision making within the APOC. References: Manual on Collaborative Air Traffic Flow Management (ATFM) (Doc 9971) Trajectory Based Operations Concept Manual on the System-wide Information Management (SWIM) Concept (Doc 10039)

2031

Ground handling agent

Operational procedures

Operations	Operational

ATM network function

ANSP

Airport operator

procedures to support integrated TAM Landside

Landside Operational procedures for passenger and luggage management.

2031

Aircraft operator

Aircraft operator

Airport operator

ANSP

Airport operator

Ground system infrastructur e

- Information

Ground handling agent

sharing systems in support of the integration of TAM

Implementation of SWIM information services to support the exchange of information between the APOC actors. References: Manual on the System-wide Information Management (SWIM) Concept (Doc 10039)

2031

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |
| Ground handling agent |  |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

Training

-

Training

requirements for integrated TAM

Training and competency monitoring in the

use of procedures, responsibilities and decision support tools.

2031

DEPLOYMENT APPLICABILITY

Operational conditions:

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Benefitting stakeholder(s)

Operational description

Type

AMET

AMET-B0/1	Meteorological observations products	Information

Sixth edition of the GANP

Main Purpose	Meteorological observations in support of flexible airspace management, improved situational awareness, collaborative decision-making and dynamically optimized flight trajectory planning.

New Capabilities	Provision of observations of additional meteorological parameters/elements. More automated observations. Higher temporal and spatial resolution for lightning, radar and satellite information.

Description 		This element represents the provision of meteorological observational products including:

Automatic Weather Observation System (AWOS) information (including real-time exchange of wind and RVR data)

Local reports (MET REPORT / SPECIAL) Aerodrome reports (METAR / SPECI) Lightning information

Ground-based weather radar information Meteorological satellite imagery

Aircraft meteorological report (ie. ADS-B, AIREP, AMDAR etc.) Vertical wind and temperature profiles

Volcano Observatory Notice for Aviation (VONA) Wind shear alerts

Maturity Level 		Ready for implementation

Human Factor Considerations

  - Does it imply a change in task by a user or affected others? No

  - Does it imply processing of new information by the user? No

  - Does it imply the use of new equipment? Yes

OPERATIONS

Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

Tactical-Pre ops

| Taxi-out | Departure |  | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- | --- |
| Turn-around |  |  |  |  |  |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology benefit

ASUR-B0/3 - Cooperative Surveillance Radar Downlink of Aircraft

Parameters (SSR-DAPS)

Relation-technology benefit

COMS-B0/2 - ADS-C (FANS 1/A) for procedural airspace

Relation-technology need

COMI-B0/7 - ATS Message Handling System (AMHS)

Relation-technology benefit

COMI-B0/1 - Aircraft Communication Addressing and Reporting System

(ACARS)

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

National regulatory framework

National framework amendment for the provision of meteorological observations products

Amendment to national regulations to include changes to the provision of quality- assured meteorological observation products to support flexible airspace management, improved situational awareness, collaborative decision-making and dynamically optimized flight trajectory planning. References: • Annex 3 - Meteorological Service for International Air Navigation • WMO No.49 Vol II - Technical Regulations - Basic Documents No. 2, Volume II – Meteorological Service for International Air Navigation • WMO No.49 Vol IV - Technical Regulations - Basic Documents No. 2, Volume IV – Quality Management • Doc. 4444 - Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM)

CAA 	2013

Operational procedures

Information exchange

Procedures for the provision of meteorological observations products

Procedures for changes to the provision of quality-assured meteorological observation products, including additional meteorological parameters and higher spatial and temporal resolution observations. References: • WMO No.8 - Guide to Meteorological Instruments and Methods of Observation • WMO No.306 - Manual on Codes – International Codes • WMO No.731 – Guide to Meteorological Observing and Information Distribution Systems for Aviation Weather Services • WMO No.732 - Guide to Practices for Meteorological Offices Serving Aviation • WMO No.782 – Aerodrome Reports and Forecasts • WMO No.1100 - Guide to the Implementation of a Quality Management System for National Meteorological and Hydrological Services • Doc. 7488 – Manual of the ICAO Standard Atmosphere

Aircraft manufacturer

Doc. 8896 - Manual of Aeronautical Meteorological Practice • Doc. 9328 - Manual of Runway Visual Range Observing and Reporting Practices • Doc 9837 - Manual on Automatic Meteorological Observing Systems at Aerodromes

ANSP

MET

Information Service Provider

2013

Airborne system capability

Aircraft system

Transmission of meteorological observations data from aircraft

Onboard sensors and communication means for the measurement and transmission of meteorological data by: ACAR, Mode-S, ADS-B & ADS-C.

2013

Ground system infrastructur e

Information exchange

Automated systems and infrastructure to support the provision of meteorological observations products

Meteorological instrumentation, IT systems and communications infrastructure for the measurement, storage, display and transmission (including AFS) of meteorological observations products. References: • WMO No.8 - Guide to Meteorological Instruments and Methods of Observation • Annex 10 - Aeronautical Telecommunications

Airport operator

ANSP

MET

| Airport operator |  |  |
| --- | --- | --- |
| ANSP | CAA |  |
| Aircraft operator |  |  |
| MET Information Service Provider |  |  |

Information Service Provider

2013

Training	-	Training requirements for the provision of meteorological observations products

Training for meteorological personnel and aviation industry stakeholders on meteorological observations products.

References: • WMO No. 1083 – Guide to the Implementation of Education and Training Standards in Meteorology and Hydrology • WMO No.1205 – Guide to Competency

2013

AMET-B0/2	Meteorological forecast and warning products	Information

Sixth edition of the GANP

Main Purpose	Meteorological forecasts, advisories and warnings in support of flexible airspace management, improved situational awareness, collaborative decision-making and dynamically optimized flight trajectory planning.

New Capabilities	Improved visualisation of meteorological forecast products. Greater resolution (spatial and temporal) of gridded WAFS information (e.g. wind, temperature, icing, turbulence, CB clouds).

Description		This element represents the provision of meteorological forecast (including advisory and warning) products including:

World Area Forecast System (WAFS) gridded products Significant Weather (SIGWX)

Low-level Area Forecast (GAMET) Aerodrome Forecast (TAF)

Trend Forecast (TREND) Take-off Forecast

Tropical Cyclone Advisory (TCA) Volcanic Ash Advisory (VAA) AIRMET

SIGMET

Aerodrome Warning Wind Shear Warning

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS

Pre-tactical

Tactical-Pre ops

Tactical-During ops

DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

### OPERATIONS

| Taxi-out | Departure |  | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- | --- |
| Turn-around |  |  |  |  |  |

Relation-operational need	AMET-B0/1 - Meteorological observations products

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

National regulatory framework

National framework amendment for the provision of meteorological forecast products and warnings

Amendment to national regulations to include changes to the provision of quality- assured meteorological forecast and warning products to support flexible airspace management, improved situational awareness, collaborative decision-making and dynamically optimized flight trajectory planning.

References: • Annex 3 - Meteorological Service for International Air Navigation • WMO No.49 Vol II - Technical Regulations

- Basic Documents No. 2, Volume II – Meteorological Service for International Air Navigation • WMO No.49 Vol IV - Technical Regulations - Basic Documents No. 2, Volume IV – Quality Management • Doc. 4444 - Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM)

CAA 	2013

Operational procedures

Information exchange

Procedures for the provision of Meteorological forecast products and warnings

Procedures for changes to the provision of  ANSP

MET

Information Service Provider

quality-assured meteorological forecast and warning products, including improved visualisation and higher spatial and temporal resolution of gridded products. References: • WMO No.306 - Manual on Codes – International Codes • WMO No.732 - Guide to Practices for Meteorological Offices Serving Aviation • WMO No.782 – Aerodrome Reports and Forecasts • WMO No.1100 - Guide to the Implementation of a Quality Management System for National Meteorological and Hydrological Services • Doc. 7488 – Manual of the ICAO Standard Atmosphere

- Doc. 8896 - Manual of Aeronautical Meteorological Practice • Doc. 9691 - Manual on Volcanic Ash, Radioactive Material and Toxic Chemical Clouds • Doc. 9766 - Handbook on the International Airways Volcano Watch (IAVW) • Regional SIGMET Guides

2013

Training	-	Training requirements for the provision of meteorological forecast products and warnings

Training for meteorological personnel and aviation industry stakeholders on meteorological forecast and warning products. References: • WMO No. 1083 – Guide to the Implementation of Education and Training Standards in Meteorology and Hydrology • WMO No.1205 – Guide to Competency

Airport operator 2013

| ANSP | CAA |  |
| --- | --- | --- |
| Aircraft operator |  |  |
| MET Information Service Provider |  |  |

Ground	Information	Systems and

system	exchange	infrastructure to infrastructur		support the

e	provision of

meteorological forecast and warning products

IT systems and communications

infrastructure for the storage, forecasting, display and transmission (including AFS) of meteorological forecast and warning products. References: • Doc. 8896 - Manual of Aeronautical Meteorological Practice

2013

MET

Information Service Provider

AMET-B0/3	Climatological and historical meteorological

products

Information

Sixth edition of the GANP

Main Purpose	Climatological products in support of the design and planning of infrastructure, flight routes and airspace management. Historical meteorological observations, forecasts, advisories and warnings in support of incident and accident investigations.

New Capabilities	Nil

Description	This element represents the provision of climatological products including:

Aerodrome climatological tables; Aerodrome climatological summaries.

This element also represents the provision of historical products including meteorological observations, forecasts, advisories and warnings.

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

### PLANNING LAYERS

Post operations

### OPERATIONS

DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-operational need	AMET-B0/1 - Meteorological observations products

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

National regulatory framework

National framework amendment for the provision of climatological meteorological information products

Amendment to national regulations to include changes to the provision of climatological and historical meteorological products in support of the design and planning of infrastructure, flight routes and airspace management and to support incident and accident investigations. References: • Annex 3 - Meteorological Service for International Air Navigation • WMO No.49 Vol II - Technical Regulations - Basic Documents No. 2, Volume II – Meteorological Service for International Air Navigation • WMO No.49 Vol IV - Technical Regulations - Basic Documents No. 2, Volume IV – Quality Management

CAA 	2013

Operational procedures

Information exchange

Procedures for the provision of climatological meteorological information products

Procedures for changes to the provision of climatological and historical meteorological, including aerodrome climatological tables and summaries.

References: • Doc. 8896 - Manual of Aeronautical Meteorological Practice

2013

MET

Information Service Provider

Training	-	Training requirements for the provision of climatological meteorological information products

Training for meteorological personnel and aviation industry stakeholders on climatological and historical meteorological products. References: • WMO No. 1083 – Guide to the Implementation of Education and Training Standards in Meteorology and Hydrology • WMO No.1205 – Guide to Competency

Airport operator 2013

| ANSP | CAA |  |
| --- | --- | --- |
| Aircraft operator |  |  |
| MET Information Service Provider |  |  |

Ground system infrastructur e

Information exchange

Systems and infrastructure to support the provision of climatological meteorological products

IT systems and communications infrastructure for the storage, display and transmission (including AFS) of climatological meteorological products. References: • Doc. 8896 - Manual of Aeronautical Meteorological Practice

2013

MET

Information Service Provider

Sixth edition of the GANP

Information

Dissemination of meteorological products

AMET-B0/4

Main Purpose 	Dissemination of meteorological products in support of flexible airspace management, improved situational awareness, collaborative decision-making and dynamically optimized flight trajectory planning

New Capabilities  Commencement of the exchange of meteorological information using the ICAO Meteorological Information Exchange Model (IWXXM), being the conversion of Traditional Alphanumeric Code (TAC), using an IWXXM schema, into XML/GML.

Description 		This element represents the dissemination of meteorological products using a variety of formats and means.

Formats include:

TAC

Gridded

Graphical (i.e., PNG format)  BUFR code

IWXXM (in XML/GML)

Dissemination means includes aeronautical fixed service (AFTN with increasing use of AMHS), and via secure internet services (ie. WIFS/SADIS).

Maturity Level 	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

PLANNING LAYERS

OPERATIONS

Does it imply a change to levels of automation? No

| Pre-tactical | Tactical-Pre ops |  | Tactical-During ops |
| --- | --- | --- | --- |
| Post operations |  |  |  |

| Taxi-out | Departure |  | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- | --- |
| Turn-around |  |  |  |  |  |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

AMET-B0/1 - Meteorological observations products

Relation-operational need

AMET-B0/2 - Meteorological forecast and warning products

Relation-technology need

COMI-B0/7 - ATS Message Handling System (AMHS)

ENABLERS

Enabler

Category

Enabler Type	Enabler Name

Description / References

Stakeholders	Year

Regulatory provisions

National regulatory framework

National framework amendment for meteorological information exchange

Amendment to national regulations to include changes to the dissemination of meteorological products in support of flexible airspace management, improved situational awareness, collaborative decision-making and dynamically optimized flight trajectory planning.

References: • Annex 3 - Meteorological Service for International Air Navigation • WMO No.49 Vol II - Technical Regulations

- Basic Documents No. 2, Volume II – Meteorological Service for International Air Navigation • Annex 10 – Aeronautical Telecommunications • Annex 15 - Aeronautical Information Services

CAA 	2013

Operational procedures

Information exchange

Procedures for meteorological information exchange

Procedures for changes to the dissemination of meteorological products, including the exchange of meteorological information using the ICAO Meteorological Information Exchange Model (IWXXM).

References: • Doc. 8896 - Manual of Aeronautical Meteorological Practice • Doc. 9377 – Manual on the Coordination between Air Traffic Services, Aeronautical Information Services and Aeronautical Meteorological Services • Doc. 9855 - Guidelines on the use of the Public Internet for Aeronautical Applications • Doc. 9880

- Manual on Detailed Technical Specifications for the Aeronautical Telecommunication Network (ATN) using ISO/OSI Standards and Protocols • Doc. 9896 – Manual on the Aeronautical Telecommunication Network (ATN) using Internet Protocol Suite (IPS) Standards and Protocol • Doc. 10003 - Manual on the Digital Exchange of Aeronautical Meteorological Information • ICAO Guidelines for the Implementation of OPMET Data Exchange using IWXXM • Regional OPMET Interface Control Documents • Regional OPMET Bulletin Exchange Handbooks

MET

Information Service Provider

ANSP

2013

Ground system infrastructur e

Information exchange

Communications infrastructure for meteorological information exchange

Ground system infrastructure and dissemination systems for meteorological products, including message switching systems and the roll-out of the Aeronautical Message Handling System (AMHS) to replace the Aeronautical Fixed Telecommunications Network (AFTN).

References: • Doc. 9880 - Manual on Detailed Technical Specifications for the Aeronautical Telecommunication Network (ATN) using ISO/OSI Standards and Protocols • Doc. 9896 – Manual on the Aeronautical Telecommunication Network (ATN) using Internet Protocol Suite (IPS) Standards and Protocol; • Regional OPMET Interface Control Documents • Regional OPMET Bulletin Exchange Handbooks

ANSP

MET

Information Service Provider

2013

Training	-	Training for meteorological information exchange

Training for meteorological and aviation IT personnel, including NOC/ROC/RODB/IROG personnel on the exchange of meteorological products.

References: • Regional OPMET Interface Control Documents • Regional OPMET Bulletin Exchange Handbooks

ANSP

MET

Information Service Provider

2013

Information exchange model

Meteorologic al information

ICAO

Meteorological Information Exchange Model (IWXXM) V1-V2

ICAO Meteorological Information Exchange Model (IWXXM) schema used to enable the provision meteorological information in XML/GML form. Version 1.1 supported Annex 3 amendment 76 (2013).

Version 2.1 supported Annex 3

amendment 77 (2016). References: • Doc. 10003 - Manual on the Digital Exchange of Aeronautical Meteorological Information • ICAO Guidelines for the Implementation of OPMET Data Exchange using IWXXM • WMO IWXXM Wiki: https://wiswiki.wmo.int/tiki- index.php? page=IWXXM&structure=WIS+up

ANSP

MET

Information Service Provider

2013

Sixth edition of the GANP

Main Purpose 		Meteorological observations information in support of automated decision processes or aids and performance based requirements, involving meteorological information, meteorological information translation, ATM impact conversion and ATM decision support.

Information

Meteorological observations information

AMET-B1/1

New Capabilities 	 Commencement of change from product-centric to data-centric information. Commencement of space weather and sulphur dioxide (SO2) services. Enhanced hazardous weather services.

Indroduction of new and enhanced space-based observations. Introduction of new observational information from both un-manned and manned aircraft (ie. observations from lidar).

Description 			Meteorological observations will begin to transition from traditional alphanumeric code (TAC) form to data-centric information to better support the common understanding on the various operational constraints, capabilities and needs. The following SWIM-compliant observational parameters and phenomena will begin to be made available to users and will include:

Wind speed and direction (aerodrome) including gusts

Wind speed and direction from departure to Top of Climb (TOC) and then Top of Descent (TOD) to landing

Wind speed and direction en-route

Air temperature and dew point temperature (aerodrome)

Air temperature and dew point temperature (or equivalent, i.e. humidity) from departure to TOC and then TOD to landing (including the following derived outputs: freezing level, lower tropospheric temperature inversions)

Air temperature and dew point temperature (or equivalent) en-route  Pressure (aerodrome) (i.e. QNH/QFE)

Visibility (aerodrome) (horizontal, slant, vertical), Runway visual range (RVR)  Cloud type (of operational significance)

Cloud coverage, bases, tops and layers

Thunderstorms, Lightning, Convection (TCU & CB)

Precipitation (ie. drizzle, rain, freezing rain, snow, hail)

Weather (ie. dust storm, sand storm, funnel cloud, squall, smoke, haze, mist, fog)  Icing, including airframe and engine

Liquid Water Content, Iced Water Content  Turbulence, Mountain waves, Wind shear  Fronts

Radioactive clouds, Toxic chemicals  Tropical cyclones

Volcanic ash

Sulphur dioxide (SO2) and other hazardous gases  Aerodrome surface (runway) temperature, state

Sea temperature, state and wave height (seaports)  Space weather events

Tsunami, Flood

Characteristics of the meteorological information include:

Time (ie. observation time) Units of measurement Resolution (spatial)

Geo Location (2D/3D/4D context, point, line or polyhedron) Movement

Severity, Accumulation, Intensity Range (Max. – Min.)

Variations

Data sample period

Auto or Human (Observed, Measured or Calculated) Amendment / Correction

Operational Status Source

Thresholds

Format (TAC, Gridded, Graphical, IWXXM)  Data quality flag

Runway identification or location identifier

Effects/impact on aviation systems (i.e. communications, navigation & surveillance systems)  Radiation (exposure)

An increase in the use of performance measures (via compliance, availability and regularity indices) of meteorological observations.

Maturity Level 		Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

OPERATIONS

Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

Tactical-Pre ops

| Taxi-out | Departure |  | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- | --- |
| Turn-around |  |  |  |  |  |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

AMET-B0/1 - Meteorological observations products

Relation-technology benefit

COMS-B1/2 - PBCS approved ADS-C (FANS 1/A+) for procedural

airspace

Relation-technology need

COMI-B0/7 - ATS Message Handling System (AMHS)

Relation-technology need

COMI-B0/1 - Aircraft Communication Addressing and Reporting System

(ACARS)

Relation-technology need

ASUR-B0/3 - Cooperative Surveillance Radar Downlink of Aircraft

Parameters (SSR-DAPS)

ENABLERS

Enabler

Category

Enabler Type	Enabler Name

Description / References

Stakeholders	Year

Regulatory provisions

National regulatory framework

National framework amendment for the provision of meteorological observations information

Amendment of national regulations to include the provision of quality-assured meteorological observations information in support of automated decision processes or aids and performance based requirements, involving meteorological information, meteorological information translation, ATM impact conversion and ATM decision support. References: • Annex 3

- Meteorological Service for International Air Navigation • WMO No.49 Vol II - Technical Regulations - Basic Documents No. 2, Volume II – Meteorological Service for International Air Navigation • WMO No.49 Vol IV - Technical Regulations - Basic Documents No. 2, Volume IV – Quality Management • Procedures for Air Navigation Services – Meteorology (PANS-MET) – being developed • Doc. 4444 - Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM)

CAA 	2019

Operational procedures

Information exchange

Procedures for the provision of meteorological observations information

Procedures for changes to the provision of  ANSP

MET

Information Service Provider

quality-assured meteorological observations information, including space weather. References: • WMO No.8 - Guide to Meteorological Instruments and Methods of Observation • WMO No.306 - Manual on Codes – International Codes • WMO No.731 – Guide to Meteorological Observing and Information Distribution Systems for Aviation Weather Services • WMO No.732 - Guide to Practices for Meteorological Offices Serving Aviation • WMO No.782 – Aerodrome Reports and Forecasts • WMO No.1100 - Guide to the Implementation of a Quality Management System for National Meteorological and Hydrological Services • Doc. 7488 – Manual of the ICAO Standard Atmosphere

- Doc. 8896 - Manual of Aeronautical Meteorological Practice • Doc. 9328 - Manual of Runway Visual Range Observing and Reporting Practices • Doc 9837 - Manual on Automatic Meteorological Observing Systems at Aerodromes • Doc. 10100 – Manual on Space Weather Information in Support of Air Navigation

2019

Airborne system capability

Ground system infrastructur e

Aircraft system

Information exchange

Transmission of meteorological observations information from aircraft

Automated systems and infrastructure to support the provision of meteorological observations information

Onboard sensors and communication means for the measurement and transmission of meteorological information by: ACAR, Mode-S, ADS-B & ADS-C

| Airport operator |  |  |
| --- | --- | --- |
| ANSP | CAA |  |
| Aircraft operator |  |  |
| MET Information Service Provider |  |  |

Meteorological instrumentation, IT systems and communications infrastructure for the measurement, storage, display and transmission (including AFS) of meteorological observations information. References: • WMO No.8 - Guide to Meteorological Instruments and Methods of Observation • Annex 10 - Aeronautical Telecommunications

2019

Aircraft manufacturer

| Airport operator |  |
| --- | --- |
| ANSP |  |
| MET Information Service Provider |  |

2019

Training	-	Training requirements for meteorological observations information

Training for meteorological personnel and aviation industry stakeholders on meteorological observations information. References: • WMO No. 1083 – Guide to the Implementation of Education and Training Standards in Meteorology and Hydrology • WMO No.1205 – Guide to Competency

2019

AMET-B1/2	Meteorological forecast and warning information	Information

Sixth edition of the GANP

Main Purpose		Meteorological forecast and warning information for automated support for decision processes or aids and performance based requirements, involving meteorological information, meteorological information translation, ATM impact conversion and ATM decision processes.

New Capabilities	Commencement of change from product-centric to data-centric information. Commencement of space weather and sulphur dioxide (SO2) services. Enhanced hazardous weather services. First steps in the provision of probabilistic information derived from ensemble prediction systems.

Description 			Meteorological forecasts and warnings will begin to transition from traditional alphanumeric code (TAC) form to data-centric information to better support the common understanding on the various operational constraints, capabilities and needs. The following SWIM-compliant forecast parameters and phenomena will begin to be made available to users and will include:

Wind speed and direction (aerodrome) including gusts and operationally significant wind shifts Air temperature and dew point temperature (aerodrome)

Upper level:

Wind (speed and direction), including departure to Top of Climb (TOC) and then Top of Descent (TOD) to landing

Air temperature and dew point temperature or equivalent (i.e. humidity), including height of freezing level and lower tropospheric temperature inversions

Flight level and temperature of tropopause

Geopotential altitude for flight levels

Pressure (aerodrome) (i.e. QNH, QFE)

Visibility (aerodrome), Runway visual range (RVR)  Cloud type (of operational significance)

Cloud coverage, bases, tops and layers

Thunderstorms, Lightning, Convection (TCU & CB)

Precipitation (ie. drizzle, rain, freezing rain, snow, hail)

Weather (ie. dust storm, sand storm, funnel cloud, squall, smoke, haze, mist, fog)  Icing (airframe and engine),

Liquid Water Content, Iced Water Content  Turbulence, Mountain waves, Wind shear  Fronts

Radioactive clouds, Toxic chemicals  Tropical cyclones

Volcanic ash

Sulphur dioxide (SO2) and other hazardous gases  Aerodrome surface (runway) temperature, state

Sea temperature, state and wave height (seaports)  Space weather events

Tsunami, Flood

Characteristics of the meteorological information include:

Time (ie. issue time, validity, commencement/cessation, lead time)  Units of measurement

Resolution (temporal & spatial)

Geo Location (2D/3D/4D context, point, line or polyhedron)  Movement

Severity, Accumulation, Intensity  Range (Max. – Min.)

Variations

Probability of occurrence

Confidence/Uncertainty of forecast  Reliability

Data sample period  Auto

Change indicator/period  Amendment / Correction  Operational Status

Source

Thresholds

Format (TAC, Gridded, Graphical, IWXXM)  Data quality flag

Runway identification or location identifier

Effects/impact on aviation systems (i.e. communications, navigation & surveillance systems)  Radiation (exposure)

Human-readable meteorological advisory and warning products start to be derived from the meteorological information/data to better suit user needs and can be based on user defined thresholds. Meteorological information to be used to assess impact.

Verification of quality (accuracy) of forecast parameters. An increased use performance measures (via compliance, availability and regularity indices) of forecast parameters.

Maturity Level 		Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

OPERATIONS

Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

Tactical-Pre ops

Pre-tactical

| Taxi-out | Departure |  | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- | --- |
| Turn-around |  |  |  |  |  |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

AMET-B0/2 - Meteorological forecast and warning products

Relation-operational need

AMET-B1/1 - Meteorological observations information

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

National regulatory framework

National framework amendment for the provision of meteorological forecast and warnings information

Amendment to national regulations to include changes to the provision of meteorological forecast and warning information in support of automated decision processes or aids and performance based requirements, involving meteorological information, meteorological information translation, ATM impact conversion and ATM decision support. References: • Annex 3

- Meteorological Service for International Air Navigation • WMO No.49 Vol II - Technical Regulations - Basic Documents No. 2, Volume II – Meteorological Service for International Air Navigation • WMO No.49 Vol IV - Technical Regulations - Basic Documents No. 2, Volume IV – Quality Management • Procedures for Air Navigation Services – Meteorology (PANS-MET) – being developed • Doc. 4444 - Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM)

CAA 	2019

Operational procedures

Information exchange

Procedures for the provision of meteorological forecast and warnings information

Procedures for changes to the provision of quality-assured meteorological forecast and warning information, including commencement of the change from product-centric to data-centric information, space weather, enhanced hazardous weather services and the provision of probabilistic information derived from ensemble prediction systems.

References: • WMO No.306 - Manual on Codes – International Codes • WMO No.732 - Guide to Practices for Meteorological Offices Serving Aviation • WMO No.782 – Aerodrome Reports and Forecasts • WMO No.1100 - Guide to the Implementation of a Quality Management System for National Meteorological and Hydrological Services • Doc. 7488 – Manual of the ICAO Standard Atmosphere

| Airport operator |  |  |
| --- | --- | --- |
| ANSP | CAA |  |
| Aircraft operator |  |  |
| MET Information Service Provider |  |  |

Doc. 8896 - Manual of Aeronautical Meteorological Practice • Doc. 9691 - Manual on Volcanic Ash, Radioactive Material and Toxic Chemical Clouds • Doc. 9766 - Handbook on the International Airways Volcano Watch (IAVW) • Doc. 10100 – Manual on Space Weather Information in Support of Air Navigation • Regional SIGMET Guides

ANSP

MET

Information Service Provider

2019

Training	-	Training requirements for Meteorological forecast and warning information

Training for meteorological personnel and aviation industry stakeholders on meteorological forecast and warning information. References: • WMO No.

MET

Information Service Provider

1083 – Guide to the Implementation of Education and Training Standards in Meteorology and Hydrology • WMO No.1205 – Guide to Competency

2019

Ground system infrastructur e

Information exchange

Systems and infrastructure to support the provision of meteorological forecast and warning information

IT systems and communications infrastructure for the storage, forecasting, display and transmission (including AFS) of meteorological forecast and warning information. References: • Doc. 8896 - Manual of Aeronautical Meteorological Practice

2019

Information

Climatological and historical meteorological

information

AMET-B1/3

Sixth edition of the GANP

Main Purpose 	Climatological information in support of the design and planning of infrastructure, flight routes and airspace management. Historical meteorological observations, forecasts, advisories and warnings in support of incident and accident investigations.

New Capabilities   Enhanced climatological data.

Description 		This element represents the provision of climatological information for the range of meteorological parameters and phenomena and their associated characteristics (metadata).

This element also represents the provision of historical information including meteorological observations and forecasts and their associated characteristics (metadata).

Climatological information services will be required to support the design and planning of infrastructure, flight routes and airspace management. The following climatology parameters and phenomena will begin to be made available to users and will include:

En-route winds

Airport parameters (i.e. air and surface temperature, wind, precipitation, etc.) Characteristics of the climatological information will include:

Averages (daily/monthly/yearly) over 10, 20, 30, 50 years

Extremes over 1, 5, 10, 20, 30 years, since start of measurement

Maturity Level 	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

PLANNING LAYERS

Post operations

OPERATIONS

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

AMET-B0/3 - Climatological and historical meteorological products

Relation-operational need

AMET-B1/1 - Meteorological observations information

ENABLERS

Enabler

Category

Enabler Type	Enabler Name

Description / References

Stakeholders	Year

Operational procedures

Information exchange

Procedures for the provision of climatological meteorological information

Procedures for changes to the provision of climatological and historical meteorological information, including en- route winds and enhanced information for the terminal area. References: • Doc.

| Airport operator |  |  |
| --- | --- | --- |
| ANSP | CAA |  |
| Aircraft operator |  |  |
| MET Information Service Provider |  |  |

8896 - Manual of Aeronautical Meteorological Practice

2019

MET

Information Service Provider

Training	-	Training requirements for climatological meteorological information

Training for meteorological personnel and aviation industry stakeholders on climatological and historical meteorological information. References: • WMO No. 1083 – Guide to the Implementation of Education and Training Standards in Meteorology and Hydrology • WMO No.1205 – Guide to Competency

2019

Regulatory provisions

National regulatory framework

National framework amendment for the provision of climatological meteorological information

Amendment to national regulations to include changes to the provision of climatological and historical meteorological information in support of the design and planning of infrastructure, flight routes and airspace management and to support incident and accident investigations. References: • Annex 3 - Meteorological Service for International Air Navigation • WMO No.49 Vol II - Technical Regulations - Basic Documents No. 2, Volume II – Meteorological Service for International Air Navigation • WMO No.49 Vol IV - Technical Regulations - Basic Documents No. 2, Volume IV – Quality Management • Procedures for Air Navigation Services – Meteorology (PANS-MET) – being developed

CAA 	2019

Ground system infrastructur e

Information exchange

Systems and infrastructure to support the provision of climatological meteorological information

IT systems and communications infrastructure for the storage, display and transmission (including AFS) of climatological meteorological information. References: • Doc. 8896 - Manual of Aeronautical Meteorological Practice

2019

MET

Information Service Provider

Sixth edition of the GANP

Information

Dissemination of meteorological information

AMET-B1/4

Main Purpose 	Dissemination of meteorological information in support of automated decision process or aids, involving meteorological information, meteorological information translation, ATM impact conversion and ATM decision support.

New Capabilities   Meteorological information in ICAO Meteorological Information Exchange Model (IWXXM) form starts to replace traditional alphanumeric code (TAC) products. Human-readable products will start to be derived from the IWXXM information (rather than the other way around). The introduction of web services allows for progressive replacement of fixed line dissemination systems.

Description 		This element represents the dissemination of meteorological products using a variety of formats, including:

Tailored products (human-readable)  Impact-translated products

Gridded

Graphical (PNG and BUFR to be phased out)

ICAO Meteorological Information Exchange Model (IWXXM) format  Traditional alphanumeric code (TAC) – being phased out

Dissemination means include aeronautical fixed service (ie. AMHS) and via secure internet services (ie. WIFS/SADIS). Commencement of SWIM-compliant web service capability to access the exact meteorological information required by users (in terms of geographical coverage, resolution etc).

Maturity Level 	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

PLANNING LAYERS

OPERATIONS

Does it imply a change to levels of automation? Yes

| Pre-tactical | Tactical-Pre ops |  | Tactical-During ops |
| --- | --- | --- | --- |
| Post operations |  |  |  |

| Taxi-out | Departure |  | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- | --- |
| Turn-around |  |  |  |  |  |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

AMET-B0/4 - Dissemination of meteorological products

Relation-operational need

AMET-B1/1 - Meteorological observations information

Relation-operational need

AMET-B1/2 - Meteorological forecast and warning information

Relation-technology need

COMI-B0/7 - ATS Message Handling System (AMHS)

Relation-information need

DAIM-B1/1 - Provision of quality-assured aeronautical data and information

Relation-information need

DAIM-B1/2 - Provision of digital Aeronautical Information Publication (AIP)

data sets

ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Information exchange

Procedures for the dissemination of meteorological information

Procedures for changes to the dissemination of meteorological information, including the exchange of meteorological information using the latest version of the ICAO Meteorological Information Exchange Model (IWXXM).

References: • Doc. 8896 - Manual of Aeronautical Meteorological Practice • Doc. 9377 – Manual on the Coordination between Air Traffic Services, Aeronautical Information Services and Aeronautical Meteorological Services • Doc. 9855 - Guidelines on the use of the Public Internet for Aeronautical Applications • Doc. 9880

- Manual on Detailed Technical Specifications for the Aeronautical Telecommunication Network (ATN) using ISO/OSI Standards and Protocols • Doc. 9896 – Manual on the Aeronautical Telecommunication Network (ATN) using Internet Protocol Suite (IPS) Standards and Protocol • Doc. 10003 - Manual on the Digital Exchange of Aeronautical Meteorological Information • Doc. 10039 - Manual on System Wide Information Management (SWIM) Concept • ICAO Guidelines for the Implementation of OPMET Data Exchange using IWXXM • WMO No.731 – Guide to Meteorological Observing and Information Distribution Systems for Aviation Weather Services • Regional OPMET Interface Control Documents • Regional OPMET Bulletin Exchange Handbooks

ANSP

MET

Information Service Provider

2019

Ground system infrastructur e

Information exchange

Communication infrastructure for meteorological information exchange

Ground system infrastructure and dissemination systems for meteorological information, including the implementation of the Aeronautical Message Handling System (AMHS). References: • Doc.

9880 - Manual on Detailed Technical Specifications for the Aeronautical Telecommunication Network (ATN) using ISO/OSI Standards and Protocols • Doc. 9896 – Manual on the Aeronautical Telecommunication Network (ATN) using Internet Protocol Suite (IPS) Standards and Protocol; • Regional OPMET Interface Control Documents • Regional OPMET Bulletin Exchange Handbooks

ANSP

MET

Information Service Provider

2019

Information exchange model

Meteorologic al information

ICAO

Meteorological Information Exchange Model (IWXXM) V3

ICAO Meteorological Information Exchange Model (IWXXM) schema used to enable the provision meteorological information in XML/GML form. Version 3 supporting Annex 3 amendment 78

(2018). References: • Doc. 10003 - Manual on the Digital Exchange of Aeronautical Meteorological Information • ICAO Guidelines for the Implementation of OPMET Data Exchange using IWXXM • WMO IWXXM Wiki:

https://wiswiki.wmo.int/tiki-index.php? page=IWXXM&structure=WIS+up

ANSP

MET

Information Service Provider

2019

Training	-	Training for the dissemination of meteorological information

Training for meteorological and aviation IT personnel, including NOC/ROC/RODB/IROG personnel on the exchange of meteorological information. References: • Doc. 10003 - Manual on the Digital Exchange of Aeronautical Meteorological Information • ICAO Guidelines for the Implementation of OPMET Data Exchange using IWXXM • Regional OPMET Interface Control Documents • Regional OPMET Bulletin Exchange Handbooks

ANSP

MET

Information Service Provider

2019

Regulatory provisions

National regulatory framework

National framework amendment for the dissemination of meteorological information

Amendment to national regulations to include changes to the dissemination of meteorological information in support of automated decision process or aids, involving meteorological information, meteorological information translation, ATM impact conversion and ATM decision support. References: • Annex 3 - Meteorological Service for International Air Navigation • WMO No.49 Vol II - Technical Regulations - Basic Documents No. 2, Volume II – Meteorological Service for International Air Navigation • WMO No.49 Vol IV - Technical Regulations - Basic Documents No. 2, Volume IV – Quality Management • Procedures for Air Navigation Services – Meteorology (PANS-MET) – being developed • Annex 10 – Aeronautical Telecommunications • Annex 15 - Aeronautical Information Services

CAA 	2019

Information

Meteorological observations information

AMET-B2/1

Sixth edition of the GANP

Main Purpose 		Integrated meteorological observations in support of enhanced ATM and airport decision-making processes, particularly in the near-term.

New Capabilities   Further development of space weather observation services. Further development of services for terminal areas. Implementation of information services to support a data-centric environment. Higher spatial and temporal resolution of meteorological observations. Automated observations which will support user-defined services using meteorological information in ICAO Meteorological Information Exchange Model (IWXXM) form.

Description 		This module builds on the meteorological information services defined in AMET-B1. Full MET-ATM integration will ensure that meteorological information is included in the logic of a decision process and the impact of the meteorological conditions on the operations are automatically derived, understood and taken into account. Increased situational awareness to support tactical in-flight avoidance of hazardous meteorological conditions.

Recognising that space weather affecting the earth’s surface or atmosphere pose a hazard to communications and navigation systems and may also pose a radiation risk to flight crew members and passengers, this module builds on AMET-B1 for space weather information services in support of safe and efficient international air navigation, particularly in the area of regional services.

Considering the impact of adverse meteorological conditions on high traffic density terminal areas around major aerodromes and air traffic management operations within those areas as well as the increasing requirements by airspace users to be able to optimise their flight profile, this module acknowledges the need to provide meteorological information services with the accuracy, resolution and frequency to support ATM operations within those areas.

Meteorological observations will increasingly include performance measurement (via compliance, availability and regularity indices).

It is assumed that aircraft will be equiped for meteorological information display capabilities, such as EFBs. Taking advantage of enhanced aircraft connectivity to maximise observation functionality of aircraft.

Maturity Level 	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

OPERATIONS

Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Post operations

Tactical-During ops

Tactical-Pre ops

| Taxi-out | Departure |  | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- | --- |
| Turn-around |  |  |  |  |  |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

AMET-B1/1 - Meteorological observations information

Relation-technology benefit

ASUR-B2/1 - Evolution of ADS-B and Mode S

Relation-technology option

COMS-B2/2 - PBCS Approved ADS-C (B2) for domestic and procedural

airspace

Relation-technology option

SWIM-B2/4 - Air/Ground SWIM for non-safety critical information

Relation-technology need

SWIM-B2/1 - Information service provision

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

National regulatory framework

National framework amendment for the provision of meteorological observations information in a SWIM

environment

Amendment of national regulations to include the provision of integrated meteorological observations information in a SWIM environment in support of enhanced operational ground and air decision-making processes, particularly in the planning phase and near-term.

References: • Annex 3 - Meteorological Service for International Air Navigation • WMO No.49 Vol II - Technical Regulations

- Basic Documents No. 2, Volume II – Meteorological Service for International Air Navigation • WMO No.49 Vol IV - Technical Regulations - Basic Documents No. 2, Volume IV – Quality Management • Procedures for Air Navigation Services – Meteorology (PANS-MET) – being developed • Doc. 4444 - Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM)

CAA 	2025

Operational procedures

Information exchange

Procedures for the provision of meteorological observations information in a SWIM

environment

Procedures for changes to the provision of integrated meteorological observations information in a SWIM environment, including enhanced terminal area, space weather, sulphur dioxide (SO2) and radioactive material services. References:

- WMO No.8 - Guide to Meteorological Instruments and Methods of Observation • WMO No.306 - Manual on Codes – International Codes • WMO No.731 – Guide to Meteorological Observing and Information Distribution Systems for Aviation Weather Services • WMO No.732 - Guide to Practices for Meteorological Offices Serving Aviation • WMO No.782 – Aerodrome Reports and Forecasts • WMO No.1100 - Guide to the Implementation of a Quality Management System for National Meteorological and Hydrological Services • Doc. 7488 – Manual of the ICAO Standard Atmosphere

Aircraft manufacturer

Doc. 8896 - Manual of Aeronautical Meteorological Practice • Doc. 9328 - Manual of Runway Visual Range Observing and Reporting Practices • Doc 9837 - Manual on Automatic Meteorological Observing Systems at Aerodromes • Doc. 10100 – Manual on Space Weather Information in Support of Air Navigation

ANSP

MET

Information Service Provider

2025

Airborne system capability

Aircraft system

Transmission of meteorological observations information from aircraft for a SWIM

environment

On-board sensors and communication means for the measurement and transmission of meteorological information by: ACAR, Mode-S, ADS-B & C.

2025

Ground system infrastructur e

Information exchange

Automated systems and infrastructure to support the provision of meteorological observations information over SWIM

Meteorological instrumentation, IT systems and communications infrastructure for the measurement, storage, display and transmission of meteorological observations information to support ground and air decision-making processes in a SWIM environment References: • WMO No.8 - Guide to Meteorological Instruments and Methods of Observation • Annex 10 - Aeronautical Telecommunications

Airport operator

ANSP

MET

Information Service Provider

2025

| Airport operator |  |  |
| --- | --- | --- |
| ANSP | CAA |  |
| Aircraft operator |  |  |
| MET Information Service Provider |  |  |

Training

-

Training

requirements for meteorological observations information over SWIM

Training for meteorological personnel and

aviation industry stakeholders on meteorological observations information in a SWIM environment. References: • WMO No. 1083 – Guide to the Implementation of Education and Training Standards in Meteorology and Hydrology • WMO No.1205 – Guide to Competency

2025

AMET-B2/2	Meteorological forecast and warning information	Information

Sixth edition of the GANP

Main Purpose		Integrated meteorological forecast and warning information in support of enhanced operational ground and air decision-making processes, particularly in the near-term.

New Capabilities		Further development of space weather services. Further development of forecast and warning services for terminal areas. Phenomena-based meteorological information is no longer constrained by Flight Information Regions (FIRs). Implementation of a data-centric information set. Higher spatial and temporal resolution of meteorological forecasts and warnings. Automated forecast and warning services derived from meteorological information in ICAO Meteorological Information Exchange Model (IWXXM) form. Further development of probabilistic information derived from ensemble prediction systems and how this type of information can be presented or integrated into user's decision processes.

Description 			This module builds on the meteorological forecast and warning services defined in AMET-B1. Full MET-ATM integration will ensure that meteorological information is included in the logic of a decision process and the impact of the meteorological conditions on the operations are automatically derived, understood and taken into account. Support for tactical in-flight avoidance of hazardous meteorological conditions (improved in-flight situational awareness) to typical near-term and planning (>20 minutes) type of decision making. Specifically, the addition of probabilistic forecasts will provide decision makers with an assessment of the likelihood of some meteorological phenomena/parameters exceeding a defined magnitude (or threshold) at a particular time and place. The probabilistic element further helps decision makers apply their own operational constraints (i.e. business rules) to determine the risk to their operations.

Recognising that space weather affecting the earth’s surface or atmosphere pose a hazard to communications and navigation systems and may also pose a radiation risk to flight crew members and passengers, this module builds on AMET-B1 for space weather information services in support of safe and efficient international air navigation, particularly in the area of regional services.

Considering the impact of adverse meteorological conditions on high traffic density terminal areas around major aerodromes and air traffic management operations within those areas as well as the increasing requirements by airspace users to be able to optimise their flight profile, this module acknowledges the need to provide meteorological information services with the accuracy, resolution and frequency to support ATM operations within those areas.

A significant evolution is planned for volcanic ash information. Next generation volcanic ash cloud forecasts will be fully implemented. It will allow decision makers to use both deterministic and probabilistic forecasts for contamination levels, taking into account their risk management practices

and the quantitative exposures allowed by the engine manufacturers. Specifically, the addition of probabilistic forecasts will provide decision makers with an assessment of the likelihood of the volcanic ash exceeding a defined magnitude (or threshold) at a particular time and place. The probabilistic element further helps decision makers apply their own operational constraints (i.e. business rules) to determine the risk to their operations.

Enhanced global MET forecasts will be provided under the World Area Forecast System (WAFS), which will include higher resolution and probabilistic information. Enhanced higher resolution regional MET forecasts will also be provided. Forecast services for the terminal area will be further enhances with the accuracy, resolution and frequency to support ATM operations within those areas.

Evolving the nature of enroute hazard forecasting into the future SWIM environment, will require the development of new services, which is planned for this module. These will be supported by new forecasting methodologies to ensure global and regional consistency and will involve a harmonised and coordinated approach between MET service providers. This module will build and demonstrate the concept, based on defined user requirements, as well as design the infrastructure and system architecture.

This enhanced global, regional and terminal area information will be integrated into flight planning, flight management and ATM decision support systems, including systems for air traffic control around and at airports.

Meteorological forecast and warning parameters and phenomena will include performance measurement (via compliance, availability and regularity indices) of some meteorological observations. Greater use of the probability of occurrence of meteorological phenomena and the level of confidence/uncertainty of the forecast will enable better risk management.

Human-readable products can be derived from the meteorological information to suit specific user needs. Meteorological advisories and warnings are derived from forecast data and based on user- defined thresholds.

Maturity Level 		Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

OPERATIONS

Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

Tactical-Pre ops

Pre-tactical

| Taxi-out | Departure |  | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- | --- |
| Turn-around |  |  |  |  |  |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

AMET-B1/2 - Meteorological forecast and warning information

Relation-operational need

AMET-B2/1 - Meteorological observations information

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

National regulatory framework

National framework amendment for the provision of meteorological forecast and warnings information in a SWIM

environment

Amendment of national regulations to include the provision of integrated meteorological forecast and warning information in a SWIM environment, in support of enhanced operational ground and air decision-making processes, particularly in the planning phase and near-term. References: • Annex 3 - Meteorological Service for International Air Navigation • WMO No.49 Vol II - Technical Regulations - Basic Documents No. 2, Volume II – Meteorological Service for International Air Navigation • WMO No.49 Vol IV - Technical Regulations - Basic Documents No. 2, Volume IV – Quality Management • Procedures for Air Navigation Services – Meteorology (PANS-MET) – being developed • Doc. 4444 - Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM)

CAA 	2025

Operational procedures

Information exchange

Procedures for the provision of meteorological forecast and warnings information in a SWIM

environment

Procedures for changes to the provision of  ANSP

MET

Information Service Provider

integrated meteorological forecast and warning information in a SWIM environment, including enhanced terminal area, space weather, sulphur dioxide (SO2) and radioactive material services. References: • WMO No.306 - Manual on Codes – International Codes • WMO No.732 - Guide to Practices for Meteorological Offices Serving Aviation • WMO No.782 – Aerodrome Reports and Forecasts • WMO No.1100 - Guide to the Implementation of a Quality Management System for National Meteorological and Hydrological Services • Doc. 7488 – Manual of the ICAO Standard Atmosphere

- Doc. 8896 - Manual of Aeronautical Meteorological Practice • Doc. 9691 - Manual on Volcanic Ash, Radioactive Material and Toxic Chemical Clouds • Doc. 9766 - Handbook on the International Airways Volcano Watch (IAVW) • Doc. 10100 – Manual on Space Weather Information in Support of Air Navigation • Regional SIGMET Guides

2025

Training	-	Training requirements for Meteorological forecast and warnings information in a SWIM

environment

Training for meteorological personnel and aviation industry stakeholders on meteorological forecast and warning information in a SWIM environment.

MET

Information Service Provider

References: • WMO No. 1083 – Guide to the Implementation of Education and Training Standards in Meteorology and Hydrology • WMO No.1205 – Guide to Competency

2025

Ground system infrastructur e

Information exchange

Systems and infrastructure to support the provision of meteorological forecast and warning information in a SWIM

environment

IT systems and communications infrastructure for the storage, forecasting, display and transmission of meteorological forecast and warning information in a SWIM environment.

References: • Doc. 8896 - Manual of Aeronautical Meteorological Practice

2025

AMET-B2/3	Climatological and historical meteorological

information

Information

| Airport operator |  |  |
| --- | --- | --- |
| ANSP | CAA |  |
| Aircraft operator |  |  |
| MET Information Service Provider |  |  |

Sixth edition of the GANP

Main Purpose	Climatological products (historical and forecast) in support of the design and planning of infrastructure, flight routes and airspace management. Historical meteorological observations, forecasts, advisories and warnings in support of incident and accident investigations.

New Capabilities	Climatological data (including satellite-based and in-situ climatological data, and a combination of the two) and climate change information available for more locations and more frequently updated.

Description		This element represents the provision of climatological information for the range of meteorological parameters and phenomena and their associated characteristics (metadata). This element also represents the provision of historical information including meteorological observations and forecasts and their associated characteristics (metadata). This element also includes the provision of information on climate change and its impact on aviation, such as upper air wind flow changes, turbulence patterns evolutions, airport temperature and wind future evolutions, etc.

Climatological information services will support the design and planning of infrastructure, flight routes and airspace management.

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

PLANNING LAYERS

Post operations

OPERATIONS

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

AMET-B1/3 - Climatological and historical meteorological information

Relation-operational need

AMET-B2/1 - Meteorological observations information

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

National regulatory framework

National framework amendment for the provision of climatological meteorological information in a SWIM

environment

Amendment of national regulations to include changes to the provision of climatological information, both historical and forecast, in a SWIM environment, in support of the design and planning of infrastructure, flight routes and airspace management and to support incident and accident investigations. References: • Annex 3 - Meteorological Service for International Air Navigation • WMO No.49 Vol II - Technical Regulations - Basic Documents No. 2, Volume II – Meteorological Service for International Air Navigation • WMO No.49 Vol IV - Technical Regulations - Basic Documents No. 2, Volume IV – Quality Management • Procedures for Air Navigation Services – Meteorology (PANS-MET) – being developed

CAA 	2025

MET

Information Service Provider

Operational procedures

Information exchange

Procedures for the provision of climatological meteorological information in a SWIM

environment

Procedures for changes to the provision of climatological information, both historical and forecast, in a SWIM environment, including satellite-based and in-situ climatological data and climate change forecasts. References: • Doc. 8896 - Manual of Aeronautical Meteorological Practice

2025

Training	-	Training for the provision of climatological meteorological information in a SWIM

environment

Training for meteorological personnel and aviation industry stakeholders on climatological information, both historical and forecast, in a SWIM environment.

MET

Information Service Provider

References: • WMO No. 1083 – Guide to the Implementation of Education and Training Standards in Meteorology and Hydrology • WMO No.1205 – Guide to Competency

2025

Ground system infrastructur e

Information exchange

Systems and infrastructure to support the provision of climatological meteorological information in a SWIM

environment

IT systems and communications infrastructure for the storage, forecasting, display and transmission of climatological meteorological information in a SWIM environment. References: • Doc. 8896 - Manual of Aeronautical Meteorological Practice

2025

| Airport operator |  |  |
| --- | --- | --- |
| ANSP | CAA |  |
| Aircraft operator |  |  |
| MET Information Service Provider |  |  |

Sixth edition of the GANP

Main Purpose

Integrated meteorological information service in the SWIM environment in support of enhanced

operational ground and air decision-making processes, particularly in the planning phase and near- term.

New Capabilities	Implementation of data-centric meteorological information services, integrated into the System Wide

Information Management (SWIM) environment. User-defined services derived from meteorological information in ICAO Meteorological Information Exchange Model (IWXXM) form. Wider use of secure web services and message brokers as part of the transition from fixed line dissemination systems. Commencement of the use of business-to-business services that allows integration of meteorological information into ATM systems.

Information

Meteorological information service in SWIM

AMET-B2/4

Description 		The establishment of standards for global exchange of the MET information within the SWIM environment.

This element represents the integration of meteorological information into the SWIM environment. Wider use of MET-SWIM services will support flexible airspace management, airborne re-routing, improved situational awareness, collaborative decision-making, including in terminal areas and at airports, dynamically optimized flight trajectory planning, ATM impact conversion and ATM decision support, hazard avoidance.

In coordination with stakeholders, the development of guidance on visualisation of some MET information, where deemed appropriate.

SWIM-compliant meteorological information to be more readily exchanged with the aircraft to improve operational awareness and decision making using air/ground data connectivity and aircraft on-board systems.

MET-SWIM information services will support request/reply or publish/subscribe access mechanisms and will provide quality & timely information to users in a range of formats to best enable their optimal decision making.

Maturity Level 	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

PLANNING LAYERS

OPERATIONS

Does it imply a change to levels of automation? Yes

| Pre-tactical | Tactical-Pre ops |  | Tactical-During ops |
| --- | --- | --- | --- |
| Post operations |  |  |  |

| Taxi-out | Departure |  | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- | --- |
| Turn-around |  |  |  |  |  |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

AMET-B1/1 - Meteorological observations information

Relation-operational need

AMET-B2/1 - Meteorological observations information

Relation-operational need

AMET-B2/2 - Meteorological forecast and warning information

Relation-information need

DAIM-B2/1 - Dissemination of aeronautical information in a SWIM

environment

Relation-operational need

SWIM-B2/1 - Information service provision

Relation-operational need

SWIM-B2/2 - Information service consumption

Relation-operational need

SWIM-B2/3 - SWIM registry

Relation-operational benefit

SWIM-B2/4 - Air/Ground SWIM for non-safety critical information

Relation-operational benefit

SWIM-B2/5 - Global SWIM processes

Relation-operational need

AMET-B2/3 - Climatological and historical meteorological information

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

National regulatory framework

National framework amendment for the provision of meteorological information service in SWIM

Amendment to national regulations to include changes to the dissemination of meteorological information in support of enhanced operational ground and air decision-making processes, particularly in the planning phase and near-term.

References: • Annex 3 - Meteorological Service for International Air Navigation • WMO No.49 Vol II - Technical Regulations

- Basic Documents No. 2, Volume II – Meteorological Service for International Air Navigation • WMO No.49 Vol IV - Technical Regulations - Basic Documents No. 2, Volume IV – Quality Management • Procedures for Air Navigation Services – Meteorology (PANS-MET) – being developed • Annex 10 – Aeronautical Telecommunications • Annex 15 - Aeronautical Information Services

CAA 	2025

Operational procedures

Information exchange

Procedures for the provision of meteorological information service in SWIM

Procedures for changes to the dissemination of meteorological information, including the exchange of meteorological information using the latest version of the ICAO Meteorological Information Exchange Model (IWXXM) and the introduction of web services.

References: • Doc. 8896 - Manual of Aeronautical Meteorological Practice • Doc. 9377 – Manual on the Coordination between Air Traffic Services, Aeronautical Information Services and Aeronautical Meteorological Services • Doc. 9855 - Guidelines on the use of the Public Internet for Aeronautical Applications • Doc. 9880

- Manual on Detailed Technical Specifications for the Aeronautical Telecommunication Network (ATN) using ISO/OSI Standards and Protocols • Doc. 9896 – Manual on the Aeronautical Telecommunication Network (ATN) using Internet Protocol Suite (IPS) Standards and Protocol • Doc. 10003 - Manual on the Digital Exchange of Aeronautical Meteorological Information • Doc. 10039 - Manual on System Wide Information Management (SWIM) Concept • ICAO Guidelines for the Implementation of OPMET Data Exchange using IWXXM • WMO No.731 – Guide to Meteorological Observing and Information Distribution Systems for Aviation Weather Services • Regional OPMET Interface Control Documents • Regional OPMET Bulletin Exchange Handbooks

ANSP

MET

Information Service Provider

2025

Ground system infrastructur e

Information exchange

Communication infrastructure for meteorological information exchange in SWIM

Ground system infrastructure and dissemination systems for meteorological information within a SWIM environment. References: • Regional OPMET Interface Control Documents • Regional OPMET Bulletin Exchange Handbooks

ANSP

MET

Information Service Provider

2025

Information exchange model

Meteorologic al information

ICAO

Meteorological Information Exchange Model (IWXXM) Vxx

ICAO Meteorological Information Exchange Model (IWXXM) schema used to enable the provision meteorological information in XML/GML form.

References: • Doc. 10003 - Manual on the Digital Exchange of Aeronautical Meteorological Information • ICAO Guidelines for the Implementation of OPMET Data Exchange using IWXXM • WMO IWXXM Wiki:

https://wiswiki.wmo.int/tiki-index.php? page=IWXXM&structure=WIS+up

ANSP

MET

Information Service Provider

2025

Training

-

Training for the

provision of a Meteorological information service

Training for meteorological and aviation IT  ANSP

personnel, including NOC/ROC/RODB/IROG and SWIM

services personnel, on the exchange of meteorological information in a SWIM environment. References: • Doc. 10003 - Manual on the Digital Exchange of Aeronautical Meteorological Information • Doc. 10039 - Manual on System Wide Information Management (SWIM) Concept

- ICAO Guidelines for the Implementation of OPMET Data Exchange using IWXXM • Regional OPMET Interface Control Documents • Regional OPMET Bulletin

Exchange Handbooks

2025

MET

Information Service Provider

AMET-B3/1	Meteorological observations information	Information

Sixth edition of the GANP

Main Purpose	Integrated high resolution meteorological observation information in support of enhanced operational ground and air decision-making processes, for all flight phases and corresponding air traffic control operations, allowing gate-to-gate seamless operations.

New Capabilities		Further development of space weather information service and of observation services for terminal areas. Higher spatial and temporal resolution of meteorological observations.

Observations to support tactical routing decisions under environmental considerations i.e. contrail or noise avoidance, if required.

Description 		This module builds on the meteorological information services defined in AMET-B2. Full MET-ATM integration will ensure that high resolution meteorological observation information is included in the logic of a decision process and the impact of the meteorological conditions on the operations are automatically derived, understood and taken into account. Increased situational awareness to support tactical in-flight avoidance of hazardous meteorological conditions.

Recognising that space weather affecting the earth’s surface or atmosphere poses a hazard to communications and navigation systems especially satellite-based systems and may also pose a radiation risk to flight crew members and passengers, this module builds on AMET-B2 for space weather information services in support of safe and efficient international air navigation.

Considering the impact of adverse meteorological conditions on high traffic density terminal areas around major aerodromes and air traffic management operations within those areas as well as the increasing requirements by airspace users to be able to optimise their flight profile, this module acknowledges the need to provide meteorological observation information services with the accuracy, resolution and frequency to support ATM operations within those areas.

Meteorological observations will increasingly include performance measurement (via compliance, availability and regularity indices).

Applicable to air traffic flow planning, en-route operations, terminal operations (arrival/departure) and surface.

Aircraft equipage is assumed in the areas of ADS-B IN/CDTI, aircraft based meteorological observations, and meteorological information display capabilities, such as EFBs.

Maturity Level 	Concept

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

OPERATIONS

Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Post operations

Tactical-During ops

Tactical-Pre ops

| Taxi-out | Departure |  | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- | --- |
| Turn-around |  |  |  |  |  |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

AMET-B2/1 - Meteorological observations information

Relation-technology need

COMS-B3/1 - Extended CPDLC (B2 incl. Adv-IM and dynamic RNP) for

dense and complex airspace

Relation-technology need

COMS-B3/2 - Extended ADS-C (B2 incl. Adv-IM and dynamic RNP) for

dense and complex airspace

Relation-operational need

SWIM-B2/1 - Information service provision

Relation-technology need

ASUR-B2/1 - Evolution of ADS-B and Mode S

Relation-operational need

SWIM-B3/1 - Air/Ground SWIM for safety critical information

### ENABLERS

| Airport operator |  |  |
| --- | --- | --- |
| ANSP | CAA |  |
| Aircraft operator |  |  |
| MET Information Service Provider |  |  |

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

MET

Information Service Provider

ANSP

Training	-	Training requirements for meteorological observations information service

References: • WMO No. 1083 – Guide to the Implementation of Education and Training Standards in Meteorology and Hydrology • WMO No.1205 – Guide to Competency

2031

Ground system infrastructur e

Information exchange

Automated systems and infrastructure to support the provision of meteorological observations information service over SWIM

Meteorological instrumentation Communications infrastructure (including AFS) Data exchange infrastructure supporting SWIM References:

2031

Aircraft manufacturer

Airborne system capability

Aircraft system

Transmission of meteorological observations from aircraft for a SWIM

environment

Transmission of meteorological information by: ACAR, Mode-S, ADS-B & C

2031

Regulatory provisions

National regulatory framework

National framework amendment for the provision of meteorological observations information service

Amendment of national regulations to include the provision of integrated high- resolution meteorological observations information in support of enhanced operational ground and air decision- making processes, for all flight phases and corresponding air traffic control operations, allowing gate-to-gate seamless operations. References: • Annex 3 - Meteorological Service for International Air Navigation • WMO No.49 Vol II - Technical Regulations - Basic Documents No. 2, Volume II – Meteorological Service for International Air Navigation • WMO No.49 Vol IV - Technical Regulations - Basic Documents No. 2, Volume IV – Quality Management • Procedures for Air Navigation Services – Meteorology (PANS-MET) – being developed • Doc. 4444 - Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM)

CAA 	2031

Operational procedures

Information exchange

Procedures for the provision of meteorological observations information service

Procedures for changes to the provision of  ANSP  integrated high-resolution meteorological observations information, including space

MET

Information Service Provider

weather and services for the terminal area. References: • WMO No.8 - Guide to Meteorological Instruments and Methods of Observation • WMO No.306 - Manual on Codes – International Codes • WMO No.731 – Guide to Meteorological Observing and Information Distribution Systems for Aviation Weather Services • WMO No.732 - Guide to Practices for Meteorological Offices Serving Aviation • WMO No.782 – Aerodrome Reports and Forecasts • WMO No.1100 - Guide to the Implementation of a Quality Management System for National Meteorological and Hydrological Services • Doc. 7488 – Manual of the ICAO Standard Atmosphere

- Doc. 8896 - Manual of Aeronautical Meteorological Practice • Doc. 9328 - Manual of Runway Visual Range Observing and Reporting Practices • Doc 9837 - Manual on Automatic Meteorological Observing Systems at Aerodromes • Doc. 10100 – Manual on Space Weather Information in Support of Air Navigation

2031

AMET-B3/2	Meteorological forecast and warning information	Information

Sixth edition of the GANP

Main Purpose	Integrated high resolution meteorological forecast and warning information in support of enhanced operational ground and air decision-making processes, for all flight phases and corresponding air traffic control operations, allowing gate-to-gate seamless operations.

New Capabilities	Further development of space weather information service and of forecast and warning services for terminal areas. Higher spatial and temporal resolution of meteorological forecasts and warnings.

Further development of probabilistic forecast information. Further development towards a fully integrated meteorological forecast service fit for the purpose of all flight phases and ATC operations, in support of gate-to-gate seamless operations.

Description		This module builds on the meteorological forecast and warning services defined in AMET-B2. Full MET-ATM integration will ensure that meteorological information is included in the logic of a decision process for each flight phase and the impact of the meteorological conditions on the operations are automatically derived, understood and taken into account.

The probabilistic element of the forecasts and warnings further helps decision makers apply their own operational constraints (i.e. business rules) to determine the risk to their operations. Greater use of the probability of occurrence of meteorological phenomena and the level of confidence/uncertainty of the forecast will enable better risk management.

Enroute SWIM services that are produced collaboratively (between MET providers) will provide tailored information on hazards.

Greater use of forecast information from ensemble prediction systems (multiple scenarios) will enable ATM stakeholders to consider multiple air traffic scenarios from air traffic management systems, flight management systems and airport management systems, and to derive uncertainty information on air traffic situation.

Meteorological forecast and warning parameters and phenomena will be made available to users only through SWIM information services.

Maturity Level	Concept

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS

Pre-tactical

Tactical-Pre ops

Tactical-During ops

DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

### OPERATIONS

| Taxi-out | Departure |  | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- | --- |
| Turn-around |  |  |  |  |  |

Evolution	AMET-B2/2 - Meteorological forecast and warning information

Relation-operational need

AMET-B3/1 - Meteorological observations information

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

National regulatory framework

National framework amendment for the provision of meteorological forecast and warnings information service

Amendment of national regulations to include the provision of integrated high- resolution meteorological forecast and warning information in support of enhanced operational ground and air decision-making processes, for all flight phases and corresponding air traffic control operations, allowing gate-to-gate seamless operations. References: • Annex 3 - Meteorological Service for International Air Navigation • WMO No.49 Vol II - Technical Regulations - Basic Documents No. 2, Volume II – Meteorological Service for International Air Navigation • WMO No.49 Vol IV - Technical Regulations - Basic Documents No. 2, Volume IV – Quality Management • Procedures for Air Navigation Services – Meteorology (PANS-MET) – being developed • Doc. 4444 - Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM)

CAA 	2031

Operational procedures

Information exchange

Procedures for the provision of meteorological forecast and warnings information service

Procedures for changes to the provision of  ANSP

MET

Information Service Provider

integrated high-resolution meteorological forecast and warning information, including space weather and services for the terminal area. References: • WMO No.306 - Manual on Codes – International Codes • WMO No.732 - Guide to Practices for Meteorological Offices Serving Aviation • WMO No.782 – Aerodrome Reports and Forecasts • WMO No.1100 - Guide to the Implementation of a Quality Management System for National Meteorological and Hydrological Services • Doc. 7488 – Manual of the ICAO Standard Atmosphere

- Doc. 8896 - Manual of Aeronautical Meteorological Practice • Doc. 9691 - Manual on Volcanic Ash, Radioactive Material and Toxic Chemical Clouds • Doc. 9766 - Handbook on the International Airways Volcano Watch (IAVW) • Doc. 10100 – Manual on Space Weather Information in Support of Air Navigation • Regional SIGMET Guides

2031

| Airport operator |  |  |
| --- | --- | --- |
| ANSP | CAA |  |
| Aircraft operator |  |  |
| MET Information Service Provider |  |  |

Training

-

Training

requirements for meteorological forecast and warnings information service

Training for meteorological personnel and

aviation industry stakeholders on meteorological forecast and warning information to support ground and air decision-making processes in a SWIM environment. References: • WMO No. 1083 – Guide to the Implementation of Education and Training Standards in Meteorology and Hydrology • WMO No.1205 – Guide to Competency

2031

Ground	Information

system	exchange infrastructur

e

Systems and

infrastructure to support the provision of meteorological forecast and warnings information service

IT systems and communications

infrastructure for the storage, forecasting, display and transmission of meteorological forecast and warnings.

References: • Doc. 8896 - Manual of

Aeronautical Meteorological Practice

2031

MET

Information Service Provider

AMET-B3/3	Climatological and historical meteorological

information

Information

Sixth edition of the GANP

Main Purpose	Climatological products in support of the design and planning of infrastructure, flight routes and airspace management. Historical meteorological observations, forecasts, advisories and warnings in support of incident and accident investigations.

New Capabilities	Climatological data and climate change information available for more locations and more frequently updated.

Description		This element represents the provision of climatological information for the range of meteorological parameters and phenomena and their associated characteristics (metadata). This element also represents the provision of historical information including meteorological observations and forecasts and their associated characteristics (metadata).

Climatological information services will support the design and planning of infrastructure, flight routes and airspace management.

Maturity Level	-

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Post operations

OPERATIONS

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

AMET-B2/3 - Climatological and historical meteorological information

Relation-operational need

AMET-B3/1 - Meteorological observations information

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

National regulatory framework

National framework amendment for the provision of a climatological meteorological information service

Amendment of national regulations to include changes to the provision of climatological information, in particular climate change forecasts, in in support of the design and planning of infrastructure, flight routes and airspace management and to support incident and accident investigations. References: • Annex 3 - Meteorological Service for International Air Navigation • WMO No.49 Vol II - Technical Regulations - Basic Documents No. 2, Volume II – Meteorological Service for International Air Navigation • WMO No.49 Vol IV - Technical Regulations - Basic Documents No. 2, Volume IV – Quality Management • Procedures for Air Navigation Services – Meteorology (PANS-MET) – being developed

CAA 	2031

Operational procedures

Information exchange

Procedures for the provision of a climatological meteorological information service

Procedures for changes to the provision of a climatological information, in particular climate change forecasts. References: • Doc. 8896 - Manual of Aeronautical Meteorological Practice

2031

MET

Information Service Provider

Training	-	Training requirements for a climatological meteorological information service

Training for meteorological personnel and aviation industry stakeholders on climatological information, in particular climate change forecasts, and its impact on aviation. References: • WMO No.

1083 – Guide to the Implementation of Education and Training Standards in Meteorology and Hydrology • WMO No.1205 – Guide to Competency

Airport operator 2031

ANSP

Aircraft operator

MET

Information Service Provider

Ground	Information

system	exchange infrastructur

e

Systems and

infrastructure to support the provision of a climatological meteorological service

IT systems and communications

infrastructure for the storage, forecasting, display and transmission of climatological meteorological service. References: • Doc. 8896 - Manual of Aeronautical Meteorological Practice

2031

MET

Information Service Provider

AMET-B3/4	Meteorological information service in SWIM	Information

Sixth edition of the GANP

Main Purpose	Integrated meteorological information service in the SWIM environment in support of enhanced operational ground and air decision-making processes, for all flight phases and corresponding air traffic control operations.

New Capabilities	Implementation of a data-centric meteorological information service, integrated into the System Wide Information Management (SWIM) environment. Enhancement of ICAO Meteorological Information Exchange Model (IWXXM) with further schemas and formats for meteorological information exchange. User-defined products automatically derived from meteorological information in ICAO Meteorological Information Exchange Model (IWXXM) form. Extensive use of secure web services, in particular business-to-business services that allows full integration of meteorological information

Description		The establishment of standards for global exchange of the MET information within the SWIM environment.

This element represents the full integration of meteorological information into the System Wide Information Management (SWIM) environment. Extensive use of MET-SWIM services will support flexible airspace management, airborne re-routing, improved situational awareness, collaborative decision-making, including in terminal areas and at airports, dynamically optimized flight trajectory planning, ATM impact conversion and ATM decision support, hazard avoidance.

Meteorological information to be more readily exchanged with the aircraft to improve operational awareness and decision making using air/ground data connectivity and aircraft on-board systems.

MET-SWIM information services will support request/reply or publish/subscribe access mechanisms and will provide quality & timely information to users in a range of formats to best enable their optimal decision making.

Maturity Level	Concept

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS 		OPERATIONS

| Pre-tactical | Tactical-Pre ops |  | Tactical-During ops |
| --- | --- | --- | --- |
| Post operations |  |  |  |

| Taxi-out | Departure |  | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- | --- |
| Turn-around |  |  |  |  |  |

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Evolution	AMET-B2/4 - Meteorological information service in SWIM Relation-operational need	AMET-B3/1 - Meteorological observations information Relation-operational need	AMET-B3/2 - Meteorological forecast and warning information Relation-operational need	SWIM-B3/1 - Air/Ground SWIM for safety critical information

Relation-operational need	AMET-B3/3 - Climatological and historical meteorological information Relation-operational need	SWIM-B2/1 - Information service provision

Relation-operational need	SWIM-B2/2 - Information service consumption Relation-operational need	SWIM-B2/3 - SWIM registry

Relation-operational need	SWIM-B2/4 - Air/Ground SWIM for non-safety critical information Relation-operational need	SWIM-B2/5 - Global SWIM processes

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

National regulatory framework

National framework amendment for the provision of a meteorological information service

Amendment to national regulations to include changes to the dissemination of meteorological information in SWIM to support of enhanced operational ground and air decision-making processes, for all flight phases and corresponding air traffic control operations, allowing gate-to-gate seamless operations. References: • Annex 3 - Meteorological Service for International Air Navigation • WMO No.49 Vol II - Technical Regulations - Basic Documents No. 2, Volume II – Meteorological Service for International Air Navigation • WMO No.49 Vol IV - Technical Regulations - Basic Documents No. 2, Volume IV – Quality Management • Procedures for Air Navigation Services – Meteorology (PANS-MET) – being developed • Annex 10 – Aeronautical Telecommunications • Annex 15 - Aeronautical Information Services

CAA 	2031

MET

Information Service Provider

Operational procedures

Information exchange

Procedures for the provision of a meteorological information service

Procedures for changes to the dissemination of meteorological information in SWIM, including the exchange of meteorological information using the latest version of the ICAO Meteorological Information Exchange Model (IWXXM). References: • Doc. 8896 - Manual of Aeronautical Meteorological Practice • Doc. 9377 – Manual on the Coordination between Air Traffic Services, Aeronautical Information Services and Aeronautical Meteorological Services • Doc. 9855 - Guidelines on the use of the Public Internet for Aeronautical Applications • Doc. 9880 - Manual on Detailed Technical Specifications for the Aeronautical Telecommunication Network (ATN) using ISO/OSI Standards and Protocols • Doc. 9896 – Manual on the Aeronautical Telecommunication Network (ATN) using Internet Protocol Suite (IPS) Standards and Protocol • Doc. 10003 - Manual on the Digital Exchange of Aeronautical Meteorological Information • Doc. 10039 - Manual on System Wide Information Management (SWIM) Concept

- ICAO Guidelines for the Implementation of OPMET Data Exchange using IWXXM • WMO No.731 – Guide to Meteorological Observing and Information Distribution Systems for Aviation Weather Services • Regional OPMET Interface Control Documents • Regional OPMET Bulletin Exchange Handbooks

2031

Information exchange model

Meteorologic al information

ICAO

Meteorological Information Exchange Model (IWXXM) Vxx (for 2031)

ICAO Meteorological Information Exchange Model (IWXXM) used to provide meteorological information in XML/GML form. References: • Doc. 10003 - Manual on the Digital Exchange of Aeronautical Meteorological Information

- ICAO Guidelines for the Implementation of OPMET Data Exchange using IWXXM • WMO IWXXM Wiki:

ATM SWIM

service provider

https://wiswiki.wmo.int/tiki-index.php? page=IWXXM&structure=WIS+up

ANSP

ATM SWIM

service provider

MET

Information Service Provider

2031

MET

Information Service Provider

ATM SWIM

service provider

ANSP

Ground system infrastructur e

Information exchange

Communication infrastructure for a meteorological information service

Ground system infrastructure and dissemination systems for meteorological information within a SWIM environment, supporting request/reply or publish/subscribe access mechanisms and providing quality & timely information to users in a range of formats to best enable their optimal decision making.

ANSP

2031

Sixth edition of the GANP

Main Purpose New Capabilities Description

Maturity Level

-

Human Factor

Considerations

PLANNING LAYERS

OPERATIONS

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

ENABLERS

There are currently no enablers.

Information

Meteorological observations information

AMET-B4/1

Tactical-During ops

Tactical-Pre ops

Training

-

Training for the

provision of meteorological information service in SWIM

Training for meteorological, aviation IT

personnel and SWIM service providers, on the exchange of meteorological information in a SWIM environment.

References: • Doc. 10003 - Manual on the Digital Exchange of Aeronautical Meteorological Information • Doc. 10039 - Manual on System Wide Information Management (SWIM) Concept • ICAO Guidelines for the Implementation of OPMET Data Exchange using IWXXM • Regional OPMET Interface Control Documents • Regional OPMET Bulletin

Exchange Handbooks

ANSP	2031

ATM SWIM

service provider

MET

Information Service Provider

| Taxi-out | Departure |  | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- | --- |
| Turn-around |  |  |  |  |  |

Sixth edition of the GANP

Main Purpose

Meteorological forecast and warning information	Information

AMET-B4/2

New Capabilities

Description

Maturity Level	-

Human Factor Considerations

PLANNING LAYERS

OPERATIONS

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

ENABLERS

There are currently no enablers.

Tactical-During ops

Tactical-Pre ops

Pre-tactical

Sixth edition of the GANP

Main Purpose New Capabilities Description

| Taxi-out | Departure |  | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- | --- |
| Turn-around |  |  |  |  |  |

ENABLERS

There are currently no enablers.

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

OPERATIONS

Human Factor

Considerations

PLANNING LAYERS

Post operations

-

Maturity Level

Information

Climatological and historical meteorological

information

AMET-B4/3

| Pre-tactical | Tactical-Pre ops |  | Tactical-During ops |
| --- | --- | --- | --- |
| Post operations |  |  |  |

| Taxi-out | Departure |  | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- | --- |
| Turn-around |  |  |  |  |  |

Sixth edition of the GANP

Main Purpose New Capabilities Description

Maturity Level

-

Human Factor

Considerations

PLANNING LAYERS

OPERATIONS

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

ENABLERS

There are currently no enablers.

Information

Meteorological information service in SWIM

AMET-B4/4

APTA

APTA-B0/1	PBN Approaches (with basic capabilities)	Operational

Sixth edition of the GANP

Main Purpose		This element represents the use of PBN in design of approach procedures to provide more flexibility to airspace planners to manage the use of airspace, and to facilitate access to airports. It includes the provision of instrument approach procedures with vertical guidance in support of stabilized approaches.

New Capabilities		Development of PBN instrument approach procedures that are independent of ground based navigation aids. The use of new approach design criteria can often achieve the lowest possible minimums due to optimized lateral and vertical paths.

The PBN procedures also support stabilized approach operations for aircraft equipped with advisory Baro-VNAV functionality. Stable vertical paths can be established using vertical guidance for aircraft equipped with either advisory Baro-VNAV functionality or by using SBAS.

Description 		PBN approaches allow for guided lateral paths and optionally, with associated advisory vertical paths based on Baro-VNAV functionality for aircraft so equipped. Such Baro-VNAV functionality enables stabilized decent operations on the final segment of the approach at airports which do not have ground infrastructure to support precision approaches. These procedures can also be implemented to allow continued approach operations in the case of failure of an existing ILS or traditional non precision approaches that are based on ground navigation aids.

Maturity Level 	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology need

NAVS-B0/3 - Aircraft Based Augmentation Systems (ABAS)

Relation-information need

AMET-B0/1 - Meteorological observations products

Relation-information need

AMET-B0/2 - Meteorological forecast and warning products

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Design	PBN approach (with basic capabilities) procedures design and use.

These operational procedures should be designed and used (aircraft operation criteria) as specified in ICAO Doc 8168 PANS-OPS Vol II and I or equivalent.

ANSP 	2013

Operational procedures

Design	PBN approach (with basic capabilities) validation, approval and publication

A flight inspection and/or validation of the procedures might be required before publication. The publication of the procedures should follow Annex 4.

References: ICAO Doc 8071 (Volume I — Testing of Ground-based Radio Navigation Systems) Doc 9906 (Quality Assurance Manual for Flight Procedure Design). Doc 9906 Vol 6 ( Quality Assurance Manual for Flight Procedure Design)

ANSP   CAA 	2013

Operational procedures

Operations	SOPs for PBN

Aircraft operator

approaches (with basic capabilities)

Procedures for the crew to follow to fly a PBN approach. Defined in the Ops Manual. Reference: Doc 9613 (PBN Manual)

2013

Aircraft operator

Operational procedures

Operations	Contingency

Aircraft operator

Aircraft manufacturer

procedures for PBN approaches (with basic capabilities)

Procedures for the crew to follow in case of abnormal events. Defined in the Ops Manual. Reference: Doc 9613 (PBN Manual)

2013

Airborne system capability

-	Aircraft capability for PBN approach (with basic capabilities)

Aircraft eligible for RNP APCH Navigation specification as defined in Doc 9613 (PBN Manual) and listed in the Aircraft Flight Manual.

2013

Training	-	Training requirements for PBN approaches (with basic capabilities)

Crew trained to fly PBN approaches (with basic capabilities). References: As defined in Doc 9613 (PBN Manual). PBN approaches (with basic capabilities) training for Air traffic controllers.

References: As defined in Doc 9613 (PBN Manual). PBN approaches (with basic capabilities) training for Procedure designers, Airspace planners.

Aircraft operator

CAA

References: PANS-OPS Vol II and Doc 9992 Manual on the Use of Performance- Based Navigation (PBN) in Airspace Design

2013

Operational Authorizatio n

-	Operational

Authorization for PBN approaches (with basic capabilities)

Aircraft operator flying a PBN approach should have an operational authorization related to the specified performance of the procedure, as described in Doc 9997 (PBN Ops Approval Manual).

2013

Aircraft operator

ANSP

DEPLOYMENT APPLICABILITY

Operational conditions:

Any runway ends with or without existing procedures. Can be used to facilitate access at aerodromes where conventional procedures are not implementable, or in support of existing procedures for contingency use.

Main intended benefits:

Benefitting stakeholder(s)

Operational description

Type

| Type | Operational description | Benefitting stakeholder(s) |  |
| --- | --- | --- | --- |
| Direct benefits | Add availability of airports/runways not served by conventional instrument approach procedures. Increase availability of airports where lower minima are achieved, or facilitate continued airport operations when conventional procedures become unserviceable. | Airport operator ANSP | Airspace user |
|  | Added safety by potentially replacing circle to land procedures. | ANSP Airspace user |  |
|  | Improve safety where both lateral and vertical guidance is provided (stabilized approach). | Airspace user |  |
| Indirect benefits | Flexibility to reduce environmental impact (noise and emissions) due to the possible avoidance of sensitive flight paths dictated by ground based navigation aids, and shorter paths to transition from en-route to the approach. | Airport operator ANSP |  |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Equip additional RWY ends with instrument approaches | ++ | KPI10: Airport peak throughput |
| Capacity | Capacity, throughput & utilization | Reduce approach minima (ceiling & visibility) | ++ | KPI10: Airport peak throughput |

Sixth edition of the GANP

Main Purpose 		Use of PBN capabilities allows more flexible placement of arrival and departure routing without the need for ground based infrastructure to support these routes.

Operational

PBN SID and STAR procedures (with basic

capabilities)

APTA-B0/2

New Capabilities 	 The flexibility of arrival path design supports the ability to connect en-route to the approach in an optimal manner, enabling better airspace management, reduced path distance, and reduced noise footprint. A precisely defined arrival path supports more optimum descent planning in operations and provides a building block for reducing ATC intervention during descent.

This flexibility is also applicable to providing more optimum departure paths to the exit of terminal airspace. These new capabilities are essential elements to support the development of Continuous Descent Operations/Continuous Climb Operations (CDO/CCO).

Description 		This element represents the use of PBN in design of arrival and departure procedures to provide more flexibility to airspace planners to manage the use of airspace for enhancing arrival and departures in terminal areas. It provides the basic capability to support the implementation of CDO and CCO operations.

Maturity Level 	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-information need

AMET-B0/1 - Meteorological observations products

Relation-information need

AMET-B0/2 - Meteorological forecast and warning products

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational Design

procedures

PBN SID and

STAR

Procedures (with basic capabilities) procedure design and use

These procedures should be designed

and used as specified in Doc 8168 (PANS-OPS Vol II and I) or equivalent.

ANSP

2013

Operational procedures

Design	PBN SID and STAR

Procedures (with basic capabilities) validation, approval and publication

A flight inspection and/or validation of the procedures might be required before publication. The publication of the procedures should follow Annex 4.

Aircraft operator

References: ICAO Doc 8071 (Volume I — Testing of Ground-based Radio Navigation Systems) Doc 9906 (Quality Assurance Manual for Flight Procedure Design). Doc 9906 Vol 6 ( Quality Assurance Manual for Flight Procedure Design).

2013

Operational procedures

Operations	SOPs for PBN

Aircraft operator

SID/STAR (with basic capabilities)

Procedures for the crew to follow to fly a PBN SID/STAR. Reference: Defined in the Ops Manual Reference: Doc 9613 (PBN Manual)

2013

Operational procedures

Operations	Contingencies

Aircraft operator

Aircraft manufacturer

for PBN SID/STAR (with basic capabilities)

Procedures for the crew to follow in case of abnormal events. Defined in the Ops Manual Reference: Doc 9613 (PBN Manual)

2013

Airborne system capability

-	Aircraft capability for PBN SID/STAR (with basic capabilities)

Aircraft eligible for applicable Navigation specification as defined in Doc 9613 (PBN Manual) and listed in the Aircraft Flight Manual.

2013

Operational Authorizatio n

-	Operational

Authorization for PBN SID/STAR

(with basic capabilities)

Aircraft operator flying a PBN SID/STAR should have an operational authorization related to the specified performance of the procedure, as described in Doc 9997 (PBN Ops Approval Manual)

2013

Aircraft operator

CAA

CAA

ANSP

Ground system infrastructur e

Navigation	NAVAIDS to

support the applicable navigation specification used for SID/STAR (with basic capabilities)

Depending on the navigation specification used, suitable ground based navigational aids will be required. See Doc 9613 (PBN Manual) for details.

ANSP 	2013

DEPLOYMENT APPLICABILITY

Operational conditions:

Applicable in all terminal areas. Arrivals facilitates descent and connects to the approach phase. Departures facilitates climb and provides lateral path top exit terminal area. The use of PBN procedures supports flexible airspace planning and development.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Training

-

Training

requirements for PBN SID/STAR

(with basic

capabilities)

Crew trained to fly PBN SID/STAR (with

basic capabilities). References: As defined in Doc 9613 (PBN Manual). PBN SID/STAR (with basic capabilities) training for Air traffic controllers.

References: As defined in Doc 9613 (PBN Manual). PBN SID/STAR (with basic capabilities) training for procedure designers, airspace planners. Ref Doc 8168 (PANS OPS Vol II); Doc 9992 (PBN

airspace design Manual); Doc 9906 (Vol

2)

ANSP

Aircraft operator

2013

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Flexible airspace planning and development. | ANSP |
|  | Increase in terminal airspace capacity due to the ability to develop optimized routes independent of the ground based navigation. | Airport operator ANSP |
| Indirect benefits | Contingencies afforded by additional procedures not reliant on ground based radio aids. | ANSP Airspace user |
|  | Increased efficiency of traffic flow through the terminal area. | ANSP Airspace user |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Increase airport arrival rate | ++ | KPI11: Airport throughput efficiency |
| Capacity | Capacity, throughput & utilization | Mitigate local airspace capacity constraints if this is the problem | ++ | KPI10: Airport peak throughput |
| Capacity | Capacity, throughput & utilization | Mitigate noise constraints if this is the problem | ++ | KPI10: Airport peak throughput |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Vertical flight efficiency | Reduce permanent (airspace and approach procedure design) and semi-permanent (ATFCM measures) altitude constraints along the descent portion of traffic flows, in en- route and terminal airspace | ++ | KPI19: Level-off during descent |
| Efficiency | Vertical flight efficiency | Reduce permanent (airspace and departure procedure design) and semi-permanent (ATFCM measures) altitude constraints (level capping) along the climb portion of traffic flows, in terminal and en-route airspace | ++ | KPI17: Level-off during climb |

APTA-B0/3	SBAS/GBAS CAT I precision approach

procedures

Operational

Sixth edition of the GANP

Main Purpose	Introduction of SBAS and GBAS CAT I procedures allow for reduced minima at aerodromes situated in areas of significant terrain, where ILS is not possible.

New Capabilities	Implementation of CAT I at airports, where ILS is precluded, increases accessibility.

It also provides a building block for aircraft with equipment such as SVS, EVS HGS to operate to decision altitudes below standard CAT I Minimums using Special operational Authorizations. For aircraft with such approvals, this increases airport availability in weather conditions that would otherwise preclude operations.

Description		This element represents the use of augmented GNSS systems to allow aircraft operation with a more precise vertical and lateral navigation capability. It also includes the development of SA CAT I operations below existing minima.

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS 		OPERATIONS

Tactical-During ops

Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology need

NAVS-B0/2 - Satellite Based Augmentation Systems (SBAS)

Relation-technology need

NAVS-B0/1 - Ground Based Augmentation Systems (GBAS)

Relation-information need

AMET-B0/1 - Meteorological observations products

Relation-information need

AMET-B0/2 - Meteorological forecast and warning products

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Design	Cat I Precision Approach Procedure design and use

These procedures should be designed and used as specified in Doc 8168 (PANS-OPS Vol II and I) or equivalent.

ANSP 	2013

Operational procedures

Design	Cat I Precision Approach procedure validation, approval and publication

A flight inspection and/or validation of the procedures might be required before publication. The publication of the procedures should follow Annex 4.

Aircraft operator

References: ICAO Doc 8071 (Volume I — Testing of Ground-based Radio Navigation Systems) Doc 9906 (Quality Assurance Manual for Flight Procedure Design). Doc 9906 Vol 6 ( Quality Assurance Manual for Flight Procedure Design)

ANSP   CAA 	2013

Operational procedures

Operations	SOPs for Cat I

Aircraft operator

Precision Approaches

Procedures for the crew to follow to fly a Cat I Precision Approach. Reference: Ops Manual defines SOPs

2013

Operational procedures

Operations	Contingencies

Aircraft operator

Aircraft manufacturer

for Cat I Precision Approaches

Procedures for the crew to follow in case of abnormal events. Reference: Ops Manual defines SOPs

2013

Airborne system capability

Navigation	Aircraft capability

for Cat I Precision Approaches

TSO C145 or C146 or equivalent required for SBAS avionics. TSO C161A required for GBAS CAT I

2013

Ground system infrastructur e

Navigation	Ground based

system for Cat I Precision Approach

GBAS Ground Station SBAS Ground stations Ref Annex 10 Vol I

Airport operator 2013

ANSP

Training	-	Training requirements for Cat I Precision Approach Procedures

Crew trained to fly Cat I precision approaches. Reference: Doc 8168 (Vol I). Cat I Precision Approach training for ATC. Reference: Doc 8168 (Vol I). Cat I Precision Approach training for procedure designers, airspace planners. Ref : Doc 8168 (PANS OPS Vol II); Doc 9906 (Vol

2)

ANSP

Aircraft operator

2013

### DEPLOYMENT APPLICABILITY

#### Operational conditions:

Runway ends where precision approach guidance is considered a requirement.

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |  |
| --- | --- | --- | --- |
| Direct benefits | Capacity: Improved airport access from reduced operating minima. | ANSP Airspace user |  |
|  | Vertical guidance for stable approach. | ANSP Airspace user |  |
|  | Reduced probability of go-around. | ANSP Airspace user |  |
| Indirect benefits | Increase in terminal airspace capacity due to the ability to conduct landing operations in adverse weather. | Airport operator ANSP | Airspace user |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Equip additional RWY ends with instrument approaches | ++ | KPI10: Airport peak throughput |
| Capacity | Capacity, throughput & utilization | Reduce approach minima (ceiling & visibility) | ++ | KPI10: Airport peak throughput |

Sixth edition of the GANP

Main Purpose

Reduce fuel burn by not requiring application or power during descent.

New Capabilities

ATC procedures to facilitate uninterrupted descent, reducing ATC/Pilot interaction.

Operational

CDO (Basic)

APTA-B0/4

Description 		Arriving aircraft are allowed to descend continuously from top of descent by employing minimum engine thrust, ideally in a low drag configuration, prior to the Initial Approach Fix (IAF).

Maturity Level 	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes In ATC.

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

APTA-B0/2 - PBN SID and STAR procedures (with basic capabilities)

Relation-operational benefit

RSEQ-B0/3 - Point merge

Relation-information need

AMET-B0/1 - Meteorological observations products

Relation-information need

AMET-B0/2 - Meteorological forecast and warning products

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Design	Development of CDO (Basic) procedures

These procedures should be designed and used as specified in Doc 8168 (PANS-OPS Vol II and I) or equivalent, with reference to Doc 9931 (CDO Manual)

ANSP

CAA

ANSP

Aircraft operator

2013

Operational procedures

Design	CDO (Basic) procedures validation, approval and publication

A flight inspection and/or validation of the procedures might be required before publication. The publication of the procedures should follow Annex 4.

References: ICAO Doc 8071 (Volume I — Testing of Ground-based Radio Navigation Systems) Doc 9906 (Quality Assurance Manual for Flight Procedure Design). Doc 9906 Vol 6 ( Quality Assurance Manual for Flight Procedure Design)

2013

Operational procedures

Operations	SOPs for CDO

Aircraft operator

(Basic) operations

Procedures for the crew to follow to facilitate the flying of a CDO. OPS Manual defines SOPs

2013

Aircraft operator

Operational procedures

Operations	Contingencies

for CDO (Basic)

Procedures for the crew to follow in case of abnormal events. Reference: Ops Manual defines SOPs

2013

Airborne system capability

Aircraft system

Aircraft Capability for CDO (Basic)

Eligibility for the applicable PBN navigation specification (if required) and vertical path capability, as defined in Doc 9613 (PBN Manual) and listed in the Aircraft Flight Manual (AFM)

2013

Aircraft operator

Aircraft manufacturer

Ground system infrastructur e

Navigation	NAVAIDS to

Aircraft operator

support the applicable navigation specification used for CDO (Basic)

Depending on the navigation specification used, suitable ground based navigational aids will be required. See Doc 9613 (PBN Manual) for details.

ANSP 	2013

Training	-	Training requirements for CDOs (Basic)

Crew trained to fly CDOs (Basic) CDO (Basic) training for Air traffic controllers CDO (Basic) training for procedure designers, Airspace planners.

DEPLOYMENT APPLICABILITY

Operational conditions:

Non-congested terminal airspace with PBN STAR routings where greater efficiency is required. Noise sensitive areas requiring reduced noise footprint.

Main intended benefits:

References: Doc 9906 (Vol 2); Doc 9992

ANSP

2013

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Reduce fuel burn by not requiring power during descent. | Airspace user |
| Indirect benefits | Increase in terminal area efficiency from reduced ATC intervention in the aircraft routing and vertical profile. | ANSP Airspace user |
|  | Reduction in noise footprint. | Local community |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Vertical flight efficiency | Avoid efficiency penalties attributable to non-optimum ToD (descent starts before or after the optimum ToD) | ++ | KPI19: Level-off during descent |
| Efficiency | Vertical flight efficiency | Avoid tactical lengthening of arrival path (eg vectoring, holding, trombone extension) because this leads to level flight | ++ | KPI19: Level-off during descent |
| Efficiency | Vertical flight efficiency | Reduce descent inefficiency attributable to altitude constraints imposed by ATM | ++ | KPI19: Level-off during descent |

APTA-B0/5	CCO (Basic)	Operational

Sixth edition of the GANP

Main Purpose	Reduce fuel burn by not requiring level-offs during climb.

New Capabilities	ATC procedures to facilitate uninterrupted climb, reducing ATC/Pilot interaction.

Description		Departing aircraft are allowed to climb continuously, to the greatest possible extent, by employing optimum engine thrust. An optimal continuous climb should start on take-off and allow the aircraft to climb efficiently using climb profiles that reduce controller pilot communications and segments of level flight until the top of climb.

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes In ATC.

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Departure

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-operational need	APTA-B0/2 - PBN SID and STAR procedures (with basic capabilities)

Relation-information need

AMET-B0/1 - Meteorological observations products

Relation-information need

AMET-B0/2 - Meteorological forecast and warning products

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Design	Development of CCO (Basic) procedures

These procedures should be designed and used as specified in Doc 8168 (PANS-OPS Vol II and I) or equivalent,

ANSP

Aircraft operator

2013

|  |  |  | with reference to Doc 9993 (CCO Manual) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operational | Design | CCO (basic) | A flight inspection and/or validation of the | ANSP | CAA | 2013 |
| procedures |  | procedures | procedures might be required before |  |  |  |
|  |  | validation, | publication. The publication of the |  |  |  |
|  |  | approval and | procedures should follow Annex 4. Ref: |  |  |  |
|  |  | publication | ICAO Doc 8071 (Volume I — Testing of |  |  |  |
|  |  |  | Ground-based Radio Navigation |  |  |  |
|  |  |  | Systems) Doc 9906 (Quality Assurance |  |  |  |
|  |  |  | Manual for Flight Procedure Design). Doc |  |  |  |
|  |  |  | 9906 Vol 6 ( Quality Assurance Manual for |  |  |  |
|  |  |  | Flight Procedure Design) |  |  |  |

Operational procedures

Operations	SOPs for CCO

Aircraft operator

(Basic) operations

Procedures for the crew to follow to facilitate the flying of a CCO. OPS Manual defines SOPs

2013

Aircraft operator

Operational procedures

Operations	Contingencies

for CCO (Basic)

Procedures for the crew to follow in case of abnormal events. Reference: Ops Manual defines SOPs

2013

Airborne system capability

Aircraft system

Aircraft Capability for CCO (Basic)

Eligibility for the applicable PBN navigation specification (if required) and vertical path capability, as defined in Doc 9613 (PBN Manual) and listed in the Aircraft Flight Manual (AFM)

2013

Aircraft operator

Aircraft manufacturer

Ground system infrastructur e

Navigation	NAVAIDS to

Aircraft operator

support the applicable navigation specification used for CCO (Basic)

Depending on the navigation specification used, suitable ground based navigational aids will be required. See Doc 9613 (PBN Manual) for details.

ANSP 	2013

Training	-	Training requirements for CCOs (Basic)

Crew trained to fly CCOs (Basic). Reference: Training to support the CCO concept. ATC trained to provides CCOs (Basic). Reference: Training to support the CCO concept. CDO (Basic) training for procedure designers, Airspace planners. References: Doc 9906 (Vol 2); Doc 9992

ANSP

2013

### DEPLOYMENT APPLICABILITY

#### Operational conditions:

Non-congested terminal airspace with PBN SID routings where greater efficiency is required. Noise sensitive areas requiring noise abatement procedures.

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Reduce fuel burn by not requiring level-offs during climb. | Airspace user |
| Indirect benefits | Increase in terminal area efficiency from reduced ATC intervention in the aircraft routing and vertical profile. | ANSP Airspace user |
|  | Reduction in noise footprint. | Local community |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Vertical flight efficiency | Reduce permanent (airspace and departure procedure design) and semi-permanent (ATFCM measures) altitude constraints (level capping) along the climb portion of traffic flows, in terminal and en-route airspace | ++ | KPI17: Level-off during climb |

Sixth edition of the GANP

Main Purpose

Helicopter unique capabilities allow IFR operations that start or terminate from any suitable point in

space (PinS), as long as visual conditions support take-off/landing capability from that point.

New Capabilities	Facilitating arrivals and departures to landing locations which do not otherwise support such

operations.

Description

PBN PinS operations include arrivals and departure procedures, specific to helicopters, that allow

visual landing and take-off operations from heliports or other landing locations.

Maturity Level

Ready for implementation

PBN Helicopter Point in Space (PinS) Operations  Operational

APTA-B0/6

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-information need

AMET-B0/1 - Meteorological observations products

Relation-information need

AMET-B0/2 - Meteorological forecast and warning products

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Design	Helicopter PBN Point in Space (PinS) procedure design

These procedures should be designed and used as specified in Doc 8168 (PANS-OPS Vol II and I) or equivalent.

ANSP 	2013

Operational procedures

Design	Helicopter PBN Point in Space (PinS) procedure validation, approval and publication

A flight inspection and/or validation of the procedures might be required before publication. The publication of the procedures should be in accordance with Doc 8168 (PANS OPS Vol II). Ref: ICAO

Aircraft operator

Doc 8071 (Volume I — Testing of Ground- based Radio Navigation Systems) Doc 9906 (Quality Assurance Manual for Flight Procedure Design). Doc 9906 Vol 6 (Quality Assurance Manual for Flight Procedure Design) The publication of the procedures should be in accordance with Doc 8168 (PANS OPS Vol II).

ANSP   CAA 	2013

Operational procedures

Operations	SOPs for

Aircraft operator

Helicopter PBN Point in Space (PinS) operations

Procedures for the crew to follow to facilitate the flying of a CDO. OPS Manual defines SOPs

2013

Operational procedures

Operations  Contingencies

for Helicopter PBN Point in Space (PinS) operations

Procedures for the crew to follow in case of abnormal events. Reference: Ops Manual defines SOPs

2019

Airborne system capability

Aircraft system

Aircraft capability for PBN Helicopter Point in Space (PinS) operations

Requires applicable PBN Navigation specification, as defined in Doc 9613 (PBN Manual) and listed in the Rotorcraft Flight Manual (RFM)

2013

Aircraft operator

Aircraft manufacturer

Operational Authorizatio n

-	Operational

Aircraft operator

ANSP

Authorization PBN Helicopter Point in Space (PinS) operations

Aircraft operator flying a PBN Helicopter Point in Space (PinS) operation should have an operational authorization related to the specified performance of the procedure, as described in Doc 9997 (PBN Ops Approval Manual)

2013

Aircraft operator

CAA

Training	-	Training requirements for Helicopter PBN Point in Space (PinS)

Crew trained to fly Helicopter PBN Point in Space (PinS) procedures. Ref.: As defined in Doc 8168 (PANS OPS Vol I) Helicopter PBN Point in Space (PinS) training for ATC Helicopter PBN Point in Space (PinS) training for procedure designers, Airspace planners.

DEPLOYMENT APPLICABILITY

Operational conditions:

Obstacle rich environments containing helicopter landing locations.

Main intended benefits:

References: Doc 8168 (PANS OPS Vol II); Doc 9906 (Vol 2).

2013

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| Type | Operational description | Benefitting stakeholder(s) |  |
| --- | --- | --- | --- |
| Direct benefits | Improve access to helicopter landing areas through approach procedures with improved minima. | ANSP | Airspace user |
|  | Reduce number of diversions. | ANSP | Airspace user |
| Indirect benefits | Airspace capacity enhancement through design flexibility and removal of existing operational restrictions. | ANSP |  |
|  | Enhance helicopter operations availability and continuity. | ANSP | Airspace user |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Mitigate local airspace capacity constraints if this is the problem | ++ | KPI10: Airport peak throughput |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Reduce approach minima (ceiling & visibility) | ++ | KPI10: Airport peak throughput |

APTA-B0/7	Performance based aerodrome operating minima – Advanced aircraft

Operational

Sixth edition of the GANP

Main Purpose	Standard Aerodrome operating minima are predicated upon aircraft equipped with the minimum required equipment (the basic aircraft) for that approach. These aerodrome operating minima relate directly to the established types and categories of operations and the associated infrastructure requirements (e.g. runway lights, approach lights). Aircraft with more advanced equipage can take advantage of existing infrastructure to obtain special authorizations for enhanced approach operations to lower minimums than basic aircraft can use.

New Capabilities		Advanced Aircraft are those aircraft with equipment in addition to that required for a Basic Aircraft for a given approach or landing operation. Examples of additional equipment could include EVS, HUD and/or autoland. The additional equipment allows the aircraft to operate to lower RVR values and/or to lower DH than would be achievable with a basic equipped aircraft. PB AOM are derived by taking account of the combined capabilities of an Advanced Aircraft, and available ground facilities.

Description	For advanced aircraft, Improvements include:

EVS operations using existing Type A or Type B CAT I procedures, requiring natural vision from 100 ft, but with significantly reduced RVR

Lower that standard CAT I (SA CAT I) operations by means of HUD or autoland. CAT II operations with less infrastructure (SA CAT II) by means of HUD or autoland.

EVS to land operations, using existing CAT I facilities but without the need to have natural visual references before landing.

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? No

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Arrival

### DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

APTA-B0/3 - SBAS/GBAS CAT I precision approach procedures

Relation-information need

AMET-B0/1 - Meteorological observations products

Relation-information need

AMET-B0/2 - Meteorological forecast and warning products

ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

National regulatory framework

Operational credits

Provisions for operational credits to enable lower minima based on advanced aircraft capabilities. Reference: Annex 6 Part I

CAA 	2013

Operational procedures

Operations	SOPs for

Aircraft operator

Performance- based Aerodrome Operating Minima (Advanced aircraft)

Procedures for the crew to operate to minima determined by the combination of aircraft equipage and ground infrastructure. Defined in the Ops Manual. Reference: Doc 9365 (AWO Manual)

2013

Operational procedures

Operations	Contingency

procedures for Performance- based Aerodrome Operating Minima (Advanced aircraft)

Procedures for the crew to follow in case of abnormal events. Defined in the Ops Manual

2013

Aircraft operator

Operational procedures

Operations	ATC procedures

for Performance- based Aerodrome Operating Minima (Advanced aircraft)

Procedures for ATC to use in order to facilitate the use of performance-based minima at aerodromes. Low visibility operating plan for aerodrome ATC

ANSP 	2013

Operational procedures

Operations	Aerodrome

Aircraft manufacturer

procedures for Performance- based Aerodrome Operating Minima (Advanced aircraft)

Procedures for ground operations by aircraft with advanced capabilities

Airport operator 2013

Aircraft operator

Airborne system capability

Aircraft system

Advanced aircraft capability

SBAS and GBAS as required EVS and HUD installation Reference: Doc 9365 (AWO Manual)

2013

Aircraft operator

Operational Authorizatio n

-	Operational

Authorization for Performance- based Aerodrome Operating Minima (Advanced aircraft)

Aircraft operator conducting PBAOM operations for low visibility operations require a specific approval detailing the operational credit applied Reference: Doc 9365 (AWO Manual)

CAA

2013

Aircraft operator

Ground system infrastructur e

Navigation	NAVAIDS to

support the intended operation

Pre threshold terrain information for advanced aircraft operations SBAS/GBAS ground stations (as required)

Airport operator 2013

ANSP

Training	-	Training requirements for Performance- based Aerodrome Operating Minima (Advanced aircraft)

Crew trained to fly using Performance- based Aerodrome Operating Minima (Advanced aircraft). Training on the use of advanced aircraft equipment such as EVS Reference: Doc 9365 (AWO Manual).

ATC trained to understand implications of Performance-based Aerodrome Operating Minima (Advanced aircraft).

Training for ATC on the application of operational credits for advanced aircraft and the effect on determining minima used by crews.

ANSP

2013

DEPLOYMENT APPLICABILITY

Operational conditions:

Approach operations in adverse weather conditions to lower than standard minima for operators of advanced aircraft.

Main intended benefits:

Benefitting stakeholder(s)

Operational description

Type

| Type | Operational description | Benefitting stakeholder(s) |  |
| --- | --- | --- | --- |
| Direct benefits | Reduced minima operations without additional infrastructure on the ground, based on aircraft equipment (SVS, EVS, HUD) Increased capacity of the airport under adverse conditions by improving access to airports. | ANSP | Airspace user |
|  | Reduced number of diversions. | ANSP | Airspace user |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Reduce approach minima (ceiling & visibility) | ++ | KPI10: Airport peak throughput |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

APTA-B0/8	Performance based aerodrome operating minima – Basic aircraft

Operational

Sixth edition of the GANP

Main Purpose	For Basic aircraft, improvements include:

Instrument approaches to non-instrument runways, improving airport access

Flexibility to gradually improve the ground infrastructure with consequent improvements in operating minima

New Capabilities	New approach procedures can be added based on GNSS, without the need for any additional ground infrastructure. Non-instrument runways can support an instrument approach procedure, but with relatively high minima. Reduction of this minima is achieved by upgrading the runway to at least a non-precision runway, along with other improvements such as lighting and visual aids.

Description		Aerodrome operating minima are predicated upon the aircraft with the minimum equipment required (the basic aircraft). These aerodrome operating minima relate directly to the established types and categories of operations and the associated infrastructure requirements. An airport operator can choose to upgrade ground based infrastructure to add approach capability and/or to enable lower minimums for increased airport availability for all operations. Examples of such infrastructure upgrades include runway lighting, approach lighting, centreline lights. This also includes upgrade of runway from non-instrument to non-precision.

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-information need

AMET-B0/1 - Meteorological observations products

Relation-information need

AMET-B0/2 - Meteorological forecast and warning products

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational Design

procedures

Design of

approach procedures to non-instrument runways

Procedures should be designed in

accordance with published criteria. Reference: Doc 8168 (PANS OPS Vol II)

ANSP

2013

Operational Design

procedures

Validation,

approval and publication of approach procedures to non-instrument runways

A flight inspection and/or validation of the

procedures might be required before publication. The publication of the procedures should follow Annex 4. Ref: ICAO Doc 8071 (Volume I — Testing of Ground-based Radio Navigation Systems) Doc 9906 (Quality Assurance Manual for Flight Procedure Design). Doc 9906 Vol 6 ( Quality Assurance Manual for Flight Procedure Design)

ANSP   CAA

2013

Operational Operations

procedures

SOPs for

operations using instrument approaches to non-instrument runways

Additional requirements for operating

crews should be detailed in the Ops Manual.

2013

Aircraft operator

Operational procedures

Operations	Contingencies

Aircraft operator

Aircraft manufacturer

for operations using instrument approaches to non-instrument runways

Specific contingencies associated with operations to non-instrument runways should be detailed in the Ops manual

2013

Airborne system capability

Aircraft system

Aircraft capability for operating in instrument approaches to non-instrument runways

Aircraft eligible for applicable Navigation specification as defined in Doc 9613 (PBN Manual) and listed in the Aircraft Flight Manual.

2013

Ground system infrastructur e

-	Ground based infrastructure to support the desired operating minima

Minimum requirement is a non-instrument runway for basic aircraft Additional elements may be used to reduce operating minim (lighting, visual aids etc.)

Airport operator 2013

Aircraft operator

### DEPLOYMENT APPLICABILITY

#### Operational conditions:

Airports with limited infrastructure wanting to implement or improve instrument approach procedures.

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |  |
| --- | --- | --- | --- |
| Direct benefits | Increased capacity of the airport under adverse conditions by improving access to airports. | Airport operator ANSP | Airspace user |
|  | Reduced number of diversions. | ANSP Airspace user |  |
| Indirect benefits | Reduced initial costs to implement vertically guided approaches. | Airport operator |  |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Equip additional RWY ends with instrument approaches | ++ | KPI10: Airport peak throughput |
| Capacity | Capacity, throughput & utilization | Reduce approach minima (ceiling & visibility) | ++ | KPI10: Airport peak throughput |

APTA-B1/1	PBN Approaches (with advanced capabilities)	Operational

Sixth edition of the GANP

Main Purpose		PBN approaches with advanced functionality allow for the introduction of more flexible approaches including the use of RF legs within the Final Approach Segment (FAS) and RNP.

New Capabilities	Use of advanced functionality as included in A-RNP and RNP AR (RF legs).

Description		This element represents the use advanced features of PBN in design of approach procedures to provide more access to airports in challenging environments, where conventional procedures are unsuitable.

Advanced RNP is the navigation specification which encompasses all elements of PBN (but excluding RNP AR APCH). It requires an FMS based on a TSO-C.

With A-RNP Ops approval, an operator can conduct any PBN operation except RNP AR APCH. RNP AR APCH requires a Specific Approval.

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Arrival

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-operational need	APTA-B0/1 - PBN Approaches (with basic capabilities) Relation-information need	AMET-B0/1 - Meteorological observations products Relation-information need	AMET-B0/2 - Meteorological forecast and warning products

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Design	PBN approach (with advanced capabilities) procedures design and use

These instrument flight procedures should be designed and used as specified in Doc 8168 (PANS-OPS Vol II and I) and Doc 9905 (Required Navigation Performance Authorization Required (RNP AR) Procedure Design Manual).

ANSP 	2019

| Operational | Design | PBN approach | A flight inspection and/or validation of the | ANSP | CAA | 2019 |
| --- | --- | --- | --- | --- | --- | --- |
| procedures |  | (with advanced | procedures might be required before |  |  |  |
|  |  | capabilities) | publication. The publication of the |  |  |  |
|  |  | validation, | procedures should follow Annex 4. Ref: |  |  |  |
|  |  | approval, and | ICAO Doc 8071 (Volume I — Testing of |  |  |  |
|  |  | publication | Ground-based Radio Navigation |  |  |  |
|  |  |  | Systems) Doc 9906 (Quality Assurance |  |  |  |
|  |  |  | Manual for Flight Procedure Design). Doc |  |  |  |
|  |  |  | 9906 Vol 6 ( Quality Assurance Manual for |  |  |  |
|  |  |  | Flight Procedure Design) |  |  |  |

Operational procedures

Operations	SOPs for PBN

Aircraft operator

approaches (with advanced capabilities)

Procedures for the crew to follow to fly a PBN approach. Defined in the Ops Manual Reference: Doc 9613 (PBN Manual)

Aircraft operator

2019

Operational procedures

Operations	Contingency

Aircraft operator

Aircraft manufacturer

procedures for PBN approaches (with advanced capabilities)

Procedures for the crew to follow in case of abnormal events. Defined in the Ops Manual Reference: Doc 9613 (PBN Manual)

2019

Airborne system capability

Navigation	Aircraft capability

Aircraft operator

CAA

for PBN approach (with advanced capabilities)

Aircraft eligible for RNP AR APCH Navigation specification, Advanced-RNP navigation specification as required.

Defined in Doc 9613 (PBN Manual) and listed in the Aircraft Flight Manual.

2019

Operational Authorizatio n

-	Operational

Aircraft operator

ANSP

Authorization for PBN approaches (with advanced capabilities)

Operations based on RNP AR APCH require a specific approval. Advanced- RNP requires an operational authorization Reference: Doc 9997 (PBN Ops Approval Manual)

2019

Training	-	Training requirements for PBN

Approaches (with advanced capabilities)

Training for ATCOs, procedure designers, airspace planners and crew to fly PBN Approaches (with advanced capabilities). References: Doc 9613 (PBN Manual).

Additional training on the use of advanced capabilities in procedure design: PANS- OPS Vol II Doc 9905 (Required Navigation Performance Authorization Required (RNP AR) Procedure Design Manual) Doc 9992 (Manual on the Use of Performance-Based Navigation (PBN) in Airspace Design) Doc 9906 (Vol 2)

2019

DEPLOYMENT APPLICABILITY

Operational conditions:

Any runway ends with or without existing procedures. Particularly relevant for airports situated in congested or challenging operating environments.

| Type | Operational description | Benefitting stakeholder(s) |  |  |
| --- | --- | --- | --- | --- |
| Direct benefits | Increased capacity of the airport under adverse conditions by Improving access to airports not served by instrument approach procedures. | Airport operator | ANSP | Airspace user |
|  | Contingencies afforded by additional procedures not reliant on ground based radio aids. | Airport operator | ANSP | Airspace user |
| Indirect benefits | Increase in runway capacity due to the possible avoidance of sensitive and critical areas for ground based navigation aids without increasing length of flying routes. | Airport operator | ANSP |  |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Equip additional RWY ends with instrument approaches | ++ | KPI10: Airport peak throughput |
| Capacity | Capacity, throughput & utilization | Reduce approach minima (ceiling & visibility) | ++ | KPI10: Airport peak throughput |

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

APTA-B1/2	PBN SID and STAR procedures (with advanced capabilities)

Operational

Sixth edition of the GANP

Main Purpose	Advanced PBN functionality further supports flexibility of route placements in airspace design.

New Capabilities		Use of advanced functionality to enable more flexibility in airspace design, such as RF legs outside of the Final Approach Segment. Introduction of RNP AR Departures.

Description		This element represents the use advanced features of PBN in design of arrival procedures to provide more flexibility in airspace design, leading to greater efficiency in the terminal area and increased capacity.

Advanced RNP is the navigation specification which encompasses all elements of PBN (but excluding RNP AR APCH). It requires an FMS based on a TSO-C115d.

With A-RNP Ops approval, an operator can conduct any PBN operation except RNP AR APCH. RNP AR APCH requires a separate Ops approval.

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

APTA-B0/2 - PBN SID and STAR procedures (with basic capabilities)

Relation-information need

AMET-B0/1 - Meteorological observations products

Relation-information need

AMET-B0/2 - Meteorological forecast and warning products

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational Design

procedures

PBN SID and

STAR

Procedures (with advanced capabilities) procedure design and use

These procedures should be designed

and used as specified in Doc 8168 (PANS-OPS Vol II and I) or equivalent.

ANSP

2019

Operational Design

procedures

PBN SID and

STAR

Procedures (with advanced capabilities) validation, approval, and publication

A flight inspection and/or validation of the

procedures might be required before publication. The publication of the procedures should follow Annex 4. Ref: ICAO Doc 8071 (Volume I — Testing of Ground-based Radio Navigation Systems) Doc 9906 (Quality Assurance Manual for Flight Procedure Design). Doc 9906 Vol 6 ( Quality Assurance Manual for Flight Procedure Design)

ANSP   CAA 	2019

Operational Operations	SOPs for PBN

procedures	SID/STAR (with advanced capabilities)

Procedures for the crew to follow to fly a

PBN SID/STAR. Defined in the Ops Manual Reference: Doc 9613 (PBN Manual)

2019

Aircraft operator

Operational procedures

Operations	Contingencies

Aircraft manufacturer

for PBN SID/STAR (with advanced capabilities)

Procedures for the crew to follow in case of abnormal events. Defined in the Ops Manual. Reference: Doc 9613 (PBN Manual)

2019

Aircraft operator

Airborne system capability

-	Aircraft capability for PBN SID/STAR (with advanced capabilities)

Aircraft eligible for applicable Navigation specification as defined in Doc 9613 (PBN Manual) and listed in the Aircraft Flight Manual.

2019

Aircraft operator

Operational Authorizatio n

-	Operational

Authorization for PBN SID/STAR

(with advanced capabilities)

Aircraft operator flying a PBN SID/STAR should have an operational authorization related to the specified performance of the procedure, as described in Doc 9997 (PBN Ops Approval Manual)

2019

Aircraft operator

CAA

Ground system infrastructur e

Navigation	NAVAIDS to

Aircraft operator

support the applicable navigation specification used for SID/STAR (with advanced capabilities)

Depending on the navigation specification used, suitable ground based navigational aids will be required. Reference: Doc 9613 (PBN Manual)

ANSP 	2019

Training	-	Training requirements for PBN SID/STAR

(with advanced capabilities)

Training for ATCOs, procedures designers and crew to fly PBN SID/STAR (with advanced capabilities). References: Doc 9613 (PBN Manual); Additional training for advanced capabilities as defined in Doc 9613 (PBN Manual) Reference: Doc 8168 (PANS OPS Vol II) Doc 9906 (Vol 2)

ANSP

2019

DEPLOYMENT APPLICABILITY

Operational conditions:

Applicable in all terminal areas. Particularly suited to airports situated in congested or challenging operating environments.

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
|  | Flexible airspace planning and development. | ANSP |

| Direct benefits Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
|  | Increase in terminal airspace capacity due to the ability to develop optimized routes independent of the ground based navigation. | ANSP Airspace user |
| Indirect benefits | Contingencies afforded by additional procedures not reliant on ground based radio aids. | Airport operator ANSP |
|  | Environmental benefits; paths can be optimized to stay away from noise sensitive areas, and shorter distances to connect from en-route to approach for reduced emissions. |  |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Increase airport arrival rate | ++ | KPI11: Airport throughput efficiency |
| Capacity | Capacity, throughput & utilization | Mitigate local airspace capacity constraints if this is the problem | ++ | KPI10: Airport peak throughput |
| Capacity | Capacity, throughput & utilization | Mitigate noise constraints if this is the problem | ++ | KPI10: Airport peak throughput |
| Efficiency | Vertical flight efficiency | Reduce permanent (airspace and approach procedure design) and semi-permanent (ATFCM measures) altitude constraints along the descent portion of traffic flows, in en- route and terminal airspace | ++ | KPI19: Level-off during descent |
| Efficiency | Vertical flight efficiency | Reduce permanent (airspace and departure procedure design) and semi-permanent (ATFCM measures) altitude constraints (level capping) along the climb portion of traffic flows, in terminal and en-route airspace | ++ | KPI17: Level-off during climb |

APTA-B1/4	CDO (Advanced)	Operational

Sixth edition of the GANP

Main Purpose	Increase the ability CDO operations to contribute to terminal airspace efficiency.

New Capabilities	High accuracy vertical navigation permits more precise paths to be flown by arriving aircraft.

Description		This element builds on the basic CDO capabilities and adds advanced vertical path management. This gives the ability to more precisely define the path flown by arriving aircraft, which allows for more flexible and efficient use of airspace. Compromise between aircraft optimal performance and airspace optimal efficiency can be achieved.

PBN with vertical navigation (VNAV) which is an altimetry-based capability is used and allows an equipped aircraft to precisely descend on a vertical path, as computed by avionics equipment, while providing the flight crew with navigation performance information through avionics. To this end, vertical windows in procedure design enable CDOs, particularly where SID/STARs cross each other.

Initially the benefits will be realised for airspace outside of the TMA, and for less congested environments.

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS 		OPERATIONS

Tactical-During ops 	 Arrival

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Evolution	APTA-B0/4 - CDO (Basic)

Relation-operational need	APTA-B0/2 - PBN SID and STAR procedures (with basic capabilities) Relation-operational benefit	RSEQ-B0/3 - Point merge

Relation-information need	AMET-B0/1 - Meteorological observations products Relation-information need	AMET-B0/2 - Meteorological forecast and warning products

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Design	Development of CDO (advanced) procedures

These procedures should be designed and used as specified in Doc 8168 (PANS-OPS Vol II and I) or equivalent, with reference to Doc 9931 (CDO Manual)

ANSP

CAA

ANSP

Aircraft operator

2019

Operational procedures

Design	CDO (advanced) procedures validation, approval and publication

A flight inspection and/or validation of the procedures might be required before publication. The publication of the procedures should follow Annex 4. Ref: ICAO Doc 8071 (Volume I — Testing of Ground-based Radio Navigation Systems) Doc 9906 (Quality Assurance Manual for Flight Procedure Design). Doc 9906 Vol 6 ( Quality Assurance Manual for Flight Procedure Design)

2019

Operational procedures

Operations	SOPs for CDO

Aircraft operator

(advanced) operations

Procedures for the crew to follow to facilitate the flying of a CDO. OPS Manual defines SOPs

2019

Aircraft operator

Operational procedures

-	Contingencies for CDO (advanced)

Procedures for the crew to follow in case of abnormal events. Reference: Ops Manual defines SOPs

2019

Airborne system capability

Aircraft system

Aircraft Capability for CDO (advanced)

Eligibility for the applicable PBN navigation specification with vertical navigation (VNAV) capability Reference: Doc 9613 (PBN Manual)

2019

Aircraft operator

Aircraft manufacturer

Ground system infrastructur e

Navigation	NAVAIDS to

Aircraft operator

support the applicable navigation specification used for CDO (advanced)

Depending on the navigation specification used, suitable ground based navigational aids will be required. Reference Doc 9613 (PBN Manual).

ANSP 	2019

Training	-	Training requirements for CDOs

(advanced)

Crew trained to fly CDOs (advanced) CDO (advanced) training for Air traffic controllers CDO (advanced) training for procedure designers. References: Doc 9906 (Vol 2) Doc 8168 (PANS OPS Vol II)

ANSP

2019

DEPLOYMENT APPLICABILITY

Operational conditions:

Congested terminal airspace with existing PBN STAR routings already utilizing CDO operations.

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
|  | Optimized vertical profile. | ANSP Airspace user |

| Type Direct benefits | Operational description | Benefitting stakeholder(s) |  |
| --- | --- | --- | --- |
|  | Increase in terminal airspace capacity from reduced vertical airspace requirements. | ANSP | Airspace user |
|  | Reduced fuel consumption. | Airspace | user |
| Indirect benefits | Increase in terminal area efficiency from reduced ATC intervention in the aircraft routing and vertical profile. | ANSP | Airspace user |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Vertical flight efficiency | Avoid efficiency penalties attributable to non-optimum ToD (descent starts before or after the optimum ToD) | ++ | KPI19: Level-off during descent |
| Efficiency | Vertical flight efficiency | Avoid tactical lengthening of arrival path (eg vectoring, holding, trombone extension) because this leads to level flight | ++ | KPI19: Level-off during descent |
| Efficiency | Vertical flight efficiency | Reduce descent inefficiency attributable to altitude constraints imposed by ATM | ++ | KPI19: Level-off during descent |

Sixth edition of the GANP

Main Purpose

Increase the ability CCO operations to contribute to terminal airspace efficiency.

New Capabilities	High accuracy vertical navigation permits more precise paths to be flown by departing aircraft.

Operational

CCO (Advanced)

APTA-B1/5

Description 		This element builds on the basic CCO capabilities and adds advanced vertical path management. This gives the ability to more precisely define the path flown by departing aircraft, which allows for more flexible and efficient use of airspace. Compromise between aircraft optimal performance and airspace optimal efficiency can be achieved.

PBN with vertical navigation (VNAV) which is an altimetry-based capability is used and allows an equipped aircraft to precisely ascend on a vertical path, as computed by avionics equipment, while providing the flight crew with navigation performance information through avionics. To this end, vertical windows in procedure design enable CCOs, particularly where SID/STARs cross each other.

Initially the benefits will be realised for airspace outside of the TMA, and for less congested environments.

Maturity Level 	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

APTA-B0/5 - CCO (Basic)

Relation-operational need

APTA-B0/2 - PBN SID and STAR procedures (with basic capabilities)

Relation-information need

AMET-B0/1 - Meteorological observations products

Relation-information need

AMET-B0/2 - Meteorological forecast and warning products

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational Design

procedures

Development of

CCO (advanced) procedures

These procedures should be designed	 ANSP

and used as specified in Doc 8168 (PANS-OPS Vol II and I) or equivalent, with reference to Doc 9993 (CCO Manual)

2019

Aircraft operator

Operational procedures

Design	CCO (advanced) procedures validation, approval and publication

A flight inspection and/or validation of the procedures might be required before publication. The publication of the procedures should follow Annex 4. Ref: ICAO Doc 8071 (Volume I — Testing of Ground-based Radio Navigation Systems) Doc 9906 (Quality Assurance Manual for Flight Procedure Design). Doc 9906 Vol 6 ( Quality Assurance Manual for Flight Procedure Design)

2019

Operational procedures

Operations	SOPs for CCO

Aircraft operator

(advanced) operations

Procedures for the crew to follow to facilitate the flying of a CCO. OPS Manual defines SOPs

2019

Aircraft operator

Operational procedures

Operations	Contingencies

Aircraft manufacturer

for CCO (advanced)

Procedures for the crew to follow in case of abnormal events. Reference: Ops Manual defines SOPs

2019

Airborne system capability

Aircraft system

Aircraft Capability for CCO (advanced)

Eligibility for the applicable PBN navigation specification with vertical navigation (VNAV) capability Reference: Doc 9613 (PBN Manual)

2019

Aircraft operator

Ground system infrastructur e

Navigation	NAVAIDS to

Aircraft operator

support the applicable navigation specification used for CCO (advanced)

Depending on the navigation specification used, suitable ground based navigational aids will be required. See Doc 9613 (PBN Manual) for details.

ANSP 	2019

Training	-	Training requirements for CCOs

(advanced)

Crew trained to fly CCOs (advanced). Training to support the CCO concept. ATC trained to provides CCOs (advanced). Training to support the CCO concept. CCO (advanced) training for procedure designers. Reference: Doc 9906 (Vol 2)

ANSP

2019

DEPLOYMENT APPLICABILITY

Operational conditions:

Congested terminal airspace with existing PBN SID routings already utilizing CCO operations.

Main intended benefits:

CAA

ANSP

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Optimized vertical profile. | ANSP Airspace user |
|  | Increase in terminal airspace capacity from reduced vertical airspace requirements. | ANSP Airspace user |

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
|  | Reduced fuel consumption. | Airspace user |
| Indirect benefits | Increase in terminal area efficiency from reduced ATC intervention in the aircraft routing and vertical profile. | ANSP Airspace user |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Vertical flight efficiency | Reduce permanent (airspace and departure procedure design) and semi-permanent (ATFCM measures) altitude constraints (level capping) along the climb portion of traffic flows, in terminal and en-route airspace | ++ | KPI17: Level-off during climb |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

APTA-B2/1	GBAS CAT II/III precision approach procedures	Operational

Sixth edition of the GANP

Main Purpose		Definition of approach procedures with Cat II and III minima based on ground-based augmentation system (GBAS). The goal of GBAS implementation is to provide an alternative to the Instrument Landing System (ILS) supporting the full range of operations.

New Capabilities	Definition of approach procedures with Cat II and III minima based on ground-based augmentation system (GBAS).

Description		The GBAS system provides several advantages such as increased coverage (all runways on an airport) and reduction of time between approaches. It can provide precision approaches where ILS cannot due to terrain constraints. The development of approach procedures with CAT II and III minima based on GBAS will allow approach procedures with lower minima even in challenging obstacle-environments and reduce the limitations due to the infrastructure constraints.

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology option

NAVS-B2/1 - Dual Frequency Multi Constellation (DF MC) GBAS

Relation-technology option

NAVS-B1/1 - Extended GBAS

Relation-information need

AMET-B2/1 - Meteorological observations information

Relation-information need

AMET-B2/2 - Meteorological forecast and warning information

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Design	GBAS CAT II/III precision approach procedure design and use

These procedures should be designed and used as specified in Doc 8168 (PANS-OPS Vol II) or equivalent.

ANSP 	2025

Operational procedures

Design	GBAS CAT II/III precision approach procedure validation, approval and publication

A flight inspection and/or validation of the procedures might be required before publication. The publication of the procedures should be as described in Doc 10066 (PANS AIM). Ref: ICAO Doc

8071 (Volume I — Testing of Ground- based Radio Navigation Systems) Doc 9906 (Quality Assurance Manual for Flight Procedure Design). Doc 9906 Vol 6 ( Quality Assurance Manual for Flight Procedure Design)

ANSP   CAA 	2025

Operational procedures

Operations	SOPs for GBAS

CAT II/III

Aircraft operator

precision approach procedures

Procedures for the crew to follow to fly a Cat I Precision Approach. Reference: Ops Manual defines SOPs

Aircraft operator

2025

Operational procedures

Operations	Contingencies

for GBAS CAT II/III precision approach procedures

Procedures for the crew to follow in case of abnormal events. Reference: Ops Manual defines SOPs

2025

Airborne system capability

-	Aircraft capability for GBAS CAT II/III precision

GBAS receiver to support CAT II/III requirements

2025

|  |  | approach procedures |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ground | Navigation | Ground based | GBAS Ground Station Ref Annex 10 Vol I | Airport operator | 2025 |
| system |  | system for GBAS |  | ANSP |  |
| infrastructur e |  | CAT II/III precision approach procedures |  |  |  |
| Training | - | Training | Crew trained to fly GBAS CAT II/III | ANSP | 2025 |
|  |  | requirements for GBAS CAT II/III | precision approach procedures. Reference: Doc 8168 (Vol I) GBAS CAT | Aircraft operator |  |

precision approach procedures

II/III precision approach procedures for ATC GBAS CAT II/III precision approach training for Procedure designers, Airspace planners. Ref : Doc 8168 (PANS OPS Vol II); Doc 9906 (Vol 2)

Aircraft operator

Aircraft manufacturer

### DEPLOYMENT APPLICABILITY

#### Operational conditions:

Runway ends with high number of hours operating under adverse weather conditions but without the facility to implement traditional Precision approach landing systems (due to terrain or other issues).

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Allows continued approach and landing operations when weather minimums are below CAT I. | ANSP Airspace user |
|  | Reduced probability of go arounds. | ANSP Airspace user |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Equip additional RWY ends with instrument approaches | ++ | KPI10: Airport peak throughput |
| Capacity | Capacity, throughput & utilization | Reduce approach minima (ceiling & visibility) | ++ | KPI10: Airport peak throughput |

APTA-B2/2	Simultaneous operations to parallel runways	Operational

Sixth edition of the GANP

Main Purpose	Introduction of Approach Procedures with Vertical Guidance (APVs) to parallel runways to permit parallel operations without ground based infrastructure.

New Capabilities	Introduces the capability to conduct simultaneous approach operations to parallel runways using any combination of APV and/or precision approach.

Description		This element adds the facility to use APV procedures (based on RNP AR APCH and RNP APCH) in parallel approach operations.

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Arrival

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-operational need	APTA-B0/1 - PBN Approaches (with basic capabilities) Relation-operational benefit	APTA-B1/1 - PBN Approaches (with advanced capabilities) Relation-information need	AMET-B2/1 - Meteorological observations information Relation-information need	AMET-B2/2 - Meteorological forecast and warning information

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Design	Development of approach procedures to support simultaneous operations

Procedures developed according to published criteria and guidance Reference Doc 8168 (PANS OPS Vol II and III) Doc 4444 (PANS ATM) Doc 9643

(Manual on Simultaneous Operations on Parallel or Near-Parallel Instrument Runways)

ANSP 	2020

| Operational | Design | Validation, | A flight inspection and/or validation of the | ANSP | CAA | 2020 |
| --- | --- | --- | --- | --- | --- | --- |
| procedures |  | approval and | procedures might be required before |  |  |  |
|  |  | publication of | publication. The publication of the |  |  |  |
|  |  | approach | procedures should follow Annex 4. Ref: |  |  |  |
|  |  | procedures to | ICAO Doc 8071 (Volume I — Testing of |  |  |  |
|  |  | support | Ground-based Radio Navigation |  |  |  |
|  |  | simultaneous | Systems) Doc 9906 (Quality Assurance |  |  |  |
|  |  | operations | Manual for Flight Procedure Design). Doc |  |  |  |
|  |  |  | 9906 Vol 6 ( Quality Assurance Manual for |  |  |  |
|  |  |  | Flight Procedure Design) |  |  |  |

Operational procedures

Operations	Operational

procedures for crew to conduct simultaneous operations

SOPs defined in the OPS manual Reference: Doc 8168 (PANS OPS Vol III)

Aircraft operator

2019

Operational procedures

Operations	Operational

Aircraft manufacturer

Procedures for ATC to conduct simultaneous operations

Reference: Doc 4444 (PANS ATM)	 ANSP 	2019

Airborne system capability

Aircraft system

Aircraft capability to conduct simultaneous operations

Eligibility for applicable PBN Navigation specification as required (depending on design of procedures) Reference: Doc 9613 (PBN Manual)

2019

Aircraft operator

Operational Authorizatio n

-	Operational

Authorization for simultaneous operations

Aircraft operator conducting simultaneous operations based on a PBN Navigation specification should have an operational authorization related to the specified performance of the procedure, as described in Doc 9997 (PBN Ops Approval Manual)

CAA

2019

Ground system infrastructur e

Navigation	NAVAIDS to

support the applicable navigation specification used for simultaneous operations

SBAS ground station if required GBAS ground station if required

ANSP 	2019

Training	-	Training requirements for simultaneous oeprations

Training for pilots to conduct simultaneous operations. Reference: Reference: Doc 8168 (PANS OPS Vol III) Training for ATC to conduct simultaneous operations Reference: Reference: Doc 4444 (PANS ATM)

ANSP 	2019

Aircraft operator

DEPLOYMENT APPLICABILITY

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Increased capacity at the aerodrome with closely spaced parallel runways by use of simultaneous approaches. | ANSP Airspace user |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Use other RWY configuration if available | ++ | KPI10: Airport peak throughput |

Operational conditions:

Aerodromes with two or more parallel runways with sufficient demand or other operational need to conduct simultaneous approaches.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

APTA-B2/3	PBN Helicopter Steep Approach Operations	Operational

Sixth edition of the GANP

Main Purpose	Helicopters can use steeper approach angles allowing additional flexibility for unique approach operations. By allowing modern helicopters to fly steeper approach angles, it will be possible to safely fly over higher obstacles or to decrease the size of the helicopter noise footprint. Where beneficial, PinS and steeper approach can also be combined.

New Capabilities	Application of steeper helicopter approach design procedures to allow helicopters to take advantage of their higher approach angle capability.

Description	PBN procedures specific to helicopter operations with approach angles exceeding 6.3 degrees.

May be used in combination with point in space procedures.

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Arrival

### DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

APTA-B0/6 - PBN Helicopter Point in Space (PinS) Operations

Relation-information need

AMET-B2/1 - Meteorological observations information

Relation-information need

AMET-B2/2 - Meteorological forecast and warning information

ENABLERS

Aircraft operator

ANSP

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Training	-	Training requirements for Helicopter PBN Point in Space (PinS)

Crew trained to fly Helicopter PBN Point in Space (PinS) procedures. Ref.: As defined in Doc 8168 (PANS OPS Vol I) Helicopter PBN Point in Space (PinS) training for ATC Helicopter PBN Point in Space (PinS) training for procedure designers, Airspace planners.

References: Doc 8168 (PANS OPS Vol II); Doc 9906 (Vol 2).

2013

Operational procedures

Design	Procedures design for PBN Helicopter Steep Approach Operations

These procedures should be designed and used as specified in Doc 8168 (PANS-OPS Vol II and I) or equivalent.

ANSP 	2022

Operational procedures

Design	Validation, approval and publication of procedures for PBN Helicopter Steep Approach Operations

A flight inspection and/or validation of the procedures might be required before publication. The publication of the procedures should follow Annex 4. Ref: ICAO Doc 8071 (Volume I — Testing of Ground-based Radio Navigation Systems) Doc 9906 (Quality Assurance Manual for Flight Procedure Design). Doc 9906 Vol 6 ( Quality Assurance Manual for Flight Procedure Design)

ANSP   CAA 	2019

Operational procedures

Operations	SOPs for PBN

Aircraft operator

Helicopter Steep Approach Operations

Procedures for the crew to follow to fly a PBN Helicopter Steep Approach. Defined in the Ops Manual

2019

Aircraft operator

Operational procedures

Operations	Contingencies

for PBN Helicopter Steep Approach Operations

Procedures for the crew to follow in case of abnormal events. Defined in the Ops Manual

2019

Airborne system capability

-	Aircraft capability for PBN Helicopter Steep Approach Operations

Eligibility for the applicable PBN navigation specification (if required) and vertical path capability, as defined in Doc 9613 (PBN Manual) and listed in the Aircraft Flight Manual (AFM)

2019

Aircraft operator

Aircraft manufacturer

DEPLOYMENT APPLICABILITY

#### Operational conditions:

Obstacle rich environments containing helicopter landing locations.

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Improve access to helicopter landing sites through approach procedures with improved minima. | Airspace user |
|  | Reduced number of diversions. | ANSP Airspace user |
|  | Helicopter procedures with steeper approach angles allow the aircraft to safely fly over higher obstacles and to decrease the size of the helicopter noise foot print. | Local community |
| Indirect benefits | Airspace capacity enhancement through design flexibility and removal of existing operational restrictions. | ANSP Airspace user |
|  | Enhanced helicopter operations availability and resilience. | Airspace user |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Mitigate local airspace capacity constraints if this is the problem | ++ | KPI10: Airport peak throughput |
| Capacity | Capacity, throughput & utilization | Reduce approach minima (ceiling & visibility) | ++ | KPI10: Airport peak throughput |

Sixth edition of the GANP

Performance based aerodrome operating minima Operational

– Advanced aircraft with SVGS

APTA-B2/4

Main Purpose  Use of advanced features on aircraft permit operations using lower than standard minima on existing procedures. This builds on the Block 0 element for PB AOM (Advanced Aircraft) and enables the use of Synthetic Vision Guidance Systems (SVGS).

New Capabilities   Definition of more precise approaches down to SA CAT I minima by use of SVGS.

Description 		This element builds on the PB AOM concept to allow operations for advanced aircraft utilizing the concept of operational credit. The use of guidance systems based on synthetic vision expands on the previous capability to use SVS for situational awareness only.

Maturity Level 	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? No

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

APTA-B0/7 - Performance based aerodrome operating minima – Advanced

aircraft

Relation-operational need

APTA-B0/3 - SBAS/GBAS CAT I precision approach procedures

Relation-information need

AMET-B0/1 - Meteorological observations products

Relation-information need

AMET-B0/2 - Meteorological forecast and warning products

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Regulatory

provisions

National

regulatory framework

Operational

credits

Provisions for operational credits to

enable lower minima based on advanced aircraft capabilities. Reference: Annex 6 Part I

CAA

2013

Operational procedures

Operations	SOPs for

Aircraft operator

Performance- based Aerodrome Operating Minima (Advanced aircraft with SVGS)

Procedures for the crew to operate to minima determined by the combination of aircraft equipage and ground infrastructure. Defined in the Ops Manual. Reference: Doc 9365 (AWO Manual)

2024

Operational procedures

Operations	Contingency

procedures for Performance- based Aerodrome Operating Minima (Advanced aircraft with SVGS)

Procedures for the crew to follow in case of abnormal events. Defined in the Ops Manual

2024

Operational procedures

Operations	ATC procedures

for Performance- based Aerodrome Operating Minima (Advanced aircraft with SVGS)

Procedures for ATC to use in order to facilitate the use of performance-based minima at aerodromes. Low visibility operating plan for aerodrome ATC

ANSP 	2024

Operational procedures

Operations	Aerodrome

Aircraft manufacturer

procedures for Performance- based Aerodrome Operating Minima (Advanced aircraft with SVGS)

Procedures for ground operations by aircraft with advanced capabilities

Airport operator 2024

Airborne system capability

Aircraft system

Advanced aircraft capability with SVGS

SBAS and GBAS as required SVGS and HUD installation Reference: Doc 9365 (AWO Manual)

2024

Aircraft operator

Aircraft operator

Ground system infrastructur e

Navigation	NAVAIDS to

support the intended operation with SVGS

Pre threshold terrain information for advanced aircraft operations SBAS/GBAS ground stations (as required)

Airport operator 2024

ANSP

Training	-	Training requirements for Performance- based aerodrome operating minima (Advanced Aircraft with SVGS)

Crew trained to fly using Performance- based Aerodrome Operating Minima (Advanced aircraft with SVGS) Training on the use of advanced aircraft equipment such as SVGS Reference: Doc 9365 (AWO Manual) ATC trained to understand implications of Performance-based Aerodrome Operating Minima (Advanced aircraft with SVGS) Training for ATC on the application of operational credits for advanced aircraft and the effect on determining minima used by crews.

ANSP

Aircraft operator

2024

### DEPLOYMENT APPLICABILITY

#### Operational conditions:

Operators wanting to obtain better minima by investing in advanced aircraft.

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Improved airport access from reduced minima (RVR, DH) for equipped operators. | Airspace user |
| Indirect benefits | Increased capacity of the airport under adverse conditions. | Airport operator ANSP Airspace user |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Reduce approach minima (ceiling & visibility) | ++ | KPI10: Airport peak throughput |

Sixth edition of the GANP

Main Purpose

Simultaneous independent parallel approach paths to parallel or near parallel runways based on

RNP capabilities are based on 3D operations. The use of A-RNP and/or RNP APCH to provide demonstrated track keeping performance will safely provide lateral performance and could enable the removal of the vertical guidance requirement for specific operations. This would enable improved access, provide flexibility in degraded environments and could optimized the vertical profile leading to increased operational efficiency and reduced noise nuisance.

New Capabilities	Independent closely spaced 2D RNP approach operations.

Operational

Parallel approaches without vertical guidance

APTA-B3/1

Description 		This element overcomes the necessity to require 3D operations for parallel approach operations providing increased flexibility and supporting aircraft in a degraded environment.

Maturity Level 	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

APTA-B0/1 - PBN Approaches (with basic capabilities)

Relation-operational benefit

APTA-B1/1 - PBN Approaches (with advanced capabilities)

Relation-information need

AMET-B2/1 - Meteorological observations information

Relation-information need

AMET-B2/2 - Meteorological forecast and warning information

### ENABLERS

Aircraft operator

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational Authorizatio n

-	Operational

Authorization for PBN

specification

Aircraft operator flying a PBN procedure should have an operational authorization related to the specified performance of the procedure, as described in Doc 9997 (PBN Ops Approval Manual).

CAA

2022

Regulatory provisions

National regulatory framework

National framework amendment for parallel approaches without vertical guidance

National regulation amendment for parallel approaches without vertical guidance.

References: DOC. 4444 – Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM)

CAA 	2026

Operational procedures

Design	PBN procedures design and use for parallel approaches without vertical guidance.

These operational procedures should be designed and used as specified in Doc 8168 (PANS-OPS Vol II and I) or

equivalent.

ANSP 	2026

| Operational | Design | PBN procedure | A flight inspection and/or validation of the | ANSP | CAA | 2026 |
| --- | --- | --- | --- | --- | --- | --- |
| procedures |  | validation, | procedures might be required before |  |  |  |
|  |  | approval and | publication. The publication of the |  |  |  |
|  |  | publication for | procedures should follow Annex 4. |  |  |  |
|  |  | parallel | References: ICAO Doc 9906 (Quality |  |  |  |
|  |  | approaches | Assurance Manual for Flight Procedure |  |  |  |
|  |  | without vertical | Design). |  |  |  |
|  |  | guidance. |  |  |  |  |
| Operational | Design | PBN procedure | A flight inspection and/or validation of the | ANSP | CAA | 2026 |
| procedures |  | validation, | procedures might be required before |  |  |  |
|  |  | approval and | publication. The publication of the |  |  |  |
|  |  | publication for | procedures should follow Annex 4. |  |  |  |
|  |  | parallel | References: ICAO Doc 9906 (Quality |  |  |  |
|  |  | approaches | Assurance Manual for Flight Procedure |  |  |  |
|  |  | without vertical | Design). |  |  |  |
|  |  | guidance |  |  |  |  |

Operational procedures

Operations	SOPs for parallel

Aircraft operator

approaches without vertical guidance.

Procedures for the crew to follow to fly a PBN approach. Defined in the Ops Manual. Reference: Doc 9613 (PBN Manual)

2026

Aircraft operator

Operational procedures

Operations	Contingency

procedures for parallel approaches without vertical guidance

Procedures for the crew to follow in case of abnormal events. Defined in the Ops Manual. Reference: Doc 9613 (PBN Manual)

2026

Operational procedures

Separation	Application of

Aircraft manufacturer

separation for parallel approaches without vertical guidance

Procedures for separation. Reference: PANS-ATM

ANSP 	2026

Airborne system capability

Navigation	PBN Approaches (with basic capabilities)

PBN Approaches (with basic capabilities)

2021

Aircraft operator

Space system infrastructur e

-	Training

requirements for parallel approaches without vertical guidance.

Crew trained to fly the procedure. References: As defined in Doc 9613 (PBN Manual). For Air traffic controllers. References: PANS-ATM.

ANSP

2026

Aircraft operator

DEPLOYMENT APPLICABILITY

Operational conditions:

Airports with simultaneous independent parallel approach paths to parallel or near parallel runways without the need

for vertical guidance. To enhance access and capacity at aerodromes with parallel or near parallel runways.

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Safety | Aircraft operator General citizen |
|  | Access | Airport operator Aircraft operator General citizen |
|  | Capacity | Airport operator ANSP Aircraft operator |
| Indirect benefits | Environment | General citizen |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Increase airport arrival rate | ++ | KPI10: Airport peak throughput |
| Efficiency | Flight time & distance | Reduce additional time in terminal airspace | ++ | KPI08: Additional time in terminal airspace |
| Efficiency | Fuel burn | Reduce fuel burn impact of impeded conditions | ++ | KPI16: Additional fuel burn |
| Efficiency | Vertical flight efficiency | Reduce ToD uncertainty by early assignment of landing RWY and approach procedure | ++ | KPI19: Level-off during descent |

Sixth edition of the GANP

Main Purpose

To provide additional flexibility to support parallel approach operations where RNP AR operations

may not be cost effective or practicable.

New Capabilities

The advent of A-RNP may allow RNP 0.3 to be coded in the initial and intermediate segments of an

approach allowing PBN to provide more flexibility in supporting parallel approach operations for non- AR operators at aerodromes with parallel runway spacings of 2224 m or greater.

Implementation of A-RNP to support non-complex Operational

simultaneous independent parallel approaches

APTA-B3/2

Description 		Parallel approach operations (Mode 1) not designed iaw ‘Established on RNP AR APCH’ must be established on the Final Approach course or track before the No Transgression Zone (NTZ) can be invoked and the 1000’ vertical separation cancelled. The performance of RNP APCH outside of the Final Approach Segment (FAS) is +/-1NM and this performance effectively minimizes the use of the RNP APCH application to support parallel approach without significant additional safety work. The A-RNP specification will provide a required navigational performance of +/-0.3 throughout the terminal airspace (missed approach might be an exception). This means that provided the aircraft are aligned with the runway centreline, by requiring A-RNP prior to the Final Approach the NTZ can be invoked earlier, the 1000’ separation cancelled, and aircraft can start their final descents from higher altitudes (and greater distances from the threshold). This should be both a cost efficiency (fuel burn) and environmental (noise) benefit.

Maturity Level 	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

APTA-B0/1 - PBN Approaches (with basic capabilities)

Relation-operational benefit

APTA-B1/5 - CCO (Advanced)

Relation-information need

AMET-B2/1 - Meteorological observations information

Relation-information need

AMET-B2/2 - Meteorological forecast and warning information

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Regulatory	National

provisions	regulatory

framework

National

framework amendment for A-RNP for independent parallel approaches

where the runway spacing of ≥ 2224m can be

assured.

National framework amendment for A-  CAA

RNP for independent parallel approaches where the runway spacing of ≥ 2224m can be assured.

2026

Operational procedures

Design	PBN procedures design and use for A-RNP for independent parallel approaches where the runway spacing of ≥ 2224m can be assured.

TPBN procedures design and use for A- RNP for independent parallel approaches where the runway spacing of ≥ 2224m can be assured.

ANSP 	2026

Operational procedures

Design	PBN approval and publication for A-RNP for independent parallel approaches where the runway spacing of ≥ 2224m can be assured..

A flight inspection and/or validation of the procedures might be required before publication. The publication of the procedures should follow Annex 4.

References: ICAO Doc 9906 (Quality Assurance Manual for Flight Procedure Design).

ANSP   CAA 	2026

Operational procedures

Operations	SOPs for A-RNP

Aircraft operator

for independent parallel approaches where the runway spacing of ≥ 2224m can be assured.

Procedures for the crew to follow to fly a PBN approach. Defined in the Ops Manual. Reference: Doc 9613 (PBN Manual)

2026

Operational procedures

Operations	Contingency

procedures for A- RNP for independent parallel approaches where the runway spacing of ≥ 2224m can be assured.

Procedures for the crew to follow in case of abnormal events. Defined in the Ops Manual. Reference: Doc 9613 (PBN Manual)

2026

Aircraft operator

Operational procedures

Separation	Application of

separation for A- RNP for independent parallel approaches where the runway spacing of ≥ 2224m can be assured.

Procedures for separation. Reference: PANS-ATM

ANSP 	2026

DEPLOYMENT APPLICABILITY

Operational conditions:

Airports with non-complex simultaneous independent parallel approaches where the runway spacing of ≥ 2224m can be assured. To provide additional flexibility to support parallel approach operations where RNP AR operations may not be cost effective or practicable.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Operational -

Authorizatio n

Operational

Authorization for A-RNP

Aircraft operator flying a PBN procedure

should have an operational authorization related to the specified performance of the procedure, as described in Doc 9997 (PBN Ops Approval Manual).

CAA

Aircraft operator

2022

Airborne	Navigation

system capability

PBN

Approaches for A-RNP

PBN Approaches for A-RNP

2026

Space	-

system infrastructur e

Training

requirements for A-RNP for independent parallel approaches (RWY spacing of

≥ 2224m)

Crew trained to fly the procedure.

References: As defined in Doc 9613 (PBN Manual). For Air traffic controllers. References: PANS-ATM.

ANSP

Aircraft operator

2026

Aircraft operator

Aircraft manufacturer

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Flexibility | Airport operator ANSP Aircraft operator |
|  | Access | Airport operator Aircraft operator General citizen |
|  | Capacity | Airport operator ANSP Aircraft operator |
| Indirect benefits | Safety | Aircraft operator |
|  | Environment | General citizen |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Increase airport arrival rate | ++ | KPI10: Airport peak throughput |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Flight time & distance | Reduce additional time in terminal airspace | ++ | KPI08: Additional time in terminal airspace |
| Efficiency | Fuel burn | Reduce fuel burn impact of impeded conditions | ++ | KPI16: Additional fuel burn |
| Efficiency | Vertical flight efficiency | Reduce ToD uncertainty by early assignment of landing RWY and approach procedure | ++ | KPI19: Level-off during descent |

GANP PORTAL

# ASBU ELEMENTS

| ASUR | COMI | COMS | CSEP |
| --- | --- | --- | --- |

| Functional Description | Enablers | Deployment Applicability | Performance Impact Assessment |
| --- | --- | --- | --- |

## ASUR

ASUR-B0/1	Automatic Dependent Surveillance – Broadcast (ADS-B)

Technology

Sixth edition of the GANP

Main Purpose		To support the provision of Air Traffic Services and operational applications at reduced cost and increased surveillance coverage.

New Capabilities		ADS-B provides precise position/velocity information in all airspace (accuracy not range-dependent as with radar). It also provides aircraft call sign and precise position/velocity information to nearby aircraft with ADS-B-In receivers.

ADS-B can also support State aircraft airspace access, however it should, when possible, leverage benefits from dual-use of State aircraft capabilities to reduce cost and technical impact.

Description	ADS-B provides an aircraft’s identification, position, altitude, velocity, and other information to any receiver (airborne or ground) within range. The broadcasted aircraft position/velocity is normally based on the global navigation satellite system (GNSS) and transmitted at least once per second.

Maturity Level	Ready for implementation

Human Factor Considerations

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Taxi-out   Departure   En-route   Arrival   Taxi-in

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-operational need	NAVS-B0/3 - Aircraft Based Augmentation Systems (ABAS) Relation-benefit	NAVS-B0/2 - Satellite Based Augmentation Systems (SBAS)

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Ground system infrastructur e

Surveillance	ADS-B ground

stations

ADS-B ground stations receive information from aircraft and transmit it to one or more Service Delivery Points Reference material: Technical standards and guidance material: ICAO Annex 10 Volume IV Chapter 2,3 and 5 ICAO Doc. 9871 Technical Provisions for Mode S Services and Extended Squitter RTCA/EUROCAE MOPS: DO-260/ED- 102, DO-260A, or DO-260B/ED-102A EUROCAE ED-129, ED-129A or ED-

129B ICAO Doc. 9924 Aeronautical Surveillance Manual

ANSP 	2008

Ground system infrastructur e

Surveillance	*Service Delivery

Point(s) for ADS- B information

Service Delivery Point(s) receive ADS-B information provides it to ATC automation for processing and display to controller Reference material: Guidance material: ICAO Doc. 9924 Aeronautical Surveillance Manual

ANSP 	2008

Ground system infrastructur e

Technical systems

HMI that supports controller awareness

Human Machine Interface (HMI) of the Air Traffic Controller Working Position (ATCo CWP) Reference: Guidance material: ICAO Doc. 9924 Aeronautical Surveillance Manual

ANSP 	2008

Airborne system capability

Surveillance  SSR Mode S

transponder with extended squitter version 0, version

1 and version 2

Reference: Technical standards and guidance material: ICAO Annex 10 Volume IV Chapter 2,3 and 5 ICAO Doc. 9871 Technical Provisions for Mode S Services and Extended Squitter RTCA/EUROCAE MOPS: DO-260/ED- 102, DO-260A, or DO-260B/ED-102A

ICAO Doc. 9924 Aeronautical Surveillance Manual

2008

Aircraft operator

Aircraft manufacturer

Training	-	Training requirements ADS-B

implementation

Depending on the ANSP implementation, some controller training on new symbology may be required. If phraseology is changed by an ANSP, then controller and pilot training on the new phraseology is required. If new ANSP equipment is installed, then training for maintenance personnel may be required (see ICAO Doc 8071).

ANSP 	2008

Airborne	Navigation	Basic Aviation

system	GNSS receiver

capability	with RAIM

Position source. Basic Aviation GNSS

receiver with RAIM. Such a receiver must comply with the technical performance requirements of either [E]TSO-C129, or [E]TSO-C196, or [E]TSO-C145/-C146.

(Note that the US/Europe and equivalent ADS-B mandates require more – see

FAA AC 20-165 or EASA CS-ACNS).

Aircraft

manufacturer	2008

Aircraft operator

ASUR-B0/2	Multilateration cooperative surveillance systems (MLAT)

Technology

Sixth edition of the GANP

Main Purpose	To provide an alternative to radar surveillance by using available aircraft transponders.

New Capabilities		MLAT allows cooperative surveillance in rough terrain such as in mountainous regions or on airport surfaces where surveillance systems requiring a rotating radar dish had performance difficulties. It may also be integrated with ADS-B ground stations to provide a surveillance capability that is more resilient to GNSS function failure.

Description		MLAT is a new technique providing independent cooperative surveillance. The MLAT system interrogates an aircraft and the transponder reply is received by multiple receivers located in different places. The reply’s times of arrival difference at the receivers allows the position of the source of signals to be determined, with an accuracy that is dependent on the number of receivers and their location relative to the aircraft. MLAT systems do not require a rotating radar dish and were initially deployed on airports to provide surface surveillance of aircraft. The technique is now used to provide surveillance over wide area (wide area MLAT system - WAM), sometimes in conjunction with ADS-

B. MLAT requires more ground stations than ADS-B, but has the early implementation advantage of using existing aircraft transponders.

Maturity Level	Ready for implementation

Human Factor Considerations

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Taxi-out   Departure   En-route   Arrival   Taxi-in

### DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Ground system infrastructur e

Surveillance	MLAT ground

stations

MLAT ground stations interrogate aircraft transponders and receive/process transponder replies to determine aircraft position; this information is then transmitted to Service Delivery Point(s) Reference material: Technical standards and guidance material: ICAO Annex 10 Volume IV Chapters 2,3,5 and 6. ICAO Doc 9871 Technical Provisions for Mode S Services and Extended Squitter RTCA/EUROCAE MOPS: DO-181D/ED-

Aircraft operator

Aircraft manufacturer

73C or subsequent versions ICAO Doc. 9924 Aeronautical Surveillance Manual

ANSP 	2008

Airborne system capability

Surveillance	SSR Mode A, C

and S transponders

References: Technical standards and guidance material: ICAO Annex 10 Volume IV Chapter 2,3 and 5 ICAO Doc.

2008

|  |  |  | 9871 Technical Provisions for Mode S Services and Extended Squitter RTCA/EUROCAE MOPS: DO-181D/ED- |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 73C or subsequent versions ICAO Doc. |  |  |
|  |  |  | 9924 Aeronautical Surveillance Manual |  |  |
| Ground | Surveillance | Service Delivery | Service Delivery Points(s) receive MLAT | ANSP | 2008 |
| system |  | Point (s) for | information and provide it to ATC |  |  |
| infrastructur |  | MLAT | automation for display to controller |  |  |
| e |  | information | Reference: Guidance material: ICAO |  |  |
|  |  |  | Doc. 9924 Aeronautical Surveillance |  |  |
|  |  |  | Manual |  |  |
| Training | - | Training | If new ANSP equipment is installed, then | ANSP | 2008 |
|  |  | requirements | training for maintenance personnel may |  |  |
|  |  | MLAT | be required (see Doc 8071). |  |  |
|  |  | implementation |  |  |  |

Sixth edition of the GANP

Main Purpose

To obtain additional information from an aircraft transponder in support of the provision of Air Traffic

Services.

New Capabilities	SSR-DAPS enables ATM systems to obtain additional information from an aircraft transponder, via

interrogation by a cooperative surveillance system (Mode S radar or MLAT). This additional information can be used to increase controller awareness and reduce the volume of air-ground voice communications, and/or to improve the performance of tracking systems or safety net systems such as STCA and MSAW.

Technology

Cooperative Surveillance Radar Downlink of

Aircraft Parameters (SSR-DAPS)

ASUR-B0/3

Description 		Downlink of Aircraft Parameters (DAPS) includes both Controller Access Parameters (CAPs) and System Access Parameters (SAPs). Possible CAPs include Magnetic Heading, Indicated Airspeed / Mach Number, Barometric rate of climb/descent, and Selected Altitude (which can also be consider a SAP). SAPs include Roll Angle, Track Angle Rate, True Track Angle, and Barometric Pressure Setting.

Maturity Level 	Ready for implementation

Human Factor Considerations

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   En-route   Arrival

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Airborne	Surveillance	SSR Mode A, C

system	and S

capability	transponders

References: Technical standards and

guidance material: ICAO Annex 10 Volume IV Chapter 2,3 and 5 ICAO Doc. 9871 Technical Provisions for Mode S Services and Extended Squitter RTCA/EUROCAE MOPS: DO-181D/ED-

73C or subsequent versions ICAO Doc.

9924 Aeronautical Surveillance Manual

2008

Ground	Surveillance	Mode S

system	cooperative

infrastructur	surveillance

e	system

Mode S cooperative surveillance system

with DAPS capability interrogates aircraft transponders to retrieve data; this information is then provided to the ATC automation system. Reference material: Technical standards and guidance material: ICAO Annex 10 Volume IV Chapter 2,3 and 5 ICAO Doc. 9871 Technical Provisions for Mode S Services and Extended Squitter: RTCA/EUROCAE MOPS: DO-181D/ED-73C or subsequent versions ICAO Doc. 9924 Aeronautical Surveillance Manual

ANSP

2008

Aircraft operator

Aircraft manufacturer

Ground	Technical

system	systems infrastructur

e

*HMI that

supports controller awareness for CAPs and automation processing for SAPs

Reference: Guidance material: ICAO Doc  ANSP

9924 Aeronautical Surveillance Manual

2008

Training

-

Training

requirements SSR-DAPS

implementation

Depending on the ANSP implementation,   ANSP

some controller training on new symbology or alerts may be required. If phraseology is changed, or new phraseology is introduced by an ANSP, then controller and pilot training on the new phraseology is required. If new ANSP equipment is installed, then training for maintenance personnel may be required (see Doc 8071).

2008

ASUR-B1/1	Reception of aircraft ADS-B signals from space (SB ADS-B)

Technology

Sixth edition of the GANP

Main Purpose		To provide surveillance coverage in locations where ground stations siting is not possible or not currently provided.

New Capabilities		SB ADS-B provides precise position/velocity information in airspace where it is not cost-effective or even feasible to place ground surveillance infrastructure.

Description	ADS-B provides an aircraft’s identification, position, altitude, velocity, and other information to a receiver on an orbiting satellite. The broadcasted aircraft position/velocity is normally based on the global navigation satellite system (GNSS) and transmitted at least once per second. Aircraft ADS-B signals are received on one or more orbiting satellites, and this information is passed through a data network to a Service Delivery Point at an Air Traffic Service facility (or facilities).

Maturity Level	Ready for implementation

Human Factor Considerations

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Departure   En-route   Arrival

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Evolution	ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-operational need

NAVS-B0/3 - Aircraft Based Augmentation Systems (ABAS)

Relation-benefit

NAVS-B0/2 - Satellite Based Augmentation Systems (SBAS)

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Ground system infrastructur e

Technical systems

HMI that supports controller awareness

Human Machine Interface (HMI) of the Air Traffic Controller Working Position (ATCo CWP) Reference: Guidance material: ICAO Doc. 9924 Aeronautical Surveillance Manual

ANSP 	2008

Airborne system capability

Surveillance  SSR Mode S

transponder with extended squitter version 0, version

1 and version 2

Reference: Technical standards and guidance material: ICAO Annex 10 Volume IV Chapter 2,3 and 5 ICAO Doc. 9871 Technical Provisions for Mode S Services and Extended Squitter RTCA/EUROCAE MOPS: DO-260/ED- 102, DO-260A, or DO-260B/ED-102A

ICAO Doc. 9924 Aeronautical Surveillance Manual

2008

Ground system infrastructur e

Surveillance	*ADS-B

receivers on orbiting satellites

ADS-B receivers on orbiting satellites receive information from aircraft and transfer this information to ground Service Delivery Point(s). Reference: Technical standards and guidance material: ICAO Annex 10 Volume IV Chapter 2,3 and 5 ICAO Doc. 9871 Technical Provisions for Mode S Services and Extended Squitter: RTCA/EUROCAE MOPS: DO-260/ED- 102, DO-260A, or DO-260B/ED-102A EUROCAE ED-129A or ED-129B ICAO

Doc. 9924 Aeronautical Surveillance Manual

ANSP 	2015

Aircraft operator

Aircraft manufacturer

Ground system infrastructur e

Surveillance	Service Delivery

Point(s) for satellite ADS-B

Service Delivery Point(s) receive information and transfer it to ATC automation for processing and display to controller. Reference: Guidance material: ICAO Doc. 9924 Aeronautical Surveillance Manual

ANSP 	2015

Training

-

Training

requirements SB ADS-B

If this capability is used to apply new

separation minima per PANS-ATM, controller training is required. Depending on the ANSP implementation, some controller training on new symbology may be required. If phraseology is changed by an ANSP, then controller and pilot training on the new phraseology is required. If new ANSP equipment is installed, then training for maintenance personnel may be required (see ICAO Doc 8071).

ANSP

2018

Airborne	Navigation	Basic Aviation

system	GNSS receiver

capability	with RAIM

Position source. Basic Aviation GNSS

receiver with RAIM. Such a receiver must comply with the technical performance requirements of either [E]TSO-C129, or [E]TSO-C196, or [E]TSO-C145/-C146.

(Note that the US/Europe and equivalent ADS-B mandates require more – see

FAA AC 20-165 or EASA CS-ACNS).

Aircraft

manufacturer	2008

Aircraft operator

ASUR-B2/1	Evolution of ADS-B and Mode S	Technology

Sixth edition of the GANP

Main Purpose		To provide new types of data in support of Air Traffic/MET Services and vehicle-to-vehicle applications.

New Capabilities		New aircraft/atmospheric information is provided via ADS-B and/or cooperative radar interrogation to support emerging ANSP/MET and vehicle-to-vehicle applications in an early TBO environment.

Description		RTCA and EUROCAE are working jointly to prepare the next version of the Mode S transponder MOPS and the ADS-B MOPS. Changes include: correction of issues identified since the MOPS were last published, to support new collision avoidance systems, to support new Detect and Avoid systems, to support the ADS-B-In Interval Management application, to support potential commercial space operations (in the atmosphere), to reduce 1090MHz spectrum congestion, to downlink certain meterological parameters, to improve ADS-B reception performance, and to remove unused capabilities. Within, this timeframe, ADS-B vehicle-to-vehicle applications are potentially provided in a different spectrum in lower airspace.

This evolution offers opportunities for compliance based on performance-equivalence for State aircraft.

Maturity Level	Validation

Human Factor Considerations

### PLANNING LAYERS 		OPERATIONS

Tactical-During ops

Taxi-out   Departure   En-route   Arrival   Taxi-in

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Evolution

ASUR-B0/2 - Multilateration cooperative surveillance systems (MLAT)

Evolution

ASUR-B0/3 - Cooperative Surveillance Radar Downlink of Aircraft

Parameters (SSR-DAPS)

Evolution

ASUR-B1/1 - Reception of aircraft ADS-B signals from space (SB ADS-B)

Relation-operational need

NAVS-B0/3 - Aircraft Based Augmentation Systems (ABAS)

Relation-benefit

NAVS-B0/2 - Satellite Based Augmentation Systems (SBAS)

Relation-benefit

NAVS-B2/2 - Dual Frequency Multi Constellation (DF MC) SBAS

Relation-benefit

NAVS-B2/3 - Dual Frequency Multi Constellation (DF MC) ABAS

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Ground

system infrastructur e

Technical

systems

HMI that supports

controller awareness

Human Machine Interface (HMI) of the Air	 ANSP

Traffic Controller Working Position (ATCo CWP) Reference: Guidance material: ICAO Doc. 9924 Aeronautical Surveillance Manual

2008

Airborne	Surveillance	SSR Mode S

system	transponder with

capability	extended squitter version 3

Reference: Technical standards and

guidance material: Future revision ICAO Annex 10 Volume IV Chapter 2,3 and 5 Future revision ICAO Doc. 9871 Technical Provisions for Mode S Services and Extended Squitter: RTCA/EUROCAE MOPS: DO-260C/ED-102C, DO-

181F/ED-73F (TBC) Future revision ICAO Doc. 9924 Aeronautical

Surveillance Manual

2022

Aircraft operator

Aircraft manufacturer

Ground	Surveillance	ADS-B receivers

system	and/or

infrastructur	cooperative

e	radars

ADS-B receivers and/or cooperative

radars receive information from aircraft and transfer this information to Service Delivery Point(s). Reference material: Technical standards and guidance material: Future revision ICAO Annex 10 Volume IV Chapter 2,3 and 5 Future revision ICAO Doc. 9871 Technical Provisions for Mode S Services and Extended Squitter RTCA/EUROCAE MOPS: DO-260C/ED-102C, DO-

181F/ED-73F, ED-129B (or subsequent

revision) Future revision ICAO Doc. 9924 Aeronautical Surveillance Manual

ANSP

2022

Ground

system infrastructur e

Surveillance

Service Delivery

Point(s) for ADS- B v3

Service Delivery Point(s) receive

information and transfers it to ATC automation for processing and display to controller. Reference: Guidance material: Future revision of ICAO Doc. 9924 Aeronautical Surveillance Manual

ANSP

2022

Airborne	Navigation	Basic Aviation

system	GNSS receiver

capability	with RAIM

Position source. Basic Aviation GNSS

receiver with RAIM. Such a receiver must comply with the technical performance requirements of either [E]TSO-C129, or [E]TSO-C196, or [E]TSO-C145/-C146.

(Note that the US/Europe and equivalent

ADS-B mandates require more – see FAA AC 20-165 or EASA CS-ACNS).

Aircraft

manufacturer	2008

Aircraft operator

ASUR-B2/2	New community based surveillance system for

airborne aircraft (low and higher airspace)

Technology

Sixth edition of the GANP

Main Purpose		To support the provision of separation services by operators for aircraft operating at very low altitudes (<500 ft) and higher airspace.

New Capabilities		A community based network solution for surveillance at a lower lifecycle cost is provided using operator reported positions into that network to support conflict detection and resolution. It substitutes shared information for visual recognition of other aircraft.

Description		Within this timeframe, vehicle identities/positions/velocities may be shared using an aviation network. Information is provided to the ANSP on aircraft location to detect airspace infringement (low altitudes) and support operator to operator (even if provided by third party acting on the operators behalf) conflict management. It extends to higher airspace to support new entrants.

Maturity Level	Concept

Human Factor

Considerations

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   En-route   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

NAVS-B0/3 - Aircraft Based Augmentation Systems (ABAS)

Relation-benefit

NAVS-B0/2 - Satellite Based Augmentation Systems (SBAS)

Relation-benefit

NAVS-B2/2 - Dual Frequency Multi Constellation (DF MC) SBAS

Relation-benefit

NAVS-B2/3 - Dual Frequency Multi Constellation (DF MC) ABAS

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Ground	Surveillance	“Public” network-

system	based

infrastructur	cooperative

e	surveillance

“Public” network-based cooperative

surveillance system collects, processes and shares “target” information; this information is then provided as needed to ANSP personnel.

ANSP

2024

ASUR-B3/1	New non-cooperative surveillance system for

airborne aircraft (medium altitudes)

Technology

Sixth edition of the GANP

Main Purpose	To provide non-cooperative and independent surveillance system at a lower cost.

New Capabilities		Non-cooperative surveillance is provided at a lower lifecycle cost using multiple, networked receivers to detect aircraft-reflected RF signals originated from continuously transmitting non-aviation transmitters such as digital TV stations, FM radio stations, or cellphone towers.

Description		Information is provided to the ANSP on aircraft location to detect airspace infringement or to safely manage aircraft with surveillance equipment failure. A performance-based surveillance framework allows ANSPs to determine the most effective means of obtaining needed surveillance information. Within this timeframe, all aircraft identities/positions/velocities can be provided/shared by operators using an aviation network. Operators not receiving safety-of-life services can be surveilled using non- aviation spectrum.

Maturity Level	Concept

Human Factor

Considerations

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   En-route   Arrival

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Ground

system infrastructur e

Surveillance

*Non-cooperative

surveillance system

Non-cooperative surveillance system

observes RF reflections large enough to be declared as “targets”; this information is then provided to the ATC display (sometimes merged with data from cooperative surveillance systems).

Reference material: Future revision of ICAO Doc 9924 – Aeronautical

Surveillance Manual

ANSP

2031

ASUR-B4/1	Further evolution of ADS-B and MLAT	Technology

Sixth edition of the GANP

Main Purpose		To support the provision of Air Traffic/MET Services and vehicle-to-vehicle applications in a full TBO environment and within a performance-based surveillance framework.

New Capabilities		Additional aircraft/atmospheric information is provided in a performance-based surveillance context to resiliently support ANSP/MET and vehicle-to-vehicle applications in a full TBO environment.

Description		This evolution will address TBO needs for surveillance information discovered in Block 2. A performance-based surveillance framework allows ANSPs to determine the most effective means of obtaining needed surveillance information.

Cooperative surveillance is expected to be the principal means of surveillance and is typically provided by ADS-B and MLAT systems; rotating radars will be replaced at end-of-life where appropriate. Within this timeframe, all aircraft identities/positions/velocities can be provided/shared by operators using internet. Operators not receiving safety-of-life services can be surveilled using non-aviation spectrum.

Maturity Level	Concept

Human Factor Considerations

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   En-route   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

ASUR-B2/1 - Evolution of ADS-B and Mode S

Relation-operational need

NAVS-B2/2 - Dual Frequency Multi Constellation (DF MC) SBAS

Relation-operational need

NAVS-B2/3 - Dual Frequency Multi Constellation (DF MC) ABAS

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Ground system infrastructur e

Technical systems

HMI that supports controller awareness

Human Machine Interface (HMI) of the Air Traffic Controller Working Position (ATCo CWP) Reference: Guidance material: ICAO Doc. 9924 Aeronautical Surveillance Manual

ANSP 	2008

Airborne system capability

Surveillance  SSR Mode S

transponder with extended squitter version 4 (TBC)

Reference: Technical standards and guidance material: ICAO Annex 10 Volume IV Chapter 2,3 and 5 ICAO Doc. 9871 Technical Provisions for Mode S Services and Extended Squitter: RTCA/EUROCAE MOPS: DO-260D/ED- 102D (TBC) ICAO Doc. 9924

Aeronautical Surveillance Manual

2032

Ground system infrastructur e

Surveillance	ADS-B receivers

and/or MLAT

ADS-B receivers and/or MLAT receive information from aircraft and transfer this information to Service Delivery Point(s) Reference material: Technical standards and guidance material: Future revision ICAO Annex 10 Volume IV Chapter 2,3 and 5 Future revision ICAO Doc. 9871 Technical Provisions for Mode S Services and Extended Squitter RTCA/EUROCAE MOPS: DO-260D/ED-102D (TBC); ED-

129B (or subsequent revision) Future revision ICAO Doc. 9924 Aeronautical Surveillance Manual

ANSP 	2032

Aircraft operator

Aircraft manufacturer

Ground system infrastructur e

Surveillance	Service Delivery

Point(s) for ADS- B v4

Service Delivery Point(s) receive information and transfers it to ATC automation for processing and display to controller. Reference: Guidance material: Future revision of ICAO Doc. 9924 Aeronautical Surveillance Manual

ANSP 	2032

Airborne	Navigation	Basic Aviation

system	GNSS receiver

capability	with RAIM

Position source. Basic Aviation GNSS

receiver with RAIM. Such a receiver must comply with the technical performance requirements of either [E]TSO-C129, or [E]TSO-C196, or [E]TSO-C145/-C146.

(Note that the US/Europe and equivalent ADS-B mandates require more – see

FAA AC 20-165 or EASA CS-ACNS).

Aircraft

manufacturer	2008

Aircraft operator

## COMI

COMI-B0/1	Aircraft Communication Addressing and

Reporting System (ACARS)

Technology

Sixth edition of the GANP

Main Purpose		The Aircraft Communications Addressing and Reporting System (ACARS) is a digital datalink system for transmission of messages between aircraft and ground stations via VHF or satellites.

New Capabilities		ACARS provides the network for the controller and pilot with the ability to exchange datalink messages and thus provides a backup to voice communications. It also provides for airline operational control messaging.

Description		ACARS is a digital datalink network system which transmits and routes messages between aircraft and ground stations via VDL Mode 0/Mode2 radios or satellites.

Maturity Level	Ready for implementation

Human Factor Considerations

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Taxi-out   Departure   En-route   Arrival   Taxi-in

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-technology option	COMI-B0/3 - VHF Data Link (VDL) Mode 0/A Relation-technology option	COMI-B0/4 - VHF Data Link (VDL) Mode 2 Basic

Relation-technology option	COMI-B0/5 - Satellite communications (SATCOM) Class C Data Relation-technology option	COMI-B0/6 - High Frequency Data Link (HFDL)

Relation-technology benefit	COMI-B1/2 - VHF Data Link (VDL) Mode 2 Multi-Frequency

Relation-technology benefit	COMI-B1/3 - SATCOM Class B Voice and Data

### ENABLERS

Aircraft manufacturer

ANSP

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Protocols	Communicati

on infrastructure

ACARS

Protocols

AEEC 618- Air Ground Character- Oriented Protocol. (AEEC 620 Datalink Ground System Standard and Interface Specification) ARINC 622- ATS Datalink Applications over ACARS Air-Ground Network. AEEC 623- Character-Oriented Air Traffic Services. ARINC 633 is the air- ground definition for sending XML over ACARS

2013

Airborne system capability

Communicati on infrastructure

ACARS Airborne Radio Transceiver

ACARS is a digital datalink system for transmission of short messages between aircraft and ground stations via airband radio or satellite. AEEC 618- Air Ground Character-Oriented Protocol. AEEC 620 Datalink Ground System Standard and Interface Specification ARINC 624 is a standard for aircraft onboard maintenance system

2013

Ground system

Communicati on

ACARS Ground Radio

ACARS is a digital datalink system for transmission of short messages between

2013

ANSP

infrastructure infrastructure

Transceiver

aircraft and ground stations via airband radio or satellite. AEEC 618- Air Ground Character-Oriented Protocol. AEEC 620 Datalink Ground System Standard and Interface Specification

Ground systems supplier

Space system

Communicati on

ACARS Space Radio

Inmarsat Classic Aero Standard and Iridium Standard (includes a satellite data

2013

Satellite provider

infrastructure infrastructure

Transceiver

unit, a high power amplifier and an antenna with a steerable beam).

Aircraft operator

Aircraft manufacturer

Sixth edition of the GANP

Main Purpose 			ATN/OSI provides a bit-oriented multi-layer protocol for exchanging ATS messages between the aircraft and ground system.

Aeronautical Telecommunication Network/Open	Technology

System Interconnection (ATN/OSI)

COMI-B0/2

New Capabilities 	 The ATN/OSI provides an alternative, dedicated ATS protocol for exchanging flight safety messages.

Description		The ATN/OSI provides a dedicated ATS commuications protocol for the usage of B1 data link applications (e.g. CPDLC and CM).

Maturity Level	Ready for implementation

Human Factor Considerations

### PLANNING LAYERS 		OPERATIONS

Tactical-During ops 	 Taxi-out   Departure   En-route   Arrival   Taxi-in

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-technology benefit	COMI-B1/2 - VHF Data Link (VDL) Mode 2 Multi-Frequency Relation-technology benefit	COMI-B1/3 - SATCOM Class B Voice and Data

Relation-technology benefit	COMI-B3/1 - VHF Data Link (VDL) Mode-2 Connectionless Relation-technology need	COMI-B0/4 - VHF Data Link (VDL) Mode 2 Basic

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Protocols	Communicati

Aircraft manufacturer

on infrastructure

ATN/OSI protocol  ICAO Doc. 9880- Manual on Detailed

Technical Specifications for the ATN/OSI.

ANSP 	2013

Airborne system capability

Communicati on infrastructure

ATN/OSI

Airborne Router

ICAO Doc. 9880- - Manual on Detailed Technical Specifications for the ATN/OSI. Connectivity is to be accomplished over

ANSP

2013

|  |  |  | multiple subnetwork types. The ATN currently recognizes a limited set of subnetworks: SSR Mode Select (Mode S), Very High Frequency (VHF) Digital Link (VDL), Aeronautical Mobile Satellite Service (AMSS), Gatelink, and High Frequency (HF). |  |  |
| --- | --- | --- | --- | --- | --- |
| Ground system | Communicati on | ATN/OSI Ground Router | ICAO Doc. 9880- Manual on Detailed Technical Specifications for the ATN/OSI. | ANSP | 2013 |

infrastructure infrastructure

COMI-B0/3	VHF Data Link (VDL) Mode 0/A	Technology

Sixth edition of the GANP

Main Purpose		VDL Mode 0/A is a data communications subnetwork that supports transmission of data link messages.

New Capabilities	Introduction of a datalink to support domestic data communications operations. a supplement to voice communications

Exchanges aviation data (AOC, CPDLC and ADS)

Description		VDL Mode 0/A is a narrow-band transceiver operating in the VHF aviation protected spectrum band which will transmit data to and from the aircraft to support data communications. The VHF digital radio system is based on the double side band AM multi-shift keying modulation to transfer 2400 bps.

Maturity Level	Ready for implementation

Human Factor Considerations

### PLANNING LAYERS 		OPERATIONS

Tactical-During ops 	 Taxi-out   Departure   En-route   Arrival   Taxi-in

### DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Protocols	Communicati

on infrastructure

“Plain Old” ACARS (POA)

protocol

AEEC 758 - Communications Management Unit ARINC 771 - Low-earth orbiting aviation satellite communication systems

ANSP

Aircraft manufacturer

2013

Airborne system capability

Communicati on infrastructure

VDL Mode 0/A airborne Analog Radio

AEEC 618 - Air/ground character-oriented protocol specification

ANSP

Aircraft manufacturer

2013

Ground system infrastructur e

Communicati on infrastructure

VDL Mode 0/A Ground Analog transceiver

Datalink Ground System Standards and Interface Specification (ARINC 620).

Defines the interfaces to the ACARS Ground System Operated by a Datalink Service Provider.

ANSP

Ground systems supplier

2013

Sixth edition of the GANP

Technology

VHF Data Link (VDL) Mode 2 Basic

COMI-B0/4

Main Purpose 		VDL Mode 2 Basic is a data communications subnetwork that supports transmission of data link messages. It provides higher performance than VDLM0/A.

New Capabilities 	 Provides an Increase in data capacity over VDL Mode O/A  Exchanges aviation data (AOC, CPDLC and ADS)

More efficient use of spectrum

Description 		VDL Mode 2 is narrow-band transceiver operating in the VHF aviation protected spectrum band, which will transmit data to support data communications between the aircraft and ground. It consists of a set of air-ground protocols that increase the data rate to 31.5 kbits.

Maturity Level 	Ready for implementation

Human Factor Considerations

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Taxi-out   Departure   En-route   Arrival   Taxi-in

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

COMI-B0/3 - VHF Data Link (VDL) Mode 0/A

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Protocols	Communicati

on infrastructure

VDL-M2

operational Usage & Protocol

VDL-M2 is the primary VDL mode being implemented operationally to support Controller Pilot Data Link Communications. • ICAO Doc 9776 VDL2 Technical Manual • VHF air-ground Digital Link (VDL) Mode 2; Technical characteristics and methods of measurement for ground-based equipment ETSI EN 301 841-1 • RTCA DO 224 Minimum Aviation System Performance Standard) • ARINC 631 VHF Digital Link (VDL) Mode 2 implementation provisions

ANSP 	2013

Airborne system capability

Communicati on infrastructure

VDL-M2 Digital Airborne Radio Transceiver

Aircraft must be fitted with a digital connection to a VHF Digital Radio (VDR) transceiver. • AEEC 631-B • DO-281B (RTCA Minimum Aviation System Performance) • ED-92B (EUROCAE MOPS)

ANSP 	2013

Ground

system infrastructur e

Communicati

on infrastructure

VDL-M2 Digital

Ground Radio Transceiver

VHF Digital Link Mode 2 system uses	 ANSP

digital radios to improve the performance of VHF analog data link. VDL Mode 2 provides a data rate of 31.5 kbit/second using a radio channel that is the same width as the channel used by the VHF analog data link system, providing a data rate of only 2.4 kbit/second. • DO-224C (RTCA Minimum Aviation System Performance Standard)

2013

COMI-B0/5	Satellite communications (SATCOM) Class C

Data

Technology

Sixth edition of the GANP

Main Purpose	To provide surveillance and communications where VHF usage is not possible or practicle.

New Capabilities		Supports improvement of surveillance and communication in airspace where procedural separation is being applied.

Description		Satellite-based, narrow-band communication provided by multiple service providers that can be used for safety and routing communciations.

Maturity Level	Ready for implementation

Human Factor Considerations

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Taxi-out   Departure   Arrival

### DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Protocols	Communicati SATCOM Class

on	C Usage and infrastructure	Data Protocol

SATCOM Performance Class C covers

the performance requirements included in the current AMS(R)S SARPs and is applicable to systems already standardized in ICAO (such as Inmarsat Classic Aero, MTSAT and Iridium SBD) as well as the Inmarsat SBB system which was recently included in the AMS(R)S Manual. Class C is effectively covering remote and oceanic operations (Performance specification DO-306/ED- 122). NGSS DO-210/DO-262

ANSP

Satellite provider

2013

Airborne	Communicati

system	on capability	infrastructure

SATCOM Class

C Management Unit (Airborne Radio Transceiver)

AMSRS Satellite SARPS with Section for

INMARSAT and Iridium AEEC 741, 761, 781-7 SATCOM Class C Form, Fit and Function

Aircraft manufacturer

Satellite provider

2013

Ground

system infrastructur e

Communicati

on infrastructure

SATCOM Class

C Ground Radio Transceiver

Minimum Aviation System Performance

Standards (MASPS) for the aeronautical mobile-satellite (R) service (AMS(R)S) as used in aeronautical data links.

ANSP

2013

Space

system infrastructur e

Communicati

on infrastructure

SATCOM Class

C Space Radio Transceiver

Aeronautical Mobile Satellite (Route)

Service Manual (Doc 9925) with a section that addresses Satcom Performance Class C

2013

Space

system infrastructur e

Communicati

on infrastructure

Communication

satellites for class C

Inmarsat I4, MTSAT and Iridium.

2013

Satellite provider

Satellite provider

Sixth edition of the GANP

Main Purpose

To communicate in areas where SATCOM and VHF are not available.

New Capabilities	HFDL is a communications media used to exchange data messages between aircraft end-systems

and corresponding ground-based HFDL ground stations.

Supports improvement of surveillance in airspace where procedural separation is being applied.

Description

The HFDL includes the HFDL network and avionics. Specifically this means the introduction of an

airborne communications router (e.g. Communications Management Unit) and an HFDL radio.

Maturity Level

Ready for implementation

Technology

High Frequency Data Link (HFDL)

COMI-B0/6

Human Factor Considerations

### PLANNING LAYERS 		OPERATIONS

Tactical-During ops 	 En-route

### DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Protocols	Communicati

on infrastructure

HF Datalink operational Usage & Protocol

HFDL is an ACARS communications media that is used to exchange data between aircraft end-systems and ground based stations. • ARINC Specification 635-4: HF Data Link Protocols • ARINC Characteristic 753-2: HF Data Link System • ARINC Report 634: HF Data Link System Design Guidance Material • ICAO Doc. 9741 HFDL

ANSP

Aircraft manufacturer

2013

Airborne system capability

Communicati on infrastructure

HFDL data radio transmission

To use High Frequency Data Links an aircraft needs to be equipped with a Communications Management Function or equivalent. This allow for airborne communications to interface with many aircraft communications systems. • DO- 265 HFDL MOPS

ANSP

Aircraft manufacturer

2013

Ground system infrastructur e

Communicati on infrastructure

HFDL ground radio transmission

- DO-277 MASPS	ANSP

Aircraft manufacturer

2013

Sixth edition of the GANP

Main Purpose

Supports improved communication over AFTN

Provide flight information coordination between ANSPs at adjacent FIRs, and with relevant military units, support separation assurance, potentially providing, when used in conjunction with other enablers (e.g. navigation capabilities), reduced separation.

Technology

ATS Message Handling System (AMHS)

COMI-B0/7

New Capabilities 	  AMHS makes use of higher speed communication than AFTN. It also allows the use of bit- oriented communications allowing greater flexibility in message types. Attachments to messages can also be supported thus allowing the exchange of graphics. Provides direct communication between adjacent FIRs using data communication to minimize the use of voice communication.

Increase performance to handle large files  Implement AFTN/AMHS gateway

Initiate the standardization of IP interface and addressing scheme

Description 		The AMHS is served as ICAO mandated communication for data exchange between ANSPs (ICAO Doc. 9880 and Annex X). AMHS is served as enabler for

- Flight Plan/Clearance

- AIDC: Flight transfer

- MET data

ATS voice service is used for emergency coordination and/or normal coordination when data communication service is not available.

AMHS is expected to be utilized to carry traffic for AIDC/Flight Plan/MET until SWIM is ready in Block 2. This is due to ANSPs need time to upgrade/implement adaptors to support SWIM interface. In the meantime, AMHS will accommodate SWIM compliance data message (IWXXM) as required. It is noted that AMHS would not be able to support FF-ICE and FIXM data.

The interface is based on IP over legacy dedicated point-to-point circuits.

Maturity Level 	Ready for implementation

Human Factor Considerations

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   En-route   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology benefit

COMI-B1/1 - Ground-Ground Aeronautical Telecommunication

Network/Internet Protocol Suite (ATN/IPS)

ENABLERS

Enabler

Category

Enabler Type	Enabler Name

Description / References

Stakeholders	Year

Protocols	Communicati AMHS

on	operational infrastructure	Usage &

Protocol

ATS Message Handling Systems (AMHS)  ANSP

is a standard for aeronautical ground-to- ground communication. AMHS was first implemented by Avitech GmbH in 1996 and the first European AMHS connection was operational in 2005. The AMHS is a set of end systems that work together to allow cooperation between human and automated users. • ICAO Doc 4444, Procedures for Air Navigation Services Air Traffic Management • ICAO Doc 7030, Regional Supplementary Procedures • ICAO Doc 10037 Edition 1 • Global Operational Datalink Manual (GOLD) • ICAO Doc 9869 Second Edition • Performance-Based Communication and Surveillance (PBCS) Manual

2013

Ground	Communicati Aviation Authority

system	on	to Aircraft pilot infrastructur infrastructure	data

e	transmission

ICAO Doc 9880 Part II defines two

fundamental levels of service within AMHS: basic and extended. The basic AMHS performs an operational role similar to the Aeronautical Fixed Telecommunication Network with some enhancements. • ED122/DO306, including Change 1 & 2, Oceanic Remote Safety and Performance Requirements • ED100A/DO258A, FANS 1/A

Interoperability Requirements

ANSP

2013

COMI-B1/1	Ground-Ground Aeronautical Telecommunication Network/Internet Protocol Suite (ATN/IPS)

Technology

Sixth edition of the GANP

Main Purpose		To provide for a more modern, more efficient, cost-effective, and robust data communications network infrastructure.

New Capabilities		It enables the efficient integration of technologies with improved integrity to support future air to ground aeronautical safety services and regularity of flight communications.

Description		The ATN/IPS internetwork consists of IPS nodes and networks operating in a multinational environment in support of Air Traffic Service Communication (ATSC) as well as Aeronautical Industry Service Communication (AINSC), such as Aeronautical Administrative Communications (AAC) and Aeronautical Operational Communications.

This evolution will support enhanced civil-military cooperation and coordination functions, if interoperability and military information security aspects are considered.

Maturity Level	Standardization

Human Factor

Considerations

PLANNING LAYERS

Tactical-During ops

OPERATIONS

En-route

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology benefit

COMI-B1/4 - Aeronautical Mobile Airport Communication System

(AeroMACS) Ground-Ground

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Protocols

Communicati

on infrastructure

IPS Protocol

The Manual on the Aeronautical

Telecommunication Network (ATN) using Internet Protocol Suite (IPS) Standards and Protocols Doc 9896

ANSP

2019

COMI-B1/2	VHF Data Link (VDL) Mode 2 Multi-Frequency	Technology

Sixth edition of the GANP

Main Purpose	Supports transmission of data link message sets to supplement current voice operations, thus reducing voice channel congestion; while increasing productivity and capacity.

Supports increased subnetwork capacity and reduces interference over the standard VDL Mode 2 system.

New Capabilities	Provides an increase in data capacity over VDL Mode 2 Exchanges aviation data (AOC, CPDLC and ADS) Provides a supplement to voice communications

More efficient use of spectrum

Description	VDL Mode 2 Multi-Frequency consists of a set of air-ground protocols that increase the data rate to

31.5 kbits. It allows transmission from a character oriented protocols to digital or bit based protocols while using VHF air and ground narrow-band transceiver operating in the VHF aviation protected spectrum band, which will transmit textual data to and from the cockpit to support data communications between the pilot and the air traffic controller.

Maturity Level	Ready for implementation

Human Factor Considerations

### PLANNING LAYERS 		OPERATIONS

Tactical-During ops 	 Taxi-out   Departure   En-route   Arrival   Taxi-in

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Evolution	COMI-B0/4 - VHF Data Link (VDL) Mode 2 Basic

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Protocols	Communicati

on infrastructure

VDL-M2 MF

operational Usage & Protocol

- ICAO Doc 9776 VDL2 Technical Manual

- ED-92B (EUROCAE MOPS) • DO-281B

(RTCA Minimum Operational Performance Standard) • AEEC 631-B • RTCA DO 224

ANSP

Aircraft manufacturer

2019

Airborne system capability

Aircraft system

Airborne VDL M- 2 MF autotune functionality

VDL Mode 2 frequency management capability allow for auto-tune of an aircraft, from the VDL common signaling channel to an alternate VDL frequency, and back to the common signaling channel. The auto-tune command is the primary building block to instruct aircraft to tune to VDL frequencies, which enables service providers to actively manage traffic load among VDL frequencies. • DO-281B (RTCA Minimum Operational Performance Standard) • AEEC 631-6 • ED-92B (EUROCAE MOPS)

ANSP

Aircraft manufacturer

2019

Ground system infrastructur e

Communicati on infrastructure

Ground VDL M-2 MF autotune functionality

VDL Mode 2 frequency management capability allow for auto-tune of an aircraft, from the VDL common signaling channel to an alternate VDL frequency, and back to the common signaling channel. The auto-tune command is the primary building block to instruct aircraft to tune to VDL frequencies, which enables service providers to actively manage traffic load among VDL frequencies. • DO-224C (RTCA Minimum Aviation System Performance Standard) • DO-343A Minimum Aviation System Performance Standards (MASPS) for the Aeronautical Mobile-Satellite (R) Service (AMS(R)S) for FANS1/A+ • RTCA DO-262 MOPS

ANSP

Aircraft manufacturer

2019

COMI-B1/3	SATCOM Class B Voice and Data	Technology

Sixth edition of the GANP

Main Purpose 	 Supports introduction of SATVOICE and SATDATA as a complement to HF voice communications.

Provides for oceanic and domestic broadband IPS based safety critical data link operations.  Supports safety critical, safety and regularity of flight operations.

New Capabilities 	 Use of SATCOM voice for all types of ATS communications (routine and emergency/urgency communications).

Provide high-speed IP based broadband networks.  Improved security

Lower cost than the traditional circuit switched services (Classic Aero).

Description 		SATCOM System is a broadband, IP based communication system that provides voice and high- speed data communications between the aircraft and the air-traffic controller.

Two constellations are expected to provide this service (INMARSAT and Iridium).

SATCOM Class B (SB-S): SwiftBroadband is available over the Inmarsat 4 satellite constellation, providing near global coverage. SwiftBroadband includes SB-S avionics, the satellite modem that accesses the service, and the aircraft antenna capable of receiving SwiftBroadband and related equipment such as diplexer, LNA, HPA and cabling.

SATCOM Class B (Certus)The Iridium NEXT (Certus) Satellite Network, with its constellation of 66 Low Earth Orbit (LEO) satellites, is a global mobile satellite communication network, with coverage of the entire Earth, including polar regions, offering ATC voice and data service.

This evolution will support enhanced civil-military cooperation and coordination functions, if interoperability and military information security aspects are considered.

Maturity Level 	Ready for implementation

Human Factor Considerations

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   En-route   Arrival

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

ENABLERS

Enabler

Category

Enabler Type	Enabler Name

Description / References

Stakeholders	Year

Aircraft manufacturer

Protocols	Communicati

on infrastructure

SATCOM Class B usage and protocols

SB-S is the INMARSAT Satellite System (2019). SATCOM Performance Class B covers more stringent (compared to Class C) performance requirements and will be applicable to commercial SATCOM systems, (i.e. the Iridium Next and the Inmarsat SB-S supporting safety services. SATCOM Performance Class B covers the performance requirements of initial 4D operations and the Baseline 2 (B2) requirements defined by RTCA and EUROCAE. Performance Class B requirements will cover remote, oceanic as well as continental/domestic operations. Global Operational Data Link Document (GOLD) Manual (Doc 10037), the PBCS Manual (DOC 9869) and the Satellite Voice Guidance Material (SVOM, Doc 10038) Performance specification DO-306/ED-122; DO-290/ED-120

Interoperability Specification DO- 258A/ED-100A DO-280B/ED-112B

Certus is the Iridium Next Satellite System (2025). SATCOM Performance Class B covers more stringent (compared to Class C) performance requirements and will be applicable to commercial SATCOM systems, (i.e. the Iridium Next and the Inmarsat SB-S supporting safety services. SATCOM Performance Class B covers the performance requirements of initial 4D operations and the Baseline 2 (B2) requirements defined by RTCA and EUROCAE. Performance Class B requirements will cover remote, oceanic as well as continental/domestic operations. Performance specification DO-306/ED-122 DO-350A/ED-228A

Interoperability Specification DO- 258A/ED-100A DO-351A/ED-229A

RTCA DO 262 MOPS Certus Technical Manual The AMS(R)S manual (Doc 9255), separate chapter for the IridiumNext (Certus) system Global Operational Data Link Document (GOLD) Manual (Doc 10037), the PBCS Manual (DOC 9869)

and the Satellite Voice Guidance Material (SVOM, Doc 10038)

ANSP

2019

Airborne	Communicati SATCOM Class

system	on	B Management capability	infrastructure	Unit (Airborne

Radio

Transceiver)

AEEC Characteristic, 771 / 781

(INMARSAT; 2019) RTCA DO 262

MOPS Certus Technical Manual (Iridium;2025) AEEC 771-2 SATCOM

Class B Form, Fit and Function

(Iridium;2025)

2019

Ground

system infrastructur e

Communicati

on infrastructure

SATCOM Class

B Ground Radio Transceiver

RTCA DO-343 MASPS

(INMARSAT;2019) DO-343B Minimum

Aviation System Performance Standards (MASPS) for the Aeronautical Mobile- Satellite (R) Service (AMS(R)S) For FANS1/A+ and ATN/IPS (Iridium;2025)

ANSP

2019

Space

system infrastructur e

Communicati

on infrastructure

Communication

satellites for class B

Swift Broadband Safety Satellite (2019):

The AMS(R)S manual (Doc 9925), separate chapter for the Inmarsat SB- Safety systems Certus Satellite (2025): Performance specification DO-306/ED- 122 DO-350A/ED-228A Interoperability Specification DO-258A/ED-100A DO- 351A/ED-229A

2019

Satellite provider

Aircraft manufacturer

Aircraft manufacturer

Sixth edition of the GANP

Main Purpose

Reduced Separation

Improved situational awareness Reduced Cost

Improved Efficiency

New Capabilities

Broadband Communications

Supports Safety communications IPS Based System

Improved Security

Description

AeroMACS is a broadband wireless communications system. It can provide network connectivity on

the airport surface for fixed wireless communications and can be used only for the safety critical and regularity of flight. The fixed assests supported by AeroMACS include multilateration sensors, weather sensors, surface radar and fixed navigation aids.

Maturity Level

Ready for implementation

Human Factor

Considerations

PLANNING LAYERS

OPERATIONS

Technology

Aeronautical Mobile Airport Communication

System (AeroMACS) Ground-Ground

COMI-B1/4

| Tactical-During ops | Taxi-out Taxi-in |  |  |
| --- | --- | --- | --- |
| DEPENDENCIES AND RELATIONS |  |  |  |
| There are currently no dependencies. |  |  |  |
| ENABLERS |  |  |  |
| Enabler Enabler Type Enabler Name | Description / References | Stakeholders | Year |
| Category |  |  |  |
| Operational - AeroMACS procedures operational description | - ED222 / Do345: Aeronautical Mobile Airport Communications System (AeroMACS) Profile Doc. • Doc 10044: AeroMACS Technical Manual | ANSP | 2017 |
| Ground Airport AeroMACS system systems Mobile Station infrastructur (Ground Radio e Unit) | - ED223 / Do346: Minimum Operational Performance Standards (MOPS) for the AeroMACS. •ICAO ANNEX-10, Volume III, Chapter 7: AeroMACS SARPs | Airport operator | 2017 |
| Ground Communicati AeroMACS system on Base Station infrastructur infrastructure e | EUROCAE ED-227: AeroMACS MASPS | ANSP | 2017 |

Sixth edition of the GANP

Main Purpose

To improve integrity of the information.

New Capabilities	This element:

provides more efficient data communications; improves information integrity.

Description

A new network infrastructure based on Internet Protocol (IP) that promises to use Commercial-Off-

The-Shelf (COTS) products to support air to ground aeronautical safety services and regularity of flight communications.

Maturity Level

Validation

Human Factor

Considerations

PLANNING LAYERS

Tactical-During ops

OPERATIONS

En-route

Technology

Air-Ground ATN/IPS

COMI-B2/1

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-technology option	COMI-B1/2 - VHF Data Link (VDL) Mode 2 Multi-Frequency Relation-technology option	COMI-B1/3 - SATCOM Class B Voice and Data

Relation-technology benefit	COMI-B3/1 - VHF Data Link (VDL) Mode-2 Connectionless Relation-technology benefit	COMI-B3/2 - SATCOM Class A voice and data

Relation-technology benefit	COMI-B3/4 - Links meeting requirements for safety critical communication Relation-technology benefit	COMI-B3/3 - L-band Digital Aeronautical Communication System (LDACS)

Relation-technology benefit	COMI-B2/2 - Aeronautical Mobile Airport Communication System

(AeroMACS) aircraft mobile connection

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Protocols	Communicati

on infrastructure

IPS Protocol	The Manual on the Aeronautical

Aircraft operator

Aircraft manufacturer

Telecommunication Network (ATN) using Internet Protocol Suite (IPS) Standards and Protocols Doc 9896

ANSP 	2019

Airborne system capability

Communicati on infrastructure

IPS Airborne Router

Aviation Profiles for Internet Protocol Suite (IPS) ICAO ANNEX-10, Volume III, IPS

SARPs ARINC Project Paper 858 IPS technical requirements

2025

Ground system

Communicati on

IPS Ground Router

IPS MASPS	 ANSP 	2025

infrastructure infrastructure

Sixth edition of the GANP

Main Purpose

To reduce separation and improve situational awareness.

New Capabilities

Broadband communications.

Supports safety communications. IPS based system.

Improved security.

Aeronautical Mobile Airport Communication	Technology

System (AeroMACS) aircraft mobile connection

COMI-B2/2

Description		AeroMACS is a broadband wireless communications system. It can provide network connectivity on the airport surface for communications to mobile assets (e.g. Aircraft, emergency vehicles, baggage trucks). It is used to transmit safety critical and regularity of flight information.

Maturity Level	Validation

Human Factor Considerations

| PLANNING LAYERS | OPERATIONS |  |  |
| --- | --- | --- | --- |
| Tactical-During ops | Taxi-out Taxi-in |  |  |
| DEPENDENCIES AND RELATIONS |  |  |  |
| There are currently no dependencies. |  |  |  |
| ENABLERS |  |  |  |
| Enabler Enabler Type Enabler Name | Description / References | Stakeholders | Year |
| Category |  |  |  |
| Operational - AeroMACS procedures mobile connection operational description | - ED222 / Do345: Aeronautical Mobile Airport Communications System (AeroMACS) Profile Doc. • Doc 10044: AeroMACS Technical Manual | ANSP | 2031 |

Airborne system capability

Communicati on infrastructure

AeroMACS Mobile Station (Avionics Radio Unit)

- ED223 / Do346: Minimum Operational Performance Standards (MOPS) for the AeroMACS. • ICAO ANNEX-10, Volume III, Chapter 7: AeroMACS SARPs • ARINC 766: Aeronautical Mobile Airport Communication System (AeroMACS) Transceiver and Aircraft Installation Standards

2031

Ground system infrastructur e

Communicati on infrastructure

AeroMACS Base Station for mobile connection

EUROCAE ED-227: AeroMACS MASPS  ANSP 	2031

Aircraft operator

Aircraft manufacturer

Sixth edition of the GANP

Links meeting requirements for non-safety critical	Technology

communication

COMI-B2/3

Main Purpose 	To:

reduce operational cost;  improve performance;

take advantage of new technologies sooner;

enable the global exchange of non-safety information.

New Capabilities 	 Improved resiliency

Reduced operational costs  Improved capacity

Broadband communications

Enable a single IP-based global operating network

Description 		Broadband satellite and cellular services are used to support non-safety critical communications. By adopting the use of available infrastructure and possibly the use of a multi-sensor concept these systems meet performance requirements, in effect leading to reduced system cost, while improving the overall performance of the network and the system resiliency.

Maturity Level 	Validation

Human Factor Considerations

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Taxi-out   Departure   En-route   Arrival   Taxi-in

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Airborne	Communicati Commercial

system	on	airborne data capability	infrastructure	transfer

In the future commercial provider (Ex.

AWS, Azure, Google) may increase their range and service offerings to allow implementation of radio systems that can be used to transfer data from airborne systems to ground systems.

2025

Ground

system infrastructur e

Communicati

on infrastructure

Commercial

ground data transfer

To reduce congestion on current air traffic

control towers, commercial cellular infrastructure and systems already in place can be used to relay data.

2025

service provider

Communication

service provider

Communication

ANSP

ANSP

Space

system infrastructur e

Communicati

on infrastructure

Commercial

satellite data transfer

To relay messages via satellites while

saving the cost on implementation, commercial broadband satellites and constellation systems (Ex. Amazon, Google, SpaceX) can be used to relay data.

ANSP

Satellite provider

2025

COMI-B3/1	VHF Data Link (VDL) Mode-2 Connectionless	Technology

Sixth edition of the GANP

Main Purpose	To make a more efficient use of the bandwith.

New Capabilities		Connection-less mode 2 uses a previously dormant feature (unnumbered information frames) instead of information frames.

Description		Connection-less VDL 2 is capable of transporting AOA messages, ATN messages and IPS messages. It reduces connection orientated complexity; while being efficient in its use, and extending the lifetime of the finite number of VDL 2 channels available. It is determined to be less complicated overall and a larger percentage of the complexity is in the ground system where it’s easier to maintain and less expensive.

Additionally, connection-less will provide better performance (fewer Provider Aborts and lower propagation times).

This evolution offers opportunities for compliance based on performance-equivalence for State aircraft.

Maturity Level	Concept

Human Factor Considerations

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Taxi-out   Departure   En-route   Arrival   Taxi-in

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Evolution	COMI-B0/4 - VHF Data Link (VDL) Mode 2 Basic

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Protocols	Communicati

on infrastructure

Connectionless VDL-M2

operational usage & protocols

- ICAO Doc 9776 VDL2 Technical Manual

- AEEC 631-6 • DO-281B (RTCA

Minimum Operational Performance Standard) • ED-92B (EUROCAE MOPS)

- DO-224C (RTCA Minimum Aviation System Performance Standard)

ANSP

Aircraft manufacturer

2030

Airborne system capability

Communicati on infrastructure

Connectionless VDL-M2 Digital Airborne Radio Transceiver

Existing VDL Mode 2 requires airplanes and ground stations to establish an explicit connection. This is less efficient than connectionless VDL Mode 2 because of resulting overhead message traffic for link establishment, ground station handoff, etc. Existing VDL Mode 2 is also less robust because airplanes will accept an uplink only from a connected ground station and ground stations will accept a downlink only from a connected airplane. • AEEC 631-6

- DO-281B (RTCA Minimum Operational Performance Standard) • ED-92B (EUROCAE MOPS)

ANSP

Aircraft manufacturer

2025

Ground system infrastructur e

Communicati on infrastructure

Connectionless VDL-M2 Digital Ground Radio Transceiver

Existing VDL Mode 2 requires airplanes and ground stations to establish an explicit connection. This is less efficient than connectionless VDL Mode 2 because of resulting overhead message traffic for link establishment, ground station handoff, etc. Existing VDL Mode 2 is also less robust because airplanes will accept an uplink only from a connected ground station and ground stations will accept a downlink only from a connected airplane. • DO-224C (RTCA Minimum Aviation System Performance Standard)

ANSP

Aircraft manufacturer

2025

Sixth edition of the GANP

Main Purpose

To improve efficiency and cost-effectiveness of SATCOM globally. SATCOM Performance Class A

defines more stringent (compared to Class B) performance requirements necessary to support future concepts of SESAR/NextGen/CARATS.

New Capabilities

Improved resiliency

Reduced operational costs Improved capacity Support B3 Message Sets

Technology

SATCOM Class A voice and data

COMI-B3/2

Description		SATCOM Class A will make use of existing and future commercially available constellations to provide multilink operations and thus improved capacity, availability and resiliency.

This evolution offers opportunities for compliance based on performance-equivalence for State aircraft.

Maturity Level	Concept

Human Factor Considerations

### PLANNING LAYERS 		OPERATIONS

Tactical-During ops 	 Taxi-out   Departure   Arrival

### DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Protocols	Communicati

on infrastructure

SATCOM Class A operational usage and protocols

SATCOM Performance Class A covers more stringent (compared to Class B) performance requirements and will be applicable to future SATCOM systems SATCOM Performance Class A covers the performance requirements of full 4D operations and operations required to support the SESAR/NextGEN/CARATS future concepts. SATCOM Performance Class A requirements correspond to the Baseline 3 (B3 ) requirements. Class A will cover remote, oceanic as well as continental operations/domestic. Class A performance requirements are expected to serve in particular as design guidelines for the definition and specification of future SATCOM systems to support the future aviation requirements.

ANSP 	2031

Sixth edition of the GANP

Technology

L-band Digital Aeronautical Communication

System (LDACS)

COMI-B3/3

Main Purpose	To:

support improved Message Sets (B2) including accurate aircraft trajectory for improved 4D flight path;

provide for more efficient data communications; increase security.

New Capabilities	Broadband communications.

Supports safety communications. IPS based system.

Improved security.

Description		LDACS (L-band Digital Aeronautical Communications System) is the future terrestrial air-ground data link. It is a frequency division duplex (FDD) configuration utilizing Orthogonal Frequency Division Multiplexing (OFDM), reservation based access control and advance network protocols (i.e. IPS).

Maturity Level	Validation

Human Factor Considerations

### PLANNING LAYERS 		OPERATIONS

Tactical-During ops 	 Taxi-out   Departure   En-route   Arrival   Taxi-in

### DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### ENABLERS

There are currently no enablers.

Sixth edition of the GANP

Main Purpose 		To:

provide more efficient data communications; increase security;

reduce cost;

improve performance;

take advantage of newer technologies sooner;

enable the global exchange of safety critical information.

Technology

Links meeting requirements for safety critical

communication

COMI-B3/4

New Capabilities 		Broadband Communications

Supports Safety communications IPS Based System

Improved Security

Enable a single IP-based global operating network for all information

Description		Broadband satellite and cellular services are used for safety-critical communications. Through the use of a multi-sensor concept, a combination of these systems can be used together to meet the safety critical and regularity of flight performance, interoperability and certification

requirements, reducing the system cost while improving the overall performance of the network and the resiliency of the system. LDACS and AeroMACS are other candidate solutions for use in the en-route/terminal area and airport surface respectively.

Maturity Level	Concept

Human Factor Considerations

### PLANNING LAYERS 		OPERATIONS

Tactical-During ops 	 Taxi-out   Departure   En-route   Arrival   Taxi-in

### DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### ENABLERS

service provider

Communication

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

service provider

Communication

Ground system infrastructur e

Communicati on infrastructure

Commercial ground system use

Future improvements to ground based commercial systems will allow safety critical information to be transferred on existing commercial systems.

2031

Space system infrastructur e

Communicati on infrastructure

Commercial space system use

Future improvements to satellite-based data transfer services as well as the addition of new satellites will allow safety critical communication to utilize commercial satellites. Likely require exploration of hybrid satellite constellation services

2031

## COMS

COMS-B0/1	CPDLC (FANS 1/A & ATN B1) for domestic and procedural airspace

Technology

Sixth edition of the GANP

Main Purpose	Supports :

reduction of voice channel congestion and increase of capacity in domestic airspace, improvement of communication and surveillance in airspace where procedural separation is being applied.

New Capabilities  As a complement to voice communications, controller–pilot data link communications (CPDLC) provides the controller and the pilot with the ability to communicate through exchange of data link messages.

Description	Depending on the specific implementations, other advantages associated with CPDLC include:

- in certain airspace, direct controller-pilot communications (DCPC) where it was not previously available;

- allowing the flight crew to print messages;

- allowing messages to be stored and reviewed as needed;

- reducing flight crew-input errors by allowing the loading of information from specific uplink messages, such as route clearances or frequency change instructions, into other aircraft systems, such as the FMS or radios;

- allowing the flight crew to request complex route clearances, which the controller can respond to without having to manually enter a long string of coordinates;

- reducing flight crew workload by supporting automatically transmitted reports when a specific event occurs, such as crossing a waypoint and the loading of clearance information directly into the flight management system; and

- reducing controller workload by providing automatic flight plan updates when specific downlink messages (and responses to some uplink messages) are received.

For domestic airspaces, ATN B1 CPDLC services provide management of the ATC data link and voice communications, check of ATC microphone stuck and exchange of a limited set of ATC clearances and requests. The vast majority of ATN B1 aircraft do not support any automatic integration of CPDLC instructions into the aircraft Flight Management System.

For procedural airspaces, FANS 1/A CPDLC services provide management of the ATC data link and voice communications, exchange of an extensive set of ATC clearance and requests, reporting of aircraft derived information (including Position Reporting). The vast majority of FANS 1/A aircraft support automatic integration of CPDLC instructions into the aircraft Flight Management System (even though implementations are not harmonized across the fleet due to lack of standardization).

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS 		OPERATIONS

Tactical-During ops

En-route

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology option

COMI-B0/1 - Aircraft Communication Addressing and Reporting System

(ACARS)

Relation-technology option

COMI-B0/2 - Aeronautical Telecommunication Network/Open System

Interconnection (ATN/OSI)

### ENABLERS

Aircraft manufacturer

ANSP

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Communicati on service

CPDLC

Aircraft operator

procedures and monitoring

Requirements and guidance to support deployment of CPDLC operations, such as : Annex 10 Vol II ICAO Doc 4444, Procedures for Air Navigation Services Air Traffic Management ICAO Doc 10037 Edition 1 Global Operational Datalink Manual (GOLD)

2013

Airborne system capability

Communicati on service

FANS 1/A

CPDLC aircraft system

Technical standards to ensure operational and technical interoperability, such as : Annex 10 Vol II ED100/DO258, FANS 1/A

Aircraft operator

Aircraft manufacturer

Interoperability Requirements

2002

Airborne system capability

Communicati on service

ATN B1 CPDLC

aircraft system

Technical standards to ensure operational and technical interoperability, such as : Annex 10 Vol II ED110B/DO280B, ATN

B1 Interoperability Requirements ED120/DO290, Domestic SPR

2007

Ground system infrastructur e

Communicati on service

FANS 1/A

CPDLC Ground System

Technical standards to ensure operational and technical interoperability, such as : Annex 10 Vol II ED100/DO258, FANS 1/A

Interoperability Requirements

ANSP

Ground systems supplier

2002

Ground system infrastructur e

Communicati on service

ATN B1 CPDLC

Ground System

Technical standards to ensure operational and technical interoperability, such as : Annex 10 Vol II ED110B & ED120

Aircraft manufacturer

ANSP

ED100/DO258, FANS 1/A Interoperability Requirements

ANSP

Ground systems supplier

2007

Aircraft operator

Aircraft operator

Aircraft manufacturer

Training	Communicati on service

Training requirements for CPDLC (FANS 1/A & ATN B1)

for domestic and procedural airspace

Flight Crew Training, Maintenance and Flight Operations Training, controllers training ICAO Doc 10037 Edition 1 Global Operational Data link Manual (GOLD)

2013

| Regulatory | Equipage | CPDLC | Airspace requirements and separation | ANSP | CAA |  | 2013 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| provisions | requirements or/and | Requirements and/or mandates | standards, with associated means of compliance. References: Amendment of | Aircraft operator |  |  |  |

mandates

Regional Supplementary Procedures –

Doc 7030 (PAC, NAT and EUR)

Regulatory	SMS

provisions

CPDLC Safety

Management System

Requirements to support local safety

cases. References: Annex 19 Annex 11

PANS ATM 4444

ANSP

2013

COMS-B0/2	ADS-C (FANS 1/A) for procedural airspace	Technology

Sixth edition of the GANP

Main Purpose	Supports improvement of surveillance in airspace where procedural separation is being applied.

New Capabilities	ADS-C (FANS 1/A) provides aircraft position and projected route information (i.e. next two waypoints ahead of aircraft).

Description		The ADS-C capability provides automatically, without pilot intervention, an ATS unit with information concerning the aircraft position and projected profile for the flight at time intervals, events or on demand dictated by the ground needs.

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

En-route

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-technology need	COMI-B0/1 - Aircraft Communication Addressing and Reporting System

(ACARS)

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational Communicati ADS-C

procedures	on service	procedures and

monitoring

Requirements and guidance to support

deployment of CPDLC operations, such as : Annex 10 Vol II ICAO Doc 4444, Procedures for Air Navigation Services Air Traffic Management ICAO Doc 10037 Edition 1 Global Operational Datalink Manual (GOLD)

2013

Aircraft manufacturer

Aircraft operator

Airborne

system capability

Communicati

on service

FANS 1/A ADS-

C aircraft system

Technical standards to ensure operational

and technical interoperability, such as : Annex 10 Vol II ED100/DO258, FANS 1/A

Interoperability Requirements

Aircraft

manufacturer	2002

Aircraft operator

Ground

system infrastructur e

Communicati

on service

FANS 1/A ADS-

C Ground System

Technical standards to ensure operational

and technical interoperability, such as : Annex 10 Vol II ED100/DO258, FANS 1/A

Interoperability Requirements

ANSP

Ground systems supplier

2002

Training

Communicati

on service

Training

requirements for ADS-C (FANS

1/A) for procedural

airspace

Flight Crew Training, Maintenance and

Flight Operations Training and air traffic controllers ICAO Doc 10037 Edition 1 Global Operational Datalink Manual (GOLD)

2013

Aircraft manufacturer

Aircraft operator

Regulatory

provisions

Equipage

requirements or/and mandates

ADS-C

Requirements and/or mandates

Airspace requirements and separation

standards, with associated means of compliance. References: Amendment of Regional Supplementary Procedures – Doc 7030 (PAC and NAT)

ANSP	CAA	2013

Regulatory	SMS

provisions

ADS-C Safety

Management System

Requirements to support local safety

cases. References: Annex 19 Annex 11

PANS ATM 4444

ANSP

2013

ANSP

ANSP

Aircraft operator

Sixth edition of the GANP

Main Purpose

Supports:

reduction of voice channel congestion and increase of capacity in domestic airspace, introduction of performance-based reduced separation minima in procedural airspace.

New Capabilities	FANS 1/A+ CPDLC systems (along with associated air-ground network and physical layers) are

demonstrated compliant with RCP240 (procedural) and RCP130 (domestic).

Technology

PBCS approved CPDLC (FANS 1/A+) for

domestic and procedural airspace

COMS-B1/1

Description 		In procedural airspace, RCP240 authorized CPDLC (FANS 1/A+) provides the controller with intervention capability, allowing when used in conjunction with other enablers (e.g. ADS-C and navigation capabilities), reduced separation minima and thus capacity increase.

In domestic airspace, RCP 130 compliant CPDLC (FANS 1/A+) provides a complementary means of communications for en-route routine communications and the extension of CPDLC for ground operations (e.g. departure clearance).

PBCS will also enhance airspace access for State aircraft as it would offer opportunities for application of performance-equivalence.

Maturity Level 	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational Communicati CPDLC

procedures	on service	procedures and

monitoring

Requirements and guidance to support

deployment of CPDLC operations, such as : Annex 10 Vol II ICAO Doc 4444, Procedures for Air Navigation Services Air Traffic Management ICAO Doc 10037 Edition 1 Global Operational Datalink Manual (GOLD)

2013

Operational Communicati Datalink

procedures	on service	performance

requirements and monitoring

Guidance to support deployment of

Performance Based operations, such as: ICAO Doc 9869 Third Edition Performance-Based Communication and Surveillance (PBCS) Manual

2019

Aircraft operator

Aircraft manufacturer

ANSP

PLANNING LAYERS

Tactical-During ops

Tactical-Pre ops

OPERATIONS

En-route

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

COMS-B0/1 - CPDLC (FANS 1/A & ATN B1) for domestic and procedural

airspace

| ANSP |  |
| --- | --- |
| Aircraft manufacturer |  |
| Aircraft operator |  |
| Communication |  |
| service provider |  |

Airborne system capability

Communicati on service

FANS 1/A+

CPDLC aircraft system

Technical standards to ensure operational and technical interoperability, such as: Annex 10 Vol II ED122/DO306, including Change 1 & 2, Oceanic Remote Safety and Performance Requirements ED100A/DO258A, FANS 1/A+

Ground systems supplier

ANSP

Interoperability Requirements

2007

Ground system infrastructur e

Communicati on service

FANS 1/A+

CPDLC Ground System

Technical standards to ensure operational and technical interoperability, such as : Annex 10 Vol II ED122/DO306, including Change 1 & 2, Oceanic Remote Safety and Performance Requirements ED100A/DO258A, FANS 1/A+

Aircraft manufacturer

ANSP

Interoperability Requirements

2007

Aircraft operator

Training	Communicati on service

Training requirements for PBCS approved CPDLC (FANS

1/A+) for domestic and procedural airspace

Controllers training, Flight Crew Training, Maintenance and Flight Operations Training. ICAO Doc 10037 Edition 2 Global Operational Datalink Manual (GOLD)

2019

Regulatory provisions

Equipage requirements or/and mandates

CPDLC PBCS

Requirements and/or mandates

Airspace requirements and separation standards, with associated means of compliance. References: Amendment of Regional Supplementary Procedures – Doc 7030 (PAC and NAT) Annex 6 Annex 11 Annex 15 ICAO Doc 9869 Second Edition Performance-Based Communication and Surveillance (PBCS) Manual

ANSP	CAA

Aircraft operator

2016

Regulatory provisions

SMS	CPDLC PBCS

Aircraft operator

Safety Management System

Requirements to support local safety cases. References: Annex 19 Annex 11

PANS ATM 4444

ANSP 	2013

Regulatory provisions

Operational Approval

PBCS Approval	Requirements and guidance for PBCS

operational authorization in Annex 6 and Doc 9869, 3rd Edition

CAA

2019

Aircraft operator

Aircraft manufacturer

Sixth edition of the GANP

Main Purpose 		Supports introduction of performance-based reduced separation minima in procedural airspace.

Technology

PBCS approved ADS-C (FANS 1/A+) for

procedural airspace

COMS-B1/2

New Capabilities 	 FANS 1/A+ ADS-C systems (along with associated air-ground network and physical layers) are demonstrated compliant with RSP180.

Description 		ADS-C (FANS 1/A) RSP180 authorized provides separation assurance, allowing when used in conjunction with other enablers (e.g. CPDLC and navigation capabilities), reduced separation minima and thus capacity increase.

PBCS will also enhance airspace access for State aircraft as it would offer opportunities for application of performance-equivalence.

Maturity Level 	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

OPERATIONS

En-route

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

COMS-B0/2 - ADS-C (FANS 1/A) for procedural airspace

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational Communicati ADS-C

procedures	on service	procedures and

monitoring

Requirements and guidance to support

deployment of CPDLC operations, such as : Annex 10 Vol II ICAO Doc 4444, Procedures for Air Navigation Services Air Traffic Management ICAO Doc 10037 Edition 1 Global Operational Datalink Manual (GOLD)

2013

Airborne	Communicati FANS 1/A+

system	on service	ADS-C aircraft capability		system

Technical standards to ensure operational

and technical interoperability, such as: Annex 10 Vol II ED122/DO306, including Change 1 & 2, Oceanic Remote Safety and Performance Requirements ED100A/DO258A, FANS 1/A+

Interoperability Requirements

2007

Aircraft operator

Aircraft manufacturer

Aircraft operator

Aircraft manufacturer

ANSP

Ground system infrastructur e

Communicati on service

FANS 1/A+

ADS-C Ground System

Technical standards to ensure operational and technical interoperability, such as: Annex 10 Vol II ED122/DO306, including Change 1 & 2, Oceanic Remote Safety and Performance Requirements ED100A/DO258A, FANS 1/A+

Aircraft manufacturer

ANSP

Interoperability Requirements

ANSP

Ground systems supplier

2007

Aircraft operator

Training	Communicati on service

Training requirements for PBCS approved ADS-C (FANS

1/A+) for procedural airspace

Controllers training, Flight Crew Training, Maintenance and Flight Operations Training. ICAO Doc 10037 Edition 1 Global Operational Datalink Manual (GOLD)

2016

Regulatory provisions

SMS	ADS-C Safety Management System

Requirements to support local safety cases. References: Annex 19 Annex 11

PANS ATM 4444

ANSP 	2013

Regulatory provisions

Equipage requirements or/and mandates

ADS-C PBCS

Requirements and/or mandates

Airspace requirements and separation standards, with associated means of compliance. References: Amendment of Regional Supplementary Procedures – Doc 7030 (PAC and NAT) Annex 6 Annex 11 Annex 15 ICAO Doc 9869 Second Edition Performance-Based Communication and Surveillance (PBCS) Manual

ANSP   CAA

2016

Operational procedures

Communicati on service

Datalink performance requirements and monitoring (ADS- C)

Guidance to support deployment of Performance Based operations, such as: ICAO Doc 9869 Second Edition Performance-Based Communication and Surveillance (PBCS) Manual

ANSP

Aircraft manufacturer

Aircraft operator

Communication

service provider

2016

Regulatory provisions

Operational Approval

PBCS Approval (RSP 180)

Requirements and guidance for PBCS operational authorization in Annex 6 and Doc 9869, 2nd Edition

CAA

2016

Aircraft operator

Aircraft operator

Sixth edition of the GANP

Main Purpose

Increase quality of voice communications in procedural airspace without VHF coverage.

New Capabilities

Use of SATVOICE for all types of ATS communications (routine and emergency/urgency

communications).

Technology

SATVOICE (incl. routine communications) for

procedural airspace

COMS-B1/3

Description 		A priority level is used by dedicated networks and the aircraft system to pre-empt calls of lower priority, if necessary, and establish precedence for incoming calls of higher priority (usually routine ATS communication will be Priority 2 out of 4, Priority 1 being dedicated to distress/urgent calls).

PBCS will also enhance airspace access for State aircraft as it would offer opportunities for application of performance-equivalence.

Maturity Level 	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Regulatory

provisions

Equipage

requirements or/and mandates

Requirements for

the use of SATVOICE

Annex 10 , Volume III Annex 4 PANS-

ATM (Doc 4444) PANS-AIM (Doc 10066)

2016

Regulatory	SMS

provisions

SATVOICE

Safety Management System

Requirements to support local safety

cases. References: Annex 19 Annex 11

PANS ATM 4444

ANSP

2013

Operational -

procedures

SATVOICE

procedures

Doc 10038, Satellite Voice Operations

Manual (SVOM)

ANSP

2019

Aircraft operator

Aircraft manufacturer

PLANNING LAYERS

Tactical-During ops

OPERATIONS

En-route

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology need

COMI-B0/5 - Satellite communications (SATCOM) Class C Data

| ANSP | CAA |  |
| --- | --- | --- |
| Aircraft manufacturer |  |  |
| Aircraft operator |  |  |
| Communication service provider |  |  |
| Satellite provider |  |  |

Aircraft manufacturer

Aircraft operator

Communication

service provider

Operational -

procedures

SATVOICE

performance monitoring

Reference: Performance-Based

Communication and Surveillance (PBCS) Manual, Second Edition.

2016

Airborne

system capability

Aircraft

system

SATVOICE

airborne system

References: Annex 10, Volume III Manual

on the Aeronautical Mobile Satellite (Route) Service. (Doc 9925)

2016

Ground

system infrastructur e

-

SATVOICE

ground system

References: Annex 10, Volume III Manual

on the Aeronautical Mobile Satellite (Route) Service. (Doc 9925)

ANSP

Ground systems supplier

2016

Training

-

Training

requirements for SATVOICE

SATVOICE training for flight crew, radio

operator and air traffic controller References: Doc 10038, Satellite Voice Operations Manual (SVOM), Edition 1

2019

Airspace user

ANSP

Aircraft operator

Aircraft manufacturer

ANSP

COMS-B2/1	PBCS approved CPDLC (B2) for domestic and procedural airspace

Technology

Sixth edition of the GANP

Main Purpose		Provision of Air Traffic Services (ATS), with the extension of data link communications use in support of various ATM enhancements among which:

Trajectory-based operations, with new CPDLC messages to uplink route amendments (with standardized automation rules to ensure consistent Flight Management System implementation among the aircraft fleet),

Surface operations, with new CPDLC TAXI (D-TAXI) messages.

New Capabilities	New generation of CPDLC application, named Baseline 2, designed to:

perpetuate legacy data link operations in procedural and domestic airspace, introduce new data link services (e.g. 4D trajectory exchange, D-TAXI).

Description		Baseline 2 data link capabilities (CPDLC and ADS-C) support provision of data communications in all operational environments e.g., continental, oceanic, and surface in support of provision of air traffic control services.

Baseline 2 CPDLC has been designed to be independent of the underlying technologies.

PBCS will also enhance airspace access for State aircraft as it would offer opportunities for application of performance-equivalence.

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

OPERATIONS

Does it imply a change to levels of automation? Yes

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational Communicati Datalink

procedures	on service	performance

requirements and monitoring

Guidance to support deployment of

Performance Based operations, such as: ICAO Doc 9869 Third Edition Performance-Based Communication and Surveillance (PBCS) Manual

2019

Operational Communicati CPDLC (B2)

procedures	on service	procedures and

monitoring

Requirements and guidance to support

deployment of CPDLC operations, such as: Annex 10 Vol II ICAO Doc 4444, Procedures for Air Navigation Services Air Traffic Management ICAO Doc 10037 Edition 2 Global Operational Datalink Manual (GOLD)

2024

Aircraft operator

Aircraft manufacturer

ANSP

ANSP

PLANNING LAYERS

Tactical-During ops

Tactical-Pre ops

| Taxi-out | Departure | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

COMS-B0/1 - CPDLC (FANS 1/A & ATN B1) for domestic and procedural

airspace

Evolution

COMS-B1/1 - PBCS approved CPDLC (FANS 1/A+) for domestic and

procedural airspace

Relation-technology need

COMI-B0/2 - Aeronautical Telecommunication Network/Open System

Interconnection (ATN/OSI)

Relation-benefit

COMI-B2/1 - Air-Ground ATN/IPS

Aircraft manufacturer

Aircraft operator

Communication

service provider

Airborne system capability

Communicati on service

Baseline 2 CPDLC aircraft system

Technical standards to ensure operational and technical interoperability, such as: Annex 10 Vol II ED228A/DO350A,

Ground systems supplier

ANSP

Baseline 2 Safety and Performance Requirements ED229A/DO351A, Baseline 2 Interoperability Requirements ED230A/DO352A, Baseline 2 FANS 1/A Accommodation Interoperability Requirements ED232A/DO353A, Baseline 2 ATN B1 Accommodation Interoperability Requirements

2016

Ground system infrastructur e

Communicati on service

Baseline 2 CPDLC Ground System

Technical standards to ensure operational and technical interoperability, such as: Annex 10 Vol II ED228A/DO350A,

Aircraft manufacturer

ANSP

Baseline 2 Safety and Performance Requirements ED229A/DO351A, Baseline 2 Interoperability Requirements ED230A/DO352A, Baseline 2 FANS 1/A Accommodation Interoperability Requirements ED232A/DO353A, Baseline 2 ATN B1 Accommodation Interoperability Requirements

2016

Aircraft operator

Training	Communicati on service

Training requirements for PBCS approved CPDLC (B2) for

CAA

domestic and procedural airspace

Controllers training, Flight Crew Training, Maintenance and Flight Operations Training. ICAO Doc 10037 Edition 2 Global Operational Data link Manual (GOLD)

2019

Regulatory provisions

Operational Approval

PBCS Approval	Requirements and guidance for PBCS

operational authorization in Annex 6 and Doc 9869, 3rd Edition

2019

Aircraft operator

Regulatory provisions

SMS	CPDLC PBCS

(B2) Safety Management System

Requirements to support local safety cases. References: Annex 19 Annex 11

PANS ATM 4444

ANSP 	2013

Aircraft operator

Aircraft manufacturer

Sixth edition of the GANP

Main Purpose 			Provision of Air Traffic Services (ATS), with the extension of data link communications use in support of various ATM enhancements among which:

Trajectory-based operations, in particular with ADS-C Extended Project Profile (i.e. predicted route ahead of the aircraft, up to 128 waypoints with their predicted level, speed and time).

PBCS Approved ADS-C (B2) for domestic and	Technology

procedural airspace

COMS-B2/2

New Capabilities 	 New generation of ADS-C applications, named Baseline 2, designed to:

perpetuate legacy data link operations in procedural airspace  new data link services (e.g.4D trajectory exchange)

Description 		Baseline 2 data link capabilities (CPDLC and ADS-C) support provision of data communications in all operational environments e.g., continental, oceanic, and surface in support of provision of air traffic control services.

Baseline 2 ADS-C has been designed to be independent of the underlying technologies.

PBCS will also enhance airspace access for State aircraft as it would offer opportunities for application of performance-equivalence.

Maturity Level 	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational Communicati Datalink

procedures	on service	performance

requirements and monitoring (B2 ADS-C)

Guidance to support deployment of

Performance Based operations, such as: ICAO Doc 9869 Fourth Edition Performance-Based Communication and Surveillance (PBCS) Manual

2024

ANSP

PLANNING LAYERS

Tactical-During ops

Tactical-Pre ops

OPERATIONS

Arrival

En-route

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

COMS-B1/2 - PBCS approved ADS-C (FANS 1/A+) for procedural

airspace

Relation-technology need

COMI-B0/2 - Aeronautical Telecommunication Network/Open System

Interconnection (ATN/OSI)

Relation-benefit

COMI-B2/1 - Air-Ground ATN/IPS

Aircraft manufacturer

Aircraft operator

Communication

service provider

Operational procedures

Communicati on service

ADS-C (B2)

Aircraft operator

procedures and monitoring

Requirements and guidance to support deployment of CPDLC operations, such as: Annex 10 Vol II ICAO Doc 4444, Procedures for Air Navigation Services Air Traffic Management ICAO Doc 10037 Edition 2 Global Operational Datalink Manual (GOLD)

2024

Airborne system capability

Communicati on service

Baseline 2 ADS- C aircraft system

Technical standards to ensure operational and technical interoperability, such as: Annex 10 Vol II ED228A/DO350A,

Baseline 2 Safety and Performance Requirements ED229A/DO351A, Baseline 2 Interoperability Requirements ED230A/DO352A, Baseline 2 FANS 1/A Accommodation Interoperability Requirements ED232A/DO353A, Baseline 2 ATN B1 Accommodation Interoperability Requirements

2016

Ground system infrastructur e

Communicati on service

Baseline 2 ADS- C Ground System

Technical standards to ensure operational and technical interoperability, such as: Annex 10 Vol II ED228A/DO350A,

Aircraft manufacturer

ANSP

Baseline 2 Safety and Performance Requirements ED229A/DO351A, Baseline 2 Interoperability Requirements ED230A/DO352A, Baseline 2 FANS 1/A Accommodation Interoperability Requirements ED232A/DO353A, Baseline 2 ATN B1 Accommodation Interoperability Requirements

ANSP

Ground systems supplier

2016

Aircraft operator

Training	Communicati on service

Training requirements for PBCS approved ADS-C (B2) for

domestic and procedural airspace

Controllers training, Flight Crew Training, Maintenance and Flight Operations Training. ICAO Doc 10037 Edition 2 Global Operational Data link Manual (GOLD)

2019

Regulatory provisions

SMS	ADS-C PBCS

Aircraft operator

(B2) Safety Management System

Requirements to support local safety cases. References: Annex 19 Annex 11

PANS ATM 4444

ANSP 	2013

Regulatory provisions

Operational Approval

PBCS Approval (RSP 160)

Requirements and guidance for PBCS operational authorization in Annex 6 and Doc 9869, 4th Edition

CAA

2024

Aircraft operator

Aircraft manufacturer

Aircraft manufacturer

ANSP

COMS-B2/3

PBCS approved SATVOICE (incl. routine

communications) for procedural airspace

Technology

Sixth edition of the GANP

Main Purpose 	Supports introduction of reduced separation minima in procedural airspace.

New Capabilities   SATVOICE systems are demonstrated compliant with a new RCP specification (to be developed).

Description 		In procedural airspace, PBCS approved SATVOICE systems provides the controller with intervention capability, allowing when used in conjunction with other enablers (e.g. ADS-C or space- based ADS-B and navigation capabilities), reduced separation minima and thus capacity increase.

PBCS will also enhance airspace access for State aircraft as it would offer opportunities for application of performance-equivalence.

Maturity Level 	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

OPERATIONS

En-route

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

COMS-B1/3 - SATVOICE (incl. routine communications) for procedural

airspace

Relation-technology need

COMI-B0/5 - Satellite communications (SATCOM) Class C Data

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Regulatory	SMS

provisions

SATVOICE

PBCS SMS for procedural airspace

Requirements to support local safety

cases. References: Annex 19 Annex 11

PANS ATM 4444

ANSP

2013

Operational -

procedures

SATVOICE

PBCS

operational procedures for procedural airspace

PANS-ATM (Doc 4444), Procedures for

Air Navigation Services — Air Traffic Management Doc 10038, Satellite Voice Operations Manual (SVOM), Edition 2

ANSP

2024

Aircraft operator

Aircraft manufacturer

Operational procedures

-	SATVOICE

PBCS

Aircraft operator

Aircraft manufacturer

performance monitoring for procedural airspace

Performance-Based Communication and Surveillance (PBCS) Manual, Edition 4

2024

Airborne system capability

Aircraft system

SATVOICE

PBCS airborne system for procedural airspace

Annex 10, Volume III Manual on the Aeronautical Mobile Satellite (Route) Service. (Doc 9925)

2024

Ground system infrastructur e

-	SATVOICE

PBCS ground system for procedural airspace

Annex 10, Volume III Manual on the Aeronautical Mobile Satellite (Route) Service. (Doc 9925)

ANSP

Ground systems supplier

2024

Regulatory provisions

Operational Approval

SATVOICE PBCS

operational approval for procedural airspace

Requirements and guidance for PBCS operational authorization in Annex 6 and Doc 9869, 4th Edition.

CAA

Aircraft operator

2024

Training	-	Training for SATVOICE

PBCS for procedural airspace

Doc 10038, Satellite Voice Operations Manual (SVOM), Edition 2

ANSP

Aircraft operator

2024

ANSP

Aircraft manufacturer

Aircraft operator

Communication

service provider

Sixth edition of the GANP

Main Purpose

Supports dynamic PBN rerouting and advanced interval management operations.

New Capabilities	Extension of B2 CPDLC services to introduce instructions for dynamic RNP and advanced interval

management.

Description

Maturity Level

Concept

Extended CPDLC (B2 incl. Adv-IM and dynamic	Technology

RNP) for dense and complex airspace

COMS-B3/1

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

Tactical-Pre ops

OPERATIONS

Arrival

En-route

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

COMS-B2/1 - PBCS approved CPDLC (B2) for domestic and procedural

airspace

### ENABLERS

Aircraft manufacturer

ANSP

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Aircraft operator

Operational procedures

Communicati on service

CPDLC (B2 incl. Adv-IM and DRNP)

procedures and monitoring

Requirements and guidance to support deployment of CPDLC operations, such as: Annex 10 Vol II ICAO Doc 4444, Procedures for Air Navigation Services Air Traffic Management ICAO Doc 10037 Edition x Global Operational Data link Manual (GOLD)

2028

service provider

Communication

Aircraft manufacturer

Operational procedures

Communicati on service

Datalink performance requirements and monitoring (B2 incl. Adv-IM and DRNP)

Guidance to support deployment of Performance Based operations, such as: ICAO Doc 9869 Edition x Performance- Based Communication and Surveillance (PBCS) Manual

2028

Aircraft operator

Aircraft manufacturer

ANSP

Airport operator

Airborne system capability

Communicati on service

Baseline 2 CPDLC (B2 incl. Adv-IM and DRNP) aircraft system

Technical standards to ensure operational and technical interoperability, such as: Annex 10 Vol II ED228B/DO350B,

Baseline 2 Safety and Performance Requirements ED229B/DO351B, Baseline 2 Interoperability Requirements ED230B/DO352B, Baseline 2 FANS 1/A Accommodation Interoperability Requirements ED232B/DO353B, Baseline 2 ATN B1 Accommodation Interoperability Requirements

2023

Ground	Communicati Baseline 2

system	on service	CPDLC (B2 incl. infrastructur		Adv-IM and

e	DRNP) Ground

System

Technical standards to ensure operational

and technical interoperability, such as: Annex 10 Vol II ED228B/DO350B,

Baseline 2 Safety and Performance Requirements ED229B/DO351B, Baseline 2 Interoperability Requirements ED230B/DO352B, Baseline 2 FANS 1/A Accommodation Interoperability Requirements ED232B/DO353B, Baseline 2 ATN B1 Accommodation

Interoperability Requirements

ANSP

Ground systems supplier

2023

Training

Communicati

on service

Training

requirements for extended CPDLC (B2 incl. Adv-IM and DRNP) for domestic airspace

Controllers Training, Flight Crew Training,

Maintenance and Flight Operations Training. ICAO Doc 10037 Edition x Global Operational Data link Manual (GOLD)

2025

Aircraft manufacturer

Aircraft operator

Regulatory	SMS

provisions

CDPLC PBCS

(B2 incl. Adv-IM and DRNP) Safety Management System

Requirements to support local safety

cases. References: Annex 19 Annex 11

PANS ATM 4444

ANSP

2013

Regulatory	Operational

provisions	Approval

PBCS Approval

CPDLC (B2 incl. Adv-IM and DRNP)

Requirements and guidance for PBCS

operational authorization in Annex 6 and Doc 9869, X Edition

CAA

2028

ANSP

Aircraft operator

Sixth edition of the GANP

Main Purpose

Supports dynamic PBN rerouting and advanced interval management operations.

New Capabilities	Extension of B2 ADS-C services to introduce reports for dynamic RNP and advanced interval

management.

Description

Maturity Level

Concept

Extended ADS-C (B2 incl. Adv-IM and dynamic	Technology

RNP) for dense and complex airspace

COMS-B3/2

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

Tactical-Pre ops

OPERATIONS

Arrival

En-route

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

COMS-B2/2 - PBCS Approved ADS-C (B2) for domestic and procedural

airspace

### ENABLERS

ANSP

Airport operator

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Aircraft manufacturer

Operational procedures

Communicati on service

Datalink performance requirements and monitoring (B2 incl. Adv-IM and DRNP)

Guidance to support deployment of Performance Based operations, such as: ICAO Doc 9869 Edition x Performance- Based Communication and Surveillance (PBCS) Manual

2028

service provider

Communication

Aircraft operator

Operational procedures

Communicati on service

ADS-C (B2 incl. Adv-IM and DRNP)

procedures and monitoring

Requirements and guidance to support deployment of CPDLC operations, such as: Annex 10 Vol II ICAO Doc 4444, Procedures for Air Navigation Services Air Traffic Management ICAO Doc 10037 Edition x Global Operational Data link Manual (GOLD)

2028

Aircraft operator

Aircraft manufacturer

Aircraft manufacturer

ANSP

Airborne system capability

Communicati on service

Baseline 2 ADS- C (B2 incl. Adv- IM and DRNP) aircraft system

Technical standards to ensure operational and technical interoperability, such as: Annex 10 Vol II ED228B/DO350B,

Baseline 2 Safety and Performance Requirements ED229B/DO351B, Baseline 2 Interoperability Requirements ED230B/DO352B, Baseline 2 FANS 1/A Accommodation Interoperability Requirements ED232B/DO353B, Baseline 2 ATN B1 Accommodation Interoperability Requirements

2023

Ground	Communicati Baseline 2 ADS-

system	on service	C (B2 incl. Adv- infrastructur		IM and DRNP)

e	Ground System

Technical standards to ensure operational

and technical interoperability, such as: Annex 10 Vol II ED228B/DO350B,

Baseline 2 Safety and Performance Requirements ED229B/DO351B, Baseline 2 Interoperability Requirements ED230B/DO352B, Baseline 2 FANS 1/A Accommodation Interoperability Requirements ED232B/DO353B, Baseline 2 ATN B1 Accommodation

Interoperability Requirements

ANSP

Ground systems supplier

2023

Training

Communicati

on service

Training

requirements for extended ADS-C (B2 incl. Adv-IM and DRNP) for domestic airspace

Controllers Training, Flight Crew Training,

Maintenance and Flight Operations Training. ICAO Doc 10037 Edition x Global Operational Data link Manual (GOLD)

2025

Aircraft manufacturer

Aircraft operator

Regulatory	SMS

provisions

ADS-C PBCS

(B2 incl. Adv-IM and DRNP) Safety Management System

Requirements to support local safety

cases. References: Annex 19 Annex 11

PANS ATM 4444

ANSP

2013

Regulatory	Operational

provisions	Approval

PBCS Approval

ADS-C (B2 incl. Adv-IM and DRNP)

Requirements and guidance for PBCS

operational authorization in Annex 6 and Doc 9869, X Edition

CAA

2028

ANSP

Aircraft operator

## CSEP

Sixth edition of the GANP

Main Purpose 		To improve traffic situational awareness in all phases of flight.

Basic airborne situational awareness during flight  Operational

operations (AIRB)

CSEP-B1/1

New Capabilities    AIRB facilitates out-the-window visual acquisition of airborne traffic within visual range and traffic situational awareness for traffic beyond visual range.

It enhances traffic situational awareness and quicker visual acquisition of targets through basic airborne situational awareness during flight operations enabled by the use of a cockpit display traffic information (CDTI).

It supplements the flight crew’s out the window scan and radio communications listening.

It supports the flight crew in integrating information from these sources into a comprehensive and accurate traffic picture including long traffic detection range.

It also enables communication with surrounding aircraft on common/local frequency (e.g. turbulence reports, uncontrolled airfield operations).

Description 		The use of cockpit displays to provide the flight crew with a graphical depiction of traffic using relative range and bearing, supplemented by altitude, flight ID and other information. This

element represents the use of the cockpit display traffic information (CDTI) with appropriate ADS-B data filtered for traffic situational awareness. The CDTI is capable of merging data with TCAS. It is recommended to use the display where ACAS information is already provided (if ACAS-equipped)

Maturity Level 	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? No

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   En-route   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology need

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-technology benefit

ACAS-B1/1 - ACAS Improvements

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Regulatory	Certification	Certification of

provisions	CDTI for airborne situational awareness

References: ICAO Annex 10 Volume IV

basic technical requirements ICAO Doc 9994 ADS-B IN equipment/function compliant with DO-317B/ED194A (2015)

2015

Operational Operations	SOPs for the use

procedures	of CDTI for AIRB

References: PANS-OPS (Doc 8168)

operation of ADS-B IN traffic display (2016) Standard Operating Procedures

2013

Aircraft operator

Aircraft manufacturer

CAA

Aircraft operator

Airborne	Surveillance

system capability

ADS-B IN

equipment/functi on

ADS-B IN equipment/function compliant

with DO-317B/ED194A (2015)

Airspace user	2015

Aircraft manufacturer

Airborne

system capability

Aircraft

system

CDTI

Cockpit Display of Traffic Information

(CDTI). The CDTI may be shared with ACAS information traffic display but we should not assume this capability is only for ACAS-equipped aircraft. References:

2013

Training

-

Training

requirements for AIRB

The appropriate use of the traffic display

could be evaluated during recurrent training on ACAS. The equipage of AIRB capability is unknown to ATC. The use of AIRB must remain transparent to ATC. References:

2013

Aircraft operator

Aircraft manufacturer

Aircraft operator

### DEPLOYMENT APPLICABILITY

#### Operational conditions:

AIRB may be introduced in a partial equipage of ADS-B OUT, in which only some aircraft are equipped with ADS-B OUT.

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |  |
| --- | --- | --- | --- |
| Direct benefits | Safety and efficiency: Improving the flight crew’s decision-making process and the resulting actions for the safe and efficient operation of the flight with earlier detection of developing unsafe situations and better flight trajectory execution. Note: NOTE: Benefits vary depending on the airspace and operational flight rules. | Airspace user |  |
|  | Safety and efficiency: Identification of the traffic ahead, enabling direct pilot-to-pilot communications, in oceanic airspace, to request turbulence information. Note: NOTE: Benefits vary depending on the airspace and operational flight rules. | Airspace user |  |
|  | Vertical flight efficiency: Saving up to 15 000 Kg fuel per aircraft annually in North Atlantic airspace (5 min separation, about 40 NM). Substantially higher benefits in larger separation environments | Aircraft operator |  |
|  | Safety: Reduced weather turbulence encounters through direct pilot-to-pilot communications Enhanced traffic situation awareness | Airspace user |  |

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
|  | Improve situational awareness of the flight crew | Airspace user |
| Indirect benefits | Improved efficiency, reduced exchanges of R/T. | ANSP |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Improve mid-air collision avoidance (safety net) | ++ | KPI20: Number of aircraft accidents KPI23: Number of airprox/TCAS alert/loss of separation/near midair collisions/midair collisions (MAC) |
| Safety |  | Improve separation provision (at a planning horizon > 2 minutes) | ++ | KPI20: Number of aircraft accidents KPI23: Number of airprox/TCAS alert/loss of separation/near midair collisions/midair collisions (MAC) |

Sixth edition of the GANP

Main Purpose

To assist pilots in maintaining own separation during successive visual approach procedures. VSA

is defined to support aircraft performing successive visual approach and landing operations.

New Capabilities	VSA is a situational awareness capability that offers a support tool for pilots to conduct existing

procedures described in PANS-ATM (6.5.3.4 and 6.5.3.5). It assist the flight crew in maintaining own separation and quicker visual acquisition of preceding aircraft during successive visual approach procedures enabled by the use of a CDTI. It supplements the flight crew’s out the window scan and radio communications listening.

Operational

Visual Separation on Approach (VSA)

CSEP-B1/2

Description 	This element is used to support existing procedures, i.e., the pilot can use the traffic display to support his visual scan of the preceding aircraft during the approach procedure if the preceding aircraft is transmitting ADS-B OUT surveillance data.

Maturity Level 	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? No

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology need

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-technology benefit

ACAS-B1/1 - ACAS Improvements

### ENABLERS

Aircraft manufacturer

CAA

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Aircraft operator

Regulatory provisions

Certification	Certification of

CDTI for airborne situational awareness

References: ICAO Annex 10 Volume IV basic technical requirements ICAO Doc 9994 ADS-B IN equipment/function compliant with DO-317B/ED194A (2015)

2015

Aircraft operator

Aircraft manufacturer

Airborne system capability

Surveillance	ADS-B IN

equipment/functi on

ADS-B IN equipment/function compliant with DO-317B/ED194A (2015)

Airspace user

Aircraft manufacturer

2015

Airborne system capability

Aircraft system

CDTI	Cockpit Display of Traffic Information (CDTI). The CDTI may be shared with ACAS information traffic display but we should not assume this capability is only for ACAS-equipped aircraft. References:

2013

Aircraft operator

Training	-	Training requirements for VSA

Training should be developed and given for the system functions, indications and interactions for each application/capability. The equipage of ADS-B IN is unknown to ATC. The use of ADS-B IN must remain transparent to ATC.

2013

Operational procedures

Operations	SOPs for the use

of CDTI for VSA

References: ICAO Doc 8168 - PANS- OPS; operation of ADS-B IN traffic display (2016)

2016

Airborne system capability

Surveillance	ADS-B OUT

equipment/functi on

ADS-B OUT capability required for the reference aircraft

2015

Aircraft operator

Aircraft operator

DEPLOYMENT APPLICABILITY

#### Operational conditions:

The application can be used by all suitably equipped aircraft during approach to any airports where own separation is used. VSA may be introduced in a partial equipage of ADS-B OUT environment, in which only some aircraft are equipped with ADS-B OUT. VSA is applicable for all types of runway configurations where successive visual approaches are in use (PANS-ATM 6.5.3).

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |  |
| --- | --- | --- | --- |
| Direct benefits | VSA improve the efficiency and safety of arrival traffic at airports. | Airport operator | ANSP |
|  | The safety of the operation will be improved through the provision of accurate data to the pilots (e.g. preceding traffic ground speed) allowing them to better predict the evolution of the traffic. | Airport operator |  |
|  | Improve situational awareness of the flight crew | Aircraft operator |  |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Improve separation provision (at a planning horizon > 2 minutes) | ++ | KPI20: Number of aircraft accidents KPI23: Number of airprox/TCAS alert/loss of separation/near midair collisions/midair collisions (MAC) |

CSEP-B1/3	Performance Based Longitudinal Separation

Minima

Operational

Sixth edition of the GANP

Main Purpose	Reduced separation allowing more flights to operate in their optimum flight levels.

New Capabilities The ability of aircraft to accurately navigate along a specified path and the ability to provide via data communication the position, intent and current navigation performance of the aircraft allow flights to be cleared to fly at reduced longitudinal separations.

Description	50 NM, 30 NM and 5 minutes longitudinal separation have been made conditional on Required Communication Performance 240 (RCP 240) and Required Surveillance Performance 180 (RSP 180). Application of the separation may also require an RNP approval.

##### (Table to be inserted)

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

En-route

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-technology option	COMI-B0/3 - VHF Data Link (VDL) Mode 0/A Relation-technology option	COMI-B0/4 - VHF Data Link (VDL) Mode 2 Basic

Relation-technology option	COMS-B0/1 - CPDLC (FANS 1/A & ATN B1) for domestic and procedural

airspace

Relation-technology option	COMS-B1/1 - PBCS approved CPDLC (FANS 1/A+) for domestic and

procedural airspace

Relation-technology option	COMS-B0/2 - ADS-C (FANS 1/A) for procedural airspace

Relation-technology option	COMS-B1/2 - PBCS approved ADS-C (FANS 1/A+) for procedural

airspace

Relation-technology option	NAVS-B0/3 - Aircraft Based Augmentation Systems (ABAS)

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

| Regulatory provisions | Operational Approval | Operational Approval for performance- based longitudinal separation | PBN, PBCS (when required) | CAA |  | 2019 |
| --- | --- | --- | --- | --- | --- | --- |
| Operational | Design and | Procedures for | Design and use of operational procedures | ANSP |  | 2019 |
| procedures | operations | PBLoSM | shall be designed and used for the application of reduced longitudinal separation minima Reference : ICAO PBN, PBCS and GOLD manual |  |  |  |
| Ground | Surveillance | Surveillance | To support RSP 180 | ANSP |  | 2019 |
| system infrastructur e |  | systems for PBLoSM |  |  |  |  |
| Ground | Navigation | Navigation | To support RNP 2, 4 or 10 | ANSP |  | 2019 |
| system infrastructur e |  | systems for PBLoSM |  |  |  |  |
| Ground | Communicati | Communication | To support RCP 240 | ANSP |  | 2019 |
| system infrastructur e | ons | systems for PBLoSM |  |  |  |  |
| Training | - | Training | ATCO Training and Flight Crew Training | ANSP |  | 2019 |
| requirements for Train for PBN Reduced Longitudinal PBLoSM Separation Minima Provide training to |  |  |  | Aircraft operator |  |  |

Aircraft manufacturer

staff prior to implementation

Airborne system capability

Communicati on service

Communication capabilities for PBLoSM

Aircraft and crew approvals for appropriate RNP specifications Reference: Aircraft and crew approvals for RCP 240 and ADS-C/CPDLC

2019

Aircraft operator

Aircraft manufacturer

Airborne system capability

Navigation	Navigation

capabilities for PBLoSM

Aircraft and crew approvals for appropriate RNP specifications

2019

Aircraft operator

Aircraft manufacturer

Airborne system capability

Surveillance	Surveillance

capabilities for PBLoSM

Aircraft and crew approvals for appropriate RNP specifications Reference: Aircraft and crew approvals for RSP 180

2019

Aircraft operator

DEPLOYMENT APPLICABILITY

Operational conditions:

Applicable to oceanic airspace and/or remote areas.

Main intended benefits:

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Improve what’s needed to reduce longitudinal separation minima | ++ | KPI06: En-route airspace capacity |
| Capacity | Capacity, throughput & utilization | Take advantage of increased navigation precision (airspace with PBN operations) to implement route networks and airspace structures with smaller lateral and vertical safety buffers | ++ | KPI06: En-route airspace capacity |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Benefitting stakeholder(s)

Operational description

Type

CSEP-B1/4	Performance Based Lateral Separation Minima	Operational

Sixth edition of the GANP

Main Purpose	To increase airspace capacity and allow optimum utilization of available airspace.

New Capabilities		The ability of aircraft to accurately navigate along a specified path and the ability to provide via data communication the position, intent and current navigation performance of the aircraft allow flights to be cleared to fly at reduced lateral separations.

Description	This element describes the application of lateral separation of 23 NM between any combination of RNP 4 and RNP 2 approved aircraft operating on parallel or non-intersecting tracks.

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

En-route

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-technology option	COMI-B0/3 - VHF Data Link (VDL) Mode 0/A

Relation-technology option

COMI-B0/4 - VHF Data Link (VDL) Mode 2 Basic

Relation-technology option

COMS-B0/1 - CPDLC (FANS 1/A & ATN B1) for domestic and procedural

airspace

Relation-technology option

COMS-B1/1 - PBCS approved CPDLC (FANS 1/A+) for domestic and

procedural airspace

Relation-technology option

COMS-B0/2 - ADS-C (FANS 1/A) for procedural airspace

Relation-technology option

COMS-B1/2 - PBCS approved ADS-C (FANS 1/A+) for procedural

airspace

Relation-technology option

NAVS-B0/3 - Aircraft Based Augmentation Systems (ABAS)

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

Operational Approval

Operational Approval for performance- based lateral separation

Approval of PBN, PBCS	 CAA 	2019

Operational procedures

Design and operations

Procedures for PBLaSM

Design and use of operational procedures for the application of reduced lateral separation minima Reference : ICAO PBN, PBCS and GOLD manual

ANSP 	2019

Training	-	Training requirements for PBLaSM

ATCO training ATCO and Flight Crew training Performance Based Lateral Separation Minima Provide training to staff prior to implementation

ANSP

2019

Aircraft operator

Aircraft manufacturer

Airborne system capability

Communicati on service

Communication capabilities for PBLaSM

Aircraft and crew approvals for RCP 240

2019

Aircraft manufacturer

Airborne system capability

Navigation	Navigation

capabilities for PBLaSM

Aircraft and crew approvals for appropriate RNP specifications

2019

Airborne system capability

Surveillance	Surveillance

capabilities for PBLaSM

Aircraft and crew approvals for RSP 180

2019

Aircraft operator

Aircraft operator

Aircraft manufacturer

Aircraft operator

Ground system infrastructur e

Communicati ons

Communication systems for PBLaSM

To support the specified RCP	 ANSP 	2019

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Improve what’s needed to reduce lateral separation minima | ++ | KPI06: En-route airspace capacity |

Ground

system infrastructur e

Navigation

Navigation

systems for PBLaSM

To support the specified RNP

ANSP

2019

Ground

system infrastructur e

Surveillance

Surveillance

systems for PBLaSM

To support the specified RSP

ANSP

2019

DEPLOYMENT APPLICABILITY

Operational conditions:

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Benefitting stakeholder(s)

Operational description

Type

CSEP-B2/1	Interval Management (IM) Procedure	Operational

Sixth edition of the GANP

Main Purpose	The objective of the IM application is to achieve and/or maintain an assigned spacing between one aircraft and the designated aircraft (referred as the reference aircraft), using the guidance on the flight deck that enables the pilot to actively manage the spacing relative to the reference aircraft.

New Capabilities	This element allows management by the pilot of the assigned spacing from a reference aircraft using airborne tools.

Description	The air traffic controller chooses to use IM operations to manage the interval between aircraft when the aircraft need to be spaced closely together in an orderly manner or their timing synchronized. This is accomplished by the air traffic controller providing a clearance to the IM aircraft which includes the assigned interval. The assigned interval can be defined in time or distance. The pilot is responsible for identifying the reference aircraft (designated by the air traffic controller) and to implement the IM clearance. While some IM clearances will keep the IM aircraft on its current route and result only in speed management, other clearances may include a controller-defined path lengthening or shortening.

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

OPERATIONS

En-route   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology need

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-technology benefit

ACAS-B1/1 - ACAS Improvements

Relation-benefit

ASUR-B2/1 - Evolution of ADS-B and Mode S

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational Operations	Procedures for

procedures	the use of IM

PANS-ATM/PANS-OPS Interval

Management Procedure (To be developed)

ANSP

Aircraft operator

2025

Airborne	Surveillance	Surveillance

system	system

capability	capabilities for IM

ADS-B IN equipment/function compliant

with DO-317B/ED194A (and subsequent versions) (2014) + ADS-B OUT capability required for the reference aircraft

2014

Airborne	Navigation

system capability

Navigation

system capabilities for IM

FIM equipment /spacing functions with

advisories

2019

Airborne

system capability

Aircraft

system

Traffic display for

IM

Cockpit display of traffic information

2019

Training

-

Training

requirements for IM

Air traffic controllers, Pilots

ANSP

Aircraft operator

2019

Ground

system infrastructur e

ATC systems

ATC tool for IM

Ground tools/capabilities that assist the

air traffic controller in issuing ATC clearances to merge and space aircraft safely and efficiently by allowing ATC to use IM clearances

ANSP

2019

Aircraft operator

Aircraft manufacturer

Aircraft operator

Aircraft manufacturer

Aircraft operator

Regulatory

provisions

Certification

IM Certification

ICAO Annex 10 Technical requirements

(to be developed) ICAO DOC9994

2025

Aircraft operator

Aircraft manufacturer

CAA

### DEPLOYMENT APPLICABILITY

#### Operational conditions:

The air traffic controller chooses to use IM operations to manage the interval between aircraft when the aircraft need to be spaced closely together in an orderly manner or their timing synchronized. This is accomplished by the air traffic controller providing a clearance to the IM aircraft which includes the assigned interval. The assigned interval can be defined in time or distance. The pilot is responsible for identifying the reference aircraft (designated by the air traffic controller) and to implement the IM clearance. While some IM clearances will keep the IM aircraft on its current route and result only in speed management, other clearances may include a controller-defined path lengthening or shortening Note.— IM may not work in all conditions. A combination of air traffic controller judgment and ATC automation will be needed to determine the conditions of which IM can be completed successfully. The assigned interval can be defined in time or distance.

#### Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | The precise management of intervals between aircraft with common or merging trajectories, maximizes airspace throughput while reducing ATC workload along with more efficient aircraft fuel burn reducing environmental impact. | Airport operator ANSP |
|  | Punctuality: IM will improve consistent spacing precision by making more frequent trajectory adjustments than is possible with a ground system alone. | ANSP Aircraft operator |
|  | Reduction flight time variability | ANSP Aircraft operator |
|  | Potential benefits of IM operations include: a) Timely speed advisories removing later requirement for path-lengthening; b) Consistent and low variance spacing between paired aircraft (e.g. at the entry to an arrival procedure and on final approach). More precise spacing can allow for higher throughput and more efficient aircraft operations; c) Optimized descent profile in high density environments; d) Reduced ATC instructions due to the reduced number of speed instructions; and e) More efficient aircraft operations for FIM-equipped aircraft, when aircraft are pre-sequenced, e.g. with the use of an arrival manager (AMAN). | ANSP Aircraft operator |

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
|  | Improve situational awareness of the flight crew | Aircraft operator |
| Indirect benefits | Reduction of radar vectoring (and of R/T exchanges between ATCO and FC). | ANSP |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Reduce ATCO workload (en- route) | ++ | KPI06: En-route airspace capacity |
| Capacity | Capacity, throughput & utilization | Resolve ATC staffing, workload, frequency congestion problems | ++ | KPI10: Airport peak throughput |
| Safety |  | Improve separation provision (at a planning horizon > 2 minutes) | ++ | KPI20: Number of aircraft accidents KPI23: Number of airprox/TCAS alert/loss of separation/near midair collisions/midair collisions (MAC) |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

CSEP-B2/2	Cooperative separation at low altitudes	Operational

Sixth edition of the GANP

Main Purpose	To maintain own separation with respect to manned aviation and other UAS in the UTM airspace.

New Capabilities	A dedicated function to support separation assurance is provided to the UAS, which is supported by detection capability as well as a guidance to remain well clear of any other UAS. Note: in the UTM, this function may be automated and not operated by a remote pilot.

Description		The UAS is capable of building a traffic picture associated to a separation assurance functionality enabling to establish and maintain the required separation from any other vehicle aka “remain well clear” using ADS-B IN for manned aviation and broadcast technologies to be determined for UAS to UAS.

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Regulatory	Certification

provisions

Low-altitude UAS

separation certification

TBD

2025

Regulatory	Operational

provisions	Approval

Low-altitude UAS

separation operational approval

TBD

UAS operator  2025

Operational -

procedures

UTM rules for

separation

TBC: in UTM, rules need to be established  UAS operator  2025

such as “remain well clear” definition, performance requirements…

Airborne

system capability

Surveillance

UAS airborne

surveillance

ADS-B IN 1090ES (to be clear of manned  UAS operator   2025

aviation) and UAT

UAS operator

CAA

UAS

manufacturer

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   En-route   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology need

CSEP-B1/1 - Basic airborne situational awareness during flight operations

(AIRB)

Relation-technology need

ASUR-B2/2 - New community based surveillance system for airborne

aircraft (low and higher airspace)

| Ground | - | Display for UTM | N/A for UAS UTM service provider and/or | UAS operator | 2025 |
| --- | --- | --- | --- | --- | --- |
| system infrastructur |  |  | Remote Pilot Ground station | UTM operator |  |
| e |  |  |  |  |  |
| Ground | Surveillance | UAT for UAS | No new requirements (maybe UAT for | UAS operator | 2025 |
| system |  |  | UAS will trigger the need for ADS-R or |  |  |
| infrastructur |  |  | TIS-R ground infrastructure? |  |  |
| e |  |  |  |  |  |

DEPLOYMENT APPLICABILITY

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Maintain or improve safety of very low-level operations | ++ |  |

Operational conditions:

At low altitudes.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Benefitting stakeholder(s)

Operational description

Type

CSEP-B2/3	Cooperative separation at higher airspace	Operational

Sixth edition of the GANP

Main Purpose	To maintain own separation with respect to manned aviation and other UAS in the upper airspace.

New Capabilities	A dedicated function to support separation assurance is provided to the UAS, which is supported by detection capability as well as a guidance to remain well clear of any other UAS. Note: in the UTM, this function may be automated and not operated by a remote pilot.

Description		The UAS is capable of building a traffic picture associated to a separation assurance functionality enabling to establish and maintain the required separation from any other vehicle aka “remain well clear” using ADS-B IN for manned aviation and broadcast technologies to be determined for UAS to UAS.

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Departure   En-route   Arrival

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-technology need	CSEP-B1/1 - Basic airborne situational awareness during flight operations

(AIRB)

Relation-technology need

ASUR-B2/2 - New community based surveillance system for airborne

aircraft (low and higher airspace)

### ENABLERS

CAA

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

Certification	Higher airspace

UAS operator

UAS separation certification

TBD

2025

UAS operator

Regulatory provisions

Operational Approval

Higher airspace UAS separation operational approval

TBD

2025

Operational procedures

-	Higher airspace rules for separation

In higher airspace, rules need to be established such as “remain well clear” definition, performance requirements…

2025

UTM operator

UAS operator

Airborne system capability

Surveillance	Higher UAS

airborne surveillance

ADS-B IN 1090ES (to be clear of manned aviation) and UAT

2025

UAS operator

Ground system infrastructur e

-	Display for higher UTM

N/A for UAS UTM service provider and/or Remote Pilot Ground station

2025

UAS operator

Ground system infrastructur e

Surveillance	UAT for higher

UAS

No new requirements (maybe UAT for UAS will trigger the need for ADS-R or TIS-R ground infrastructure?

2025

Other	-	MEL, performance compliance for higher UAS

Before reaching the higher airspace, the UAS will cross ATM airspace and may have to comply with the airspace requirements in terms of MEL, performance,… It is possible that in transition period, such operations will be conducted in temporarily segregated airspace to ensure safe and limited impact on ATM operations.

2025

UAS

manufacturer

UAS operator

UAS operator

DEPLOYMENT APPLICABILITY

Operational conditions:

At higher airspace.

Main intended benefits:

Benefitting stakeholder(s)

Operational description

Type

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Maintain or improve safety of higher airspace operations | ++ |  |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

CSEP-B3/1	Interval Management (IM) Procedure with complex geometries

Operational

Sixth edition of the GANP

Main Purpose		The objective of the IM application is to achieve and/or maintain an assigned spacing between one aircraft and the designated aircraft (referred as the reference aircraft), using the guidance on the flight deck that enables the pilot to actively manage the spacing relative to the reference aircraft supported by data link to facilitate exchange of complex clearances in a diversity of operational environments.

New Capabilities		This element allows management by the pilot of the assigned spacing from a reference aircraft using airborne tools and data link in a diversity of operational environments.

Description		The air traffic controller chooses to use IM operations to manage the interval between aircraft when the aircraft need to be spaced closely together in an orderly manner or their timing synchronized.

This is accomplished by the air traffic controller providing a clearance to the IM aircraft which includes the assigned interval. The assigned interval can be defined in time or distance.

The pilot is responsible for identifying the reference aircraft (designated by the air traffic controller) and to implement the IM clearance. While some IM clearances will keep the IM aircraft on its current route and result only in speed management, other clearances may include a controller-defined path lengthening or shortening.

Maturity Level	Concept

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Departure   En-route   Arrival

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-technology need	ASUR-B2/1 - Evolution of ADS-B and Mode S

Relation-technology need	COMS-B3/1 - Extended CPDLC (B2 incl. Adv-IM and dynamic RNP) for

dense and complex airspace

Evolution	CSEP-B2/1 - Interval Management (IM) Procedure

### ENABLERS

There are currently no enablers.

### DEPLOYMENT APPLICABILITY

#### Operational conditions:

Same as in Block 2 but, this element is also applicable in environments with complex geometries.

Main intended benefits:

Type

Operational description

Benefitting stakeholder(s)

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Reduce ATCO workload (en- route) | ++ | KPI06: En-route airspace capacity |
| Capacity | Capacity, throughput & utilization | Resolve ATC staffing, workload, frequency congestion problems | ++ | KPI10: Airport peak throughput |
| Safety |  | Improve separation provision (at a planning horizon > 2 minutes) | ++ | KPI20: Number of aircraft accidents KPI23: Number of airprox/TCAS alert/loss of separation/near midair collisions/midair collisions (MAC) |

Sixth edition of the GANP

Main Purpose 			Aims at maintaining own separation with respect to any manned and/or unmanned aircraft in all phases of flight.

Operational

Remain Well Clear (RWC) functionality for

UAS/RPAS

CSEP-B3/2

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

New Capabilities 	 “Remain well clear” is a dedicated function to support separation assurance is provided to the aircraft, which must remain independent of the collision avoidance logic (ACAS).

While the detection capability can be the same, the objective of the function is different and must be supported by a specific logic based on separation minima developed to suit the new operations.

Description 		The remote pilot ensures separation from suitably equipped aircraft (manned and/or unmanned) thanks to a traffic picture (CDTI) associated to a separation assurance functionality enabling to establish and maintain the required separation (remain well clear) in all phases of flight.

Maturity Level 	Validation

Human Factor Considerations

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   En-route   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology need

ASUR-B2/1 - Evolution of ADS-B and Mode S

Relation-operational need

ACAS-B2/2 - New collision avoidance capability as part of an overall detect

and avoid system for RPAS

ENABLERS

There are currently no enablers.

DEPLOYMENT APPLICABILITY

Operational conditions: Main intended benefits:

Benefitting stakeholder(s)

Operational description

Type

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Reduce ATCO workload (en- route) | ++ | KPI06: En-route airspace capacity |
| Capacity | Capacity, throughput & utilization | Resolve ATC staffing, workload, frequency congestion problems | ++ | KPI10: Airport peak throughput |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Improve separation provision (at a planning horizon > 2 minutes) | ++ | KPI20: Number of aircraft accidents KPI23: Number of airprox/TCAS alert/loss of separation/near midair collisions/midair collisions (MAC) |

CSEP-B4/1	Airborne separation	Operational

Sixth edition of the GANP

Main Purpose	Aims at maintaining own separation with respect to one aircraft via a delegation of responsibility from the ATCO to the flight crew in all phases of flight.

New Capabilities	A dedicated function to support separation assurance is provided to the aircraft, which must remain independent of the collision avoidance logic (ACAS-Bx). While the detection capability can be the same, the objective of the function is different and must be supported by a specific logic based on airborne separation minima developed to suit the new operations.

Description		Airborne Separation = temporary delegation of responsibility to the flight deck for separation provision with suitably equipped designated aircraft.

The flight crew ensures separation from suitably equipped designated aircraft as communicated in new clearances, which relieve the controller from the responsibility for separation between these aircraft. However, the controller retains responsibility for separation from aircraft that are not part of these clearances. The flight crew/remote pilot is provided with a traffic picture (CDTI) associated to a separation assurance functionality enabling to establish and maintain the required separation (remain well clear for RPAS).

Maturity Level	Validation

Human Factor Considerations

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Departure   En-route   Arrival

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-operational need	ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B) Relation-technology need	ASUR-B2/1 - Evolution of ADS-B and Mode S

Relation-operational need	ACAS-B2/1 - New collision avoidance system

### ENABLERS

There are currently no enablers.

### DEPLOYMENT APPLICABILITY

Operational conditions:

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Improve situational awareness of the flight crew | Aircraft operator |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Reduce ATCO workload (en- route) | ++ | KPI06: En-route airspace capacity |
| Capacity | Capacity, throughput & utilization | Resolve ATC staffing, workload, frequency congestion problems | ++ | KPI10: Airport peak throughput |
| Safety |  | Improve separation provision (at a planning horizon > 2 minutes) | ++ | KPI20: Number of aircraft accidents KPI23: Number of airprox/TCAS alert/loss of separation/near midair collisions/midair collisions (MAC) |

GANP PORTAL

# ASBU ELEMENTS

| DAIM | DATS | FICE | FRTO |
| --- | --- | --- | --- |

| Functional Description | Enablers | Deployment Applicability | Performance Impact Assessment |
| --- | --- | --- | --- |

## DAIM

DAIM-B1/1	Provision of quality-assured aeronautical data and information

Information

Sixth edition of the GANP

Main Purpose	The main purpose of this element is to ensure that aeronautical data and information comply with quality standards in order to meet the needs of airspace users and support the safety of flight operations.

New Capabilities	Provision of high quality aeronautical data and information; consistent interpretation and timely exchange of aeronautical data and information; and, automatic management, processing, verification, usage and exchange of aeronautical data and information.

Description		This element ensures that processes, procedures and systems are improved to allow for an enhanced quality of aeronautical information products and services. This element includes:

- Full move into an automated data-centric environment so that the management, processing, verification, usage and exchange can be done in a structured, automatic manner and human intervention is reduced.

- Aeronautical data and information is of high quality if it is aggregated and provided by authoritative sources. This requires to properly control relationships along the whole data chain from the origination to the distribution to the next intended user (formal arrangements with data originators, neighbouring States, data and information service providers and others).

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS 		OPERATIONS

Strategical

Pre-tactical

| Taxi-out | Departure | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- |

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Information exchange

Procedures for the provision of aeronautical information services in an AIM environment

ANSPs should define in an AIM Manual of operations the procedures to be followed for the provision of quality assured aeronautical information. Reference ICAO documents: Annex 15 - Aeronautical Information Services PANS – Aeronautical Information Management ICAO Doc 8126 - Aeronautical Information Services

ANSP 	2018

Operational procedures

Quality	Procedures for the application of a quality management system to the AIM processes.

ANSPs should define in a Quality Manual the procedures to be followed to apply a quality management system to the AIM processes. Reference ICAO documents: Annex 15 - Aeronautical Information Services PANS – Aeronautical Information Management Doc 9839 - Manual on the Quality Management System for Aeronautical Information Services

ANSP 	2018

Ground system infrastructur e

Information exchange

Automated aeronautical information management systems and infrastructure.

Automated systems and infrastructure to support the collection, processing, distribution and quality control of the aeronautical information products and services.

ANSP 	2018

Training	-	Training requirements for the provision of quality-assured aeronautical data and information

Training for AIS/AIM personnel	 ANSP 	2018

Regulatory provisions

National regulatory framework

National framework for the provision of quality assured aeronautical data and information

Development of State Regulatory framework to support the provision of quality-assured aeronautical data and information. Reference ICAO documents: Annex 15 - Aeronautical Information Services PANS – Aeronautical Information Management

CAA 	2018

Regulatory

provisions

Formal

Formal

arrangements arrangements for

data quality assurance

Formal arrangements between ANSP and	ANSP	CAA	2018

the aeronautical data originators to ensure data quality. Reference ICAO documents: Aeronautical Data Catalogue (PANS – Aeronautical Information Management, Appendix 1)

DAIM-B1/2	Provision of digital Aeronautical Information

Publication (AIP) data sets

Information

Sixth edition of the GANP

Main Purpose		To make available digital AIP data and information in an interoperable manner and mutually- understood manner.

New Capabilities	Provision of digital AIP data sets. This will facilitate the exchange of aeronautical information that becomes easy to be integrated and filtered, thus increasing cost effectiveness and efficiency.

Description		The need for interoperable exchange of AIP data and information requires providing them in digital form and complying with digital data exchange requirements. This element consists in the replacement of existing sections of the AIP by digital AIP data sets. Therefore, this element supports the migration to a data-centric environment where aeronautical data and information (AIP) will be provided in a structured and digital form through the use of information exchange models (e.g.

AIXM).

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS

| Strategical | Pre-tactical |  | Tactical-Pre ops |  |
| --- | --- | --- | --- | --- |
| Tactical-During ops |  | Post operations |  |  |

DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

### OPERATIONS

| Taxi-out | Departure | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- |

Relation-operational need	DAIM-B1/1 - Provision of quality-assured aeronautical data and information

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Information exchange

Procedures for the provision of digital AIP data set

Updated AIM Manual of operations to include information about the provisions of digital AIP data sets. Reference ICAO documents: Annex 15 - Aeronautical Information Services PANS - Aeronautical Information Management Doc 8126 - Aeronautical Information Services

ANSP 	2018

Information exchange model

Aeronautical Information

Aeronautical Information Exchange Model (AIXM) v 5.1

The objective of the Aeronautical Information Exchange Model (AIXM) is to enable the provision in digital format of the aeronautical information that is in the scope of Aeronautical Information Services (AIS). Reference material: AIXM Confluence site/AIS Manual Volume 4

ANSP 	2018

Ground system infrastructur e

Information exchange

Automated systems and infrastructure to support the provision of digital AIP data sets using AIXM

Automated systems and infrastructure to support the processing and distribution of the digital Aeronautical Information Publication (AIP) data sets.

ANSP 	2018

Ground system infrastructur e

Information exchange

Airspace user systems to be updated to leverage the benefits of digital AIP data sets using AIXM.

Automated systems and infrastructure for the users to make use of the digital Aeronautical Information Publication (AIP) data sets.

Airspace user  2018

Training	-	Training requirements for the provision of digital AIP data sets

Training for AIS/AIM personnel, Airspace users

ANSP

Airspace user

2018

Regulatory provisions

National regulatory framework

National framework for the provision of digital Aeronautical Information Publication (AIP) data sets

Development of State Regulatory framework to support the provision of digital Aeronautical Information Publication (AIP) data sets Reference ICAO documents: Annex 15 - Aeronautical Information Services PANS – Aeronautical Information Management Doc 8126 – Aeronautical Information Services

CAA 	2018

Information

Provision of digital terrain data sets

DAIM-B1/3

Sixth edition of the GANP

Main Purpose 		To make available digital terrain data and information in an interoperable manner and mutually- understood manner.

New Capabilities   Provision of terrain data as digital data sets. This will facilitate the exchange of terrain data that becomes easy to integrate and easily filtered, thus increasing cost effectiveness and efficiency.

Description 		The need for interoperable exchange of terrain data requires providing the data in digital form and complying with digital data exchange requirements. This element consists in the replacement of existing terrain data by digital terrain data sets. Therefore, this element supports the migration to a data-centric environment where terrain data will be provided in a digital form and in a structured way.

Maturity Level 	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? Yes

PLANNING LAYERS

OPERATIONS

Does it imply a change to levels of automation? Yes

| Strategical | Pre-tactical |  | Tactical-Pre ops |  |
| --- | --- | --- | --- | --- |
| Tactical-During ops |  | Post operations |  |  |

| Taxi-out | Departure | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

DAIM-B1/1 - Provision of quality-assured aeronautical data and information

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational Information	Procedures for

procedures	exchange	the provision of

digital terrain data set

Updated AIM Manual of operations to

include information about the provisions of digital terrain data sets. ICAO reference documents : Annex 15 - Aeronautical Information Services PANS- Aeronautical Information Management Doc 8126 - Aeronautical Information Services

ANSP

2018

Ground	Information

system	exchange infrastructur

e

Automated

systems and infrastructure to support the provision of digital terrain data sets using AIXM

Automated systems and infrastructure to

support the processing and distribution of the digital terrain data sets.

ANSP

2018

Ground	Information

system	exchange infrastructur

e

Airspace user

systems to be updated to leverage the benefits of digital terrain data sets using AIXM.

Automated systems and infrastructure for

the users to make use of the digital terrain data sets.

Airspace user

2018

Training

-

Training

requirements for the provision of digital terrain data sets

Training for AIS/AIM personnel, Airspace

users

ANSP 	2018

Airspace user

Regulatory	National

provisions	regulatory

framework

National

framework for the provision of digital terrain data sets

Development of State Regulatory

framework to support the provision of digital terrain data sets Reference ICAO documents: Annex 15 - Aeronautical Information Services PANS – Aeronautical Information Management Doc 8126 – Aeronautical Information Services

CAA

2018

DAIM-B1/4	Provision of digital obstacle data sets	Information

Sixth edition of the GANP

Main Purpose	To make available digital obstacle data in an interoperable and mutually-understood manner.

New Capabilities	Provision of obstacle data as digital data sets. This will facilitate the exchange of obstacle data that becomes easy to integrate and easily filtered, thus increasing cost effectiveness and efficiency.

Description		The need for interoperable exchange of obstacle data requires providing the data in digital form and complying with digital data exchange requirements. This element consists in the replacement of existing obstacle data by digital obstacle data sets. Therefore, this element supports the migration to a data centric environment where obstacle data will be provided in a structured and digital form through the use through the use of information exchange models (e.g. AIXM).

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? Yes

PLANNING LAYERS

OPERATIONS

Does it imply a change to levels of automation? Yes

| Strategical | Pre-tactical |  | Tactical-Pre ops |  |
| --- | --- | --- | --- | --- |
| Tactical-During ops |  | Post operations |  |  |

| Taxi-out | Departure | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

DAIM-B1/1 - Provision of quality-assured aeronautical data and information

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Information exchange model

Aeronautical Information

Aeronautical Information Exchange Model (AIXM) v 5.1

The objective of the Aeronautical Information Exchange Model (AIXM) is to enable the provision in digital format of the aeronautical information that is in the scope of Aeronautical Information Services (AIS). Reference material: AIXM Confluence site/AIS Manual Volume 4

ANSP 	2018

Operational procedures

Information exchange

Procedures for the provision of digital obstacle data set

Updated AIM Manual of operations to include information about the provisions of digital obstacle data sets. Reference ICAO document: Annex 15 - Aeronautical Information Services PANS – Aeronautical Information Management Doc 8126 – Aeronautical Information Services

ANSP 	2018

Ground system infrastructur e

Information exchange

Automated systems and infrastructure to support the provision of digital obstacle data sets using AIXM

Automated systems and infrastructure to support the processing and distribution of the digital obstacle data sets.

ANSP 	2018

Ground	Information

system	exchange infrastructur

e

Airspace user

systems to be updated to leverage the benefits of digital obstacle data sets using AIXM.

Automated systems and infrastructure for

the users to make use of the digital obstacle data sets.

Airspace user

2018

Training

-

Training

requirements for the provision of digital obstacle data sets

Training for AIS/AIM personnel, Airspace

users

ANSP 	2018

Airspace user

Regulatory	National

provisions	regulatory

framework

National

framework for the provision of digital obstacle data sets

Development of State Regulatory

framework to support the provision of digital obstacles data sets Reference ICAO documents: Annex 15 - Aeronautical Information Services PANS – Aeronautical Information Management Doc 8126 – Aeronautical Information Services

CAA

2018

DAIM-B1/5	Provision of digital aerodrome mapping data sets  Information

Sixth edition of the GANP

Main Purpose		To make available digital aerodrome mapping data and information in an interoperable and mutually- understood manner.

New Capabilities	Provision of aerodrome mapping data as digital data sets. This will facilitate the exchange of aerodrome mapping data that becomes easy to integrate and easily filtered, thus increasing cost effectiveness and efficiency.

Description		The need for interoperable exchange of aerodrome mapping data requires providing the data in digital form and complying with digital data exchange requirements. This element consists in the replacement of existing aerodrome mapping data by digital aerodrome mapping data sets.

Therefore, this element supports the migration to a data centric environment where aerodrome mapping data will be provided in a structured and digital form through the use through the use of information exchange models (e.g. AIXM).

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

| Strategical | Pre-tactical |  | Tactical-Pre ops |  |
| --- | --- | --- | --- | --- |
| Tactical-During ops |  | Post operations |  |  |

| Taxi-out | Departure | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

DAIM-B1/1 - Provision of quality-assured aeronautical data and information

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Information exchange model

Aeronautical Information

Aeronautical Information Exchange Model (AIXM) v 5.1

The objective of the Aeronautical Information Exchange Model (AIXM) is to enable the provision in digital format of the aeronautical information that is in the scope of Aeronautical Information Services (AIS). Reference material: AIXM Confluence site/AIS Manual Volume 4

ANSP 	2018

Operational procedures

Information exchange

Procedures for the provision of digital aerodrome mapping data set

Updated AIM Manual of operations to include information about the provisions of digital aerodrome mapping data sets.

Reference ICAO documents: Annex 15 - Aeronautical Information Services PANS – Aeronautical Information Management Doc 8126 – Aeronautical Information Services

ANSP 	2018

Ground system infrastructur e

Information exchange

Automated systems and infrastructure to support the provision of digital aerodrome mapping data sets using AIXM

Automated systems and infrastructure to support the processing and distribution of the digital aerodrome mapping data sets.

ANSP 	2018

PLANNING LAYERS

OPERATIONS

Ground system infrastructur e

Information exchange

Airspace user systems to be updated to leverage the benefits of digital aerodrome mapping data sets using AIXM.

Automated systems and infrastructure for the users to make use of the digital aerodrome mapping data sets.

Airspace user  2018

Training

-

Training

requirements for the provision of digital aerodrome mapping data sets

Training for AIS/AIM personnel, Airspace

users

ANSP 	2018

Airspace user

Regulatory	National

provisions	regulatory

framework

National

framework for the provision of digital aerodrome mapping data sets

Development of State Regulatory

framework to support the provision of digital aerodrome mapping data sets Reference ICAO documents: Annex 15 - Aeronautical Information Services PANS – Aeronautical Information Management Doc 8126 – Aeronautical Information Services

CAA

2019

DAIM-B1/6	Provision of digital instrument flight procedure

data sets

Information

Sixth edition of the GANP

Main Purpose		To make available digital instrument flight procedure data in an interoperable and mutually- understood manner.

New Capabilities	Provision of instrument flight procedure data as digital data sets; and, compliance with the navigation specifications, consistency in design, coding and operation of performance-based navigation (PBN) procedures to avoid differences in the aircraft behaviour in response to the coded path terminators by the use specific criteria for coding instrument flight procedures. This will facilitate the exchange of instrument flight procedure data that becomes easy to integrate and easily filtered, thus increasing cost effectiveness and efficiency.

Description		The need for interoperable exchange of instrument flight procedure data requires providing the data in digital form and complying with digital data exchange requirements. This element consists in the replacement of existing instrument flight procedure data by digital instrument flight procedure data sets. In addition, it includes consistent coding of procedures to match the procedure design intent and ensure more repeatable flight paths. Applying new rules for coding Instrument flight procedures will limit the number of allowable path terminators for PBN procedures in compliance with the PBN Navigation Specifications. Therefore, this element supports the migration to a data centric environment where instrument flight procedure data will be provided in a structured and digital form through the use of information exchange models (e.g. AIXM).

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

| Strategical | Pre-tactical |  | Tactical-Pre ops |  |
| --- | --- | --- | --- | --- |
| Tactical-During ops |  | Post operations |  |  |

OPERATIONS

Arrival

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

DAIM-B1/1 - Provision of quality-assured aeronautical data and information

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Information exchange model

Aeronautical Information

Aeronautical Information Exchange Model (AIXM) v 5.1

The objective of the Aeronautical Information Exchange Model (AIXM) is to enable the provision in digital format of the aeronautical information that is in the scope of Aeronautical Information Services (AIS). Reference material: AIXM Confluence site/AIS Manual Volume 4

ANSP 	2018

Operational procedures

Information exchange

Procedures for the provision of digital instrument flight procedures data set

Updated AIM Manual of operations to include information about the provisions of digital instrument flight procedure data sets. Reference ICAO documents: Annex 15 - Aeronautical Information Services PANS – Aeronautical Information Management Doc 8126 – Aeronautical Information Services

ANSP 	2018

Ground system infrastructur e

Information exchange

Automated systems and infrastructure to support the provision of digital instrument flight procedures data sets using AIXM

Automated systems and infrastructure to support the processing and distribution of the digital instrument flight procedures data sets.

ANSP 	2018

PLANNING LAYERS

Ground system infrastructur e

Information exchange

Airspace user systems to be updated to leverage the benefits of digital instrument flight procedure data sets using AIXM.

Automated systems and infrastructure for the users to make use of the digital instrument flight procedures data sets.

Airspace user  2018

Training

-

Training

requirements for the provision of digital instrument flight procedure data sets

Training for AIS/AIM personnel, Airspace

users

ANSP	2018

Airspace user

Regulatory	National

provisions	regulatory

framework

National

framework for the provision of digital instrument flight procedures sets

Development of State Regulatory

framework to support the provision of digital instrument flight procedures data sets Reference ICAO documents: Annex 15 - Aeronautical Information Services PANS – Aeronautical Information Management Doc 8126 – Aeronautical Information Services

CAA

2018

DAIM-B1/7	NOTAM improvements	Information

Sixth edition of the GANP

Main Purpose	To provide timely and relevant information about status and condition of the ANS infrastructure to the next intended users via NOTAM.

New Capabilities	Identification of clear operational conditions to determine when a NOTAM shall or shall not be originated, thus ensuring that the information provided meets the needs of the users; provision of digital NOTAMs to enhance the quality of the information provided and allow the graphical representation and better filtering of information to assist operators in retrieving the relevant information.

Description		In order to meet the operational needs of the users, it is essential to provide information that is timely and fit for purpose. This can be done by refining the criteria to ensure that the users receive the right information. This element consists in the identification of clear operational conditions under which a NOTAM shall or shall not be originated and replacement of paper NOTAMs by a digital version through the use of information exchange models (e.g. AIXM).

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

| Pre-tactical | Tactical-Pre ops |  | Tactical-During ops |
| --- | --- | --- | --- |
| Post operations |  |  |  |

DEPENDENCIES AND RELATIONS

OPERATIONS

| Taxi-out | Departure | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- |

Type of Dependencies

ASBU Element

Relation-operational need

DAIM-B1/1 - Provision of quality-assured aeronautical data and information

Relation-information need

DAIM-B1/2 - Provision of digital Aeronautical Information Publication (AIP)

data sets

Relation-information need

DAIM-B1/4 - Provision of digital obstacle data sets

Relation-information need

DAIM-B1/5 - Provision of digital aerodrome mapping data sets

Relation-information need

DAIM-B1/6 - Provision of digital instrument flight procedure data sets

Relation-information benefit

SWIM-B2/1 - Information service provision

Relation-information need

SWIM-B2/2 - Information service consumption

ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Information exchange model

Aeronautical Information

Aeronautical Information Exchange Model (AIXM) v 5.1

The objective of the Aeronautical Information Exchange Model (AIXM) is to enable the provision in digital format of the aeronautical information that is in the scope of Aeronautical Information Services (AIS). Reference material: AIXM Confluence site/AIS Manual Volume 4

ANSP 	2018

Operational procedures

Information exchange

Procedures for the provision of an enhanced NOTAM service

Updated AIM Manual of Operations to include information about the provisions of NOTAM. Reference ICAO documents: Annex 15 - Aeronautical Information Services PANS – Aeronautical Information Management Doc 8126 – Aeronautical Information Services

ANSP 	2019

Ground system infrastructur e

Information exchange

Automated systems and infrastructure to support the provision of NOTAM using AIXM.

Automated systems and infrastructure to support the processing and distribution of NOTAM

ANSP 	2019

Ground system infrastructur e

Information exchange

Airspace user systems need to be updated to leverage the benefits of digital NOTAM using AIXM.

Automated systems and infrastructure for the users to make use of NOTAM

Airspace user  2019

Training

-

Training

requirements for the provision of enhanced NOTAM

Training for AIS/AIM personnel, Airspace

users

ANSP	2019

Airspace user

Regulatory	National

provisions	regulatory

framework

National

framework for the provision of NOTAM

Development of State Regulatory

framework to support the provision of NOTAM Reference ICAO documents: Annex 15 - Aeronautical Information Services PANS – Aeronautical Information Management Doc 8126 – Aeronautical Information Services

CAA

2018

DAIM-B2/1	Dissemination of aeronautical information in a

SWIM environment

Information

Sixth edition of the GANP

Main Purpose	Integrated aeronautical information service in a SWIM environment in support of enhanced operational ground and air decision-making processes for all phases of flight.

New Capabilities	Implementation of a data-centric AIM, integrated into the System Wide Information Management (SWIM) environment; user-defined products derived from aeronautical information in a standardized Information Exchange Model (e.g. AIXM) that can be provided alone or integrated with other information domains (e.g. Meteorological or FF-ICE information); wider use of secure web services and starts the decommissioning of current distribution mechanism; commencement of the use of business-to-business services that allows integration of aeronautical information into ATM systems.

Description	This element represents the full integration of aeronautical information into the SWIM environment.

The use of AIM SWIM services will allow the user to access relevant and mutually understood aeronautical information in an interoperable manner. This will include the ability not only to communicate and exchange aeronautical information but also to interpret it in a meaningful manner.

AIM-SWIM information services will support request/reply or publish/subscribe access mechanisms and will provide quality & timely information to users in a range of formats to best enable their optimal decision making. AIM-SWIM information services will also include web-services supporting the graphical representation of aeronautical information in a geo-referenced environment.

Maturity Level	Validation

Human Factor Considerations

PLANNING LAYERS

| Pre-tactical | Tactical-Pre ops |  | Tactical-During ops |
| --- | --- | --- | --- |
| Post operations |  |  |  |

DEPENDENCIES AND RELATIONS

OPERATIONS

| Taxi-out | Departure | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- |

Type of Dependencies	ASBU Element

Evolution	DAIM-B1/2 - Provision of digital Aeronautical Information Publication (AIP) data sets

Evolution	DAIM-B1/3 - Provision of digital terrain data sets

Evolution	DAIM-B1/4 - Provision of digital obstacle data sets

Evolution	DAIM-B1/5 - Provision of digital aerodrome mapping data sets

Evolution	DAIM-B1/6 - Provision of digital instrument flight procedure data sets Relation-information need	DAIM-B2/3 - Aeronautical information to support higher airspace operations Relation-information need	DAIM-B2/4 - Aeronautical information requirements tailored to UTM Relation-operational need	SWIM-B2/1 - Information service provision

Relation-operational need	SWIM-B2/2 - Information service consumption Relation-operational need	SWIM-B2/3 - SWIM registry

Relation-operational need	DAIM-B2/2 - Daily Airspace Management information to support flight and

flow

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Information exchange model

Aeronautical Information

Aeronautical Information Exchange Model (AIXM) v 5.1

The objective of the Aeronautical Information Exchange Model (AIXM) is to enable the provision in digital format of the aeronautical information that is in the scope of Aeronautical Information Services (AIS). Reference material: AIXM Confluence site/AIS Manual Volume 4

ANSP 	2018

Ground system infrastructur e

Information exchange

Automated systems and infrastructure to support the provision of aeronautical information over SWIM

Automated systems and infrastructure for the dissemination of aeronautical information in a SWIM environment.

ANSP 	2025

Ground system infrastructur e

Information exchange

Airspace user systems to be updated to subscribe or access the aeronautical information services over SWIM

Automated systems and infrastructure for the users to subscribe or access the aeronautical information in a SWIM environment.

Airspace user  2025

Training

-

Training

requirements for the provision of aeronautical information over SWIM

Training for AIS/AIM personnel

ANSP

2025

Information	Aeronautical	Aeronautical

exchange	Information	Information model		Exchange Model

(AIXM) v 5.x

The objective of the Aeronautical

Information Exchange Model (AIXM) is to enable the provision in digital format of the aeronautical information that is in the scope of Aeronautical Information Services (AIS). Reference material: AIXM Confluence site/AIS Manual Volume 4

ANSP

2025

DAIM-B2/2	Daily Airspace Management information to support flight and flow

Information

Sixth edition of the GANP

Main Purpose		To establish common practices and data formats for daily airspace management initiatives continuously updated as events take place for flight and flow planning and execution.

New Capabilities		Daily Airspace Management operational changes are packaged and made accessible for use in all local and regional flight and flow operations.

Description		Airspace Management, in daily operations, will continuously adjust airspace status, adjust airspace volumes (e.g. advanced flexible use of airspace) and add temporary airspace initiatives. This element mirrors the modernization efforts ongoing for scheduled AIP modifications with global best practices for packaging and making this information available for dissemination to improve local and regional flight planning in keeping with the flight and flow initiatives engendered in Block 2 FICE and NOPS. This best practice DAIM service enable by SWIM will ensure that information regarding status airspace configurations (Fixes, FIR Boundaries, static zones etc…) and information regarding airspace evolution (reroutings, sector configurations, airspace use plan and updated airspace use plan, airspace reservations, route restrictions and availability, dynamic zones etc.) will be available in formats that support NOPS and FICE automation.

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS 		OPERATIONS

| Pre-tactical | Tactical-Pre ops |  | Tactical-During ops |
| --- | --- | --- | --- |
| Post operations |  |  |  |

Arrival

En-route

Departure

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-operational need	DAIM-B2/1 - Dissemination of aeronautical information in a SWIM

environment

Relation-information need	FRTO-B0/1 - Direct routing (DCT)

Relation-information need	FRTO-B0/2 - Airspace planning and Flexible Use of Airspace (FUA)

Relation-information need	FRTO-B0/3 - Pre-validated and coordinated ATS routes to support flight and

flow

Relation-information need	FRTO-B1/1 - Free Route Airspace (FRA)

Relation-information need	FRTO-B1/2 - Required Navigation Performance (RNP) routes

Relation-information need	FRTO-B1/3 - Advanced Flexible Use of Airspace (FUA) and management

of real time airspace data

Relation-information need	FRTO-B1/4 - Dynamic sectorization

Relation-information need	FRTO-B2/1 - Local components of integrated ATFM and ATC Planning

function (INAP)

Relation-information need	FRTO-B2/2 - Local components of Dynamic Airspace Configurations (DAC) Relation-information need	FRTO-B2/3 - Large Scale Cross Border Free Route Airspace (FRA)

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational Information	Operational

procedures	exchange	procedures for

aeronautical information requirements to support network operations

New procedures will be required to

produce and distribute the new aeronautical information in support of network operations. Updated AIM Manual of operations to address the requirements for aeronautical information to support network operations. Reference ICAO documents: Annex 15 - Aeronautical Information Services PANS- Aeronautical Information Management Doc 8126 - Aeronautical Information Services

2025

ATM network function

Ground	Information

system	exchange infrastructur

e

Automated

systems and infrastructure to include aeronautical information requirements to support network operations

Update to the national aeronautical data

management system, including systems supporting data collection, verification and distribution. For instance, automated functions should be introduced to generate the airspace usage plan information service.

2025

Training

-

Training

requirements for aeronautical information requirements regarding network operations

For personnel managing the ATM

information and for their users if the interfaces and access conditions change.

2025

Information	Aeronautical	Aeronautical

exchange	Information	Information model		Exchange Model

(AIXM) v 5.x

The objective of the Aeronautical

Information Exchange Model (AIXM) is to enable the provision in digital format of the aeronautical information that is in the scope of Aeronautical Information Services (AIS). Reference material: AIXM Confluence site/AIS Manual Volume 4

ANSP

2025

ATM network function

ATM network function

Sixth edition of the GANP

Main Purpose

To provide higher airspace management information in an airspace service that is complementary to

standard AIS.

New Capabilities

Higher airspace definitions and status are packaged and made accessible to operators flying in the

airspace.

Description

Long endurance and near space tourism operators’ space crafts are subject to all applicable AIS. In

addition, there may be a need to manage operations in the airspace by changing the status of volumes of airspace solely related to these operators. Since this information is applicable only to these operators, there is limited value in including this as information in ATM airspace management. A complementary, separate service is desired.

Maturity Level

Validation

Information

Aeronautical information to support higher

airspace operations

DAIM-B2/3

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

PLANNING LAYERS

Does it imply a change to levels of automation? No

| Strategical | Pre-tactical |  | Tactical-Pre ops |
| --- | --- | --- | --- |
| Tactical-During ops |  | Post operations |  |

OPERATIONS

En-route

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

DAIM-B1/2 - Provision of digital Aeronautical Information Publication (AIP)

data sets

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Information exchange

Operational procedures to support higher airspace operations

Rules to describe higher airspace information. Updated AIM Manual of operations to include information about the requirements for aeronautical information in higher airspace. Standards and guidance material: ICAO Annex 15 - Aeronautical Information Services PANS- AIM ICAO Doc 7383 - Aeronautical Information Services Provided by States

ANSP 	2025

Ground system infrastructur

Information exchange

Automated systems and infrastructure to

Update to the national aeronautical data management system, including data collection, verification and distribution.

ANSP

2025

ATM network function

| e | include aeronautical information requirements to support higher airspace operations |  |  |  |
| --- | --- | --- | --- | --- |
| Training | - Training requirements for aeronautical information requirements regarding higher airspace operations. | Training for AIS/AIM personnel | ANSP | 2025 |

Information	Aeronautical	Aeronautical

exchange	Information	Information model		Exchange Model

(AIXM) v 5.x

The objective of the Aeronautical

Information Exchange Model (AIXM) is to enable the provision in digital format of the aeronautical information that is in the scope of Aeronautical Information Services (AIS). Reference material: AIXM Confluence site/AIS Manual Volume 4

ANSP

2025

DAIM-B2/4	Aeronautical information requirements tailored to UTM

Information

Sixth edition of the GANP

Main Purpose		To provide low altitude airspace management information in a UAS Traffic Management (UTM) service that is complementary to standard AIS.

New Capabilities	UTM airspace definitions and status are packaged and made accessible to UAS operators flying in UTM.

Description	UAS flying in UTM are subject to all applicable AIS.

In addition, UTM will manage UAS in the airspace by changing the status of volumes of airspace solely related to UAS operations. For example, the publishing of maps where UAS may operate near airports in class B, or locations where state operations are occurring for which UAS need to stay clear.

Given the manner in which UAS operations occur it is likely that the operator must be in continuous contact with the network to access this information.

Since the volume of such airspace management information would overwhelm ATM airspace management, a complementary, separate service is desired.

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

### PLANNING LAYERS

| Pre-tactical | Tactical-Pre ops |  | Tactical-During ops |
| --- | --- | --- | --- |
| Post operations |  |  |  |

DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

### OPERATIONS

| Taxi-out | Departure | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- |

Relation-operational need	DAIM-B1/2 - Provision of digital Aeronautical Information Publication (AIP)

data sets

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Information exchange

Operational procedures for aeronautical information requirements to support UTM

New procedures will be required to produce and distribute the AIP data for UTM users. Updated AIM Manual of operations to address the requirements for aeronautical information in low-altitude. Standards and guidance material: ICAO Annex 15 - Aeronautical Information Services PANS-AIM ICAO Doc 7383 - Aeronautical Information Services Provided by States

ANSP 	2025

Ground system infrastructur e

Information exchange

Automated systems and infrastructure to include aeronautical information requirements to support UTM

Automated systems and infrastructure to include aeronautical information requirements to support provision of digital AIM data for UTM users.

ANSP 	2025

Airborne system capability

Information exchange

UAS logic to make use of the AIM information

Automated systems and infrastructure for UAS to make use of the AIM information.

Airspace user  2025

Training	-	Training requirements for aeronautical information requirements regarding UTM

Training for AIS/AIM personnel	 ANSP 	2025

Information exchange model

Aeronautical Information

Aeronautical Information Exchange Model (AIXM) v 5.x

The objective of the Aeronautical Information Exchange Model (AIXM) is to enable the provision in digital format of the aeronautical information that is in the scope of Aeronautical Information Services (AIS). Reference material: AIXM Confluence site/AIS Manual Volume 4

ANSP 	2025

Sixth edition of the GANP

Information

NOTAM replacement

DAIM-B2/5

Main Purpose 		To introduce a more efficient mechanism to exchange aeronautical information that is currently provided by the NOTAM system.

New Capabilities  Airspace user will be allowed to tailor the AIM updates to their operational needs as well as subscribe to airspace constraint alerts so that changes to any constraint are immediately available.

Description 		This element consists in the establishment of an information service through SWIM that serves as a replacement for the information currently provided by NOTAMs. The replacement of the current NOTAM system by this information service is expected to solve identified deficiencies such as NOTAM proliferation or the operational irrelevance of the information provided.

Maturity Level 	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

PLANNING LAYERS

OPERATIONS

Does it imply a change to levels of automation? Yes

| Pre-tactical | Tactical-Pre ops |  | Tactical-During ops |
| --- | --- | --- | --- |
| Post operations |  |  |  |

| Taxi-out | Departure |  | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- | --- |
| Turn-around |  |  |  |  |  |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

DAIM-B1/7 - NOTAM improvements

Relation-operational need

SWIM-B2/1 - Information service provision

Relation-operational need

SWIM-B2/2 - Information service consumption

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational Information	Operational

procedures	exchange	procedures to

produce and distribute the new "NOTAM"

Updated AIM Manual of operations to

include information about the new "NOTAM". Reference ICAO documents: Annex 15 - Aeronautical Information Services PANS – Aeronautical Information Management Doc 8126 – Aeronautical Information Services

ANSP

2025

Ground	Information

system	exchange infrastructur

e

Automated

systems and infrastructure to support the provision new "NOTAM"

Automated systems and infrastructure to

support the provision of dynamic AIM updates including data collection, verification and distribution.

ANSP

2025

Ground	Information

system	exchange infrastructur

e

Airspace user

systems need to be updated to take advantage of the new “NOTAM”

Automated systems and infrastructure for

the users to make use of the new “NOTAM”.

Airspace user

2025

Training

-

Training

requirements for the provision of new "NOTAM"

Training for AIS/AIM personnel

ANSP

2025

Regulatory	National

provisions	regulatory

framework

National

framework for the provision of the new “NOTAM”

Development of State Regulatory

framework to support the provision of the new “NOTAM” Reference ICAO documents: Annex 15 - Aeronautical Information Services PANS – Aeronautical Information Management Doc 8126 – Aeronautical Information Services

CAA

2025

## DATS

DATS-B1/1	Remotely Operated Aerodrome Air Traffic

Services

Operational

Sixth edition of the GANP

Main Purpose	To provide ATS at aerodromes not from a traditional on-site tower, but remotely from either a local or a distant location. The service provided may be a control service or flight information service as appropriate.

New Capabilities	Provision of an aerodrome ATS from a remote location using digital video or surveillance technologies, or non-surveillance procedures.

Description 			This element represents the provision of Aerodrome Control or Aerodrome Flight Information Services (AFIS) at aerodromes from other than an on-site facility. This could be achieved by utilizing either video surveillance, digital surveillance, procedural processes, or a combination thereof, which is commensurate with the complexities and traffic demands at the aerodrome. A Remote Tower Centre (RTC) will be remotely connected to one or more aerodromes and consist of one or more Controller Working Positions (CWP), dependent on the requirements of the connected aerodrome(s).

Maturity Level 		Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

OPERATIONS

Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

Tactical-Pre ops

| Taxi-out | Departure | Arrival | Taxi-in |
| --- | --- | --- | --- |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology benefit

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-technology benefit

ASUR-B0/2 - Multilateration cooperative surveillance systems (MLAT)

Relation-technology benefit

ASUR-B0/3 - Cooperative Surveillance Radar Downlink of Aircraft

Parameters (SSR-DAPS)

Relation-operational benefit

SURF-B0/1 - Basic ATCO tools to manage traffic during ground operations

Relation-information need

AMET-B1/1 - Meteorological observations information

Relation-information need

AMET-B1/2 - Meteorological forecast and warning information

Relation-information need

AMET-B1/4 - Dissemination of meteorological information

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

National regulatory framework

Amendment of the national regulatory framework for the provision of remote ATS at aerodromes

Depending on existing state regulations, new provisions on remotely operated ATS may be required, based on ICAO Annex 10, 11 and/or PANS-ATM.

CAA 	2019

Operational procedures

Regular operations

Procedures for the provision of remote ATS at aerodromes

Procedures may be required for the operation depending on the capabilities of the implementation.

ANSP 	2019

Operational procedures

Contingency operations

Procedures for the provision of remote ATS at aerodromes in contingency situations.

Contingency procedures in case of full or partial failure of the RTC based on ICAO PANS-ATM.

ANSP 	2019

ANSP

Training	-	Training requirements for the provision of remote ATS at aerodromes

ATCO Training in the RTC operational standards, limitations and procedures. ATSEP training on the RTC Equipment and Datalink Systems.

2019

ANSP

Ground system infrastructur e

Surveillance  Surveillance

means at the remote tower centre

At the remote facility: Visual reproduction of the out-of-the window view on data/monitor screens, projectors or similar technical solutions or Procedural Systems. References:

2019

Ground system infrastructur e

Surveillance	Surveillance

ANSP

means at the aerodrome where remote ATS are provided

At the aerodrome where ATS are provided: visual surveillance cameras, surface movement radar, surveillance radar, multilateration or other positioning and surveillance implementations as required.

2019

Ground system infrastructur e

Communicati ons

Communication means between the remote tower centre and the aerodrome where remote ATS are provided

Suitable communication and data transfer capabilities between the airports and the RTC including suitable redundancies for technical infrastructure at the aerodrome, remote facility and data links.

2019

Regulatory provisions

Operational Approval

Operational approval required for remote tower centre

Operational Approval required for Remote Tower Facility and associated equipment and system components linking the RTC to the aerodrome.

2019

ANSP

Regulatory provisions

Certification	ATS Unit

Certification to include level of service to be provided by the RTC.

ATS Unit Certification to include level of service to be provided by the RTC.

2019

ANSP

ANSP

DEPLOYMENT APPLICABILITY

Operational conditions:

Provide ATS for an aerodrome which due to its location has limited support living facilities for staff and results in increased overall operational costs to build and maintain a conventional on-site tower. Provide a contingency ATC Tower Facility from a remote tower in the event the regular on-site tower is unavailable. A remote facility may be sited at a location which is ideally located to provide better living conditions for operational staff. Providing a remote facility may enable substantial cost savings in construction. The deployment may enable provision of ATS at aerodromes where it would otherwise be uneconomical or unsustainable. Provide an ATC Tower Facility from a remote tower located on the aerodrome or within close proximity to the aerodrome as the main facility which may enable a more cost effective and efficient service compared to that of a conventional tower.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| Type Operational description Benefitting stakeholder(s) |  |
| --- | --- |
| Direct benefits | Enabling options for an ATS Facility to be constructed at a cost efficient, easily ANSP accessible location particularly when the aerodrome is situated in a remote region. |
|  | Facility construction, equipage and maintenance cost benefits. Minimise cost to Aerodromes for provision of ATS ANSP Services at a low traffic density aerodromes Staffing benefits. |
|  | Service Improvement through use of digital cameras and systems to provide an improved level of surveillance as compared to a conventional tower with basically only ANSP direct out of the window surveillance and possible limitations to viewing all relevant parts of an aerodrome or its associated circuit or airspace. |
|  | Minimise cost to Aerodromes for provision of ATS Services at low traffic density ANSP aerodromes. Reduced ATS and Maintenance staff requirements. |
|  | Enabling options to combine multiple ANSP aerodromes ATS from one ATS facility. |
|  | Enabling ATS provision at new locations which were previously uneconomical or Airspace user unsustainable. |
|  | Improve situational awareness of ATCO ANSP |
| Staffing and Human Resources benefits through options for ATS Facility being Indirect benefits located in easily assessable areas with ANSP better access to facilities such as schools, shops and hospitals. |  |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Cost effectiveness |  | Reduce costs in the Air Navigation System | ++ |  |
| Flexibility |  | Improve flexibility of the Air Navigation System | ++ |  |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Maintain or improve safety during surface movement | ++ | KPI20: Number of aircraft accidents |
| Safety |  | Maintain or improve safety on the runway | ++ | KPI20: Number of aircraft accidents |

## FICE

FICE-B0/1	Automated basic inter facility data exchange

(AIDC)

Information

Sixth edition of the GANP

Main Purpose	To improve the efficiency of coordination and tranfer of control between ATS units.

New Capabilities	Replacement of voice communication between ATS units by automatic message exchange.

Description		This element represents a first automation step in the evolution of the coordination and transfer of control between neighbouring ATS units to guarantee that all related and necessary flight information will be available to the other unit as per agreement.

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS 		OPERATIONS

Tactical-Pre ops

Tactical-During ops

Departure

En-route

Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-technology benefit	COMI-B0/7 - ATS Message Handling System (AMHS)

### ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Regulatory

provisions

SMS

Apply SMS

Apply Safety Management System in

accordance with the national requirements and guidance.

ANSP

2013

Ground

system infrastructur e

Flight and

Flow information

HMI and FDPS

Upgrade the ground system to support the  ANSP

composition, exchange and processing of messages.

2013

Operational Flight and flow Procedures for

procedures

Procedures for message composition and  ANSP

exchange. References: PANS-ATM ICAO Doc 4444 - Procedures for Air Navigation Services and regional interface control (ICD) documents.

2013

information

AIDC

Ground

system

Flight and

Flow

Interconnectivity

Connectivity between ATSU systems

through IP, AMHS, etc.

ANSP

2013

infrastructure information

Training

Flight and

Flow information

Training

requirements for AIDC

Training for ATCO and CNS staff

regarding AIDC.

ANSP

2013

Regulatory	National

provisions	regulatory

framework

National

regulatory framework amendment for the use of AIDC

If applicable, CAA may need to amend the  CAA

national regulatory provisions on the use of AIDC. References: PANS-ATM (ICAO Doc 4444) and Regional Interface Control (ICD) documents.

2013

Sixth edition of the GANP

Main Purpose

To allow aircraft operator to obtain constraint feedback while informing the relevant service providers

of their intentions.

New Capabilities	Ability to determine relevant constraints applicable to a flight and feed them back to the operator. In

this way, service providers will benefit from the availability of preliminary flight plans for resource planning (e.g. ATFM).

A service provider that supports Preliminary Flight Plans and provides feedback should also provide a “re-evaluation” capability, i.e. updates to the feedback that reflect changes to constraints after the initial feedback is provided.

Information

Planning Service

FICE-B2/1

Description 		The FF-ICE Planning Service permits an operator to submit a Preliminary Flight plan prior to filing an ATS Flight plan (Filed Flight Plan) for evaluation by the service provider. The flight plan represents the intention of the operator and is used by the service provider for resource planning. Service providers indicate the acceptability of the flight plan to the operator, and provide feedback regarding constraints that will be applicable to the flight. It allows for a collaborative, iterative planning process to optimize the plan and reduce any surprises after a filed flight plan is submitted.

Maturity Level 	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes It is a new method and people have to learn the new method.

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Pre-tactical   Tactical-Pre ops

OPERATIONS

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-benefit	AMET-B2/1 - Meteorological observations information Relation-operational need	SWIM-B2/1 - Information service provision

Relation-operational need	SWIM-B2/2 - Information service consumption Relation-operational need	SWIM-B2/3 - SWIM registry

Relation-information need	AMET-B2/2 - Meteorological forecast and warning information Relation-benefit	AMET-B2/4 - Meteorological information service in SWIM

Relation-information benefit	DAIM-B1/1 - Provision of quality-assured aeronautical data and information

Relation-information need	DAIM-B1/2 - Provision of digital Aeronautical Information Publication (AIP)

data sets

Relation-information need	DAIM-B1/3 - Provision of digital terrain data sets Relation-information need	DAIM-B1/4 - Provision of digital obstacle data sets

Relation-information need	DAIM-B1/5 - Provision of digital aerodrome mapping data sets Relation-information need	DAIM-B1/6 - Provision of digital instrument flight procedure data sets Relation-information need	DAIM-B1/7 - NOTAM improvements

Relation-information need	DAIM-B2/1 - Dissemination of aeronautical information in a SWIM

environment

Relation-information need	DAIM-B2/2 - Daily Airspace Management information to support flight and

flow

Relation-information need

DAIM-B2/5 - NOTAM replacement

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational

Flight and flow Procedures for

Procedures for FF-ICE flight data

2022

ANSP

procedures

information

the provision and use of FF-ICE Planning Service

exchange, preliminary flight plan, flight plan update, flight plan revaluation, flight cancellation and flight data request.

Airspace user

References: PANS-ATM ICAO Doc 4444

ANSP

- Procedures for Air Navigation Services and ICAO Doc 9965 - Manual on Flight and Flow Information for a Collaborative Environment (FF-ICE) 1st Edition

ATM network function

Information

Flight and flow Flight Information

References: PANS-ATM ICAO Doc 4444

2020

exchange model

information

Exchange Model (FIXM) Version 4.2.0

- Procedures for Air Navigation Services and ICAO Doc 9965- FF-ICE Manual 1st Edition.

| Ground Flight and Capability to Upgrade the ground system to process system Flow process FFICE FF-ICE data exchange using FIXM and |  |  |  | ANSP 2024 Airspace user |  |
| --- | --- | --- | --- | --- | --- |
| infrastructure | information | data exchange | including assignment/recognition of GUFI. |  |  |
|  |  |  | References: PANS-ATM ICAO Doc 4444 |  |  |
|  |  |  | - Procedures for Air Navigation Services |  |  |
|  |  |  | and ICAO Doc 9965- FF-ICE Manual 1st |  |  |
|  |  |  | Edition. |  |  |
| Ground | Flight and | Capability to | Upgrade the ground system to use AMET | ANSP | 2024 |
| system | Flow | obtain and use | and DAIM information obtained via SWIM |  |  |
| infrastructur | information | necessary | services to determine constraints |  |  |
| e |  | information for | applicable to a flight. References: ICAO |  |  |
|  |  | FF-ICE Services | Doc 4444 - PANS-ATM, ICAO Doc 9965 - |  |  |
|  |  |  | FF-ICE Manual 1st Edition, Annex 3 - |  |  |
|  |  |  | Meteorological Service for International |  |  |
|  |  |  | Air Navigation, ICAO Doc 10066 - PANS- |  |  |
|  |  |  | AIM, ICAO Doc (xxxxx) - PANS-IM and |  |  |
|  |  |  | Annex 15 - Aeronautical Information |  |  |
|  |  |  | Services. |  |  |
| Training | Flight and | Training | Training for flight data staff, dispatch, | ANSP | 2024 |
|  | Flow information | requirements for FICE Planning | ATFM staff on FICE Planning service. | Airspace user |  |
|  |  | service. |  |  |  |
| Regulatory | SMS | Apply SMS | Apply Safety Management System in | ANSP | 2013 |
| provisions |  |  | accordance with the national requirements |  |  |
|  |  |  | and guidance. |  |  |

Ground

system

Flight and

Flow

infrastructure information

Capability to

provide and use FF-ICE Planning Service

Upgrade the ground system to provide

and use FF-ICE Planning Service (e.g. send and process flight plans and constraints). References: PANS-ATM ICAO Doc 4444 - Procedures for Air Navigation Services and ICAO Doc 9965

- FF-ICE Manual 2nd Edition.

ANSP	2024

Airspace user

Regulatory	National

provisions	regulatory

framework

National

regulatory framework amendment for the provision and use of FF-ICE Planning Service

If applicable, the national regulatory

framework may need to be amended to allow the provision and use of FF-ICE Planning Service. References: PANS- ATM (Doc 4444) and FF-ICE Manual (ICAO Doc 9965, 2nd Edition).

CAA

2022

FICE-B2/2	Filing Service	Information

Sixth edition of the GANP

Main Purpose		To enhance ATS flight plan processing including constraints evaluation and enhanced flight information sharing.

New Capabilities		Ability to accept Filed Flight Plans provided by the aircraft operator in FIXM format and to take advantage of the more advanced content permitted in them, for example:

Climb and descent performance data to allow more accurate and consistent calculation of 4D Trajectories by the service provider.

Operator-calculated 4D-Trajectory to allow understanding by the service provider of the operator expectations.

Description		FF-ICE will permit submission of a filed flight plan using FIXM in lieu of the teletype-format FPL used today. FF-ICE also provides feedback to the operator regarding whether the flight plan was successfully processed, the flight plan status and where necessary any constraints with which the flight plan does not comply.

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes It is a new method and people have to learn the new method.

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS 		OPERATIONS

Pre-tactical

Tactical-Pre ops

DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-operational need	SWIM-B2/1 - Information service provision Relation-operational need	SWIM-B2/2 - Information service consumption Relation-operational need	SWIM-B2/3 - SWIM registry

Relation-operational need	FICE-B2/1 - Planning Service

Relation-benefit	AMET-B2/1 - Meteorological observations information Relation-information need	AMET-B2/2 - Meteorological forecast and warning information Relation-benefit	AMET-B2/4 - Meteorological information service in SWIM

Relation-information benefit	DAIM-B1/1 - Provision of quality-assured aeronautical data and information

Relation-information need	DAIM-B1/2 - Provision of digital Aeronautical Information Publication (AIP)

data sets

Relation-information need	DAIM-B1/3 - Provision of digital terrain data sets Relation-information need	DAIM-B1/4 - Provision of digital obstacle data sets

Relation-information need	DAIM-B1/5 - Provision of digital aerodrome mapping data sets Relation-information need	DAIM-B1/6 - Provision of digital instrument flight procedure data sets Relation-information need	DAIM-B1/7 - NOTAM improvements

Relation-information need	DAIM-B2/1 - Dissemination of aeronautical information in a SWIM

environment

Relation-information need	DAIM-B2/2 - Daily Airspace Management information to support flight and

flow

Relation-information need	DAIM-B2/5 - NOTAM replacement

ENABLERS

Enabler

Category

Enabler Type	Enabler Name

Description / References

Stakeholders	Year

Information	Flight and flow Flight Information

exchange model

References: PANS-ATM ICAO Doc 4444

- Procedures for Air Navigation Services and ICAO Doc 9965- FF-ICE Manual 1st Edition.

2020

information

Exchange Model

(FIXM) Version 4.2.0

Ground

system

Flight and

Flow

Capability to

process FFICE data exchange

Upgrade the ground system to process

FF-ICE data exchange using FIXM and including assignment/recognition of GUFI. References: PANS-ATM ICAO Doc 4444

- Procedures for Air Navigation Services and ICAO Doc 9965- FF-ICE Manual 1st

Edition.

2024

infrastructure information

Airspace user

ANSP

ATM network function

ANSP

| Ground system infrastructur e | Flight and Flow information | Capability to obtain and use necessary information for | Upgrade the ground system to use AMET and DAIM information obtained via SWIM services to determine constraints applicable to a flight. References: ICAO | ANSP | 2024 |
| --- | --- | --- | --- | --- | --- |
|  |  | FF-ICE Services | Doc 4444 - PANS-ATM, ICAO Doc 9965 - |  |  |
|  |  |  | FF-ICE Manual 1st Edition, Annex 3 - |  |  |
|  |  |  | Meteorological Service for International |  |  |
|  |  |  | Air Navigation, ICAO Doc 10066 - PANS- |  |  |
|  |  |  | AIM, ICAO Doc (xxxxx) - PANS-IM and |  |  |
|  |  |  | Annex 15 - Aeronautical Information |  |  |
|  |  |  | Services. |  |  |
| Regulatory | SMS | Apply SMS | Apply Safety Management System in | ANSP | 2013 |
| provisions |  |  | accordance with the national requirements |  |  |
|  |  |  | and guidance. |  |  |
| Operational | Flight and flow | Procedures for | Procedures for FF-ICE flight data | ANSP | 2022 |
| procedures | information | provision and use of FF-ICE | exchange, filed flight plan, flight plan update, flight plan re-evaluation, flight plan | Airspace user |  |
|  |  | Filing Service | cancellation and flight data |  |  |
|  |  |  | request/Reference: PANS-ATM (Doc |  |  |
|  |  |  | 4444) and FF-ICE Manual (Doc 9965, 2nd |  |  |
|  |  |  | Edition). |  |  |
| Training | Flight and | Training | Flight data staff, Dispatch, ATCOs, Flight | ANSP | 2022 |
|  | Flow information | requirements for the filing service | Crew, AIS staff, ATM/ASM staff | Airspace user |  |
| Ground | Flight and | Capability to | Upgrade to the ground system to provide | ANSP | 2024 |
| system infrastructure | Flow information | provide and use FF-ICE Filing | and use FF-ICE Filing Service (e.g. send and process FF-ICE filed flight plans and | Airspace user |  |
|  |  | Service | constraints). Reference: PANS-ATM (Doc |  |  |
|  |  |  | 4444) and FF-ICE Manual (Doc 9965, 2nd |  |  |
|  |  |  | Edition). |  |  |
| Regulatory | National | National | If applicable, CAA may need to amend the | CAA | 2022 |
| provisions | regulatory | regulatory | national regulatory provisions to allow the |  |  |
|  | framework | framework | provision and use of FF-ICE Filing |  |  |
|  |  | amendment for | Service/Reference: PANS-ATM (Doc |  |  |
|  |  | the provision and | 4444) and FF-ICE Manual (Doc 9965, 2nd |  |  |
|  |  | use of FF-ICE | Edition). |  |  |
|  |  | Filing Service |  |  |  |

Sixth edition of the GANP

Main Purpose

To provide the aircraft operator with the ability to obtain feedback on a possible change without

impacting the flight plan currently being used by the service provider.

New Capabilities

Allows an aircraft operator to assess the impact of a potential change to a Filed or Preliminary Flight

Plan before committing to the change.

Information

Trial Service

FICE-B2/3

Description 		The aircraft operator submits a potential change to a Filed or Preliminary Flight Plan, then, the service provider analyses the proposed change and provides the flight plan acceptability and constraints expected to affect the flight. The service provider does not retain any information about trial requests. A trial request does not indicate intent and therefore is not used by the service provider. If the aircraft operator is satisfied with the solution they will submit a Flight Plan update.

Maturity Level 	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes It is a new method and people have to learn the new method.

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Pre-tactical   Tactical-Pre ops

OPERATIONS

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-benefit	FICE-B2/1 - Planning Service Relation-operational need	FICE-B2/2 - Filing Service

Relation-benefit	AMET-B2/1 - Meteorological observations information Relation-information need	AMET-B2/2 - Meteorological forecast and warning information Relation-benefit	AMET-B2/4 - Meteorological information service in SWIM Relation-operational need	SWIM-B2/1 - Information service provision

Relation-operational need	SWIM-B2/2 - Information service consumption Relation-operational need	SWIM-B2/3 - SWIM registry

Relation-information need	DAIM-B1/1 - Provision of quality-assured aeronautical data and information

Relation-information need	DAIM-B1/2 - Provision of digital Aeronautical Information Publication (AIP)

data sets

Relation-information need	DAIM-B1/3 - Provision of digital terrain data sets Relation-information need	DAIM-B1/4 - Provision of digital obstacle data sets

Relation-information need	DAIM-B1/5 - Provision of digital aerodrome mapping data sets Relation-information need	DAIM-B1/6 - Provision of digital instrument flight procedure data sets Relation-information need	DAIM-B1/7 - NOTAM improvements

Relation-information need	DAIM-B2/1 - Dissemination of aeronautical information in a SWIM

environment

Relation-information need

DAIM-B2/2 - Daily Airspace Management information to support flight and

flow

Relation-information need

DAIM-B2/5 - NOTAM replacement

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Information

Flight and flow Flight Information

References: PANS-ATM ICAO Doc 4444

ANSP

2020

exchange model

information

Exchange Model (FIXM) Version 4.2.0

- Procedures for Air Navigation Services and ICAO Doc 9965- FF-ICE Manual 1st Edition.

ATM network function

Ground system

Flight and Flow

Capability to process FFICE

Upgrade the ground system to process FF-ICE data exchange using FIXM and

2024

ANSP

infrastructure information

data exchange

including assignment/recognition of GUFI. References: PANS-ATM ICAO Doc 4444

Airspace user

- Procedures for Air Navigation Services and ICAO Doc 9965- FF-ICE Manual 1st Edition.

Ground system infrastructur e

Flight and Flow information

Capability to obtain and use necessary information for FF-ICE Services

Upgrade the ground system to use AMET and DAIM information obtained via SWIM services to determine constraints applicable to a flight. References: ICAO Doc 4444 - PANS-ATM, ICAO Doc 9965 -

FF-ICE Manual 1st Edition, Annex 3 - Meteorological Service for International Air Navigation, ICAO Doc 10066 - PANS- AIM, ICAO Doc (xxxxx) - PANS-IM and Annex 15 - Aeronautical Information Services.

ANSP 	2024

Regulatory provisions

SMS	Apply SMS	Apply Safety Management System in

accordance with the national requirements and guidance.

ANSP 	2013

Operational

Flight and flow Procedures for

Procedures for submission and

ANSP

2022

procedures

information

provision and use of FF-ICE Trial Service

processing of trial requests/Reference: PANS-ATM (Doc 4444) and FF-ICE

Manual (Doc 9965, 2nd Edition).

Airspace user

Ground system

Flight and Flow

Capability to provide and use

Upgrade to the ground system to provide and use FF-ICE Trial Service (send and

ANSP

Airspace user

2024

infrastructure information

FF-ICE Trial Service

process trial requests and constraints). Reference: PANS-ATM (Doc 4444) and FF-ICE Manual (Doc 9965, 2nd Edition).

Training	Flight and Flow information

Training requirements for trial service

Training for flight data staff, Dispatch, ATCOs, Flight Crew, AIS staff, ATM/ASM staff related to the trial service.

ANSP

Airspace user

2024

Regulatory	National

provisions	regulatory

framework

National

regulatory framework amendment for the provision and use of FF-ICE Trial Service

If applicable, CAA may need to amend the  CAA

national regulatory provisions to allow the provision and use of FF-ICE Trial Service. Reference PANS-ATM (Doc 4444) and (Doc 9965, 2nd Edition).

2022

FICE-B2/4	Flight Data Request Service	Information

Sixth edition of the GANP

Main Purpose		To make available a query and reply service allowing an operator or authorized stakeholders to query the service providers for information on one of its flights - allows an operator to verify the status of a flight previously submitted.

New Capabilities		To allow a service provider or operator to request a Filed or Preliminary Flight Plan, supplementary information (SAR data), or Flight status information. In addition, a service provider will be able to request an aircraft operator to resend a previously submitted flight plan.

Description		PANS-ATM Chapter 11 describes the use of the RQP and RQS messages for an ATS Unit to request a flight plan or specific information about a flight. This element will permit a service provider to make available a query and reply service allowing an operator to query information about its flights, for example a flight plan previously submitted, search and rescue data, or status of a flight.

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes It is a new method and people have to learn the new method.

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS 		OPERATIONS

Pre-tactical

Tactical-Pre ops

Tactical-During ops

DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-operational need	SWIM-B2/3 - SWIM registry

Relation-operational need	SWIM-B2/1 - Information service provision Relation-operational need	SWIM-B2/2 - Information service consumption Relation-operational need	FICE-B2/2 - Filing Service

Relation-operational benefit	SWIM-B2/4 - Air/Ground SWIM for non-safety critical information

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Information

Flight and flow Flight Information

References: PANS-ATM ICAO Doc 4444

ANSP

2020

exchange model

information

Exchange Model (FIXM) Version 4.2.0

- Procedures for Air Navigation Services and ICAO Doc 9965- FF-ICE Manual 1st Edition.

ATM network function

Ground system

Flight and Flow

Capability to process FFICE

Upgrade the ground system to process FF-ICE data exchange using FIXM and

2024

ANSP

infrastructure information

data exchange

including assignment/recognition of GUFI. References: PANS-ATM ICAO Doc 4444

Airspace user

- Procedures for Air Navigation Services and ICAO Doc 9965- FF-ICE Manual 1st Edition.

Regulatory provisions

SMS	Apply SMS	Apply Safety Management System in

accordance with the national requirements and guidance.

ANSP 	2013

Operational

Flight and flow Procedures for

Procedures for submission and

ANSP

2022

procedures

information

FF-ICE Flight Data Request Service

processing of flight data requests. Reference: PANS-ATM (Doc 4444) and FF-ICE Manual (Doc 9965, 2nd Edition).

Airspace user

Ground system

Flight and Flow

Capability to provide and use

Upgrade to the ground system to provide and use FF-ICE Flight Data Request

ANSP

Airspace user

2024

infrastructure information

FF-ICE Flight Data Request Service

Service (query flight information and provide flight plan in response).

Reference: PANS-ATM (Doc 4444) and FF-ICE Manual (Doc 9965, 2nd Edition).

Training	Flight and Flow information

Training requirements for flight data request service

Training for flight data staff and dispatch regarding flight data request service.

ANSP

Airspace user

2024

Regulatory provisions

National regulatory framework

National regulatory framework amendment for the provision and use of FF-ICE Flight Data Request Service

If applicable, CAA may need to amend the national regulatory provisions to allow the provision and use of FF-ICE Flight Data Request Service. Reference PANS-ATM (Doc 4444) and (Doc 9965, 2nd Edition).

CAA 	2022

Information

Notification Service

FICE-B2/5

Sixth edition of the GANP

Main Purpose 	To allow a service provider or operator to notify other parties of the departure or arrival of a flight.

New Capabilities   Allow the complete transition of the existing ATS messages to FIXM.

Description 		The service provider or operator sends a message to other parties upon departure or arrival of a flight based on local agreement.

Maturity Level 	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes It is a new method and people have to learn the new method.

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

SWIM-B2/1 - Information service provision

Relation-operational need

SWIM-B2/2 - Information service consumption

Relation-operational need

SWIM-B2/3 - SWIM registry

Relation-operational need

FICE-B2/2 - Filing Service

ENABLERS

Enabler

Category

Enabler Type	Enabler Name

Description / References

Stakeholders	Year

Information

exchange model

Flight and flow Flight Information

information

Exchange Model

(FIXM) Version 4.2.0

References: PANS-ATM ICAO Doc 4444

- Procedures for Air Navigation Services and ICAO Doc 9965- FF-ICE Manual 1st Edition.

ANSP

ATM network function

2020

Ground

system

Flight and

Flow

Capability to

process FFICE data exchange

Upgrade the ground system to process

FF-ICE data exchange using FIXM and including assignment/recognition of GUFI. References: PANS-ATM ICAO Doc 4444

- Procedures for Air Navigation Services and ICAO Doc 9965- FF-ICE Manual 1st

Edition.

2024

infrastructure information

Airspace user

ANSP

Regulatory

provisions

SMS

Apply SMS

Apply Safety Management System in

accordance with the national requirements and guidance.

ANSP

2013

Operational

procedures

Flight and flow Procedures for

information

FF-ICE

Notification Service

Procedures for provision of FF-ICE

Notification Service.

ANSP

2024

Training

Flight and

Flow information

Training

requirements for notification service

Training requirements for flight data staff

and dispatch regarding the notification service.

Airport operator 2024

ANSP

Airspace user

Regulatory	National

provisions	regulatory

framework

National

regulatory framework amendment for the provision and use of FF-ICE Notification Service

If applicable, CAA may need to amend the  CAA

national regulatory provisions to allow the provision and use of FF-ICE Notification Service. Reference PANS-ATM (Doc 4444) and (Doc 9965, 2nd Edition).

2022

Sixth edition of the GANP

Main Purpose

To ensure consistent flight information and data is available to all stakeholders. This information can

be used to improve ATM decision-making processes.

New Capabilities

Allow the sharing of FF-ICE with the ATM community via FIXM.

Description

General publication of information about flights being handled by the ASP to authorized subscribers

via a SWIM information service.

Maturity Level

Validation

Human Factor

Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes It is a new method and people have to learn the new method.

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

OPERATIONS

DEPENDENCIES AND RELATIONS

Information

Publication Service

FICE-B2/6

Tactical-During ops

Tactical-Pre ops

Pre-tactical

Type of Dependencies	ASBU Element

Relation-operational need	FICE-B2/2 - Filing Service

Relation-operational need	SWIM-B2/1 - Information service provision Relation-operational need	SWIM-B2/2 - Information service consumption Relation-operational need	SWIM-B2/3 - SWIM registry

Relation-operational benefit	SWIM-B2/4 - Air/Ground SWIM for non-safety critical information

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Information

Flight and flow Flight Information

References: PANS-ATM ICAO Doc 4444

ANSP

2020

exchange model

information

Exchange Model (FIXM) Version 4.2.0

- Procedures for Air Navigation Services and ICAO Doc 9965- FF-ICE Manual 1st Edition.

ATM network function

Ground system

Flight and Flow

Capability to process FFICE

Upgrade the ground system to process FF-ICE data exchange using FIXM and

2024

ANSP

infrastructure information

data exchange

including assignment/recognition of GUFI. References: PANS-ATM ICAO Doc 4444

Airspace user

- Procedures for Air Navigation Services and ICAO Doc 9965- FF-ICE Manual 1st Edition.

Regulatory provisions

SMS	Apply SMS	Apply Safety Management System in

accordance with the national requirements and guidance.

ANSP 	2013

Operational

Flight and flow Procedures for

Procedures to provide FF-ICE Publication

ANSP 	2022

procedures

information

FF-ICE

Publication Service

Service

Training	Flight and Flow information

Training requirements for publication service

Training requirements for flight data staff and dispatch regarding publication service.

ANSP 	2024

Regulatory provisions

National regulatory framework

National regulatory framework amendment for the provision and use of FF-ICE Publication Service

If applicable, CAA may need to amend the national regulatory provisions to allow the provision and use of Publication Service. Reference PANS-ATM (Doc 4444) and (Doc 9965, 2nd Edition).

CAA 	2022

FICE-B2/7	Flight information management service for higher airspace operations

Information

Sixth edition of the GANP

Main Purpose		Higher airspace operations will have a different multi-national flavour worldwide. The FF-ICE capabilities support a strategic collaborative flight planning environment.

New Capabilities	Trajectory operations based on the flight business/mission needs

High level of accuracy in the strategic planning of flight business/mission intent

Based on community flight planning rules, an operator will generate business trajectory and publically share via SWIM.

Updates of the intended trajectory will be provided if intended trajectory deviates from the agreed trajectory

Description		A joint multi-national capability to support operations at these altitudes provides for strategic separation based on shared intent. Vehicles at these altitudes exhibit the widest range of operational conditions. They share in common the ability to provide long-term precise intent which allows for flight planning that support strategic conflict management. This ability to share long term intent as well as the lower number of participants allows this operational capability to be a shared as opposed to a centralized command and control state or regional based function.

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS 		OPERATIONS

| Strategical | Pre-tactical |  | Tactical-Pre ops |
| --- | --- | --- | --- |
| Tactical-During ops |  |  |  |

Departure

En-route

Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-operational need	SWIM-B2/3 - SWIM registry

Relation-operational need	SWIM-B2/1 - Information service provision Relation-operational need	SWIM-B2/2 - Information service consumption

Relation-information need	DAIM-B2/3 - Aeronautical information to support higher airspace operations

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

SMS	Apply SMS	Apply Safety Management System in

accordance with the national requirements and guidance.

ANSP 	2013

Operational

Flight and flow Procedures for

PANS-ATM ICAO Doc 4444 - Procedures

ANSP 	2018

procedures

information

message composition and exchange

for Air Navigation Services

Ground system infrastructur e

Flight and Flow information

Capability to support the assignment/reco gnition of GUFI

ICAO Doc 4444 - PANS-ATM and ICAO

Doc 9965 - FF-ICE Manual 2nd edition.

ANSP

Airspace user

2024

Ground system infrastructur e

Flight and Flow information

FDP system that accesses AMET and DAIM information via SWIM services.

ICAO Doc 4444 - PANS-ATM, Annex 3 -

Meteorological Service for International Air Navigation, ICAO Doc 10066 - PANS- AIM, ICAO Doc (xxxxx) PANS-IM, and Annex 15 – Aeronautical Information Services.

ANSP

Airspace user

2018

Ground system infrastructur e

Flight and Flow information

FDP System that exposes the flight information to global community via SWIM services.

PANS-ATM ICAO Doc 4444 - Procedures for Air Navigation Services

ANSP 	2018

Ground system

Flight and Flow

Capability to provide and use

Upgrade to the ground system to provide and use FF-ICE Filing Service (e.g. send

ANSP

Airspace user

2024

infrastructure information

FF-ICE Filing Service

and process FF-ICE filed flight plans and constraints). Reference: PANS-ATM (Doc 4444) and FF-ICE Manual (Doc 9965, 2nd Edition).

Training	Flight and Flow information

Training requirements for flight management service for higher airspace operations

Training for flight data staff and dispatch regarding the flight management service of higher airspace operations.

ANSP 	2018

ATM network function

Information

Flight and flow Flight Information

Reference: PANS-ATM (Doc 4444) and

ANSP

2024

exchange model

information

Exchange Model (FIXM) Version x.x.x

FF-ICE Manual (Doc 9965).

Ground system infrastructur e

Flight and Flow information

FDP system able to process FIXM Version x.x.x

ICAO Doc 4444 - PANS-ATM and ICAO

Doc 9965 - FF-ICE Manual 2nd Edition.

ANSP 	2024

FICE-B2/8	Flight information management service for low-

altitude operations

Information

Sixth edition of the GANP

Main Purpose		Operators at the lowest altitudes, outside of manned flight terminal operations, have unique shared operating environment to support beyond visual line of sight operations.

New Capabilities	Operators provide intent before operating

UTM flight trajectory requirements and exchanges (Block 1)

A community network of intent sharing is in place to improve safe operations

Description		The large number of operations occurring in what has been traditionally a visual flight regime extends well beyond the capability from a CNS, automation and controllers to manage as a traditional IFR environment. From a flight planning management perspective these operations require the operators to share intent before flying so that they strategically de-conflict, provides for the exchange models to support these operations, and a flight management system that complements this shared environment.

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS 		OPERATIONS

Pre-tactical

Tactical-Pre ops

Tactical-During ops

Departure

En-route

Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-operational need	SWIM-B2/3 - SWIM registry

Relation-operational need	SWIM-B2/1 - Information service provision Relation-operational need	SWIM-B2/2 - Information service consumption

Relation-information need	DAIM-B2/4 - Aeronautical information requirements tailored to UTM

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Information

Flight and flow Flight Information

References: PANS-ATM ICAO Doc 4444

ANSP

2020

exchange model

information

Exchange Model (FIXM) Version 4.2.0

- Procedures for Air Navigation Services and ICAO Doc 9965- FF-ICE Manual 1st Edition.

ATM network function

Regulatory provisions

SMS	Apply SMS	Apply Safety Management System in

accordance with the national requirements and guidance.

2013

ANSP

Operational

Flight and flow Procedures for

PANS-ATM ICAO Doc 4444 - Procedures

2018

ANSP

procedures

information

message composition and exchange

for Air Navigation Services

Airspace user

ANSP

Ground system infrastructur e

Flight and Flow information

Capability to support the assignment/reco gnition of GUFI

ICAO Doc 4444 - PANS-ATM and ICAO

Doc 9965 - FF-ICE Manual 2nd edition.

2024

Ground system infrastructur e

Flight and Flow information

FDP system that accesses AMET and DAIM information via SWIM services.

ICAO Doc 4444 - PANS-ATM, Annex 3 -

ANSP

Meteorological Service for International Air Navigation, ICAO Doc 10066 - PANS- AIM, ICAO Doc (xxxxx) PANS-IM, and Annex 15 – Aeronautical Information Services.

2018

Ground system infrastructur e

Flight and Flow information

FDP System that exposes the flight information to global community via SWIM services.

PANS-ATM ICAO Doc 4444 - Procedures for Air Navigation Services

2018

Training	Flight and Flow information

Training requirements for low-altitude unmanned aircraft operations

Training for flight data staff and dispatch regarding the flight management service low-altitude unmanned aircraft operations.

2018

Ground system infrastructur e

Flight and Flow information

FDP system able to process FIXM Version x.x.x

ICAO Doc 4444 - PANS-ATM and ICAO

ANSP

Doc 9965 - FF-ICE Manual 2nd Edition.

2024

Airspace user

ANSP

ANSP

Sixth edition of the GANP

Main Purpose 			To enable aircraft operators and service providers (ATFM functions) to coordinate the re- optimization of flights based upon changing circumstances. Trajectory changes are limited to those occurring beyond an operationally-appropriate horizon. Service providers (ATFM functions) provide full constraint evaluation on proposed changes.

Flight information management support for inflight Information

re-planning

FICE-B2/9

New Capabilities 	  Collaborative planning via flight information exchange applications available between traffic management and airspace users.

Synchronization process to align trajectories for shared planning.

Information standards and protocols for sharing network operations objectives and information exchanges supporting collaborative in-flight re-planning. Information models allow for consistent, integrated flight, flow and constraint information.

Operator constraints that the ATM service provider can consider when re-planning.

Description 		Globally consistent processes and information exchanges are applied to support collaborative inflight re-planning between the AU and ASPs, integrating applicable RSEQ and NOPS planning processes. Information exchange models (e.g., MET, Aeronautical, Flow and Flight) support the application of consistent methods for evaluation of expected impacts on flows and individual flights as circumstances change. Automated applications employ these methods in support of in-flight re- planning. The flight is cleared to the new flight plan by ATC as appropriate.

Maturity Level 	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Taxi-out   Departure   En-route   Arrival

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-operational need	FICE-B0/1 - Automated basic inter facility data exchange (AIDC) Relation-operational need	FICE-B2/1 - Planning Service

Relation-operational need	FICE-B2/2 - Filing Service Relation-operational need	FICE-B2/3 - Trial Service

Relation-operational need	FICE-B2/4 - Flight Data Request Service Relation-operational need	FICE-B2/5 - Notification Service Relation-operational need	FICE-B2/6 - Publication Service

Relation-benefit	AMET-B2/1 - Meteorological observations information Relation-information need	AMET-B2/2 - Meteorological forecast and warning information Relation-benefit	AMET-B2/4 - Meteorological information service in SWIM Relation-operational need	SWIM-B2/1 - Information service provision

Relation-operational need	SWIM-B2/2 - Information service consumption

Relation-operational need

SWIM-B2/3 - SWIM registry

Relation-information benefit

SWIM-B2/4 - Air/Ground SWIM for non-safety critical information

Relation-information need

DAIM-B2/1 - Dissemination of aeronautical information in a SWIM

environment

Relation-information need

DAIM-B2/2 - Daily Airspace Management information to support flight and

flow

Relation-information need

DAIM-B2/5 - NOTAM replacement

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

SMS	Apply SMS	Apply Safety Management System in

accordance with the national requirements and guidance.

ANSP 	2013

Operational

Flight and flow Procedures to

Procedures for flight plan evaluation,

ANSP 	2025

procedures

information

coordinate re- optimization of flight

submission and clearance delivery while in-flight. Reference: PANS-ATM (Doc 4444) and FF-ICE Manual (Doc 9965, 3rd Edition)

Airborne system capability

Flight and Flow information

Flight deck communication capability

Upgrade the ground system to allow the participation of the flight deck in replanning (e.g., A/G with FOC or A/G SWIM as applicable). References: TBD

Airspace user  2025

Ground system infrastructur e

Flight and Flow information

Automated capability to coordinate re- optimization of flights

Upgrade the ground system to provide applicable constraints and operational acceptability of a proposed change to a flight plan for an active flight. References: TBD

ANSP 	2025

Ground system infrastructur e

Flight and Flow information

Synchronization of trajectory information

Upgrade of the gorund system to enable the synchronization of trajectory information suitable for RSEQ and NOPS planning. References: TBD

ANSP 	2025

Ground system

Flight and Flow

Provision of updated plan to

Upgrade of the ground system to enable the provision of updated flight plan to

ANSP 	2025

infrastructure information

ATCO position

ATCO position for clearance delivery. References: TBD

Training	Flight and Flow information

Training requirements for inflight re- planning

Training for flight data staff, dispatch, ATCOs, flight crew, AIS staff and ATM/ASM staff regarding inflight re- planning.

ANSP

Airspace user

2025

Ground system

Flight and Flow

Enhance flight planning

Upgrade of the ground systems to allow the update and exchange flight plans and

Airspace user  2025

infrastructure information

capability

process constraints received. References: TBD

Information

exchange model

Flight and flow Flight Information	Reference: PANS-ATM (Doc 4444) and

2024

information

Exchange Model

(FIXM) Version x.x.x

FF-ICE Manual (Doc 9965).

ANSP

ATM network function

FICE-B3/1	Flight information management services for

enhanced trajectory operations

Information

Sixth edition of the GANP

Main Purpose	To provide automatic exchanges of up-to-date flight trajectories tailored to the individual flight operation in support of post-departure coordination and trajectory sharing.

New Capabilities	Mechanisms are in place to support exchange and synchronization of individual trajectory intent/projections (FDP/FMS/DST) for use across planning and tactical operations Enhancement of automated coordination between ATS facilities supporting seamless delivery of Agreed trajectory

Support for flight-specific, dynamic constraint and separation application considering operator constraints, preferences and capabilities

Description		Mechanisms are in place to support exchange of intent allowing for advanced applications which can synchronize and share a common trajectory intent agreed to facilitate TBO in execution. The use of synchronized trajectories by automation and for ATC Coordination support a wider range of operational techniques within and across ANSP boundaries. Constraints are applied only where and when necessary and tailored to flight-specific capabilities.

Maturity Level	Concept

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Departure   En-route   Arrival

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-operational need	SWIM-B2/1 - Information service provision Relation-operational need	SWIM-B2/2 - Information service consumption Relation-operational need	SWIM-B2/3 - SWIM registry

Relation-operational need	SWIM-B2/5 - Global SWIM processes

Relation-operational need

SWIM-B3/1 - Air/Ground SWIM for safety critical information

Relation-benefit

AMET-B3/1 - Meteorological observations information

Relation-benefit

AMET-B3/2 - Meteorological forecast and warning information

Relation-benefit

AMET-B3/4 - Meteorological information service in SWIM

Relation-information need

FICE-B2/7 - Flight information management service for higher airspace

operations

Relation-information need

FICE-B2/9 - Flight information management support for inflight re-planning

Relation-information need

DAIM-B2/1 - Dissemination of aeronautical information in a SWIM

environment

Relation-information need

DAIM-B2/2 - Daily Airspace Management information to support flight and

flow

Relation-information need

DAIM-B2/5 - NOTAM replacement

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

SMS	Apply SMS	Apply Safety Management System in

accordance with the national requirements and guidance.

ANSP 	2013

Operational

Flight and flow Procedures for

Procedures for enhanced automated

ANSP 	2031

procedures

information

enhanced automated coordination

coordination between ATS units. References: TBD

Airborne system capability

Flight and Flow information

Provision of aircraft-derived information

Update of the ground systems to allow the provision of aircraft-derived information supporting trajectory synchronization.

References: TBD

Airspace user  2031

Ground system

Flight and Flow

Trajectory synchronization

Update of the ground system to allow trajectory synchronisation across ground

ANSP 	2031

infrastructure information

automation and using aircraft-derived information, including both the sharing and use of trajectory information. References: TBD

Ground system

Flight and Flow

ATCO

awareness and

Update of the ground systems for enhanced HMI supporting ATCO

ANSP 	2031

infrastructure information

flight-specific application

awareness and flight-specific constraint application. References: TBD

Ground system infrastructur e

Flight and Flow information

Trajectory-based automated coordination between ATS facilities.

Update of the ground system to allow trajectory-based automated coordination between ATS units. References: TBD

ANSP 	2031

Sixth edition of the GANP

Main Purpose New Capabilities Description

Maturity Level

Concept

Human Factor

Considerations

PLANNING LAYERS

OPERATIONS

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

ENABLERS

There are currently no enablers.

Integrated flight information management system	Information

for end-to-end global flight planning

FICE-B4/1

Tactical-During ops

Tactical-Pre ops

Pre-tactical

Training

Flight and

Flow information

Training

requirements for enhanced trajectory operations.

Training for flight data staff, ATCOs, flight

crew and ATM/ASM staff regarding enhanced trajectory operations.

ANSP

Airspace user

2031

| Taxi-out | Departure | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- |

Sixth edition of the GANP

Main Purpose New Capabilities Description

Maturity Level

Concept

Human Factor

Considerations

Information

Real-Time Participation of operators in flight

information

FICE-B4/2

| Taxi-out | Departure | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- |

PLANNING LAYERS

OPERATIONS

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

ENABLERS

There are currently no enablers.

Tactical-During ops

Tactical-Pre ops

Pre-tactical

FRTO

FRTO-B0/1	Direct routing (DCT)	Operational

Sixth edition of the GANP

Main Purpose	Direct routings are established with the aim of providing airspace users with additional flight planning route options on a larger scale across FIRs such that overall planned leg distances are reduced in comparison with the fixed route network.

New Capabilities		Direct routings (DCTs) are established at national and regional levels and made available for flight planning (with published conditions of use). DCTs should be considered as an early iteration of the Free Route Airspace (FRA) concept. Direct routing operations allow airspace users to optimize flight and fuel planning.

Description 	DCTs could be implemented in a limited way e.g.:

time constraint (fixed or depending on traffic/availability);  traffic constraint (based on flow and/or level of traffic);

flight level;

lateral constraints;

entry/exit conditions.

The extension of direct routings within and across the FIR boundaries also requires Network and ANSPs ground system upgrades for airspace management and flight data processing.

The following procedures and process might need to be considered:

identify the direct routing airspace volume (lateral and vertical) and applicable time;  direct routings may co-exists with ATS route structure;

identify direct routing entry and exit points;

adapt airspace design and ensure direct routing horizontal and vertical connectivity;  ATFM direct routing procedures;

adapt the LoA with adjacent ATS units;

publish relevant data for direct routing in AIP;

airspace management procedure for the implementation of direct routings;

ATC procedures to cover direct routing co-ordination and transfer of control, trajectory change in direct routing; environment, conflict detection.

The ATM system upgrades of FDP and CWP, if required, are related to:

upgrade of network flight planning and ASM/ATFM system for DCTs;  direct routing clearances;

rerouting capabilities in cases the direct routing traversed the military airspace; differentiation between different traffic type airspaces;

direct route beyond AoR;

calculation of 4D trajectory with AoI;  editing function for 4D trajectories.

Maturity Level 	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

PLANNING LAYERS

Does it imply a change to levels of automation? Yes

| ATM planning | Strategical |  | Pre-tactical |
| --- | --- | --- | --- |
| Tactical-Pre ops |  | Tactical-During ops |  |

OPERATIONS

En-route

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

NOPS-B0/1 - Initial integration of collaborative airspace management with

air traffic flow management

Relation-operational need

FRTO-B0/2 - Airspace planning and Flexible Use of Airspace (FUA)

Relation-operational need

FRTO-B0/4 - Basic conflict detection and conformance monitoring

Relation-information need

FICE-B0/1 - Automated basic inter facility data exchange (AIDC)

### ENABLERS

ATM network function

ANSP

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Design and operations

DCT Procedures	Design and use of operational procedures

Aircraft operator

for direct routes. Reference: EUROCONTROL European Route Network Improvement Plan (ERNIP) - Part 1: European Airspace Design Methodology - Guidelines - Edition December 2018 (https://www.eurocontrol.int/publications/eu ropean-route-network-improvement-plan- ernip-part-1-european-airspace-design)

2013

Ground system infrastructur e

ATC systems ATC system

upgrades for monitoring aids functions

ATC systems to be upgraded for DCT clearances, notification and co-ordination data exchanges and management of relevant airspace data. Reference: EUROCONTROL specification for the on- line Data exchanges (OLDI) https://eurocontrol.int/sites/default/files/pub lication/files/EUROCONTROL%20Specifi cation%20OLDI%204.3.pdf

ANSP 	2013

Ground system infrastructur e

CFSP

systems

Upgrade of AOs Flight Planning Systems for DCTs

AO-CFSPS systems to be upgraded to enable flight planning of DCTs Reference: EUROCONTROL NM Flight Planning Requirements document December 2018 https://www.eurocontrol.int/publications/nm

ATM network function

-flight-planning-requirements-guidelines

2013

Aircraft operator

Aircraft operator

Training	-	Training requirements for direct routing

ATCO Training, AO Training, ATM Network Training Training for DCT Provide training to staff prior to implementation

ANSP

2013

Ground system infrastructur e

ATM systems ATFM system for

FUA

Upgrade ATFM/flight planning systems to support FUA Reference: EUROCONTROL NM Flight Planning Requirements document December 2018 (https://www.eurocontrol.int/publications/n m-flight-planning-requirements-guidelines)

2013

ATM network function

DEPLOYMENT APPLICABILITY

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Flight time & distance | Overcome route selection inefficiencies associated with route network design | ++ | KPI04: Filed flight plan en-route extension |

Operational conditions:

This element will bring benefits in en-route airspace with low, medium and high complexity traffic.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Benefitting stakeholder(s)

Operational description

Type

FRTO-B0/2	Airspace planning and Flexible Use of Airspace (FUA)

Operational

Sixth edition of the GANP

Main Purpose		Establish the Flexible Use of Airspace (FUA) process and improve data exchange between civil and military stakeholders by automation to enable a more efficient use of airspace based on transparency and due regard to national security needs.

New Capabilities		Deployment of automated airspace management (ASM) support systems to manage the airspace reservations based on the airspace user needs and resulting from civil-military collaborative decision-making process and more flexibly according to the airspace user needs.

Description		This element addresses strategic/long term airspace management, pre-tactical planning and tactical operations. Automated ASM support systems improve airspace management processes and flexible airspace planning including time horizon specifications in all flight phases (strategic, pre- tactical and tactical time horizon) by providing mutual visibility on civil and military requirements. They also support flexible airspace planning according to civil and military ANSPs and airspace user requirements, including permit cross border and use of segregated areas operations regardless of national boundaries.

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS 		OPERATIONS

| ATM planning | Strategical |  | Pre-tactical |  |
| --- | --- | --- | --- | --- |
| Tactical-Pre ops |  | Tactical-During ops |  |  |

Arrival

En-route

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

FRTO-B0/1 - Direct routing (DCT)

Relation-operational need

NOPS-B0/1 - Initial integration of collaborative airspace management with

air traffic flow management

### ENABLERS

ICAO

CAA

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

Operational Approval

Operational approval to provide FUA

Follow regulations for regulatory approval Reference : ICAO Circular 330 Civil/Military Coordination European Union Regulation (EC) No 2150/2005 of 23 December 2005 laying down common rules for the flexible use of airspace. https://www.eurocontrol.int/articles/flexible- use-airspace-fua-mandate

2013

Operational procedures

Design and operations

FUA Procedures for Dynamic Airspace Management

Design and use of operational procedures Reference: EUROCONTROL European Route Network Improvement Plan (ERNIP)

- Part 1: European Airspace Design Methodology - Guidelines - Edition December 2018 https://www.eurocontrol.int/publications/eur opean-route-network-improvement-plan- ernip-part-1-european-airspace-design EUROCONTROL European Route Network Improvement Plan (ERNIP) - Part 3: Airspace Management Handbook - Edition December 2018 https://www.eurocontrol.int/sites/default/file s/publication/files/ernip-part-3-asm- handbook-edition-5-v5-5.pdf

ANSP

Aircraft operator

2013

Ground system infrastructur e

ATM systems Tools and

System to support FUA

Tools to be implemented and existing systems upgraded to conduct FUA operations. Reference European Route Network Improvement Plan (ERNIP) - Part 3: Airspace Management Handbook - Annex 12 Edition December 2018 (https://www.eurocontrol.int/sites/default/fil es/publication/files/ernip-part-3-asm- handbook-edition-5-v5-5.pdf) EUROCONTROL Local And sub-Regional Airspace Management support system (https://www.eurocontrol.int/services/local- and-sub-regional-airspace-management- support-system-lara)

ANSP 	2013

ATM network function

Training	-	Training requirements for FUA

Training for FUA Provide training to staff prior to implementation

ANSP

2013

Ground system infrastructur e

ATM systems ATFM system for

Aircraft operator

FUA

Upgrade ATFM/flight planning systems to support FUA Reference: EUROCONTROL NM Flight Planning Requirements document December 2018 (https://www.eurocontrol.int/publications/n m-flight-planning-requirements-guidelines)

2013

DEPLOYMENT APPLICABILITY

Operational conditions:

This element will bring benefits in en-route airspace with low, medium and high complexity traffic.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Ensure that the right airspace is available at the right time for the mission | ANSP ATM network function Aircraft operator |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Access and equity |  | Improve airspace reservation management | ++ |  |
| Efficiency | Flight time & distance | Facilitate direct routing of portions of the flight (if this does not cause network problems) | ++ | KPI05: Actual en- route extension |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Flight time & distance | Overcome route selection inefficiencies associated with route & airspace availability as known at the flight planning stage | ++ | KPI04: Filed flight plan en-route extension |
| Efficiency | Flight time & distance | Reduce need for tactical ATFM rerouting to circumnavigate airspace closed at short notice | ++ | KPI05: Actual en- route extension |
| Efficiency | Flight time & distance | Reduce need to avoid airspace because of lack of confirmation that it will be open | ++ | KPI04: Filed flight plan en-route extension |
| Efficiency | Vertical flight efficiency | Reduce altitude restrictions during climb to avoid Special Use Airspace | ++ | KPI17: Level-off during climb |
| Efficiency | Vertical flight efficiency | Reduce altitude restrictions during cruise to avoid Special Use Airspace | ++ | KPI18: Level capping during cruise |
| Efficiency | Vertical flight efficiency | Reduce altitude restrictions during descent to avoid Special Use Airspace | ++ | KPI19: Level-off during descent |

Sixth edition of the GANP

Main Purpose

A collection of routes that have been pre-validated and coordinated with impacted air route traffic

control centers and airspace users.

New Capabilities	There are three main options for the flexible routings:

Preferred Routes Playbook Routes

Coded Departure Routes (CDR)

Operational

Pre-validated and coordinated ATS routes to

support flight and flow

FRTO-B0/3

Description 		There are many instances when ATC needs to move air traffic away from, or into, a particular area of airspace. When this happens, traffic managers will typically implement reroutes – a common route, or set of routes, that they want aircraft to use in a particular area. These routes are predetermined and applied to the certain sector/airport accordingly. Routes are available through ANSP database and are published for the airspace users.

Preferred routes are the normal, everyday routes that ATC would like operators to file. These routes were developed to increase system efficiency and capacity by having balanced traffic flows among high-density airports, as well as de-conflicting traffic flows where possible. Preferred routes are those that operators will most commonly file.

Playbook routes are a set of standard routes that ATC can utilize to fit a particular set of circumstances, when the preferred routes are not available. These routes were created to allow for rapid implementation as needed.

CDRs are a combination of coded air traffic routings and refined coordination procedures, designed to reduce the amount of information that needs to be exchanged between ATC and flight crews.

Maturity Level 	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

PLANNING LAYERS

Tactical-During ops

Tactical-Pre ops

Pre-tactical

OPERATIONS

Arrival

En-route

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

FRTO-B0/1 - Direct routing (DCT)

Relation-operational need

FRTO-B0/2 - Airspace planning and Flexible Use of Airspace (FUA)

Relation-information need

AMET-B0/1 - Meteorological observations products

Relation-information need

AMET-B0/2 - Meteorological forecast and warning products

Relation-information need

AMET-B0/4 - Dissemination of meteorological products

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational Design and	Procedures on

procedures	operations	flexible routing

tools

Design and operational procedures for

ANSPs and AOs on how to use flexible routing tools.

ANSP

Aircraft operator

2013

Ground system infrastructur e

ATM systems Tools and system

to support Flexible routing

ANSP implemented tools to support preferred, playbook, and CDR routes. Reference: https://www.fly.faa.gov/Products/products.j sp

ANSP 	2013

Ground system infrastructur e

CFSP

systems

Upgrade of AOs Flight Planning Systems for Flexible routings (optional)

AO-CFSPS systems to be upgraded to enable flight planning of flexible routings. Although AOs can just look up these routes on the weblink, they can also integrate these data into their flight planning system and have it in their tool. Reference: https://www.fly.faa.gov/Products/products.j sp

2013

Training	-	Training requirements for flexible routings

Training provided to TMC, ATCO, FOC on  ANSP

Aircraft operator

how to use flexible routing

2013

Aircraft operator

### DEPLOYMENT APPLICABILITY

#### Operational conditions:

This element will bring benefits in en-route airspace with low, medium and high complexity traffic.

Main intended benefits:

Type

Operational description

Benefitting stakeholder(s)

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | Establish/update/publish the catalogue of strategic ATFM measures designed to respond to a variety of possible/typical/recurring events degrading the airspace system (e.g. predefined action plans) | ++ |  |
| Flexibility |  | Improve flexibility of the Air Navigation System | ++ |  |

Sixth edition of the GANP

Operational

Basic conflict detection and conformance

monitoring

FRTO-B0/4

Main Purpose 	Reduction of ATCO’s workload via early and systematic conflict detection and conformance monitoring.

New Capabilities   Deployment of basic conflict detection tools (Medium Term Conflict Detection Tool- MTCD) and conformance monitoring warnings.

Description 		MTCD assists the controller in conflict identification and planning tasks by providing automated early detection of potential conflicts; facilitating identification of flexible routing/conflict free trajectories; identifying aircraft constraining the resolution of a conflict or occupying a flight level requested by another aircraft.

The monitoring aids (MONA) function provides the controller with warnings if aircraft deviate from a clearance or planned trajectories and reminders related to the ATCO instructions to be issued.

MONA might include the flight progress monitoring as well as the lateral, longitudinal, vertical and Cleared Flight Level (CFL) deviations.

Maturity Level 	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   En-route   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

FRTO-B0/1 - Direct routing (DCT)

ENABLERS

Enabler

Category

Enabler Type

Enabler Name	Description / References

Stakeholders	Year

Operational Design and

procedures	operations

ATC Procedures	Design and use of operational procedures  ANSP

for Medium Term Conflict Detection (MTCD) and Monitoring Aids (MONA)

2013

Ground system infrastructur e

ATC systems ATM system

Upgrades for MTCD and MONA functions

ATC systems to be upgraded to ensure conformance monitoring of flights and conflict detection for ATC planning purposes. Reference: EUROCONTROL Monitoring Aids (MONA) specification 3 March 2017. This document provides system requirements for Monitoring Aids (MONA).https://www.eurocontrol.int/standa rds?page= Reference: EUROCONTROL Medium-Term Conflict Detection (MTCD) specification 3 March 2017.This document provides system requirements for Medium-Term Conflict Detection (MTCD).

https://www.eurocontrol.int/standards? page=4 EUROCONTROL Trajectory Prediction Specification Edition 2.0 March 2017

(https://www.eurocontrol.int/publications/tr ajectory-prediction-specification)

ANSP 	2013

DEPLOYMENT APPLICABILITY

Operational conditions:

This element will bring benefits in en-route airspace with low, medium and high complexity traffic.

Main intended benefits:

Training	-	Training requirements for MTCD and MONA

ATCO Training Provide training to staff prior to implementation

ANSP 	2013

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Improve situational awareness of ATCO | ANSP |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Reduce ATCO workload (en- route) | ++ | KPI06: En-route airspace capacity |
| Safety |  | Avoid vertical & lateral navigation errors during flight (cases of non-conformance with clearance) | ++ | KPI20: Number of aircraft accidents |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Improve early detection of conflicting ATC Clearances (CATC) (en-route / departure / approach) | ++ | KPI20: Number of aircraft accidents KPI23: Number of airprox/TCAS alert/loss of separation/near midair collisions/midair collisions (MAC) |
| Safety |  | Improve separation provision (at a planning horizon > 2 minutes) | ++ | KPI20: Number of aircraft accidents KPI23: Number of airprox/TCAS alert/loss of separation/near midair collisions/midair collisions (MAC) |

Sixth edition of the GANP

Main Purpose

The Free Route Airspace (FRA) concept brings significant flight efficiency benefits and a choice of

user preferred routes to airspace users.

As a step to full trajectory-based operations, the FRA concept brings increased flight predictability, reduced uncertainty for the ATM network function, which in turn can lead to potential capacity increases for ATM, which will also benefit the user.

New Capabilities

FRA is a specified volume of airspace within which users may freely plan a route between a defined

entry point and a defined exit point, with the possibility to route via intermediate (published or unpublished) waypoints, without reference to the ATS route network, subject to airspace availability. Within this airspace, flights remain subject to air traffic control.

FRA enables airspace users to fly as close as possible to what they consider the optimal trajectory

without the constraints of a fixed route network structure.

Operational

Free Route Airspace (FRA)

FRTO-B1/1

Description 	FRA implementation can be customized for instance:

laterally and vertically;

during specific periods;

with a set of entry/exit conditions;  with initial system upgrades.

The extension of FRA within and across the FIR boundaries also requires upgrades of the ATM network function system and the ANSPs ground system for airspace management and flight data processing.

The following procedures and process might need to be considered:

FRA airspace volume (lateral and vertical) and applicable time (not necessary H24 7/7);

FRA entry and exit points, arrival transition point and departure transition point, and intermediate points;

adapt airspace design and ensure FRA horizontal and vertical connectivity;  ATFM FRA procedures;

adapt the LoA with adjacent -and military- ATS units;  publish relevant data for FRA in AIP;

charts for FRA operations;

airspace management procedure for the implementation of free routes operation;

ATC procedures to cover free route co-ordination and transfer of control, trajectory change in a free route environment, conflict detection.

The upgrades of ATM systems for flight data processing and controller working postition, if required, are related to:

ATC clearances beyond AoR;

differentiation between different traffic type airspaces;  calculation of 4D trajectory with AoI;

editing function for 4D trajectories;

coordination point management for FRA;  coordination with military agencies;

enhance conflict management and controller HMI functions to support conflict detection and resolution.

Maturity Level 	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

PLANNING LAYERS

Does it imply a change to levels of automation? Yes

| ATM planning | Strategical |  | Pre-tactical |  |
| --- | --- | --- | --- | --- |
| Tactical-Pre ops |  | Tactical-During ops |  |  |

OPERATIONS

Arrival

En-route

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

NOPS-B1/5 - Full integration of airspace management with air traffic flow

management

Relation-operational need

FRTO-B1/4 - Dynamic sectorization

Relation-operational need

FRTO-B1/3 - Advanced Flexible Use of Airspace (FUA) and management

of real time airspace data

Relation-information need

FICE-B0/1 - Automated basic inter facility data exchange (AIDC)

Relation-operational need

FRTO-B1/5 - Enhanced Conflict Detection Tools and Conformance

Monitoring

Relation-information need

DAIM-B2/2 - Daily Airspace Management information to support flight and

flow

Evolution

FRTO-B0/1 - Direct routing (DCT)

### ENABLERS

ATM network function

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Design and operations

Procedures for FRA Airspace Design

Design and use of operational procedures. Reference: EUROCONTROL European Route Network Improvement Plan (ERNIP) - Part 1: European Airspace Design Methodology - Guidelines - Edition December 2018 https://www.eurocontrol.int/publications/eur opean-route-network-improvement-plan- ernip-part-1-european-airspace-design

ANSP

2019

Ground system infrastructur e

ATM systems ATC System

Upgrade for FRA

AATM system upgrades for MTCD and MONA functions. ATC systems to be upgraded to ensure conformance monitoring of flights and conflict detection for ATC planning purposes Reference: EUROCONTROL Monitoring Aids (MONA) specification 3 March 2017. This document provides system requirements for Monitoring Aids (MONA).https://www.eurocontrol.int/standa rds?page= Reference: EUROCONTROL Medium-Term Conflict Detection (MTCD) specification 3 March 2017.This document provides system requirements for Medium-Term Conflict Detection (MTCD).

https://www.eurocontrol.int/standards? page=4 EUROCONTROL Trajectory Prediction Specification Edition 2.0 March 2017

Aircraft operator

https://www.eurocontrol.int/publications/traj ectory-prediction-specification

ANSP 	2019

Ground system infrastructur e

CFSP

systems

Upgrade of AOs flight planning systems for FRA

AO-CFSPS systems to be upgraded to enable flight planning of FRA operations. Reference: EUROCONTROL NM Flight Planning Requirements document December 2018 https://www.eurocontrol.int/publications/nm

ATM network function

-flight-planning-requirements-guidelines

2019

Aircraft operator

Training	-	Training requirements for FRA

ATCO Training: FRA Training Provide training to staff prior to implementation Reference: EUROCONTROL European Free Route Airspace Developments 2016 https://www.eurocontrol.int/sites/default/file s/publication/files/free-route-airspace- brochure-20161216.pdf AO and ATM Network Function Training: FRA Training Provide training to staff prior to implementation Reference: EUROCONTROL European Free Route Airspace Developments 2016 https://www.eurocontrol.int/sites/default/file s/publication/files/free-route-airspace- brochure-20161216.pdf

ANSP

2019

ATM network function

Ground system infrastructur e

ATM systems ATFM system for

FRA

Upgrade ATFM/flight planning systems to support FRA

2019

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Flight time & distance | Overcome route selection inefficiencies associated with route network design | ++ | KPI04: Filed flight plan en-route extension |

DEPLOYMENT APPLICABILITY

Operational conditions:

This element will bring benefits in en-route airspace with low, medium and high complexity traffic.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Benefitting stakeholder(s)

Operational description

Type

FRTO-B1/2	Required Navigation Performance (RNP) routes	Operational

Sixth edition of the GANP

Main Purpose		RNP routes should be deployed within en-route airspace where Free Route Airspace (FRA) is not planned or if FRA is deployed the RNP routes should ensure the connectivity between FRA and TMAs.

The objective is to provide consistent navigation using the most appropriate PBN type, infrastructure and navigation applications.

New Capabilities		Performance-based navigation (PBN) specifications allow aircraft to fly a specific path between two 3D-defined points in space. The new capability refers to the Implementation of PBN/RNP routes within en-route airspace.

Description		With the introduction of a RNP navigation specification, the advantages gained from RNAV will be further enhanced by on-board performance monitoring and alerting and the execution of more predictable aircraft behavior.

Design of optimized routes which may include closely spaced parallel routes, Fixed Radius Transition (FRT) and Tactical Parallel Offset (TPO) functionality in en-route, supported by infrastructure and system improvements to support PBN routes.

The adequate navigation infrastructure is required. GNSS or DME ground infrastructure needs to be optimised to support RNP operations and main reversionary capability in case of GNSS outages.

PBN requires a full digital chain, to critical data quality levels, for aeronautical data provided to the airborne systems. The system improvements for controller support tools which might be required are covered by other FRTO elements (MTCD, monitoring aids) or other threads (Safety Nets).

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

Tactical-Pre ops

Pre-tactical

OPERATIONS

En-route

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

APTA-B0/1 - PBN Approaches (with basic capabilities)

Relation-operational need

APTA-B1/1 - PBN Approaches (with advanced capabilities)

Relation-operational need

SNET-B0/1 - Short Term Conflict Alert (STCA)

### ENABLERS

ICAO

CAA

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

Operational Approval

Operational approval to provide RNP Routes

Provide appropriate terrestrial navigation infrastructure to support RNP operations - ground based stations Reference: ICAO Docs, 9613 (Ed 5 – when it is released), Airspace Design – 9992, Operation Approval – 9997. Annexes 6, 11 and 15 (for the data) and the PANS, 4444, 8168 and the new PANS AIM

2019

Operational procedures

Design and operations

Procedures to design the RNP routes

Design and use of operational procedures Reference: EUROCONTROL European Route Network Improvement Plan (ERNIP)

Aircraft operator

Aircraft manufacturer

- Part 1: European Airspace Design Methodology - Guidelines - Edition December 2018 https://www.eurocontrol.int/publications/eur opean-route-network-improvement-plan- ernip-part-1-european-airspace-design European Airspace Concept Handbook for PBN Implementation Edition 3.0 2013 https://www.eurocontrol.int/sites/default/file s/publication/files/handbook-pbn- implement-2013-ed-3a.pdf

ANSP 	2019

Airborne system capability

Aircraft system

Install appropriate RNP equipment

Equip aircraft eligible for RNP operations as defined in ICAO DOC 9613

2019

Aircraft operator

Ground system infrastructur e

ATC systems Adapt ATC

ground system HMI for RNP routes

Upgrade HMI to provide presentation of PBN equipage to ATC

ANSP 	2019

Training	-	Training requirements for RNP routes

Flight Crew Training: Train flight crews in RNP Provide training to staff prior to implementation ATCO Training: Train ATCOs in RNP Provide training to staff prior to implementation

ANSP

2019

### DEPLOYMENT APPLICABILITY

#### Operational conditions:

The element will bring benefit in an en-route medium to high complexity traffic environment.

Main intended benefits:

Type

Operational description

Benefitting stakeholder(s)

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Overcome capacity limitations attributable to route network design | ++ | KPI06: En-route airspace capacity |
| Capacity | Capacity, throughput & utilization | Take advantage of increased navigation precision (airspace with PBN operations) to implement route networks and airspace structures with smaller lateral and vertical safety buffers | ++ | KPI06: En-route airspace capacity |

Sixth edition of the GANP

Main Purpose 			FUA and airspace mangement (ASM) need to be enhanced with collaborative airspace data sharing between all ATM actors, negotiation procedures, system support and real time ASM data integration.

Operational

Advanced Flexible Use of Airspace (FUA) and

management of real time airspace data

FRTO-B1/3

New Capabilities 	 FUA procedures are enhanced by ASM data sharing between the ATM network function, ASM actors, airspace users and ATC. ASM data regarding the planning and tactical management of airspace reservations are continuously exchanged and integrated in real time between the ATM systems. Continuous exchange of ASM data between civil and military national actors will be enhanced.

Description 		Automated ASM systems to ensure uninterrupted data flow between ATM Network functions and the neighbouring ASM systems from the pre-tactical planning to the real time airspace status.

ASM is enhanced by automated data exchange services during the pre-tactical and tactical execution phases continuously in real time. ASM information is shared between ASM systems

and ATS units/systems, and communicated to the ATM network function in the tactical and execution phases. These data, consisting of pre-notification of activation, notification of activation, de- activation, modification and release are collected, saved and processed. Furthermore, these

data need to be exchanged between ASM stakeholders and made available to other actors and relevant airspace users not involved in ASM processed.

Maturity Level 	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

PLANNING LAYERS

Does it imply a change to levels of automation? Yes

| Pre-tactical | Tactical-Pre ops |  | Tactical-During ops |
| --- | --- | --- | --- |
| Post operations |  |  |  |

OPERATIONS

Arrival

En-route

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

FRTO-B0/2 - Airspace planning and Flexible Use of Airspace (FUA)

Relation-operational need

FRTO-B1/1 - Free Route Airspace (FRA)

Relation-operational need

NOPS-B1/5 - Full integration of airspace management with air traffic flow

management

ENABLERS

Enabler

Category

Enabler Type	Enabler Name

Description / References

Stakeholders	Year

| Regulatory | Operational | Operational | Follow regulations for regulatory approval | CAA | ICAO | 2019 |
| --- | --- | --- | --- | --- | --- | --- |
| provisions | Approval | approval to | Reference : ICAO Circular 330 |  |  |  |
|  |  | provide | Civil/Military Coordination European |  |  |  |
|  |  | Advanced FUA | Union Regulation (EC) No 2150/2005 of |  |  |  |
|  |  |  | 23 December 2005 laying down common |  |  |  |
|  |  |  | rules for the flexible use of airspace. |  |  |  |
|  |  |  | https://www.eurocontrol.int/articles/flexible- |  |  |  |
|  |  |  | use-airspace-fua-mandate |  |  |  |

Aircraft operator

Operational procedures

Design and operations

Procedures for Exchange of Real Time Airspace Data

Design and use of operational procedures. Reference: EUROCONTROL Centralised Advanced Flexible Use of Airspace Support Service Concept of Operations (CONOPS) Edition 2.1 October 2014 EUROCONTROL

ATM network function

ANSP

European Route Network Improvement Plan (ERNIP) - Part 3: Airspace Management Handbook - Edition December 2018 https://www.eurocontrol.int/sites/default/file s/publication/files/ernip-part-3-asm- handbook-edition-5-v5-5.pdf ICAO Guidance for Civil/Military Cooperation Reference: 10088 ICAO Doc Civil Military co-operation

2019

Ground system infrastructur e

ATM systems System

Upgrades for Exchange of Real Time Airspace Data

Upgrade systems for partners to exchange real time data Reference: EUROCONTROL European Route Network Improvement Plan (ERNIP) - Part 3: Airspace Management Handbook - Annex 12 Edition December 2018 https://www.eurocontrol.int/sites/default/file s/publication/files/ernip-part-3-asm- handbook-edition-5-v5-5.pdf

2019

ATM network function

ANSP

Ground system infrastructur e

ATC systems Integration and

Management of ASM real-time Data

Upgrade systems to handle real time data in ATM systems and AU flight planning systems Reference: EUROCONTROL European Route Network Improvement Plan (ERNIP) - Part 3: Airspace Management Handbook - Annex 12 Edition December 2018 https://www.eurocontrol.int/sites/default/file s/publication/files/ernip-part-3-asm- handbook-edition-5-v5-5.pdf EUROCONTROL Local And sub-Regional Airspace Management support system https://www.eurocontrol.int/services/local- and-sub-regional-airspace-management- support-system-lara

ANSP 	2019

DEPLOYMENT APPLICABILITY

Operational conditions:

The element will bring benefit in an en-route medium to high complexity traffic environment.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Training

-

Training

requirements for Advanced FUA

Training for the real time ASM data

exchanges Provide training to staff prior to implementation

2019

Ground

system infrastructur e

CFSP

systems

Upgrade of AOs

flight planning systems for FUA

Upgrade systems to handle real time data

in AU flight planning systems Reference: EUROCONTROL European Route Network Improvement Plan (ERNIP) - Part 3: Airspace Management Handbook - Annex 12 Edition December 2018 https://www.eurocontrol.int/sites/default/file s/publication/files/ernip-part-3-asm- handbook-edition-5-v5-5.pdf

2019

Aircraft operator

ATM network function

ANSP

Aircraft operator

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Improve sitautional awareness of network managers, aircraft operators, ANSPs, flight crew, ATCO and airspace managers. | ANSP ATM network function Aircraft operator |
|  | Ensure that the right airspace is available at the right time for the mission | ANSP ATM network function Aircraft operator |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Access and equity |  | Improve airspace reservation management | ++ |  |
| Efficiency | Flight time & distance | Facilitate direct routing of portions of the flight (if this does not cause network problems) | ++ | KPI05: Actual en- route extension |
| Efficiency | Flight time & distance | Overcome route selection inefficiencies associated with route & airspace availability as known at the flight planning stage | ++ | KPI04: Filed flight plan en-route extension |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Flight time & distance | Reduce need for tactical ATFM rerouting to circumnavigate airspace closed at short notice | ++ | KPI05: Actual en- route extension |
| Efficiency | Flight time & distance | Reduce need to avoid airspace because of lack of confirmation that it will be open | ++ | KPI04: Filed flight plan en-route extension |
| Efficiency | Vertical flight efficiency | Reduce altitude restrictions during climb to avoid Special Use Airspace | ++ | KPI17: Level-off during climb |
| Efficiency | Vertical flight efficiency | Reduce altitude restrictions during cruise to avoid Special Use Airspace | ++ | KPI18: Level capping during cruise |
| Efficiency | Vertical flight efficiency | Reduce altitude restrictions during descent to avoid Special Use Airspace | ++ | KPI19: Level-off during descent |

FRTO-B1/4	Dynamic sectorization	Operational

Sixth edition of the GANP

Main Purpose		Dynamically adapt ATC sectorization to respond to traffic demand without increasing the number of controllers working position in use.

New Capabilities		This tool will provide real-time support to the operations room supervisor to select the most appropriate sector configuration according to the traffic demand and complexity, taking into account predefined sector configuration as well as the change of the ATC sector shapes by adding/removing the elementary sectors.

Description		The sectorization tool enables the dynamic management of a large number of possible sector configurations. Based on the volume of pre-defined ATC sector configurations, the automated system continuously evaluates traffic demand and complexity in the future and proposes optimum sectorization solutions.

This tool supports real-time shaping of the airspace volumes allocated to the physical controller working position by adding/removing elementary sectors in order to respond to the predicted traffic demand and complexity.

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

PLANNING LAYERS

Does it imply a change to levels of automation? Yes

DEPENDENCIES AND RELATIONS

| Strategical | Pre-tactical |  | Tactical-Pre ops |  |
| --- | --- | --- | --- | --- |
| Tactical-During ops |  | Post operations |  |  |

OPERATIONS

Arrival

En-route

Departure

| Type of Dependencies | ASBU Element |  |
| --- | --- | --- |
| Relation-operational need | NOPS-B1/6 - Initial Dynamic Airspace configurations |  |
| Relation-operational need | FRTO-B1/2 - Required Navigation Performance (RNP) routes |  |
| Relation-operational need | NOPS-B1/4 - Dynamic Traffic Complexity Management |  |
| ENABLERS |  |  |
| Enabler Enabler Type | Enabler Name Description / References Stakeholders | Year |
| Category |  |  |
| Operational Design and procedures operations | ATC Operational Design and use of operational procedures ANSP Supervisor Procedures for Dynamic Sectorization | 2019 |
| Ground ATC systems system infrastructur e | ATC System ATC system Upgrade relevant to dynamic ANSP Tools for Support handling of airspace volumes of Dynamic Sectorization | 2019 |
| Training - | Training ATCO/Operational Supervisor Training for ANSP requirements for Dynamic Sectorization Provide training dynamic to staff prior to implementation sectorization | 2019 |

DEPLOYMENT APPLICABILITY

Operational conditions:

The element will bring benefit in an en-route medium to high complexity traffic environment.

Main intended benefits:

Benefitting stakeholder(s)

Operational description

Type

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Improve flexibility of sector configuration management | ++ |  |
| Capacity | Capacity, throughput & utilization | Improve flexibility to modify sector configuration at short notice to cope with traffic pattern variations | ++ |  |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

FRTO-B1/5	Enhanced Conflict Detection Tools and

Conformance Monitoring

Operational

Sixth edition of the GANP

Main Purpose		Enhancements of basic mid-term conflict detection (MTCD)/ monitoring alert (MONA) functions and thus further improving the ATCO productivity and reducing the workload.

New Capabilities		Conflict Detection Tools (CDT) will include enhancements to MTCD (up to 20 minutes planning horizon) function, plus tactical functions based on a shorter look-ahead time, typically from 10 to 2 minutes ahead. MTCD will be complemented by basic a conflict resolution advisor and a what if function.

Monitoring aids (MONA) provide controllers with reminders and warnings and are enhanced via the integration of Aircraft Derived Data (ADD). The provision of a trajectory warning with respect to downlinked parameters, as well as the additional reminders for change of frequency and manual coordination enhance support provided to controllers.

Description		CDT provides real-time assistance to the en-route controllers (both planning and tactical) in conflict detection and resolution. It is based on new approaches that enhance and refine the existing tools yielding more efficient and usable services.

MTCD aids the planning ATCO by showing only the most probable conflicts within the predefined look-ahead time, discarding detected conflicts with lower probabilities. The MTCD includes the what if probe function showing the problems that would occur if the given clearances is applied and identify the contextual traffic that may impair the manual identified conflict resolution.

The tactical tool is based on the tactical trajectories and identifies the conflicts within the sectors, including the what-if capabilities.

MONA provides the en-route controller with warnings if aircraft deviate from the calculated ground system trajectory or the ATCOs tactical clearances (e.g. heading, vertical rate).

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

DEPENDENCIES AND RELATIONS

Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   En-route   Arrival

| Type of Dependencies | ASBU Element |  |
| --- | --- | --- |
| Evolution | FRTO-B0/4 - Basic conflict detection and conformance monitoring |  |
| Relation-operational need | FRTO-B1/1 - Free Route Airspace (FRA) |  |
| Relation-operational need | FRTO-B1/6 - Multi-Sector Planning |  |
| ENABLERS |  |  |
| Enabler Enabler Type | Enabler Name Description / References Stakeholders | Year |
| Category |  |  |
| Operational Design and procedures operations | ATC procedures Design and use of operational ANSP for enhanced CDT and MONA | 2019 |
| Ground ATC systems system infrastructur e | ATC system Upgrade ATC systems to provide ANSP upgrades for enhanced monitoring capabilities as well enhanced CDT as detection of planned/tactical conflicts and MONA Reference: EUROCONTROL Monitoring functions Aids (MONA) specification 3 March 2017. This document provides system requirements for Monitoring Aids (MONA).https://www.eurocontrol.int/standa rds?page= Reference: EUROCONTROL Medium-Term Conflict Detection (MTCD) specification 3 March 2017.This document provides system requirements for Medium-Term Conflict Detection (MTCD). https://www.eurocontrol.int/standards? page=4 EUROCONTROL Trajectory Prediction Specification Edition 2.0 March 2017 https://www.eurocontrol.int/publications/traj ectory-prediction-specification | 2019 |
| Training - | Training ATCO Training for CDT and MONA ANSP requirements for Provide training to staff prior to enhanced CDT implementation and MONA | 2019 |

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Improve situational awareness of ATCO | ANSP |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

DEPLOYMENT APPLICABILITY

Operational conditions:

The element will bring benefit in an en-route medium to high complexity traffic environment.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

FRTO-B1/6	Multi-Sector Planning	Operational

Sixth edition of the GANP

Main Purpose		This element is applicable only to en-route sectors that are currently staffed by two ATCOs (planning and tactical).

The multi-sector planning (MSP) function defines a new organization of controller team(s) and new operating procedures to enable the planning controller to provide support to several tactical controllers operating in different adjacent sectors.

This function might reduce the ATCO workload related to intra/inter centre coordination. The workload conversion to potential capacity gains might vary considerably depending on the sector configurations.

New Capabilities	New tools and operating procedures are needed for the planning controller to provide support to several tactical controllers operating in different sectors. The Multi Sector Planning (MSP) controller ensures suitable coordination agreements between sectors and assists in managing the workload of the tactical controllers.

Description		The ATM system functions are enhanced to allow a single planner role to be associated to multiple sector tactical roles and the planner and tactical roles to be combined on a controller work position. The multi-sector planner needs to have an access to flight data, system tracks, trajectory, warnings and tools for the airspace of several ATC sectors allocated to him.

The multi-sector planner needs to provide an extended planner functions within the sectors allocated to this role, including the coordination function.

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

OPERATIONS

En-route

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

FRTO-B1/3 - Advanced Flexible Use of Airspace (FUA) and management

of real time airspace data

Relation-operational need

FRTO-B1/5 - Enhanced Conflict Detection Tools and Conformance

Monitoring

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Design and operations

ATC procedures for MSP

Design and use of operational procedures  ANSP 	2019

Ground system infrastructur e

ATC systems ATC system

Upgrade for Support of MSP Role

Upgrade ATC systems with the capabilities to support the planning controller taking the responsibility of more than one sector. The tactical responsibilities are distributed to several tactical controllers. Reference: EUROCONTROL Medium-Term Conflict Detection (MTCD) specification 3 March 2017.This document provides system requirements for Medium-Term Conflict Detection (MTCD). https://www.eurocontrol.int/standards? page=4 EUROCONTROL Trajectory Prediction Specification Edition 2.0 March 2017

https://www.eurocontrol.int/publications/traj ectory-prediction-specification

ANSP 	2019

Training	-	Training requirements for MSP

ATCO Training Provide training to staff prior to implementation

ANSP 	2019

DEPLOYMENT APPLICABILITY

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Cost effectiveness |  | Reduce costs in the Air Navigation System | ++ |  |

Operational conditions:

The element will bring benefit in an en-route medium to high complexity traffic environment.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Benefitting stakeholder(s)

Operational description

Type

FRTO-B1/7	Trajectory Options Set (TOS)	Operational

Sixth edition of the GANP

Main Purpose		To give airspace users greater flexibility and control over their trajectory with respect to airspace constraints.

New Capabilities		Airspace users have the capability to develop and file a set of desired reroute options (called a Trajectory Options Set or TOS) that is the operator’s preference for routing around airspace constraints.

From these options, ATFM will choose the one that the operator is expected to fly. Operators have te capability to receive and process these notifications.

Description	Trajectory Options Sets (TOS) are used when airspace users are participating in Collaborative Option Programs (CTOP). These work as follows:

- ATFM creates an airspace boundary and establishes flow control on any air traffic that crosses that boundary. (This is a NOPS action).

- Airspace Users based on the notice of the airspace constraint develop and submit in advance of the issuance of the program, a set of desired reroute options (called a Trajectory Options Set or TOS) that is the operator’s preference for routing around the constraint.

- CTOP uses the preferred options to automatically assign delays or reroutes to flights in order to dynamically manage the demand as conditions change.

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-Pre ops

Pre-tactical

Strategical

OPERATIONS

Arrival

En-route

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational benefit

FRTO-B1/1 - Free Route Airspace (FRA)

Relation-operational benefit

FRTO-B1/2 - Required Navigation Performance (RNP) routes

Relation-operational benefit

FRTO-B1/3 - Advanced Flexible Use of Airspace (FUA) and management

of real time airspace data

Evolution

FRTO-B0/3 - Pre-validated and coordinated ATS routes to support flight and

flow

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

Advisory circular

CTOP advisory circular

Guidance to customers how to manage flights in the constrained area Reference: FAA AC 90-115

CAA 	2019

Operational procedures

Operations	Operational

procedures for the use of CTOP

Operational guidelines on CTOP usage	 ANSP

2019

Aircraft operator

Ground system infrastructur e

ATM systems Tools and system

Aircraft operator

to support CTOP

Tools and systems in place at ANSP to support CTOP operations Reference: https://cdm.fly.faa.gov/?page_id=983

ANSP 	2019

Aircraft operator

Ground system infrastructure

CFSP

systems

CSFP to support CTOP

Tools and systems in place at AOs to support CTOP operations Reference: https://cdm.fly.faa.gov/?page_id=983

2019

Training	-	Training requirements for CTOP

ATM Training, Flight Dispatcher training Training is provided on how to use CTOP system to increase flight operation efficiency

ANSP

2019

DEPLOYMENT APPLICABILITY

Operational conditions:

The element will bring benefit in an en-route medium to high complexity traffic environment.

Main intended benefits:

Benefitting stakeholder(s)

Operational description

Type

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

FRTO-B2/1	Local components of integrated ATFM and ATC Planning function (INAP)

Operational

Sixth edition of the GANP

Main Purpose		Bridge the planning gap between local ATFM and ATC processes and facilitate layered ATM planning in the execution phase.

New Capabilities	1. Transparency and exchange of information between local ATFM and ATC in respect of necessary action the execution phase.

- Relevant and shared information on the predicted complexity of the operation with workload impact in short and medium term up to 45 minutes.

- Informed decision making via integration with local resource management (scheduling tools)

- Full use of DAC system capabilities.

- Use of the information in the local ATFM and ATC decision support tools in a harmonised and integrated manner.

Description	In the ATM layered planning the gap between the ATFM planning and ATC planning is being closed.

On one side ATFM planning traditionally dealing in pre-departure is being extended into the execution phase). On the other side ATC planning, due better precision of the Trajectory Prediction and thus ATC support tools, is pushed further than the traditional sector entry and exist planning with the horizon of 8 to 15 minutes to extended ATC planning with the horizon up to 45 minutes, especially with further progression of the concept like MSP and Flight Centric Operations. Basis for this integration is the use and exchange of predicted workload information based on complexity assessment as a best approximation for ATCO workload.

The enhanced tool will take into account all available information combine them and deliver resolutions for overload situations with the less disruption and most performant network effect. The tool is the main enabler for optimization of the two main resources of the ATM - the airspace and the ATCOs. It is designed in such a way to keep the airspace and ATCO workload related constrains always at the optimal level in the real time. Therefore, what was considered as a planning gap is becoming an overlap requiring integration and harmonization of Local ATFM and ATC operations. This integration will eventually result in complete transformation and disappearance of the “sector defensive” traditional ATC planning into seamless ATM planning process.

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

PLANNING LAYERS

Tactical-During ops

OPERATIONS

En-route

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

FRTO-B1/6 - Multi-Sector Planning

Evolution

NOPS-B1/4 - Dynamic Traffic Complexity Management

Relation-operational need

NOPS-B2/1 - Optimised ATM Network Services in the initial TBO context

Relation-operational need

NOPS-B2/6 - ATFM adapted for cross-border Free Route Airspace (FRA)

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational Design and	Operational

procedures	operations	procedures for

INAP

Design and use of operational procedures

for Dynamic Capacity Balancing - DCB and Extended ATC planning (INAP)

ANSP

ATM network function

2025

Ground	ATM systems

system infrastructur e

ATC/ATFM

system upgrade for support of INAP

Upgrade the ATC/ATFM system to

support the integration of ATFM and ATC planning function

ANSP

ATM network function

2025

Training

-

Training

requirements for INAP

ATCO Training for INAP FMP training for

INAP Provide training to staff prior to implementation

ANSP

2025

DEPLOYMENT APPLICABILITY

Operational conditions:

The element will bring benefit in an en-route medium to high complexity traffic environment.

Main intended benefits:

Benefitting stakeholder(s)

Operational description

Type

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Optimise actual capacity (capacity monitoring values and sector configurations actually used on the day of operation) | ++ |  |

FRTO-B2/2	Local components of Dynamic Airspace Configurations (DAC)

Operational

Sixth edition of the GANP

Main Purpose	To provide optimum use of the airspace, with minimum impact on the AU business needs

New Capabilities	Re-organisation and re-structuring of the airspace in the execution phase and in real time including transition from between different type of airspace configurations and related ATC operations

Description		In order to satisfy stakeholders business needs, it will be required that the airspace configuration changes and transfer from one type of ATC operation to another happens more dynamically. This is enabled by automation and existence of large number of airspace configuration scenarios adapted for different traffic flow or complexity situations.

It is clear the efficient forms of configurations supporting Free-route or concepts like Fight Centric Operations become impracticable when certain level of complexity is reached. Dynamic Airspace Configurations using local tools and procedures in short term are supposed to optimize these transitions and make them possible in real time.

Dynamic mobile areas introduction will allow for restricted/reserved areas to be either planned along a trajectory or a protective area that will accompany the user during the whole or part of the trajectory to ensure the safety of all airspace users.

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS 		OPERATIONS

Tactical-Pre ops

Tactical-During ops

Departure

En-route

Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Evolution	NOPS-B1/6 - Initial Dynamic Airspace configurations

Relation-operational need	NOPS-B2/1 - Optimised ATM Network Services in the initial TBO context Relation-operational need	FRTO-B2/1 - Local components of integrated ATFM and ATC Planning

function (INAP)

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

| Regulatory | Operational | Operational | Follow regulations for approval Adapt | CAA | ICAO | 2025 |
| --- | --- | --- | --- | --- | --- | --- |
| provisions | Approval | approval for DAC | present airspace classification. Define responsibility and liability in DAC environment |  |  |  |

Operational procedures

Design and operations

Operational procedures for DAC

Design and use of operational procedures: Transitional procedures for different ATC operations in DAC situations. Procedures for instantaneous airspace delegation. Procedures for dynamic change of mode of ATC operations in a portion of an airspace.

2025

ATM network function

ANSP

Ground system infrastructur e

ATM systems ATM system

Upgrade Provide upgrade to support of DAC

Upgrade ATC/ATFM system with automated tools capable of predicting traffic and complexity evolution and applying low-altitude DAC based resolution scenarios.

2025

ATM network function

ANSP

Training	-	Training requirements for DAC

ATC, Local Airspace Mangers, supervisors, Network function staff and FMP training Provide training to staff prior to implementation

2025

ATM network function

ANSP

DEPLOYMENT APPLICABILITY

#### Operational conditions:

The element will bring benefit in an en-route medium to high complexity traffic environment.

Main intended benefits:

Type

Operational description

Benefitting stakeholder(s)

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Improve flexibility of sector configuration management | ++ |  |
| Capacity | Capacity, throughput & utilization | Optimise actual capacity (capacity monitoring values and sector configurations actually used on the day of operation) | ++ |  |

Sixth edition of the GANP

Large Scale Cross Border Free Route Airspace	Operational

(FRA)

FRTO-B2/3

Main Purpose 	The Free Route Airspace (FRA) concept brings significant flight efficiency benefits and a choice of user preferred routes to airspace users. Large -scale operations based on regional implementation can provide even greater freedom and benefits to AUs to fly according to their own business models leading to full trajectory based operations. Large scale FRA will continue to provide increased flight predictability, reduced uncertainty for the ATM Network function, which in turn can lead to potential capacity increases for ATM, which will also benefit the user.

New Capabilities   FRA is specified airspace within which users may freely plan a route between a defined entry point and a defined exit point, with the possibility to route via intermediate (published or unpublished) waypoints, without reference to the ATS route network, subject to airspace availability. Within this airspace, flights remain subject to air traffic control. In large scale operations the concept is applied across a large scale geographical area typically changing from State implementation to regional implementation.

Description 		The initial implementation of FRA comes with certain limitations e.g. FL, dimensions, timing, functions and tools, Lat/Long use as a significant point etc. Large scale operations of FRA will ensure that these limitation are solved and no longer required leading to seamless operations in a large volume of airspace crossing State borders.

Maturity Level 	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-Pre ops   Tactical-During ops

OPERATIONS

En-route

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

FRTO-B1/1 - Free Route Airspace (FRA)

Relation-operational need

FRTO-B1/3 - Advanced Flexible Use of Airspace (FUA) and management

of real time airspace data

Relation-operational need

FRTO-B1/6 - Multi-Sector Planning

Relation-operational need

FRTO-B1/5 - Enhanced Conflict Detection Tools and Conformance

Monitoring

Relation-operational need

NOPS-B1/4 - Dynamic Traffic Complexity Management

Relation-operational need

NOPS-B2/6 - ATFM adapted for cross-border Free Route Airspace (FRA)

Relation-operational need

FRTO-B2/4 - Enhanced Conflict Resolution Tools

Relation-operational need

FRTO-B2/1 - Local components of integrated ATFM and ATC Planning

function (INAP)

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Design and operations

Cross border FRA Airspace Design

Design and use of operational procedures Reference: EUROCONTROL European Route Network Improvement Plan (ERNIP)

- Part 1: European Airspace Design Methodology - Guidelines - Edition 5 December 2018 https://www.eurocontrol.int/publications/eur opean-route-network-improvement-plan- ernip-part-1-european-airspace-design

ANSP

ATM network function

2025

Ground system infrastructur e

ATM systems Upgrade ATM

systems for improvements for cross-border FRA

ATM system upgraded for cross-border FRA clearances, notification and co- ordination data exchanges relevant for cross –border FRA and management of relevant cross-border FRA airspace data. Reference: EUROCONTROL Monitoring Aids (MONA) specification 3 March 2017. This document provides system requirements for Monitoring Aids (MONA).https://www.eurocontrol.int/standa rds?page= Reference: EUROCONTROL Medium-Term Conflict Detection (MTCD) specification 3 March 2017.This document provides system requirements for Medium-Term Conflict Detection (MTCD).

https://www.eurocontrol.int/standards? page=4 EUROCONTROL Trajectory Prediction Specification Edition 2.0 March 2017

https://www.eurocontrol.int/publications/traj ectory-prediction-specification EUROCONTROL specification for the on- line Data exchanges (OLDI) https://eurocontrol.int/sites/default/files/pub lication/files/EUROCONTROL%20Specifi cation%20OLDI%204.3.pdf

ANSP

ATM network function

2025

| Ground | CFSP | Upgrade of AOs | Upgrade of AOs flight planning systems | Aircraft operator | 2025 |
| --- | --- | --- | --- | --- | --- |
| system | systems | Flight Planning | Reference: EUROCONTROL NM Flight |  |  |
| infrastructur |  | Systems for | Planning Requirements document 10 |  |  |
| e |  | cross-border | December 2018 |  |  |
|  |  | FRA | https://www.eurocontrol.int/publications/nm |  |  |
|  |  |  | -flight-planning-requirements-guidelines |  |  |
| Training | - | Training | ATCO, AO and ATM Network Function | ANSP | 2025 |
|  |  | requirements for cross-border | Training Training for Cross-Border FRA Provide training to staff prior to | ATM network function |  |
| FRA |  |  | implementation | Aircraft operator |  |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Flight time & distance | Overcome route selection inefficiencies associated with route network design | ++ | KPI04: Filed flight plan en-route extension |

Ground

system infrastructur e

ATM systems

ATFM system for

cross border FRA

Upgrade ATFM/flight planning systems to

support cross border FRA. Reference: EUROCONTROL NM Flight Planning Requirements document December 2018 https://www.eurocontrol.int/publications/nm

-flight-planning-requirements-guidelines

2025

DEPLOYMENT APPLICABILITY

Operational conditions:

The element will bring benefit in an en-route medium to high complexity traffic environment.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Benefitting stakeholder(s)

Operational description

Type

ATM network function

FRTO-B2/4	Enhanced Conflict Resolution Tools	Operational

Sixth edition of the GANP

Main Purpose	Providing conflict resolution support for ATCOs considering elements of ATM layered planning and overall network effect.

New Capabilities	1. Taking into account additional INAP and network effect elements when providing conflict resolutions.

- Harmonisation and integration with the ATFM support tools resulting in informed decision making

- Integration of foreseen conflict resolution options for workload assessment and prediction

Description 			Basic conflict resolution tools are designed to provide resolution solutions for immanent conflict taking into account certain time horizon for which the resolution trajectory has to be conflict free. In the new planning environment with extended panning horizons and integration between local ATFM and ATC processes this feature is insufficient. In order to provide informed decision on conflict resolution, which will have no negative network effect, enhanced resolution tools are to be designed. They will exploit the SWIM enabled capability to exchange and probe possible conflict resolutions taking into account the network effect to some extend (compatibility with Target Times, CTA and CTO) and the effect on the workload on other ATCOs on a longer planning scale. This approach induces substantial changes in the ATCO’s working methods but on other side should provide for the flexibility and required level of freedom for safe separation in human centric environment.

Maturity Level 		Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   En-route   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

FRTO-B1/5 - Enhanced Conflict Detection Tools and Conformance

Monitoring

Relation-operational need

FRTO-B2/3 - Large Scale Cross Border Free Route Airspace (FRA)

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

Operational Approval

Operational Approval for enhanced conflict resolution tools

Obtain Regulatory Approval Provisions for limited delegation of Responsibility and Liability to the automation will be required

CAA 	2025

Operational procedures

-	Operational

Aircraft manufacturer

procedures for enhanced conflict resolution tools

Adaptation of Manuals Design and use of operational Procedures

ANSP 	2025

Airborne system capability

Aircraft system

Capability	Downlink of airborne calculation of the

Trajectory and other aircraft derived data for improvement of TP

Airspace user

2025

Ground system infrastructure

ATC systems Integrate

enhanced Conflict Resolution Tools

Upgrade ATC system with advance conflict resolution capabilities

ANSP 	2025

Training	-	Training requirements for enhanced conflict resolution tools

Training for the use of the function Provide training to staff prior to implementation

ANSP 	2025

DEPLOYMENT APPLICABILITY

Operational conditions:

The element will bring benefit in an en-route medium to high complexity traffic environment.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Benefitting stakeholder(s)

Operational description

Type

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Avoid vertical & lateral navigation errors during flight (cases of non-conformance with clearance) | ++ | KPI20: Number of aircraft accidents |
| Safety |  | Improve early detection of conflicting ATC Clearances (CATC) (en-route / departure / approach) | ++ | KPI20: Number of aircraft accidents KPI23: Number of airprox/TCAS alert/loss of separation/near midair collisions/midair collisions (MAC) |

GANP PORTAL

# ASBU ELEMENTS

| GADS | NAVS | NOPS | OPFL |
| --- | --- | --- | --- |

| Functional Description | Enablers | Deployment Applicability | Performance Impact Assessment |
| --- | --- | --- | --- |

## GADS

GADS-B1/1	Aircraft Tracking	Operational

Sixth edition of the GANP

Main Purpose	To provide support to the ATSU Alerting Service in areas without ATS surveillance with an update rate of the aircraft position of at least once per 15 mins. The objective is to assist the relevant stakeholders in the timely identification and location of aircraft in distress, to reduce reliance on the procedural methods for determining aircraft position and helping to ensure the availability and sharing of aircraft position data.

New Capabilities	Aircraft operator will be able to track the aircraft, detect missing position reports, notify if necessary the relevant ATSUs and timely share relevant information including last known position(s).

Description		Aircraft tracking is one of the Global Aeronautical Distress and Safety System (GADSS) functions (ref, GADSS ConOPS V6). Aircraft tracking is a process, established by the operator, that maintains and updates, at standardised intervals, a ground based record of the four dimensional position of individual aircraft in flight. (ICAO Annex 6)

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

If the aircraft operator is not already tracking its aircraft.

- Does it imply a change to levels of automation? Yes

Tactical-During ops

Tactical-Pre ops

PLANNING LAYERS

OPERATIONS

En-route

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-operational need	GADS-B1/2 - Operational Control Directory

Relation-technology option

COMS-B0/2 - ADS-C (FANS 1/A) for procedural airspace

Relation-technology option

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-technology option

ASUR-B1/1 - Reception of aircraft ADS-B signals from space (SB ADS-B)

Relation-technology option

COMI-B0/5 - Satellite communications (SATCOM) Class C Data

Relation-technology option

COMI-B1/3 - SATCOM Class B Voice and Data

Relation-technology option

COMS-B1/2 - PBCS approved ADS-C (FANS 1/A+) for procedural

airspace

### ENABLERS

Aircraft operator

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Search and rescue

Procedures for aircraft tracking

Operator aircraft tracking policy, process and procedures. References: ICAO Annex 6 – Aircraft Tracking and ICAO Circular 347 - Aircraft Tracking Implementation Guidelines

2018

Airborne system capability

Search and rescue

Airborne aircraft tracking system

Airborne aircraft tracking capability. Note: copy table in the circular. Reference: ICAO Annex 6 – Aircraft Tracking and ICAO Circular 347 - Aircraft Tracking Implementation Guidelines.

2018

Ground system infrastructur e

Search and rescue

Data link for aircraft tracking

Airborne aircraft tracking capability. Note: copy table in the circular. Reference: ICAO Annex 6 – Aircraft Tracking and ICAO Circular 347 - Aircraft Tracking Implementation Guidelines.

2018

Ground system infrastructur e

Search and rescue

Ground aircraft tracking system

System with capability to process and monitor aircraft tracking data. Airborne aircraft tracking capability. Note: copy table in the circular. Reference: ICAO Annex 6 – Aircraft Tracking and ICAO Circular 347 - Aircraft Tracking Implementation Guidelines.

2018

Training	Search and rescue

Aircraft tracking training

Aircraft Operator procedures for detecting missing position reports and notifying ATSU’s

2018

Aircraft operator

Aircraft operator

Aircraft operator

Aircraft operator

DEPLOYMENT APPLICABILITY

Operational conditions:

This element is only applicable in oceanic airspace lacking ATS surveillance with a position update rate of at least

once per 15 mins.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Sixth edition of the GANP

Main Purpose

To ensure that Point of Contact (PoC) information is available and can be accessed by Rescue

Coordination Centres (RCCs), ATSUs and aircraft operators in support of emergency situations.

New Capabilities

Access to point of contact information of ATSUs, Aircraft Operators and Rescue Coordination

Centres.

Description

Point of Contact repository is part of the Global Aeronautical Distress and Safety System (GADSS)

and is used to enable timely contact between the persons relevant to an emergency situation involving an aircraft in a specified area.

Maturity Level

Ready for implementation

Human Factor

Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

PLANNING LAYERS

OPERATIONS

En-route

Operational

Operational Control Directory

GADS-B1/2

| ANSP | Aircraft operator |  | RCC |
| --- | --- | --- | --- |
| SAR authority |  |  |  |

| Type Operational description Benefitting stakeholder(s) |  |
| --- | --- |
| Direct benefits | Earlier detection of aircraft in operational ANSP Aircraft operator anomalies |
|  | Better position information of aircraft in distress |
|  | Improve situational awareness of aircraft Aircraft operator operator |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

| Strategical | Tactical-Pre ops |  | Tactical-During ops |
| --- | --- | --- | --- |
| Post operations |  |  |  |

### DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Search and rescue

Procedures for Operational Control Directory

Procedures for maintaining PoC information and making PoC updates available. Procedures for using PoC repository. Reference: Annex 11; Annex 12 and ICAO Circular 347 - Aircraft Tracking Implementation Guidelines

ANSP

Aircraft operator

RCC

2018

### DEPLOYMENT APPLICABILITY

#### Operational conditions:

This element is applicable in emergency situations.

| ANSP | Aircraft operator |  | RCC |
| --- | --- | --- | --- |
| SAR authority |  |  |  |

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Faster and more effective response to emergency situations |  |
|  | Improve situational awareness of aircraft operator, ANSP and RCCs. |  |

| ANSP | Aircraft operator |  | RCC |
| --- | --- | --- | --- |
| SAR authority |  |  |  |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

Sixth edition of the GANP

Main Purpose

To ensure tracking of aircraft in distress and timely and accurate location of end of flight position to

accurately direct search and rescue (SAR) operations in an efficient and effective way.

New Capabilities

To be able to determine the position of an aircraft in distress at least once every minute. This

capability is resilient to failures of the aircraft’s electrical power, navigation and communication systems.

Operational

Location of an aircraft in Distress

GADS-B2/1

Description 		The localisation of an aircraft in distress is one of the Global Aeronautical Distress and Safety System (GADSS) functions. This function uses on board systems to broadcast aircraft position (latitude and longitude), or distinctive distress signals from which the aircraft position and time can be derived. The aircraft position information will be transmitted, without the need for flight crew action, at least once every minute, when an aircraft is in a distress condition.

An aircraft is in a distress condition when it is in a state that, if the aircraft behaviour event is left uncorrected, may result in an accident. The operator is responsible for ensuring that this information is made available to the actors involved in the emergency.

Maturity Level 	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   En-route   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology need

GADS-B1/2 - Operational Control Directory

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Search and rescue

Procedures for location of aircraft in distress

References: ICAO Annex 6 – Location of an aircraft in distress and ICAO Doc 10054 Manual on Location of Aircraft in Distress and Flight Recorder Data Recovery

ANSP

Aircraft operator

Aircraft operator

RCC

2018

Aircraft operator

Airborne system capability

Search and rescue

Location of aircraft in distress

Applicable to new production aircraft from 2021. Reference: ICAO Annex 6 – Location of an aircraft in distress

2023

Ground system infrastructur e

Search and rescue

Capability to receive a distress signal

Distress Tracking Repository. Reference: ICAO Annex 6 – Location of an aircraft in distress and ICAO Doc 10054 Manual on Location of Aircraft in Distress and Flight Recorder Data Recovery

2023

Training

Search and

rescue

Training

requirements for location of aircraft in distress

Aircraft Operator procedures for ADT

2023

DEPLOYMENT APPLICABILITY

Operational conditions:

This element is applicable in emergency situations.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Aircraft operator

| ANSP | Aircraft operator |  | RCC |
| --- | --- | --- | --- |
| SAR authority |  |  |  |

| ANSP | Aircraft operator |  | RCC |
| --- | --- | --- | --- |
| SAR authority |  |  |  |

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Faster and more effective response to emergency situations |  |
|  | More reliable accident site location |  |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

Sixth edition of the GANP

Main Purpose

To ensure distress tracking information is available and can be accessed by RCCs, ATSUs and

aircraft operators in support of emergency procedures.

New Capabilities	Access to location of aircraft in distress data.

Description

Distress tracking information is of vital importance for efficient and effective handling of distress

situations and SAR operations. Distress tracking information management provides a means to make the last known position of an aircraft in distress available to the relevant stakeholders in atimely manner.

Maturity Level

Ready for implementation

Operational

Distress tracking information management

GADS-B2/2

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

DEPLOYMENT APPLICABILITY

Operational conditions:

This element is applicable in emergency situations.

Main intended benefits:

| ANSP | Aircraft operator |  | RCC |
| --- | --- | --- | --- |
| SAR authority |  |  |  |

| ANSP | Aircraft operator |  | RCC |
| --- | --- | --- | --- |
| SAR authority |  |  |  |

| ANSP | Aircraft operator |  | RCC |
| --- | --- | --- | --- |
| SAR authority |  |  |  |

PLANNING LAYERS

Post operations

Tactical-During ops

OPERATIONS

Arrival

En-route

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational option

SWIM-B2/1 - Information service provision

Relation-operational option

SWIM-B2/2 - Information service consumption

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational Search and	Procedures for

procedures	rescue	making available

and accessing distress tracking information

Reference: ICAO Annex 6 – Location of

an aircraft in distress/Flight Recorders and ICAO Doc 10054 Manual on Location of Aircraft in Distress and Flight Recorder Data Recovery.

ANSP

Aircraft operator

2021

RCC

| Type Operational description Benefitting stakeholder(s) |  |
| --- | --- |
| Direct benefits | Faster and more effective response to emergency situations |
|  | More reliable accident site location |
|  | Improve situational awareness of aircraft operator, ANSP and RCCs. |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

GADS-B2/3	Post Flight Localization	Operational

Sixth edition of the GANP

Main Purpose		To enable efficient and effective SAR operations by providing accurate position information of the end of flight location following a crash.

New Capabilities	The capability to locate the end of flight with a level of accuracy required by SAR.

Description		Post Flight Localization is one of the GADSS functions. When an accident occurs there is a phase beginning immediately at the end of flight where the rescue of possible survivors has the immediate and highest priority. Accurate aircraft position information (1 NM or better) is provided through the Post Flight Localization function to guide SAR services on site.

Maturity Level	Standardization

Human Factor Considerations

### PLANNING LAYERS

Post operations

### OPERATIONS

Departure   En-route   Arrival

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-operational benefit	GADS-B2/1 - Location of an aircraft in Distress

### ENABLERS

Aircraft operator

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Search and rescue

Procedures regarding the operation of the system intended for timely recovery of flight recorder data

Description of the operator capabilities. Reference: Annex 6 – Location of an aircraft in distress and ICAO Doc 10054 Manual on Location of Aircraft in Distress and Flight Recorder Data Recovery

2021

Airborne system capability

Search and rescue

ELT	Reference: ICAO Annex 6 – Location of an aircraft in distress and ICAO Doc 10054 Manual on Location of Aircraft in Distress and Flight Recorder Data Recovery

2019

Airborne system capability

Search and rescue

ULD	Reference: ICAO Annex 6 – Location of an aircraft in distress and ICAO Doc 10054 Manual on Location of Aircraft in Distress and Flight Recorder Data Recovery

2019

RCC

Aircraft operator

RCC

Aircraft operator

DEPLOYMENT APPLICABILITY

#### Operational conditions:

This element is applicable in emergency situations.

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Faster and more effective response to emergency situations | Aircraft operator RCC SAR authority |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

GADS-B2/4	Flight Data Recovery	Operational

Sixth edition of the GANP

Main Purpose	To enable timely and efficiently recovery of information for accident investigation.

New Capabilities		Timely recovery and availability of information relevant for accident investigation. Avoiding long and costly search operations to recover Flight Data Recorder.

Description		To ensure accident investigation authorities obtain timely access to the flight recorder information, the aircraft will be equipped with a means, approved by the State of the aircraft operator, to recover the flight recorder data and make it available in a timely manner. The requirements for approving the means to make flight recorder data available in a timely manner are detailed in ICAO Annex 6.

Maturity Level	Ready for implementation

Human Factor Considerations

PLANNING LAYERS

Post operations

OPERATIONS

Departure   En-route   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational benefit

GADS-B2/1 - Location of an aircraft in Distress

### ENABLERS

Aircraft operator

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Search and rescue

Procedures regarding the operation of the system intended for timely recovery of flight recorder data

Description of the operator capabilities. Reference: Annex 6 – Location of an aircraft in distress and ICAO Doc 10054 Manual on Location of Aircraft in Distress and Flight Recorder Data Recovery

2021

Airborne system capability

Search and rescue

System intended for timely recovery of flight recorder data

Reference: ICAO Annex 6 – Location of an aircraft in distress and ICAO Doc 10054 Manual on Location of Aircraft in Distress and Flight Recorder Data Recovery

2025

DEPLOYMENT APPLICABILITY

Operational conditions:

This element is applicable in emergency situations.

Main intended benefits:

Ground system infrastructur e

Search and rescue

C,N,S	No CNS needed for Automatic Deployable Flight Recorder. Satellite Link needed for flight recorder streaming before impact.

2025

Aircraft operator

Aircraft operator

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Timely retrieval flight recorder data | Aircraft operator Accident Investigation Authority |
|  | Improve AIA understanding of the flight environment pre the end of flight. | Accident Investigation Authority |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

## NAVS

NAVS-B0/1	Ground Based Augmentation Systems (GBAS)	Technology

Sixth edition of the GANP

Main Purpose	Support Precision Approach and landing operations at a specific airport (one system may support all runway ends). As an option, may support arrival and departure phases of flight.

New Capabilities	Category I performance using GBAS Approach Service Type C (GAST-C).

As an option, PBN in terminal area (RNAV 1 and RNP 1 operations) can be supported using GBAS positioning service.

Description		This element introduces improved accuracy, integrity and availability through a local airport based differential satellite navigation and monitoring system. A local network of reference receivers is deployed at or near an airport. Observations from these reference receivers are used to compute corrections for each satellite as well as to monitor for system integrity. The information is broadcast to users via a VHF Data Broadcast link (operating in the 108 to 118 MHz band).

Maturity Level	Ready for implementation

Human Factor Considerations

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Departure   Arrival

### DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational Navigation	GBAS Cat I

procedures	Instrument flight procedures

Instrument flight procedures validated and

published. Reference: ICAO Doc 4444 PANS Air Traffic Management; ICAO Doc 8168 PANS Aircraft operations ; ICAO Doc 9849 Global Navigation Satellite System (GNSS) Manual; ICAO Doc 9365 Manual of All-Weather Operations; ICAO EUR Doc 013 European guidance material on All Weather Operations at Aerodromes and ICAO Doc 8071 Manual on Testing of Radio Navigation Aids.

ANSP

Aircraft operator

2013

Airborne

system capability

Navigation

GBAS receiver

GAST C

GBAS receivers. Reference: ICAO Annex

10 Vol I technical requirements; RTCA/EUROCAE MOPS.

Aircraft

manufacturer	2013

Aircraft operator

Ground

system infrastructur e

Navigation

GBAS ground

stations GAST C

GBAS ground stations. Reference: ICAO

Annex 10 Vol I; provisions supporting system certification/approval by the designated authority; RTCA/EUROCAE MOPS.

2013

ANSP

Ground systems supplier

Training

Navigation	Training

requirements for GBAS for CAT I

Adaptation of aircrews due to ILS look

alike concept implementation and procedures designers. Reference: States/Regions regulations.

2013

ANSP

Aircraft operator

Space

system infrastructur e

Navigation

GNSS core

constellation

Core constellation. Reference:

documentation specific to each core constellation defining provisions supporting performance commitment.

Ground systems supplier

Satellite provider

2013

Airport operator

Airport operator

Sixth edition of the GANP

Main Purpose

Support PBN in all phases of flight with an increased accuracy, integrity and availability compared to

ABAS. Increases accuracy and integrity for the vertical guidance.

New Capabilities	Support all PBN navigation specifications, with a deployment emphasis over RNP APCH down to

LPV or LP minima at 250 ft (APV I performance) or 200 ft /550 m (Category I performance)

Satellite Based Augmentation Systems (SBAS)	Technology

NAVS-B0/2

Description 		This element introduces improvements in the availability, accuracy and integrity of satellite navigation through a wide area differential satellite navigation position and integrity monitoring system. A network of ground reference systems is deployed in a region and connected via a data-network.

Observations from the reference systems are used to monitor satellite signals and produce correction and integrity information which is then broadcast over a geostationary satellite link to aircraft. The LPV service volume is mainly determined by the distribution of the monitoring network, depending on the implementation, a wider service volume may be achieved supporting RNP 0.3 and RNP 0.1 performance.

Maturity Level 	Ready for implementation

Human Factor Considerations

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   En-route   Arrival

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational Navigation	SBAS instrument

procedures	flight procedures

Procedures validated and published.

Reference material: ICAO Doc 4444 PANS Air Traffic Management; ICAO Doc 8168 PANS Aircraft operations or United States Standards for Terminal Instrument Procedures (TERPS); ICAO Doc 9849 Global Navigation Satellite System (GNSS) Manual; ICAO Doc 9613

Performance-based Navigation (PBN) Manual and ICAO Doc 8071 Manual on

Testing of Radio Navigation Aids.

2013

Airborne

system capability

Navigation

SBAS receiver

SBAS receivers integrated with aircraft

navigation systems. Reference: ICAO Annex 10 Vol I technical requirements; RTCA/EUROCAE MOPS.

2013

Aircraft operator

Aircraft manufacturer

Aircraft operator

Aircraft manufacturer

ANSP

Ground

system infrastructur e

Navigation

SBAS ground

stations

Ground stations to provide a wide area

monitoring network. No local airport infrastructure is required. However, a network of ground reference systems and a highly reliable data network with processing facilities and satellite signal generation capability is required.

References: ICAO Annex 10 Vol I; document specific to each SBAS; provisions supporting system certification/approval by the designated

authority and performance commitments.

2013

Training

Navigation	Training

requirements for SBAS

Adaptation of aircrews due to ILS look

alike concept implementation and procedures designers. Reference: ICAO Doc 9613 Performance-based Navigation (PBN) Manual and States/Regions regulations.

ANSP	2013

Aircraft manufacturer

Aircraft operator

Space

system infrastructur e

Navigation

GNSS core

constellation

Core constellation. Reference:

documentation specific to each core constellation defining provisions supporting performance commitment.

Ground systems supplier

Satellite provider

2013

Space

system infrastructur e

Navigation

GNSS

augmentation satellites

These satellites broadcast the SBAS

messages to the different users. Reference: document specific to each SBAS provider

2013

SBAS service provider

SBAS service provider

Sixth edition of the GANP

Main Purpose

Support non-precision (LNAV) and vertically guided (LNAV/VNAV) approaches with BaroVNAV and

other terminal and enroute navigations.

New Capabilities	ABAS supports all PBN navigation specifications with the exception of RNP APCH down to

LPV/LP minima. Specific ABAS configurations are required to support RNP AR APCH.

Description

This element supports non-precision and vertically guided approaches using GNSS lateral

navigation and barometric vertical guidance.

Maturity Level

Ready for implementation

Human Factor

Considerations

PLANNING LAYERS

OPERATIONS

Technology

Aircraft Based Augmentation Systems (ABAS)

NAVS-B0/3

Tactical-During ops 	 Departure   En-route   Arrival

### DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### ENABLERS

Aircraft operator

ANSP

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Navigation	ABAS instrument

flight procedures

ABAS instrument flight procedures validated and published. Reference: ICAO Doc 4444 PANS Air Traffic Management; ICAO Doc 8168 PANS Aircraft operations or United States Standard for Terminal Instrument Procedures (TERPS); ICAO Doc 9849 Global Navigation Satellite System (GNSS) Manual; ICAO Doc 9613 Performance-based Navigation (PBN) Manual and ICAO Doc 8071 Manual on Testing of Radio Navigation Aids.

2013

Airborne system capability

Navigation	ABAS avionics	ABAS without Baro-VNAV capability to

Ground systems supplier

support non-precision (LNAV) or with Baro-VNAV capability to support vertically guided (LNAV/VNAV) approaches. ABAS is capable of supporting other phase of flight such as oceanic, en-route and terminal operations. Reference: ICAO Annex 10 Volume I and ICAO Doc 9613 Performance-based Navigation (PBN) Manual for technical requirements; RTCA/EUROCAE MOPS.

2013

Aircraft operator

ANSP

Space system infrastructur e

Navigation	GNSS core

constellation

Core constellation. Reference: documentation specific to each core constellation defining provisions supporting performance commitment.

2013

Satellite provider

Training	Navigation	Training

requirements for ABAS

Pilot training requirements for RNP APCH and procedure designers. Specific training required for RNP AR APCH.

2013

Aircraft manufacturer

Airport operator

Sixth edition of the GANP

Technology

Navigation Minimal Operating Networks (Nav.

MON)

NAVS-B0/4

Main Purpose	To adjust conventional navaids networks through the increased deployment of satellite based navigation systems and procedures to ensure the necessary levels of resilience for navigation. To provide a minimum level of capabilities to accommodate State aircraft operations where there is a mismatch in terms of aircraft equipage.

To make a more efficient use of the frequency spectrum

New Capabilities	Provision of a navigation backup.

Description		This element allows the rationalization of the ground based conventional infrastructure through the definition of minimal networks of ground navaids. Consultations and agreements from airspace users and aircraft operators are required to define this element.

The MON should be revisited with the introduction of new navigation capabilities.

Maturity Level	Ready for implementation

Human Factor Considerations

### PLANNING LAYERS 		OPERATIONS

Tactical-During ops 	 Departure   En-route   Arrival

### DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

| Other | Navigation | Plan for minimal | Airspace users consultation, dedicated | ANSP | CAA |  | 2013 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | operating networks (MON). | studies and safety cases are required to mitigate the withdrawal of existing ground | Aircraft operator |  |  |  |

networks. References: ICAO Regional Air Navigation Plans

Sixth edition of the GANP

Main Purpose 			To support precision approach and landing operations at a specific airport (one system may support all runway ends). As an option, may support arrival and departure phases of flight

Technology

Extended GBAS

NAVS-B1/1

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Space

system infrastructur e

Navigation

GNSS core

constellation

Core constellation. Reference:

documentation specific to each core constellation defining provisions supporting performance commitment.

2013

Airborne

system capability

Navigation

GBAS receiver

GAST D

GBAS receivers. Reference: ICAO Annex

10 Vol I technical requirements; RTCA/EUROCAE MOPS.

2019

Ground

system infrastructur e

Navigation

GBAS ground

stations GAST D

GBAS ground stations. Reference: ICAO

Annex 10 Vol I; provisions supporting system certification/approval by the designated authority; RTCA/EUROCAE MOPS.

2019

Aircraft operator

Aircraft manufacturer

Satellite provider

Ground systems supplier

New Capabilities 	  CAT II operations utilizing GBAS Approach Service Type C (GAST C) in conjunction with enhanced ionospheric monitoring and airplane augmentations.

Category II/III operations supported by GBAS Approach Service Type D (GAST D).

Enhanced VHF Data Broadcast (VDB) airborne equipment performance to support interoperability of VDB, ILS and VOR in the 108 - 118 MHz band.

Description 		This element introduces improved accuracy, integrity and availability through a local airport based differential satellite navigation and monitoring system. A local network of reference receivers is deployed at or near an airport. Observations from these reference receivers are used to compute corrections for each satellite as well as to monitor for system integrity. The information is broadcast to users via a VHF Data Broadcast link (operating in the 108 to 118 MHz band). This element extends the capability of Block 0 by adding improved ionospheric error monitoring and mitigation as well as enhanced VDB receiver performance to support interoperability and coexistence of ILS, VOR and VDB at any airport.

Maturity Level 	Standardization

Human Factor Considerations

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   Arrival

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

| Airport operator |  |
| --- | --- |
| ANSP |  |
| Ground systems supplier |  |

Operational Operations	GBAS Cat II/III

procedures	Instrument flight procedures

Instrument flight procedures validated and

published. Reference: ICAO Doc 4444 PANS Air Traffic Management; ICAO Doc 8168 PANS Aircraft operations ; ICAO Doc 9849 Global Navigation Satellite System (GNSS) Manual; ICAO Doc 9365 Manual of All-Weather Operations; ICAO EUR Doc 013 European guidance material on All Weather Operations at Aerodromes and ICAO Doc 8071 Manual on Testing of Radio Navigation Aids.

ANSP

Aircraft operator

2019

Training

-

Training

requirements for GBAS for CAT II/III

Adaptation of aircrews due to ILS look

alike concept implementation and procedures designers. Reference: States/Regions regulations.

2019

ANSP

Aircraft operator

Airport operator

NAVS-B2/1	Dual Frequency Multi Constellation (DF MC)

GBAS

Technology

Sixth edition of the GANP

Main Purpose	More robust and less vulnerable to atmospheric propagation perturbations, supports Cat I,II, III GBAS landing operations in all regions of the world.

New Capabilities	GBAS provide dual frequency and multi constellation additional augmentation information.

Description	This element builds upon the basic GBAS systems introduced within Block 1.

New core constellations and signals: (e.g. Beidou, Galileo) or dual frequencies such as GPS, GLONASS will enhance satellite based navigation services through GBAS.

DF MC GBAS will provide additional navigation augmentation information.

Maturity Level	Validation

Human Factor Considerations

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Departure   Arrival

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Evolution	NAVS-B0/1 - Ground Based Augmentation Systems (GBAS)

### ENABLERS

Satellite provider

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Space system infrastructur e

Navigation	New core

constellations

New satellite constellations (Beidou, Galileo) or dual frequencies such as GPS or GLONASS provide opportunities to enhance satellite based navigation services by increasing performance and robustness to failures.

2019

Operational procedures

Navigation	DF MC GBAS

Cat I, II, III

Instrument flight procedures

New core constellations (Beidou, Galileo) and signals (GPS, GLONASS) will provide opportunities to enhance satellite based navigation services through GBAS. New signals/constellations are expected to support existing procedures with an increased performance/robustness, therefore, additional procedures might be validated and published. Reference: ICAO Doc 4444 PANS Air Traffic Management; ICAO Doc 8168 PANS Aircraft operations

; ICAO Doc 9849 Global Navigation Satellite System (GNSS) Manual; ICAO Doc 9365 Manual of All-Weather Operations; ICAO EUR Doc 013 European guidance material on All Weather Operations at Aerodromes and ICAO Doc 8071 Manual on Testing of Radio Navigation Aids.

2025

Airborne system capability

Ground system infrastructur e

Navigation	DFMC GBAS

receiver

Navigation	DFMC GBAS

ground stations

New core constellations (Beidou, Galileo) and signals (GPS, GLONASS) will provide opportunities to enhance satellite based navigation services through GBAS. Dual Frequency multi constellation (DFMC) GBAS receivers. Reference: ICAO Annex 10 Vol I technical requirements; RTCA/EUROCAE MOPS.

New core constellations (Beidou, Galileo) and signals (GPS, GLONASS) will provide opportunities to enhance satellite based navigation services through GBAS. DFMC GBAS ground stations at the airport are required. Reference: ICAO Annex 10 Vol I; provisions supporting system certification/approval by the designated authority; RTCA/EUROCAE MOPS.

2025

Aircraft operator

Aircraft manufacturer

| Airport operator |  |
| --- | --- |
| ANSP |  |
| Ground systems supplier |  |

2025

Aircraft operator

ANSP

NAVS-B2/2	Dual Frequency Multi Constellation (DF MC)

SBAS

Technology

Sixth edition of the GANP

Main Purpose	To increase availability and expand coverage.

To reduce cost through the reduction of the need for ground stations.

New Capabilities	More robust and wider area landing services, including Cat I autoland, possibly Cat II (R&D on- going).

Description	This module builds upon the basic SBAS systems introduced within Block 0.

New core constellations and signals: (e.g. Beidou, Galileo) or dual frequencies such as GPS, GLONASS will enhance satellite based navigation services through SBAS.

DF MC SBAS provide dual frequency and multi constellation additional navigation signals.

Maturity Level	Validation

Human Factor Considerations

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Departure   En-route   Arrival

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Evolution	NAVS-B0/2 - Satellite Based Augmentation Systems (SBAS)

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Navigation	Enhanced SBAS

Aircraft operator

instrument flight procedures

New core constellations (Beidou, Galileo) and signals (GPS, GLONASS) will provide opportunities to enhance satellite based navigation services through SBAS. New signals/constellations are expected to support existing procedures with an increased performance/robustness, therefore, additional procedures might be validated and published. Reference material: ICAO Doc 4444 PANS Air Traffic Management; ICAO Doc 8168 PANS Aircraft operations or United States Standards for Terminal Instrument Procedures (TERPS); ICAO Doc 9849 Global Navigation Satellite System (GNSS) Manual; ICAO Doc 9613

Aircraft operator

Aircraft manufacturer

Performance-based Navigation (PBN) Manual and ICAO Doc 8071 Manual on Testing of Radio Navigation Aids.

2025

Airborne system capability

Navigation	DFMC SBAS

receiver

New core constellations (Beidou, Galileo) and signals (GPS, GLONASS) will provide opportunities to enhance satellite based navigation services through SBAS. Dual Frequency multi constellation (DFMC) SBAS receivers integrated with aircraft navigation systems. Reference: ICAO Annex 10 Vol I technical requirements; RTCA/EUROCAE MOPS.

2025

Ground system infrastructur e

Navigation	DFMC SBAS

ground stations

New core constellations (Beidou, Galileo) and signals (GPS, GLONASS) will provide opportunities to enhance satellite based navigation services through SBAS. Ground stations will provide a wide area monitoring network. No local airport infrastructure is required. However, a network of ground reference systems and a highly reliable data network with processing facilities and satellite signal generation capability is required.

Satellite provider

References: ICAO Annex 10 Vol I; document specific to each SBAS; provisions supporting system certification/approval by the designated authority and performance commitments.

2025

Aircraft manufacturer

ANSP

SBAS service provider

Space system infrastructur e

Navigation	New core

constellations

New satellite constellations (Beidou, Galileo) or dual frequencies such as GPS or GLONASS provide opportunities to enhance satellite based navigation services by increasing performance and robustness to failures.

2019

Space

system infrastructur e

Navigation

GNSS DFMC

augmentation satellites

These satellites broadcast the DFMC

SBAS messages to the different users. Reference: document specific to each SBAS provider

2025

SBAS service provider

NAVS-B2/3	Dual Frequency Multi Constellation (DF MC)

ABAS

Technology

Sixth edition of the GANP

Main Purpose		More robust navigation services (in particular versus loss of a single frequency, or of a single constellation).

New Capabilities		ABAS algorithms use dual frequency and multi constellation additional signals. Only horizontal (2D) enhanced ABAS navigation services are expected within Block 2. Vertical (3D) enhanced ABAS navigation services will be introduced within Block 3.

Description	This element builds upon the basic ABAS systems introduced within Block 0.

New core constellations and signals: (e.g. Beidou, Galileo) or dual frequencies such as GPS, GLONASS will enhance satellite based navigation services through ABAS.

DF MC ABAS will use additional navigation signals and new integrity algorithmes (A-RAIM).

Maturity Level	Validation

Human Factor Considerations

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Departure   En-route   Arrival

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Evolution	NAVS-B0/3 - Aircraft Based Augmentation Systems (ABAS)

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Space system infrastructur e

Navigation	New core

constellations

New satellite constellations (Beidou, Galileo) or dual frequencies such as GPS or GLONASS provide opportunities to enhance satellite based navigation services by increasing performance and robustness to failures.

2019

Satellite provider

Training	Navigation	Training

requirements for enhanced ABAS

In principle no additional training than for NAV-B0/3 will be required. Reference: Pilot training requirements for RNP APCH and procedure designers. Specific training required for RNP AR APCH.

2025

Aircraft operator

ANSP

Operational procedures

Navigation	Enhanced ABAS

instrument flight procedures

New core constellations (Beidou, Galileo) and signals (GPS, GLONASS) will provide opportunities to enhance satellite based navigation services through ABAS. New signals/constellations are expected to support existing procedures with an increased performance/robustness, therefore, additional procedures might be validated and published. Reference: ICAO Doc 4444 PANS Air Traffic Management; ICAO Doc 8168 PANS Aircraft operations or United States Standard for Terminal Instrument Procedures (TERPS); ICAO Doc 9849 Global Navigation Satellite System (GNSS) Manual; ICAO Doc 9613 Performance-based Navigation (PBN) Manual and ICAO Doc 8071 Manual on Testing of Radio Navigation Aids

2025

Airborne system capability

Navigation	DFMC ABAS

receiver

DFMC ABAS receiver integrated with aircraft navigation systems. Reference: ICAO Annex 10 Volume I technical requirements; RTCA/EUROCAE MOPS.

2025

Aircraft operator

ANSP

Aircraft operator

Aircraft manufacturer

NOPS

Sixth edition of the GANP

Main Purpose 		Introduce ASM/ATFM techniques, procedures and tools for the initial establishment of an integrated collaborative airspace management and air traffic flow and capacity management process applicable to the strategic through to the tactical phases of operations.

Operational

Initial integration of collaborative airspace

management with air traffic flow management

NOPS-B0/1

New Capabilities 	 Collaborative airspace planning process is extended by harmonizing the ASM/ATFM rules and procedures for the establishment, allocation and use of airspace structures in response to ATFM requirements.

Description 		This element represents the initial step to enhancing the common situational awareness supporting optimum availability of airspace and ATC capacity to meet air traffic demands. It will result in a dynamic/rolling process supporting the enhancement of network operations. It will improve the cross border operations and optimise network operations based on the richest and more accurate information. It requires the implementation of new tools/systems and processes notably:

ASM/ATFM process for the provision of the airspace use plan;

Improved ASM/ATFM process for the provision of updated airspace use plan;  System/tools for provision of airspace plan to ATM network function;

Improved notification process for the ASM/ATFM purposes;  Improved accuracy of airspace booking;

Interoperability between local ASM and ATFM systems.

Maturity Level 	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

Coordination between different airspace planning actors is more efficient and the need for paper/phone coordination is minimised.

- Does it imply processing of new information by the user? Yes Integrated airspace planning implies an utilisation of new data stream.

- Does it imply the use of new equipment? Yes ASM tool.

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

The manual process of airspace notification is semi-automated.

| Strategical | Pre-tactical |  | Tactical-Pre ops |
| --- | --- | --- | --- |
| Tactical-During ops |  |  |  |

OPERATIONS

Arrival

En-route

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-information need

AMET-B0/1 - Meteorological observations products

Relation-operational need

FRTO-B0/2 - Airspace planning and Flexible Use of Airspace (FUA)

ENABLERS

Enabler

Category

Enabler Type	Enabler Name

Description / References

Stakeholders	Year

Operational procedures

-	Procedures for dynamic co- operative management of the airspace

Develop the ASM/ATFM procedures related to dynamic co-operative management of the airspace (improved ASM/ATFM process via e.g. Airspace Use Plan/Updated airspace Use Plan). Reference: ICAO Doc 9971 Manual on Collaborative ATFM.

2013

ATM network function

ANSP

Operational procedures

-	Procedures for improved notification process

Improved ASM/ATFM notification process. References: ICAO Doc 9971 Manual on Collaborative ATFM

2013

ATM network function

ANSP

Operational procedures

-	Procedures for released of reserved airspace

Develop the ASM/ATFM procedures to identify and release previously reserved airspace. References: Doc 9971 Manual on Collaborative ATFM

2013

ATM network function

ANSP

Operational procedures

-	Procedures for promulgation and notification of receipt

Develop the ASM/ATFM procedure for promulgation and notification of receipt of ASM data- Doc 9971 Manual on Collaborative ATFM

2013

ATM network function

ANSP

Ground system infrastructur e

ATM systems Distribution of

planned airspace usage information

Enhance the Airspace Management System and ATFM systems to distribute planned airspace usage information.

2013

ATM network function

ANSP

ATM network function

ANSP

Ground system infrastructure

ATM systems Integrated

airspace planning process

Upgrade the Airspace Management System and ATFM system to support an integrated airspace planning process

2013

ATM network function

ANSP

Ground system infrastructur e

ATM systems Pre-tactical

scenario management

Enhancements of Scenario management sub-system equipped with function to support pre-tactical CDM

2013

ANSP

Ground system infrastructur e

ATM systems Airspace status

information

Upgrade the Airspace Management Systems to provide airspace status information

2013

Aircraft operator

Ground system infrastructur e

ATM systems ATFM systems

Aircraft operator

interoperability with ASM system

Interoperability of ATFM systems for ASM zone shapes and timing with local ASM tools

2013

ATM network function

ATM network function

ANSP

Ground system infrastructure

ATM systems Reception of

planned and actual airspace status

Enhance to ASM/ATFCM tools to receive information on planned and actual airspace status and support decision- making based on this information

2013

DEPLOYMENT APPLICABILITY

Operational conditions:

This element should be undertaken by all ANSPs, AUs and the ATM Network function affecting both en-route and TMA operations.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Training

-

Training

requirements for initial integration of collaborative ASM with ATFM

Collaborative Airspace management

training. Training on new procedures and tools.

2013

Aircraft operator

ATM network function

ANSP

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Capacity. Flight delays will be reduced through better utilization of airspace resources within and across airspace boundaries. | ANSP Airspace user ATM network function |
|  | Flight efficiency increased through the chance to plan more optimum routes/trajectories allowing lower fuel burn. | Airspace user |
|  | Improve situational awareness of network manager | ATM network function |
| Indirect benefits | Safety. Better knowledge of traffic environment, common situational awareness, and some enhancement through reduction in controller workload. | ANSP Airspace user ATM network function |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Flight time & distance | Facilitate tactical decisions leading to a shorter actual route than in the FPL | ++ | KPI05: Actual en- route extension |
| Efficiency | Flight time & distance | Overcome route selection inefficiencies associated with route & airspace availability as known at the flight planning stage | ++ | KPI04: Filed flight plan en-route extension |
| Efficiency | Flight time & distance | Reduce need for tactical ATFM rerouting to circumnavigate airspace closed at short notice | ++ | KPI05: Actual en- route extension |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Vertical flight efficiency | Reduce altitude restrictions during climb introduced to avoid airspace above | ++ | KPI17: Level-off during climb |
| Efficiency | Vertical flight efficiency | Reduce altitude restrictions during cruise introduced to avoid airspace above | ++ | KPI18: Level capping during cruise |
| Efficiency | Vertical flight efficiency | Reduce altitude restrictions during descent to avoid Special Use Airspace | ++ | KPI19: Level-off during descent |

NOPS-B0/2	Collaborative Network Flight Updates	Operational

Sixth edition of the GANP

Main Purpose	Improve ATFM situation awareness in order to facilitate re-routings and coordinated application of ATFM measures.

New Capabilities		Seamless exchange and processing of correlated position information, flight activation status and up to date flight plan information for airborne flights. Such data are required within the Area of Responsibility (AOR) of the ATFM unit, but also within the Area of Interest (AOI) of the ATFM unit for all flights entering the ATFM area.

Description	This element will ensure:

Effective interface between ATC and ATFM with regard to deviations from the current flight plan. Enhanced tactical flow management service based on real-time aircraft position data and flight activation information resulting to more accurate ATFM measures and thus better use of scarce airspace resources.

It will require implementation ATFM/ATC systems related to provision, processing and presentation of ATFM messages.

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes Manual notification disappeared.

- Does it imply processing of new information by the user? Yes

ATFM message, CPRs and flight plan proposals are new items that were not previously exchanged.

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   En-route   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-information need

AMET-B0/2 - Meteorological forecast and warning products

### ENABLERS

ATM network function

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

-	Network Planning procedures

Develop the ATFM procedures to incorporate information received from multiple sources into the Network Planning.- Doc 9971 Manual on Collaborative ATFM

2008

Operational procedures

-	Procedures for updated flight plan information

Develop the ATFM/ATC procedures for provision of updated flight plan information.- Doc 9971 Manual on Collaborative ATFM

2008

ATM network function

ANSP

Ground system infrastructure

ATM systems Correlated

ATM network function

ANSP

Position Reports

Upgrade of ATFM/ATC system related to the provision and reception of correlated position reports for airborne flights.

2008

ATM network function

ANSP

Ground system infrastructur e

ATM systems ATFM message

ATM network function

ANSP

exchanges

Enhancement of ATFM/ATC system related to the provision and processing of ATFM messages.

2008

ATM network function

ANSP

Ground system infrastructur e

ATM systems Flight activation

messages

Upgrade of ATFM/ATC system related to the flight activation.

2008

Ground system infrastructure

ATM systems Updated flight

plan info

Upgrade the ATFM/ATC system for handling of flight plan info for airborne flights.

2008

DEPLOYMENT APPLICABILITY

Operational conditions:

This element will involve all ANSPs, AUs and the ATM Network Function for the collaborative updates of the flight status within an ATFM area. This will enhance predictability and better utilisation of available capacity.

Main intended benefits:

Benefitting stakeholder(s)

Operational description

Type

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Capacity. Better use of the available network capacity hence reducing delays. | ANSP ATM network function |
|  | Predictability | ANSP Airspace user ATM network function |
|  | Improve situational awareness of network manager. | ATM network function |
| Indirect benefits | Safety. Prevention of ATCO overload. | ANSP |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | Ensure that the measures applied are absolutely necessary and that unnecessary measures are avoided | ++ |  |
| Capacity | Capacity shortfall & associated delay | Establish/improve the capability to use opportunities to mitigate disturbances, originating from: More precise surveillance data | ++ |  |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

NOPS-B0/3	Network Operation Planning basic features	Operational

Sixth edition of the GANP

Main Purpose	The Network Operation Planning provides an overview of the situation from strategic planning through real time operations with ever increasing accuracy up to and including the day of operations by a common situational awareness for all ATFM actors within and adjacent to the ATFM area and allowing network wide demand and capacity balancing.

New Capabilities		A Network Operations Plan will be accessible online by stakeholders for consultation and update as needed.

Description	Network Operation Planning is based on enhanced participation in a dynamically updated collaborative planning process. This requires the sharing of the latest flight status and intentions; airport and airspace component, associated demand and capacity balancing measures in a frequently updated plan which is aimed to be realised as target by all actors. The elements and formats of the plan need to be established and harmonized, taking into account the requirements of the users of these plans. It will be possible for them to access and extract data for selected areas to support their operation and, if required, to create their specific operations plan.

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

PLANNING LAYERS

Does it imply a change to levels of automation? Yes

| Strategical | Pre-tactical |  | Tactical-Pre ops |
| --- | --- | --- | --- |
| Tactical-During ops |  |  |  |

OPERATIONS

Arrival

En-route

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-information need

AMET-B0/2 - Meteorological forecast and warning products

Relation-information need

AMET-B0/3 - Climatological and historical meteorological products

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational -

procedures

Tactical changes

procedures

Develop the ATFM procedures to modify

the Network operations planning in real- time in response to tactical changes to trajectories and airport/airspace capacities..- References: ICAO Doc 9971 Manual on Collaborative ATFM

2012

Operational -

procedures

Capacity

balancing procedures

Develop the ATFM procedures for

systematically incorporating changes to capacity balance as revised information enables the updating of the Network operations plan. References: ICAO Doc 9971 Manual on Collaborative ATFM

2012

Operational -

procedures

Coordination

procedures

Develop the ATFM procedures for

coordinating refined plans between ANSP, Airspace Users and Airport Operators.

2012

Operational -

procedures

Network

Operation Planning procedures

Develop the ATFM procedures for on-line

collaborative determination access/update to the Network Operation Planning and notification of updates.

2013

Aircraft operator

ATM network function

Aircraft operator

ATM network function

Aircraft operator

ATM network function

ANSP

ANSP

ANSP

Airport operator

Airport operator

Airport operator

ATM network function

Operational -

procedures

Predefined

scenario management

Develop the ATFM procedures for

identifying the appropriate scenario from the catalogue of scenarios through collaborative decision making and initiating its implementation. References: ICAO Doc 9971 Manual on Collaborative ATFM

2013

Operational -

procedures

Dynamic

sectorization procedures

Develop the ATFM procedures for

initiating dynamic sectorization responses in collaboration with the ANSPs.

References: ICAO Doc 9971 Manual on

Collaborative ATFM

2013

Ground

system infrastructur e

ATM systems

Exchange of

ATFM related data

Tool for exchange, presentation,

visualization and update of ATFM related data.

2013

Ground

system infrastructure

ATM systems

Capacity

planning

Upgrade the capacity planning and

scenario management with sector management tool to assist ANSPs in defining sector configurations.

2013

Training

-

Training

requirements for network operation planning basic features

Network Operation Planning (basic

features) training. Training on new procedures and tools

2013

ATM network function

ATM network function

ATM network function

DEPLOYMENT APPLICABILITY

Operational conditions:

The Network Operation Planning will involve all the operational stakeholders providing an overview of the situation from strategic planning to real time operations with ever increasing accuracy optimising the efficiency of the ATM system while balancing demand with capacity.

Main intended benefits:

| Airport operator | ANSP |  | Airspace user |
| --- | --- | --- | --- |
| ATM network function |  |  |  |

| Airport operator | ANSP |  | Airspace user |
| --- | --- | --- | --- |
| ATM network function |  |  |  |

| Airport operator | ANSP |  | Airspace user |
| --- | --- | --- | --- |
| ATM network function |  |  |  |

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |

| Type Operational description Benefitting stakeholder(s) |  |  |
| --- | --- | --- |
| Direct benefits | Capacity. |  |
|  | Predictability. |  |
|  | Improve situational awareness of stakeholders. |  |

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Indirect benefits | Safety by planning scarce resources in the most optimum way. | ANSP |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

NOPS-B0/4	Initial Airport/ATFM slots and A-CDM Network

Interface

Operational

Sixth edition of the GANP

Main Purpose	Initial integration of airports into the ATM network function.

New Capabilities		Stakeholders will be able to share relevant airport and flight turnaround information with ATM network function resulting in better predictability and better use of existing capacity whilst considering user preferences and requirements.

Description		The first objective is the A-CDM (Airport Collaborative Decision Making) integration with ATFM via exchanges of specific messages. The second objective is to ensure ATFM slot adherence and limited ATFM slot swapping in order to meet airline demands in line with capacity declarations.

Convergence is ensured between airport slots, and flight plans, together with airport slot monitoring processes in order to improve consistency. That will require the deployment of new systems and processes for A-CDM and ATFM slot swapping:

ATFM and airports system modules related to data exchanges for A-CDM Tools for airport and ATFM slot monitoring post-ops

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes Phone coordination is reduced.

- Does it imply processing of new information by the user? Yes

ATFM data presentation and scenario management are new data streams.

- Does it imply the use of new equipment? Yes

Network Operation planning is a new tool, it might require specialised equipment for data access.

- Does it imply a change to levels of automation? Yes Manual process is semi-automated.

### PLANNING LAYERS 		OPERATIONS

Tactical-During ops

Tactical-Pre ops

Turn-around

Arrival

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

ACDM-B0/1 - Airport CDM Information Sharing (ACIS)

Relation-operational need

ACDM-B0/2 - Integration with ATM Network function

### ENABLERS

ATM network function

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

-	Airport slot

procedure

Develop procedures for the collection of the Airport slots. References: ICAO Doc 9971 Manual on Collaborative ATFM

2013

ATM network function

Aircraft operator

Operational procedures

-	ATFM A-CDM

procedure

Develop the ATFM Collaborative procedures for improving Airport Operations in Adverse Conditions. References: ICAO Doc 9971 Manual on Collaborative ATFM

2012

Ground system infrastructur e

ATM systems Airport Slot

Aircraft operator

Aircraft operator

ANSP

Airport operator

Monitoring Tool

Tools supporting Airport Slot Monitoring post ops.

2012

Ground system infrastructur e

ATM systems Departure

ATM network function

Airport operator

planning estimate

Enhance the ATFM system for provision of planned departure information. Enhance the ATC and airport systems for reception and processing of planned departure information.

Airport operator

ANSP

ATM network function

2010

Ground system infrastructure

ATM systems Flight data for

airborne flights

Enhance the ATFM system for provision of real time flight data for airborne flights.

Enhance the ATC and airport systems for reception and processing of real time flight data for airborne flights

Airport operator

ANSP

ATM network function

2010

DEPLOYMENT APPLICABILITY

Operational conditions:

It concerns airports which have implemented A-CDM. The integration of airport planning with ATFM will involve all respective stakeholders in a collaborative decision facilitating slot adherence and some AUs preferences (limited slot swapping).

Main intended benefits:

Benefitting stakeholder(s)

Operational description

Type

| Airport operator | ANSP |  | Airspace user |
| --- | --- | --- | --- |
| ATM network function |  |  |  |

| Airport operator | ANSP |  | Airspace user |
| --- | --- | --- | --- |
| ATM network function |  |  |  |

| Type Operational description Benefitting stakeholder(s) |  |  |
| --- | --- | --- |
| Direct benefits | Capacity. |  |
|  | Predictability. |  |
|  | Improve situational awareness of network ATM network function manager. |  |
| Airspace users specific preferences Indirect benefits Airspace user /efficiency criteria of operations. |  |  |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | For a given airspace entry slot: let airspace users swap the slot to another flight (slot substitution or UDPP – User Driven Prioritisation Process) | ++ |  |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

NOPS-B0/5	Dynamic ATFM slot allocation	Operational

Sixth edition of the GANP

Main Purpose	Provision of dynamic departure ATFM slot allocation including Calculated Take-off Time (CTOT) for regulated flights to avoid ATFM congestions.

New Capabilities		ATM network function to provide the departure ATFM slots, including CTOT for regulated flight to all concerned operational stakeholders. ANSPs/ Airport/ AU to be capable to receive and process CTOT and update Estimated Take-off Time (EOBT) in accordance with the agreed operational procedures.

Description		The CTOT is defined as a time at which the aircraft shall take-off. CTOT is sent to AU / ATS when a flight becomes regulated (e.g. new flight entering the system, new period of regulation in the system, change of runway in use) at a system parameter time before the last received EOBT. AU/ATS/Airport need to adhere with the CTOT. The calculation of take-off times takes into account the off-block times and an average taxiing time for the runway in use at the airfield concerned.

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes Constrains need to be handled.

- Does it imply processing of new information by the user? Yes CTOT and DPI are new items.

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes

Instead being active user, only monitoring of data exchange and reacting in abnormal situations.

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational -

procedures

Slot revision

procedures

Develop the ATFM procedures for slot

revision. References: ICAO Doc 9971 Manual on Collaborative ATFM

2000

Ground

system infrastructure

ATM systems

CTOT

System upgrade for provision, exchange

and processing of CTOT

2000

DEPLOYMENT APPLICABILITY

Operational conditions:

It concerns airports integrated in ATFM. All operational stakeholders participate in smoothing traffic flows facilitating ATFM slot adherence.

Main intended benefits:

PLANNING LAYERS

Tactical-Pre ops

OPERATIONS

Departure

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
|  | Predictability. | Airport operator ANSP Airspace user ATM network function |
|  | Fuel consumption. | Airspace user |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| Airport operator | ANSP |  | Airspace user |
| --- | --- | --- | --- |
| ATM network function |  |  |  |

| Direct benefits Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
|  | ATFM delays. |  |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | Implement TMIs to delay take-off times | ++ | KPI07: En-route ATFM delay |
| Capacity | Capacity shortfall & associated delay | Use ATFM oriented flow management: delay push-back of inbound traffic | ++ |  |

Sixth edition of the GANP

Main Purpose

Short Term ATFM Measures (STAM) intends to smooth sector workloads by reducing traffic peaks

through short-term applications of minor ground delays, appropriate flight level capping, timing and modalities of ATC re-sectorisation These measures are capable of reducing the traffic complexity for ATC with minimum curtailing impact on the airspace users.

New Capabilities	Stakeholders can optimize capacity throughput by adopting and improving the tactical capacity

management procedures with the use of STAM.

Operational

Short Term ATFM measures

NOPS-B1/1

Description 		The rigid application of ATFM measures based on standard capacity thresholds as the pre- dominant tactical capacity measure needs to be replaced by a close working relationship between ANSP, AU and ATM Network function, which monitors both the real demand and the effective capacity of sectors having taken into account the complexity of expected traffic situation.

In order to close the gap between ATC and ATFM, new tools and local operational procedures need to be developed. The aim is to improve the efficiency of the system using flow management techniques close to the real time operations with direct impact on tactical capacity management and tactical action on traffic.

The target of the Short Term ATFM Measures is to replace en-route measures for situations where the capacity is nominal. These measures are capable of reducing the traffic complexity for ATC with minimum constraints for the airspace users. STAM tools and procedures are based on accurate short-term occupancy counts. The tactical capacity management procedures can be supported by the ATFM Tools (system based STAM with the hot-spot detections in the network view, the “what-if” function and capabilities of promulgation and implementation of STAM measures, including CDM). This will require the introduction of:

Pre-tactical and Tactical Demand Capacity Balancing (DCB) evaluation tools;  DCB tool based on occupancy counts;

Enhanced monitoring techniques;  DCB Coordination tools;

DCB What-if function;

DCB Network impact assessment;

ATFM procedures to enable application of flow management closer to real time.

Maturity Level 	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes New task for all ATFM actors.

- Does it imply processing of new information by the user? Yes STAM measures are new items.

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes Reduced need for late and tactical interventions.

PLANNING LAYERS

Tactical-Pre ops

OPERATIONS

En-route   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

NOPS-B0/3 - Network Operation Planning basic features

ENABLERS

Enabler

Category

Enabler Type	Enabler Name

Description / References

Stakeholders	Year

Aircraft operator

Operational procedures

-	DCB change

procedure

Develop the ATFM procedures to respond to change of demand/ capacity balance. References: ICAO Doc 9971 Manual on Collaborative ATFM

2014

ATM network function

ANSP

Operational procedures

-	DCB

optimisation procedure

Develop the ATFM procedures for demand/ capacity balance optimization. References: ICAO Doc 9971 Manual on Collaborative ATFM

2014

Aircraft operator

ATM network function

Operational procedures

-	STAM

procedures

Develop the ATFM procedures to enable application of flow management techniques on traffic streams closer to real-time. References: ICAO Doc 9971 Manual on Collaborative ATFM

2019

ATM network function

ANSP

Ground system infrastructure

ATM systems Strategic DCB

ATM network function

ANSP

tools

Strategic and pre-tactical demand- capacity balancing evaluation, simulation and display tools.

2014

Aircraft operator

Ground system infrastructure

ATM systems Routings and

ATM network function

Flow changes

Upgrade the Capacity planning and scenario management with tools to identify the most beneficial routings and flows changes implemented within the Collaborative Decision Making processes.

2014

Ground system infrastructur e

ATM systems Basic STAM tool	Demand capacity balancing tool base on

occupancy counts.

ANSP

ATM network function

2014

ATM network function

Ground system infrastructure

ATM systems Sector

configuration integration

Integration of ANSPs sector configuration into ATFM Systems

ANSP

ATM network function

2014

Ground system infrastructur e

ATM systems Enhanced STAM tool

Enhanced STAM tool (Coordination, what- if, network impact assessment).

2019

ATM network function

Ground system infrastructur e

ATM systems Local STAM tool	Local tool and interface with ATFM tools	 ANSP 	2019

Aircraft operator

Training	-	Training requirements for STAM

Staff training	 ANSP

2014

DEPLOYMENT APPLICABILITY

#### Operational conditions:

It is applicable in areas where significant ATFM measures are required. This element will involve ANSPs as well AUs and the ATM Network function as required in a collaborative manner, during tactical planning in applying demand and capacity balancing actions for a limited time and affecting selected flights.

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Capacity. Effective capacity is globally optimized thanks to the application of certain techniques affecting only selected flights. | ANSP Airspace user ATM network function |
| Indirect benefits | Safety. Small enhancement through the resolution of some conflicts through STAM measures, Predictability | ANSP Airspace user ATM network function |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | Establish/improve the capability to use opportunities to mitigate disturbances | ++ |  |
| Capacity | Capacity shortfall & associated delay | TMI-based optimisation (only impacts traffic when a TMI or restriction is manually activated for one or more constraint satisfaction points) | ++ |  |

Sixth edition of the GANP

Main Purpose 			Collaborative Trajectory Options Programs are Traffic Management Initiatives (TMI) that allow ATFM to choose the best possible balance between ATFM dalay and rerouting by using airspace user provided Trajectory Option Sets (TOS) to mitigate the operational impact of weather or traffic demand airspace constraints.

Collaborative Trajectory Options Program (CTOP) Operational

NOPS-B1/10

New Capabilities 	 ATFM has the capability to receive and process Trajectory Option Sets (TOS) provided by airspace users. These are ranked trajectories that represent the operator’s preference for the trade-off between receiving ATFM delay and routing around airspace constraints.

When there is an airspace constraint, ATFM has the flexibility to use the trajectory options provided by all participating operators to optimize the choice between accepting a subset of the flights to use the available airspace capacity, applying ATFM delay to others, and rerouting the remaining traffic around the constraint.

Finally, ATFM has the capability to electronically notify the participating airspace users of the chosen trajectory that they are expected to fly.

Description 	CTOP works as follows:

- ATFM creates an airspace boundary and establishes flow control on any air traffic that crosses that boundary.

- Airspace Users based on the notice of the airspace constraint develop and submit in advance of the issuance of the program, a set of desired reroute options (called a Trajectory Options Set or TOS) that is the operator’s preference for routing around the constraint.

- CTOP uses the preferred options to automatically assign delays or reroutes to flights in order to dynamically manage the demand as conditions change.

Maturity Level 	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-Pre ops

Pre-tactical

OPERATIONS

En-route

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

FRTO-B1/7 - Trajectory Options Set (TOS)

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Regulatory	Advisory

provisions	circular

CTOP advisory

circular

Guidance to customers how to manage	 CAA

flights in the constrained area Reference: FAA AC 90-115

2019

Operational Operations

procedures

Operational

procedures for the use of CTOP

Operational guidelines on CTOP usage	 ANSP

2019

Aircraft operator

Aircraft operator

Ground system infrastructur e

ATM systems Tools and system

to support CTOP

Tools and systems in place at ANSP to support CTOP operations Reference: https://cdm.fly.faa.gov/?page_id=983

ANSP 	2019

Aircraft operator

Ground system infrastructure

CFSP

systems

CSFP to support CTOP

Tools and systems in place at AOs to support CTOP operations Reference: https://cdm.fly.faa.gov/?page_id=983

2019

Training	-	Training requirements for CTOP

ATM Training, Flight Dispatcher training Training is provided on how to use CTOP system to increase flight operation efficiency

ANSP

2019

### DEPLOYMENT APPLICABILITY

Benefitting stakeholder(s)

Operational description

Type

Operational conditions: Main intended benefits:

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | For a given flight: at flight plan filing time airspace users provide network management with a range of trajectory options and associated trade-off criteria, from which one solution is chosen (CTOP – Collaborative Trajectory Options Program) | ++ | KPI04: Filed flight plan en-route extension KPI07: En-route ATFM delay KPI18: Level capping during cruise |

Sixth edition of the GANP

Main Purpose

The Network Operations Planning needs to be enhanced to achieve collaborative planning with the

support of services which can be automated (B2B interfaces/SWIM services).

New Capabilities

Tools and procedures to be deployed to enhance Network Operations planning.

Operational

Enhanced Network Operations Planning

NOPS-B1/2

Description 		The Network Operations Planning process will be enhanced to continuously provide up-to-date situational information on all components of the network. Furthermore, it will provide access to initial network performance objectives and support to network performance assessment in post- operations.

The required technological platform will use the state-of-the-art technologies for creation of a virtual operations room for the physically distributed network operations, in support of collaborative Network Operations Planning. These interfaces will support the network collaborative dynamic/rolling processes from strategic to real-time operations, including capabilities for online performance monitoring integrated and feeding back into the collaborative network planning.

The information and dialogue tools shall be accessed via different interfaces. Access to information is done in a secure way, tailored according to stakeholders needs and subject to access control rules, so that only those who have an operational need to access particular information are able to do so. A common interface to all stakeholders needs to be developed to enable the collaborative decision-making processes used to build and execute the Network Operations Planning.

The following new features will be introduced:

Enhanced Network Operations Planning interfaces (B2B/SWIM based);

Initial steps related to the Network Operations Planning extended functions (crisis management and network disruption);

Tools for on-line performance monitoring;  Tools for network impact assessments.

Maturity Level 	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes New role and responsibilities for some ATFM actor.

- Does it imply processing of new information by the user? Yes

New info stream are handled as impact assessment and crisis management.

- Does it imply the use of new equipment? Yes New platform and interfaces are needed.

- Does it imply a change to levels of automation? Yes New function were added , those that did not exist before.

PLANNING LAYERS

Tactical-Pre ops

Pre-tactical

Strategical

OPERATIONS

Arrival

En-route

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

NOPS-B0/3 - Network Operation Planning basic features

Relation-information need

AMET-B1/3 - Climatological and historical meteorological information

Relation-information benefit

FICE-B2/4 - Flight Data Request Service

Relation-operational benefit

SWIM-B2/1 - Information service provision

Relation-operational benefit

SWIM-B2/2 - Information service consumption

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Operational procedures

Operational procedures

Operational procedures

- Enhanced NOP dissemination procedure

- Enhanced NOP notification procedure

- Critical event

procedure

- Airspace

availability procedure

Develop the ATFM procedures for communicating refined Network plans to ANSP, Airspace Users and Airport Operators...- Doc 9971 Manual on Collaborative ATFM

Develop the ATFM procedures for on-line access/update to the Network Plan and notification of updates. References: ICAO Doc 9971 Manual on Collaborative ATFM

Develop the ATFM procedures for handling of a critical event.

Aircraft operator

Develop the ATFM procedures to ensure that Network is constantly updated to reflect all changes to the airspace availability and airspace users requests. References: ICAO Doc 9971 Manual on Collaborative ATFM

2019

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |

2019

ATM network function

2019

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |

2019

Operational procedures

- B2B/SWIM

services procedure

Real-time technical support procedures for B2B(2019)/SWIM(2020) services. References: ICAO Doc 9971 Manual on Collaborative ATFM

2019

Ground system infrastructur e

- Enhanced NOP platform

Enhance the ATFM technical platform.

2019

Ground system infrastructure

Ground system infrastructure

ATM systems Enhanced NOP

ATM network function

functions

ATM systems Enhanced NOP

interfaces

Upgrade the ATFM system with extended function (crisis management, impact assessment, performance monitoring).

B2B (2019)/ SWIM (2020) Network

system interfaces with concerned stakeholders.

2019

ATM network function

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |

2019

Training	-	Training requirements for enhanced network operations planning

Training in new operational procedures and ground systems.

2019

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |

DEPLOYMENT APPLICABILITY

#### Operational conditions:

An enhanced common platform is available to all Stakeholders needs enabling the collaborative decision making processes used to maintain and execute the Network Operations Planning.

#### Main intended benefits:

Type

Operational description

Benefitting stakeholder(s)

Direct benefits

| Airport operator | ANSP |  | Airspace user |
| --- | --- | --- | --- |
| ATM network function |  |  |  |

Capacity Enhanced planning will ensure better use of available capacity and reduce delays.

| Airport operator | ANSP |  | Airspace user |
| --- | --- | --- | --- |
| ATM network function |  |  |  |

Cost Efficiency Enhanced planning will allow better use of human resources.

ATM network function

| Airport operator | ANSP |  | Airspace user |
| --- | --- | --- | --- |
| ATM network function |  |  |  |

Improve situational awareness of network manager

Indirect benefits

Safety Enhanced by improved sharing of the network situation awareness.

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | Establish/update the crisis management capabilities and plans (to cope with the risk of large scale disruptions) | ++ |  |

Sixth edition of the GANP

Main Purpose 		Integrate the airport operations planning into the network operations planning.

Operational

Enhanced integration of Airport operations

planning with network operations planning

NOPS-B1/3

New Capabilities 	 The airport operations plan will be a single common and collaborative agreed dynamic/rolling plan available to all airport stakeholders. This plan will be integrated with the enhanced network operations planning.

Description 		The airport operations plan will contain all data and information related to the different status of planning phases and will be a dynamic/rolling plan, which naturally evolves over time. The integration of airport operations planning within the network operations planning provides a dynamic/rolling picture of the network situation to be used by all operational stakeholders to prepare their plans and their inputs to the network CDM processes.

The data exchanges are based on the subset of B2B/SWIM services that are most widely available to all stakeholders, communicating with local airport A-CDM systems to exchange relevant operational information.

This element is a step prior to the full integration of the airport operations planning to the network operations planning.

Maturity Level 	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes Yes, manual coordination is automated.

- Does it imply processing of new information by the user? Yes AOP/NOP interfaces contain a full set of new data exchange items.

- Does it imply the use of new equipment? Yes In some cases. New modules and interfaces.

PLANNING LAYERS

OPERATIONS

Does it imply a change to levels of automation? Yes Reduced need for phone coordination.

| ATM planning | Strategical |  | Pre-tactical |
| --- | --- | --- | --- |
| Tactical-Pre ops |  |  |  |

| Taxi-out | Departure | Arrival | Taxi-in | Turn-around |
| --- | --- | --- | --- | --- |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

NOPS-B0/4 - Initial Airport/ATFM slots and A-CDM Network Interface

Relation-operational need

ACDM-B2/1 - Airport Operations Plan (AOP)

Relation-operational need

NOPS-B1/2 - Enhanced Network Operations Planning

Relation-operational benefit

SWIM-B2/1 - Information service provision

Relation-operational benefit

SWIM-B2/2 - Information service consumption

ENABLERS

ATM network function

Airport operator

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

- AOP/NOP

procedure

Develop the ATFM/Airport procedures for AOP/Network Operation Planning integration. References: ICAO Doc 9971 Manual on Collaborative ATFM

2019

Ground system infrastructure

Airport systems

AOP	Develop the Airport Operations Plan module

Airport operator 2019

Ground system infrastructure

ATM systems AOP/NOP

interface

Develop the Airport Operations/ /Network Operations Planning interfaces

Airport operator 2019

ATM network function

### DEPLOYMENT APPLICABILITY

#### Operational conditions:

It concerns airports which have already implemented AOP. This element will involve Airports and the ATM Network function. Airport planning will be integrated in the Network operations planning. The integration of Airport operations planning within the Network operations planning.

| Airport operator | ANSP |  | Airspace user |
| --- | --- | --- | --- |
| ATM network function |  |  |  |

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Capacity. An integrated planning will facilitate better utilization of available airspace and airport capacities and will reduce delays. Support optimal use of facilities and services, better use of airport and ATFM slots. Predictability Efficiency Decrease in fuel burn through better timed operations. |  |
|  | Improve situational awareness of stakeholders |  |

| Airport operator | ANSP |  | Airspace user |
| --- | --- | --- | --- |
| ATM network function |  |  |  |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

Sixth edition of the GANP

Main Purpose 		Enhanced traffic complexity assessment by automation.

Operational

Dynamic Traffic Complexity Management

NOPS-B1/4

New Capabilities 	 The predicted complexity coupled with traffic demand enables the ATM Network function to take timely action to better address demand and capacity balancing, or request the trajectory changes in coordination with ATC and Airspace Users.

Description 	The rigid application of ATFM measures based on standard capacity thresholds as the pre-dominant tactical capacity measure needs to be replaced by a close working relationship between ANSPs and ATM Network function, which would monitor both the real demand and the effective capacity of sectors having taken into account the complexity of expected traffic situation. The local traffic complexity assessment continuously monitors sector demand and evaluate traffic complexity (by applying predefined complexity metrics) according to a predetermined qualitative scale. It provides support in the determination of solutions in order to plan airspace, sectors and staff to handle the predicted traffic. The local complexity assessment would benefit by receiving processing and integrating the ATM Network function information in order to supplement the local traffic counts with the relevant flight plan data. This will improve the quality of the planned trajectory and further enhance the traffic complexity management.

Maturity Level 	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes New task to manage traffic complexity.

- Does it imply processing of new information by the user? Yes New stream of data.

- Does it imply the use of new equipment? Yes

It could be module of existing system or separate system.

- Does it imply a change to levels of automation? Yes

Mental activates are automated, complexity tool provides additional data to facilitate the traffic de- confliction well in advance.

PLANNING LAYERS

Tactical-During ops

OPERATIONS

En-route

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational -

procedures

Complexity

management procedure

Complexity management procedures.

References: ICAO Doc 9971 Manual on Collaborative ATFM.

ANSP

ATM network function

2018

Ground

system infrastructure

ATM systems

Local Traffic load	Local Traffic Load Management tool.

ANSP

2014

ATM network function

Ground system infrastructur e

ATM systems Local Traffic

complexity

Local Traffic Complexity tools	 ANSP 	2016

ATM network function

Ground system infrastructur e

ATM systems ATFM Planned

Trajectory (basic feature)

Provision and integration of ATFM Planned Trajectory.

ANSP

2016

ATM network function

Ground system infrastructur e

ATM systems ATFM Planned

Trajectory ( enhanced feature)

Upgrade the ATFM systems with the Planned Trajectory improvements

2018

Ground system infrastructur e

ATM systems Network traffic

complexity

Enhancements related to ATFM Traffic complexity assessment.

2019

### DEPLOYMENT APPLICABILITY

#### Operational conditions:

It is applicable only in areas with dense and complex traffic. This element needs to be addressed by the ANSPs and the ATM Network function in their endeavour to find optimum solutions to accommodate the traffic demand.

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Capacity | ANSP ATM network function |
|  | Cost-Efficiency | ANSP |
|  | Improve situational awareness of network manager | ATM network function |
| Indirect benefits | Safety | ANSP |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Overcome operational ATC service delivery limitations if these are the blocking factor | ++ | KPI06: En-route airspace capacity |

NOPS-B1/5	Full integration of airspace management with air traffic flow management

Operational

Sixth edition of the GANP

Main Purpose	Ensure a continuous, seamless and iterative airspace management and air traffic flow management approach.

New Capabilities	Fully dynamic/rolling ASM/ ATFM integration process supporting information collection, processing and sharing with all concerned stakeholders.

Description		The full dynamic/rolling ASM/ATFM process focuses on improving airspace planning. It will ensure a continuous, seamless and iterative airspace planning and management/allocation based on airspace requests at any time period within strategic, pre-tactical and tactical ASM levels. It will result in a rolling process, supporting the enhancement of dynamic Network Operations Planning.

The real time ASM data exchanges relates to the automated exchange services of ASM data during the tactical phase continuously in real time. ASM information (real-time Airspace Reservation status) is shared between different systems and Stakeholders and communicated to ATFM in the tactical phase.

Several new improvements are introduced as:

Process/system modules supporting a full rolling ASM/ATFM and dynamic ASM/ATFM process allowing data sharing to all operational stakeholders,

Process/System changes for initial Collaborative Decision Making (CDM) between ATFM function and the local designated authorities and between neighboring ASM actors.

ASM information sharing addresses requirements aiming to improve the notification to airspace users based on automated data exchange processes

Procedural and system modules for exchange of real time airspace status data;

The Flexible Use of Airspace (FUA) process is improved with more dynamic airspace management enabling dynamic responses to airspace requests (or very short term changes) Real-time ASM coordination is further enhanced through "what-if" functionalities and automated support to airspace booking and airspace management.

Real-time ASM data are exchanged between ASM support systems and ATC system

Integration and management of ASM real-time data into ANSPs’ ATM systems and into AUs flight planning systems;

The full dynamic/rolling ASM/ATFM process will be supported by the sharing of civil-military airspace data and by continuously updating Airspace Reservation information with other demand information among the authorized operational stakeholders in order to enhance the coordination of Cross-Border Operations, and to optimise the whole network operations based on the most timely and correct information. The process is enhanced with “what-if” functionalities enabling a better use of available capacity. ASM real-time data exchanges consisting of pre-notification of activation, notification of activation, de-activation, modification and release of airspace are collected, saved and processed in order to be exchanged between stakeholders and be made available to ATM actors, including Airspace Users (AUs). ATM systems need to be upgraded to exchange real-time ASM data messages containing real time activation status of predefined airspace structures with local ASM support systems and to display airspace status data at the Controller Working Position (CWP).

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes Real time ASM data is new data stream.

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes Automation increased compared with B0.

PLANNING LAYERS

Tactical-Pre ops

Pre-tactical

Strategical

OPERATIONS

Arrival

En-route

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

NOPS-B0/1 - Initial integration of collaborative airspace management with

air traffic flow management

Relation-operational need

FRTO-B1/1 - Free Route Airspace (FRA)

Relation-operational need

FRTO-B1/2 - Required Navigation Performance (RNP) routes

Relation-operational need

FRTO-B1/3 - Advanced Flexible Use of Airspace (FUA) and management

of real time airspace data

Relation-operational need

FRTO-B1/4 - Dynamic sectorization

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

-	Rolling

ASM/ATFM

procedure

Develop the Procedures and processes for a dynamic/rolling ASM/ATFM process. References: ICAO Doc 9971 Manual on Collaborative ATFM

ANSP

ATM network function

ATM network function

2019

Operational procedures

-	ASM data

sharing procedure

Develop the Procedures in support of ASM data sharing. References: ICAO Doc 9971 Manual on Collaborative ATFM

2019

Aircraft operator

Operational procedures

-	ASM real time exchanges procedure

Develop the Procedures related to real- time ASM data exchanges. References: ICAO Doc 9971 Manual on Collaborative ATFM

ANSP

ATM network function

2019

Operational procedures

-	Advanced FUA procedure

Develop the to advanced FUA procedure. References: ICAO Doc 9971 Manual on Collaborative ATFM

ANSP 	2019

DEPLOYMENT APPLICABILITY

Operational conditions:

This element is an evolution of element NOPS-B0/1 and should be undertaken by all ANSPs, AUs and the ATM Network function affecting both en-route and TMA operations.

Main intended benefits:

Operational -

procedures

Airspace

changes procedure

Procedures to respond to changes

submitted by Airspace users resulting from changes in airspace availability. References: ICAO Doc 9971 Manual on Collaborative ATFM

ANSP

ATM network function

2019

Ground

system infrastructur e

ATM systems

Rolling

ASM/ATFM

Adapt ASM/ATFM/AUs systems to

support a rolling ASM/ATFM process

2019

Ground

system infrastructur e

ATM systems

ASM notification

process

Upgrade the ATFM System modules for

improved ASM notification process

2019

Ground

system infrastructur e

ATM systems

ASM real time

exchanges-ATC part

Upgrade the ATC systems for real-time

ASM data exchanges

2019

Ground

system infrastructur e

ATM systems

ASM real time

exchanges-ASM and ATFM part

Adapt ASM system, ATFM and AU

systems for real-time ASM data exchanges

2019

Training

-

Training

requirements for full integration of ASM with ATFM

Rolling ASM/ATFCM process training.

Training in new operational procedures and ground system changes

2019

Aircraft operator

ATM network function

Aircraft operator

ATM network function

Aircraft operator

ATM network function

ANSP

ANSP

ANSP

ATM network function

ATM network function

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Capacity Flight delays will be reduced through better utilisation of airspace resources within and across airspace boundaries. | ANSP Airspace user ATM network function |
|  | Flight efficiency Increased through the chance to plan more optimum routes/trajectories allowing lower fuel burn. | Airspace user |

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
|  | Improve situational awareness of network manager and airspace manager | ANSP ATM network function |
| Indirect benefits | Safety Better knowledge of traffic environment, common situational awareness, and some enhancement through reduction in controller workload. | ANSP Airspace user ATM network function |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Flight time & distance | Facilitate tactical decisions leading to a shorter actual route than in the FPL | ++ | KPI05: Actual en- route extension |
| Efficiency | Flight time & distance | Overcome route selection inefficiencies associated with route & airspace availability as known at the flight planning stage | ++ | KPI04: Filed flight plan en-route extension |
| Efficiency | Flight time & distance | Reduce need for tactical ATFM rerouting to circumnavigate airspace closed at short notice | ++ | KPI05: Actual en- route extension |
| Efficiency | Vertical flight efficiency | Reduce altitude restrictions during climb introduced to avoid airspace above | ++ | KPI17: Level-off during climb |
| Efficiency | Vertical flight efficiency | Reduce altitude restrictions during cruise introduced to avoid airspace above | ++ | KPI18: Level capping during cruise |
| Efficiency | Vertical flight efficiency | Reduce altitude restrictions during descent to avoid Special Use Airspace | ++ | KPI19: Level-off during descent |

Sixth edition of the GANP

Main Purpose

ASM solutions and initial dynamic airspace configurations for ATFM planning, synchronisation of

traffic flows and demand/capacity balancing

New Capabilities

Availability of optimised Airspace solutions/Initial Dynamic Airspace configurations based on traffic

demand and dynamic sectors management taken into account for ATFM purposes.

Operational

Initial Dynamic Airspace configurations

NOPS-B1/6

Description 	This element addresses the following ASM/ATFM improvements:

Airspace solutions

Pre-defined airspace configurations

ANSPs/ ATM Network function data exchanges pertinent to pre-defined airspace configurations

The ASM solutions process is aimed at delivering ASM options/solutions that can help reducing or even alleviate the ATFM measures and address capacity issues identified in any particular area as well as to improve flight efficiency assessing impact on capacity and ensuring the synchronised availability of optimized airspace structures based on traffic demand and dynamic sectors management.

The Airspace configurations are pre-defined and coordinated airspace structures and ATC dynamic sectorisation, to meet the ATFM and airspace needs in terms of capacity and/or flight efficiency. The implementation of pre-defined airspace configuration exchange covers the improvements of ATFM systems, to allow exchange of predefined airspace configurations information.

The decisions required for dynamic sectorisation could benefit from real time exchanges with ATM Network function for ATFM planning, synchronisation of traffic flows and demand/capacity balancing. The notification of Airspace Configurations will be based on automatic flows of information between the different stakeholders supported by the ATM Network function. The airspace configurations and flexible sector configurations are already used when the flows and constraints can be predicted well in advance (e.g. weekend routes or seasonal flows of traffic).

A more efficient and dynamic process involving the ATM Network function, ATC would require new functionalities and procedures and well defined collaborative decision making processes at pre- tactical level. The ANSPs systems needs to support the dynamic sectorisation by dynamic resizing and change of sector shapes and volumes based on pre-defined airspace configurations.

Maturity Level 	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes Utilisation of pre-defined airspace configuration.

- Does it imply processing of new information by the user? Yes Airspace structure and ATC sectorisation are new data information.

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes Manual Task are semi-automated.

PLANNING LAYERS

Pre-tactical

Strategical

OPERATIONS

Arrival

En-route

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

FRTO-B0/1 - Direct routing (DCT)

Relation-operational need

FRTO-B0/2 - Airspace planning and Flexible Use of Airspace (FUA)

Relation-operational need

FRTO-B0/3 - Pre-validated and coordinated ATS routes to support flight and

flow

Relation-operational need

FRTO-B1/1 - Free Route Airspace (FRA)

Relation-operational need

FRTO-B1/2 - Required Navigation Performance (RNP) routes

Relation-operational need

FRTO-B1/3 - Advanced Flexible Use of Airspace (FUA) and management

of real time airspace data

Relation-operational need

FRTO-B1/4 - Dynamic sectorization

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

-	Airspace solution procedure

Develop the ASM/ATFM procedures for airspace solution. References: ICAO Doc 9971 Manual on Collaborative ATFM

ANSP

ATM network function

2016

Operational procedures

-	Predefined

airspace configuration procedure

Develop the ASM/ATFM procedures for pre-defined airspace configurations.

References: ICAO Doc 9971 Manual on Collaborative ATFM

ANSP

ATM network function

2018

Operational procedures

-	Constrain

management procedure

Develop the ASM/ATFM procedures for dynamic sectorization and constrain management. References: ICAO Doc 9971 Manual on Collaborative ATFM

ANSP 	2019

ATM network function

Ground system infrastructure

ATM systems Airspace solution  Upgrade the ATFM system modules

related to the airspace solution

2016

ATM network function

Ground system infrastructure

ATM systems Predefined

airspace configuration

Upgrade the ATFM system modules related to the pre-defined airspace configurations

2018

ATM network function

Ground system infrastructur e

ATM systems Dynamic

sectorization

Enhance the ATC system capabilities with dynamic sectorization and constraint management.

ANSP 	2019

Ground system infrastructure

ATM systems Airspace

configuration data exchanges (basic feature)

SWIM data exchanges for pre-defined airspace configurations

ANSP

2019

DEPLOYMENT APPLICABILITY

Operational conditions:

This element will be addressed by ANSPs and the ATM Network function as required ensuring a synchronised availability of optimised airspace structures supported by dynamic sectors management to better address traffic demand.

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Capacity | ANSP ATM network function |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | Establish/update/publish the catalogue of strategic ATFM measures designed to respond to a variety of possible/typical/recurring events degrading the airspace system (e.g. predefined action plans) | ++ |  |

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

NOPS-B1/7	Enhanced ATFM slot swapping	Operational

Sixth edition of the GANP

Main Purpose	Improve the Airspace Users driven ATFM slot swapping process

New Capabilities	The swapping of ATFM measure impacted flights within the same ATFM measure will be extended to all ATFM measure impacted flights.

Description		ATFM slot swapping allows Airspace Users (AU) to request a rearrangement of their own flights subject to an ATFM measure in order to better suit their needs. The enhanced ATFM Slot Swapping improves the slot swapping currently used by Airspace Users (AU), by allowing the function to be extended gradually to all airspace users, by re-prioritizing their flights during the pre-departure part of operations. The Enhanced Slot swapping increases flexibility for Airspace Users; and provides a wider range of possibilities, by facilitating the identification of possible swaps for an ATFM Measure impacted flight (through B2B/SWIM-based Network Operations Planning interfaces) and by reducing the rate of rejection of swap requests by refining current processes. The AUs requests for these changes in flight priority will be introduced at the initiative of the AUs themselves, airport authorities or the ATM Network function.

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes Change of tasks for many ATFM actors.

- Does it imply processing of new information by the user? Yes New data stream for slot swapping and airport slot monitoring.

- Does it imply the use of new equipment? Yes Change of system and interfaces.

- Does it imply a change to levels of automation? Yes Coordination is semi-automated.

PLANNING LAYERS

Tactical-Pre ops

OPERATIONS

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

NOPS-B0/4 - Initial Airport/ATFM slots and A-CDM Network Interface

Relation-operational need

NOPS-B1/2 - Enhanced Network Operations Planning

Relation-operational benefit

SWIM-B2/1 - Information service provision

Relation-operational benefit

SWIM-B2/2 - Information service consumption

### ENABLERS

Aircraft operator

ATM network function

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

-	Slot swapping procedure

Develop the Procedures for ATFM slot swapping. References: ICAO Doc 9971 Manual on Collaborative ATFM

2019

ATM network function

Ground system infrastructur e

ATM systems FOC interface to

slot swapping module

Upgrade the The Flight Operations Centre (FOC) system modules with interfaces, automation, and decision-support for ATFM slot swapping

2019

Aircraft operator

ATM network function

Ground system infrastructur e

ATM systems ATFM slot

Aircraft operator

swapping module

Enhance the ATFM systems with ATFM slot swapping capabilities supporting as well the Airport Slot Monitoring in real time

2019

Training	-	Training requirements for slot swapping

Slot swapping training. Train Flight Operation Centre personnel

2019

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Indirect benefits | Efficiency of airlines operations. Airlines cost savings related to each slot swap that is executed. | Airspace user |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | For a given airspace entry slot: let airspace users swap the slot to another flight (slot substitution or UDPP – User Driven Prioritisation Process) | ++ |  |

DEPLOYMENT APPLICABILITY

Operational conditions:

This element will support AUs businesses by reprioritizing their flights during the pre-tactical part of operations if and as requested by them. The ATM Network function will keep all impacted stakeholders in the loop in the context of the Collaborative Decision Making processes.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

NOPS-B1/8	Extended Arrival Management supported by the ATM Network function

Operational

Sixth edition of the GANP

Main Purpose	ATM Network function contributions to extended Arrival Management.

New Capabilities	Extended Arrival Management information is taken on board by the Network ATM function to improve the quality of the ATFM service.

Description 			The ATM Network function involvement in extended Arrival Management process is addressed by this element. It does include the following elements:

Enhancements of ATFM Planned Trajectory about the accuracy/predictability of estimates to meet the extended arrival management operational requirements;

Provision of ATFM Planned Trajectory to ANSPs;

Reception and processing of ANSPs extended Arrival Management info by ATM Network function;

ATFM assessment tool for extended Arrival Management.

Bilateral agreements need be established between the sectors involved that can be in different ATC units and also in different countries, including the ATM Network function for the notification purposes. The ATFM procedures need to be revised for the management of the extended Arrival Management information.

Maturity Level 		Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

Propagation of delay further en-route will increase the task for some ATFM actors.

- Does it imply processing of new information by the user? Yes

AMAN delay is propagated further en-route and the Network function is notified.

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes

Some ATFM actors got new tasks for the optimisation of arrival traffic flows.

PLANNING LAYERS

Tactical-During ops

Tactical-Pre ops

OPERATIONS

Arrival

En-route

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

RSEQ-B1/1 - Extended arrival metering

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational -

procedures

Extended AMAN

LoA

Define the data exchanges and

operational procedures with ANSP. References: ICAO Doc 9971 Manual on Collaborative ATFM

ANSP	2017

ATM network function

Operational -

procedures

ATFM procedure

for Extended AMAN

Develop the ATFM procedures for

management of extended Arrival Management information. References: ICAO Doc 9971 Manual on Collaborative ATFM

2017

Ground

system infrastructur e

ATM systems

ATFM extended

AMAN module

Upgrade the ATFM system modules to

support extended Arrival Management

2017

ATM network function

ATM network function

DEPLOYMENT APPLICABILITY

Operational conditions:

It is applicable in areas where extended AMAN tools and procedures are implemented. This element will be addressed by the ANSPs at high density TMAs, if and as required, in improving arrival management. The ATM Network function will ensure that all impacted stakeholders (different ATC units also maybe in different countries) are properly involved and appropriate ATFM actions are undertaken.

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Capacity Operational Efficiency: Improved arrival flow. | ANSP Airspace user ATM network function |
|  | Improve situational awareness of network managers and ANSPs | ANSP ATM network function |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

NOPS-B1/9	Target Times for ATFM purposes	Operational

Sixth edition of the GANP

Main Purpose	Use of Target Times for ATFM purposes including an initial level of arrival sequencing in case of an arrival ATFM measure.

New Capabilities	Calculation and provision of Target Times by the ATM Network function in addition to CTOT, for the most penalised measure.

Description		In order to improve the flight predictability at the entry of the congested area, a target time of entry at the congested area (most penalised measure) will be provided by ATM Network function. At this stage, the target times will be applied for ATFM purpose only, including an initial level of arrival sequencing in case of an arrival ATFM measure. The ATM Network function will provide the calculated Target Time (TT) at the most penalising measure reference point in addition to Calculated Take-Off Time (CTOT) to all concerned users. TT will be distributed by data exchanges with the concerned Stakeholders. Stakeholders using TTs should be able to receive, extract and present the target times delivered by ATM Network function. ANSPs have access to the relevant information on flights that are subject to a Target Time to manage these flights as required in accordance with local procedures that need to be developed. The Flight Operating centres should provide Target Times to pilots prior to departure; pilots should endeavour to adhere to the Target Times to the extent possible.

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes Target time adherence by AUs and ANSPs.

- Does it imply processing of new information by the user? Yes Target time to be presented to affected users.

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes AUs and ANSPs to process and comply with target times.

PLANNING LAYERS

Tactical-Pre ops

OPERATIONS

En-route   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

RSEQ-B1/1 - Extended arrival metering

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational -

procedures

Target Time

procedure

Develop the ATFM Target Times

procedures and processes. References: ICAO Doc 9971 Manual on Collaborative ATFM

2018

Ground

system infrastructur e

ATM systems

Target time

module

Upgrade the ATFM Systems to support

Target Time processing and sharing

2019

Ground

system infrastructur e

CFSP

systems

Target time

interface

AUs Operation Centre systems to extract

and distribute Target Times

2019

Training

-

Training

requirements for Target Times

Target Time training. Target Time training

ANSP

Aircraft operator

2019

Aircraft operator

ATM network function

ANSP

Aircraft operator

ATM network function

DEPLOYMENT APPLICABILITY

Operational conditions:

This element will involve ANSPs, Airspace Users and the ATM Network function in their endeavour to improve predictability of operations through the sharing and use of Target Times (Target Times Over/Target Times of Arrival).

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Predictability. Capacity | ANSP Airspace user ATM network function |
|  | Improve situational awareness of ANSPs and flight crew | ANSP Aircraft operator |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | TMI-based optimisation (only impacts traffic when a TMI or restriction is manually activated for one or more constraint satisfaction points) | ++ |  |

Sixth edition of the GANP

Main Purpose

Optimised Air Traffic Flow and Capacity Management stemming from enhanced Trajectory

prediction and further integration of ATC with ATFM.

New Capabilities	Availability of “BIG” data and enhanced statistical approaches will improve Trajectory forecast and

provide more accurate and consistent end to end 4D trajectories which will in turn optimise the provision of Flow and Capacity Management Services also in the context of the global ATFM concept. New systems, tools and procedures supporting the ATFM/ATC integration (e.g. advanced STAM, TT exchanges) will be introduced.

Operational

Optimised ATM Network Services in the initial

TBO context

NOPS-B2/1

Description 		Computation and sharing of more accurate and consistent end to end 4D trajectories will constitute the basis for forecasting traffic complexity for the relevant look-ahead time horizons, improved ATFM scenario management and the provision of an enhanced common network view as a key enabler to optimise collaborative network management. Interactions between trajectory management and for instance, ATFM / STAM processes and Dynamic Airspace Configuration will be further enhanced.

Tools and systems are required for improved network prediction and performances such as:

Tools that provide functionality for simulating, evaluating the balance between demand and capacity taking into account the data from different stakeholders;

ATFM scenario management;

Assessment of performance of network operations with stakeholders able to evaluate the impact of their intentions and decisions on capacity and other performance indicators;

Automated tool supporting ATFM/ATC planning environment to manage traffic complexity in order to alleviate traffic complexity, density and traffic flow problems by planning individual trajectories using advanced planning tools;

The management of Hotspot resolution and more frequent STAM Measures in the planning and execution phase is supported by advanced capabilities (preparation, implementation, monitoring).

The Target-Time Management: to manage the hotspot resolution, DCB actors can constrain the Time of Entry of flights into the hotspot with TTO (Target Time Over the congested en-route point) and TTA (target Time of Arrival at congested Airport) in order to smooth the traffic.

Maturity Level 	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes Additional data are exchanged.

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes STAM measure coordination is automated.

PLANNING LAYERS

Tactical-During ops

Tactical-Pre ops

OPERATIONS

Arrival

En-route

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

NOPS-B1/1 - Short Term ATFM measures

Evolution

NOPS-B1/4 - Dynamic Traffic Complexity Management

Evolution

NOPS-B1/8 - Extended Arrival Management supported by the ATM Network

function

Evolution

NOPS-B1/9 - Target Times for ATFM purposes

Relation-operational need

NOPS-B2/3 - Collaborative Network Operation Planning

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Operational procedures

Operational procedures

Operational procedures

Ground system infrastructur e

- Scenario

management procedure

- Network

Performance Assessment procedure

- Hotspot

resolution procedure

- Target Time

reconciliation procedure

ATM systems Further traffic

complexity enhancements

Develop the ATFM the Scenario Management procedures. References: ICAO Doc 9971 Manual on Collaborative ATFM

Develop the ATFM procedures for the Network performance assessments. References: ICAO Doc 9971 Manual on Collaborative ATFM

ATFM procedures for Hotspot resolution and more frequent STAM measures.

References: ICAO Doc 9971 Manual on Collaborative ATFM

ATFM/ATC/Airport procedures for TT management. References: ICAO Doc 9971 Manual on Collaborative ATFM

Further enhancement of ATC/ATFCM traffic complexity tools

2022

ATM network function

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |

2022

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |

2022

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |

2022

ATM network function

ANSP

2024

Ground system infrastructur e

Ground system infrastructur e

Ground system infrastructur e

ATM systems Scenario

management module

ATM systems Network

Performance Assessment module

ATM systems Trajectory

forecast module

Upgrade the ATFM systems with the Scenario Management module

Upgrade the ATFM systems with the Network Performance Assessment module

Develop the Trajectory forecast modules

2024

ATM network function

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |

2024

| Airport operator |  |  |
| --- | --- | --- |
| ANSP |  |  |
| Airspace user |  |  |
| ATM network function |  |  |

2025

Ground system infrastructur e

ATM systems Further

enhancements of STAM tool

Upgrade the ATFM system to support the management of Hotspot resolution and more frequent STAM Measures in the planning and execution phase (preparation, implementation, monitoring)

2024

Ground system infrastructur e

ATM systems Target Time

ATM network function

reconciliation module

Target Time management tools for the reconciliation of extended AMAN and AOP

2022

| Airport operator |  |
| --- | --- |
| ANSP |  |
| ATM network function |  |
| Aircraft operator |  |

DEPLOYMENT APPLICABILITY

Benefitting stakeholder(s)

Operational description

Type

Operational conditions: Main intended benefits:

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | Address demand/capacity imbalance handled at the tactical ATFM stage (on the day of operations) | ++ |  |

Sixth edition of the GANP

Main Purpose

Improve the management of airspace including restricted / reserved areas and if possible

dynamic mobile areas and fully integrate ASM with ATFM.

New Capabilities	Airspace configurations modified more dynamically based on evolutions anticipated in the forecast

of traffic will be introduced. Airspace configurations will better match scarce resources with anticipated Demand and Capacity imbalances thus minimising the need for ATFM measures. The Air Traffic Flow Management and the Airspace Management will be further integrated.

Operational

Enhanced dynamic airspace configuration

NOPS-B2/2

Description 		New concepts, systems and procedures will allow full dynamicity of airspace volumes management and sector configurations. DACs integration in ATFM will provide additional means regarding the collaborative optimisation of traffic flows (strategic, pre-tactical, tactical) making best use of available scarce resources. Airspace volumes that satisfy specific operational requirements are progressively introduced to facilitate the ATM network optimization and minimize the impact on the expected traffic flows.

Maturity Level 	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes Data related to dynamic airspace configurations.

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes The automation of DAC process is increased.

PLANNING LAYERS

Tactical-Pre ops

Pre-tactical

Strategical

OPERATIONS

Arrival

En-route

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

NOPS-B1/6 - Initial Dynamic Airspace configurations

Evolution

NOPS-B1/5 - Full integration of airspace management with air traffic flow

management

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational

procedures

-

DAC procedure

Develop the ASM/ATFM procedures for

DAC. References: ICAO Doc 9971 Manual on Collaborative ATFM.

ANSP	2024

ATM network function

Operational -

procedures

Airspace volume

procedures

Develop the ASM/ATFM procedures for

Dynamic Airspace Volumes. References: ICAO Doc 9971 Manual on Collaborative ATFM

ANSP	2024

ATM network function

Ground

system infrastructur e

ATM systems

DAC/DMA

integration

Enhancement of ATFM system modules

related to the DAC/DMA integration

2024

ATM network function

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | Mitigate demand/capacity imbalance in en-route airspace | ++ |  |
| Capacity | Capacity, throughput & utilization | Optimise en-route airspace capacity | ++ |  |

Ground

system infrastructur e

ATM systems DAC/DMA

interfaces

Development of data exchange interfaces

for DAC/DMA

ANSP

ATM network function

2024

DEPLOYMENT APPLICABILITY

Operational conditions:

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Benefitting stakeholder(s)

Operational description

Type

NOPS-B2/3	Collaborative Network Operation Planning	Operational

Sixth edition of the GANP

Main Purpose	Enhance Network Operation Planning to support the Collaborative Network Management in the context of more dynamic /rolling planning processes and the initial Trajectory Based Operations (TBO).

New Capabilities	System and procedures for the variety of collaborative mechanisms. It includes the tools, roles, responsibilities, rules and procedures concerning the ATFM initiation, delegation, coordination, implementation and monitoring.

Description		The Collaborative Network Operation Planning provides all the information to the ATFM actors/functions to support network and local demand capacity balancing activities as:

Conciliating multiple constraint resolution strategies from all the stakeholders in order to identify the best measure given nature, scope and time horizon;

The provision of network consolidated imbalances figures; The provision of enhanced “what-if” functionalities;

The provision of what-else capabilities to propose alternate solution; The provision of Network information;

Monitoring Network performance;

Drawing latest information shared via SWIM.

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes Integration of ATC and ATFM roles.

- Does it imply processing of new information by the user? Yes

New data are handled and needs to presented to ATFM and ATC planning actors.

- Does it imply the use of new equipment? Yes

In some cases. It could be handled via modules and interfaces or separate sub-systems.

- Does it imply a change to levels of automation? Yes User task for extended planning horizon are automated.

PLANNING LAYERS

Tactical-Pre ops

Pre-tactical

Strategical

OPERATIONS

Arrival

En-route

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

NOPS-B1/2 - Enhanced Network Operations Planning

Evolution

NOPS-B1/1 - Short Term ATFM measures

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational -

procedures

Collaborative

NOP procedure

Develop the ATFM procedures for the

collaborative access and update of the Network Operation Planning. References: ICAO Doc 9971 Manual on Collaborative ATFM

2025

Ground

system infrastructur e

ATM systems

Collaborative

NOP platform

Develop the Collaborative Network

Operation Planning platform

2025

Ground

system infrastructur e

ATM systems

Extended ATFM

functions

Upgrade the ATFM system with extended

functions (consolidation of imbalances, ECDM modules “what-if” and “what-else” functions, monitoring of network performances).

2025

Ground

system infrastructur e

ATM systems

Collaborative

NOP SWIM

interfaces

SWIM Network system interfaces with

concerned Stakeholders.

2025

Aircraft operator

ATM network function

Aircraft operator

ATM network function

Aircraft operator

ATM network function

ANSP

ANSP

ANSP

Airport operator

ATM network function

Airport operator

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Improve situational awareness of stakeholders |  |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

DEPLOYMENT APPLICABILITY

Operational conditions:

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

NOPS-B2/4	Multi ATFM slot swapping and Airspace Users

priorities

Operational

Sixth edition of the GANP

Main Purpose	Introduce s Airspace Users preferences and priorities based on their business models to select flights for ATFM measures

New Capabilities	Capabilities to accommodate Airspace Users priorities are introduced enabling AUs to recommend a priority order for flights affected by delays on departure, arrival and en-route, and to share preferences with other ATM stakeholders in capacity-constrained situations. The swapping of ATFM regulated flights will be extended/enhanced (different AUs, multi-swapping, substitution on cancelations, etc.).

Description	The multi ATFM slot swapping is supported by a tool that:

Identification of viable swaps;

Perform multi-swap (e.g. weather deterioration);

Slot substitution on Cancellation: capability to substitute the ATFM slot of a cancelled flight for another flight.

The systems and tools for Airspace Users priorities include:

AU decision making tool can interface with Airport CDM system to propose flight reordering; Interface with the departure sequencing tool;

Interface with the ATM network function related to the swapped flights and actions to be undertaken ( e.g. new CTOT)

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes Change of tasks for many ATFM actors.

- Does it imply processing of new information by the user? Yes New data stream for UDPP data.

- Does it imply the use of new equipment? Yes Change of system and interfaces.

- Does it imply a change to levels of automation? Yes Airspace Users preferences are fully taken into account.

PLANNING LAYERS

Tactical-Pre ops

OPERATIONS

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

NOPS-B1/7 - Enhanced ATFM slot swapping

Evolution

NOPS-B1/2 - Enhanced Network Operations Planning

Evolution

NOPS-B0/4 - Initial Airport/ATFM slots and A-CDM Network Interface

Evolution

NOPS-B0/2 - Collaborative Network Flight Updates

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational -

procedures

Multi-swap

procedures

Develop the procedures for multi ATFM

slot swapping and AUs priorities. References: ICAO Doc 9971 Manual on Collaborative ATFM

2025

Ground

system infrastructur e

CFSP

systems

Multi Swap

/UDPP interfaces

The Flight Operations Centre (FOC)

system modules with tools and interfaces for multi ATFM slot swapping and UDPP

2025

Ground

system infrastructur e

Airport

systems

Airport modules

for AUs priority

Upgrade the Airport system adapted to

accommodate AUs priorities

Airport operator 2025

Ground

system infrastructur e

ATM systems

Multi Swap

/UDPP modules

Develop the ATFM Tool for ATFM multi-

swap and AUs priorities

2025

Aircraft operator

ATM network function

ATM network function

Aircraft operator

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | For a given airspace entry slot: let airspace users swap the slot to another flight (slot substitution or UDPP – User Driven Prioritisation Process) | ++ |  |

Training

-

Training

requirements for multi slot swapping

Multi Swap /UDPP training. Train Flight

Operation Centre personnel

2025

DEPLOYMENT APPLICABILITY

Operational conditions:

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Benefitting stakeholder(s)

Operational description

Type

Aircraft operator

ATM network function

Airport operator

NOPS-B2/5	Further airport integration within Network Operation Planning

Operational

Sixth edition of the GANP

Main Purpose	Integrated Network planning with additional Airport Operation Planning (AOP) components into the Network Operation Planning.

New Capabilities	Additional AOP components (airport performance data, airport constrains, landside part) will be integrated with the enhanced Network operations planning.

Description		The integration of additional AOP data within the Network operations planning provides an enhanced dynamic/rolling picture of the network situation to be used by all operational stakeholders to prepare their plans and their inputs to the network CDM processes. The data exchanges are based on the subset of SWIM services that are most widely available to all stakeholders, communicating with local airport A-CDM systems to exchange relevant operational information.

The concept aims to improve integration of departure planning data from medium/small size airports when serving a complex airspace with dense traffic through improved availability of aircraft pre- departure information to the ATM Network function by specific TWR tools providing accurate electronic pre-departure information.

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes Manual coordination is automated.

- Does it imply processing of new information by the user? Yes Extended AOP/NOP interfaces contain an additional set of data items.

- Does it imply the use of new equipment? Yes In some cases. New modules and interfaces.

OPERATIONS

Does it imply a change to levels of automation? Yes Reduced need for phone coordination.

PLANNING LAYERS

Tactical-Pre ops

Pre-tactical

Strategical

| Taxi-out | Departure | Arrival | Taxi-in | Turn-around |
| --- | --- | --- | --- | --- |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

NOPS-B0/4 - Initial Airport/ATFM slots and A-CDM Network Interface

Evolution

ACDM-B2/1 - Airport Operations Plan (AOP)

Evolution

NOPS-B1/3 - Enhanced integration of Airport operations planning with

network operations planning

Relation-operational need

NOPS-B2/2 - Enhanced dynamic airspace configuration

### ENABLERS

ATM network function

Airport operator

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

-	Enhanced

AOP/NOP

procedure

Develop the ATFM/Airport procedures for additional AOP components/Network Operation Planning integration.

References: ICAO Doc 9971 Manual on Collaborative ATFM

2025

Ground system infrastructure

Airport systems

Enhanced AOP	Enhance the Airport Operations Plan

(AOP) with additional components

Airport operator 2025

Ground system infrastructure

ATM systems Enhanced

AOP/NOP

interface

Develop the Advanced AOP/Network Operations Planning interfaces

Airport operator 2025

ATM network function

ATM network function

Ground system infrastructure

ATC systems ATC DPI	Enhance the TWR systems for provision of  ANSP 	2016

ATC departure planning data

Ground system infrastructure

ATM systems ATC DPI

integration

Departure planning data interface and integration

ANSP

2016

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Improve situational awareness of network manager, airport operator, aircraft operator and ATCOs. |  |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

DEPLOYMENT APPLICABILITY

Operational conditions:

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

NOPS-B2/6	ATFM adapted for cross-border Free Route

Airspace (FRA)

Operational

Sixth edition of the GANP

Main Purpose	Adapt ATFM to accommodate cross border Free Route operations.

New Capabilities		The ATFM system tools and procedures are adapted to support the complexity management, demand capacity balancing and ATFM/ATC integration for large cross border FRA deployments.

Description	Large cross-border FRA will require the modification of existing ATFM tools and processes as:

Demand Prediction Uncertainty in FRA environment; Complexity methodologies in FRA environment; STAM measures managed in FRA environment; ATFM/ATC integration in FRA environment.

Maturity Level	Validation

| Airport operator | ANSP |  |  |
| --- | --- | --- | --- |
| ATM network function |  | Aircraft operator |  |

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

Additional and more complex data are exchanged (hotspot resolution, prediction uncertainty).

- Does it imply the use of new equipment? Yes

In some cases. It could be considered as upgrade of existing tool via new modules and interfaces, or development of new tool with SWIM interfaces.

- Does it imply a change to levels of automation? Yes

The level of automation compared with B0 and B1 elements is increased.

PLANNING LAYERS

Tactical-Pre ops

Pre-tactical

Strategical

OPERATIONS

En-route

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

NOPS-B1/1 - Short Term ATFM measures

Evolution

NOPS-B1/4 - Dynamic Traffic Complexity Management

Evolution

FRTO-B1/2 - Required Navigation Performance (RNP) routes

Relation-operational need

NOPS-B2/3 - Collaborative Network Operation Planning

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

-	Cross-border

FRA procedure

Develop the ATFM/ ATC procedures for the large cross border FRA. References: ICAO Doc 9971 Manual on Collaborative ATFM

ANSP

ATM network function

2016

ATM network function

Ground system infrastructur e

ATM systems Integration of

ATFM/ATC

planning function

Develop the tools supporting the integration of ATFM/ATC planning function

ANSP

ATM network function

2025

Ground system infrastructur e

ATM systems Advanced STAM tool

Upgrade the ATFM system to support the management of Hotspot resolution and more frequent STAM Measures in the planning and execution phase (preparation, implementation, monitoring)

2024

ATM network function

Ground system infrastructur e

ATM systems Traffic

Complexity enhancements

Further enhancement of ATC/ATFCM traffic complexity tools

ANSP

ATM network function

2024

Ground system infrastructur e

ATM systems Traffic prediction

module

Develop the Demand Prediction Uncertainty tools

2025

DEPLOYMENT APPLICABILITY

Operational conditions: Main intended benefits:

Benefitting stakeholder(s)

Operational description

Type

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Improve situational awareness of network manager | ATM network function |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | Mitigate demand/capacity imbalance in en-route airspace | ++ |  |
| Capacity | Capacity, throughput & utilization | Optimise en-route airspace capacity | ++ |  |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

NOPS-B2/7	UTM Network operations	Operational

Sixth edition of the GANP

Main Purpose New Capabilities Description

Maturity Level	-

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes New role and responsibilities.

- Does it imply processing of new information by the user? Yes New data stream for UTM traffic.

- Does it imply the use of new equipment? Yes Flight planning modules for drone operators.

- Does it imply a change to levels of automation? No

### PLANNING LAYERS 		OPERATIONS

Strategical

Pre-tactical

Tactical-Pre ops

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Operations	UTM Network

procedure

Develop the Network procedures to manage UTM traffic concerning the airspace reservation and constraint management. Additional item as trajectory management and ATFM measure might be included if relevant.

ANSP

UAS operator

ATM network function

2025

Ground system infrastructur e

ATM systems UTM Network

system upgrades

Upgrade of ATFM and flight planning function to manage UTM.

ANSP

UAS operator

ATM network function

2025

Training	-	Training requirements for UTM Network operations

Training of ATM Network function personnel, Drone operators and ATCOs concerning the UTM operations

ANSP

UAS operator

ATM network function

2025

### DEPLOYMENT APPLICABILITY

Benefitting stakeholder(s)

Operational description

Type

Operational conditions: Main intended benefits:

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Maintain or improve safety of very low-level operations | ++ |  |

Sixth edition of the GANP

Main Purpose New Capabilities Description

Maturity Level

-

Operational

High upper airspace network operations

NOPS-B2/8

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes New role and responsibilities.

- Does it imply processing of new information by the user? Yes New data stream for the higher altitude operation.

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

PLANNING LAYERS

Tactical-Pre ops

Pre-tactical

Strategical

OPERATIONS

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Operations	Higher airspace

Network procedure

Develop the Network procedures to manage higher airspace traffic concerning the airspace reservation and constraint management.

ANSP

UAS operator

ATM network function

2025

Ground system infrastructur e

ATM systems Higher airspace

operation Network system upgrades

Upgrade ATFCM and flight planning functions to coordinate any flow /trajectory constraints due to unforeseen weather phenomena, transition operations or airspace restrictions.

ANSP

UAS operator

ATM network function

2025

Training	-	Training requirements for higher airspace network operations

Training of ATM Network function personnel, higher altitude vehicle operators and ATCO.

ANSP

UAS operator

ATM network function

2025

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

DEPLOYMENT APPLICABILITY

Operational conditions: Main intended benefits:

Benefitting stakeholder(s)

Operational description

Type

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

| SKaPfAety | Focus Areas | MMaoisnttasipneocriifmicpproevrefosramfeatnycoef hoibgjheecrtiavires(psa) cseupoppeorratetiodns | KPI ++ Impact | KPI |
| --- | --- | --- | --- | --- |

NOPS-B3/1	ATM Network Services in full TBO context	Operational

Sixth edition of the GANP

Main Purpose	Air Traffic Flow and Capacity Management in the TBO environment

New Capabilities	Improved trajectory predictions will permit capacity buffers to be reduced. Trajectory clustering will allow Flow-centric operations for Demand and Capacity Balancing.

Description		Automated network management using AI and machine leaning for DCB decision support tools and hotspot resolution.

Dynamic Airspace Configurations (DAC) and Demand and Capacity Balance will be fully integrated. In this context conflict free routes will be automatically allocated and the most optimal division FL structure for specific periods of time will be calculated and flexibly implemented. Finer ATFCM measures (Smart Trajectory Adjustment Measures), taken in a timeframe particularly close to the time of occurrence of a complex situation, will be implemented in a cooperative manner.

Maturity Level	-

Human Factor Considerations

### PLANNING LAYERS 		OPERATIONS

Tactical-Pre ops

Tactical-During ops

Departure

En-route

Arrival

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### ENABLERS

There are currently no enablers.

### DEPLOYMENT APPLICABILITY

Benefitting stakeholder(s)

Operational description

Type

Operational conditions: Main intended benefits:

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | Address demand/capacity imbalance handled at the tactical ATFM stage (on the day of operations) | ++ |  |
| Flexibility |  | Improve flexibility of the Air Navigation System | ++ |  |

NOPS-B3/2	Cooperative Network Operations Planning	Operational

Sixth edition of the GANP

Main Purpose	Network planning will transit from collaborative to cooperative in support of performance steering in the TBO environment.

New Capabilities		Network planning will accommodate the needs of all actors identifying all possible constraints and striving for the “Absolute Priority” unifying flight priorities and specificities.

Description		High level of automation enabling the reconciliation of multiple flight and flow centric measures and scenarios with minimum adverse impact on performance. It will allow connecting SBT reconciliation with RBT synchronisation mechanism. The Network Operations Planning will cover the global/inter- regional coordination process in all phases including post-ops analysis focusing on performance impact.

Maturity Level	-

Human Factor Considerations

### PLANNING LAYERS 		OPERATIONS

| Strategical | Pre-tactical |  | Tactical-Pre ops |  |
| --- | --- | --- | --- | --- |
| Tactical-During ops |  | Post operations |  |  |

Departure

En-route

Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Evolution	NOPS-B2/3 - Collaborative Network Operation Planning

### ENABLERS

There are currently no enablers.

### DEPLOYMENT APPLICABILITY

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | Mitigate demand/capacity imbalance at airports and/or associated terminal airspace | ++ |  |
| Capacity | Capacity shortfall & associated delay | Mitigate demand/capacity imbalance in en-route airspace | ++ |  |

Operational conditions:

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Benefitting stakeholder(s)

Operational description

Type

NOPS-B3/3	Innovative airspace architecture	Operational

Sixth edition of the GANP

Main Purpose	Service oriented airspace architecture/concept/design/management to fulfil the needs of all kind of users/actors.

New Capabilities	New airspace concept with associated management/institutional structures in place to incorporate all actors and operations including very low and sub-orbital commercial space.

Description		A holistic airspace/architecture including design and management/institutional structures envisaged to protect a “flight” along its path. Specific operational characteristics/type of operations for each actor/user will be accommodated. Different taxonomy/procedures management functions will be in place. Required infrastructure and its performance will be relevant to the need including regional centralised systems, contingency and emergency cases. Dynamic management of moving areas will be in place (e.g. management of areas that are potentially unsafe due to weather phenomena that can evolve in 4D) and Variable Profile Areas will be integrated in DAC.

Maturity Level	-

Human Factor Considerations

### PLANNING LAYERS 		OPERATIONS

| Strategical | Pre-tactical |  | Tactical-Pre ops |
| --- | --- | --- | --- |
| Tactical-During ops |  |  |  |

Departure

En-route

Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Access and equity |  | Improve access and equity | ++ |  |

Evolution

NOPS-B2/2 - Enhanced dynamic airspace configuration

ENABLERS

There are currently no enablers.

DEPLOYMENT APPLICABILITY

Operational conditions:

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Benefitting stakeholder(s)

Operational description

Type

OPFL

OPFL-B0/1	In Trail Procedure (ITP)	Operational

Sixth edition of the GANP

Main Purpose	To enable aircraft to reach a more satisfactory flight level for flight efficiency or to avoid turbulence for safety.

New Capabilities	The procedure couples the capability of the controller to receive the current position and intent from a pair(s) of aircraft with the ability of the trailing aircraft to space itself accurately from the preceding aircraft(s) to allow for the safe issuance of the ITP clearance.

Description		ITP is primarily intended to help facilitate access to optimum flight levels for aircraft operating in airspace where no ATS surveillance service is available. The ITP aircraft must acquire and process position broadcast (ADS-B) data from up to two non-manoeuvring aircraft. Aircraft identification, altitude, position and ground speed of reference aircraft would be assessed by the ITP aircraft’s on- board equipment (on-board decision support system) to determine whether an ITP climb or descent is possible. Based on the processed broadcast data from the reference aircraft(s), a pilot can make an ITP climb or descent request to air traffic control (ATC). Pilots are responsible for using the on- board equipment to evaluate the situation and provide the required information to the controller.

Maturity Level	Ready for implementation

Human Factor Considerations

### PLANNING LAYERS 		OPERATIONS

Tactical-During ops 	 En-route

### DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### ENABLERS

There are currently no enablers.

### DEPLOYMENT APPLICABILITY

Operational conditions:

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Improve situational awareness of flight crew and ATCO | ANSP Aircraft operator |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Vertical flight efficiency | Increase acceptance of pilot requests for higher cruise level | ++ | KPI18: Level capping during cruise |
| Efficiency | Vertical flight efficiency | Reduce level restrictions during cruise issued by ATCOs for conflict resolution purposes | ++ | KPI18: Level capping during cruise |

Sixth edition of the GANP

Main Purpose

The CDP was designed to improve service to appropriately equipped aircraft by providing an air

traffic controller with another option for initiating an altitude change when existing separation minima do not allow an aircraft to climb or descend through the altitude of a blocking aircraft.

New Capabilities	The capability for the controller to request current position and intent from pair(s) (ADS-C capability)

aircraft provides the situational awareness to allow the controller to use the simultaneous reporting of position to support the procedure at less than the nominal separation.

Operational

Climb and Descend Procedure (CDP)

OPFL-B1/1

DEPLOYMENT APPLICABILITY

Operational conditions: Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Description 		The CDP utilizes existing ADS-C aircraft equipage and air traffic control (ATC) capabilities to allow more flights to achieve their preferred vertical profiles. Integral to the CDP is the use of advanced communication and surveillance capabilities (i.e. ADS-C and CPDLC). The CDP is conceptually modelled after existing in-trail distance measuring equipment (DME) rules set forth in the Procedures for Air Navigation Services — Air Traffic Management (PANS-ATM, Doc 4444), paragraph 5.4.2.3.4. Aircraft pair distance verification is performed by the ground automation system using simultaneous ADS-C demand contract reports.

Maturity Level 	Standardization

Human Factor Considerations

PLANNING LAYERS

Tactical-During ops

OPERATIONS

En-route

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

ENABLERS

There are currently no enablers.

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Improve situational awareness of flight crew and ATCO | ANSP Aircraft operator |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Vertical flight efficiency | Increase acceptance of pilot requests for higher cruise level | ++ | KPI18: Level capping during cruise |
| Efficiency | Vertical flight efficiency | Reduce level restrictions during cruise issued by ATCOs for conflict resolution purposes | ++ | KPI18: Level capping during cruise |

OPFL-B2/1	Separation minima using ATS surveillance

systems where VHF voice communications are not available

Operational

Sixth edition of the GANP

Main Purpose		With the advent of Space-based ADS-B data in oceanic and remote continental areas, there are projected Capacity, Efficiency and Environmental advantages in SADS-B derived separation minima.

ICAO has published reduced lateral and longitudinal separation minima that capitalize on the increased frequency of position information update, while retaining the procedural foundation of requiring aircraft to remain on tracks to ensure separation, thereby making accessible such capacity, efficiency and environmental advantages.

New Capabilities		Implementation of the new lateral and longitudinal separation minima will provide alleviation from level congestion, and a consequent increase in access to optimal flight levels.

Description		The availability of additional usable flight levels will increase airspace efficiency while decreasing congestion at lower flight levels. This will result in reduced fuel burn and consequently reduce greenhouse gas emissions.

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

| PLANNING LAYERS | OPERATIONS |  |  |
| --- | --- | --- | --- |
| Tactical-During ops | En-route |  |  |
| DEPENDENCIES AND RELATIONS |  |  |  |
| There are currently no dependencies. |  |  |  |
| ENABLERS |  |  |  |
| Enabler Enabler Type Enabler Name | Description / References | Stakeholders | Year |
| Category |  |  |  |
| Regulatory National National provisions regulatory framework framework amendment for new separation standard | National regulation amendment for new separation standard. References: DOC. 4444 – Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM) | CAA | 2020 |

Aircraft operator

Aircraft manufacturer

Operational procedures

Separation	Procedures for

separation.

Defined in PANS-ATM.	 ANSP 	2020

Aircraft operator

Aircraft manufacturer

Airborne system capability

Navigation	RNP4/RNP2	Aircraft should be equipped with

RNP4/RNP2. Reference: Doc 9613 (PBN Manual)

2020

Airborne system capability

Communicati on service

RCP 240	Aircraft should be equipped with RCP

Aircraft operator

Aircraft manufacturer

240. Reference: ICAO Doc 10037 Edition 1 Global Operational Datalink Manual (GOLD)

2020

Airborne system capability

Surveillance	Surveillance

capability for the new separation minima.

Surveillance capability such as ADS-B OUT transponder. Reference: Annex 10 Vol IV.

2020

Ground system infrastructur e

Surveillance	Surveillance

system for the new separation minima.

To support the separation application. Reference: Annex 10 Vol IV.

ANSP 	2020

Aircraft operator

Ground system infrastructur e

ATC systems ATC tool for

separation minima

Ground tools/capabilities that assist the air traffic controller in providing separation.

ANSP 	2020

DEPLOYMENT APPLICABILITY

Operational conditions:

Oceanic and continental en-route environments. To increase capacity and to allow a more optimum flight level.

Main intended benefits:

Training	-	Training requirements for separation minima using ATS surveillance systems where VHF voice communications are not available

For Air traffic controllers. References: Annex 1, PANS-ATM. And for crew for awareness.

ANSP

2020

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Efficiency | Aircraft operator |
|  | Capacity | ANSP Aircraft operator |
|  | Access | Aircraft operator |
| Indirect benefits | Environment | General citizen |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Flight time & distance | Improve route selection after the flight planning stage | ++ | KPI05: Actual en- route extension |
| Efficiency | Flight time & distance | Improve route selection at the flight planning stage | ++ | KPI04: Filed flight plan en-route extension |
| Efficiency | Fuel burn | Reduce fuel burn impact of impeded conditions | + | KPI16: Additional fuel burn |
| Efficiency | Vertical flight efficiency | Reduce vertical flight inefficiency during the cruise phase | ++ | KPI18: Level capping during cruise |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

OPFL-B3/1	Helicopter RNP 0.3 Terminal and En-Route

Operations

Operational

Sixth edition of the GANP

Main Purpose		Existing PBN track separation guidance for fixed wing airplane routes does not fully serve the helicopter IFR mission profiles which may require routes to be established at low altitude in mountainous and obstacle-rich environments or over densely populated regions with approaches to Points-In-Space (PINS) or helipads/heliports in airspace not typically used for fixed wing operations.

Unlike fixed wing aircraft, helicopter normal operating capability, i.e. en-route cruising speed and manoeuvring ability is suited to RNP 0.3 for en-route as well as the terminal environment. Studies have shown that increasing the availability of IFR service to helicopter operations has the potential to decrease the risk of accidents for helicopter operations in marginal weather conditions.

New Capabilities		Facilitating arrivals and departures, and en-route IFR Helicopter operations in terrain rich environments.

Description		Evolution of lateral track separation for parallel helicopter RNP 0.3 routes to facilitate safe and efficient IFR helicopter operations in all phases of flight.

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   En-route   Arrival

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

National regulatory framework

National framework amendment for performance- based separation provisions for helicopter specific navigation capability

National regulation amendment for performance-based separation provisions for helicopter specific navigation capability. References: DOC. 4444 – Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM)

CAA 	2028

Operational procedures

Design	PBN procedures design and use

These operational procedures should be designed and used as specified in Doc 8168 (PANS-OPS Vol II and I) or

equivalent.

ANSP 	2028

Operational procedures

Design	PBN procedure validation, approval and publication

A flight inspection and/or validation of the procedures might be required before publication. The publication of the procedures should follow Annex 4.

Aircraft operator

References: ICAO Doc 9906 (Quality Assurance Manual for Flight Procedure Design).

ANSP   CAA 	2028

Operational procedures

Operations	Procedures for

the crew to follow to fly a PBN approach.

Defined in the Ops Manual. Reference: Doc 9613 (PBN Manual)

2028

Operational procedures

Separation	Procedures for

separation

Defined in the Ops Manual. Reference: PANS-ATM

ANSP 	2028

Operational Authorizatio n

-	Operational

Authorization for PBN

specification

Aircraft operator flying a PBN procedure should have an operational authorization related to the specified performance of the procedure, as described in Doc 9997 (PBN Ops Approval Manual).

Aircraft operator

CAA

2022

Aircraft operator

ANSP

Airborne system capability

Navigation	Helicopter

capability RNP 0.3

Helicopters should be equipped with RNP

0.3. Reference: Doc 9613 (PBN Manual)

2021

Aircraft operator

Aircraft manufacturer

Training	-	Training requirements for Helicopter RNP

0.3 Terminal and En-Route Operations

Crew trained to fly the procedure. References: As defined in Doc 9613 (PBN Manual). For Air traffic controllers. References: PANS-ATM. References: PANS-OPS Vol II and Doc 9992 Manual on the Use of Performance-Based Navigation (PBN) in Airspace Design

2028

### DEPLOYMENT APPLICABILITY

#### Operational conditions:

For operations of helicopters in terminal and en-Route in complex urban environment or areas with abrupt terrain. Preserving safety and providing access to helicopters for emergency purposes for example.

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |  |
| --- | --- | --- | --- |
| Direct benefits | Safety | Aircraft operator | General citizen |
|  | Access | Aircraft operator | General citizen |
| Indirect benefits | Environment | General citizen |  |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Increase planned capacity (the maximum configuration capacity established as part of [multi- year] ATM planning) | + | KPI06: En-route airspace capacity |
| Efficiency | Flight time & distance | Optimise horizontal flight efficiency in the en-route phase | + |  |
| Safety |  | Avoid controlled flight into terrain (CFIT) and obstacle collision risk | ++ | KPI20: Number of aircraft accidents |

Expansion of upper limit of the Reduced Vertical	Operational

Separation Minima (RVSM) band of flight levels

OPFL-B3/2

Sixth edition of the GANP

Main Purpose 		The demand for flights in the existing RVSM band of flight levels (FL290- FL410) has increased to the point where, at certain altitudes, it is generating congestion similar to that which prevailed before the implementation of RVSM.

The introduction of RVSM added six flight levels, the immediate impact and benefit of which were mostly observed in the FL 330 to FL 370 band of flight levels as this is where the majority of civil flights operated. Initially the upper levels of RVSM airspace were sparsely occupied. With the proliferation of modern civil aircraft capable of efficient operations at higher flight levels, occupation of FL 400 and FL 410 has increased with routing options becoming limited as a result of congestion, with consequent inefficiency and fuel burn/emissions affected adversely. Expansion of the upper bound of the RVSM band of flight levels will provide alleviation from level congestion.

New Capabilities   Expansion of the upper bound of the RVSM band of flight levels will provide alleviation from level congestion, and a consequent increase in access to optimal flight levels.

Description 		The availability of additional usable flight levels will increase airspace efficiency while decreasing congestion at lower flight levels. This will result in reduced fuel burn and consequently reduce greenhouse gas emissions.

Maturity Level 	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

| PLANNING LAYERS | OPERATIONS |  |  |
| --- | --- | --- | --- |
| Tactical-During ops | En-route |  |  |
| DEPENDENCIES AND RELATIONS |  |  |  |
| There are currently no dependencies. |  |  |  |
| ENABLERS |  |  |  |
| Enabler Enabler Type Enabler Name | Description / References | Stakeholders | Year |
| Category |  |  |  |
| Regulatory National National provisions regulatory framework framework amendment for allowing qualified aircraft to operate at the additional RVSM levels | National regulation amendment for allowing qualified aircraft to operate at the additional RVSM levels. References: Annex 2, Annex 11; Annex 6; DOC. 4444 - Procedures for Air Navigation Services - Air Traffic Management (PANS-ATM) | CAA | 2028 |

DEPLOYMENT APPLICABILITY

Operational conditions:

Oceanic and continental en-route environments. To increase capacity and to allow a more optimum flight level.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Operational Design and

procedures	operations

Procedures for

the use of additional RVSM levels

Design and use of additional flight levels.

Reference: Annex 2

ANSP

2028

Operational -

Authorizatio n

Operational

Authorization for extended RVSM

Aircraft operator flying an additional

RVSM level should have an authorization as per Doc 9760.

CAA

2028

Airborne

system capability

Altimeter

Height keeping

capability

Aircraft should be equipped with

appropriate height keeping capability. Reference: Doc 9937.

2028

Training

-

Training

requirements for the expansion of upper limit of the Reduced Vertical Separation Minima (RVSM) band of flight levels

Crew trained to fly the procedure.

References: Annex 6. For Air traffic controllers. References: PANS-ATM.

ANSP

2028

Aircraft operator

Aircraft manufacturer

Aircraft operator

Aircraft operator

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Efficiency | Aircraft operator |
|  | Capacity | ANSP Aircraft operator |
|  | Access | Aircraft operator |
| Indirect benefits | Environment | General citizen |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Increase planned capacity (the maximum configuration capacity established as part of [multi- year] ATM planning) | ++ | KPI06: En-route airspace capacity |
| Efficiency | Fuel burn | Reduce fuel burn impact of impeded conditions | + | KPI16: Additional fuel burn |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Vertical flight efficiency | Reduce cruise level inefficiency attributable to altitude constraints imposed by ATM | ++ | KPI18: Level capping during cruise |

OPFL-B3/3	Target-to-target separations using Space-based ADS-B data

Operational

Sixth edition of the GANP

Main Purpose		With the advent of Space-based ADS-B data in oceanic and remote continental areas, there are projected Capacity, Efficiency and Environmental advantages in SADS-B derived separation minima.

A “radar-like” target-to-target separation minima that capitalizes on the increased frequency of position information updates but makes use of datalink communications capability (for circumstances where VHF communications do not exist) will offer increased flexibility and free aircraft form needing to adhere to specific tracks.

New Capabilities  Implementation of the target-to-target separation minima will provide alleviation track adherence requirements associated with procedural separation minima. And offer a consequent increase in access to optimal flight levels.

Description		The availability of the flexibility offered by target-to-target separation will increase airspace efficiency, resulting in reduced fuel burn and consequently reduced greenhouse gas emissions.

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

En-route

### DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

National regulatory framework

National framework amendment for target-to-target separation

National regulation amendment for target- to-target separation. References: DOC. 4444 – Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM)

CAA 	2024

Aircraft operator

Aircraft manufacturer

Operational procedures

Separation	Target to target

separation

Procedures for target-to-target separation.  ANSP 	2024

Defined in PANS-ATM.

Airborne system capability

Communicati on service

RCP 240 .	Aircraft should be equipped with RCP

Aircraft operator

Aircraft manufacturer

240. Reference: ICAO Doc 10037 Edition 1 Global Operational Datalink Manual (GOLD)

2024

Airborne system capability

Surveillance	Surveillance

capability for the new target-to- target separation minima.

Surveillance capability such as ADS-B OUT transponder. Reference: Annex 10 Vol IV.

2024

Ground system infrastructur e

Surveillance	Surveillance

system for the new target-to- target separation minima.

To support the separation application. Reference: Annex 10 Vol IV.

ANSP 	2024

Aircraft operator

Ground system infrastructure

ATC systems ATC tool for

target to target separation

Ground tools/capabilities that assist the air traffic controller in providing target to target separation.

ANSP 	2024

DEPLOYMENT APPLICABILITY

Operational conditions:

Oceanic and continental en-route environments. To increase capacity and to allow a more optimum flight level.

Main intended benefits:

Training	-	Training requirements for target-to-target separation

For Air traffic controllers. References: Annex 1, PANS-ATM. And for crew for awareness.

ANSP

2024

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Efficiency | Aircraft operator |
|  | Capacity | ANSP Aircraft operator |
|  | Access | Aircraft operator |
| Indirect benefits | Environment | General citizen |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Flight time & distance | Improve route selection after the flight planning stage | ++ | KPI05: Actual en- route extension |
| Efficiency | Flight time & distance | Improve route selection at the flight planning stage | ++ | KPI04: Filed flight plan en-route extension |
| Efficiency | Fuel burn | Reduce fuel burn impact of impeded conditions | + | KPI16: Additional fuel burn |
| Efficiency | Vertical flight efficiency | Reduce vertical flight inefficiency during the cruise phase | ++ | KPI18: Level capping during cruise |

GANP PORTAL

# ASBU ELEMENTS

RSEQ

SNET

SURF

| Functional Description | Enablers | Deployment Applicability | Performance Impact Assessment |
| --- | --- | --- | --- |

## RSEQ

RSEQ-B0/1	Arrival Management	Operational

Sixth edition of the GANP

Main Purpose	To optimize sequencing for arrivals.

New Capabilities	Arrival management metering and sequencing by ATC is based on inbound traffic prediction information, and decision making support.

Description		This element represents management of arrival sequences, thereby allowing aircraft to fly more efficiently to the necessary fix and to reduce the use of holding stacks, especially at low altitude.

Based on inbound traffic prediction information and decision making support, ATC operational techniques (metering points, speed-control, Time-To-Gain/Time-To-Lose, etc.) will be used to sequence inbound flights at minimum separation on final approach (time or distance based) so as to optimise runway utilization. Time-based metering (as opposed to time-based separations) is the practice of planning a sequence of traffic by time rather than distance. Typically, the relevant ATC authorities will assign a time in which a flight must arrive at the aerodrome or at a specific control point, and/or advises subject flights of speed changes as required to achieve the optimal separation on final approach. Besides inbound traffic predication information, input can include aerodrome capacity, terminal airspace capacity, aircraft capability, wind and other meteorological factors. Time- based metering is the primary mechanism in which arrival sequencing is achieved.

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Arrival

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-information benefit	AMET-B0/1 - Meteorological observations products Relation-information benefit	AMET-B0/2 - Meteorological forecast and warning products

Relation-operational benefit	WAKE-B2/1 - Wake turbulence separation minima based on 7 aircraft

groups

Relation-operational benefit	WAKE-B3/3 - Wake turbulence separation minima based on leader/follower

static pairs-wise

Relation-operational benefit	WAKE-B2/2 - Time based wake separation minima for final approach Relation-operational benefit	SURF-B0/2 - Comprehensive situational awareness of surface operations Relation-operational benefit	SURF-B1/4 - Routing service to support ATCO surface operations

management

Relation-operational benefit	ACDM-B0/1 - Airport CDM Information Sharing (ACIS) Relation-operational benefit	ACDM-B0/2 - Integration with ATM Network function

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Operations	Arrival

Sequencing Procedure

Local ATM procedure for arrival sequencing

ANSP 	2013

Aircraft operator

Ground system infrastructur e

ATC systems Arrival

Sequencing ATC Automation system

Automation for calculating optimum arrival sequence with presentation to ATCOs

ANSP 	2013

Training	-	Training requirements for arrival management

ATCO Arrival Sequencing Training - ATCOs trained to use arrival sequencing automation, supported by arrival sequencing procedure. Pilot Time-Based Metering Training - Pilots trained to use airborne system to arrive at waypoint specified by ATCOs at specific timing

ANSP

2013

Regulatory provisions

SMS	Arrival Sequencing Safety Assessment

Safety assessment of arrival sequencing operation

ANSP 	2013

DEPLOYMENT APPLICABILITY

Operational conditions:

Runways and terminal manoeuvring area in major hubs and metropolitan areas.

Main intended benefits:

| Type Operational description Benefitting stakeholder(s) |  |
| --- | --- |
| Direct benefits | Reduce holding and low level vectoring ANSP Aircraft operator |
|  | Enable dynamic scheduling ANSP |
|  | Decreased uncertainties in ANSP aerodrome/terminal demand prediction |
|  | Increase aerodrome throughput Airport operator ANSP |
|  | Increase aerodrome capacity Airport operator ANSP |
|  | Efficiency Airport operator ANSP Aircraft operator |
|  | Flexibility Aircraft operator |
|  | Predictability ANSP Aircraft operator |
| Indirect benefits Fuel consumption Aircraft operator |  |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Apply arrival balancing | + | KPI10: Airport peak throughput |
| Capacity | Capacity, throughput & utilization | Apply smart sequencing to harmonise final approach speeds (arrival) | ++ | KPI10: Airport peak throughput |
| Capacity | Capacity, throughput & utilization | Apply smart sequencing to optimise wake vortex separations (arrival) | ++ | KPI10: Airport peak throughput |
| Capacity | Capacity, throughput & utilization | Improve arrival sequencing and metering to fill all arrival slots | ++ | KPI11: Airport throughput efficiency |
| Efficiency | Flight time & distance | Apply TTA and en-route speed reduction if traffic is already airborne | ++ | KPI08: Additional time in terminal airspace |
| Efficiency | Flight time & distance | Reduce need to fine-tune traffic spacing in terminal airspace (arrival) | ++ | KPI08: Additional time in terminal airspace |

RSEQ-B0/2	Departure Management	Operational

Sixth edition of the GANP

Main Purpose	To optimize departure operations.

New Capabilities	Departure management sequences the aircraft for optimized utilization of ground infrastructure and efficiently meet en-route and destination airport constraints, taking on board user preferences.

Description		Departure management, like its arrival counterpart, serves to optimize departure operation to ensure the most efficient utilization of aerodrome and terminal resources. Slots assignment and adjustments will be supported by departure management automation like department management or departure flow management. Dynamic ATFM slot allocation will foster smoother integration into overhead streams and help airspace users to better meet metering points and comply with other ATM requirements. It will sequence aircraft, based on the ground and airspace structure, wake turbulence, aircraft capability, en-route and destination ATFM constraints, and airspace users' preferences. This will serve to increase aerodrome throughput and compliance with allotted departure time. Where Airport CDM is implemented, departure management will interface with the associated A-CDM processes (including the pre-departure sequencing of A-CDM) in determining optimal departure sequencing.

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS

Tactical-Pre ops   Tactical-During ops

### OPERATIONS

Departure

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-information benefit	AMET-B0/1 - Meteorological observations products Relation-information benefit	AMET-B0/2 - Meteorological forecast and warning products Relation-operational benefit	ACDM-B0/1 - Airport CDM Information Sharing (ACIS) Relation-operational benefit	ACDM-B0/2 - Integration with ATM Network function

Relation-operational benefit	SURF-B1/4 - Routing service to support ATCO surface operations

management

Relation-operational benefit	WAKE-B2/1 - Wake turbulence separation minima based on 7 aircraft

groups

Relation-operational benefit	WAKE-B3/3 - Wake turbulence separation minima based on leader/follower

static pairs-wise

Relation-operational benefit

WAKE-B3/6 - Time based wake separation minima for departure based on

leader/follower static pair-wise

Relation-operational benefit

SURF-B0/2 - Comprehensive situational awareness of surface operations

Relation-operational benefit

APTA-B0/2 - PBN SID and STAR procedures (with basic capabilities)

Relation-information benefit

NOPS-B0/5 - Dynamic ATFM slot allocation

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Operations	Departure

Sequencing Procedure

Local ATM procedure for departure sequencing

ANSP 	2013

Ground system infrastructur e

ATC systems Departure

Sequencing ATC Automation system

Automation for calculating an optimum departure sequence with presentation to ATCOs

ANSP 	2013

Training	-	Training requirements for departure management

ATCO Departure Sequencing Training - ATCOs trained to use departure sequencing automation, supported by departure sequencing procedure

ANSP 	2013

Regulatory provisions

SMS	Departure Sequencing Safety Assessment

Safety assessment of departure sequencing operation

ANSP 	2013

DEPLOYMENT APPLICABILITY

Operational conditions:

Runways and terminal manoeuvring area in major hubs and metropolitan areas. It will streamline departure traffic flow and smooth transition into en-route airspace. Automated dissemination of departure information and clearances.

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |  |  |
| --- | --- | --- | --- | --- |
| Direct benefits | Decreased lead time for departure request | Aircraft operator |  |  |
|  | Decrease time between call for release and departure time | Airport operator | ANSP | Aircraft operator |
|  | Enable dynamic scheduling | ANSP |  |  |
|  | Increase aerodrome throughput | Airport operator | ANSP |  |
|  | Increase aerodrome capacity | Airport operator | ANSP |  |
|  | Efficiency | Airport operator | ANSP | Aircraft operator |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| Type | Operational description Benefitting stakeholder(s) |  |
| --- | --- | --- |
|  | Flexibility | ANSP Aircraft operator |
|  | Predictability | ANSP Aircraft operator |
| Indirect benefits | Fuel consumption | Aircraft operator |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Maintain or improve departure rate of the RWY | ++ | KPI10: Airport peak throughput |
| Efficiency | Flight time & distance | Avoid additional holding time after line up caused by departure metering not factored in during pushback planning | ++ | KPI02: Taxi-out additional time |
| Efficiency | Flight time & distance | Improve the delivery of departing traffic into the overhead stream | ++ | KPI02: Taxi-out additional time |

Sixth edition of the GANP

Main Purpose

To allow merging of arrival flows.

New Capabilities

Sequencing using pre-defined legs equidistant from a point that are used for shortening or stretching

the arrival path.

Description

This element represents a procedural concept that uses existing technology to merge arrival flows.

Its purpose is to improve and harmonize arrival operations by enabling continuous descent operations (CDO) and increasing arrival predictability, thereby enhancing airport capacity and limiting the environmental impact of aircraft emissions. Point Merge is based on a specific route structure that is made of a point (the merge point) with pre-defined legs (the sequencing legs) equidistant from this point that are used for shortening or stretching the arrival path.

Maturity Level

Ready for implementation

Operational

Point merge

RSEQ-B0/3

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational benefit

WAKE-B2/1 - Wake turbulence separation minima based on 7 aircraft

groups

Relation-operational benefit

WAKE-B3/3 - Wake turbulence separation minima based on leader/follower

static pairs-wise

Relation-operational benefit

WAKE-B2/2 - Time based wake separation minima for final approach

Relation-information benefit

AMET-B0/1 - Meteorological observations products

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational Operations

procedures

Arrival Traffic

Merging Procedure

Local ATM procedure to merge traffic

during arrivals

ANSP

2013

Training

-

Training

requirements for point merge

ATCO Point Merge Arrival Traffic Merging  ANSP

Training - ATCOs trained to merge arrival traffic using point merge procedure

2013

Training

Awareness	Pilot Point Merge

Briefing

Pilots briefed on airport’s point merge

procedure

2013

Regulatory	SMS

provisions

Point Merge

Safety Assessment

Safety assessment of point merge

operation

ANSP

2013

Aircraft operator

DEPLOYMENT APPLICABILITY

Operational conditions:

Runways and terminal manoeuvring area in major hubs and metropolitan areas.

Main intended benefits:

Benefitting stakeholder(s)

Operational description

Type

| Type Operational description Benefitting stakeholder(s) |  |
| --- | --- |
| Direct benefits | Reduce holding and low level vectoring Aircraft operator |
|  | Enable dynamic scheduling ANSP |
|  | Decreased uncertainties in ANSP aerodrome/terminal demand prediction |
|  | Increase aerodrome throughput Airport operator ANSP |
|  | Increase aerodrome capacity Airport operator ANSP |
|  | Efficiency Airport operator ANSP Aircraft operator |
|  | Flexibility Aircraft operator |
|  | Predictability ANSP Aircraft operator |
| Indirect benefits Fuel consumption Aircraft operator |  |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Apply merging & synchronisation of arrival flows | ++ | KPI10: Airport peak throughput |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

RSEQ-B1/1	Extended arrival metering	Operational

Sixth edition of the GANP

Main Purpose	To enhance predictability and ATM decision compliance.

New Capabilities		Synchronization between adjacent FIRs, arrival management taking into account extended metering requirements.

Description 			Extended metering will enhance predictability and ATM decision compliance. The ATS units will be able to meter across FIR boundaries. Extended metering will enable ATS units to continue metering during high volume traffic and will improve metering accuracy. This will also facilitate synchronization between adjacent FIRs. With extended metering, delays can be shifted to higher attitudes or even to the departure gate, where it can be more efficiently absorbed by incoming flights. This metering will provide extended arrival management, increasing arrival management effectiveness and benefits (e.g. in terms of reduced holding time) while reducing approach ATC workload. Extended metering may set requirements on flights pre-departure, if departing within the arrival metering range of the destination airport.

Maturity Level 		Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   En-route   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

RSEQ-B0/1 - Arrival Management

Relation-operational benefit

NOPS-B1/8 - Extended Arrival Management supported by the ATM Network

function

Relation-information benefit

AMET-B1/1 - Meteorological observations information

Relation-information benefit

AMET-B1/2 - Meteorological forecast and warning information

Relation-operational benefit

APTA-B1/4 - CDO (Advanced)

Relation-information benefit

SWIM-B2/1 - Information service provision

Relation-information benefit

FICE-B3/1 - Flight information management services for enhanced trajectory

operations

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Operations	Extended Arrival

Metering Procedure

ATM procedure for extended arrival metering including exchange of metering information with affected neighboring ATC units

ANSP 	2019

Operational procedures

Letter of agreement

Extended Arrival Metering Letter of Agreement

Letter of Agreement or equivalent document outlining procedure to exchange metering information with affected neighboring ATC units

ANSP 	2019

Operational procedures

-	Procedure to

Reconcile ATFM Constraint with Extended Metering Requirements

ATM procedure for reconciling ATFM constraints with Extended Arrival Metering requirements

ANSP 	2019

Ground system infrastructur e

ATM systems Extended Arrival

Metering Local ATM Automation systems

ATM automation for calculation and presentation of extended arrival metering to ATCOs and exchange metering information with affected neighboring ATC units

ANSP 	2019

Training	-	Training requirements for extended arrival management

ATCO Extended Arrival Metering Training

- ATCOs trained to use extended arrival metering automation, supported by extended arrival metering procedure and Letter of Agreement with affected neighboring ATC units, along with Procedure to Reconcile ATFM Constraint with Extended Metering Requirements.

Pilot Time-Based Metering Training - Pilots trained to use airborne system to arrive at waypoint specified by ATCOs at specific timing

Aircraft operator

ANSP

2019

Regulatory provisions

SMS	Extended Arrival Metering Safety Assessment

Safety assessment of extended arrival metering operation

ANSP 	2019

DEPLOYMENT APPLICABILITY

Operational conditions:

Runways and terminal manoeuvring areas in major hubs and metropolitan areas.

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Reduction in airborne delay/holding | ANSP Aircraft operator |
|  | Traffic flow synchronization between en- route and terminal domain | ANSP |
|  | Enables dynamic scheduling | ANSP |
|  | Decrease uncertainties in aerodrome/terminal demand prediction | ANSP |
|  | Efficiency | Airport operator ANSP Aircraft operator |
|  | Flexibility | ANSP |
|  | Predictability | ANSP |
| Indirect benefits | Reduction in fuel burn and environment impact (emission and noise) | Aircraft operator |
|  | Capacity at the airport | Airport operator |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | Apply (unplanned) airborne holding to inbound traffic | ++ |  |
| Capacity | Capacity shortfall & associated delay | Delay take-off of inbound traffic (sequencing & metering measures) | ++ |  |
| Capacity | Capacity shortfall & associated delay | Slow down inbound traffic during en-route | ++ |  |
| Efficiency | Flight time & distance | Extend arrival management to a greater radius around the destination airport | ++ | KPI08: Additional time in terminal airspace |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

RSEQ-B2/1	Integration of arrival and departure management	Operational

Sixth edition of the GANP

Main Purpose	To set up detailed integrated arrival and departure sequence for the same runway.

New Capabilities	Integration of arrival and departure management sequences into a single runway or dependent runways.

Description		A fully integrated and throughput-optimized sequence of arrivals and departures for the same runway (or for dependent runways) is set up by an algorithm considering minimum separations. The sequence is characterized by high planning stability and all controllers working towards delivering this sequence. Thus, in order to meter arrival and sequence pre-departure, controllers will follow Target Take-Off Time (TTOT) and Target Landing Time (TLDT) as closely as possible. Feeder controllers will provide the required gaps in the arrival sequence to allow for the respective departure flights.

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS 		OPERATIONS

Tactical-Pre ops

Tactical-During ops

Departure

Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

RSEQ-B0/2 - Departure Management

Relation-operational benefit

AMET-B2/1 - Meteorological observations information

Relation-information benefit

SWIM-B2/1 - Information service provision

Relation-information benefit

SWIM-B2/2 - Information service consumption

Relation-operational benefit

AMET-B2/2 - Meteorological forecast and warning information

Relation-information benefit

FICE-B3/1 - Flight information management services for enhanced trajectory

operations

Relation-operational benefit

APTA-B1/4 - CDO (Advanced)

Evolution

RSEQ-B1/1 - Extended arrival metering

ENABLERS

ANSP

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Operations	Departure

ANSP

Sequencing Procedure

Local ATM procedure for departure sequencing

2013

ANSP

Ground system infrastructur e

ATC systems Departure

Sequencing ATC Automation system

Automation for calculating an optimum departure sequence with presentation to ATCOs

2013

Operational procedures

Operations	Extended Arrival

Metering Procedure

ATM procedure for extended arrival metering including exchange of metering information with affected neighboring ATC units

2019

Operational procedures

Letter of agreement

Extended Arrival Metering Letter of Agreement

Letter of Agreement or equivalent document outlining procedure to exchange metering information with affected neighboring ATC units

2019

ANSP

Operational procedures

-	Procedure to

ANSP

Reconcile ATFM Constraint with Extended Metering Requirements

ATM procedure for reconciling ATFM constraints with Extended Arrival Metering requirements

2019

ANSP

Ground system infrastructur e

ATM systems Extended Arrival

Metering Local ATM Automation systems

ATM automation for calculation and presentation of extended arrival metering to ATCOs and exchange metering information with affected neighboring ATC units

2019

Operational procedures

Operations	Common

Operating Procedure for Departure – Arrival Sequencing Integration

Common Operating Procedure in support of departure and arrival sequencing integration

2025

ANSP

Operational procedures

-	Departure –

Arrival Sequencing Integration Procedure

Local ATM procedure in support of departure and arrival sequencing integration

ANSP 	2025

Ground system infrastructur e

ATM systems Departure –

Arrival Sequencing Automation

Automation for calculating optimum departure and arrival sequencing especially for airports with mixed-mode runway used for both arriving and departing flights considering minimum separations applicable

ANSP 	2025

Training	-	Training requirements for integration of arrival and departure management

ATCO Departure Sequencing Training - ATCOs trained to use departure sequencing automation, supported by departure sequencing procedure ATCO Extended Arrival Metering Training - ATCOs trained to use extended arrival metering automation, supported by extended arrival metering procedure and Letter of Agreement with affected neighboring ATC units, along with Procedure to Reconcile ATFM Constraint with Extended Metering Requirements ATCO Departure – Arrival Sequencing Training: ATCOs trained to use Departure – Arrival Sequencing Automation supported by Departure – Arrival Sequencing Integration Procedure Pilot Time-Based Metering Training - Pilots trained to use airborne system to arrive at waypoint specified by ATCOs at specific timing

ANSP

2025

Regulatory provisions

SMS	Arrival-Departure Management Integration Safety Assessment

Safety assessment of integrated arrival – departure management operation

ANSP 	2025

Airport operator

Aircraft operator

DEPLOYMENT APPLICABILITY

Operational conditions:

At aerodrome with existing arrival management and departure management operation, interrelated through significant use of mixed-mode operations.

Main intended benefits:

| Type Operational description Benefitting stakeholder(s) |  |
| --- | --- |
| Direct benefits | Decreased lead time for departure request Aircraft operator |
|  | Decrease time between call for release Airport operator ANSP Aircraft operator and departure time |
|  | Reduce holding and low level vectoring ANSP Aircraft operator |
|  | Enable dynamic scheduling ANSP |
|  | Decreased uncertainties in ANSP aerodrome/terminal demand prediction |
|  | Increase aerodrome throughput Airport operator ANSP |
|  | Increase aerodrome capacity Airport operator ANSP |
|  | Efficiency Airport operator ANSP Aircraft operator |
|  | Flexibility Aircraft operator |
|  | Predictability ANSP Aircraft operator |
| Indirect benefits Fuel Consumption Aircraft operator |  |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Introduce integrated arrival and departure sequencing for a single runway or dependent runways of the same airport | ++ | KPI11: Airport throughput efficiency |

Sixth edition of the GANP

Main Purpose

To provide more efficient arrival management support for TMAs with multiple airports.

New Capabilities	Further development of arrival management in support of TMAs with multiple airports and associated

interaction among traffic flows into airports within the same TMA.

Operational

Arrival management in terminal airspace with

multiple airports

RSEQ-B3/2

Description 		In terminal airspace with multiple airports, advisory notices from arrival management operations for these airports need to be harmonized in the form of traffic flows to ensure that bunching does not occur in sectors adjacent to TMAs where inbound routes merge or intersect as well as other sectors in adjacent FIRs.

Maturity Level 	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

DEPENDENCIES AND RELATIONS

Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   En-route   Arrival

| Type of Dependencies | ASBU Element |  |
| --- | --- | --- |
| Evolution | RSEQ-B1/1 - Extended arrival metering |  |
| Relation-operational benefit | APTA-B1/4 - CDO (Advanced) |  |
| Relation-information benefit | AMET-B2/4 - Meteorological information service in SWIM |  |
| ENABLERS |  |  |
| Enabler Enabler Type | Enabler Name Description / References Stakeholders | Year |
| Category |  |  |
| Operational Operations procedures | Extended Arrival ATM procedure for extended arrival ANSP Metering metering including exchange of metering Procedure information with affected neighboring ATC units | 2019 |
| Operational Letter of procedures agreement | Extended Arrival Letter of Agreement or equivalent ANSP Metering Letter document outlining procedure to exchange of Agreement metering information with affected neighboring ATC units | 2019 |
| Operational - procedures | Procedure to ATM procedure for reconciling ATFM ANSP Reconcile ATFM constraints with Extended Arrival Metering Constraint with requirements Extended Metering Requirements | 2019 |

Ground system infrastructur e

ATM systems Extended Arrival

Metering Local ATM Automation systems

ATM automation for calculation and presentation of extended arrival metering to ATCOs and exchange metering information with affected neighboring ATC units

ANSP 	2019

Operational procedures

Operations	Mixed Arrival

Stream Traffic Sequencing Procedure

ATC procedure to accommodate mixed traffic streams into multiple aerodromes

ANSP 	2026

Operational procedures

Letter of agreement

Mixed Arrival Stream Traffic Sequencing Letter of Agreement

ATC Unit Letter of Agreement specifying procedure to accommodate mixed traffic streams into multiple aerodromes

ANSP 	2026

Ground system infrastructur e

ATM systems Mixed Arrival

Stream Traffic Sequencing ATM Automation systems

ATM automation to support metering of interacting flows of traffic to multiple aerodromes within same terminal airspace

ANSP 	2026

Training	-	Training requirements for arrival management for multi-airport terminal airspace

ATCO Extended Arrival Metering Training

- ATCOs trained to use extended arrival metering automation, supported by extended arrival metering procedure and Letter of Agreement with affected neighboring ATC units, along with Procedure to Reconcile ATFM Constraint with Extended Metering Requirements ATCO Mixed Arrival Stream Traffic Sequencing Training - ATCOs trained to use Mixed Arrival Stream Traffic Sequencing ATM automation supported by Mixed Arrival Stream Traffic Sequencing Procedure and Letter of Agreement Pilot Time-Based Metering Training- Pilots trained to use airborne system to arrive at waypoint specified by ATCOs at specific timing

ANSP 	2026

Regulatory provisions

SMS	Arrival Management for Multi-Airport Terminal Airspace Safety Assessment

Safety assessment of arrival management operation for multi-airport terminal airspace

ANSP 	2026

DEPLOYMENT APPLICABILITY

Operational conditions:

In terminal airspace with multiple aerodromes requiring arrival management operations.

Main intended benefits:

| Type Operational description Benefitting stakeholder(s) |  |
| --- | --- |
| Direct benefits | Reduce holding and low level vectoring ANSP Aircraft operator |
|  | Enable dynamic scheduling ANSP |
|  | Decreased uncertainties in ANSP aerodrome/terminal demand prediction |
|  | Increase aerodrome throughput Airport operator ANSP |
|  | Increase aerodrome capacity Airport operator ANSP |
|  | Efficiency Airport operator ANSP Aircraft operator |
|  | Flexibility Aircraft operator |
|  | Predictability ANSP Aircraft operator |
| Indirect benefits Fuel Consumption Aircraft operator |  |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Increase multi-airport terminal airspace arrival rate | ++ | KPI10: Airport peak throughput |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

RSEQ-B3/3	Increased utilization of runway capacity by

improved real-time runway scheduling

Operational

Sixth edition of the GANP

Main Purpose	Increase the achieve utilization of runway capacity.

New Capabilities		Uncertainty buffers are reduced to reflect the increased accuracy of constraints in terms of volume and active time as well as increased accuracy in both the aircraft current and future state characteristics.

Description 			This element represents the combined efforts in weather forecasting and more refined translations into the characteristics of the severity, size and duration of trajectory interacting weather phenomena, refinement of network operational initiatives through big data analysis and machine learning, and the increased information on the aircrafts current states and intent. This improves the real-time ability to schedule runaway usage without impacting flight efficiency balancing these twin goals at a better equilibrium.

Maturity Level 		Concept

Human Factor Considerations

### PLANNING LAYERS 		OPERATIONS

Tactical-During ops 	 Departure   Arrival

### DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### ENABLERS

There are currently no enablers.

### DEPLOYMENT APPLICABILITY

Benefitting stakeholder(s)

Operational description

Type

Operational conditions: Main intended benefits:

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Improve airport capacity utilization (throughput efficiency) | ++ | KPI11: Airport throughput efficiency |

Sixth edition of the GANP

Main Purpose

To improve operator fleet management in runway sequencing by improving the ability of the operator

to manage their fleet priorities among their real-time schedule allocation.

New Capabilities	The ubiquitous sharing of airspace and flight information as well schedule assignments provides an

environment where operators can adjust their flights in the assigned sequence to meet operational and connectivity needs for passengers and cargo.

Improved operator fleet management in runway	Operational

sequencing

RSEQ-B3/4

Description		While a real-time master schedule for runway usage is developed and maintained by the ANSP to assure equity and maximum capacity utilization, users have increased flexibility in resequencing aircraft among their slot assignments. The capability is supported by the sharing of all state information with respect to configurations, traffic initiatives as well as position and intent information on all flights in the planning horizon. This allows the operator to evaluate and test potential sequencing with respect to feasibility increasing the likelihood of ASNSP approval without unceasing workload and complexity to the ANSP’s schedule management. This allow the operator to increase their opportunity to meet fleet business objectives which are unknown to the ANS during schedule development. This uses the ability of the operator to use real-time information for their decision making versus increased a priori delivery of business requirements to ANSP as well as increased complexity of ANSP schedule algorithms to have the ANSP deliver against business objectives.

Maturity Level	Concept

Human Factor Considerations

### PLANNING LAYERS 		OPERATIONS

Tactical-Pre ops

Tactical-During ops

Departure

Arrival

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### ENABLERS

There are currently no enablers.

### DEPLOYMENT APPLICABILITY

Benefitting stakeholder(s)

Operational description

Type

Operational conditions: Main intended benefits:

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Flexibility |  | Improve flexibility of the Air Navigation System | ++ |  |

RSEQ-B4/1

Departure management in terminal airspace from  Operational

multiple airports

Sixth edition of the GANP

Main Purpose 	To provide departure management support for TMAs with multiple airports.

New Capabilities  Further development of departure management in support of TMAs with multiple airports and associated interaction among traffic flows out of airports within the same TMA.

Description 		In terminal airspace with multiple airports, departure management operations among airports involved need to be harmonized in the form of traffic flows to enable a more consistent and manageable delivery into En-Route phase of flight. Harmonization among these departure management operations may advise SID-rebalancing in the event of demand-capacity imbalance predicted. Departure sequence may be adjusted if other capacity cannot be utilized.

Maturity Level 	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

PLANNING LAYERS

Tactical-Pre ops   Tactical-During ops

OPERATIONS

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

RSEQ-B0/2 - Departure Management

Relation-information benefit

AMET-B2/1 - Meteorological observations information

Relation-information benefit

AMET-B2/2 - Meteorological forecast and warning information

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational Operations

procedures

Terminal

Airspace Congestion Identification and Mitigation Procedure

Procedure to identify terminal airspace

congestion and routing aircraft onto offload departure routes where possible

ANSP

2032

Regulatory	SMS

provisions

Departure

Management for Multi-Airport Terminal Airspace Safety Assessment

Safety assessment of departure

management operation for multi-airport terminal airspace

ANSP

2032

Ground system infrastructur e

ATM systems Terminal

Airspace Congestion Identification and Mitigation Procedure automation systems

ATM system supporting identification of terminal airspace congestion and suggesting offload departure route to de- conflict

ANSP 	2032

DEPLOYMENT APPLICABILITY

Operational conditions:

In terminal airspace with multiple aerodromes requiring departure management operations.

Main intended benefits:

Training	-	ATCO Terminal Airspace Congestion Identification and Mitigation Training

ATCO trained to identify and mitigate terminal airspace congestion, offloading aircraft onto other departure routes where appropriate, based on operational procedure using appropriate ATM automation

ANSP 	2032

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| Type Operational description Benefitting stakeholder(s) |  |
| --- | --- |
| Direct benefits | Decreased lead time for departure request Aircraft operator |
|  | Decrease time between call for release Airport operator ANSP Aircraft operator and departure time |
|  | Enable dynamic scheduling ANSP |
|  | Decreased uncertainties in ANSP aerodrome/terminal demand prediction |
|  | Increase aerodrome throughput Airport operator ANSP |
|  | Increase aerodrome capacity Airport operator ANSP |
|  | Efficiency Airport operator ANSP Aircraft operator |
|  | Flexibility Aircraft operator |
|  | Predictability ANSP Aircraft operator |
| Indirect benefits Fuel Consumption Aircraft operator |  |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Increase multi-airport terminal airspace departure rate | ++ | KPI10: Airport peak throughput |
| Efficiency | Flight time & distance | Improve the delivery of departing traffic into the overhead stream | ++ | KPI02: Taxi-out additional time |

RSEQ-B4/2	Extended arrival management supporting

overlapping operations into multiple airports

Operational

Sixth edition of the GANP

Main Purpose	To enable en-route sectors to support extended arrival metering from multiple airports / TMAs.

New Capabilities	En-route support to deconflict concurrent arrival management strategies into independent TMAs, with overlapping arrival management horizons and ATFM measures such as Target Times, while also considering non-arrival traffic within the sector.

Description		En-route sectors are expected to contribute to the arrival sequencing towards multiple TMA simultaneously with potentially conflicting sequencing constraints. The process integrates information from arrival management systems operating out to extended range with local traffic/sector information and balances the needs of each.

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

En-route

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Evolution	RSEQ-B1/1 - Extended arrival metering

Relation-information benefit	AMET-B2/1 - Meteorological observations information Relation-information benefit	AMET-B2/2 - Meteorological forecast and warning information

Relation-information need	FICE-B2/9 - Flight information management support for inflight re-planning

### ENABLERS

ANSP

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Operations	Extended Arrival

ANSP

Management Conflicting Instruction Detection & Resolution Procedure

Procedure to identify conflicting overlapping instructions from various extended arrival management operations and resolution thereof.

2032

Regulatory provisions

SMS	Extended Arrival Management Conflicting Instruction Detection & Resolution Safety Assessment

Safety assessment of procedure to identify conflicting overlapping instructions from various extended arrival management operations and resolution thereof.

2032

Ground system infrastructur e

ATM systems Extended Arrival

ANSP

Management Conflicting Instruction Detection & Resolution ATM Automation

ATM automation to support detection and resolution of conflicting extended arrival management instructions.

2032

DEPLOYMENT APPLICABILITY

Operational conditions:

In en route Area Control Centers with supporting multiple overlapping arrival management operations.

Main intended benefits:

Training	-	ATCO Extended Arrival Management Conflicting Instruction Detection & Resolution Training

ATCOs trained to identify and resolve conflicting overlapping instructions from various extended arrival management operations based on procedure using appropriate ATM automation

2032

ANSP

| Type | Operational description | Benefitting stakeholder(s) |  |  |
| --- | --- | --- | --- | --- |
| Direct benefits | Enable dynamic scheduling | ANSP |  |  |
|  | Decreased uncertainties in aerodrome/terminal demand prediction | ANSP |  |  |
|  | Efficiency | Airport operator | ANSP | Aircraft operator |
|  | Flexibility | Aircraft operator |  |  |

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
|  | Predictability | Airport operator ANSP |
| Indirect benefits | Fuel Consumption | Aircraft operator |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Apply arrival management | ++ | KPI10: Airport peak throughput |
| Capacity | Capacity, throughput & utilization | Improve arrival sequencing and metering to fill all arrival slots | ++ | KPI11: Airport throughput efficiency |
| Capacity | Capacity, throughput & utilization | Increase multi-airport terminal airspace arrival rate | ++ | KPI10: Airport peak throughput |
| Efficiency | Flight time & distance | Extend arrival management to a greater radius around the destination airport | ++ | KPI08: Additional time in terminal airspace |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

SNET

Sixth edition of the GANP

Main Purpose

To assist the air traffic controller in preventing collision between aircraft, using position data from

ground surveillance.

New Capabilities	STCA systems alert the controller when a given separation between two aircraft is actually lost or

may be lost within a given amount of time.

Operational

Short Term Conflict Alert (STCA)

SNET-B0/1

Description 		Surveillance data from ground radars and ADS-B stations is used to track aircraft. For each pair of aircraft which are sufficiently close, a short term conflict alert is raised if at least one of the following tests is true:

(current proximity test) their current horizontal separation is lower than a horizontal threshold and their current vertical separation is lower than a vertical threshold; or

(linear prediction test) at any of their future positions within a given amount of time (warning time), as linearly extrapolated from their current track, their horizontal separation will be lower than a horizontal threshold and their vertical separation will be lower than a vertical threshold.

The horizontal and vertical thresholds may be different in each test but are equal or lower than the ATC separation standards for the airspace covered by the STCA system. The warning time for the linear prediction may depend on the control unit specificities but is typically equal to or lower than 2 minutes.

The above parameters may be configured differently in defined geographic areas of the control unit. Additionally, inhibitions of alerts may be set up for a list of aircraft and for defined geographic areas.

On noticing the alert, the controller has to analyse the situation and, if deemed necessary, issue an avoiding instruction to one or both aircraft, with the appropriate emergency phraseology.

Maturity Level 	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? No

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   En-route   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology option

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-technology option

ASUR-B0/2 - Multilateration cooperative surveillance systems (MLAT)

Relation-technology benefit

ASUR-B0/3 - Cooperative Surveillance Radar Downlink of Aircraft

Parameters (SSR-DAPS)

ENABLERS

Enabler

Category

Enabler Type	Enabler Name

Description / References

Stakeholders	Year

Operational procedures

Safety nets	Controller

reaction to short term conflict alerts

Procedures for air traffic controllers reaction to short term conflict alerts. References: Doc 4444 - Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM)

ANSP 	2013

Airborne system capability

Surveillance	Surveillance

system capabilities required for short term conflict alerts

SSR mode S transponder with Ext.squitter version 0, version 1 and version 2 ADS-B out compliant with … References: Doc 100xx - Ground-based Safety Nets Manual http://www.eurocontrol.int/publications/eur ocontrol-guidelines-short-term-conflict- alert-stca-part-i-iii

Airspace user

2013

Aircraft manufacturer

Ground system infrastructur e

Safety nets	Surveillance

ground capabilities required for short term conflict alerts

SSR radar ADS-B in station References: Doc 100xx - Ground-based Safety Nets Manual Details in Eurocontrol documents at http://www.eurocontrol.int/publications/eur ocontrol-guidelines-short-term-conflict- alert-stca-part-i-iii

ANSP

2013

Ground system infrastructur e

Safety nets	Display for short

Ground systems supplier

term conflict alerts

Capability to indicate alerts on the radar screen of the controller working positions. References: Doc 100xx - Ground-based Safety Nets Manual Details in Eurocontrol documents at http://www.eurocontrol.int/publications/eur ocontrol-guidelines-short-term-conflict- alert-stca-part-i-iii

ANSP

2013

DEPLOYMENT APPLICABILITY

Operational conditions:

STCA systems are applicable in all controlled airspace for all aircraft for which a controller has responsibility for separation or traffic information. Before operational use, the system must have been configured for the target airspace, to maximize the number of relevant alerts while keeping the number of unnecessary alerts to an acceptable level.

Main intended benefits:

Ground systems supplier

Training	-	Training requirements for short term conflict alerts

Air traffic controller knowledge and reaction to alerts. References: Indications in Doc 100xx - Ground-based Safety Nets Manual

ANSP 	2013

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | STCA use improves safety by making controllers aware of potentially dangerous situations that may otherwise have been missed or detected later. | ANSP Airspace user |
|  | Improve situational awareness of ATCO | ANSP |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Improve mid-air collision avoidance (safety net) | ++ | KPI20: Number of aircraft accidents KPI23: Number of airprox/TCAS alert/loss of separation/near midair collisions/midair collisions (MAC) |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

SNET-B0/2	Minimum Safe Altitude Warning (MSAW)	Operational

Sixth edition of the GANP

Main Purpose		To assist the air traffic controller in preventing controlled flight into terrain accidents by generating, in a timely manner, an alert of aircraft proximity to terrain or obstacles.

New Capabilities		MSAW systems warns the controller about the increased risk of controlled flight into terrain accidents by generating, in a timely manner, an alert of aircraft proximity to terrain or obstacles.

##### Note: MSAW systems are providing protection on all aircraft in particular those not equipped with Ground Proximity Warning Systems.

Description		Surveillance data (including tracked pressure altitude), flight data (including cleared flight levels) and environment data (including terrain and obstacle data) are input to the MSAW system to generate the alerts to the controller working position.

On noticing the alert, the controller has to analyse the situation and, if deemed necessary, issue an instruction to the aircraft, with the appropriate emergency phraseology.

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? No

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

En-route

Departure

Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology option

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-operational benefit

SNET-B0/1 - Short Term Conflict Alert (STCA)

Relation-technology benefit

ASUR-B0/3 - Cooperative Surveillance Radar Downlink of Aircraft

Parameters (SSR-DAPS)

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Safety nets	Controller

reaction to minimum safe altitude warning

Procedures for air traffic controllers to react to minimum safe altitude warnings. Reference: Doc 4444 - Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM); Doc 100xx - Ground-based Safety Nets Manual - Details in Eurocontrol documents at http://www.eurocontrol.int/publications/eur ocontrol-guidelines-minimum-safe- altitude-warning-msaw-part-i-iii

ANSP 	2013

Airborne system capability

Safety nets	Surveillance

system capabilities required for minimum safe altitude warning

SSR transponder compliant with … ADS- B out compliant with … Reference: Doc 100xx - Ground-based Safety Nets Manual

Ground systems supplier

- Details in Eurocontrol documents at http://www.eurocontrol.int/publications/eur ocontrol-guidelines-minimum-safe- altitude-warning-msaw-part-i-iii

Airspace user

2013

Aircraft manufacturer

Ground system infrastructur e

Safety nets	Surveillance

ground capabilities required for minimum safe altitude warning

SSR radar ADS-B in station Reference: Doc 100xx - Ground-based Safety Nets Manual - Details in Eurocontrol documents at http://www.eurocontrol.int/publications/eur ocontrol-guidelines-minimum-safe- altitude-warning-msaw-part-i-iii

ANSP

2013

Ground system infrastructur e

Safety nets	Display for

minimum safe altitude warnings

Capability to indicate alerts on the controller working position. Reference: Doc 100xx - Ground-based Safety Nets Manual - Details in Eurocontrol documents at http://www.eurocontrol.int/publications/eur ocontrol-guidelines-minimum-safe- altitude-warning-msaw-part-i-iii

ANSP

2013

Ground systems supplier

Training	-	Training requirements for minimum safe altitude warnings

Air traffic controller knowledge and reaction to alerts. Reference: Doc 100xx - Ground-based Safety Nets Manual

ANSP 	2013

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Improve situational awareness of ATCO | ANSP |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Avoid controlled flight into terrain (CFIT) and obstacle collision risk | ++ | KPI20: Number of aircraft accidents |

DEPLOYMENT APPLICABILITY

Operational conditions:

All controlled airspace for all aircraft.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

SNET-B0/3	Area Proximity Warning (APW)	Operational

Sixth edition of the GANP

Main Purpose		APW is designed, configured and used to make a significant positive contribution to the prevention of accidents arising from unauthorized penetration of an airspace volume.

New Capabilities	APW systems warn the air traffic controller about unauthorised penetration into the airspace (either restricted or controlled) by a flight (either controlled or uncontrolled).

Description		Surveillance data (including tracked pressure altitude), flight data (including cleared flight levels and RVSM status) and environment data (including airspace volumes) are input to the APW system to generate the alerts to the controller working position(s).

On noticing the alert, the controller has to analyse the situation and, if deemed necessary, issue an instruction to the aircraft, with the appropriate emergency phraseology.

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? No

### PLANNING LAYERS 		OPERATIONS

Tactical-During ops

Departure   En-route   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology option

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Safety nets	Controller

reaction to area proximity warning

Procedures for air traffic controllers to react to area proximity warnings.

Aircraft manufacturer

Reference: Future amendment of Doc 4444 - Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM); Doc 100xx - Ground-based Safety Nets Manual - Details in Eurocontrol documents at http://www.eurocontrol.int/publications/eur ocontrol-guidelines-minimum-safe- altitude-warning-msaw-part-i-iii

ANSP 	2013

Airborne system capability

Safety nets	Surveillance

system capabilities required for area proximity warning

SSR transponder compliant with … ADS- B out compliant with … Reference: Doc 100xx - Ground-based Safety Nets Manual

-

Ground systems supplier

http://www.eurocontrol.int/publications/eur ocontrol-guidelines-minimum-safe- altitude-warning-msaw-part-i-iii

Aircraft operator

2013

Ground system infrastructur e

Safety nets	Surveillance

ground capabilities required for area proximity warning

SSR radar ADS-B in station Reference: Doc 100xx - Ground-based Safety Nets Manual - http://www.eurocontrol.int/publications/eur ocontrol-guidelines-minimum-safe- altitude-warning-msaw-part-i-iii

ANSP

2013

Ground system infrastructur e

Safety nets	Display for area

proximity warning

Capability to indicate alerts on the controller working position. Reference: Doc 100xx - Ground-based Safety Nets Manual - http://www.eurocontrol.int/publications/eur ocontrol-guidelines-minimum-safe- altitude-warning-msaw-part-i-iii

ANSP

2013

Ground systems supplier

Training	-	Training requirements for area proximity warnings

Air traffic controller knowledge and reaction to alerts. Reference: Doc 100xx - Ground-based Safety Nets Manual - Details in Eurocontrol documents at http://www.eurocontrol.int/publications/eur ocontrol-guidelines-minimum-safe- altitude-warning-msaw-part-i-iii

ANSP 	2013

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Improve situational awareness of ATCO | ANSP |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Avoid unauthorized penetration of segregated airspace | ++ | KPI20: Number of aircraft accidents |

DEPLOYMENT APPLICABILITY

Operational conditions:

All controlled airspace for all aircraft.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

SNET-B0/4	Approach Path Monitoring (APM)	Operational

Sixth edition of the GANP

Main Purpose	APM is a ground-based safety net intended to warn the controller about increased risk of controlled flight into terrain accidents by generating, in a timely manner, an alert of aircraft proximity to terrain or obstacles during final approach.

New Capabilities	APM is designed, configured and used to make a significant positive contribution to avoidance of controlled flight into terrain accidents by generating, in a timely manner, an alert of aircraft proximity to terrain or obstacles during final approach.

Description		Surveillance data (including tracked pressure altitude), flight data (including concerned sectors) and environment data (including terrain and obstacle data) are input to the APM system to generate the alerts to the controller working position(s).

On noticing the alert, the controller has to analyse the situation and, if deemed necessary, issue an instruction to the aircraft, with the appropriate emergency phraseology.

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? No

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology option

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Safety nets	Controller

reaction to approach path monitoring alerts

Procedures for air traffic controllers to react to approach path monitoring alerts. Reference: Future amendment of Doc 4444 - Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM); Doc 100xx - Ground-based Safety Nets Manual - Details in Eurocontrol documents at http://www.eurocontrol.int/publications/eur ocontrol-guidelines-minimum-safe- altitude-warning-msaw-part-i-iii

ANSP 	2013

Airborne system capability

Safety nets	Surveillance

system capabilities required for approach path monitoring alerts

SSR transponder compliant with … ADS- B out compliant with … Reference: Doc 100xx - Ground-based Safety Nets Manual

-

Ground systems supplier

http://www.eurocontrol.int/publications/eur ocontrol-guidelines-minimum-safe- altitude-warning-msaw-part-i-iii

Aircraft operator

2013

Aircraft manufacturer

Ground system infrastructur e

Safety nets	Surveillance

ground capabilities required for approach path monitoring alerts

SSR radar ADS-B in station Reference: Doc 100xx - Ground-based Safety Nets Manual - http://www.eurocontrol.int/publications/eur ocontrol-guidelines-minimum-safe- altitude-warning-msaw-part-i-iii

ANSP

2013

Ground system infrastructur e

Safety nets	Display for

approach path monitoring alerts

Capability to indicate alerts on the controller working position. Reference: Doc 100xx - Ground-based Safety Nets Manual - http://www.eurocontrol.int/publications/eur ocontrol-guidelines-minimum-safe- altitude-warning-msaw-part-i-ii

2013

Aircraft operator

Training	-	Training requirements for approach path monitoring alerts

Air traffic controller knowledge and reaction to alerts. Reference: Doc 100xx - Ground-based Safety Nets Manual - Details in Eurocontrol documents at http://www.eurocontrol.int/publications/eur ocontrol-guidelines-minimum-safe- altitude-warning-msaw-part-i-iii

ANSP 	2013

Aircraft manufacturer

### DEPLOYMENT APPLICABILITY

#### Operational conditions:

All controlled airspace for all aircraft in final approach.

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Improve situational awareness of ATCO | ANSP |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Avoid controlled flight into terrain (CFIT) and obstacle collision risk | ++ | KPI20: Number of aircraft accidents |

Sixth edition of the GANP

Main Purpose

Assist the air traffic controller in preventing collision between aircraft, using position data from ground

surveillance and flight intent reported by aircraft.

New Capabilities	Using aircraft intent parameters allows STCA systems to reduce the number of unnecessary alerts,

to increase the number of relevant alerts and to alert earlier, compared to the basic STCA system in Block 0.

Operational

Enhanced STCA with aircraft parameters

SNET-B1/1

Description 		This enhanced STCA works the same as the basic STCA system in Block 0, but stops the linear extrapolation of the vertical position of an aircraft when it reached the Selected Flight Level information reported from ADS-B or downlinked from Mode S transponders.

Maturity Level 	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? No

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   En-route   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology option

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-technology need

ASUR-B0/3 - Cooperative Surveillance Radar Downlink of Aircraft

Parameters (SSR-DAPS)

Evolution

SNET-B0/1 - Short Term Conflict Alert (STCA)

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational Safety nets	Controller

procedures	reaction to short term conflict alerts

Procedures for air traffic controllers

reaction to short term conflict alerts. References: Doc 4444 - Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM)

ANSP

2013

Airborne	Safety nets

system capability

Surveillance

system capabilities required for enhanced short term conflict alerts

SSR transponder compliant with … ADS-

B out compliant with … References: Doc 100xx - Ground-based Safety Nets Manual http://www.eurocontrol.int/publications/eur ocontrol-guidelines-short-term-conflict- alert-stca-part-i-iii

2019

Aircraft operator

Aircraft manufacturer

Ground system infrastructur e

Safety nets	Surveillance

ground capabilities required for enhanced short term conflict alerts

SSR radar ADS-B in station References: Doc 100xx - Ground-based Safety Nets Manual Details in Eurocontrol documents at http://www.eurocontrol.int/publications/eur ocontrol-guidelines-short-term-conflict- alert-stca-part-i-iii

ANSP

Ground systems supplier

2019

Ground system infrastructur e

Safety nets	Display for

enhanced short term conflict alerts

Capability to indicate alerts on the radar screen of the controller working positions. References: Doc 100xx - Ground-based Safety Nets Manual Details in Eurocontrol documents at http://www.eurocontrol.int/publications/eur ocontrol-guidelines-short-term-conflict- alert-stca-part-i-iii

ANSP

Ground systems supplier

2019

Training	-	Training requirements for enhanced short term conflict alerts

Air traffic controller knowledge and reaction to alerts. Reference: Doc 100xx - Ground-based Safety Nets Manual - Details in Eurocontrol documents at http://www.eurocontrol.int/publications/eur ocontrol-guidelines-minimum-safe- altitude-warning-msaw-part-i-iii

ANSP 	2019

DEPLOYMENT APPLICABILITY

Operational conditions:

Enhanced STCA systems with aircraft parameters are applicable in all controlled airspace for all aircraft for which a controller has responsibility for separation or traffic information. Before operational use, the system must have been configured for the target airspace, to maximize the number of relevant alerts while keeping the number of unnecessary alerts to an acceptable level. It will bring incremental benefits as the number of ADS-B out capable aircraft or Mode S EHS capable aircraft increase.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Enhanced STCA use improves on basic STCA safety benefits by providing more relevant and earlier alerts for aircraft in vertical evolution. It also improves controllers’ efficiency by reducing the number of unnecessary alerts for aircraft in vertical evolution. | ANSP Aircraft operator |
|  | Improve situational awareness of ATCO | ANSP |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Improve mid-air collision avoidance (safety net) | ++ | KPI20: Number of aircraft accidents KPI23: Number of airprox/TCAS alert/loss of separation/near midair collisions/midair collisions (MAC) |

SNET-B1/2	Enhanced STCA in complex TMAs	Operational

Sixth edition of the GANP

Main Purpose	Assist the air traffic controller in preventing collision between aircraft, using position data from ground surveillance and taking into account possible crew intents linked to traffic patterns and ATC practices in complex TMAs.

New Capabilities Taking into account traffic patterns and ATC practices allows STCA systems to reduce the number of unnecessary alerts, to increase the number of relevant alerts and to alert earlier, compared to the basic STCA system in Block 0.

Description		This enhanced STCA works the same as the basic STCA system in Block 0, but, in addition of the current proximity test and the linear prediction test, performs the following tests:

(level-off prediction test) The vertical positions of aircraft in vertical evolution are extrapolated to level-off at the next reasonable FL.

(turn prediction test) The horizontal positions of aircraft in proximity of a final approach path are extrapolated to turn in alignment with this final approach path.

Care is also taken to setup a specific set of alerting parameters (horizontal threshold, vertical threshold and warning time) for each approach area within the TMAS, where unnecessary alerts could affect runway throughputs.

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? No

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

En-route

Departure

Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology option

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-technology option

ASUR-B0/3 - Cooperative Surveillance Radar Downlink of Aircraft

Parameters (SSR-DAPS)

Evolution

SNET-B0/1 - Short Term Conflict Alert (STCA)

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Safety nets	Controller

reaction to short term conflict alerts

Procedures for air traffic controllers reaction to short term conflict alerts. References: Doc 4444 - Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM)

ANSP 	2013

Airborne system capability

Safety nets	Surveillance

Ground systems supplier

system capabilities required for enhanced short term conflict alerts in complex TMAs

SSR transponder compliant with … ADS- B out compliant with … References: Doc 100xx - Ground-based Safety Nets Manual http://www.eurocontrol.int/publications/eur ocontrol-guidelines-short-term-conflict- alert-stca-part-i-iii

2019

Aircraft operator

Ground system infrastructur e

Safety nets	Surveillance

Ground systems supplier

ground capabilities required for enhanced short term conflict alerts in complex TMAs

SSR radar ADS-B in station References: Doc 100xx - Ground-based Safety Nets Manual Details in Eurocontrol documents at http://www.eurocontrol.int/publications/eur ocontrol-guidelines-short-term-conflict- alert-stca-part-i-iii

ANSP

2019

Ground system infrastructur e

Safety nets	Display for

enhanced short term conflict alerts in complex TMAs

Capability to indicate alerts on the radar screen of the controller working positions. References: Doc 100xx - Ground-based Safety Nets Manual Details in Eurocontrol documents at http://www.eurocontrol.int/publications/eur ocontrol-guidelines-short-term-conflict- alert-stca-part-i-iii

ANSP

2019

Aircraft manufacturer

Training	-	Training requirements for enhanced short term conflict alerts in complex TMAs

Air traffic controller knowledge and reaction to alerts. Reference: Doc 100xx - Ground-based Safety Nets Manual - Details in Eurocontrol documents at http://www.eurocontrol.int/publications/eur ocontrol-guidelines-minimum-safe- altitude-warning-msaw-part-i-iii

ANSP 	2019

### DEPLOYMENT APPLICABILITY

#### Operational conditions:

Enhanced STCA systems in complex TMAs are applicable in all controlled complex TMAs for all aircraft for which a controller has responsibility for separation or traffic information. The deployment of such an enhanced STCA is necessary in TMAs where the basic STCA would produce an unacceptable level of unnecessary alerts or would fail to produce timely alerts for traffic with frequent trajectory changes. Before operational use, the system must have been configured for the target airspace, to maximize the number of relevant alerts while keeping the number of unnecessary alerts to an acceptable level.

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Enhanced STCA use improves on basic STCA safety benefits by providing more relevant and earlier alerts for aircraft in horizontal and vertical evolution. It also improves controllers’ efficiency by reducing the number of unnecessary alerts for aircraft in horizontal and vertical evolution. | ANSP Aircraft operator |
|  | Improve situational awareness of ATCO | ANSP |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Improve mid-air collision avoidance (safety net) | ++ | KPI20: Number of aircraft accidents KPI23: Number of airprox/TCAS alert/loss of separation/near midair collisions/midair collisions (MAC) |

## SURF

Sixth edition of the GANP

Basic ATCO tools to manage traffic during ground Operational

operations

SURF-B0/1

Main Purpose 		To improve safety and efficiency during ground operations by providing proper indications to pilots and vehicle drivers.

New Capabilities   The guiding and routing service is delivered using visual aids and signals on the platform. Information is managed by the controller to provide pilots and vehicle drivers all necessary information to taxi and avoid incursion on the runway.

Description 		This element represents the provision of guidance and routing information to the pilot in order to manage the traffic in a safe and efficient way by the controller:

to confirm the routing of all aircraft and vehicles according to the defined identification procedures;  to prevent incursions on the runway using visual aids, stop bars in particular.

The Controller monitors and commands the lighting systems.

Maturity Level 	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

DEPLOYMENT APPLICABILITY

Operational conditions:

Runway incursion is traditionally prevented using lighting systems on the Airport. Stop bars and other systems are highly contributing to the fluidity and safety of taxi operations.

Main intended benefits:

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Taxi-out   Taxi-in

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

ENABLERS

There are currently no enablers.

| Type | Operational description | Benefitting stakeholder(s) |  |
| --- | --- | --- | --- |
|  | Safety of surface operations | Airport operator | ANSP Aircraft operator |
|  | Efficiency of surface operations | Airport operator | ANSP Aircraft operator |
|  | Improved Management of the platform | Airport operator |  |

| Direct benefits Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
|  | Prevention of Runway incursion | Airport operator ANSP Aircraft operator |
|  | Improve situational awareness of flight crew and vehicle driver |  |

| Airport operator | ANSP |  | Aircraft operator |
| --- | --- | --- | --- |
| Ground handling agent |  |  |  |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Flight time & distance | Avoid taxi-in additional time resulting from adverse conditions | ++ | KPI13: Taxi-in additional time |
| Efficiency | Flight time & distance | Avoid taxi-out additional time resulting from adverse conditions | ++ | KPI02: Taxi-out additional time |
| Safety |  | Avoid incorrect entries of aircraft or vehicles onto the runway protected area (without or contrary to ATC clearance or due to incorrect ATC clearance) | ++ | KPI20: Number of aircraft accidents KPI21: Number of runway incursions |
| Safety |  | Avoid incorrect runway crossings by aircraft or vehicles (without or contrary to ATC clearance or due to incorrect ATC clearance) | ++ | KPI20: Number of aircraft accidents KPI21: Number of runway incursions |
| Safety |  | Avoid incorrect taxiing (cases of non-conformance with clearance) | ++ | KPI20: Number of aircraft accidents |

Sixth edition of the GANP

Main Purpose

To better maintain ATCO awareness of ground operations.

New Capabilities   The surveillance service of A-SMGCS provides airport traffic situational awareness through the

position, identification and tracking of aircraft and vehicle suitably equipped on the aerodrome surface. Information is presented on the controller and airport operator display independent of visibility conditions and controller line of sight.

Comprehensive situational awareness of surface	Operational

operations

SURF-B0/2

Description 		This service represents the provision of surveillance information to the controller in order to manage the traffic in a more efficient way and allows the controller:

to confirm the identity of all participating vehicles according to the defined identification procedures;

to prevent collisions between all aircraft and vehicles especially in conditions when visual contact cannot be maintained;

to manually correlate (link a target with a call sign) targets for the rare cases where there is an operational need to, e.g. areas of poor cooperative surveillance coverage and the need to track non-cooperative targets such as towed aircraft;

to detect and indicate the position of potential intruders.

Maturity Level 	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

DEPLOYMENT APPLICABILITY

Operational conditions:

On complex Airports, the introduction of SMGCS functions is highly contributing to the safety and efficiency of surface operations by providing to the ATCO the necessary situation awareness to control operations.

Main intended benefits:

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Taxi-out   Taxi-in

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology option

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-technology option

ASUR-B0/2 - Multilateration cooperative surveillance systems (MLAT)

Relation-technology benefit

ASUR-B0/3 - Cooperative Surveillance Radar Downlink of Aircraft

Parameters (SSR-DAPS)

ENABLERS

There are currently no enablers.

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
|  | Safety of surface operations | Airport operator ANSP Aircraft operator |

| Type Direct benefits | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
|  | Efficiency of surface operations | Airport operator ANSP Aircraft operator |
|  | Improved management of the platform | Airport operator |
|  | Prevention of runway incursion | Airport operator ANSP Aircraft operator |
|  | Improve situational awareness of ATCO | ANSP |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Improve collision avoidance during taxi operations (safety net) | ++ | KPI20: Number of aircraft accidents |
| Safety |  | Avoid incorrect entries of aircraft or vehicles onto the runway protected area (without or contrary to ATC clearance or due to incorrect ATC clearance) | ++ | KPI20: Number of aircraft accidents KPI21: Number of runway incursions |
| Safety |  | Avoid incorrect presence of vacating aircraft or vehicles onto the runway protected area | ++ | KPI20: Number of aircraft accidents KPI21: Number of runway incursions |
| Safety |  | Avoid incorrect runway crossings by aircraft or vehicles (without or contrary to ATC clearance or due to incorrect ATC clearance) | ++ | KPI20: Number of aircraft accidents KPI21: Number of runway incursions |

Sixth edition of the GANP

Main Purpose

Detection by the ATCO of potentially unsafe situations with regard to runway operations.

New Capabilities

The ATCO will be provided with a short term conflicting alerting tool (A-SMGCS initial alerting

service) that monitors movements on or near the runway and detects conflicts between an aircraft and another vehicle as well as runway incursion by intruders. Appropriate alerts will be visualized on the ATCO display.

Operational

Initial ATCO alerting service for surface

operations

SURF-B0/3

Description 		This element represents the first step of A-SMGCS alerting service and is based on A-SMGCS surveillance. It takes into account elements such as:

the runway configuration of the airport (e.g. one, two or more runways);

the associated procedures (e.g. multiple line ups and reduced separation on the runway when approved by the ATS authorities);

the position and type of the aircraft and vehicles (e.g. arrival, departure or vehicle) according to the set time parameters and their relative speeds and positions when within or about to enter a predefined area around the runway;

aircraft in the vicinity of the runway (e.g. on final approach, climb out and helicopters crossing);  meteorological conditions.

Maturity Level 	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes

DEPLOYMENT APPLICABILITY

Operational conditions:

On complex airports, the complexity of the infrastructure and the traffic can induce possibility of errors in the management of taxi operations.

Main intended benefits:

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Taxi-out   Departure   Arrival   Taxi-in

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology option

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-technology option

ASUR-B0/2 - Multilateration cooperative surveillance systems (MLAT)

Relation-technology benefit

ASUR-B0/3 - Cooperative Surveillance Radar Downlink of Aircraft

Parameters (SSR-DAPS)

Evolution

SURF-B0/1 - Basic ATCO tools to manage traffic during ground operations

ENABLERS

There are currently no enablers.

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
|  | Safety of surface operations | Airport operator ANSP Aircraft operator |

| Type Direct benefits | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
|  | Prevention of runway incursion | Airport operator ANSP Aircraft operator |
|  | Improve situational awareness of ATCO | ANSP |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Improve runway collision avoidance (safety net) | ++ | KPI20: Number of aircraft accidents |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

SURF-B1/1	Advanced features using visual aids to support

traffic management during ground operations

Operational

Sixth edition of the GANP

Main Purpose	To improve surface operations with the aim to reduce taxi time and fuel burn, potential mistakes.

New Capabilities		Advanced features including “Follows the Greens” (FTG) and Variable Message Panels are used to optimize routing during taxi operations. The lighting system is used to direct the aircraft, making the guidance safer, as errors are minimized.

Lighting system for other vehicles than aircraft is connected to the SMGCS in order to optimize ground circulation and prevent collision.

Description		Advanced features including FTG and Variable Message Panels are used to optimize routing during taxi operations. The lighting system is used to direct the aircraft, making the guidance safer, as errors are minimized.

Lighting system for other vehicles than aircraft is connected to the SMGCS in order to optimize ground circulation and prevent collision.

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? No

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Taxi-out

Taxi-in

DEPLOYMENT APPLICABILITY

Operational conditions:

On complex airports, the management of the platform is introducing numerous changes and an increased complexity in managing maintenance or construction together with ensuring safety and efficiency of operations. The introduction of dynamic aids is highly improving accuracy of the navigation on the surface and as such safety and efficiency.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| Airport operator | ANSP |  | Aircraft operator |
| --- | --- | --- | --- |
| Ground handling agent |  |  |  |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

SURF-B0/1 - Basic ATCO tools to manage traffic during ground operations

Relation-operational need

ACDM-B0/1 - Airport CDM Information Sharing (ACIS)

ENABLERS

There are currently no enablers.

| Type Operational description Benefitting stakeholder(s) |  |
| --- | --- |
| Direct benefits | Safety of surface operations Airport operator ANSP Aircraft operator |
|  | Airport operator ANSP Efficiency of surface operations Aircraft manufacturer |
|  | Improved management of the platform Airport operator |
|  | Prevention of runway incursion Airport operator ANSP Aircraft operator |
|  | Improve situational awareness of flight crew and vehicle driver |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Flight time & distance | Avoid taxi-in additional time resulting from adverse conditions | ++ | KPI13: Taxi-in additional time |
| Efficiency | Flight time & distance | Avoid taxi-out additional time resulting from adverse conditions | ++ | KPI02: Taxi-out additional time |
| Safety |  | Improve collision avoidance during taxi operations (safety net) | ++ | KPI20: Number of aircraft accidents |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Avoid incorrect taxiing (cases of non-conformance with clearance) | ++ | KPI20: Number of aircraft accidents |

SURF-B1/2	Comprehensive pilot situational awareness on the airport surface

Operational

Sixth edition of the GANP

Main Purpose	To improve ground operations based on increasing pilot’s situational awareness and safety especially at taxiway and runway intersections, as well as for aircraft landing and taking off.

New Capabilities In addition to display of the airport layout (showing taxiways, runways, fixed obstacles) and the own aircraft position, the pilot has an improved situational awareness thanks to the additional display of surrounding traffic (incl. both aircraft and optionally airport vehicles).

Description		The pilot can visualize surrounding traffic to be presented on traffic computer and display. Different technologies enable this capability, among which ADS-B OUT/ADS-B IN. In order to maximize the benefits, it is suitable that all aircraft be equipped in a homogeneous manner. However, a transition period can be observed and a partial equipage will result in the display of only the appropriately equipped aircraft.

Maturity Level	Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Taxi-out   Departure   Arrival   Taxi-in

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-technology need	ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-operational need	CSEP-B1/1 - Basic airborne situational awareness during flight operations

(AIRB)

### ENABLERS

There are currently no enablers.

### DEPLOYMENT APPLICABILITY

#### Operational conditions:

On complex airports, the complexity of the infrastructure and the traffic can induce possibility of errors in the management of taxi operations. Full pilot situational awareness will first compensate possible mistakes from the controllers but also assist in improving the efficient management of taxi operations.

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |  |  |
| --- | --- | --- | --- | --- |
| Direct benefits | Safety of surface operations | Airport operator | ANSP | Aircraft operator |
|  | Efficiency of surface operations | Airport operator | ANSP | Aircraft operator |
|  | Improved management of the platform | Airport operator |  |  |
|  | Prevention of runway incursion | Airport operator | ANSP | Aircraft operator |
|  | Improve situational awareness of flight crew | Aircraft operator |  |  |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Improve collision avoidance during taxi operations (safety net) | ++ | KPI20: Number of aircraft accidents |
| Safety |  | Avoid incorrect entries of aircraft or vehicles onto the runway protected area (without or contrary to ATC clearance or due to incorrect ATC clearance) | ++ | KPI20: Number of aircraft accidents KPI21: Number of runway incursions |
| Safety |  | Avoid incorrect presence of vacating aircraft or vehicles onto the runway protected area | ++ | KPI20: Number of aircraft accidents KPI21: Number of runway incursions |
| Safety |  | Avoid incorrect runway crossings by aircraft or vehicles (without or contrary to ATC clearance or due to incorrect ATC clearance) | ++ | KPI20: Number of aircraft accidents KPI21: Number of runway incursions |

SURF-B1/3	Enhanced ATCO alerting service for surface

operations

Operational

Sixth edition of the GANP

Main Purpose		The enhanced A-SMGCS alerting service anticipates potential runway conflicts, runway incursion and other hazardous situations on the aerodrome surface.

New Capabilities	Early detection of aircraft and vehicles that are not following given clearances/ instructions or provision of alerts when clearances given by the controller do not comply with local ATC rules/procedures.

Description		The A-SMGCS Alerting service for controllers is complemented with the detection of conflicting ATC Clearances (CATC) given by the controller (e.g. Line-up versus Land on same runway) and with the detection of non-conformance to procedures or instructions (e.g. route deviation). An electronic clearance input means is used by the controller to make the clearances known to the system.

Surveillance data and routing information are also used by the logic to generate alerts to the controller.

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Taxi-out   Departure   Arrival   Taxi-in

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-operational need	SURF-B0/2 - Comprehensive situational awareness of surface operations Evolution	SURF-B0/3 - Initial ATCO alerting service for surface operations

Relation-operational need	SURF-B1/4 - Routing service to support ATCO surface operations

management

### ENABLERS

There are currently no enablers.

### DEPLOYMENT APPLICABILITY

#### Operational conditions:

On complex airports, the complexity of the infrastructure and the traffic can induce possibility of errors in the management of taxi operations.

| Type | Operational description | Benefitting stakeholder(s) |  |  |
| --- | --- | --- | --- | --- |
| Direct benefits | Safety of surface operations | Airport operator | ANSP | Aircraft operator |
|  | Prevention of runway incursion | Airport operator | ANSP | Aircraft operator |
|  | Improve situational awareness of ATCO | ANSP |  |  |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Improve early detection of conflicting ATC Clearances (CATC) related to runway usage | ++ | KPI20: Number of aircraft accidents |
| Safety |  | Improve early detection of conflicting ATC Clearances (CATC) related to taxi operations | ++ | KPI20: Number of aircraft accidents |

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

SURF-B1/4	Routing service to support ATCO surface

operations management

Operational

Sixth edition of the GANP

Main Purpose		To improve pre-departure and departure sequencing by provision of accurate taxi times and efficient routing service.

New Capabilities		The A-SMGCS routing service calculates individual routes for mobiles for representation to the controller in order to support the runway sequencing strategy.

Description		The A-SMGCS routing service calculates individual routes for mobiles based on known airport parameters and constraints or following an interaction by the controller. The controller is presented with planned or cleared routes and has means to modify these routes or to create new route if necessary. Information is updated in real time in order to improve predictability of surface operations.

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

DEPLOYMENT APPLICABILITY

Operational conditions:

On complex airports, the management of the platform is introducing numerous changes and an increased complexity in managing maintenance or construction together with ensuring safety and efficiency of operations. Appropriate and potentially tailored routing services can highly improve safety and efficiency of airport surface management. When fully consistent with ACDM and Runway sequencing strategies, it clearly contributes to the performance of the airport and surrounding airspace management.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

PLANNING LAYERS

Tactical-During ops

Tactical-Pre ops

OPERATIONS

Taxi-in

Taxi-out

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

SURF-B0/2 - Comprehensive situational awareness of surface operations

Relation-operational need

RSEQ-B0/1 - Arrival Management

Relation-operational need

RSEQ-B0/2 - Departure Management

ENABLERS

There are currently no enablers.

| Type | Operational description | Benefitting stakeholder(s) |  |
| --- | --- | --- | --- |
| Direct benefits | Safety of surface operations | Airport operator ANSP | Aircraft operator |
|  | Efficiency of surface operations | Airport operator ANSP | Aircraft operator |
|  | Improved management of the platform | Airport operator |  |
|  | Prevention of runway incursions | Airport operator ANSP | Aircraft operator |
|  | Improve situational awareness of ATCO | ANSP |  |
| Indirect benefits | Improved efficiency of terminal airspace management and network operations. | ANSP Aircraft operator |  |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Flight time & distance | Avoid taxi-in additional time resulting from adverse conditions | ++ | KPI13: Taxi-in additional time |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Flight time & distance | Avoid taxi-out additional time resulting from adverse conditions | ++ | KPI02: Taxi-out additional time |
| Efficiency | Flight time & distance | Introduce 4D planning of taxi-in surface movements | ++ | KPI13: Taxi-in additional time |
| Efficiency | Flight time & distance | Introduce 4D planning of taxi-out surface movements | ++ | KPI02: Taxi-out additional time |

SURF-B1/5	Enhanced vision systems for taxi operations	Operational

Sixth edition of the GANP

Main Purpose		Allow for improved navigation by visual reference, even during conditions of low-light or weather obscuration such as fog.

New Capabilities	The addition of cockpit enhanced vision capabilities will improve flight crew awareness of own ship position, and reduce navigation errors during periods of reduced visibility. In addition, improved situational awareness of aircraft position will allow for more confidence by the flight crew in the conduct of the taxi operation during periods of reduced visibility and ensure accurate application of received clearances.

Description		Additional avionics add electromagnetic sensors outside the visible light spectrum (e.g., infrared cameras, millimetre wave radar). These sensors will allow for improved navigation by visual reference, even during conditions of low-light or weather obscuration such as fog. Presentation to the flight crew may be through an instrument panel display or via heads-up display (HUD), etc.

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? No

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Taxi-out   Taxi-in

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-information benefit	AMET-B0/1 - Meteorological observations products

DEPLOYMENT APPLICABILITY

Operational conditions:

On complex airports, the capacity of the airport may decrease a lot in LVC due to surface operations. The introduction of enhance vision systems on board aircraft able to recognize lightings and ground indications can highly improve accuracy of the navigation on the surface and as such safety and efficiency and limit negative impact.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

ENABLERS

There are currently no enablers.

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Efficiency of surface operations | Airport operator ANSP Aircraft operator |
|  | Improve situational awareness of flight crew | Aircraft operator |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Flight time & distance | Avoid longer taxi-in due to taxi errors | ++ | KPI13: Taxi-in additional time |
| Efficiency | Flight time & distance | Avoid longer taxi-out routes due to taxi errors | ++ | KPI02: Taxi-out additional time |
| Efficiency | Flight time & distance | Avoid slow taxi-in due to ATC and/or pilot | ++ | KPI13: Taxi-in additional time |
| Efficiency | Flight time & distance | Avoid slow taxi-in due to weather conditions | ++ | KPI13: Taxi-in additional time |
| Efficiency | Flight time & distance | Avoid slow taxi-out due to ATC and/or pilot | ++ | KPI02: Taxi-out additional time |
| Efficiency | Flight time & distance | Avoid slow taxi-out due to weather conditions | ++ | KPI02: Taxi-out additional time |
| Efficiency | Flight time & distance | Reduce ATC constraints during low visibility taxi-in | ++ | KPI13: Taxi-in additional time |
| Efficiency | Flight time & distance | Reduce ATC constraints during low visibility taxi-out | ++ | KPI02: Taxi-out additional time |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Avoid incorrect entries of aircraft or vehicles onto the runway protected area (without or contrary to ATC clearance or due to incorrect ATC clearance) | ++ | KPI20: Number of aircraft accidents KPI21: Number of runway incursions |
| Safety |  | Avoid incorrect presence of vacating aircraft or vehicles onto the runway protected area | ++ | KPI20: Number of aircraft accidents KPI21: Number of runway incursions |
| Safety |  | Avoid incorrect runway crossings by aircraft or vehicles (without or contrary to ATC clearance or due to incorrect ATC clearance) | ++ | KPI20: Number of aircraft accidents KPI21: Number of runway incursions |
| Safety |  | Avoid incorrect taxiing (cases of non-conformance with clearance) | ++ | KPI20: Number of aircraft accidents |
| Safety |  | Improve early detection of conflicting ATC Clearances (CATC) related to taxi operations | ++ | KPI20: Number of aircraft accidents |

SURF-B2/1	Enhanced surface guidance for pilots and vehicle drivers

Operational

Sixth edition of the GANP

Main Purpose		To improve the guidance of pilots and vehicle drivers on the aerodrome surface. Depending from the level of equipage of aircrafts and vehicles, the operational objective may be achieved either by airport ground equipment or through on-board capabilities.

New Capabilities	Automatic triggering of airport ground signs according to the route and clearances issued by ATC.

Description		The A-SMGCS guidance service is using the routing service in conjunction with ATCO inputs to allow the automated switching of Taxiway Centreline Lights (TCL) and/or stop bars. The guidance service improves the movement of mobiles on the movement area and reduces the workload of controllers.

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes

DEPLOYMENT APPLICABILITY

Operational conditions:

On complex airports with very demanding traffic, the accuracy of the ground trajectory management is conditioning the overall performance of the management of the platform together with ensuring safety and efficiency of operations.

Appropriate and potentially tailored routing services can highly improve safety and efficiency of Airport surface management. When fully consistent with ACDM and Runway sequencing strategies, it clearly contributes to the performance of the Airport and surrounding Airspace management.

Main intended benefits:

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Taxi-out   Taxi-in

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

SURF-B0/1 - Basic ATCO tools to manage traffic during ground operations

Evolution

SURF-B1/1 - Advanced features using visual aids to support traffic

management during ground operations

ENABLERS

There are currently no enablers.

| Type | Operational description | Benefitting stakeholder(s) |  |
| --- | --- | --- | --- |
| Direct benefits | Safety of surface operations | Airport operator ANSP | Aircraft operator |
|  | Safety of surface operations | Airport operator ANSP | Aircraft operator |
|  | Improved management of the platform | Airport operator |  |
|  | Prevention of runway incursion | Airport operator ANSP | Aircraft operator |
|  | Improve situational awareness of flight crew and vehicle driver | Airport operator ANSP Ground handling agent | Aircraft operator |
| Indirect benefits | Improved efficiency of terminal airspace management and network operations | ANSP Aircraft operator |  |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Flight time & distance | Avoid taxi-in additional time resulting from adverse conditions | ++ | KPI13: Taxi-in additional time |
| Efficiency | Flight time & distance | Avoid taxi-out additional time resulting from adverse conditions | ++ | KPI02: Taxi-out additional time |
| Safety |  | Improve collision avoidance during taxi operations (safety net) | ++ | KPI20: Number of aircraft accidents |
| Safety |  | Avoid incorrect taxiing (cases of non-conformance with clearance) | ++ | KPI20: Number of aircraft accidents |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

SURF-B2/2	Comprehensive vehicle driver situational

awareness on the airport surface

Operational

Sixth edition of the GANP

Main Purpose		Expansion of situation awareness to vehicle’s drivers by the provision of own position and surrounding traffic position on a display in the vehicle. Considered vehicles can be small UAS used for airport specific functions. The vehicle driver is informed about potential and actual risk of collision with aircraft and infringement of restricted or closed areas.

New Capabilities		Information regarding the surrounding traffic (including both aircraft and airport vehicles) during taxi and runway operations is displayed in the vehicle driver's cockpit. The system detects hazardous situations and provides the vehicle driver with the appropriate alert, either generated by the on-board system or uplinked from a centralized airport function.

Description		For the vehicles operating on the manoeuvring area of the airport, a display on-board of the vehicle is showing the surrounding traffic and own position to enhance the situational awareness of the vehicle driver and providing the appropriate alerts to ensure safer operations of vehicles on the manoeuvring area.

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

- Does it imply a change to levels of automation? Yes

DEPLOYMENT APPLICABILITY

Operational conditions:

On complex airports with very demanding traffic, vehicle drivers may have difficulty to get the full situation awareness. The provision of a comprehensive situational awareness and associated alerts through appropriate means associated with appropriate alerts will ensure safety and efficiency of operations.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Taxi-out   Departure   Arrival   Taxi-in

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

ENABLERS

There are currently no enablers.

| Type | Operational description | Benefitting stakeholder(s) |  |
| --- | --- | --- | --- |
| Direct benefits | Safety of surface operations | Airport operator | ANSP Aircraft operator |
|  | Efficiency of surface operations | Airport operator | ANSP Aircraft operator |
|  | Improved management of the platform | Airport operator |  |
|  | Prevention of runway incursion | Airport operator | ANSP Aircraft operator |
|  | Improve situational awareness of flight crew | Aircraft operator |  |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Improve collision avoidance during taxi operations (safety net) | ++ | KPI20: Number of aircraft accidents |
| Safety |  | Avoid incorrect entries of aircraft or vehicles onto the runway protected area (without or contrary to ATC clearance or due to incorrect ATC clearance) | ++ | KPI20: Number of aircraft accidents KPI21: Number of runway incursions |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Avoid incorrect presence of vacating aircraft or vehicles onto the runway protected area | ++ | KPI20: Number of aircraft accidents KPI21: Number of runway incursions |
| Safety |  | Avoid incorrect runway crossings by aircraft or vehicles (without or contrary to ATC clearance or due to incorrect ATC clearance) | ++ | KPI20: Number of aircraft accidents KPI21: Number of runway incursions |
| Safety |  | Improve runway collision avoidance (safety net) | ++ | KPI20: Number of aircraft accidents |

SURF-B2/3	Conflict alerting for pilots for runway operations	Operational

Sixth edition of the GANP

Main Purpose		To improve safety during runway operations by providing traffic indication and alerts to pilots and/or vehicle drivers.

New Capabilities	The on-board system detects potential and actual risk of collision with other traffic during runway operations and provides the pilot with the appropriate alert.

Description		This enhancement represents a key on-board feature to significantly decrease the risk of conflict with any mobile on or near the runway, improving safety on airport surface. Aircraft data is broadcasted with the proper level of performance and quality in order to provide adequate alerts to the pilots.

Broadcasted aircraft data can also be presented on board airport ground vehicles

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Taxi-out   Departure   Arrival   Taxi-in

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Evolution	SURF-B1/2 - Comprehensive pilot situational awareness on the airport surface

### ENABLERS

There are currently no enablers.

### DEPLOYMENT APPLICABILITY

#### Operational conditions:

Runway incursion prevention.

| Airport operator | ANSP |  | Aircraft operator |
| --- | --- | --- | --- |
| Ground handling agent |  |  |  |

Main intended benefits:

| Type Operational description Benefitting stakeholder(s) |  |
| --- | --- |
| Direct benefits | Prevention of runway incursions. Airport operator ANSP Aircraft operator |
|  | Improve situational awareness of flight crew and vehicle driver |
| The enabling infrastructure comprising traffic position broadcast as well as Data Indirect benefits Airport operator ANSP Aircraft operator Link services is serving at least the general operational efficiency of the airport. |  |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Improve runway collision avoidance (safety net) | ++ | KPI20: Number of aircraft accidents |

Sixth edition of the GANP

Main Purpose

The objective of this element is to achieve the expected performance of the surface

management taking account of all aspects of ground operations.

New Capabilities

Taking in account all aspects of ground operations and based on optimization algorithms, the A-

SMGCS routing and guiding service is providing automation for the management of surface traffic in complex situations following ACDM agreed strategies. The controller workload is reduced. The ATCO keeps the ability to be in manual mode.

Operational

Optimization of surface traffic management in

complex situations

SURF-B3/1

Description 		Using the capabilities provided in the previous blocks, the management of surface traffic in complex situations can be automated. The controller keeps the possibility to operate in manual, semi- automated or automated mode.

Maturity Level 	Concept

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? Yes

DEPLOYMENT APPLICABILITY

Operational conditions:

On complex airports with very demanding traffic, full and reliable knowledge of the situation and associated routing services is allowing for automation and optimization of surface management in complex situations and at all conditions.

Main intended benefits:

PLANNING LAYERS

Tactical-During ops

Tactical-Pre ops

OPERATIONS

Taxi-in

Taxi-out

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

SURF-B1/3 - Enhanced ATCO alerting service for surface operations

Relation-operational need

SURF-B2/1 - Enhanced surface guidance for pilots and vehicle drivers

Relation-operational need

SURF-B2/2 - Comprehensive vehicle driver situational awareness on the

airport surface

ENABLERS

There are currently no enablers.

| Type Operational description Benefitting stakeholder(s) |  |  |
| --- | --- | --- |
| Direct benefits | Safety of surface operations | Airport operator ANSP Aircraft operator |
|  | Efficiency and predictability of surface Airport operator ANSP Aircraft operator operations |  |
|  | Improved management of the platform Airport operator |  |
|  | Prevention of runway incursion Airport operator ANSP Aircraft operator |  |

Type

Operational description

Benefitting stakeholder(s)

Indirect benefits

Improved efficiency of terminal airspace

management and network operations

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Aircraft operator

ANSP

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Efficiency | Flight time & distance | Avoid taxi-in additional time resulting from adverse conditions | ++ | KPI13: Taxi-in additional time |
| Efficiency | Flight time & distance | Avoid taxi-out additional time resulting from adverse conditions | ++ | KPI02: Taxi-out additional time |
| Efficiency | Flight time & distance | Introduce 4D planning of taxi-in surface movements | ++ | KPI13: Taxi-in additional time |
| Efficiency | Flight time & distance | Introduce 4D planning of taxi-out surface movements | ++ | KPI02: Taxi-out additional time |

GANP PORTAL

# ASBU ELEMENTS

SWIM

TBO

WAKE

| Functional Description | Enablers | Deployment Applicability | Performance Impact Assessment |
| --- | --- | --- | --- |

## SWIM

SWIM-B2/1	Information service provision	Information

Sixth edition of the GANP

Main Purpose	To define the requirements for an information service provider to make aviation-related information available as an information service.

New Capabilities		Once an information service is created by an information service provider, it can be discovered by the ATM community through its service overview made available via a registry. The service overview includes metadata specifying the characteristics of the provided information service including the means by which the service is accessed by the authorized users.

SWIM Information services typically apply publish/subscribe or request/reply message exchange patterns. SWIM information services facilitate integration with automation systems.

Description		The service overview is the means for the information service provider to publicize the characteristics of an information service. Based on the service characteristics provided, potential information service consumers can evaluate whether or not to use that information service. The service overview includes, for example, a description of the information, the exchange format, the service performance, and the access rules. The service overview should be exposed preferably via a registry and available to all stakeholders, while the access to the information service may only be granted to authorized users.

An information service provider can provide information falling into one of the traditional information domains (i.e., AIM, FF-ICE, MET or surveillance), or any other information deemed appropriate.

Information service providers have to have a quality management system in place to ensure the quality of the information and the quality of the information service provided.

State aviation entities involved in civil-military cooperation and coordination –ground centres or aircraft– will be able to act as SWIM information service provider.

Maturity Level	Validation

Human Factor Considerations

| ATM planning | Strategical |  | Pre-tactical |  |
| --- | --- | --- | --- | --- |
| Tactical-Pre ops |  | Tactical-During ops |  |  |
| Post operations |  |  |  |  |

| Taxi-out | Departure |  | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- | --- |
| Turn-around |  |  |  |  |  |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational benefit

SWIM-B2/5 - Global SWIM processes

Relation-operational benefit

SWIM-B2/3 - SWIM registry

Relation-technology need

COMI-B1/1 - Ground-Ground Aeronautical Telecommunication

Network/Internet Protocol Suite (ATN/IPS)

### ENABLERS

ATM SWIM

service provider

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Information exchange

Procedures for how to publish and access a service overview

Reference guidance: Manual on System Wide Information Management (SWIM) Implementation

2023

Operational procedures

Information exchange

Procedures for defining the content of a service overview and how an information service can be discovered via a registry.

Standards and procedures: Future ICAO PANS-IM

2023

PLANNING LAYERS

OPERATIONS

Information service provider

Information service provider

Operational procedures

Information exchange

Procedures for quality management system

Procedures for quality management system to ensure the quality of the information services. Reference guidance material: Manual on System Wide Information Management (SWIM) Implementation

2023

Ground	Information	Automated

system	exchange	systems capable infrastructur		to provide

e	information over

an IP network following the required message exchange patterns.

Automated systems capable to exchange

information: - Over IP network (OSI Layer 1 to 5) Standards and guidance material: ICAO Annex 10 - Aeronautical Telecommunications Vol III -Rules to access an IP network Future Manual on System Wide Information Management (SWIM) Implementation - Following exchange patterns required to support the service provision (e.g. publish/subscribe or request/reply) (OSI Layer 7)

2023

Ground	Information

system	exchange infrastructur

e

Automated

systems with logon and authentication mechanisms

OSI Layer 5 Guidance material:Future

Manual on system wide information management (SWIM) Implementation

2023

Information	-

exchange model

Information

exchange models

Information exchange models such as

FIXM, AIXM, IWXXM,... These exchange models are to be selected by the information service providers depending on the domain of the information to be provided.

Airport operator 2018

ANSP

Airspace user

Training

-

Training

requirements for information service provision.

Training on how to define, develop and

provide information services.

Information

service provider 2023

ATM SWIM

service provider

ATM SWIM

service provider

Information service provider

Sixth edition of the GANP

Main Purpose

To define the requirements for an information service consumer to discover and access aviation-

related information provided via information services.

New Capabilities	An information service consumer has access to a registry to discover available information services.

A registry's search and filter capability helps an information service consumer discover an information service appropriate to their specific needs. The information service consumer needs to implement publish/subscribe and request/reply message exchange patterns over an internet protocol-based communication infrastructure in order to consume information services.

Information

Information service consumption

SWIM-B2/2

Description 	An information service consumer makes use of a registry to discover available information services.

A registry contains a listing of service overviews which provide details about the information services. The information service consumer makes use of the registry’s search and filter capability to identify and select the information service appropriate to their specific needs (e.g. quality of service requirements).

Once an information service is selected, assuming that the information service consumer is authorized to access it, the information service consumer obtains the access point information and implements one of the available message exchange patterns to obtain the information. The information (e.g. AIM, FF-ICE or MET) can be readily consumed by automation systems.

State aviation entities involved in civil-military cooperation and coordination –ground centres or aircraft– will be able to act as SWIM information service consumers.

Maturity Level 	Validation

PLANNING LAYERS

OPERATIONS

Human Factor Considerations

| ATM planning | Strategical |  | Pre-tactical |  |
| --- | --- | --- | --- | --- |
| Tactical-Pre ops |  | Tactical-During ops |  |  |
| Post operations |  |  |  |  |

| Taxi-out | Departure |  | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- | --- |
| Turn-around |  |  |  |  |  |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

SWIM-B2/1 - Information service provision

Relation-operational benefit

SWIM-B2/3 - SWIM registry

Relation-technology need

COMI-B1/1 - Ground-Ground Aeronautical Telecommunication

Network/Internet Protocol Suite (ATN/IPS)

Relation-operational benefit

SWIM-B2/5 - Global SWIM processes

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Information	-

exchange model

Information

exchange models

Information exchange models such as

FIXM, AIXM, IWXXM,... These exchange models are to be selected by the information service providers depending on the domain of the information to be provided.

2018

Airspace user

ANSP

Airport operator

Operational Information	Procedures to

procedures	exchange	access registry

and information services

Procedures to be followed to access the

registry and how to access information services as defined in the service overview. Guidance material: Future Manual on System Wide Information Management (SWIM) Implementation.

2023

Ground	Information	Automated

system	exchange	systems capable infrastructur		to consume

e	information over

an IP network following required message exchange patterns

Automated systems capable to exchange

information: - Over IP network (OSI Layer 1 to 5) Standards and guidance material: ICAO Annex 10 - Aeronautical Telecommunications Vol III -Rules to access an IP network Future Manual on System Wide Information Management (SWIM) Implementation - Following exchange patterns required to support the service provision (e.g. publish/subscribe or request/reply) (OSI Layer 7)

2023

Ground	Information

system	exchange infrastructur

e

Automated

systems capable of accessing a registry

Guidance material: Future Manual on

System Wide Information Management (SWIM) Implementation

2023

Information service consumer

Information service consumer

Information service consumer

SWIM-B2/3	SWIM registry	Information

Sixth edition of the GANP

Main Purpose		A registry is a means for the information service producer to publicize and for an information service consumer to discover information services within a SWIM environment.

New Capabilities		A registry contains a listing of service overviews. It has a search and filter capability to identify and select information services, permits managing service overviews, and provides user access control mechanism.

Description 			The registry is a means to link information providers with information consumers and thereby facilitates the exchange of information. The main purpose of a registry is to enable discoverability by making available a service overview describing information services in a structured and searchable format.

The registry permits an information service provider to enter and update a service overview, has a search and filter capability, and provides controlled user access. In addition, it facilitates service lifecycle management, including versioning of an information service. A registry can be provided at State, regional or multi-national levels.

State aviation entities involved in civil-military cooperation and coordination –ground centres or aircraft– will be enabled to access the SWIM registry.

Maturity Level 		Ready for implementation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

- Does it imply processing of new information by the user? Yes

- Does it imply the use of new equipment? Yes

PLANNING LAYERS

OPERATIONS

Does it imply a change to levels of automation? Yes

| ATM planning | Strategical |  | Pre-tactical |  |
| --- | --- | --- | --- | --- |
| Tactical-Pre ops |  | Tactical-During ops |  |  |
| Post operations |  |  |  |  |

| Taxi-out | Departure |  | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- | --- |
| Turn-around |  |  |  |  |  |

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### ENABLERS

SWIM Registry Manager

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Ground system infrastructur e

Information exchange

*Automated system capable of operating and managing a registry

Guidance Material: Future Manual on System Wide Information Management (SWIM) Implementation

2023

Operational procedures

Information exchange

Procedures to populate the registry

Procedures to publish the service overview of an information service in the registry. Reference: Guidance material: Future Manual on System Wide Information Management (SWIM) Implementation

2023

SWIM Registry Manager

Operational procedures

Information exchange

Procedure for registry management

Provisions addressing registry management and content. Reference: PANS-IM

2023

SWIM Registry Manager

Operational procedures

Information exchange

Procedures to find the registry

Procedures for publishing the reference to the registry that contain service overviews of the information services provided.

SWIM Registry Manager

Reference: Future Manual on System Wide Information Management (SWIM) Implementation

2023

SWIM Registry Manager

Training	-	Training requirements for SWIM registry

Training on how to set up and operate a registry for the SWIM registry manager.

2023

SWIM-B2/4	Air/Ground SWIM for non-safety critical

information

Information

Sixth edition of the GANP

Main Purpose	Exchange of non-safety critical information with the aircraft to improve operational awareness and efficiency.

New Capabilities		Air/Ground (A/G) SWIM leverages inflight internet connectivity (e.g., broadband) capabilities, along with the air navigation service provider’s ground SWIM infrastructure, to enable information exchange with the aircraft.

Description		A/G SWIM will enable airspace users, specifically flight crew, to make information available to the air navigation service provider (ANSP), including reroute preferences and air reports / airspace conditions. A/G SWIM will also allow the flight crew to have access to more information in a timely manner. In an A/G SWIM environment, the management and use of information on-board the aircraft is expanded.

A/G SWIM expands information exchange between the aircraft (including its automation systems) and ANSPs without the constraints imposed by voice communications. A/G SWIM requires flight deck applications like electronic flight bags (EFBs) or other devices to be enabled for the exchange of information. Flight deck application access to SWIM will allow the flight crew to obtain, for example, airspace constraint information and flow restrictions to assist them in re-planning their flights, provide them with information that supports negotiation with ATFM, or enable coordination of flight plan updates initiated by an airline operations center.

State aircraft will have access to A/G SWIM to obtain and share information needed for their operations –especially when operating in civil controlled airspace–, while giving due regard for military information security aspects.

Maturity Level	Validation

Human Factor Considerations

### PLANNING LAYERS

Tactical-Pre ops

Tactical-During ops

DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

### OPERATIONS

| Taxi-out | Departure |  | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- | --- |
| Turn-around |  |  |  |  |  |

Evolution	SWIM-B2/1 - Information service provision

Evolution	SWIM-B2/2 - Information service consumption

Relation-technology need	COMI-B1/1 - Ground-Ground Aeronautical Telecommunication

Network/Internet Protocol Suite (ATN/IPS)

Relation-technology need	COMI-B2/3 - Links meeting requirements for non-safety critical

communication

### ENABLERS

Information service consumer

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Information exchange

Procedures to access registry and information services

Procedures to be followed to access the registry and how to access information services as defined in the service overview. Guidance material: Future Manual on System Wide Information Management (SWIM) Implementation.

2023

Ground system infrastructur e

Information exchange

Automated systems capable to consume information over an IP network following required message exchange patterns

Automated systems capable to exchange information: - Over IP network (OSI Layer 1 to 5) Standards and guidance material: ICAO Annex 10 - Aeronautical Telecommunications Vol III -Rules to access an IP network Future Manual on System Wide Information Management (SWIM) Implementation - Following exchange patterns required to support the service provision (e.g. publish/subscribe or request/reply) (OSI Layer 7)

2023

Operational procedures

Information exchange

Procedures for the exchange of non-safety critical information with the aircraft

Procedures to be followed for the exchange of non-safety critical information with the aircraft. Reference: Future Manual on System Wide Information Management (SWIM) Implementation

Airspace user  2025

Airborne system capability

Information exchange

Airborne SWIM capability for exchanging non- safety critical information

Onboard systems capable of exchanging non-safety critical information (e.g. EFB).

Airspace user  2025

Ground system infrastructur e

Information exchange

G/G SWIM

communications between ANSP and airspace user to exchange non-safety critical information

G/G SWIM communications between ANSP and airspace user to exchange non-safety critical information.

ANSP

Airspace user

2025

Information service consumer

Sixth edition of the GANP

Information

Global SWIM processes

SWIM-B2/5

Main Purpose 		SWIM governance helps establish global access to aviation-related information by connecting information service providers and consumers within a global interoperability framework.

New Capabilities   SWIM governance comprises a set of standards, policies and processes in support of, for example, rights of usage of information, quality of service aspects and trust. Within a global interoperability framework, registries will be inter-connected to provide a single access point for information services.

Description 		SWIM governance comprises a set of standards, policies and processes to ensure that globally interoperable information services are provided by reliable and trusted information service providers and accessed by authorized consumers. SWIM governance helps establish confidence by addressing topics such as rights of usage and aspects related to quality of service.

The establishment of SWIM governance entails activities which can be applied at different levels (e.g., national, regional, global and organizational). These activities include establishment of a common set of standards, policies and processes for information, information services and technical infrastructure; interconnection of registries; definition and establishment of governance structures; promotion of interoperability within a global framework; definition of common processes to be followed; and definition of the transition to a SWIM environment through national or regional arrangements. The interconnection of registries will provide information consumers with a single access point at a global level to all available information services.

SWIM governance considers and supports involvement of State aviation authorities, with the aim of increasing civil-military cooperation and coordination functions in SWIM, while giving due regard for military information security aspects.

Maturity Level 	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? No

- Does it imply the use of new equipment? No

PLANNING LAYERS

OPERATIONS

Does it imply a change to levels of automation? No

| ATM planning | Strategical |  | Pre-tactical |  |
| --- | --- | --- | --- | --- |
| Tactical-Pre ops |  | Tactical-During ops |  |  |
| Post operations |  |  |  |  |

| Taxi-out | Departure |  | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- | --- |
| Turn-around |  |  |  |  |  |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

SWIM-B2/1 - Information service provision

Evolution

SWIM-B2/2 - Information service consumption

Evolution

SWIM-B2/3 - SWIM registry

Relation-technology need

COMI-B1/1 - Ground-Ground Aeronautical Telecommunication

Network/Internet Protocol Suite (ATN/IPS)

Sixth edition of the GANP

Main Purpose

Exchange of safety critical information with the aircraft to improve operational awareness and

efficiency.

New Capabilities

A/G SWIM capability is extended to address the exchange of safety critical information with the

aircraft.

Description

A/G SWIM will be extended to enable the exchange of safety critical information.

Maturity Level

Concept

Human Factor

Considerations

PLANNING LAYERS

OPERATIONS

Information

Air/Ground SWIM for safety critical information

SWIM-B3/1

Tactical-During ops

Tactical-Pre ops

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Operational Information

procedures	exchange

Procedure to

provide access to SWIM information consumers

Provide SWIM information consumers a

single access point. Reference: Future Manual on System Wide Information Management (SWIM) Implementation.

2025

Ground

system infrastructur e

Information

exchange

Interconnection of

SWIM registries

Standards and procedures needed to

interconnect registries at a local and/or regional level to form a single access point for information service consumers. Reference: Future Manual on System Wide Information Management (SWIM) Implementation.

ATM SWIM

service provider 2025

SWIM Registry Manager

Regulatory	National

provisions	regulatory

framework

National

Regulatory framework amendment to address SWIM governance

Standards, policy and procedures needed  CAA

for SWIM governance.

2021

ATM SWIM

service consumer

| Taxi-out | Departure |  | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- | --- |
| Turn-around |  |  |  |  |  |

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-technology need

COMI-B3/4 - Links meeting requirements for safety critical communication

Evolution

SWIM-B2/4 - Air/Ground SWIM for non-safety critical information

Relation-technology need

COMI-B1/1 - Ground-Ground Aeronautical Telecommunication

Network/Internet Protocol Suite (ATN/IPS)

### ENABLERS

Information service consumer

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Information exchange

Procedures to access registry and information services

Procedures to be followed to access the registry and how to access information services as defined in the service overview. Guidance material: Future Manual on System Wide Information Management (SWIM) Implementation.

2023

Ground system infrastructur e

Information exchange

Automated systems capable to consume information over an IP network following required message exchange patterns

Automated systems capable to exchange information: - Over IP network (OSI Layer 1 to 5) Standards and guidance material: ICAO Annex 10 - Aeronautical Telecommunications Vol III -Rules to access an IP network Future Manual on System Wide Information Management (SWIM) Implementation - Following exchange patterns required to support the service provision (e.g. publish/subscribe or request/reply) (OSI Layer 7)

2023

Operational procedures

Information exchange

Procedures for the exchange of safety critical information with the aircraft

Procedures to be followed for the exchange of safety critical information with the aircraft. Reference: Future Manual on System Wide Information Management (SWIM) Implementation

Airspace user  2031

Airborne system capability

Information exchange

Airborne SWIM capability for exchanging safety critical information

Onboard systems capable of exchanging safety critical information (e.g. EFB).

Airspace user  2031

Information service consumer

Ground system infrastructur e

Information exchange

G/G SWIM

communications between ANSP and airspace user to exchange safety critical information

G/G SWIM communications between ANSP and airspace user to exchange safety critical information.

ANSP

Airspace user

2031

## TBO

TBO-B0/1	Introduction of time-based management within a flow centric approach.

Operational

Sixth edition of the GANP

Main Purpose		Provides for more efficient flight operation by using time-based scheduling versus more tactical measures such as holding to manage tactical synchronization.

New Capabilities		Strategic and tactical time based management are introduced via initial decision-making processes for network operations (demand capacity balancing) and runway sequencing (traffic synchronization).

Description		Individual time-based initiatives are available in decision making processes related to network operations or flight sequencing. The individual time-based initiatives are not synchronized, and any synchronization of individual time advisories is left to the tactical ATCO. The main focus is on the traffic flow activity without consideration to individual flights or gate-to gate focus.

Maturity Level	Ready for implementation

Human Factor Considerations

### PLANNING LAYERS

| Strategical | Pre-tactical |  | Tactical-Pre ops |
| --- | --- | --- | --- |
| Tactical-During ops |  |  |  |

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### OPERATIONS

| Taxi-out | Departure | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- |

ENABLERS

There are currently no enablers.

### DEPLOYMENT APPLICABILITY

Benefitting stakeholder(s)

Operational description

Type

Operational conditions: Main intended benefits:

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | Mitigate demand/capacity imbalance at airports and/or associated terminal airspace | ++ |  |
| Capacity | Capacity shortfall & associated delay | Mitigate demand/capacity imbalance in en-route airspace | ++ |  |

TBO-B1/1	Initial Integration of time-based decision making processes

Operational

Sixth edition of the GANP

Main Purpose	Provides initial support to network operations by integrating network applied constraints into local arrival and departure management. Overall operations are still locally conducted with time-based decision-making tools.

New Capabilities	Network operations and runway sequencing are the main contributors. Coordination is conducted between the two contributors. Some strategic and tactical decisions are locally coordinated but not necessarily fully synchronized.

Description		Information about individual and some sets of flights are available for time-based decision-making tools. Some pre-departure and in-flight synchronization is conducted locally via ATCO and automation. Flights are subject to local/regional initial synchronization processes.

Maturity Level	Standardization

Human Factor Considerations

### PLANNING LAYERS

| Strategical | Pre-tactical |  | Tactical-Pre ops |
| --- | --- | --- | --- |
| Tactical-During ops |  |  |  |

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### OPERATIONS

| Taxi-out | Departure | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- |

ENABLERS

There are currently no enablers.

### DEPLOYMENT APPLICABILITY

#### Operational conditions: Main intended benefits:

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | Mitigate demand/capacity imbalance at airports and/or associated terminal airspace | ++ |  |
| Capacity | Capacity shortfall & associated delay | Mitigate demand/capacity imbalance in en-route airspace | ++ |  |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Benefitting stakeholder(s)

Operational description

Type

TBO-B2/1	Pre-departure trajectory synchronization within a flight centric and network performance approach

Operational

Sixth edition of the GANP

Main Purpose	To achieve pre-departure synchronisation of aircraft trajectories through time-based decision- making processes. The objective of the pre-departure synchronization is to improve the performance of the network by sharing information and negotiating trajectories.

New Capabilities	Access to time targets from time-based decision-making processes to be used for local synchronization of either the insertion of a flight into ongoing time-based management or adherence to the advisories for flights in pre-departure. AU and ANSP planning is consistent based upon information exchange, including trajectory information, and information/knowledge of the evolution of the constraints.

Description		The move from data exchange to consistent information sharing provides the basis for managing departure times to meet time advisories provided by the different decision-making processes. Flow Managers provide synchronisation of trajectories (within their local (e.g. FIR) remit) via adjustments to departures times and flight profiles. The lack of automated processes across local boundaries to support synchronisation of a larger set of trajectories means that flights with non-aligned trajectories and potentially conflicting time constraints are still managed as in Block 0.

Maturity Level	Validation

Human Factor Considerations

### PLANNING LAYERS

Tactical-Pre ops

### OPERATIONS

Taxi-out   Departure   En-route   Arrival   Taxi-in

### DEPENDENCIES AND RELATIONS

There are currently no dependencies.

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | Mitigate demand/capacity imbalance at airports and/or associated terminal airspace | ++ |  |
| Capacity | Capacity shortfall & associated delay | Mitigate demand/capacity imbalance in en-route airspace | ++ |  |

ENABLERS

There are currently no enablers.

DEPLOYMENT APPLICABILITY

Operational conditions:

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Benefitting stakeholder(s)

Operational description

Type

TBO-B2/2	Extended time-based management across

multiple FIRs for active flight synchronization

Operational

Sixth edition of the GANP

Main Purpose	Achieve a locally efficiently-converging coordination process across time-based capabilities leading to stable, consistent and robust local trajectory solutions to improve individual flight efficiency while optimising the overall network performance. Trajectory information is shared between the AU and ANSPs for all flight phases. Post-departure trajectory sharing and negotiation with the AU allows the meeting of flight-specific business objectives and improved ground-based trajectory predictions

New Capabilities		Ability locally to link and synchronize various time advisories and paths for flights and individual aircraft across flight segments to provide equitable efficient operations. New performance specifications to meet the time component of the 4D intended trajectory to be taken into account for the local on-demand synchronization. Trajectory predictions are consistent and accurate for the aircraft that become “connected” aircraft (FMS, AOC and ANSPs have consistent predictions and each prediction is modified as a result of any change as this change is always shared and reflected in the position). Maintain custom trajectory predictions for each aircraft within the area of jurisdiction. Trajectory predictions are sufficiently synchronised across ANSPs to allow for traffic synchronization.

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Description 		Based on the move from data exchange to information sharing and providing that information to all components of individual or regional ANSP(s), AOC and the aircraft operator/pilot, automation exists which supports the synchronization of time and trajectory advisories within a state or region. Synchronization is based on rules and processes which take into consideration not only the objectives of the individual time-based decision-making processes, also but both dynamic and static constraints which may change/set new priorities rather than a single set of fixed rules.

UTM Rules are established to define airspace use by the different users and to locally synchronize the decision-making processes, based on the overall optimization of the performance.

Global rules for transitioning from different operating environments (including higher airspace and lower airspace) should be established based on local synchronization of decision making processes [coupled time-based management] in the affected airspace.

##### Note: TBO provisions are envisaged at this point. In order for a 4DT capability that is consistent across ANSPs, processes and information exchanges are required, which will include the definition and application of tolerances for various purposes (e.g. trajectory updates).

Maturity Level 	Validation

OPERATIONS

Human Factor Considerations

PLANNING LAYERS

Tactical-During ops

Tactical-Pre ops

| Taxi-out | Departure | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- |

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

ENABLERS

There are currently no enablers.

DEPLOYMENT APPLICABILITY

Operational conditions: Main intended benefits:

Benefitting stakeholder(s)

Operational description

Type

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | Mitigate demand/capacity imbalance at airports and/or associated terminal airspace | ++ |  |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | Mitigate demand/capacity imbalance in en-route airspace | ++ |  |

TBO-B3/1	Network based on-demand synchronization of trajectory based operations

Operational

Sixth edition of the GANP

Main Purpose	Achieve a network based, efficiently-converging coordination process, across all time-based decision-making processes. In this process a flight is subject to an end-to-end stable, consistent and robust trajectory solution.

New Capabilities		Ability to link and synchronize relevant time advisories and paths for flights and individual aircraft across all flight segments to provide equitable efficient operations. Trajectories and intents are synchronized along ANSPs, AOC and FMS. Synchronisation between airborne and ground systems is realised by means of exchanging and reconciling flight data including the airborne trajectory prediction.

Description		Based on the global availability of information and its sharing to all downstream ANSPs, AOC and the aircraft operator/pilot, automation exists which supports the synchronization of time and path in all flight phases. Synchronization is based on rules and processes which take into consideration not only the objectives of the individual time-based decision-making processes but both dynamic and static constraints which may alter the priority of solutions.

Note: TBO provisions are envisaged at this point. Full integration of strategic with tactical is implied. This includes the integration of multiple decision support tools within large ANSPs and between ANSPs. Strategic and tactical decisions will need to be compatible. As mentioned, rules and the conditions under which various rules are applicable will need to be defined. Convergence involving distributed decision-making is needed. Provisions & guidance will be required to standardize the distributed decision making and the sharing of information.

Maturity Level	Concept

Human Factor Considerations

### PLANNING LAYERS

| Strategical | Pre-tactical |  | Tactical-Pre ops |
| --- | --- | --- | --- |
| Tactical-During ops |  |  |  |

DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### OPERATIONS

| Taxi-out | Departure | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | Mitigate demand/capacity imbalance at airports and/or associated terminal airspace | ++ |  |
| Capacity | Capacity shortfall & associated delay | Mitigate demand/capacity imbalance in en-route airspace | ++ |  |

ENABLERS

There are currently no enablers.

DEPLOYMENT APPLICABILITY

Operational conditions:

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

Benefitting stakeholder(s)

Operational description

Type

TBO-B4/1	Total airspace management performance system  Operational

Sixth edition of the GANP

Main Purpose	Realize safe minimum separation trajectories (spatially and temporarily) to allow the users, through better information exchange, to manage their business trajectories.

New Capabilities	All constraints are provided to the airspace users so that they can determine and execute their business trajectories.

Description		ANSPs to provide on time minimum constraints, shifting the management of the trajectory from ANSP to airspace user centric. This is a shift from the ANSP choosing preferred airspace users’ trajectories to full provision of constraints so that the airspace user can manage their own business trajectories.

Note: TBO provisions are envisaged at this point. Provisions to define the interactions between trajectories and constraints are foreseen.

Maturity Level	Concept

Human Factor Considerations

### PLANNING LAYERS 		OPERATIONS

Strategical

Pre-tactical

Tactical-Pre ops

| Taxi-out | Departure | En-route | Arrival | Taxi-in |
| --- | --- | --- | --- | --- |

Post operations

Tactical-During ops

Turn-around

### DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### ENABLERS

There are currently no enablers.

### DEPLOYMENT APPLICABILITY

Benefitting stakeholder(s)

Operational description

Type

Operational conditions: Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity shortfall & associated delay | Mitigate demand/capacity imbalance at airports and/or associated terminal airspace | ++ |  |
| Capacity | Capacity shortfall & associated delay | Mitigate demand/capacity imbalance in en-route airspace | ++ |  |

## WAKE

Sixth edition of the GANP

Main Purpose

To safely optimise the separation minima to be applied between groups of aircraft due to wake

turbulence on arrival and departure phases of flight.

New Capabilities

Alternative of the aircraft wake turbulence categories defined in ICAO PANS-ATM by aircraft wake

turbulence groups based on safety and operational requirement criteria.

Description

This element defines new wake turbulence separation minima between wake turbulence groups.

The placement of aircraft into groups may be based on new aircraft designs, aircraft performance, collected wake and wind data, and flight trials.

Wake turbulence separation minima based on 7	Operational

aircraft groups

WAKE-B2/1

Maturity Level 		Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

Adaptation of working methods to separate traffic based on wake turbulence groups.

- Does it imply processing of new information by the user? Yes

Processing of more wake turbulence categories/groups and associated separation minima.

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-information need

AMET-B1/3 - Climatological and historical meteorological information

Relation-operational option

APTA-B0/3 - SBAS/GBAS CAT I precision approach procedures

Relation-operational option

APTA-B1/2 - PBN SID and STAR procedures (with advanced capabilities)

Relation-technology option

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-technology option

NAVS-B0/1 - Ground Based Augmentation Systems (GBAS)

Relation-technology option

NAVS-B0/3 - Aircraft Based Augmentation Systems (ABAS)

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Operational procedures

Operations	ATC Procedure

to separate arrival and departure traffic based on Wake Turbulence Groups

Procedure, working methods and separation look-up tables for Approach and Tower Air Traffic Controllers to operate Wake Turbulence separation based 7 aircraft Wake Turbulence Groups (WTG). References:

ANSP 	2020

Ground system infrastructur e

ATC systems Flight Plan

Processing System adaptation to operate WTG

Association in the local flight data processing system of an aircraft type to the corresponding Wake Turbulence Groups. References:

ANSP 	2019

Ground system infrastructur e

ATC systems ATC system

adaptation to display the WTG

Display on the approach and aerodrome controller’s surveillance display systems of the Wake Turbulence Group code associated to an aircraft type. References:

ANSP 	2019

Training	-	Training requirements for WTG

ATC Training to separate arrival and departure traffic based on WTG: Training of Approach and Tower Air Traffic Controllers on separation minima and delivery using 7 wake turbulence groups.

ANSP 	2019

Regulatory provisions

National regulatory framework

National framework amendment for Arrival and departure separation provisions based on Wake Turbulence Groups

National regulation amendment for arrival and departure separation provisions based on of WTG. References: DOC.

4444 – Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM) to be amended with distance-based separation (under ATS surveillance service) based 7 aircraft Wake Turbulence Groups (WTG)

CAA 	2020

Ground system infrastructur e

Surveillance	Surveillance

capabilities for separation in arrivals and departures

Provide the necessary surveillance means to comply with applicable surveillance performance requirements.

ANSP 	2013

Ground system infrastructur e

Navigation	Navigation

Aircraft operator

capabilities for separation in arrivals and departures

Provide the necessary navigation capabilities (e.g. ILS for precision approach).

ANSP 	2013

Training	Awareness	Awareness

requirements for WTG

Flight Crew awareness of separation based on WTG: Briefing material for Flight Crew awareness of the change in applicable wake separation minima.

2019

DEPLOYMENT APPLICABILITY

Operational conditions:

Aerodromes with demand that nears or exceeds peak capacity during periods of the operational day or overall daily capacity resulting in arrival and departure delay, and aerodromes where additional operational resilience is required to manage peak traffic in response to unplanned events.

Main intended benefits:

| Airport operator | ANSP |  | Airspace user |
| --- | --- | --- | --- |
| ATM network function |  |  |  |

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Reduce separation minima | Airport operator ANSP Airspace user |
|  | Reduce delay |  |
|  | Increase peak capacity | Airport operator ANSP Airspace user |
|  | Efficiency (temporal efficiency, i.e. delay) | ANSP Airspace user |

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Indirect benefits | Fuel savings: from taxi-out and arrival track miles | Airspace user |
|  | Resilience: additional headroom for spacing and sequence management | ANSP |
|  | Safety: reduced under spacing, number of go-arounds | Airspace user |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Improved categorisation of aircraft (arrival) | ++ | KPI10: Airport peak throughput |
| Capacity | Capacity, throughput & utilization | Reduce wake vortex separation penalty from other traffic using the departure RWY | ++ | KPI10: Airport peak throughput |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

WAKE-B2/2	Time based wake separation minima for final

approach

Operational

Sixth edition of the GANP

Main Purpose		To improve runway throughput and resilience due to wake turbulence on arrival phase of flight and to mitigate the impact of strong headwind conditions by safely optimising the separation minima to be applied between aircraft pairs by time rather than distance.

New Capabilities	Time based separation minima for aircraft pairs supporting the safe reduction of wake turbulence separations which can be frequent in traffic;

Mitigating the impact of strong headwind conditions;

System monitored real time wind and improved forecast meteo information.

Description		This element defines a new set of time based wake turbulence separations on final approach under ATS surveillance service, derived from distance-based separation (DBS) minima, allowing a dynamic DBS application for stabilizing landing rates across headwind conditions. It consists

of optimized wake separation pairings that can be exploited by the ATCO with system support, for enhancing resilience and mitigating the impact of strong headwinds.

Maturity Level	Concept

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

Adaptation of working methods to separate based on time-based wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

- Does it imply the use of new equipment? Yes

Adaptation of working methods to separate based on time-based wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

- Does it imply a change to levels of automation? Yes

Adaptation of working methods to separate based on time-based wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

WAKE-B3/3 - Wake turbulence separation minima based on leader/follower

static pairs-wise

Relation-information need

AMET-B1/3 - Climatological and historical meteorological information

Relation-information need

AMET-B2/1 - Meteorological observations information

Relation-operational option

APTA-B0/3 - SBAS/GBAS CAT I precision approach procedures

Relation-operational option

APTA-B1/2 - PBN SID and STAR procedures (with advanced capabilities)

Relation-technology option

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-technology option

NAVS-B0/1 - Ground Based Augmentation Systems (GBAS)

Relation-technology option

NAVS-B0/3 - Aircraft Based Augmentation Systems (ABAS)

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Regulatory	National

provisions	regulatory

framework

National

framework amendment for time-based pair- wise wake turbulence separation provision on approach (TBS- PWS-A)

National regulation amendment for Time-	 CAA

based pair-wise wake turbulence separation provision on Approach . References: DOC. 4444 – Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM) to be amended for allowing time-based separation on final appproach (under ATS surveillance service) derived from distance wake turbulence pair-wise minima

2024

Regulatory provisions

National regulatory framework

Local TBS-PWS- A minima separation rules

Local establishment of time-based pair- wise minima applicable to local traffic, converted from local reference air speed profiles per aircraft type over the distance separation minimum in calm wind (e.g.from analysis of surveillance data). References:

2024

CAA

ANSP

Operational procedures

Operations	TBS-PWS-A

Operational Procedure

Design of procedure and working methods for Approach and Tower Air Traffic Controllers to operate TBS on final approach. References:

ANSP 	2024

MET

Information Service Provider

Airborne system capability

Aircraft system

Aircraft-derived wind to feed the ATC Separation delivery tool

Aircraft-derived wind speed and direction on final approach. References:

ANSP

2024

Aircraft manufacturer

Ground system infrastructur e

ATC systems ATC separation

delivery tool to operate wake turbulence longitudinal TBS- PWS-A minima

Processing and display on the approach and aerodrome controller’s ATC surveillance display systems: - the distance separation minimum, or spacing constraint applicable to a sequenced pair of aircraft, - a spacing aid indicator to assist Air Traffic Controllers for optimising Separation Delivery taking into account predicted aircraft speed profiles (optional), - a safety alerting function of imminent risk of separation infringement or wrong aircraft being turned on to a separation indicator, fed by flight data, surveillance data, meteo data and separation rules. References: EUROCONTROL Specification for Time Based Separation (TBS) support tool for Final Approach, ed 1.0 (2017)

ANSP 	2024

Ground system infrastructur e

MET

instrument

Wind profile information to feed the ground- based ATC Separation delivery tool

Actual final approach wind profile information shall be provided to the TBS function, based on ground-based and/or aircraft measurements. References:

2024

MET

Information Service Provider

Training	-	Training requirements for time based wake separation minima for arrivals based on leader/follower static pairs-wise

TBS-PWS-A training on working methods and tool: Training of Approach and Tower Air Traffic Controllers on separation delivery using an TBS function, in nominal and degraded mode of operations.

ANSP 	2024

Ground system infrastructur e

Surveillance	Surveillance

capabilities for separation in arrivals and departures

Provide the necessary surveillance means to comply with applicable surveillance performance requirements.

ANSP 	2013

Ground system infrastructur e

Navigation	Navigation

Aircraft operator

capabilities for separation in arrivals and departures

Provide the necessary navigation capabilities (e.g. ILS for precision approach).

ANSP 	2013

Training	Awareness	Awareness

DEPLOYMENT APPLICABILITY

Operational conditions:

Aerodromes with demand nears or exceeds peak capacity during periods of the operational day or overall daily capacity with a dense heterogeneous traffic mix resulting in arrival and departure delay, and aerodromes where additional operational resilience, predictability and head wind mitigation is required to manage peak traffic in response to unplanned events.

Main intended benefits:

requirements for time based wake separation minima for arrivals based on leader/follower static pairs-wise

TBS-PWS-A awareness by Fligh Crews: Briefing material for Flight Crew awareness of the change in applicable wake separation minima.

2024

| Airport operator | ANSP |  | Airspace user |
| --- | --- | --- | --- |
| ATM network function |  |  |  |

| Airport operator | ANSP |  | Airspace user |
| --- | --- | --- | --- |
| ATM network function |  |  |  |

| Type Operational description Benefitting stakeholder(s) |  |
| --- | --- |
| Direct benefits | Reduce separation minima Airport operator ANSP Airspace user |
|  | Reduce delay |
|  | Prevent reduction of arrival rate during headwind conditions |
|  | Increase peak capacity Airport operator ANSP Airspace user |
| Indirect benefits | Efficiency (temporal efficiency, i.e. delay) ANSP Airspace user |
|  | Predictability: delivery of traffic more ANSP Airspace user consistent, allowing increased throughput |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Apply time-based separation instead of distance-based (arrival) | ++ | KPI10: Airport peak throughput |
| Capacity | Capacity, throughput & utilization | Improved categorisation of aircraft (arrival) | ++ | KPI10: Airport peak throughput |
| Capacity | Capacity, throughput & utilization | Reduce wake vortex separation penalty from other traffic using the departure RWY | ++ | KPI10: Airport peak throughput |

WAKE-B3/1	Dependent parallel approaches	Operational

Sixth edition of the GANP

Main Purpose	To safely optimise the wake turbulence separation applied to instrument landing operations on parallel runways with centre lines spaced less than 760m (2500ft) through reduced separation.

New Capabilities	Simultaneous use of parallel runways through the definition of new landing and go around procedures with a revised wake vortex separation 7 group separation minimum.

Description		This element defines a dependent paired approach procedure to parallel runways, with centre lines spaced less than 760m (2500ft) apart, threshold staggers, and/or glide path height differences, under ILS Category I minimums, or the minimums depicted for an RNAV or LPV approach. It covers airports applying ICAO Wake Turbulence Categories or wake turbulence aircraft groups.

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

Adaptation of working methods to separate traffic based on wake turbulence groups.

- Does it imply processing of new information by the user? Yes

Processing of more wake turbulence categories/groups and associated separation minima.

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Arrival

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Relation-information benefit	AMET-B1/2 - Meteorological forecast and warning information

Relation-information need

AMET-B1/3 - Climatological and historical meteorological information

Relation-operational need

WAKE-B2/1 - Wake turbulence separation minima based on 7 aircraft

groups

Relation-operational option

APTA-B0/3 - SBAS/GBAS CAT I precision approach procedures

Relation-operational option

APTA-B1/2 - PBN SID and STAR procedures (with advanced capabilities)

Relation-operational option

APTA-B2/2 - Simultaneous operations to parallel runways

Relation-technology option

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-technology option

NAVS-B0/1 - Ground Based Augmentation Systems (GBAS)

Relation-technology option

NAVS-B0/3 - Aircraft Based Augmentation Systems (ABAS)

Relation-operational benefit

SNET-B0/4 - Approach Path Monitoring (APM)

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

National regulatory framework

National framework amendment for wake turbulence separation for staggered dependent parallel approaches

National regulation amendment for wake turbulence separation for staggered dependent parallel approaches.

References: DOC. 4444 – Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM) to be amended with distance-based separation (under ATS surveillance service) between staggered dependent parallel approaches based on Wake Turbulence Categories or Wake Turbulence Groups (WTG)

CAA 	2028

Operational procedures

Operations	ATC Procedure

to separate staggered dependent parallel approaches

Design of local procedure and minima (if wind-dependent), working methods and separation look-up tables for Approach and Tower Air Traffic Controllers to operate wake turbulence separation for staggered dependent parallel approaches based on Wake Turbulence Categories (WTC) or Wake Turbulence Groups (WTG). References:

ANSP 	2028

Ground system infrastructure

Ground infrastructure

Adaptation of runway threshold location enabling staggered dependent parallel approaches

Displacement of one landing threshold location to create a stagger with the other threshold, if not already existing.

References:

ANSP 	2019

Ground system infrastructur e

ATC systems Flight Plan

Processing System adaptation to operate staggered dependent parallel approaches based on WTG

Local association of an aircraft type to the Wake Turbulence Groups in the flight data processing system (required when operating separation based on 7 WTG). References:

ANSP 	2019

Ground system infrastructur e

ATC systems ATC system

adaptation to display the WTG for staggered dependent parallel approaches

Display on the approach and aerodrome controller’s surveillance display systems on the Wake Turbulence Group code associated to an aircraft type (required when operating separation based on 7 WTG). References:

ANSP 	2019

Training	-	Training requirements for dependent parallel approaches

ATC Training to separate traffic under staggered dependent parallel approaches: Training of Approach and Tower Air Traffic Controllers to operate separation minima for staggered dependent parallel approaches based

ANSP 	2019

Ground system infrastructur e

Surveillance	Surveillance

capabilities for separation in arrivals and departures

Provide the necessary surveillance means to comply with applicable surveillance performance requirements.

ANSP 	2013

Ground system infrastructur e

Navigation	Navigation

Aircraft operator

capabilities for separation in arrivals and departures

Provide the necessary navigation capabilities (e.g. ILS for precision approach).

ANSP 	2013

Training	Awareness	Awareness

requirements for dependent parallel approaches

Wake Turbulence Categories (WTC) or Wake Turbulence Groups (WTG) Flight Crew awareness of separation applicable under staggered dependent parallel approaches: Briefing material for Flight Crew awareness of the change in applicable wake separation minima for staggered dependent parallel approaches

2019

DEPLOYMENT APPLICABILITY

Operational conditions:

Aerodromes with demand that exceeds peak capacity during periods of the operational day or overall daily capacity resulting in arrival and departure delay, and aerodromes where additional operational resilience is required to manage peak traffic in response to unplanned events operating dependent parallel approaches with parallel runways having centrelines spaced less than 760 m (2500 feet) apart.

| Airport operator | ANSP |  | Airspace user |
| --- | --- | --- | --- |
| ATM network function |  |  |  |

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Reduce separation minima on approach | Airport operator ANSP Airspace user |
|  | Reduce delay |  |
|  | Increase peak capacity by using parallel RWY operations | Airport operator ANSP Airspace user |
| Indirect benefits | Efficiency (temporal efficiency, i.e. delay) | ANSP Airspace user |
|  | Resilience: additional headroom for spacing and sequence management | ANSP |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Reduce wake turbulence impact from parallel RWY during crosswind (arrival) | ++ | KPI10: Airport peak throughput |

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

WAKE-B3/2	Independent segregated parallel operations	Operational

Sixth edition of the GANP

Main Purpose	To safely optimise the wake turbulence separation applied to segregated operations on parallel runways, with centre lines spaced less than 760m (2500ft) apart for wake independent departure and arrival operations based on wind transport of wake turbulence.

New Capabilities	Simultaneous use of parallel runways through the definition of new landing, departure, go around and departure procedures;

Revised wake vortex separation 7 group separation minimum.

Description		This element defines independent segregated parallel operations procedures to parallel runways with centre lines spaced less than 760m (2500ft) apart based on detailed wind and wake analysis. It is aerodrome specific in terms of runway layout and weather conditions. It covers airports exploiting ICAO 3 Category or Wake Vortex Separation of 7 groups.

Maturity Level	Standardization

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

Adaptation of working methods to separate traffic based on wake turbulence groups.

- Does it imply processing of new information by the user? Yes

Processing of more wake turbulence categories/groups and associated separation minima.

- Does it imply the use of new equipment? No

- Does it imply a change to levels of automation? No

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

WAKE-B2/1 - Wake turbulence separation minima based on 7 aircraft

groups

Relation-information need

AMET-B1/3 - Climatological and historical meteorological information

Relation-operational option

APTA-B0/3 - SBAS/GBAS CAT I precision approach procedures

Relation-operational option

APTA-B1/2 - PBN SID and STAR procedures (with advanced capabilities)

Relation-technology option

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-technology option

NAVS-B0/1 - Ground Based Augmentation Systems (GBAS)

Relation-technology option

NAVS-B0/3 - Aircraft Based Augmentation Systems (ABAS)

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

National regulatory framework

National framework amendment for wake- independent segregated parallel operations

National regulation amendment for wake- independent segregated parallel operations. References: DOC. 4444 – Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM) to be amended with wake-independent segregated parallel operations for closely spaced parallel runways based on Wake Turbulence Categories or Wake Turbulence Groups (WTG)

CAA 	2028

Operational procedures

Operations	ATC Procedure

for wake- independent segregated parallel operations

Design of local procedure, working methods and separation look-up tables for Approach and Tower Air Traffic Controllers to operate based on Wake Turbulence Categories (WTC) or Wake Turbulence Groups (WTG). References:

ANSP 	2028

Ground system infrastructur e

ATC systems Flight Plan

Processing System adaptation to operate wake- independent segregated parallel operations based on WTG

Local assocation of an aircraft type to the Wake Turbulence Groups in the flight data processing system (required when operating separation based on 7 WTG). References:

ANSP 	2019

Ground system infrastructur e

ATC systems ATC system

adaptation to display the WTG for wake- independent segregated parallel operations

Display on the approach and aerodrome controller’s surveillance display systems on the Wake Turbulence Group code associated to an aircraft type (required when operating separation based on 7 WTG). References:

ANSP 	2019

Training	-	Training requirements for wake- independent segregated parallel operations

ATC Training to separate traffic unde Wake-independent segregated parallel operations: Training of Approach and Tower Air Traffic Controllers to operate based Wake Turbulence Categories (WTC) or Wake Turbulence Groups (WTG).

ANSP 	2019

Ground system infrastructur e

Surveillance	Surveillance

capabilities for separation in arrivals and departures

Provide the necessary surveillance means to comply with applicable surveillance performance requirements.

ANSP 	2013

Ground system infrastructur e

Navigation	Navigation

Aircraft operator

capabilities for separation in arrivals and departures

Provide the necessary navigation capabilities (e.g. ILS for precision approach).

ANSP 	2013

Training	Awareness	Awareness

requirements for wake- independent segregated parallel operations

Flight Crew awareness of separation applicable under wake-independent segregated parallel operations: Briefing material for Flight Crew awareness of the change in applicable wake separation minima for closely spaced parallel runway operations.

2019

DEPLOYMENT APPLICABILITY

Operational conditions:

Aerodromes with demand that exceeds peak capacity during periods of the operational day or overall daily capacity resulting in arrival and departure delay, and aerodromes where additional operational resilience is required to manage peak traffic in response to unplanned events operating Independent segregated parallel operations with

| Type | Operational description | Benefitting stakeholder(s) |  |  |
| --- | --- | --- | --- | --- |
| Direct benefits | Reduce separation minima independently on parallel approach and departure runways. | Airport operator | ANSP | Airspace user |
|  | Increase peak capacity | Airport operator | ANSP | Airspace user |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Reduce wake turbulence impact from parallel RWY during crosswind (arrival) | ++ | KPI10: Airport peak throughput |
| Capacity | Capacity, throughput & utilization | Reduce wake turbulence impact from parallel RWY during crosswind (departures) | ++ | KPI10: Airport peak throughput |

parallel runways having centrelines spaced less than 760 m (2500 feet) apart.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

WAKE-B3/3	Wake turbulence separation minima based on

leader/follower static pairs-wise

Operational

Sixth edition of the GANP

Main Purpose	To safely optimise the separation minima to be applied between aircraft pairs due to wake turbulence on arrival and departure phases of flight.

New Capabilities		Aircraft separated by a tailored 7 (or more) separation groups or leader / follower pair-wise static matrix of aircraft type wake separation pairings with system support, for a specific airport or terminal area.

Description		This element defines new tailored wake turbulence separations for frequent aircraft pairs based on the performance characteristics of the leading aircraft generating wake turbulence and the following aircraft that might encounter the wake turbulence. It consists of a leader / follower pair-wise static matrix of aircraft type wake separation pairings that can be exploited by the ATCO with system support, or used to tailor a wake turbulence separation group system for a given traffic mix for a specific airport or terminal area. Existing categorisation or grouping systems will be used to determine separation minima for all types of aircraft pairs not specifically included in the pairwise separation matrix.

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

Adaptation of working methods to separate based on wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

- Does it imply processing of new information by the user? Yes

Adaptation of working methods to separate based on wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

- Does it imply the use of new equipment? Yes

Adaptation of working methods to separate based on wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

- Does it imply a change to levels of automation? Yes

Adaptation of working methods to separate based on wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

WAKE-B2/1 - Wake turbulence separation minima based on 7 aircraft

groups

Relation-information need

AMET-B1/3 - Climatological and historical meteorological information

Relation-operational option

APTA-B0/3 - SBAS/GBAS CAT I precision approach procedures

Relation-operational option

APTA-B1/2 - PBN SID and STAR procedures (with advanced capabilities)

Relation-technology option

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-technology option

NAVS-B0/1 - Ground Based Augmentation Systems (GBAS)

Relation-technology option

NAVS-B0/3 - Aircraft Based Augmentation Systems (ABAS)

ENABLERS

Enabler

Category

Enabler Type

Enabler Name

Description / References

Stakeholders	Year

Regulatory	National

provisions	regulatory

framework

National

framework amendment for wake turbulence longitudinal separation provisions based on pair-wise minima

National regulation amendment for the	 CAA

provision of separation based on pair- wise minima. References: DOC. 4444 – Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM) to be amended with distance-based separation (under ATS surveillance service) based on wake turbulence pair- wise minima

2030

Operational procedures

Operations	ATC Procedure

to separate arrival and departure traffic based on wake turbulence longitudinal pair- wise minima

Design of procedure and working methods for Approach and Tower Air Traffic Controllers to operate Wake Turbulence separation based on pair-wise minima with ATC separation delivery function. References:

ANSP 	2030

Ground system infrastructur e

ATC systems ATC separation

delivery tool to operate wake turbulence longitudinal pair- wise minima

Processing and display on the approach and aerodrome controller’s ATC surveillance display systems OF: - a separation deliver indicator (the distance separation minimum, or spacing constraint) applicable to a sequenced pair of aircraft, - a spacing aid indicator to assist Air Traffic Controllers for optimising Separation Delivery taking into account predicted aircraft speed profiles (optional), - a safety alerting function of imminent risk of separation infringement or wrong aircraft being turned on to a separation indicator, fed by flight data, surveillance data and separation rules

ANSP 	2030

Training	-	Training requirements for wake turbulence longitudinal separation provisions based on Pair-wise minima

ATC Training to separate arrival and departure traffic based on Pair-wise minima: Training of Approach and Tower Air Traffic Controllers on separation delivery using an separation delivery function, in nominal and degraded mode of operations .

ANSP 	2030

Ground system infrastructur e

Surveillance	Surveillance

capabilities for separation in arrivals and departures

Provide the necessary surveillance means to comply with applicable surveillance performance requirements.

ANSP 	2013

Ground system infrastructur e

Navigation	Navigation

Aircraft operator

capabilities for separation in arrivals and departures

Provide the necessary navigation capabilities (e.g. ILS for precision approach).

ANSP 	2013

Training	Awareness	Awareness

requirements for wake turbulence longitudinal separation provisions based on Pair-wise minima

Flight Crew awareness of applicable separation based on Pair-wise minima: Briefing material for Flight Crew awareness of the change in applicable wake separation minima.

2030

### DEPLOYMENT APPLICABILITY

#### Operational conditions:

Aerodromes with demand nears or exceeds peak capacity during periods of the operational day or overall daily capacity with a dense heterogeneous traffic mix resulting in arrival and departure delay, and aerodromes where additional operational resilience and predictability is required to manage peak traffic in response to unplanned events.

| Airport operator | ANSP |  | Airspace user |
| --- | --- | --- | --- |
| ATM network function |  |  |  |

| Airport operator | ANSP |  | Airspace user |
| --- | --- | --- | --- |
| ATM network function |  |  |  |

Main intended benefits:

| Type Operational description Benefitting stakeholder(s) |  |  |
| --- | --- | --- |
| Direct benefits | Reduce separation minima | Airport operator ANSP Airspace user |
|  | Reduce delay |  |
|  | Prevent reduction of arrival rate during headwind conditions |  |
|  | Increase peak capacity Airport operator ANSP Airspace user |  |
| Indirect benefits | Efficiency (temporal efficiency, i.e. delay) ANSP Airspace user |  |
|  | Predictability: delivery of traffic more ANSP Airspace user consistent, allowing increased throughput |  |
|  | Fuel savings: from taxi-out and arrival track Airspace user miles |  |
|  | Resilience: additional headroom for ANSP spacing and sequence management |  |
|  | Safety: reduced under-spacing, number of Airspace user go-arounds |  |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Base minima on more accurate wake vortex characteristics of aircraft pairs (static) (arrival) | ++ | KPI10: Airport peak throughput |
| Capacity | Capacity, throughput & utilization | Reduce wake vortex separation penalty from other traffic using the departure RWY | ++ | KPI10: Airport peak throughput |

WAKE-B3/4	Enhanced dependent parallel approaches	Operational

Sixth edition of the GANP

Main Purpose	To safely optimise the wake turbulence separation applied to instrument landing operations on parallel runways with centre lines spaced less than 760m (2500ft) through reduced tailored or pairwise minimum.

New Capabilities	Simultaneous use of runways through the definition of new landing and go around procedures System monitored real time wind and improved forecast meteo information (optional)

A tailored wake turbulence separation category system for a specific airport or terminal area or Aircraft separated by leader / follower pair-wise static matrix of aircraft type wake separation pairings with system support.

Description		This element defines a dependent paired approach procedure to parallel runways, with centre lines spaced less than 760m (2500ft) apart, threshold staggers, and/or glide path height differences, under ILS Category I minimums, or the minimums depicted for an RNAV or LPV approach. It covers aircraft separation by leader / follower pair-wise static matrix of aircraft type wake separation pairings with system support, or a customised a wake turbulence separation category system for a specific airport or terminal area . It may include improved forecast meteo information available (optional).

Maturity Level	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

Adaptation of working methods to separate based on wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

- Does it imply processing of new information by the user? Yes

Adaptation of working methods to separate based on wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

- Does it imply the use of new equipment? Yes

Adaptation of working methods to separate based on wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

- Does it imply a change to levels of automation? Yes

Adaptation of working methods to separate based on wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

### PLANNING LAYERS

Tactical-During ops

### OPERATIONS

Arrival

### DEPENDENCIES AND RELATIONS

Type of Dependencies	ASBU Element

Evolution	WAKE-B3/1 - Dependent parallel approaches

Relation-operational need	WAKE-B3/3 - Wake turbulence separation minima based on leader/follower

static pairs-wise

Relation-information need

AMET-B1/3 - Climatological and historical meteorological information

Relation-operational option

APTA-B0/3 - SBAS/GBAS CAT I precision approach procedures

Relation-operational option

APTA-B1/2 - PBN SID and STAR procedures (with advanced capabilities)

Relation-operational option

APTA-B2/2 - Simultaneous operations to parallel runways

Relation-technology option

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-technology option

NAVS-B0/1 - Ground Based Augmentation Systems (GBAS)

Relation-technology option

NAVS-B0/3 - Aircraft Based Augmentation Systems (ABAS)

Relation-operational benefit

SNET-B0/4 - Approach Path Monitoring (APM)

Relation-information benefit

AMET-B2/2 - Meteorological forecast and warning information

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

National regulatory framework

National framework amendment for wake turbulence separation for staggered dependent parallel approaches based on pair- wise minima

National regulation amendment for staggered dependent parallel approaches based on pair-wise minima. References: DOC. 4444 – Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM) to be amended with distance-based separation (under ATS surveillance service) between staggered dependent parallel approaches based on pair-wise minima

CAA 	2030

Operational procedures

Operations	ATC Procedure

to separate staggered dependent parallel approaches based on PWM

Design of local procedure and minima (if wind-dependent), working methods and separation look-up tables for Approach and Tower Air Traffic Controllers to operate wake turbulence separation for staggered dependent parallel approaches based on pair-wise minima with ATC separation delivery function. References:

ANSP 	2030

Ground system infrastructur e

Ground infrastructure

Adaptation of runway threshold location enabling staggered dependent parallel approaches based on PWM

Displacement of one landing threshold location to create a stagger with the other threshold, if not already existing.

References:

ANSP 	2030

Ground system infrastructur e

ATC systems ATC separation

delivery tool to operate staggered dependent parallel approaches based on wake turbulence longitudinal pair- wise minima

Processing and display on the approach and aerodrome controller’s ATC surveillance display systems of: - a separation delivery indicator (the distance separation minimum, or spacing constraint) applicable to a sequenced pair of aircraft, - a spacing aid indicator to assist Air Traffic Controllers for optimising Separation Delivery taking into account predicted aircraft speed profiles (optional), - a safety alerting function of imminent risk of separation infringement or wrong aircraft being turned on to a separation indicator, fed by flight data, surveillance data and separation rules.

MET

Information Service Provider

References:

ANSP 	2030

Ground system infrastructur e

MET

instrument

MET tool to operate staggered dependent parallel approaches based on pair- wise minima

Actual final approach wind profile information shall be provided to the TBS function, based on ground-based and/or aircraft measurements (optional if separation are wind-dependent).

References:

ANSP

2030

Training	-	Training requirements for enhanced dependent parallel approaches

ATC Training to separate traffic under staggered dependent parallel approaches: Training of Approach and Tower Air Traffic Controllers on separation delivery using an separation delivery function, in nominal and degraded mode of operations.

ANSP 	2030

Ground system infrastructur e

Surveillance	Surveillance

capabilities for separation in arrivals and departures

Provide the necessary surveillance means to comply with applicable surveillance performance requirements.

ANSP 	2013

Ground system infrastructur e

Navigation	Navigation

Aircraft operator

capabilities for separation in arrivals and departures

Provide the necessary navigation capabilities (e.g. ILS for precision approach).

ANSP 	2013

Training	Awareness	Awareness

requirements for enhanced dependent parallel approaches

Flight Crew awareness of separation applicable under staggered dependent parallel approaches: Briefing material for Flight Crew awareness of the change in applicable wake separation minima.

2030

### DEPLOYMENT APPLICABILITY

#### Operational conditions:

Aerodromes with parallel runways having centrelines spaced less than 760 m (2500 feet) apart operating enhanced dependent parallel approaches with demand near to or exceeds peak capacity during periods of the operational day or overall daily capacity with a dense heterogeneous traffic mix resulting in arrival delay, and aerodromes where additional operational resilience and predictability is required to manage peak traffic in response to unplanned events.

| Airport operator | ANSP |  | Airspace user |
| --- | --- | --- | --- |
| ATM network function |  |  |  |

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Reduce separation minima on approach | Airport operator ANSP Airspace user |
|  | Reduce delay |  |
|  | Increase peak capacity by using parallel RWY operations | Airport operator ANSP Airspace user |
| Indirect benefits | Efficiency (temporal efficiency, i.e. delay) | ANSP Airspace user |
|  | Resilience: additional headroom for spacing and sequence management | ANSP |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Reduce wake turbulence impact from parallel RWY during crosswind (arrival) | ++ | KPI10: Airport peak throughput |

Sixth edition of the GANP

Main Purpose 		To safely optimise the wake turbulence separation applied to segregated operations on parallel runways, with centre lines spaced less than 760m (2500ft) apart for wake independent departure and arrival operations based on wind transport of wake turbulence and reduced tailored or pairwise separation minimum.

Operational

Enhanced independent segregated parallel

operations

WAKE-B3/5

New Capabilities 	  Simultaneous use of parallel runways for segregated operations on parallel runways through the definition of new landing, go around and departure procedures;

System monitored real time wind and improved forecast meteo information (optional);

Aircraft separated by leader / follower pair-wise static matrix of aircraft type wake separation pairings with system support, or a customised a wake turbulence separation category system for a specific airport or terminal area.

Description 		This element defines an independent segregated parallel operation procedure to parallel runways, with centre lines spaced less than 760m (2500ft) apart under ILS Category I minimum, or the minimums depicted for an RNAV or LPV approach. It covers independent landing and departing aircraft separation by leader / follower pair-wise static matrix of aircraft type wake separation pairings with system support, or a customised a wake turbulence separation category system for a specific airport or terminal area. It may include improved forecast meteo information and predicted and monitored winds on final approach and along the airport parallel runways that determine if the wake turbulence of arriving aircraft will be mitigated by crosswinds from moving into the path of departing aircraft on the adjacent parallel runway (optional). It is aerodrome specific in terms of runway layout and weather conditions.

Maturity Level 	Validation

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

Adaptation of working methods to separate based on wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

- Does it imply processing of new information by the user? Yes

Adaptation of working methods to separate based on wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

- Does it imply the use of new equipment? Yes

Adaptation of working methods to separate based on wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

- Does it imply a change to levels of automation? Yes

Adaptation of working methods to separate based on wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Relation-operational need

WAKE-B3/3 - Wake turbulence separation minima based on leader/follower

static pairs-wise

Evolution

WAKE-B3/2 - Independent segregated parallel operations

Relation-information need

AMET-B1/3 - Climatological and historical meteorological information

Relation-operational option

APTA-B0/3 - SBAS/GBAS CAT I precision approach procedures

Relation-operational option

APTA-B1/2 - PBN SID and STAR procedures (with advanced capabilities)

Relation-technology option

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-technology option

NAVS-B0/1 - Ground Based Augmentation Systems (GBAS)

Relation-technology option

NAVS-B0/3 - Aircraft Based Augmentation Systems (ABAS)

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

National regulatory framework

National framework amendment for wake- independent segregated parallel operations based on longitudinal pair- wise minima

National regulation amendment for wake- independent segregated parallel operations based on longitudinal pair- wise minima. References: DOC. 4444 – Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM) to be amended with wake-independent segregated parallel operations for closely spaced parallel runways based on pair- wise minima

CAA 	2030

Operational procedures

Operations	ATC Procedure

for wake- independent segregated parallel operations based on PWM

Design of local procedure, working methods and separation look-up tables for Approach and Tower Air Traffic Controllers to operate based on pair-wise minima with ATC separation delivery function. References:

ANSP 	2030

Ground system infrastructur e

ATC systems ATC separation

delivery tool to operate wake- independent segregated parallel operations based on longitudinal pair- wise minima

Processing and display on the approach and aerodrome controller’s ATC surveillance display systems: - the distance separation minimum, or spacing constraint applicable to a sequenced pair of aircraft, - a spacing aid indicator to assist Air Traffic Controllers for optimising Separation Delivery taking into account predicted aircraft speed profiles (optional), - a safety alerting function of imminent risk of separation infringement or wrong aircraft being turned on to a separation indicator, fed by flight data, surveillance data and separation rules.

References:

ANSP 	2030

Training	-	Training requirements for enhanced independent segregated parallel operations

ATC Training to separate traffic unde Wake-independent segregated parallel operations: Training of Approach and Tower Air Traffic Controllers on separation delivery using an separation delivery function, in nominal and degraded mode of operations.

ANSP 	2030

Ground system infrastructur e

Surveillance	Surveillance

capabilities for separation in arrivals and departures

Provide the necessary surveillance means to comply with applicable surveillance performance requirements.

ANSP 	2013

Ground system infrastructur e

Navigation	Navigation

Aircraft operator

capabilities for separation in arrivals and departures

Provide the necessary navigation capabilities (e.g. ILS for precision approach).

ANSP 	2013

Training	Awareness	Awareness

DEPLOYMENT APPLICABILITY

Operational conditions:

Aerodromes with parallel runways having centrelines spaced less than 760 m (2500 feet) apart operating independent segregated parallel approaches with demand near to or exceeds peak capacity during periods of the operational day or overall daily capacity with a dense heterogeneous traffic mix resulting in arrival delay, and aerodromes where additional operational resilience and predictability is required to manage peak traffic in response to unplanned events.

Main intended benefits:

requirements for enhanced independent segregated parallel operations

Flight Crew awareness of separation applicable under wake-independent segregated parallel operations: Briefing material for Flight Crew awareness of the change in applicable wake separation minima.

2030

| Airport operator | ANSP |  | Airspace user |
| --- | --- | --- | --- |
| ATM network function |  |  |  |

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Reduce separation minima independently on parallel approach and departure runways. | Airport operator ANSP Airspace user |
|  | Reduce delay |  |
|  | Increase peak capacity | Airport operator ANSP Airspace user |
|  | Efficiency (temporal efficiency, i.e. delay) | ANSP Airspace user |

| Indirect benefits Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
|  | Resilience: additional headroom for spacing and sequence management | ANSP |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Reduce wake turbulence impact from parallel RWY during crosswind (arrival) | ++ | KPI10: Airport peak throughput |
| Capacity | Capacity, throughput & utilization | Reduce wake turbulence impact from parallel RWY during crosswind (departures) | ++ | KPI10: Airport peak throughput |

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

WAKE-B3/6	Time based wake separation minima for

departure based on leader/follower static pair- wise

Operational

Sixth edition of the GANP

Main Purpose		To improve runway throughput and resilience due to wake turbulence on departure phases of flight by safely optimising the separation minima delivery between aircraft pairs.

New Capabilities	Optimised delivery of Time based separation minima for aircraft pairs supporting the safe reduction of wake turbulence separations which can be frequent in traffic.

Description		This element defines a new ATC tool for assisting Tower Runway Controllers in delivering time- based pair-wise wake turbulence separations on departure, taking into account take-off rolling distance and speed profiles per aircraft type (e.g. obtained from analysis of surveillance data).

The tool processes and displays on the aerodrome tower controller’s surveillance systems:

an aircraft positioning indicator to visualise in distance the applicable equivalent time-based pair- wise separation minimum, or spacing constraint to be delivered by the controller, or

a timer indication to provide the take-off clearance for delivering the time-based pair-wise minima in an optimum manner, and

a safety alerting function of imminent risk of separation infringement.

Maturity Level	Concept

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

Adaptation of working methods to separate based on time-based wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

- Does it imply processing of new information by the user? Yes

Adaptation of working methods to separate based on time-based wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

- Does it imply the use of new equipment? Yes

Adaptation of working methods to separate based on time-based wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

- Does it imply a change to levels of automation? Yes

Adaptation of working methods to separate based on time-based wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

WAKE-B3/3 - Wake turbulence separation minima based on leader/follower

static pairs-wise

Relation-information need

AMET-B1/3 - Climatological and historical meteorological information

Relation-information need

AMET-B2/1 - Meteorological observations information

### ENABLERS

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

Regulatory provisions

National regulatory framework

National framework amendment for time-based pair- wise wake turbulence separation provision on departure (TBS- PWS-D)

National regulation amendment for Time- based pair-wise wake turbulence separation provision on Departure.

References: DOC. 4444 – Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM) to be amended with static time-based pair-wise separation delivery based on distance wake turbulence pair-wise separation minima

CAA 	2030

Regulatory provisions

National regulatory framework

Local TBS-PWS- D minima separation rules

Local establishment of static time-based pair-wise minima applicable to local traffic, based on local reference departure speed and take-off distance profiles per aircraft type (e.g. obtained from analysis of surveillance data). References:

ANSP   CAA 	2030

Operational procedures

Operations	TBS-PWS-D

Oprational Procedure

Design of procedure and working methods for Tower Air Traffic Controllers to operate TBS on final approach.

References:

ANSP 	2030

Ground system infrastructur e

ATC systems TBS-PWS-D

ATC system capability

Provide flight data, surveillance data , meteorological data and separation rules to the TBS Optimised Separation Delivery (OSD) function. References:

ANSP 	2030

Ground system infrastructur e

ATC systems TBS-PWS-D

ATC system capability

Processing and display on the aerodrome tower controller’s surveillance display systems: - an aircraft positioning indicator to visualise in distance the applicable equivalent time-based pair-wise separation minimum, or spacing constraint to be delivered by the controller, or - a timer indication to provide the take- off clearance for delivering the time-based pair-wise minima in an optimum manner, and - a safety alerting function of imminent risk of separation infringement

ANSP 	2030

Aircraft operator

Training	-	Training requirements for time based wake separation minima for departures based on leader/follower static pairs-wise

TBS-PWS-D training on working methods and tool Training of Tower Air Traffic Controllers ontime-based pair-wise separation delivery, in nominal and degraded mode of operations

ANSP 	2030

Training	Awareness	Awareness

requirements for time based wake separation minima for departures based on leader/follower static pairs-wise

TBS-PWS-D awareness by Flight Crews Briefing material for Flight Crew awareness of the change in applicable wake separation minima

2030

DEPLOYMENT APPLICABILITY

Operational conditions:

Aerodromes with demand nears or exceeds peak capacity during periods of the operational day or overall daily capacity with a dense traffic mix resulting in departure delay, and aerodromes where additional operational resilience and predictability is required to manage peak traffic in response to unplanned events.

Main intended benefits:

Benefitting stakeholder(s)

Operational description

Type

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| Airport operator | ANSP |  |  |
| --- | --- | --- | --- |
| ATM network function |  | Aircraft operator |  |

| Type Operational description Benefitting stakeholder(s) |  |
| --- | --- |
| Direct benefits | Reduce delay |
|  | Airport operator ANSP Increase peak capacity Aircraft manufacturer |
| Indirect benefits | Efficiency (temporal efficiency, i.e. delay) ANSP Aircraft operator |
|  | Predictability: delivery of traffic more ANSP Aircraft operator consistent, allowing increased throughput |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Reduce wake vortex separation penalty from other traffic using the departure RWY | ++ | KPI10: Airport peak throughput |

Sixth edition of the GANP

Main Purpose

To improve runway throughput and resilience due to wake turbulence on arrival phase of flight and to

mitigate the impact of strong headwind conditions for instrument landing operations on parallel runways with centre lines spaced less than 760m (2500ft) apart by safely optimising the separation minima to be applied between aircraft pairs by time rather than distance, applied.

New Capabilities

Time based separation minima for aircraft pairs supporting the safe reduction of wake turbulence

which can be frequent in traffic applied during the simultaneous use of closely spaced parallel runways through the definition of new landing and go around procedures;

Mitigating the impact of strong headwind conditions;

System monitored real time wind and improved forecast meteo information.

Operational

Time based dependent parallel approaches

WAKE-B3/7

Description 		This element defines a dependent paired approach procedure to parallel runways, with centre lines spaced less than 760m (2500ft) apart, threshold staggers, and/or glide path height differences, under ILS Category I minimums, or the minimums depicted for an RNAV or LPV approach.

Aircraft are separated by time based wake turbulence separations rather than distance, based on the performance characteristics of the leading aircraft generating wake turbulence and the following aircraft that might encounter the wake turbulence.

The controller is provided with system support to separate aircraft with the goal of increasing runway throughput, enhancing resilience and mitigating the impact of strong headwinds,

Existing categorisation systems may be used to determine separation minima for types of aircraft pairs not specifically included in the pairwise separation matrix.

Maturity Level 	Concept

Human Factor Considerations

- Does it imply a change in task by a user or affected others? No

- Does it imply processing of new information by the user? Yes

Adaptation of working methods to separate based on time-based wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

- Does it imply the use of new equipment? Yes

Adaptation of working methods to separate based on time-based wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

- Does it imply a change to levels of automation? Yes

Adaptation of working methods to separate based on time-based wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

WAKE-B3/4 - Enhanced dependent parallel approaches

Relation-information need

AMET-B1/3 - Climatological and historical meteorological information

Relation-information benefit

AMET-B2/2 - Meteorological forecast and warning information

Relation-operational option

APTA-B0/3 - SBAS/GBAS CAT I precision approach procedures

Relation-operational option

APTA-B1/2 - PBN SID and STAR procedures (with advanced capabilities)

Relation-operational option

APTA-B2/2 - Simultaneous operations to parallel runways

Relation-technology option

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-technology option

NAVS-B0/1 - Ground Based Augmentation Systems (GBAS)

Relation-technology option

NAVS-B0/3 - Aircraft Based Augmentation Systems (ABAS)

Relation-operational benefit

SNET-B0/4 - Approach Path Monitoring (APM)

Relation-operational need

WAKE-B2/2 - Time based wake separation minima for final approach

Relation-information need

AMET-B2/1 - Meteorological observations information

### ENABLERS

Aircraft manufacturer

ANSP

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

MET

Information Service Provider

Airborne system capability

Aircraft system

Aircraft-derived wind to feed the ATC Separation delivery tool

Aircraft-derived wind speed and direction on final approach. References:

2024

Ground system infrastructur e

MET

instrument

Wind profile information to feed the ground- based ATC Separation delivery tool

Actual final approach wind profile information shall be provided to the TBS function, based on ground-based and/or aircraft measurements. References:

2024

MET

Information Service Provider

Regulatory provisions

National regulatory framework

National framework amendment for wake turbulence separation provision for staggered dependent parallel approaches based on TBS- PWS-A minima

National regulation amendment for wake turbulence separation provision for staggered dependent parallel approaches based on TBS-PWS-A minima.

References: DOC. 4444 – Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM) to be amended for allowing time-based separation between staggered dependent parallel approaches based on distance wake turbulence pair-wise minima

CAA 	2030

Regulatory provisions

National regulatory framework

Local TBS-PWS- A minima separation rules for staggered dependent parallel approaches

Local establishment of time-based pair- wise minima applicable to local traffic for staggered dependent parallel approache, converted from local reference air speed profiles per aircraft type over the distance separation minimum in calm wind (e.g.from analysis of surveillance data).

References:

ANSP   CAA 	2030

Operational procedures

Operations	ATC Procedure

to separate staggered dependent parallel approaches based on TBS- PWS-A minima

Design of local procedure, working methods and separation look-up tables for Approach and Tower Air Traffic Controllers to operate with ATC separation delivery function wake turbulence separation for staggered dependent parallel approaches based on time-based pair-wise minima. Refrences:

ANSP 	2030

Ground system infrastructur e

ATC systems ATC separation

delivery tool to operate staggered dependent parallel approaches based on wake turbulence longitudinal TBS- PWS-A minima

Processing and display on the approach and aerodrome controller’s ATC surveillance display systems of: - a separation delivery indicator (the distance separation minimum, or spacing constraint) applicable to a sequenced pair of aircraft, - a spacing aid indicator to assist Air Traffic Controllers for optimizing Separation Delivery taking into account predicted aircraft speed profiles (optional), - a safety alerting function of imminent risk of separation infringement or wrong aircraft being turned on to a separation indicator, fed by flight data, surveillance data, meteo data and separation rules. References: ICAO camera ready TBS

ANSP 	2030

Training	-	Training requirements for time based dependent parallel approaches

ATC Training to separate traffic under staggered dependent parallel approaches: Training of Approach and Tower Air Traffic Controllers on separation delivery using an TBS function, in nominal and degraded mode of operations.

ANSP 	2030

Ground system infrastructur e

Surveillance	Surveillance

capabilities for separation in arrivals and departures

Provide the necessary surveillance means to comply with applicable surveillance performance requirements.

ANSP 	2013

Ground system infrastructur e

Navigation	Navigation

Aircraft operator

capabilities for separation in arrivals and departures

Provide the necessary navigation capabilities (e.g. ILS for precision approach).

ANSP 	2013

Training	Awareness	Awareness

requirements for time based dependent parallel approaches

Flight Crew awareness of separation applicable under staggered dependent parallel approaches: Briefing material for Flight Crew awareness of the change in applicable wake separation minima.

2030

### DEPLOYMENT APPLICABILITY

#### Operational conditions:

Aerodromes with parallel runways having centrelines spaced less than 760 m (2500 feet) apart operating time based dependent parallel approaches with demand near to or exceeds peak capacity during periods of the operational day or overall daily capacity with a dense heterogeneous traffic mix resulting in arrival delay, and aerodromes where additional operational resilience and predictability is required to manage peak traffic in response to unplanned events.

| Airport operator | ANSP |  | Airspace user |
| --- | --- | --- | --- |
| ATM network function |  |  |  |

Main intended benefits:

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Reduce separation minima on approach | Airport operator ANSP Airspace user |
|  | Prevent reduction of arrival rate during headwind conditions |  |
|  | Increase peak capacity by using parallel RWY operations | Airport operator ANSP Airspace user |
| Indirect benefits | Efficiency (temporal efficiency, i.e. delay) | ANSP Airspace user |
|  | Resilience: additional headroom for spacing and sequence management | ANSP |

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Apply time-based separation instead of distance-based (arrival) | ++ | KPI10: Airport peak throughput |
| Capacity | Capacity, throughput & utilization | Improved categorisation of aircraft (arrival) | ++ | KPI10: Airport peak throughput |
| Capacity | Capacity, throughput & utilization | Reduce wake turbulence impact from parallel RWY during crosswind (arrival) | ++ | KPI10: Airport peak throughput |

Sixth edition of the GANP

Operational

Time based independent segregated parallel

operations

WAKE-B3/8

Main Purpose 	To improve runway throughput and resilience due to wake turbulence on arrival and departure phases of flight and to mitigate the impact of strong headwind conditions for segregated operations on parallel runways, with centre lines spaced less than 760m (2500ft) apart covering wake independent departure and arrival operations through the safe application of separation minima between aircraft pairs by time rather than distance.

New Capabilities 	 Time based separation minima for aircraft pairs supporting the safe reduction of wake turbulence which can be frequent in traffic, for independent arrivals and departures, applied during the simultaneous use of closely spaced parallel runways through the definition of new landing and go around procedures;

Mitigating the impact of strong headwind conditions;

System monitored real time wind and improved forecast meteo information.

Description 		This element defines an independent segregated parallel operation procedure to parallel runways, with centre lines spaced less than 760m (2500ft) apart under ILS Category I minimums, or the minimums depicted for an RNAV or LPV approach.

Aircraft are separated by time based wake turbulence separations rather than distance, based on the performance characteristics of the leading aircraft generating wake turbulence and the following aircraft that might encounter the wake turbulence.

The controller is provided with system support to separate aircraft with the goal of increasing runway throughput, enhancing resilience and mitigating the impact of strong headwinds,

Existing categorisation systems may be used to determine separation minima for types of aircraft pairs not specifically included in the pairwise separation matrix.

Maturity Level 	Concept

Human Factor Considerations

- Does it imply a change in task by a user or affected others? Yes

Adaptation of working methods to separate based on time-based wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

- Does it imply processing of new information by the user? Yes

Adaptation of working methods to separate based on time-based wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

- Does it imply the use of new equipment? Yes

Adaptation of working methods to separate based on time-based wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

- Does it imply a change to levels of automation? Yes

Adaptation of working methods to separate based on time-based wake turbulence pair-wise minima with ATC support tool with separation delivery indicator(s).

PLANNING LAYERS

Tactical-During ops

OPERATIONS

Departure   Arrival

DEPENDENCIES AND RELATIONS

Type of Dependencies

ASBU Element

Evolution

WAKE-B3/5 - Enhanced independent segregated parallel operations

Relation-information need

AMET-B1/3 - Climatological and historical meteorological information

Relation-operational need

WAKE-B2/2 - Time based wake separation minima for final approach

Relation-operational need

WAKE-B3/6 - Time based wake separation minima for departure based on

leader/follower static pair-wise

Relation-operational option

APTA-B0/3 - SBAS/GBAS CAT I precision approach procedures

Relation-operational option

APTA-B1/2 - PBN SID and STAR procedures (with advanced capabilities)

Relation-technology option

ASUR-B0/1 - Automatic Dependent Surveillance – Broadcast (ADS-B)

Relation-technology option

NAVS-B0/1 - Ground Based Augmentation Systems (GBAS)

Relation-technology option

NAVS-B0/3 - Aircraft Based Augmentation Systems (ABAS)

Relation-information need

AMET-B2/2 - Meteorological forecast and warning information

### ENABLERS

Aircraft manufacturer

ANSP

Enabler Category

Enabler Type	Enabler Name	Description / References	Stakeholders	Year

MET

Information Service Provider

Airborne system capability

Aircraft system

Aircraft-derived wind to feed the ATC Separation delivery tool

Aircraft-derived wind speed and direction on final approach. References:

2024

Ground system infrastructur e

MET

instrument

Wind profile information to feed the ground- based ATC Separation delivery tool

Actual final approach wind profile information shall be provided to the TBS function, based on ground-based and/or aircraft measurements. References:

2024

MET

Information Service Provider

Regulatory provisions

National regulatory framework

National framework amendment for wake- independent segregated parallel operations with time-based pair- wise minima

National regulation amendment for wake- independent segregated parallel operations with Time-based pair-wise minima. References: DOC. 4444 – Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM) to be amended for allowing wake independent time-based pair-wise separation between segregated approaches operations for closely spaced parallel runways

CAA 	2030

Regulatory provisions

National regulatory framework

Wake- independent segregated parallel operations with Time-based pair- wise minima

Local establishment of time-based pair- wise minima applicable to local traffic for independent segregated parallel operations, converted from local reference air speed profiles per aircraft type over the distance separation minimum in calm wind (e.g.from analysis of surveillance data)

ANSP   CAA 	2030

Operational procedures

Operations	ATC Procedure

for independent segregated parallel operations with time-based pair- wise minima

Design of local procedure, working methods and separation look-up tables for Approach and Tower Air Traffic Controllers to operate with ATC separation delivery function wake independent time-based pair-wise separation between segregated approaches operations for closely spaced parallel runways. References:

ANSP 	2030

Ground system infrastructur e

ATC systems ATC separation

delivery tool to operate Wake- independent segregated parallel operations based Time- based Pair-wise minima

Processing and display on the approach and aerodrome controller’s ATC surveillance display systems of: - a separation delivery indicator (the distance separation minimum, or spacing constraint) applicable to a sequenced pair of aircraft, - a spacing aid indicator to assist Air Traffic Controllers for optimising Separation Delivery taking into account predicted aircraft speed profiles (optional), - a safety alerting function of imminent risk of separation infringement or wrong aircraft being turned on to a separation indicator, fed by flight data, surveillance data, meteo data and separation rules. References:

ANSP 	2030

Training	-	Training requirements for time based independent segregated parallel operations

ATC Training to separate traffic unde Wake-independent segregated parallel operations based Time-based Pair-wise minima: Training of Approach and Tower Air Traffic Controllers on separation delivery using an TBS function, in nominal and degraded mode of operations.

ANSP 	2030

Ground system infrastructur e

Surveillance	Surveillance

capabilities for separation in arrivals and departures

Provide the necessary surveillance means to comply with applicable surveillance performance requirements.

ANSP 	2013

Ground system infrastructur e

Navigation	Navigation

capabilities for separation in arrivals and departures

Provide the necessary navigation capabilities (e.g. ILS for precision approach).

ANSP 	2013

DEPLOYMENT APPLICABILITY

Operational conditions:

Aerodromes with parallel runways having centrelines spaced less than 760 m (2500 feet) apart operating time based independent segregated parallel approaches with demand near to or exceeds peak capacity during periods of the operational day or overall daily capacity with a dense heterogeneous traffic mix resulting in arrival delay, and aerodromes where additional operational resilience and predictability is required to manage peak traffic in response to unplanned events.

Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| Airport operator | ANSP |  | Airspace user |
| --- | --- | --- | --- |
| ATM network function |  |  |  |

Training

Awareness	Awareness

requirements for time based independent segregated parallel operations

Flight Crew awareness of separation

applicable under wake-independent segregated parallel operations based Time-based Pair-wise minima: Briefing material for Flight Crew awareness of the change in applicable wake separation minima

2030

Aircraft operator

| Type | Operational description | Benefitting stakeholder(s) |
| --- | --- | --- |
| Direct benefits | Reduce separation minima independently on parallel approach and departure runways | Airport operator ANSP Airspace user |
|  | Prevent reduction of arrival rate during headwind conditions |  |
|  | Increase peak capacity | Airport operator ANSP Airspace user |
| Indirect benefits | Efficiency (temporal efficiency, i.e. delay) | ANSP Airspace user |
|  | Resilience: additional headroom for spacing and sequence management | ANSP |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Apply time-based separation instead of distance-based (arrival) | ++ | KPI10: Airport peak throughput |
| Capacity | Capacity, throughput & utilization | Improved categorisation of aircraft (arrival) | ++ | KPI10: Airport peak throughput |
| Capacity | Capacity, throughput & utilization | Reduce penalty caused by parallel dependent RWYs (departures) | ++ | KPI10: Airport peak throughput |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Capacity | Capacity, throughput & utilization | Reduce wake turbulence impact from parallel RWY during crosswind (arrival) | ++ | KPI10: Airport peak throughput |

WAKE-B4/1	En-route Wake Encounter Ground based

Prediction

Operational

Sixth edition of the GANP

Main Purpose New Capabilities Description

Maturity Level	-

Human Factor Considerations

### PLANNING LAYERS 		OPERATIONS

Tactical-During ops 	 En-route

### DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### ENABLERS

There are currently no enablers.

### DEPLOYMENT APPLICABILITY

Benefitting stakeholder(s)

Operational description

Type

Operational conditions: Main intended benefits:

### INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Avoid en-route wake vortex encounters | ++ | KPI20: Number of aircraft accidents |

WAKE-B4/2	En-Route Wake Encounter on-board flight management/mitigation

Operational

Sixth edition of the GANP

Main Purpose New Capabilities Description

Maturity Level	-

Human Factor Considerations

### PLANNING LAYERS 		OPERATIONS

Tactical-During ops 	 En-route

### DEPENDENCIES AND RELATIONS

There are currently no dependencies.

### ENABLERS

There are currently no enablers.

### DEPLOYMENT APPLICABILITY

Benefitting stakeholder(s)

Operational description

Type

Operational conditions: Main intended benefits:

INTENDED PERFORMANCE IMPACT ON SPECIFIC KPAS AND KPIS

| KPA | Focus Areas | Most specific performance objective(s) supported | KPI Impact | KPI |
| --- | --- | --- | --- | --- |
| Safety |  | Avoid en-route wake vortex encounters | ++ | KPI20: Number of aircraft accidents |